| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS#63 |
| Source:* | Peter Niblett, IBM |
| Date:* | 2024-02-26 |
| Reason for Change/s:* | Answer question raised by IANA expert |
| CR against: Release* | 5 |
| CR against: WI* | Active MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0008 v5 |
| Clauses * |  |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | n/a |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

In releases of TS-0008 up to and including R4 the oneM2M primitive parameters are mapped into CoAP options (the URI-Path, URI-Query or oneM2M-specific custom options).

When we applied to register the custom option numbers with IANA, the CoAP experts we talked to advised that our use of CoAP options could cause problems if the primitive parameter values were large, so this CR has been prepared to allow the Primitive Parameters to be carried in the CoAP message payload instead.

This change applies only from R5 onwards. In R4 and earlier the primitivs parameters will continue to be carried as CoAP options.

See SDS-2023-0177R07 for more background and a discussion of alternative approaches.

R01. Minor updates to the text for better clarity.

R02.

- Added text to 6.2.2.4 to explain the tradeoff between passing parameters as CoAP options vs including them in the payload.

- Proposed new numbers being used for the oneM2M-specific Content-Formats (10001 and 10002) that have now been assigned to other protocols.

- Simplified the RSC mapping by saying that the RSC is always required (previously you could omit it if you thought it could be uniquely deduced from the CoAP response code). Added a requirement that the RSC is always passed as a CoAP option (this means that we don’t need to introduce payloads in the non-blocking acknowledgements).

- Note: Figures 6.3.3-1 and 6.3.3-2 need some small corrections. These will be handled via an issue once this CR is approved.

### ********** Start of change 1 **********

## 2.1	Normative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

The following referenced documents are necessary for the application of the present document.

[1]	IETF RFC 7252: "The Constrained Application Protocol (CoAP)".

[2]	oneM2M TS-0004: " Service Layer Core Protocol Specification".

[3]	IETF RFC 7959: "Block-Wise Transfers in the Constrained Application Protocol (CoAP)".

[4]	oneM2M TS-0003: "Security solutions".

[5]	IETF RFC 6347: "Datagram Transport Layer Security Version 1.2".

[6]	IETF RFC 8132: "PATCH and FETCH Methods for the Constrained Application Protocol (CoAP)".

### ********** End of change 1 **********

### ********** Start of change 2 **********

## 5.1	Required Features

This clause explicitly specifies the required features of the CoAP layer for oneM2M to properly bind oneM2M primitives into CoAP messages:

- The 4-byte binary CoAP message header is defined in section 3 of IETF RFC 7252 [1].

- Confirmable (CON), Non-confirmable (NON), Acknowledgement (ACK) and Reset (RST) messages shall be supported. The Reset message is used to send an error message in response to a malformed Confirmable message in CoAP layer.

- GET, PUT, POST and DELETE methods shall be supported, as shall the FETCH method defined in IETF RFC 8132 [6]. oneM2M primitives map to these methods.

- The CoAP Response Codes specified in clause 6.2.4 shall be supported for oneM2M Response Status Code parameter mapping.

- The Uri-Host, Uri-Port, Uri-Path, and Uri-Query shall be supported.

- The Content-Type Option shall be used to indicate the media types of the payload.

- Block-wise transfers feature may be supported to carry large payloads.

- The Caching feature may be supported.

### ********** End of change 2 **********

### ********** Start of change 3**********

### 6.2.1	Header

This clause specifies how to configure CoAP header information:

- The Version field shall be configured as 1.

- The Type field shall be configured according to clause 6.3. The Reset message is used to indicate an error in response to a malformed message in the CoAP layer.

- In case of a request, the Code field indicates the CoAP Method. If the oneM2M operation is sent as a Blocking request the oneM2M Operation parameter shall be mapped to a CoAP Method according to the table 6.2.1-1. In non-blocking and flex blocking cases, the request shall use the CoAP POST method, and the Operation parameter shall be mapped as described in clause 6.2.2.3.

- In case of a response, the Code field indicates the CoAP Response Code. The oneM2M Response Status Code parameter shall be mapped to a CoAP Response Code as specified in clause 6.2.4.

- The Originator and Receiver shall set the 16 bit MessageId in accordance with the CoAP specification [1] and shall retry transmission of all unacknowledged Confirmable messages, as required by that specification.

Table 6.2.1-1: oneM2M Operation Parameter Mapping

| oneM2M Operation Parameter | CoAP Method | CoAP Method Code |
| --- | --- | --- |
| CREATE | POST | 0.02 |
| RETRIEVE | FETCH or GET | 0.05 (FETCH) or 0.01 (GET) |
| UPDATE | PUT | 0.03 |
| DELETE | DELETE | 0.04 |
| NOTIFY | POST | 0.02 |

The Originator shall use the GET method when sending a RETRIEVE request to a Receiver that is running Release 4 or earlier, otherwise it shall use FETCH as defined in RFC 8132 [6]. See clause 6.6 for more details on the use of FETCH.

At the Receiver, a CoAP request message with a POST method that does not carry an Operation parameter shall be mapped to a oneM2M CREATE or NOTIFY operation in accordance with the existence of the Resource Type parameter. If a Resource Type parameter exists then the value of the Operation parameter is CREATE and if the Resource Type parameter does not exist, the value of the Operation parameter is NOTIFY.

### ********** End of change 3 **********

### ********** Start of change 4**********

#### 6.2.2.2	Content Format Negotiation Options

The CoAP Accept Option may be used to indicate which Content-Format is acceptable to an Originator. If a Hosting CSE supports the Content-Format specified in Accept Option of the request, the Hosting CSE shall respond with that Content-Format. If the Hosting CSE does not support the Content-Format specified in Accept Option of the request, 4.06 "Not Acceptable" shall be sent as a response, unless another error code takes precedence for this response.

Possible values for Content-Format and Accept options are listed below:

- application/xml (41);

- application/json (50);

- application/cbor (60);

- media types specified in clause 6.7 "oneM2M specific MIME media types" of oneM2M TS-0004 [2].

Numeric values for oneM2M defined media types are listed in table 6.2.2.2-1.

Table 6.2.2.2-1: CoAP oneM2M Specific Content-Formats

| oneM2M Specific Media Type | ID |
| --- | --- |
| application/vnd.onem2m-res+xml | 10014 |
| application/vnd.onem2m-res+json | 10015 |
| application/vnd.onem2m-ntfy+xml | 10016 |
| application/vnd.onem2m-ntfy+json | 10003 |
| application/vnd.onem2m-preq+xml | 10006 |
| application/vnd.onem2m-preq+json | 10007 |
| application/vnd.onem2m-prsp+xml | 10008 |
| application/vnd.onem2m-prsp+json | 10009 |
| application/vnd.onem2m-res+cbor | 10010 |
| application/vnd.onem2m-ntfy+cbor | 10011 |
| application/vnd.onem2m-preq+cbor | 10012 |
| application/vnd.onem2m-prsp+cbor | 10013 |

#### 6.2.2.3	URI Options

This clause describes how to configure CoAP Uri-Host, Uri-Port, Uri-Path, and Uri-Query Options.

Host and port part of the address specified in pointOfAccess attribute of <remoteCSE> resource shall be mapped to Uri-Host and Uri-Port respectively.

If To parameter contains absolute format, then the first URI-Path Option shall contain a letter "_" and map To parameter removing starting "//" into next URI-Path Option(s).

If To parameter contains SP-relative format, then the first URI-Path Option shall contain a letter "~" and map To parameter removing starting "/" into next URI-Path Option(s).

If To parameter contains CSE-relative format, then To parameter shall be mapped to URI-Path Option(s).

Table 6.2.2.3-1 shows valid mappings between the To request primitive parameter and the Uri-Path of the CoAP.

CSEBase represents the resource name of a <CSEBase> resource, CSEBase/ae12/cont27/contInst696 represents a structured CSE-relative resource ID, and cin00856 an unstructured CSE-relative resource ID.

Table 6.2.2.3-1: Mapping examples between To parameter and Uri-Path of the CoAP

| Method |  | Request Scope |  |  |
| --- | --- | --- | --- | --- |
|  |  | CSE-Relative | SP-Relative | Absolute |
| Structured | To | CSEBase/ae12/cont27/contInst696 | /CSE178/CSEBase/ae12/cont27/contInst696 | //mym2msp.org/CSE178/CSEBase/ae12/cont27/contInst696 |
|  | Uri-Path | CSEBase |  | _ |
|  |  |  | ~ | mym2msp.org |
|  |  |  | CSE178 | CSE178 |
|  |  |  | CSEBase | CSEBase |
|  |  | ae12 | ae12 | ae12 |
|  |  | cont27 | cont27 | cont27 |
|  |  | contInst696 | contInst696 | contInst696 |
| Unstructured | To | cin00856 | /CSE178/cin00856 | //mym2msp.org/CSE178/cin00856 |
|  | Uri-Path | cin00856 |  | _ |
|  |  |  | ~ | mym2msp.org |
|  |  |  | CSE178 | CSE178 |
|  |  |  | cin00856 | cin00856 |
| NOTE: How to read this table: To primitive - from left to right, Uri-Path - from top to bottom. |  |  |  |  |

If either the Originator or the Receiver is running as oneM2M Release 4 or earlier the responseTypeValue element of Response Type, the elements of Filter Criteria and the Result Persistence, Delivery Aggregation, Result Content, Desired Identifier Result Type, Token Request Indicator, Tokens, Token IDs, Local Token IDs,  Role IDs, Authorization Relationship Indicator, Authorization Signature Indicator, Semantic Query Indicator and Operation parameters shall be carried, if needed, in the Uri-Query Option in a short name form as specified in clause 8.2.2 of oneM2M TS-0004 [2]. If neither the Originator nor the Receiver are Release 4 or earlier these elements and parameters, if needed, shall be carried in the CoAP payload as described in clause 6.3.

#### 6.2.2.4	Definition of New Options

##### 6.2.2.4.0	Introduction

This clause describes new CoAP Options used for binding several oneM2M request/response parameters. Table 6.2.2.4.0-1 contains definitions of the new CoAP Options and sub-clauses specify oneM2M parameter mapping with the newly-defined CoAP Options in the table 6.2.2.4.0-1.

Table 6.2.2.4.0-1: Definition of New Options

| No | C | U | N | R | Name | Format | Length | Default |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 279 | X | X |  |  | oneM2M-FR | string | 0-255 | (None) |
| 283 | X | X |  |  | oneM2M-RQI | string | 0-255 | (None) |
| 259 | X | X |  |  | oneM2M-OT | string | 15 | (None) |
| 291 | X | X |  |  | oneM2M-RQET | string | 15 | (None) |
| 295 | X | X |  |  | oneM2M-RSET | string | 15 | (None) |
| 299 | X | X |  |  | oneM2M-OET | string | 15 | (None) |
| 263 | X | X |  |  | oneM2M-RTURI | string | 0-255 | (None) |
| 303 | X | X |  |  | oneM2M-EC | uint | 1 | (None) |
| 307 | X | X |  |  | oneM2M-RSC | uint | 2 | (None) |
| 311 | X | X |  |  | oneM2M-GID | string | 0-255 | (None) |
| 267 | X | X |  |  | oneM2M-TY | uint | 2 | (None) |
| 319 | X | X |  |  | oneM2M-CTO | uint | 2 | (None) |
| 323 | X | X |  |  | oneM2M-CTS | uint | 2 | (None) |
| 327 | X | X |  |  | oneM2M-ATI | string | 0-255 | (None) |
| 271 | X | X |  |  | oneM2M-RVI | string | 1-2 | (None) |
| 331 | X | X |  |  | oneM2M-VSI | string | 0-255 | (None) |
| 335 | X | X |  |  | oneM2M-GTM | string | 0-512 | (None) |
| 339 | X | X |  |  | oneM2M-AUS | string | 0-255 | (None) |
| 275 | X | X |  |  | oneM2M-ASRI | string | 0-255 | (None) |
| 343 | X | X |  |  | oneM2M-OMR | string | 0-255 | (None) |
| 347 | X | X |  |  | oneM2M-PRPI | string | 0-255 | (None) |
| 351 | X | X |  |  | oneM2M-MSU | string | 0-255 | (None) |
| NOTE 1: C, U, N, R means Critical, Unsafe, NoCacheKey and Repeatable respectively [1]. Table 6.2.2.4.0-1 follows the template used in clause 5.10 Option Definitions of CoAP specification [1]. NOTE 2: CoAP Option numbers specified in table 6.2.2.4.0-1 are subject to change after review by IANA registration. |  |  |  |  |  |  |  |  |

If either the Originator or Receiver is using oneM2M release 4 or earlier the oneM2M primitive parameters listed in the subclauses below shall be mapped to the CoAP options shown in table 6.2.2.4.0-1.

The Response Status Code, if present, shall be carried using the CoAP option shown in table 6.2.2.4.0-1 regardless of the release level being used by Originator or Receiver.

If both Originator and Receiver are release 5 or later the following parameters shall be carried in the serialised primitive included in the CoAP payload as described in clause 6.3. The corresponding CoAP options shown in table 6.2.2.4.0-1 shall not be used:

- From

- The notificationURI of Response Type

- Group Request Identifier

- Assigned Token Identifiers

- Vendor Information

- Group Request Target Members

- Authorization Signatures

- Authorization Signature Request Information

- Ontology Mapping Resources

- Primitive Profile Identifier

- M2M Service User

If both Originator and Receiver are release 5 or later any parameter in the list below can be carried either using the CoAP option shown in table 6.2.2.4.0-1 or in the CoAP payload as described in clause 6.3.

- The payload approach shall be used if the parameter value is longer than the maximum length permitted for the CoAP option (see the length column in table 6.2.2.4.0-1)

- The CoAP option approach gives a more efficient encoding if the whole primitive is small enough to fit in a single CoAP message, however it is less efficient if the primitive has to be split into multiple blocks (see clause 6.5) as COAP options have to be repeated in each block.

- Request Identifier

- Originating Timestamp

- Request Expiration Timestamp

- Result Expiration Timestamp

- Operation Execution Time

- Event Category

- Resource Type

- Content Offset

- Content Status

- Release Version Indicator

6.2.2.4.1	From

The From parameter shall be mapped to the oneM2M-FR Option.

##### 6.2.2.4.2	Request Identifier

The Request Identifier parameter shall be mapped to the oneM2M-RQI Option.

##### 6.2.2.4.3	Void

##### 6.2.2.4.4	Originating Timestamp

The Originating Timestamp parameter shall be mapped to the oneM2M-OT Option.

##### 6.2.2.4.5	Request Expiration Timestamp

The Request Expiration Timestamp parameter shall be mapped to the oneM2M-RQET Option.

##### 6.2.2.4.6	Result Expiration Timestamp

The Result Expiration Timestamp parameter shall be mapped to the oneM2M-RSET Option.

##### 6.2.2.4.7	Operation Execution Time

The Operation Execution Time parameter shall be mapped to the oneM2M-OET Option.

##### 6.2.2.4.8	notificationURI of Response Type

The notificationURI element of Response Type parameter shall be mapped to the oneM2M-RTURI Option.

##### 6.2.2.4.9	Event Category

The Event Category parameter shall be mapped to the oneM2M-EC Option.

##### 6.2.2.4.10	Response Status Code

The Response Status Code parameter shall be mapped to the oneM2M-RSC Option.

##### 6.2.2.4.11	Group Request Identifier

The Group Request Identifier parameter shall be mapped to the oneM2M-GID Option.

##### 6.2.2.4.12	Resource Type

The Resource Type parameter shall be mapped to the oneM2M-TY Option.

##### 6.2.2.4.13	Content Offset

The Content Offset parameter shall be mapped to the oneM2M-CTO Option.

##### 6.2.2.4.14	Content Status

The Content Status parameter shall be mapped to the oneM2M-CTS Option.

##### 6.2.2.4.15	Assigned Token Identifiers

The Assigned Token Identifiers parameter shall be mapped to the oneM2M-ATI Option. The format of the oneM2M-ATI option shall be represented as a sequence of lti-value:tkid-value pairs separated by a colon ':' and multiple pairs separated by a '+' character.

EXAMPLE:	The option looks as follows:

oneM2M-ATI: lti-value1:tkid-value1 + lti-value2:tkid-value2 + …

if the XML representation of the Assigned Token Identifiers parameter is given as (using short element names):

<ati>

<ltia>

<lti>lti-value1</lti>

<tkid>tkid-value1</tkid>

</ltia>

<ltia>

<lti>lti-value2</lti>

<tkid>tkid-value2</tkid>

</ltia>

…

</ati>

The data type m2m:dynAuthlocalTokenIdAssignments of the Assigned Token Identifiers parameter is defined in clause 6.3.5.43 of oneM2M TS-0004 [2].

##### 6.2.2.4.16	Release Version Indicator

The Release Version Indicator parameter shall be mapped to the oneM2M-RVI Option.

##### 6.2.2.4.17	Vendor Information

The Vendor Information parameter shall be mapped to the oneM2M-VSI Option.

##### 6.2.2.4.18	Group Request Target Members

The Group Request Target Members parameter shall be mapped to the oneM2M-GTM Option.

##### 6.2.2.4.19	Authorization Signatures

The Authorization Signatures parameter shall be mapped to the oneM2M-AUS Option.

##### 6.2.2.4.20	Authorization Signature Request Information

The Authorization Signature Request Information parameter shall be mapped to the oneM2M-ASRI Option.

##### 6.2.2.4.21	Ontology Mapping Resources

The Ontology Mapping Resources parameter shall be mapped to the oneM2M-OMR Option. The format of the oneM2M-OMR option shall be represented as a sequence of oneM2M resource identifiers separated by a '+'.

EXAMPLE:	The option looks as follows:

oneM2M-OMR: /IN-CSE-0001/omr1+/IN-CSE-0001/omr2+…

##### 6.2.2.4.22	Primitive Profile Identifier

The Primitive Profile Identifier parameter shall be mapped to the oneM2M-PRPI Option.

##### 6.2.2.4.22	M2M Service User

The M2M Service User parameter shall be mapped to the oneM2M-MSU Option.

### 6.2.3	Payload

If both the Originator and Receiver are using oneM2M Release 5 or later the CoAP payload shall contain the Request or Response primitive serialised using the rules given in clause 8 of oneM2M TS-0004 [2] applied to m2m:requestPrimitive defined in clause 6.4.1 of oneM2M TS-0004 [2] or the m2m:responsePrimitive defined in clause 6.4.1 of oneM2M TS-0004 [2].

- Clause 6.2.2 requires some Primitive Parameters (e.g. the To and Response Status Code parameters) to be mapped to CoAP Options and it permits some other parameters to be passed as CoAP options. If a parameter is passed as a CoAP option it shall not be included in the serialized request or response primitive.

If either the Originator or Receiver is using oneM2M Release 4 or earlier the CoAP payload shall contain the value of the Token Request Information parameter (if present in a response primitive) otherwise it shall contain the value of the Content parameter (if present). All other Primitive Parameters that are needed shall be passed as CoAP options, as described in clause 6.2.2.

The CoAP Blockwise transfer mechanism shall be used to deliver the payload if it is too large to fit into one CoAP message. Refer to clause 6.5 for more information.

If the Content parameter contains URI and resource representation in a response to a create request, the URI shall be mapped to the CoAP Location-Path Option instead of being included in the CoAP payload.

The CoAP Content-Format option shall be included and shall be set to a value that complies with the data representation.

6.2.4	Response Codes Mapping

Table 6.2.4-1 defines the mapping between oneM2M Response Status Code parameter specified in [2] and CoAP Response Code that is referenced in clause 6.2.1.

Table 6.2.4-1: Mapping between oneM2M Response Status Code and CoAP Response Code

### ********** End of change 4 **********

### ********** Start of change 5**********

## 6.6	Use of the CoAP FETCH method

When both Originator and Receiver are using oneM2M Release 5 or later all Blocking RETRIEVE requests shall be sent using the CoAP FETCH method defined in RFC 8132 [6]. The FETCH method differs from the GET method in that it contains a CoAP payload.

The CoAP payload shall be set to contain the oneM2M Request primitive, serialized as described in clause 6.2.2.

RFC 8132 requires that the CoAP Content-Format option be present in the request to indicate the nature of the payload. The Content-Format shall be set to one of the oneM2M-specific MIME media types values defined in clause 6.2.2.2. The value to use depends on the encoding used for the request as follows:

- application/vnd.onem2m-preq+xml 	if the request is encoded using XML

- application/vnd.onem2m-preq+json 	if the request is encoded using JSON

- application/vnd.onem2m-preq+cbor 	if the request is encoded using CBOR

### ********** End of change 5 **********

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?