| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS 65 |
| Source:* | Rahul Jha, C-DOT, rahulj@cdot.in Rahul Kumar, C-DOT, krahul@cdot.in Anupama, C-DOT, anupama@cdot.in Poornima, C-DOT, poornima@cdot.in |
| Date:* | 2024-06-24 |
| Reason for Change/s:* | See the Introduction |
| CR against: Release* | Rel 5 |
| CR against: WI* | Active <Work Item number> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0001, V5.5.0 |
| Clauses * | 9.6.19, 9.6.68 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Other TS/TR(s) impacted | TS-0001, V4.23.0 TS-0004, V4.21.0 |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2020 (do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

If this is  a correction, and the change applies to previous releases, a separate “mirror CR” should be posted at the same time as this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made e.g. a change impacting 5 tables should not only include a proposal to change only 3 tables. Include any changes to references, definitions, and abbreviations in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar.

Use change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the proposed new clause is located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to the content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

The <m2mServiceSubscriptionProfile> resource defines the maxNumContainers, maxNumFlexContainers but does not define the current number of <container> and <flexContainer> resources to restrict the resources because of which we will not be able to check if the service subscription limits have been exceeded. Thus this change request defines the currentNumContainers and currentNumFlexContainers attribute to keep the count of <container> and <flexContainer> resource at any moment in time. Thus enforcing the service subscription limits.

The <subscription> resource is an widely used and essential resource and thus also needs to be restricted. Hence this change request also defines the currentNumSubscriptions attribute to keep the count of <subscriptions> resource and maxNumSubscriptions attribute which is the service subscription limit of <subscription> resource. Thus enforcing the service subscription limits on <subscription> resource.

This CR proposes to add following attributes to <m2mServiceSubscriptionProfile> resource :

- currentNumContainers

- currentNumFlexContainers

- currentNumSubscriptions

- maxNumSubscriptions

This CR proposes to add following attributes to the <serviceSubscribedUserProfile> resource :

- maxNumSubscriptions

R01 includes the following changes :

  - The <serviceSubscribedUserProfile> should also contains maxNumSubscriptions attribute as service subscription limits on the <subscription> resource is needed.

R02 includes following changes :

This revision proposes to change the mentions of “Hosting CSE” in attribute definitions to “IN-CSE” as from the start the intend was to make the <m2mServiceSubscriptionProfile> and <serviceSubscribedUserProfile> resource for global management and thus to reside on IN-CSE.

### -----------------------Start of change 1-------------------------------------------

### 9.6.19	Resource Type m2mServiceSubscriptionProfile

The <m2mServiceSubscriptionProfile> resource represents an M2M Service Subscription. It is used to represent all data pertaining to the M2M Service Subscription, i.e. the technical part of the contract between an M2M Application Service Provider and an M2M Service Provider and is only stored on IN-CSE. The data is also represented in <serviceSubscribedNode>, <serviceSubscribedAppRule> and <serviceSubscribedUserProfile> resources as well as <m2mServiceSubscriptionProfile> resource. The relationship among those resource types is depicted as follows. Note that the diagram does not capture all attributes and child resources. Those resource types shall only be instantiated on IN-CSE.

Figure 9.6.19-1: Relationship among M2M Service Subscription related resources

The <m2mServiceSubscriptionProfile> resource shall contain the child resources specified in table 9.6.19-1.

Table 9.6.19-1: Child resources of <m2mServiceSubscriptionProfile> resource

| Child Resources of <m2mServiceSubscriptionProfile> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 |
| [variable] | <serviceSubscribedNode> | 0..n | See clause 9.6.20 |
| [variable] | <transaction> | 0..n | See clause 9.6.48 |
| [variable] | <serviceSubscribedUserProfile> | 0..n | See clause 9.6.68 |

The <m2mServiceSubscriptionProfile> resource shall contain the attributes specified in table 9.6.19-2.

Table 9.6.19-2: Attributes of <m2mServiceSubscriptionProfile> resource

| Attributes of <m2mServiceSubscriptionProfile> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3. |
| resourceID | 1 | RO | See clause 9.6.1.3. |
| resourceName | 1 | WO | See clause 9.6.1.3. |
| parentID | 1 | RO | See clause 9.6.1.3. |
| expirationTime | 1 | RW | See clause 9.6.1.3. |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3. |
| creationTime | 1 | RO | See clause 9.6.1.3. |
| labels | 0..1 (L) | RW | See clause 9.6.1.3. |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3. |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See clause 9.6.1.3. |
| custodian | 0..1 | RW | See clause 9.6.1.3. |
| M2M-Sub-ID | 1 | WO | The identifier assigned by the M2M Service Provder for this M2M Service Subscription. |
| M2M-SS-ID | 0..1 | RW | The identifier assigned by the M2M Service Provder to the M2M Service Subscriber associated with this M2M Service Subscription. |
| status | 1 | RO | Indicates the active/inactive status of this M2M Service Subscription. The IN-CSE shall set this attribute to "ACTIVE" when the value of activate attribute is set to TRUE and "INACTIVE" when the value of the activate attribute is set to FALSE. |
| activate | 1 | RW | Used to activate or deactivate this M2M Service Subscription. When this attribute is set to a value of TRUE, the IN-CSE shall activate this M2M Service Subscription. When this attribute is set to a value of FALSE, the IN-CSE shall deactivate this M2M Service Subscription. The default value of the attribute is FALSE. The attribute shall be set to value of FALSE by the IN-CSE on end of serviceSubscriptionDuration. To activate the service subscription again, this attribute shall be set to a value of TRUE. |
| activationTime | 0..1 | RO | M2M Service Subscription Activation Time. It is the time from which an M2M Service Provider activates the services of an M2M subscriber. The value of this attribute shall be set to current time when activate is set to TRUE. |
| deactivationTime | 0..1 | RO | M2M Service Subscription Deactivation Time. It is the time on which an M2M Service Provider deactivates the services of an M2M subscriber. The value of this attribute shall set to the time when activate transition from TRUE to FALSE. |
| serviceSubscriptionDuration | 0..1 | RW | It indicates the duration of the contract between an M2M Subscriber and M2M Service Provider. When a service subscription reaches the end of this duration, the IN-CSE shall set the activate attribute to a value of FALSE. When an ACTIVE service subscription is deactivated by setting activate to FALSE, the IN-CSE shall recalculate the duration of time for which the service subscription was active and update the value of this attribute. |
| currentNumAEs | 1 | RO | For this M2M Service Subscription, the IN-CSE shall keep track of the current number of <AE> resources created . |
| maxNumAEs | 0..1 | RW | For this M2M Service Subscription, the IN-CSE shall limit the maximum number of <AE> resources created to this limit, if configured. If a request is received to create an <AE> resource once this limit is reached, the IN-CSE shall return an error indicating that the maximum number of allowed <AE> resources has been reached. |
| currentNumNodes | 1 | RO | For this M2M Service Subscription, the IN-CSE shall keep track of the current number of <node> resources created . |
| maxNumNodes | 0..1 | RW | For this M2M Service Subscription, the IN-CSE shall limit the maximum number of <node> resources created to this limit, if configured. If a request is received to create a <node> resource once this limit is reached, the IN-CSE shall return an error indicating the maximum number of allowed <node> resources has been reached. |
| maxNumBytes | 0..1 | RW | For this M2M Service Subscription, the IN-CSE shall limit the aggregate amount of bytes stored to this limit, if configured. This limit shall be applied to Content Sharing Resources only (i.e. <container>, <contentInstance>, <flexContainer>, <flexContainerInstance>, <timeSeries> and <timeSeriesInstance>). If a request is received to create a Content Sharing Resource that will result in exceeding this limit, the IN-CSE shall return an error that the maximum number of allowed bytes has been reached. A IN-CSE may take the value of the maxNumBytes attribute as well as other attributes (e.g. maxNumContainers, maxNumContentInstances, etc.) into account when determining a limit to enforce on the maxByteSize attribute of individual Content Sharing Resources, if applicable. How a IN-CSE allocates the maxNumBytes across individual Content Sharing Resources is based on local policy. |
| maxNumUsers | 0..1 | RW | Maximum number of allowed M2M Service Users associated with this M2M Service Subscription. |
| maxRequestRate | 0..1 | RW | If a IN-CSE receives requests for this M2M Service Subscription at a rate which exceeds the limit defined by this attribute, the IN-CSE may throttle the processing of the requests. The IN-CSE may throttle processing of requests by delaying responses and/or by returning an error indicating that the maximum allowed request rate has been reached. |
| currentNumContainers | 1 | RO | For this M2M Service Subscription, the IN-CSE shall keep track of the current number of <container> resources created . |
| maxNumContainers | 0..1 | RW | For this M2M Service Subscription, the IN-CSE shall limit the maximum number of <container> resources created to this limit, if configured. If a request is received to create a <container> resource once this limit is reached, the IN-CSE shall return an error indicating the maximum number of allowed <container> resources has been reached. |
| maxNumInstancesPerContainer | 0..1 | RW | For this M2M Service Subscription, the IN-CSE shall limit the value of the maxNrOfInstances attribute of a <container> resource to this limit, if configured. |
| maxNumTimeSeries | 0..1 | RW | For this M2M Service Subscription, the IN-CSE shall limit the maximum number of <timeSeries> resources created to this limit, if configured. If a request is received to create a <timeSeries> resource once this limit is reached, the IN-CSE shall return an error indicating the maximum number of <timeSeries> resources has been reached. |
| maxNumInstancesPerTimeSeries | 0..1 | RW | For this M2M Service Subscription, the IN-CSE shall limit the value of the maxNrOfInstances attribute of a <timeSeries> resource to this limit, if configured. |
| maxMembersPerGroup | 0..1 | RW | For this M2M Service Subscription, the IN-CSE shall limit the value of the maxNrOfMembers attribute of a <group> resource to this limit, if configured. |
| maxNotificationRate | 0..1 |  | The IN-CSE shall restrict the maximum rate of notifications for this M2M Service Subscription to this limit, if configured. A IN-CSE shall also restrict the maximum value of the rateLimit attribute of <subscription> resources associated with this M2M Service Subscription to this limit,if configured. |
| currentNumFlexContainers | 1 | RO | For this M2M Service Subscription, the IN-CSE shall keep track of the current number of <flexContainer> resources created . |
| maxNumFlexContainers | 0..1 | RW | For this M2M Service Subscription, the IN-CSE shall limit the maximum number of <flexContainer> resources created to this limit, if configured. If a request is received to create a <flexContainer> resource once this limit is reached, the IN-CSE shall return an error indicating the maximum number of allowed <flexContainer> resources has been reached. |
| maxNumInstancesPerFlexContainer | 0..1 | RW | For this M2M Service Subscription, the IN-CSE shall limit the value of the maxNrOfInstances attribute of a <flexContainer> resource to this limit, if configured. |
| currentNumSubscriptions | 1 | RO | For this M2M Service Subscription, the IN-CSE shall keep track of the current number of <subscription> resources created . |
| maxNumSubscriptions | 0..1 | RW | For this M2M Service Subscription, the IN-CSE shall limit the maximum number of <subscription> resources created to this limit, if configured. If a request is received to create a <subscription> resource once this limit is reached, the IN-CSE shall return an error indicating the maximum number of allowed <subscription> resources has been reached. |
| defaultAccessControlPrivileges | 0..1(L) | RW | A list of resource identifiers of default access control policies to link to a resource if/when the accessControlPolicyIDs attribute of a resource created by an entity associated with this M2M Service Subscription is not configured. |

### -----------------------End of change 1---------------------------------------------

### -----------------------Start of change 2---------------------------------------------

### 9.6.68	Resource Type serviceSubscribedUserProfile

The <serviceSubscribedUserProfile> resource contains user profile information for a given M2M Service User such as its M2M-User-ID. A oneM2M <m2mServiceSubscriptionProfile> resource shall have a <serviceSubscribedUserProfile> child resource for each authorized M2M Service User associated with the M2M Service Subscription. A <serviceSubscribedAppRule> resource may have an allowedUsers attribute that can be configured with a list of <serviceSubscribedUserProfile> resource identifiers, wherein the allowedUsers represents the users that are allowed to originate requests via the allowed AEs specified by the <serviceSubscribedAppRule>.

The <serviceSubscribedUserProfile> resource shall contain the child resources specified in table 9.6.68-1.

Table 9.6.68-1: Child resources of <serviceSubscribedUserProfile> resource

| Child Resources of <serviceSubscribedUserProfile> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <subscription> | 0..n | See clause 9.6.8 |
| [variable] | <transaction> | 0..n | See clause 9.6.48 |

The <serviceSubscribedUserProfile> resource shall contain the attributes specified in table 9.6.68-2.

Table 9.6.68-2: Attributes of <serviceSubscribedUserProfile> resource

| Attributes of <serviceSubscribedUserProfile> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3. |
| resourceID | 1 | RO | See clause 9.6.1.3. |
| resourceName | 1 | WO | See clause 9.6.1.3. |
| parentID | 1 | RO | See clause 9.6.1.3. |
| expirationTime | 1 | RW | See clause 9.6.1.3. |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3. |
| creationTime | 1 | RO | See clause 9.6.1.3. |
| labels | 0..1 (L) | RW | See clause 9.6.1.3. |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3. |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See clause 9.6.1.3. |
| custodian | 0..1 | RW | See clause 9.6.1.3. |
| M2M-User-ID | 1 | RW | The M2M-User-ID that is assigned to this M2M Service User. |
| maxNumAEs | 0..1 | RW | For this M2M Service User, the IN-CSE shall limit the maximum number of <AE> resources createdto this limit, if configured. If a request is received to create an <AE> resource once this limit is reached, the IN-CSE shall return an error indicating that the maximum number of allowed <AE> resources has been reached. |
| maxNumNodes | 0..1 | RW | For this M2M Service User, the IN-CSE shall limit the maximum number of <node> resources createdto this limit, if configured. If a request is received to create a <node> resource once this limit is reached, the IN-CSE shall return an error indicating the maximum number of allowed <node> resources has been reached. |
| maxNumBytes | 0..1 | RW | For this M2M Service User, the IN-CSE shall limit the aggregate amount of bytes stored to this limit, if configured. This limit shall be applied to Content Sharing Resources only (i.e. <container>, <contentInstance>, <flexContainer>, <flexContainerInstance>, <timeSeries> and <timeSeriesInstance>). If a request is received to create a Content Sharing Resource that will result in exceeding this limit, the IN-CSE shall return an error that the maximum number of allowed bytes has been reached. A IN-CSE may take the value of the maxNumBytes attribute as well as other attributes (e.g. maxNumContainers, maxNumContentInstances, etc.) into account when determining a limit to enforce on the maxByteSize attribute of individual Content Sharing Resources, if applicable. How a IN-CSE allocates the maxNumBytes across individual Content Sharing Resources is based on local policy. |
| maxRequestRate | 0..1 | RW | If a IN-CSE receives requests from this M2M Service User at a rate which exceeds the limit defined by this attribute, the IN-CSE may throttle the processing of the requests. The IN-CSE may throttle processing of requests by delaying responses and/or by returning an error indicating that the maximum allowed request rate has been reached. |
| maxNumContainers | 0..1 | RW | For this M2M Service User, the IN-CSE shall limit the maximum number of <container> resources created to this limit, if configured. If a request is received to create a <container> resource once this limit is reached, the IN-CSE shall return an error indicating the maximum number of allowed <container> resources has been reached. |
| maxNumInstancesPerContainer | 0..1 | RW | For this M2M Service User, the IN-CSE shall limit the value of the maxNrOfInstances attribute of a <container> resource to this limit, if configured. |
| maxNumTimeSeries | 0..1 | RW | For this M2M Service User, the IN-CSE shall limit the maximum number of <timeSeries> resources created to this limit, if configured. If a request is received to create a <timeSeries> resource once this limit is reached, the IN-CSE shall return an error indicating the maximum number of <timeSeries> resources has been reached. |
| maxNumInstancesPerTimeSeries | 0..1 | RW | For this M2M Service User, the IN-CSE shall limit the value of the maxNrOfInstances attribute of a <timeSeries> resource to this limit, if configured. |
| maxMembersPerGroup | 0..1 | RW | For this M2M Service User, the IN-CSE shall limit the value of the maxNrOfMembers attribute of a <group> resource to this limit, if configured. |
| maxNotificationRate | 0..1 | RW | The IN-CSE shall restrict the maximum rate of notifications sent to this M2M Service User to this limit, if configured. A IN-CSE shall also restrict the maximum value of the rateLimit attribute of a <subscription> resource for this M2M Service User to this limit, if configured. |
| maxNumFlexContainers | 0..1 | RW | For this M2M Service User, the IN-CSE shall limit the maximum number of <flexContainer> resources created to this limit, if configured. If a request is received to create a <flexContainer> resource once this limit is reached, the IN-CSE shall return an error indicating the maximum number of allowed <flexContainer> resources has been reached. |
| maxNumSubscriptions | 0..1 | RW | For this M2M Service Subscription, the IN-CSE shall limit the maximum number of <subscription> resources created to this limit, if configured. If a request is received to create a <subscription> resource once this limit is reached, the IN-CSE shall return an error indicating the maximum number of allowed <subscription> resources has been reached. |
| maxNumInstancesPerFlexContainer | 0..1 | RW | For this M2M Service User, the IN-CSE shall limit the value of the maxNrOfInstances attribute of a <flexContainer> resource to this limit, if configured. |
| defaultAccessControlPrivileges | 0..1(L) | RW | A list of resource identifiers of default access control policies to link to a resource if/when the accessControlPolicyIDs attribute of a resource created by this M2M Service User is not configured. This list of default access control policies is combined with any default access control policies specified by the defaultAccessControlPrivileges attribute of the parent <m2mServiceSubscriptionProfile> resource. |

### -----------------------End of change 2---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?