| oneM2M Technical Report |  |
| --- | --- |
| Document Number | TR-0071-V-0.2.0 |
| Document Name: | AI enablement to oneM2M (Stage 2) |
| Date: | 2025-03-31 |
| Abstract: | The document provides a summary of identified AI/ML-related requirements at Stage 1. It then investigates potential architectural solutions to support these identified AI/ML requirements. Based on the findings of this technical report, new features will be proposed to enable AI/ML in the oneM2M system. |
| 'Template Version: January 2019 (do not modify) |  |

The present document is provided for future development work within oneM2M only. The Partners accept no liability for any use of this report.

The present document has not been subject to any approval process by the oneM2M Partners Type 1.  Published oneM2M specifications and reports for implementation should be obtained via the oneM2M Partners' Publications Offices.

About oneM2M

The purpose and goal of oneM2M is to develop technical specifications which address the need for a common M2M Service Layer that can be readily embedded within various hardware and software, and relied upon to connect the myriad of devices in the field with M2M application servers worldwide.

More information about oneM2M may be found at:  http//www.oneM2M.org

Copyright Notification

© 2023, oneM2M Partners Type 1 (ARIB, ATIS, CCSA, ETSI, TIA, TSDSI, TTA, TTC).

All rights reserved.

The copyright and the foregoing restriction extend to reproduction in all media.

Notice of Disclaimer & Limitation of Liability

The information provided in this document is directed solely to professionals who have the appropriate degree of experience to understand and interpret its contents in accordance with generally accepted engineering or other professional standards and applicable regulations. No recommendation as to products or vendors is made or should be implied.

NO REPRESENTATION OR WARRANTY IS MADE THAT THE INFORMATION IS TECHNICALLY ACCURATE OR SUFFICIENT OR CONFORMS TO ANY STATUTE, GOVERNMENTAL RULE OR REGULATION, AND FURTHER, NO REPRESENTATION OR WARRANTY IS MADE OF MERCHANTABILITY OR FITNESS FOR ANY PARTICULAR PURPOSE OR AGAINST INFRINGEMENT OF INTELLECTUAL PROPERTY RIGHTS. NO oneM2M PARTNER TYPE 1 SHALL BE LIABLE, BEYOND THE AMOUNT OF ANY SUM RECEIVED IN PAYMENT BY THAT PARTNER FOR THIS DOCUMENT, WITH RESPECT TO ANY CLAIM, AND IN NO EVENT SHALL oneM2M BE LIABLE FOR LOST PROFITS OR OTHER INCIDENTAL OR CONSEQUENTIAL DAMAGES. oneM2M EXPRESSLY ADVISES ANY AND ALL USE OF OR RELIANCE UPON THIS INFORMATION PROVIDED IN THIS DOCUMENT IS AT THE RISK OF THE USER.

# Contents

Contents	3

1	Scope	4

2	References	4

2.1	Normative references	4

2.2	Informative references	4

3	Definition of terms, symbols and abbreviations	4

3.1	Terms	4

3.2	Symbols	5

3.3	Abbreviations	5

4	Conventions	5

5	Introduction	5

6	Identified AI/ML Requirements	5

7	Proposed Solutions	6

7.1	AI/ML model management	6

7.1.1	Overview	6

7.1.2	Resource types	7

7.1.2.1	Resource Type modelRepo	7

7.1.2.2	Resource Type mlModel	7

7.1.2.3	Resource Type modelDeploymentList	8

7.1.2.4	Resource Type modelDeployment	9

7.1.3	Procedures	10

7.1.3.1	Procedures for <modelRepo>	10

7.1.3.2	Procedures for <mlModel>	10

7.1.3.3	Procedures for <modelDeploymentList>	10

7.1.3.4	Procedures for <modelDeployment>	10

7.2	AI/ML dataset management	11

7.2.1	Overview	11

7.2.2	Resource types	12

7.2.2.1	Resource Type mlDatasetPolicy	12

7.2.2.2	Resource Type dataset	14

7.2.2.3	Resource Type datasetFragment	14

7.2.3	Procedures	15

7.2.3.1	Procedures for <mlDatasetPolicy>	15

7.2.3.2	Procedures for <dataset>	15

7.2.3.3	Procedures for <datasetFragment>	15

7.n	Solution n	16

8	Conclusions	16

Annex <A>: Title of annex	17

Annex <y>: Bibliography	17

History	17

# 1	Scope

The current document provides a summary of identified AI/ML-related requirements at Stage 1. It then investigates potential architectural solutions to support these identified AI/ML requirements. Based on the findings of this technical report, new features will be proposed to enable AI/ML in the oneM2M system.

# 2	References

The following text block applies.

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references,only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

## 2.1	Normative references

As a Technical Report (TR) is entirely informative it shall not list normative references.

The following referenced documents are necessary for the application of the present document.

[1]	oneM2M TS-0001: "Functional Architecture- Release 4".

[2]	oneM2M TS-0004 "Service Layer Core protocol Specification - Release 4".

## 2.2	Informative references

Clause 2.2 shall only contain informative references which are cited in the document itself.

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	oneM2M Drafting Rules  (http://member.onem2m.org/Static_pages/Others/Rules_Pages/oneM2M-Drafting-Rules-V1_0.doc)

# 3	Definition of terms, symbols and abbreviations

Delete from the above heading the word(s) which is/are not applicable.

## 3.1	Terms

Clause numbering depends on applicability.

A definition shall not take the form of, or contain, a requirement.

The form of a definition shall be such that it can replace the term in context. Additional information shall be given only in the form of examples or notes (see below).

The terms and definitions shall be presented in alphabetical order.

For the purposes of the present document, the [following] terms and definitions [given in ... and the following] apply:

Definition format

<defined term>: <definition>

If a definition is taken from an external source, use the format below where [N] identifies the external document which must be listed in Section 2 References.

<defined term>[N]: <definition>

example 1: text used to clarify abstract rules by applying them literally

NOTE:	This may contain additional information.

## 3.2	Symbols

Clause numbering depends on applicability.

For the purposes of the present document, the [following] symbols [given in ... and the following] apply:

Symbol format

<symbol>	<Explanation>

<2nd symbol>	<2nd Explanation>

<3rd symbol>	<3rd Explanation>

## 3.3	Abbreviations

Abbreviations should be ordered alphabetically.

Clause numbering depends on applicability.

For the purposes of the present document, the [following] abbreviations [given in ... and the following] apply:

Abbreviation format

<ABBREVIATION1>	<Explanation>

<ABBREVIATION2>	<Explanation>

<ABBREVIATION3>	<Explanation>

# 4	Conventions

The key words “Shall”, ”Shall not”, “May”, ”Need not”, “Should”, ”Should not” in this document are to be interpreted as described in the oneM2M Drafting Rules [i.1]

# 5	Introduction

Editor’s Note: This section summarises the contents of this Technical Report.

# 6	Identified AI/ML Requirements

Editor’s Note: The section provides identified AI/ML related requirements at Stage 1.

# 7	Proposed Solutions

Editor’s Note: The section provides various AI/ML solutions for potential requirements.

## 7.1	AI/ML model management

### 7.1.1	Overview

In this AI/ML model repository and deployment management feature, there are four new resource types suggested.  For a CSE, there can be one or more <modelRepo> resource(s) for different services. The repository contains <mlModel> resources and each model contains an ML model and its meta data.

For an AE which represents an AIoT device (e.g. AI robot), AI service users can deploy model(s) to the device. To deploy models, a <modelDeploymentList> resource which works as the container of ML model deployments needs to be created first. A <modelDeployment> resource includes the IDs to the ML model, inference input and output resources. Multiple ML models can be deployed on an AIoT device which has one or more sensor(s) that provides inference input.

Figure 7.1.1-1: Resource tree structure for ML model management

NOTE: An alternative approach regarding the exising oneM2M device management capability is FFS

### 7.1.2	Resource types

#### 7.1.2.1	Resource Type modelRepo

The <modelRepo> resource represents a repository of ML models while each model is stored in a child <mlModel> resource. There can be more than one ML model repositories in a CSE for multiple ML-based services.

Table 7.1.2.1-1: Child resources of <modelRepo> resource

| Child Resources of <modelRepo> | Child Resource Type | Multiplicity | Description | <modelRepoAnnc> Child Resource Types |
| --- | --- | --- | --- | --- |
| [variable] | <semanticDescriptor> | 0..n | See clause 9.6.30 [1] | <semanticDescriptor>, <semanticDescriptorAnnc> |
| [variable] | <subscription> | 0..n | See clause 9.6.8 [1] | <subscription> |
| [variable] | <mlModel> | 0..n | See clause 7.1.2.2 | <mlModelAnnc> |
| [variable] | <latest> | 1 | See clause 9.6.27 [1] | None |
| [variable] | <oldest> | 1 | See clause 9.6.28 [1] | None |

The <modelRepo> resource shall contain the attributes specified in table 7.1.2.1-2.

Table 7.1.2.1-2: Attributes of <modelRepo> resource

| Attributes of <modelRepo> | Multiplicity | RW/ RO/ WO | Description | <modelRepoAnnc> Attributes |
| --- | --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| resourceID | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| resourceName | 1 | WO | See clause 9.6.1.3 [1]. | NA |
| parentID | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| expirationTime | 1 | RW | See clause 9.6.1.3 [1] | MA |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | MA |
| labels | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | MA |
| creationTime | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| announceTo | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | NA |
| announcedAttribute | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | NA |
| announceSyncType | 0..1 | RW | See clause 9.6.1.3 [1]. | MA |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | OA |
| creator | 0..1 | RO | See clause 9.6.1.3 [1]. | NA |
| custodian | 0..1 | RW | See clause 9.6.1.3 [1]. | NA |
| currentNumberOfModels | 1 | RO | The current number of ML models in the repository. | OA |
| currentByteOfModels | 1 | RO | The current total byte size of ML models in the repository. | OA |
| maxNumberOfModels | 0..1 | RW | The maximum number of ML models which can be stored in the repository. | OA |
| maxByteOfModels | 0..1 | RW | The maximum byte size of ML models which can be stored in the repository. | OA |

#### 7.1.2.2	Resource Type mlModel

The <mlModel> resource represents an ML model and is a child of a <modelRepo> resource.

Table 7.1.2.2-1: Child resources of <mlModel> resource

| Child Resources of <mlModel> | Child Resource Type | Multiplicity | Description | <mlModelAnnc> Child Resource Types |
| --- | --- | --- | --- | --- |
| [variable] | <semanticDescriptor> | 0..n | See clause 9.6.30 [1] | <semanticDescriptor>, <semanticDescriptorAnnc> |
| [variable] | <subscription> | 0..n | See clause 9.6.8 [1] | <subscription> |

The <mlDatasetPolicy> resource shall contain the attributes specified in table 7.1.2.2-2.

Table 7.1.2.2-2: Attributes of <mlModel> resource

| Attributes of <mlModel> | Multiplicity | RW/ RO/ WO | Description | <mlModelAnnc> Attributes |
| --- | --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| resourceID | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| resourceName | 1 | WO | See clause 9.6.1.3 [1]. | NA |
| parentID | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| expirationTime | 1 | RW | See clause 9.6.1.3 [1] | MA |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | MA |
| labels | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | MA |
| creationTime | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| announceTo | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | NA |
| announcedAttribute | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | NA |
| announceSyncType | 0..1 | RW | See clause 9.6.1.3 [1]. | MA |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | OA |
| creator | 0..1 | RO | See clause 9.6.1.3 [1]. | NA |
| custodian | 0..1 | RW | See clause 9.6.1.3 [1]. | NA |
| name | 0..1 | RW | The human readable name of the ML model. | OA |
| version | 1 | RW | The version of the ML model. | OA |
| platform | 1 | RW | The ML platform that trained the model. (e.g. tensorFlow) | OA |
| mlType | 1 | RW | The type of ML algorithm (e.g. regression, classification). | OA |
| description | 0..1 | RW | The human readable description of the ML model. | OA |
| inputSample | 0..1 | RW | The sample data of inference input. | OA |
| outputSample | 0..1 | RW | The sample data of inference output. | OA |
| mlModel | 0..1 | RW | The Base 64 encodedbinary ML model. When a model consists more than one file, than the zipped file is stored in this attribute. This cannot be present with mlModelURL. | OA |
| mlModelSize | 0..1 | RO | The byte size of the ML model stored in mlModel. | OA |
| mlModelURL | 0..1 | RW | The URL of the ML model. This cannot be present with mlModel. | OA |

#### 7.1.2.3	Resource Type modelDeploymentList

The <modelDeploymentList> resource represents the meta data of ML model deployements while each ML model deployment is respresented in a child <modelDeployment> resource.  This resource can be created as a child of an <AE> resource which represents an IoT device, so the deployment list resouce can work as a list of remote ML model deployments.

Table 7.1.2.3-1: Child resources of <modelDeploymentList> resource

| Child Resources of <modelDeployment>List> | Child Resource Type | Multiplicity | Description | <modelDeploymentList> Child Resource Types |
| --- | --- | --- | --- | --- |
| [variable] | <semanticDescriptor> | 0..n | See clause 9.6.30 [1] | <semanticDescriptor>, <semanticDescriptorAnnc> |
| [variable] | <subscription> | 0..n | See clause 9.6.8 [1] | <subscription> |
| [variable] | <modelDeployment> | 0..n | See 7.1.2.4 | <modelDeploymentAnnc> |

The <modelDeploymentList> resource shall contain the attributes specified in table 7.1.2.3-2.

Table 7.1.2.3-2: Attributes of <modelDeploymentList> resource

| Attributes of <modelDeploymentList> | Multiplicity | RW/ RO/ WO | Description | <modelDeploymentListAnnc> Attributes |
| --- | --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| resourceID | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| resourceName | 1 | WO | See clause 9.6.1.3 [1]. | NA |
| parentID | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| expirationTime | 1 | RW | See clause 9.6.1.3 [1] | MA |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | MA |
| labels | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | MA |
| creationTime | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| announceTo | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | NA |
| announcedAttribute | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | NA |
| announceSyncType | 0..1 | RW | See clause 9.6.1.3 [1]. | MA |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | OA |
| creator | 0..1 | RO | See clause 9.6.1.3 [1]. | NA |
| custodian | 0..1 | RW | See clause 9.6.1.3 [1]. | NA |
| numberOfDeployedModels | 1 | RO | The the number of ML models whose status is “deployed” among the models in this deployment list. | OA |
| numberOfRunningModels | 1 | RO | The the number of ML models whose status is “running” among the models in this deployment list. | OA |
| numberOfStoppedModels | 1 | RO | The the number of ML models whose status is “stopped” among the models in this deployment list. | OA |

#### 7.1.2.4	Resource Type modelDeployment

The <modelDeployment> resource represents an ML model deployment. An ML AE (e.g. AIoT device) is expected to subscribe to the parent <modelDeploymentList> resource for the newly created <modelDeployment> resource. When the AE gets the new ML model deployment, it retrieves the ML model by the modelID attribute. Once the AE serves the model, then it gets the inference input by the inputResource and run the ML model. Then the AE stores the inference result into the resource indicated by the outputResource. A referred resource by inputResource and outputResource are aresource sharing resources (e.g. <container> resource).

Another AE can remotely manipulate the model status with the modelCommand attribute. It can run or stop the deployed model.

Table 7.1.2.1-1: Child resources of <modelDeployment> resource

| Child Resources of <modelDeployment> | Child Resource Type | Multiplicity | Description | <modelDeploymentAnnc> Child Resource Types |
| --- | --- | --- | --- | --- |
| [variable] | <semanticDescriptor> | 0..n | See clause 9.6.30 [1] | <semanticDescriptor>, <semanticDescriptorAnnc> |
| [variable] | <subscription> | 0..n | See clause 9.6.8 [1] | <subscription> |

The <modelDeployment> resource shall contain the attributes specified in table 7.1.2.1-2.

Table 7.1.2.1-2: Attributes of <modelDeployment> resource

| Attributes of <modelDeployment> | Multiplicity | RW/ RO/ WO | Description | <modelDeploymentAnnc> Attributes |
| --- | --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| resourceID | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| resourceName | 1 | WO | See clause 9.6.1.3 [1]. | NA |
| parentID | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| expirationTime | 1 | RW | See clause 9.6.1.3 [1] | MA |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | MA |
| labels | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | MA |
| creationTime | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| announceTo | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | NA |
| announcedAttribute | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | NA |
| announceSyncType | 0..1 | RW | See clause 9.6.1.3 [1]. | MA |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | OA |
| creator | 0..1 | RO | See clause 9.6.1.3 [1]. | NA |
| custodian | 0..1 | RW | See clause 9.6.1.3 [1]. | NA |
| modelID | 1(L) | WO | The resource ID of the <mlModel> resource that is deployed. | OA |
| modelCommand | 1 | RW | This attribute is not returned in a response, but can be included in an Update request to update the modelStatus. Allowed values for an Update request is “run” and “stop”. | NA |
| modelStatus | 1 | RO | The status of the deployed ML model which is “deployed”, “running” or “stopped”. The default is “deployed” | OA |
| inputResource | 1 | RW | The resource ID of the inference input resource. | OA |
| outputResource | 1 | RW | The resource ID of the inference output. | OA |

### 7.1.3	Procedures

#### 7.1.3.1	Procedures for <modelRepo>

No change from the CRUD procedures in clause 10.1 [1].

#### 7.1.3.2	Procedures for <mlModel>

No change from the CRUD procedures in clause 10.1 [1].

#### 7.1.3.3	Procedures for <modelDeploymentList>

No change from the CRUD procedures in clause 10.1 [1].

#### 7.1.3.4	Procedures for <modelDeployment>

In a <modelDeployment> Update procedure, when the modelCommand attribute is included in the Update request, depending on the requested value, the Hosting CSE shall update the modelStatus attribute. The status value by default after the resource creation is “deployed”. For the start “run” and “stop” command, the status shall be updated as “running” and “stopped”, respectively. When the modelStatus attribute is updated, the Hosting CSE shall update the corresponding attributes (i.e. numberOfDeployedModels, numberOfRunningModels, numberOfStoppedModels) of a parent <modelDeploymentList> resource as well.

No change from the Create, Retrieve and Delete procedures in clause 10.1 [1].

## 7.2	AI/ML dataset management

### 7.2.1	Overview

The mechanism and the resource structure of AI/ML dataset management is similar to the <locationPolicy>, <container>, <contentInstance> and resource and corresponding handling procedures [1]. When an AE creates the <mlDatasetPolicy> resource with preferred policies for the Hosting CSE, the AI/ML dataset is created and managed by the Hosting CSE on behalf of the AE. Once a dataset is managed, other AEs can use the datasets for different applications.

Figure 7.2.1-1: Resource structure for AI/ML dataset management

When the resources get instantiated it would look like the figure 7.2.1-2. One dataset policy resource can have two dataset resources: one for historical dataset which is used for AI/ML model training, and the other is the live dataset for inference with the trained models. Each dataset resource contains dataset fragments.

The datasetFormat attribute in the <mlDatasetPolicy> resource as well as the <datasetFragment> resource represents the format of the dataset. As it is widely used in AI/ML model training, CSV (i.e. MIME type “text/csv") is suggested as one of the format options.

Figure 7.2.1-2: Resource tree example

### 7.2.2	Resource types

#### 7.2.2.1	Resource Type mlDatasetPolicy

The <mlDatasetPolicy> resource represents a policy to create a machine learning dataset with the data from other data sharing resources. This is used to create training datasets with historical data for building machine learning models. When dataset preparation is done, the CSE creates <dataset> and <datasetFragment> resources to store the dataset. Then data scientists fetch, process the dataset and train models. After that, for newly created resources, inference input data is created and stored the by CSE with the policies. Newly created inference input data can be retrieved or notified with subscription to the machine leaerning applications which deploys the trained models.

Table 7.2.2.1-1: Child resources of <mlDatasetPolicy> resource

| Child Resources of <mlDatasetPolicy> | Child Resource Type | Multiplicity | Description | <mlDatasetPolicy> Child Resource Types |
| --- | --- | --- | --- | --- |
| [variable] | <semanticDescriptor> | 0..n | See clause 9.6.30 [1] | <semanticDescriptor>, <semanticDescriptorAnnc> |
| [variable] | <subscription> | 0..n | See clause 9.6.8 [1] | <subscription> |

The <mlDatasetPolicy> resource shall contain the attributes specified in table 7.2.2.1-2.

Table 7.2.2.1-2: Attributes of <mlDatasetPolicy> resource

| Attributes of <mlDatasetPolicy> | Multiplicity | RW/ RO/ WO | Description | <mlDatasetPolicyAnnc> Attributes |
| --- | --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| resourceID | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| resourceName | 1 | WO | See clause 9.6.1.3 [1]. | NA |
| parentID | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| expirationTime | 1 | RW | See clause 9.6.1.3 [1] | MA |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | MA |
| labels | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | MA |
| creationTime | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| announceTo | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | NA |
| announcedAttribute | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | NA |
| announceSyncType | 0..1 | RW | See clause 9.6.1.3 [1]. | MA |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | OA |
| creator | 0..1 | RO | See clause 9.6.1.3 [1]. | NA |
| custodian | 0..1 | RW | See clause 9.6.1.3 [1]. | NA |
| sourceResourceIDs | 1(L) | WO | The resource IDs which builds a dataset When the target resource is a <container> resource, content attribute of the children <contentInstance> resouces get included in the dataset. When the target resource is a <timeSeries> resource, content attribute of the cchildren <timeSeriesInstance> resouces get included in the dataset. When the target resource is a <flexContainer> resource, custom attributes of the children <flexContainerInstance> resouces get included in the dataset. | OA |
| datasetStartTime | 0..1 | WO | The timestamp filter as the start time of source data resources. creationTime or dataGenerationTime depending on the source resource type, gets filtered. If datasetStartTime and datasetEndTime both are not provided, then all data instanes of source resources get included in the dataset. | OA |
| datasetEndTime | 0..1 | WO | The timestamp filter as the end time of source data resources. creationTime or dataGenerationTime depending on the source resource type, gets filtered. If datasetStartTime and datasetEndTime both are not provided, then all data instanes of source resources get included in the dataset. | OA |
| timeCorrelationStartTime | 0..1 | WO | When more than one data from source resources get batched as one, this timestamp indicates the start time of each recurring windows. | OA |
| timeCorrelationDuration | 0..1 | WO | This indicates duration for each data batch window. | OA |
| nullValuePolicy | 0..1 | WO | This indicates null(empty) value handling policy (e.g. leave as null, fill with last-known values) for the created dataset. | OA |
| datasetFormat | 1 | WO | The serialization format of the dataset. (e.g. CSV, JSON) Note: CSV format is not supported in oneM2M yet. | OA |
| historicalDatasetID | 0..1 | RO | The ID of the <dataset> resource for a training dataset which gets generated with existing source resources (i.e. historical data) referred by the sourceResourceIDs attribute. | OA |
| numberOfRowsForHistoricalDataset | 0..1 | WO | The number of data fragment of the dataset is stored in a single <datasetFragment> resource. Default is all. | OA |
| numberOfRowsForLiveDataset | 0..1 | WO | The number of data fragment of the dataset is stored in a single <datasetFragment> resource. Default is 1(one). | OA |
| liveDatasetID | 0..1 | RO | The ID of the <dataset> resource for a dataset which gets generated with newly created source resources after creation of the <mlDatasetPolicy> resource. When the is numberOfDataForInference set, a separate <dataset> resource gets created for newly created source resources. | OA |

#### 7.2.2.2	Resource Type dataset

The <dataset> resource represents a container for a dataset. This is created by a CSE when <mlDatasetPolicy> resource is created. When it is created, the links are stored in trainingDatasetID and  infefenceInputDataID in the <mlDatasetPolicy> resource, and mlDatasetPolicyID attribute in the <dataset> resource.

The <dataset> resource works similar to <container> resource, but has additional attributes to have dataset related aspects. This supports <latest> and <oldest> resources. The dataset is fragmented and stored in each child <datasetFragment> resource.

Table 7.2.2.2-1: Child resources of <dataset> resource

| Child Resources of <dataset> | Child Resource Type | Multiplicity | Description | <dataset> Child Resource Types |
| --- | --- | --- | --- | --- |
| [variable] | <semanticDescriptor> | 0..n | See clause 9.6.30 [1] | <semanticDescriptor>, <semanticDescriptorAnnc> |
| [variable] | <subscription> | 0..n | See clause 9.6.8 [1] | <subscription> |
| la | <latest> | 1 | This virual resource refers the latest <datasetFragment> resource. | None |
| ol | <oldest> | 1 | This virual resource refers the oldest <datasetFragment> resource. | None |
| [variable] | <datasetFragment> | 0..n | See clause 7.1.2.3 | <datasetFragmentAnnc> |

The <dataset> resource shall contain the attributes specified in table 7.2.2.2-2.

Table 7.2.2.2-2: Attributes of <dataset> resource

| Attributes of <dataset> | Multiplicity | RW/ RO/ WO | Description | <datasetAnnc> Attributes |
| --- | --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| resourceID | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| resourceName | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| parentID | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| expirationTime | 1 | RO | See clause 9.6.1.3 [1] | MA |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | MA |
| labels | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | MA |
| creationTime | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| announceTo | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | NA |
| announcedAttribute | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | NA |
| announceSyncType | 0..1 | RW | See clause 9.6.1.3 [1]. | MA |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See clause 9.6.1.3 [1]. | OA |
| creator | 0..1 | RO | See clause 9.6.1.3 [1]. | NA |
| custodian | 0..1 | RW | See clause 9.6.1.3 [1]. | NA |
| mlDatasetPolicyID | 1 | RO | The ID of the <mlDatasetPolicyID> resource which generated this <dataset> resource. | OA |
| listOfFeatures | 1 | RO | The list of dataset feature names. | OA |

NOTE: management mechanism for location attribute of a dataset which merges more than one data sharing resources is FFS

### 7.2.2.3	Resource Type datasetFragment

The <datasetInstance> resource represents a fragment of a dataset. A whole dataset is split into the number of data items which is configured in the corresponding <mlDatasetPolicy> resource of the parent <dataset> resource.

The <datasetFragment> resource shall contain the attributes specified in table 7.2.2.3-1.

Table 7.2.2.3-1: Attributes of <datasetFragment> resource

| Attributes of <datasetFragment> | Multiplicity | RW/ RO/ WO | Description | <datasetFragmentAnnc> Attributes |
| --- | --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| resourceID | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| resourceName | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| parentID | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| labels | 0..1 (L) | WO | See clause 9.6.1.3 [1]. | MA |
| creationTime | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| expirationTime | 1 | WO | See clause 9.6.1.3 [1]. | NA |
| announceTo | 0..1 (L) | WO | See clause 9.6.1.3 [1]. | NA |
| announcedAttribute | 0..1 (L) | WO | See clause 9.6.1.3 [1]. | NA |
| announceSyncType | 0..1 | RW | See clause 9.6.1.3 [1]. | MA |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3 [1]. | NA |
| datasetFragmentStartTime | 1 | RO | The oldest timestamp among the data in the dataset attribute. | OA |
| datasetFragmentEndTime | 1 | RO | The latest timestamp among the data in the dataset attribute. | OA |
| numberOfRowsInFragment | 1 | RO | The number of data in the dataset attribute. | OA |
| datasetFragment | 1 | RO | The dataset fragment generated by the <mlDatasetPolicy> resource, referred by mlDatasetPolicyID of the parent <dataset> resource. | OA |
| datasetFormat | 1 | RO | The serialization format of the dataset. (e.g. CSV, JSON) Note: CSV format is not supported in oneM2M yet. | OA |

### 7.2.3	Procedures

#### 7.2.3.1	Procedures for <mlDatasetPolicy>

No change from the  CRUD procedures in clause 10.1 [1].

#### 7.2.3.2	Procedures for <dataset>

A <dataset> resource is created by the <mlDatasetPolicy> resource hosting CSE, so the Create prodecure is not specified as an API.

When the CSE creates <dataset> resource after the creation of corresponding <mlDatasetPolicy> resource, the custodian attribute of <dataset> resource is set as the same as <mlDatasetPolicy> resource, if present.

No change from the Retrieve, Update and Delete procedures in clause 10.1 [1].

#### 7.2.3.3	Procedures for <datasetFragment>

A <datasetFragment> resource is created by the <dataset> resource hosting CSE, so the Create prodecure is not specified as an API. Also, this resource is immutable so Update procedure is not specified.

No change from the  Retrieve and Delete procedures in clause 10.1 [1].

Editor’s Note: How to merge structured data into a single dataset is FFS. (e.g. flattening structured data to merge into one dataset table)

## 7.n	Solution n

# 8	Conclusions

Editor’s Note: This section provides a summary of the conclusions drawn during the study.

## Annex <A>:
Title of annex

<Text>

<PAGE BREAK>

Annex <B>:
Title of annex (style H9)

<Text>

B.1	First clause of the annex (style H1)

<Text>

B.1.1	First subdivided clause of the annex (style H2)

<Text>

<PAGE BREAK>

Annex <y>:
Bibliography

The annex entitled "Bibliography" is optional.

It shall contain a list of standards, books, articles, or other sources on a particular subject which are not mentioned in the document itself.

It shall not include references mentioned in the document.

Use the Heading 9 style for the title and B1+ or Normal for the text.

<Publication>: "<Title>".

OR

<Publication>: "<Title>".

<PAGE BREAK>

# History

This clause shall be the last one in the document and list the main phases (all additional information will be removed at the publication stage).

| Publication history |  |  |
| --- | --- | --- |
| V1.1.1 | <yyyy-mm-dd> | <Milestone> |

| Draft history (to be removed on publication) |  |  |
| --- | --- | --- |
| V0.0.1 | 2023-06-27 | Skeleton of the TR. |
| V0.1.0 | 2024-06-26 | This baseline contains agreed contributions from SDS#62: - SDS-2023-0172R01-solutions_for_ML_model_management |
| V0.2.0 | 2025-03-31 | This baseline contains agreed contributions from SDS#68: - SDS-2023-01711R03-solutions_for_ML_dataset_management |