	Doc# SDS-2025-0148R01-SSP_Status_Checks_For_Targeted_Resource_R5
	[image: image3.png]






	


	CHANGE REQUEST

	Meeting ID:*
	SDS 71

	Source:*
	Rahul Jha, CDOT, rahulj@cdot.in
Poornima Shandilya, CDOT, poornima@cdot.in

	Date:*
	2025-09-08

	Reason for Change/s:*
	See Introduction

	CR  against:  Release*
	Rel 5

	CR  against:  WI*
	 FORMCHECKBOX 
 Active <Work Item number>  

 FORMCHECKBOX 
 MNT maintenance / < Work Item number(optional)>
Is this a mirror CR? Yes  FORMCHECKBOX 
 No  FORMCHECKBOX 

mirror CR number: (Note to Rapporteur - use latest agreed revision)
 STE Small Technical Enhancements / < Work Item number (optional)>
Only ONE of the above shall be ticked

	CR  against:  TS/TR*
	TS-0004 v5.1.0

	Clauses *
	7.3.3.1

	Type of change: *
	 FORMCHECKBOX 
 Editorial change
 Bug Fix or Correction
 Change to existing feature or functionality
 New feature or functionality
Only ONE of the above shall be ticked

	Other TS/TR(s) impacted
	TS-0001

	Post Freeze checking:*
	This CR contains only essential changes and corrections?  YES 
  NO 
This CR may break backwards compatibility with the last approved version of the TS?       YES 
  NO 


	Template Version: January 2020 (do not modify)


oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.
If this is  a correction, and the change applies to previous releases, a separate “mirror CR” should be posted at the same time as this CR
Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).
Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made e.g. a change impacting 5 tables should not only include a proposal to change only 3 tables. Include any changes to references, definitions, and abbreviations in the same deliverable.
Follow the drafting rules.
All pictures must be editable.
Check spelling and grammar.
Use change bars for modifications.
The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the proposed new clause is located.
Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.
When subsequent changes are made to the content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text. 
Introduction
The current spec dictates that for a request, the Registrar CSE shall ensure that the Originator is associated with a valid and an ACTIVE <m2mServiceSubscriptionProfile>.
[image: image1.png]WM
E IN-CSE

Checks the originators Service
Subscription Restrictions

A

Belongs to this
Service Subscription

CS
S

ends oneM2M Request





But incase where the originator belongs to one <m2mServiceSubscriptionProfile> while the targeted resource belongs to another <m2mServiceSubscriptionProfile>, it is unclear as to which <m2mServiceSubscriptionProfile> should be checked for status.

[image: image2.png]Belongs to this Belongs to this
Service Subscription . Service Subscription

m AE-1 wants to create
<container> in AE-2 <container>





This issue was discussed in the TP-70 with discussion contribution SDS-2025-0059, it was concluded that both originator and targeted resources’ <m2mServiceSubscriptionProfile> should be checked for status ACTIVE.
This CR corresponds to the discussed change for TS-0004.
-----------------------Start of change 1-------------------------------------------
7.3.3.1
Check existence of the targeted resource and Primitive Profile
Upon receiving a request, a Hosting CSE shall first check if a Primitive Profile Identifier parameter is specified in the request.  If specified, the Hosting CSE shall check if the <primitiveProfile> resource referenced by the Primitive Profile Identifier parameter is present and the originator has Retrieve privileges for the <primitiveProfile> resource.  If the referenced <primitiveProfile> resource is not present or the originator does not have Retrieve privileges, the Hosting CSE shall reject the request with a Response Status Code indicating "PRIMITIVE_PROFILE_NOT_ACCESSIBLE".
If the Primitive Profile Identifier parameter is specified and the referenced <primitiveProfile> resource is present and is retrievable by the request originator, the Hosting CSE shall check whether the primitive profile’s IDList, resourceTypes, operations, resourceIDs and releaseVersions attributes match the received request and the applicability attribute includes the value “REQUESTS_TO_CSE”.  If a match exists, the Hosting CSE shall apply the <primitiveProfile> resource to the request by adding, replacing or deleting any applicable request parameters or attributes defined in the additions or deletions attributes of the referenced <primitiveProfile> resource. If a match does not exist, the Hosting CSE shall attempt to process the primitive without applying the <primitiveProfile> resource.

If the Hosting CSE detects any names of request parameters or resource attributes defined in the additions attribute of the referenced <primitiveProfile> resource are not valid names of request parameters or attributes defined for the type of resource targeted by the request, the Hosting CSE shall reject the request with a Response Status Code indicating "PRIMITIVE_PROFILE_BAD_REQUEST".
If the Hosting CSE detects that a name of a request parameter or resource attribute, defined in the deletions attribute of the referenced <primitiveProfile> resource, is a mandatory request parameter or mandatory attribute defined for the type of resource targeted by the request, the Hosting CSE shall reject the request with a Response Status Code indicating "PRIMITIVE_PROFILE_BAD_REQUEST".
If the Request Expiration Timestamp is given in the request and has expired, the Hosting CSE shall reject the request with a "REQUEST_TIMEOUT" Response Status Code parameter value. Otherwise, the Hosting CSE should handle the request before the time specified in Request Expiration Timestamp.
The Hosting CSE shall check if the resource addressed by the To parameter exists in the repository. If the resource does not exist, the Hosting CSE shall reject the request with a Response Status Code indicating "NOT_FOUND" error.

If the parameter filterUsage is configured as "Conditional Operation", the Hosting CSE shall also check the existence of the target resource based on any Filter Criteria Matching Conditions present in the Create/Retrieve/Update/Delete request. If there is no matching target resource, the Hosting CSE shall reject the request with a Response Status Code indicating "NOT_FOUND" error. 
If the Hosting CSE does not support the content format (i.e. type of serialization) requested by the originator, the request shall be rejected with a Response Status Code indicating "NOT_ACCEPTABLE" error.

If the Hosting CSE does not support the content format sent by the originator, the request shall be rejected with a Response Status Code indicating "UNSUPPORTED_MEDIA_TYPE" error.
The Hosting CSE shall check whether the targeted resource is associated with a valid <m2mServiceSubscriptionProfile>. If such a profile exists and its status attribute has the value INACTIVE, then the Hosting CSE shall reject the request with the error “SERVICE_SUBSCRIPTION_NOT_ACTIVE”.
-----------------------End of change 1---------------------------------------------

CHECK LIST

· Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?
· Does this CR contain changes related to only one particular issue/problem?
· Have any mirror CRs been posted?
· Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?
· Are all pictures editable?
· Have you checked the spelling and grammar?
· Have you used change bars for all modifications?
· Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)
· Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?
© 2020 oneM2M Partners
                                                                                                   Page 2 (of 4)



[image: image3.png]