| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | TDE#44.2 |
| Source:* | Poornima Shandilya ,C-DOT, poornima@cdot.in Chaitan Kumar Yadav, C-DOT, chaitan.yadav@cdot.in |
| Date:* | 2020-03-26 |
| Reason for Change/s:* | Addition of test cases. |
| CR against: Release* | 3.6.0 |
| CR against: WI* | Active MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0018 v3.6.0 |
| Clauses * |  |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | Not Applicable |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

As per definition of ACR attribute of <accessControlPolicy> resource,  accessControlOriginators attribute(as per section 9.6.2.1 of TS-0001) can  have resourceID of <group> resource, the current release of TS-0018 don’t have any testcase for this scenario , this CR attempts to add testcase for same.

Table 9.6.2.1-1: Types of Parameters in accessControlOriginators

| Name | Description |
| --- | --- |
| domain | A SP domain or SP sub-domain |
| originatorID | CSE-ID, AE-ID or the resource-ID of a <group> resource that contains the AE or CSE that represents the Originator. |
| all | Any Originators are allowed to access the resource within the accessControlOriginators constraints |
| Role-ID | A Role Identifier as defined in clause 7.1.14 |

********** Start of change 1 **********

TP/oneM2M/CSE/SEC/ACP/CRE/005

| TP Id |  | TP/oneM2M/CSE/SEC/ACP/CRE/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds successfully when an AE creates an accessControlPolicy resource having accessControlOriginators element set to a group resource identifier which contains AE resource as member |  |
| Reference |  | TS-0001 [1], clause 9.6.2 |  |
| Config Id |  | CF01 |  |
| Parent release |  | Release 3 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the AE1 having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS and the IUT having registered the AE2 and the IUT having created a group resource at GROUP_RESOURCE_ADDRESS containing memberIDs set to AE2_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request containing To set to TARGET_RESOURCE_ADDRESS and From set to AE1 and Content containing accessControlPolicy resource containing privileges containing accessControlRule element containing accessControlOriginators element set to GROUP_RESOURCE_ADDRESS } |  | IUT  AE1 |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) } |  | IUT  AE1 |

********** End of change 1 **********

### ********** Start of change 2 **********

TP/oneM2M/CSE/SEC/ACP/015

| TP Id |  | TP/oneM2M/CSE/SEC/ACP/015 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds successfully when an AE tries an OPERATION on the AE1 resource whose accessControlPolicyIDs attribute is set. |  |
| Reference |  | TS-0001 [1], clause 9.6.2 |  |
| Config Id |  | CF01 |  |
| Parent release |  | Release 3 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and this IUT having <group> resource GRP1 having AE2 as memberIds and the <AE> resource of AE1 having accessControlPolicyIDs attribute set and this referenced <accessControlPolicy> having atleast single ACR’ accessControlOriginators attribute having resourceID of <group> resource GRP1 and accessControlOperations to OPERATION } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request containing To set to AE1_RESOURCE_ADDRESS and From set to AE2_ID } |  | IUT  AE2 |
|  | then { the IUT sends a valid Response containing Response Status Code set to RESPONSE_STATUS_CODE } |  | IUT  AE2 |

| TP Id | OPERATION | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/SEC/ACP/015_CRE | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/SEC/ACP/015_RET | RETRIEVE | 2000 (OK) |
| TP/oneM2M/CSE/SEC/ACP/015_UPD | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/SEC/ACP/015_DEL | DELETE | 2002 (DELETED) |

********** End of change 2 **********

### ********** Start of change 3 **********

TP/oneM2M/CSE/SEC/ACP/016

| TP Id |  | TP/oneM2M/CSE/SEC/ACP/016 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT responds with an error when an AE tries an OPERATION on the AE1 resource whose accessControlPolicyIDs attribute is set. |  |
| Reference |  | TS-0001 [1], clause 9.6.2 |  |
| Config Id |  | CF01 |  |
| Parent release |  | Release 3 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the IUT having registered the AE3 and this IUT having <group> resource GRP1 having AE3 as member only. and the <AE> resource of AE1 having accessControlPolicyIDs attribute set and this referenced <accessControlPolicy> having at least single ACR’ accessControlOriginators attribute having resourceID of <group> resource GRP1 and accessControlOperations to OPERATION } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid OPERATION Request containing To set to AE1_RESOURCE_ADDRESS and From set to AE2_ID } |  | IUT  AE2 |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4103 (ORIGINATOR_HAS_NO_PRIVILEGE) } |  | IUT  AE2 |

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/SEC/ACP/016_CRE | CREATE |
| TP/oneM2M/CSE/SEC/ACP/016_RET | RETRIEVE |
| TP/oneM2M/CSE/SEC/ACP/016_UPD | UPDATE |
| TP/oneM2M/CSE/SEC/ACP/016_DEL | DELETE |

### ********** End of change 3 **********

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?