| CHANGE REQUEST |  |
| --- | --- |
| Meeting:* | ARC#19 |
| Source:* | SeungMyeong JEONG (seungmyeong.jeong@lge.com) |
| Date:* | 2015-08-31 |
| Contact:* | SeungMyeong JEONG (seungmyeong.jeong@lge.com) |
| Reason for Change/s:* | See the introduction. |
| CR against: Release* | Rel-1 |
| CR against: WI* | Active: MNT Maintenace / < Work Item number(optional)> STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0001 v1.10.0 |
| Clauses/Sub Clauses* | 7.1.7 M2M Request Identifier (M2M-Request-ID) |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR is a mirror CR? YES NO if YES, please indicate the document number of the original CR: ARC-2015-2006-MNT_CR_request_ID_clarification_(R1) |
| Template Version:23 February 2015 (Dot not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete sections need not show surrounding clauses as long as the proposed section number clearly shows where the new section is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

Uncertainty from the original text and proposals:

- Uniqueness of the Request ID – globally unique? or unique in a Registrar CSE only?

  - This CR suggests to make it globally unique over all Transit CSEs including Registrar CSEs.

- How do we make the Request ID globally unique?

  - This CR suggests to pre-pend a SP-relative-ID of the Originator.

    - When an AE is the Originator, the Registrar prepends the AE’s SP-relative-ID (c.f., currently AE-ID-Stem).

    - When a CSE is the Originator, the Originator CSE itself prepends its (SP-Relative) CSE-ID and send it to the Registrar CSE.

Other minor changes are:

- Need a discussion on Mcc’ aspect. This should be included over Mcc’ also. (paragraph 1)

- Removed the “M2M-” from the ID. This would be the same discussion which oneM2M defined ID should have “M2M-“ prefix. There’s no specific rule, therefore some IDs have the prefix while the others don’t. (See clause 7.1). If there’s no rule, then it’s better to remove all the prefix in the IDs.

- Moved the paragraph (i.e., starting with “In case an IN-CSE needs to send”) to the end considering the clause 7.1.7 text structure. The flow is, 1) ID role/feature, 2) ID for AE initiated request, 3) ID for CSE initiated request, 4) Request ID spanning over request/response chain, 5) ID duplication issue, and 6) Mcn issue.

R01:globally unique request ID  SP-relative request ID. The request ID is Service Provider domain unique.

R02:

- maintains “M2M-“ prefix in the name of the ID (needs further discussion later on this prefix)

- makes it “globally” unique again, this is the important point

- adds the text to cover Mcc’ aspect

- ----------------------- Start of change 1 -----------------------

### 7.1.7	M2M-Request Identifier (M2M-Request-ID)

The Request-ID tracks a Request initiated by an AE over the Mca reference point, and by a CSE over the Mcc or Mcc’ reference point, if applicable, end to end. It is also included in the Response to the Request.

To enable an Originator AE to track Requests and corresponding Responses over the Mca reference point, AEs shall include a distinct Request Identifier per request over the Mca reference point to the Registrar CSE for any initiated request.

The Registrar CSE of the AE shall make such Request Identifier globally unique by prepending the SP-Relative-AE-ID (see clause 7.2) and slash(‘/’)  in front of it (e.g., CSE01/C190XX7T/001, S200YY1S/001) .

If the Registrar CSE creates an Request Identifier by pending AE-ID-Stem, then the CSE shall maintain a binding between the Request Identifier received from the AE and the Request Identifier it created in its interactions towards other peer CSEs. The Registrar CSE shall include the Request Identifier received from the AE in its Response to the AE. This binding shall be maintained by the Registrar CSE until the Request message sequence is completed.

To enable an Originator CSE to track Requests and corresponding Responses over the Mcc reference point, CSEs shall include a distinct Request Identifier per request over the Mcc reference point to the Registrar CSE for any initiated request. The Originator CSE shall make such Request Identifier globally unique by including its SP-Relative-CSE-ID in the beginning of the Request Identifier (e.g., CSE01/001). When the request is forwarded over Mcc’ by IN-CSE, the IN-CSE shall prepend its M2M-SP-ID to the Request Identifier in the received request.

Note that the Request initiated by the CSE could be the result of an application Request, or a request initiated autonomously by the CSE to fulfil a M2M Service.

A CSE receiving a Request from a peer CSE shall include the received Request Identifier in all additional Requests unspanned (i.e. 1:1) it has to generate (including propagation of the incoming Request) and that are associated with the incoming Request, where applicable.

If a Receiver CSE receives a request from an Originator for which another request with the same Request Identifier is already pending, the request shall be rejected. Otherwise - even if the same Request Identifier was already used by the same Originaor sometime in the past, the request shall be treated as a new request.

In case an IN-CSE needs to send a request to a receiving CSE that is not reachable over any of the Underlying Networks, the IN-CSE initiates the procedure for "waking up" the Node hosting the receiving CSE by using procedures such as device triggering over the Mcn reference point. For Device Triggering, the triggering reference number to co-relate device triggering response is independent of the M2M Request Identifier. An IN-CSE may use the same value of an Request-Identifier in an incoming request for the triggering reference number in its interaction with the underlying network.

- ----------------------- End of change 1 -----------------------

CHECK LIST

- Does this change request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Does this change request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable?

- Does this change request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete sections need not show surrounding clauses as long as the proposed section number clearly shows where the new section is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?