| Work Item |  |
| --- | --- |
| Work Item Title:* | Optimized Group-based Operation |
| Document Number* | WI-0031-Optimized Group-based Operation-V1.0.0 |
| Supporting Members or Partner type 2* | Huawei Technologies Co., Ltd.; InterDigital; KETI; China Unicom |
| Date:* | 2015-03-24 |
| Abstract:* | The WI is to investigate optimization on group operations. |
| Template Version:23 February 2015 (Dot not modify) |  |

oneM2M Copyright statement

No part may be reproduced except as authorized by written permission.

The copyright and the foregoing restriction extend to reproduction in all media.

All rights reserved.

1	Title (Acronym)

Optimized Group-based Operation

2	Justification

In the current TS0001 and TS0004, groups in oneM2M are defined for fan out operations of all kinds of CSEs to multiple devices one by one using unicast. But group operations can be performed in other ways, depending on the underlying network capabilities. In the oneM2M system, as the amount of device connected with the system is very large, group-based features are very important to organize devices that are intended with the same functionality together.

According to the solution delivery experience, group operations are of great importance for the oneM2M system. And there are new use cases and requirements pertaining to group operations.

For example, currently oneM2M can only fan out group operation one by one using unicast. But some underlying network (e.g. 3GPP, IETF etc) provides multicast or broadcast method. For efficiency consideration, how to utilize the underlying network for broadcasting/ multicasting? Is it possible to use capabilities offered by the underlying network over Mcn for affecting (communicating with, receiving information about …) a group of AEs? Another example is that in oneM2M, if we need to grant access priviledge to multiple AEs which have the same role to multiple places, we need to repeat the same access control rule multiple times with just the accessControlOriginators different. And it is not convient to modify the priviledge. Is it possible to use group to grant access privilege to multiple AEs that has the same role? And so on.

3	Intended Output

| Tick all the appropriate cases |  |
| --- | --- |
| √ | Change request(s) to existing Technical Specification(s) |
| √ | Change request(s) to existing Technical Reports(s) |
|  | New Normative Technical Specifications(s) |
|  | New Permanent Technical Reports(s) |
|  | New Temporary Technical Reports(s) |

4	Impact

4.1	oneM2M Work Items

WI0016 – The proposed work item focuses on how to enhance group operations, which potentially have dependencies on group authentication methods. If needed, WG4 will also be involved for discussion on this topic.

5	Scope

Objectives may include:

-	Study (and standardize) the mechanism for group based communication by leveraging underlying network service capabilities over Mcn, such as, broadcast or multi-cast based mechanisms,

-	Study (and standardize) the mechanism for granting access previledge to a group of entities,

-	Other features related to group operation.

As a result, the scope of the proposed new WI includes:

-	Call for and analyse use cases and requirements on group operations.

-	Analyse if the current oneM2M group mechanism can fulfil the use cases and requirements.

-	Analyse the potential group related new features to be brought to oneM2M.

-	Change Requests to architecture and protocol specification to contribute the new features needed.

6	Schedule and impacted specifications

| CRs to existing specifications (if any) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Impacted TS/TR | CR number (when known) | Subject of the CR | Approved at plenary# | Impacted WGs | Comments |
| TS0002 | N/A | Group enhancement requirements | TP#19 | WG1 | N/A |
| TS0001 | N/A | Enhancement in architecture | TP#20 | WG2 | N/A |
| TS0004 | N/A | Enhancement in protocol | TP#21 | WG3 | N/A |
| TR0001 | N/A | Group related use cases | TP#19 | WG1 | N/A |
| TR0012 | N/A | Potential CR related to group authentication | TP#19 | WG4 | N/A |

7	Work Item Rapporteur(s)

Jiaxin Yin, Huawei Technologies Co., Ltd. (For all impacted WGs)

8	History

| Document history |  |  |
| --- | --- | --- |
| V0.0.1 | 14 March 2015 | Initial proposal |

-------------------------------