| oneM2M Technical Specification |  |
| --- | --- |
| Document Number | TS-0007 -V0.8.0 |
| Document Name: | Service Components |
| Date: | 2015-March-30 |
| Abstract: | This document specifies the M2M Services provided by the M2M Services Platform, the integration and interworking of the M2M Services functional architecture of the oneM2M Services Platform and informatively illustrates the use of the M2M Services within the context of complex business services. |

This Specification is provided for future development work within oneM2M only. The Partners accept no liability for any use of this Specification.

The present document has not been subject to any approval process by the oneM2M Partners Type 1. Published oneM2M specifications and reports for implementation should be obtained via the oneM2M Partners’ Publications Offices.

About oneM2M

The purpose and goal of oneM2M is to develop technical specifications which address the need for a common M2M Service Layer that can be readily embedded within various hardware and software, and relied upon to connect the myriad of devices in the field with M2M application servers worldwide.

More information about oneM2M may be found at: http//www.oneM2M.org

Copyright Notification

No part of this document may be reproduced, in an electronic retrieval system or otherwise, except as authorized by written permission.

The copyright and the foregoing restriction extend to reproduction in all media.

© 2014, oneM2M Partners Type 1 (ARIB, ATIS, CCSA, ETSI, TIA, TTA, TTC).

All rights reserved.

Notice of Disclaimer & Limitation of Liability

The information provided in this document is directed solely to professionals who have the appropriate degree of experience to understand and interpret its contents in accordance with generally accepted engineering or other professional standards and applicable regulations. No recommendation as to products or vendors is made or should be implied.

NO REPRESENTATION OR WARRANTY IS MADE THAT THE INFORMATION IS TECHNICALLY ACCURATE OR SUFFICIENT OR CONFORMS TO ANY STATUTE, GOVERNMENTAL RULE OR REGULATION, AND FURTHER, NO REPRESENTATION OR WARRANTY IS MADE OF MERCHANTABILITY OR FITNESS FOR ANY PARTICULAR PURPOSE OR AGAINST INFRINGEMENT OF INTELLECTUAL PROPERTY RIGHTS. NO oneM2M PARTNER TYPE 1 SHALL BE LIABLE, BEYOND THE AMOUNT OF ANY SUM RECEIVED IN PAYMENT BY THAT PARTNER FOR THIS DOCUMENT, WITH RESPECT TO ANY CLAIM, AND IN NO EVENT SHALL oneM2M BE LIABLE FOR LOST PROFITS OR OTHER INCIDENTAL OR CONSEQUENTIAL DAMAGES. oneM2M EXPRESSLY ADVISES ANY AND ALL USE OF OR RELIANCE UPON THIS INFORMATION PROVIDED IN THIS DOCUMENT IS AT THE RISK OF THE USER.

# Contents

Contents	3

1	Scope	8

2	References	8

2.1	Normative references	8

2.2	Informative references	8

3	Definitions and abbreviations	8

3.1	Definitions	8

3.3	Abbreviations	9

4	Conventions	9

5	M2M Services Architecture	10

5.1	Introduction	10

5.2	Reference Points	11

5.2.1	Mca Reference Point	11

5.2.2	Msc Reference Point	11

5.2.2.1	 Message Exchange Patterns	11

5.2.2.2	 M2M Identifiers	12

5.2.2.2.1	 M2M Service Capability Identifier (M2M-Serv-Cap-ID)	12

5.2.3	Mcc’ Reference Point	13

5.2.4	Mcn Reference Point	13

5.3	Configurations support by M2M Service Architecture	13

6	M2M Services	14

6.1	Introduction	14

6.2	Service Subscription	14

6.2.1	Overview	14

6.2.1.1	M2M Service Subscription Entities	14

6.2.1.2	M2M Service Subscription Broker Entities	15

6.2.1.3	M2M Service Subscription Management Adapter Entities	15

6.2.1.4	M2M Service Subscription Transport Adapter Entities	16

6.2.2	Service Capabilities	17

6.2.2.1	validateServiceSubscription	17

6.2.2.2	getAE	18

6.2.2.3	getBroker	20

6.2.2.4	getManagementAdapter	21

6.2.2.5	getTransportAdapter	22

6.2.2.6	associateAEWithServiceSubscription	23

6.2.2.7	disassociateAEFromServiceSubscription	25

6.2.2.8	refreshAEAssociationWithServiceSubscription	27

6.3	Authorization	29

6.3.1	Service Capabilities	29

6.3.1.1	authorizeServiceCapability	29

6.4	Data Exchange	31

6.4.1	Overview	31

6.4.1.1	Subscribe-Publish-Notify Data Exchange	31

6.4.1.2	Request-Response Data Exchange	32

6.4.2	Service Capabilities	32

6.4.2.1	subscribe	32

6.4.2.2	publish	35

6.4.2.3	notify	37

6.4.2.4	sendMessage	38

6.5	Broker	40

6.5.1	Overview	40

6.5.2	Service Capabilities	40

6.5.2.1	subscribe	40

6.5.2.2	publish	41

6.5.2.3	notify	42

6.5.2.4	sendMessage	45

6.6	Device Management	46

6.6.1	Overview	46

6.6.1.1	Device Management	46

6.6.2	Service Capabilities	53

6.6.2.1	downloadFirmware	53

6.6.2.2	installFirmware	55

6.6.2.3 	getFirmwareInformation	57

6.6.2.4 	getFirmwareExecStatus	59

6.6.2.5	deviceManagementReport	61

6.6.2.6	upgradeFirmware	62

6.6.2.7	getDeviceInformation	64

6.6.2.8	getDeviceCapabilities	66

6.6.2.9 	enableDeviceCapability	68

6.6.2.10	disableDeviceCapability	70

6.6.2.11	getAreaNetworks	72

6.6.2.12	updateDeviceForAreaNetwork	74

6.6.2.13	rebootDevice	76

6.6.2.14	resetDevice	78

6.6.2.15 	uploadDeviceLog	80

6.6.2.16

getDeviceLogs	82

6.6.2.17	getDeviceLogInformation	84

6.6.2.18	getSoftwareInformation	86

6.6.2.19	downloadSoftware	88

6.6.2.20 	installSoftware	90

6.6.2.21	 activateSoftware	92

6.6.2.22 	deactivateSoftware	94

6.6.2.23	removeSoftware	96

6.7	Management Adapter	98

6.7.1	Overview	98

6.7.2	Service Capabilities	98

6.7.2.1	downloadFirmware	98

6.7.2.2	installFirmware	98

6.7.2.3	getFirmwareInformation	100

6.7.2.4	getFirmwareExecStatus	101

6.7.2.5	deviceManagementReport	102

6.7.2.6	getDeviceInformation	104

6.7.2.7	getDeviceCapabilities	105

6.7.2.8	enableDeviceCapability	106

6.7.2.9	disableDeviceCapability	107

6.7.2.10	getAreaNetworks	108

6.7.2.11	updateDeviceForAreaNetwork	109

6.7.2.12	rebootDevice	110

6.7.2.13	resetDevice	111

6.7.2.14	uploadDeviceLog	112

6.7.2.15	getDeviceLogs	113

6.7.2.16	getDeviceLogInformation	114

6.7.2.17	getSoftwareInformation	115

6.7.2.18	downloadSoftware	116

6.7.2.19	installSoftware	117

6.7.2.20	activateSoftware	118

6.7.2.21	deactivateSoftware	119

6.7.2.22	removeSoftware	120

6.8	Service Administration	121

6.8.1	Overview	121

6.8.1.1	M2M Service Entity	121

6.8.2	Service Capabilities	123

6.8.2.1	createM2MService	123

6.8.2.2	deleteM2MService	125

6.8.2.3	addRoleToM2MService	127

6.8.2.4	deleteRoleFromM2MService	129

6.8.2.5	getM2MService	131

6.8.2.6	addServiceCapabilityToRole	132

6.8.2.7	deleteServiceCapabilityFromRole	133

6.8.2.8	getServiceCapability	134

6.9	Service Subscription Administration	136

6.9.1	Overview	136

6.9.1.1	Service Subscription Entity	136

6.9.1.2	M2M Node Entity for Service Subscription	136

6.9.1.2.1	M2M Node Filter Criteria	137

6.9.1.3	Application Rule	137

6.9.1.3.1	Application Rule Filter Criteria	138

6.9.1.4	Supporting Rules	138

6.9.2	Service Capabilities	139

6.9.2.1	createServiceSubscription	139

6.9.2.2	deleteServiceSubscription	141

6.9.2.3	updateServiceSubscription	143

6.9.2.4	addRoleToServiceSubscription	145

6.9.2.5	deleteRoleFromServiceSubscription	147

6.9.2.6	getServiceSubscription	149

6.9.2.7	addNodeToServiceSubscription	151

6.9.2.8	deleteNodesFromServiceSubscription	153

6.9.2.9	getNodesForServiceSubscription	155

6.9.2.10	createApplicationRule	157

6.9.2.11	deleteApplicationRules	159

6.9.2.12	getApplicationRules	161

6.9.2.13	updateNodeForServiceSubscription	163

6.9.2.14	updateApplicationRule	165

6.9.2.15	addReachabilitySchedule	167

6.9.2.16	updateReachabilitySchedule	169

6.9.2.17	getReachabilitySchedule	171

6.9.2.18	deleteReachabilitySchedule	173

6.10	Event Collection	174

6.10.1	Overview	174

6.10.1.1	Event Collection Types	174

6.10.2	Service Capabilities	175

6.10.2.1	setEventCollectionPolicy	175

6.10.2.2	getEventCollectionPolicy	177

6.10.2.3	setEventCollectionTriggers	179

6.10.2.4	getEventCollectionTriggers	181

6.10.2.5	recordEvent	183

6.10.2.6	getEventRecords	185

6.11	Registration	187

6.11.1	Overview	187

6.11.2	Service Capabilities	187

6.11.2.1	registerAE	187

6.11.2.2	refreshAERegistration	189

6.11.2.3	deregisterAE	191

6.12	Registration Administration	193

6.12.1	Overview	193

6.12.2	Service Capabilities	193

6.12.2.1	getRegistrationStatus	193

6.12.2.2	revokeAERegistration	195

6.12.2.3	subscribeInitialAERegistrationEvent	197

6.12.2.4	unsubscribeInitialAERegistrationEvent	199

7	M2M Service Components	201

7.1	Introduction	201

7.1.1	Service Component Interaction Cross Reference	201

7.2	Infrastructure Component (INF)	201

7.2.1	INF to Service Cross Reference	201

7.3	Service Subscription Component (SSUB)	202

7.3.1	SSUB to Service Cross Reference	202

7.4	Transport Adapter (TRA)	202

7.4.1	TRA to Service Cross Reference	202

7.5	Accounting (ACC)	202

7.5.1	Accounting to Service Cross Reference	202

7.6	Service Exposure (SE)	202

7.6.1	Service Exposure to Service Cross Reference	202

7.7	Management Adapter (MA)	203

7.7.1	Management Adapter to Service Cross Reference	203

7.8	Device Management (DM)	203

7.8.1	Device Management to Service Cross Reference	203

Proforma copyright release text block	204

Annex A: Common Request Processing	205

A.1	Overview	205

A.2	Mca Common Request Processing	205

A.2.1	Mca Common Request Parameters (Normative)	205

A.2.2	Authentication and Authorization of Requests	206

A.2.2.1	Pre-conditions	206

A.2.2.2	Common M2M Service Capability Parameters for Request Authentication and Authorization	206

A.2.2.3	Service Interactions	206

A.2.2.4	Post-Conditions	207

A.2.2.5	Exceptions	207

A.2.2.6	Policies for Use	207

A.3	Msc Common Request Processing	208

A.3.1	Msc Common Request Data Types (Normative)	208

A.4	Common Entity Attributes	208

A.4.1	System Generated Attributes (Normative)	208

A.4.2	User Defined Discovery Attributes (Normative)	208

A.5	Using Filter Criteria	209

Annex B (Informative): Data Exchange Services	210

B.1	Overview	210

B.2	Supporting Services	210

B.2.1	Subscribe-Publish-Notify Message Exchange	210

B.2.1.1	Service Capabilities	210

B.2.2	Request-Response Message Exchange	220

B.2.2.1	Service Capabilities	220

Annex C (Informative): Service Subscription Administration Services	223

C.1	Overview	223

C.2	Supporting Services	223

C.2.1	Service Capabilities	223

Annex D (Informative): Device Management Services	241

D.1	Overview	241

D.2	Supporting Services	241

D.2.1	Message Exchange	241

D.2.1.1	Service Capabilities	241

Annex E (Informative): Device On-boarding Service	287

E.1	Overview	287

E.2	Supporting Services	287

E.2.1	Remote Administration	287

E.2.1.1	Service Capabilities	287

Annex F (Informative): Registration Services	289

F.1	Overview	289

F.2	Mca Registration Service Processing and Supporting Services	289

F.2.1	Message Exchanges	289

F.2.1.1	Service Capabilities	289

Annex X(Informative): M2M Service Capability Template	295

X.1	<serviceCapabilityName>	295

X.1.1	Signature	295

X.1.2	Pre-conditions	296

X.1.3	Service Interactions	296

X.1.4	Post-Conditions	296

X.1.5	Exceptions	296

X.1.6	Policies for Use	296

X.1.7	oneM2M Resource Interworking	296

Annex Y(Informative): oneM2M Service Requirements	297

History	298

# 1	Scope

This specification describes the M2M Services provided by the oneM2M Services Platform, the integration and interworking of the M2M Services functional architecture of the oneM2M Services Platform and informatively illustrates the use of the M2M Services within the context of complex business services.

# 2	References

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

## 2.1	Normative references

The following referenced documents are necessary, partially or totally, for the application of the present document. Their use in the context of this TS is specified by the normative statements that are referring back to this clause.

[1]	oneM2M TS-0002: “oneM2M Requirements”

[2]	oneM2M TS-0001: “oneM2M Functional Architecture”

## 2.2	Informative references

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	oneM2M Drafting Rules (http://member.onem2m.org/Static_pages/Others/Rules_Pages/oneM2M-Drafting-Rules-V1_0.doc)

[i.2]	W3C WD-wsdl20-patterns-20040326:”Web Services Description Language (WSDL) Version 2.0 Part 2: Message Exchange Patterns.”

[i.3]	 OASIS soa-rm: “Reference Model for Service Oriented Architecture 1.0”

[i.4]	oneM2M TS-0003: “oneM2M Security Solutions”.

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions apply:

M2M Service: A Service that creates added-value to the providing M2M Service Platform and is consumed by one or more Supporting Services. The Data Exchange Service is an example of a M2M Service.

M2M Service Capability: A Service Capability that is identified within the M2M Service Platform

M2M Service Registration: When a Service is deployed into a SOA environment; the SOA environment’s Service Registry is updated so that the Service can be accessed by other Services.

Proprietary Reference Point: A Reference Point as defined in Functional Architecture [2] where the realization of the communication flows are implementation specific.

Service[i.3]: A mechanism to enable access to one or more Service Capabilities, where the access is provided using a prescribed interface and is exercised consistent with constraints and policies as specified by the service description.

Service Capability[i.3]: A real-world effect that a service provider is able to provide to a service consumer.

Service Cluster: A collection of distributed and related Services that are gathered to solve a business problem.

Service Component: An entity of the M2M Services Architecture which may contain one or more M2M Services.

Service Execution Environment: An Execution Environment is a logical entity that represents an environment capable of running Services.

Service Exposure Component: A Service Component that provides an enforcement point for specific oneM2M Reference Points (e.g. Mca, Mcn, Mcc’).

Service Registry: A component of an underlying SOA environment that maintains information about Services and provides facilities to publish and discover Services to entities that would utilize the Service.

Supporting Service: A Service that creates added-value to an organization and is relevant to the business process of the organization that consumes Services from the M2M Services Platform. Supporting Services are typically complex, orchestrated Services which consume M2M Services among other Services. A Device Onboarding Service is an example of a Supporting Service.

## 3.3	Abbreviations

For the purposes of the present document, the following apply:

AE	Application Entity

BSS	Business Support System

CSE	Common Service Entity

ESB	Enterprise Service Bus

MEP	Message Exchange Pattern

M2M	Machine-to-Machine

M2M-Serv-Cap-ID	M2M Service Capability Identifier

NMS	Network Management System

OSS	Operational Support System

SEE	Service Execution Environment

# 4	Conventions

The key words “Shall”, ”Shall not”, “May”, ”Need not”, “Should”, ”Should not” in this document are to be interpreted as described in the oneM2M Drafting Rules [i.1]

# 5	M2M Services Architecture

This clause describes the architecture the M2M Services within the context of the M2M Services Platform.

## 5.1	Introduction

The architecture of the M2M Service Platform as defined in the oneM2M Functional Architecture [2] includes an entity described as the Common Service Entity (CSE). The CSE comprises a set of service functions that are common to the M2M environment and are exposed through the Mca and Mcc’ Reference Points. These Reference Points are described in the oneM2M Functional Architecture [2]. The M2M Service Architecture described in this specification is primarily suitable for the Infrastructure Domain where the CSE is viewed as a set of Service Components.

The M2M Service Architecture augments the oneM2M Functional Architecture by specifying M2M Services provided to M2M Application and M2M Service Providers.

These M2M Services are consumed by:

AEs across the Mca Reference Point via the Service Exposure Component

Other Infrastructure CSEs across the Mcc’ Reference Point via the Remote Service Exposure Component

Other Service Components across the Msc Reference Point

These M2M Services utilize services of the:

Underlying Network across the Mcn Reference Point via the Network Service Utilization Component

Figure 5.1-2: oneM2M Services Architecture

The oneM2M Services architecture in Figure 5.1-2 comprises of the following entities:

- Application Entity (AE): Defined by the oneM2M Functional Architecture [2], the Application Entity provides Application logic for the end-to-end M2M solutions.

- Common Services Entity (CSE): Defined by the oneM2M Functional Architecture [2], a Common Services Entity comprises the set of “service functions” that are common to the M2M environments and specified by oneM2M. For oneM2M Services, this definition of a CSE is a logical representation where “service functions” that are exposed through the Mca and Mcc’ Reference Points via the corresponding Service Exposure and Remote Service Exposure Components and the Network Service Utilization Component utilizes services of the Underlying Network through the Mcn referent point. In addition Service Components consume and provide M2M Services with other Service Components.

As a logical representation of loosely coupled Service Components, the CSE is entity that in itself is identifiable but not directly addressable. Instead the addressable entities are the corresponding M2M Service Capabilities within the Service Exposure Components of the Reference Points.

  - Service Exposure Component: The Service Exposure Component exposes M2M Service Capabilities to AEs.

  - Network Service Utilization Component: The Network Service Utilization Component consumes Service Capabilities from the NSE.

  - Remote Service Exposure Component: The Remote Service Exposure connects M2M Service Capabilities from different M2M environments.

The Service Exposure, Network Service Utilization and Remote Service Exposure Components follow the CSE Public Domain Names convention described in section 6.5.1.1 of the Functional Architecture [2] but are extended as a sub-domain of the Infrastructure Node public domain name.

## 5.2	Reference Points

The M2M Service Architecture exposes M2M Services across the Mca and Msc Reference Points. The enhancement of the Mcc and Mcc’ Reference Point to expose M2M Services through the Remote Service Exposure Component is FFS. Likewise the Service Capabilities consumed from the NSE are FFS.

### 5.2.1	Mca Reference Point

The Mca Reference Point is defined within the oneM2M Functional Architecture [2]. This specification defines a set of service oriented capabilities that is used for the communication flow between the M2M Service Platform and the AE.

Note: In this specification, when an AE uses the Data Exchange M2M Service Capabilities defined in this specification across the Mca Reference Point, the CSE transmits the payload through the M2M Service Platform; offering additional M2M Services (e.g., Authorization, Event Collection) associated with the transmittal of the payload.

### 5.2.2	Msc Reference Point

The Msc Reference Point is a Proprietary Reference Point that specifies set of interactions between the Service Capabilities of different Service Components.

### 5.2.2.1	 Message Exchange Patterns

Communication between M2M Service Components which pass over the Msc Reference Point utilizes a web services approach, e.g., Web Services Message Exchange Patterns (MEP) defined by WSDL[i.2]:

- In-Only

- Robust In-Only

- In-Out

- In-Optional-Out

- Out-Only

- Robust Out-Only

- Out-In

- Out-Optional-In

Figure 5.2.2.1-1: Message Exchange Patterns

The MEP(s) utilized by a M2M Service Capability is documented within the Service Capabilities Policy clause.

Note: A subset of the MEPs provided by a M2M Service Capability would be exposed by the Service Exposure Component or the Remote Service Exposure Component.

### 5.2.2.2	 M2M Identifiers

### 5.2.2.2.1	 M2M Service Capability Identifier (M2M-Serv-Cap-ID)

Service Components contain multiple M2M Services (M2M-Serv-ID). Each M2M Service contains multiple M2M Service Capabilities.  The M2M Service Capability Identifier is assigned to each M2M Service Capability.

| Identifier | Assigned by | Assigned to | Assigned during | Lifetime | Uniqueness | Used during | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M2M Service Capability Identifier | M2M SP | M2M Service Capability | M2M Service Registration. This is the identifier from the Services Registry. | Life of the M2M Service Registration. | Global | Message exchange between Service Components across the Msc and Mcc’. Message exchange across the Mca. |  |

Table 5.2.2.2.1-1 M2M Service Capability Identifier

### 5.2.3	Mcc’ Reference Point

The Mcc’ Reference Point is defined within the oneM2M Functional Architecture [2]. The oneM2M Service Architecture utilizes communications over the Mcc’ Reference Point to construct a Service Cluster for service sharing and capacity expansion. A messaging exchange protocol (e.g., AMQP, XMPP, JMS) between two service execution environments (SEE) (e.g., ESB, Apache Service Mix) accommodates the SEEs with same and different ownership.

Note: The M2M Service Capabilities that are exposed across the Mcc’ Reference Point is FFS. Likewise, the protocol used for communication between SEEs with different ownership may require additional security capabilities and is FFS.

### 5.2.4	Mcn Reference Point

The Mcn Reference Point is defined within the oneM2M Functional Architecture [2].

Note: The Service Capabilities that are consumed across the Mcn Reference Point is FFS.

## 5.3	Configurations support by M2M Service Architecture

M2M Services exposed across the Mca, Mcn, Mcc’ and Msc reference are supported for Infrastructure Nodes as defined in the oneM2M Functional Architecture [2].

# 6	M2M Services

This clause describes the M2M Services provided by the M2M Services Platform.

## 6.1	Introduction

The M2M Services defined in this clause are utilized by the Supporting Services described in the Annexes of this specification.

Figure 6.1-1: M2M Services

## 6.2	Service Subscription

### 6.2.1	Overview

The Service Subscription service provides the ability to:

Validate the M2M Service Capabilities (e.g., requests, notifications) that go across the Mca Reference Point by ensuring M2M Service requested by the originator is permitted within the context of the M2M Service Subscription

Retrieve the Application Entity and associated M2M Service Subscription for the Application Entity

Assist in returning the Broker and Transport Adapter  service instances needed to exchange data for data exchange service capabilities

Assist in returning the Management Adapter service instance needed to manage the device

#### 6.2.1.1	M2M Service Subscription Entities

The following diagram describes the associations between the AE, Service Roles and the M2M Service Capabilities to obtain an Application Entity and validate the M2M Service Subscription:

The M2M Service Subscription(s) for a specified M2M Service Capability is validated through the associations the Service Role has to the M2M Service Capability and M2M Service Subscription

The Application Entity and associated M2M Service Subscription is obtained through the Application Entity’s to the  M2M Service Subscription that is created during registration of the Application Entity

Figure 6.2.1.1-1 Service Subscription Entity Relationships

#### 6.2.1.2	M2M Service Subscription Broker Entities

In this specification Brokers  are used to exchange data between AEs where the AEs use the publish-subscribe data exchange pattern. Brokers are typically implemented using types of Message Queue technologies (e.g., MQTT, XMPP).

The following diagram describes the associations between the AE, M2M Service Subscription, M2M Service Capability and the Broker resource in order to obtain the Broker used to exchange the data

The Broker is obtained through the association of the Broker Resource and the M2M Service Capability.

The Broker Resource is associated with Applications which are themselves associated with the Application Entity in the context of a M2M Service Subscription

Figure 6.2.1.1-1 M2M Service Subscription for Broker Entity Relationships

#### 6.2.1.3	M2M Service Subscription Management Adapter Entities

The following diagram describes the associations between the M2M Service Subscription, M2M Nodes associated with the M2M Service Subscription and M2M Service Capability in order to obtain the Management Adapter.

The Management Adapter is obtained through the association of the M2M Node and the M2M Service Capability.

The M2M Node is associated with the M2M Service Subscription

Figure 6.2.1.2-1 M2M Service Subscription For Management Adapter Entity Relationships

#### 6.2.1.4	M2M Service Subscription Transport Adapter Entities

In this specification Transport Adapters for request-reply message exchange patterns. Transport Adapters are  typically implemented using types of Message technologies (e.g., WSDL, REST).

The following diagram describes the associations between the M2M Service Subscription, AE and M2M Service Capability in order to obtain the Transport Adapter.

The Transport Adapter is obtained through the association of the Application and the M2M Service Capability

The Application is associated with the M2M Service Subscription

The AE is associated with an Application and M2M Service Subscription AE registration

Figure 6.2.1.3-1 M2M Service Subscription For Transport Adapter Entity Relationships

### 6.2.2	Service Capabilities

#### 6.2.2.1	validateServiceSubscription

This service capability ensures that a valid M2M Service Subscription exists for the requested M2M Service Capability requested that is either received by the M2M Service Platform from an AE or is transmitted to the AE from the M2M Service Platform.

##### 6.2.2.1.1	Pre-conditions

A correlation between the M2M Service Capability, M2M Service Role(s) and the M2M Service Subscription has been defined in order to validate the M2M Service Subscription.

##### 6.2.2.1.2	Signature – validateServiceSubscription

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceSubscriptionId | IN | NO | The M2M Service Subscription Identifier (M2M-Service-Profile-ID) |
| serviceCapId | IN | NO | The M2M Service Capability Identifier (M2M-Serv-Cap-ID) |
| responseType | OUT | NO | Response types: - M2M Service Capability does not exist for requested M2M Service Subscription - M2M Service Capability does exist for the requested M2M Service Subscription |

Table 6.2.2.1.2-1 Service Subscription – validateServiceSubscription capability

##### 6.2.2.1.3	Post-Conditions

Not Applicable

##### 6.2.2.1.4	Exceptions

Not Applicable

##### 6.2.2.1.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.2.2.1.6	oneM2M Resource Interworking

This service capability interworks with the following oneM2M Resource operations that are interworked are:

RETRIEVE <m2MServiceSubscriptionProfile> resource retrieves the associated Service Roles. A M2M Service Capability is associated with the M2M Service Roles. The M2M Service Subscription is validated through the Service Roles associated with the M2M Service Capability.

#### 6.2.2.2	getAE

This service capability provides the ability to retrieve the Application Entity information and associated M2M Service Subscription Identifier associated with the Application Entity Identifier.

##### 6.2.2.2.1	Pre-conditions

Not Applicable

##### 6.2.2.2.2	Signature – getAE

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| aeId | IN | NO | The Application Entity Id (AE-ID) |
| applicationId | OUT | NO | The Application Identifier (App-ID) for the AE |
| serviceSubscriptionId | OUT | YES | The M2M Service Subscription (M2M-Service-Profile-ID) for the AE |
| externalIds | OUT | YES | The external identifiers of the device that hosts the AE |
| reachabilitySchedule | OUT | YES | The contact reachability schedule information of the AE associated with the device node. The absence of this parameter implies the AE associated with the device node is always contact reachable. Type Schedule, see 6.9.1.4.1. |
| responseType | OUT | YES | Unique response types for this service. - AE does not exist |

Table 6.2.2.2.2-1 Service Subscription – getAE capability

##### 6.2.2.2.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the service capability

Figure 6.2.2.2.3-1 Service Subscription – getAE Diagram

##### 6.2.2.2.4	Post-Conditions

Not Applicable

##### 6.2.2.2.5	Exceptions

Not Applicable

##### 6.2.2.2.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.2.2.2.7	oneM2M Resource Interworking

When an AE is registered using a CREATE operation on the <AE> resource, the AE is associated with the M2M Service Subscription which is returned along with the correlated AE information (e.g., Application, Device).

#### 6.2.2.3	getBroker

This service capability obtains the Broker instance that will be used to subscribe and publish requests based on the requested M2M Service Capability (e.g., publishRequest, subscribeRequest), AE, Resource and M2M Service Subscription.

For the publishRequest service capability the AE is the from AE-ID and the resource is the toResource representing a leaf node from the publication resource.

For the subscribeRequest service capability the AE is the from AE-ID and the resource is the publication resource.

##### 6.2.2.3.1	Pre-conditions

A correlation between a M2M Service Subscription, AE, Service Capability, Resource and Broker exist.

##### 6.2.2.3.2	Signature – getBroker

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceSubscriptionId | IN | NO | The M2M Service Subscription Identifier (M2M-Service-Profile-ID). |
| serviceCapId | IN | NO | The M2M Service Capability Identifier (M2M-Serv-Cap-ID). |
| aeId | IN | NO | The identifier of the AE |
| resource | IN | NO | The identifier of the resource that is associated with the AE and M2M Service Subscription |
| broker | OUT | NO | The returned instance of the Broker service associated with the M2M Service Subscription. |
| responseType | OUT | NO | Response types: - Broker does not exist for requested M2M Service Capability, M2M Service Subscription and Resource |

Table 6.2.2.3.2-1 Service Subscription – getBroker capability

##### 6.2.2.3.3	Post-Conditions

Not Applicable

##### 6.2.2.3.4	Exceptions

Not Applicable

##### 6.2.2.3.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.2.2.3.6	oneM2M Resource Interworking

This service capability interworks with the following oneM2M Resource operations that are interworked are:

Not Applicable

#### 6.2.2.4	getManagementAdapter

This service capability obtains the Management Adapter instance that will be used to manage device based on the requested M2M Service Capability (e.g., downloadFirmware, installFirmware) and device.

##### 6.2.2.4.1	Pre-conditions

A correlation between a Management Adapter, the M2M Service Capability and device exist.

##### 6.2.2.4.2	Signature – getManagementAdapter

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceSubscriptionId | IN | YES | The M2M Service Subscription Identifier (M2M-Service-Profile-ID) , is optional when the deviceId is unique across M2M Service Subscription. |
| serviceCapId | IN | NO | The M2M Service Capability Identifier (M2M-Serv-Cap-ID). |
| deviceId | IN | YES | The unique device identifier in the context of the M2M Service Subscription. |
| managementAdapter | OUT | NO | The returned instance of the Management Adapter service associated with the subscription. |
| responseType | OUT | NO | Response types: - Management Adapter does not exist |

Table 6.2.4.1.2-1 Service Subscription – getManagementAdapter capability

##### 6.2.2.4.3	Post-Conditions

Not Applicable

##### 6.2.2.4.4	Exceptions

Not Applicable

##### 6.2.2.4.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.2.2.4.6	oneM2M Resource Interworking

This service capability interworks with the following oneM2M Resource operations that are interworked are:

Not Applicable

#### 6.2.2.5	getTransportAdapter

This service capability obtains the Transport adapter instance that will be used to send requests based on the requested M2M Service Capability, AE and M2M Service Subscription.

##### 6.2.2.5.1	Pre-conditions

A correlation between a M2M Service Subscription, AE and type of transport adapter (e.g., Broker) service exist.

##### 6.2.2.5.2	Signature – getTransportAdapter

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceSubscriptionId | IN | NO | The M2M Service Subscription Identifier (M2M-Service-Profile-ID)’ |
| serviceCapId | IN | NO | The M2M Service Capability Identifier (M2M-Serv-Cap-ID). |
| aeId | IN | NO | The identifier of the AE that is to target of the request (“to”) (AE-ID) |
| transportAdapter | OUT | NO | The returned instance of the service (e.g., Broker) associated with the M2M Service Subscription. |
| responseType | OUT | NO | Response types: - Transport Adapter does not exist for requested M2M Service Capability, M2M Service Subscription and target |

Table 6.2.2.5.2-1 Service Subscription – getTransportAdapter capability

##### 6.2.2.5.3	Post-Conditions

Not Applicable

##### 6.2.2.5.4	Exceptions

Not Applicable

##### 6.2.2.5.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.2.2.5.6	oneM2M Resource Interworking

Not Applicable

#### 6.2.2.6	associateAEWithServiceSubscription

This service capability provides the ability to associate an AE with a M2M Service Subscription. This service capability shall be restricted to the Msc Reference Point.

##### 6.2.2.6.1	Pre-conditions

One of the following combinations has to be unique within the M2M System for the AE to be associated with an M2M Service Subscription:

Identifier of Application Entity (AE-ID) or

Identifier of the M2M Application (App-ID) and the credential identifier for the M2M Application.

The M2M System is pre-provisioned with the Application Rule (clause 6.9.1.3) that includes one of the above combinations in order for a M2M Service Subscription to the returned.

##### 6.2.2.6.2	Signature – associateAEWithServiceSubscription

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| aeId | IN | NO | The Application Entity Identifier (AE-ID). |
| credentialId | IN | YES | The Application Credential Identifier |
| applicationId | IN | YES | The Application Identifier (App-ID) |
| serviceSubscriptionId | OUT | NO | The M2M Service Subscription Identifier (M2M-Service-Profile-ID) |
| responseType | OUT | YES | Unique response types for this service. - M2M Service Subscription does not exist. - Application Entity Identifier is not unique within the context of the M2M System. |

Table 6.2.2.6.2-1 Service Subscription – associateAEWithServiceSubscription capability

##### 6.2.2.6.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue this capability

Figure 6.2.2.6.3-1 Service Subscription – associateAEWithServiceSubscription Diagram

##### 6.2.2.6.4	Post-Conditions

The Application Entity Identifier (AE-ID) provided as the input parameter is associated with the M2M Service Subscription.

##### 6.2.2.6.5	Exceptions

Not Applicable

##### 6.2.2.6.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.2.2.6.7	oneM2M Resource Interworking

This service capability associates the AE-ID with the M2M Service Subscription Profile. Currently TS-0001 [2] does not define how an AE is associated with a M2M Service Subscription Profile as part of a registration flow.

#### 6.2.2.7	disassociateAEFromServiceSubscription

This service capability provides the ability to remove the association of  an AE from a M2M Service Subscription. This service capability shall be restricted to the Msc Reference Point.

##### 6.2.2.7.1	Pre-conditions

An AE has successfylly registered with the M2M system

##### 6.2.2.7.2	Signature – disassociateAEFromServiceSubscription

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| aeId | IN | NO | The Application Entity Identifier (AE-ID). |
| serviceSubscriptionId | IN | NO | The M2M Service Subscription Identifier (M2M-Service-Profile-ID) |
| responseType | OUT | YES | Unique response types for this service. - M2M Service Subscription does not exist. - Application Entity Identifier is not associated with the M2M Service Subscription. |

Table 6.2.2.7.2-1 Service Subscription – disassociateAEFromServiceSubscription capability

##### 6.2.2.7.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue this capability

Figure 6.2.2.7.3-1 Service Subscription – disassociateAEFromServiceSubscription Diagram

##### 6.2.2.7.4	Post-Conditions

The Application Entity registration association is removed from the M2M Service Subscription.

##### 6.2.2.7.5	Exceptions

Not Applicable

##### 6.2.2.7.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.2.2.7.7	oneM2M Resource Interworking

This service capability removes the association of the AE-ID from the M2M Service Subscription Profile. Currently TS-0001 [2] does not define how an AE is disassociated from a M2M Service Subscription Profile as part of a AE lifecycle flow.

#### 6.2.2.8	refreshAEAssociationWithServiceSubscription

This service capability provides the ability to refresh an existing registration association between an Application Entity and a M2M Service Subscription. This service capability shall be restricted to the Mca and Msc Reference Points.

##### 6.2.2.8.1	Pre-conditions

An AE has successfully registered with the M2M system

##### 6.2.2.8.2	Signature – refreshAEAssociationWithServiceSubscription

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| aeId | IN | NO | The Application Entity Identifier (AE-ID). |
| serviceSubscriptionId | IN | NO | The M2M Service Subscription Identifier (M2M-Service-Profile-ID) |
| responseType | OUT | YES | Unique response types for this service. - M2M Service Subscription does not exist. - Application Entity Identifier is not associated with the M2M Service Subscription. |

Table 6.2.2.8.2-1 Service Subscription – refreshAEAssociationFWithServiceSubscription capability

##### 6.2.2.8.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue this capability

Figure 6.2.2.8.3-1 Service Subscription – refreshAEAssociationWithServiceSubscription Diagram

##### 6.2.2.8.4	Post-Conditions

Not Applicable

##### 6.2.2.8.5	Exceptions

Not Applicable

##### 6.2.2.8.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.2.2.8.7	oneM2M Resource Interworking

This service capability refreshes the association of the Application Entity (AE-ID) with the M2M Service Subscription Profile. Currently TS-0001 [2] does not define how an AE registration association is refreshed with a M2M Service Subscription Profile as part of a AE lifecycle flow.

## 6.3	Authorization

The Authorization service provides the ability to:

- Authorize the Originator for the service capability

Editors Note: Need to determine how the entity instance based access management is configured in order to ensure no cross ASP access to other ASPs data.

### 6.3.1	Service Capabilities

#### 6.3.1.1	authorizeServiceCapability

This service capability authenticates an Originator is a valid Originator of the request and then authorizes the M2M Service Capability within the context of a M2M Service Subscription between the from AE-ID and to AE-ID.

##### 6.3.1.1.1	Pre-conditions

The M2M Identifiers for the request Originator, M2M Service Subscription from AE-ID and to AE-ID are assigned to the M2M Service Subscription.

A correlation between the M2M Service Capability, M2M Service and the M2M authorization event has been defined in order to authorize the M2M Service Capability.

##### 6.3.1.1.2	Common M2M Service Capability Parameters for Request Authentication and Authorization

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| originator | IN | NO | The identifier of the Originator that is issuing the request. |
| from | IN | NO | The identifier of the Originator of the request (AE-ID) |
| to | IN | NO | The identifier of the AE that is to target of the request (AE-ID) |
| serviceCapId | IN | NO | The identifier of the M2M Service Capability (M2M-Serv-Cap-ID) |
| serviceSubscriptionId | IN | NO | The M2M Service Subscription Identifier (M2M-Service-Profile-ID) |
| responseType | OUT | YES | Response types that are relevant to the Authentication and Authorizations of requests from AEs across the Mca Reference Point. - Originator is not authenticated - Originator does not have a M2M Service Subscription - Originator not authorized for the M2M Service Capability |

Table 6.3.1.1.2-1 Common M2M Service Capability Parameters for Request Authentication and Authorization

##### 6.3.1.1.3	Service Interactions

Not Applicable

##### 6.3.1.1.4	Post-Conditions

Not Applicable

##### 6.3.1.1.5	Exceptions

Not Applicable

##### 6.3.1.1.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.3.1.1.7	oneM2M Resource Interworking

This service capability interworks with the following oneM2M Resource operations that are interworked are:

- FFS – The End-to-End Security Framework is expected to develop APIs that can be issued for this purposed.

## 6.4	Data Exchange

### 6.4.1	Overview

The Data Exchange service provides the ability to exchange payloads between AEs across the Mca Reference Point using supported data exchange patterns. This service utilizes an underlying transport adapter to provide the transportation functions.

The supported data exchange patterns are:

Subscribe-Publish-Notify

Request-Response

#### 6.4.1.1	Subscribe-Publish-Notify Data Exchange

This data exchange pattern permits AEs to subscribe to publication resources. When an AE publishes data to a specific publication resource, the AEs that have subscribed to the publication resource are notified of the publication by receiving the published data.

##### 6.4.1.1.1	Supporting Rules

A publication resource is an entity that represents the mechanism used by underlying publish and subscribe protocols which represent one or more queues in the underlying protocol. A publication resource in the data exchange service provides capabilities to reference one or more queues in the underlying protocol using a tree mechanism where wildcards are placed to represent all elements of the tree branch or leaf.

###### 6.4.1.1.1.1	Publication Resources

When a AE publishes a payload to a publication resource, the publication resource is evaluated to obtain the leaf node of the tree. The AEs that have subscribed to leaf nodes of the tree receive the message via the notify M2M Service Capability.

###### 6.4.1.1.1.2	Delivery Policy

As part of the notify M2M Service Capability the Service Layer attempts to honor the delivery policy of the subscriber and publisher. The delivery policy is the attempt by the Service Layer to provide a reliable delivery of the payload that was published using the mechanisms of the underlying Broker. The reliable delivery of a payload is defined in the deliveryPolicy type.

| Attribute | Description |
| --- | --- |
| retryLimit | The number of retries by the Broker to connected subscribers before assuming failure of the notification. |

Table 6.4.1.1.1..2-1 Type: deliveryPolicy

###### 6.4.1.1.1.3	Message Retainment Policy

When a message is published, the M2M Service Layer, using the resources of the underlying Broker, attempts to retain messages according to a retainment policy. Retainment of messages permits AEs that are subscribed to the publication resource but not connected to the underlying Broker to receive messages, up to the retainLimit once the AE connects to the Broker. The message retainment policy is defined in the retainmentPolicy type.

| Attribute | Description |
| --- | --- |
| retainLimit | The maximum number of messages retained by the Broker for a subscriber that is not connected to the underlying Broker. |

Table 6.4.1.1.1..3-1 Type: retainmentPolicy

###### 6.4.1.1.1.4	Service Subscription Integration

In order for the Data Exchange Service to take advantage of the underlying transport, there shall be an association between the AE Resource and the identifier of the underlying transport within the context of the M2M Service Subscription.

#### 6.4.1.2	Request-Response Data Exchange

This data exchange pattern permits AEs to synchronously send messages to other AEs utilizing the underlying transport.

##### 6.4.1.2.1	Supporting Rules

Messages are sent to from an AE to another AE via the Service Layer. When sending a message the originating AE can provide a delivery policy to be used by the transport adapter when sending the message to the target AE.

###### 6.4.1.1.2.2	Delivery Policy

As part of the message sending capability the Service Layer attempts to honor the delivery policy of the message Originator. The delivery policy is the attempt by the Service Layer to provide a reliable delivery of the payload that was sent using the mechanisms of the transport adapter. The reliable delivery of a payload is defined in the deliveryPolicy type.

| Attribute | Description |
| --- | --- |
| retryLimit | The number of retries by the transport adapter to target AEs before assuming failure of the notification. |

Table 6.4.1.1.2.2-1 Type: deliveryPolicy

###### 6.4.1.1.2.3	Service Subscription Integration

In order for the Data Exchange Service to take advantage of the underlying transport, there shall be an association between the target AE and the identifier of the underlying transport within the context of the M2M Service Subscription.

### 6.4.2	Service Capabilities

#### 6.4.2.1	subscribe

This service capability provides the ability for AEs to subscribe to receive payloads based on a publication resource.

As part of the subscription, the subscribing AE can provide delivery and retainment policies to enhance the robustness of the notification to AEs that have subscribed to the publication resource identified in the request.

##### 6.4.2.1.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a M2M Service Subscription, publication resource, subscribing AE and Broker exist.

##### 6.4.2.1.2	Signature – subscribe

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| publicationResource | IN | NO | The publication resource. See 6.4.1.1.1.1 |
| deliveryPolicy | IN | YES | The delivery policy when notifying the subscriber. See 6.4.1.1.1.2 |
| retainmentPolicy | IN | YES | The retainment policy for unconnected subscribers. See 6.4.1.1.1.3 |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Exception: Request may not have been completed |

Table 6.4.2.1.2-1 Data Exchange Service – subscribe capability

##### 6.4.2.1.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the subscribe request to the Supporting Service to validate the request and obtain the Broker instance for the subscription request.

- Issue the request to the broker to subscribe the AE-ID for the publication resource

- Issue the subscribe complete subscription request to the Supporting Service

Figure 6.4.2.1.3-1 Data Exchange Service – subscribe capability

##### 6.4.2.1.4	Post-Conditions

Not Applicable

##### 6.4.2.1.5	Exceptions

Not Applicable

##### 6.4.2.1.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.4.2.1.7	oneM2M Resource Interworking

This service capability interworks with the following oneM2M Resource operations that are interworked are:

Not Applicable

#### 6.4.2.2	publish

This service capability provides the ability for AEs to publish payloads to a leaf node of a publication resource.

As part of the publication, the publishing AE can provide a delivery policy to enhance the robustness of the publication to AEs that have subscribed to the Resource identified in the request.

##### 6.4.2.2.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a M2M Service Subscription, resource to publish the payload, publishing AE and Broker exist.

##### 6.4.2.2.2	Signature – publish

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| toResource | IN | NO | The leaf node of a publication resource. See 6.4.1.1.1.1 |
| deliveryPolicy | IN | YES | The delivery policy when notifying the subscriber. See 6.4.1.1.1.2 |
| payload | IN | NO | The payload to published. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Exception: Request may not have been completed |

Table 6.4.2.2.2-1 Data Exchange Service – publish capability

##### 6.4.2.2.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the publish request to the Supporting Service to validate the request

- Issue the request to the broker to publish the payload from the publishing AE to the resource in the request

- Issue the complete publication request to the Supporting Service

Figure 6.4.2.2.3-1 Data Exchange Service –publish (In-Out) capability

##### 6.4.2.2.4	Post-Conditions

Not Applicable

##### 6.4.2.2.5	Exceptions

Not Applicable

##### 6.4.2.2.6	Policies for Use

Message Exchange Patterns: In-Out, In-Only

The In-Only message exchange pattern would return a transport layer acknowledgement if supported by the underlying transport layer protocol. Response types or Exceptions are not returned in this message exchange pattern.

Transaction Pattern: Participation allowed

##### 6.4.2.2.7	oneM2M Resource Interworking

This service capability interworks with the following oneM2M Resource operations that are interworked are:

Not Applicable

#### 6.4.2.3	notify

This service capability provides the ability to notify AEs that have subscribed to the publication resource identified in a publish request.

##### 6.4.2.3.1	Pre-conditions

A correlation between a M2M Service Subscription, subscribed AE, publication resource and Broker exist.

##### 6.4.2.3.2	Signature – notify

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| fromResource | IN | NO | The leaf node of a publication resource. See 6.4.1.1.1.1 |
| deliveryPolicy | IN | YES | The delivery policy when notifying the subscriber. See 6.4.1.1.1.2 |
| payload | IN | NO | The Payload of the message to notify to the AE |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Exception: Request may not have been completed |

Table 6.4.2.3.2-1 Data Exchange Service – notify capability

##### 6.4.2.3.3	Post-Conditions

Not Applicable

##### 6.4.2.3.4	Exceptions

Not Applicable

##### 6.4.2.3.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.4.2.3.6	oneM2M Resource Interworking

This service capability interworks with the following oneM2M Resource operations that are interworked are:

Not Applicable

#### 6.4.2.4	sendMessage

This service capability provides the ability for AEs to send messages to a target AE.

As part of the message send request, the originating AE can provide a delivery policy to enhance the robustness of the message to the AE.

##### 6.4.2.4.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a M2M Service Subscription, Originating AE and Transport Adapter exist.

##### 6.4.2.4.2	Signature – sendMessage

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deliveryPolicy | IN | YES | The delivery policy when sending to the target AE. See 6.4.1.1.2.2 |
| payload | IN | NO | The payload to be sent. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Exception: Request may not have been completed |

Table 6.4.2.4.2-1 Data Exchange Service – sendMessage capability

##### 6.4.2.4.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the sendMessage request to the Supporting Service to validate the request

- Issue the request to the transport adapter (in this case the Broker service) to send the payload from the Originator to the target AE.

- Issue the complete sendMessage request to the Supporting Service

Figure 6.4.2.4.3-1 Data Exchange Service –sendMessage capability

##### 6.4.2.4.4	Post-Conditions

Not Applicable

##### 6.4.2.4.5	Exceptions

Not Applicable

##### 6.4.2.4.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.4.2.4.7	oneM2M Resource Interworking

Not Applicable

## 6.5	Broker

### 6.5.1	Overview

The Broker service provides the ability to adapt the Publish-Subscribe-Notify and Request Response data exchange patterns to the underlying transport protocol and deployment configuration.

The Broker maintains the subscriptions to the publication resources

### 6.5.2	Service Capabilities

#### 6.5.2.1	subscribe

This service capability provides the ability for AEs to subscribe to receive payloads based on a publication resource.

As part of the publication, the publishing AE can provide delivery and retainment policies to enhance the robustness of the notification to AEs that have subscribed to the Resource identified in the request.

##### 6.5.2.1.1	Pre-conditions

Not Applicable

##### 6.5.2.1.2	Signature – subscribe

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| publicationResource | IN | NO | The publication resource. See 6.4.1.1.1.1 |
| deliveryPolicy | IN | YES | The delivery policy when notifying the subscriber. See 6.4.1.1.1.2 |
| retainmentPolicy | IN | YES | The retainment policy for unconnected subscribers. See 6.4.1.1.1.3 |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Exception: Request may not have been completed |

Table 6.5.2.2.2-1 Broker – subscribe capability

##### 6.5.2.1.3	Post-Conditions

Not Applicable

##### 6.5.2.1.4	Exceptions

Not Applicable

##### 6.5.2.1.5	Policies for Use

Message Exchange Patterns: In-Out

The In-Only message exchange pattern would return a transport layer acknowledgement if supported by the underlying Transaction Pattern: Participation allowed

#### 6.5.2.2	publish

This service capability provides the ability for AEs to publish payloads to a leaf node of a publication resource.

As part of the publication, the publishing AE can provide a delivery policy to enhance the robustness of the publication to AEs that have subscribed to the Resource identified in the request.

##### 6.5.2.2.1	Pre-conditions

Not Applicable.

##### 6.5.2.2.2	Signature – publish

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| toResource | IN | NO | The leaf node of a publication resource. See 6.4.1.1.1.1 |
| deliveryPolicy | IN | YES | The delivery policy when notifying the subscriber. See 6.4.1.1.1.2 |
| payload | IN | NO | Payload to be published. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Exception: Request may not have been completed |

Table 6.5.2.2.2-1 Broker – publish capability

##### 6.5.2.3.3	Post-Conditions

Not Applicable

##### 6.5.2.3.4	Exceptions

Not Applicable

##### 6.5.2.3.5	Policies for Use

Message Exchange Patterns: In-Out

The In-Only message exchange pattern would return a transport layer acknowledgement if supported by the underlying Transaction Pattern: Participation allowed

##### 6.5.2.3.6	oneM2M Resource Interworking

This service capability interworks with the following oneM2M Resource operations that are interworked are:

Not Applicable

#### 6.5.2.3	notify

This service capability provides the ability for the Broker to notify AEs that have subscribed to the publication resource identified in the request.

##### 6.5.2.3.1	Pre-conditions

The Broker maintains the AEs (to AE-ID, clientId) that have subscribed to publicationResources.

The Broker maintains the M2M Service Subscription (serviceSubscriptionId) for AE subscriptions to publicationResources.

##### 6.5.2.3.2	Signature – notify

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| payload | IN | NO | See Table A.2.1-1 |
| fromResource | IN | NO | The leaf node of a publication resource. See 6.4.1.1.1.1 |
| deliveryPolicy | IN | YES | The delivery policy when notifying the subscriber. See 6.4.1.1.1.2 |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Originator does not have a Broker for the requested resource - Originator has requested an invalid delivery policy - Exception: Request may not have been completed |

Table 6.5.2.3.2-1 Broker – notify capability

##### 6.5.2.3.3	Service Interactions

For each AE subscribed to the resource perform the interactions of service capabilities required for this service capability:

- Issue the notify request to the Supporting Service to validate the request

- Issue the request to the SE: Data Exchange

- Issue the complete request to the Supporting Service

Figure 6.5.2.3.3-1 Broker –notify (In-Out) capability

##### 6.5.2.3.4	Post-Conditions

Not Applicable

##### 6.5.2.3.5	Exceptions

Not Applicable

##### 6.5.2.3.6	Policies for Use

Message Exchange Patterns: In-Out, In-Only

The In-Only message exchange pattern would return a transport layer acknowledgement if supported by the underlying transport layer protocol. Response types or Exceptions are not returned in this message exchange pattern.

Transaction Pattern: Participation allowed

##### 6.5.2.3.7	oneM2M Resource Interworking

This service capability interworks with the following oneM2M Resource operations that are interworked are:

Not Applicable

#### 6.5.2.4	sendMessage

This service capability provides the ability for AEs to send message payloads to a target AE.

As part of the send message, the originating AE can provide delivery to enhance the robustness of the publication to AEs that have subscribed to the Resource identified in the request.

##### 6.5.2.4.1	Pre-conditions

The Broker service has connectivity to the target AE where the target AE has subscribed to the Broker to receive send message requests from the Broker.

##### 6.5.2.4.2	Signature – sendMessage

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deliveryPolicy | IN | YES | The delivery policy when notifying the subscriber. See 6.4.1.1.2.2 |
| payload | IN | NO | Payload to be published. |
| responseType | OUT | YES | Unique response types for this service. - Target AE is not subscribed with the Broker to receive messages - Originator has requested an invalid delivery policy Note: Consumed services also provide response types. - Exception: Request may not have been completed |

Table 6.5.2.4.2-1 Broker – sendMessage capability

##### 6.5.2.4.3	Post-Conditions

Not Applicable

##### 6.5.2.4.4	Exceptions

Not Applicable

##### 6.5.2.4.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.5.2.4.6	oneM2M Resource Interworking

Not Applicable

## 6.6	Device Management

### 6.6.1	Overview

The Device Management service provides for the capability for an AE to manage devices across the Mca Reference Point using supported device management message exchange patterns. This service utilizes management adapters to provide the management functions.

The supported device management message exchanges patterns are:

- Request-Response

- Request- Asynchronous Report

#### 6.6.1.1	Device Management

The Device Management service permits AEs to request management operations (e.g., device configuration operations, device troubleshooting operations, device firmware management operations, and application software management operations) and provide the result or report/retrieve the status of the operations to the originating AE.

AEs have the option to request to manage a targeted set of devices that include:

- Individual device

- Multiple devices through the use of lists of devices

- A group that designates as list of devices using the groupId.

AEs have option to request management operations to be performed based on a schedule.

AEs have the option to request that responses to requests be delivered synchronous (the request is held until all responses are received) or asynchronously (Reports of the operation are transmitted to the AE when an operation has completed) to the request submission. For asynchronous requests, reports are transmitted based on a report policy configured as part of the original request.

AEs also have the capability to request to manage devices by complex operations (e.g. upgradefirmware), where the M2M service platform can orchestrate management operations (e.g. installfirmware, downloadware) according to the business process.

##### 6.6.1.1.1	Supporting Rules

###### 6.6.1.1.1.1	Report Policy Type

The report policy is used to determine how a report is sent by the M2M System to the AE. There is a choice of parameters used for batchReport, pendingReport, latestReport.

| Parameter | Optional | Description |
| --- | --- | --- |
| groupId | YES | The group identifier. |
| operationMonitorList | YES | List of monitored operations. Enum OperationMonitor, see 6.6.1.1.1.29. |
| batchReport | YES | This parameter indicates that AE wants to receive batches of reports rather than receiving them one at a time. This attribute e may include two values: the number of reports to be batched for delivery and a duration. If batchReport is used simultaneously with latestReport, only the latest report shall be sent. Type BatchReport, see 6.6.1.1.1.2. |
| pendingReport | YES | This parameter indicates how missed reports due to a period of connectivity. Enum PendingReport, see 6.6.1.1.1.28. |
| latestReport | YES | This parameter indicates if AE wants only the latest report. |

Table 6.6.1.1.1.1-1 Type:ReportPolicy

###### 6.6.1.1.1.2	Batch Report Type

The BatchReport is complex type that defines the batch for reports.

| Parameter | Optional | Description |
| --- | --- | --- |
| number | YES | The number of reports to be batched. |
| duration | YES | The duration in which all the reports to be batched. |

Table 6.6.1.1.1.2-1 Type: BatchReport

###### 6.6.1.1.1.3	Firmware Report Type

The FirmwareReport is complex type that defines the current state of a firmware management operation for a specific device.

| Parameter | Optional | Description |
| --- | --- | --- |
| groupId | YES | The group identifier. |
| deviceId | NO | The unique device identifier in the context of the M2M Service Subscription. |
| operation | YES | The firmware management operation.(e.g. installFirmware, downloadFirmware) |
| operationResult | YES | The firmware management operation execution result. |
| operationStatus | YES | The firmware management operation execution status. It can be Initiated, Started, Finished, Cancelled, or Deleted. |

Table 6.6.1.1.1.3-1 Type: FirmwareReport

###### 6.6.1.1.1.4	Orchestration Rule Type

The OrchestrationRule is a complex type that describes the target and associated schedule of the request for the execution of the management operation.

| Parameter | Optional | Description |
| --- | --- | --- |
| groupId | YES | The group identifier. |
| deviceId | NO | The unique device identifier in the context of the M2M Service Subscription. |
| operation | NO | The orchestrated operation. (e.g. installFirmware, downloadFirmware) |
| schedule | YES | The time when to execute firmware operation. If it is not provided, the operation shall be executed immediately. |

Table 6.6.1.1.1.4-1 Type: OrchestrationRule

###### 6.6.1.1.1.5	Firmware Info Type

The FirmwareInfo is a complex type that describes the version, name, URL of the device firmware.

| Parameter | Optional | Description |
| --- | --- | --- |
| version | YES | The version of the firmware. |
| name | YES | The name of the firmware to be used on the device. |
| url | YES | The URL from which the firmware image can be downloaded. |

Table 6.6.1.1.1.5-1 Type: FirmwareInfo

###### 6.6.1.1.1.6	Service Subscription Integration

In order for the Device Management Service to take advantage of the Management Adapter, there shall be association between the device associated with a Service Subscription and the service capability to the Management Adapter.

###### 6.6.1.1.1.7	Device Info Type

The DeviceInfo is a complex type that describes the device label, manufacturer, model, device type, firmware version, software version, hardware version.

| Parameter | Optional | Description |
| --- | --- | --- |
| deviceLabel | YES | Unique device label assigned by the manufacturer. The uniqueness may be global or only valid within a certain domain (e.g. vendor-wise or for a certain deviceType). |
| manufacturer | YES | The name/identifier of the device manufacturer. |
| model | YES | The name/identifier of the device mode assigned by the manufacturer. |
| deviceType | YES | The type (e.g. cell phone, photo frame, smart meter) or product class (e.g. X-series) of the device. |
| fwVersion | YES | The firmware version of the device. NOTE: If the device only supports one kind of Software this is identical to softwareVersion. |
| swVersion | YES | The software version of the device. |
| hwVersion | YES | The hardware version of the device. |

Table 6.6.1.1.1.7-1 Type: DeviceInfo

###### 6.6.1.1.1.8	Memory Type

The Memory is a complex type that describes the current available amount of memory and the total amount of memory.

| Parameter | Optional | Description |
| --- | --- | --- |
| memAvailable | YES | The current available amount of memory in bytes. |
| memTotal | YES | The total amount of memory in bytes. |

Table 6.6.1.1.1.8-1 Type: Memory

###### 6.6.1.1.1.9	Battery Type

The Battery is a complex type that describes the current battery level and the status of the battery.

| Parameter | Optional | Description |
| --- | --- | --- |
| level | YES | The current battery level as a percent of the battery capacity |
| status | YES | Indicates the status of the battery. Enum BatteryStatus, see 6.6.1.1.1.11. |

Table 6.6.1.1.1.9-1 Type: Battery

###### 6.6.1.1.1.10	DeviceCapability Type

The DeviceCapability is a complex type that describes the name of the capability, attach label, capability action status.

| Parameter | Optional | Description |
| --- | --- | --- |
| name | YES | The name of the capability. Enum DeviceCapabilityName, see 6.6.1.1.1.12. |
| attached | YES | Indicates whether the capability is attached to the device or not. |
| state | NO | Indicates if the capability is enabled or disabled |

Table 6.6.1.1.1.10-1 Type: DeviceCapability

###### 6.6.1.1.1.11	BatteryStatus Enum

| Battery Status | Description |
| --- | --- |
| Normal | The battery is operating normally and not on power. |
| Charging | The battery is currently charging. |
| Charge Complete | The battery is fully charged and still on power. |
| Damaged | The battery has some problem. |
| Low Battery | The battery is low on charge. |
| Not Installed | The battery is not installed. |
| Unknown | The battery information is not available. |

Table 6.6.1.1.1.11-1 Enum: BatteryStatus

###### 6.6.1.1.1.12	DeviceCapabilityName Enum

| Device Capability Name | Description |
| --- | --- |
| Hardware: ExternalMemory | Used to enable/disable the removable card memory or storage disk. |
| Hardware: Display | Used to enable/disable the display screen. |
| Hardware: Camera | Used to enable/disable the camera. |
| Hardware: Speaker | Used to enable/disable the speaker. |
| I/O: Tethering | Used to enable/disable the device to be attached to other devices. |
| I/O: AudioInputEncoder | Used to enable/disable the audio input encoder. |
| I/O: AttachedDevice | Used to enable/disable the capability to allow other devices to be attached to this device. |
| I/O: Keyboard | Used to enable/disable the keyboard |
| I/O: InputPeripheral | Used to enable/disable the input peripheral. |
| I/O: OutputPeripheral | Used to enable/disable the output peripheral. |
| I/O: USB | Used to enable/disable the USB port. |
| I/O: SerialPort | Used to enable/disable the serial port. |
| I/O: ParallelPort | Used to enable/disable the Parallel port. |
| I/O: GPS | Used to enable/disable GPS capability |
| I/O: GNSS | Used to enable/disable GNSS capability |
| Connectivity: Bluetooth | Used to enable/disable the Bluetooth connectivity. |
| Connectivity: WLAN | Used to enable/disable the WLAN connectivity. |
| Connectivity: Infrared | Used to enable/disable the Infrared connectivity. |
| Connectivity: WCDMA | Used to enable/disable the WCDMA connectivity |
| Connectivity: GPRS | Used to enable/disable the GPRS connectivity |
| Connectivity: EDGE | Used to enable/disable the EDGE connectivity |
| Connectivity: CDMA | Used to enable/disable the CDMA connectivity |
| Connectivity: WiMAX | Used to enable/disable the WiMAX connectivity |
| Connectivity: LTE | Used to enable/disable the LTE connectivity |
| Connectivity: NFC | Used to enable/disable the NFC connectivity |
| Connectivity: TD-SCDMA | Used to enable/disable the TD-SCDMA connectivity |
| LAN Interfaces: USB | Used to enable/disable the USB Interface |
| LAN Interfaces: Wi-Fi | Used to enable/disable the Wi-Fi Radio |
| LAN Interfaces: HomePlug | Used to enable/disable the HomePlug Interface |
| LAN Interfaces: MoCA | Used to enable/disable the MoCA Interface |
| LAN Interfaces: UPA | Used to enable/disable the UPA Interface |
| Hardware: SmartCardReader | Used to enable/disable the SmartCardReader |

Table 6.6.1.1.1.11-1 Enum: DeviceCapabilityName

###### 6.6.1.1.1.13	LockStatus Enum

| Lock Status | Description |
| --- | --- |
| locked | The device is locked. |
| unlocked | The device is unlocked. |

Table 6.6.1.1.1.11-1 Enum: LockedStatus

###### 6.6.1.1.1.14	Area Network Info Type

The AreaNwkInfo is a complex type that describes the information about a M2M area network. Information includes the area network identifier, area network name, area network type, and the list of devices in the M2M Area Network.

| Parameter | Optional | Description |
| --- | --- | --- |
| areaNwkId | NO | The unique area network identifier. |
| areaNwkName | YES | The Area Network name. |
| areaNwkType | YES | The areaNwkType is an implementation-chosen string that indicates the type of M2M Area Network. |
| listOfDevices | YES | Indicates the list of devices in the M2M Area Network. Type AreaNwkDevice, see 6.6.1.1.1.15. From listOfDevices, the topology of the area network can be discovered and retrieved. |

Table 6.6.1.1.1.14-1 Type: AreaNwkInfo

###### 6.6.1.1.1.15	Area Network Device Type

The AreaNwkDevice is a complex type that describes the device information for a device in the context of a M2M Area Network. Information includes the device identifier, area network identifier and the area network device specific information.

| Parameter | Optional | Description |
| --- | --- | --- |
| deviceId | NO | The unique device identifier in the context of the M2M Service Subscription. |
| areaNwkDeviceInfo | YES | The area network device information. Type AreaNwkDeviceInfo, see 6.6.1.1.1.16. |

Table 6.6.1.1.1.15-1 Type: AreaNwkDevice

###### 6.6.1.1.1.16	Area Network Device Info Type

The AreaNwkDeviceInfo is a complex type that describes information about the device in the context of the M2M Area Network. Information includes the type of device, the interval of time between two sleep cycles, the time duration of each sleep cycle, the status of the device and any neighbor devices.

| Parameter | Optional | Description |
| --- | --- | --- |
| deviceType | YES | The type (e.g. cell phone, photo frame, smart meter) or product class (e.g. X-series) of the device. |
| sleepInterval | YES | The interval between two sleep cycles. |
| sleepDuration | YES | The time duration of each sleep cycle. |
| status | YES | The status of the device (sleeping or waked up). |
| listOfNeighbors | YES | Indicates the neighbour devices of the same area network. When modified, the connection relationship of the devices shall be modified accordingly. |

Table 6.6.1.1.1.16-1 Type: AreaNwkDeviceInfo

###### 6.6.1.1.1.17	LogTypeId Enum

| LogTypeId | Description |
| --- | --- |
| system | System log |
| security | Security log |
| event | Event log |
| trace | Trace log |
| panic | Panic log |

Table 6.6.1.1.1.17-1 Enum: LogTypeId

###### 6.6.1.1.1.18	LogStatus Enum

| LogStatus | Description |
| --- | --- |
| Started | the logging activity is started |
| Stopped | the logging activity is stopped |
| Unknown | the current status of the logging activity is unknown. |
| NotPresent | the log data is not present and the logData attribute shall be ignored. |
| Error | error conditions for the logging activities, and the logging is stopped. |

Table 6.6.1.1.1.18-1 Enum: LogStatus

###### 6.6.1.1.1.19		LogInfo Type

The LogInfo is a complex type that describes the log type, diagnostic data, log action status.

| Parameter | Optional | Description |
| --- | --- | --- |
| logTypeId | NO | The type of the log. E.g. security log, system log. Enum LogTypeId, see 6.6.1.1.1.17. |
| logData | NO | Diagnostic data logged upon event of interests defined by the diagnostic function. |
| logActionStatus | NO | Indicates the status of the Action. E.g. Started, Stopped. Enum LogStatus, see 6.6.1.1.1.18. |

Table 6.6.1.1.1.19-1 Type: LogInfo

###### 6.6.1.1.1.20		TroubleshootingReport Type

The TroubleshootingReport is complex type that defines the current state of a troubleshooting operation for a specific device.

| Parameter | Optional | Description |
| --- | --- | --- |
| groupId | YES | The group identifier. |
| deviceId | NO | The unique device identifier in the context of the M2M Service Subscription. |
| operation | YES | The troubleshooting operation.(e.g. rebootDevice, resetDevice, uploadDeviceLog) |
| operationResult | YES | The troubleshooting operation execution result. |
| operationStatus | YES | The troubleshooting operation execution status. It can be Initiated, Started, Finished, Cancelling, or Cancelled. |

Table 6.6.1.1.1.20-1 Type: TroubleshootingReport

###### 6.6.1.1.1.21		Log Type

The Log is a complex type that describes the log information, log URL.

| Parameter | Optional | Description |
| --- | --- | --- |
| logInfo | NO | The log information. Type LogInfo, see 6.6.1.1.1.19. |
| logURL | NO | The URL from which the log can be accessed. |

Table 6.6.1.1.1.21-1 Type: Log

###### 6.6.1.1.1.22	Log Filter Criteria

| Criterion name | Description |
| --- | --- |
| logTypeId | The type of the log. E.g. security log, system log. Enum LogTypeId, see 6.6.1.1.1.17. |
| startTime | The start time of log. |
| endTime | The end time of log. |

Table 6.6.1.1.1.22 Log Filter Criteria

###### 6.6.1.1.1.24	Action Enum

| Software Action | Description |
| --- | --- |
| Install | The action that installs the software. |
| Uninstall | The action that un-installs the software. |
| Activate | The action that activates software previously installed. |
| Deactivate | The action that deactivates software. |

Table 6.6.1.1.1.24-1 Enum: Action

###### 6.6.1.1.1.25	Status Enum

| Status | Description |
| --- | --- |
| Successful | The status is successful. |
| Failure | The status is failure. |
| In-Process | The status is in process. |

Table 6.6.1.1.1.25-1 Enum: Status

###### 6.6.1.1.1.26		ActionStatus Type

The ActionStatus is a complex type that describes the action and related status.

| Parameter | Optional | Description |
| --- | --- | --- |
| action | NO | The action being performed Enum Action, see 6.6.1.1.1.24. |
| status | NO | Indicates the status of the operation is successful, failure or in process. Enum Status, see 6.6.1.1.1.25. |

Table 6.6.1.1.1.26-1 Type: ActionStatus

###### 6.6.1.1.1.27		SoftwareReport Type

The SoftwareReport is complex type that defines the current state of an application software management operation for a specific device.

| Parameter | Optional | Description |
| --- | --- | --- |
| groupId | YES | The group identifier. |
| deviceId | NO | The unique device identifier in the context of the M2M Service Subscription. |
| operation | YES | The application software management operation.(e.g. downloadSoftware, installSoftware, activateSoftware, deactivateSoftware, removeSoftware) |
| operationResult | YES | The application software management operation execution result. |
| installStatus | YES | Indicates the status of the install. Enum ActionStatus, see 6.6.1.1.1.26. |
| activeStatus | YES | The status of active or deactivate action. Enum ActionStatus, see 6.6.1.1.1.26. |

Table 6.6.1.1.1.27-1 Type: SoftwareReport

###### 6.6.1.1.1.28	PendingReport Enum

| PendingReport | Description |
| --- | --- |
| SendLatest | Send the latest report. |
| SendAllPending | Send all pending report. |

Table 6.6.1.1.1.28-1 Enum: PendingReport

###### 6.6.1.1.1.29	OperationMonitor Enum

| Monitored Operation | Description |
| --- | --- |
| DownloadFirmware | The downloadFirmware operation. |
| InstallFirmware | The installFirmware operation. |
| RebootDevice | The rebootDevice operation. |
| ResetDevice | The resetDevice operation. |
| UploadDeviceLog | The uploadDeviceLog operation. |
| DownloadSoftware | The downloadSoftware operation. |
| InstallSoftware | The installSoftware operation. |
| ActiveSoftware | The activeSoftware operation. |
| DeactivateSoftware | The deactivateSoftware operation. |
| RemoveSoftware | The removeSoftware operation. |

Table 6.6.1.1.1.29-1 Enum: OperationMonitor

### 6.6.2	Service Capabilities

#### 6.6.2.1	downloadFirmware

This service capability provides the ability for AEs to download device firmware.

##### 6.6.2.1.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the service capability and device exist.

##### 6.6.2.1.2	Signature – downloadFirmware

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| firmwareInfo | IN | NO | The device firmware information. Type FirmwareInfo, see 6.6.1.1.1.5 |
| deviceId | IN | YES | The unique device identifier in the context of the M2M Service Subscription. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. Type ReportPolicy, see 6.6.1.1.1.1. |
| requestId | OUT | NO | The M2M Request Identifier (M2M-Request-ID) |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Issuer does not have a Management Adapter for the requested operation - Report policy does not supported by the Management Adapter - Firmware download failure,e.g. firmware miss dependency, firmware is too large for available program or data memory |

Table 6.6.2.1.2-1 Device Management Service –downloadFirmware capability

##### 6.6.2.1.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.6.2.1.3-1 downloadFirmware Diagram

##### 6.6.2.1.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to download firmware.

##### 6.6.2.1.5	Exceptions

Not Applicable

##### 6.6.2.1.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.2.1.7	oneM2M Resource Interworking

This service capability is used to download firmware. The service capability aligns with the <firmware> resource and maps to the UPDATE procedure for the attribute update of the resource. The service capability also aligns with the <mgmtCmd> resource and maps to the EXECUTE procedure for the resource.

#### 6.6.2.2	installFirmware

This service capability provides the ability for AEs to install firmware on the device.

##### 6.6.2.2.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the service capability and device exist.

The firmware to be installed exists on the device.

##### 6.6.2.2.2	Signature –installFirmware

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| firmwareInfo | IN | NO | The device firmware information. Type FirmwareInfo, see 6.6.1.1.1.5 |
| deviceId | IN | YES | The unique device identifier in the context of the M2M Service Subscription. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. Type ReportPolicy, see 6.6.1.1.1.1. |
| requestId | OUT | NO | The M2M Request Identifier (M2M-Request-ID) |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Issuer does not have a Management Adapter for the requested operation - Report policy does not supported by the Management Adapter - Firmware install failure, e.g. firmware is missing dependency on the device, firmware is too large for available program or data memory |

Table 6.6.2.2.2-1 Device Management Service –installFirmware capability

##### 6.6.2.2.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.6.2.2.3-1 installFirmware Diagram

##### 6.6.2.2.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to install firmware.

##### 6.6.2.2.5	Exceptions

Not Applicable

##### 6.6.2.2.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.2.2.7	oneM2M Resource Interworking

This service capability is used to install firmware. The service capability aligns with the <firmware> resource and maps to the UPDATE procedure for the attribute update of the resource. The service capability also aligns with the <mgmtCmd> resource and maps to the EXECUTE procedure for the resource.

#### 6.6.2.3 	getFirmwareInformation

This service capability provides the ability for AEs to get device firmware information.

##### 6.6.2.3.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the service capability and device exist.

##### 6.6.2.3.2	Signature –getFirmwareInformation

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | YES | The unique device identifier in the context of the M2M Service Subscription. |
| requestId | OUT | NO | The M2M Request Identifier (M2M-Request-ID) |
| firmwareInfo | OUT | YES | The device firmware information. Type FirmwareInfo, see 6.6.1.1.1.5 |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Issuer does not have a Management Adapter for the requested operation |

Table 6.6.2.3.2-1 Device Management Service –getFirmwareInformation capability

##### 6.6.2.3.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.6.2.3.3-1 getFirmwareInformation Diagram

##### 6.6.2.3.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get firmware information.

##### 6.6.2.3.5	Exceptions

Not Applicable

##### 6.6.2.3.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.2.3.7	oneM2M Resource Interworking

This service capability is used to get firmware information. The service capability aligns with the <firmware> resource and maps to the RETRIEVE procedure for the attribute version, name, URL of the resource.

#### 6.6.2.4 	getFirmwareExecStatus

This service capability provides the ability for AEs to get state of firmware management operation for a specific device.

##### 6.6.2.4.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and previously submitted firmware request exist.

##### 6.6.2.4.2	Signature –getFirmwareExecStatus

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| operationRequestId | IN | NO | The M2M Request Identifier of previously submitted firmware request (M2M-Request-ID) |
| deviceId | IN | YES | The unique device identifier in the context of the M2M Service Subscription. |
| requestId | OUT | NO | The M2M Request Identifier (M2M-Request-ID) |
| firmwareReport | OUT | YES | The firmware management operation execution result or status. Type FirmwareReport, see 6.6.1.1.1.3. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Issuer does not have a Management Adapter for the requested operation - The previously submitted firmware request does not exist |

Table 6.6.2.4.2-1 Device Management Service –getFirmwareExecStatus capability

##### 6.6.2.4.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.6.2.4.3-1 getFirmwareExecStatus Diagram

##### 6.6.2.4.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get firmware state of management operation.

##### 6.6.2.4.5	Exceptions

Not Applicable

##### 6.6.2.4.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.2.4.7	oneM2M Resource Interworking

This service capability is used to get state of firmware management operation. The service capability aligns with the <firmware> resource and maps to the RETRIEVE procedure for the attribute updateStatus of the resource. The service capability also aligns with the <execInstance> resource and maps to the RETRIEVE procedure for the attribute execStatus, execResult of the resource.

#### 6.6.2.5	deviceManagementReport

This service capability provides the ability to report the device management operation execution result or status information to the AE.

##### 6.6.2.5.1	Pre-conditions

A correlation between a Management Adapter, the M2M Service Capability and previously submitted request exist.

##### 6.6.2.5.2	Signature –deviceManagementReport

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| isLastReport | IN | NO | Boolean, whether it is the last report. |
| sequenceNumber | IN | NO | The report sequence number. |
| firmwareReportList | IN | YES | Array of firmwareReport. Type FirmwareReport, see 6.6.1.1.1.3. |
| troubleshootingReportList | IN | YES | Array of troubleshootingReport. Type TroubleshootingReport, see 6.6.1.1.1.20. |
| softwareReportList | IN | YES | Array of softwareReport. Type SoftwareReport, see 6.6.1.1.1.27. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - The target AE does not exist. |

Table 6.6.2.5.2-1 Device Management Service –deviceManagementReport capability

##### 6.6.2.5.3	Post-Conditions

The AE has received a report of firmware operation execution status or result. The M2M System shall maintain the state of the report (i.e., sequence number) according to the report policy.

##### 6.6.2.5.4	Exceptions

Not Applicable

##### 6.6.2.5.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.2.5.6	oneM2M Resource Interworking

Not Applicable

#### 6.6.2.6	upgradeFirmware

This service capability is a complex operation for AEs to upgrade firmware on one or more devices. The service capability utilizes orchestration rules for the related management operations such as downloadFirmware, installFirmware, getFirmwareInformation, getFirmwareExecInstance and reportFirmwareStatus according to the business process.

This service capability permits AEs to upgrade the firmware on individual device, multiple devices or a group of devices. In addition the upgrade of the firmware is permitted based on a schedule for each of the operations.

##### 6.6.2.6.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between Management Adapter, the service capability and devices or device group exist.

##### 6.6.2.6.2	Signature – upgradeFirmware

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| orchestrationRuleList | IN | NO | List of OrchestrationRule. Type OrchestrationRule, see clause 6.6.1.1.1.4. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. Type ReportPolicy, see 6.6.1.1.1.1. |
| requestId | OUT | NO | The M2M Request Identifier (M2M-Request-ID) |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Report policy is not supported by the Management Adapter - The Orchestration Rules are not supported by the Management Adapter - Operation exception: The device’s reachability schedule is inconsistent with the schedule of the Orchestration rule. |

Table 6.6.2.6.2-1 Device Management Service –upgradeFirmware capability

##### 6.6.2.6.3	Service Interactions

The interactions of service capabilities required for this service capability:

1) 	Issue the request to the Supporting Service to perform the operation

Figure 6.6.2.6.3-1 upgradeFirmware Diagram

##### 6.6.2.6.4	Post-Conditions

The Management Adapter has submitted a set of requests to the Management Server to manage firmware.

##### 6.6.2.6.5	Exceptions

Not Applicable

##### 6.6.2.6.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.2.6.7	oneM2M Resource Interworking

Not Applicable

#### 6.6.2.7	getDeviceInformation

This service capability provides the ability for AEs to get device information.

##### 6.6.2.7.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

##### 6.6.2.7.2	Signature – getDeviceInformation

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| deviceInfo | OUT | NO | The device information. Type DeviceInfo, see 6.6.1.1.1.7. |
| memory | OUT | YES | The memory information. Type Memory, see 6.6.1.1.1.8. |
| battery | OUT | YES | The battery information. Type Battery, see 6.6.1.1.1.9. |
| lockStatus | OUT | YES | The device lock status. Enum LockStatus, see 6.6.1.1.1.13. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Issuer does not have a Management Adapter for the requested operation |

Table 6.6.2.7.2-1 Device Management Service –getDeviceInformation capability

##### 6.6.2.7.3	Service Interactions

The interactions of service capabilities required for this service capability:

1)  Issue the request to the Supporting Service to perform the operation

Figure 6.6.2.7.3-1 getDeviceInformation Diagram

##### 6.6.2.7.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get device information.

Based on the capabilities supported by the device, the memory, battery and lock status information may or may not be returned.

##### 6.6.2.7.5	Exceptions

Not Applicable

##### 6.6.2.7.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.2.7.7	oneM2M Resource Interworking

This service capability is used to get device information. The service capability aligns with the <deviceInfo> resource and maps to the Retrieve procedure for the resource. The service capability aligns with the <memory> resource and maps to the Retrieve procedure for the resource. The service capability aligns with the <battery> resource and maps to the Retrieve procedure for the resource.

#### 6.6.2.8	getDeviceCapabilities

This service capability provides the ability for AEs to get the device capabilities.

##### 6.6.2.8.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

##### 6.6.2.8.2	Signature –getDeviceCapabilities

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| deviceCapabilities | OUT | YES | List of deviceCapability. Type DeviceCapability, see 6.6.1.1.1.10. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Issuer does not have a Management Adapter for the requested operation |

Table 6.6.2.8.2-1 Device Management Service –getDeviceCapabilities capability

##### 6.6.2.8.3	Service Interactions

The interactions of service capabilities required for this service capability:

1)  Issue the request to the Supporting Service to perform the operation

Figure 6.6.2.8.3-1 getDeviceCapabilities Diagram

##### 6.6.2.8.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get device capabilities.

##### 6.6.2.8.5	Exceptions

Not Applicable

##### 6.6.2.8.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.2.8.7	oneM2M Resource Interworking

This service capability is used to get device capabilities. The service capability aligns with the<deviceCapability> resource and maps to the RETRIEVE procedure for the resource.

#### 6.6.2.9 	enableDeviceCapability

This service capability provides the ability for AEs to enable device capability.

##### 6.6.2.9.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

##### 6.6.2.9.2	Signature –enableDeviceCapability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| Name | IN | NO | The name of the capability. Enum DeviceCapabilityName, see 6.6.1.1.1.12. |
| state | OUT | NO | Indicates if the capability is enabled or disabled. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Issuer does not have a Management Adapter for the requested operation |

Table 6.6.2.9.2-1 Device Management Service –enableDeviceCapability capability

##### 6.6.2.9.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.6.2.9.3-1 enableDeviceCapability Diagram

##### 6.6.2.9.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to enable device capability.

##### 6.6.2.9.5	Exceptions

Not Applicable

##### 6.6.2.9.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.2.9.7	oneM2M Resource Interworking

This service capability is used to enable device capability. The service capability aligns with the <deviceCapability> resource and maps to the UPDATE procedure for the attribute enable of the resource.

#### 6.6.2.10	disableDeviceCapability

This service capability provides the ability for AEs to disable device capability.

##### 6.6.2.10.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the service capability and device exist.

##### 6.6.2.10.2	Signature –disableDeviceCapability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| name | IN | NO | The name of the capability. Enum DeviceCapabilityName, see 6.6.1.1.1.12. |
| state | OUT | NO | Indicates if the capability is enabled or disabled. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Issuer does not have a Management Adapter for the requested operation |

Table 6.6.2.10.2-1 Device Management Service –disableDeviceCapability capability

##### 6.6.2.10.3	Service Interactions

The interactions of service capabilities required for this service capability:

1)  Issue the request to the Supporting Service to perform the operation

Figure 6.6.2.10.3-1 disableDeviceCapability Diagram

##### 6.6.2.10.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to disable device capability.

##### 6.6.2.10.5	Exceptions

Not Applicable

##### 6.6.2.10.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.2.10.7	oneM2M Resource Interworking

This service capability is used to disable device capability. The service capability aligns with the <deviceCapability> resource and maps to the UPDATE procedure for the attribute disable of the resource.

#### 6.6.2.11	getAreaNetworks

This service capability provides the ability for AEs to get area networks information.

##### 6.6.2.11.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and area network exist.

##### 6.6.2.11.2	Signature – getAreaNetworks

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| areaNwks | OUT | YES | Array of area network. Type AreaNwkInfo, see 6.6.1.1.1.14. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Issuer does not have a Management Adapter for the requested operation |

Table 6.6.2.11.2-1 Device Management Service –getAreaNetworks capability

##### 6.6.2.11.3	Service Interactions

The interactions of service capabilities required for this service capability:

1)  Issue the request to the Supporting Service to perform the operation

Figure 6.6.2.11.3-1 getAreaNetworks Diagram

##### 6.6.2.11.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get area networks information.

##### 6.6.2.11.5	Exceptions

Not Applicable

##### 6.6.2.11.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.2.11.7	oneM2M Resource Interworking

This service capability is used to get area networks information. The service capability aligns with the <areaNwkInfo> resource and maps to the RETRIEVE procedure for the resource. The service capability aligns with the <areaNwkDeviceInfo> resource and maps to the Retrieve procedure for the resource.

#### 6.6.2.12	updateDeviceForAreaNetwork

This service capability provides the ability for AEs to update device information for area network.

##### 6.6.2.12.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability, the device and area network exist.

##### 6.6.2.12.2	Signature –updateDeviceForAreaNetwork

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| areaNwkId | IN | NO | The unique area network identifier. |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| areaNwkDeviceInfo | IN | YES | The existing areaNwkDeviceInfo are replaced with the information in this parameter. Type AreaNwkDeviceInfo, see 6.6.1.1.1.16. |
| lastModifiedTime | OUT | NO | The modified time. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Issuer does not have a Management Adapter for the requested operation |

Table 6.6.2.12.2-1 Device Management Service –updateDeviceForAreaNetwork capability

##### 6.6.2.12.3	Service Interactions

The interactions of service capabilities required for this service capability:

1)  Issue the request to the Supporting Service to perform the operation

Figure 6.6.2.12.3-1 updateDeviceForAreaNetwork Diagram

##### 6.6.2.12.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to update device information for area network.

##### 6.6.2.12.5	Exceptions

Not Applicable

##### 6.6.2.12.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.2.12.7	oneM2M Resource Interworking

The service capability aligns with the <areaNwkDeviceInfo> resource and maps to the Update procedure for attribute devType , sleepInterval , sleepDuration , status , listOfNeighbors of the resource.

#### 6.6.2.13	rebootDevice

This service capability provides the ability for AEs to reboot device.

##### 6.6.2.13.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

##### 6.6.2.13.2	Signature – rebootDevice

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. Type ReportPolicy, see 6.6.1.1.1.1. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Issuer does not have a Management Adapter for the requested operation - report policy does not supported by the Management Adapter |

Table 6.6.2.13.2-1 Device Management Service –rebootDevice capability

##### 6.6.2.13.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.6.2.13.3-1 rebootDevice Diagram

##### 6.6.2.13.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to reboot device.

##### 6.6.2.13.5	Exceptions

Not Applicable

##### 6.6.2.13.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.2.13.7	oneM2M Resource Interworking

This service capability is used to reboot device. The service capability aligns with the <reboot> resource and maps to the Execute procedure for the attribute reboot of the resource.

#### 6.6.2.14	resetDevice

This service capability provides the ability for AEs to reset device.

##### 6.6.2.14.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

##### 6.6.2.14.2	Signature –resetDevice

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. Type ReportPolicy, see 6.6.1.1.1.1. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Issuer does not have a Management Adapter for the requested operation - report policy does not supported by the Management Adapter |

Table 6.6.2.14.2-1 Device Management Service –resetDevice capability

##### 6.6.2.14.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.6.2.14.3-1 resetDevice Diagram

##### 6.6.2.14.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to reset device.

##### 6.6.2.14.5	Exceptions

Not Applicable

##### 6.6.2.14.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.2.14.7	oneM2M Resource Interworking

This service capability is used to reset device. The service capability aligns with the <reboot> resource and maps to the Execute procedure for the attribute factoryReset of the resource.

#### 6.6.2.15 	uploadDeviceLog

This service capability provides the ability for AEs to upload device log.

##### 6.6.2.15.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

##### 6.6.2.15.2	Signature –uploadDeviceLog

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| logInfo | IN | NO | The log information. Type LogInfo, see 6.6.1.1.1.19. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.1.1.1.1. |
| logURL | OUT | YES | The URL from which the log can be uploaded. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Issuer does not have a Management Adapter for the requested operation - report policy does not supported by the Management Adapter |

Table 6.6.2.15.2-1 Device Management Service –uploadDeviceLog capability

##### 6.6.2.15.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.6.2.15.3-1 uploadDeviceLog Diagram

##### 6.6.2.15.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to upload device log.

##### 6.6.2.15.5	Exceptions

Not Applicable

##### 6.6.2.15.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.2.15.7	oneM2M Resource Interworking

This service capability is used to upload device log. The service capability aligns with the <eventLog> resource and maps to the Update procedure for the resource.

#### 6.6.2.16	getDeviceLogs

This service capability provides the ability for AEs to get all logs of a device.

##### 6.6.2.16.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

##### 6.6.2.16.2	Signature – getDeviceLogs

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| logList | OUT | YES | Array of log. Type Log, see 6.6.1.1.1.21. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Issuer does not have a Management Adapter for the requested operation |

Table 6.6.2.16.2-1 Device Management Service –getDeviceLogs capability

##### 6.6.2.16.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.6.2.16.3-1 getDeviceLogs Diagram

##### 6.6.2.16.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get all logs of a device.

##### 6.6.2.16.5	Exceptions

Not Applicable

##### 6.6.2.16.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.2.16.7	oneM2M Resource Interworking

This service capability is used to get all logs of a device. The service capability aligns with the <eventLog> resource and maps to the Retrieve procedure for the resource.

#### 6.6.2.17	getDeviceLogInformation

This service capability provides the ability for AEs to get a device log information.

##### 6.6.2.17.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

##### 6.6.2.17.2	Signature – getDeviceLogInformation

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| filterCriteria | IN | YES | See Table 6.6.1.1.1.22 |
| logURL | IN | YES | The URL from which the log can be accessed. |
| log | OUT | YES | Type Log, see 6.6.1.1.1.21. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Issuer does not have a Management Adapter for the requested operation |

Table 6.6.2.17.2-1 Device Management Service –getDeviceLogInformation capability

##### 6.6.2.17.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.6.2.17.3-1 getDeviceLogInformation Diagram

##### 6.6.2.17.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get a device log information.

##### 6.6.2.17.5	Exceptions

Not Applicable

##### 6.6.2.17.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.2.17.7	oneM2M Resource Interworking

This service capability is used to a device log information. The service capability aligns with the <eventLog> resource and maps to the Retrieve procedure for the resource.

#### 6.6.2.18	getSoftwareInformation

This service capability provides the ability for AEs to get application software information.

##### 6.6.2.18.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

##### 6.6.2.18.2	Signature – getSoftwareInformation

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| version | OUT | YES | The version of the software. |
| name | OUT | YES | The name of the software. |
| URL | OUT | YES | The URL from which the software package can be downloaded. |
| installStatus | OUT | YES | Indicates the status of the install. Enum ActionStatus, see 6.6.1.1.1.26. |
| activeStatus | OUT | YES | The status of active or deactivate action. Enum ActionStatus, see 6.6.1.1.1.26. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Issuer does not have a Management Adapter for the requested operation |

Table 6.6.2.18.2-1 Device Management Service –getSoftwareInformation capability

##### 6.6.2.18.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.6.2.18.3-1 getSoftwareInformation Diagram

##### 6.6.2.18.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get application software information.

##### 6.6.2.18.5	Exceptions

Not Applicable

##### 6.6.2.18.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.2.18.7	oneM2M Resource Interworking

This service capability is used to get application software information. The service capability aligns with the <software> resource and maps to the Retrieve procedure for the resource.

#### 6.6.2.19	downloadSoftware

This service capability provides the ability for AEs to download application software.

##### 6.6.2.19.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

##### 6.6.2.19.2	Signature –downloadSoftware

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| version | IN | NO | The version of the software. |
| name | IN | NO | The name of the software. |
| URL | IN | NO | The URL from which the software package can be downloaded. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.1.1.1.1. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Issuer does not have a Management Adapter for the requested operation - report policy does not supported by the Management Adapter |

Table 6.6.2. 19.2-1 Device Management Service –downloadSoftware capability

##### 6.6.2.19.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.6.2. 19.3-1 downloadSoftware Diagram

##### 6.6.2.19.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to download application software.

##### 6.6.2.19.5	Exceptions

Not Applicable

##### 6.6.2.19.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.2.19.7	oneM2M Resource Interworking

This service capability is used to download application software. The service capability aligns with the <software> resource and maps to the UPDATE procedure for the attribute install of the resource. The service capability also aligns with the <mgmtCmd> resource and maps to the EXECUTE procedure for the resource.

#### 6.6.2.20 	installSoftware

This service capability provides the ability for AEs to install application software.

##### 6.6.2.20.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

##### 6.6.2.20.2	Signature –installSoftware

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| version | IN | NO | The version of the software. |
| name | IN | NO | The name of the software. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.1.1.1.1. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Issuer does not have a Management Adapter for the requested operation - report policy does not supported by the Management Adapter |

Table 6.6.2.20.2-1 Device Management Service –installSoftware capability

##### 6.6.2.20.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.6.2.20.3-1 installSoftware Diagram

##### 6.6.2.20.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to install application software.

##### 6.6.2.20.5	Exceptions

Not Applicable

##### 6.6.2.20.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.2.20.7	oneM2M Resource Interworking

This service capability is used to install application software. The service capability aligns with the <software> resource and maps to the UPDATE procedure for the attribute install of the resource. The service capability also aligns with the <mgmtCmd> resource and maps to the EXECUTE procedure for the resource.

#### 6.6.2.21	 activateSoftware

This service capability provides the ability for AEs to activate software previously installed.

##### 6.6.2.21.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

##### 6.6.2.21.2	Signature –activateSoftware

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| version | IN | NO | The version of the software. |
| name | IN | NO | The name of the software. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.1.1.1.1. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Issuer does not have a Management Adapter for the requested operation - report policy does not supported by the Management Adapter |

activateSoftware capability

##### 6.6.2.21.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.6.2.21.3-1 activateSoftware Diagram

##### 6.6.2.21.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to activate software previously installed.

##### 6.6.2.21.5	Exceptions

Not Applicable

##### 6.6.2.21.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.2.21.7	oneM2M Resource Interworking

This service capability is used to activate software previously installed. The service capability aligns with the <software> resource and maps to the UPDATE procedure for the attribute activate of the resource. The service capability also aligns with the <mgmtCmd> resource and maps to the EXECUTE procedure for the resource.

#### 6.6.2.22 	deactivateSoftware

This service capability provides the ability for AEs to deactivates software.

##### 6.6.2.22.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and the device exist.

##### 6.6.2.22.2	Signature –deactivateSoftware

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| version | IN | NO | The version of the software. |
| name | IN | NO | The name of the software. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.1.1.1.1. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Issuer does not have a Management Adapter for the requested operation - report policy does not supported by the Management Adapter |

deactivateSoftware capability

##### 6.6.2.22.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.6.2.22.3-1 deactivateSoftware Diagram

##### 6.6.2.22.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to deactivates software.

##### 6.6.2.22.5	Exceptions

Not Applicable

##### 6.6.2.22.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.2.22.7	oneM2M Resource Interworking

This service capability is used to deactivates software. The service capability aligns with the <software> resource and maps to the UPDATE procedure for the attribute deactivate of the resource. The service capability also aligns with the <mgmtCmd> resource and maps to the EXECUTE procedure for the resource.

#### 6.6.2.23	removeSoftware

This service capability provides the ability for AEs to uninstall the software.

##### 6.6.2.23.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and the device exist.

##### 6.6.2.23.2	Signature – removeSoftware

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| version | IN | NO | The version of the software. |
| name | IN | NO | The name of the software. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.1.1.1.1. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Issuer does not have a Management Adapter for the requested operation - report policy does not supported by the Management Adapter |

Table 6.6.2.23.2-1 Device Management Service –removeSoftware capability

##### 6.6.2.23.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.6.2.23.3-1 removeSoftware Diagram

##### 6.6.2.23.4	Post-Conditions

The Management Adapter has submitted a set of requests to the Management Server to uninstall the software.

##### 6.6.2.23.5	Exceptions

Not Applicable

##### 6.6.2.23.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.2.23.7	oneM2M Resource Interworking

This service capability is used to install application software. The service capability aligns with the <software> resource and maps to the UPDATE procedure for the attribute uninstall of the resource. The service capability also aligns with the <mgmtCmd> resource and maps to the EXECUTE procedure for the resource.

## 6.7	Management Adapter

### 6.7.1	Overview

The Management Adapter service provides the ability to adapt the M2M Service Layer operations to the technology specific operations of the Management Server.

### 6.7.2	Service Capabilities

#### 6.7.2.1	downloadFirmware

This service capability provides the ability to download the specific device firmware.

##### 6.7.2.1.1	Pre-conditions

A correlation between a Management Server, the service capability and device exist.

##### 6.7.2.1.2	Signature – downloadFirmware

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| firmwareInfo | IN | NO | The device firmware information. Type FirmwareInfo, see 6.6.1.1.1.5 |
| deviceId | IN | YES | The unique device identifier in the context of the M2M Service Subscription. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.1.1.1.1. |
| requestId | IN | NO | The M2M Request Identifier (M2M-Request-ID) |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Report policy is not supported by the Management Adapter - Firmware download failure, e.g. firmware is missing dependency on the device, firmware is too large for available program or data memory |

Table 6.7.2.1.2-1 Management Adapter –downloadFirmware capability

##### 6.7.2.1.3	Post-Conditions

The Management Adapter has submitted a request to the Management Server to download the firmware.

##### 6.7.2.1.4	Exceptions

Not Applicable

##### 6.7.2.1.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.1.6	oneM2M Resource Interworking

This service capability is used to download firmware. The service capability aligns with the <firmware> resource and maps to the UPDATE procedure for the attribute update of the resource. The service capability also aligns with the <mgmtCmd> resource and maps to the EXECUTE procedure for the resource.

#### 6.7.2.2	installFirmware

This service capability provides the ability to install firmware on a device.

##### 6.7.2.2.1	Pre-conditions

A correlation between a Management Server, the M2M Service Capability and device exist.

##### 6.7.2.2.2	Signature –installFirmware

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| firmwareInfo | IN | NO | The device firmware information. Type FirmwareInfo, see 6.6.1.1.1.5 |
| deviceId | IN | YES | The unique device identifier in the context of the M2M Service Subscription. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. |
| requestId | IN | NO | The M2M Request Identifier (M2M-Request-ID) |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Report policy does not supported by the Management Adapter - Firmware install failure, e.g. firmware is missing dependency on the device, firmware is too large for available program or data memory |

Table 6.7.2.2.2-1 Management Adapter –installFirmware capability

##### 6.7.2.2.3	Post-Conditions

The Management Adapter has submitted a request to the Management Server to install firmware.

##### 6.7.2.2.4	Exceptions

Not Applicable

##### 6.7.2.2.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.2.6	oneM2M Resource Interworking

This service capability is used to install firmware. The service capability aligns with the <firmware> resource and maps to the UPDATE procedure for the attribute update of the resource. The service capability also aligns with the <mgmtCmd> resource and maps to the EXECUTE procedure for the resource.

#### 6.7.2.3	getFirmwareInformation

This service capability provides the ability to retrieve the information related to the firmware on a device..

##### 6.7.2.3.1	Pre-conditions

A correlation between a Management Server, the M2M Service Capability and device exist.

##### 6.7.2.3.2	Signature –getFirmwareInformation

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | YES | The unique device identifier in the context of the M2M Service Subscription. |
| requestId | IN | NO | The M2M Request Identifier (M2M-Request-ID) |
| firmwareInfo | OUT | YES | The device firmware information. Type FirmwareInfo, see 6.6.1.1.1.5 |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Exception: Request may not have been completed |

Table 6.7.2.3.2-1 Management Adapter –getFirmwareInformation capability

##### 6.7.2.3.3	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get the firmware information.

##### 6.7.2.3.4	Exceptions

Not Applicable

##### 6.7.2.3.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.3.6	oneM2M Resource Interworking

This service capability is used to get firmware information. The service capability aligns with the <firmware> resource and maps to the RETRIEVE procedure for the attribute version, name, URL of the resource.

#### 6.7.2.4	getFirmwareExecStatus

This service capability provides the ability to get state of firmware management operation for a specific device.

##### 6.7.2.4.1	Pre-conditions

A correlation between a Management Server, the M2M Service Capability and previously submitted firmware request exist.

##### 6.7.2.4.2	Signature –getFirmwareExecInstance

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| operationRequestId | IN | NO | The M2M Request Identifier of previously submitted firmware request (M2M-Request-ID) |
| deviceId | IN | YES | The unique device identifier in the context of the M2M Service Subscription. When provided the FirmwareReport type is generated for only the specified device. |
| firmwareReport | OUT | YES | The firmware management operation execution result or status. Type FirmwareReport, see 6.6.1.1.1.3. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - The previously submitted firmware request does not exist |

Table 6.7.2.4.2-1 Management Adapter –getFirmwareExecStatus capability

##### 6.7.2.4.3	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get the firmware operation execution status or result for each device in the operation request.

##### 6.7.2.4.4	Exceptions

Not Applicable

##### 6.7.2.4.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.4.6	oneM2M Resource Interworking

This service capability is used to get state of firmware management operation. The service capability aligns with the <firmware> resource and maps to the RETRIEVE procedure for the attribute updateStatus of the resource. The service capability also aligns with the <execInstance> resource and maps to the RETRIEVE procedure for the attribute execStatus, execResult of the resource.

#### 6.7.2.5	deviceManagementReport

This service capability provides the ability for the Management Adapter to report the device management operation execution result or status information. The Management Adapter can send one or multiple reports according to the report policy.

##### 6.7.2.5.1	Pre-conditions

A correlation between a Management Server, the M2M Service Capability and previously submitted requests exist.

##### 6.7.2.5.2	Signature –deviceManagementReport

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| to | IN | NO | The identifier of the AE that is to receive the data(M2M-AppInst-ID) |
| isLastReport | IN | NO | Boolean, whether it is the last report. |
| sequenceNumber | IN | NO | The report sequence number. |
| firmwareReportList | IN | YES | Array of firmwareReport. Type FirmwareReport, see 6.6.1.1.1.3. |
| troubleshootingReportList | IN | YES | Array of troubleshootingReport. Type TroubleshootingReport, see 6.6.1.1.1.20. |
| softwareReportList | IN | YES | Array of softwareReport. Type SoftwareReport, see 6.6.1.1.1.27. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - The target AE does not exist. |

Table 6.7.2.5.2-1 Management Adapter –deviceManagementReport capability

##### 6.7.2.5.3	Service Interactions

The interactions of service capabilities required for this service capability:

1)  Issue the request to the Supporting Service to perform the operation

Figure 6.7.2.5.3-1 deviceManagementReport Diagram

##### 6.7.2.5.4	Post-Conditions

The M2M System maintains the state of the report (i.e., sequence number) according to the report policy.

##### 6.7.2.5.5	Exceptions

Not Applicable

##### 6.7.2.5.6	Policies for Use

Message Exchange Patterns: Not Applicable

Transaction Pattern: Not Applicable

##### 6.7.2.5.7	oneM2M Resource Interworking

Not Applicable

#### 6.7.2.6	getDeviceInformation

This service capability provides the ability to get the specific device information.

##### 6.7.2.6.1	Pre-conditions

A correlation between a Management Server, the M2M Service Capability and device exist.

##### 6.7.2.6.2	Signature –getDeviceInformation

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| deviceInfo | OUT | YES | The device information. Type DeviceInfo, see 6.6.1.1.1.7. |
| memory | OUT | YES | The memory information. Type Memory, see 6.6.1.1.1.8. |
| battery | OUT | YES | The battery information. Type Battery, see 6.6.1.1.1.9. |
| lockStatus | OUT | YES | The device lock status. Enum LockStatus, see 6.6.1.1.1.13. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Exception: Request may not have been completed |

Table 6.7.2.6.2-1 Management Adapter –getDeviceInformation capability

##### 6.7.2.6.3	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get the device information.

##### 6.7.2.6.4	Exceptions

Not Applicable

##### 6.7.2.6.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.6.6	oneM2M Resource Interworking

This service capability is used to get device information. The service capability aligns with the <deviceInfo> resource and maps to the RETRIEVE procedure for the resource. The service capability aligns with the <memory> resource and maps to the Retrieve procedure for the resource. The service capability aligns with the <battery> resource and maps to the Retrieve procedure for the resource.

#### 6.7.2.7	getDeviceCapabilities

This service capability provides the ability to get device capabilities.

##### 6.7.2.7.1	Pre-conditions

A correlation between a Management Server, the M2M Service Capability and device exist.

##### 6.7.2.7.2	Signature –getDeviceCapabilities

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| deviceCapabilities | OUT | YES | Array of deviceCapability. Type DeviceCapability, see 6.6.1.1.1.10. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Exception: Request may not have been completed |

Table 6.7.2.7.2-1 Management Adapter –getDeviceCapabilities capability

##### 6.7.2.7.3	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get device capabilities.

##### 6.7.2.7.4	Exceptions

Not Applicable

##### 6.7.2.7.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.7.6	oneM2M Resource Interworking

This service capability is used to get device capabilities. The service capability aligns with the<deviceCapability> resource and maps to the RETRIEVE procedure for the resource.

#### 6.7.2.8	enableDeviceCapability

This service capability provides the ability to enable device capability.

##### 6.7.2.8.1	Pre-conditions

A correlation between a Management Server, the M2M Service Capability and device exist.

##### 6.7.2.8.2	Signature –enableDeviceCapability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| name | IN | NO | The name of the capability. Enum DeviceCapabilityName, see 6.6.1.1.1.12. |
| state | OUT | NO | Indicates if the capability is enabled or disabled. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Exception: Request may not have been completed |

Table 6.7.2.8.2-1 Management Adapter –enableDeviceCapability capability

##### 6.7.2.8.3	Post-Conditions

The Management Adapter has submitted a request to the Management Server to enable device capability.

##### 6.7.2.8.4	Exceptions

Not Applicable

##### 6.7.2.8.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.8.6	oneM2M Resource Interworking

This service capability is used to enable device capability. The service capability aligns with the <deviceCapability> resource and maps to the UPDATE procedure for the attribute enable of the resource.

#### 6.7.2.9	disableDeviceCapability

This service capability provides the ability to disable device capability.

##### 6.7.2.9.1	Pre-conditions

A correlation between a Management Server, the M2M Service Capability and device exist.

##### 6.7.2.9.2	Signature –disableDeviceCapability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| name | IN | NO | The name of the capability. Enum DeviceCapabilityName, see 6.6.1.1.1.12. |
| state | OUT | NO | Indicates if the capability is enabled or disabled. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Exception: Request may not have been completed |

Table 6.7.2.9.2-1 Management Adapter –disableDeviceCapability capability

##### 6.7.2.9.3	Post-Conditions

The Management Adapter has submitted a request to the Management Server to disable device capability.

##### 6.7.2.9.4	Exceptions

Not Applicable

##### 6.7.2.9.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.9.6	oneM2M Resource Interworking

This service capability is used to disable device capability. The service capability aligns with the <deviceCapability> resource and maps to the UPDATE procedure for the attribute disable of the resource.

#### 6.7.2.10	getAreaNetworks

This service capability provides the ability to get area networks information.

##### 6.7.2.10.1	Pre-conditions

A correlation between a Management Server, the M2M Service Capability and area network exist.

##### 6.7.2.10.2	Signature –getAreaNetworks

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| AreaNwks | OUT | YES | Array of area network. Type AreaNwkInfo, see 6.6.1.1.1.13. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Exception: Request may not have been completed |

Table 6.7.2.10.2-1 Management Adapter –getAreaNetworks capability

##### 6.7.2.10.3	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get area networks information.

##### 6.7.2.10.4	Exceptions

Not Applicable

##### 6.7.2.10.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.10.6	oneM2M Resource Interworking

This service capability is used to get area networks information. The service capability aligns with the <areaNwkInfo> resource and maps to the RETRIEVE procedure for the resource. The service capability aligns with the <areaNwkDeviceInfo> resource and maps to the Retrieve procedure for the resource.

#### 6.7.2.11	updateDeviceForAreaNetwork

This service capability provides the ability for the Management Adapter to update device information for area network.

##### 6.7.2.11.1	Pre-conditions

A correlation between a Management Server, the M2M Service Capability, the device and area network exists.

##### 6.7.2.11.2	Signature –updateDeviceForAreaNetwork

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| areaNwkId | IN | NO | The unique area network identifier. |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| areaNwkDeviceInfo | IN | YES | The existing areaNwkDeviceInfo are replaced with the information in this parameter. Type AreaNwkDeviceInfo, see 6.6.1.1.1.15. |
| lastModifiedTime | OUT | NO | The modified time. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Exception: Request may not have been completed |

Table 6.7.2.11.2-1 Management Adapter –updateDeviceForAreaNetwork capability

##### 6.7.2.11.3	Post-Conditions

The Management Adapter has submitted a request to the Management Server to update device information for area network.

##### 6.7.2.11.4	Exceptions

Not Applicable

##### 6.7.2.11.5	Policies for Use

Message Exchange Patterns: Not Applicable

Transaction Pattern: Not Applicable

##### 6.7.2.11.6	oneM2M Resource Interworking

The service capability aligns with the <areaNwkDeviceInfo> resource and maps to the Update procedure for attribute devType , sleepInterval , sleepDuration , status , listOfNeighbors of the resource.

#### 6.7.2.12	rebootDevice

This service capability provides the ability to reboot the specific device.

##### 6.7.2.12.1	Pre-conditions

A correlation between a Management Server, the M2M Service Capability and device exist.

##### 6.7.2.12.2	Signature –rebootDevice

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.1.1.1.1. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Exception: Request may not have been completed |

Table 6.7.2.12.2-1 Management Adapter –rebootDevice capability

##### 6.7.2.12.3	Post-Conditions

The Management Adapter has submitted a request to the Management Server to reboot the device.

##### 6.7.2.12.4	Exceptions

Not Applicable

##### 6.7.2.12.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.12.6	oneM2M Resource Interworking

This service capability is used to reboot device. The service capability aligns with the <reboot> resource and maps to the Execute procedure for the attribute reboot of the resource.

#### 6.7.2.13	resetDevice

This service capability provides the ability to reset device.

##### 6.7.2.13.1	Pre-conditions

A correlation between a Management Server, the M2M Service Capability and device exist.

##### 6.7.2.13.2	Signature –resetDevice

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.1.1.1.1. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Exception: Request may not have been completed |

Table 6.7.2.13.2-1 Management Adapter –resetDevice capability

##### 6.7.2.13.3	Post-Conditions

The Management Adapter has submitted a request to the Management Server to reset device.

##### 6.7.2.13.4	Exceptions

Not Applicable

##### 6.7.2.13.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.13.6	oneM2M Resource Interworking

This service capability is used to reset device. The service capability aligns with the <reboot> resource and maps to the Execute procedure for the attribute factoryReset of the resource.

#### 6.7.2.14	uploadDeviceLog

This service capability provides the ability to upload device log.

##### 6.7.2.14.1	Pre-conditions

A correlation between a Management Server, the M2M Service Capability and device exist.

##### 6.7.2.14.2	Signature –uploadDeviceLog

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| logInfo | IN | NO | The log information. Type LogInfo, see 6.6.1.1.1.19. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.1.1.1.1. |
| logURL | OUT | YES | The URL from which the log can be uploaded. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Exception: Request may not have been completed |

Table 6.7.2.14.2-1 Management Adapter –uploadDeviceLog capability

##### 6.7.2.14.3	Post-Conditions

The Management Adapter has submitted a request to the Management Server to upload device log.

##### 6.7.2.14.4	Exceptions

Not Applicable

##### 6.7.2.14.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.14.6	oneM2M Resource Interworking

This service capability is used to upload device log. The service capability aligns with the <eventLog> resource and maps to the Update procedure for the resource.

#### 6.7.2.15	getDeviceLogs

This service capability provides the ability to get all logs of a device.

##### 6.7.2.15.1	Pre-conditions

A correlation between a Management Server, the M2M Service Capability and device exist.

##### 6.7.2.15.2	Signature –getDeviceLogs

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| logList | OUT | YES | Array of log. Type Log, see 6.6.1.1.1.21. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Exception: Request may not have been completed |

Table 6.7.2.15.2-1 Management Adapter –getDeviceLogs capability

##### 6.7.2.15.3	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get all logs of a device.

##### 6.7.2.15.4	Exceptions

Not Applicable

##### 6.7.2.15.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.15.6	oneM2M Resource Interworking

This service capability is used to get all logs of a device. The service capability aligns with the <eventLog> resource and maps to the Retrieve procedure for the resource.

#### 6.7.2.16	getDeviceLogInformation

This service capability provides the ability to get a device log information.

##### 6.7.2.16.1	Pre-conditions

A correlation between a Management Server, the M2M Service Capability and device exist.

##### 6.7.2.16.2	Signature –getDeviceLogInformation

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| filterCriteria | IN | YES | See Table 6.6.1.1.1.22 |
| logURL | IN | YES | The URL from which the log can be accessed. |
| log | OUT | YES | Type Log, see 6.6.1.1.1.21. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Exception: Request may not have been completed |

Table 6.7.2.16.2-1 Management Adapter –getDeviceLogInformation capability

##### 6.7.2.16.3	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get a device log information.

##### 6.7.2.16.4	Exceptions

Not Applicable

##### 6.7.2.16.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.16.6	oneM2M Resource Interworking

This service capability is used to get device log information. The service capability aligns with the <eventLog> resource and maps to the Retrieve procedure for the resource.

#### 6.7.2.17	getSoftwareInformation

This service capability provides the ability to get application software information.

##### 6.7.2.17.1	Pre-conditions

A correlation between a Management Server, the M2M Service Capability and device exist.

##### 6.7.2.17.2	Signature –getSoftwareInformation

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| version | OUT | YES | The version of the software. |
| name | OUT | YES | The name of the software. |
| url | OUT | YES | The URL from which the software package can be downloaded. |
| installStatus | OUT | YES | Indicates the status of the install. Enum ActionStatus, see 6.6.1.1.1.26. |
| activeStatus | OUT | YES | The status of active or deactivate action. Enum ActionStatus, see 6.6.1.1.1.26. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Exception: Request may not have been completed |

Table 6.7.2.17.2-1 Management Adapter –getSoftwareInformation capability

##### 6.7.2.17.3	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get application software information.

##### 6.7.2.17.4	Exceptions

Not Applicable

##### 6.7.2.17.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.17.6	oneM2M Resource Interworking

This service capability is used to get application software information. The service capability aligns with the <software> resource and maps to the Retrieve procedure for the resource.

#### 6.7.2.18	downloadSoftware

This service capability provides the ability to download application software.

##### 6.7.2.18.1	Pre-conditions

A correlation between a Management Server, the M2M Service Capability and device exist.

##### 6.7.2.18.2	Signature –downloadSoftware

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| version | IN | NO | The version of the software. |
| name | IN | NO | The name of the software. |
| url | IN | NO | The URL from which the software package can be downloaded. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.1.1.1.1. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Exception: Request may not have been completed |

Table 6.7.2.18.2-1 Management Adapter –downloadSoftware capability

##### 6.7.2.18.3	Post-Conditions

The Management Adapter has submitted a request to the Management Server to download application software.

##### 6.7.2.18.4	Exceptions

Not Applicable

##### 6.7.2.18.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.18.6	oneM2M Resource Interworking

This service capability is used to download application software. The service capability aligns with the <software> resource and maps to the UPDATE procedure for the attribute install of the resource. The service capability also aligns with the <mgmtCmd> resource and maps to the EXECUTE procedure for the resource.

#### 6.7.2.19	installSoftware

This M2M Service Capability provides the ability to install application software.

##### 6.7.2.19.1	Pre-conditions

A correlation between a Management Server, the service capability and device exist.

##### 6.7.2.19.2	Signature –installSoftware

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| version | IN | NO | The version of the software. |
| name | IN | NO | The name of the software. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.1.1.1.1. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Exception: Request may not have been completed |

19.2-1 Management Adapter –installSoftware capability

##### 6.7.2.19.3	Post-Conditions

The Management Adapter has submitted a request to the Management Server to install application software.

##### 6.7.2.19.4	Exceptions

Not Applicable

##### 6.7.2.19.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.19.6	oneM2M Resource Interworking

This service capability is used to install application software. The service capability aligns with the <software> resource and maps to the UPDATE procedure for the attribute install of the resource. The service capability also aligns with the <mgmtCmd> resource and maps to the EXECUTE procedure for the resource.

#### 6.7.2.20	activateSoftware

This service capability provides the ability to activate software previously installed.

##### 6.7.2.20.1	Pre-conditions

A correlation between a Management Server, the M2M Service Capability and device exist.

##### 6.7.2.20.2	Signature –activateSoftware

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| version | IN | NO | The version of the software. |
| name | IN | NO | The name of the software. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.1.1.1.1. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Exception: Request may not have been completed |

Table 6.7.2.21.2-1 Management Adapter –activateSoftware capability

##### 6.7.2.20.3	Post-Conditions

The Management Adapter has submitted a request to the Management Server to activate software previously installed.

##### 6.7.2.20.4	Exceptions

Not Applicable

##### 6.7.2.20.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.20.6	oneM2M Resource Interworking

This service capability is used to activate software previously installed. The service capability aligns with the <software> resource and maps to the UPDATE procedure for the attribute activate of the resource. The service capability also aligns with the <mgmtCmd> resource and maps to the EXECUTE procedure for the resource.

#### 6.7.2.21	deactivateSoftware

This service capability provides the ability for the Management Adapter to deactivates software.

##### 6.7.2.21.1	Pre-conditions

A correlation between a Management Server, the M2M Service Capability and device exist.

##### 6.7.2.21.2	Signature –deactivateSoftware

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| version | IN | NO | The version of the software. |
| name | IN | NO | The name of the software. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.1.1.1.1. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Exception: Request may not have been completed |

Table 6.7.2.21.2-1 Management Adapter –deactivateSoftware capability

##### 6.7.2.21.3	Post-Conditions

The Management Adapter has submitted a request to the Management Server to deactivates software.

##### 6.7.2.21.4	Exceptions

Not Applicable

##### 6.7.2.21.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.21.6	oneM2M Resource Interworking

This service capability is used to deactivates software. The service capability aligns with the <software> resource and maps to the UPDATE procedure for the attribute deactivate of the resource. The service capability also aligns with the <mgmtCmd> resource and maps to the EXECUTE procedure for the resource.

#### 6.7.2.22	removeSoftware

This service capability provides the ability for the Management Adapter to uninstall the software.

##### 6.7.2.22.1	Pre-conditions

A correlation between a Management Server, the M2M Service Capability and device exist.

##### 6.7.2.22.2	Signature –removeSoftware

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| version | IN | NO | The version of the software. |
| name | IN | NO | The name of the software. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.1.1.1.1. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Exception: Request may not have been completed |

Table 6.7.2.22.2-1 Management Adapter –removeSoftware capability

##### 6.7.2.22.3	Post-Conditions

The Management Adapter has submitted a request to the Management Server to uninstall the software.

##### 6.7.2.22.4	Exceptions

Not Applicable

##### 6.7.2.22.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.22.6	oneM2M Resource Interworking

This service capability is used to install application software. The service capability aligns with the <software> resource and maps to the UPDATE procedure for the attribute uninstall of the resource. The service capability also aligns with the <mgmtCmd> resource and maps to the EXECUTE procedure for the resource.

## 6.8	Service Administration

### 6.8.1	Overview

The Service Administration service provides the capability to administer the following across the Msc Reference Points:

Administration of M2M Service attributes

Association of M2M Services to corresponding Service Roles

Association of Service Roles to M2M Service Capabilities

The information elements that relate M2M Services to Service Roles and Service Roles to M2M Service Capabilities is depicted below:

Each instance of an M2M Service contains zero or instances of a Service Role

The M2M Service can be obtained from the Service Role. The Service Roles from the M2M Service can be obtained from the M2M Service

Each instance of a Service Role is associated with zero or more instances of a M2M Service Capability

The M2M Service can be obtained from the Service Role. The Service Roles associated with the M2M Service can be obtained from the M2M Service

Each instance of a M2M Service Capability is associated with zero or more instances of a Service Role

The M2M Service Capabilities that are associated with a Service Role can be obtained from the Service Role. Likewise the Service Roles associated with the M2M Service Capability can be obtained from the M2M Service Capability

Each operation that is defined within the Service Exposure Component is assigned to exactly one M2M Service Capability

Figure 6.8.1-1 M2M Service Entity Relationships

#### 6.8.1.1	M2M Service Entity

The Service is a primary entity that provides the attributes to identify the M2M Service and associated Service Roles within the oneM2M System. These M2M Services and associated Service Roles shall be maintained by the M2M Service Provider.

| Attribute name | Description |
| --- | --- |
| serviceId | The M2M Service Identifier (M2M-Serv-ID). |
| serviceRoleIds | List of Service Role Identifiers (SRole-ID) associated with the Service. |

Table 6.8.1.1-1 M2M Service Entity

##### 6.8.1.1.1	M2M Service Filter Criteria

| Criterion name | Description |
| --- | --- |
| serviceIds | List of the Service Identifiers (M2M-Serv-ID). |
| serviceRoleIds | One or more Service Role Identifiers (SRole-ID) associated with the M2M Service entity. |

Table 6.8.1.1.1-1 M2M Service Filter Criteria

### 6.8.2	Service Capabilities

#### 6.8.2.1	createM2MService

This service capability provides the ability to create a M2M Service.

##### 6.8.2.1.1	Pre-conditions

Not Applicable

##### 6.8.2.1.2	Signature – createM2MService

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| name | IN | NO | The unique name of the M2M Service. |
| description | IN | YES | The description of the M2M Service. |
| serviceId | OUT | NO | The identifier of the created M2M Service (M2M-Serv-ID) |
| responseType | OUT | YES | Unique response types for this service. - M2M Service exists for the name parameter - M2M Service Role exists for M2M Service |

Table 6.8.2.1.2-1 Service Administration – createM2M Service capability

##### 6.8.2.1.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue this capability

Figure 6.8.2.1.3-1 Service Administration – createM2MService Diagram

##### 6.8.2.1.4	Post-Conditions

Not Applicable

##### 6.8.2.1.5	Exceptions

Not Applicable

##### 6.8.2.1.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.8.2.1.7	oneM2M Resource Interworking

This service capability creates the M2M Service. The M2M Service entity is described in the Annex G of the oneM2M Functional Architecture [2]. The M2M-Serv-ID created using this capability shall map to the M2M-Serv-ID of the oneM2M Functional Architecture [2].

#### 6.8.2.2	deleteM2MService

This service capability provides the ability to delete a M2M Service. This service capability shall be restricted to the Msc Reference Point.

##### 6.8.2.2.1	Pre-conditions

The M2M Service’s Service Roles are not associated with any M2M Service Subscriptions.

##### 6.8.2.2.2	Signature – deleteM2MService

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceId | IN | NO | The M2M Service Identifier (M2M-Serv-ID). |
| responseType | OUT | YES | Unique response types for this service. M2M Service does not exist M2M Service has Service Roles associated with M2M Service Subscriptions |

Table 6.8.2.2.2-1 Service Administration – deleteM2MService capability

##### 6.8.2.2.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue this capability

Figure 6.8.2.2.3-1 Service Administration – deleteM2MService Diagram

##### 6.8.2.2.4	Post-Conditions

The M2M Service is deleted along with any associations has to the Service Roles. The Node information associated with the M2M Service is also deleted.

##### 6.8.2.2.5	Exceptions

Not Applicable

##### 6.8.2.2.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.8.2.2.7	oneM2M Resource Interworking

This service capability deletes the M2M Service. The M2M Service entity is described in the Annex G of the oneM2M Functional Architecture [2].

#### 6.8.2.3	addRoleToM2MService

This service capability provides the ability to add new Service Roles to a M2M Service. This service capability shall be restricted to the Msc Reference Point.

##### 6.8.2.3.1	Pre-conditions

M2M Service exists.

##### 6.8.2.3.2	Signature – addRoleToM2MService

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceId | IN | NO | The M2M Service Identifier (M2M-Serv-ID). |
| serviceRoleIds | IN | NO | List of Service Role Identifiers (SRole-ID) to be associated with the M2M Service. If a SRole-ID in the parameter is already associated with the M2M Service, nothing is done for the SRole-ID as it already is associated. |
| responseType | OUT | YES | Unique response types for this service. M2M Service does not exist |

Table 6.8.2.3.2-1 Service Administration – addRoleToM2MService capability

##### 6.8.2.3.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue this capability

Figure 6.8.2.3.3-1 Service Administration – addRoleToM2MService Diagram

##### 6.8.2.3.4	Post-Conditions

The associations that M2M Service has to the Service Role(s) are updated.

##### 6.8.2.3.5	Exceptions

Not Applicable

##### 6.8.2.3.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.8.2.3.7	oneM2M Resource Interworking

This service capability updates the M2M Service with the respective M2M Service Roles. The M2M Service entity is described in the Annex G of the oneM2M Functional Architecture [2].

#### 6.8.2.4	deleteRoleFromM2MService

This service capability provides the ability to delete existing Service Roles from a M2M Service. This service capability shall be restricted to the Msc Reference Point.

##### 6.8.2.4.1	Pre-conditions

The Service Role is not associated with any M2M Service Subscriptions.

##### 6.8.2.4.2	Signature – deleteRoleFromM2MService

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceId | IN | NO | The M2M Service Identifier (M2M-Serv-ID). |
| serviceRoleIds | IN | NO | List of Service Role Identifiers (SRole-ID) to be deleted from the association with the M2M Service. |
| responseType | OUT | YES | Unique response types for this service. - M2M Service does not exist - Service Role does not exist - Service Role is associated with a M2M Service Subscription |

Table 6.8.2.4.2-1 Service Administration – deleteRoleFromM2MService capability

##### 6.8.2.4.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue this capability

Figure 6.8.2.4.3-1 Service Administration – deleteRoleFromM2MService Diagram

##### 6.8.2.4.4	Post-Conditions

The M2M Service is updated with associations of the identified Service Roles deleted.

##### 6.8.2.4.5	Exceptions

Not Applicable

##### 6.8.2.4.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.8.2.4.7	oneM2M Resource Interworking

This service capability updates the M2M Service with the respective Service Roles removed. The M2M Service entity is described in the Annex G of the oneM2M Functional Architecture [2].

#### 6.8.2.5	getM2MService

This service capability provides the ability to retrieve the existing M2M Service. This service capability shall be restricted to the Msc Reference Points.

##### 6.8.2.5.1	Pre-conditions

Not Applicable

##### 6.8.2.5.2	Signature – getM2MService

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| filterCriteria | IN | NO | See M2M Service Filter Criteria Table 6.8.1.1.1-1 |
| services | OUT | NO | The resulting M2M Service entities in Table 6.8.1.1-1. |
| responseType | OUT | YES | Unique response types for this service. - None |

Table 6.8.2.5.2-1 Service Administration – getM2MService capability

##### 6.8.2.5.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue this capability

Figure 6.8.2.5.3-1 Service Administration – getM2MService Diagram

##### 6.8.2.5.4	Post-Conditions

Not Applicable

##### 6.8.2.5.5	Exceptions

Not Applicable

##### 6.8.2.5.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.8.2.5.7	oneM2M Resource Interworking

This service capability retrieves the M2M Service. The M2M Service entity is described in the Annex G of the oneM2M Functional Architecture [2].

#### 6.8.2.6	addServiceCapabilityToRole

This service capability adds a M2M Service Capability to one or more Service Roles.

##### 6.8.2.6.1	Pre-conditions

Service Roles exist.

##### 6.8.2.6.2	Signature – addServiceCapabilityToRole

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceCapId | IN | NO | The M2M Service Capability Identifier (M2M-Serv-Cap-ID). |
| serviceRoleIds | IN | NO | List of Service Role Identifiers (SRole-ID) to be associated with the M2M Service Capability. |
| responseType | OUT | YES | Response types: - Service Role does not exist - M2M Service Capability is not part of the capabilities of M2M Service associated with the Service Role |

Table 6.8.2.6.2-1 Service Administration – addServiceCapabilityToRole capability

##### 6.8.2.6.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue this capability

Figure 6.8.2.6.3-1Service Administration – addServiceCapabilityToRole Diagram

##### 6.8.2.6.4	Post-Conditions

Not Applicable

##### 6.8.2.6.5	Exceptions

Not Applicable

##### 6.8.2.6.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.8.2.6.7	oneM2M Resource Interworking

Not Applicable

#### 6.8.2.7	deleteServiceCapabilityFromRole

This service capabilitydelete a M2M Service Capability from one or more Service Roles.

##### 6.8.2.7.1	Pre-conditions

The association between the M2M Service Capability and Service Role exists.

##### 6.8.2.7.2	Signature – deleteServiceCapabilityFromRole

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceCapId | IN | NO | The M2M Service Capability Identifier (M2M-Serv-Cap-ID). |
| serviceRoleIds | IN | NO | List of Service Role Identifiers (SRole-ID) to be associated with the M2M Service Capability to be removed |
| responseType | OUT | YES | Response types: - Service Role does not exist - Service Role is not associated with a M2M Service Capability |

Table 6.8.2.7.2-1 Service Administration – deleteServiceCapabilityFromRole capability

##### 6.8.2.7.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue this capability

Figure 6.8.2.7.3-1Service Administration – deleteServiceCapabilityFromRole Diagram

##### 6.8.2.7.4	Post-Conditions

The association of the M2M Service Capability and Service Role is deleted.

##### 6.8.2.7.5	Exceptions

Not Applicable

##### 6.8.2.7.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.8.2.7.7	oneM2M Resource Interworking

Not Applicable

#### 6.8.2.8	getServiceCapability

This service capability retrieves the M2M Service Capability and associated Service Roles for a specified requested operation.

##### 6.8.2.8.1	Pre-conditions

There is a correlation to the service capabilities’ operation (signature) and the M2M Service Capability. In many deployments the service capabilities’ signature serves as the M2M Service Capability.

##### 6.8.2.8.2	Signature – getServiceCapability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| operation | IN | NO | Request operation – Common to all requests |
| serviceCapId | OUT | NO | The M2M Service Capability Identifier (M2M-Serv-Cap-ID). |
| serviceRoleIds | OUT | NO | List of the M2M Service Roles Identifiers (SRole-ID) associated with the M2M Service Capabilities. |
| responseType | OUT | YES | Response types: - Unable to determine M2M Service Capability from the operation |

Table 6.8.2.8.2-1 Service Administration – getServiceCapability capability

##### 6.8.2.8.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue this capability

Figure 6.8.2.8.3-1 Service Administration – getServiceCapability Diagram

##### 6.8.2.8.4	Post-Conditions

Not Applicable

##### 6.8.2.8.5	Exceptions

Not Applicable

##### 6.8.2.8.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.8.2.8.7	oneM2M Resource Interworking

Not Applicable

## 6.9	Service Subscription Administration

### 6.9.1	Overview

The Service Subscription Administration service provides the capability to administer the following across the Mca and Msc Reference Points:

Maintenance of M2M Services Subscriptions

Association of M2M Nodes and Application Rules to the M2M Service Subscription

The M2M Service Subscription defines the technical part of the contract between a M2M Subscriber (typically an M2M Application Service Provider) and a M2M Service Provider. Each M2M Service Subscription has a unique identifier (M2M-Service-Profile-ID).

A M2M Service Subscription establishes an association between one or more M2M Application Rules and one or more M2M Nodes. In addition, the M2M Service Subscription is associated with one or more Service Roles.

A M2M Service Subscription shall be used for the following purposes:

Serve as a basis for authorization for service capabilities

Serve as the basis for charging

Identify which devices and M2M Applications are part of this M2M Service Subscription

#### 6.9.1.1	Service Subscription Entity

| Attribute name | Description |
| --- | --- |
| serviceSubscriptionId | The M2M Service Subscription Identifier (M2M-Service-Profile-ID). |
| serviceRoleIds | List of Service Role Identifiers (SRole-ID) associated with the M2M Service Subscription. |

Table 6.9.1.1-1 Service Subscription Entity

##### 6.9.1.1.1	Service Subscription Filter Criteria

| Criterion name | Description |
| --- | --- |
| serviceSubscriptionIds | List of the M2M Service Subscription Identifiers (M2M-Service-Profile-ID). |
| serviceRoleIds | List of Service Role Identifiers (SRole-ID) associated with the M2M Service Subscription entity. |

Table 6.9.1.1.1-1 Service Subscription Filter Criteria

#### 6.9.1.2	M2M Node Entity for Service Subscription

| Attribute name | Description |
| --- | --- |
| serviceSubscriptionId | The M2M Service Subscription Identifier (M2M-Service-Profile-ID) for the Device. |
| nodeId | The unique M2M Node identifier in the context of the M2M Service Subscription. |
| externalIds | List of URNs that represent the external identifiers associated with this M2M Node. |
| applicationRuleIds | List of Application Rules Indentifiers associated with the M2M Node. |

Table 6.9.1.2-1 M2M Node Entity for Service Subscription

#### 6.9.1.2.1	M2M Node Filter Criteria

| Criterion name | Description |
| --- | --- |
| serviceSubscriptionIds | List of the M2M Service Subscription Identifier (M2M-Service-Profile-ID) for the Device. |
| nodeIds | List of the unique M2M Node identifier in the context of the M2M Service Subscription. |
| externalIds | List of URNs that represent the external identifiers associated with this M2M Node. |
| applicationRuleIds | List of Application Rule identifiers associated with the M2M Node. |

Table 6.9.1.2.1-1 M2M Node Filter Criteria

#### 6.9.1.3	Application Rule

| Criterion name | Description |
| --- | --- |
| applicationRuleId | The unique Application Rule identifier in the context of the M2M Service Subscription. |
| credentialIds | List of credential identifiers for which this Application Rule is applicable, i.e. for registration requests coming into a CSE via a Security Association Endpoint (SEA) [i.4], that was authenticated using credentials that match with any of these credential identifiers, the current Application Rule applies. This can contain a ‘*’ for any credential identifier or ‘None’ for not authenticated case. Also Wildcards within an element of this list are possible (e.g. ‘C123*X’ for any credential identifier that starts with ‘C123’ and ends with ‘X’) to define sets or ranges of credential identifiers. |
| allowedApplicationIds | List of M2M Application Identifiers (App-ID) that shall be considered to be allowed for AE registration requests received via Security Association Endpoint (SEA) [i.4]. This can contain ‘*’ for any App-ID. Also Wildcards within an element of this list are possible (e.g. ‘C123*X’ for any App-ID that starts with ‘C123’ and ends with ‘X’) to define sets or ranges of App-IDs. |
| allowedAEIds | List of M2M Application Entity Identifiers (AE-ID) that shall be considered to be allowed for AE registration requests. This can contain zero or more specific AE identifier values, ‘S*’ for any Service Provider AE-ID, ‘C*’ for any system assigned AE-ID, or ‘*’ for any AE-ID. Also Wildcards within an element of this list are possible (e.g. ‘C123*X’ for any AE-ID that starts with ‘C123’ and ends with ‘X’) to define sets or ranges of AE-ID. |

Table 6.9.1.3-1 Application Rule Entity

#### 6.9.1.3.1	Application Rule Filter Criteria

| Criterion name | Description |
| --- | --- |
| applicationRuleIds | List of the unique Application Rule identifier in the context of the M2M Service Subscription. |
| credentialIds | List of credential identifiers for which this Application Rule is applicable, i.e. for registration requests coming into the M2M System via a Security Association Endpoint (SEA) [i.1], that was authenticated using credentials that match with any of these credential identifiers, the current Application Rule applies. This can contain a ‘*’ for any credential identifier or ‘None’ for not authenticated case. Also Wildcards within an element of this list are possible (e.g. ‘C123*X’ for any credential identifier that starts with ‘C123’ and ends with ‘X’) to define sets or ranges of credential identifiers. |
| allowedApplicationIds | List of M2M Application Identifiers (App-ID) that shall be considered to be allowed for AE registration requests received via Security Association Endpoint (SEA) [i.2]. This can contain ‘*’ for any App-ID. Also Wildcards within an element of this list are possible (e.g. ‘C123*X’ for any App-ID that starts with ‘C123’ and ends with ‘X’) to define sets or ranges of App-IDs. |
| allowedAEIds | List of M2M Application Entity Identifiers (AE-ID) that shall be considered to be allowed for AE registration requests. This can contain zero or more specific AE identifier values, ‘S*’ for any Service Provider AE-ID, ‘C*’ for any system assigned AE-ID, or ‘*’ for any AE-ID. Also Wildcards within an element of this list are possible (e.g. ‘C123*X’ for any AE-ID that starts with ‘C123’ and ends with ‘X’) to define sets or ranges of AE-ID. |

Table 6.9.1.3.1-1 Application Rule Filter Criteria

#### 6.9.1.4	Supporting Rules

##### 6.9.1.4.1	Schedule Type

The Schedule is a simple type that describes the schedule information.

| Parameter | Optional | Description |
| --- | --- | --- |
| scheduleElement | NO | Expresses time periods defined by second, minute, hour day of month, month, and year. Supports repeating periods, and wildcards expressed as a list. The definition of a schedule is defined in clause 7.3.9 of TS-0004. |

Table 6.9.1.4.1-1 Type: Schedule

### 6.9.2	Service Capabilities

#### 6.9.2.1	createServiceSubscription

This service capability provides the ability to create a M2M Service Subscription. This service capability shall be restricted to the Msc Reference Point.

##### 6.9.2.1.1	Pre-conditions

Not Applicable

##### 6.9.2.1.2	Signature – createServiceSubscription

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceSubscriptionId | IN-OUT | YES | The M2M Service Subscription Identifier (M2M-Service-Profile-ID). If the attribute is not provided on input, the oneM2M System shall assign the M2M-Service-Profile-ID. The M2M-Service-Profile-ID is unique for the oneM2M System. |
| serviceRoleIds | IN | NO | List of Service Role Identifiers (SRole-ID) to be associated with the M2M Service Subscription. |
| responseType | OUT | YES | Unique response types for this service. - M2M Service Subscription exists - Service Role does not exist |

Table 6.9.2.1.2-1 Service Subscription Administration – createServiceSubscription capability

##### 6.9.2.1.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the createServiceSubscription capability

Figure 6.9.2.1.3-1 Service Subscription Administration – createServiceSubscription Diagram

##### 6.9.2.1.4	Post-Conditions

The M2M Service Subscription is created and any Service Roles are associated with the M2M Service Subscription.

##### 6.9.2.1.5	Exceptions

Not Applicable

##### 6.9.2.1.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.2.1.7	oneM2M Resource Interworking

This service capability creates the M2M Service Subscription with the identifier (M2M-Service-Profile-ID) and maps to the CREATE procedure for the <m2mServiceSubscriptionProfile> resource.

This service capability utilizes the access management services with the privileges associated with the associated Service Role. As such the accessControlPolicyIDs attribute of the <m2mServiceSubscriptionProfile> resource is not used within this service capability.

#### 6.9.2.2	deleteServiceSubscription

This service capability provides the ability to delete a M2M Service Subscription. This service capability shall be restricted to the Msc Reference Point.

##### 6.9.2.2.1	Pre-conditions

Not Applicable

##### 6.9.2.2.2	Signature – deleteServiceSubscription

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceSubscriptionId | IN | NO | The M2M Service Subscription Identifier (M2M-Service-Profile-ID). |
| responseType | OUT | YES | Unique response types for this service. - M2M Service Subscription does not exist |

Table 6.9.2.2.2-1 Service Subscription Administration – deleteServiceSubscription capability

##### 6.9.2.2.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the deleteServiceSubscription capability

Figure 6.9.2.2.3-1 Service Subscription Administration – deleteServiceSubscription Diagram

##### 6.9.2.2.4	Post-Conditions

The M2M Service Subscription is deleted along with any associations of the M2M Service Subscription has to the Service Roles. The Node information (e.g., devices, M2M Applications) associated with the M2M Service Subscription is also deleted.

##### 6.9.2.2.5	Exceptions

Not Applicable

##### 6.9.2.2.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.2.2.7	oneM2M Resource Interworking

This service capability deletes the M2M Service Subscription with the identifier (M2M-Service-Profile-ID) and maps to the DELETE procedure for the <m2mServiceSubscriptionProfile> resource.

#### 6.9.2.3	updateServiceSubscription

This service capability provides the ability to update a M2M Service Subscription. This service capability shall be restricted to the Msc Reference Point.

##### 6.9.2.3.1	Pre-conditions

Not Applicable

##### 6.9.2.3.2	Signature – updateServiceSubscription

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceSubscriptionId | IN | NO | The M2M Service Subscription Identifier (M2M-Service-Profile-ID). |
| serviceRoleIds | IN | YES | List of Service Role Identifiers (SRole-ID) to be associated with the M2M Service Subscription. If supplied, SRole-IDs in the list that are not already associated with the M2M Service Subscription are added. SRole-IDs that are associated with the M2M Service Subscription but not in the list are deleted. |
| roleIdResult | OUT | YES | If the parameter serviceRoleIds is present, this parameter provides the results (added, deleted) for each affected Service Roles. |
| responseType | OUT | YES | Unique response types for this service. - M2M Service Subscription does not exist - Service Role does not exist for a Supporting Service |

Table 6.9.2.3.2-1 Service Subscription Administration – updateServiceSubscription capability

##### 6.9.2.3.3	Service Interactions

- The interactions of service capabilities required for this service capability:

- Issue the updateServiceSubscription capability

Figure 6.9.2.3.3-1 Service Subscription Administration – updateServiceSubscription Diagram

##### 6.9.2.3.4	Post-Conditions

The M2M Service Subscription is updated along with any associations of the M2M Service Subscription has to the Service Roles.

##### 6.9.2.3.5	Exceptions

Not Applicable

##### 6.9.2.3.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.2.3.7	oneM2M Resource Interworking

This service capability updates the M2M Service Subscription with the identifier (M2M-Service-Profile-ID) and maps to the UPDATE procedure for the <m2mServiceSubscriptionProfile> resource.

#### 6.9.2.4	addRoleToServiceSubscription

This service capability provides the ability to add new Service Roles to a M2M Service Subscription. This service capability shall be restricted to the Msc Reference Point.

##### 6.9.2.4.1	Pre-conditions

Not Applicable

##### 6.9.2.4.2	Signature – addRoleToServiceSubscription

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceSubscriptionId | IN | NO | The M2M Service Subscription Identifier (M2M-Service-Profile-ID). |
| serviceRoleIds | IN | NO | List of Service Role Identifiers (SRole-ID) to be associated with the M2M Service Subscription. If a SRole-ID in the parameter is already associated with the M2M Service Subscription, nothing is done for the SRole-ID as it already is associated. |
| responseType | OUT | YES | Unique response types for this service. - M2M Service Subscription does not exist - Service Role does not exist for a Supporting Service |

Table 6.9.2.4.2-1 Service Subscription Administration – addRoleToServiceSubscription capability

##### 6.9.2.4.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the addRoleToServiceSubscription capability

Figure 6.9.2.4.3-1 Service Subscription Administration – addRoleToServiceSubscription Diagram

##### 6.9.2.4.4	Post-Conditions

The M2M Service Subscription is updated with any associations of the M2M Service Subscription has to the Service Roles.

##### 6.9.2.4.5	Exceptions

Not Applicable

##### 6.9.2.4.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.2.4.7	oneM2M Resource Interworking

This service capability updates the M2M Service Subscription with the identifier (M2M-Service-Profile-ID) and maps to the UPDATE procedure for the <m2mServiceSubscriptionProfile> resource for the serviceRoles attribute.

#### 6.9.2.5	deleteRoleFromServiceSubscription

This service capability provides the ability to delete existing Service Roles from a M2M Service Subscription. This service capability shall be restricted to the Msc Reference Point.

##### 6.9.2.5.1	Pre-conditions

Not Applicable

##### 6.9.2.5.2	Signature – deleteRoleFromServiceSubscription

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceSubscriptionId | IN | NO | The M2M Service Subscription Identifier (M2M-Service-Profile-ID). |
| serviceRoleIds | IN | NO | List of Service Role Identifiers (SRole-ID) to be deleted from the association with the M2M Service Subscription. |
| responseType | OUT | YES | Unique response types for this service. - M2M Service Subscription does not exist - Service Role does not exist for a Supporting Service |

Table 6.9.2.5.2-1 Service Subscription Administration – deleteRoleFromServiceSubscription capability

##### 6.9.2.5.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the deleteRoleFromServiceSubscription capability

Figure 6.9.2.5.3-1 Service Subscription Administration – deleteRoleFromServiceSubscription Diagram

##### 6.9.2.5.4	Post-Conditions

The M2M Service Subscription is updated with associations of the identified Service Roles deleted.

##### 6.9.2.5.5	Exceptions

Not Applicable

##### 6.9.2.5.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.2.5.7	oneM2M Resource Interworking

This service capability updates the M2M Service Subscription with the identifier (M2M-Service-Profile-ID) and maps to the UPDATE procedure for the <m2mServiceSubscriptionProfile> resource for the serviceRoles attribute.

#### 6.9.2.6	getServiceSubscription

This service capability provides the ability to retrieve the existing M2M Service Subscription. This service capability shall be restricted to the Msc and Mca Reference Points.

##### 6.9.2.6.1	Pre-conditions

Not Applicable

##### 6.9.2.6.2	Signature – getServiceSubscription

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| filterCriteria | IN | NO | See Table 6.9.1.1.1-1 |
| serviceSubscriptions | OUT | NO | The resulting M2M Service Subscription entities in Table 6.9.1.2-1. |
| responseType | OUT | YES | Unique response types for this service. - None |

Table 6.9.2.6.2-1 Service Subscription Administration – getServiceSubscription capability

##### 6.9.2.6.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the getServiceSubscription capability

Figure 6.9.2.6.3-1 Service Subscription Administration – getServiceSubscription Diagram

##### 6.9.2.6.4	Post-Conditions

Not Applicable

##### 6.9.2.6.5	Exceptions

Not Applicable

##### 6.9.2.6.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.2.6.7	oneM2M Resource Interworking

This service capability retrieves the M2M Service Subscription for the specified filter criteria and maps to the RETRIEVE procedure for the <m2mServiceSubscription> resource.

#### 6.9.2.7	addNodeToServiceSubscription

This service capability provides the ability to add a M2M Node to the existing M2M Service Subscription. This service capability shall be restricted to the Msc and Mca Reference Points.

##### 6.9.2.7.1	Pre-conditions

Not Applicable

##### 6.9.2.7.2	Signature – addNodeToServiceSubscription

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceSubscriptionId | IN | NO | The M2M Service Subscription (M2M-Service-Profile-ID) to add the Device |
| externalIds | IN | YES | A List of URNs that represent the external identifiers associated with this M2M Node. |
| applicationRuleIds | IN | YES | A list of Application Rules associated with this Node. |
| nodeId | OUT | NO | The unique M2M Node identifer for this entity. |
| responseType | OUT | YES | Unique response types for this service. - M2M Service Subscription does not exist. |

Table 6.9.2.7.2-1 Service Subscription Administration – addNodeToServiceSubscription capability

##### 6.9.2.7.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the addNodeToServiceSubscription capability

Figure 6.9.2.7.3-1 Service Subscription Administration – addNodeToServiceSubscription Diagram

##### 6.9.2.7.4	Post-Conditions

Not Applicable

##### 6.9.2.7.5	Exceptions

Not Applicable

##### 6.9.2.7.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.2.7.7	oneM2M Resource Interworking

This service capability adds the M2M Node information for the M2M Service Subscription. This capability maps to the CREATE procedure of the <serviceSubscribedNode> resource.

#### 6.9.2.8	deleteNodesFromServiceSubscription

This service capability provides the ability to delete M2M Nodes from the existing M2M Service Subscription. This service capability shall be restricted to the Msc and Mca Reference Points.

##### 6.9.2.8.1	Pre-conditions

Not Applicable

##### 6.9.2.8.2	Signature – deleteNodesFromServiceSubscription

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceSubscriptionId | IN | NO | The M2M Service Subscription (M2M-Service-Profile-ID) for the Devices |
| nodeIds | IN | NO | List of M2M Node identifiers to be deleted from the M2M Service Subscription. |
| responseType | OUT | YES | Unique response types for this service. - M2M Service Subscription does not exist. - M2M Node does not exist for this M2M Service Subscription |

Table 6.9.2.8.2-1 Service Subscription Administration – deleteNodesFromServiceSubscription capability

##### 6.9.2.8.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the deleteNodesFromServiceSubscription capability

Figure 6.9.2.8.3-1 Service Subscription Administration – deleteNodesFromServiceSubscription Diagram

##### 6.9.2.8.4	Post-Conditions

Not Applicable

##### 6.9.2.8.5	Exceptions

Not Applicable

##### 6.9.2.8.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.2.8.7	oneM2M Resource Interworking

This service capability removes the M2M Node from the M2M Service Subscription. This capability maps to the DELETE procedure of the <serviceSubscribedNode> resource.

#### 6.9.2.9	getNodesForServiceSubscription

This service capability provides the ability to retrieve M2M Nodes for an existing M2M Service Subscription. This service capability shall be restricted to the Msc and Mca Reference Points.

##### 6.9.2.9.1	Pre-conditions

Not Applicable

##### 6.9.2.9.2	Signature – getNodesForServiceSubscription

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| filterCriteria | IN | NO | See Table 6.9.1.2.1-1 |
| nodes | OUT | NO | The resulting M2M Node entities in Table 6.9.1.2-1. |
| responseType | OUT | YES | Unique response types for this service. - None |

Table 6.9.2.9.2-1 Service Subscription Administration – getNodesForServiceSubscription capability

##### 6.9.2.9.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the getNodesForServiceSubscription capability

Figure 6.9.2.9.3-1 Service Subscription Administration – getNodesForServiceSubscription Diagram

##### 6.9.2.9.4	Post-Conditions

Not Applicable

##### 6.9.2.9.5	Exceptions

Not Applicable

##### 6.9.2.9.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.2.9.7	oneM2M Resource Interworking

This service capability retrieves the devices for a M2M Service Subscription for the specified filter criteria and maps to the RETRIEVE procedure for the <serviceSubscribedNode> resource.

#### 6.9.2.10	createApplicationRule

This service capability provides the ability to create a M2M Application Rule to to be associated with one or more M2M Nodes. This service capability shall be restricted to the Msc and Mca Reference Points.

##### 6.9.2.10.1	Pre-conditions

Not Applicable

##### 6.9.2.10.2	Signature – createApplicationRule

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| credentialIds | IN | YES | List of credential identifiers for which this Application Rule is applicable, i.e. for registration requests coming into the M2M System via a Security Association Endpoint (SEA) [i.3], that was authenticated using credentials that match with any of these credential identifiers, the current Application Rule applies. |
| allowedApplicationIds | IN | YES | List of M2M Application Identifiers (App-ID) that shall be considered to be allowed for AE registration requests received via Security Association Endpoint (SEA) [i.4]. |
| allowedAEIds | IN | YES | List of M2M Application Entity Identifiers (AE-ID) that shall be considered to be allowed for AE registration requests. |
| applicationRuleId | OUT | NO | The unique Application Rule identifier in the context of the M2M Service Subscription. |
| responseType | OUT | YES | Unique response types for this service. - None |

Table 6.9.2.10.2-1 Service Subscription Administration – createApplicationRule capability

##### 6.9.2.10.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the createApplicationRule capability

Figure 6.9.2.10.3-1 Service Subscription Administration – createApplicationRule Diagram

##### 6.9.2.10.4	Post-Conditions

Not Applicable

##### 6.9.2.10.5	Exceptions

Not Applicable

##### 6.9.2.10.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.2.10.7	oneM2M Resource Interworking

This service capability adds an Application Rule. This capability maps to the CREATE procedure of the <serviceSubscribedAppRule> resource..

#### 6.9.2.11	deleteApplicationRules

This service capability provides the ability to delete Application Rules and any references to the Application Rule made by M2M Nodes to the Application Rules. This service capability shall be restricted to the Msc and Mca Reference Points.

##### 6.9.2.11.1	Pre-conditions

Not Applicable

##### 6.9.2.11.2	Signature – deleteApplicationRules

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| applicationRuleIds | IN | NO | A list of Application Rule identifiers. |
| responseType | OUT | YES | Unique response types for this service. - Application Rule does not exist |

Table 6.9.2.11.2-1 Service Subscription Administration – deleteApplicationRules capability

##### 6.9.2.11.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the deleteApplicationRules capability

Figure 6.9.2.11.3-1 Service Subscription Administration – deleteApplicationRules Diagram

##### 6.9.2.11.4	Post-Conditions

Not Applicable

##### 6.9.2.11.5	Exceptions

Not Applicable

##### 6.9.2.11.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.2.11.7	oneM2M Resource Interworking

This service capability adds Application Rules. This capability maps to the DELETE procedure of the <serviceSubscribedAppRule> resource.

.

#### 6.9.2.12	getApplicationRules

This service capability provides the ability to retrieve the Application Rules. This service capability shall be restricted to the Msc and Mca Reference Points.

##### 6.9.2.12.1	Pre-conditions

Not Applicable

##### 6.9.2.12.2	Signature – getApplicationRules

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| filterCriteria | IN | NO | See Table 6.9.1.3.1-1 |
| applicationRules | OUT | NO | The resulting M2M Node entities in Table 6.9.1.3-1. |
| responseType | OUT | YES | Unique response types for this service. - None |

Table 6.9.2.12.2-1 Service Subscription Administration – getApplicationRules capability

##### 6.9.2.12.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the getApplicationRules capability

Figure 6.9.2.12.3-1 Service Subscription Administration – getApplicationRules Diagram

##### 6.9.2.12.4	Post-Conditions

Not Applicable

##### 6.9.2.12.5	Exceptions

Not Applicable

##### 6.9.2.12.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.2.12.7	oneM2M Resource Interworking

This service capability retrieves Application Rules. This capability maps to the RETRIEVE procedure of the <serviceSubscribedAppRule> resource..

#### 6.9.2.13	updateNodeForServiceSubscription

This service capability provides the ability to update the Application Rules and External Identifiers to be associated to a Node within the context of a M2M Service Subscription. This service capability shall be restricted to the Msc and Mca Reference Points.

##### 6.9.2.13.1	Pre-conditions

Not Applicable

##### 6.9.2.13.2	updateNodeForServiceSubscription

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceSubscriptionId | IN | NO | The M2M Service Subscription (M2M-Service-Profile-ID) for the device |
| nodeId | IN | NO | The unique M2M Node identifier in the context of the M2M Service Subscription. |
| applicationRuleIds | IN | YES | A list of Application Rule Identifier. When present this list will overlay the list of Applications associated with the Node. |
| externalIds | IN | YES | List of URNs that represent the external identifiers associated with this device. When present this list will overlay the list of external identifiers associated with the Node. |
| responseType | OUT | YES | Unique response types for this service. - M2M Service Subscription does not exist - M2M Node does not exist for the M2M Service Subscription |

Table 6.9.2.13.2-1 Service Subscription Administration – updateNodeForServiceSubscription capability

##### 6.9.2.13.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the updateNodeForServiceSubscription capability

Figure 6.9.2.13.3-1 Service Subscription Administration – updateNodeForServiceSubscription Diagram

##### 6.9.2.13.4	Post-Conditions

When parameters are present, the M2M Applications and External Identifiers are replaced for the M2M Node.

##### 6.9.2.13.5	Exceptions

Not Applicable

##### 6.9.2.13.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.2.13.7	oneM2M Resource Interworking

This service capability updates the M2M Application and/or External Identifier list(s) for the Node in the context of the M2M Service Subscription Profile. This capability maps to the UPDATE procedure of the <serviceSubscribedNode> resource.

#### 6.9.2.14	updateApplicationRule

This service capability provides the ability to update an Application Rule. This service capability shall be restricted to the Msc and Mca Reference Points.

##### 6.9.2.14.1	Pre-conditions

Not Applicable

##### 6.9.2.14.2	Signature – updateApplicationRule

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| applicationRuleId | IN | NO | The unique Application Rule identifier in the context of the M2M Service Subscription. |
| credentialIds | IN | YES | List of credential identifiers for which this Application Rule is applicable, i.e. for registration requests coming into the M2M System via a Security Association Endpoint (SEA) [i.5], that was authenticated using credentials that match with any of these credential identifiers, the current Application Rule applies. When present this list will overlay the list of credential identifiers. |
| allowedApplicationIds | IN | YES | List of M2M Application Identifiers (App-ID) that shall be considered to be allowed for AE registration requests received via Security Association Endpoint (SEA) [i.6]. When present this list will overlay the list of allowed App-IDs. |
| allowedAEIds | IN | YES | List of M2M Application Entity Identifiers (AE-ID) that shall be considered to be allowed for AE registration requests. When present this list will overlay the list of allowed AE-IDs. |
| responseType | OUT | YES | Unique response types for this service. - Application Rule does not exist. |

Table 6.9.2.14.2-1 Service Subscription Administration – updateApplicationRule capability

##### 6.9.2.14.3	Service Interactions

The interactions of service capabilities required for this service capability:

Issue the updateApplicationRule capability

Figure 6.9.2.14.3-1 Service Subscription Administration – updateApplicationRule Diagram

##### 6.9.2.14.4	Post-Conditions

When parameters are present, the Allowed Application, AEs and Credentials are replaced for the Application Rule.

##### 6.9.2.14.5	Exceptions

Not Applicable

##### 6.9.2.14.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.2.14.7	oneM2M Resource Interworking

This service capability updates an Application Rule. This capability maps to the UPDATE procedure of the <serviceSubscribedAppRule> resource.

#### 6.9.2.15	addReachabilitySchedule

This service capability provides the ability to add a reachability schedule to an AE.This service capability shall be restricted to the Msc and Mca Reference Points.

##### 6.9.2.15.1	Pre-conditions

Not Applicable

##### 6.9.2.15.2	Signature –addReachabilitySchedule

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| aeId | IN | NO | The Application Entity Identifier (AE-ID). |
| reachabilitySchedule | IN | YES | The contact reachability schedule information of the AE associated with the device node. The absence of this parameter implies the AE associated with the device node is always contact reachable. Type Schedule, see 6.9.1.4.1. |
| responseType | OUT | YES | Unique response types for this service. - AE successfully added |

Table 6.9.2.15.2-1 Service Subscription Administration –addReachabilitySchedule capability

##### 6.9.2.15.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the addReachabilitySchedule capability

Figure 6.9.2.15.3-1 addReachabilitySchedule Diagram

##### 6.9.2.15.4	Post-Conditions

When parameters are present, the reachability schedule is added for the AE.

##### 6.9.2.15.5	Exceptions

Not Applicable

##### 6.9.2.15.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.2.15.7	oneM2M Resource Interworking

This service capability is used to add AE reachability schedule information. The service capability aligns with the <schedule> resource and maps to the CREATE procedure. However the linkage between the <schedule> resource and the AE does not exist.

#### 6.9.2.16	updateReachabilitySchedule

This service capability provides the ability to update an AE’s reachability schedule information.This service capability shall be restricted to the Msc and Mca Reference Points.

##### 6.9.2.16.1	Pre-conditions

Not Applicable

##### 6.9.2.16.2	Signature –updateReachabilitySchedule

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| aeId | IN | NO | The Application Entity Identifier (AE-ID). |
| reachabilitySchedule | IN | YES | The contact reachability schedule information of the AE associated with the device node. The absence of this parameter implies the AE associated with the device node is always contact reachable. Type Schedule, see 6.9.1.4.1. |
| responseType | OUT | YES | Unique response types for this service. - AE successfully updated |

Table 6.9.2.16.2-1 Service Subscription Administration –updateReachabilitySchedule capability

##### 6.9.2.16.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the updateReachabilitySchedule capability

Figure 6.9.2.16.3-1 updateReachabilitySchedule Diagram

##### 6.9.2.16.4	Post-Conditions

When parameters are present, the reachability schedule is updated for the AE.

##### 6.9.2.16.5	Exceptions

Not Applicable

##### 6.9.2.16.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.2.16.7	oneM2M Resource Interworking

This service capability is used to update AE’s reachability schedule information. The service capability aligns with the <schedule> resource and maps to the UPDATE procedure. The reference between an AE and a <schedule> resource does not exist as an oneM2M Resource attribute.

#### 6.9.2.17	getReachabilitySchedule

This service capability provides the ability to get an AE’s reachability schedule information. This service capability shall be restricted to the Msc and Mca Reference Points.

##### 6.9.2.17.1	Pre-conditions

Not Applicable

##### 6.9.2.17.2	Signature –getReachabilitySchedule

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| aeId | IN | NO | The Application Entity Identifier (AE-ID). |
| reachabilitySchedule | OUT | YES | The contact reachability schedule information of the AE associated with the device node. The absence of this parameter implies the AE associated with the device node is always contact reachable. Type Schedule, see 6.9.1.4.1. |
| responseType | OUT | YES | Unique response types for this service. - not successfully |

Table 6.9.2.17.2-1 Service Subscription Administration –getReachabilitySchedule capability

##### 6.9.2.17.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the getReachabilitySchedule capability

Figure 6.9.2.17.3-1 getReachabilitySchedule Diagram

##### 6.9.2.17.4	Post-Conditions

When parameters are present, the reachability schedule is gotten from the AE.

##### 6.9.2.17.5	Exceptions

Not Applicable

##### 6.9.2.17.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.2.17.7	oneM2M Resource Interworking

This service capability is used to get an AE’s  reachability schedule information. The service capability aligns with the <schedule> resource and maps to the RETRIEVE procedure. The reference between an AE and a <schedule> resource does not exist as an oneM2M Resource attribute.

#### 6.9.2.18	deleteReachabilitySchedule

This service capability provides the ability to delete a reachability schedule for an AE. This service capability shall be restricted to the Msc and Mca Reference Points.

##### 6.9.2.18.1	Pre-conditions

Not Applicable

##### 6.9.2.18.2	Signature –deleteReachabilitySchedule

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| aeId | IN | NO | The Application Entity Identifier (AE-ID). |
| responseType | OUT | YES | Unique response types for this service. - AE successfully deleted |

Table 6.9.2.18.2-1 Service Subscription Administration –deleteReachabilitySchedule capability

##### 6.9.2.18.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the deleteReachabilitySchedule capability

Figure 6.9.2.18.3-1 deleteReachabilitySchedule Diagram

##### 6.9.2.18.4	Post-Conditions

When parameters are present, the reachability schedule is deleted from the AE.

##### 6.9.2.18.5	Exceptions

Not Applicable

##### 6.9.2.18.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.2.18.7	oneM2M Resource Interworking

This service capability is used to delete an AE’s reachability schedule information. The service capability aligns with the <schedule> resource and maps to the DELETE procedure. The reference between an AE and a <schedule> resource does not exist as an oneM2M Resource attribute.

## 6.10	Event Collection

### 6.10.1	Overview

The Event Collection service provides the capability to record events for accounting purposes.

#### 6.10.1.1	Event Collection Types

##### 6.10.1.1.1	Event Collection Record

| Information Element | Optional | Description |
| --- | --- | --- |
| eventRecordID | NO | It is the unique ID that identifies a specific event record. |
| eventCollectionTriggerID | NO | It is the unique ID that identifies a specific event collection triggering scenario, which triggers information recording for a specific event. |
| collectingEntityID | NO | This is the unique ID of the entity that collects the statistics. It can be an AE-ID or M2M Service Capability Identifier (M2M-Serv-Cap-ID) |
| collectedEntityID | NO | This is the unique ID of the entity whose service layer operation statistics are being collected. It can be an AE-ID or M2M Service Capability Identifier (M2M-Serv-Cap-ID). |
| serviceProfileID | YES | An M2M Service Profile Identifier defines M2M Services and/or M2M Service Roles and AEs applicable to M2M Nodes. |
| eventDescription | NO | Describes the event triggered at the service. It is a subset of the eventConfig data structure. The specific structure depends on the specific event defined by the Event Collection Trigger. |
| Vendor Specific Information | YES | Defines Vendor specific information |

Table 6.10.1.1-1 Type: Event Record Template

##### 6.10.1.1.2	Event Configuration Type

| Parameter name | Optional | Description |
| --- | --- | --- |
| eventType | NO | This attribute indicates the type of the event, such as “subscribeComplete”, “downloadFirmware “, “timer based”,, “storage based”, etc. The eventType can be a combination of multiple sub-types, for example, it can be based on both “subscribeComplete” and a timer. |
| serviceRoleIds | YES | Defines the specific service role. |
| eventStart | YES | This attribute indicates the start time of the event. It is mandatory if the eventType includes “timer based”. |
| eventEnd | YES | This attribute indicates the end time of the event. It is mandatory if the eventType includes “timer based”. |
| dataSize | YES | This attribute defines the data size if an event is triggered when the stored data exceeds a certain size. It is mandatory if the eventType is “storage based”. |

Table 6.10.1.1.2-1 Type: Event Collection – eventConfig

##### 6.10.1.1.3 	getEventCollectionPolicy Filter Criteria

| Criterion name | Description |
| --- | --- |
| eventType | See Table 6.10.1.1.2-1 |
| eventStart | See Table 6.10.1.1.2-1. This criterion may only be used when the eventType criterion is set to “timer based”. |
| eventEnd | See Table 6.10.1.1.2-1. This criterion may only be used when the eventType criterion is set to “timer based”. |
| dataSize | See Table 6.10.1.1.2-1. This criterion may only be used when the eventType criterion is set to “storage based”. |

Table 6.10.1.1.3-1 getEventCollectionPolicy Filter Criteria

##### 6.10.1.1.4 	getEventCollectionTriggers Filter Criteria

| Criterion name | Description |
| --- | --- |
| eventCollectionPolicyID | See Table 6.10.2.3.2-1. |
| collectingEntityID | See Table 6.10.2.3.2-1. |
| collectedEntityID | See Table 6.10.2.3.2-1. |
| eventReceiverID | See Table 6.10.2.3.2-1. |
| serviceID | See Table 6.10.2.3.2-1. |
| triggerStatus | See Table 6.10.2.3.2-1. |

Table 6.10.1.1.4-1 getEventCollectionTriggers Filter Criteria

##### 6.10.1.1.5 	getEventRecords Filter Criteria

| Criterion name | Description |
| --- | --- |
| eventCollectionTriggerID | See Table 6.10.1.1-1. |
| collectingEntityID | See Table 6.10.1.1-1. |
| collectedEntityID | See Table 6.10.1.1-1. |
| serviceProfileID | See Table 6.10.1.1-1. |
| eventDescription | See Table 6.10.1.1-1. In the filterCriteria, the eventDescription can be a string that partially matches the eventDescription parameter in the record. |

Table 6.10.1.1.5-1 getEventRecords Filter Criteria

### 6.10.2	Service Capabilities

#### 6.10.2.1	setEventCollectionPolicy

This service capability provides the ability for AEs to configure event collection policies for statistics and charging purposes. This service capability applies to the Mca Reference Point.

##### 6.10.2.1.1	Pre-conditions

The Originator can be an AE, and the Originator wants to configure event collection policies at a receiving service capability. The receiving service capability conducts event collection according to the policies. The receiving service capability may establish its own event collection policies.

The Originator, if different from the receiving service capability, is subscribed and registered to the receiving service capability.

##### 6.10.2.1.2	Signature – setEventCollectionPolicy

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceId | IN | NO | The M2M Service Identifier (M2M-Serv-ID) |
| eventConfig | IN | NO | The configuration of the event collection policy. Defined in Table 6.10.1.1.2-1. |
| eventCollectionPolicyID | OUT | NO | The receiving service capability generates a unique policy ID for each policy. |
| responseType | OUT | YES | Unique response types for this service. Exception: - Policy already exists |

Table 6.10.2.1.2-1 Event Collection – setEventCollectionPolicy capability

##### 6.10.2.1.3	Post-Conditions

Not Applicable

##### 6.10.2.1.4	Exceptions

The Originator has no access right to create the policy

##### 6.10.2.1.5	Policies for Use

Message Exchange Patterns: In-Out

##### 6.10.2.1.6	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.10.2.1.6-1 setEventCollectionPolicy Diagram

##### 6.10.2.1.7	oneM2M Resource Interworking

The service capability aligns with the <eventConfig> resource and maps to the CREATE procedure for the resource.

#### 6.10.2.2	getEventCollectionPolicy

This service capability provides the ability for entities, such as an AE or a service capability, to retrieve the existing policies stored at a service capability. This service capability applies to the Mca and Msc Reference Point.

##### 6.10.2.2.1	Pre-conditions

The Originator can be an AE or a service capability, and the Originator wants to retrieve event collection policies at a receiving service capability.

The Originator, if different from the receiving service capability, is subscribed and registered to the receiving service capability. The Originator is only allowed to retrieve with the proper access right.

##### 6.10.2.2.2	Signature – getEventCollectionPolicy

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| eventCollectionPolicyID | IN | NO | See Table 6.10.2.1.2-2. The eventCollectionPolicyID specifies the specific event collection policy to retrieve, if the Originator has such information. If the Originator does not have the information regarding eventCollectionPolicyID, it indicates eventCollectionPolicyID with a specific value that represents “any IDs”, and uses the filterCriteria to filter selected policies. |
| filterCriteria | IN | YES | See Table 6.10.1.1.3-1. |
| responseType | OUT | YES | Unique response types for this service. Exception: - eventCollectionPolicyID does not exist |

Table 6.10.2.2.2-1 Event Collection – getEventCollectionPolicy capability

##### 6.10.2.2.3	Post-Conditions

Not Applicable

##### 6.10.2.2.4	Exceptions

The Originator has no access right to retrieve the policy

##### 6.10.2.2.5	Policies for Use

Message Exchange Patterns: In-Out

##### 6.10.2.2.6	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.10.2.2.6-1 getEventCollectionPolicy Diagram

##### 6.10.2.2.7	oneM2M Resource Interworking

The service capability aligns with the <eventConfig> resource and maps to the RETRIEVE procedure for the resource.

#### 6.10.2.3	setEventCollectionTriggers

This service capability provides the ability for AEs and service capabilities to configure specific triggers for event collection, based on the Event Collection Policy. This service capability applies to the Mca and Msc Reference Point.

##### 6.10.2.3.1	Pre-conditions

The Originator can be an AE or a service capability, and the Originator wants to configure event collection triggers based on existing event collection policy available at the collecting service capability.

##### 6.10.2.3.2	Signature – setEventCollectionTriggers

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| eventCollectionPolicyID | IN | NO | This attribute specify the policy used to define the specific event collection trigger. |
| collectingEntityID | IN | NO | This is the ID of the entity where the event is being collected. If not specified, the default collecting entity is the receiving service capability. |
| collectedEntityID | IN | NO | This is the ID of the entity that triggered the event collection. If not specified, the default is all entities triggered the event at the collecting entity. |
| eventReceiverID | IN | YES | In cases when there is another entity, other than the collecting entity (such as a service capability), wants to receive the collected event, this attribute provides the unique ID of this entity. |
| serviceRoleIds | IN | YES | Specifies the service roles that trigger the event collection. |
| serviceId | IN | YES | Specifies the services that trigger the event collection. |
| triggerStatus | IN | YES | This attribute indicates if the trigger is activated or not. If not defined, the trigger is activated. |
| eventCollectionTriggerID | OUT | NO | The receiving service capability generates a unique ID for each event collection trigger. |
| responseType | OUT | YES | Unique response types for this service. Exception: - eventCollectionPolicyID is invalid - collectingEntityID is invalid - collectedEntityID is invalid - eventReceiverID is invalid - triggerStatus is invalid |

Table 6.10.2.3.2-1 Event Collection – setEventCollectionTriggers capability

##### 6.10.2.3.3	Post-Conditions

After the successful creation of event collection triggers, when the defined event happens at the collectingEntity, and when the event collection trigger status is ACTIVE, the collectingEntity shall collect the event. The supporting service shall send a recordEvent message to the Event Collection entity.

##### 6.10.2.3.4	Exceptions

The Originator has no access right to create the event collection triggers

##### 6.10.2.3.5	Policies for Use

Message Exchange Patterns: In-Out

##### 6.10.2.3.6	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.10.2.3.6-1 setEventCollectionTriggers Diagram

##### 6.10.2.3.7	oneM2M Resource Interworking

The service capability aligns with the <statsCollect> resource and maps to the CREATE procedure for the resource.

#### 6.10.2.4	getEventCollectionTriggers

This service capability provides the ability for AEs and service capabilities to retrieve event collection triggers at a receiving service capability. This service capability applies to the Mca and Msc Reference Point.

##### 6.10.2.4.1	Pre-conditions

Originating AEs and service capabilities are subscribed and registered to the receiving service capability. Originator has the access right to retrieve.

##### 6.10.2.4.2	Signature – getEventCollectionTriggers

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| eventCollectionTriggerID | IN | YES | This attribute specifies which event collection triggers to retrieve. If not specified, any event collection trigger matching the filter criteria should be retrieved. If the Originator does not have the information regarding eventCollectionTriggerID, it indicates with a specific value that represents “any IDs”, and uses the filterCriteria to filter selected event triggers. |
| filterCriteria | IN | YES | See Table 6.10.1.1.4-1. |
| responseType | OUT | YES | Unique response types for this service. Exception: - eventCollectionTriggerID does not exist |

Table 6.10.2.4.2-1 Event Collection – getEventCollectionTriggers capability

##### 6.10.2.4.3	Post-Conditions

Not Applicable

##### 6.10.2.4.4	Exceptions

The Originator has no access right to retrieve the event collection triggers

##### 6.10.2.4.5	Policies for Use

Message Exchange Patterns: In-Out

##### 6.10.2.4.6	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.10.2.4.6-1 getEventCollectionTriggers Diagram

##### 6.10.2.4.7	oneM2M Resource Interworking

The service capability aligns with the <statsCollect> resource and maps to the RETRIEVE procedure for the resource.

#### 6.10.2.5	recordEvent

This service capability provides the ability for a service (such as Data Exchange service) to trigger the event collection service to record an event. This service applies to the Msc Reference Point.

##### 6.10.2.5.1	Pre-conditions

Event Collection Triggers have been created by the setEventCollectionTriggers capability.

##### 6.10.2.5.2	Signature – recordEvent

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| eventCollectionPolicyID | IN | YES | This attribute specify the policy used to define the specific event collection trigger. |
| eventCollectionTriggerID | IN | NO | The service where the trigger happens provides the eventCollectionTriggerID being used. |
| eventDescription | IN | NO | This attribute describes the event triggered at the service. It is a subset of the eventConfig data structure. It contains one or more parameters defined in the eventConfig data type. |
| recordID | OUT | NO | The event collection service generates a unique ID. |
| responseType | OUT | YES | Unique response types for this service. Exception:. - Event recording cannot be completed |

Table 6.10.2.5.2-1 Event Collection – recordEvent capability

##### 6.10.2.5.3	Post-Conditions

Not Applicable

##### 6.10.2.5.4	Exceptions

Not Applicable

##### 6.10.2.5.5	Policies for Use

Message Exchange Patterns: In-Out

##### 6.10.2.5.6	Service Interactions

The request comes from the service where the event was triggered to the Event Collection entity.

Figure 6.10.2.5.6-1 recordEvent Diagram

##### 6.10.2.5.7	oneM2M Resource Interworking

Not Applicable

#### 6.10.2.6	getEventRecords

This service capability provides the ability for AEs and service capabilities to retrieve the recorded events for statistical or charging purposes. This service capability applies to the Mca and Msc Reference Point.

##### 6.10.2.6.1	Pre-conditions

Originating AEs and service capabilities are subscribed and registered to the receiving service capability.

##### 6.10.2.6.2	Signature – getEventRecords

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| eventRecordID | IN | NO | This attribute specifies which record to retrieve. If not specified, any event records matching the filter criteria should be retrieved. If the Originator does not have the information regarding the eventRecordID, it indicates with a specific value that represents “any IDs”, and use the filterCriteria to filter selected records of events. |
| filterCriteria | IN | YES | See Table 6.10.1.1.5-1 |
| responseType | OUT | YES | Unique response types for this service. Exception: - The eventRecordID |

Table 6.10.2.6.2-1 Event Collection – getRecords capability

##### 6.10.2.6.3	Post-Conditions

Not Applicable

##### 6.10.2.6.4	Exceptions

Not Applicable

##### 6.10.2.6.5	Policies for Use

Message Exchange Patterns: In-Out

##### 6.10.2.6.6	Service Interactions

The Originator sends the request to the Event Collection entity to obtain the event records that it is

interested in.

Figure 6.10.2.6.6-1 getEventRecords Diagram

##### 6.10.2.6.7	oneM2M Resource Interworking

Not Applicable

## 6.11	Registration

### 6.11.1	Overview

The Registration service provides the ability for:

- Initial Registration of an AE

- Refresh an existing Registration of an AE

- AE initiated termination of an existing Registration

- Removal of subscriptions to AE Registration events

### 6.11.2	Service Capabilities

#### 6.11.2.1	registerAE

This service capability enables an AE with the proper authorization to register with the M2M System. This service capability shall be restricted across the Mca Reference Point.

##### 6.11.2.1.1	Pre-conditions

The AE has not registered with the M2M System. An AE identifier may have been already allocated to the registering AE and acquired by the AE by means outside the scope of this specification.

##### 6.11.2.1.2	Signature – registerAE

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| pointOf Access | IN | NO | The point of Access of the registered AE. |
| applicationId | IN | NO | The Application Identifier (App-ID). |
| credentialId | IN | YES | Application Credential-ID used to identify the Application Entity for the Application. It is used to retrieve the corresponding M2M Service Subscription. |
| expirationTime | IN | NO | The expiration time of the registration as requested by the Originator. |
| reachabilitySchedule | IN | YES | The contact reachability schedule information of the AE associated with the device node. The absence of this parameter implies the AE associated with the device node is always contact reachable. Type Schedule, see 6.9.1.4.1. |
| aeId | IN-OUT | Optional for IN only-Not Optional for OUT | Application Entity (AE-ID) is provided back in the response. |
| responseType | OUT | NO | Unique response types for this service: - AE successfully registered - AE not successfully registered - M2M Service Subscription does not exist - Invalid Application Credential Identifier. |

Table 6.11.2.1.2-1 Registration – registerAE capability

##### 6.11.2.1.3	Services Interaction

The interactions of service capabilities required for this service capability:

- Associate the AE with the M2M Service Subscription

- Issue the request to the Supporting Service.

- Send a notification to the Device Onboarding Supporting Service for a first-time application registration  in the event there is a subscription of the first contact event by the Supporting Service.

Figure 6.11.2.1.3-1 Registration – registerAE Diagram

##### 6.11.2.1.4	Post-Conditions

AE is registered and can start using service capabilities

The returned AE-ID shall be used in any subsequent operation related to that application

##### 6.11.2.1.5	Exceptions

Not Applicable

##### 6.11.2.1.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.11.2.1.7	oneM2M Resource Interworking

This service capability maps to the CREATE procedure of the <AE> resource.

#### 6.11.2.2	refreshAERegistration

This service capability enables an AE to refresh an existing AE registration with the M2M System. This service capability shall be restricted to the Mca Reference Points.

##### 6.11.2.2.1	Pre-conditions

The AE has successfully registered with the M2M System.

##### 6.11.2.2.2	Signature – refreshAERegistration

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| aeId | IN | NO | The Application Entity Identifier (AE-ID) of the request originator. |
| pointOfAccess | IN | YES | The point of Access of the registered AE. POA is optional only if identical to the one in the refreshed registration |
| expirationTime | IN | NO | The expiration time of the registration as requested by the Originator. |
| reachabilitySchedule | IN | YES | The contact reachability schedule information of the AE associated with the device node. The absence of this parameter implies the AE associated with the device node is always contact reachable. Type Schedule, see 6.9.1.4.1. |
| responseType | OUT | NO | Unique response types for this service: - AE successfully refreshed - Registration does not exist |

Table 6.11.2.2.2-1 Registration – refreshAERegistration capability

##### 6.11.2.2.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue a request to refresh the AE association with the M2M Service Profile

- Issue the request to the Supporting Service

Figure 6.11.2.2.3-1 Registration – refreshAERegistration Diagram

##### 6.11.2.2.4	Post-Conditions

The Application Entity registration is refereshed and can continue using service capabilities.

##### 6.11.2.2.5	Exceptions

Not Applicable

##### 6.11.2.2.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.11.2.2.7	oneM2M Resource Interworking

This service capability maps to the UPDATE procedure of the <AE> resource.

#### 6.11.2.3	deregisterAE

This service capability enables an AE to deregister from the M2M System. This service capability shall be restricted to the Mca Reference Points.

##### 6.11.2.3.1	Pre-conditions

The AE successfully registered with the M2M System.

##### 6.11.2.3.2	Signature – deregisterAE

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| aeId | IN | NO | The Application Identifier (AE-ID) |
| responseType | OUT | NO | Unique response types for this service: - AE successfully De-registered - Registration does not exist |

Table 6.11.2.3.2-1 Registration – deregisterAE capability

##### 6.11.2.3.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Disassociate the AE with Service Subscription

- Issue the request to Supporting Services.

- Send a notification  to device onboarding service supporting service for application de-registration in the event there is a subscription

Figure 6.11.2.3.3-1 Registration – deregisterAE Diagram

##### 6.11.2.3.4	Post-Conditions

AE is no longer registered and cannot use any services.

##### 6.11.2.3.5	Exceptions

Not Applicable

Transaction Pattern: Participation allowed

##### 6.11.2.3.6	oneM2M Resource Interworking

This service capability maps to the DELETE procedure of the <AE> resource.

## 6.12	Registration Administration

### 6.12.1	Overview

The Registration administration service provides the ability for:

- Retrieve the AE’s Registration status

- Revoke an AE existing Registration

- Permit subscriptions to AE Registration events

### 6.12.2	Service Capabilities

#### 6.12.2.1	getRegistrationStatus

This service capability enables a Supporting Service authorized for this operation, to retrieve the registration status for an AE. This service capability is restricted to the Msc Reference Point.

##### 6.12.2.1.1	Pre-conditions

Not Applicable

##### 6.12.2.1.2	Signature – getRegistrationStatus

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| aeId | IN | NO | The Application Entity Identifier (AE-ID) |
| responseType | OUT | NO | Unique response types for this service: - AE is registered - AE registration does not exist |

Table 6.12.2.1.2-1 Registration Administration – getRegistrationStatus capability

##### 6.12.2.1.3	Service Interaction

The interactions of service capabilities required for this service capability:

- Retrieve the Application Entity information

Figure 6.12.2.1.3-1 Registration Administration – getRegistrationStatus Diagram

##### 6.12.2.1.4	Post-Conditions

Not Applicable

##### 6.12.2.1.5	Exceptions

Not Applicable

##### 6.12.2.1.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.12.2.1.7	oneM2M Resource Interworking

This service capability maps to the RETRIEVE procedure of the <AE> resource.

#### 6.12.2.2	revokeAERegistration

This service capability enables a Supporting Service authorized for this operation to revoke an existing AE registration. This service capability shall be restricted to the Msc Reference Points

##### 6.12.2.2.1	Pre-conditions

The AE is successfully registered with the M2M System.

##### 6.12.2.2.2	Signature – revokeAERegistration

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| aeId | IN | NO | Application Entity Identifier (AE-ID) |
| responseType | OUT | NO | Unique response types for this service: - AE successfully De-registered - Registration does not exist |

Table 6.12.2.2.2-1 Registration Administration – revokeAERegistration capability

##### 6.12.2.2.3	Service Interaction

The interactions of service capabilities required for this service capability:

- Issue a request to the Service Subscription Administration to fetch the service profile.

- Disassociate the AE from the M2M Service Subscription

Figure 6.12.2.2.3-1 Registration Administration – revokeAERegistration Diagram

##### 6.12.2.2.4	Post-Conditions

The Application Entity is no longer registered and cannot use any services

##### 6.12.2.2.5	Exceptions

Not Applicable

##### 6.12.2.2.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.12.2.2.7	oneM2M Resource Interworking

This service capability maps to the DELETE procedure of the <AE> resource.

#### 6.12.2.3	subscribeInitialAERegistrationEvent

This service capability enables subscription to the initial (first time only) registration of an AE. This capability shall be restricted to the Msc Reference Point.

##### 6.12.2.3.1	Pre-conditions

Not Applicable

##### 6.12.2.3.2	Signature – subscribeInitialAERegistrationEvent

| Parameter name | Direction |  | Optional | Description |
| --- | --- | --- | --- | --- |
| subscriptionId |  | OUT | NO | Return subscriptionId in case of a success. |
| responseType |  | OUT | NO | Unique response types for this service: - Successfully subscribed - Not successfully subscribed |

6.12.2.3.2-1 Registration Administration – subscribeInitialAERegistrationEvent capability

##### 6.12.2.3.3	Service Interaction

The interactions of service capabilities required for this service capability:

- Subscribe the requesting AE for the initial registration event.

Figure 6.12.2.3.3-1 Registration Administration– subscribeInitialAERegistrationEvent Diagram

##### 6.12.2.3.4	Post-Conditions

A Subscription is created for the Originator to receive initial contact events.

##### 6.12.2.3.5	Exceptions

Not Applicable

##### 6.12.2.3.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.12.2.3.7	oneM2M Resource Interworking

Not Applicable

#### 6.12.2.4	unsubscribeInitialAERegistrationEvent

This service capability enables a Supoorting Service for this operation to remove an existing subscription against an AE initial registration. This capability shall be restricted to the Msc Reference Point.

##### 6.12.2.4.1	Pre-conditions

Not Applicable

##### 6.12.2.4.2	Signature – unsubcribeInitialAERegistrationEvent

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| subscriptionId | IN | NO | The subscriptionId for the AE initial registration event. |
| responseType | OUT | NO | Unique response types for this service: - Successfully unsubscribed - Subscription does not exist |

Table 6.12.2.4.2-1 unsubscribeInitialAERegistrationEvent capability

##### 6.12.2.4.3	Service Interaction

The interactions of service capabilities required for this service capability:

- Remove the subscription

Figure 6.12.2.4.3-1 Registration Administration – unsubscribeInitialAERegistrationEvent Diagram

##### 6.12.2.4.4	Post-Conditions

Subscription to initial registration event is removed.

##### 6.12.2.4.5	Exceptions

Not Applicable

##### 6.12.2.4.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.12.2.4.7	oneM2M Resource Interworking

Not Applicable

# 7	M2M Service Components

This clause describes the M2M Service Components provided by the M2M Services Platform.

## 7.1	Introduction

An M2M Service Component defines the set of M2M Services that are related in the sense that the M2M Services that comprise the M2M Service Component are typically deployed together in order to support one or more Supporting Services.

The M2M Service Architecture augments the oneM2M Functional Architecture by specifying M2M Services provided to M2M Service Providers. These M2M Services are exposed to the AE across the Mca Reference Point as well as other Service Components across the Msc Reference Point.

## 7.1.1	Service Component Interaction Cross Reference

Table 7.1-1 Service Component Cross Reference

## 7.2	Infrastructure Component (INF)

The INF Component provides services that are exposed to other M2M Service Components across the Msc Reference Point. The Services within the INF Component are common and utility type Services needed to enable other M2M Services.

## 7.2.1	INF to Service Cross Reference

Authorization: All Service Capabilities

Registration Services: All Service Capabilities

## 7.3	Service Subscription Component (SSUB)

The SSUB Component provides services that are necessary to maintain the M2M Service, the associations (e.g., Services Roles) to the M2M Service, M2M Service Subscriptions, the associations (e.g., AE, M2M Nodes) to the M2M Service Subscription and any external identifiers associated with AE and M2M Nodes. In addition the services used to manage devices and AEs are part of this component.

## 7.3.1	SSUB to Service Cross Reference

Service Subscription: All Service Capabilities

Service Administration: All Service Capabilities

Service Subscription Administration: All Service Capabilities

## 7.4	Transport Adapter (TRA)

The TRA component provides services are used to integrate or adapt Data Exchange requests to the underlying Data Exchange transport (e.g., XMPP or MQTT Broker).

## 7.4.1	TRA to Service Cross Reference

Broker: All Service Capabilities

## 7.5	Accounting (ACC)

The ACC Component provides services that are record events necessary to charge for the Services provided to AEs.

## 7.5.1	Accounting to Service Cross Reference

Event Collection: All Service Capabilities

## 7.6	Service Exposure (SE)

The SE Component acts as the primary interface for AEs to interact with the M2M Service Layer across the Mca Reference Point.

## 7.6.1	Service Exposure to Service Cross Reference

Data Exchange: subscribe, publish, sendMessage

Service Subscription Administration: getServiceSubscription, addNodeToServiceSubscription, deleteNodesFromServiceSubscription, getNodesForServiceSubscription, updateNodeForServiceSubscription, addApplicationRule, deleteApplicationRules, getApplicationRules, updateApplicationRule, addReachabilitySchedule, updateReachabilitySchedule, getReachabilitySchedule, deleteReachabilitySchedule

Device Management:  downloadFirmware, installFirmware, getFirmwareInformation, getFirmwareExecStatus, upgradeFirmware, getDeviceInformation, getDeviceCapabilities, enableDeviceCapability, disableDeviceCapability, getAreaNetworks, updateDeviceForAreaNetwork, rebootDevice, resetDevice, uploadDeviceLog, getDeviceLogs, getDeviceLogInformation, getSoftwareInformation, downloadSoftware, installSoftware, activateSoftware, deactivateSoftware, removeSoftware

Event Collecton: setEventCollectionPolicy, getEventCollectionPolicy, setEventCollectionTriggers, getEventCollectionTriggers, getEventRecords

Registration Services: registerAE, refreshAERegistration, deregisterAE

## 7.7	Management Adapter (MA)

The Management Adapter component provides services that are used to integrate or adapt Device Management requests to the Management Servers of existing device management technologies(e.g., TR-069, OMA-DM, and LWM2M) .

## 7.7.1	Management Adapter to Service Cross Reference

Management Adapter: All Service Capabilities

Device Management: deviceManagementReport

## 7.8	Device Management (DM)

The Device Management component provides services to manage devices without consideration of the technology of Underlying Network.

### 7.8.1	Device Management to Service Cross Reference

Device Management: All Service Capabilities

The following text is to be used when appropriate:

# Proforma copyright release text block

This text box shall immediately follow after the heading of an element (i.e. clause or annex) containing a proforma or template which is intended to be copied by the user. Such an element shall always start on a new page.

Notwithstanding the provisions of the copyright clause related to the text of the present document, OneM2M grants that users of the present document may freely reproduce the <proformatype> proforma in this {clause|annex} so that it can be used for its intended purposes and may further publish the completed <proformatype>.

# Annex A: Common Request Processing

## A.1	Overview

This annex illustrates the usage of the common services described in this technical specification for implementation requests received from AEs across the Mca Reference Point.

## A.2	Mca Common Request Processing

AEs communicate with the M2M System over the Mca Reference Point through the SE component. The SE component accepts the request from the AE and provides the request to the Supporting Service in order allow the M2M Service Provider to ensure the request is properly authorized and recorded. In addition, the M2M Service Provider is able to orchestrate other capabilities that have not been specified.

### A.2.1	Mca Common Request Parameters (Normative)

Each request that is received by the SE component across the Mca Reference Point shall have a common set of input parameters as described in Table A.2.1-1 Mca: Common Request Input Parameters.

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| from | IN | NO | The identifier of the Originator of the request (AE-ID) |
| requestId | IN-OUT | YES | The request identifier. When supplied by the Originator, the same value is returned in the response if the response is applicable for the message exchange pattern. If not supplied by the Originator, the M2M System supplied a globally unique value in the response if the response is applicable for the message exchange pattern. |
| operation | IN | NO | The signature of the operation that is received or transmitted across the Mca reference point. |

Table A.2.1-1 Mca: Common Request Input Parameters

Service capabilities in the SE component whose message exchange patterns require a response shall have a common set of output parameters as described in Table A.2.1-2 Mca: Common Request Output Parameters.

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| responseType | OUT | YES | Response types that are relevant to requests from AEs across the Mca Reference Point. |

Table A.2.1-2 Mca: Common Request Output Parameters

In addition to the parameters in Table A.2.1.-1, each request that addresses a target AE (e.g., Data Exchange) t hat is received by the SE component across the Mca Reference Point shall have a common set of input parameters as described in Table A.2.1-3 Mca: Common Request Input Parameters For Target AEs.

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| to | IN | NO | The identifier of the AE that is the target of the request (AE-ID) |

Table A.2.1-3 Mca: Common Request Input Parameters For Target AEs

### A.2.2	Authentication and Authorization of Requests

Upon reception of requests across the Mca reference, the request Originator is authenticated and the requested M2M Service Capability is authorized when the:

- Request Originator has a M2M Service Subscription for the requested operation.

- Request Originator is authorized for the requested operation.

#### A.2.2.1	Pre-conditions

The external identifiers for the from parameter’s AE-ID are assigned to a M2M Service Subscription (M2M-Service-Profile-ID).

A correlation between the service capability and the M2M authorization event has been defined in order to authorize the service capability.

#### A.2.2.2	Common M2M Service Capability Parameters for Request Authentication and Authorization

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Mca common request input parameters | IN | NO | The Mca common request input parameters defined in Table A.2.1-1. |
| responseType | OUT | YES | Response types that are relevant to the Authentication and Authorizations of requests from AEs across the Mca Reference Point. - Originator is not authenticated - Originator does not have a M2M Service Subscription - Originator not authorized for the M2M Service Capability |

Table A.2.2.2-1 Common M2M Service Capability Parameters for Request Authentication and Authorization

#### A.2.2.3	Service Interactions

The interactions of service capabilities required to authenticate and authorize a request:

- Obtain the M2M Service Capability from the requested operation

- Obtain the Application Entity and associated M2M Service Subscription

- Validate the M2M Service Subscription for the Service Capability.

- Authorize the use of the M2M Service Capability

Figure A.2.2.3-1 Mca Common Request Authorization Diagram

#### A.2.2.4	Post-Conditions

Success case: The request is permitted.

Failure case: The request is not permitted and an optional response type is transmitted back the request Originator.

#### A.2.2.5	Exceptions

Not Applicable

#### A.2.2.6	Policies for Use

Message Exchange Patterns: In-Out, In-Only, Robust In-Only, In-Optional-Out

Transaction Pattern: Participation allowed

## A.3	Msc Common Request Processing

Service Components communicate with each other across the Msc Reference Point. Additional input or output parameters are not necessary when invoking service capabilities across the Msc Reference Point. As such signature of the service capability is sufficient for implementing the service capability in clause 6 when the capability is invoked across the Msc Reference Point.

### A.3.1	Msc Common Request Data Types (Normative)

Each request that is received by the M2M Component across the Msc Reference Point shall have a common set of input parameters as described in Table A.3.1-1 Msc: Common Request Input Parameters.

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| from | IN | NO | The identifier of the Originator of the request. This is typically the Service Capability identifier of the M2M System. |
| requestId | IN-OUT | NO | The request identifier. When supplied by the Originator, the same value is returned in the response if the response is applicable for the message exchange pattern. If not supplied by the Originator, the M2M System supplied a globally unique value in the response if the response is applicable for the message exchange pattern. |

Table A.3.1-1 Msc: Common Request Input Parameters

## A.4	Common Entity Attributes

Information Entities (e.g., M2M Service Subscription) that are maintained using the M2M Service Capabilities have a set of common attributes that are associated with the entity when the information entity is created, updated and retrieved.

### A.4.1	System Generated Attributes (Normative)

When an information entity is created or updated the system shall update the common attributes as described in Table A.4.1-1.  When an information entity is retrieved the system shall return the common attributes as described in Table A.4.1-1.

| Parameter name | Description |
| --- | --- |
| creationTime | The date and time that the system created the information entity. |
| lastModifiedTime | The date and time that the system modified the information entity. The system modifies this attribute when the information entity is created or updated. |

Table A.4.1-1 System Generated Attributes

### A.4.2	User Defined Discovery Attributes (Normative)

Information entities are retrieved through the system’s service capabilities.  Some service capabilities allow for discovery of information elements using user defined discovery attributes.

| Parameter name | Description |
| --- | --- |
| labels | The list of labels that a user can add or modify in creation and update requests for information entities. When a service capability is used to retrieve a list of information elements, the service capability shall have this attribute as a common request input attribute. |

Table A.4.2-1 User Defined Discovery Attributes

## A.5	Using Filter Criteria

Service Capabilities have the ability to define a filter criteria entity as part of the Service Capability’s input parameters.  Filter criteria within the system shall be evaluated using the following rules:

different elements of the filter criteria entity shall use the “AND” logical operation

same elements of the filter criteria entity shall use the “OR” logical operation

# Annex B (Informative): Data Exchange Services

## B.1	Overview

This annex illustrates the usage of the services for requests to exchange data from AEs across the Mca Reference Point.

### B.2	Supporting Services

### B.2.1	Subscribe-Publish-Notify Message Exchange

This service provides the capability to exchange data using the publish and subscribe message patterns where AEs:

Request to receive payloads from a publication resource by subscribing to the publication resource.

Request to publish a payload to a resource.

#### B.2.1.1	Service Capabilities

##### B.2.1.1.1	subscribeRequest

This service capability provides the ability to validate a subscription request from an AE.

###### B.2.1.1.1.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a M2M Service Subscription, subscribing AE, publication resource and Broker exist.

###### B.2.1.1.1.2	Signature – subscribeRequest

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input parameters | - | - | See Table A.2.1-3 |
| publicationResource | IN | NO | The publication resource. See 6.4.1.1.1.1 |
| deliveryPolicy | IN | YES | The delivery policy when notifying the subscriber. See 6.4.1.1.1.2 |
| retainmentPolicy | IN | YES | The retainment policy for unconnected subscribers. See 6.4.1.1.1.3 |
| transportAdapter | OUT | YES | The instance of the transport adapter service (broker) |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Originator does not have a Broker for the requested Resource - Delivery policy not supported by the transport adapter - Retainment policy not supported by the transport adapter |

Table B.2.1.1.1.2-1 Data Exchange Service – subscribeRequest capability

###### B.2.1.1.1.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Determine the Broker to be used for the originating AE and publication resource

- Perform the Common Request Services for requests across the Mca Reference Point

- Validate the delivery and retainment policies

Figure B.2.1.1.1.3-1 subscribeRequest and subscribeComplete Diagram

###### B.2.1.1.1.4	Post-Conditions

Success case: The request is permitted.

Failure case: The request is not permitted and a response type is transmitted back to the AE.

###### B.2.1.1.1.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### B.2.1.1.1.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 250ms

##### B.2.1.1.2	subscribeComplete

This service capability provides the ability to complete the actions required once the subscription request has been completed in the Broker.

###### B.2.1.1.2.1	Pre-conditions

Not Applicable

###### B.2.1.1.2.2	Signature – subscribeComplete

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input parameters | - | - | See Table A.2.1-3 |
| publicationResource | IN | NO | The publication resource. See 6.4.1.1.1.1 |
| deliveryPolicy | IN | YES | The delivery policy when notifying the subscriber. See 6.4.1.1.1.2 |
| retainmentPolicy | IN | YES | The retainment policy for unconnected subscribers. See 6.4.1.1.1.3 |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Exception: Request may not have been completed |

Table B.2.1.1.2.2-1 Data Exchange Service – subscribeComplete capability

###### B.2.1.1.2.3	Service Interactions

The interactions of service capabilities required for this service capability as shown in Figure B.2.1.1.1.3-1:

- Upon a successful subscribe by the Broker, record the event for accounting purposes

###### B.2.1.1.2.4	Post-Conditions

Success case: The request is permitted.

Failure case: The request is not permitted and a response type is transmitted back to the AE.

###### B.2.1.1.2.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### B.2.1.1.2.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 50ms

##### B.2.1.1.3	publishRequest

This service capability provides the ability to validate a publication request from an AE..

As part of the publication, the publishing AE can provide a delivery policy to enhance the robustness of the publication to AEs that have subscribed to the Resource identified in the request.

###### B.2.1.1.3.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a M2M Service Subscription, publishing AE, resource to publish the payload and Broker exist.

###### B.2.1.1.3.2	Signature – publishRequest

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input parameters | - | - | See Table A.2.1-3 |
| toResource | IN | NO | The leaf node of a publication resource. See 6.4.1.1.1.1 |
| deliveryPolicy | IN | YES | The delivery policy when notifying the subscriber. See 6.4.1.1.1.2 |
| transportAdapter | OUT | YES | The instance of the transport adapter service (broker) |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Originator does not have a Broker for the requested resource - Originator has requested an invalid deliveryPolicy - Originator has requested an invalid retainmentPolicy - Delivery policy not supported by the transport adapter - Exception: Request may not have been completed |

Table B.2.1.1.3.2-1 Data Exchange Service – publishRequest capability

###### B.2.1.1.3.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Determine the Broker to be used for the originating AE and resource

- Perform the Common Request Services for requests across the Mca Reference Point

- Validate the delivery policy for the Broker

- Upon a successful publication to the Broker, record the event for accounting purposes

Figure B.2.1.1.3.3-1 publishRequest and publishComplete Diagram

###### B.2.1.1.3.4	Post-Conditions

Success case: The request is permitted.

Failure case: The request is not permitted and a response type is transmitted back to the AE.

###### B.2.1.1.3.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### B.2.1.1.3.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### B.2.1.1.4	publishComplete

This service capability provides the ability to complete the actions required once the publication request has been completed in the Broker.

###### B.2.1.1.4.1	Pre-conditions

Not Applicable

###### B.2.1.1.4.2	Signature – publishComplete

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input parameters | - | - | See Table A.2.1-3 |
| toResource | IN | NO | The leaf node of a publication resource. See 6.4.1.1.1.1 |
| deliveryPolicy | IN | YES | The delivery policy when notifying the subscriber. See 6.4.1.1.1.2 |
| retainmentPolicy | IN | YES | The retainment policy for unconnected subscribers. See 6.4.1.1.1.3 |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Exception: Request may not have been completed |

Table B.2.1.1.4.2-1 Data Exchange Service – pubishComplete capability

###### B.2.1.1.4.3	Service Interactions

The interactions of service capabilities required for this service capability as shown in Figure B.2.1.1.3.3-1:

- Upon a successful publish by the Broker, record the event for accounting purposes

###### B.2.1.1.4.4	Post-Conditions

Success case: The request is permitted.

Failure case: The request is not permitted and a response type is transmitted back to the AE.

###### B.2.1.1.4.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### B.2.1.1.4.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 50ms

##### B.2.1.1.5	notifyRequest

This service capability provides the ability to validate a notification request to an AE.

As part of the notification, delivery policy that is requested to be used is validated and adjusted if necessary.

###### B.2.1.1.5.1	Pre-conditions

A correlation between a M2M Service Subscription, subscribing AE, subscribed resource and Broker exist.

The pre-conditions for Mca Received Requests are met.

###### B.2.1.1.5.2	Signature – notifyRequest

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input parameters | - | - | See Table A.2.1-1 |
| fromResource | IN | NO | The leaf node of a publication resource. See 6.4.1.1.1.1 |
| deliveryPolicy | IN-OUT | YES | The delivery policy when notifying the subscriber. See 6.4.1.1.1.2 |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Originator does not have a Broker for the requested resource - Originator has requested an invalid delivery policy - Exception: Request may not have been completed |

Table B.2.1.1.5.2-1 Data Exchange Service – notifyRequest capability

###### B.2.1.1.5.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca Reference Point

- Validate that the delivery policy requested by the Broker is allowed for the subscribing AE and M2M Service Subscription

Figure B.2.1.1.5.3-1 notifyRequest and notifyComplete Diagram

###### B.2.1.1.5.4	Post-Conditions

Success case: The request is permitted.

Failure case: The request is not permitted and a response type is transmitted back to the transport adapter.

###### B.2.1.1.5.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### B.2.1.1.5.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### B.2.1.1.6	notifyComplete

This service capability provides the ability to complete the actions required once the notification request has been completed in the Data Exchange service.

###### B.2.1.1.6.1	Pre-conditions

Not Applicable

###### B.2.1.1.6.2	Signature – notifyComplete

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input parameters | - | - | See Table A.2.1-3 |
| fromResource | IN | NO | The leaf node of a publication resource. See 6.4.1.1.1.1 |
| deliveryPolicy | IN | YES | The delivery policy when notifying the subscriber. See 6.4.1.1.1.2 |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Exception: Request may not have been completed |

Table B.2.1.1.6.2-1 Data Exchange Service – notifyComplete capability

###### B.2.1.1.6.3	Service Interactions

The interactions of service capabilities required for this service capability as shown in Figure B.2.1.1.5.3-1:

- Upon a successful notification by the Data Exchange, record the event for accounting purposes

###### B.2.1.1.6.4	Post-Conditions

Success case: The request is permitted.

Failure case: The request is not permitted and a response type is transmitted back to the transport adapter.

###### B.2.1.1.6.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### B.2.1.1.6.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 50ms

### B.2.2	Request-Response Message Exchange

This service provides the capability to exchange data using the synchronous request-response message pattern where AEs request to send a message to an AE and wait for a response to the message.

#### B.2.2.1	Service Capabilities

##### B.2.2.1.1	sendMessageRequest

This service capability provides the ability to validate a send message request from an AE.

###### B.2.2.1.1.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a M2M Service Subscription, originating AE and Transport Adapter exist.

###### B.2.2.1.1.2	Signature – sendMessageRequest

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input parameters | - | - | See Table A.2.1-3 |
| deliveryPolicy | IN | YES | The delivery policy when sending a request. See 6.4.1.1.1.2 |
| transportAdapter | OUT | YES | The instance of the transport adapter service. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Originator does not have a transport adaptor for the requested AE. - Delivery policy not supported by the underlying transport adapter. |

Table B.2.2.1.1.2-1 Data Exchange Service – sendMessageRequest capability

###### B.2.2.1.1.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Determine the transport adapter to be used for the originating AE and target AE. This example shows the use of the Broker service for the transport adapter.

- Perform the Common Request Services for requests across the Mca Reference Point

- Validate the delivery policy

Figure B.2.2.1.1.3-1 sendMessageRequest and sendMessageComplete Diagram

###### B.2.2.1.1.4	Post-Conditions

Success case: The request is permitted.

Failure case: The request is not permitted and a response type is transmitted back to the AE.

###### B.2.2.1.1.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### B.2.2.1.1.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 250ms

##### B.2.2.1.2	sendMessageComplete

This service capability provides the ability to complete the actions required once the request has been completed in the Broker.

###### B.2.2.1.2.1	Pre-conditions

Not Applicable

###### B.2.2.1.2.2	Signature – sendMessageComplete

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input parameters | - | - | See Table A.2.1-3 |
| deliveryPolicy | IN | YES | The delivery policy when notifying the subscriber. See 6.4.1.1.1.2 |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Exception: Request may not have been completed |

Table B.2.2.1.2.2-1 Data Exchange Service – sendMessageComplete capability

###### B.2.2.1.2.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Upon a successful message send and received from the transport adapter, record the event for accounting purposes

###### B.2.2.1.2.4	Post-Conditions

Success case: The request is permitted.

Failure case: The request is not permitted and a response type is transmitted back to the AE.

###### B.2.2.1.2.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### B.2.2.1.2.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 50 ms

# Annex C (Informative): Service Subscription Administration Services

## C.1	Overview

This annex illustrates the usage of the services for requests to administer M2M Service Subscriptions from AEs across the Mca Reference Point.

### C.2	Supporting Services

This service provides the capability for AEs to administer the M2M Service Subscriptions that have been previously created by the M2M Service Provider. The following capabilities are provided.

Retrieval of M2M Service Subscriptions based on a set of filter criteria.

Administration of the M2M Nodes associated with a M2M Service Subscriptions as well as the Application Rules associated with the M2M Node.

#### C.2.1	Service Capabilities

##### C.2.1.1	getServiceSubscription

This service capability provides the ability for an AE to retrieve the M2M Service Subscription across the Mca Reference Point.

###### C.2.1.1.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

###### C.2.1.1.2	Signature – getServiceSubscription

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Mca Common request input parameters | - | - | See Table A.2.1-1 |
| filterCriteria | IN | NO | See Table 6.9.1.1.1-1 |
| serviceSubscriptions | OUT | NO | The resulting M2M Service Subscription entities in Table 6.9.1.2-1. |
| responseType | OUT | YES | Unique response types for this service. None |

Table C.2.2.1.1.2-1 Service Subscription Administration – getServiceSubscription capability

###### C.2.1.1.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the common request authorization

- Retrieve the M2M Service Subscriptions for the supplied criteria

- Account for the event

Figure C.2.1.1.3-1 getServiceSubscription Diagram

###### C.2.1.1.4	Post-Conditions

Not Applicable

###### C.2.1.1.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### C.2.1.1.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### C.2.1.2	addNodeToServiceSubscription

This service capability adds a M2M Node to a M2M Service Subscription.

###### C.2.1.2.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

###### C.2.1.2.2	Signature – addNodeToServiceSubscription

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Mca Common request input parameters | - | - | See Table A.2.1-1 |
| externalIds | IN | YES | A List of URNs that represent the external identifiers associated with this M2M Node. |
| applicationRuleIds | IN | YES | A list of Application Rules associated with this Node. |
| nodeId | OUT | NO | The unique M2M Node identifer for this entity. |
| responseType | OUT | YES | Unique response types for this service. M2M Service Subscription does not exist. |
| applicationRuleIds | IN | YES | - A list of Application Rules associated with this Node. |

Table C.2.1.2.2-1 Service Subscription Administration – addDeviceToServiceSubscription capability

###### C.2.1.2.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the common request authorization

- Add the M2M Nodes to the M2M Service Subscription

- Account for the event

Figure C.2.1.2.3-1 addNodeToServiceSubscription Diagram

###### C.2.1.2.4	Post-Conditions

The M2M Node is added to the M2M Service Subscription returning the oneM2M System.

###### C.2.1.2.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### C.2.1.2.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### C.2.1.3	deleteNodesFromServiceSubscription

This service capability deletes M2M Nodes from a M2M Service Subscription.

###### C.2.1.3.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

###### C.2.1.3.2	Signature – deleteNodesFromServiceSubscription

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input parameters | - | - | See Table A.2.1-1 |
| nodeIds | IN | NO | List of M2M Node identifiers to be deleted from the M2M Service Subscription. |
| responseType | OUT | YES | Unique response types for this service. - M2M Service Subscription does not exist. M2M Node does not exist for this M2M Service Subscription |

Table C.2.1.3.2-1 Service Subscription Administration – deleteNodesFromServiceSubscription capability

###### C.2.1.3.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the common request authorization

- Delete the M2M Nodes from the M2M Service Subscription

- Account for the event

Figure C.2.1.3.3-1 deleteNodesFromServiceSubscription Diagram

###### C.2.1.3.4	Post-Conditions

The M2M Nodes are deleted from the M2M Service Subscription.

###### C.2.1.3.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### C.2.1.3.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### C.2.1.4	getNodesForServiceSubscription

This service capability retrieves the M2M Nodes for a M2M Service Subscription.

###### C.2.1.4.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

###### C.2.1.4.2	Signature – getNodesForServiceSubscription

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input parameters | - | - | See Table A.2.1-1 |
| filterCriteria | IN | NO | See Table 6.9.1.2.1-1 |
| nodes | OUT | NO | The resulting M2M Node entities in Table 6.9.1.2-1. |
| responseType | OUT | YES | Unique response types for this service. - None |

Table C.2.1.4.2-1 Service Subscription Administration – getNodesForServiceSubscription capability

###### C.2.1.4.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the common request authorization

- Retrieve the M2M Nodes with a filter condition

- Account for the event

Figure C.2.1.4.3-1 getNodesForServiceSubscription Diagram

###### C.2.1.4.4	Post-Conditions

Not Applicable

###### C.2.1.4.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### C.2.1.4.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### C.2.1.5	createApplicationRule

This service capability provides the ability to create a M2M Application Rule to to be associated with one or more M2M Nodes.

###### C.2.1.5.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

###### C.2.1.5.2	Signature – createApplicationRule

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input parameters | - | - | See Table A.2.1-1 |
| credentialIds | IN | YES | List of credential identifiers for which this Application Rule is applicable, i.e. for registration requests coming into the M2M System via a Security Association Endpoint (SEA) [i.7], that was authenticated using credentials that match with any of these credential identifiers, the current Application Rule applies. |
| allowedApplicationIds | IN | YES | List of M2M Application Identifiers (App-ID) that shall be considered to be allowed for AE registration requests received via Security Association Endpoint (SEA) [i.8]. |
| allowedAEIds | IN | YES | List of M2M Application Entity Identifiers (AE-ID) that shall be considered to be allowed for AE registration requests. |
| applicationRuleId | OUT | NO | The unique Application Rule identifier in the context of the M2M Service Subscription. |
| responseType | OUT | YES | Unique response types for this service. - None |

Table C.2.1.5.2-1 Service Subscription Administration – createApplciationRule capability

###### C.2.1.5.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the common request authorization

- Create the Application Rule

- Account for the event

Figure C.2.1.5.3-2 createApplicationRule Diagram

###### C.2.1.5.4	Post-Conditions

The Application Rule is creared.

###### C.2.1.5.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### C.2.1.5.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### C.2.1.6	deleteApplicationRules

This service capability provides the ability to delete Application Rules and any references to the Application Rule made by M2M Nodes to the Application Rules.

###### C.2.1.6.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

###### C.2.1.6.2	Signature – deleteApplicationRules

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input parameters | - | - | See Table A.2.1-1 |
| applicationRuleIds | IN | NO | A list of Application Rule identifiers. |
| responseType | OUT | YES | Unique response types for this service. Application Rule does not exist |

Table C.2.1.6.2-1 Service Subscription Administration – deleteApplicationRules capability

###### C.2.1.6.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the common request authorization

- Delete the Application Rules

- Account for the event

Figure C.2.1.6.3-1 deleteApplicationRules Diagram

###### C.2.1.6.4	Post-Conditions

Application Rules and any references to the Application Rule made by M2M Nodes to the Application Rules are deleted.

###### C.2.1.6.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### C.2.1.6.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### C.2.1.7	getApplicationRules

This service capability provides the ability to retrieve the Application Rules.

###### C.2.1.7.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

###### C.2.1.7.2	Signature – getApplicationRules

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input parameters | - | - | See Table A.2.1-1 |
| filterCriteria | IN | NO | See Table 6.9.1.3.1-1 |
| applicationRules | OUT | NO | The resulting M2M Node entities in Table 6.9.1.3-1. |
| responseType | OUT | YES | Unique response types for this service. - None |

Table C.2.1.7.2-1 Service Subscription Administration – getApplicationRulescapability

###### C.2.1.7.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the common request authorization

- Retrieve the M2M Applications of the M2M Service Subscription

- Account for the event

Figure C.2.1.7.3-1 getApplicationRules Diagram

###### C.2.1.7.4	Post-Conditions

Not Applicable

###### C.2.1.7.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### C.2.1.7.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### C.2.1.8	updateApplicationRule

This service capability provides the ability to update an Application Rule.

###### C.2.1.8.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

###### C.2.1.8.2	Signature – updateApplicationRule

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Mca Common request input parameters | - | - | See Table A.2.1-1 |
| applicationRuleId | IN | NO | The unique Application Rule identifier in the context of the M2M Service Subscription. |
| credentialIds | IN | YES | List of credential identifiers for which this Application Rule is applicable, i.e. for registration requests coming into the M2M System via a Security Association Endpoint (SEA) [i.9], that was authenticated using credentials that match with any of these credential identifiers, the current Application Rule applies. When present this list will overlay the list of credential identifiers. |
| allowedApplicationIds | IN | YES | List of M2M Application Identifiers (App-ID) that shall be considered to be allowed for AE registration requests received via Security Association Endpoint (SEA) [i.10]. When present this list will overlay the list of allowed App-IDs. |
| responseType | OUT | YES | Unique response types for this service. - Application Rule does not exist. |

Table C.2.1.8.2-1 Service Subscription Administration – updateApplicationForDevice capability

###### C.2.1.8.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the common request authorization

- Update the Application Rule.

- Account for the event

Figure C.2.1.8.3-1 updateApplicationRule Diagram

###### C.2.1.8.4	Post-Conditions

The Applicaton Rule is updated.

###### C.2.1.8.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### C.2.1.8.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### C.2.1.9	updateNodeForServiceSubscription

This service capability provides the ability to update the Application Rules and External Identifiers to be associated to a Node within the context of a M2M Service Subscription.

###### C.2.1.9.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

###### C.2.1.9.2	Signature – updateNodeForServiceSubscription

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Mca Common request input parameters | - | - | See Table A.2.1-1 |
| nodeId | IN | NO | The unique M2M Node identifier in the context of the M2M Service Subscription. |
| applicationRuleIds | IN | YES | A list of Application Rule Identifier. When present this list will overlay the list of Applications associated with the Node. |
| externalIds | IN | YES | List of URNs that represent the external identifiers associated with this device. When present this list will overlay the list of external identifiers associated with the Node. |
| responseType | OUT | YES | Unique response types for this service. - M2M Service Subscription does not exist - M2M Node does not exist for the M2M Service Subscription |

Table C.2.1.9.2-1 Service Subscription Administration – updateNodeForServiceSubscription capability

###### C.2.1.9.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the common request authorization

- Update the M2M Node information for the M2M Service Subscription

- Account for the event

Figure C.2.1.9.3-1 updateNodeForServiceSubscription Diagram

###### C.2.1.9.4	Post-Conditions

The M2M Node for the M2M Service Subscription is updated.

###### C.2.1.9.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### C.2.1.9.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

# Annex D (Informative): Device Management Services

## D.1	Overview

This annex illustrates the usage of the services for requests to manage device from AEs across the Mca Reference Point.

### D.2	Supporting Services

### D.2.1	Message Exchange

This service provides the capability to manage device using the request-response or request-asynchronous report message patterns where AEs:

- Request to manage device using device specific service capabilities or more complex service capabilities that the Support Service will orchestrate into the individual service capabilities according to their business process.

- Request to obtain management operation execution results or status using the reporting capability. Reports can be generated or batched by the report policy.

#### D.2.1.1	Service Capabilities

##### D.2.1.1.1	downloadFirmware

This service capability provides the ability to execute a downloadFirmware request from an AE.

###### D.2.1.1.1.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

###### D.2.1.1.1.2	Signature – downloadFirmware

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input attributes | - | - | See Table A.2.1-1 |
| firmwareInfo | IN | NO | The device firmware information. Type FirmwareInfo, see 6.6.1.1.1.5 |
| deviceId | IN | YES | The unique device identifier in the context of the M2M M2M Service Subscription. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.1.1.1.1. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. |

Table D.2.1.1.1.2-1 Device Management Service –downloadFirmware capability

###### D.2.1.1.1.3	Service Interactions

The interactions of service capabilities required for this service capability:

1) Perform the Common Request Services for requests across the Mca Reference Point

2) Retrieves the Management Adapter to be used for the device and M2M Service Subscription

3) Issues the request to the Management Adapter to download the firmware

4) Records the event

Figure D.2.1.1.1.3-1 downloadFirmware Diagram

###### D.2.1.1.1.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to download firmware.

The event has been recorded.

###### D.2.1.2.1.5	Exceptions

Not Applicable

###### D.2.1.2.1.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### D.2.1.1.2	installFirmware

This service capability provides the ability to execute a installFirmware request from an AE.

###### D.2.1.1.2.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

###### D.2.1.1.2.2	Signature – installFirmware

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input attributes | - | - | See Table A.2.1-1 |
| firmwareInfo | IN | NO | The device firmware information. Type FirmwareInfo, see 6.6.1.1.1.5 |
| deviceId | IN | YES | The unique device identifier in the context of the M2M M2M Service Subscription. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. |

Table D.2.1.1.2.2-1 Device Management Service –installFirmware capability

###### D.2.1.1.2.3	Service Interactions

The interactions of service capabilities required for this service capability:

1) Perform the Common Request Services for requests across the Mca Reference Point

2) Retrieves the Management Adapter to be used for the device and M2M Service Subscription

3) Issues the request to the Management Adapter to install the firmware

4) Records the event

Figure D.2.1.1.2.3-1 installFirmware Diagram

###### D.2.1.1.2.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to install firmware.

The event has been recorded.

###### D.2.1.1.3.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### D.2.1.1.3.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### D.2.1.1.3	getFirmwareInformation

This service capability provides the ability to execute a getFirmwareInformation request from an AE.

###### D.2.1.1.3.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

###### D.2.1.1.3.2	Signature – getFirmwareInformation

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input attributes | - | - | See Table A.2.1-1 |
| deviceId | IN | YES | The unique device identifier in the context of the M2M M2M Service Subscription. |
| firmwareInfo | OUT | YES | The device firmware information. Type FirmwareInfo, see 6.6.1.1.1.5 |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. |

Table D.2.1.1.3.2-1 Device Management Service –getFirmwareInformation capability

###### D.2.1.1.3.3	Service Interactions

The interactions of service capabilities required for this service capability:

1) Perform the Common Request Services for requests across the Mca Reference Point

2) Retrieves the Management Adapter to be used for the device and M2M Service Subscription

3) Issues the request to the Management Adapter to get the firmware information

4) Records the event

Figure D.2.1.1.3.3-1 getFirmwareInformation Diagram

###### D.2.1.1.3.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get firmware information.

The event is recorded

###### D.2.1.1.3.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### D.2.1.1.3.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### D.2.1.1.4	getFirmwareExecStatus

This service capability provides the ability to retrieve the execution status or result of a requested firmware operation.

###### D.2.1.1.4.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and previously submitted firmware request exist.

###### D.2.1.1.4.2	Signature – getFirmwareExecStatus

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input attributes | - | - | See Table A.2.1-1 |
| operationRequestId | IN | NO | The M2M Request Identifier of previously submitted firmware request (M2M-Request-ID) |
| deviceId | IN | YES | The unique device identifier in the context of the M2M M2M Service Subscription. |
| firmwareReport | OUT | YES | The firmware management operation execution result or status. Type FirmwareReport, see 6.6.1.1.1.3. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - The previously submitted firmware request does not exist |

Table D.2.1.1.4.2-1 Device Management Service –getFirmwareExecStatus capability

###### D.2.1.1.4.3	Service Interactions

The interactions of service capabilities required for this service capability:

1) Perform the Common Request Services for requests across the Mca Reference Point

2) Retrieves the Management Adapter to be used for the device and M2M Service Subscription

3) Issues the request to the Management Adapter to retrieve the firmware operations execution status or result

4) Records the event

Figure D.2.1.1.4.3-1 getFirmwareExecStatus Diagram

###### D.2.1.1.4.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get firmware operation execution status or result.

Record the event.

###### D.2.1.1.4.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### D.2.1.1.4.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### D.2.1.1.5	deviceManagementReport

This service capability provides the ability to report an AE about the status or result of management operations for  previously submitted operation requests.

###### D.2.1.1.5.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and previously submitted management requests exist.

###### D.2.1.1.5.2	Signature –deviceManagementReport

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input attributes | - | - | See Table A.2.1-1 |
| isLastReport | IN | NO | Boolean, whether it is the last report. |
| sequenceNumber | IN | NO | The report sequence number. |
| firmwareReportList | IN | NO | Array of firmwareReport. Type FirmwareReport, see 6.6.1.1.1.3. |
| troubleshootingReportList | IN | YES | Array of troubleshootingReport. Type TroubleshootingReport, see 6.6.1.1.1.20. |
| softwareReportList | IN | YES | Array of softwareReport. Type SoftwareReport, see 6.6.1.1.1.27. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. |

Table D.2.1.1.5.2-1 Device Management Service –deviceManagementReport capability

###### D.2.1.1.5.3	Service Interactions

The interactions of service capabilities required for this service capability:

1) Perform the Common Request Services for requests across the Mca Reference Point

2) Record the event.

Figure D.2.1.1.5.3-3 deviceManagementReport Diagram

###### D.2.1.1.5.4	Post-Conditions

The AE has received a report of management operation execution status or result.

The event is recorded.

###### D.2.1.1.5.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### D.2.1.1.5.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### D.2.1.1.6	upgradeFirmware

This service capability permits AEs to upgrade the firmware on individual device, multiple devices or a group of devices. In addition the upgrade of the firmware is permitted based on a schedule for each of the operations

###### D.2.1.1.6.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between Management Adapter, the M2M Service Capability and devices or device group exist.

###### D.2.1.1.6.2	Signature – upgradeFirmware

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input attributes | - | - | See Table A.2.1-1 |
| orchestrationRuleList | IN | NO | List of OrchestrationRule. Type OrchestrationRule, see clause 6.6.1.1.1.4. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.1.1.1.1. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. |

Table D.2.1.1.6.2-1 Device Management Service –upgradeFirmware capability

###### D.2.1.1.6.3	Service Interactions

The interactions of service capabilities required for this service capability:

1) Perform the Common Request Services for requests across the Mca Reference Point

2) Retrieves the Management Adapter to be used for the device and M2M Service Subscription

3) Issues a series of operations to the Management Adapter to upgrade the firmware on a the devices in the orchestration rule list. Each entry in the orchestration rule list contains the operation, the devices and schedule for the operation.

4) Records the event

Figure D.2.1.1.6.3-4 upgradeFirmware Diagram

###### D.2.1.1.6.4	Post-Conditions

The Management Adapter has submitted a set of requests to the Management Server to upgrade the firmware.

The event is recorded.

###### D.2.1.1.6.5	Exceptions

The reachability schedule of the device doesn’t not match the schedule of the orchestration rule.

###### D.2.1.1.6.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### D.2.1.1.7	getDeviceInformation

This service capability provides the ability for an AE to retrieve the information about a device across the Mca Reference Point.

###### D.2.1.1.7.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

###### D.2.1.1.7.2	Signature – getDeviceInformation

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input attributes | - | - | See Table A.2.1-1 |
| deviceId | IN | NO | The unique device identifier in the context of the M2M M2M Service Subscription. |
| deviceInfo | OUT | NO | The device information. Type DeviceInfo, see 6.6.1.1.1.7. |
| memory | OUT | YES | The memory information. Type Memory, see 6.6.1.1.1.8. |
| battery | OUT | YES | The battery information. Type Battery, see 6.6.1.1.1.9. |
| lockStatus | OUT | YES | The device lock status. Enum LockStatus, see 6.6.1.1.1.13. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. |

Table D.2.1.1.7.2-1 Device Management Service –getDeviceInformation capability

###### D.2.1.1.7.3	Service Interactions

The interactions of service capabilities required for this service capability:

1) Perform the Common Request Services for requests across the Mca Reference Point

2) Retrieves the Management Adapter to be used for the device and M2M Service Subscription

3) Issues the request to the Management Adapter to get device information

4) Records the event

Figure D.2.1.1.7.3-1 getDeviceInformation Diagram

###### D.2.1.1.7.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get device information.

Based on the capabilities supported by the device, the memory, battery and lock status information may or may not be returned.

The event has been recorded.

###### D.2.1.1.7.5	Exceptions

Not Applicable

###### D.2.1.1.7.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### D.2.1.1.8	getDeviceCapabilities

This service capability provides the ability to execute a getDeviceCapabilities request from an AE.

###### D.2.1.1.8.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

###### D.2.1.1.8.2	Signature – getDeviceCapabilities

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input attributes | - | - | See Table A.2.1-1 |
| deviceId | IN | NO | The unique device identifier in the context of the M2M M2M Service Subscription. |
| deviceCapabilities | OUT | YES | Array of deviceCapability. Type DeviceCapability, see 6.6.1.1.1.10. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. |

Table D.2.1.1.8.2-1 Device Management Service –getDeviceCapabilities capability

###### D.2.1.1.8.3	Service Interactions

The interactions of service capabilities required for this service capability:

1) Perform the Common Request Services for requests across the Mca Reference Point

2) Retrieves the Management Adapter to be used for the device and M2M Service Subscription

3) Issues the request to the Management Adapter to get device capabilities

4) Records the event

Figure D.2.1.1.8.3-1 getDeviceCapabilities Diagram

###### D.2.1.1.8.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get device capabilities.

The event has been recorded.

###### D.2.1.1.8.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### D.2.1.1.8.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### D.2.1.1.9	enableDeviceCapability

This service capability provides the ability to execute a enableDeviceCapability request from an AE.

###### D.2.1.1.9.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

###### D.2.1.1.9.2	Signature – enableDeviceCapability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input attributes | - | - | See Table A.2.1-1 |
| deviceId | IN | NO | The unique device identifier in the context of the M2M M2M Service Subscription. |
| name | IN | NO | The name of the capability. Enum DeviceCapabilityName, see 6.6.1.1.1.12. |
| state | OUT | NO | Indicates if the capability is enabled or disabled. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. |

Table D.2.1.1.9.2-1 Device Management Service –enableDeviceCapability capability

###### D.2.1.1.9.3	Service Interactions

The interactions of service capabilities required for this service capability:

1) Perform the Common Request Services for requests across the Mca Reference Point

2) Retrieves the Management Adapter to be used for the device and M2M Service Subscription

3) Issues the request to the Management Adapter to enable device capability

4) Records the event

Figure D.2.1.1.9.3-1 enableDeviceCapability Diagram

###### D.2.1.1.9.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to enable device capability.

The event is recorded

###### D.2.1.1.9.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### D.2.1.1.9.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### D.2.1.1.10	disableDeviceCapability

This service capability provides the ability to execute a disableDeviceCapability request from an AE.

###### D.2.1.1.10.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

###### D.2.1.1.10.2	Signature – disableDeviceCapability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input attributes | - | - | See Table A.2.1-1 |
| deviceId | IN | NO | The unique device identifier in the context of the M2M M2M Service Subscription. |
| name | IN | NO | The name of the capability. Enum DeviceCapabilityName, see 6.6.1.1.1.12. |
| state | OUT | NO | Indicates if the capability is enabled or disabled. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. |

Table D.2.1.1.10.2-1 Device Management Service –disableDeviceCapability capability

###### D.2.1.1.10.3	Service Interactions

The interactions of service capabilities required for this service capability:

1) Perform the Common Request Services for requests across the Mca Reference Point

2) Retrieves the Management Adapter to be used for the device and M2M Service Subscription

3) Issues the request to the Management Adapter to disable device capability

4) Records the event

Figure D.2.1.1.10.3-1 disableDeviceCapability Diagram

###### D.2.1.1.10.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to disable device capability.

Record the event.

###### D.2.1.1.10.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### D.2.1.1.10.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### D.2.1.1.11	getAreaNetworks

This service capability provides the ability to execute a getAreaNetworks request from an AE.

###### D.2.1.1.11.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and area network exist.

###### D.2.1.1.11.2	Signature –getAreaNetworks

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input attributes | - | - | See Table A.2.1-1 |
| areaNwks | OUT | YES | Array of area network. Type AreaNwkInfo, see 6.6.1.1.1.13. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. |

Table D.2.1.1.11.2-1 Device Management Service –getAreaNetworks capability

###### D.2.1.1.11.3	Service Interactions

The interactions of service capabilities required for this service capability:

1) Perform the Common Request Services for requests across the Mca Reference Point

2) Retrieves the Management Adapter to be used for the device and M2M Service Subscription

3) Issues the request to the Management Adapter to get area networks information

4) Records the event

Figure D.2.1.1.11.3-1 getAreaNetworks Diagram

###### D.2.1.1.11.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get area networks information.

The event has been recorded.

###### D.2.1.1.11.5	Exceptions

Not Applicable

###### D.2.1.1.11.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### D.2.1.1.12	updateDeviceForAreaNetwork

This service capability provides the ability to execute a updateDeviceForAreaNetwork request from an AE.

###### D.2.1.1.12.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability, the device and area network exist.

###### D.2.1.1.12.2	Signature –updateDeviceForAreaNetwork

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input attributes | - | - | See Table A.2.1-1 |
| areaNwkId | IN | NO | The unique area network identifier. |
| deviceId | IN | NO | The unique device identifier in the context of the M2M M2M Service Subscription. |
| areaNwkDeviceInfo | IN | YES | The existing areaNwkDeviceInfo are replaced with the information in this parameter. Type AreaNwkDeviceInfo, see 6.6.1.1.1.15. |
| lastModifiedTime | OUT | NO | The modified time. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. |

Table D.2.1.1.12.2-1 Device Management Service –updateDeviceForAreaNetwork capability

###### D.2.1.1.12.3	Service Interactions

The interactions of service capabilities required for this service capability:

1) Perform the Common Request Services for requests across the Mca Reference Point

2) Retrieves the Management Adapter to be used for the device and M2M Service Subscription

3) Issues the request to the Management Adapter to update device information for area network

4) Records the event

Figure D.2.1.1.12.3-5 updateDeviceForAreaNetwork Diagram

###### D.2.1.1.12.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to update device information for area network.

The event is recorded.

###### D.2.1.1.12.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### D.2.1.1.12.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### D.2.1.1.13	rebootDevice

This service capability provides the ability to execute a rebootDevice request from an AE.

###### D.2.1.1.13.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

###### D.2.1.1.13.2	Signature –rebootDevice

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input attributes | - | - | See Table A.2.1-1 |
| deviceId | IN | NO | The unique device identifier in the context of the M2M M2M Service Subscription. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.1.1.1.1. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. |

Table D.2.1.1.13.2-1 Device Management Service –rebootDevice capability

###### D.2.1.1.13.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca Reference Point

- Retrieves the Management Adapter to be used for the device and M2M Service Subscription

- Issues the request to the Management Adapter to reboot device

- Records the event

Figure D.2.1.1.13.3-1 rebootDevice Interaction Diagram

###### D.2.1.1.13.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to reboot device.

The event has been recorded.

###### D.2.1.1.13.5	Exceptions

Not Applicable

###### D.2.1.1.13.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### D.2.1.1.14	resetDevice

This service capability provides the ability to execute a resetDevice request from an AE.

###### D.2.1.1.14.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

###### D.2.1.1.14.2	Signature –resetDevice

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input attributes | - | - | See Table A.2.1-1 |
| deviceId | IN | NO | The unique device identifier in the context of the M2M M2M Service Subscription. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.1.1.1.1. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. |

Table D.2.1.1.14.2-1 Device Management Service –resetDevice capability

###### D.2.1.1.14.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca Reference Point

- Retrieves the Management Adapter to be used for the device and M2M Service Subscription

- Issues the request to the Management Adapter to reset device

- Records the event

Figure D.2.1.1.14.3-1 resetDevice Interaction Diagram

###### D.2.1.1.14.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to reset device.

The event has been recorded.

###### D.2.1.1.14.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### D.2.1.1.14.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### D.2.1.1.15	uploadDeviceLog

This service capability provides the ability to execute a uploadDeviceLog request from an AE.

###### D.2.1.1.15.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

###### D.2.1.1.15.2	Signature –uploadDeviceLog

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input attributes | - | - | See Table A.2.1-1 |
| deviceId | IN | NO | The unique device identifier in the context of the M2M M2M Service Subscription. |
| logInfo | IN | NO | The log information. Type LogInfo, see 6.6.1.1.1.19. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.1.1.1.1. |
| logURL | OUT | YES | The URL from which the log can be uploaded. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. |

Table D.2.1.1.15.2-1 Device Management Service –uploadDeviceLog capability

###### D.2.1.1.15.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca Reference Point

- Retrieves the Management Adapter to be used for the device and M2M Service Subscription

- Issues the request to the Management Adapter to upload device log

- Records the event

Figure D.2.1.1.15.3-1 uploadDeviceLog Interaction Diagram

###### D.2.1.1.15.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to upload device log.

The event is recorded

###### D.2.1.1.15.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### D.2.1.1.15.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### D.2.1.1.16	getDeviceLogs

This service capability provides the ability to execute a getDeviceLogs request from an AE.

###### D.2.1.1.16.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

###### D.2.1.1.16.2	Signature –getDeviceLogs

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input attributes | - | - | See Table A.2.1-1 |
| deviceId | IN | NO | The unique device identifier in the context of the M2M M2M Service Subscription. |
| logList | OUT | YES | Array of log. Type Log, see 6.6.1.1.1.21. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. |

Table D.2.1.1.16.2-1 Device Management Service –getDeviceLogs capability

###### D.2.1.1.16.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca Reference Point

- Retrieves the Management Adapter to be used for the device and M2M Service Subscription

- Issues the request to the Management Adapter to get all logs of a device

- Records the event

Figure D.2.1.1.16.3-1 getDeviceLogs Interaction Diagram

###### D.2.1.1.16.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get all logs of a device.

The event is recorded

###### D.2.1.1.16.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### D.2.1.1.16.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### D.2.1.1.17	getDeviceLogInformation

This service capability provides the ability to execute a getDeviceLogInformation request from an AE.

###### D.2.1.1.17.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

###### D.2.1.1.17.2	Signature –getDeviceLogInformation

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input attributes | - | - | See Table A.2.1-1 |
| deviceId | IN | NO | The unique device identifier in the context of the M2M M2M Service Subscription. |
| filterCriteria | IN | YES | See Table 6.6.1.1.1.22 |
| logURL | IN | YES | The URL from which the log can be accessed. |
| log | OUT | YES | Type Log, see 6.6.1.1.1.21. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. |

Table D.2.1.1.17.2-1 Device Management Service –getDeviceLogInformation capability

###### D.2.1.1.17.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca Reference Point

- Retrieves the Management Adapter to be used for the device and M2M Service Subscription

- Issues the request to the Management Adapter to get a device log information

- Records the event

Figure D.2.1.1.17.3-1 getDeviceLogInformation Interaction Diagram

###### D.2.1.1.17.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get a device log information.

The event is recorded

###### D.2.1.1.17.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### D.2.1.1.17.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### D.2.1.1.18	getSoftwareInformation

This service capability provides the ability to execute a getSoftwareInformation request from an AE.

###### D.2.1.1.18.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

###### D.2.1.1.18.2	Signature –getSoftwareInformation

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input attributes | - | - | See Table A.2.1-1 |
| deviceId | IN | NO | The unique device identifier in the context of the M2M M2M Service Subscription. |
| version | OUT | YES | The version of the software. |
| name | OUT | YES | The name of the software. |
| URL | OUT | YES | The URL from which the software package can be downloaded. |
| installStatus | OUT | YES | Indicates the status of the install. Enum ActionStatus, see 6.6.1.1.1.26. |
| activeStatus | OUT | YES | The status of active or deactivate action. Enum ActionStatus, see 6.6.1.1.1.26. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. |

Table D.2.1.1.18.2-1 Device Management Service –getSoftwareInformation capability

###### D.2.1.1.18.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca Reference Point

- Retrieves the Management Adapter to be used for the device and M2M Service Subscription

- Issues the request to the Management Adapter to get application software information

- Records the event

Figure D.2.1.1.18.3-1 getSoftwareInformation Interaction Diagram

###### D.2.1.1.18.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get application software information.

The event has been recorded.

###### D.2.1.1.18.5	Exceptions

Not Applicable

###### D.2.1.1.18.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### D.2.1.1.19	downloadSoftware

This service capability provides the ability to execute a downloadSoftware request from an AE.

###### D.2.1.1.19.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

###### D.2.1.1.19.2	Signature –downloadSoftware

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input attributes | - | - | See Table A.2.1-1 |
| deviceId | IN | NO | The unique device identifier in the context of the M2M M2M Service Subscription. |
| version | IN | NO | The version of the software. |
| name | IN | NO | The name of the software. |
| URL | IN | NO | The URL from which the software package can be downloaded. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.1.1.1.1. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. |

Table D.2.1.1.19.2-1 Device Management Service –downloadSoftware capability

###### D.2.1.1.19.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca Reference Point

- Retrieves the Management Adapter to be used for the device and M2M Service Subscription

- Issues the request to the Management Adapter to download application software

- Records the event

Figure D.2.1.1.19.3-1 downloadSoftware Interaction Diagram

###### D.2.1.1.19.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to download application software.

The event has been recorded.

###### D.2.1.1.19.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### D.2.1.1.19.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### D.2.1.1.20	installSoftware

This service capability provides the ability to execute a installSoftware request from an AE.

###### D.2.1.1.20.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

###### D.2.1.1.20.2	Signature –installSoftware

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input attributes | - | - | See Table A.2.1-1 |
| deviceId | IN | NO | The unique device identifier in the context of the M2M M2M Service Subscription. |
| version | IN | NO | The version of the software. |
| name | IN | NO | The name of the software. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.1.1.1.1. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. |

Table D.2.1.1.21.2-1 Device Management Service –installSoftware capability

###### D.2.1.1.20.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca Reference Point

- Retrieves the Management Adapter to be used for the device and M2M Service Subscription

- Issues the request to the Management Adapter to install application software

- Records the event

Figure D.2.1.1.20.3-1 installSoftware Interaction Diagram

###### D.2.1.1.20.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to install application software.

The event is recorded

###### D.2.1.1.20.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### D.2.1.1.20.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### D.2.1.1.21	activateSoftware

This service capability provides the ability to execute an activateSoftware request from an AE.

###### D.2.1.1.21.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

###### D.2.1.1.21.2	Signature –activateSoftware

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input attributes | - | - | See Table A.2.1-1 |
| deviceId | IN | NO | The unique device identifier in the context of the M2M M2M Service Subscription. |
| version | IN | NO | The version of the software. |
| name | IN | NO | The name of the software. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.1.1.1.1. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. |

Table D.2.1.1.21.2-1 Device Management Service –activateSoftware capability

###### D.2.1.1.21.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca Reference Point

- Retrieves the Management Adapter to be used for the device and M2M Service Subscription

- Issues the request to the Management Adapter to activate software previously installed

- Records the event

Figure D.2.1.1.21.3-1 activateSoftware Interaction Diagram

###### D.2.1.1.21.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to activate software previously installed.

Record the event.

###### D.2.1.1.21.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### D.2.1.1.21.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### D.2.1.1.22	deactivateSoftware

This service capability provides the ability to execute a deactivateSoftware request from an AE.

###### D.2.1.1.22.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

###### D.2.1.1.22.2	Signature –deactivateSoftware

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input attributes | - | - | See Table A.2.1-1 |
| deviceId | IN | NO | The unique device identifier in the context of the M2M M2M Service Subscription. |
| version | IN | NO | The version of the software. |
| name | IN | NO | The name of the software. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.1.1.1.1. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. |

Table D.2.1.1.22.2-1 Device Management Service –deactivateSoftware capability

###### D.2.1.1.22.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca Reference Point

- Retrieves the Management Adapter to be used for the device and M2M Service Subscription

- Issues the request to the Management Adapter to deactivates software

- Records the event

Figure D.2.1.1.22.3-1 deactivateSoftware Interaction Diagram

###### D.2.1.1.22.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to deactivates software.

The event is recorded.

###### D.2.1.1.22.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### D.2.1.1.22.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

##### D.2.1.1.23	removeSoftware

This service capability provides the ability to execute a removeSoftware request from an AE.

###### D.2.1.1.23.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

###### D.2.1.1.23.2	Signature –removeSoftware

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input attributes | - | - | See Table A.2.1-1 |
| deviceId | IN | NO | The unique device identifier in the context of the M2M M2M Service Subscription. |
| version | IN | NO | The version of the software. |
| name | IN | NO | The name of the software. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.1.1.1.1. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. |

Table D.2.1.1.23.2-1 Device Management Service –removeSoftware capability

###### D.2.1.1.23.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca Reference Point

- Retrieves the Management Adapter to be used for the device and M2M Service Subscription

- Issues the request to the Management Adapter to uninstall the software

- Records the event

Figure D.2.1.1.23.3-1 removeSoftware Interaction Diagram

###### D.2.1.1.23.4	Post-Conditions

The Management Adapter has submitted a set of requests to the Management Server to uninstall the software.

The event is recorded.

###### D.2.1.1.23.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### D.2.1.1.23.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

# Annex E (Informative): Device On-boarding Service

## E.1	Overview

This annex illustrates the usage of services to onboard a device or AE when the M2M Node or AE contacts the M2M System.

The Device On-boarding supporting service is a process by which a M2M Node is provided the artifacts (e.g., configuration information, software and firmware) needed to successfully register an AE with the M2M Service Layer and invoke or receive the authorized service capabilities.

### E.2	Supporting Services

### E.2.1	Remote Administration

This service provides remote administration of the device and/or AE when an AE registers or registers with the M2M System. The mechanism is implemented using the first contact subscription and notification services provided/used by the Registration Service.

#### E.2.1.1	Service Capabilities

##### E.2.1.1.1	notifyRegistrationContact

This service capability provides the capability to invoke a process to update the M2M Service Layer artifacts (e.g., credentials, device firmware, and application software).

###### E.2.1.1.1.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

A correlation between a M2M Service Subscription and the contacting entity (M2M Node, AE) exist.

The Supporting Service subscribed to receive registration events using the subscribeInitialAERegistrationEvent service capability.

###### E.2.1.1.1.2	Signature – notifyRegistrationContact

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Mca Common request input parameters | - | - | See Table A.2.1-1 |
| aeId | IN | NO | The AE-ID of the registered AE. |
| event | IN | NO | The event that caused the registration contact. Enumeration of: - FirstContact |

Table E.2.1.1.1.2-1 Device On-boarding Service – notifyRegistrationContact capability

###### E.2.1.1.1.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca Reference Point

Determine the Application information (Device, Subscription, and Application Id) for the registering AE.

Determine from the Device information if the device need be updated

Determine from the Application information if Applications need updated on the Device

Record the event for accounting purposes.

Figure E.2.1.1.1.3-1	notifyRegistrationContact Diagram

###### E.2.1.1.1.4	Post-Conditions

Success case: The request is permitted and the associated.

Failure case: The request is recorded as a failure.

###### E.2.1.1.1.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are recorded by this service capability.

###### E.2.1.1.1.6	Policies for Use

Message Exchange Patterns: In-Only

Transaction Pattern: Creates Transaction

Maximum Response: Not applicable for the In-Only message exchange pattern

# Annex F (Informative): Registration Services

## F.1	Overview

This annex illustrates the usage of the services for requests to register AEs across the Mca Reference Point and between M2M Service Components across the Msc Reference Point.

### F.2	Mca Registration Service Processing and Supporting Services

AEs communicate with the M2M System over the Mca Reference Point through the SE component. The SE component accepts the request from the AE and provides the request to the Supporting Service in order allow the M2M Service Provider to ensure the request is properly authorized and recorded. In addition, the M2M Service Provider is able to orchestrate other capabilities that have not been specified.

### F.2.1	Message Exchanges

#### F.2.1.1	Service Capabilities

##### F.2.1.1.1	registerAE

This service capability enables an AE to register with the M2M System. This service capability shall be restricted across the Mca Reference Point

###### F.2.1.1.1.1	Pre-conditions

The AE has not registered with the M2M System.

The pre-conditions for Mca Received Requests are met.

###### F.2.1.1.1.2	Signature – registerAE

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Mca Common request input parameters | - | - | See Table A.2.1-1 |
| pointOf Access | IN | NO | The point of Access of the registered AE. |
| applicationId | IN | NO | The Application Identifer (App-ID). |
| credentialId | IN | YES | Application Credentials. It is used to retrieve the corresponding Service Subscription |
| expirationTime | IN | NO | The expiration time of the registration as requested by the Originator |
| aeId | IN-OUT | Option for IN only-Not an Option for OUT | The Application Entity Identiifer (AE-ID) is provided back in the response |
| responseType | OUT | NO | Unique response types for this service: - AE successfully registered - AE not successfully registered |

Table  F.2.1.1.1.2-1  Registration Services – registerAE capability

###### F.2.1.1.1.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca Reference Point

- Record the event

Figure F.2.1.1.1.3-1 Registration Services – registerAE Diagram

###### F.2.1.1.1.4	Post-Conditions

Event is recorded

###### F.2.1.1.1.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### F.2.1.1.1.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 250ms

##### F.2.1.1.2	refreshAERegistration

This service capability enables an AE to refresh an existing registration with the M2M System

###### F.2.1.1.2.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

The AE is registered and allocated an aeId by the M2M System.

###### F.2.1.1.2.2	Signature – refreshAERegsitration

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Mca Common request input parameters | - | - | See Table A.2.1-1 |
| aeId | IN | NO | The Application Entity Identifier (AE-ID) to be refreshed. |
| pointOfAccess | IN | YES | The point of Access of the registered AE. POA is optional only if identical to the one in the refreshed registration |
| expirationTime | IN | NO | The expiration time of the registration as requested by the Originator. |
| reachabilitySchedule | IN | YES | The contact reachability schedule information of the AE associated with the device node. The absence of this parameter implies the AE associated with the device node is always contact reachable. Type Schedule, see 6.9.1.4.1. |
| responseType | OUT | NO | Unique response types for this service: - AE successfully refreshed - Registration does not exist |

Table F.2.1.1.2.2-1 Registration Services – refreshAERegistration capability

###### F.2.1.1.2.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Msc Reference Point

- Record the event

Figure F.2.1.1.2.3-1 Registration Services – refreshAERegistration Diagram

###### F.2.1.1.2.4	Post-Conditions

Success case: The request is permitted.

Failure case: The request is not permitted and a response type is transmitted to the AE

###### F.2.1.1.2.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### F.2.1.1.2.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 50ms

##### F.2.1.1.3	deregisterAE

This service capability enables an AE to deregister from the M2M System.

###### F.2.1.1.3.1	Pre-conditions

The pre-conditions for Mca Received Requests are met.

The Application Entity is registered.

###### F.2.1.1.3.2	Signature – deregisterAE

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Mca Common request input parameters | - | - | See Table A.2.1-1 |
| aeId | IN | NO | The Application Entity Identifier (AE-ID) to be deregistered. |
| responseType | OUT | NO | Unique response types for this service: - AE successfully De-registsred - Registration does not exist |

Table F.2.1.1.3.2-1 RegistrationServcies – deregisterAE capability

###### F.2.1.1.3.3	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Msc Reference Point

- Record the event

Figure F.2.1.1.3.3-1 Registration Services – deregisterAE Diagram

###### F.2.1.1.3.4	Post-Conditions

Success case: The request is permitted.

Failure case: The request is not permitted and a response type is transmitted back to the AE

###### F.2.1.1.3.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### F.2.1.1.3.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

# Annex X(Informative): M2M Service Capability Template

The M2M Service Capability template is used as a specification that defines the contract between the consumer of the service capability and the producer of the service capability. The specification contract includes documenting the provided Signature, constraints (pre-conditions, post-conditions, exceptions), policies and runtime expectations.

### X.1	<serviceCapabilityName>

<Document introductory description of the service capability>

The <serviceCapabilityName> reflects the operational functionality provided by the M2M Service Capability. M2M Service Capabilities are named using a construct [verb][noun][context] where the:

verb: Used to indicate what will happen to the noun

noun: Is the target of the noun

context: Is optional and further defines the service capability

### X.1.1	Signature

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Name of the parameter. | IN, OUT, IN-OUT | YES, NO | Description of the parameter in the context of the service capability. |

Table X.1.1-1 M2M Service Capability Signature

Direction: The direction is relative to the entity that provides (implements) the M2M Service Capability. The value “IN” means that the entity expects to receive a value for the parameter from the consumer (sender) of the M2M Service Capability request. The value “Out” means that the entity will send a value for the parameter to the consumer (sender) of the M2M Service Capability request. A value of “IN-OUT” means that the entity will receive the value for the parameter from the consumer and then send a value (not necessarily the same value) for the parameter back to the consumer.

Optional: Indicates if the parameter is required in requests (Direction: IN, IN-OUT) or responses (Direction: IN-OUT, OUT). A value of YES means that the parameter is optional. A value of NO means that the parameter is mandatory.

### X.1.2	Pre-conditions

<Document the pre-conditions that may only be valid to run provided some testable prior business rule check or other value to be changed is actually present and available for the service capability to act upon. Pre-conditions are documented that are in addition to that provided by the internal resource operations and procedures>

### X.1.3	Service Interactions

<Document the detail procedure for <serviceCapabilityName>. The detail procedure for the service capability is written as an Diagram in a time order and step-by-step. The Originator and what the Originator carries out (a task, function, accessing to resources or other services) are defined in each step.

### X.1.4	Post-Conditions

<Document the post-conditions of the <serviceCapabilityName>. If the service capability has no defined output that can be validated except a notification of success (or failure) to set a value in the target of the service capability, a testable post condition of the service capability is documented. Post-conditions are documented that are in addition to that provided by the internal resource operations and procedures.>

### X.1.5	Exceptions

< Document the exception or error conditions that may be returned by the <serviceCapabilityName>. Exceptions are documented that are in addition to that provided by the internal resource operations and procedures.>

### X.1.6	Policies for Use

Message Exchange Patterns : See section 5.2.2.1Message Exchange Patterns for allowable patterns

Transaction Pattern:

- Creates Transaction: This service capability starts and completes a transaction

- Participation allowed: This service capability is able to participate in a transaction without violating the ongoing transaction’s properties (i.e. Atomicity, Consistency, Isolation, Durability)

- Participation not allowed: This service capability is unable to participate in a transaction

Maximum Response: The maximum time in milliseconds permitted to complete this service capability.

### X.1.7	oneM2M Resource Interworking

<Document the related oneM2M Resources operations to <serviceCapabilityName> here. Interworking rules are defined for each Resource operation. If there is no related internal resource, this resource list can be empty (e.g., the service capability is ‘sendTrigger’).>.

# Annex Y(Informative): oneM2M Service Requirements

The primary motivation of this technical specification is to integrate oneM2M Service capabilities with existing M2M deployments without affecting that existing M2M solution’s underlying deployment framework and protocols. For example if an existing M2M solution uses a XMPP based framework to communicate between applications in the network and devices, then M2M Service capabilities are efficiently exposed to the XMPP based framework without requiring modifications to the existing M2M solution’s deployment framework or protocol. A secondary motivation for this technical specification is to offer M2M Service capabilities to M2M service provider’s who utilize service oriented architectural frameworks.

This annex defines requirements that have not been captured in the oneM2M Requirements [1].

| Requirement ID | Description |
| --- | --- |
| SCA-001 | The M2M System shall be capable of integrating oneM2M service capabilities with existing M2M deployments without affecting the existing M2M solution’s underlying deployment framework and protocols. |
| SCA-002 | The M2M System shall be capable of offering oneM2M service capabilities to SOA architectural frameworks. |

Annex <y>:
Bibliography

The annex entitled “Bibliography” is optional.

It shall contain a list of standards, books, articles, or other sources on a particular subject which are not mentioned in the document itself

It shall not include references mentioned in the document.

Use the Heading 9 style for the title and B1+ or Normal for the text.

<Publication>: “<Title>”.

OR

<Publication>: “<Title>”.

<PAGE BREAK>

# History

This clause shall be the last one in the document and list the main phases (all additional information will be removed at the publication stage).

| Publication history |  |  |
| --- | --- | --- |
| V1.1.1 | <dd-Mmm-yyyy> | <Milestone> |

| Draft history (to be removed on publication) |  |  |
| --- | --- | --- |
| V0.0.1 | 19-Feb-2014 | Rapporteur Input – Draft Skeleton |
| V.0.1.0 | 22-Feb-2014 | Added agreed contributions from TP9 ARC-2014-0081R02 |
| V.0.2.0 | 15-May-2014 | Added agreed contributions from TP10 and interim conference calls ARC-2014-1259-TS-0007-Reference_Update ARC-2014-1260R02-TS-0007-Mca-Request_Services ARC-2014-1324-TS-0007_Attribute_Alignment ARC-2014-1261R02-TS-0007-Mca-Sub-Pub-Notify |
| V.0.3.0 | 17-June-2014 | Added agreed contributions from TP11 ARC-2014-1390-TS-0007_IN-OUT_DeSignation ARC-2014-1391-TS-0007_Alignment_with_TS-0001_terms ARC-2014-1398-TS-0007_Clause-Role-Interworking-Removal ARC-2014-1401R02-TS-0007_Response_Code_Cleanup ARC-2014-1404-TS-0007-Service_Adminstration ARC-2014-1405-TS-0007-Service_Subscription_Adminstration |
| V.0.4.0 | 04-August-2014 | Added agreed contributions from TP12 ARC-2014-1411R02-TS-0007-Mca_Device_Firmware_management ARC-2014-1473R01-TS-0007-Mca_Device_Configuration_service ARC-2014-1474R01-TS-0007-Mca_Device_Topology_Management_service |
| V.0.5.0 | 01-October-2014 | Added agreed contributions from TP12 conference calls ARC-2014-1519R01-TS-0007-Request-Response-Data-Exchange-Pattern Added agreed contributions from TP13 Updated Normative Reference text ARC-2014-1534-TS-0007-Annex-A-Rework ARC-2014-1523R03-TS-0007-Device-Onboarding ARC-2014-1548R02-TS_0007_Event_Collection_service ARC-2014-1554-TS-0007-Section_7_DM_update ARC-2014-1560R01-TS-0007-Mca_Device_Troubleshooting_service ARC-2014-1555-TS-0007-Mca_Application_Software_Management_Service ARC-2014-1540R01-TS0007-Registration-Service ARC-2014-1541R01-TS0007-Supporting-Service-Registration-Services |
| V.0.5.1 | 17-October-2014 | Added agreed contributions from TP13 conference calls ARC-2014-1600-TS-0007-_ServiceCapabilities_CommonRequest_Alignment |
| V.0.6.0 | 24-November-2014 | Added agreed contributions from TP13.4 conference call (email) and TP14 ARC-2014-1615-TS-0007-M2M_Service_Capability_Alignment ARC-2014-1643-TS-0007-downloadFirmware&installFirmware_update ARC-2014-1644R01-TS-0007-Section_6_getManagementAdapter_update |
| V.0.6.1 | 05-January-2015 | Added agreed contributions from TP14.2 conference call ARC-2014-1688R02-TS-0007-Service_Subscription_Alignment ARC-2014-1689R06-TS0007-Registration_Alignment_with_Service_Subscription |
| V0.7.0 | 27-January-2015 | Added agreed contributions from TP15 plenary ARC-2015-0001-Msc-TS0007-Common-Request-Parameters ARC-2015-0002-TS-0007-Section_3_5_Editor_Notes_Cleanup ARC-2015-0004R02-TS_0007_clean_up_-_event_collection_filterCriteria ARC-2015-0008R04-TS-0007-DM_editor_notes_cleanup |

| V0.8.0 | 30-March-2015 | Added agreed contributions from TP16 plenary ARC-2015-1737-TS-0007-Non-DM-MSC-Notation_correction ARC-2015-1742-TS-0007-DM-MSC-Notation_correction ARC-2015-1743-TS-0007-DM_editor_note_cleanup |
| --- | --- | --- |