	ARC-2016-0335R01-EventConfig_attributes
	[image: image3.png]






	


	CHANGE REQUEST

	Meeting:*
	TP24

	Source:*
	InterDigital

	Date:*
	2016-07-11

	Contact:*
	Dale Seed (dale.seed@interdital.com)

Bob Flynn (bob.flynn@interdigital.com)

Bhargavi Nagaraja Rao (BhargaviNagarajaRao.Chanakesapura@InterDigital.com )

	Reason for Change/s:*
	Providing more flexibility and granularity for configuration of Charging Events.

	CR  against:  Release*
	Rel-3, track 1

	CR  against:  WI*
	 FORMCHECKBOX 
 Active <Work Item number>  
 FORMCHECKBOX 
 MNT Maintenace / < Work Item number(optional)>
 FORMCHECKBOX 
 STE Small Technical Enhancements / < Work Item number (optional)>

Only ONE of the above shall be ticked

	CR  against:  TS/TR*
	TS-0001-V2.9.0

	Clauses/Sub Clauses*
	9.6.24 and 10.2.15.6

	Type of change: *
	 FORMCHECKBOX 
 Editorial change

 FORMCHECKBOX 
 Bug Fix or Correction

 FORMCHECKBOX 
 Change to existing feature or functionality

 FORMCHECKBOX 
 New feature or functionality
Only ONE of the above shall be ticked

	Post Freeze checking:*
	This CR contains only essential changes and corrections?  YES  FORMCHECKBOX 
  NO  FORMCHECKBOX 

This CR is a mirror CR? YES  FORMCHECKBOX 
   NO  FORMCHECKBOX 
  if YES, please indicate the document number of the original CR: 
<Document Number)<CR Number of the original CR to the current Release>

	Template Version:23 February 2015 (Dot not modify)


oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.
Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.
Follow the drafting rules.
All pictures must be editable.
Check spelling and grammar to the extent practicable.
Use Change bars for modifications.
The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete sections need not show surrounding clauses as long as the proposed section number clearly shows where the new section is proposed to be located.
Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.
When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text. 
Introduction
This STE CR proposes to add a three optional attributes  to the <eventConfig> resource defined in clause 9.6.24 of  TS0001 v2.9.0 :
1. “eventResourceTypes” which shall indicate list of types of resource for which the charging event is supposed to be captured and reported. This could be used to differentiate the same operation on different types of information that triggers the charging activity. 

2. “eventResourceIDs” which shall indicate the list of resourceIDs for which an event is to be captured and reported.

3. “locationRestriction” shall be used to restrict the event to a specific location or a certain area. This attribute holds the ID of a <locationPolicy> resource.
In addition to the existing operationType,  eventStart, eventEnd and dataSize attributes used for defining an event, these attributes define a list of conditions/rules based on which an event information will be collected. This will provide more flexibility and granularity for configuring charging event. 
----------------------- Start of change 1 in TS0001 -----------------------
9.6.24
Resource Type eventConfig

<eventConfig> sub-resource shall be used to define events that trigger statistics collection. Below are some examples of events that can be generated:

· Collection based on a certain operation: collects any RETRIEVE operations on the data (i.e. resources) stored in the IN-CSE.

· Collection based on storage size: collects the size of storage when a <container> resource stored in the IN-CSE exceeds a quota.

· Combined configuration: collects all RETRIEVE operations on the data stored in the IN-CSE during a period of time.


[image: image2.emf]<


eventConfig


>


1


eventID


1


eventType


0


..


1


eventStart


0


..


1


eventEnd


0


..


1


dataSize


0


..


1 


(


L


)


operation


Type


<


subscription


>


0


..


n


0


..


1


locationRestriction


0


..


1 


(


L


)


eventResourceTypes


0


..


1 


(


L


)


eventResourceIDs




<eventConfig>

1

eventID

1

eventType

0..1

eventStart

0..1

eventEnd

0..1

dataSize

0..1 (L)

operationType

<subscription>

0..n

0..1

locationRestriction

0..1 (L)

eventResourceTypes

0..1 (L)

eventResourceIDs


Figure 9.6.24-1: Structure of <eventConfig> resource

The <eventConfig> resource shall contain the child resource specified in table 9.6.24-1.

Table 9.6.24-1: Child resources of <eventConfig> resource

	Child Resources of <eventConfig>
	Child Resource Type
	Multiplicity
	Description

	[variable]
	<subscription>
	0..n
	See clause 9.6.8 where this type of resource is described.


The <eventConfig> resource shall contain the attributes specified in table 9.6.24-2.

Table 9.6.24-2: Attributes of <eventConfig> resource

	Attributes of <eventConfig>
	Multiplicity
	RW/

RO/

WO
	Description

	resourceType
	1
	RO
	See clause 9.6.1.3.

	resourceID
	1
	RO
	See clause 9.6.1.3.

	resourceName
	1
	WO
	See clause 9.6.1.3.

	parentID
	1
	RO
	See clause 9.6.1.3.

	accessControlPolicyIDs
	1 (L)
	RW
	See clause 9.6.1.3.

	creationTime
	1
	RO
	See clause 9.6.1.3.

	expirationTime
	1
	RW
	See clause 9.6.1.3.

	lastModifiedTime
	1
	RO
	See clause 9.6.1.3.

	labels
	0..1 (L)
	RW
	See clause 9.6.1.3.

	dynamicAuthorizationConsultationIDs
	0..1 (L)
	RW
	See clause 9.6.1.3.

	creator
	0..1
	RO
	 See clause 9.6.1.3.

	eventID
	1
	RO
	This attribute uniquely identifies the event to be collected for statistics for AEs.

	eventType
	1
	RW
	This attribute indicates the type of the event, such as timer based, data operation, storage based, etc. 

	eventStart
	0..1
	RW
	This attribute indicates the start time of the event.

	eventEnd
	0..1
	RW
	This attribute indicates the end time of the event

	operationType
	0..1 (L)
	RW
	This attribute defines the type of the operation to be collected by statistics, such as CREATE, RETRIEVE. 

	dataSize
	0..1
	RW
	This attribute defines the data size if an event is triggered when the stored data exceeds a certain size. 

	eventResourceTypes
	0..1 (L)
	RW
	This attribute indicates the list of types of resource for which an event is to be captured and reported. This could be used to differentiate the same operation on different types of resources that triggers the charging activity.

	eventResourceIDs
	0..1 (L)
	RW
	This attribute indicates the list of resourceIDs for which the event is to be captures and reported. Whenever an operation is performed on the resourceIDs in this list, event will be recorded provided other event criterias are met such as eventResourceType, locationRestriction and the event information based on the type of event.

	
	
	
	



----------------------- End of change 1 in TS0001-----------------------
----------------------- Start of change 2 in TS0001 -----------------------
10.2.15.6
Create <eventConfig>
This procedure shall be used to create <eventConfig> resource.

Table 10.2.15.6-1: <eventConfig> CREATE

	<eventConfig> CREATE

	Associated Reference Points
	Mca

	Information in Request message
	From: Identifier of the AE that initiates the Request

To: The address of the <statsConfig> resource where the <eventConfig> sub‑resource is intended to be Created

Content: The representation of the <eventConfig> resource for which the attributes are described in clause 9.6.24
Other information in the Request message is defined according to clause 10.1.1.1

	Processing at Originator before sending Request
	The Originator shall be an AE. The Originator shall request to Create a new <eventConfig>  resource by addressing to the <statsConfig> resource of a Hosting CSE

	Processing at Receiver
	The Receiver shall be an IN-CSE:

The Receiver shall verify whether the eventID is unique or not, and if not, provides a new value
The Receiver shall verify that the eventEnd time is greater than the eventStart time if these two attributes are present
The Receiver shall verify that the dataSize attribute is present and contains a value greater to zero if the eventType is set to "Storage based"
When eventType is set to “StorageBased” and if the eventResourceType attribute is empty or is set to any other value other than container or contentInstance, by default the eventResourceType will be set to contentInstance.

	Information in Response message
	According to clause 10.1.1.1

	Processing at Originator after receiving Response 
	None

	Exceptions
	According to clause 10.1.1.1


----------------------- End of change 2 in TS0001-----------------------
CHECK LIST

· Does this change request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?
· Does this CR contain changes related to only one particular issue/problem?
· Does this change request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable?
· Does this change request follow the drafting rules?
· Are all pictures editable?
· Have you checked the spelling and grammar?
· Have you used change bars for all modifications?
· Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete sections need not show surrounding clauses as long as the proposed section number clearly shows where the new section is proposed to be located.)
· Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?
© 2016 oneM2M Partners
                                                                                                   Page 1 (of 6)



[image: image3.png]_1524138131.vsd

