| oneM2M Technical Report |  |
| --- | --- |
| Document Number | oneM2M-TR-0006-Study_of_Management_Capability_Enablement-V0_4_0 |
| Document Name: | Study of Management Capability Enablement Technologies for Consideration by oneM2M |
| Date: | 2013-Aug-16 |
| Abstract: | Collect and describe the state-of-the-art technologies (e.g. OMA DM and BBF TR-069) that are relevant to oneM2M management capabilities. Analyze the collected technologies to match with the oneM2M requirements on management aspects; Evaluate the possibility of leveraging all or part of those technologies by oneM2M to enable its management capability. |

oneM2M IPR and copyright statements

“Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of an agreement to be bound by all provisions of IPR policy of the admitting Partner Type 1 and permission that all communications and statements, oral or written, or other information disclosed or presented, and any translation or derivative thereof, may without compensation, and to the extent such participant or attendee may legally and freely grant such copyright rights, be distributed, published, and posted on oneM2M’s web site, in whole or in part, on a non-exclusive basis by oneM2M or oneM2M Partners Type 1 or their licensees or assignees, or as oneM2M SC directs.

# Contents

Contents	2

2	References	4

2.1	Normative references	4

2.2	Informative references	4

3	Definitions, symbols, abbreviations  and acronyms	5

3.1	Definitions	5

3.2	Symbols	6

3.3	Abbreviations	6

3.4	Acronyms	6

5	Introduction of existing technologies	7

5.2	TR-069 Family of Specifications	7

5.2.1	Description	7

5.2.2	Architecture	7

5.2.2.1	TR-069 Proxy Management	8

5.2.2.1.1	Proxied Device Deployment Archirecture	8

5.2.3	Reference points	9

5.2.4	Protocols	10

5.2.4.1	ACS to CPE Protocol	10

5.2.4.2	CPE to BSS	11

5.2.4.2.1	IPDR Reference Points	12

5.2.4.3	CPE to Device Protocol	12

5.2.4.3.1	UPnP DM Proxy	12

5.2.4.3.2	ZigBee Proxy	13

5.2.5	Functions	15

6	Gap analysis of existing relevant technolgoies	15

6.1	Management related requirements gap analysis reference	15

6.2	MGR-001	16

6.2.1	Requirement Description	16

6.2.2	OMA DM 1.3	16

6.2.3	BBF TR-069	17

6.2.4	OMA DM LWM2M	17

6.2.5	OMA DM 2.0	17

6.3	MGR-002	18

6.3.1	Requirement Description	18

6.3.2	OMA DM 1.3 and OMA DM 2.0	18

6.3.3	BBF TR-069	18

6.3.4	OMA DM LWM2M	19

6.4	MGR-003	19

6.4.1	Requirement Description	19

6.4.2	OMA DM 1.3 and OMA DM 2.0	19

6.4.3	BBF TR-069	19

6.4.4	OMA DM LWM2M	19

6.5	MGR-004	20

6.5.1	Requirement Description	20

6.5.2	OMA DM 1.3 and OMA DM 2.0	20

6.5.3	BBF TR-069	20

6.5.4	OMA DM LWM2M	20

6.6	MGR-005	20

6.6.1	Requirement Description	20

6.6.2	OMA DM 1.3 and OMA DM 2.0	20

6.6.3	BBF TR-069	20

6.6.4	OMA DM LWM2M	21

6.7	MGR-006	21

6.7.1	Requirement Description	21

6.7.2	OMA DM 1.3 and OMA DM 2.0	21

6.7.3	BBF TR-069	21

6.7.4	OMA DM LWM2M	21

6.8	MGR-007	21

6.8.1	Requirement Description	21

6.8.2	OMA DM 1.3 and OMA DM 2.0	21

6.8.3	BBF TR-069	22

6.8.4	OMA DM LWM2M	22

6.9	MGR-008	22

6.9.1	Requirement Description	22

6.9.2	OMA DM 1.3 and OMA DM 2.0	22

6.9.3	BBF TR-069	23

6.9.4	OMA DM LWM2M	23

6.10	MGR-009	23

6.10.1	Requirement Description	23

6.10.2	OMA DM 1.3 and OMA DM 2.0	23

6.10.3	BBF TR-069	23

6.10.4	OMA DM LWM2M	23

6.11	MGR-010	23

6.11.1	Requirement Description	23

6.11.2	OMA DM 1.3 and OMA DM 2.0	23

6.11.3	BBF TR-069	24

6.11.4	OMA DM LWM2M	24

6.12	MGR-011	24

6.12.1	Requirement Description	24

6.12.2	OMA DM 1.3 and OMA DM 2.0	24

6.12.3	BBF TR-069	24

6.12.4	OMA DM LWM2M	24

6.13	MGR-012	24

6.13.1	Requirement Description	24

6.13.2	OMA DM 1.3 and OMA DM 2.0	24

6.13.3	BBF TR-069	25

6.13.4	OMA DM LWM2M	25

6.14	MGR-013	25

6.14.1	Requirement Description	25

6.14.2	OMA DM 1.3 and OMA DM 2.0	25

6.14.3	BBF TR-069	25

6.14.4	OMA DM LWM2M	25

6.15	MGR-014	26

6.15.1	Requirement Description	26

6.15.2	OMA DM 1.3 and OMA DM 2.0	26

6.15.3	BBF TR-069	26

6.15.4	OMA DM LWM2M	26

6.16	MGR-015	26

6.16.1	Requirement Description	26

6.16.2	OMA DM 1.3 and OMA DM 2.0	26

6.16.3	BBF TR-069	26

6.16.4	OMA DM LWM2M	27

6.17	MGR-016	27

6.17.1	Requirement Description	27

6.17.2	OMA DM 1.3 and OMA DM 2.0	27

6.17.3	BBF TR-069	27

6.17.4	OMA DM LWM2M	27

# 2	References

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

## 2.1	Normative references

Not applicable.

## 2.2	Informative references

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

Use the EX style, add the letter "i" (for informative) before the number (which shall be in square brackets) and separate this from the title with a tab (you may use sequence fields for automatically numbering references, (see example).

[i.1]	oneM2M Drafting Rules  (http://member.onem2m.org/Static_pages/Others/Rules_Pages/oneM2M-Drafting-Rules-V1_0.doc)

[i.2]	OMA-AD-DM-V1_3 “Device Management Architecture”

[i.3]	OMA-TS-DM_Protocol-V1_3 “OMA Device Management Protocol”

[i.4]	OMA-TS-DM_RepPro-V1_3 “OMA Device Management Representation Protocol”

[i.5]	OMA-TS-DM_StdObj-V1_3 “OMA Device Management Standardized Objects”

[i.6]	OMA-TS-DCMO-V1-0: “Device Capability Management Object”

[i.7]	OMA-TS-LAWMO-V1-0: “Lock and Wipe Management Object”

[i.8]	OMA-TS-DM-FUMO-V1-0: “Firmware Update Management Object”,.

[i.9]	OMA-TS-DM-SCOMO-V1-0: “Software Component Management Object”, Version 1.0.

[i.10]	OMA-TS-GwMO-V1-0: “Gateway Management Object Technical Specification”, Version 1.0.

[i.11]	OMA-TS-DiagMonFunctions-1-0: “DiagMon Functions Supplemental Specification”, Version 1.0.

[i.12]	OMA-AD-GwMO-V1_1-20130214-D “Gateway Management Object Architecture”

[i.13]	BBF TR-069 CPE WAN Management Protocol Issue: 1 Amendment 4, July 2011

[i.14]	BBF MR-239 Broadband Forum Value Proposition for Connected Home  Issue: 1, April 2011

[i.15]	BBF TR-232 Bulk Data Collection  Issue: 1, May 2012

[i.16]	TMForum IPDR Service Specification Design Guide, Version 3.8, Release 1.0, 2009

[i.17]	ZigBee Alliance ZigBee Specification: ZigBee Documents 053474r17, 2008

[i.18]	OMA-RD-LightweightM2M-V1_0”OMA Lightweight Machine to Machine Requirement”

[i.19]	OMA-AD-LightweightM2M-V1_0 “OMA Lightweight Machine to Machine Architecture”

[i.20]	OMA-TS-LightweightM2M-V1_0 “OMA Lightweight Machine to Machine Protocol” (work on progress)

[i.21]	Shelby, Z., Hartke, K., Bormann, C., and B. Frank, "Constrained Application Protocol (CoAP)", draft-ietf-core-coap-14 (work in progress), Sept 2012.

[i.22]	Rescorla, E. and N. Modadugu, "Datagram Transport Layer Security Version 1.2", RFC 6347, January 2012.

[i.23]	oneM2M-TS-0002-Requirements-V0_4_0

[i.24]	ETSI M2M TS 103 092 OMA DM compatible Management Objects for ETSI M2M v2.1.1

[i.25]	“Device Management Requirements”, Version 2.0, Open Mobile Alliance™, OMA-RD-DM-V2_0, http://www.openmobilealliance.org/

[i.26]	“Device Management Architecture”, Version 2.0, Open Mobile Alliance™, OMA-AD-DM-V2_0, http://www.openmobilealliance.org/

[i.27]	“OMA Device Management Protocol”, Version 2.0, Open Mobile Alliance™, OMA-TS-DM-V2_0, http://www.openmobilealliance.org/

[i.28]	“Enabler Release Definition for Firmware Update Management Object”, Version 1.0, Open Mobile Alliance™, OMA-ERELD-FUMO-V1_0, http://www.openmobilealliance.org/

[i.29]	“DM Client Side API Framework (DMClientAPIfw)” OMA-ER-DMClientAPIfw-V1_0

[i.30]	C. Bormann and M. Ersue, “Terminology for Constrained Node Networks”, draft-ietf-lwig-terminology-05, July 09 2012.

[i.31]	A. Sehgal, V. Perelman, S Kuryla and J Schonwalder, “Management of Resource Constrained Devices in the Internet of Things”, IEEE Communication Magazine (Vol.50, Issue.12), Dec 2012.

[i.32]	BBF TR-143 Enabling Network Throughput Performance Tests and Statistical Monitoring, Corrigendum 1, December 2008.

[i.33]	BBF TR-131 ACS Northbound Interface Requirements, November 2009.

[i.34]	BBF TR-181 Device Data Model for TR-069, Issue 2 Amendment 6, November 2012.

[i.35]	BBF TR-135 Device Data Model for TR-069 Enabled STB, Amendment 3, November 2012.

[i.36]	BBF TR-104 Provisioning Parameters for VoIP CPE, September 2005.

[i.37]	BBF TR-196 Femto Access Point Service Data Model, Issue 2, November 2012.

[i.38]	BBF TR-140 TR-069 Data Model for Storage Service Enabled Devices, Issue 1.1, December 2007.

# 3	Definitions, symbols, abbreviations  and acronyms

## 3.1	Definitions

For the purposes of the present document, the [following] terms and definitions [given in ... and the following] apply:

mc: The interface between the management server and the management client. This interface can be realized by the existing device management technologies such as BBF TR-069, OMA DM, etc.

ms: The interface between the management adapter and the management server in the underlying network domain or in the M2M service domain for use by other systems. Using this interface, systems can perform management operations on devices through the management server.

mp: The interface that is exposed by the proxy management client in the area network for devices that connect to a Proxy. This interface is realized by existing LAN based protocols (e.g., ZigBee, UPnP) as well as existing device management technologies (e.g., OMA-DM).

<defined term>: <definition>

<defined term>[N]: <definition>

example 1: text used to clarify abstract rules by applying them literally

NOTE:	This may contain additional information.

## 3.2	Symbols

For the purposes of the present document, the [following] symbols [given in ... and the following] apply:

<symbol>	<Explanation>

<2nd symbol>	<2nd Explanation>

<3rd symbol>	<3rd Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the [following] abbreviations [given in ... and the following] apply:

## 3.4	Acronyms

For the purposes of the present document, the [following] abbreviations [given in ... and the following] apply:

ACS	Auto-Configuration Server

BBF	Broadband Forum

BSS	Business Support System

CoAP	Constrained Application Protocol

CPE	Customer Premises Equipment

CPU	Centralized Processing Unit

CWMP	CPE WAN Management Protocol

DM	Device Management

DTLS	Datagram Transport Layer Security

FTP	File Transfer Protocol

GW	Gateway

HTTP	Hypertext Transfer Protocol

IP	Internet Protocol

IPDR	Internet Protocol Detail Record

IrDA	Infrared Data Association

MO	Management Object

OBEX	OBject EXchange

OMA	Open Mobile Alliance

OSS 	Operation Support System

OTA	Over The Air

PAN	Personal Area Network

RPC 	Remote Procedure Call

SCTP	Stream Control Transmission Protocol

SE	Service Element

SIP	Session Initiation Protocol

SOAP	Simple Object Access Protocol

SMS	Short Message Service

SSL	Secure Session Layer

TCP	Transmission Control Protocol

TMForum	Telemanagement Forum

TR	Technical Report

TLS	Transport Layer Security

UDP	User Datagram Protocol

UPnP DM	Universial Plug and Play Device Management

UI	User Interaction

WAP	Wireless Application Protocol

WSP	Wireless Session Protocol

XDR	External Data Representation

XML	Extensible Markup Language

ZC	ZigBee Coordinator

ZDO	ZigBee Device Object

ZED	ZigBee End Device

ZR	ZigBee Router

# 5	Introduction of existing technologies

## 5.2	TR-069 Family of Specifications

### 5.2.1	Description

The Broadband Forum has developed a series of specifications that have been termed the TR-069 Family of Specifications. These specifications provides the capability to manage CPEs within the connected home. MR-239 Broadband Forum Value Proposition for Connected Home [i.14] provides an overview of the value proposition for utilizing the TR-069 family of specifications for the connected home.

### 5.2.2	Architecture

The TR-069 family of specifications is anchored by the TR-069 [i.13] specification for the CPE WAN Management Protocol (CWMP), TR-143 [i.32] for diagnostic tests. TR-131 [i.33] for requirements related to the ACS North Bound Interface.

In addition Service Providers are increasingly interested in retrieving large quantities of data from their installed CPE base at regular intervals. The amount of data being requested represents a significant portion of the CPE’s data model and is thus a large amount of data. In response to this, the Broadband Forum has documented a data collection solution in TR-232 Bulk Data Collection [i.15]. This specification is based on the IPDR protocol from the TMForum.

Devices within the Connected Home are managed via a set of data models for CWMP Enabled Devices. These data models are anchored by TR-181i2 [i.34] which defines the objects and attributes for management for most capabilities offered by a device (e.g., physical interfaces, bridging and routing, firewalls, NAT, software modules). Likewise services and capabilities specific to a type of device are included in a separate set of specifications. For example, CWMP enabled STB are managed using TR-135 [i.35]; Femto cell devices are managed using TR-196 [i.37]; VoIP capable devices are managed using TR-104 [i.36]. Not all devices within the Connected Home are CWMP enabled; in this situation TR-069 provides the capability for a CWMP enabled device to act a proxy for the device.

Figure 5.2.1: TR-069 Family of Specifications

#### 5.2.2.1	TR-069 Proxy Management

CWMP can be extended to devices that do not have a native CWMP Endpoint of their own, but instead support management of devices with another management protocol or “Proxy Protocol”.  A CPE Proxier is a CPE that supports a CWMP Endpoint(s) and also supports one or more Proxy Protocols (example services include UPnP DM, Z-Wave etc.). A CPE Proxier uses these Proxy Protocols to manage the devices connected to it, i.e. the Proxied Devices. This approach is designed to support Proxy Protocols of all types that can exist in the CPE network now or in the future. Annex J of the TR-069 [i.13] provides an overview of CWMP Proxy Management.

Figure 5.2.3: Proxy management terminology

##### 5.2.2.1.1	Proxied Device Deployment Archirecture

Figure 5.2.4: TR-069 UPnP DM Proxied Device depicts an example scenario where a proxied device that supports the UPnP DM protocol is managed using CWMP.

Figure 5.2.4: TR-069 UPnP DM Proxied Device

The entities include the Service Provider OSS/BSS systems that interface with the ACS (1); the CWMP and IPDR protocols between the ACS and the TR-069 enabled CPE (2) and the Home Area Network protocol UPnP DM (3).

### 5.2.3	Reference points

### The CWMP enabled devices in the Connected Home typically communicates with three (3) entities, the ACS, OSS/BSS and devices within the Connected Home via standardized reference points.

### These references points are defined as:

### ACS to CPE

### CPE to BSS

### CPE to Device

Figure 5.2.3: TR-069 Reference Points

### 5.2.4	Protocols

#### 5.2.4.1	ACS to CPE Protocol

The protocol that is supported on the ACS to CPE reference point is CWMP as defined in  BBF TR-069 [i.13]. CWMP takes a layered approach to the protocol based on several standard protocols for transport and exchange of messages. The protocol stack defined by CWMP is shown in Figure 5.2.5. A brief description of each layer is provided in Table 5.2.1.

Figure 5.2.5: Protocol stack

| Layer | Description |
| --- | --- |
| CPE/ACS Application | The application uses the CPE WAN Management Protocol on the CPE and ACS, respectively. The application is locally defined and not specified as part of the CPE WAN Management Protocol. |
| RPC Methods | The specific RPC methods that are defined by the CPE WAN Management Protocol. These methods are specified in CPE WAN Management Protocol. |
| SOAP | A standard XML-based syntax used here to encode remote procedure calls. Specifically SOAP 1.1, as specified in Simple Object Access Protocol (SOAP) 1.1. |
| HTTP | HTTP 1.1, as specified in RFC 2616, Hypertext Transfer Protocol -- HTTP/1. |
| TLS | The standard Internet transport layer security protocol. Specifically, TLS 1.2 (Transport Layer Security) as defined in RFC 5246, The Transport Layer Security (TLS) Protocol, Version 1.2 (or a later version). Note that previous versions of this specification referenced SSL 3.0 and TLS 1.0. |
| TCP/IP | Standard TCP/IP. |

Table 5.2.1: Protocol layer summary

#### 5.2.4.2	CPE to BSS

The protocol that is supported on the CPE to BSS reference point is the IPDR protocol. The IPDR reference architecture is presented in Figure 5.2.6 is defined in TMForum IPDR Service Specification Design Guide [i.16]. The figure depicts a Service Element communicating to an IPDR Recorder that sends messages to the IPDR Transmitter and optionally to an IPDR Store. TR-232 [i.15] utilizes the A and D interfaces of this specification where the Service Element is a device within the Connected Home.

Figure 5.2.6: IPDR Reference Architecture

From the perspective of the Broadband Forum, the CPE or device is the Service Element and IPDR Exporter. The IPDR Data Collector is the BSS.  As described in Annex A IPDR Theory of Operaton of TR-232 [i.15], the IPDR documentation clarifies that the following scenario, where the Service Element directly communicates to the BSS, is valid and simply means that the IPDR Recorder and IPDR Transmitter (collectively the IPDR Exporter in this use case) are all incorporated into the Service Element. The Service Element is permitted to directly interface with the BSS if it supports the “D” interface specifications including backing stores and retransmission of IPDR documents.

Figure 5.2.7: Simplified IPDR Architecture

##### 5.2.4.2.1	IPDR Reference Points

TR-232 [i.15] defines 6 interfaces and 4 defintions for the IPDR Reference Model:

| Interface | Description |
| --- | --- |
| A | Vendor proprietary. High-volume with high granularity void of context. This interface is not part of the IPDR Protocol. |
| B | IPDR Data Interface. From IPDR Recorders to IPDR Stores or IPDR Transmitters. |
| C | IPDR Store Export Interface. |
| D | BSS Interface. XML or XDR data from IPDR Exporter to IPDR Collector |
| E | Settlement Interface. Connects Service Delivery Business Management Systems. |
| F | Financial System Interface. This interface is not part of the IPDR Protocol. |

Table 5.2.2: IPDR Interfaces

The IPDR File Transfer Protocol uses FTP or HTTP to transfer files that contain IPDR records from the SE to the BSS.  The IPDR Streaming Protocol uses SCTP or TCP to transfer IPDR records from the SE to the BSS using highly efficient XDR encoding as described in the IPDR/XDR Encoding Format document or an XML encoding as described in the IPDR/XML File Encoding Format document.

#### 5.2.4.3	CPE to Device Protocol

The TR-069 proxy mechanism is designed to incorporate any protocol for area networks within the customer premises. The following protocols have been standardized or are currently in development:

UPnP DM

ZigBee

##### 5.2.4.3.1	UPnP DM Proxy

The CPE Proxier consists of three logical modules: CWMP client, TR-069/UPnP DM Proxy Module and UPnP DM Control Point. CWMP requests received by the CWMP client from the ACS are translated by the TR-069/UPnP DM Proxy Module to the UPnP DM actions, and then passed to the UPnP DM Control Point to be sent to the UPnP DM devices. When an UPnP action response or event is received by the UPnP DM Control Point, the action response and event is passed to the TR-069/UPnP DM Proxy Module to be converted to a CWMP response or sent to the ACS using the CWMP event notification mechanism.

Figure 5.2.8: TR-069/UPnP DM Proxy Management Architecture

##### 5.2.4.3.2	ZigBee Proxy

Figure 5.2.9 and Figure 5.2.10 present the principle and an example basic sequence for the management of ZigBee devices by using TR-069 with the ZigBee data model.

The ZigBee devices reside behind a GW and communicate with the ACS via this GW. The GW resides normally in a CPE such as a broadband router (home gateway or business gateway). The GW has a proxy function to change a CWMP message to a ZDO function invocation based on the ZigBee data model object. The proxy function changes messages by referring to a mapping of ZigBee data model objects and CWMP methods to ZDO functions and their parameters. A management example is shown in Figure 5.2.10.

Figure 5.2.9: Usage of the data model to manage ZigBee devices with TR-069

Figure 5.2.10: Example sequence diagram of ZigBee management with TR-069

This example shows how the ACS gets a ZigBee device’s network address by using TR-069 communication based on the ZigBee data model. The ACS sends a CWMP message which includes the “GetParameterValues” as a method and the part of the ZigBee data model “Device.ZigBee.ZDO.{i}.NetworkAddress”, which refers to the network address, as a parameter name. The proxy function in the GW changes the received message to a ZDO handling message to call some ZDO function on the ZC. The ZC manages the ZigBee devices according to the called ZDO function and sends the result (the searched network address, in this case) to the proxy. The proxy function changes the ZDO management result to a CWMP message which is denoted in Figure 5.2.10 as “GetParameterValuesResponse”. The name of the parameter list is “Device.ZigBee.ZDO.{i}.NetworkAddress” and the value of the parameter list is “0x0fE3” (network address instance).

### 5.2.5	Functions

The TR-069 family of specifications is intended to support a variety of functionalities to manage a collection of devices, including the following primary capabilities:

Auto-configuration and dynamic service provisioning

Software/firmware image management

Software module management

Status and performance monitoring

Diagnostics

Proxy Management

Bulk data collection

# 6	Gap analysis of existing relevant technolgoies

Editor’s Note: The chapter lists all the requirements related to management in oneM2M and analyzes how the technologies can fulfil the requirements and identifies potential gaps. The chapter also gives reference to the specification of the analyzed technologies.

## 6.1	Management related requirements gap analysis reference

Editor’s Note: The table below gives cross reference to the requirements and the technologies. The matrix represents whether or not the technology can fulfil the requirement. At the same time, in the following chapters, detailed description of the depth of how well the technology can fulfil the requirement should be shown.

Some MGR requirements (e.g., MGR-004) target functionality outside the scope of the existing technology (e.g., TR-069, OMA-DM). In those cases the analysis of the technology against the requirement determines if the the functionality that is outside the scope of the technology is allowed.

The definitions for the values in below table are:

FULL: the requirement can be fulfilled by the technology alone

PARTIAL: the requirement can be partially fulfilled by the technology

ALLOWED: Adopting this technology will allow this requirement to be implemented

NOT ALLOWED: This technology does not fulfill the requirement AND adopting this technology would not allow the requirement to be implemented

| Requirement Support |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | OMA DM 1.3 | BBF TR-069 | OMA DM LWM2M | OMA DM 2.0 |
| MGR-001 | Partial | Partial | Full | Partial |
| MGR-002 | Full | Full | Full | Full |
| MGR-003 | Full | Full | Full | Full |
| MGR-004 | Allowed | Allowed | Allowed | Allowed |
| MGR-005 | Partial | Partial | Partial | Partial |
| MGR-006 | Full | Full | Full | Full |
| MGR-007 | Full | Full | Full | Full |
| MGR-008 | Full | Full | Partial | Full |
| MGR-009 | Full | Full | Full | Full |
| MGR-010 | Partial | Partial | Partial | Partial |
| MGR-011 | Full | Full | Full | Full |
| MGR-012 | Full | Full | Partial | Full |
| MGR-013 | Allowed | Allowed | Allowed | Allowed |
| MGR-014 | Full | Full | Full | Full |
| MGR-015 | Full | Full | Full | Full |
| MGR-016 | Full | Full | Full | Full |

Table 6.1.1: Requirements fulfilment reference

## 6.2	MGR-001

Editor’s Note: This chapter gives descriptions in depth how the technologies can fulfil each management related requirements defined in oneM2M as well as the gaps related to the requirements.

### 6.2.1	Requirement Description

Editor’s note: This section is a copy paste of the requirement text in the requirement TS. Revision number of the requirement TS should be explicitly referenced.

The M2M System shall support management and configuration of M2M Gateways/ Devices including resource constrained M2M Devices.[i.23]

Note: See the Annex A as a guidance about the definition of resource constrained and what kinds of the existing management technologies are suitable to apply the constrained devices.

### 6.2.2	OMA DM 1.3

OMA DM 1.3 provides PARTIAL support for this requirement.

OMA DM 1.3 requires an OMA DM compliance device shall have at least one of the protocol stacks among TCP/IP, IrDA or WSP. And the devices shall also have a capability to parse the xml file. Because the DM Representation OMA DM uses to deliver the DM Message is in the format of XML. The OMA DM devices shall also be capable of store a certain amount of information which is the MO trees to carry the management functions. For constrained devices that serve very simple functions and have the basic capability of parsing short XML and small amount of storage to store the MO, OMA DM 1.3 can be used for device management. As a result, OMA DM can be applied to some resource constrained devices but not those very limited in resources (no memory, cannot parse the XML, no communication module).

OMA DM 1.3 can also configure devices with DM Client using the ClientAPI between DM Client and the local application. With the local application defined by oneM2M, devices can be configured using service layer protocols.

### 6.2.3	BBF TR-069

TR-069  provides PARTIAL support for this requirement.

The TR-069 provides support for resource constrained devices that are CWMP enabled through the use of its standard CWMP protocols. For resource constrained devices that are not CWMP enabled (e.g., ZigBee devices, IP devices without CWMP stack), TR-069 provides mechanisms to access the constrained devices through a CWMP enabled device called a CWMP Proxy. Section 5.2.1.1 TR-069 Proxy Management describes this architecture. A technology constraint exists in that the CWMP Proxy must have connectivity, typically LAN, with the non-CWMP enabled device. As such, the TR-69 Proxy Management functions generally reside on a M2M Gateway within the customer premises.

Resource constrained devices that are CWMP enabled requires, at a minimum, the support for the:

Protocol stack as defined in Section 5.1.4.1 ACS to CPE Protocol

Implementation of the TR-181i2 Baseline:3 profile [i.34]

Resources required to implement a CWMP stack have been advertised as low as 150 Kilobytes storage and 30 Kilobytes DRAM (heap and stack) on an Android operating system.

Many resource constrained devices require monitoring of the device’s environment (e.g., processor, memory, battery, temperature), the TR-181i2 data model provides support for many of these objects (processor, memory, temperature) where these objects may be monitored and alarmed using the FaultMgmt objects of the data model or using the Active/Passive notification mechanism described in Section 3.7.1.5 of TR-069 [i.13]. While TR-181i2 provides support for many objects within a resource constrained device, the current data model does not provide support for a Battery resource. This type of resource may be implemented using Vendor specific extensions or submitted to the Broadband Forum for inclusion in a revision of the TR-181i2 data model.

### 6.2.4	OMA DM LWM2M

OMA DM LWM2M provides FULL support for this requirement.

Since the main focusing M2M device of LWM2M is resource constraint device, LWM2M is specialized in managing and configuring resource constraint devices.

For resource constraint device, LWM2M has the several features which are listed below:

- CoAP with minimum set of required features for header and options

- Either UDP or SMS as transport layer binding

- Binary TLV format (Tag, Length, Value) for conveying values of multiple Resources

- JSON format is optionally supported

Editor’s Note: According to the definition of resource constrained, this analysis can be reformulated.

### 6.2.5	OMA DM 2.0

OMA DM 2.0 provides PARTIAL support for this requirement.

The device that conforms to OMA DM 2.0 shall support TCP/IP, HTTP protocol stack, which might be not applicable for severely resource-constrained devices (e.g., 8-bit microcontrollers with small accounts of memory).

OMA DM 2.0 also requires the HTTP client in the device that can be used to retrieve management data from the Data Repository. Note that this is a mandatory feature of OMA DM 2.0. Optionally, OMA DM 2.0 might require the Web Browser Component in the device to support the web-based user interaction.

If the resource constrained device can support TCP/IP and HTTP protocol, OMA DM 2.0 can be used to manage those devices with the simple DM package representations based on the JSON format.

Compared to OMA DM 1.3, OMA DM 2.0 has different factors to support resource constrained devices as follows:

Supporting only HTTP transport-binding,

Providing the simple JSON-based DM package representations,

Requiring the HTTP Client to interact with the Data Repository,

Optionally requiring the Web Browser Component for the user interaction.

## 6.3	MGR-002

### 6.3.1	Requirement Description

The M2M System shall provide the capability to discover the M2M Area Networks including information about devices on those networks and the parameters (e.g. topology, protocol) of those networks.[i.23]

### 6.3.2	OMA DM 1.3 and OMA DM 2.0

OMA DM 1.3 provides FULL support for this requirement.

In the setup phase of the OMA DM protocols. The MO DevInfo is transported from DM Client to DM Server. And the MO DevDetail can be requested by DM Server if necessary. In the MO DevInfo and DevDetail, information about how the device can be reached, the protocol, the address, port number, required security parameters is transferred from device to DM server.

For devices in the local area network that are attached to the DM Gateway, GwMO can be used to get the address information.

Also some work has been done in ETSI M2M to define MANMO and MANDMO [i.24] to enable the DM Server to get the information about the topology and protocol of the local area network.

In this way, the DM Server can get to know the connection related parameters (protocol) from the device.GwMO defines how DM Server can manage device in a local area network through DM Gateway. DM Gateway can work in three modes which are transparent mode, proxy mode and adaptor mode. OMA DM devices and non-OMA DM devices can all be managed using DM Gateway. Combined with other MOs defined by OMA DM, devices in the local area network can be managed by DM Server.

### 6.3.3	BBF TR-069

TR-069  provides FULL support for this requirement.

The TR-069 provides support for discovery of  devices in the associated Local Area Networks for CWMP enabled devices.

TR-069 proxy management has mechanisms where a CPE Proxier discovers devices using device discovery mechanisms as described in Appendix I of TR-069 [i.13]. These mechanisms rely on the discovery of the device using the device’s native protocol (e.g., UPnP DM, ZigBee, Z-wave).

The discovery of the topology of the area network in which the device exists is constrained by the device’s native protocol support for topology discovery. For example – UPnP DM doesn’t provide any support for discovery of topologies while ZigBee topologies can be inferred by evaluating the routing tables of the ZigBee nodes. The TR-181 data model exposes these elements (e.g., ZigBee routing tables) but rely on the management systems to develop the topologies.

The TR-181i2 data model [reference TR-181 Device Data Model for TR-069 Issue: 02 Amendment 6 November 2012] provides support for the following LAN topologies:

Ethernet

WiFi

USB

HPNA

MoCA

G.hn

HomePlug

Universal Powerline Association (UPA)

UPnP

In addition the ZigBee Pro topology support is expected to be included in the next release of TR-181i2.

### 6.3.4	OMA DM LWM2M

OMA DM LWM2M provides FULL support for this requirement.

Connectivity Monitoring Object contains which networks are available and which network is currently used.

Also, it has parent router IP address of each M2M Device so the M2M System can discover entire topology of M2M Local Area Network.

Protocol (e.g., WLAN, Bluetooth) used by M2M Device is specified in Connectivity Object also.

## 6.4	MGR-003

### 6.4.1	Requirement Description

The M2M System shall provide the capability to maintain and describe the management information model of devices and parameters (e.g. topology, protocol) of M2M Area Networks. [i.23]

### 6.4.2	OMA DM 1.3 and OMA DM 2.0

OMA DM 1.3 provides FULL support for this requirement.

For devices in the local area network, some work has been done in ETSI M2M to define MANMO and MANDMO [i.24] to maintain and describe the information model about the topology and protocol of the local area network. The MANMO and MANDMO have been submitted for registration in OMA.

### 6.4.3	BBF TR-069

TR-069  provides FULL support for this requirement.

TR-069 provides capabilities to describe and maintain the management information model of CWMP and non-CWMP enabled devices as through the Supported Data Model and  Software/Firmware Management features of the protocol. Within TR-069 all devices and services that are of interest to the problem space are modelled using the TR-069 XML meta-model. These models are a description of the device and services that are under management. These models can be either configured within the device or CWMP Proxy (if the device is not CWMP enabled) or the device can report its Supported Data Model to the ACS.

### 6.4.4	OMA DM LWM2M

OMA DM LWM2M provides FULL support for this requirement.

Connectivity Monitoring Object contains lots of Resources to maintain and describe the information model about the topology and protocol of the local area network.

LWM2M has several features to be able to manage LWM2M Clients in M2M Local Area Network. The main barrier is how to reach LWM2M Clients from LWM2M Server.

- Let a LWM2M Server know when IP Address is changed at LWM2M Client by sending Registration Update logical operation.

- Support Queue mode which makes LWM2M Server queue the request until LWM2M Client is online. If LWM2M Client with Queue mode is within M2M Local Area Network, LWM2M Client sends Update message triggered by time or event. After LWM2M Server receives Update message, the LWM2M Server reaches LWM2M Client so the LWM2M Server sends queued message.

## 6.5	MGR-004

### 6.5.1	Requirement Description

The M2M System shall support common means to manage devices enabled by different management technologies (e.g. OMA DM, BBF TR-069).[i.23]

### 6.5.2	OMA DM 1.3 and OMA DM 2.0

OMA DM 1.3 can ALLOW the fulfilment of the requirement.

OMA DM does not provide features that translate between OMA DM and other management protocols (e.g., BBF TR-069, oneM2M). As such there is an expectation that the M2M System would translate between management protocols.

The management of devices defined by OMA DM is fulfilled by sending DM Messages from DM Server to DM Client. The management related information is carried by Management Objects. For the oneM2M system to support common means to manage devices through OMA DM, the oneM2M system could include the DM Server, DM Client and DM Gateway in the oneM2M system and could have some abstraction to include the MO trees in the oneM2M system to enable the device management in spite of the detailed technologies.

### 6.5.3	BBF TR-069

TR-069  ALLOWS  fulfilment of the requirement by Service Layer mechanisms.

The TR-069 family of specifications do not provide features that translate between TR-069 and other management protocols (e.g., OMA-DM, oneM2M). As such there is an expectation that the M2M System would translate between management protocols.

TR-069 provides access to manage devices through its Auto-configuration Server. The Auto-configuration Server has interfaces with the NMS/OSS and BSS systems of the Service provider. As such the ACS would have expected to interface with the M2M System.

### 6.5.4	OMA DM LWM2M

OMA DM LWM2M ALLOWS fulfilment of the requirement when common means between the oneM2M Server and LWM2M Server are defined. OMA DM LWM2M only defines the protocol between the LWM2M Server and LWM2M Client.

LWM2M does not provide features that translate between LWM2M and other management protocols (e.g., OMA-DM, oneM2M, TR-069).

## 6.6	MGR-005

### 6.6.1	Requirement Description

The M2M System shall provide the capability to manage multiple devices in a grouped manner. [i.23]

### 6.6.2	OMA DM 1.3 and OMA DM 2.0

OMA DM 1.3 provides PARTIAL support for this requirement.

The requirement can be fulfilled both in service layer and by OMA DM technology.

For service layer, one DM Server could manage multiple DM Clients. The oneM2M system is able to manage multiple devices in a grouped manner by utilize one DM Server. The M2M System could have multiple devices in a group. When the M2M System needs to send identical DM Message to each device in the group, the M2M System could pass the command to DM Server to send DM Messages one by one or simultaneously.

For manage group of devices in OMA DM. GwMO is defined how to fan out DM Command to a group devices.

### 6.6.3	BBF TR-069

TR-069  provides PARTIAL support for this requirement.

The TR-069 family of specifications defines management actions that are destined for a single CWMP enabled device. The concept of groups within the TR-069 family of specifications does not exist. Grouping, while not specified within the CWMP protocol is implemented within ACS or the NMS/OSS/BSS systems. As such, the M2M System would be required to implement the grouping feature.

### 6.6.4	OMA DM LWM2M

OMA DM LWM2M provides PARTIAL support for this requirement.

The requirement can be fulfilled at service layer.

A LWM2M Server could manage multiple LWM2M Clients. The oneM2M system is able to manage multiple devices in a grouped manner by utilizing one LWM2M Server. The M2M System could have multiple devices in a group. When the oneM2M System needs to send identical message to each device in the group, the oneM2M System could pass the command to the LWM2M Server to send the messages one by one.

## 6.7	MGR-006

### 6.7.1	Requirement Description

The M2M System shall provide the capability for provisioning and configuration of devices in M2M Area Networks. [i.23]

### 6.7.2	OMA DM 1.3 and OMA DM 2.0

OMA DM 1.3 provides FULL support for this requirement.

For devices in the local area network, GwMO can be used to provision and configure End Devices that are attached to the DM Gateway. Gateway Config MO contains the information related to each attached End Device.\

### 6.7.3	BBF TR-069

TR-069  provides FULL support for this requirement.

TR-069 provides support for provisioning and configuration of  devices in the Local Area Networks for CWMP enabled and non-CWMP enabled devices. TR-069 has the capability to manage a device through the device’s life-cycle (bootstrap through decommissioning).

For CWMP enabled devices that reside behind a Gateway with Firewalls and Network Address Translation features enabled, the CWMP protocol provides a mechanism that allows the ACS to communicate those devices. This mechanism is described in as described in Annex G of TR-069 [i.13].

### 6.7.4	OMA DM LWM2M

OMA DM LWM2M provides FULL support for this requirement.

In LWM2M, bootstrap mechanism is used to provision and configure the M2M Device. Four approaches are introduced in TS: Manufacturer Pre-configuration, SmartCard Provisioning, Client Initiated Bootstrap, and Server Initiated Bootstrap. And detail bootstrap steps are described.

## 6.8	MGR-007

### 6.8.1	Requirement Description

The M2M System shall provide the capability for monitoring and diagnostics of M2M Gateways/Devices in M2M Area Networks. [i.23]

### 6.8.2	OMA DM 1.3 and OMA DM 2.0

OMA DM 1.3 provides FULL support for this requirement.

The capability of monitoring and diagnostics of OMA DM is mainly achieved by MO DiagMon. DiagMon supports diagnostics policies management, fault reporting, performance monitoring, device interrogation, remote diagnostics procedure invocation and remote device repairing. The monitoring and diagnostics of the devices in the local area network can be fulfilled by DiagMon plus GwMO.

### 6.8.3	BBF TR-069

TR-069  provides FULL support for this requirement.

TR-069 provides support for monitoring and diagnostics of  devices in the Local Area Networks for CWMP enabled and non-CWMP enabled devices. Monitoring can include notification support for devices that need immediate attention as well as passive monitoring of device information and statistics that may be collected in a periodic manner. Likewise, TR-069 provides the capability to execute diagnostics in a synchronous or asynchronous fashion; allowing for long lived diagnostics to be executed on a device,

For non-CWMP enabled devices, the device is implemented as a non-CWMP enabled Virtual Device where the procedure is documented in Appendix I of TR-069 [i.13]

CWMP and non-CWMP enabled Virtual and Embedded devices support the following diagnostic operations:

Reboot

Factory Reset

In addition CWMP enabled and non-CWMP enabled Virtual Devices also support the following standard diagnostics:

IP Diagnostics (Ping, Trace Route, HTTP or FTP Download or Upload, UDP Echo)

DNS – NS Lookup

HPNA Diagnostics

UPA Diagnostics

Device Self Tests

DSL Line

ATM Interface

These tests are documented within the TR-181i2 data model [reference TR-181 Device Data Model for TR-069 Issue: 02 Amendment 6 November 2012].

### 6.8.4	OMA DM LWM2M

OMA DM LWM2M provides FULL support for this requirement.

In LWM2M, Connectivity Monitoring Object and Connectivity Statistics Object are used to monitor current connection and collect connection information to configure parameters based on the collected result respectively.

LWM2M enabler also has concept to send diagnostic information such as GPS module failure, IP connectivity failure, and Peripheral malfunction.

## 6.9	MGR-008

### 6.9.1	Requirement Description

The M2M System shall provide the capability for software management of devices in M2M Area Networks. [i.23]

### 6.9.2	OMA DM 1.3 and OMA DM 2.0

OMA DM 1.3 provides FULL support for this requirement.

Software management capability is fulfilled by the MO SCOMO (Software Component Management Object). SCOMO can be used to remotely manage a software component within a device. The functionalities provided by SCOMO includes delivery, download, installation, update, removal, activation, and de-activation of software. The software management of devices in local area network can be fulfilled by SCOMO plus GwMO.

### 6.9.3	BBF TR-069

TR-069  provides FULL support for this requirement.

TR-069 provides support for software and firmware management of  devices in the Local Area Networks for CWMP enabled and non-CWMP enabled devices. Software and firmware management of non-CWMP enabled devices may be performed using Software modules within the CWMP enabled M2M Gateway or may be downloaded directly to device by implementing the non-CWMP enabled device as a Non-CWMP enabled Virtual Device.

### 6.9.4	OMA DM LWM2M

OMA DM LWM2M provides PARTIAL support for this requirement.

Since LWM2M talks about resource constraint M2M devices, LWM2M enabler provides the capability for management of a full package, firmware. So firmware contains all the software which is necessary for all the services provided by M2M Service Platform.

## 6.10	MGR-009

### 6.10.1	Requirement Description

The M2M System shall provide the capability for rebooting and/or resetting of M2M Gateways/Devices and other devices in M2M Area Networks. [i.23]

### 6.10.2	OMA DM 1.3 and OMA DM 2.0

OMA DM 1.3 provides FULL support for this requirement.

The functionality for reset the device is provided by the MO LAWMO. LAWMO is for remotely locking and wiping the device. The functionality of rebooting a device is enabled by DiagMon. In this way, OMA DM can reset the device to its original state and reboot the device as well. The reset and reboot of a device in local area network can be enabled by DCMO or DiagMon plus GwMO.

### 6.10.3	BBF TR-069

TR-069  provides FULL support for this requirement.

TR-069 provides support for resetting (boot, factory) of  devices in the Local Area Networks for CWMP enabled and non-CWMP enabled devices. For non-CWMP enabled devices, the device is implemented as a Non-CWMP enabled Virtual Device.

In the scenario where the device in the M2M Local Area network is a CWMP enabled device, TR-069 provides a mechanism where the device can communicate through a Gateway (which might be NAT or Firewall enabled) to the ACS. Annex G of TR-069 [i.13] describes this mechanism.

### 6.10.4	OMA DM LWM2M

OMA DM LWM2M provides FULL support for this requirement.

Device Object has a Reset Resource which is used for resetting M2M Devices. LWM2M also provides factory reset function which makes M2M Device initial state of deployment.

## 6.11	MGR-010

### 6.11.1	Requirement Description

The M2M System shall provide the capability for authorizing devices to access M2M Area Networks. [i.23]

### 6.11.2	OMA DM 1.3 and OMA DM 2.0

OMA DM 1.3 provides PARTIAL support for this requirement.

Configuration methods of OMA DM can be used to detach certain device from the network. Coordinator and router can also be configured to block the new access request to deny devices to be attached to the local area network. In this way, devices can be authorized to access M2M Local Area Network.

### 6.11.3	BBF TR-069

TR-069  provides PARTIAL support for this requirement.

TR-069 does not provide direct support for authorizing devices for access to Local Area Networks but does allow for configuration of credentials and other properties that these networks utilize. TR-069 does this for CWMP and non-CWMP enabled devices.

### 6.11.4	OMA DM LWM2M

OMA DM LWM2M provides PARTIAL support for this requirement.

If M2M devices have configuration for accessing M2M Local Area Networks prior to deployment, the M2M devices can be authorized based on the configured information.

## 6.12	MGR-011

### 6.12.1	Requirement Description

The M2M System shall provide the capability for modifying the topology of devices in M2M Area Networks. [i.23]

### 6.12.2	OMA DM 1.3 and OMA DM 2.0

OMA DM 1.3 provides FULL support for this requirement.

OMA DM can be used to modify the topology of devices in M2M Local Area Network by activate and de-activate devices that serve as coordinator or router in the area network. For example, if a coordinator or router is de-activated, devices attached to the coordinator or router will automatically find other coordinators or routers to access the local area network.

Configuration methods can also be used to configure the router or coordinator to accept or deny access request of new devices. The topology is modified.

### 6.12.3	BBF TR-069

TR-069  provides FULL support for this requirement.

TR-069 does allow for configuration  of properties for Local Area Networks that can determine which devices are to be included within a Local Area Network. TR-069 does this for CWMP and non-CWMP enabled devices.

### 6.12.4	OMA DM LWM2M

OMA DM LWM2M provides FULL support for this requirement.

OMA DM LWM2M can be used to modify the topology of devices in M2M Local Area Network by disabling M2M devices which serve as coordinator in the area network. For example, if a coordinator is disabled, devices in M2M Local Area Network attached to the coordinator will automatically find other coordinators to access the local area network.

## 6.13	MGR-012

### 6.13.1	Requirement Description

Upon detection of a new device the M2M Gateway shall be able to be provisioned by the M2M Service Infrastructure with an appropriate configuration which is required to handle the detected device. [i.23]

### 6.13.2	OMA DM 1.3 and OMA DM 2.0

OMA DM 1.3 provides FULL support for this requirement.

In a DM Server assisted bootstrap procedure defined in GwMO. Whenever a new device is detected, DM Server will install the End Device credentials to the gateway through Gateway Config MO.

In OMA DM 1.3, DM Gateways with DM Client can be also provisioned using the ClientAPI between DM Client and the local application. With the local application defined by oneM2M, devices can be configured using service layer protocols.

### 6.13.3	BBF TR-069

TR-069  provides FULL support for this requirement.

TR-069 provides support for detection of new devices in the Local Area Networks. TR-069 has the capability to detect new devices via active and passive notification mechanisms described in Section 3.7.1.5 of TR-069 [i.13]  as well as using the CWMP protocol’s inform event (e.g., Bootstrap) mechanisms for CWMP enabled devices.

In the most common M2M deployment scenarios, the M2M Gateway would include a CWMP Agent making the M2M Gateway a CWMP-enabled device with a CWMP Proxier as defined in Annex J of TR-069 [i.13].

In this scenario, the M2M Gateway’s CWMP agent could detect and report the new device to the ACS via the M2M Gateway’s CWMP agent. Once a device is reported to the ACS, the ACS can inform the M2M System about the device addition for further configuration and software/firmware management activities. Possible configuration activities could include:

Software management (Annex H of TR-069[i.13])

Device configuration via Proxy management (Appendix I of TR-069[i.13])

### 6.13.4	OMA DM LWM2M

OMA DM LWM2M provides FULL support for this requirement.

LWM2M doesn’t have gateway concept so to be able to handle the detected devices, the M2M Gateway needs to have LWM2M Server feature which means that device thinks gateway as LWM2M Server. In this case, M2M Gateway can manage the device detected.

## 6.14	MGR-013

### 6.14.1	Requirement Description

The M2M System shall be able to identify and manage M2M Service status of M2M Devices. [i.23]

### 6.14.2	OMA DM 1.3 and OMA DM 2.0

OMA DM 1.3 and 2.0 ALLOWS fulfilment of the requirement by development of a oneM2M Service Layer data model for management purposes.

DCMO defined in OMA DM can be used to enable or disable device capabilities, such as hardware, IO and connectivity. OMA DM 1.3 and 2.0, as a device management protocol, is able to manage the M2M Service layer using the DM protocol to configuration and retrieval of M2M Services that execute within M2M Devices. The only thing necessary is that a data model of the M2M Service be defined for management purposes.

### 6.14.3	BBF TR-069

TR-069 ALLOWS fulfilment of the requirement by development of a oneM2M Service Layer data model for management purposes.

The TR-069 family of specifications, as a device management protocol, is able to manage the M2M Service layer using the CWMP protocol to configuration and retrieval of M2M Services that execute within M2M Devices. The only thing necessary is that a data model of the M2M Service be defined for management purposes.

### 6.14.4	OMA DM LWM2M

OMA DM LWM2M ALLOWS fulfilment of requirement by development of a M2M Service Layer data model for management purposes.

The LWM2M specification, as a device management protocol, is able to manage the M2M Service layer using the LWM2M protocol to configuration and retrieval of M2M Services that execute within M2M Devices. The only thing necessary is that a data model of the M2M Service be defined for management purposes.

## 6.15	MGR-014

### 6.15.1	Requirement Description

The M2M System shall be able to retrieve events and information logged by M2M Gateways/ Devices and other devices in M2M Area Networks. [i.23]

### 6.15.2	OMA DM 1.3 and OMA DM 2.0

OMA DM 1.3 and 2.0 provides FULL support for this requirement..

Related technologies are defined in DiagMon:Trap which describes how the DM Client monitors the performance of the device. The DiagMon Client can send notification to DM Server or collect trap event together to response the retrieve request. As a result, OMA DM 1.3 and 2.0 can fully fulfill the requirement.

With regard to devices in the M2M Area Network, DiagMon is combined with GwMO to fulfill the requirement.

### 6.15.3	BBF TR-069

TR-069 provides FULL support for this requirement.

The TR-069 family of specifications, as a device management protocol, is able to retrieve events and logs for M2M Gateway/Devices and devices in M2M Area Networks through the use of the CWMP protocol using the standard data model and proxy mechanisms. Information can be retrieved using the CWMP get RPC as well as file transfer mechanisms (e.g., FTP, HTTP). In addition, the IPDR protocol can be used to retrieve information that would be considered bulk transfer.

### 6.15.4	OMA DM LWM2M

OMA DM LWM2M provides FULL support for this requirement.

The LWM2M Client has capability to collect error or event information such as GPS module failure, out of memory, SMS failure, and low battery power. The LWM2M has functionality to collect data to be notified due to subscription when the LWM2M Client is offline or the LWM2M Server is temporarily disabled.

## 6.16	MGR-015

### 6.16.1	Requirement Description

The M2M System shall be able to support firmware management (e.g. update) of M2M Gateways/ Devices and other devices in M2M Area Networks.. [i.23]

### 6.16.2	OMA DM 1.3 and OMA DM 2.0

OMA DM 1.3 and 2.0 provides FULL support for this requirement.

It is defined in FUMO about how to support firmware management. FUMO defined in OMA DM can be used to initiate firmware update, exchange device information, download update package, install update package, notify firmware update.

With regard to devices in the M2M Area Network, FUMO is combined with GwMO to fulfill the requirement.

### 6.16.3	BBF TR-069

TR-069 provides FULL support for this requirement.

The TR-069 family of specifications, as a device management protocol, is able to download firmware for M2M Gateway/Devices and devices in M2M Area Networks through the use of the CWMP protocol using the standard data model and proxy mechanisms. Firmware can be retrieved using the CWMP download RPC file transfer mechanisms (e.g., FTP, HTTP). One constraint exists in the proxy mechanism for downloading firmware to device in a M2M Area Network. A device in a M2M Area Network must be a Virtual Device to have allow for the download RPC to be utilized.

### 6.16.4	OMA DM LWM2M

OMA DM LWM2M provides FULL support for this requirement.

LWM2M has Firmware Object to update firmware of the LWM2M Client. LWM2M supports two download mechanisms: directly write firmware package, and give URI and let the LWM2M Client download firmware package. After updating firmware, LWM2M has capability to inform the connected LWM2M Servers of what functionalities are added in the LWM2M Client.

## 6.17	MGR-016

### 6.17.1	Requirement Description

The M2M System shall be able to retrieve information related to the Static and Dynamic Device/Gateway Context for M2M Gateways/Devices as well as Device Context for other devices in M2M Area Networks. [i.23]

### 6.17.2	OMA DM 1.3 and OMA DM 2.0

OMA DM 1.3 and 2.0 provides FULL support for this requirement.

DiagMon defined by OMA DM 1.3 and 2.0 enable the DM Server to acquire information related to the devices local context including battery level, available memory as well as some network capabilities.

DiagMon also defines Trap method which can be used to collect events related to the change of dynamic context in the device. The collected events can be retrieved by the DM Server.

ClientAPI defined in OMA DM 1.3 and 2.0 also provides interfaces that can be used to retrieve MO information from DM Client which can be static device context.

### 6.17.3	BBF TR-069

TR-069 provides FULL support for this requirement.

The TR-069 family of specifications, as a device management protocol, is able to retrieve device specific information for M2M Gateway/Devices and devices in M2M Area Networks through the use of the CWMP protocol using the standard data model and proxy mechanisms.  As the TR-069 model utilizes data models for representing information that is easily extended either through standardization activities or vendor specific attributes.

### 6.17.4	OMA DM LWM2M

OMA DM LWM2M provides FULL support for this requirement.

LWM2M has Static Context such as manufacturer, model number, and serial number, and Dynamic Context such as battery level, memory free, location, time, IP address, current network (e.g., WCDMA, GSM, LTE, Bluetooth, WiFi), and serving cell id. LWM2M has capability to expose parent IP address, which can make topology of M2M Area Network.