| INPUT CONTRIBUTION |  |
| --- | --- |
| Group Name:* | ARC WG |
| Title:* | Device management resource message flows |
| Source:* | InterDigital Communications |
| Contact: | Catalina Mladin, Catalina.Mladin@InterDigital.com |
| Date:* | 2013-12-01 |
| Abstract:* | The contribution provides description of message flows for the management command |
| Agenda Item:* | TBD |
| Work item(s): | M2M Architecture |
| Document(s) Impacted* | Architecture TS |
| Intended purpose of document:* | Decision Discussion Information Other <specify> |
| Decision requested or recommendation:* | Incorporate the texts into the TS if approved. |

oneM2M IPR STATEMENT

Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by all provisions of IPR policy of the admitting Partner Type 1 and permission that all communications and statements, oral or written, or other information disclosed or presented, and any translation or derivative thereof, may without compensation, and to the extent such participant or attendee may legally and freely grant such copyright rights, be distributed, published, and posted on oneM2M’s web site, in whole or in part, on a non-exclusive basis by oneM2M or oneM2M Partners Type 1 or their licensees or assignees, or as oneM2M SC directs.

#### 10.3..X	External management operations through <mgmtCmd>

##### 10.3.X.1 Introduction

This clause describes how RESTful management operations may be performed using <mgmtCmd> resources over the Mca and Mcc reference points.  The <mgmtCmd> resource, together with its attributes or sub-resources, may be used in the process of translating between RESTful operations and management commands and procedures from existing management technologies (e.g. BBF TR-069). These procedures can then be performed on the remote entity, using the  Management Adapterand the procedures described in the following clauses.

##### 10.3..X.2 Create <mgmtCmd>

A CREATE request is used by an originator to create a specific <mgmtCmd> resource in a hosting CSE.

The created <mgmtCmd> will be mapping a RESTful method to commands and/or procedures from existing management protocols (e.g. BBF TR-069). At run-time the hosting CSE can expose the translated commands, over the Mcc reference point, to the remote entities (i.e. D/G CSE).

Originator: shall request to create a new mgmtCmd type resource to be named as "<mgmtCmd>" by using a CREATE verb. The request shall address baseURI resource of the hosting CSE. The request may also provide the attributes of the <mgmtCmd> to be created as described in clause 9.X.1 such as "cmdType".

The originator may be:

- An AE registered to the N-CSE.

- The local CSE on the managed entity: In this case, the local CSE transforms supported management command into the <mgmtCmd> resource representation, then requests the hosting CSE to create the corresponding <mgmtCmd> resource.

NOTE 1:	The hosting CSE in the network domain may also create the <mgmtCmd> resource locally by itself. The details are out of scope. Then an AE can discover the created <mgmtCmd> and manipulate it.

NOTE 2:	The <mgmtCmd> resource may also be created in the hosting CSE by other offline provisioning means which are out of scope.

Receiver: shall check if the originator has the CREATE permission on the addressed baseURI resource (or the parent <mgmtCmd> in the case of child resource creation). The hosting CSE shall also check the validity of provided attributes. Upon successful validation, a new resource with name "<mgmtCmd>" shall be created in the hosting CSE with the provided attributes. The hosting CSE shall maintain the mapping between the created <mgmtCmd> and corresponding nonRESTful commands represented by the <cmdType> attribute of <mgmtCmd> resource. The hosting CSE shall respond to the originator with the appropriate generic. It shall also provide in the response the URL of the created new resource.

The hosting CSE shall be:

- An N-CSE.

Table 10.3.x.2.1 <mgmtCmd> CREATE flow

|  | Description |
| --- | --- |
| Call flow type | CREATE |
| Pre-conditions | Originating D/G-CSE shall first collect local management command |
| Information on Request message | op: C fr: Originator App-Inst-ID or CSE-ID to: Receiver CSE-ID cn: Name of <mgmtCmd> resource The mandatory and/or optional attributes defined in 9.6.X, as needed. |
| Local processing on Hosting CSE | No change from the basic procedure (clause 10.1.1.1) |
| Information on Response message | fr: Receiver CSE-ID to: Originator App-Inst-ID or CSE-ID cn: URI of created <mgmtCmd> resource |
| Post-condition | None |
| Exceptions | If the creation is not allowed, the <mgmtCmd> resource shall not be created, and a proper error code shall be returned to the originator in the Response message |

##### 10.3.X.3 Retrieve <mgmtCmd>

A RETRIEVE request is used by an originator to retrieve all or part information from an existing <mgmtCmd> resource on a hosting CSE. Alternatively, the originator can request to retrieve only a specific attribute or part of an attribute.

Originator: shall request to retrieve all or part of information from an existing <mgmtCmd> resource by using a RETRIEVE verb. The request shall address a specific "<mgmtCmd>" resource to retrieve all attributes and the references to the sub-resources of the <mgmtCmd> resource.  Alternatively, the request shall address the individual attribute of the specific "<mgmtCmd>" resource to retrieve the corresponding attribute value.

The originator may be:

- An AE.

- A D/G-CSE.

Receiver: shall check if the originator has the READ permission on the addressed <mgmtCmd> resource. Upon successful validation, the hosting CSE shall retrieve the corresponding attributes the <mgmtCmd> resource and the references to the sub-resources from its repository and shall respond to the originator with the appropriate generic responses.

The hosting CSE shall be:

- An N-CSE.

Table 10.3.x.3.1 <mgmtCmd> RETRIEVE flow

|  | Description |
| --- | --- |
| Call flow type | RETRIEVE |
| Pre-conditions | Originator needs to create a resource |
| Information on Request message | op: R fr: Originator App-Inst-ID or CSE-ID to: Receiver CSE-ID cn: Name of <mgmtCmd> resource The mandatory and/or optional attributes defined in 9.6.X, as needed. |
| Local processing on Hosting CSE | No change from the basic procedure (clause 10.1.1.2) |
| Information on Response message | No change from the basic procedure (clause 10.1.1.2) |
| Post-condition | None |
| Exceptions | If the retrieval is not allowed or the specific <mgmtCmd> resource does not exist in the N-CSE, a proper error code shall be returned to AE (or D/G-CSE) |

##### 10.3.X.4 Update <mgmtCmd>

An UPDATE request is used by an originator to update all or part information of an existing <mgmtCmd> resource on a hosting CSE with new attributes. Alternatively, the originator can request to update only a specific attribute or part of an attribute.

Originator: shall request to update all or part information of an existing <mgmtCmd> resource by using a UPDATE verb. The request shall address a specific "<mgmtCmd>" resource of a CSE to update all attributes of the <mgmtCmd> resource with the provided new value.  Alternatively, the request shall address the individual attribute of the specific "<mgmtCmd>" resource to update the corresponding attribute with the provided new value.

The originator may be:

- An AE.

- A D/G-CSE.

Receiver: shall check if the originator has the WRITE permission on the address <mgmtCmd> resource. The hosting CSE shall also check the validity of provided attributes if any. Upon successful validation, the hosting CSE shall overwrite the corresponding attributes of the <mgmtCmd> resource with the provided new data. The CSE shall respond to the originator with the appropriate generic responses.

The hosting CSE shall be:

- An N-CSE.

Table 10.3.x.4.1 <mgmtCmd> UPDATE flow

|  | Description |
| --- | --- |
| Call flow type | UPDATE |
| Pre-conditions | Originator needs to retrieve a resource |
| Information on Request message | op: U fr: Originator App-Inst-ID or CSE-ID to: Receiver CSE-ID cn: Name of <mgmtCmd> resource The mandatory and/or optional attributes defined in 9.6.X, as needed. |
| Local processing on Hosting CSE | No change from the basic procedure (clause 10.1.1.3) |
| Information on Response message | No change from the basic procedure (clause 10.1.1.3) |
| Post-condition | None |
| Exceptions | If the update is not allowed or the specific <mgmtCmd> resource (or its sub-resource) does not exist in the N-CSE, a proper error code shall be returned to AE (or D/G-CSE) |

##### 10.3.X.5 Delete <mgmtCmd>

A DELETE request is used by the originator to delete an existing <mgmtCmd> resource on a hosting CSE. An AE may also use this procedure to cancel all initiated <execInstance> of an <mgmtCmd> if applicable.

Originator: shall request to delete an existing <mgmtCmd> resource by using a DELETE verb. The request shall address the specific "<mgmtCmd>" resource on the hosting CSE.

The originator may be:

- The D/G-CSE on the manageable entity: In this case, the local D/G-CSE issues the request to the hosting CSE to hide the corresponding management command from being exposed by the <mgmtCmd> resource.

- An AE: In this case, the AE requests the hosting CSE to delete the <mgmtCmd> resource from the hosting CSE and cancel all initiated <execInstance> of an <mgmtCmd> if applicable.

NOTE 1:	The hosting CSE in the network domain may also delete an <mgmtCmd> resource locally by itself. This internal procedure is out of scope.

NOTE 2:	The <mgmtCmd> resource may also be deleted in the hosting CSE by other offline provisioning means which are out of scope.

Receiver: shall check if the originator has the DELETE permission on the address <mgmtCmd> resource. Upon successful validation, the hosting CSE shall remove the resource from its repository. If the originator is an AE and there is any initiated <execInstance> under the <mgmtCmd>that can be cancelled by a corresponding management command. The hosting CSE shall also issue the management command to the remote entity to cancel those initiated <execInstance> based on existing management protocol (i.e. BBF TR-069). Then the CSE shall respond to the originator with the appropriate generic responses.

The hosting CSE shall be:

- An N-CSE.

Table 10.3.x.5.1 <mgmtCmd> DELETE by D/G-CSE flow

|  | Description |
| --- | --- |
| Call flow type | DELETE |
| Pre-conditions | Before issuing a DELETE request to the N-CSE, the D/G-CSE may perform cancelling of the corresponding management command locally. |
| Information on Request message | op: D fr: Originator App-Inst-ID or CSE-ID to: Receiver CSE-ID cn: Name of <mgmtCmd> resource The mandatory and/or optional attributes defined in 9.6.X, as needed. |
| Local processing on Hosting CSE | The <mgmtCmd> resource shall be deleted from the repository of the N-CSE. |
| Information on Response message | No change from the basic procedure (clause 10.1.1.4) |
| Post-condition | None |
| Exceptions | If the deletion is not allowed or the specific <mgmtCmd> resource does not exist in the N-CSE, the deletion shall be failed and a proper error code shall be returned to D/G-CSE |

Table 10.3.x.1.1 <mgmtCmd> DELETE by AE flow

|  | Description |
| --- | --- |
| Call flow type | DELETE |
| Pre-conditions | Originator needs to delete the resource |
| Information on Request message | op: D fr: Originator App-Inst-ID or CSE-ID to: Receiver CSE-ID cn: Name of <mgmtCmd> resource The mandatory and/or optional attributes defined in 9.6.X, as needed. |
| Local processing on Hosting CSE | If there is any initiated <execInstance> under <mgmtCmd> and it is cancellable, N-CSE shall cancel those initiated <execInstance> from the remote entity using corresponding management procedures in existing management protocol (i.e. CancelTransfer RPC in BBF TR-069). The <mgmtCmd> resource shall be deleted from the repository of the N-CSE. |
| Information on Response message | No change from the basic procedure (clause 10.1.1.4) |
| Post-condition | None |
| Exceptions | If the deletion is not allowed or the specific <mgmtCmd> resource does not exist, there is no local processing in the N-CSE and a proper error code shall be returned to AE If the corresponding initiated commands cannot be deleted from remote entity due to some reason (e.g. not found) al response with the proper indication shall be returned to AE |

##### 10.3.X.6 Execute < mgmtCmd >

The Execute procedure is used by an originator for execution of a specific management command on a remote entity, through an existing <mgmtCmd> resource on the hosting CSE.

Originator: shall request to execute a specific management command which is represented by an existing <mgmtCmd> resource by using an UPDATE verb. The UPDATE request shall address the execEnable attribute of the <mgmtCmd> resource without any payload. Alternatively, the UPDATE request shall address the URI provided as the value of the execEnable attribute of the <mgmtCmd> resource.

After issuing the execution request, the originator may request to retrieve the execution result or status from an <execInstance> sub-resource of the <mgmtCmd>by using a RETRIEVE method as described in clause 10.3.X.8.

The originator shall be:

- An AE.

Receiver: shall check if the originator has the WRITE permission on the addressed <mgmtCmd> resource. Upon successful validation, the hosting CSE shall perform command conversion and mapping, and send the converted management command over mId reference point to execute the corresponding management command with the provided arguments on the remote entity based on existing device management protocol (i.e. BBF TR-069). Then the hosting CSE shall create a corresponding <execInstance> resource under <mgmtCmd> for this command execution. And the hosting CSE shall respond to the originator with the appropriate generic responses. It shall also provide in the response the URL of the created <execInstance> resource.

Upon receiving from the remote entity a management notification (i.e. BBF TR-069 "Inform" message) regarding the execution result or status, the hosting CSE may update the corresponding <execInstance> sub-resource locally.

The hosting CSE shall be:

- An N-CSE.

.

Table 10.3.x.6.1 <mgmtCmd> EXECUTE flow

|  | Description |
| --- | --- |
| Call flow type | UPDATE |
| Pre-conditions | Originator needs to retrieve a resource |
| Information on Request message | op: U fr: Originator App-Inst-ID or CSE-ID to: Receiver CSE-ID cn: <mgmtCmd>/execEnable The UPDATE request shall address to "<mgmtCmd>/execEnable" without any payload. The mandatory and/or optional attributes defined in 9.6.X, as needed. |
| Local processing on Hosting CSE | a. If the execution is allowed, Hosting CSE will do command conversion and mapping. b. The Hosting CSE shall trigger existing device management procedures (i.e. BBF TR-069) to execute the corresponding management command on the remote entity. c. The Hosting CSE shall perform local processing: if Step b is successful, the N-CSE shall create an <execInstance> resource under the triggered <mgmtCmd> to maintain status and results for this execution. |
| Information on Response message | No change from the basic procedure (clause 10.1.1.3) |
| Post-condition | The following processing on the Hosting CSE is dependent on the type of the command and execution status and may occur after the Response message has been sent. d. After the command execution is finished, the remote entity sends response including execution results to the Hosting CSE, who will store the execution results in corresponding <execInstance> resource. This step may occur any time after the Response message has been sent and might occur after step b below. e. The AE may use normal RETRIEVE procedure to retrieve the execution results or status of an <execInstance>. f .After receiving the RETRIEVE request from the AE, the Hosting CSE can retrieve the execution status or results on the remote entity using existing management protocol. If step d. occurs before step e, step f. may not be needed. g. A response shall be returned to the AE. |
| Exceptions | If the execution is not allowed or the specified <mgmtCmd> resource or its attribute/sub-resource does not exist in the N-CSE, no further processing is required in the Hosting CSE and a proper error code shall be returned to AE in the Message response If the corresponding management command cannot be executed in remote entity due to some reason, step 004 shall be skipped and a proper error code shall be returned to AE in step 005. |

##### 10.3.x.7 Cancel <execInstance>

The Cancel procedureis used by an originating AE to disable/stop/cancel an initiated management command execution on the remote entity, through an existing <execInstance> resource on the hosting CSE.

Originator: shall request to disable/stop/cancel an initiated management command execution which is represented by an existing <execInstance> resource, by using an UPDATE verb. The UPDATE request shall address the execDisable attribute without any payload.

The originator shall be:

- An AE.

Receiver: shall check if the originator has the WRITE permission on the addressed <mgmtCmd> resource. Upon successful validation, the hosting CSE shall perform command conversion and mapping, then use existing management protocol (i.e. BBF TR-069) to cancel the corresponding management command execution initiated on the remote entity, and the hosting CSE shall respond to the originator with the appropriate responses.

The hosting CSE shall be:

- An N-CSE.

Table 10.3.x.9.1 <execInstance> CANCEL flow

|  | Description |
| --- | --- |
| Call flow type | UPDATE |
| Pre-conditions | Originator needs to disable/stop/cancel an initiated management command execution on the remote entity |
| Information on Request message | op: U fr: Originator App-Inst-ID or CSE-ID to: Receiver CSE-ID cn: <mgmtCmd>/execInstance The mandatory and/or optional attributes defined in 9.6.X, as needed. |
| Local processing on Hosting CSE | If the cancellation is allowed, N-CSE will do command conversion and mapping. The N-CSE shall cancel the <execInstance> from the remote entity using corresponding management procedures in existing management protocol (i.e. CancelTransfer RPC in BBF TR-069). The N-CSE shall delete the corresponding <execInstance> resource. |
| Information on Response message | No change from the basic procedure (clause 10.1.1.3) |
| Post-condition | None |
| Exceptions | If the execution is not allowed or the specified <execInstance> resource does not exist in the N-CSE or the <execInstance> is finished, the local processing on Hosting CSE shall be skipped and a proper error code shall be returned to AE in the Response message. If the corresponding management command cannot be executed in remote entity due to some reason, the N-CSE shall still delete the corresponding <execInstance> resource and return a Response message. |

##### 10.3.X.8 Retrieve <execInstance>

A RETRIEVE request is used by an originator to retrieve an existing <execInstance> resource, including individual attributes, from a hosting CSE. Alternatively, the originator can request to retrieve only a specific attribute or part of an attribute.

Originator: shall request to retrieve all or part of information from an existing <execInstance> resource by using a RETRIEVE verb. The request shall address a specific "<execInstance>" resource of a hosting CSE to retrieve all attributes of the <execInstance> resource. Or the request shall address the individual attribute of the specific "<execInstance>" resource to retrieve the corresponding attribute value.

The originator shall be:

- An AE.

Receiver: shall check if the originator has the READ permission on the addressed <mgmtCmd> resource. Upon successful validation, the CSE shall retrieve the corresponding attributes of the <execInstance> resource and the references to the sub-resources from its repository and shall respond to the originator with the appropriate generic responses.

The hosting CSE may be:

- An N-CSE.

Table 10.3.x.7.1 <execInstance> RETRIEVE flow

|  | Description |
| --- | --- |
| Call flow type | RETRIEVE |
| Pre-conditions | Originator needs to create a resource |
| Information on Request message | op: R fr: Originator App-Inst-ID or CSE-ID to: Receiver CSE-ID cn: Name of <mgmtCmd> resource The mandatory and/or optional attributes defined in 9.6.X, as needed. |
| Local processing on Hosting CSE | If the retrieval is allowed, the N-CSE can retrieve the execution status or results on the remote entity using existing management protocol (i.e. BBF TR-069). If the retrieval is allowed, the addressed attributes of the <execInstance> resource shall be retrieved from the repository of the N-CSE. |
| Information on Response message | No change from the basic procedure (clause 10.1.1.2) |
| Post-condition | None |
| Exceptions | If the retrieval is not allowed or the specific <execInstance> resource does not exist in the N-CSE, there is no local processing on the hosting CSE and a proper error code shall be returned to AE in the Response Message. |

##### 10.3.X.9 Delete <execInstance>

The DELETE request is used by an originating AE  to delete an existing <execInstance> resource on a hosting CSE.

Originator: shall request to delete an existing <execInstance> resource by using a DELETE verb. The request shall address the specific "<execInstance>" resource of an <mgmtCmd> on the hosting CSE.

The originator shall be:

- An AE.

NOTE 1:	The hosting CSE in the network domain may also delete an <execInstance> resource locally by itself. This internal procedure is out of scope.

NOTE 2:	The <execInstance> resource may also be deleted in the hosting CSE by other offline provisioning means which are out of scope.

Receiver: shall check if the originator has the DELETE permission on the addressed <execInstance> resource. Upon successful validation, the hosting CSE shall remove the resource from its repository. If the <execInstance> is not finished on the remote entity and it is cancellable, the hosting CSE shall also use existing management protocols 
(i.e. BBF TR-069 CancelTransfer RPC) to cancel the corresponding management currently initiated at the remote entity. Then the CSE shall respond to the originator with the appropriate generic responses.

The hosting CSE shall be:

- An N-CSE.

Table 10.3.x.8.1 <execInstance> DELETE flow

|  | Description |
| --- | --- |
| Call flow type | DELETE |
| Pre-conditions | Originator needs to delete a resource |
| Information on Request message | op: D fr: Originator App-Inst-ID or CSE-ID to: Receiver CSE-ID cn: Name of <mgmtCmd> resource The mandatory and/or optional attributes defined in 9.6.X, as needed. |
| Local processing on Hosting CSE | If the <execInstance> is not finished yet and it is cancellable, the N-CSE shall cancel the <execInstance> from the remote entity using corresponding management procedures in existing management protocol (i.e. CancelTransfer RPC in BBF TR-069 [10]). The <execInstance> resource shall be deleted from the repository of the N-CSE. If the corresponding initiated commands cannot be deleted from remote entity due to some reason (e.g. not found), the <execInstance> resource shall be still deleted be performed |
| Information on Response message | No change from the basic procedure (clause 10.1.1.4) |
| Post-condition | None |
| Exceptions | If the deletion is not allowed or the specific <execInstance> resource does not exist in the N-CSE, there is no processing on the Hosting CSE and a proper error code shall be returned to AE in the Response message If the <execInstance> is already complete or it is not cancellable, there is no processing on the Hosting CSE |