| INPUT CONTRIBUTION |  |
| --- | --- |
| Group Name:* | WG5 |
| Title:* | TS-0007 Mca Device Firmware management service |
| Source:* | Weixiang Shao, ZTE Corporation, shao.weixiang@zte.com.cn |
| Contact: | Weixiang Shao, ZTE Corporation, shao.weixiang@zte.com.cn |
| Date:* | 2014-05-30 |
| Abstract:* | Add services for Device Firmware management requests across the Mca |
| Agenda Item:* | TP#11 |
| Work item(s): | TS-0007 WI |
| Document(s) Impacted* | TS-0007 |
| Intended purpose of document:* | Decision Discussion Information Other <specify> |
| Decision requested or recommendation:* | Update TS for inclusion |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

6	M2M Services	4

6.1	Introduction	4

6.2	Service Subscription	4

6.2.1	Service Capabilities	4

6.2.1.4	getManagementAdapter	4

6.2.1.4.1	Pre-conditions	4

6.2.1.4.2	Signature	4

6.2.1.4.3	Post-Conditions	4

6.2.1.4.4	Exceptions	5

6.2.1.4.5	Policies for Use	5

6.2.1.4.6	oneM2M Resource Interworking	5

6.2.1.4.7	oneM2M Roles Association	5

6.6	Device Management	6

6.6.1	Overview	6

6.6.1.1	Device Management	6

6.6.1.1.1	Supporting Rules	6

6.6.2	Service Capabilities	7

6.6.2.1	downloadFirmware	7

6.6.2.1.1	Pre-conditions	7

6.6.2.1.2	Signature - downloadFirmware	8

6.6.2.1.3	Service Interactions	8

6.6.2.1.4	Post-Conditions	9

6.6.2.1.5	Exceptions	9

6.6.2.1.6	Policies for Use	9

6.6.2.1.7	oneM2M Resource Interworking	9

6.6.2.2	installFirmware	10

6.6.2.2.1	Pre-conditions	10

6.6.2.2.2	Signature –installFirmware	10

6.6.2.2.3	Service Interactions	10

6.6.2.2.4	Post-Conditions	11

6.6.2.2.5	Exceptions	11

6.6.2.2.6	Policies for Use	11

6.6.2.2.7	oneM2M Resource Interworking	12

6.6.2.3 	getFirmwareInformation	13

6.6.2.3.1	Pre-conditions	13

6.6.2.3.2	Signature –getFirmwareInformation	13

6.6.2.3.3	Service Interactions	13

6.6.2.3.4	Post-Conditions	14

6.6.2.3.5	Exceptions	14

6.6.2.3.6	Policies for Use	14

6.6.2.3.7	oneM2M Resource Interworking	14

6.6.2.4 	getFirmwareExecStatus	15

6.6.2.4.1	Pre-conditions	15

6.6.2.4.2	Signature –getFirmwareExecInstance	15

6.6.2.4.3	Service Interactions	15

6.6.2.4.4	Post-Conditions	16

6.6.2.4.5	Exceptions	16

6.6.2.4.6	Policies for Use	16

6.6.2.4.7	oneM2M Resource Interworking	16

6.6.2.5	reportFirmwareStatus	17

6.6.2.5.1	Pre-conditions	17

6.6.2.5.2	Signature –reportFirmwareStatus	17

6.6.2.5.3	Post-Conditions	17

6.6.2.5.4	Exceptions	17

6.6.2.5.5	Policies for Use	17

6.6.2.5.6	oneM2M Resource Interworking	17

6.6.2.6	upgradeFirmware	18

6.6.2.6.1	Pre-conditions	18

6.6.2.6.2	Signature - upgradeFirmware	18

6.6.2.6.3	Service Interactions	18

6.6.2.6.4	Post-Conditions	19

6.6.2.6.5	Exceptions	19

6.6.2.6.6	Policies for Use	19

6.6.2.6.7	oneM2M Resource Interworking	19

6.7	Management Adapter	20

6.7.1	Overview	20

6.7.2	Service Capabilities	20

6.7.2.1	downloadFirmware	20

6.7.2.1.1	Pre-conditions	20

6.7.2.1.2	Signature – downloadFirmware	20

6.7.2.1.3	Post-Conditions	20

6.7.2.1.4	Exceptions	20

6.7.2.1.5	Policies for Use	20

6.7.2.1.6	oneM2M Resource Interworking	20

6.7.2.2	installFirmware	21

6.7.2.2.1	Pre-conditions	21

6.7.2.2.2	SIgnature –installFirmware	21

6.7.2.2.3	Post-Conditions	21

6.7.2.2.4	Exceptions	21

6.7.2.2.5	Policies for Use	21

6.7.2.2.6	oneM2M Resource Interworking	21

6.7.2.3	getFirmwareInformation	22

6.7.2.3.1	Pre-conditions	22

6.7.2.3.2	SIgnature –getFirmwareInformation	22

6.7.2.3.3	Post-Conditions	22

6.7.2.3.4	Exceptions	22

6.7.2.3.5	Policies for Use	22

6.7.2.3.6	oneM2M Resource Interworking	22

6.7.2.4	getFirmwareExecStatus	23

6.7.2.4.1	Pre-conditions	23

6.7.2.4.2	SIgnature –getFirmwareExecInstance	23

6.7.2.4.3	Post-Conditions	23

6.7.2.4.4	Exceptions	23

6.7.2.4.5	Policies for Use	23

6.7.2.4.6	oneM2M Resource Interworking	23

6.7.2.5	reportFirmwareStatus	24

6.7.2.5.1	Pre-conditions	24

6.7.2.5.2	Signature –reportFirmwareStatus	24

6.7.2.5.3	Service Interactions	24

6.7.2.5.4	Post-Conditions	25

6.7.2.5.5	Exceptions	25

6.7.2.5.6	Policies for Use	25

6.7.2.5.7	oneM2M Resource Interworking	25

7	M2M Service Components	26

7.1.1	Service Component Interaction Cross Reference	26

7.6	Service Exposure (SE)	26

7.6.1	Service Exposure to Service Cross Reference	26

7.7	Management Adapter(MA)	26

7.7.1	Management Adapter to Service Cross Reference	26

Annex B (Informative): Device Management Services	27

B.1	Overview	27

B.2	Supporting Services	27

B.2.2	Message Exchange	27

B.2.2.1	Service Capabilities	27

B.2.2.1.1	downloadFirmware	27

B.2.2.1.2	installFirmware	29

B.2.2.1.3	getFirmwareInformation	31

B.2.2.1.4	getFirmwareExecStatus	33

B.2.2.1.5	reportFirmwareStatus	35

B.2.2.1.6	upgradeFirmware	37

6	M2M Services

This clause describes the M2M Services provided by the M2M Services Platform.

Editor’s note – Response types should only be documented in the originating service capability and should not be duplicated (documented) in the consuming capabilities.

6.1	Introduction

The M2M Services defined in this clause are utilized by the Supporting Services described in the Annexes of this specification.

# Figure 6.1-1: oneM2M Services

6.2	Service Subscription

The Service Subscription service provides the ability to:

- Assist in returning the Management Adapter instance needed to manage device.

6.2.1	Service Capabilities

6.2.1.4	getManagementAdapter

This service capability obtains the Management Adapter instance that will be used to manage device based on the requested service capability (e.g., downloadFirmware, installFirmware) and device.

6.2.1.4.1	Pre-conditions

A correlation between a Management Adapter, the sevice capability and device exist.

6.2.1.4.2	Signature

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| subscriptionId | IN | NO | The M2M Service Subscription Identifier (M2M-Sub-ID) |
| serviceId | IN | NO | The M2M Service Identifier (M2M-Serv-ID) of the service capability |
| deviceId | IN | YES | The unique device identifier in the context of the M2M Service Subscription. |
| managementAdapter | OUT | NO | The returned instance of the Management Adapter service associated with the subscription. |
| responseType | OUT | NO | Response types: - Management Adapter does not exist |

Table 6.2.4.1.2-1 getManagementAdapter Signature

6.2.1.4.3	Post-Conditions

Not Applicable

6.2.1.4.4	Exceptions

Not Applicable

6.2.1.4.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

6.2.1.4.6	oneM2M Resource Interworking

This service capability interworks with the following oneM2M Resource operations that are interworked are:

Not Applicable

6.2.1.4.7	oneM2M Roles Association

Not Applicable

6.6	Device Management

6.6.1	Overview

The Device Management service provides for the capability for an AE to manage devices across the Mca reference point using supported device management message exchange patterns. This service utilizes management adapters to provide the management functions.

The supported device management message exchanges patterns are:

Request-Response

Request- Asynchronous Notification

6.6.1.1	Device Management

The Device Management service permits AEs to request management operations (e.g.,  device configuration operations, device troubleshooting operations, device firmware management operations, application software management operations) and provide the result or report/retrieve the status of the operations to the originating AE..

AEs have the option to request to manage a targeted set of devices that include:

Individual device

Multiple devices through the use of lists of devices

A group that designates as  list of devices using the groupID.

AEs have option  to request management operations to be performed based on a schedule.

AEs have the option to request that responses to requests be delivered synchronous (the request is held until all responses are received) or asynchronously (Reports of the operation are transmitted  to the AE when an operation has completed) to the request submission. For asynchronous requests, reports are transmitted based on a report policy configured as part of the original request.

AEs also have the capability to  request to manage devices by complex operations (e.g. upgradefirmware), where the M2M service platform can orchestrate management operations (e.g. installfirmware, downloadware) according to the business process.

6.6.1.1.1	Supporting Rules

6.6.1.1.1.1	Report Policy Parameter

The reportPolicy parameter is used to determine when a report is sent by the M2M System to the AE. The report period attribute is defined in the reportPolicy parameter.

| Parameter | Description |
| --- | --- |
| reportPeriod | Describes the period which reports for a operation are sent to the AE. |

Table 6.6.1.1.1.1-1 Parameter:reportPolicy

6.6.1.1.1.2	Aggregation Policy Parameter

The aggregationPolicy parameter  is used to determine the time period for aggregating the result of  the management operation prior to the reports being sent to the AE. The aggregation period attribute is defined in the aggregationPolicy parameter.

| Parameter | Description |
| --- | --- |
| aggregationPeriod | The time period of aggregating reports prior to transmission to the the AE. |

Table 6.6.1.1.1.2-1 Parameter: aggregationPolicy

6.6.1.1.1.3	Firmware Report Type

The FirmwareReport  is complex type that defines the current state of a firmware management operation for a specific device.

| Parameter | Optional | Description |
| --- | --- | --- |
| groupId | YES | The group identifier. |
| deviceId | NO | The unique device identifier in the context of the M2M Service Subscription. |
| operation | YES | The firmware management operation.(e.g. installFirmware, downloadFirmware) |
| operationResult | YES | The firmware management operation execution result. |
| operationStatus | YES | The firmware management operation execution status. It can be Initiated, Started, Finished, Cancelled, or Deleted. |

Table 6.6.1.1.1.3-1 Type: FirmwareReport

6.6.1.1.1.4	Orchestration Rule Type

The OrchestrationRule is a complex type that describes the target and associated schedule of the request for the execution of the management operation.

| Parameter | Optional | Description |
| --- | --- | --- |
| groupId | YES | The group identifer. |
| deviceId | NO | The unique device identifier in the context of the M2M Service Subscription. |
| operation | NO | The orchestrated operation. (e.g. installFirmware, downloadFirmware) |
| schedule | YES | The time when to execute firmware operation. If it is not provided, the operation shall be execute immediately. |

Table 6.6.1.1.1.4-1 Type: OrchestrationRule

6.6.1.1.1.5	Firmware Info Type

The FirmwareInfo is a complex type that describes the version, name, URL of the device firmware.

| Parameter | Optional | Description |
| --- | --- | --- |
| version | YES | The version of the firmware. |
| name | YES | The name of the firmware to be used on the device. |
| url | YES | The URL from which the firmware image can be downloaded. |

Table 6.6.1.1.1.5-1 Type: FirmwareInfo

6.6.1.1.1.6	Service Subscription Integration

In order for the Device Management Service to take advantage of the Management Adapter, there shall be association between the device associated with a Service Subscription and the service capability to the Management Adapter.

6.6.2	Service Capabilities

Editors note: Exceptions and responses from consumed services should not be redefined or duplicatrion. Only the originating service should document the response code or exception.

6.6.2.1	downloadFirmware

This service capability provides the ability for AEs to download device firmware.

6.6.2.1.1	Pre-conditions

The pre-conditions for MCA Received Requests are met.

A correlation between a Management Adapter, the sevice capability and device exist.

6.6.2.1.2	Signature - downloadFirmware

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| firmwareInfo | IN | NO | The device firmware information. Type FirmwareInfo, see 6.6.1.1.1.5 |
| deviceId | IN | YES | The unique device identifier in the context of the M2M Service Subscription. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.1.1.1.1. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Issuer does not have a Management Adapter for the requested operation - report policy does not supported by the Management Adapter - Operation exception: Firmware miss dependency - Operation exception: The new firmware is too large for available program or data memory |
| requestId | OUT | NO | The M2M Request Identifier (M2M-Request-ID) |
| firmwareReport | OUT | YES | The firmware management operation execution result or status. Type FirmwareReport, see 6.6.1.1.1.3. |

Table 6.6.2.1.2-1 Device Management Service –downloadFirmware capability

6.6.2.1.3	Service Interactions

The interactions of  service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

# Figure 6.6.2.1.3-1 downloadFirmware Diagram

6.6.2.1.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to download firmware.

6.6.2.1.5	Exceptions

- Missing dependency within the device for the firmware

- New firmware is too large for available program or data memory

When invoking the service capability meet these exceptions, the response feedback and no subsequence notifications. AE can continue issue the request after M2M Area network meet the requirement or invoke other service capabilities to troubleshoot related devices.

# Editors note: The highlighted text is unclear; we need to clarify the exception handling

6.6.2.1.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

6.6.2.1.7	oneM2M Resource Interworking

This service capability is used to download firmware. The service capability aligns with the <firmware> resource and maps to the UPDATE procedure for the attribute update of the resource. The service capability also aligns with the <mgmtCmd> resource and maps to the EXECUTE procedure for the resource.

6.6.2.2	installFirmware

This service capability provides the ability for AEs to install firmware on the device.

6.6.2.2.1	Pre-conditions

The pre-conditions for MCA Received Requests are met.

A correlation between a Management Adapter, the sevice capability and device exist.

# The firmware to be installed exists on the device.

6.6.2.2.2	Signature –installFirmware

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| firmwareInfo | IN | NO | The device firmware information. Type FirmwareInfo, see 6.6.1.1.1.5 |
| deviceId | IN | YES | The unique device identifier in the context of the M2M Service Subscription. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Issuer does not have a Management Adapter for the requested operation - Report policy does not supported by the Management Adapter - Operation exception: Firmware is missing dependency on the device - Operation exception: The new firmware is too large for available program or data memory |
| requestId | OUT | NO | The M2M Request Identifier (M2M-Request-ID) |
| firmwareReport | OUT | YES | The firmware management operation execution result or status. Type FirmwareReport, see 6.6.1.1.1.3. |

Table 6.6.2.2.2-1 Device Management Service –installFirmware capability

6.6.2.2.3	Service Interactions

The interactions of  service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

# Figure 6.6.2.2.3-1 installFirmware Diagram

6.6.2.2.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to install firmware.

6.6.2.2.5	Exceptions

Missing dependency of the firmware on the device

New firmware is too large for available program or data memory

When invoking the service capability meet these exceptions, the response feedback and no subsequence notifications. AE can continue issue the request after M2M Area network meet the requirement or invoke other service capabilities to troubleshoot related devices.

# Editors note: The highlighted text is unclear; we need to clarify the exception handling

6.6.2.2.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

6.6.2.2.7	oneM2M Resource Interworking

This service capability is used to install firmware. The service capability aligns with the <firmware> resource and maps to the UPDATE procedure for the attribute update of the resource. The service capability also aligns with the <mgmtCmd> resource and maps to the EXECUTE procedure for the resource.

6.6.2.3 	getFirmwareInformation

This service capability provides the ability for AEs to get device firmware information.

6.6.2.3.1	Pre-conditions

The pre-conditions for MCA Received Requests are met.

A correlation between a Management Adapter, the sevice capability and device exist.

6.6.2.3.2	Signature –getFirmwareInformation

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | YES | The unique device identifier in the context of the M2M Service Subscription. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Issuer does not have a Management Adapter for the requested operation - Report policy does not supported by the Management Adapter |
| requestId | OUT | NO | The M2M Request Identifier (M2M-Request-ID) |
| firmwareInfo | OUT | YES | The device firmware information. Type FirmwareInfo, see 6.6.1.1.1.5 |

Table 6.6.2.3.2-1 Device Management Service –getFirmwareInformation capability

6.6.2.3.3	Service Interactions

The interactions of  service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

# Figure 6.6.2.3.3-1 getFirmwareInformation Diagram

6.6.2.3.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get firmware information.

6.6.2.3.5	Exceptions

Not Applicable

6.6.2.3.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

6.6.2.3.7	oneM2M Resource Interworking

This service capability is used to get firmware information. The service capability aligns with the <firmware> resource and maps to the RETRIEVE procedure for the attribute version, name, URL of the resource.

6.6.2.4 	getFirmwareExecStatus

This service capability provides the ability for AEs to get state of firmware management operation for a specific device.

6.6.2.4.1	Pre-conditions

The pre-conditions for MCA Received Requests are met.

A correlation between a Management Adapter, the sevice capability and previously submitted firmware request exist.

6.6.2.4.2	Signature –getFirmwareExecInstance

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| operationRequestId | IN | NO | The M2M Request Identifier ofpreviously submitted firmware request (M2M-Request-ID) |
| deviceId | IN | YES | The unique device identifier in the context of the M2M Service Subscription. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Issuer does not have a Management Adapter for the requested operation - Report policy is not supported by the Management Adapter - Thepreviously submitted firmware request does not exist |
| requestId | OUT | NO | The M2M Request Identifier (M2M-Request-ID) |
| firmwareReport | OUT | YES | The firmware management operation execution result or status. Type FirmwareReport, see 6.6.1.1.1.3. |

Table 6.6.2.4.2-1 Device Management Service –getFirmwareExecInstance capability

6.6.2.4.3	Service Interactions

The interactions of  service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

# Figure 6.6.2.3.3-1 getFirmwareExecStatus Diagram

6.6.2.4.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get firmware state of management operation.

6.6.2.4.5	Exceptions

Not Applicable

6.6.2.4.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

6.6.2.4.7	oneM2M Resource Interworking

This service capability is used to get state of firmware management operation. The service capability aligns with the <firmware> resource and maps to the RETRIEVE procedure for the attribute updateStatus of the resource. The service capability also aligns with the <execInstance> resource and maps to the RETRIEVE procedure for the attribute execStatus, execResult of the resource.

6.6.2.5	reportFirmwareStatus

This service capability provides the ability to report the firmware management operation execution result or status information to the AE.

6.6.2.5.1	Pre-conditions

A correlation between a Management Adapter, the sevice capability and previously submitted firmware request exist.

6.6.2.5.2	Signature –reportFirmwareStatus

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| isLastReport | IN | NO | Boolean, whether it is the last report. |
| sequenceNumber | IN | NO | The report sequence number. |
| firmwareReportList | IN | NO | Array of firmwareReport. Type FirmwareReport, see 6.6.1.1.1.3. |
| aggregationPolicy | IN | YES | The policy used to aggregate the result of the management operation prior to the reports being sent. See clause 6.6.1.1.1.2. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - The target AE does not exist. - Aggregation policy does not supported |

Table 6.3.2.5.2-1 Device Management Service –reportFirmwareStatus capability

6.6.2.5.3	Post-Conditions

The AE has received a report of firmware operation execution status or result.

Editor’s note;  There are post conditions that requires the M2M System to maintain the “state” of the report (i.e., sequence number.

6.6.2.5.4	Exceptions

Not Applicable

6.6.2.5.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

6.6.2.5.6	oneM2M Resource Interworking

Not Applicable

6.6.2.6	upgradeFirmware

This service capability is a complex operation for AEs to upgrade firmware on one or more devices. The service capability utlizes orchestration rules for the related management operations such as downloadFirmware, installFirmware, getFirmwareInformation, getFirmwareExecInstance and reportFirmwareStatus according to the business process.

This service capability permits AEs to upgrade the firmware on individual device, multiple devices or a group of devices. In addition the upgrade of the firmware is permitted based on a schedule for each of the operations.

6.6.2.6.1	Pre-conditions

The pre-conditions for MCA Received Requests are met.

A correlation between Management Adapter, the sevice capability and devices or device group exist.

6.6.2.6.2	Signature - upgradeFirmware

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| orchestrationRuleList | IN | NO | List of OrchestrationRule. Type OrchestrationRule, see clause 6.6.1.1.1.4. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.1.1.1.1. |
| aggregationPolicy | IN | YES | The policy used to aggregate the result of the management operation prior to the reports being sent. See clause 6.6.1.1.1.2. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Report policy is not supported by the Management Adapter - Aggregation policy is not supported by the Management Adapter - The Orchestration Rules are not supported by the Management Adapter - Operation exception: The device’s reachability schedule is inconsistent with the schedule of the Orchestration rule. |
| requestId | OUT | NO | The M2M Request Identifier (M2M-Request-ID) |
| firmwareReportList | OUT | YES | Array of firmwareReport. Type FirmwareReport, see 6.6.1.1.1.3. |

Table 6.6.2.6.2-1 Device Management Service –upgradeFirmware capability

# Editors Note: Need to clarify the device’s reachability schedule.

6.6.2.6.3	Service Interactions

The interactions of  service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

# Figure 6.6.2.1.6-1 upgradeFirmware Diagram

6.6.2.6.4	Post-Conditions

The Management Adapter has submitted a set of requests to the Management Server to manage firmware.

6.6.2.6.5	Exceptions

The device’s reachability schedule is inconsistent with the schedule of  the Orchestration rule.

6.6.2.6.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

6.6.2.6.7	oneM2M Resource Interworking

Not Applicable

6.7	Management Adapter

6.7.1	Overview

The Management Adapter service provides the ability to adapt the M2M Service Layer operations to the technology specific operations of the Management Server.

6.7.2	Service Capabilities

6.7.2.1	downloadFirmware

This service capability provides the ability to download the specific device firmware.

6.7.2.1.1	Pre-conditions

A correlation between a Management Server, the sevice capability and device exist.

6.7.2.1.2	Signature – downloadFirmware

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| firmwareInfo | IN | NO | The device firmware information. Type FirmwareInfo, see 6.6.1.1.1.5 |
| deviceId | IN | YES | The unique device identifier in the context of the M2M Service Subscription. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.1.1.1.1. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Report policy is not supported by the Management Adapter |
| requestId | IN | NO | The M2M Request Identifier (M2M-Request-ID) |
| firmwareReport | OUT | YES | The firmware management operation execution result or status. Type FirmwareReport, see 6.6.1.1.1.3. |

Table 6.7.2.1.2-1 Management Adapter –downloadFirmware capability

6.7.2.1.3	Post-Conditions

The Management Adapter has submitted a request to the Management Server to download the firmware.

6.7.2.1.4	Exceptions

New firmware is too large for available program or data memory

When these exceptions are thrown, the requested operation is completed and any corresponding reports for the device are not sent.

Editors Note: These exceptions should be considered as whether they are exceptions or should be error response code. Exceptions stop processing immediately; errors response conplere the request-response interaction. Both will be reported to the target AE.

6.7.2.1.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

6.7.2.1.6	oneM2M Resource Interworking

This service capability is used to download firmware. The service capability aligns with the <firmware> resource and maps to the UPDATE procedure for the attribute update of the resource. The service capability also aligns with the <mgmtCmd> resource and maps to the EXECUTE procedure for the resource.

6.7.2.2	installFirmware

This service capability provides the ability to install firmware on a device.

6.7.2.2.1	Pre-conditions

A correlation between a Management Server, the sevice capability and device exist.

6.7.2.2.2	SIgnature –installFirmware

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| firmwareInfo | IN | NO | The device firmware information. Type FirmwareInfo, see 6.6.1.1.1.5 |
| deviceId | IN | YES | The unique device identifier in the context of the M2M Service Subscription. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Issuer does not have a Management Adapter for the requested operation - Report policy does not supported by the Management Adapter |
| requestId | IN | NO | The M2M Request Identifier (M2M-Request-ID) |
| firmwareReport | OUT | YES | The firmware management operation execution result or status. Type FirmwareReport, see 6.6.1.1.1.3. |

Table 6.7.2.2.2-1 Management Adapter –installFirmware capability

6.7.2.2.3	Post-Conditions

The Management Adapter has submitted a request to the Management Server to install firmware.

6.7.2.2.4	Exceptions

Missing dependency in the device for the firmware and requested operation

New firmware is too large for available program or data memory

When these exceptions are thrown, the requested operation is completed and any corresponding reports for the device are not sent.

6.7.2.2.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

6.7.2.2.6	oneM2M Resource Interworking

This service capability is used to install firmware. The service capability aligns with the <firmware> resource and maps to the UPDATE procedure for the attribute update of the resource. The service capability also aligns with the <mgmtCmd> resource and maps to the EXECUTE procedure for the resource.

6.7.2.3	getFirmwareInformation

This service capability provides the ability to retrieve the information related to the firmware on a device..

6.7.2.3.1	Pre-conditions

A correlation between a Management Server, the sevice capability and device exist.

6.7.2.3.2	SIgnature –getFirmwareInformation

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | YES | The unique device identifier in the context of the M2M Service Subscription. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - Report policy is not supported by the Management Adapter |
| requestId | IN | NO | The M2M Request Identifier (M2M-Request-ID) |
| firmwareInfo | OUT | YES | The device firmware information. Type FirmwareInfo, see 6.6.1.1.1.5 |

Table 6.7.2.3.2-1 Management Adapter –getFirmwareInformation capability

6.7.2.3.3	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get the firmware information.

6.7.2.3.4	Exceptions

Not Applicable

6.7.2.3.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

6.7.2.3.6	oneM2M Resource Interworking

This service capability is used to get firmware information. The service capability aligns with the <firmware> resource and maps to the RETRIEVE procedure for the attribute version, name, URL of the resource.

6.7.2.4	getFirmwareExecStatus

This service capability provides the ability to get state of firmware management operation for a specific device.

6.7.2.4.1	Pre-conditions

A correlation between a Management Server, the sevice capability and previously submitted firmware request exist.

6.7.2.4.2	SIgnature –getFirmwareExecInstance

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| operationRequestId | IN | NO | The M2M Request Identifier of previously submitted firmware request (M2M-Request-ID) |
| deviceId | IN | YES | The unique device identifier in the context of the M2M Service Subscription. When provided the FirmwareReport type is generated for only the specified device. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - The previously submitted firmware request does not exist |
| firmwareReports | OUT | YES | The list of firmware management operation execution result or status for each device in the operation request. Type FirmwareReport, see 6.6.1.1.1.3. |

Table 6.7.2.4.2-1 Management Adapter –getFirmwareExecStatus capability

6.7.2.4.3	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get the  firmware operation execution status or result for each device in the operation request.

6.7.2.4.4	Exceptions

Not Applicable

6.7.2.4.5	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

6.7.2.4.6	oneM2M Resource Interworking

This service capability is used to get state of firmware management operation. The service capability aligns with the <firmware> resource and maps to the RETRIEVE procedure for the attribute updateStatus of the resource. The service capability also aligns with the <execInstance> resource and maps to the RETRIEVE procedure for the attribute execStatus, execResult of the resource.

6.7.2.5	reportFirmwareStatus

This service capability provides the ability for the Management Adapter to report the firmware management operation execution result or status information. The Management Adapter can send one or multiple firmware reports according to the aggregation policy.

6.7.2.5.1	Pre-conditions

A correlation between a Management Server, the sevice capability and previously submitted firmware requests exist.

6.7.2.5.2	Signature –reportFirmwareStatus

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| to | IN | NO | The identifier of the AE that is to receive the data(M2M-AppInst-ID) |
| isLastReport | IN | NO | Boolean, whether it is the last report. |
| sequenceNumber | IN | NO | The report sequence number. |
| firmwareReportList | IN | NO | Array of firmwareReport. Type FirmwareReport, see 6.6.1.1.1.3. |
| aggregationPolicy | IN | YES | The policy used to aggregate the result of the management operation prior to the reports being sent. See clause 6.6.1.1.1.2. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - The target AE does not exist. - Aggregation policy does not supported |

Table 6.7.2.5.2-1 Management Adapter –reportFirmwareStatus capability

6.7.2.5.3	Service Interactions

The interactions of  service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

# Figure 6.7.2.5.3-1 firmwareReport Diagram

6.7.2.5.4	Post-Conditions

Not Applicable

# Editors note: We need to clarify the post conditions around the sequence number of the report.

6.7.2.5.5	Exceptions

Not Applicable

6.7.2.5.6	Policies for Use

Message Exchange Patterns: Not Applicable

Transaction Pattern: Not Applicable

6.7.2.5.7	oneM2M Resource Interworking

Not Applicable

7	M2M Service Components

This clause describes the M2M Service Components provided by the M2M Services Platform.

7.1.1	Service Component Interaction Cross Reference

| Service Component | INF | SSUB | TRA | ACC | SE | MA |
| --- | --- | --- | --- | --- | --- | --- |
| INF | NA | C |  |  |  |  |
| SSUB | P | NA |  |  |  |  |
| TRA |  | C | NA |  | C |  |
| ACC |  |  |  | NA |  |  |
| SE |  |  | P |  | NA | P |
| MA |  | C |  |  | C | NA |
| Legend: |  |  |  |  |  |  |
| NA: Not Applicable |  |  |  |  |  |  |
| C: Left hand Service Component consumes services |  |  |  |  |  |  |
| P: Left hand Service Component provides services |  |  |  |  |  |  |

Table 7.1-1 Service Component Cross Reference

7.6	Service Exposure (SE)

The SE Component acts as the primary interface for AEs to interact with the M2M Service Layer across the Mca reference point.

7.6.1	Service Exposure to Service Cross Reference

SE Device Management: All Service Capabilities

Service Subscription: getManagementAdapter

Management Adapter: All Service Capabilities

7.7	Management Adapter(MA)

The Management Adapter component provides Services are used to integrate or adapt Device Management requests to the Management Servers of existing device management technologies(e.g., TR-069, OMA-DM, and LWM2M) .

7.7.1	Management Adapter to Service Cross Reference

Management Adapter: All Service Capabilities

SE: Device Management: reportFirmwareStatus

Annex B (Informative): Device Management Services

B.1	Overview

This annex illustrates the usage of the services for  requests to manage device from AEs across the Mca Reference Point.

B.2	Supporting Services

B.2.2	Message Exchange

This service provides the capability to manage device using the request-response or request-asynchronous notification message patterns where AEs:

Request to manage device using device specific service capabilities or more complex service capabilies that the Support Service will orchestrate into the individual service capabilities according to their business process.

Request to obtain management operation execution results or status using the reporting capability. Reports can be generated immediately or periodically based on the report policy. Reports can also be aggregated prior to reporting.

B.2.2.1	Service Capabilities

B.2.2.1.1	downloadFirmware

This service capability provides the ability to execute a downloadFirmware request from an AE.

B.2.2.1.1.1	Pre-conditions

The pre-conditions for MCA Received Requests are met.

A correlation between a Management Adapter, the sevice capability and device exist.

B.2.2.1.1.2	Signature - downloadFirmware

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| firmwareInfo | IN | NO | The device firmware information. Type FirmwareInfo, see 6.6.1.1.1.5 |
| deviceId | IN | YES | The unique device identifier in the context of the M2M Service Subscription. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.1.1.1.1. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. |
| firmwareReport | OUT | YES | The firmware management operation execution result or status. Type FirmwareReport, see 6.6.1.1.1.3. |

Table B.2.2.1.1.2-1 Device Management Service –downloadFirmware capability

B.2.2.1.1.3	Service Interactions

The interactions of  service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca reference point

- Retrieves the Management Adapter to be used for the device and Service Subscription

- Issues the request to the Management Adapter to download the firmware

- Records the event

# Figure B.2.2.1.1.3-1 downloadFirmware Interaction Diagram

B.2.2.1.1.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to download firmware.

# The event has been recorded.

B.2.2.2.1.5	Exceptions

# Not Applicable

B.2.2.2.1.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern:  Participation allowed

Maximum Response: 300ms

B.2.2.1.2	installFirmware

This service capability provides the ability to execute a installFirmware request from an AE.

B.2.2.1.2.1	Pre-conditions

The pre-conditions for MCA Received Requests are met.

A correlation between a Management Adapter, the sevice capability and device exist.

B.2.2.1.2.2	Signature - installFirmware

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| firmwareInfo | IN | NO | The device firmware information. Type FirmwareInfo, see 6.6.1.1.1.5 |
| deviceId | IN | YES | The unique device identifier in the context of the M2M Service Subscription. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. |
| firmwareReport | OUT | YES | The firmware management operation execution result or status. Type FirmwareReport, see 6.6.1.1.1.3. |

Table B.2.2.1.2.2-1 Device Management Service –installFirmware capability

B.2.2.1.2.3	Service Interactions

The interactions of  service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca reference point

- Retrieves the Management Adapter to be used for the device and Service Subscription

- Issues the request to the Management Adapter to install the firmware

- Records the event

# Figure B.2.2.1.2.3-1 installFirmware Interaction Diagram

B.2.2.1.2.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to install firmware.

# The event has been recorded.

B.2.2.1.3.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

B.2.2.1.3.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

B.2.2.1.3	getFirmwareInformation

This service capability provides the ability to execute a getFirmwareInformation request from an AE.

B.2.2.1.3.1	Pre-conditions

The pre-conditions for MCA Received Requests are met.

A correlation between a Management Adapter, the sevice capability and device exist.

B.2.2.1.3.2	Signature - getFirmwareInformation

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | YES | The unique device identifier in the context of the M2M Service Subscription. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. |
| firmwareInfo | OUT | YES | The device firmware information. Type FirmwareInfo, see 6.6.1.1.1.5 |

Table B.2.2.1.3.2-1 Device Management Service –getFirmwareInformation capability

B.2.2.1.3.3	Service Interactions

The interactions of  service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca reference point

- Retrieves the Management Adapter to be used for the device and Service Subscription

- Issues the request to the Management Adapter to get the firmware information

- Records the event

# Figure B.2.2.1.3.3-1 getFirmwareInformation Interaction Diagram

B.2.2.1.3.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get firmware information.

# The event is recorded

B.2.2.1.3.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

B.2.2.1.3.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

B.2.2.1.4	getFirmwareExecStatus

This service capability provides the ability to retrieve the execution status or result of a requested firmware operation.

B.2.2.1.4.1	Pre-conditions

The pre-conditions for MCA Received Requests are met.

A correlation between a Management Adapter, the sevice capability and previously submitted firmware request exist.

B.2.2.1.4.2	Signature - getFirmwareExecStatus

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| operationRequestId | IN | NO | The M2M Request Identifier of previously submitted firmware request (M2M-Request-ID) |
| deviceId | IN | YES | The unique device identifier in the context of the M2M Service Subscription. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. - The previously submitted firmware request does not exist |
| firmwareReport | OUT | YES | The firmware management operation execution result or status. Type FirmwareReport, see 6.6.1.1.1.3. |

Table B.2.2.1.4.2-1 Device Management Service –getFirmwareExecStatus capability

B.2.2.1.4.3	Service Interactions

The interactions of  service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca reference point

- Retrieves the Management Adapter to be used for the device and Service Subscription

- Issues the request to the Management Adapter to retrieve the firmware operations execution status or result

- Records the event

# Figure B.2.2.1.4.3-1 getFirmwareExecStatus Interaction Diagram

B.2.2.1.4.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get firmware operation execution status or result.

# Record the event.

B.2.2.1.4.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

B.2.2.1.4.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

B.2.2.1.5	reportFirmwareStatus

This service capability provides the ability to notify an AE about the status or result of a firmware operation for a previously submitted operation request.

B.2.2.1.5.1	Pre-conditions

The pre-conditions for MCA Received Requests are met.

A correlation between a Management Adapter, the sevice capability and previously submitted firmware requests exist.

B.2.2.1.5.2	Signature - reportFirmwareStatus

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| isLastReport | IN | NO | Boolean, whether it is the last report. |
| sequenceNumber | IN | NO | The report sequence number. |
| firmwareReportList | IN | NO | Array of firmwareReport. Type FirmwareReport, see 6.6.1.1.1.3. |
| aggregationPolicy | IN | YES | The policy used to aggregate the result of the management operation prior to the reports being sent. See clause 6.6.1.1.1.2. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. |

Table B.2.2.1.5.2-1 Device Management Service –reportFirmwareStatus capability

B.2.2.1.5.3	Service Interactions

The interactions of  service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca reference point

- Record the event.

Figure B.2.2.1.5.3-1 reportFirmwareStatus Interaction Diagram

B.2.2.1.5.4	Post-Conditions

The AE has received a report of firmware operation execution status or result.

# The event is recorded.

B.2.2.1.5.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

B.2.2.1.5.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300ms

B.2.2.1.6	upgradeFirmware

This service capability permits AEs to upgrade the firmware on individual device, multiple devices or a group of devices. In addition the upgrade of the firmware is permitted based on a schedule for each of the operations

B.2.2.1.6.1	Pre-conditions

The pre-conditions for MCA Received Requests are met.

A correlation between Management Adapter, the sevice capability and devices or device group exist.

B.2.2.1.6.2	Signature - upgradeFirmware

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| orchestrationRuleList | IN | NO | List of OrchestrationRule. Type OrchestrationRule, see clause 6.6.1.1.1.4. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.1.1.1.1. |
| aggregationPolicy | IN | YES | The policy used to aggregate the result of the management operation prior to the reports being sent. See clause 6.6.1.1.1.2. |
| responseType | OUT | YES | Unique response types for this service. Note: Consumed services also provide response types. |
| firmwareReportList | OUT | YES | List of firmware reports. Type FirmwareReport, see 6.6.1.1.1.3. |

Table B.2.2.1.6.2-1 Device Management Service –upgradeFirmware capability

B.2.2.1.6.3	Service Interactions

The interactions of  service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca reference point

- Retrieves the Management Adapter to be used for the device and Service Subscription

- Issues a series of  operations to the Management Adapter to upgrade the firmware on a the devices in the orchestration rule list. Each entry in the orchestration rule list contains the operation, the devices and schedule for the operation.

- Records the event

- Aggregate the report status or results

- Records the event

# Figure B.2.2.1.6.3-2 upgradeFirmware Interaction Diagram

# Editors Note: We need a common service capability for Aggregate reports. Flow needs to account for the Report Policy.

# Aggregate reports need further study.

B.2.2.1.6.4	Post-Conditions

The Management Adapter has submitted a set of requests to the Management Server to upgrade the firmware.

# The event is recorded.

B.2.2.2.6.5	Exceptions

The reachability schedule of the device doesn not match the schedule of the orchestration rule.

B.2.2.2.6.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern:  Participation allowed

Maximum Response: 300ms