# 7	Mapping of resources

This clause contains all information on how to map management resources from TS-0004 [2] to managed objects and parameters as defined in the TR-181 [6] data model or the Remote Procedure Calls (RPCs) in TR-069 [4].

## 7.1	General mapping assumptions

TR-069 [4] specifies a protocol for communication between a CPE (Customer Premises Equipment) and an ACS (Auto-Configuration Server).  Any TR-069 enabled device has to follow the data model as described in the TR-106 [5] and TR-181 [6] as well as RPCs described in TR-069 [4].

As TR-181 [6] is the model that the Resources are mapped, all Resources shall have the bjected of the TR-181[6] namespace (e.g., “urn:broadband-forum-org:tr-181-2-7-0”).

### 7.1.1	Mapping of Device identifiers

The Device identifiers for CPEs are mapped to the Resource Typse [deviceInfo].

For CPE and Virtual Devices map their Device Identifiers (OUI-[PC-]SN) to the manufacturer, deviceType and deviceLabel attributes of the Resource [deviceInfo].

For Embedded Devices, the ControllerID and ProxiedDeviceID parameters of the Device.ManagementServer.EmbeddedDevice.{i} object instance are mapped to the deviceLabel attribute of the Resource [deviceInfo] as a comma separated list: “Device.ManagementServer.EmbeddedDevice.{i}.ControllerID, Device.ManagementServer.EmbeddedDevice.{i}.ProxiedDeviceID”.

### 7.1.2	Mapping of Embedded Devices

The TR-181 [6] specification does not provide a mechanism where Embedded Devices provide information related to the Device.DeviceInfo objects and sub-objects. Instead the TR-181 [6] provides this information in a manner that is reliant on the Embedded Device’s underlying technology (e.g., ZigBee®, UpnP).

As such the mapping of the [memory] and [battery] Resources are implementation specific for each underlying technology and is outside the scope of this specification.

## 7.2	Resource [deviceInfo]

The Resource [deviceInfo] is a read-only Resource that shall map to the Device.DeviceInfo object of TR-181 [6] for CPE and Virtual Devices.

The information shall be retrieved using the GetParameterValues RPC of TR-069 [4].

Note: The SerialNumber, ModelNumber, ProductClass attributes for a Virtual device are the same values as the Device.ManagementServer.VirtualDevice.{i} object in the CPE Proxier.

Table 7.2-1: Resource [deviceInfo] for CPE and Virtual Devices

| Attribute Name of [deviceInfo] | TR-181 Parameter |
| --- | --- |
| deviceLabel | Device.DeviceInfo.SerialNumber |
| manufacturer | Device.DeviceInfo.Manufacturer |
| model | Device.DeviceInfo.ModelNumber |
| deviceType | Device.DeviceInfo.ProductClass |
| fwVersion | Device.DeviceInfo.SoftwareVersion if the device supports only 1 software version. If the device support multiple software versions this shall map to Device.DeviceInfo.AdditionalSoftwareVersion |
| swVersion | Device.DeviceInfo.SoftwareVersion |
| hwVersion | Device.DeviceInfo.HardwareVersion |

Table 7.2-2: Resource [deviceInfo] for Embedded Devices

| Attribute Name of [deviceInfo] | TR-181 Parameter |
| --- | --- |
| deviceLabel | Comma separated list: “Device.ManagementServer.EmbeddedDevice.{i}.ControllerID, Device.ManagementServer.EmbeddedDevice.{i}.ProxiedDeviceID |
| manufacturer | No mapping available |
| model | No mapping available |
| deviceType | No mapping available |
| fwVersion | No mapping available |
| swVersion | No mapping available |
| hwVersion | No mapping available |

## 7.3	Resource [memory]

The Resource [memory] is a read-only Resource that shall map to the Device.DeviceInfo.MemoryStatus object of TR-181 [6] for CPE and Virtual Devices.

The information shall be retrieved using the GetParameterValues RPC of TR-069 [4].

Attempts to modify the attributes of the memory Resource causes an error code “operation unsupported” to be returned.

Table 7.3-1: Resource [memory]

| Attribute Name of [memory] | TR-181 Parameter |
| --- | --- |
| memAvailable | Device.DeviceInfo.MemoryStatus.Free |
| memTotal | Device.DeviceInfo.MemoryStatus.Total |

## 7.4	Resource [battery]

The Resource [battery]  is a read-only Resource that shall map to an instance of Device.DeviceInfo.X_oneM2M_org_BatteryStatus.Battery.{i} object for CPE and Virtual Devices.

The information shall be retrieved using the GetParameterValues RPC of TR-069 [4].

Table 7.4-1: Resource [battery]

| Attribute Name of [battery] | TR-181 Parameter |
| --- | --- |
| batteryLevel | Device.DeviceInfo.X_oneM2M_org_BatteryStatus.Battery.{i}.Level |
| batteryStatus | Device.DeviceInfo.X_oneM2M_org_BatteryStatus.Battery.{i}.Status |

## 7.5	Resource [areaNwkInfo]

The Resource [areaNwkInfo] is a multi-instance Resource where each instance of the Resource shall map to an instance of Device.X_oneM2M_org_CSE.{i}.M2MareaNetwork.{i} object.

As the Resource [areaNwkInfo] is a multi-instance Resource, the M2MareaNetwork object is a multi-object instance that can be created and deleted.

The M2MareaNetwork instance shall be created  using the Add Object RPC of TR-069 [4].

The M2MareaNetwork instance shall be deleted  using the Delete Object RPC of TR-069 [4].

The information of an M2MareaNetwork shall be retrieved using the GetParameterValues RPC of TR-069 [4].

The information of an M2MareaNetwork shall be modified using the SetParameterValues RPC of TR-069 [4].

Table 7.5-1: Resource [areaNwkInfo]

| Attribute Name of [areaNwkInfo] | X_oneM2M_org Parameter |
| --- | --- |
| areaNwkType | Device.X_oneM2M_org_CSE.{i}.M2MareaNetwork.{i}.Type |
| listOfDevices | Device.X_oneM2M_org_CSE.{i}.M2MareaNetwork.{i}.ListOfDevices |

## 7.6	Resource [areaNwkDeviceInfo]

The Resource [areaNwkDeviceInfo] is a multi-instance Resource where each instance of the Resource shall map to an instance of Device.X_oneM2M_org_CSE.{i}.AreaNetworkDevice.{i} object.

As the Resource [areaNwkDeviceInfo] is a multi-instance Resource, the AreaNetworkDevice object is a multi-object instance that can be created and deleted.

Instances of the Resource [areaNwkDeviceInfo]  are referenced in the listOfDevices attribute of the associated Resource [areaNwkInfo].

The M2MareaNetworkDevice instance shall be created  using the Add Object RPC of TR-069 [4].

The M2MareaNetworkDevice instance shall be deleted  using the Delete Object RPC of TR-069 [4].

The information of an M2MareaNetworkDevice shall be retrieved using the GetParameterValues RPC of TR-069 [4].

The information of an M2MareaNetworkDevice shall be modified using the SetParameterValues RPC of TR-069 [4].

Table 7.6-1: Resource [areaNwkDeviceInfo]

| Attribute Name of [areaNwkDeviceInfo] | X_oneM2M_org Parameter |
| --- | --- |
| devId | Device.X_oneM2M_org_CSE.{i}.M2MareaNetworkDevice.{i}.Host |
| devType | Device.X_oneM2M_org_CSE.{i}.M2MareaNetworkDevice.{i}.Type |
| areaNwkId | Reference to Device.X_oneM2M_org_CSE.{i}.M2MareaNetworkDevice.{i}.M2MareaNetwork |
| sleepInterval | Device.X_oneM2M_org_CSE.{i}.M2MareaNetworkDevice.{i}.SleepInterval |
| sleepDuration | Device.X_oneM2M_org_CSE.{i}.M2MareaNetworkDevice.{i}.SleepDuration |
| status | Device.X_oneM2M_org_CSE.{i}.M2MareaNetworkDevice.{i}.Status |
| listOfNeighbors | Device.X_oneM2M_org_CSE.{i}.M2MareaNetworkDevice.{i}.Neighbors |

## 7.7	Resource [eventLog]

The Resource [eventLog] is a multi-instance Resource where each instance of the Resource shall map to an instance of Device.DeviceInfo.X_oneM2M_org_Diagnostics.EventLog.{i} object.

The EventLog instance shall be created using the Add Object RPC of TR-069 [4].

The EventLog instance shall be deleted using the Delete Object RPC of TR-069 [4].

The information of an EventLog instance shall be retrieved using the GetParameterValues RPC of TR-069 [4].

The information of an EventLog instance shall be updated using the SetParameterValues RPC of TR-069 [4].

Table 7.7-1: Resource [eventLog]

| Attribute Name of [eventLog] | TR-181 Parameter |
| --- | --- |
| logTypeId | Device.DeviceInfo.X_oneM2M_org_Diagnostics.EventLog.{i}.Type |
| logData | Device.DeviceInfo.X_oneM2M_org_Diagnostics.EventLog.{i}.Data |
| logStatus | Device.DeviceInfo.X_oneM2M_org_Diagnostics.EventLog.{i}.Status |
| logStart | Set to “True” , the Device.DeviceInfo.X_oneM2M_org_Diagnostics.EventLog.{i}.Enable parameter is set to “True”. |
| logStop | Set to “True” , the Device.DeviceInfo.X_oneM2M_org_Diagnostics.EventLog.{i}.Enable parameter is set to “False”. |

## 7.8	Resource [deviceCapability]

The Resource [deviceCapability] represents a capability of device that can be administratively enabled or disabled. The lists of capabilities that are managed are defined in the enumeration of the capabilityName attribute.  The TR-181 [6] data model defines a subset of capabilities listed in the deviceCapability enumeration. The supported device capabilities within TR-181 [6] include:

LAN Interfaces: USB, Wi-Fi, HomePlug, MoCA, UPA

Hardware Capabilities: SmartCardReader

The information shall be retrieved using the GetParameterValues RPC of TR-069 [4].

The capabilities shall be enabled and disabled using the SetParameterValues RPC of TR-069 [4].

Table 7.8-1: Resource [capabilityInstance]

| Attribute Name of [capabilityInstance] | TR-181 Parameter |
| --- | --- |
| capabilityName | This attribute is fixed based on the value of the capabilityName attribute. |
| attached | Returns “True” |
| capabilityActionStatus | Status is defined as: - Success if the SetParameterValues RPC indicates that the operation was sucessfull. - Failure if the response to the SetParameterValues RPCs indicates that the operation failed. - In process if the SetParameterValues RPC is initiatied but the response to the SetParameterValues RPC has not been received. |
| currentState | USB: Device.USB.Interface.{i}.Enable Wi-Fi: Device.Wi-Fi.Radio.{i}.Enable HomePlug: Device.HomePlug.Interface.{i}.Enable MoCA: Device.MoCA.Interface.{i}.Enable UPA: Device.UPA.Interface.{i}.Enable SmartCardReader: Device.SmartCardReaders.SmartCardReader.{i}.Enable |
| enable | USB: Device.USB.Interface.{i}.Enable Wi-Fi: Device.Wi-Fi.Radio.{i}.Enable HomePlug: Device.HomePlug.Interface.{i}.Enable MoCA: Device.MoCA.Interface.{i}.Enable UPA: Device.UPA.Interface.{i}.Enable SmartCardReader: Device.SmartCardReaders.SmartCardReader.{i}.Enable |
| disable | Same parameter is used to disable a capability as the enable attribute. |

## 7.9	Resource [firmware]

The Resource [firmware] represents a firmware instance and is not considered a TR-069 managed entity within the device until the firmware Resource’s update attribute has been written a value of “True”. When this occurs, the TR-069 Download RPC shall be invoked.

Note: In many instances, the server from which the firmware is downloaded requires authentication in the form of Username and Password credentials. The CSE that executes firmware download shall maintain the mapping of the username and password of the download server needed to download the firmware outside the lifecycle of the specific firmware.

Table 7.9-1: Resource [firmware]

| Attribute Name of [firmware] | RPC Download Arguments |
| --- | --- |
| URL | URL |
| update | When set to the value of “True” executes the Download operations with a FileType “1 Firmware Upgrade Image” is performed. |
|  | Username: Received from the CSE for the download server where the update is set to “True”. |
|  | Password: Received from the CSE for the download server where the update is set to “True”. |
|  | CommandKey: Automatically set by the CSE where the update is set to “True” in order to correlate the TransferComplete response. |
|  | FileSize: 0 (not used) |
|  | TargetFileName: <empty> (not used) |
|  | DelaySeconds: 0 (immediate) |
|  | SuccessURL: <empty> (not used) |
|  | FailureURL: <empty> (not used) |

## 7.10	Resource [software]

The Resource [software] is a multi-instance Resource where each instance of the Resource maps directly to an instance of Device.SoftwareModules.DeploymentUnit.{i} object for the deployment aspects (install, uninstall) of the Resource [software].  The install and uninstall operation of the Resource [software] is performed using a combination of the ChangeDUState and ChangeDUStateComplete RPCs.

Once a Resource [software] has been installed, the Resource shall be mapped to the associated Device.SoftwareModules.ExecutionUnit.{i} objects in order to activate and deactivate the associated execution unit.

The Resource [software] version and name shall be retrieved using the GetParameterValues RPC of TR-069 [4].

The activate and deactivate operations of the Resource [software] shall be performed by manipulating the Device.SoftwareModules.ExecutionUnit.{i}.RequestedState parameter using the SetParameterValues RPC.

Note: The Resource [software] provides support for only 1 Execution Unit per Deployment Unit. If a Deployment Unit is discovered by the M2M Service Layer that contains multiple Execution Units for a Deployment Unit; only 1 Execution Unit is exposed. The selection of which Execution Unit is implementation specific.

Table 7.10-1: Resource [software]

| Attribute Name of [software] | Description |
| --- | --- |
| version | Device.SoftwareModules.DeploymentUnit.{i}.Version |
| name | Device.SoftwareModules.DeploymentUnit.{i}.Name |
| URL | Device.SoftwareModules.DeploymentUnit.{i}.URL |
| install | Use the ChangeDUState:InstallOpStruct |
| installStatus | Status is defined as: - Success if the ChangeDUStateComplete RPC indicates that the operation was sucessfull. - Failure if the response to the ChangeDUState or ChangeDUStateComplete RPCs indicates that the operation failed. - In process if the ChangeDUState RPC is initiatied but the ChangeDUStateComplete RPC has not been received. |
| activate | The action that activates software previously installed. |
| deactivate | The action that deactivates software. |
| activeStatus | Status is defined as: - Success if the SetParameterValues RPC indicates that the operation was sucessfull. - Failure if the response to the SetParameterValues RPCs indicates that the operation failed. - In process if the SetParameterValues RPC is initiatied but the response to the SetParameterValues RPC has not been received. |

Table 7.10-2: RPC ChangeDUState:InstallOpStruct Arguments

RPC ChangeDUState:InstallOpStruct Argument

URL: URL of the Server that M2M Node uses to download the DU.

Username: Username credential of Server that the CPE uses to download the DU – Supplied by the CSE.

Password: Password credential of Server that the CPE uses to download the DU – Supplied by the CSE.

UUID: Supplied by the CSE and used to correlate the DU for the uninstall operation.

ExecurtionEnvRef: <empty> not used

Table 7.10-3: RPC ChangeDUState:UninstallOpStruct Arguments

RPC ChangeDUState:Uninstall OpStruct Argument

UUID: UUID of the DU that was installed – Maintained by the CSE.

ExecutionEnvRef: <empty> not used

## 7.11	Resource [reboot]

The Resource [reboot] maps to either the Reboot RPC or FactoryReset RPC of TR-069 [4].

When the reboot attribute of the Resource [reboot] is set to “True”, the CSE shall execute the Reboot RPC of TR-069[4].

When the factoryReset attribute of Resource [reboot] is set to “True”, the CSE shall execute the FactoryReset RPC of TR-069[4].

Table 7.11-1: Resource [reboot]

| Attribute Name of [reboot] | Description |
| --- | --- |
| reboot | Executes the Reboot RPC |
| factoryReset | FactoryReset RPC |

Table 7.11-2: RPC Reboot Arguments

RPC Reboot Arguments

CommandKey: Automatically set by the CSE where the reboot is set to “True” in order to correlate the “M-Reboot” Event from the next Inform.

## 7.12	Resource [cmdhPolicy]

The Resource [cmdhPolicy] represents a set of rules defining which CMDH parameters will be used by default when a request issued by a local originator contains the ec (event category) parameter but not all other CMDH parameters, see clause D.12 of TS-0001 [1].

The Resource [cmdhPolicy] is a multi-instance Resource where each instance of the Resource shall map to an instance of Device.X_oneM2M_org_CSE.{i}.CMDH.Policy.{i} object.

The Policy instance shall be created using the Add Object RPC of TR-069 [4].

The Policy instance shall be deleted using the Delete Object RPC of TR-069 [4].

The information of a Policy instance shall be retrieved using the GetParameterValues RPC of TR-069 [4].

The information of a Policy instance shall be updated using the SetParameterValues RPC of TR-069 [4].

Table 7.12-1: Resource [cmdhPolicy]

| Attribute Name of [cmdhPolicy] | X_oneM2M_org Parameter |
| --- | --- |
| name | Device.X_oneM2M_org_CSE.{i}.CMDH.Policy.{i}.Name |
| cmdhDefaults | Device.X_oneM2M_org_CSE.{i}.CMDH.Policy.{i}.DefaultRule |
| cmdhLimits | Device.X_oneM2M_org_CSE.{i}.CMDH.Policy.{i}.LimitRules |
| cmdhNetworkAccessRules | Device.X_oneM2M_org_CSE.{i}.CMDH.Policy.{i}.NetworkAccessECRules |
| cmdhBuffer | Device.X_oneM2M_org_CSE.{i}.CMDH.Policy.{i}.BufferRules |

### 7.12.1	Resource [activeCmdhPolicy]

The Resource [activeCmdhPolicy] provides a link to the currently active set of CMDH policies, see clause D.12.1 of TS-0001 [1].

The Resource [activeCmdhPolicy]  is mapped to the Enable parameter of the Device.X_oneM2M_org_CSE.{i}.CMDH.Policy.{i} object.

The information of a Policy instance shall be updated using the SetParameterValues RPC of TR-069 [4].

Table 7.12.1-1: Resource [activeCmdhPolicy]

| Attribute Name of [activeCmdhPolicy] | X_oneM2M_org Parameter |
| --- | --- |
| cmdhPolicy | Device.X_oneM2M_org_CSE.{i}.CMDH.Policy.{i}.Enable At most one Policy instance shall be enabled at a time. As such the Policy instance that has the Enable parameter with a value of “True” is the active CMDH policy. |

### 7.12.2	Resource [cmdhDefaults]

The Resource [cmdhDefaults] defines default CMDH policy values, see clause D.12.2 of TS-0001 [1].

The Resource [cmdhDefaults] is a multi-instance Resource where each instance of the Resource shall map to an instance of Device.X_oneM2M_org_CSE.{i}.CMDH.Default.{i} object.

The Default instance shall be created using the Add Object RPC of TR-069 [4].

The Default instance shall be deleted using the Delete Object RPC of TR-069 [4].

The information of a Default instance shall be retrieved using the GetParameterValues RPC of TR-069 [4].

The information of a Default instance shall be updated using the SetParameterValues RPC of TR-069 [4].

Table 7.12.2-1: Resource [cmdhDefaults]

| Attribute Name of [cmdhDefaults] | X_oneM2M_org Parameter |
| --- | --- |
| cmdhDefEcValue | Device.X_oneM2M_org_CSE.{i}.CMDH.Default.{i}.DefaultECRules |
| cmdhEcDefParamValues | Device.X_oneM2M_org_CSE.{i}.CMDH.Default.{i}.DefaultECParamRules |

### 7.12.3	Resource [cmdhDefEcValue]

The Resource [cmdhDefEcValue] represents a value for the ec (event category) parameter of an incoming request, see clause D.12.3 of TS-0001 [1].

The Resource [cmdhDefEcValue] is a multi-instance Resource where each instance of the Resource shall map to an instance of Device.X_oneM2M_org_CSE.{i}.CMDH.DefaultECRule.{i} object.

The DefaultECRule instance shall be created using the Add Object RPC of TR-069 [4].

The DefaultECRule instance shall be deleted using the Delete Object RPC of TR-069 [4].

The information of a DefaultECRule instance shall be retrieved using the GetParameterValues RPC of TR-069 [4].

The information of a DefaultECRule instance shall be updated using the SetParameterValues RPC of TR-069 [4].

Table 7.12.3-1: Resource [cmdhDefEcValue]

| Attribute Name of [cmdhDefEcValue] | X_oneM2M_org Parameter |
| --- | --- |
| order | Device.X_oneM2M_org_CSE.{i}.CMDH.DefaultECRule.{i}.Order |
| defEcValue | Device.X_oneM2M_org_CSE.{i}.CMDH.DefaultECRule.{i}.EventCategory |
| requestOrigin | Device.X_oneM2M_org_CSE.{i}.CMDH.DefaultECRule.{i}.RequestOrigin |
| requestContext | Device.X_oneM2M_org_CSE.{i}.CMDH.DefaultECRule.{i}.RequestContext |
| requestContextNotification | Device.X_oneM2M_org_CSE.{i}.CMDH.DefaultECRule.{i}.RequestContextNotificationEnable |
| requestCharacteristics | Device.X_oneM2M_org_CSE.{i}.CMDH.DefaultECRule.{i}.RequestCharacteristics |

### 7.12.4	Resource [cmdhEcDefParamValues]

The Resource [cmdhEcDefParamValues] represents a specific set of default values for the CMDH related parameters rqet (request expiration timestamp), rset (result expiration timestamp), oet (operational execution time), rp (response persistence) and da (delivery aggregation) that are applicable for a given ec (event category) if these parameters are not specified in the request, see clause D.12.4 of TS-0001 [1].

The Resource [cmdhEcDefParamValues] is a multi-instance Resource where each instance of the Resource shall map to an instance of Device.X_oneM2M_org_CSE.{i}.CMDH.DefaultECParamRule.{i} object.

The DefaultECParamRule instance shall be created using the Add Object RPC of TR-069 [4].

The DefaultECParamRule instance shall be deleted using the Delete Object RPC of TR-069 [4].

The information of a DefaultECParamRule instance shall be retrieved using the GetParameterValues RPC of TR-069 [4].

The information of a DefaultECParamRule instance shall be updated using the SetParameterValues RPC of TR-069 [4].

Table 7.12.4-1: Resource [cmdhEcDefParamValues]

| Attribute Name of [cmdhEcDefParamValues] | X_oneM2M_org Parameter |
| --- | --- |
| applicableEventCategory | Device.X_oneM2M_org_CSE.{i}.CMDH.DefaultECParamRule.{i}.EventCategories |
| defaultRequestExpTime | Device.X_oneM2M_org_CSE.{i}.CMDH.DefaultECParamRule.{i}.RequestExpTime |
| defaultResultExpTime | Device.X_oneM2M_org_CSE.{i}.CMDH.DefaultECParamRule.{i}.ResultExpTime |
| defaultOpExecTime | Device.X_oneM2M_org_CSE.{i}.CMDH.DefaultECParamRule.{i}.OperationExecTime |
| defaultRespPersistence | Device.X_oneM2M_org_CSE.{i}.CMDH.DefaultECParamRule.{i}.ResponsePersistence |
| defaultDelAggregation | Device.X_oneM2M_org_CSE.{i}.CMDH.DefaultECParamRule.{i}.DeliveryAggregation |

### 7.12.5	Resource [cmdhLimits]

The Resource [cmdhLimits] represents limits for CMDH related parameter values, see clause D.12.5 of TS-0001 [1].

The Resource [cmdhLimits] is a multi-instance Resource where each instance of the Resource shall map to an instance of Device.X_oneM2M_org_CSE.{i}.CMDH.Limit.{i} object.

The Limit instance shall be created using the Add Object RPC of TR-069 [4].

The Limit instance shall be deleted using the Delete Object RPC of TR-069 [4].

The information of a Limit instance shall be retrieved using the GetParameterValues RPC of TR-069 [4].

The information of a Limit instance shall be updated using the SetParameterValues RPC of TR-069 [4].

Table 7.12.5-1: Resource [cmdhLimits]

| Attribute Name of [cmdhLimits] | X_oneM2M_org Parameter |
| --- | --- |
| order | Device.X_oneM2M_org_CSE.{i}.CMDH.Limit.{i}.Order |
| requestOrigin | Device.X_oneM2M_org_CSE.{i}.CMDH.Limit.{i}.RequestOrigin |
| requestContext | Device.X_oneM2M_org_CSE.{i}.CMDH.Limit.{i}.RequestContext |
| requestContextNotification | Device.X_oneM2M_org_CSE.{i}.CMDH.Limit.{i}.RequestContextNotificationEnable |
| requestCharacteristics | Device.X_oneM2M_org_CSE.{i}.CMDH.Limit.{i}.RequestCharacteristics |
| limitsEventCategory | Device.X_oneM2M_org_CSE.{i}.CMDH.Limit.{i}.EventCategories |
| limitsRequestExpTime | Device.X_oneM2M_org_CSE.{i}.CMDH.Limit.{i}.RequestExpTime |
| limitsResultExpTime | Device.X_oneM2M_org_CSE.{i}.CMDH.Limit.{i}.ResultExpTime |
| limitsOpExecTime | Device.X_oneM2M_org_CSE.{i}.CMDH.Limit.{i}.OperationExecTime |
| limitsRespPersistence | Device.X_oneM2M_org_CSE.{i}.CMDH.Limit.{i}.ResponsePersistence |
| limitsDelAggregation | Device.X_oneM2M_org_CSE.{i}.CMDH.Limit.{i}.DeliveryAggregation |

### 7.12.6	Resource [cmdhNetworkAccessRules]

The Resource [cmdhNetworkAccessRules] defines the usage of underlying networks for forwarding information to other CSEs during processing of CMDH-related requests in a CSE, see clause D.12.6 of TS-0001 [1].

The Resource [cmdhNetworkAccessRules] is a multi-instance Resource where each instance of the Resource shall map to an instance of Device.X_oneM2M_org_CSE.{i}.CMDH.NetworkAccessECRule.{i} object.

The NetworkAccessECRule instance shall be created using the Add Object RPC of TR-069 [4].

The NetworkAccessECRule instance shall be deleted using the Delete Object RPC of TR-069 [4].

The information of a NetworkAccessECRule instance shall be retrieved using the GetParameterValues RPC of TR-069 [4].

The information of a NetworkAccessECRule instance shall be updated using the SetParameterValues RPC of TR-069 [4].

Table 7.12.6-1: Resource [cmdhNetworkAccessRules]

| Attribute Name of [cmdhNetworkAccessRules] | X_oneM2M_org Parameter |
| --- | --- |
| applicableEventCategories | Device.X_oneM2M_org_CSE.{i}.CMDH.NetworkAccessECRule.{i}.EventCategories |
| cmdhNwAccessRule | Device.X_oneM2M_org_CSE.{i}.CMDH.NetworkAccessECRule.{i}.NetworkAccessRules |

### 7.12.7	Resource [cmdhNwAccessRule]

The Resource [cmdhNwAccessRule] define limits in usage of specific underlying networks for forwarding information to other CSEs during processing of CMDH-related requests, see clause D.12.7 of TS-0001 [1].

The Resource [cmdhNwAccessRule] is a multi-instance Resource where each instance of the Resource shall map to an instance of Device.X_oneM2M_org_CSE.{i}.CMDH.NetworkAccessECRule.{i} object.

The NetworkAccessRule instance shall be created using the Add Object RPC of TR-069 [4].

The NetworkAccessRule instance shall be deleted using the Delete Object RPC of TR-069 [4].

The information of a NetworkAccessRule instance shall be retrieved using the GetParameterValues RPC of TR-069 [4].

The information of a NetworkAccessRule instance shall be updated using the SetParameterValues RPC of TR-069 [4].

Table 7.12.7-1: Resource [cmdhNwAccessRule]

| Attribute Name of [cmdhNwAccessRule] | X_oneM2M_org Parameter |
| --- | --- |
| targetNetwork | Device.X_oneM2M_org_CSE.{i}.CMDH.NetworkAccessRule.{i}.TargetNetworks |
| minReqVolume | Device.X_oneM2M_org_CSE.{i}.CMDH.NetworkAccessRule.{i}.MinimumReqVolume |
| backOffParameters | Device.X_oneM2M_org_CSE.{i}.CMDH.NetworkAccessRule.{i}.BackoffTime |
|  | Device.X_oneM2M_org_CSE.{i}.CMDH.NetworkAccessRule.{i}.BackoffTimeIncrement |
|  | Device.X_oneM2M_org_CSE.{i}.CMDH.NetworkAccessRule.{i}.MaximumBackoffTime |
| otherConditions | Device.X_oneM2M_org_CSE.{i}.CMDH.NetworkAccessRule.{i}.OtherConditions |
| allowedSchedule | Device.X_oneM2M_org_CSE.{i}.CMDH.NetworkAccessRule.{i}.AllowedSchedule |

### 7.12.8	Resource [cmdhBuffer]

The Resource [cmdhBuffer] represents limits in usage of buffers for temporarily storing information that needs to be forwarded to other CSEs during processing of CMDH-related requests in a CSE, see clause D.12.8 of TS-0001 [1].

The Resource [cmdhBuffer] is a multi-instance Resource where each instance of the Resource shall map to an instance of Device.X_oneM2M_org_CSE.{i}.CMDH.Buffer.{i} object.

The Buffer instance shall be created using the Add Object RPC of TR-069 [4].

The Buffer instance shall be deleted using the Delete Object RPC of TR-069 [4].

The information of a Buffer instance shall be retrieved using the GetParameterValues RPC of TR-069 [4].

The information of a Buffer instance shall be updated using the SetParameterValues RPC of TR-069 [4].

Table 7.12.8-1: Resource [cmdhBuffer]

| Attribute Name of [cmdhBuffer] | X_oneM2M_org Parameter |
| --- | --- |
| applicableEventCategory | Device.X_oneM2M_org_CSE.{i}.CMDH.Buffer.{i}.EventCategories |
| maxBufferSize | Device.X_oneM2M_org_CSE.{i}.CMDH.Buffer.{i}.MaximumBufferSize |
| storagePriority | Device.X_oneM2M_org_CSE.{i}.CMDH.Buffer.{i}.StoragePriority |

## 7.13	Resource Type <mgmtCmd>

Each mgmtCmd Resource shall map to BBF TR-069 RPC commands based on the value of cmdType. Accordingly, execReqArgs shall contain arguments related to the corresponding BBF TR-069 RPCs. The details about corresponding procedure mapping are described in section 8.2.

Table 7.13-1: Resource Type <mgmtCmd>

| Attribute cmdType of mgmtCmd | Attribute execReqArgs of mgmtCmd |
| --- | --- |
| cmdType = RESET | Shall include all arguments related to BBF FactoryReset RPC |
| cmdType = REBOOT | Shall include all arguments related to BBF Reboot RPC |
| cmdType = UPLOAD | Shall include all arguments related to BBF Reboot RPC |
| cmdType = DOWNLOAD | Shall contain all arguments related to BBF Reboot RPC |
| cmdType = SOFTWAREINSTALL | Shall contain all arguments related to BBF ChangeDUState RPC which shall contain “InstallOpStruct” structure. |
| cmdType = SOFTWAREUNINSTALL | Shall contain all arguments related to BBF ChangeDUState RPC which shall contain “UninstallOpStruct” structure. |

## 7.14 	Resource Type <execInstance>

The <execInstance> resource  from TS-0004 [2] shall map to BBF CancelTransfer RPC commands when it is disabled/cancelled using a Update operation or deleted using a Delete operation. The details are described in section 8.2.