
[image: image1.png]



	oneM2M
Technical Specification

	Document Number
	TS-0005-V3.2.0

	Document Name:
	Management enablement (BBF)

	Date:
	2017-Feb-20

	Abstract:
	Specifies the usage of the BBF TR-069 protocol and the corresponding message flows including normal cases as well as error cases to fulfil the oneM2M management requirements.

· Protocol mapping between the oneM2M service layer and BBF TR-069 protocol. The Mca reference point, ms interface and la interface are possibly involved in this protocol mapping.

· Mapping between the oneM2M management related resources and the TR-069 protocol RPCs and TR-181i2 data model.
Specification of new TR-181 data model elements to fulfil oneM2M specific management requirements that cannot be currently translated.

	Template Version: January 2017 (Do not modify)


The present document is provided for future development work within oneM2M only. The Partners accept no liability for any use of this specification.
The present document has not been subject to any approval process by the oneM2M Partners Type 1.  Published oneM2M specifications and reports for implementation should be obtained via the oneM2M Partners' Publications Offices.
About oneM2M 

The purpose and goal of oneM2M is to develop technical specifications which address the need for a common M2M Service Layer that can be readily embedded within various hardware and software, and relied upon to connect the myriad of devices in the field with M2M application servers worldwide. 

More information about oneM2M may be found at:  http//www.oneM2M.org

Copyright Notification

© 2017, oneM2M Partners Type 1 (ARIB, ATIS, CCSA, ETSI, TIA, TSDSI, TTA, TTC).

All rights reserved.
The copyright extends to reproduction in all media.

Notice of Disclaimer & Limitation of Liability 

The information provided in this document is directed solely to professionals who have the appropriate degree of experience to understand and interpret its contents in accordance with generally accepted engineering or other professional standards and applicable regulations. No recommendation as to products or vendors is made or should be implied. 

NO REPRESENTATION OR WARRANTY IS MADE THAT THE INFORMATION IS TECHNICALLY ACCURATE OR SUFFICIENT OR CONFORMS TO ANY STATUTE, GOVERNMENTAL RULE OR REGULATION, AND FURTHER, NO REPRESENTATION OR WARRANTY IS MADE OF MERCHANTABILITY OR FITNESS FOR ANY PARTICULAR PURPOSE OR AGAINST INFRINGEMENT OF INTELLECTUAL PROPERTY RIGHTS. NO oneM2M PARTNER TYPE 1 SHALL BE LIABLE, BEYOND THE AMOUNT OF ANY SUM RECEIVED IN PAYMENT BY THAT PARTNER FOR THIS DOCUMENT, WITH RESPECT TO ANY CLAIM, AND IN NO EVENT SHALL oneM2M BE LIABLE FOR LOST PROFITS OR OTHER INCIDENTAL OR CONSEQUENTIAL DAMAGES. oneM2M EXPRESSLY ADVISES ANY AND ALL USE OF OR RELIANCE UPON THIS INFORMATION PROVIDED IN THIS DOCUMENT IS AT THE RISK OF THE USER.

Contents

Contents
3
1
Scope
5
2
References
5
2.1
Normative references
5
2.2
Informative references
5
3
Definitions, abbreviations and acronyms
5
3.1
Definitions
5
3.2
Abbreviations
6
3.3
Acronyms
6
4
Conventions
6
5
Mapping of basic data types
6
6
Mapping of identifiers
7
6.0
Introduction
7
6.1
Mapping of Device identifiers to the Node Resource
7
6.2
Identifier of an object instance
8
7
Mapping of resources
8
7.0
Introduction
8
7.1
General mapping assumptions
8
7.1.0
Introduction
8
7.1.1
Mapping of Device Identifiers
8
7.1.2
Mapping of Embedded Devices
8
7.2
Resource [deviceInfo]
9
7.3
Resource [memory]
9
7.4
Resource [battery]
10
7.5
Resource [areaNwkInfo]
10
7.6
Resource [areaNwkDeviceInfo]
10
7.7
Resource [eventLog]
11
7.8
Resource [deviceCapability]
11
7.9
Resource [firmware]
12
7.10
Resource [software]
13
7.11
Resource [reboot]
14
7.12
Resource [cmdhPolicy]
15
7.12.0
Introduction
15
7.12.1
Resource [activeCmdhPolicy]
15
7.12.2
Resource [cmdhDefaults]
16
7.12.3
Resource [cmdhDefEcValue]
16
7.12.4
Resource [cmdhEcDefParamValues]
17
7.12.5
Resource [cmdhLimits]
17
7.12.6
Resource [cmdhNetworkAccessRules]
18
7.12.7
Resource [cmdhNwAccessRule]
18
7.12.8
Resource [cmdhBuffer]
18
7.13
Resource Type <mgmtCmd>
19
7.14
Resource Type <execInstance>
19
7.15
Resource [registration]
19
7.16
Resource [dataCollection]
20
7.17
Security Solutions
21
7.17.1
Introduction
21
7.17.2
Resource [authenticationProfile]
21
7.17.3
Resource [trustAnchorCred]
21
7.17.4
Resource [myCertFileCred]
22
7.17.5
Resource [MAFClientRegCfg]
22
8
Mapping of procedures for management
23
8.0
Introduction
23
8.1
Resource Type <mgmtObj> primitive mappings
23
8.1.0
Introduction
23
8.1.1
Alias-Based Addressing Mechanism
23
8.1.2
Create primitive mapping
23
8.1.2.0
Introduction
23
8.1.2.1
M2M Service Layer Resource Instance Identifier mapping
24
8.1.3
Delete primitive mapping
24
8.1.3.1
Delete primitive mapping for deletion of Object Instances
24
8.1.3.2
Delete primitive mapping for software un-install operation
24
8.1.4
Update primitive mapping
26
8.1.4.1
Update primitive mapping for Parameter modifications
26
8.1.4.2
Update primitive mapping for upload file transfer operations
26
8.1.4.3
Update primitive mapping for download file transfer operations
27
8.1.4.4
Update primitive mapping for reboot operation
29
8.1.4.5
Update primitive mapping for factory reset operation
29
8.1.4.6
Update primitive mapping for software install operation
29
8.1.5
Retrieve primitive mapping
31
8.1.6
Notify primitive mapping
31
8.1.6.0
Introduction
31
8.1.6.1
Procedure for subscribed Resource attributes.
31
8.1.6.2
Notification primitive mapping
32
8.2
<mgmtCmd> and <execInstance> resource primitive mappings
32
8.2.1
Update (Execute) primitive for the <mgmtCmd> resource
32
8.2.1.0
Introduction
32
8.2.1.1
Execute File Download
33
8.2.1.2
Execute File Upload Operations
33
8.2.1.3
Report Results using TransferComplete RPC
34
8.2.1.4
Execute Software Operations with ChangeDUState RPC
35
8.2.1.5
Report Results with ChangeDUStateComplete RPC
36
8.2.1.6
Execute Reboot operation
38
8.2.1.7
Execute Factory Reset operation
38
8.2.2
Delete <mgmtCmd> resource primitive mapping
38
8.2.3
Update (Cancel) <execInstance> primitive mapping
39
8.2.4
Delete <execInstance> primitive mapping
39
8.3
Resource [myCertFileCred] primitive mappings
40
8.3.1
Introduction
40
8.3.2
Creation of Resource [myCertFileCred]
40
8.3.2.1
Introduction
40
8.3.2.2
Procedure for creation of Resource [myCertFileCred]
40
9
Server Interactions
41
9.0
Introduction
41
9.1
Communication Session Establishment
42
9.1.1
IN-CSE to ACS Communication Session Establishment
42
9.1.2
ACS to IN-CSE Communication Session Establishment
42
9.2.3
ACS and IN-CSE Communication Session Requirements
42
9.2
Processing of Requests and Responses
42
9.2.1
Request and Notification Formatting
42
9.2.2
ACS Request Processing Requirements
42
9.2.3
ACS Notification Processing Requirements
43
9.3
Discovery and Synchronization of Resources
43
9.4
Access Management
43
9.4.0
Introduction
43
9.4.1
Access Management Requirements
43
10
New Management Technology Specific Resources
43
History
44


1
Scope

The present document describes the protocol mappings between the management Resources for oneM2M and the BBF TR-181i2 Data Model [6].
2
References

2.1
Normative references

References are either specific (identified by date of publication and/or edition number or version number) or non‑specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

The following referenced documents are necessary for the application of the present document.

 [1]
oneM2M TS-0001: “Functional Architecture”.
[2]
oneM2M TS-0004: “Service Layer Core Protocol Specification”.

[3]
oneM2M TS-0011: “Common Terminology”.

[4]
BBF: “TR-069 CPE WAN Management Protocol” Issue: 1 Amendment 5, November 2013.

[5]
BBF: “TR-106 Data Model Template for TR-069-Enabled Devices”, Issue 1, Amendment 7, September 2013.
[6]
BBF: “TR-181 Device Data Model for TR-069, Issue 2 Amendment 11”, July 2016.
[7]
BBF: “TR-131 ACS Northbound Interface Requirements, Issue:1”, November 2009.
[8]
oneM2M TS-0022: “Field Device Configuration”.
2.2
Informative references

References are either specific (identified by date of publication and/or edition number or version number) or non‑specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.
[i.1]
oneM2M Drafting Rules

NOTE:
Available at http://member.onem2m.org/Static-pages/Others/Rules-Pages/oneM2M-Drafting-Rules-V1-0.doc.

3
Definitions, abbreviations and acronyms
3.1
Definitions

For the purposes of the present document, the terms and definitions given in TS-0011 [3] and the following apply.

CPE Proxier: CPE that is capable of proxying the communication between an ACS and a Proxied Device as defined in TR-069 [4]

3.2
Abbreviations

For the purposes of the present document, the abbreviations given in TS-0011 [3] and the following apply.

ACS
Auto-Configuration Server

ADN
Application Dedicated Node

AE
Application Entity

ASN
Application Service Node

BBF
Broadband Forum

CMDH
Communication Management and Delivery Handling

CPE
Customer Premise Equipment

CSE
Common Services Entity

CWMP
CPE WAN Management Protocol

DM
Device Management

DU
Deployment Unit

IN-CSE
CSE which resides in the Infrastructure Node

LAN
Local Area Network
MAF
M2M Authentication Function
MN
Middle Node

OUI
Organizationally Unique Identifier

PC
Product Class

RPC
Remote Procedure Call

SN
Serial Number

UPA
Universal Powerline Association

URI
Uniform Resource Identifier

URL
Uniform Resource Locator

USB
Universal Serial Bus

UUID
Universal Unique Identifier

XML
Extensible Markup Language

3.3
Acronyms

For the purposes of the present document, the acronyms given in TR-0004 [3] apply.

4
Conventions 

The key words “Shall”, “Shall not”, “May”, “Need not”, “Should”, “Should not” in the present document are to be interpreted as described in the oneM2M Drafting Rules [i.1]

5
Mapping of basic data types
TR-106 [5] specifies the object structure supported by TR-069 enabled devices and specifies the structural requirements for the data hierarchy. This clause includes the mapping attribute data types to TR-181 [6] parameters which follows the conventions of section 3 of TR-106 [5] and data types described in Table 4 of TR-106 [5].

Table 5-1: Data Type Mapping

	oneM2M Data Types
	Mapping to data types in TR-106
	Conversion Notes

	xs:boolean
	boolean
	

	xs:string
	string
	Mapping is constrained to the size of the string

	xs:unsignedInt
	unsignedInt
	

	xs:unsignedLong
	unsignedLong
	

	xs:integer
	long
	Mapping is constrained to the size of the long data type.

	Xs:positiveInteger
	unsignedLong
	Mapping is constrained to a lower limit of 1 and the size of the unsignedLong data type.

	Xs:nonNegativeInteger
	unsignedLong
	Mapping is constrained the size of the unsignedLong data type.

	Comma separated Lists
	Comma separated Lists
	Data structure is represented by comma separated list as described in section 3.2.3 of TR-106 [5].


In some instances the conversion of the contents between data types will cause an error to occur (e.g. xs:integer to long). When an error occurs in the conversion of a data type, the STATUS_BAD_REQUEST response status code.

6
Mapping of identifiers

6.0
Introduction

The TR-069 [4] specification defines three (3) types of devices, known as CPEs, that are capable of being managed from the perspective of the TR-069 agent:

· CPE that hosts the TR-069 agent: Section A.3.3.1 Inform of TR-069 [4] defines the required fields for a CPE to be identified. These fields include the OUI and Serial Number of the CPE assigned by the CPE manufacturer. Optionally the manufacturer may assign a Product Class to the CPE. The format of the identifier is as follows: OUI-[PC-]SN.

· Virtual Device: This type of device is addressed as a CPE. The Virtual Device has its own OUI-[PC-]SN as represented by the CPE Proxier. The CPE Proxier emulates a CWMP agent for each Virtual Device.

· Embedded Device: This type of device is addressed as one or more objects within the data model of the CPE that hosts the TR-069 agent.

6.1
Mapping of Device identifiers to the Node Resource

Node Resources are identified for each instance of an ADN, ASN and MN node and are identified using the M2M Node Identifier (M2M-Node-ID) defined in the oneM2M Functional [1].

CPE Device identifiers shall map to the nodeID attribute of the <node> resource. The CPE Device identifiers are obtained from the contents of the following attributes:

· Device.DeviceInfo.ManufacturerOUI

· Device.DeviceInfo.ProductClass

· Device.DeviceInfo.SerialNumber

Virtual Device identifiers shall map to the nodeID attribute of the <node> resource. The Virtual Device identifiers are obtained from the CPE Proxier using the contents of the attributes:

· Device.ManagementServer.VirtualDevice.{i}.ManufacturerOUI

· Device.ManagementServer.VirtualDevice.{i}.ProductClass

· Device.ManagementServer.VirtualDevice.{i}.SerialNumber

Embedded Device identifiers shall map to the nodeID attribute of the <node> resource. The Embedded Device identifiers are obtained using the containing CPE Device or Virtual Device identifiers along with the contents of the attributes of the:

· Device.ManagementServer.EmbeddedDevice.{i}.ControllerID

· Device.ManagementServer.EmbeddedDevice.{i}.ProxiedDeviceID

6.2
Identifier of an object instance

The TR-069 [4] specification permits objects to have multiple object instances where each object instance is contained within the objectPath attribute of the Resource within the context of the Resource’s objectId as defined in clause 7.1.

In order to allow the AE or CSE that originated the request that manipulates a Resource to easily align the M2M Service Layer with the Resource’s external technology identifier, the value of the object instance “{i}” should be a part of the identifier of the Resource in the M2M Service Layer where possible. For example if the [areaNetwork] resource has an object instance identifier of “Device.X_oneM2M_org_CSE.1.M2MareaNetworkDevice.[foo]” then the M2M Service Layer Resource should be identified using the object instance of the underlying technology (e.g. “/foo” for the Resource areaNetwork). 

7
Mapping of resources
7.0
Introduction

This clause contains all information on how to map management resources from TS-0004 [2] to managed objects and parameters as defined in the TR-181 [6] data model or the Remote Procedure Calls (RPCs) in TR-069 [4].

7.1
General mapping assumptions

7.1.0
Introduction

TR-069 [4] specifies a protocol for communication between a CPE (Customer Premises Equipment) and an ACS (Auto-Configuration Server). Any TR-069 enabled device has to follow the data model as described in the TR-106 [5] and TR‑181 [6] as well as RPCs described in TR-069 [4].

As TR-181 [6] is the model that the Resources are mapped, all Resources shall have the objects of the TR-181 [6] namespace (e.g. “urn:broadband-forum-org:tr-181-2-7-0”).

7.1.1
Mapping of Device Identifiers

The Device identifiers for CPEs are mapped to the Resource Types [deviceInfo]. 

For CPE and Virtual Devices map their Device Identifiers (OUI-[PC-]SN) to the manufacturer, deviceType and deviceLabel attributes of the Resource [deviceInfo]. 

For Embedded Devices, the ControllerID and ProxiedDeviceID parameters of the Device.ManagementServer.EmbeddedDevice.{i} object instance are mapped to the deviceLabel attribute of the Resource [deviceInfo] as a comma separated list: “Device.ManagementServer.EmbeddedDevice.{i}.ControllerID, Device.ManagementServer.EmbeddedDevice.{i}.ProxiedDeviceID”.

7.1.2
Mapping of Embedded Devices

The TR-181 [6] specification does not provide a mechanism where Embedded Devices provide information related to the Device.DeviceInfo objects and sub-objects. Instead the TR-181 [6] provides this information in a manner that is reliant on the Embedded Device’s underlying technology (e.g. ZigBee®, UpnP).

As such the mapping of the [memory] and [battery] Resources are implementation specific for each underlying technology and is outside the scope of the present document.

7.2
Resource [deviceInfo]

The Resource [deviceInfo] is a read-only Resource that shall map to the Device.DeviceInfo object of TR-181 [6] for CPE and Virtual Devices.

The information shall be retrieved using the GetParameterValues RPC of TR-069 [4].

NOTE:
The SerialNumber, ModelNumber, ProductClass attributes for a Virtual device are the same values as the Device.ManagementServer.VirtualDevice.{i} object in the CPE Proxier.
Table 7.2-1: Resource [deviceInfo] for CPE and Virtual Devices

	Attribute Name of [deviceInfo]
	TR-181 Parameter

	deviceLabel
	Device.DeviceInfo.SerialNumber

	manufacturer
	Device.DeviceInfo.Manufacturer

	model
	Device.DeviceInfo.ModelNumber

	deviceType
	Device.DeviceInfo.ProductClass

	fwVersion
	Device.DeviceInfo.SoftwareVersion if the device supports only 1 software version. If the device support multiple software versions this shall map to Device.DeviceInfo.AdditionalSoftwareVersion

	swVersion
	Device.DeviceInfo.SoftwareVersion

	hwVersion
	Device.DeviceInfo.HardwareVersion


Table 7.2-2: Resource [deviceInfo] for Embedded Devices

	Attribute Name of [deviceInfo]
	TR-181 Parameter

	deviceLabel
	Comma separated list: “Device.ManagementServer.EmbeddedDevice.{i}.ControllerID, Device.ManagementServer.EmbeddedDevice.{i}.ProxiedDeviceID

	manufacturer
	No mapping available

	model
	No mapping available

	deviceType
	No mapping available

	fwVersion
	No mapping available

	swVersion
	No mapping available

	hwVersion
	No mapping available


7.3
Resource [memory]

The Resource [memory] is a read-only Resource that shall map to the Device.DeviceInfo.MemoryStatus object of TR‑181 [6] for CPE and Virtual Devices. 

The information shall be retrieved using the GetParameterValues RPC of TR-069 [4]. 

Attempts to modify the attributes of the memory Resource causes an error code “operation unsupported” to be returned.

Table 7.3-1: Resource [memory]

	Attribute Name of [memory]
	TR-181 Parameter

	memAvailable
	Device.DeviceInfo.MemoryStatus.Free

	memTotal
	Device.DeviceInfo.MemoryStatus.Total


7.4
Resource [battery]

The Resource [battery] is a read-only Resource that shall map to an instance of Device.DeviceInfo.X_oneM2M_org_BatteryStatus.Battery.{i} object for CPE and Virtual Devices. 

The information shall be retrieved using the GetParameterValues RPC of TR-069 [4]. 

Table 7.4-1: Resource [battery]

	Attribute Name of [battery]
	TR-181 Parameter

	batteryLevel
	Device.DeviceInfo.X_oneM2M_org_BatteryStatus.Battery.{i}.Level

	batteryStatus
	Device.DeviceInfo.X_oneM2M_org_BatteryStatus.Battery.{i}.Status


7.5
Resource [areaNwkInfo]

The Resource [areaNwkInfo] is a multi-instance Resource where each instance of the Resource shall map to an instance of Device.X_oneM2M_org_CSE.{i}.M2MareaNetwork.{i} object. 

As the Resource [areaNwkInfo] is a multi-instance Resource, the M2MareaNetwork object is a multi-object instance that can be created and deleted.

The M2MareaNetwork instance shall be created using the Add Object RPC of TR-069 [4].

The M2MareaNetwork instance shall be deleted using the Delete Object RPC of TR-069 [4]. 

The information of an M2MareaNetwork shall be retrieved using the GetParameterValues RPC of TR-069 [4]. 

The information of an M2MareaNetwork shall be modified using the SetParameterValues RPC of TR-069 [4]. 

Table 7.5-1: Resource [areaNwkInfo]

	Attribute Name of [areaNwkInfo]
	X_oneM2M_org Parameter

	areaNwkType
	Device.X_oneM2M_org_CSE.{i}.M2MareaNetwork.{i}.Type

	listOfDevices
	Device.X_oneM2M_org_CSE.{i}.M2MareaNetwork.{i}.ListOfDevices


7.6
Resource [areaNwkDeviceInfo]

The Resource [areaNwkDeviceInfo] is a multi-instance Resource where each instance of the Resource shall map to an instance of Device.X_oneM2M_org_CSE.{i}.AreaNetworkDevice.{i} object. 

As the Resource [areaNwkDeviceInfo] is a multi-instance Resource, the AreaNetworkDevice object is a multi-object instance that can be created and deleted.

Instances of the Resource [areaNwkDeviceInfo] are referenced in the listOfDevices attribute of the associated Resource [areaNwkInfo]. 

The M2MareaNetworkDevice instance shall be created using the Add Object RPC of TR-069 [4].

The M2MareaNetworkDevice instance shall be deleted using the Delete Object RPC of TR-069 [4]. 

The information of an M2MareaNetworkDevice shall be retrieved using the GetParameterValues RPC of TR-069 [4]. 

The information of an M2MareaNetworkDevice shall be modified using the SetParameterValues RPC of TR-069 [4].

Table 7.6-1: Resource [areaNwkDeviceInfo]

	Attribute Name of [areaNwkDeviceInfo]
	X_oneM2M_org Parameter

	devId
	Device.X_oneM2M_org_CSE.{i}.M2MareaNetworkDevice.{i}.Host

	devType
	Device.X_oneM2M_org_CSE.{i}.M2MareaNetworkDevice.{i}.Type

	areaNwkId
	Reference to Device.X_oneM2M_org_CSE.{i}.M2MareaNetworkDevice.{i}.M2MareaNetwork

	sleepInterval
	Device.X_oneM2M_org_CSE.{i}.M2MareaNetworkDevice.{i}.SleepInterval

	sleepDuration
	Device.X_oneM2M_org_CSE.{i}.M2MareaNetworkDevice.{i}.SleepDuration

	status
	Device.X_oneM2M_org_CSE.{i}.M2MareaNetworkDevice.{i}.Status

	listOfNeighbors
	Device.X_oneM2M_org_CSE.{i}.M2MareaNetworkDevice.{i}.Neighbors


7.7
Resource [eventLog]

The Resource [eventLog] is a multi-instance Resource where each instance of the Resource shall map to an instance of Device.DeviceInfo.X_oneM2M_org_Diagnostics.EventLog.{i} object. 

The EventLog instance shall be created using the Add Object RPC of TR-069 [4].

The EventLog instance shall be deleted using the Delete Object RPC of TR-069 [4]. 

The information of an EventLog instance shall be retrieved using the GetParameterValues RPC of TR-069 [4]. 

The information of an EventLog instance shall be updated using the SetParameterValues RPC of TR-069 [4]. 

Table 7.7-1: Resource [eventLog]

	Attribute Name of [eventLog]
	TR-181 Parameter

	logTypeId
	Device.DeviceInfo.X_oneM2M_org_Diagnostics.EventLog.{i}.Type

	logData
	Device.DeviceInfo.X_oneM2M_org_Diagnostics.EventLog.{i}.Data

	logStatus
	Device.DeviceInfo.X_oneM2M_org_Diagnostics.EventLog.{i}.Status

	logStart
	Set to “True” , the Device.DeviceInfo.X_oneM2M_org_Diagnostics.EventLog.{i}.Enable parameter is set to “True”.

	logStop
	Set to “True” , the Device.DeviceInfo.X_oneM2M_org_Diagnostics.EventLog.{i}.Enable parameter is set to “False”.


7.8
Resource [deviceCapability]

The Resource [deviceCapability] represents a capability of device that can be administratively enabled or disabled. The lists of capabilities that are managed are defined in the enumeration of the capabilityName attribute. The TR-181 [6] data model defines a subset of capabilities listed in the deviceCapability enumeration. The supported device capabilities within TR-181 [6] include:

· LAN Interfaces: USB, Wi-Fi, HomePlug, MoCA, UPA

· Hardware Capabilities: SmartCardReader

The information shall be retrieved using the GetParameterValues RPC of TR-069 [4]. 

The capabilities shall be enabled and disabled using the SetParameterValues RPC of TR-069 [4]. 

Table 7.8-1: Resource [capabilityInstance]

	Attribute Name of [capabilityInstance]
	TR-181 Parameter

	capabilityName
	This attribute is fixed based on the value of the capabilityName attribute.

	Attached
	Returns “True”

	capabilityActionStatus
	· Status is defined as:

· Success if the SetParameterValues RPC indicates that the operation was successful.
· Failure if the response to the SetParameterValues RPCs indicates that the operation failed.
· In process if the SetParameterValues RPC is initiated but the response to the SetParameterValues RPC has not been received.

	currentState
	USB: Device.USB.Interface.{i}.Enable

Wi-Fi: Device.Wi-Fi.Radio.{i}.Enable

HomePlug: Device.HomePlug.Interface.{i}.Enable

MoCA: Device.MoCA.Interface.{i}.Enable

UPA: Device.UPA.Interface.{i}.Enable

SmartCardReader: Device.SmartCardReaders.SmartCardReader.{i}.Enable

	enable
	USB: Device.USB.Interface.{i}.Enable

Wi-Fi: Device.Wi-Fi.Radio.{i}.Enable

HomePlug: Device.HomePlug.Interface.{i}.Enable

MoCA: Device.MoCA.Interface.{i}.Enable

UPA: Device.UPA.Interface.{i}.Enable

SmartCardReader: Device.SmartCardReaders.SmartCardReader.{i}.Enable

	disable
	Same parameter is used to disable a capability as the enable attribute.


7.9
Resource [firmware]

The Resource [firmware] represents a firmware instance and is not considered a TR-069 managed entity within the device until the firmware Resource’s update attribute has been written a value of “True”. When this occurs, the TR‑069 Download RPC shall be invoked.

Note: In many instances, the server from which the firmware is downloaded requires authentication in the form of Username and Password credentials. The CSE that executes firmware download shall maintain the mapping of the username and password of the download server needed to download the firmware outside the lifecycle of the specific firmware.

Table 7.9-1: Resource [firmware]

	Attribute Name of [firmware]
	RPC Download Arguments

	URL
	URL

	update
	When set to the value of “True” executes the Download operations with a FileType “1 Firmware Upgrade Image” is performed.

	
	Username: Received from the CSE for the download server where the update is set to “True”.

	
	Password: Received from the CSE for the download server where the update is set to “True”.

	
	CommandKey: Automatically set by the CSE where the update is set to “True” in order to correlate the TransferComplete response.

	
	FileSize: 0 (not used)

	
	TargetFileName: <empty> (not used)

	
	DelaySeconds: 0 (immediate)

	
	SuccessURL: <empty> (not used)

	
	FailureURL: <empty> (not used)


7.10
Resource [software]

The Resource [software] is a multi-instance Resource where each instance of the Resource maps directly to an instance of Device.SoftwareModules.DeploymentUnit.{i} object for the deployment aspects (install, uninstall) of the Resource [software]. The install and uninstall operation of the Resource [software] is performed using a combination of the ChangeDUState and ChangeDUStateComplete RPCs.

Once a Resource [software] has been installed, the Resource shall be mapped to the associated Device.SoftwareModules.ExecutionUnit.{i} objects in order to activate and deactivate the associated execution unit.

The Resource [software] version and name shall be retrieved using the GetParameterValues RPC of TR-069 [4].

The activate and deactivate operations of the Resource [software] shall be performed by manipulating the Device.SoftwareModules.ExecutionUnit.{i}.RequestedState parameter using the SetParameterValues RPC.

NOTE:
The Resource [software] provides support for only 1 Execution Unit per Deployment Unit. If a Deployment Unit is discovered by the M2M Service Layer that contains multiple Execution Units for a Deployment Unit; only 1 Execution Unit is exposed. The selection of which Execution Unit is implementation specific.

Table 7.10-1: Resource [software]

	Attribute Name of [software]
	Description

	version
	Device.SoftwareModules.DeploymentUnit.{i}.Version

	name
	Device.SoftwareModules.DeploymentUnit.{i}.Name

	URL
	Device.SoftwareModules.DeploymentUnit.{i}.URL

	install
	Use the ChangeDUState:InstallOpStruct

	installStatus
	Status is defined as:

· Success if the ChangeDUStateComplete RPC indicates that the operation was  successful.
· Failure if the response to the ChangeDUState or ChangeDUStateComplete RPCs indicates that the operation failed.
· In process if the ChangeDUState RPC is initiated but the ChangeDUStateComplete RPC has not been received.

	Activate
	The action that activates software previously installed.

	Deactivate
	The action that deactivates software.

	activeStatus
	Status is defined as:

· Success if the SetParameterValues RPC indicates that the operation was  successful.
· Failure if the response to the SetParameterValues RPCs indicates that the operation failed.
· In process if the SetParameterValues RPC is initiated but the response to the SetParameterValues RPC has not been received.


Table 7.10-2: RPC ChangeDUState:InstallOpStruct Arguments

	RPC ChangeDUState:InstallOpStruct Argument

	URL: URL of the Server that M2M Node uses to download the DU.

	Username: Username credential of Server that the CPE uses to download the DU – Supplied by the CSE.

	Password: Password credential of Server that the CPE uses to download the DU – Supplied by the CSE.

	UUID: Supplied by the CSE and used to correlate the DU for the uninstall operation.

	ExecurtionEnvRef: <empty> not used


Table 7.10-3: RPC ChangeDUState:UninstallOpStruct Arguments

	RPC ChangeDUState:Uninstall OpStruct Argument

	UUID: UUID of the DU that was installed – Maintained by the CSE.

	ExecutionEnvRef: <empty> not used


7.11
Resource [reboot]

The Resource [reboot] maps to either the Reboot RPC or FactoryReset RPC of TR-069 [4].

When the reboot attribute of the Resource [reboot] is set to “True”, the CSE shall execute the Reboot RPC of TR‑069 [4].

When the factoryReset attribute of Resource [reboot] is set to “True”, the CSE shall execute the FactoryReset RPC of TR-069 [4].

Table 7.11-1: Resource [reboot]

	Attribute Name of [reboot]
	Description

	reboot
	Executes the Reboot RPC

	factoryReset
	FactoryReset RPC


Table 7.11-2: RPC Reboot Arguments

	RPC Reboot Arguments

	CommandKey: Automatically set by the CSE where the reboot is set to “True” in order to correlate the “M-Reboot” Event from the next Inform.


7.12
Resource [cmdhPolicy]

7.12.0
Introduction

The Resource [cmdhPolicy] represents a set of rules defining which CMDH parameters will be used by default when a request issued by a local originator contains the ec (event category) parameter but not all other CMDH parameters, see clause D.12 of TS-0001 [1]. 

The Resource [cmdhPolicy] is a multi-instance Resource where each instance of the Resource shall map to an instance of Device.X_oneM2M_org_CSE.{i}.CMDH.Policy.{i} object. 

The Policy instance shall be created using the Add Object RPC of TR-069 [4].

The Policy instance shall be deleted using the Delete Object RPC of TR-069 [4]. 

The information of a Policy instance shall be retrieved using the GetParameterValues RPC of TR-069 [4]. 

The information of a Policy instance shall be updated using the SetParameterValues RPC of TR-069 [4]. 

Table 7.12-1: Resource [cmdhPolicy]

	Attribute Name of [cmdhPolicy]
	X_oneM2M_org Parameter

	name
	Device.X_oneM2M_org_CSE.{i}.CMDH.Policy.{i}.Name

	cmdhDefaults
	Device.X_oneM2M_org_CSE.{i}.CMDH.Policy.{i}.DefaultRule

	cmdhLimits
	Device.X_oneM2M_org_CSE.{i}.CMDH.Policy.{i}.LimitRules

	cmdhNetworkAccessRules
	Device.X_oneM2M_org_CSE.{i}.CMDH.Policy.{i}.NetworkAccessECRules

	cmdhBuffer
	Device.X_oneM2M_org_CSE.{i}.CMDH.Policy.{i}.BufferRules


7.12.1
Resource [activeCmdhPolicy]

The Resource [activeCmdhPolicy] provides a link to the currently active set of CMDH policies, see clause D.12.1 of TS-0001 [1].

The Resource [activeCmdhPolicy] is mapped to the Enable parameter of the Device.X_oneM2M_org_CSE.{i}.CMDH.Policy.{i} object. 

The information of a Policy instance shall be updated using the SetParameterValues RPC of TR-069 [4]. 

Table 7.12.1-1: Resource [activeCmdhPolicy]

	Attribute Name of [activeCmdhPolicy]
	X_oneM2M_org Parameter

	cmdhPolicy
	Device.X_oneM2M_org_CSE.{i}.CMDH.Policy.{i}.Enable

At most one Policy instance shall be enabled at a time. As such the Policy instance that has the Enable parameter with a value of “True” is the active CMDH policy.


7.12.2
Resource [cmdhDefaults]

The Resource [cmdhDefaults] defines default CMDH policy values, see clause D.12.2 of TS-0001 [1].

The Resource [cmdhDefaults] is a multi-instance Resource where each instance of the Resource shall map to an instance of Device.X_oneM2M_org_CSE.{i}.CMDH.Default.{i} object. 

The Default instance shall be created using the Add Object RPC of TR-069 [4].

The Default instance shall be deleted using the Delete Object RPC of TR-069 [4]. 

The information of a Default instance shall be retrieved using the GetParameterValues RPC of TR-069 [4].

The information of a Default instance shall be updated using the SetParameterValues RPC of TR-069 [4]. 

Table 7.12.2-1: Resource [cmdhDefaults]

	Attribute Name of [cmdhDefaults]
	X_oneM2M_org Parameter

	cmdhDefEcValue
	Device.X_oneM2M_org_CSE.{i}.CMDH.Default.{i}.DefaultECRules

	cmdhEcDefParamValues
	Device.X_oneM2M_org_CSE.{i}.CMDH.Default.{i}.DefaultECParamRules


7.12.3
Resource [cmdhDefEcValue]

The Resource [cmdhDefEcValue] represents a value for the ec (event category) parameter of an incoming request, see clause D.12.3 of TS-0001 [1].

The Resource [cmdhDefEcValue] is a multi-instance Resource where each instance of the Resource shall map to an instance of Device.X_oneM2M_org_CSE.{i}.CMDH.DefaultECRule.{i} object. 

The DefaultECRule instance shall be created using the Add Object RPC of TR-069 [4].

The DefaultECRule instance shall be deleted using the Delete Object RPC of TR-069 [4]. 

The information of a DefaultECRule instance shall be retrieved using the GetParameterValues RPC of TR-069 [4].

The information of a DefaultECRule instance shall be updated using the SetParameterValues RPC of TR-069 [4]. 

Table 7.12.3-1: Resource [cmdhDefEcValue]

	Attribute Name of [cmdhDefEcValue]
	X_oneM2M_org Parameter

	order
	Device.X_oneM2M_org_CSE.{i}.CMDH.DefaultECRule.{i}.Order

	defEcValue
	Device.X_oneM2M_org_CSE.{i}.CMDH.DefaultECRule.{i}.EventCategory

	requestOrigin
	Device.X_oneM2M_org_CSE.{i}.CMDH.DefaultECRule.{i}.RequestOrigin

	requestContext
	Device.X_oneM2M_org_CSE.{i}.CMDH.DefaultECRule.{i}.RequestContext

	requestContextNotification
	Device.X_oneM2M_org_CSE.{i}.CMDH.DefaultECRule.{i}.RequestContextNotificationEnable

	requestCharacteristics
	Device.X_oneM2M_org_CSE.{i}.CMDH.DefaultECRule.{i}.RequestCharacteristics


7.12.4
Resource [cmdhEcDefParamValues]

The Resource [cmdhEcDefParamValues] represents a specific set of default values for the CMDH related parameters rqet (request expiration timestamp), rset (result expiration timestamp), oet (operational execution time), rp (response persistence) and da (delivery aggregation) that are applicable for a given ec (event category) if these parameters are not specified in the request, see clause D.12.4 of TS-0001 [1].

The Resource [cmdhEcDefParamValues] is a multi-instance Resource where each instance of the Resource shall map to an instance of Device.X_oneM2M_org_CSE.{i}.CMDH.DefaultECParamRule.{i} object. 

The DefaultECParamRule instance shall be created using the Add Object RPC of TR-069 [4].

The DefaultECParamRule instance shall be deleted using the Delete Object RPC of TR-069 [4]. 

The information of a DefaultECParamRule instance shall be retrieved using the GetParameterValues RPC of TR-069 [4].

The information of a DefaultECParamRule instance shall be updated using the SetParameterValues RPC of TR-069 [4]. 

Table 7.12.4-1: Resource [cmdhEcDefParamValues]

	Attribute Name of [cmdhEcDefParamValues]
	X_oneM2M_org Parameter

	applicableEventCategory
	Device.X_oneM2M_org_CSE.{i}.CMDH.DefaultECParamRule.{i}.EventCategories

	defaultRequestExpTime
	Device.X_oneM2M_org_CSE.{i}.CMDH.DefaultECParamRule.{i}.RequestExpTime

	defaultResultExpTime
	Device.X_oneM2M_org_CSE.{i}.CMDH.DefaultECParamRule.{i}.ResultExpTime

	defaultOpExecTime
	Device.X_oneM2M_org_CSE.{i}.CMDH.DefaultECParamRule.{i}.OperationExecTime

	defaultRespPersistence
	Device.X_oneM2M_org_CSE.{i}.CMDH.DefaultECParamRule.{i}.ResponsePersistence

	defaultDelAggregation
	Device.X_oneM2M_org_CSE.{i}.CMDH.DefaultECParamRule.{i}.DeliveryAggregation


7.12.5
Resource [cmdhLimits]

The Resource [cmdhLimits] represents limits for CMDH related parameter values, see clause D.12.5 of TS-0001 [1].

The Resource [cmdhLimits] is a multi-instance Resource where each instance of the Resource shall map to an instance of Device.X_oneM2M_org_CSE.{i}.CMDH.Limit.{i} object. 

The Limit instance shall be created using the Add Object RPC of TR-069 [4].

The Limit instance shall be deleted using the Delete Object RPC of TR-069 [4]. 

The information of a Limit instance shall be retrieved using the GetParameterValues RPC of TR-069 [4].

The information of a Limit instance shall be updated using the SetParameterValues RPC of TR-069 [4]. 

Table 7.12.5-1: Resource [cmdhLimits]

	Attribute Name of [cmdhLimits]
	X_oneM2M_org Parameter

	order
	Device.X_oneM2M_org_CSE.{i}.CMDH.Limit.{i}.Order

	requestOrigin
	Device.X_oneM2M_org_CSE.{i}.CMDH.Limit.{i}.RequestOrigin

	requestContext
	Device.X_oneM2M_org_CSE.{i}.CMDH.Limit.{i}.RequestContext

	requestContextNotification
	Device.X_oneM2M_org_CSE.{i}.CMDH.Limit.{i}.RequestContextNotificationEnable

	requestCharacteristics
	Device.X_oneM2M_org_CSE.{i}.CMDH.Limit.{i}.RequestCharacteristics

	limitsEventCategory
	Device.X_oneM2M_org_CSE.{i}.CMDH.Limit.{i}.EventCategories

	limitsRequestExpTime
	Device.X_oneM2M_org_CSE.{i}.CMDH.Limit.{i}.RequestExpTime

	limitsResultExpTime
	Device.X_oneM2M_org_CSE.{i}.CMDH.Limit.{i}.ResultExpTime

	limitsOpExecTime
	Device.X_oneM2M_org_CSE.{i}.CMDH.Limit.{i}.OperationExecTime

	limitsRespPersistence
	Device.X_oneM2M_org_CSE.{i}.CMDH.Limit.{i}.ResponsePersistence

	limitsDelAggregation
	Device.X_oneM2M_org_CSE.{i}.CMDH.Limit.{i}.DeliveryAggregation


7.12.6
Resource [cmdhNetworkAccessRules]

The Resource [cmdhNetworkAccessRules] defines the usage of underlying networks for forwarding information to other CSEs during processing of CMDH-related requests in a CSE, see clause D.12.6 of TS-0001 [1].

The Resource [cmdhNetworkAccessRules] is a multi-instance Resource where each instance of the Resource shall map to an instance of Device.X_oneM2M_org_CSE.{i}.CMDH.NetworkAccessECRule.{i} object. 

The NetworkAccessECRule instance shall be created using the Add Object RPC of TR-069 [4].

The NetworkAccessECRule instance shall be deleted using the Delete Object RPC of TR-069 [4]. 

The information of a NetworkAccessECRule instance shall be retrieved using the GetParameterValues RPC of TR-069 [4].

The information of a NetworkAccessECRule instance shall be updated using the SetParameterValues RPC of TR-069 [4]. 

Table 7.12.6-1: Resource [cmdhNetworkAccessRules]

	Attribute Name of [cmdhNetworkAccessRules]
	X_oneM2M_org Parameter

	applicableEventCategories
	Device.X_oneM2M_org_CSE.{i}.CMDH.NetworkAccessECRule.{i}.EventCategories

	cmdhNwAccessRule
	Device.X_oneM2M_org_CSE.{i}.CMDH.NetworkAccessECRule.{i}.NetworkAccessRules


7.12.7
Resource [cmdhNwAccessRule]

The Resource [cmdhNwAccessRule] define limits in usage of specific underlying networks for forwarding information to other CSEs during processing of CMDH-related requests, see clause D.12.7 of TS-0001 [1].

The Resource [cmdhNwAccessRule] is a multi-instance Resource where each instance of the Resource shall map to an instance of Device.X_oneM2M_org_CSE.{i}.CMDH.NetworkAccessECRule.{i} object. 

The NetworkAccessRule instance shall be created using the Add Object RPC of TR-069 [4].

The NetworkAccessRule instance shall be deleted using the Delete Object RPC of TR-069 [4]. 

The information of a NetworkAccessRule instance shall be retrieved using the GetParameterValues RPC of TR-069 [4].

The information of a NetworkAccessRule instance shall be updated using the SetParameterValues RPC of TR-069 [4]. 

Table 7.12.7-1: Resource [cmdhNwAccessRule]

	Attribute Name of [cmdhNwAccessRule]
	X_oneM2M_org Parameter

	targetNetwork
	Device.X_oneM2M_org_CSE.{i}.CMDH.NetworkAccessRule.{i}.TargetNetworks

	minReqVolume
	Device.X_oneM2M_org_CSE.{i}.CMDH.NetworkAccessRule.{i}.MinimumReqVolume

	backOffParameters
	Device.X_oneM2M_org_CSE.{i}.CMDH.NetworkAccessRule.{i}.BackoffTime

	
	Device.X_oneM2M_org_CSE.{i}.CMDH.NetworkAccessRule.{i}.BackoffTimeIncrement

	
	Device.X_oneM2M_org_CSE.{i}.CMDH.NetworkAccessRule.{i}.MaximumBackoffTime

	otherConditions
	Device.X_oneM2M_org_CSE.{i}.CMDH.NetworkAccessRule.{i}.OtherConditions

	allowedSchedule
	Device.X_oneM2M_org_CSE.{i}.CMDH.NetworkAccessRule.{i}.AllowedSchedule


7.12.8
Resource [cmdhBuffer]

The Resource [cmdhBuffer] represents limits in usage of buffers for temporarily storing information that needs to be forwarded to other CSEs during processing of CMDH-related requests in a CSE, see clause D.12.8 of TS-0001 [1].

The Resource [cmdhBuffer] is a multi-instance Resource where each instance of the Resource shall map to an instance of Device.X_oneM2M_org_CSE.{i}.CMDH.Buffer.{i} object. 

The Buffer instance shall be created using the Add Object RPC of TR-069 [4].

The Buffer instance shall be deleted using the Delete Object RPC of TR-069 [4]. 

The information of a Buffer instance shall be retrieved using the GetParameterValues RPC of TR-069 [4].

The information of a Buffer instance shall be updated using the SetParameterValues RPC of TR-069 [4]. 

Table 7.12.8-1: Resource [cmdhBuffer]

	Attribute Name of [cmdhBuffer]
	X_oneM2M_org Parameter

	applicableEventCategory
	Device.X_oneM2M_org_CSE.{i}.CMDH.Buffer.{i}.EventCategories

	maxBufferSize
	Device.X_oneM2M_org_CSE.{i}.CMDH.Buffer.{i}.MaximumBufferSize

	storagePriority
	Device.X_oneM2M_org_CSE.{i}.CMDH.Buffer.{i}.StoragePriority


7.13
Resource Type <mgmtCmd>

Each mgmtCmd Resource shall map to BBF TR-069 RPC commands based on the value of cmdType. Accordingly, execReqArgs shall contain arguments related to the corresponding BBF TR-069 RPCs. The details about corresponding procedure mapping are described in clause 8.2.

Table 7.13-1: Resource Type <mgmtCmd>

	Attribute cmdType of mgmtCmd
	Attribute execReqArgs of mgmtCmd

	cmdType = RESET
	Shall include all arguments related to BBF FactoryReset RPC

	cmdType = REBOOT
	Shall include all arguments related to BBF Reboot RPC

	cmdType = UPLOAD
	Shall include all arguments related to BBF Reboot RPC

	cmdType = DOWNLOAD
	Shall contain all arguments related to BBF Reboot RPC

	cmdType = SOFTWAREINSTALL
	Shall contain all arguments related to BBF ChangeDUState RPC which shall contain “InstallOpStruct” structure.

	cmdType = SOFTWAREUNINSTALL
	Shall contain all arguments related to BBF ChangeDUState RPC which shall contain “UninstallOpStruct” structure.


7.14
Resource Type <execInstance>

The <execInstance> resource from TS-0004 [2] shall map to BBF CancelTransfer RPC commands when it is disabled/cancelled using a Update operation or deleted using a Delete operation. The details are described in clause 8.2.
7.15
Resource [registration]

The Resource [registration] represents the configuration information needed to register and AE or CSE with a Registrar CSE.

The Resource [registration] is a multi-instance object where the key of the object is the originatorID (i.e., AE-ID, CSE-ID). The following rules are used to determine the object instance based on the originatorID:

· When the originatorID resource is for a CSE-ID, the TR-069 object instance Device.X_oneM2M_org_CSE.{i} shall be used for the specified CSE-ID.

· When the originatorID resource is for an AE-ID, the TR-069 object instance Device.X_oneM2M_org_AE.{i} shall be used for the specified AE-ID. 

The information shall be created using the Add Object RPC of TR-069 [4].

The information shall be deleted using the Delete Object RPC of TR-069 [4]. 

The information shall be retrieved using the GetParameterValues RPC of TR-069 [4]. 

The capabilities shall be enabled and disabled using the SetParameterValues RPC of TR-069 [4].

Table 7.15-1: Resource [registration] for CSE
	Attribute Name of [registration]
	TR-181 Parameter (X_oneM2M_org_CSE)

	originatorID
	ID – See description of the type of object to instantiate.

	poA
	PointOfAccess

	externalID
	ExternalID

	triggerRecipientID
	TriggerRecipientID

	mgmtLink [authenticationProfile]
	AuthenticationProfile (TR-069 reference parameter that references a row in the Device.X_oneM2M_org_SecuritySolution.AuthenticationProfile table)


Table 7.15-2: Resource [registration] for AE
	Attribute Name of [registration]
	TR-181 Parameter (X_oneM2M_org_AE)

	originatorID
	ID – See description of the type of object to instantiate.

	poA
	PointOfAccess

	appID
	ApplicationID







	mgmtLink [authenticationProfile]
	AuthenticationProfile (TR-069 reference parameter that references a row in the Device.X_oneM2M_org_SecuritySolution.AuthenticationProfile table)

	
	

	
	

	
	


7.16
Resource [dataCollection]

The Resource [dataCollection] represents data collection (measurement) and transmittal (reporting) properties for an AE.

The information shall be retrieved using the GetParameterValues RPC of TR-069 [4]. 

The capabilities shall be enabled and disabled using the SetParameterValues RPC of TR-069 [4].

Table 7.15-1: Resource [dataCollection]

	Attribute Name of [dataCollection]
	TR-181 Parameter

	containerPath
	ContainerPath

	reportingSchedule
	ReportingSchedule

	measurementSchedule
	CollectionSchedule


7.17
Security Solutions
7.17.1
Introduction

This clauses in the section of the this present document contains information on how to map the security specific management resources from TS-0022 [8] to managed objects and parameters as defined in the TR-181 [6] data model or the Remote Procedure Calls (RPCs) in TR-069 [4].
7.17.2
Resource [authenticationProfile]

The Resource [authenticationProfile] represents configuration information regarding establishing mutually-authenticated secure communications. The security principal using this configuration information can be a CSE or AE or the Managed ADN/ASN/MN acting as security principal on behalf of AEs on the Node, see clause 7.1.4 of TS-0022 [8].

The Resource [authenticationProfile] is a multi-instance Resource where each instance of the Resource shall map to an instance of Device.X_oneM2M_org_SecuritySolution.AuthenticationProfile.{i} object. 

The AuthenticationRule instance shall be created using the Add Object RPC of TR-069 [4].

The AuthenticationRule instance shall be deleted using the Delete Object RPC of TR-069 [4]. 

The information of a AuthenticationProfile instance shall be retrieved using the GetParameterValues RPC of TR-069 [4].

The information of a AuthenticationProfile instance shall be updated using the SetParameterValues RPC of TR-069 [4]. 

Table 7.17.2-1: Resource [auhenticationProfile]

	Attribute Name of [authenticationProfile]
	Parameters of
Device.X_oneM2M_org_SecuritySolution.AuthenticationProfile.{i}



	SUID
	SUID

	TLSCiphersuites
	TLSCiphersuites

	symmKeyID
	SymmetricKeyID

	symmKeyValue
	SymmetricKeyValue

	MAFFQDN
	MAFFQDN

	MAFKeyRegDuration
	MAFKeyRegDuration

	mycertFingerprint
	MyCert (reference)

	rawPubKeyID
	RawPubKeyID

	mgmtLink [trustAnchorCred]
	TrustAnchorCredentials (list of references)


The parameter MyCert is a TR-069 reference parameter that references a row in the Device.Security.Certificate table where the value of the mycertFingerprint attribute matches the value of a Device.Security.Certficate.{i}.X_oneM2M_org_Fingerprint parameter. The X_oneM2M_org_Fingerprint parameter shall be a unique key for the Device.Security.Certificate table. 
The parameter TrustAnchorCredentials is a list of TR-069 reference parameter where each entry in the list references a row in the Device.X_oneM2M_org_SecuritySolution.TrustAnchorCredential table.

7.17.3
Resource [trustAnchorCred]

The Resource [trustAnchorCred] represents configuration information regarding certificates provided by certificate authorities used be managed entities to authenticate peer endpoints, see clause 7.1.6 of TS-0022 [8].

The Resource [trustAnchorCred] is a multi-instance Resource where each instance of the Resource shall map to an instance of Device.X_oneM2M_org_SecuritySolution.TrustAnchorCredential.{i} object. 

The TrustAnchorCredential instance shall be created using the Add Object RPC of TR-069 [4].

The TrustAnchorCredential instance shall be deleted using the Delete Object RPC of TR-069 [4]. 

The information of a TrustAnchorCredential instance shall be retrieved using the GetParameterValues RPC of TR-069 [4].

The information of a TrustAnchorCredential instance shall be updated using the SetParameterValues RPC of TR-069 [4]. 

Table 7.17.3-1: Resource [trustAnchorCred]

	Attribute Name of [trustAnchorCred]
	Parameters of

Device.X_oneM2M_org_SecuritySolution.TrustAnchorCredential.{i}



	certFingerprint
	Fingerprint

	URI
	RemoteTrustStore


7.17.4
Resource [myCertFileCred]

The Resource [myCertFileCred] represents configuration information regarding certificates presented by the managed entity to remote entities for the establishment of secure communications, see clause 7.1.5 of TS-0022 [8].

The Resource [myCertFileCred] is a multi-instance Resource where each instance of the Resource shall map to an instance of Device.Security.Certificate.{i} object. 

The Certificate instance shall be created either using the Download RPC of TR-069 [4] or via an out-of-band mechanism.
The Certificate instance shall be deleted using using the Download RPC of TR-069 [4] or via an out-of-band mechanism.
The information of a Certificate instance shall be retrieved using the GetParameterValues RPC of TR-069 [4].

The information of a Certificate instance shall be updated using the SetParameterValues RPC of TR-069 [4]. 

Table 7.17.4-1: Resource [myCertFileCred]

	Attribute Name of [myCertFileCred]
	Parameters of

Device.Security.Credential.{i}



	SUIDs
	X_oneM2M_org_SUIDs

	myCertFileFormat
	X_oneM2M_org_Format

	myCertFileContent
	The certificate is downloaded as part of the Download RPC of TR-069


The parameter AuthenticationProfile is a TR-069 reference parameter that references a row in the Device.X_oneM2M_org_SecuritySolution.AuthenticationProfile table where the value of the mycertFingerprint attribute matches the value of a Device.Security.Certficate.{i}.X_oneM2M_org_Fingerprint parameter. The X_oneM2M_org_Fingerprint parameter shall be a unique key for the Device.Security.Certificate table
7.17.5
Resource [MAFClientRegCfg]

The Resource [MAFClientRegCfg] represents configuration information that permits a MAF client to register with a MAF in order to securely exchange keys used for secure communications, see clause 7.1.5 of TS-0022 [8].

The Resource [MAFClientRegCfg] is a multi-instance Resource where each instance of the Resource shall map to an instance of Device.X_oneM2M_org_SecuritySolution.MAFClientRegistration.{i} object. 

The MAFClientRegistration instance shall be created using the Add Object RPC of TR-069 [4].

The MAFClientRegistration instance shall be deleted using the Delete Object RPC of TR-069 [4]. 

The information of a MAFClientRegistration instance shall be retrieved using the GetParameterValues RPC of TR-069 [4].

The information of a MAFClientRegistration instance shall be updated using the SetParameterValues RPC of TR-069 [4]. 

Table 7.17.5-1: Resource [MAFClientRegCfg]

	Attribute Name of [MAFClientRegCfg]
	Parameters of

Device.X_oneM2M_org_SecuritySolution.MAFClientRegistration.{i}

	mgmtLink [authenticationProfile\
	AuthenticationProfile (TR-069 reference parameter that references a row in the Device.X_oneM2M_org_SecuritySolution.AuthenticationProfile table)

	URI
	URI

	httpPort
	HTTPPort

	coapPort
	CoAPPort

	websocketPort
	WebsocketPort

	expirationTime
	ExpirationTimeStamp


8
Mapping of procedures for management
8.0
Introduction

This clause contains all information on how to map management resource primitives from TS-0004 [2] to the Remote Procedure Calls (RPCs) in TR-069 [4].

8.1
Resource Type <mgmtObj> primitive mappings

8.1.0
Introduction

This clause contains all information on how to map Resource Type <mgmtObj> primitives from TS-0004 [2] to the Remote Procedure Calls (RPCs) in TR-069 [4].

8.1.1
Alias-Based Addressing Mechanism 

In order to utilize the Alias-Based Addressing Mechanism, the mechanism has to be supported by the ACS and CPE in order to map the M2M Service Layer identifier for the Resource instance to the CPE object instance. If the Alias‑Based Addressing Mechanism feature is not supported by either the ACS or CPE, the CSE has to retain the mapping of the these M2M Resource instance identifiers.

8.1.2
Create primitive mapping

8.1.2.0
Introduction

The Create Request and Response primitives shall map to the AddObject RPC. The AddObject RPC is defined in TR-069 [4] as a synchronous RPC and returns a successful response or one of the following fault codes in Table 8.1.2-1.

Table 8.1.2-1: AddObject Fault Code Mapping
	Fault code
	Description
	Response Status Code

	9001
	Request denied (no reason specified)
	STATUS_BAD_REQUEST

	9002
	Internal error
	STATUS_BAD_REQUEST

	9003
	Invalid arguments
	STATUS_BAD_REQUEST

	9004
	Resources exceeded (when used in association with SetParameterValues, this cannot be used to indicate Parameters in error)
	STATUS_BAD_REQUEST

	9005
	Invalid Parameter name (associated with Set/GetParameterValues, GetParameterNames, Set/GetParameterAttributes, AddObject, and DeleteObject)
	STATUS_NOT_IMPLEMENTED


8.1.2.1
M2M Service Layer Resource Instance Identifier mapping

When the Resource is a multi-instance Resource, the AddObject RPC should utilize the Alias-Based Addressing Mechanism as defined in Section 3.6.1 of TR-069 [4] in order to use the Resource instance value of the URI.

8.1.3
Delete primitive mapping

8.1.3.1
Delete primitive mapping for deletion of Object Instances

The Delete Request and Response primitives that results in the deletion of a Resource shall map to the DeleteObject RPC. The DeleteObject RPC is defined in TR-069 [4] as a synchronous RPC and returns a successful response or one of the following fault codes in Table 8.1.3.1-1.

Table 8.1.3.1-1: DeleteObject Fault Code Mapping
	Fault code
	Description
	Response Status Code

	9001
	Request denied (no reason specified)
	STATUS_BAD_REQUEST

	9002
	Internal error
	STATUS_BAD_REQUEST

	9003
	Invalid arguments
	STATUS_BAD_REQUEST

	9005
	Invalid Parameter name (associated with Set/GetParameterValues, GetParameterNames, Set/GetParameterAttributes, AddObject, and DeleteObject)
	STATUS_NOT_IMPLEMENTED


8.1.3.2
Delete primitive mapping for software un-install operation

The Delete Request and Response primitives that results in a software un-install operation (e.g. Resource [software]) shall use the ChangeDUState mechanism defined in TR-069 [4]. The ChangeDUState mechanism is an asynchronous command that consists of the synchronous ChangeDUState RPC for the un-installation request and the asynchronous ChangeDUStateComplete RPC. The ChangeDUState RPC returns a successful response or one of the following fault codes in Table 8.1.3.2-1. A successful response means that the CPE has accepted the ChangeDUState RPC.

Table 8.1.3.2-1: ChangeDUState Fault Code Mapping

	Fault code
	Description
	Response Status Code

	9000
	Method not supported
	STATUS_BAD_REQUEST

	9001
	Request denied (no reason specified)
	STATUS_BAD_REQUEST

	9002
	Internal error
	STATUS_BAD_REQUEST

	9004
	Resources exceeded (when used in association with SetParameterValues, this cannot be used to indicate Parameters in error)
	STATUS_BAD_REQUEST


Once the CPE has attempted to change the state of the deployment unit, the CPE reports the result of the state change operation using the ChangeDUStateComplete RPC. The ChangeDUStateComplete RPC indicates a successful operation or one of the following fault codes in Table 8.1.3.2-2.

Table 8.1.3.2-2: ChangeDUStateComplete Fault Code Mapping
	Fault code
	Description
	Response Status Code

	9001
	Request denied (no reason specified)
	STATUS_BAD_REQUEST

	9003
	Invalid arguments
	STATUS_BAD_REQUEST

	9012
	File transfer server authentication failure (associated with Upload, Download, TransferComplete, AutonomousTransferComplete, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_BAD_REQUEST

	9013
	Unsupported protocol for file transfer (associated with Upload, Download, ScheduleDownload, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_BAD_REQUEST

	9015
	File transfer failure: unable to contact file server (associated with Download, TransferComplete, AutonomousTransferComplete, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_BAD_REQUEST

	9016
	File transfer failure: unable to access file (associated with Download, TransferComplete, AutonomousTransferComplete, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_BAD_REQUEST

	9017
	File transfer failure: unable to complete download (associated with Download, TransferComplete, AutonomousTransferComplete, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_BAD_REQUEST

	9018
	File transfer failure: file corrupted or otherwise unusable (associated with Download, TransferComplete, AutonomousTransferComplete, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_BAD_REQUEST

	9022
	Invalid UUID Format (associated with DUStateChangeComplete or AutonomousDUStateChangeComplete methods: Install, Update, and Uninstall)
	STATUS_BAD_REQUEST

	9023
	Unknown Execution Environment (associated with DUStateChangeComplete or AutonomousDUStateChangeComplete methods: Install only)
	STATUS_BAD_REQUEST

	9024
	Disabled Execution Environment (associated with DUStateChangeComplete or AutonomousDUStateChangeComplete methods: Install, Update, and Uninstall)
	STATUS_BAD_REQUEST

	9025
	Deployment Unit to Execution Environment Mismatch (associated with DUStateChangeComplete or AutonomousDUStateChangeComplete methods: Install and Update)
	STATUS_BAD_REQUEST

	9026
	Duplicate Deployment Unit (associated with DUStateChangeComplete or AutonomousDUStateChangeComplete methods: Install only)
	STATUS_BAD_REQUEST

	9027
	System Resources Exceeded (associated with DUStateChangeComplete or AutonomousDUStateChangeComplete methods: Install and Update)
	STATUS_BAD_REQUEST

	9028
	Unknown Deployment Unit (associated with DUStateChangeComplete or AutonomousDUStateChangeComplete methods: Update and Uninstall)
	STATUS_BAD_REQUEST

	9029
	Invalid Deployment Unit State (associated with DUStateChangeComplete or AutonomousDUStateChangeComplete methods: Install, Update and Uninstall)
	STATUS_BAD_REQUEST

	9030
	Invalid Deployment Unit Update – Downgrade not permitted (associated with DUStateChangeComplete or AutonomousDUStateChangeComplete methods: Update only)
	STATUS_BAD_REQUEST

	9031
	Invalid Deployment Unit Update – Version not specified (associated with DUStateChangeComplete or AutonomousDUStateChangeComplete methods: Update only)
	STATUS_BAD_REQUEST

	9032
	Invalid Deployment Unit Update – Version already exists (associated with DUStateChangeComplete or AutonomousDUStateChangeComplete methods: Update only)
	STATUS_BAD_REQUEST


8.1.4
Update primitive mapping

8.1.4.1
Update primitive mapping for Parameter modifications

The Update Request and Response primitives that modifies the value of Resource attributes shall map to the SetParameterValues RPC. The SetParametersValue RPC is defined in TR-069 [4] as a synchronous RPC and returns a successful response or one of the following fault codes in Table 8.1.4.1-1.

Table 8.1.4.1-1: SetParameterValues Fault Code Mapping
	Fault code
	Description
	Response Status Code

	9001
	Request denied (no reason specified)
	STATUS_BAD_REQUEST

	9002
	Internal error
	STATUS_BAD_REQUEST

	9003
	Invalid arguments
	STATUS_BAD_REQUEST

	9004
	Resources exceeded (when used in association with SetParameterValues, this cannot be used to indicate Parameters in error)
	STATUS_BAD_REQUEST

	9005
	Invalid Parameter name (associated with Set/GetParameterValues, GetParameterNames, Set/GetParameterAttributes, AddObject, and DeleteObject)
	STATUS_NOT-IMPLEMENTED

	9006
	Invalid Parameter type (associated with SetParameterValues)
	STATUS_BAD_REQUEST

	9007
	Invalid Parameter value (associated with SetParameterValues)
	STATUS_BAD_REQUEST

	9008
	Attempt to set a non-writable Parameter (associated with SetParameterValues)
	STATUS_BAD_REQUEST


8.1.4.2
Update primitive mapping for upload file transfer operations

The Update Request and Response primitives that results in an upload file transfer operation (e.g. logStop attribute of the Resource [eventLog]) shall use the Upload mechanism defined in TR-069 [4]. The Upload mechanism is an asynchronous command that consists of the synchronous Upload RPC for the Upload and the asynchronous TransferComplete RPC. The Upload RPC returns a successful response or one of the following fault codes in Table 8.1.4.2-1. A successful response means that the CPE has accepted the Upload RPC.

Table 8.1.4.2-1: Upload Fault Code Mapping
	Fault code
	Description
	Response Status Code

	9000
	Method not supported
	STATUS_BAD_REQUEST

	9001
	Request denied (no reason specified)
	STATUS_BAD_REQUEST

	9002
	Internal error
	STATUS_BAD_REQUEST

	9003
	Invalid arguments
	STATUS_BAD_REQUEST

	9004
	Resources exceeded (when used in association with SetParameterValues, this cannot be used to indicate Parameters in error)
	STATUS_BAD_REQUEST

	9011
	Upload failure (associated with Upload, TransferComplete or Autonomous​Trans​ferComplete methods).
	STATUS_BAD_REQUEST

	9012
	File transfer server authentication failure (associated with Upload, Download, TransferComplete, AutonomousTransferComplete, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_BAD_REQUEST

	9013
	Unsupported protocol for file transfer (associated with Upload, Download, ScheduleDownload, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_BAD_REQUEST


Once the CPE has attempted to upload the file, the CPE reports the result of the Upload operation using the TransferComplete RPC. The TransferComplete RPC indicates a successful operation or one of the following fault codes in Table 8.1.4.2-2. 

Table 8.1.4.2-2: TransferComplete Fault Code Mapping

	Fault code
	Description
	Response Status Code

	9001
	Request denied (no reason specified)
	STATUS_BAD_REQUEST

	9002
	Internal error
	STATUS_BAD_REQUEST

	9010
	File transfer failure (associated with Download, ScheduleDownload, TransferComplete or AutonomousTransferComplete methods).
	STATUS_BAD_REQUEST

	9011
	Upload failure (associated with Upload, TransferComplete or Autonomous​Trans​ferComplete methods).
	STATUS_BAD_REQUEST

	9012
	File transfer server authentication failure (associated with Upload, Download, TransferComplete, AutonomousTransferComplete, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_BAD_REQUEST

	9014
	File transfer failure: unable to join multicast group (associated with Download, TransferComplete or AutonomousTransferComplete methods).
	STATUS_BAD_REQUEST

	9015
	File transfer failure: unable to contact file server (associated with Download, TransferComplete, AutonomousTransferComplete, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_BAD_REQUEST

	9016
	File transfer failure: unable to access file (associated with Download, TransferComplete, AutonomousTransferComplete, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_BAD_REQUEST

	9017
	File transfer failure: unable to complete download (associated with Download, TransferComplete, AutonomousTransferComplete, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_BAD_REQUEST

	9018
	File transfer failure: file corrupted or otherwise unusable (associated with Download, TransferComplete, AutonomousTransferComplete, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_BAD_REQUEST

	9019
	File transfer failure: file authentication failure (associated with Download, TransferComplete or AutonomousTransferComplete methods).
	STATUS_BAD_REQUEST

	9020
	File transfer failure: unable to complete download within specified time windows (associated with TransferComplete method).
	STATUS_BAD_REQUEST


8.1.4.3
Update primitive mapping for download file transfer operations

The Update Request and Response primitives that results in a download file transfer operation (e.g. update attribute of Resource [firmware]) shall use the Download mechanism defined in TR-069 [4]. The Download mechanism is an asynchronous command that consists of the synchronous Download RPC for the Download and the asynchronous TransferComplete RPC. The Download RPC returns a successful response or one of the following fault codes in Table 8.1.4.3-1. A successful response means that the CPE has accepted the Download RPC.

Table 8.1.4.3-1: Download Fault Code Mapping

	Fault code
	Description
	Response Status Code

	9000
	Method not supported
	STATUS_BAD_REQUEST

	9001
	Request denied (no reason specified)
	STATUS_BAD_REQUEST

	9002
	Internal error
	STATUS_BAD_REQUEST

	9003
	Invalid arguments
	STATUS_BAD_REQUEST

	9004
	Resources exceeded (when used in association with SetParameterValues, this cannot be used to indicate Parameters in error)
	STATUS_BAD_REQUEST

	9010
	File transfer failure (associated with Download, ScheduleDownload, TransferComplete or AutonomousTransferComplete methods).
	STATUS_BAD_REQUEST

	9012
	File transfer server authentication failure (associated with Upload, Download, TransferComplete, AutonomousTransferComplete, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_BAD_REQUEST

	9013
	Unsupported protocol for file transfer (associated with Upload, Download, ScheduleDownload, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_BAD_REQUEST


Once the CPE has attempted to download the file, the CPE reports the result of the download operation using the TransferComplete RPC. The TransferComplete RPC indicates a successful operation or one of the following fault codes in Table 8.1.4.3-2.

Table 8.1.4.3-2: TransferComplete Fault Code Mapping
	Fault code
	Description
	Response Status Code

	9001
	Request denied (no reason specified)
	STATUS_BAD_REQUEST

	9002
	Internal error
	STATUS_BAD_REQUEST

	9010
	File transfer failure (associated with Download, ScheduleDownload, TransferComplete or AutonomousTransferComplete methods).
	STATUS_BAD_REQUEST

	9011
	Upload failure (associated with Upload, TransferComplete or Autonomous​Trans​ferComplete methods).
	STATUS_BAD_REQUEST

	9012
	File transfer server authentication failure (associated with Upload, Download, TransferComplete, AutonomousTransferComplete, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_BAD_REQUEST

	9014
	File transfer failure: unable to join multicast group (associated with Download, TransferComplete or AutonomousTransferComplete methods).
	STATUS_BAD_REQUEST

	9015
	File transfer failure: unable to contact file server (associated with Download, TransferComplete, AutonomousTransferComplete, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_BAD_REQUEST

	9016
	File transfer failure: unable to access file (associated with Download, TransferComplete, AutonomousTransferComplete, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_BAD_REQUEST

	9017
	File transfer failure: unable to complete download (associated with Download, TransferComplete, AutonomousTransferComplete, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_BAD_REQUEST

	9018
	File transfer failure: file corrupted or otherwise unusable (associated with Download, TransferComplete, AutonomousTransferComplete, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_BAD_REQUEST

	9019
	File transfer failure: file authentication failure (associated with Download, TransferComplete or AutonomousTransferComplete methods).
	STATUS_BAD_REQUEST

	9020
	File transfer failure: unable to complete download within specified time windows (associated with TransferComplete method).
	STATUS_BAD_REQUEST


8.1.4.4
Update primitive mapping for reboot operation

The Update Request and Response primitives that results in a reboot operation (e.g. reboot attribute of Resource [reboot]) shall use the Reboot RPC defined in TR-069 [4]. The Reboot RPC is asynchronous command. The Reboot RPC returns a successful response or one of the following fault codes in Table 8.1.4.4-1. 

Table 8.1.4.4-1: Reboot Fault Code Mapping

	Fault code
	Description
	Response Status Code

	9001
	Request denied (no reason specified)
	STATUS_BAD_REQUEST

	9002
	Internal error
	STATUS_BAD_REQUEST

	9003
	Invalid arguments
	STATUS_BAD_REQUEST


8.1.4.5
Update primitive mapping for factory reset operation

The Update Request and Response primitives that results in a factory reset operation (e.g. factoryReset attribute of Resource [ reboot]) shall use the FactoryReset RPC defined in TR-069 [4]. The FactoryReset RPC is an asynchronous command. The FactoryReset RPC returns a successful response or one of the following fault codes in Table 8.1.4.5-1. 

Table 8.1.4.5-1: FactoryReset Fault Code Mapping

	Fault code
	Description
	Response Status Code

	9000
	Method not supported
	STATUS_BAD_REQUEST

	9001
	Request denied (no reason specified)
	STATUS_BAD_REQUEST

	9002
	Internal error
	STATUS_BAD_REQUEST

	9003
	Invalid arguments
	STATUS_BAD_REQUEST


8.1.4.6
Update primitive mapping for software install operation

The Update Request and Response primitives that results in a software installation operation (e.g. install attribute of Resource [software]) shall use the ChangeDUState mechanism defined in TR-069 [4]. The ChangeDUState mechanism is an asynchronous command that consists of the synchronous ChangeDUState RPC for the download and the asynchronous ChangeDUStateComplete RPC. The ChangeDUState RPC returns a successful response or one of the following fault codes in Table 8.1.4.6-1. A successful response means that the CPE has accepted the ChangeDUState RPC.

Table 8.1.4.6-1: ChangeDUState Fault Code Mapping

	Fault code
	Description
	Response Status Code

	9000
	Method not supported
	STATUS_BAD_REQUEST

	9001
	Request denied (no reason specified)
	STATUS_BAD_REQUEST

	9002
	Internal error
	STATUS_BAD_REQUEST

	9004
	Resources exceeded (when used in association with SetParameterValues, this cannot be used to indicate Parameters in error)
	STATUS_BAD_REQUEST


Once the CPE has attempted to change the state of the deployment unit, the CPE reports the result of the state change operation using the ChangeDUStateComplete RPC. The ChangeDUStateComplete RPC indicates a successful operation or one of the following fault codes in Table 8.1.4.6-2.

Table 8.1.4.6-2: ChangeDUStateComplete Fault Code Mapping
	Fault code
	Description
	Response Status Code

	9001
	Request denied (no reason specified)
	STATUS_BAD_REQUEST

	9003
	Invalid arguments
	STATUS_BAD_REQUEST

	9012
	File transfer server authentication failure (associated with Upload, Download, TransferComplete, AutonomousTransferComplete, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_BAD_REQUEST

	9013
	Unsupported protocol for file transfer (associated with Upload, Download, ScheduleDownload, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_BAD_REQUEST

	9015
	File transfer failure: unable to contact file server (associated with Download, TransferComplete, AutonomousTransferComplete, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_BAD_REQUEST

	9016
	File transfer failure: unable to access file (associated with Download, TransferComplete, AutonomousTransferComplete, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_BAD_REQUEST

	9017
	File transfer failure: unable to complete download (associated with Download, TransferComplete, AutonomousTransferComplete, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_BAD_REQUEST

	9018
	File transfer failure: file corrupted or otherwise unusable (associated with Download, TransferComplete, AutonomousTransferComplete, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_BAD_REQUEST

	9022
	Invalid UUID Format (associated with DUStateChangeComplete or AutonomousDUStateChangeComplete methods: Install, Update, and Uninstall)
	STATUS_BAD_REQUEST

	9023
	Unknown Execution Environment (associated with DUStateChangeComplete or AutonomousDUStateChangeComplete methods: Install only)
	STATUS_BAD_REQUEST

	9024
	Disabled Execution Environment (associated with DUStateChangeComplete or AutonomousDUStateChangeComplete methods: Install, Update, and Uninstall)
	STATUS_BAD_REQUEST

	9025
	Deployment Unit to Execution Environment Mismatch (associated with DUStateChangeComplete or AutonomousDUStateChangeComplete methods: Install and Update)
	STATUS_BAD_REQUEST

	9026
	Duplicate Deployment Unit (associated with DUStateChangeComplete or AutonomousDUStateChangeComplete methods: Install only)
	STATUS_BAD_REQUEST

	9027
	System Resources Exceeded (associated with DUStateChangeComplete or AutonomousDUStateChangeComplete methods: Install and Update)
	STATUS_BAD_REQUEST

	9028
	Unknown Deployment Unit (associated with DUStateChangeComplete or AutonomousDUStateChangeComplete methods: Update and Uninstall)
	STATUS_BAD_REQUEST

	9029
	Invalid Deployment Unit State (associated with DUStateChangeComplete or AutonomousDUStateChangeComplete methods: Install, Update and Uninstall)
	STATUS_BAD_REQUEST

	9030
	Invalid Deployment Unit Update – Downgrade not permitted (associated with DUStateChangeComplete or AutonomousDUStateChangeComplete methods: Update only)
	STATUS_BAD_REQUEST

	9031
	Invalid Deployment Unit Update – Version not specified (associated with DUStateChangeComplete or AutonomousDUStateChangeComplete methods: Update only)
	STATUS_BAD_REQUEST

	9032
	Invalid Deployment Unit Update – Version already exists (associated with DUStateChangeComplete or AutonomousDUStateChangeComplete methods: Update only)
	STATUS_BAD_REQUEST


8.1.5
Retrieve primitive mapping

The Retrieve Request and Response primitives shall map to the GetParameterValues RPC. The GetParametersValue RPC is defined in TR-069 [4] as a synchronous RPC and returns a successful response or one of the following fault codes in Table 8.1.5-1.

Table 8.1.5-1: GetParameterValues Fault Code Mapping
	Fault code
	Description
	Response Status Code

	9001
	Request denied (no reason specified)
	STATUS_BAD_REQUEST

	9002
	Internal error
	STATUS_BAD_REQUEST

	9003
	Invalid arguments
	STATUS_BAD_REQUEST

	9004
	Resources exceeded (when used in association with SetParameterValues, this cannot used to indicate Parameters in error)
	STATUS_BAD_REQUEST

	9005
	Invalid Parameter name (associated with Set/GetParameterValues, GetParameterNames, Set/GetParameterAttributes, AddObject, and DeleteObject)
	STATUS_BAD_REQUEST


8.1.6
Notify primitive mapping

8.1.6.0
Introduction

The NotifyRequest and Response primitives permit notifications to AE or CSEs that have subscribed to a Resource. 

While TR-069 [4] has the capability to notify the subscribed ACS when an object’s parameter has been modified, TR‑069 [4] does not have the capability for an ACS to be notified if any parameter within the object has been modified unless the ACS individually subscribes to all the parameters of the object.

As such the procedure for mapping the Notify Request and Response primitives for TR-069 [4] is not possible unless the CSE subscribes to receive notification to all the parameters of an Object that are mapped to the Resource’s attributes.

NOTE:
In many implementations, subscribing to all the parameters of an Object that are mapped to the Resource can cause performance issues in the CPE as well as the CSE. As such using the attribute based subscription capabilities of TR-069 [4] for subscription of Resources should be avoided when possible.

8.1.6.1
Procedure for subscribed Resource attributes.

When a <subscription> Resource for a <mgmtObj> Resource is Created, Deleted or Updated the CSE shall map to the SetParameterAttributes RPC in the following manner:

· TR-069 [4] provides the capability to subscribe to changes of a specific attribute through the use of the SetParameterAttributes RPC using the “Active” value for the Notification parameter.

· TR-069 [4] provides the capability to un-subscribe to changes of a specific attribute through the use of the SetParameterAttributes RPC using the “None” value for the Notification parameter.

The SetParametersAttributes RPC is defined in TR-069 [4] as a synchronous RPC and returns a successful response or one of the following fault codes in Table 8.1.6.1-1.

Table 8.1.6.1-1: SetParameterAttributes Fault Code Mapping
	Fault code
	Description
	Response Status Code

	9000
	Method not supported
	STATUS_BAD_REQUEST

	9001
	Request denied (no reason specified)
	STATUS_BAD_REQUEST

	9002
	Internal error
	STATUS_BAD_REQUEST

	9003
	Invalid arguments
	STATUS_BAD_REQUEST

	9004
	Resources exceeded (when used in association with SetParameterValues, this cannot be used to indicate Parameters in error)
	STATUS_BAD_REQUEST

	9010
	File transfer failure (associated with Download, ScheduleDownload, TransferComplete or AutonomousTransferComplete methods).
	STATUS_BAD_REQUEST


8.1.6.2
Notification primitive mapping

Notify Request and Response primitives shall map to the TR-069 notification mechanism. CPEs produce notifications for subscribed attributes using the TR-069 Inform method, the Inform method has an argument Event that has as one of the EventCodes with the value “4 VALUE CHANGE” indicating that a subscribed parameter’s value has changed. The parameter(s) that have changed are included ParameterList argument of the Inform method.

The ParameterList argument is list of name-value pairs; the name is parameter name and shall be mapped to the objectPath attribute of the Resource while the value is the most recent value of the parameter.

NOTE:
TR-069 CPEs do not report value changes of parameters that were modified by the ACS.

8.2
<mgmtCmd> and <execInstance> resource primitive mappings

8.2.1
Update (Execute) primitive for the <mgmtCmd> resource

8.2.1.0
Introduction

When the Update Request primitive for <mgmtCmd> resource addresses the execEnable attribute of the <mgmtCmd> resource, it effectively triggers an Execute <mgmtCmd> procedure.

The Hosting CSE performs command conversion of its <execInstance> sub-resources. The mapping between the <execInstance> attributes and the TR-069 [4] RPC procedures triggered is based on the value of the cmdType attribute of the <mgmtCmd> resource defined in Table 8.2.1-1. The CPE acceptance of the corresponding RPC procedures is indicated by returning a successful Response primitive to the initial Update Request.

The Fault Codes which may be returned by the CPE to the Hosting CSE are mapped onto execStatus codes and stored in the corresponding <execInstance> attributes, and are detailed in the following clauses:

Table 8.2.1-1 Mapping of Execute <mgmtCmd> primitives to BBF TR-069 RPC
	cmdType value 
	BBF TR-069 RPCs

	 “DOWNLOAD”
	Download RPC (see clause 8.2.1.1) and TransferComplete RPC (clause 8.2.1.3)

	“UPLOAD”
	Upload RPC (clause 8.2.1.2) and TransferComplete RPC (clause 8.2.1.3)

	“SOFTWAREINSTALL”
	ChangeDUState RPC (clause 8.2.1.4) and ChangeDUStateComplete RPC (clause 8.2.1.5)

	 “SOFTWAREUNINSTALL”
	ChangeDUState RPC (clause 8.2.1.4) and ChangeDUStateComplete RPC (clause 8.2.1.5)

	 “REBOOT”
	Reboot RPC (clause 8.2.1.6)

	 “RESET”
	Factory reset RPC (clause 8.2.1.7)


8.2.1.1
Execute File Download 

The download file transfer operation may use the Download mechanism defined in TR-069 [4]. The Download mechanism is an asynchronous command which returns a successful response or one of the following fault codes mapped onto execStatus values as detailed in Table 8.2.1.1-1. A successful response to the Update primitive triggering the Execute procedure means that the CPE has accepted the Download RPC.

Table 8.2.1.1-1: Download Fault Code Mapping

	Fault code
	Description
	execStatus Code

	9000
	Method not supported
	STATUS_REQUEST_UNSUPPORTED

	9001
	Request denied (no reason specified)
	STATUS_REQUEST_DENIED

	9002
	Internal error
	STATUS_INTERNAL_ERROR

	9003
	Invalid arguments
	STATUS_INVALID_ARGUMENTS

	9004
	Resources exceeded (when used in association with SetParameterValues, this cannot be used to indicate Parameters in error)
	STATUS_RESOURCES_EXCEEDED

	9010
	File transfer failure (associated with Download, ScheduleDownload, TransferComplete or AutonomousTransferComplete methods).
	STATUS_FILE_TRANSFER_FAILED

	9012
	File transfer server authentication failure (associated with Upload, Download, TransferComplete, AutonomousTransferComplete, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods, not associated with Scheduled Download method).
	STATUS_FILE_TRANSFER_SERVER_AUTHENTICATION_FAILURE

	9013
	Unsupported protocol for file transfer (associated with Upload, Download, ScheduleDownload, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_UNSUPPORTED_PROTOCOL


8.2.1.2
Execute File Upload Operations

The upload file transfer operation shall use the Upload mechanism defined in TR-069 [4]. The Upload mechanism is an asynchronous command that consists of the synchronous Upload RPC for the Upload and the asynchronous TransferComplete RPC. The Upload RPC returns a successful response or one of the following fault codes mapped onto execStatus values as detailed in Table 8.2.1.2-1. A successful response to the Update primitive triggering the execute procedure means that the CPE has accepted the Upload RPC in Table 8.2.1.2-1.

Table 8.2.1.2-1: Upload Fault Code Mapping
	Fault code
	Description
	execStatus Code

	9000
	Method not supported
	STATUS_REQUEST_UNSUPPORTED 

	9001
	Request denied (no reason specified)
	STATUS_REQUEST DENIED

	9002
	Internal error
	STATUS_INTERNAL_ERROR 

	9003
	Invalid arguments
	STATUS_INVALID_ARGUMENTS 

	9004
	Resources exceeded (when used in association with SetParameterValues, this cannot be used to indicate Parameters in error)
	STATUS_RESOURCES_EXCEEDED 

	9011
	Upload failure (associated with Upload, TransferComplete or Autonomous​Trans​ferComplete methods).
	STATUS_UPLOAD_FAILED 

	9012
	File transfer server authentication failure (associated with Upload, Download, TransferComplete, AutonomousTransferComplete, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_FILE_TRANSFER_SERVER_AUTHENTICATION_FAILURE

	9013
	Unsupported protocol for file transfer (associated with Upload, Download, ScheduleDownload, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_UNSUPPORTED_PROTOCOL 


8.2.1.3
Report Results using TransferComplete RPC
After a File Download or Upload has been attempted, the result of the operation is reported using the TransferComplete RPC. The TransferComplete RPC indicates a successful operation or one of the following fault codes mapped onto execStatus values in Table 8.2.1.3-2.

Table 8.2.1.3-2: TransferComplete Fault Code Mapping

	Fault code
	Description
	execStatus Code

	9001
	Request denied (no reason specified)
	STATUS_REQUEST DENIED 

	9002
	Internal error
	STATUS_INTERNAL_ERROR 

	9010
	File transfer failure (associated with Download, ScheduleDownload, TransferComplete or AutonomousTransferComplete methods).
	STATUS_FILE_TRANSFER_FAILED 

	9011
	Upload failure (associated with Upload, TransferComplete or Autonomous​Trans​ferComplete methods).
	STATUS_UPLOAD_FAILED

	9012
	File transfer server authentication failure (associated with Upload, Download, TransferComplete, AutonomousTransferComplete, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_FILE_TRANSFER_SERVER_AUTHENTICATION_FAILURE

	9014
	File transfer failure: unable to join multicast group (associated with Download, TransferComplete or AutonomousTransferComplete methods).
	STATUS_FILE_TRANSFER_FAILED_MULTICAST_GROUP_UNABLE_JOIN

	9015
	File transfer failure: unable to contact file server (associated with Download, TransferComplete, AutonomousTransferComplete, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_FILE_TRANSFER_FAILED_SERVER_CONTACT_FAILED

	9016
	File transfer failure: unable to access file (associated with Download, TransferComplete, AutonomousTransferComplete, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_FILE_TRANSFER_FAILED_FILE_ACCESS_FAILED 

	9017
	File transfer failure: unable to complete download (associated with Download, TransferComplete, AutonomousTransferComplete, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_FILE_TRANSFER_FAILED_DOWNLOAD_INCOMPLETE 

	9018
	File transfer failure: file corrupted or otherwise unusable (associated with Download, TransferComplete, AutonomousTransferComplete, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_FILE_TRANSFER_FAILED_FILE_CORRUPTED

	9019
	File transfer failure: file authentication failure (associated with Download, TransferComplete or AutonomousTransferComplete methods).
	STATUS_FILE_TRANSFER_FILE_AUTHENTICATION_FAILURE

	9020
	File transfer failure: unable to complete download within specified time windows (associated with TransferComplete method).
	STATUS_FILE_TRANSFER_WINDOW_EXCEEDED


8.2.1.4
Execute Software Operations with ChangeDUState RPC

The software installation and uninstall operations shall use the ChangeDUState mechanism defined in TR-069 [4]. The ChangeDUState mechanism is an asynchronous command that consists of the synchronous ChangeDUState RPC and returns a successful response or one of the fault codes mapped onto execStatus values as detailed in Table 8.2.1.4.-1. A successful response to the Update primitive triggering the Execute procedure means that the CPE has accepted the ChangeDUState RPC.

Table 8.2.1.4-1: ChangeDUState Fault Code Mapping

	Fault code
	Description
	execStatus Code

	9000
	Method not supported
	STATUS_REQUEST_UNSUPPORTED

	9001
	Request denied (no reason specified)
	STATUS_REQUEST DENIED 

	9002
	Internal error
	STATUS_INTERNAL_ERROR 

	9004
	Resources exceeded (when used in association with SetParameterValues, this cannot be used to indicate Parameters in error)
	STATUS_RESOURCES_EXCEEDED 


8.2.1.5
Report Results with ChangeDUStateComplete RPC

After software installation and uninstall operations using a ChangeDUState mechanism as defined in TR-069 [4], the result of the state change operation is retrieved using the ChangeDUStateComplete RPC. The ChangeDUStateComplete RPC indicates a successful operation or one of the fault codes mapped onto execStatus values as detailed in Table 8.2.1.5.-1.

Table 8.2.1.5-1: ChangeDUStateComplete Fault Code Mapping
	Fault code
	Description
	execStatus Code

	9001
	Request denied (no reason specified)
	STATUS_REQUEST_DENIED 

	9003
	Invalid arguments
	STATUS_INVALID_ARGUMENTS 

	9012
	File transfer server authentication failure (associated with Upload, Download, TransferComplete, AutonomousTransferComplete, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_FILE_TRANSFER_SERVER_AUTHENTICATION_FAILURE

	9013
	Unsupported protocol for file transfer (associated with Upload, Download, ScheduleDownload, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_UNSUPPORTED_PROTOCOL 

	9015
	File transfer failure: unable to contact file server (associated with Download, TransferComplete, AutonomousTransferComplete, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_FILE_TRANSFER_FAILED_SERVER_CONTACT_FAILED 

	9016
	File transfer failure: unable to access file (associated with Download, TransferComplete, AutonomousTransferComplete, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_FILE_TRANSFER_FAILED_FILE_ACCESS_FAILED 

	9017
	File transfer failure: unable to complete download (associated with Download, TransferComplete, AutonomousTransferComplete, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_FILE_TRANSFER_FAILED_DOWNLOAD_INCOMPLETE 

	9018
	File transfer failure: file corrupted or otherwise unusable (associated with Download, TransferComplete, AutonomousTransferComplete, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_FILE_TRANSFER_FAILED_FILE_CORRUPTED 

	9022
	Invalid UUID Format (associated with DUStateChangeComplete or AutonomousDUStateChangeComplete methods: Install, Update, and Uninstall)
	STATUS_INVALID_UUID_FORMAT

	9023
	Unknown Execution Environment (associated with DUStateChangeComplete or AutonomousDUStateChangeComplete methods: Install only)
	STATUS_UNKNOWN_EXECUTION_ENVIRONMENT

	9024
	Disabled Execution Environment (associated with DUStateChangeComplete or AutonomousDUStateChangeComplete methods: Install, Update, and Uninstall)
	STATUS_DISABLED_EXECUTION_ENVIRONMENT 

	9025
	Deployment Unit to Execution Environment Mismatch (associated with DUStateChangeComplete or AutonomousDUStateChangeComplete methods: Install and Update)
	STATUS_EXECUTION_ENVIRONMENT_MISMATCH 

	9026
	Duplicate Deployment Unit (associated with DUStateChangeComplete or AutonomousDUStateChangeComplete methods: Install only)
	STATUS_DUPLICATE_DEPLOYMENT_UNIT 

	9027
	System Resources Exceeded (associated with DUStateChangeComplete or AutonomousDUStateChangeComplete methods: Install and Update)
	 STATUS_SYSTEM_RESOURCES_EXCEEDED

	9028
	Unknown Deployment Unit (associated with DUStateChangeComplete or AutonomousDUStateChangeComplete methods: Update and Uninstall)
	STATUS_UNKNOWN_DEPLOYMENT_UNIT

	9029
	Invalid Deployment Unit State (associated with DUStateChangeComplete or AutonomousDUStateChangeComplete methods: Install, Update and Uninstall)
	STATUS_INVALID_DEPLOYMENT_UNIT_STATE

	9030
	Invalid Deployment Unit Update – Downgrade not permitted (associated with DUStateChangeComplete or AutonomousDUStateChangeComplete methods: Update only)
	STATUS_INVALID_DEPLOYMENT_UNIT_UPDATE_DOWNGRADE_DISALLOWED

	9031
	Invalid Deployment Unit Update – Version not specified (associated with DUStateChangeComplete or AutonomousDUStateChangeComplete methods: Update only)
	STATUS_INVALID_DEPLOYMENT_UNIT_UPDATE_UPGRADE_DISALLOWED

	9032
	Invalid Deployment Unit Update – Version already exists (associated with DUStateChangeComplete or AutonomousDUStateChangeComplete methods: Update only)
	STATUS_INVALID_DEPLOYMENT_UNIT_UPDATE_VERSION_EXISTS


8.2.1.6
Execute Reboot operation

The reboot operation shall use the Reboot RPC defined in TR-069 [4]. The Reboot RPC is a synchronous command. A successful response to the Update primitive triggering the Execute procedure means that the CPE has accepted the Reboot RPC. The Reboot RPC returns a successful response or one of the fault codes mapped onto execStatus values as detailed in Table 8.2.1.6-1. 

Table 8.2.1.6-1: Reboot Fault Code Mapping

	Fault code
	Description
	execStatus Code

	9001
	Request denied (no reason specified)
	STATUS_REQUEST_DENIED 

	9002
	Internal error
	STATUS_INTERNAL_ERROR 

	9003
	Invalid arguments
	STATUS_INVALID_ARGUMENTS


8.2.1.7
Execute Factory Reset operation

The factory reset operation shall use the FactoryReset RPC defined in TR-069 [4]. The FactoryReset RPC is a synchronous command. A successful response to the Update primitive triggering the Execute procedure means that the CPE has accepted the FactoryReset RPC. The FactoryReset RPC returns a successful response or one of the fault codes mapped onto execStatus values as detailed in Table 8.2.1.7-1. 

Table 8.2.1.7-1: FactoryReset Fault Code Mapping

	Fault code
	Description
	execStatus Code

	9000
	Method not supported
	STATUS_REQUEST_UNSUPPORTED 

	9001
	Request denied (no reason specified)
	STATUS_REQUEST_DENIED 

	9002
	Internal error
	STATUS_INTERNAL_ERROR

	9003
	Invalid arguments
	STATUS_INVALID_ARGUMENTS 


8.2.2
Delete <mgmtCmd> resource primitive mapping

The Delete Request primitive for the <mgmtCmd> resource may initiate TR-069 [4] RPC commands for the corresponding <execInstance> sub-resources as follows:

· If there are no <execInstance> sub-resources with RUNNING execStatus, a successful response to the Delete primitive is returned and the <mgmtCmd> resource is deleted without triggering any TR-069 [4] RPCs.

· If there are <execInstance> sub-resources with RUNNING execStatus that resulted in cancellable TR-069 [4] RPCs (e.g. File Upload and File Download RPCs), a TR-069 [4] CancelTransfer RPC shall be initiated for each cancellable operation. Upon completion of all the cancellation operations, if any fault codes are returned by the CPE, an unsuccessful Response to the Delete primitive with status code “Delete mgmtCmd- execInstance cancellation error” is returned, and the <mgmtCmd> resource is not deleted. The execStatus attribute of each specific <execInstance> is set to CANCELLED for successful RPCs or is determined from the RPC fault codes as detailed in Table 8.2.2-1. If all cancellation operations are successful on the managed entity, a successful Response to the Delete primitive is returned and the <mgmtCmd> resource is deleted.

· If there is at least one <execInstance> sub-resource with RUNNING execStatus that resulted in non-cancellable TR-069 [4] RPCs (e.g. RPCs other than File Upload and File Download RPCs), the execStatus attribute of the specific <execInstance> is changed to STATUS_NON_CANCELLABLE. An unsuccessful Response to the Delete primitive with status code “Delete mgmtCmd- execInstance cancellation error” is returned and the <mgmtCmd> resource is not deleted.

Table 8.2.2-1: CancelTransfer Fault Code Mapping for Delete <mgmtCmd>

	Fault code
	Description
	Response Status Code

	9000
	Method not supported
	STATUS_REQUEST_UNSUPPORTED 

	9001
	Request denied (no reason specified)
	STATUS_REQUEST DENIED 

	9021
	Cancelation of file transfer not permitted in current transfer state
	STATUS_CANCELLATION_DENIED 


8.2.3
Update (Cancel) <execInstance> primitive mapping

When the Update Request primitive for an <execInstance> sub-resource addresses the execDisable attribute of the <execInstance > sub-resource, it effectively triggers a Cancel <execInstance> resource procedure. 

The hosting CSE determines whether the <execInstance> resource has a RUNNING execStatus and weather the resulting TR-069 [4] RPCs are cancellable. Currently, only the TR-069 File Upload and File Download RPCs are cancellable using the TR-069 [4] CancelTransfer RPC.

· If the addressed <execInstance> sub-resource has an execStatus other than RUNNING, an un-successful Response to the Update primitive is returned with status code “Cancel execInstance – already complete”.

· If the addressed <execInstance> sub-resources has RUNNING execStatus and resulted in cancellable TR-069 [4] RPCs (e.g. File Upload and File Download RPCs), a BBF TR-069 [4] CancelTransfer RPC shall be initiated. For a successful CancelTransfer RPC the execStatus attribute of the specific <execInstance> is set to CANCELLED and a successful Response is sent to the Update primitive. For an unsuccessful CancelTransfer RPC the execStatus attribute is determined from the RPC fault codes as detailed in Table 8.2.3-1 and an unsuccessful Response is sent to the Update primitive with status code “Cancel execInstance – cancellation error”.

· If the addressed <execInstance> sub-resources has RUNNING execStatus and resulted non-cancellable TR-069 [4] RPCs (e.g. RPCs other than File Upload and File Download RPCs), the execStatus attribute of the specific <execInstance> is changed to STATUS_NON_CANCELLABLE. An unsuccessful Response is sent to the Update primitive with status code “Cancel execInstance – not cancellable”.

Table 8.2.3-1: CancelTransfer Fault Code Mapping for Update (Cancel) <execInstance>

	Fault code
	Description
	execStatus Code

	9000
	Method not supported
	STATUS_REQUEST_UNSUPPORTED 

	9001
	Request denied (no reason specified)
	STATUS_REQUEST DENIED 

	9021
	Cancelation of file transfer not permitted in current transfer state
	STATUS_REQUEST_UNSUPPORTED 


8.2.4
Delete <execInstance> primitive mapping

The Delete Request primitive for an <execInstance> sub-resource may initiate TR-069 [4] RPC commands for the corresponding <execInstance> sub-resources as follows: 

· If the addressed <execInstance> sub-resource has an execStatus other than RUNNING, an successful Response to the Delete primitive is returned and the <execInstance> sub-resource is deleted without triggering any TR-069 [4] RPCs. 

· If the addressed <execInstance> sub-resource has RUNNING execStatus and resulted in cancellable TR-069 [4] RPCs (e.g. File Upload and File Download RPCs), a BBF TR-069 [4] CancelTransfer RPC shall be initiated. For a successful CancelTransfer RPC a successful response is sent to the Delete primitive and the <execInstance> sub-resource is deleted. For an unsuccessful CancelTransfer RPC the execStatus attribute is determined from the RPC fault codes as detailed in Table 8.2.4-1 and an unsuccessful Response is sent to the Delete primitive with status code “Delete execInstance – cancellation failed”.

· If the addressed <execInstance> sub-resource has RUNNING execStatus and resulted non-cancellable TR-069 [4] RPCs (e.g. RPCs other than File Upload and File Download RPCs), the execStatus attribute is set to STATUS_NOT_CANCELLABLE and an unsuccessful Response is sent to the Update primitive with status code “Delete execInstance – not cancellable”
Table 8.2.4-1: CancelTransfer Fault Code Mapping for Delete <execInstance>

	Fault code
	Description
	execStatus Code

	9000
	Method not supported
	STATUS_REQUEST_UNSUPPORTED 

	9001
	Request denied (no reason specified)
	STATUS_REQUEST DENIED 

	9021
	Cancelation of file transfer not permitted in current transfer state
	STATUS_CANCELLATION_DENIED 


8.3
Resource [myCertFileCred] primitive mappings

8.3.1
Introduction

This clause contains information regarding the procedures for establishing a certificates presented by the managed entity in order for the peer to authenticate the managed entity.
8.3.2
Creation of Resource [myCertFileCred]

8.3.2.1
Introduction
The creation of a [myCertFileCred] resource requires the use of the TR-069 Download RPC to establish the credential on the managed entity. Once the managed entity has obtained the credential, the Device.Security.Certificate.{i} instance’s SUIDs parameter is set from the [myCertFileCred] attribute using the TR-069 Set RPC.
8.3.2.2
Procedure for creation of Resource [myCertFileCred]

The Create Request and Response primitives for Resource [myCertFileCred] that results in a download file transfer shall use the Download mechanism defined in TR-069 [4]. The Download mechanism is an asynchronous command that consists of the synchronous Download RPC for the Download and the asynchronous TransferComplete RPC. The Download RPC returns a successful response or one of the following fault codes in Table 8.3.2.2-1. A successful response means that the CPE has accepted the Download RPC.

Table 8.3.2.2-1: Download Fault Code Mapping

	Fault code
	Description
	Response Status Code

	9000
	Method not supported
	STATUS_BAD_REQUEST

	9001
	Request denied (no reason specified)
	STATUS_BAD_REQUEST

	9002
	Internal error
	STATUS_BAD_REQUEST

	9003
	Invalid arguments
	STATUS_BAD_REQUEST

	9004
	Resources exceeded (when used in association with SetParameterValues, this cannot be used to indicate Parameters in error)
	STATUS_BAD_REQUEST

	9010
	File transfer failure (associated with Download, ScheduleDownload, TransferComplete or AutonomousTransferComplete methods).
	STATUS_BAD_REQUEST

	9012
	File transfer server authentication failure (associated with Upload, Download, TransferComplete, AutonomousTransferComplete, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_BAD_REQUEST

	9013
	Unsupported protocol for file transfer (associated with Upload, Download, ScheduleDownload, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_BAD_REQUEST


Once the CPE has attempted to download the file, the CPE reports the result of the download operation using the TransferComplete RPC. The TransferComplete RPC indicates a successful operation or one of the following fault codes in Table 8.3.2.2-2.

Table 8.3.2.2-2: TransferComplete Fault Code Mapping
	Fault code
	Description
	Response Status Code

	9001
	Request denied (no reason specified)
	STATUS_BAD_REQUEST

	9002
	Internal error
	STATUS_BAD_REQUEST

	9010
	File transfer failure (associated with Download, ScheduleDownload, TransferComplete or AutonomousTransferComplete methods).
	STATUS_BAD_REQUEST

	9011
	Upload failure (associated with Upload, TransferComplete or Autonomous​Trans​ferComplete methods).
	STATUS_BAD_REQUEST

	9012
	File transfer server authentication failure (associated with Upload, Download, TransferComplete, AutonomousTransferComplete, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_BAD_REQUEST

	9014
	File transfer failure: unable to join multicast group (associated with Download, TransferComplete or AutonomousTransferComplete methods).
	STATUS_BAD_REQUEST

	9015
	File transfer failure: unable to contact file server (associated with Download, TransferComplete, AutonomousTransferComplete, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_BAD_REQUEST

	9016
	File transfer failure: unable to access file (associated with Download, TransferComplete, AutonomousTransferComplete, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_BAD_REQUEST

	9017
	File transfer failure: unable to complete download (associated with Download, TransferComplete, AutonomousTransferComplete, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_BAD_REQUEST

	9018
	File transfer failure: file corrupted or otherwise unusable (associated with Download, TransferComplete, AutonomousTransferComplete, DUStateChangeComplete, or AutonomousDUStateChangeComplete methods).
	STATUS_BAD_REQUEST

	9019
	File transfer failure: file authentication failure (associated with Download, TransferComplete or AutonomousTransferComplete methods).
	STATUS_BAD_REQUEST

	9020
	File transfer failure: unable to complete download within specified time windows (associated with TransferComplete method).
	STATUS_BAD_REQUEST


Upon successful TransferComplete notification from the CPE, the newly created Device.Security.Certificate.{i} object instance shall be assigned the values of the SUIDs attribute using the procedure for updating parameters in clause 8.1.4.
9
Server Interactions
9.0
Introduction
This clause specifies how the IN-CSE interacts with an ACS in order to manage the Resources described in the present document. The IN-CSE interaction with an ACS includes:

· Establishment of the communication session between the IN-CSE and ACS

· Processing of requests and notifications between the IN-CSE and the ACS

· Discovery

NOTE:
The Broadband Forum has not defined a protocol specification for the Northbound Interface of an ACS. As such, the present document only describes the expectations of this interface in the form of requirements on the ACS.

9.1
Communication Session Establishment

9.1.1
IN-CSE to ACS Communication Session Establishment

When the IN-CSE detects that it has to delegate an interaction with a device resource to an ACS, the IN-CSE establishes a communication session with the ACS. The establishment of a communication session between the IN‑CSE and ACS provides security dimensions for Access control, Authentication, Non-repudiation, Data confidentiality, Communication security, Data integrity and Privacy adhering to the following TR-131 [7] Architectural requirement A7.

The IN-CSE may establish multiple sessions with an ACS based on the security model utilized between the IN-CSE and the ACS.

9.1.2
ACS to IN-CSE Communication Session Establishment

When the ACS detects a change to resources it manages that the IN-CSE has expressed interest, the ACS requests the IN-CSE to establish a session if a session does not exist for the resource being managed. The establishment of a communication session between the IN-CSE and ACS provides security dimensions for Access control, Authentication, Non-repudiation, Data confidentiality, Communication security, Data integrity and Privacy adhering to the following TR-131 [7] Architectural requirement A7.
The ACS may establish multiple sessions with an IN-CSE based on the security model utilized between the IN-CSE and the ACS.

While a session between the ACS and IN-CSE is not established, the ACS retains any notifications or changes in the resources based on an Event retention policy (i.e. time, number of events).

When an ACS to IN-CSE interaction is required and a session does not exist, the ACS requests to initiate a session based on a Session Initiation Policy (i.e. Periodic contact establishment (schedule), upon event detection with timeframe window).

9.2.3
ACS and IN-CSE Communication Session Requirements

When establishing a session from the ACS to the IN-CSE:

· If a session does not exist between the IN-CSE and ACS, the ACS shall retain any notifications or changes in the resources based on an Event retention policy (i.e. time, number of events).

· When an ACS to IN-CSE interaction is required and a session does not exist, the ACS shall be capable to initiate a session based on a Session Initiation Policy (i.e. Periodic contact establishment (schedule), upon event detection with timeframe window) 

9.2
Processing of Requests and Responses

9.2.1
Request and Notification Formatting

Requests and Notifications mechanisms between the IN-CSE and the DM Server format the XML schema of the CPE methods defined in TR-069 [4] as an ACS would format the CPE methods that it would pass to the CPE. The IN-CSE would then also process the CPE methods as defined in TR-069 [4]. Likewise the ACS would send notifications in the format of the XML schema of the CPE for sending events using the Inform RPC.

9.2.2
ACS Request Processing Requirements

When receiving requests from the IN-CSE the ACS shall be capable of defining mechanisms to support triggering of immediate operations to device. If the device is not available the ACS returns an appropriate error code.

The ACS shall provide capability for the IN-CSE to indicate request policies to include: Retry policy, Request Time out.

9.2.3
ACS Notification Processing Requirements

When sending notifications to the IN-CSE:

· The ACS shall be capable of providing a mechanism for the IN-CSE to subscribe to events.

· The ACS shall be capable of providing a list of events for which the IN-CSE can subscribe.

· The ACS shall be capable of providing a mechanism for the IN-CSE to unsubscribe from events.

· The ACS shall be capable of providing an event delivery mechanism.

· The ACS shall be capable of providing the capability for the IN-CSE to request event filters including: Event Code; Specific parameters changing value; Device; Any combination of the previous criteria. 

· The IN-CSE shall be capable of subscribing to be notified of changes to resources it manages.

· The ACS shall be capable of notifying the IN-CSE of changes to resources to which the client has subscribed.

9.3
Discovery and Synchronization of Resources

For devices under management, the IN-CSE may discover resources of interest (metadata and values) within a device using the ACS.

For resources of interest, the IN-CSE may also express an interest to be notified of a resource if a resource is changed (added, deleted, updated).

The IN-CSE shall be capable to discover and subscribe to changes of resources in order to synchronize the IN-CSE with resources of interest of the ACS.

9.4
Access Management

9.4.0
Introduction

Once a request has performed an Access Decision by the IN-CSE to allow the request, the IN-CSE shall select the appropriate ACS along with elements the ACS would need to implement access management within the ACS. These would include the Identity of the subject (oneM2M Originator) of the request which is needed in scenarios where the original issuer of the request is needed to be known – this could be done by correlating principals (e.g. Roles, Accounts) used by the IN-CSE and ACS.

9.4.1
Access Management Requirements

· The ACS shall be capable of providing a mechanism for the IN-CSE to discover the Access Management elements used to authorize and authenticate access to resources controlled by the ACS.

· The IN-CSE shall be capable of correlating Access Management elements provided by the ACS to Access Management elements used by the IN-CSE.

· The IN-CSE shall be capable of providing secured storage of Access Management elements within the IN‑CSE.

10
New Management Technology Specific Resources
TR-181 [6] provides a list of management objects that have been standardized by the Broadband Forum and where possible, clause 7 provides a mapping of the Resources to standardized management objects. This clause provides the oneM2M vendor specific extensions to the TR-181 [6] data model as specified in the ts-0006-1-2-0.xml.

History
	Publication history

	V1.0.1
	30 Jan 2015
	Release 1 – Publication

	V1.1.4
	29 February 2016
	Updated Release 1 – Publication

	V2.0.1
	30 August 2016 
	Release 2 – Publication

	
	
	

	
	
	


	Draft history (to be removed on publication)

	V3.0.0
	27-Oct-2016
	Updated with agreed contribution from MAS 25

MAS-2016-0211R01-TS-0006-Updates_for_TS-0022_v_0_6_0

	V3.1.0
	07-Nov-2016
	Updated with agreed contributions from MAS 25.1

MAS-2016-0250-TS-0006-Updates_for_TS-0022_v_0_7_0

	V.3.2.0
	20-Feb-2017
	Updated with  agreed contributions from MAS 27

MAS-2017-0002-TS-0006-Data_collection_schedules

	
	
	



© oneM2M Partners Type 1 (ARIB, ATIS, CCSA, ETSI, TIA, TSDSO, TTA, TTC)
Page 5 of 10
This is a draft oneM2M document and should not be relied upon; the final version, if any, will be made available by oneM2M Partners Type 1.

