	Doc# MAS-2018-0094R01-Smart_City_model_-_street_light.doc
Change Request
	[image: image1.png]






	


	CHANGE REQUEST

	Meeting ID:*
	MAS #36

	Source:*
	Hongna Chang, HUAWEI, changhongna@huawei.com
Yongjing Zhang, HUAWEI,  zhangyongjing@huawei.com

	Date:*
	2018-7-3

	Reason for Change/s:*
	Adding modules to support managing street light controllers for smart city.

	CR  against:  Release*
	Release 4

	CR  against:  WI*
	 FORMCHECKBOX 
 Active WI-0084
 FORMCHECKBOX 
 MNT maintenance / < Work Item number(optional)>
Is this a mirror CR? Yes  FORMCHECKBOX 
 No  FORMCHECKBOX 

mirror CR number: (Note to Rapporteur - use latest agreed revision)
 FORMCHECKBOX 
 STE Small Technical Enhancements / < Work Item number (optional)>
Only ONE of the above shall be ticked

	CR  against:  TS/TR*
	TS-0023   V4.0.0

	Clauses *
	5.3, 5.5, 5.6

	Type of change: *
	 FORMCHECKBOX 
 Editorial change
 FORMCHECKBOX 
 Bug Fix or Correction
 Change to existing feature or functionality
 FORMCHECKBOX 
 New feature or functionality
Only ONE of the above shall be ticked

	Impacted other TS/TR(s)
	NA

	Post Freeze checking:*
	This CR contains only essential changes and corrections?  YES  FORMCHECKBOX 
  NO  FORMCHECKBOX 

This CR may break backwards compatibility with the last approved version of the TS?       YES 
  NO 


	Template Version: January 2017 (Do not modify)


oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.
In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR
Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).
Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.
Follow the drafting rules.
All pictures must be editable.
Check spelling and grammar to the extent practicable.
Use Change bars for modifications.
The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.
Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.
When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text. 
Introduction
This contribution proposes to add some module classes and  a device model of street light controllers for smart city.

-----------------------Start of change 1-------------------------------------------

5.5.2.1
deviceStreetLightController
A street light controller is used to control the opening and closing of the street light.
Table 5.5.xx‑1: Modules of deviceStreetLightController Device model
	Module Instance Name
	Module Class Name
	Optional
	Description

	geoLocation
	geoLocation
	true
	See clause 5.3.1.37.

	connectivity
	connectivity
	true
	See clause 5.3.1.xx.

	periodicalReportConfig
	periodicalReportConfig
	true
	See clause 5.3.1.xx.

	slcReportInfo
	slcReportInfo
	false
	See clause 5.3.2.xx.

	slcParaSetting
	slcParaSetting
	false
	See clause 5.3.2.xx.

	slcAlarm
	slcAlarm
	false
	See clause 5.3.2.xx.


-----------------------End of change 1---------------------------------------------

-----------------------Start of change 2-------------------------------------------

5.3.2.xx slcReportInfo
This ModuleClass provides information of status of the street light controller.
Table 5.3.2.xx‑1: DataPoints of slcReportInfo ModuleClass
	Name
	Type
	R/W
	Optional
	Unit
	Documentation

	voltageFrequency
	xs:float
	R
	true
	Hz
	The frequency of voltage.

	switchStatus
	xs:boolean
	RW
	false
	
	This value indicates the status of light switch. ”False”: off, “True”: on.

	onlineStatus
	xs:boolean
	R
	true
	
	This value indicates the communication status of light. ”False”: offline, “True”: online.

	lightVoltage
	xs:float
	R
	false
	Volt
	The voltage of the street light controller.

	lightCurrent
	xs:float
	R
	false
	Ampere
	The current of the street light controller.

	lightDimmingValue
	xs:integer
	RW
	false
	%
	The dimming value of the street light.

	lightPowFactor
	xs:float
	R
	false
	pF
	The light power factor is the ratio of active power to apparent power. Power factor is a factor that measures the efficiency of electrical equipment. The value range is [0,1].

	lightActivePower
	xs:float
	R
	true
	kW
	Active power is the electrical power required to keep the electrical equipment running normally, that is, the electrical power that converts electrical energy into other forms of energy, such as mechanical, optical, thermal, and so on.

	lightReactivePower
	xs:float
	R
	true
	kvar
	The reactive power is the electrical power required to establish an alternating magnetic field and induced magnetic flux.

	lightApparentPower
	xs:float
	R
	true
	kVA
	This value indicates the apparent power that is mainly used to calculate the energy consumption of the street light.

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	


5.3.2.xx slcParaSetting
This ModuleClass provides capabilities to set service parameters. 
Table 5.3.2.xx‑1: DataPoints of slcParaSetting ModuleClass
	Name
	Type
	R/W
	Optional
	Unit
	Documentation

	lightCount
	xs:integer
	RW
	true
	
	Number of lampholders controlled by the street light controller.

	timePlanStatus
	xs:boolean
	RW
	true
	
	“False” indicates the time plan is not used. “Ture” indicates the time plan is being used.

	timeRangeCount
	xs:integer
	RW
	true
	
	The number of time ranges for the time plan.

	timeRange
	xs:string

	RW
	true
	
	A array of sequencial time points which define the time plan. Each time point is the start time of the next time range as well as the end of previous time range in the time plan.

	timeRangeLightDimmingValue
	xs:string
	RW
	true
	
	A array containing the dimming values in different time ranges. In the case that lightCount is larger than 1, it may be a 2-dimenstional array describing the dimming value of each lampholder in each time range.


5.3.2.xx slcAlarm
This ModuleClass provides capabilities to provide alarm information of street light controller. 
Table 5.3.2.xx‑1: DataPoints of slcAlarm ModuleClass
	Name
	Type
	R/W
	Optional
	Unit
	Documentation

	overCurrentThreshold
	xs:float
	RW
	true
	Ampere
	The threshold of over current.

	overCurrentDuration
	xs:integer
	RW
	true
	minute
	The duration of over current to trigger the alarm.

	underCurrentThreshold
	xs:float
	RW
	true
	Ampere
	The threshold of under current.

	underCurrentDuration
	xs:integer
	RW
	true
	minute
	The duration of under current to trigger the alarm.

	overVoltageThreshold
	xs:float
	RW
	true
	Volt
	The threshold of over voltage.

	overVoltageDuration
	xs:integer
	RW
	true
	minute
	The duration of over voltage to trigger the alarm.

	underVoltageThreshold
	xs:float
	RW
	true
	Volt
	The threshold of under voltage.

	underVoltageDuration
	xs:integer
	RW
	true
	minute
	The duration of under voltage to trigger the alarm.

	overCurrentAlarm
	xs:boolean
	R
	false
	
	The alarm of over current.

	underCurrentAlarm
	xs:boolean
	R
	false
	
	The alarm of under current.

	overVoltageAlarm
	xs:boolean
	R
	false
	
	The alarm of over voltage.

	underVoltageAlarm
	xs:boolean
	R
	false
	
	The alarm of under voltage.

	overVoltagePercent
	xs:integer
	RW
	true
	%
	The threshold of over current in terms of percentage. The value range is [0,100]. This is the alternative trigger of the overVoltageAlarm and should be mutually exclusive to the overVoltageThreshold. 

	underVoltagePercent
	xs:integer
	RW
	true
	%
	The duration of under current in terms of percentage. The value range is [0,100]. This is the alternative trigger of the underVoltageAlarm and should be mutually exclusive to the underVoltageThreshold.

	referenceVoltage
	xs:float
	RW
	true
	Volt
	The reference voltage used as the basis of the overVoltagePercent and underVoltagePercent.


-----------------------End of change 2---------------------------------------------
CHECK LIST

· Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?
· Does this CR contain changes related to only one particular issue/problem?
· Have any mirror CRs been posted?
· Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?
· Are all pictures editable?
· Have you checked the spelling and grammar?
· Have you used change bars for all modifications?
· Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)
· Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?
�sdt:array?





© 2018 oneM2M Partners
                                                                                                   Page 5 (of 6)



[image: image1.png]