	MAS-2018-0099-Adding_a_new_element_to_TS-0023
	[image: image1.png]






	


	CHANGE REQUEST

	Meeting ID:*
	MAS#36

	Source:*
	Yawen Niu, CMCC, niuyawen@chinamobile.com
Yongjing Zhang,Huawei, zhangyongjing@huawei.com

	Date:*
	2018-06-12

	Reason for Change/s:*
	Adding a few new elements for dataPoints of bodyCompositionAnalyser ModuleClass

	CR  against:  Release*
	Rel.3 

	CR  against:  WI*
	 FORMCHECKBOX 
 Active WI-0017
 FORMCHECKBOX 
 MNT maintenance / < Work Item number(optional)>
Is this a mirror CR? Yes  FORMCHECKBOX 
 No  FORMCHECKBOX 

mirror CR number: (Note to Rapporteur - use latest agreed revision)
 FORMCHECKBOX 
 STE Small Technical Enhancements / < Work Item number (optional)>
Only ONE of the above shall be ticked

	CR  against:  TS/TR*
	TS-0023 V3.7.0

	Clauses *
	New clauses in 5.3.14

	Type of change: *
	 FORMCHECKBOX 
 Editorial change

 FORMCHECKBOX 
 Bug Fix or Correction

 Change to existing feature or functionality

 FORMCHECKBOX 
 New feature or functionality
Only ONE of the above shall be ticked

	Impacted other TS/TR(s)
	

	Post Freeze checking:*
	This CR contains only essential changes and corrections?  YES  FORMCHECKBOX 
  NO  FORMCHECKBOX 

This CR may break backwards compatibility with the last approved version of the TS?       YES  FORMCHECKBOX 
  NO  FORMCHECKBOX 



	Template Version: January 2017 (Do not modify)


oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

Introduction
This CR introduces a few new elements for dataPoints of bodyCompositionAnalyser ModuleClass
-----------------------Start of change 1-------------------------------------------

5.3.14
bodyCompositionAnalyser
This ModuleClass provides the capability to report the measurement of body composition analyser characteristics.

Table 5.3.14‑1: DataPoints of body composition analyser ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	bodyLength
	xs:float
	true
	false
	true
	The measurement of body length by Weight scale and Body composition analyser. The unit of measure is centimeter(cm).

	Bmi
	xs:float
	true
	false
	true
	The measurement of Body Mass Index (BMI) by a weight scale and a body composition analyser. The unit of measure is kg/m2.

	fatFreeMass
	xs:float
	true
	false
	true
	The measurement of fat free mass by a weight scale and a body composition analyser. The unit of measure is kilogram (kg).

	softLeanMass
	xs:float
	true
	false
	true
	The measurement of soft lean mass by a weight scale and a body composition analyser. The unit of measure is kilogram (kg).

	muscleMass
	xs:float
	true
	false
	true
	The measurement of muscle mass by a weight scale and a body composition analyser. The unit of measure is kilogram (kg).

	basalMetabolism
	xs:float
	true
	false
	true
	The measurement of basal metabolism by a weight scale and a body composition analyser. The unit of measure is kilocalorie (kcal).

	impedance
	xs:float
	true
	false
	true
	The measurement of impedance by a weight scale and a body composition analyser. The unit of measure is ohm.

	proteinMass
	xs:float
	true
	false
	true
	The measurement of protein mass by a weight scale and a body composition analyser. The unit of measure is kilogram (kg).

	bodyMoistureMass
	xs:float
	true
	false
	true
	The measurement of body moisture mass by a weight scale and a body composition analyser. The unit of measure is kilogram (kg).

	inorganicSaltMass
	xs:float
	true
	false
	true
	The measurement of inorganic salt mass by a weight scale and a body composition analyser. The unit of measure is kilogram (kg).

	somatotype
	xs:String
	true
	false
	true
	The measurement of somatotype by Weight scale and Body composition analyser. 

	
	
	
	
	
	


-----------------------End of change 1---------------------------------------------

© 2018 oneM2M Partners
                                                                                                   Page 1 (of 2)



[image: image1.png]