	Doc MAS-2018-0062-Adaptation_of_TS-0023_to_Release_4_assumptions
Change Request
	[image: image2.png]






	


	CHANGE REQUEST

	Meeting ID:*
	MAS #35

	Source:*
	Goluch Maciej, ORANGE, maciej.goluch@orange.com

Strzemecki Paweł, ORANGE, pawel.strzemecki2@orange.com
Saïd Gharout, ORANGE, said.gharout@orange.com

	Date:*
	2018-05-22

	Reason for Change/s:*
	See introduction below

	CR  against:  Release*
	Release 4

	CR  against:  WI*
	 FORMCHECKBOX 
 Active WI-0084  

 FORMCHECKBOX 
 MNT maintenance / < Work Item number(optional)>
Is this a mirror CR? Yes  FORMCHECKBOX 
 No  FORMCHECKBOX 

mirror CR number: (Note to Rapporteur - use latest agreed revision)
 FORMCHECKBOX 
 STE Small Technical Enhancements / < Work Item number (optional)>
Only ONE of the above shall be ticked

	CR  against:  TS/TR*
	TS-00023 v.3.7.1

	Clauses *
	5

	Type of change: *
	 FORMCHECKBOX 
 Editorial change
 FORMCHECKBOX 
 Bug Fix or Correction
 FORMCHECKBOX 
 Change to existing feature or functionality
 FORMCHECKBOX 
 New feature or functionality
Only ONE of the above shall be ticked

	Impacted other TS/TR(s)
	<TS/TR number>, <Version Number>, and <Description on which aspect should be reflected in this TS/TR>

	Post Freeze checking:*
	This CR contains only essential changes and corrections?  YES  FORMCHECKBOX 
  NO  FORMCHECKBOX 

This CR may break backwards compatibility with the last approved version of the TS?       YES 
  NO 


	Template Version: January 2017 (Do not modify)


oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.
In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR
Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).
Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.
Follow the drafting rules.
All pictures must be editable.
Check spelling and grammar to the extent practicable.
Use Change bars for modifications.
The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.
Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.
When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text. 
Introduction
This CR introduces division of Module Classes and Devices into 3 domain: common domain, home domain, industry domain. 
-----------------------Start of change 1-------------------------------------------
5 Harmonised Information Model

5.1 Introduction

The present document intends to provide the unified means in the oneM2M system by defining a home appliance information model for the home domain devices such as TV, refrigerator, air conditioner, clothes washer, oven, and robot cleaner. For the reasons of interworking with external technologies and efficiency, the principle of the home appliance information model is designed based on HGI SDT 3.0 [1].

The principle of defining the home appliance information model is introduced in clause 5.2. ModuleClasses which oneM2M systems support are explained in clause 5.3. In the subsequent clause 5.5, Device models are defined.
Editor’s note: this clause has to be updated (remove specific references to Home).
5.2 Design Principle of the Harmonised Information Model
 Editor’s note: this clause has to be updated (removed specific references to Home).
5.2.1 Basic design principle of information modelling
The design principle of the oneM2M abstract information model of home appliance, is to use SDT 3.0 as introduced in oneM2M TR‑0017 [i.2]. Note that those terms starting with a capital letter in this clause are SDT terms and are explained in [1].

Domain is a unique name which acts like a namespace (e.g., "org.oneM2M.home.modules"). It is set by the organization creating the SDT, allowing reference to a package of definitions for the contained ModuleClasses and Device models.

ModuleClasses specifies a single service (e.g., audioVolume, powerOn/Off) with one or more Actions, Properties, DataPoints and Events. Each service which is described as a ModuleClass can be re-used in many Devices.

Device model is a physical, addressable, identifiable appliance, sensor and actuator with one or more Modules, Properties and SubDevices.

SubDevice is a device which may be embedded in a Device and/or is addressed via another Device.

Module is an instantiation of a ModuleClass for a specific Device or SubDevice.

Figure 5.2.1‑1 depicts the basic structure of SDT 3.0.
Specifications of new Device models and ModuleClasses are encouraged to re-use the definitions specified in this document as much as possible. If re-use is not possible and new Device and/or ModuleClases definitions are necessary, it is strongly advised to closely follow the guidelines and definition style from this document.

[image: image1.png]Domain

* Modules : ModuleClass
* Devices : Device

ModuleClass
@id: 1D @ name : text
= Doc : Doc @ optional : boolean = false
* imports

Device

@id : Name

- Doc: Doc

* Property : Properties

* Modules : Module

* SubDevices : SubDevice

SubDevice
@id : Name
- Doc : Doc

* Property : Properties
* Modules : Module

- Doc : Doc

- extends
@domain : IDRF
@class : text

* Property : Properties

* Actions : Action

* Data : DataPoint

* Events : Event

=2 Action

=2 DataPoint

> Module

>f Property

@ name : text

@ optional : boolean = false
@ value : text

- Doc : Doc

- DataType : SimpleType

> Event
@ name : text
@ optional : boolean = false

@ name: text

@ optional : boolean = false
- Doc: Doc

- DataType : DataType

* Args : Arg

@ name : text

@ optional  boolean = false
@writable : boolean
@ readable : boolean
@ eventable : boolean
- Doc : Doc

- DataType : DataType

- Doc : Doc
* Data : DataPoint

(B9

>f Arg

@ name ; text

- Doc : Doc

- DataType : DataType

Doc





Figure 5.2.1‑1: Design Structure of the Home Appliance Information Model using SDT 3.0
5.2.2 Description rules for Module Classes and Device models 

When the Home Appliances Information Model is described based on SDT, the following rules shall be applied:

· Rule 1: CamelCase rule:

· When naming each element, lowerCamelCase shall be used as the Java coding rules [2].

· Rule 2: Rule for description of Action, DataPoint:

· DataPoint shall be used to represent stateless operations. (e.g. powerState of binarySwitch for on/off operations).

· Action shall be used when describing stateful condition, handling unknown internal state conditions (e.g. upVolume/downVolume by increasing/decreasing the audioVolume in steps, handling transactional procedures, or checking integrity using username plus password at the same time).

· Rule 3: Rule for description of DataPoint and Property:

· Non-functional information shall be described as a Property. Functional information shall be described as a DataPoint. (E.g. non-functional information: version, id; functional information: targetTemperature, targetVolume).

· Rule 4: Definition of the Domain:

· The Domain, in the case of the Home Appliance Information Model, is specified as "org.onem2m.home".

· The sub-domain for Device and ModuleClass shall be specified as "org.onem2m.home.devices" and "org.onem2m.home.moduleclasses" respectively.
· Rule 5: Naming rule for the element:

· The name of each element should be concise and avoid repeating its parent element name; but

· It may include the name of its parent element for readability. (e.g., lightDimmerUp, lightDimmerDown under lightDimmer).

· Rule 6: Criteria for marking elements as optional or mandatory:

· An element shall only be defined as mandatory if it's foreseen to be universally mandatory to all implementing technologies.

· Rule 7: Enumeration type:

· When describing the meaning of values for enumeration type elements, they may be described in another clause.
· The enumeration types for Home Appliance Information Model are based on <xs:integer>, and the numeric values are interpreted as specified in clause 5.6. 
· The name of an enumeration type shall start with the prefix “enum”. This prefix shall not be used with non-enumaration type names.
· Rule 8: Rule for unit  in documentation :

· SI (International Systems of Units in [z]) measurement (e.g. meter, kilogram, second.) should be considered as first candidate.
· Otherwise, it may be kept consistency with implementing technologies such as other SDO’s specification..
Editor's note: Popular unit in particular industrial domain should be considered  (e.g. cm for human height, calories for energy consumption in healthcare domain). It should be made coherent in the document, as possible.
· Rule 9: Rule for type :

· Measured and/or calculated values should be represented in float (without taking care of resolution of values).

Editor’s note: It should be made coherent in the document, as possible. Unit shall not be fixed as a rule but be decided with correspondence to each device or module.
· Rule 10: Inheritance of ModuleClasses :
· A ModuleClass may inherit from another existing ModuleClass in order to provide additional functionalities based on the existing ModuleClass. However, inheritance from multiple ModuleClasses is not allowed (due to the “diamond problem” [i.6]).

· Inheritance of ModuleClass shall only be used in the case that extending an existing ModuleClass is not appropriate, i.e. the functionality to be added is irrelevant to the original design purpose of the existing ModuleClass (e.g. adding a ‘time’ DataPoint to a ‘binarySwitch’ ModuleClass). 
· Rule 11: When to differentiate between current and target Data Points in ModuleClasses:
· Device operations, which are executed when setting data points to specific values, may take some time to reach the desired result. For example, setting a new temperature to a heater does not immediately change the room temperature, but it may take some time for the heater to increase the temperature. Therefore, it is sometimes necessary to distinguish between current and target data points.
· A ModuleClass must provide an additional “target” data point when the “current” data point …
· is writable, and

· the functionality that is mapped to the data point is an operation, not a configuration function, and

· the operation may take some time to start and/or to complete, or reach the desired result.  

· When a ModuleClass provides current and target data points then the name for the current data point must have the prefix “current”, and the name for the target data point must have the prefix “target”. Both data points must have the same suffix, for example “currentTemperature” and “targetTemperature”.

5.3 ModuleClasses
5.3.1 Common domain
5.3.1.1 3Dprinter
This ModuleClass provides capabilities for a 3D printer. 

Table 5.3.1‑1: Actions of 3Dprinter
	Return Type
	Name
	Argument
	Optional
	Documentation

	none
	start3Dprint
	none
	true
	Start 3D printing.

	none
	stop3Dprint
	none
	true
	Stop 3D printing.


Table 5.3.1‑2: DataPoints of 3Dprinter ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	printType
	hd:enum3DprinterTechnology
	true
	false
	false
	The type of printing technology (see clause 5.6.1).

	printSizeX
	xs:float
	true
	false
	false
	This data pointrepresents the maximum size of a printing object in the direction of X-axis.

The unit of measure is “mm”. (1/10cm).

	printSizeY
	xs:float
	true
	false
	false
	This data poin represents the maximum size of printing object in the direction of Y-axis.

The unit of measure is “mm”. (1/10cm).

	printSizeZ
	xs:float
	true
	false
	false
	This data point represents the maximum size of printing object in the direction of Z-axis.

The unit of measure is “mm”. (1/10cm).

	network
	xs:boolean
	true
	false
	false
	This value indicates the Wide Area Network (WAN) connectivity of the 3D printer, such as Internet or GSM.

”False” indicates that the printer does not have network connectivity to a WAN. “True” indicates that the printer has WAN network connectivity.

	memorySize
	xs:float
	true
	false
	false
	This value represents the total memory size of the printer. The unit of measure is MB(Mega Bytes).


5.3.1.2 acousticSensor

This ModuleClass provides capabilities for an acoustic sensor. 
Table 5.3.2‑1: DataPoints of acousticSensor ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	loudness
	xs:float
	true
	false
	false
	The unit of measure of the loudness is “dB2”.

	acousticStatus
	xs:integer
	true
	false
	true
	The acousticStatus is expressed in percent, whereas a value of 0 means “no sound” and a value of 100 means “most noisy”.


5.3.1.3 airConJobMode
This ModuleClasses provides capabilities to control and monitor the job modes of an air conditioner.
Table 5.3.3‑1: DataPoints of airConJobMode ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	currentJobMode
	hd:enumAirConJobMode
	true
	true
	false
	Currently active job mode (see clause 5.6.3). 

	currentJobModeName
	xs:string
	true
	false
	true
	Name of the current job mode as a string. This can be used when currentJobMode is vendor-specific.

	jobModes
	list of hd:enumAirConJobMode
	true
	false
	false
	List of possible job states the device supports (see clause 5.6.3). 


5.3.1.4 airFlow

This ModuleClass provides capabilities for controlling the air flow of a device.
Table 5.3.4‑1: DataPoints of airFlow ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	speed
	xs:integer
	true
	true
	false
	The current speed level in the range of the [minSpeed, maxSpeed] data poins.

	minSpeed
	xs:integer
	true
	false
	true
	The minimum value for the speed level. If not present, the default is 0.

	maxSpeed
	xs:integer
	true
	false
	true
	The maximum value for the speed level. If not present, the default is 100.

	verticalDirection
	hd:enumVerticalDirection
	true
	true
	true
	The vertical direction of the air flow (see clause 5.6.34).

	supportedVerticalDirection
	List of hd:enumVerticalDirection
	true
	false
	true
	List of supported vertical directions.

	horizontalDirection
	hd:enumHorizontalDirection
	true
	true
	true
	The horizontal direction of the air flow (see clause 5.6.20).

	supportedHorizontalDirection
	List of hd:enumHorizontalDirection
	true
	false
	true
	List of supported horizontal directions.

	automode
	xs:Boolean
	true
	true
	true
	Status of the automode feature. “True” indicates that the speed is set by the device, “False” indicates that the device is not controlling the speed.


5.3.1.5 airPurifierJobMode
This ModuleClasses provides capabilities to control and monitor the job modes of an airPurifier.
Table 5.3.5‑1: DataPoints of airPurifierJobMode ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	currentJobMode
	hd:enumAirPurifierJobMode
	true
	true
	false
	Currently active job mode (see clause 5.6.4). 

	currentJobModeName
	xs:string
	true
	false
	true
	Name of the current job mode as a  string. This can be used when currentJobMode is vendor-specific.

	jobModes
	list of hd:enumAirPurifierJobMode
	true
	false
	false
	List of possible job states the device supports (see clause 5.6.4).


5.3.1.6 airQualitySensor
This ModuleClass provides capabilities for a monitoring sensor that measures the air quality.
Table 5.3.6‑1: DataPoints of airQualitySense ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	sensorPM1
	xs:integer
	true
	false
	true
	The concentration of particle matter under 1㎛.. The minimum value is 0, and the maximum value is 1000.


	sensorPM2
	xs:integer
	true
	false
	true
	The concentration of particle matter under 2.5㎛. The minimum value is 0, and the maximum value is 1000.

	sensorPM10
	xs:integer
	true
	false
	true
	The concentration of particle matter under 10㎛. The minimum value is 0, and the maximum value is 1000.

	sensorOdor
	xs:integer
	true
	false
	true
	The concentration of odor that reflects air pollution. The minimum value is 0, and the maximum valu is 1000.

	sensorHumidity
	xs:integer
	true
	false
	true
	The measured humidity. The minimum value is 0, and the maximum value is 100.

	monitoringEnabled
	xs:boolean
	true
	true
	true
	The current status of monitoring. "True" indicates enabled, and "False" indicates not enabled.

	co2
	xs:integer
	true
	false
	true
	This value indicates the CO2 level in ppm (parts per million).

	co
	xs:integer
	true
	false
	true
	This value indicates the CO level in ppm.

	ch2o
	xs:integer
	true
	false
	true
	This value indicates the CH2O level in ppm.

	voc
	xs:integer
	true
	false
	true
	This value indicates the VOC (Volatile Organic Compounds) value in ppm.


5.3.1.7 alarmSpeaker
This ModuleClass provides the capabilites to initiate and monitor an alarm.

Table 5.3.7‑1: DataPoints of alarmSpeaker ModuleClass

	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	tone 
	hd:enumTone
	true
	true
	true
	Representing the tones of the alarm (see clause 5.6.32).

	light
	hd:enumAlertColourCode
	true
	true
	true
	Representing the lighting mode of the alarm (see clause 5.6.5).

	alarmStatus
	xs:boolean
	true
	true
	false
	"True" indicates the alarm start while "False" indicates the alarm stop.


5.3.1.8 audioVolume

This ModuleClass provides capabilities to control and monitor volume
Table 5.3.8‑1: Actions of audioVolume
	Return Type
	Name
	Argument
	Optional
	Documentation

	none
	upVolume
	none
	true
	Increase the volume by the amount of the stepValue up to the maxValue.

	none
	downVolume
	none
	true
	Decrease the volume by the amount of the stepValue down to 0.


Table 5.3.8‑2: DataPoints of audioVolume
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	volumePercentage
	xs:integer
	true
	true
	false
	The rounded percentage of the current volume in the range of [0, maxValue]. 0 percentage shall mean no sound produced.

	stepValue
	xs:integer
	true
	false
	true
	Step value used by the “UpVolume” and “DownVolume” actions.

	maxValue
	xs:integer
	true
	false
	true
	Maximum value allowed for Volume. maxValue is 100 by default if “maxValue” is not provided.

	muteEnabled
	xs:boolean
	true
	true
	false
	The current status of the mute enablement. "True" indicates enabled (that is, no sound), and "False" indicates not enabled (that is, sound is played).


5.3.1.9 autoDocumentFeeder
This ModuleClasses provides capabilities to monitor the state of autoDocumentFeeder (ADF). ADF is a feature which takes several pages and feeds the paper one page at a time into a scanner or printer, allowing the user to scan, print or fax, multiple-page documents without having to manually replace each page.
Table 5.3.9‑1: DataPoints of autoDocumentFeeder ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	currentAdfState
	hd:enumAdfState
	true
	false
	false
	Current state of the ADF. 

	adfStates
	list of hd:enumAdfState
	true
	false
	false
	List of possible ADF states that are supported by the device (see clause 5.6.2).


5.3.1.10 battery

This ModuleClass provides capabilities to indicate the detection of low battery and gives an alarm if the triggering criterion is met. The level data point in the ModuleClass represents the current battery charge level.

Table 5.3.10‑1: DataPoints of battery ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	level
	xs:integer
	true
	false
	false
	The rounded percentage of the current charging level of a battery in the range of [0, 100]. 0 percentage shall mean that no charge remains.

	capacity
	xs:integer
	true
	false
	true
	The total capacity of battery in mAh.

	charging
	xs:boolean
	true
	false
	true
	The status of charging. "True" indicates enabled, and "False" indicates not enabled. 

	discharging
	xs:boolean
	true
	false
	true
	The status of discharging. "True" indicates charging, and "False" indicates not charging.

	lowBattery
	xs:boolean
	true
	false
	true
	To indicate that the battery is on a low charge level.

	batteryThreshold
	xs:integer
	true
	true
	true
	When a battery’s “level” is less than “batteryThreshold” then “lowBattery” is set to “True”. This datapoint can be used to raise an alarm, depending on the implementation.

	electricEnergy
	xs:integer
	true
	false
	true
	Rated electric energy. The unit of measure is ampere (A).

	voltage
	xs:integer
	true
	false
	true
	Rated voltage. The unit of measure is volts (V).

	material
	xs:string
	true
	false
	true
	The material of the cell (for example lithium ion, nickel and lead.


5.3.1.11 binaryObject
This ModuleClass describes the handling of a binary object (blob). 
Table 5.3.11‑1: DataPoints of binaryObject ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	object
	xs:string
	true
	true
	false
	This data point contains the base64 encoded binary object.

	objectType
	xs:string
	true
	true
	false
	This data point contains the type and subtype of the binary object as a MIME type.

	size
	xs:integer
	true
	true
	true
	The size of the decoded binary object.

	hash
	xs:string
	true
	true
	true
	The hash code of the blob. If present, it is used to check the decoded content of the “object” data point for integrity. The algorithm used for generating the hash value is SHA-2 [15]. The data point contains the hash as a hex encoded value.


5.3.1.12 binarySwitch

This ModuleClass provides capabilities to control and monitor the state of power.
Table 5.3.12‑1: Actions of binarySwitch ModuleClass
	Return Type
	Name
	Argument
	Optional
	Documentation

	none
	toggle
	none
	true
	Toggle the switch.


Table 5.3.12‑2: DataPoints of binarySwitch ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	powerState
	xs:boolean
	true
	true
	false
	The current status of the binarySwitch. "True" indicates turned-on, and "False" indicates turned-off.


5.3.1.13 bioElectricalImpedanceAnalysis
This ModuleClass provides the analysis of human body tissue based on impedance measurement.
Table 5.3.13‑1: DataPoints of bioElectricalImpedanceAnalysis ModuleClass

	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	water
	xs:float
	true
	false
	false
	The water content measurement from the BIA. The unit of measue is percentage.

	fat
	xs:float
	true
	false
	false
	The fat content measurement from the BIA. The unit of measure is percentage.

	muscle
	xs:float
	true
	false
	false
	The muscle content measurement from the BIA. The unit of measure is percentage.

	bone
	xs:float
	true
	false
	false
	The bone content measurement from the BIA. The unit of measure is percentage.

	visceraFat
	xs:float
	true
	false
	false
	The viscera fat content measurement from the BIA. The unit of measure is percentage.

	kcal
	xs:float
	true
	false
	false
	The kcal (kilocalories) measurement from the BIA.

	resistance
	xs:float
	true
	false
	false
	The resistance of human body. The unit of measure is ohm.


5.3.1.14 bodyCompositionAnalyser
This ModuleClass provides the capability to report the measurement of body composition analyser characteristics.

Table 5.3.14‑1: DataPoints of body composition analyser ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	bodyLength
	xs:float
	true
	false
	true
	The measurement of body length by Weight scale and Body composition analyser. The unit of measure is centimeter(cm).

	Bmi
	xs:float
	true
	false
	true
	The measurement of Body Mass Index (BMI) by a weight scale and a body composition analyser. The unit of measure is kg/m2.

	fatFreeMass
	xs:float
	true
	false
	true
	The measurement of fat free mass by a weight scale and a body composition analyser. The unit of measure is kilogram (kg).

	softLeanMass
	xs:float
	true
	false
	true
	The measurement of soft lean mass by a weight scale and a body composition analyser. The unit of measure is kilogram (kg).

	muscleMass
	xs:float
	true
	false
	true
	The measurement of muscle mass by a weight scale and a body composition analyser. The unit of measure is kilogram (kg).

	basalMetabolism
	xs:float
	true
	false
	true
	The measurement of basal metabolism by a weight scale and a body composition analyser. The unit of measure is kilocalorie (kcal).

	impedance
	xs:float
	true
	false
	true
	The measurement of impedance by a weight scale and a body composition analyser. The unit of measure is ohm.


5.3.1.15 boiler

This ModuleClass provides capabilities to control the status of the boiling functionality for water heaters.
Table 5.3.15‑1: DataPoints of boiler ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	status
	xs:boolean
	true
	true
	false
	The status of boiling. “True” indicates boiling, “False” indicates not boiling.


5.3.1.16 brewing

This ModuleClass provides capabilities to control and monitor a brewing process. It is intended to be part of devices that prepare hot drinks such as a coffee or a tea.

Table 5.3.16‑1: DataPoints of brewing ModuleClass

	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	cupsNumber
	xs:integer
	true
	true
	false
	The current number of the cups requested to brew.

	strength
	hd:enumTasteStrength
	true
	true
	true
	The current strength of the drink taste (see clause 5.6.31). A higher value indicates a stronger taste.


5.3.1.17 brightness
This ModuleClass provides capabilities to control and monitor the brightness of a light for example from a lamp. Brightness is scaled as a percentage. A lamp or a monitor can be adjusted to a level of light between very dim (0% is the minimum brightness) and very bright (100% is the maximum brightness).

Table 5.3.17‑1: DataPoints of brightness ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	brightness
	xs:integer
	true
	true
	false
	The status of brightness level in percentage.


5.3.1.18 clock

This ModuleClass provides capabilities to control and monitor time and date information.
Table 5.3.18‑1: DataPoints of clock ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	currentTime
	xs:time
	true
	true
	false
	Information of the current time 

	currentDate
	xs:date
	true
	true
	false
	Information of the current date

	currentTimeZone
	xs:string
	true
	true
	true
	Name of current time zone according to the IANA Timezone data format (TZ) [17], for example, “America/New York”. 


5.3.1.19 clothesDryerJobMode
This ModuleClasses provides capabilities to control and monitor the job modes of a clothes dryer.
Table 5.3.19‑1: DataPoints of clothesDryerJobMode ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	currentJobMode
	hd:enumClothesDryerJobMode
	true
	true
	false
	Currently active job mode  (see clause 5.6.7). 

	currentJobModeName
	xs:string
	true
	false
	true
	The name of current job mode as a string. This can be used when “currentJobMode” is vendor-specific.

	jobModes
	list of hd:enumClothesDryerJobMode
	true
	false
	false
	List of possible job states the device supports.


5.3.1.20 clothesWasherJobMode
This ModuleClasses provides capabilities to control and monitor the job mode of a washer.
Table 5.3.20‑1: DataPoints of clothesWasherJobMode ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	currentJobMode
	hd: enumClothesWasherJobMode
	true
	true
	false
	Currently active job mode (see clause 5.6.8).

	currentJobModeName
	xs:string
	true
	true
	true
	The name of the current job mode as a string. This can be used when the currentJobMode is vendor-specific.

	jobModes
	list of hd:enumClothesWasherJobMode
	true
	false
	false
	List of possible job states that the device supports (see clause 5.6.8).


5.3.1.21 clothesWasherDryerJobMode

This ModuleClasses provides capabilities to control and monitor the job modes of clothesWasherDryer.
Table 5.3.21‑1: DataPoints of clothesWasherDryerJobMode ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	currentJobMode
	hd: enumClothesWasherDryerJobMode
	true
	true
	false
	Currently active job (see clause 5.6.8).

	currentJobModeName
	xs:string
	true
	true
	true
	The name of the current job mode as a string. This can be used when currentJobMode is vendor-specific.

	jobModes
	list of hd:enumClothesWasherDryerJobMode
	true
	false
	false
	List of possible job states the device supports (see clause 5.6.8).


5.3.1.22 clothesWasherJobModeOption
This ModuleClasses provides capabilities to control and monitor the washing job mode options of a washer.
Table 5.3.22‑1: DataPoints of clothesWasherJobModeOption ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	washTemp
	hd:enumGeneralTemperature
	true 
	true
	true
	This data point represents the water temperature level  (see clause 5.6.17).

	soilLevel
	hd:enumGeneralLevel
	true 
	true
	true
	This data point represents the washing level (see clause 5.6.15).

	spinSpeed
	hd:enumGeneralSpeed
	true 
	true
	true
	This data point represents the spin-dry speed level (see clause 5.6.16).

	preWash
	xs:boolean
	true 
	true
	true
	This data point indicates pre-wash. “True” indicates enabled, “False” indicates disabled.

	speedWash
	xs:boolean
	true 
	true
	true
	This data point indicates speed wash. “True” indicates enabled, “False” indicates disabled.

	steamTreat
	xs:boolean
	true 
	true
	true
	This data point indicates steam treat. “True” indicates enabled, “False” indicates disabled. 

	coldWash
	xs:boolean
	true 
	true
	true
	This data point indicates cold wash. “True” indicates enabled, “False” indicates disabled.

	extraRinse
	xs:boolean
	true 
	true
	true
	This data point indicates extra rinse. “True” indicates enabled, “False” indicates disabled.


5.3.1.23 colour
This ModuleClass provides the capabilities to set the value of the Red, Green, and Blue colour channels for a colour device.

Table 5.3.23‑1: DataPoints of colour ModuleClass

	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	red
	xs:integer
	true
	true
	false
	The value of the Red colour channel of RGB. The range is [0,255].

	green
	xs:integer
	true
	true
	false
	The value of the Green colour channel of RGB. The range is [0,255].

	blue
	xs:integer
	true
	true
	false
	The value of the Blue colour channel of RGB. The range is [0,255].


5.3.1.24 colourSaturation
This ModuleClass provides cababilities to control and monitor a colour saturation value.

Table 5.3.24‑1: DataPoints of colourSaturation ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	colourSaturation
	xs:integer
	true
	true
	false
	The status of colour saturation level. “colourSaturation” has a range of [0,100]. A “colourSaturation” value of 0 means that a device displays or produces black and white images. A “colourSaturation” value of 50 means that a  device displays or produces normal colour images. A “colourSaturation” value of 100 means that a device displays or produces very colourfull images.


5.3.1.25 cookerHoodJobMode
This ModuleClasses provides capabilities to control and monitor the job modes of a cookerHood.
Table 5.3.25‑1: DataPoints of cookerHoodJobMode ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	currentJobMode
	hd: enumCookerHoodJobMode
	true
	true
	false
	The currently active job mode. 

	currentJobModeName
	xs:string
	true
	false
	true
	The name of the current job mode as a string. This can be used when currentJobMode is vendor-specific.

	jobModes
	list of hd: enumCookerHoodJobMode
	true
	false
	false
	List of possible job states the device supports (see clause 5.6.10).


5.3.1.26 credentials

This ModuleClass provides the capability to manage user credentials which allows a user to authenticate on an appliance or a server that is associated with the appliance. The authentication depends on a user login and password, or on a token. An example appliance which may include this ModuleClass is a camera. 
Table 5.3.26‑1: DataPoints of credentials ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	loginName
	xs:string
	false
	true
	true
	The user’s login name.

	password
	xs:string
	false
	true
	true
	The user’s password.

	token
	xs:string
	false
	true
	true
	An authentication token, for example an OAuth token.


5.3.1.27 dehumidifierJobMode
This ModuleClasses provides capabilities to control and monitor the job modes of a dehumidifier device.
Table 5.3.27‑1: DataPoints of dehumidifierJobMode ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	currentJobMode
	hd:enumDehumidifierJobMode
	true
	true
	false
	The currently active job mode (see clause 5.6.11). 

	currentJobModeName
	xs:string
	true
	false
	true
	The name of the current job mode as a string. This can be used when currentJobMode is vendor-specific.

	jobModes
	list of hd:enumDehumidifierJobMode
	true
	false
	false
	List of possible job states the device supports. 


5.3.1.28 dishWasherJobMode
This ModuleClasses provides capabilities to control and monitor the job modes of a dishWasher.
Table 5.3.28‑1: DataPoints: DataPoints of dishWasherJobMode ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	currentJobMode
	hd: enumDishWasherJobMode
	true
	true
	false
	The currently active job mode. 

	currentJobModeName
	xs:string
	true
	false
	true
	The name of the current job mode as a string. This can be used when the currentJobMode is vendor-specific.

	jobModes
	list of hd: enumDishWasherJobMode
	true
	false
	false
	List of possible job states the device supports (see clause 5.6.12)


5.3.1.29 doorStatus

This ModuleClass provides the status of a door. It is intended to be part of a device such as a refrigerator and an oven that might have multiple doors. 
Table 5.3.29‑1: DataPoints of doorStatus ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	doorState
	hd:enumDoorState
	true
	false
	false
	Current state of the door (see clause 5.6.13).

	openDuration
	m2m:timestamp
	true
	false
	true
	The time duration the door has been open. 

	openAlarm
	xs:boolean
	true
	true
	true
	The state of the door open alarm. "True" indicates that the open alarm is active. "False" indicates that the open alarm is not active.


5.3.1.30 electricVehicleConnector
This ModuleClass provides information about charging/discharging devices for electric vehicles.
Table 5.3.30‑1: DataPoints of electricVehicleConnector ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	Status
	xs:boolean
	true
	false
	false
	The status of connection. “True” means connected, “False” means not connected.

	chargingCapacity
	xs:integer
	true
	false
	true
	Rated charging capacity. The unit of measure is ampere hours.

	dischargingCapacity
	xs:integer
	true
	false
	true
	Rated discharging capacity. The unit of measure is ampere hours.


5.3.1.31 energyConsumption

This ModuleClass describes the measured energy consumed by the device since power up. One particular use case for the energyConsumption ModuleClass is a smart meter.
Table 5.3.31‑1: DataPoints of energyConsumption ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	power
	xs:float
	true
	false
	false
	The power of the device. The unit of measure is Watt (W).

	absoluteEnergyConsumption
	xs:float
	true
	false
	true
	The absolute energy consumption, reflecting the real measurement of accumulative energy. The unit of measure is Watt-hour (Wh).

	roundingEnergyConsumption
	xs: integer
	true
	false
	true
	This energy consumption data is calculated by multiplying significantDigits with multiplyingFactors, and rounding down the result.

	significantDigits
	xs:integer
	true
	false
	true
	The number of effective digits for data.

	multiplyingFactors
	xs:float
	true
	false
	true
	The unit for data multiplying factors, for example 1 kWh, 0,1 kWh, 0,01 kWh etc.

	voltage
	xs:float
	true
	false
	true
	The voltage of the device. The unit of measure is volts (V).

	current
	xs:float
	true
	false
	true
	The current of the device. The unit of measure is ampere (A).

	frequency
	xs:float
	true
	false
	true
	The frequency of the device. The unit of measure is hertz (Hz).

	measuringScope
	xs:string
	true
	true
	true
	The measuring scope of the meter, for example the whole house, a room, or a device.


5.3.1.32 energyGeneration

This ModuleClass provides information about generation data on electric generator devices such as a photo voltaic power system, fuel cells, or microgeneration.
Table 5.3.32‑1: DataPoints of energyGenerationModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	powerGenerationData
	xs:float
	true
	false
	true
	Amount of instaneous generation data. The unit of measure is Watt (W).

	roundingEnergyGeneration
	xs:integer
	true
	false
	true
	This energy consumption data is calculated by multiplying significantDigits with multiplyingFactors, and rounding down the result.

	significantDigits
	xs:integer
	true
	false
	true
	The number of effective digits for data.

	multiplyingFactors
	xs:floatr
	true
	false
	true
	The unit for data multiplying factors, for example 1 kWh, 0,1 kWh, 0,01 kWh etc.

	generationSource
	xs:string
	true
	true
	false
	The type of generating source.


5.3.1.33 faultDetection

This ModuleClass provides information about whether a fault has occurred in a device.
Table 5.3.33‑1: DataPoints of faultDetection ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	status
	xs:boolean
	true
	false
	false
	The status of fault detection.

	code
	xs:integer
	true
	false
	true
	The numeric representation of the fault.

	description
	xs:string
	true
	false
	true
	The message representation of the fault.


5.3.1.34 filterInfo
This ModuleClass is for monitoring filter information of a device.
Table 5.3.34‑1: DataPoints of filterInfo ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	usedTime
	xs:integer
	true
	false
	false
	The cumulative used time in seconds of a filter

	needsReplacement
	xs:boolean
	true
	false
	true
	This value indicates that the filter needs to be replaced.

	filterLifetime
	xs:integer
	true
	false
	true
	Percentage life time remaining for the water filter.


5.3.1.35 foaming

This ModuleClass provides capabilities to control and monitor desired parameters of foam e.g. for foaming milk. It is initially intended to be part of a device that prepare drinks with milk (for example a coffee machine or hot chocolate machine).

Table 5.3.35‑1: DataPoints of foaming ModuleClass

	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	foamingStrength
	hd:enumFoamStrength
	true
	true
	false
	The current strength of foamed milk. A higher value indicates more foamed milk (see clause 5.6.14)


5.3.1.36 geoLocation
This ModuleClass provides the capability to get or set geo-location information.
Table 5.3.36‑1: DataPoints of geoLocation ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	latitude
	xs:float
	true
	true
	false
	The current latitude part of a geo-location. The unit of measures is degrees.

	longitude
	xs:float
	true
	true
	false
	The current longitude part of a geo-location. The unit of measures is degrees.

	altitude
	xs:float
	true
	true
	true
	The optional current altitude part of a geo-location. The unit of measures is meters.

	heading
	xs:float
	true
	true
	true
	The azimuth of a device measured in degrees to true north. North is 0.0 degrees, east is 90.0 degrees, south is 180.0 degrees, west is 270.0 degrees. A negative value indicates an unknown heading.

	horizontalAccuracy
	xs:float
	true
	false
	true
	The optional current horizontal accuracy of the geo-location. The unit of measures is meters and describes a radius around the latitude/longitude coordinate.

	verticalAccuracy
	xs:float
	true
	false
	true
	The optional current vertical accuracy of the altitude. The unit of measures is meters.

	headingAccuracy
	xs:float
	true
	false
	true
	The optional current maximum deviation between the heading and the true geomagnetic heading. The unit of measures is degrees.

	targetLatitude
	xs:float
	true
	true
	true
	The optional target latitude part of a geo-location. This can be used to move a device to a new location. The unit of measures is degrees.

	targetLongitude
	xs:float
	true
	true
	true
	The optional target longitude part of a geo-location. This can be used to move a device to a new location. The unit of measures is degrees.

	targetAltitude
	xs:float
	true
	true
	true
	The optional target altitude part of a geo-location. This can be used to move a device to a new altitude. The unit of measures is meters.


5.3.1.37 glucometer
This ModuleClass provides the capability to report the measurement of glucose characteristics. 

Table 5.3.37‑1: DataPoints of glucometer ModuleClass

	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	concentration
	xs:float
	true
	false
	false
	The measurement of concentration by Glucometer. The unit of measure is mg/dl.

	hba1c
	xs:float
	true
	false
	true
	The measurement of HbA1c by Glucometer. The unit of measure is percentage.

	contextExercise
	xs:float
	true
	false
	true
	The measurement of context exercise by Glucometer. The unit of measure is percentage.

	contextMedication
	xs:float
	true
	false
	true
	The measurement of context medication by Glucometer. The unit of measure is mg/dl.

	contextCarbohydratesAmount
	xs:float
	true
	false
	true
	The measurement of context carbohydrates by Glucometer. The unit of measure is gram (g).

	contextCarbohydratesSource
	xs:string
	true
	false
	true
	The timing of meals (for example “breakfast carbohydrates”).

	contextMeal
	xs:string
	true
	false
	true
	The style of meals (for examople “casual”).

	contextLocation
	xs:string
	true
	false
	true
	The body location where the Glucometer is worn (for example “finger”).

	contextTester
	xs:string
	true
	false
	true
	The test style (for example “self”).

	contextHealth
	xs:string
	true
	false
	true
	The severity of symptoms (for example “minor”),


5.3.1.38 grinder

This ModuleClass is for controlling a grinder, for example in a coffee machine. 

Table 5.3.38‑1: DataPoints of grinder ModuleClass

	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	useGrinder
	xs:boolean
	true
	true
	false
	The current status of the grinder enablement. "True" indicates enabled, and "False" indicates disabled.

	coarseness
	hd:enumGrindCoarseness
	true
	true
	true
	The wished coarseness of the solid supplies after grinding, for example for coffee beans (see clause 5.6.19).

	grainsRemaining
	hd:enumGrainsLevel
	true
	false
	true
	The level of remaining grains in a machine having a grinder, for example for remaining coffee beans in the coffee machine grinder (see clause 5.6.18).


5.3.1.39 heatingZone
This ModuleClass provides the capabilities to monitor the status of the heating zone, for example for a cooktop.
Table 5.3.39‑1: DataPoints of heatingZone ModuleClass

	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	heatingLevel
	xs:integer
	true
	false
	false
	The current heating level of the zone. The value range is from 0 (indicating that the zone is not heating) up to the maxHeatingLevel.

	maxHeatingLevel
	xs:integer
	true
	false
	false
	The maximum value allowed for the heating level of the zone.


5.3.1.40 height
This ModuleClass provides the capability to report the measurement of height.

Table 5.3.40‑1: DataPoints of height ModuleClass

	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	height
	xs:float
	true
	false
	false
	The height measurement. The unit of measure is centimetre (cm).


5.3.1.41 hotWaterSupply
This ModuleClass provides information about the status of supplying hot water into tanks or bath tubs.
Table 5.3.41‑1: DataPoints of hotWaterSupply ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	bath
	xs:boolean
	true
	true
	true
	The status of whether a bath tub is filled.


5.3.1.42 impactSensor

This ModuleClass describes the capabilities on an impact sensor. The impact is a high force or shock over a short time period and the impactSensor detects this.
Table 5.3.42‑1: DataPoints of impactSensor ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	impactStatus
	xs:boolean
	true
	false
	false
	The “impactStatus” indicates as follows:

“True”  means that a physical impact is detected, “False” means indicates a normal status (no impact detected).

	impactLevel
	xs:float
	true
	false
	true
	The “impactLevel” provides the level of impact. The unit of measure is “g” (G-force).

	impactDirectionHorizontal
	xs:float
	true
	false
	true
	The “impactDirection” indicates the horizontal direction where the impact comes from. The value is 0° to 360°. 0 is the front of the sensor and with clockwise increment.  

	impactDirectionVertical
	xs:float
	true
	false
	true
	The “impactDirection” indicates the vertical direction where the impact comes from. The value is 0° to 360°. 0 is the front of the sensor and with upward increment.


5.3.1.43 keepWarm
This module allows to control the ‘keep warm’ feature in devices like coffe machines, kettles etc. It allows to keep water warm for a desired time. This ModuleClass inherits from binarySwitch (see clause 5.3.12) to store setting for the ‘keep warm’ feature. If the “powerState” data point in a keepWarmSwitch is “True” then the ‘keep warm’ function will be performed just after boiling (or heating) process is finished (otherwise this function will not be applied).

Table 5.3.43‑1: DataPoints of keepWarm ModuleClass

	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	time
	xs:integer
	true
	true
	true
	The desired duration of ‘keep water warm’ function. It indicates how long water shall be kept warm, for example after the boiling in a kettle. The value indicates a time expressed in minutes.


5.3.1.44 keypad
This ModuleClass provides the capability to perform a user defined service through the key-in number. For example, a user can define key 1 as "perform a takeout from a restaurant with combo meal 1". The IoT service provider or user can define the services. 

Table 5.3.44‑1: DataPoints of keypad ModuleClass

	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	keyNumber
	xs:integer
	true
	false
	false
	The number of the pressed key.


5.3.1.45 liquidLevel

This ModuleClass provides the desired level of water (or other liquid)  for an appliance, for example the desired level of milk for a cup of coffee from a coffee machine. 

Table 5.3.45‑1: DataPoints of liquidLevel ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	liquidLevel
	hd:enumLiquidLevel
	true
	true
	false
	The desired level of water or other liquid, for example the desired level of milk in a cup of coffee (see clause 5.6.22).


5.3.1.46 liquidRemaining

This ModuleClass provides the status of water level (or other liquid) for an appliance, for example the level of remaining milk in a coffee machine.

Table 5.3.46‑1: DataPoints of liquidRemaining ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	liquidRemaining
	hd:enumLiquidLevel
	true
	false
	false
	The remaining level of water or other liquid in an appliance (see clause 5.6.22).


5.3.1.47 lock

This ModuleClass provides the function to lock and unlock an object. 
Table 5.3.47‑1: DataPoints of lock ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	lock
	xs:boolean
	true
	true
	false
	"True" indicates the object is locked, while "False" indicates the object is not locked.


5.3.1.48 mediaSelect
This ModuleClass provides capabilities to control and monitor media input and output of device such as TV or SetTopBox.
Table 5.3.48‑1: DataPoints of mediaSelect ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	mediaID
	xs:integer
	true
	true
	false
	The numeric index of the activated media in the supported media sources list “supportedMediaSources”.

	supportedMediaSources
	list of hd:enumSupportedMediaSource
	true
	false
	false
	List of supported input or output media for the given device (see clause 5.6.30).

	mediaName
	xs:string
	true
	false
	true
	Specifies a pre-defined media input or output.

	status
	xs:boolean
	true
	false
	true
	Specifies whether the specific media instance is selected (“True”) or not (“False”).

	mediaType
	hd:enumSupportedMediaSource
	true
	false
	false
	Specifies the type of the media (see clause 5.6.30).


5.3.1.49 motionSensor
This ModuleClass provides the capabilities to indicate the occurrence of motion and raising of an alarm if the triggering criterion is met. 

Table 5.3.49‑1: DataPoints of motionSensor ModuleClass

	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	alarm
	xs:boolean
	true
	false
	false
	The detection of the motion occurrence.

	silentTime
	xs:integer
	true
	true
	true
	The time that a motionSensor restrains from sending an alarm in case continous motions are detected after one alarm is produced. This data point can be used to avoid repeated alarm reports. The unit of measure is seconds.

	sensitivity
	xs:integer
	true
	true
	true
	The level of the detection accuracy of the motion sensor. This data point can be used to control the number of the report.


5.3.1.50 numberValue
This ModuleClass provides the capabilities to represent a number. It also has capabilities for controlled increment and decrement a counter. It can be used to present a number-related functionality in a technology where there is only a weak semantic specification of that functionality. 

Table 5.3.50‑1: Actions of numberValue ModuleClass
	Return Type
	Name
	Argument
	Optional
	Documentation

	none
	decrementNumberValue
	none
	true
	Decrement the “numberValue” by the value of “step”, down to the value of “minimum”.

	none
	incrementNumberValue
	none
	true
	Increment the "numberValue" by the value of "stepValue", up to the value of "maxValue".

	none
	resetNumberValue
	none
	true
	Reset the “numberValue” to its “defaultValue”.


Table 5.3.50‑2: DataPoints of numberValue ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	numberValue
	xs:float
	true
	true
	false
	The actual value of the number.

	minValue
	xs:float
	true
	true
	true
	The optional minimum value of the number. The default is the system-specific minimum value for a float value.

	maxValue
	xs:float


	true
	true
	true
	The optional maximum value of the number. The default is the system-specific maximum value for a float value.

	defaultValue
	xs:float
	true
	true
	true
	The optional default value for the number. The default is 0.0 .

	step
	xs:float
	true
	true
	true
	The optional step size for controlled increment and decrement. The default is 1.0 , even when this data point is not implemeneted.


5.3.1.51 openLevel
This ModuleClass provides the capabilities to control and monitor the open status of an entity, for example a curtain.
Table 5.3.51‑1: Actions of openLevel ModuleClass
	Return Type
	Name
	Argument
	Optional
	Documentation

	none
	open
	None
	true
	Increase the open level by the amount of the “stepValue” up to the “maxLevel”.

	none
	close
	None
	true
	Decrease the open level by the amount of the “stepValue” down to the “minLevel”.


Table 5.3.51‑2: DataPoints of openLevel ModuleClass

	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	openLevel
	xs:integer
	true
	true
	false
	The rounded percentage of the current open level of entity in the range of [0, 100]. 0 percentage shall mean the entity is closed.

	stepValue
	xs:integer
	true
	true
	true
	The step value used by the “open” and “close” actions.

	minLevel
	xs:integer
	true
	true
	true
	The minimum value allowed for the “openLevel” status. The default value is 0, which means fully closed.

	maxLevel
	xs:integer
	true
	true
	true
	The maximum value allowed for the “openLevel” status. The default value is 100, which means fully opened.


5.3.1.52 operationMode
This ModuleClasses provides capabilities to control or monitor the operation mode of appliances.
Table 5.3.52‑1: DataPoints of operationMode ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	startPause
	xs:boolean
	true
	true
	false
	A value of “True” triggers or starts an operation, and “False” pauses the operation.


5.3.1.53 overcurrentSensor

This ModuleClass provides capabilities for an over-current sensor. 
Table 5.3.53‑1: DataPoints of overcurrentSensor ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	overcurrentStatus
	xs:boolean
	true
	false
	false
	The overcurrentStatus indicates as follows:

“True” indicates that an over-current is detected, and “False” indicates a normal status, this means that an over-current is not detected.

	detectedTime
	m2m:timestamp
	true
	false
	true
	The time when the over-current was detected.

	duration
	xs:float
	true
	false
	true
	The duration of the detected over-current.
The unit of measure is “ms”.


5.3.1.54 oximeter
This ModuleClass provides the capability to report the measurement of blood oxygen characteristics. 

Table 5.3.54‑1: DataPoints of oximeter ModuleClass

	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	oxygenSaturation
	xs:integer
	true
	false
	false
	The measurement of oxygensaturation by Oximeter. The unit of measure is in percentage.


5.3.1.55 ozoneMeter

This ModuleClass provides capabilities for an ozone meter. The “ozoneValue…” attributes are optional, but one of them SHALL be provided.
Table 5.3.55‑1: DataPoints of ozoneMeter ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	ozoneValuePPM
	xs:float
	true
	false
	true
	The unit of measure of ozone measurement is “ppm”.

	ozoneValueMG
	xs:float
	true
	false
	true
	The unit of measure of ozone measurement is “mg/m3“.

	ozoneStatus
	hd:enumOzoneStatus
	true
	false
	true
	The ozoneStatus indicates the level of ozone status. (see clause 5.6.24). 

	maxValue
	xs:float
	true
	false
	true
	The mazimum value shows the measurement range of the ozone meter (for example maxValue=5 means the range is 0 to 5 ppm). This attribute is only used that the ozoneMeter provides “ppm” value.


5.3.1.56 phoneCall
This ModuleClass provides the capability get or set the caller and receipient IDs as well as to initate and terminate a call.
Table 5.3.56‑1: Actions of phoneCall ModuleClass
	Return Type
	Name
	Argument
	Optional
	Documentation

	none
	call
	none
	true
	Initiate an outgoing call.

	none
	answer
	none
	true
	Answer (pickup) an incoming call.

	none
	hangup
	none
	true
	Hangup an established call.


Table 5.3.56‑2: DataPoints of phoneCall ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	callerID
	xs:string
	true
	true
	false
	This data point represents the initiating caller identiﬁcation of a call. In case of an outgoing call this would be the local line ID. This data point is optional. When it is empty for an incomming call, then the caller ID is unknown. When it is empty for an outgoing call, then it is exepected that it is set by the PBX or the operator. The format of caller ID is not speciﬁed here.

	recipientID
	xs:string
	true
	true
	false
	This data point represents the receiving caller identiﬁcation of a call. In case of an incoming call this would be the local line ID and optionally extension. The format of caller ID is not speciﬁed here.

	callState
	hd:enumCallState
	true
	false
	true
	This data point represents the current state of an associated phone device regarding calls.


5.3.1.57 playerControl
This ModuleClass provides capabilities to control and monitor the operational modes of a media player functionality.
Table 5.3.57‑1: Actions of playerControl ModuleClass
	Return Type
	Name
	Argument
	Optional
	Documentation

	none
	nextTrack
	none
	true
	Go forward to a next chapter, section or similar marker in the media.

	none
	previousTrack
	none
	true
	Go back to a previous chapter, section or similar marker in the media.


Table 5.3.57‑2: DataPoints of playerControl ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	currentPlayerMode
	hd:enumPlayerMode
	true
	true
	false
	The current mode of the player.

	currentPlayerModeName
	xs:string
	true
	false
	true
	Name of current player mode in string. This can be used when “currentPlayerMode” is vendor-specific.

	supportedPlayerModes
	list of hd:enumPlayerMode
	true
	false
	false
	List of supported modes for a player.

	speedFactor
	xs:float
	true
	true
	true
	The optional factor of speeding up or slowing down playback, rewind or fast forward.


5.3.1.58 powerSave 

This ModuleClass provides capabilities to enable the power saving mode of a device and monitor the current status.
Table 5.3.58‑1: DataPoints of powerSave ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	powerSaveEnabled
	xs:boolean
	true
	true
	false
	The current status of the power saving mode. "True" indicates enabled, and "False" indicates not enabled.


5.3.1.59 printQueue

This ModuleClass provides the capabilities for monitoring printing list information. 
Table 5.3.59‑1: DataPoints of printQueue ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	uri
	list of

xs:uri
	true
	false
	false
	The URI of the printing file. The URI could be “file://www.example.com/file.extension”

	printingState
	list of hd:enumJobState
	true
	false
	false
	The printingState is indicating the status of the printing file.


5.3.1.60 pulsemeter
This ModuleClass provides the capability to report the measurement of pulse characteristics. 

Table 5.3.60‑1: DataPoints of pulsemeter ModuleClass

	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	pulseRate
	xs:float
	true
	false
	false
	The measurement of pulserate by pulsemeter. The unit of measure is in beats per minute.

	rr
	xs:float
	true
	false
	true
	The measurement of RR interval by pulsemeter. The unit of measure is millisecond (ms).

	energy
	xs:float
	true
	false
	true
	The measurement of energy by pulsemeter. The unit of mease is kilocalorie per hour.

	modality
	xs:string
	true
	false
	true
	The modality of a particular SpO2 measurement.


5.3.1.61 pushButton
This ModuleClass provides the capability to indicate the operation of a push button style switch. A typical application can be an SOS button.
Table 5.3.61‑1: DataPoints of pushButton ModuleClass

	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	pushed
	xs:boolean
	true
	false
	false
	This data point indicates the press of the button.


5.3.1.62 recorder
This ModuleClass provides the capability to record video/audio for a defined duration.

Table 5.3.62‑1: DataPoints of recorder ModuleClass

	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	duration
	xs:integer
	true
	true
	false
	The duration for video/audio recording. Set to trigger the recorder. The unit of measure is seconds.


5.3.1.63 refrigeration
This ModuleClass provides capabilities for a refrigeration function.

Table 5.3.63‑1: DataPoints of refrigeration ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	rapidFreeze
	xs:boolean
	true
	True
	true
	Controls the rapid freeze capability. “True” indicates active, “False” indicates inactive.

	rapidCool
	xs:boolean
	true
	True
	true
	Controls the rapid cool capability. “True” indicates active, “False” indicates inactive.

	defrost
	xs:boolean
	true
	True
	true
	Controls the defrost cycle. “True” indicates active, “False” indicates inactive.


5.3.1.64 relativeHumidity
This ModuleClass provides the capability for a device to report the humidity based on a specified rule that is vendor dependent.

Table 5.3.64‑1: DataPoints of relativeHumidity ModuleClass

	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	relativeHumidity
	xs:float
	true
	false
	false
	The measurement of the relative humidity value; the unit of measure is percentage.

	desiredHumidity
	xs:float
	true
	true
	true
	Desired value for humidity. This data point indicates the desired humidity.


5.3.1.65 remoteControlEnable
This ModuleClasses provides capabilities to monitor the remote controllability of the appliance. 
Table 5.3.65‑1: DataPoints of remoteControlEnable ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	remoteControlEnabled
	xs:boolean
	true
	false
	false
	This data point enables or disables remote controllability and is set by a user locally. “True” indicates  enabled remote access, and “False” indicates disabled remote access.


5.3.1.66 robotCleanerJobMode

This ModuleClasses provides capabilities to control and monitor the job modes of a robotCleaner.
Table 5.3.66‑1: DataPoints of robotCleanerJobMode ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	currentJobMode
	hd:enumRobotCleanerJobMode
	true
	true
	false
	Currently active job mode (see clause 5.6.26). 

	currentJobModeName
	xs:string
	true
	false
	true
	Name of current job mode as a string. This can be used when “currentJobMode” is vendor-specific.

	jobModes
	list of hd:enumRobotCleanerJobMode
	true
	false
	false
	List of possible job states the device supports (see clause 5.6.26).


5.3.1.67 runState
This ModuleClasses provides capabilities to control and the monitor machine state of appliances.
Table 5.3.67‑1: DataPoints of runState ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	currentMachineState
	hd:enumMachineState
	true
	true
	false 
	The currently active machine state  (see clause 5.6.23).

	machineStates
	list of hd:enumMachineState
	true 
	false
	false
	A list of possible machine states the device supports (see clause 5.6.23).

	currentJobState
	hd:enumJobState
	true
	false
	true 
	The currently active job state at the level of some transaction being executed by the device  (see clause 5.6.21).

	jobStates
	list of hd:enumJobState
	true 
	false
	true
	The list of possible job states that the device supports (see clause 5.6.21).

	progressPercentage
	xs:float
	true
	false
	true
	The indication of current job progress in percentage.


5.3.1.68 securityMode
This ModuleClasses provides capabilities to control and monitor a security mode.
Table 5.3.68‑1: DataPoints of securityMode ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	currentSecurityMode
	hd:enumSecurityMode
	true
	true
	false 
	Current security mode (see clause 5.6.27).

	securityModes
	list of hd:enumSecurityMode
	true 
	false
	false
	List of possible security modes the device supports (see clause 5.6.27).


5.3.1.69 sessionDescription
This ModuleClass provides the capabilities for a sessionDescription containing a URL at twhich the specified media can be accessed and the definition of media using SDP.
Table 5.3.69‑1: DataPoints of mediaType ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	url
	xs:uri
	true
	false
	true
	A URL at which the specified media can be accessed.

	sdp
	xs:string
	true
	false
	true
	Media description using SDP. One or more comma separated multiple SDP lines (SDP media or attribute line) can be included using SDP description syntax as defined in the SDP specification in RFC4566 [16].


5.3.1.70 signalStrength
This ModuleClass provides the capability to monitor the strength of the signal.

Table 5.3.70‑1: DataPoints of signalStrength ModuleClass

	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	lqi
	xs:integer
	true
	false
	false
	The current value of link quality indicator, which reflects the scaling of rssi by dividing the received signal strength over reference signal strength. The common unit for lqi is percentage [0,100]. For the detailed definition, please see IEEE 802.15.4 [i.4], clause 6.7.8.

	rssi
	xs:float
	true
	false
	true
	The current value of received signal strength indicator, which reflects the raw signal level.


5.3.1.71 smokeSensor
This ModuleClass provides the capabilities to indicate the detection of smoke and raising an alarm if the triggering criterion is met. 

Table 5.3.71‑1: DataPoints of smokeSensor ModuleClass

	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	alarm
	xs:boolean
	true
	false
	false
	The alarm is indicated as follows:

“True” indicates that smoke has been detected, “False” indicates a normal status, that means that smoke is not detected.

	detectedTime
	m2m:timestamp
	true
	true
	true
	The date and time the smoke is detected.

	smokeThreshhold
	xs:integer
	true
	true
	true
	The threshhold to trigger the alarm. The unit of measure is ppm.

	currentValue
	xs:integer
	true
	false
	true
	The current data value of the smoke sensor.


5.3.1.72 sphygmomanometer
This ModuleClass provides the capability to report the measurement of blood pressure characteristics. 

Table 5.3.72‑1: DataPoints of sphygmomanometer ModuleClass

	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	diastolicPressure 
	xs:float
	true
	false
	false
	The measurement of diastolic pressure by sphygmomanometer. The unit of measure is millimetre of mercury (mmHg).

	systolicPressure
	xs:float
	true
	false
	false
	The measurement of systolic pressure by sphygmomanometer. The unit of measure is millimetre of mercury (mmHg).

	meanPressure
	xs:float
	true
	false
	false
	The measurement of mean arterial pressure by sphygmomanometer. The unit of measure is millimetre of mercury (mmHg).


5.3.1.73 spinLevel

This ModuleClass provides capabilities to control and monitor the level of spin. It is intended to be part of devices which use spinning function such as a washing machine and a dryer.
Table 5.3.73‑1: DataPoints of spinLevel ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	spinLevelStrength


	hd:enumSpinLevelStrength
	true
	true
	false
	The value of spin-dry level (see clause 5.6.28). A higher value indicates a higher spin level.


5.3.1.74 steamClosetJobMode

This ModuleClasses provides capabilities to control and monitor the job modes of steamCloset.
Table 5.3.74‑1: DataPoints of steamClosetJobMode ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	currentJobMode
	hd:enumSteamClosetJobMode
	true
	true
	false
	Currently active job mode (see clause 5.6.29). 

	currentJobModeName
	xs:string
	true
	false
	true
	Name of current job mode as a string. This can be used when “currentJobMode” is vendor-specific.

	jobModes
	list of hd:enumSteamClosetJobMode
	true
	false
	false
	List of possible job states the device supports (see clause 5.6.29).


5.3.1.75 televisionChannel

This ModuleClass provides capabilities to set and get channels of a device that has a channel list.
Table 5.3.75‑1: Actions of televisionChannel ModuleClass
	Return Type
	Name
	Argument
	Optional
	Documentation

	none
	upChannel
	None
	true
	Change the current channel to the next channel in the stored list of available channels. If the current channel is the last one in the list, the new set channel may be the first one in the list.

	none
	downChannel
	None
	true
	Change the current channel to the previous channel in the stored list of available channels. If the current channel is the first one in the list, the new set channel may be the last one in the list.


Table 5.3.75‑2: DataPoints of televisionChannel ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	channelNumber
	xs:integer
	true
	true
	false
	Current channel number. 

	availableChannels
	list of xs:integer
	true
	false
	true
	The list of available channel numbers which may be build by automatic scan and/or manual selction.

	previousChannel
	xs:integer
	true
	false
	true
	The channel number which was selected previously.

	channelName
	xs:string
	true
	false
	true
	Current human-friendly channel name in string, for example ‘CNN’.


5.3.1.76 temperature

This ModuleClass provides capabilities to represent the current temperature and target temperature of devices such as an air conditioner, refrigerator, oven etc.
Table 5.3.76‑1: DataPoints of temperature ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	currentTemperature
	xs:float
	true
	false
	false
	The current temperature.

	targetTemperature
	xs:float
	true
	true
	true
	The desired temperature to reach.

	unit
	xs:string
	true
	false
	true
	The unit of measure for the temperature values. The default is celsius (C).

	minValue
	xs:float
	true
	false
	true
	Minimum value of “targetTemperature”.

	maxValue
	xs:float
	true
	false
	true
	Maximum value of “targetTemperature”.

	stepValue
	xs:float
	true
	false
	true
	Step value allowed for “targetTemperature”.


5.3.1.77 temperatureAlarm
This ModuleClass provides the capabilities to indicate the detection of abnormal temperatures and raises an alarm if the triggering criterion is met. 

Table 5.3.77‑1: DataPoints of temperatureAlarm ModuleClass

	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	alarm
	xs:boolean
	true
	false
	false
	This data point indicates the status of detection of an abnormal temperature. “True” indicates an abnormal temperature, “False” indicates a normal temperature.

	temperature
	xs:float
	true
	false
	true
	To report the value of the temperature. The unit of measure is celsius (C).

	temperatureThreshhold
	xs:integer
	true
	true
	true
	The threshhold to trigger the alarm.


5.3.1.78 textMessage
This ModuleClass provides capabilities to set and get a text message.
Table 5.3.78‑1: Actions of textMessage ModuleClass
	Return Type
	Name
	Argument
	Optional
	Documentation

	none
	resetTextMessage
	none
	true
	Reset  the receiver of the message to the “defaultValue”.


Table 5.3.78‑2: DataPoints of textMessage ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	textMessage
	xs:string
	true
	true
	false
	The current message value.

	supportedMessageValues
	list of xs:string
	true
	false
	true
	List of supported values for the message. Each of the values in this list must be URL-encoded. An encoded value must not contain white spaces.

	minLength
	xs:integer
	true
	false
	true
	The optional minimum length in characters of the message. The default is 0.

	maxLength
	xs:integer
	true
	false
	true
	The optional maximum length in characters of the message. The default is unlimited.

	messageEncoding
	xs:string
	true
	false
	true
	The optional expected method for character encoding of the message. The default is "UTF-8".

	defaultValue
	xs:string
	true
	true
	true
	The optional default value for “textMessage”. The default is an empty string.


5.3.1.79 timer

This ModuleClass provides capabilities to monitor and control the times when the appliance executes its operations, that means when it starts, when it ends etc.
Table 5.3.79‑1: Actions of timer ModuleClass
	Return Type
	Name
	Argument
	Optional
	Documentation

	none
	activateClockTimer
	None
	true
	Activate current clock timer.

	none
	deactivateClockTimer
	None
	true
	Deactivate current clock timer.


Table 5.3.79‑2: DataPoints of timer ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	referenceTimer
	xs:integer
	true
	false
	true
	A timer (for example. a time-based value, App Defined Epoch, Progressive) expressed in seconds. The value indicates a time counter to be used as reference for the other time-based data points of this ModuleClass. Usually it is the time since the last event of power-on of the producer (or more in detail the time since the boot of its connectivity node).

	targetTimeToStart
	xs:integer 
	true
	true
	true
	A time span (for example a time-based value, App Defined Epoch, Fixed) expressed in seconds. The value indicates the time when the appliance is expected to start its operation, starting counting from the last ”referenceTimer”.

	targetTimeToStop
	xs:integer 
	true
	true
	true
	A time span (for example a time-based value, App Defined Epoch, Fixed) expressed in seconds. The value indicates the time when the appliance is expected to stop its operation, starting counting from the last ”referenceTimer”.

	estimatedTimeToEnd
	xs:integer
	true
	false
	true
	A timer (for example a time-based value, App Defined Epoch, Progressive) expressed in seconds. The value indicates the time to the end of an appliance’s operations. It is calculated at runtime by the device itself during the execution of its operation.

	runningTime
	xs:integer
	true
	false
	true
	A timer (for example a time-based value, App Defined Epoch, Progressive) expressed in seconds. It indicates the time of the current operation. Usually its value is increasing one value per  second. It starts counting from 0 when the operation starts and stops counting when the operation ends.

	targetDuration
	xs:integer
	true
	false
	true
	A time span (for a time-based value, App Defined Epoch, Fixed) expressed in seconds. The value indicates a time that represents the target duration of the operation as per user selection. 

	absoluteStartTime
	m2m:timestamp
	true
	true
	true
	An absolute time to specify the start time.

	absoluteStopTime
	m2m:timestamp
	true
	true
	true
	An absolute time to specify the stop time.


5.3.1.80 turbo

This ModuleClass provides capabilities to enable turbo mode and monitor the current status of the turbo function. It is intended to be part of devices which use turbo function such as an air conditioner, a washing machine etc.
Table 5.3.80‑1: DataPoints of turbo ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	turboEnabled
	xs:boolean
	true
	true
	false
	The current status of the turbo mode. "True" indicates enabaled, and "False" indicates not enabled.


5.3.1.81 uvSensor

This ModuleClass describes the capabilities of an ultraviolet sensor. 
Table 5.3.81‑1: DataPoints of uvSensor ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	uvValue
	xs:float
	true
	false
	false
	The unit of measure of the UV intensity of radiation is “mW/cm2”.

	uvStatus
	hd:enumUvStatus
	true
	false
	true
	The “uvStatus” indicates the level of the UV radiation status (see clause 5.6.33). 


5.3.1.82 waterFlow

This ModuleClass provides capabilities for controlling the water strength of a device.
Table 5.3.82‑1: DataPoints of waterFlow ModuleClass
	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	waterLevelStrength
	hd:enumWaterFlowStrength
	true
	true
	false
	The desired level of water flow (see clause 5.6.35). A higher value indicates higher water flow.


5.3.1.83 waterSensor
This ModuleClass provides the capabilities to indicate whether or not water has been sensed, and raising an alarm if the triggering criterion is met. 

Table 5.3.83‑1: DataPoints of waterSensor ModuleClass

	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	alarm
	xs:boolean
	True
	false
	false
	The detection of water.
The alarm is indicated as follows:

“True” indicates that water has been detected, “False” indicates a normal status, that means that water is not detected.


5.3.1.84 weight
This ModuleClass provides the capability to report the measurement of weight.

Table 5.3.84‑1: DataPoints of weight ModuleClass

	Name
	Type
	Readable
	Writable
	Optional
	Documentation

	weight
	xs:float
	true
	false
	false
	The weight measurement. The common unit is kilogram (kg).


5.3.2 City domain
5.3.3 Health domain
5.3.4 Home domain

5.3.5 Industry domain
5.3.6 Vehicular domain

5.4 
SubDevice models
5.4.1 Common domain
5.4.1.1 subDeviceCuff
A cuff is a subDevice that expresses the attachment device for measuring blood pressure.
Table 5.4.1‑1: Modules of subDeviceCuff model
	Module Instance Name
	Module Class Name
	Optional
	Description

	binarySwitch
	binarySwitch
	true
	See clause 5.3.12.

	sphygmomanometer
	sphygmomanometer
	false
	See clause 5.3.72.

	pulsemeter
	pulsemeter
	false
	See clause 5.3.60.


5.4.1.2 subDevicePowerOutlet

A powerOutlet is a subDevice that specifies the attachment device for deviceSmartPlug.
Table 5.4.2‑1: Modules of subDevicePowerOutlet model
	Module Instance Name
	Module Class Name
	Optional
	Description

	binarySwitch
	binarySwitch
	true
	See clause 5.3.12.

	energyConsumption
	energyConsumption
	true
	See clause 5.3.31.

	overcurrentSensor
	overcurrentSensor
	true
	See clause 5.3.53.

	dimmingLevel
	numberValue
	true
	See clause 5.3.50.

This provides the capability to change the energy.


5.4.2 City domain
5.4.3 Health domain
5.4.4 Home domain

5.4.5 Industry domain
5.4.6 Vehicular domain

5.4.7 
5.4.8 
5.5 Device models
5.5.1 Common domain
5.5.1.1 device3DPrinter
A 3D printer is a smart home appliance to provide 3D printing capabilities.
Table 5.5.1‑1: Modules of device3DPrinter Device model
	Module Instance Name
	Module Class Name
	Optional
	Description

	binarySwitch
	binarySwitch
	false
	See clause 5.3.12.

	faultDetection
	faultDetection
	true
	See clause 5.3.33.

	3Dprinter
	3Dprinter
	false
	See clause 5.3.1.

	runState
	runState
	false
	See clause 5.3.67.

	temperature
	temperature
	false
	See clause 5.3.76.

This value of “currentTemperature” in this module instance represents the temperature of the nozzle. This value SHALL be a float number in a range from 0.0 to 1000.0.

	printQueue
	printQueue
	false
	See clause 5.3.59.


5.5.1.2 deviceAirConditioner
An air conditioner is a home appliance used to alter the properties of air (primarily temperature and humidity) to more comfortable conditions. This air conditioner information model provides capabilities to control and monitor air conditioner specific functions and resources.

Table 5.5.2‑1: Modules of deviceAirConditioner Device model
	Module Instance Name
	Module Class Name
	Optional
	Description

	binarySwitch
	binarySwitch
	false
	See clause 5.3.12.

	runState
	runState
	true
	See clause 5.3.67.

	airConJobMode
	airConJobMode
	true
	See clause 5.3.1.

	airConOperationMode
	operationMode
	true
	See clause 5.3.52.

This module instance is used to trigger an airCon operation that is pre-set in “airConJobMode”. If this data point is not present, then the air conditioner’s job mode can be triggered by setting the “airConJobMode”.

	airCleanOperationMode
	operationMode
	true
	See clause 5.3.52.

This module instance is used to trigger airClean operation.

	temperature
	temperature
	true
	See clause 5.3.76.

	timer
	timer
	true
	See clause 5.3.79.

	sleepTimer
	timer
	true
	See clause 5.3.79.

The sleep function, which is vendor-specific algorithm (for example increasing the remperature by one degree for every 30 minutes), is triggered instantly when “targetDuration” is set, and it indicates the time to the end of appliance operation. It is set at runtime by a user application.

	turbo
	turbo
	true
	See clause 5.3.80.

	airFlow
	airFlow
	true
	See clause 5.3.4.

	powerSave
	powerSave
	true
	See clause 5.3.58.

	airQualitySensor
	airQualitySensor
	true
	See clause 5.3.6.

	filterInfo
	filterInfo
	true
	See clause 5.3.34.


5.5.1.3 deviceAirPurifier
An airPurifieris a home appliance is used to prevent dust and other particles from air by filtering, washing or electrostatic precipitation. This airPurifier information model provides capabilities to control and monitor airPurifier specific functions and resources.

Table 5.5.3‑1: Modules of deviceAirPurifier Device model
	Module Instance Name
	Module Class Name
	Optional
	Description

	binarySwitch
	binarySwitch
	false
	See clause 5.3.12.

	runState
	runState
	true
	See clause 5.3.67.

	airPurifierJobMode
	airPurifierJobMode
	true
	See clause 5.3.5.

	airPurifierOperationMode
	operationMode
	true
	See clause 5.3.52.

This module instance is used to trigger the airPurifier operation.

	timer
	timer
	true
	See clause 5.3.79.

	powerSave
	powerSave
	true
	See clause 5.3.58.

	airQualitySensor
	airQualitySensor
	true
	See clause 5.3.6.

	filterInfo
	filterInfo
	true
	See clause 5.3.34.


5.5.1.4 deviceAirQualityMonitor
An air quality monitor is a home appliance for monitoring the air quality. This airQualityMonitor information model provides capabilities to monitor the airQualityMonitor functions and resources.

Table 5.5.4‑1: Modules of deviceAirQualityMonitor Device model
	Module Instance Name
	Module Class Name
	Optional
	Description

	airQualitySensor
	airQualitySensor
	false
	See clause 5.3.6.


5.5.1.5 deviceAudioReceiver
An audio receiver is a device that receives audio signals from a number of sources, processing them to drive speakers.
Table 5.5.5‑1: Modules of deviceAudioReceiver model
	Module Instance Name
	Module Class Name
	Optional
	Description

	binarySwitch
	binarySwitch
	false
	See clause 5.3.12.

	audioVolume
	audioVolume
	false
	See clause 5.3.8.

	mediaInput
	mediaSelect
	true
	See clause 5.3.48.

	mediaOutput
	mediaSelect
	true
	See clause 5.3.48.


5.5.1.6 deviceBloodPressureMonitor
A blood pressure monitor is a device that can be used to monitor the blood pressure and is composed of one or more cuffs and a main montor machine.
Table 5.5.6‑1: Modules of deviceBloodPressureMonitor Device model
	Module Instance Name
	Module Class Name
	Optional
	Description

	battery
	battery
	false
	See clause 5.3.10

	binarySwitch
	binarySwitch
	true
	See clause 5.3.12


Table 5.5.6‑2: Subdevice of deviceBloodPressureMonitor Device model
	Subdevice Instance Name
	Subdevice Name
	Optional
	Description

	cuff0
	subDeviceCuff
	false
	See clause 5.4.1

	cuff1
	subDeviceCuff
	true
	See clause 5.4.1

	cuff2
	subDeviceCuff
	true
	See clause 5.4.1

	cuff3
	subDeviceCuff
	true
	See clause 5.4.1

	cuff4
	subDeviceCuff
	true
	See clause 5.4.1

	cuff5
	subDeviceCuff
	true
	See clause 5.4.1


5.5.1.7 
deviceCamera
A camera is an optical instrument for recording or capturing images, which may be stored locally or transmitted to another locations.
Table 5.5.7‑1: Modules of deviceCamera Device model
	Module Instance Name
	Module Class Name
	Optional
	Description

	sessionDescription
	sessionDescription
	false
	See clause 5.3.69.

	playerControl
	playerControl
	true
	See clause 5.3.57.

	motionSensor
	motionSensor
	true
	See clause 5.3.49.


5.5.1.8 deviceClothesDryer
A clothes dryer is a home appliance for drying clothes. This clothesDryer information model provides capabilities to control and monitor clothes dryer specific functions and resources.

Table 5.5.8‑1: Modules of deviceClothesDryer Device model
	Module Instance Name
	Module Class Name
	Optional
	Description

	binarySwitch
	binarySwitch
	false
	See clause 5.3.12.

	runState
	runState
	true
	See clause 5.3.67.

	clothesDryerJobMode
	clothesDryerJobMode
	true
	See clause 5.3.19.

	clothesDryerOperationMode
	operationMode
	true
	See clause 5.3.52.

This module instance is used to trigger the clothesDryer operation.


5.5.1.9 deviceClothesWasher
A clothes washer is a home appliance that is used to wash laundry, such as clothing and sheets. This information model provides capabilities to interact with specific functions and resources of clothes washers.

Table 5.5.9‑1: Modules of deviceClothesWasher Device model
	Module Instance Name
	Module Class Name
	Optional
	Description

	binarySwitch
	binarySwitch
	false
	See clause 5.3.12.

	clothesWasherOperationMode
	operationMode
	true
	See clause 5.3.52.

	runState
	runState
	false
	See clause 5.3.67.

	clothesWasherJobMode
	clothesWasherJobMode
	false
	See clause 5.3.20.

	clothesWasherJobModeOption
	clothesWasherJobModeOption
	true
	See clause 5.3.22.

	remoteControlEnable
	remoteControlEnable
	true
	See clause 5.3.65.

	timer
	timer
	true
	See clause 5.3.79.


5.5.1.10 deviceClothesWasherDryer
A clothes washer dryer is a home appliance that is a combination of cloth washer and cloth dryer in a single cabinet. This information model provides capabilities to interact with specific functions and resources of clothes washers and dryers.

Table 5.5.10‑1: Modules of deviceClothesWasherDryer Device model
	Module Instance Name
	Module Class Name
	Optional
	Description

	binarySwitch
	binarySwitch
	false
	See clause 5.3.12.

	clothesWasherDryerOperationMode
	operationMode
	true
	See clause 5.3.52.

	runState
	runState
	false
	See clause 5.3.67.

	clothesWasherDryerJobMode
	clothesWasherDryerJobMode
	false
	See clause 5.3.21.

	clothesWasherJobModeOption
	clothesWasherJobModeOption
	true
	See clause 5.3.22.

	remoteControlEnable
	remoteControlEnable
	true
	See clause 5.3.65.

	timer
	timer
	true
	See clause 5.3.79.


5.5.1.11 deviceCoffeeMachine

A coffee machine is a device that is used to brew a coffee, may add foamed milk, and may include some variants, for example a grinder.
Table 5.5.11‑1: Modules of deviceCoffeeMachine Device model

	Module Instance Name
	Module Class Name
	Optional
	Description

	faultDetection
	faultDetection
	true
	See clause 5.3.33.

	runState
	runState
	true
	See clause 5.3.67.

	clock
	clock
	true
	See clause 5.3.18.

	brewing
	brewing
	false
	See clause 5.3.16.

	waterStatus
	liquidRemaining
	true
	See clause 5.3.46.

	milkStatus
	liquidRemaining
	true
	See clause 5.3.46.

	grinder
	grinder
	true
	See clause 5.3.38.

	milkFoaming
	foaming
	true
	See clause 5.3.35.

	milkQuantity
	liquidLevel
	true
	See clause 5.3.45.

	brewingSwitch
	binarySwitch
	false
	See clause 5.3.12.

	keepWarm
	keepWarm
	true
	See clause 5.3.43.


5.5.1.12 deviceCookerHood
A cooker hood is a device containing a mechanical fan that hangs above the stove or cooktop in the kitchen.
Table 5.5.12‑1: Modules of deviceCookerHood model
	Module Instance Name
	Module Class Name
	Optional
	Description

	binarySwitch
	binarySwitch
	false
	See clause 5.3.12.

	runState
	runState
	true
	See clause 5.3.67.

	airFlow
	airFlow
	true
	See clause 5.3.4.

	cookerHoodJobMode
	cookerHoodJobMode
	true
	See clause 5.3.25.


5.5.1.13 deviceCooktop
A cooktop is a device that is a kitchen appliance designed for the purpose of cooking food.
Table 5.5.13‑1: Modules of deviceCooktop model
	Module Instance Name
	Module Class Name
	Optional
	Description

	heatingZone0
	heatingZone
	false
	See clause 5.3.39. 

	heatingZone1
	heatingZone
	true
	See clause 5.3.39.

	heatingZone2
	heatingZone
	true
	See clause 5.3.39.

	heatingZone3
	heatingZone
	true
	See clause 5.3.39.

	heatingZone4
	heatingZone
	true
	See clause 5.3.39.

	heatingZone5
	heatingZone
	true
	See clause 5.3.39.


5.5.1.14 deviceDehumidifier
A dehumidifier is a device that is used to monitor or control the state of a dehumidifying appliance.
Table 5.5.14‑1: Modules of deviceDehumidifier Device model
	Module Instance Name
	Module Class Name
	Optional
	Description

	binarySwitch
	binarySwitch
	false
	See clause 5.3.12.

	relativeHumidity
	relativeHumidity
	true
	See clause 5.3.64.

	runState
	runState
	true
	See clause 5.3.67.

	dehumidifierJobMode
	dehumidifierJobMode
	true
	See clause 5.3.27.

	dehumidifierOperationMode
	operationMode
	true
	See clause 5.3.52.

This module instance is used to trigger dehumidifier operation.

	Timer
	timer
	true
	See clause 5.3.79.

	powerSave
	powerSave
	true
	See clause 5.3.58.


5.5.1.15 deviceDishWasher
A dish washer is a home appliance used to wash dishes. This information model provides capabilities to interact with specific functions and resources of a dish washer. 

Table 5.5.15‑1: Modules of deviceDishWasher Device model
	Module Instance Name
	Module Class Name
	Optional
	Description

	binarySwitch
	binarySwitch
	false
	See clause 5.3.12.

	runState
	runState
	true
	See clause 5.3.67.

	dishWasherJobMode
	dishWasherJobMode
	true
	See clause 5.3.28.


5.5.1.16 deviceDoor
A door is a device that is used to open and close a door.
Table 5.5.16‑1: Modules of deviceDoor model
	Module Instance Name
	Module Class Name
	Optional
	Description

	openLevel
	openLevel
	true
	See clause 5.3.51.

	doorlock
	lock
	true
	See clause 5.3.47.

	doorStatus
	doorStatus
	true
	See clause 5.3.28.


5.5.1.17 deviceDoorLock

A door lock is a device that can be used to lock, for example, a door.
Table 5.5.17‑1: Modules of deviceDoorLock Device model
	Module Instance Name
	Module Class Name
	Optional
	Description

	doorLock
	lock
	false
	See clause 5.3.47.

	doorStatus
	doorStatus
	true
	See clause 5.3.28.

	battery
	battery
	true
	See clause 5.3.10.


5.5.1.18 deviceElectricVehicleCharger

An electric vehicle charger is a device that is used for charging or discharging electric vehicles.

Table 5.5.18‑1: Modules of deviceElectricVehicleCharger Device model
	Module Instance Name
	Module Class Name
	Optional
	Description

	faultDetection
	faultDetection
	false
	See clause 5.3.33.

	binarySwitch
	binarySwitch
	false
	See clause 5.3.12.

	runState
	runState
	false
	See clause 5.3.67.

	battery
	battery
	false
	See clause 5.3.10.

	electricVehicleConnector
	electricVehicleConnector
	false
	See clause 5.3.30.


5.5.1.19 deviceFan
A fan is a device that is used to monitor or control the state of a fanning device.
Table 5.5.19‑1: Modules of deviceFan model
	Module Instance Name
	Module Class Name
	Optional
	Description

	binarySwitch
	binarySwitch
	false
	See clause 5.3.12.


5.5.1.20 deviceFoodProbe
A food probe is a device that is used to measure the internal temperature of food.
Table 5.5.20‑1: Modules of deviceFoodProbe model
	Module Instance Name
	Module Class Name
	Optional
	Description

	temperature
	temperature
	false
	See clause 5.3.76.


5.5.1.21 deviceFreezer
A freezer is a large container like a fridge in which the temperature is kept below freezing point, so that food can be storeed inside of it for long periods. This freezer information model provides capabilities to monitor freezer specific functions and resources.

Table 5.5.21‑1: Modules of deviceFreezer Device model
	Module Instance Name
	Module Class Name
	Optional
	Description

	temperature
	temperature
	false
	See clause 5.3.76.


5.5.1.22 deviceGlucosemeter
A glucometer is a device that can be used to monitor the blood glucose level.
Table 5.5.22‑1: Modules of deviceGlucoseMeter Device model
	Module Instance Name
	Module Class Name
	Optional
	Description

	glucometer
	glucometer
	false
	See clause 5.3.37.

	battery
	battery
	false
	See clause 5.3.10.


5.5.1.23 deviceHeartRateMonitor
A heart rate monitor is a device that can be used to monitor the heart rate.

Table 5.5.23‑1: Modules of deviceHeartRateMonitor Device model

	Module Instance Name
	Module Class Name
	Optional
	Description

	pulsemeter
	pulsemeter
	false
	See clause 5.3.60.

	battery
	battery
	false
	See clause 5.3.10.


5.5.1.24 deviceHomeCCTV
A home CCTV is a smart home appliance to provide monitoring capabilities when people stay way from their home or a room, or to monitor the environmental status of their home or room.
Table 5.5.24‑1: Modules of deviceHomeCCTV Device model
	Module Instance Name
	Module Class Name
	Optional
	Description

	binarySwitch
	binarySwitch
	false
	See clause 5.3.12.

	sessionDescription
	sessionDescription
	false
	See clause 5.3.69.

	playerControl
	playerControl
	true
	See clause 5.3.57.

	runState
	runState
	true
	See clause 5.3.67.

	motionSensor
	motionSensor
	true
	See clause 5.3.49.

	airQualitySensor
	airQualitySensor
	true
	See clause 5.3.6.

	ozoneMeter
	ozoneMeter
	true
	See clause 5.3.55.

	smokeSensor
	smokeSensor
	true
	See clause 5.3.71.

	acousticSensor
	acousticSensor
	true
	See clause 5.3.1.

	impactSensor
	impactSensor
	true
	See clause 5.3.42.

	faultDetection
	faultDetection
	true
	See clause 5.3.33.

	alarmSpeaker
	alarmSpeaker
	true
	See clause 5.3.7.


5.5.1.25 deviceHumidifier
A humidifier is a device that is used to monitor or control the state of a humidifying appliance.
Table 5.5.25‑1: Modules of Device model
	Module Instance Name
	Module Class Name
	Optional
	Description

	binarySwitch
	binarySwitch
	false
	See clause 5.3.12.


5.5.1.26 deviceKettle
Kettle is a device used to boil water. It may set a desired temperature for water and may keep water warm for a desired time.

Table 5.5.26‑1: Modules of deviceKettle Device model

	Module Instance Name
	Module Class Name
	Optional
	Description

	faultDetection
	faultDetection
	true
	See clause 5.3.33.

	runState
	runState
	true
	See clause 5.3.67.

	waterStatus
	liquidRemaining
	true
	See clause 5.3.46.

	boilingSwitch
	binarySwitch
	false
	See clause 5.3.12.

	temperature
	temperature
	true
	See clause 5.3.76.

	keepWarm
	keepWarm
	true
	See clause 5.3.43.


5.5.1.27 deviceLight

A light is a device that is used to control the state of an illumination appliance.
Table 5.5.27‑1: Modules of deviceLight Device model
	Module Instance Name
	Module Class Name
	Optional
	Description

	faultDetection
	faultDetection
	true
	See clause 5.3.33.

	binarySwitch
	binarySwitch
	false
	See clause 5.3.12.

	runState
	runState
	true
	See clause 5.3.67.

	colour
	colour
	true
	See clause 5.3.23.

	colourSaturation
	colourSaturation
	true
	See clause 5.3.24.

	brightness
	brightness
	true
	See clause 5.3.17.


5.5.1.28 deviceMicrogeneration

A microgeneration is a Home Energy Management System (HEMS) device that is used to generate energy. Examples of microgeneration devices are photovoltaics device or fuel cells.

Table 5.5.28‑1: Modules of deviceMicrogeneration Device model
	Module Instance Name
	Module Class Name
	Optional
	Description

	faultDetection
	faultDetection
	true
	See clause 5.3.33.

	binarySwitch
	binarySwitch
	true
	See clause 5.3.12.

	runState
	runState
	true
	See clause 5.3.67.

	energyGeneration
	energyGeneration
	false
	See clause 5.3.32.


5.5.1.29 deviceMultiFunctionPrinter
A Multi Function Printer (MFP) is an office machine which incorporates the functionality of multiple devices in one, so as to have a smaller footprint in home or office. A typical MFP may act as a combination of printer, scanner and more. This MFP information model provides capabilities to control and monitor MFP specific functions and resources.

Table 5.5.29‑1: Modules of deviceMultiFunctionPrinter Device model
	Module Instance Name
	Module Class Name
	Optional
	Description

	binarySwitch
	binarySwitch
	false
	See clause 5.3.12.

	printerRunState
	runState
	false
	See clause 5.3.67.

	scannerRunState
	runState
	true
	See clause 5.3.67.

	autoDocumentFeeder
	autoDocumentFeeder
	true
	See clause 5.3.9.

	printQueue
	printQueue
	true
	See clause 5.3.59.


5.5.1.30 deviceOutdoorLamp
An outdoor lamp is a smart home appliance to provide lights and information for outside of home with smart sensing capabilities such as ultraviolet sensing.
Table 5.5.30‑1: Modules of deviceOutdoorLamp Device model
	Module Instance Name
	Module Class Name
	Optional
	Description

	binarySwitch
	binarySwitch
	false
	See clause 5.3.12.

	brightness
	brightness
	true
	See clause 5.3.17.

	motionSensor
	motionSensor
	true
	See clause 5.3.49.

	airQualitySensor
	airQualitySensor
	true
	See clause 5.3.6.

	uvSensor
	uvSensor
	true
	See clause 5.3.81.

	timer
	timer
	true
	See clause 5.3.79.

The timer is used to set duration of giving lights from the moment of triggering by the “brightness” module or “motionSensor” module.

	faultDetection
	faultDetection
	true
	See clause 5.3.33.


5.5.1.31 deviceOven
An oven is a home appliance used to roast and heat food in a complete stove. This information model is applicable to different types of ovens: gas ovens, electrical ovens, steam ovens, microwave ovens, etc. This information model provides capabilities to interact with specific functions and resources of ovens. 

Table 5.5.31‑1: Modules of deviceOven Device model
	Module Instance Name
	Module Class Name
	Optional
	Description

	binarySwitch
	binarySwitch
	false
	See clause 5.3.12.

Depending on the manufacturer policies or regulations, the binary switch might only be used to turn the device off.

	runState
	runState
	true
	See clause 5.3.67.

	timer
	timer
	true
	See clause 5.3.79.

	temperature
	temperature
	false
	See clause 5.3.76.


5.5.1.32 devicePrinter
A printer is a device that is used to monitor or control the state of a printing appliance.
Table 5.5.32‑1: Modules of devicePrinter Device model
	Module Instance Name
	Module Class Name
	Optional
	Description

	binarySwitch
	binarySwitch
	false
	See clause 5.3.12.

	runState
	runState
	true
	See clause 5.3.67.

	printQueue
	printQueue
	true
	See clause 5.3.59.


5.5.1.33 devicePulseOximeter

A pulseoximeter is a device that can be used to monitor the blood characteristics.
Table 5.5.33‑1: Modules of devicePulseOxiMeter Device model
	Module Instance Name
	Module Class Name
	Optional
	Description

	pulsemeter
	pulsemeter
	true
	See clause 5.3.60.

When the “oximeter” module doesn’t exist, then the “pulsemeter” module is mandatory.

	oximeter
	oximeter
	true
	See clause 5.3.54.

When the “pulsemeter” module doesn’t exist, then the “oximeter” module is mandatory.

	battery
	battery
	false
	See clause 5.3.10.


5.5.1.34 deviceRefrigerator
A refrigerator is a home appliance used to store food at temperatures which are a few degrees above the freezing point of water. This information model provides capabilities to interact with specific functions and resource of refrigerators.

Table 5.5.34‑1: Modules of deviceRefrigerator Device model
	Module Instance Name
	Module Class Name
	Optional
	Description

	binarySwitch
	binarySwitch
	true
	See clause 5.3.12.

	powerSave
	powerSave
	true
	See clause 5.3.58.

	doorStatus
	doorStatus
	true
	See clause 5.3.28.

	frozenTemperature
	temperature
	true
	See clause 5.3.76.

	fridgeTemperature
	temperature
	false
	See clause 5.3.76.

	customTemperature
	temperature
	true
	See clause 5.3.76.

This module can be configured to fridge temperature or frozen temperature based on its usage by manufacturer

	refrigeration
	refrigeration
	true
	See clause 5.3.63.

	controlPanelLock
	lock
	true
	See clause 5.3.47.

	waterFilterInfo
	filterInfo
	true
	See clause 5.3.34.


5.5.1.35 deviceRobotCleaner
A robot cleaner is an autonomous robotic vacuum cleaner that has intelligent programming and a limited vacuum cleaning system. This robot cleaner information model provides capabilities to control and monitor robot cleaner specific functions and resources. 

Table 5.5.35‑1: Modules of deviceRobotCleaner Device model
	Module Instance Name
	Module Class Name
	Optional
	Description

	binarySwitch
	binarySwitch
	false
	See clause 5.3.12.

	runState
	runState
	true
	See clause 5.3.67.

	robotCleanerJobMode
	robotCleanerJobMode
	false
	See clause 5.3.66.

	robotCleanerOperationMode
	operationMode
	true
	See clause 5.3.52.

	battery
	battery
	true
	See clause 5.3.10.

	timer
	timer
	true
	See clause 5.3.79.


5.5.1.36 deviceScanner
A scanner is a device that optically scans images, printed text, handwriting or an object, and converts it to a digital image.
Table 5.5.36‑1: Modules of deviceScanner model
	Module Instance Name
	Module Class Name
	Optional
	Description

	binarySwitch
	binarySwitch
	false
	See clause 5.3.12.

	runState
	runState
	false
	See clause 5.3.67.

	autoDocumentFeeder
	autoDocumentFeeder
	true
	See clause 5.3.9.


5.5.1.37 deviceSecurityPanel
A security pannel is a device that can change the security mode of, for example, an alarm system.
Table 5.5.37‑1: Modules of deviceSecurityPanel model
	Module Instance Name
	Module Class Name
	Optional
	Description

	securityMode
	securityMode
	false
	See clause 5.3.68.


5.5.1.38 deviceSetTopBox
A set top box is a device that in general contains a TV tuner input and displays output to a TV.
Table 5.5.38‑1: Modules of deviceSetTopBox model
	Module Instance Name
	Module Class Name
	Optional
	Description

	binarySwitch
	binarySwitch
	false
	See clause 5.3.12.

	audioVolume
	audioVolume
	true
	See clause 5.3.8.

	Channel
	televisionChannel
	true
	See clause 5.3.75.

	mediaInput
	mediaSelect
	true
	See clause 5.3.48.

	mediaOutput
	mediaSelect
	true
	See clause 5.3.48.


5.5.1.39 deviceSmartElectricMeter

A smart electric meter is a metering device that is used to measure consumption data for electrictricity.

Table 5.5.39‑1: Modules of deviceSmartElectricMeter Device model
	Module Instance Name
	Module Class Name
	Optional
	Description

	faultDetection
	faultDetection
	true
	See clause 5.3.33.

	binarySwitch
	binarySwitch
	true
	See clause 5.3.12.

	runState
	runState
	true
	See clause 5.3.67.

	clock
	clock
	true
	See clause 5.3.18.

	energyConsumption
	energyConsumption
	false
	See clause 5.3.31.

	energyGeneration
	energyGeneration
	true
	See clause 5.3.32.


5.5.1.40 deviceSmartPlug
A smart plug is a device that can turn on and off a connected appliance.
Table 5.5.40‑1: Modules of deviceSmartPlug model
	Module Instance Name
	Module Class Name
	Optional
	Description

	binarySwitch
	binarySwitch
	true
	See clause 5.3.12.

	faultDetection
	faultDetection
	true
	See clause 5.3.33.

	remoteControlEnable
	remoteControlEnable
	true
	See clause 5.3.65.


Table 5.5.40‑2: Subdevice of deviceSmartPlug Device model
	Subdevice Instance Name
	Subdevice Name
	Optional
	Description

	powerOutlet0
	subDevicePowerOutlet
	false
	See clause 5.4.2.

	powerOutlet1
	subDevicePowerOutlet
	true
	See clause 5.4.2.

	powerOutlet2
	subDevicePowerOutlet
	true
	See clause 5.4.2.

	powerOutlet3
	subDevicePowerOutlet
	true
	See clause 5.4.2.

	powerOutlet4
	subDevicePowerOutlet
	true
	See clause 5.4.2.

	powerOutlet5
	subDevicePowerOutlet
	true
	See clause 5.4.2.


5.5.1.41 deviceSteamCloset
A deviceSteamCloset is a home appliance that de-wrinkles, sanitizes and dries to clean fabrics similar to a dry cleaner. This information model provides capabilities to interact with specific functions and resources of the steam closet. 

Table 5.5.41‑1: Modules of deviceSteamCloset Device model
	Module Instance Name
	Module Class Name
	Optional
	Description

	binarySwitch
	binarySwitch
	false
	See clause 5.3.12.

	runState
	runState
	true
	See clause 5.3.67.

	steamClosetJobMode
	steamClosetJobMode
	false
	See clause 5.3.74.

	steamClosetOperationMode
	operationMode
	true
	See clause 5.3.52.


5.5.1.42 deviceStorageBattery

A storage battery is a Home Energy Management System HEMS device that is used to provide the home with electrical energy. 

Table 5.5.42‑1​​: Modules of deviceStorageBattery Device model
	Module Instance Name
	Module Class Name
	Optional
	Description

	faultDetection
	faultDetection
	true
	See clause 5.3.33.

	binarySwitch
	binarySwitch
	true
	See clause 5.3.12.

	runState
	runState
	true
	See clause 5.3.67.

	battery
	battery
	false
	See clause 5.3.10.


5.5.1.43 deviceSwitch

A switch is a device that is used to control and monitor the state of power.
Table 5.5.43‑1: Modules of deviceSwitch Device model
	Module Instance Name
	Module Class Name
	Optional
	Description

	binarySwitch
	binarySwitch
	false
	See clause 5.3.12.


5.5.1.44 deviceTelevision
A television (TV) is a home appliance used to show audio and visual content such as broadcasting programs and network streaming. This TV information model provides capabilities to control and monitor TV specific resources.

Table 5.5.44‑1: Modules of deviceTelevision Device model
	Module Instance Name
	Module Class Name
	Optional
	Description

	binarySwitch
	binarySwitch
	false
	See clause 5.3.12.

	audioVolume
	audioVolume
	true
	See clause 5.3.8.

	televisionChannel
	televisionChannel
	true
	See clause 5.3.75.

	playerControl
	playerControl
	true
	See clause 5.3.57.

	mediaInput
	mediaSelect
	true
	See clause 5.3.48.

	mediaOutput
	mediaSelect
	true
	See clause 5.3.48.


5.5.1.45 deviceThermometer

A thermometer is a device that can be used to check, for example, the body or other temperatures.
Table 5.5.45‑1: Modules of deviceThermoMeter Device model
	Module Instance Name
	Module Class Name
	Optional
	Description

	temperature
	temperature
	false
	See clause 5.3.76.

	battery
	battery
	false
	See clause 5.3.10.


5.5.1.46 deviceThermostat

A thermostat is used to control the ambient temperature of rooms within, for example, a house. This information model provides capabilities to interact with specific functions of thermostats.

Table 5.5.46‑1: Modules of deviceThermostat Device model

	Module Instance Name
	Module Class Name
	Optional
	Description

	runState
	runState
	true
	See clause 5.3.67.
The possible values of the “supportedModes” datapoint for the thermostat device are included in clause 5.6.21.

	timer
	timer
	true
	See clause 5.3.79.

	temperature
	temperature
	false
	See clause 5.3.76.


5.5.1.47 deviceWaterHeater

A water heater is a device that is used to provide hot water through home facilities.

Table 5.5.47‑1: Modules of deviceWaterHeater Device model
	Module Instance Name
	Module Class Name
	Optional
	Description

	faultDetection
	faultDetection
	true
	See clause 5.3.33.

	binarySwitch
	binarySwitch
	false
	See clause 5.3.12.

	runState
	runState
	true
	See clause 5.3.67.

	clock
	clock
	true
	See clause 5.3.18.

	boiler
	boiler
	true
	See clause 5.3.15.

	hotWaterSupply
	hotWaterSupply
	true
	See clause 5.3.41.


5.5.1.48 deviceWaterValve
A water valve is a device that is used to turn the water supply ON or OFF remotely.
Table 5.5.48‑1: Modules of waterValve model
	Module Instance Name
	Module Class Name
	Optional
	Description

	openLevel
	openLevel
	false
	See clause 5.3.51.


5.5.1.49 deviceWeightScaleAndBodyCompositionAnalyser
A weight scale and body composition analyser is a device that can be used to monitor the weight and body composition.

Table 5.5.49‑1: Modules of deviceWeightScaleAndBodyCompositionAnalyser Device model

	Module Instance Name
	Module Class Name
	Optional
	Description

	weight
	weight
	false
	See clause 5.3.84.

	bodyCompositionAnalyser
	bodyCompositionAnalyser
	false
	See clause 5.3.14.

	bioElectricalImpedanceAnalysis
	bioElectricalImpedanceAnalysis
	false
	See clause 5.3.13.

	battery
	battery
	false
	See clause 5.3.10.


5.5.1.50 deviceWindowShade

The window shade is an appliance that provides the ability to cover windows. This device type includes but not limited to roller shades, drapes, and tilt-only blinds.

Table 5.5.50‑1: Modules of deviceWindowShade Device model
	Module Instance Name
	Module Class Name
	Optional
	Description

	openLevel
	openLevel
	false
	See clause 5.3.51.

	battery
	battery
	true
	See clause 5.3.10.


5.5.2 City domain
5.5.3 Health domain
5.5.4 Home domain

5.5.5 Industry domain
5.5.6 Vehicular domain

-----------------------End of change 1---------------------------------------------

CHECK LIST

· Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?
· Does this CR contain changes related to only one particular issue/problem?
· Have any mirror CRs been posted?
· Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?
· Are all pictures editable?
· Have you checked the spelling and grammar?
· Have you used change bars for all modifications?
· Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)
· Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?
© 2018 oneM2M Partners
                                                                                                   Page 1 (of 50)



[image: image2.png]