PRO-2014-0144-Device_triggering_for_3GPP 

	INPUT CONTRIBUTION

	Group Name:*
	WG3

	Title:*
	Device Triggering for 3GPP

	Source:*
	NEC, NTT Docomo

	Contact:
	Barbara Pareglio (Barbara.Pareglio@neclab.eu) , Tetsuo Inoue (t-inoue@fp.jp.nec.com) , Syed Husain (shusain2016@gmail.com) 

	Date:*
	2014-03-

	Abstract:*
	The contribution provide a mapping of the commands for Device Triggering on Tsp interface of 3GPP

	Agenda Item:*
	Input Contribution

	Work item(s):
	

	Document(s) 

Impacted*
	TS-0004 v0.3.0

	Intended purpose of

document:*
	 FORMCHECKBOX 
 Decision

 FORMCHECKBOX 
 Discussion

 Information

 Other <specify>

	Decision requested or recommendation:*
	To agree to include the new clauses in the TS.


oneM2M Notice
The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.
1
Introduction

Right now the template of the TS does not include any clause that should contain underlying network specific interfaces and how to map oneM2M information into features/interfaces from the underlying network.

2
Proposal

This contribution proposes to add new clauses that cover only Device Triggering feature for 3GPP network by means of Tsp.

-----------------------Start of change 1-------------------------------------------
X.Y
Device Triggering
X.Y.1 Providing Device Triggering service by means of 3GPP networks
3GPP Underlying Network has defined a dedicated interface for requesting device triggering. The normative references for applicable interfaces are as follows: 3GPP [TS 23.682]. The specification for the interface Tsp is described in [TS 29.368]. Tsp interface uses Diameter Base Protocol as specified in [IETF RFC 3588], in order to use such an interface the CSE shall act as a Diameter client as described in [RFC 6733].
X.Y.1.1 Device Action Request command
When a CSE needs to issue a device triggering request from MTC-IWF, the CSE shall send a Device-Action-Request (DAR) command (see [TS 29.368]). Following tables provide an overview of the parameters and the parameter values needed for the DAR command.
Table x.y.1.1-1: AVPs in Device Action Request (DAR) command
	AVP name
	AVP description
	Data Type
	AVP Code
	Value

	Session-Id
	It is used to identify a specific session (see [RFC 3588])
	UTF8String
	263
	CSE implementation specific

	Auth-Application-Id
	This parameter transport the Tsp application identifier as specified in [TS 29.368 caluse 6.1.3]
	Enumerated
	274
	16777309  

	Auth-Session-State
	The Tsp session is terminated after each request/answer pair and according to [TS 29.368] the parameter shall indicate that the session state shall not be maintained
	Enumerated
	277
	NO_STATE_MAINTAINED (see [TS 29.368])

	Origin-Host
	It identifies the endpoint that originated the Diameter message (see [RFC 3588])
	DiamIdent  
	264
	CSE implementation specific

	Origin-Realm
	It identifies the Realm of the originator of  theDiameter message (see [RFC 3588])
	DiamIdent
	296
	CSE implementation specific

	Destination-Realm
	It identifies the realm the message is to be routed to. (see [RFC 3588])
	DiamIdent  
	283
	Domain Identifier part from M2M-Ext-ID



	Destination-Host
	The absence of the Destination-Host causes the message to be sent to any Diameter server supporting the application within the specified realm. (see [RFC 3588])
	DiamIdent  
	293
	NULL

	Origin-State-Id
	It is an increasing value that is advanced whenever a Diameter entity restarts with loss of previous state, for example upon reboot (see [RFC 3588])
	Unsigned32
	278
	CSE implementation specific

	Device-Action
	It contains the specific action for the device (see [TS 29.368])
	Grouped    
	3001
	Specific format described in table x.y.1.1-2

	Proxy-Info
	It’s a grouped AVP which provides the proxy state and host (see [RFC 3588])
	Grouped    
	284
	CSE implementation specific

	Route-Record
	The  identity added in this AVP is be the same as the one received in  the Origin-Host (see [RFC 3588])
	DiamIdent  
	282
	Not applicable


Table x.y.1.1-2: AVPs in Device Action AVP (Grouped AVP) 

	AVP name
	AVP description
	Data Type
	AVP Code
	Value

	External-Id
	It is the identifier of the device as specified in [TS 29.336]
	UTF8String
	3111
	M2M-Ext-ID

	MSISDN
	It is the identifier of the device as specified in [TS 29.329]
	OctetString
	701
	M2M-Ext-ID

	SCS-Identifier
	It identifies the CSE issuing the request (see [TS 29.368])
	OctetString
	3104
	CSE-ID

	Reference-number
	It uniquely identifies a transaction (see [TS 29.368])
	Unsigned32
	3007
	CSE implementation specific 

Editor’s note: possibility to maybe use M2M-Request-ID is FFS

	Action-Type
	It informs the MTC-IWF of the required action
	Enumerated
	3005
	1 (Device Trigger Request)

	Trigger-Data
	
	Grouped    
	3003
	See table x.y.1.1-3

	Validity-Time
	The validity time in seconds for the specific action requested. (see [TS 29.368])
	Unsigned32
	448
	CSE implementation specific


Table x.y.1.1-3: AVPs in Trigger Data AVP (Grouped AVP) 

	AVP name
	AVP description
	Data Type
	AVP Code
	Value

	Payload
	It contains the payload to be transferred to the addressed device. (see [TS 29.368])
	OctetString
	3004
	NULL
Editor’s Note: in the current TS-0001 there is no need to use payload

	Priority-Indication
	It identifies priority of the device trigger. (see [TS 29.368])
	Enumerated
	3006
	CSE implementation specific (possible values are 0 or 1)

	Application-Port-Identifier
	It uniquely identifies the triggering entity addressed in the device. (see [TS 29.368])
	Unsigned32
	3010
	Trigger-Recipient-ID


X.Y.1.2 Device Action Answer command
As a result of device triggering request to MTC-IWF, the CSE receives a Device-Action-Answer (DAA) command (see [TS 29.368]). The relevant parameters for a CSE are the following ones:
· Result-Code AVP: see [RFC 3588] for detail.
· Session-Id AVP: see [RFC 3588] for detail.
· Other AVPs for Diameter Base Protocol: see [RFC 3588] for detail.
Following AVPs are set in Device-Notification AVP (Grouped AVP):
· Either MSISDN or External-Id: the CSE shall check that the identifier of the target CSE is correct.
· SCS-Identifier: the CSE shall check that the value corresponds to its CSE-ID.
· Reference-Number: the CSE can use this value for correlating the DAA with the corresponding Device-Action-Request command issued previously by the CSE.
· Action-Type: the CSE shall check that the value is se to 1 (Trigger Request) as same value of DAR command;
· Request-Status: see table x.y.1.2-1 for the list of possible value.
· 
Table x.y.1.2-1: Result Status 

	Result-Satatus value 
	name
	Parameter description

	0
	SUCCESS
	This value indicates that device action requested is confirmed.

	201
	TEMPORARYERROR
	See [TS 29.368] for detail.

	101
	INVALIDPAYLOAD
	See [TS 29.368] for detail.

	102
	INVEXTID
	See [TS 29.368] for detail.

	103
	INVSCSID
	See [TS 29.368] for detail.

	104
	INVPERIOD
	See [TS 29.368] for detail.

	105
	NOTAUTHORIZED
	See [TS 29.368] for detail.

	106
	SERVICEUNAVAILABLE
	See [TS 29.368] for detail.

	107
	PERMANENTERROR
	See [TS 29.368] for detail.

	108
	QUOTAEXCEEDED
	See [TS 29.368] for detail.

	109
	RATEEXCEEDED
	See [TS 29.368] for detail.


X.Y.1.3 Device Notification Request command
As a report of the result for device triggering delivery by 3GPP network, the CSE receives a Device-Notification-Request (DNR) command (see [TS 29.368]). The relevant parameters for a CSE are the following ones:

· Result-Code: see [RFC 3588] for detail.
· Session-Id AVP: see [RFC 3588] for detail.
· Other AVPs for Diameter Base Protocol: see [RFC 3588] for detail.
Following AVPs are set in Device-Notification AVP (Grouped AVP):
· Either MSISDN or External-Id: the CSE shall check that the identifier of the target CSE is correct.
· SCS-Identifier: the CSE shall check that the value corresponds to its CSE-ID.
· Reference-Number: the CSE can use this value for correlating the DNR with the corresponding Device-Action-Request command issued previously by the CSE.

· Action-Type: the CSE shall check that the value is se to 2 (Delivery Report);
· Delivery-Outcome: see table x.y.1.2-2 for the list of possible value

Table x.y.1.3-1: Delivery Outcome 

	Delivery-Outcome value 
	name
	Parameter description

	0
	SUCCESS
	This value indicates that the device action request was successfully completed.

	1
	EXPIRED
	See [TS 29.368] for detail.

	2
	TEMPORARYERROR
	See [TS 29.368] for detail.

	3
	UNDELIVERABLE
	See [TS 29.368] for detail.

	4
	UNCONFIRMED
	See [TS 29.368] for detail.


X.Y.1.4 Device Notification Answer command
As a result of device notification request to MTC-IWF, the CSE sends a Device-Notification-Answer (DNA) command (see [TS 29.368]). The relevant parameters for a CSE are the following ones:

· Auth-Application-ID: set to 16777309  as indicated in in [TS 29.368])
· Auth-Session-State: set to NO_STATE_MAINTAINED as indicated in [TS 29.368]
· Other AVPs for Diameter Base Protocol: see [RFC 3588] for detail, the value of such AVP is CSE implementation specific.
-----------------------End of change 1---------------------------------------------

�3gpp does not specify  for MSISDN what it is.


�CSE implementation specific for MSISDN case.





© 2014 oneM2M Partners

Page 2 (of 6)

