	Doc# PRO-2015-0649-CoAP_New_Options_Descriptions 
Input Contribution
	[image: image1.png]






	INPUT CONTRIBUTION

	Group Name:*
	PRO

	Title:*
	CoAP New Options Descriptions

	Source:*
	LG Electronics

	Contact:
	Seongyoon Kim, LG Electronics, seongyoon.kim@lge.com

	Date:*
	2014-01-15

	Abstract:*
	CoAP New Options Descriptions

	Agenda Item:*
	PRO

	Work item(s):
	WI 0009

	Document(s) 

Impacted*
	Technical Specification – CoAP Protocol Binding

	Intended purpose of

document:*
	 FORMCHECKBOX 
 Decision

 FORMCHECKBOX 
 Discussion

 Information

 Other <specify>

	Decision requested or recommendation:*
	Approve the proposed change and integrate it into the TS document


oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.
Rationale

This contribution defines new CoAP options used to bind oneM2M parameters.

· Mandatory parameters are marked as Critical; Optional parameters are marked as Elective.
· Since we don’t use CoAP proxy function, Unsafe & NoCacheKey are not used
========================Start of 1st Change==========================
6.2.4
New Options Definitions
This clause describes new CoAP options used for binding several oneM2M request/response parameters. The table 6.2.4-1 contains definitions of the new CoAP options and sub-clauses specify oneM2M parameter mapping with the newly defined CoAP options in table 6.2.4-1.
Table 6.2.4-1: New Options Definitions

	No
	C
	U
	N
	R
	Name
	Format
	Length
	Default

	256
	X
	
	
	
	oneM2M-RI
	string
	0-255
	(None)

	257
	
	
	
	
	oneM2M-OT
	string
	0-255
	(None)

	258
	
	
	
	
	oneM2M-RET
	string
	0-255
	(None)

	259
	
	
	
	
	oneM2M-ECT
	uint
	1
	1

	260
	
	
	
	
	oneM2M-ECN
	uint
	1
	1

	261
	
	
	
	
	oneM2M-RSC
	uint
	2
	(None)


Note: table 6.2.4-1 follows the template used in section 5.10 Option Definitions of CoAP specification [1].
6.2.4.1
oneM2M-RI
The oneM2M-RI option shall be mapped to the request identifier parameter.
6.2.4.2
Originating Timestamp

The oneM2M-OT option shall be mapped to the originating timestamp parameter.
6.2.4.3
Result Expiration Timestamp

The oneM2M-RET option shall be mapped to the result expiration timestamp parameter.


6.2.4.4
Event Category

The oneM2M-ECT option shall be mapped to the event category parameter if the eventCatType element is used.
6.2.4.5
Event Category

The oneM2M-ECN option shall be mapped to the event category parameter if the eventCatNum element is used.
6.2.4.6
Response Status Code

The oneM2M-RSC option shall be mapped to the response status code parameter.
========================End of 1st Change==========================
© 2013 oneM2M Partners
                                                                                                     Page 3 (of 3)



[image: image1.png]