	Doc# PRO-2015-0649R02-CoAP_New_Options_Descriptions 
Input Contribution
	[image: image1.png]






	INPUT CONTRIBUTION

	Group Name:*
	PRO

	Title:*
	CoAP New Options Descriptions

	Source:*
	LG Electronics

	Contact:
	Seongyoon Kim, LG Electronics, seongyoon.kim@lge.com

	Date:*
	2014-01-15

	Abstract:*
	CoAP New Options Descriptions

	Agenda Item:*
	PRO

	Work item(s):
	WI 0009

	Document(s) 

Impacted*
	Technical Specification – CoAP Protocol Binding

	Intended purpose of

document:*
	 FORMCHECKBOX 
 Decision

 FORMCHECKBOX 
 Discussion

 Information

 Other <specify>

	Decision requested or recommendation:*
	Approve the proposed change and integrate it into the TS document


oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.
Rationale

This contribution defines new CoAP options used to bind oneM2M parameters.

· Mandatory parameters are marked as Critical; Optional parameters are marked as Elective.
· Since we don’t use CoAP proxy function, Unsafe & NoCacheKey are not used

R01: 
1. Move From, Request Expiration Timestamp , Operation Execution Time, responseType element of Response Type , Name, Group Request Identifier (Variable Values) from query string to CoAP Option.

2. Combine Event Category Type and Event Category Name into Event Category aligned with TS-0004.

3. Add Resource Type parameter which was missing.

4. Move Filter Criteria to Query String

========================Start of 1st Change==========================
6.2.4
New Options Definitions
This clause describes new CoAP Options used for binding several oneM2M request/response parameters. The table 6.2.4-1 contains definitions of the new CoAP Options and sub-clauses specify oneM2M parameter mapping with the newly defined CoAP Options in table 6.2.4-1.
Table 6.2.4-1: New Options Definitions

	No
	C
	U
	N
	R
	Name
	Format
	Length
	Default

	256
	X
	
	
	
	oneM2M-FR
	string
	0-255
	(None)

	257
	X
	
	
	
	oneM2M-RQI
	string
	0-255
	(None)

	258
	
	
	
	
	oneM2M-NM
	string
	0-255
	(None)

	259
	
	
	
	
	oneM2M-OT
	string
	15
	(None)

	260
	
	
	
	
	oneM2M-RQET
	string
	15
	(None)

	261
	
	
	
	
	oneM2M-RSET
	string
	15
	(None)

	262
	
	
	
	
	oneM2M-OET
	string
	15
	(None)

	263
	
	
	
	
	oneM2M-RTURI
	string
	0-255
	(None)

	264
	
	
	
	
	oneM2M-EC
	uint
	1
	1

	
	
	
	
	
	
	
	
	

	265
	
	
	
	
	oneM2M-RSC
	uint
	2
	(None)

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	266
	
	
	
	
	oneM2M-GID
	string
	0-255
	(None)


Note: table 6.2.4-1 follows the template used in section 5.10 Option Definitions of CoAP specification [1].
6.2.4.1
From
The From parameter shall be mapped to the oneM2M-FR Option.
6.2.4.2
Request Identifier
The Request Identifier parameter shall be mapped to the oneM2M-RQI Option.
6.2.4.3
Name
The Name parameter shall be mapped to the oneM2M-NM Option.
6.2.4.4
Originating Timestamp

The Originating Timestamp parameter shall be mapped to the oneM2M-OT Option.
6.2.4.5
Request Expiration Timestamp

The Request Expiration Timestamp parameter shall be mapped to the oneM2M-RQET Option.
6.2.4.6
Result Expiration Timestamp
The Result Expiration Timestamp parameter shall be mapped to the oneM2M-RSET Option.


6.2.4.7
Operation Execution Time
The Operation Execution Time parameter shall be mapped to the oneM2M-OET Option.
6.2.4.8
notificationURI of Response Type
The notificationURI element of Response Type parameter shall be mapped to the oneM2M-RTURI Option.
6.2.4.9
Event Category

The Event Category parameter shall be mapped to the oneM2M-EC Option.


6.2.4.10
Response Status Code

The Response Status Code parameter shall be mapped to the oneM2M-RSC Option.






6.2.4.11
Group Request Identifier
The Group Request Identifier parameter shall be mapped to the oneM2M-GID Option.
========================End of 1st Change==========================
========================Start of 1st Change==========================
6.2.2.3
URI Options
This clause describes how to configure CoAP Uri-Host, Uri-Port, Uri-Path, and Uri-Query Options.

Host and port part of the address specified in pointOfAccess attribute of <remoteCSE> resource shall be mapped to Uri-Host and Uri-Port respectively.

To parameter shall be mapped to Uri-Path.

Resource Type, responseType element of Response Type, Result Persistence, parameters of Filter Criteria, Delivery Aggregation, Result Content, and Discovery Result Type parameters shall be carried in Uri-Query Option in a short name form as specified in clause 8.2.2 [2].
========================End of 1st Change==========================
© 2013 oneM2M Partners
                                                                                                     Page 5 (of 5)



[image: image1.png]