	Doc# PRO-2015-0679-CR_HTTP_AE_CSE_registration.doc
Change Request
	[image: image1.png]






	


	CHANGE REQUEST

	Meeting:*
	PRO 14.3

	Source:*
	FUJITSU

	Date:*
	2015-01-19

	Contact:*
	Shingo Fujimoto, FUJITSU, Shingo_fujimoto@jp.fujitsu.com

	Reason for Change/s:*
	While registrar CSE will behave as HTTP-proxy, CSE must provide address to accept AE/CSE registration in offline manner.
Additionaly, having dedicated sub-clause for Request-Line and Response-Line looks strange and suggested to merge them as part of Introduction texts.

	CR  against:  Release*
	1.0 (DRAFT)

	CR  against:  TS/TR*
	TS-0009-V0.6.0

	Clauses/Sub Clauses*
	

	Type of change: *
	 FORMCHECKBOX 
 Editorial change
 FORMCHECKBOX 
 Bug Fix or Correction
 FORMCHECKBOX 
 Change/correction to existing feature or functionality
 New feature or functionality

	Post Freeze checking:*
	This CR contains only essential changes and corrections

                           YES  FORMCHECKBOX 
             NO  FORMCHECKBOX 



oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.
Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.
Follow the drafting rules.
All pictures must be editable.
Check spelling and grammar to the extent practicable.
Use Change bars for modifications.
The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete sections need not show surrounding clauses as long as the proposed section number clearly shows where the new section is proposed to be located.
Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.
When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text. 
Introduction
Relationship between TS-0001 should be explained in overview of HTTP Binding.
-----------------------Start of change 1-------------------------------------------
5
Overview of HTTP Binding

This clause describes what oneM2M primitive parameters can be mapped to HTTP request/response messages.

5.1

Introduction

The present specification assumes AE has the capability of HTTP Client, and CSE has the capability of both HTTP Client and Server.

Single request primitive will be mapped to single HTTP request message, and single response primitive will be mapped to single HTTP response message. HTTP-Version of the HTTP transport shall be HTTP 1.1[1]. 
An HTTP request message consists of Request-Line, headers and message-body. An HTTP response message consists of Status-Line, headers and message-body [4]. This clause describes how oneM2M request/response primitives are mapped to HTTP messages at a high level. Corresponding details of each sub-clause are specified in clause 6.


The Method in HTTP-Request message is mapped to the oneM2M operation parameter, and 
the HTTP Request-URI is derived from the oneM2M to parameter, including a query string which carries specific primitive parameters. 




The Status-Code and Reason-Phrase in HTTP-response meeesage are derived from the oneM2M responseStatusCode parameter of the response primitive.
The Registrar CSE shall behave as ‘proxy server’(see[1]).
The Registrar may provide AE and/or CSE the URI with credential to allow issuing Registration request targeted to the Registrar. 
-----------------------End of change 1---------------------------------------------

CHECK LIST

· Does this change request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?
· Does this CR contain changes related to only one particular issue/problem?
· Does this change request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable?
· Does this change request follow the drafting rules?
· Are all pictures editable?
· Have you checked the spelling and grammar?
· Have you used change bars for all modifications?
· Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete sections need not show surrounding clauses as long as the proposed section number clearly shows where the new section is proposed to be located.)
· Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?
© 2015 oneM2M Partners
                                                                                                   Page 3 (of 3)



[image: image1.png]