ETSI TS 118 104 V1.1.0 (2016-03)

oneM2M; 
Service Layer Core Protocol Specification

(oneM2M TS-0004 version 1.6.0 Release 1)

<

Technical Specification

Reference

RTS/oneM2M-000004v110

Keywords

IoT, M2M, protocol

ETSI

650 Route des Lucioles

F-06921 Sophia Antipolis Cedex - FRANCE

Tel.: +33 4 92 94 42 00   Fax: +33 4 93 65 47 16

Siret N° 348 623 562 00017 - NAF 742 C

Association à but non lucratif enregistrée à la

Sous-Préfecture de Grasse (06) N° 7803/88

Important notice

The present document can be downloaded from:
http://www.etsi.org/standards-search

The present document may be made available in electronic versions and/or in print. The content of any electronic and/or print versions of the present document shall not be modified without the prior written authorization of ETSI. In case of any existing or perceived difference in contents between such versions and/or in print, the only prevailing document is the print of the Portable Document Format (PDF) version kept on a specific network drive within ETSI Secretariat.

Users of the present document should be aware that the document may be subject to revision or change of status. Information on the current status of this and other ETSI documents is available at https://portal.etsi.org/TB/ETSIDeliverableStatus.aspx

If you find errors in the present document, please send your comment to one of the following services:
https://portal.etsi.org/People/CommiteeSupportStaff.aspx

Copyright Notification

No part may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm except as authorized by written permission of ETSI.
The content of the PDF version shall not be modified without the written authorization of ETSI.
The copyright and the foregoing restriction extend to reproduction in all media.

© European Telecommunications Standards Institute 2016.

All rights reserved.

DECTTM, PLUGTESTSTM, UMTSTM and the ETSI logo are Trade Marks of ETSI registered for the benefit of its Members.
3GPPTM and LTE™ are Trade Marks of ETSI registered for the benefit of its Members and
of the 3GPP Organizational Partners.
GSM® and the GSM logo are Trade Marks registered and owned by the GSM Association.

Contents

Intellectual Property Rights	14

Foreword	14

1	Scope	15

2	References	15

2.1	Normative references	15

2.2	Informative references	16

3	Definitions and abbreviations	17

3.1	Definitions	17

3.2	Abbreviations	17

4	Conventions	18

5	Protocol design principles and requirements	19

5.1	General introduction	19

5.2	Introduction	19

5.2.0	General	19

5.2.1	Interfaces to the underlying networks	19

5.3	API design guidelines	20

5.4	Primitives	20

5.4.1	Introduction	20

5.4.2	Primitives modelling	21

5.4.3	Primitive principles	22

5.4.4	Serialization of primitives	22

5.5	Design principles	22

5.5.1	Introduction	22

5.5.2	Extensibility	22

5.5.3	Scalability	22

5.5.4	Fault tolerance and robustness	23

5.5.5	Efficiency	23

5.5.6	Inter-operability	23

5.5.7	Self-operation and self-management	23

6	oneM2M protocols/API overview	23

6.1	Introduction	23

6.2	Addressing	24

6.2.1	Introduction	24

6.2.2	Summary of oneM2M Identifiers	24

6.2.3	oneM2M Entity Addressing	25

6.2.4	oneM2M Resource Addressing	26

6.3	Common data types	27

6.3.1	Introduction	27

6.3.2	Simple data types incorporated from XML schema	27

6.3.3	oneM2M simple data types	29

6.3.4	oneM2M enumerated data types	32

6.3.4.1	Introduction	32

6.3.4.2	Enumeration type definitions	32

6.3.4.2.1	m2m:resourceType	32

6.3.4.2.2	m2m:cseTypeID	33

6.3.4.2.3	m2m:locationSource	33

6.3.4.2.4	m2m:stdEventCats	33

6.3.4.2.5	m2m:operation	33

6.3.4.2.6	m2m:responseType	33

6.3.4.2.7	m2m:resultContent	34

6.3.4.2.8	m2m:discResType	34

6.3.4.2.9	m2m:responseStatusCode	34

6.3.4.2.10	m2m:requestStatus	34

6.3.4.2.11	m2m:memberType	35

6.3.4.2.12	m2m:consistencyStrategy	35

6.3.4.2.13	m2m:cmdType	35

6.3.4.2.14	m2m:execModeType	36

6.3.4.2.15	m2m:execStatusType	36

6.3.4.2.16	m2m:execResultType	36

6.3.4.2.17	m2m:pendingNotification	37

6.3.4.2.18	m2m:notificationContentType	37

6.3.4.2.19	m2m:notificationEventType	37

6.3.4.2.20	m2m:status	38

6.3.4.2.21	m2m:batteryStatus	38

6.3.4.2.22	m2m:mgmtDefinition	38

6.3.4.2.23	m2m:logTypeId	39

6.3.4.2.24	m2m:logStatus	39

6.3.4.2.25	m2m:eventType	39

6.3.4.2.26	m2m:statsRuleStatusType	40

6.3.4.2.27	m2m:statModelType	40

6.3.4.2.28	m2m:encodingType	40

6.3.4.2.29	m2m:accessControlOperations	40

6.3.4.2.30	m2m:SRole-ID	41

6.3.4.2.31	m2m:filterUsage	41

6.3.5	Complex data types	41

6.3.5.1	Introduction	41

6.3.5.2	m2m:deliveryMetaData	41

6.3.5.3	m2m:aggregatedRequest	41

6.3.5.4	m2m:metaInformation	42

6.3.5.5	m2m:primitiveContent	42

6.3.5.6	m2m:batchNotify	42

6.3.5.7	m2m:eventNotificationCriteria	42

6.3.5.8	m2m:filterCriteria	43

6.3.5.9	m2m:attribute	43

6.3.5.10	(void)	43

6.3.5.11	m2m:scheduleEntries	43

6.3.5.12	m2m:aggregatedNotification	44

6.3.5.13	m2m:notification	44

6.3.5.14	m2m:actionStatus	45

6.3.5.15	m2m:anyArgType	45

6.3.5.16	m2m:resetArgsType	45

6.3.5.17	m2m:rebootArgsType	45

6.3.5.18	m2m:uploadArgsTypes	45

6.3.5.19	m2m:downloadArgsType	46

6.3.5.20	m2m:softwareInstallArgsType	46

6.3.5.21	m2m:softwareUpdateArgsType	46

6.3.5.22	m2m:softwareUninstallArgsType	46

6.3.5.23	m2m:execReqArgsListType	47

6.3.5.24	m2m:mgmtLinkRef	47

6.3.5.25	m2m:resourceWrapper	47

6.3.5.26	m2m:setOfAcrs	48

6.3.5.27	m2m:accessControlRule	48

6.3.5.28	m2m:locationRegion	49

6.3.5.29	m2m:childResourceRef	49

6.3.5.30	m2m:responseTypeInfo	49

6.3.5.31	m2m:rateLimit	49

6.3.5.32	m2m:operationResult	50

6.3.5.33	m2m:aggregatedResponse	50

6.3.6	Universal and Common attributes	50

6.4	Message parameter data types	53

6.4.1	Request primitive parameter data types	53

6.4.2	Response primitive parameter data types	54

6.5	Resource data types	54

6.5.1	Description	54

6.5.2	resource	55

6.5.2.1	Description	55

6.5.2.2	Reference	55

6.5.2.3	Usage	55

6.5.3	regularResource	55

6.5.3.1	Description	55

6.5.3.2	Reference	55

6.5.3.3	Usage	55

6.5.4	announceableResource	56

6.5.4.1	Description	56

6.5.4.2	Reference	56

6.5.4.3	Usage	56

6.5.5	announcedResource	56

6.5.5.1	Description	56

6.5.5.2	Reference	56

6.5.5.3	Usage	56

6.5.6	announceableSubordinateResource	56

6.5.6.1	Description	56

6.5.6.2	Reference	56

6.5.6.3	Usage	56

6.5.7	announcedSubordinateResource	57

6.5.7.1	Description	57

6.5.7.2	Reference	57

6.5.7.3	Usage	57

6.6	Response status codes	57

6.6.1	Introduction	57

6.6.2	RSC framework overview	57

6.6.3	Definition of Response Status Codes	57

6.6.3.1	Overview	57

6.6.3.2	Informational response class	57

6.6.3.3	Successful response class	57

6.6.3.4	Redirection response class	58

6.6.3.5	Originator Error response class	58

6.6.3.6	Receiver Error response class	58

6.6.3.7	Network System Error response class	58

6.7	oneM2M specific MIME media types	59

6.8	Virtual Resources	60

7	oneM2M procedures	60

7.1	Introduction	60

7.2	Primitive format and generic procedure	60

7.2.1	Primitive format	60

7.2.1.1	Request primitive format	60

7.2.1.2	Response primitive format	61

7.2.2	Description of generic procedures	62

7.2.2.1	Generic resource request procedure for originator	62

7.2.2.2	Generic request procedure for receiver	64

7.3	Common operations	68

7.3.1	Originator actions	68

7.3.1.1	Compose request primitive	68

7.3.1.2	Send a request to the receiver CSE	69

7.3.1.3	Wait for response primitive	69

7.3.1.4	Retrieve the <request> resource	69

7.3.2	Receiver CSE actions	69

7.3.2.1	Check the validity of received request primitive	69

7.3.2.2	Create <request> resource locally	70

7.3.2.3	Create a success response (acknowledgement)	71

7.3.2.4	Send response primitive (acknowledgement)	72

7.3.2.5	Update <request> resource	72

7.3.2.6	Forwarding	72

7.3.2.7	Check Service Subscription Profile	72

7.3.3	Hosting CSE actions	73

7.3.3.1	Check the supported resource types	73

7.3.3.2	Check existence of the addressed resource	73

7.3.3.3	Check validity of resource representation for CREATE	73

7.3.3.4	Check validity of resource representation for UPDATE	74

7.3.3.5	Create the resource	74

7.3.3.6	Retrieve the resource	75

7.3.3.7	Update the resource	75

7.3.3.8	Delete the resource	76

7.3.3.9	Notify re-targeting	76

7.3.3.10	Announce the resource or attribute	77

7.3.3.11	De-announce the resource or attribute	78

7.3.3.12	Create a success response	79

7.3.3.13	Create an error response	79

7.3.3.14	Resource discovery procedure	79

7.3.3.15	Check authorization of the originator	80

7.3.3.16	Send response primitive	80

7.3.3.17	Using Filter Criteria for identification of target resources	80

7.3.3.17.0	Introduction	80

7.3.3.17.1	Conditions on the creationTime attribute	81

7.3.3.17.2	Conditions on the lastModifiedTime attribute	81

7.3.3.17.3	Conditions on stateTag attribute	82

7.3.3.17.4	Conditions on expirationTime attribute	82

7.3.3.17.5	Conditions on labels attribute	82

7.3.3.17.6	Conditions on resourceType attribute	82

7.3.3.17.7	Conditions on contentSize attribute	83

7.3.3.17.8	Conditions on typeOfContent of contentInfo attribute	83

7.3.3.17.9	Conditions on attribute name and value pairs	83

7.3.3.17.10	Constraint on number of retrieved resources by limit element	83

7.3.3.17.11	Filter Usage request parameter	83

7.3.4	Management common operations	84

7.3.4.1	Identify the managed entity and the technology specific protocol	84

7.3.4.2	Locate the technology specific data model objects to be managed on the managed entity	84

7.3.4.3	Establish a management session with the managed entity or management server	84

7.3.4.4	Send the management request(s) to the managed entity corresponding to the received Request primitive	84

7.4	Resource type-specific procedures and definitions	85

7.4.1	Introduction	85

7.4.2	Resource type specification conventions	85

7.4.2.0	Introduction	85

7.4.2.1	Resource type definition conventions	85

7.4.2.2	Resource type-specific procedure conventions	86

7.4.3	Resource type <accessControlPolicy>	86

7.4.3.1	Introduction	86

7.4.3.2	accessControlPolicy resource specific procedure on CRUD operations	87

7.4.3.2.0	Introduction	87

7.4.3.2.1	Create	87

7.4.3.2.2	Retrieve	87

7.4.3.2.3	Update	87

7.4.3.2.4	Delete	87

7.4.4	Resource Type <CSEBase>	88

7.4.4.1	Introduction	88

7.4.4.2	<CSEBase> resource specific procedure on CRUD operations	88

7.4.4.2.1	Create	88

7.4.4.2.2	Retrieve	89

7.4.4.2.3	Update	89

7.4.4.2.4	Delete	89

7.4.5	Resource Type <remoteCSE>	89

7.4.5.1	Introduction	89

7.4.5.2	<remoteCSE> resource specific procedure on CRUD operations	90

7.4.5.2.0	Introduction	90

7.4.5.2.1	Create	91

7.4.5.2.2	Retrieve	91

7.4.5.2.3	Update	91

7.4.5.2.4	Delete	91

7.4.6	Resource Type <AE>	91

7.4.6.1	Introduction	91

7.4.6.2	<AE> resource specific procedure on CRUD+N operations	92

7.4.6.2.1	Introduction	92

7.4.6.2.2	Create	92

7.4.6.2.3	Retrieve	93

7.4.6.2.4	Update	93

7.4.6.2.5	Delete	93

7.4.6.2.6	Notify	93

7.4.7	Resource Type <container>	93

7.4.7.1	Introduction	93

7.4.7.2	<container> resource specific procedure on CRUD operations	94

7.4.7.2.0	Introduction	94

7.4.7.2.1	Create	95

7.4.7.2.2	Retrieve	95

7.4.7.2.3	Update	95

7.4.7.2.4	Delete	95

7.4.8	Resource Type <contentInstance>	95

7.4.8.1	Introduction	95

7.4.8.2	<contentInstance> resource specific procedure on CRUD operations	96

7.4.8.2.1	Create	96

7.4.8.2.2	Retrieve	96

7.4.8.2.3	Update	97

7.4.8.2.4	Delete	97

7.4.9	Resource Type <subscription>	97

7.4.9.1	Introduction	97

7.4.9.2	<subscription> resource specific procedure on CRUD operations	98

7.4.9.2.1	Create	98

7.4.9.2.2	Retrieve	99

7.4.9.2.3	Update	99

7.4.9.2.4	Delete	99

7.4.10	Resource Type <schedule>	99

7.4.10.1	Introduction	99

7.4.10.2	<schedule> resource specific procedure on CRUD operations	101

7.4.10.2.0	Introduction	101

7.4.10.2.1	Create	101

7.4.10.2.2	Retrieve	101

7.4.10.2.3	Update	101

7.4.10.2.4	Delete	102

7.4.11	Resource Type <locationPolicy>	102

7.4.11.1	Introduction	102

7.4.11.2	<locationPolicy> resource specific procedure on CRUD Operations	103

7.4.11.2.0	Introduction	103

7.4.11.2.1	Create	103

7.4.11.2.2	Retrieve	104

7.4.11.2.3	Update	104

7.4.11.2.4	Delete	104

7.4.12	Resource Type <delivery>	105

7.4.12.1	Introduction	105

7.4.12.2	<delivery> resource specific procedure on CRUD operations	106

7.4.12.2.0	Introduction	106

7.4.12.2.1	Create	106

7.4.12.2.2	Retrieve	106

7.4.12.2.3	Update	106

7.4.12.2.4	Delete	107

7.4.13	Resource Type <request>	107

7.4.13.1	Introduction	107

7.4.13.2	<request> resource specific procedure on CRUD operations	108

7.4.13.2.0	Introduction	108

7.4.13.2.1	Create	108

7.4.13.2.2	Retrieve	108

7.4.13.2.3	Update	109

7.4.13.2.4	Delete	109

7.4.14	Resource Type <group>	109

7.4.14.1	Introduction	109

7.4.14.2	<group> resource specific procedure on CRUD operations	110

7.4.14.2.1	Introduction	110

7.4.14.2.2	Create	110

7.4.14.2.3	Retrieve	111

7.4.14.2.4	Update	111

7.4.14.2.5	Delete	111

7.4.15	Resource Type <fanOutPoint>	111

7.4.15.1	Introduction	111

7.4.15.2	<fanOutPoint> operations	112

7.4.15.2.1	Validate the type of  resource to be created	112

7.4.15.2.2	Sub-group creation for members residing on the same CSE	112

7.4.15.2.3	Assign URI for aggregation of notification	113

7.4.15.2.4	Fanout Request to each member	113

7.4.15.3	<fanOutPoint> resource specific procedure on CRUD operations	113

7.4.15.3.1	Introduction	113

7.4.15.3.2	Create	113

7.4.15.3.3	Retrieve	114

7.4.15.3.4	Update	114

7.4.15.3.5	Delete	115

7.4.16	Resource Type <mgmtObj>	115

7.4.16.1	Introduction	115

7.4.16.2	<mgmtObj> resource specific procedure on CRUD operations	116

7.4.16.2.0	Introduction	116

7.4.16.2.1	Create	116

7.4.16.2.2	Retrieve	117

7.4.16.2.3	Update	117

7.4.16.2.4	Delete	117

7.4.17	Resource Type <mgmtCmd>	118

7.4.17.1	Introduction	118

7.4.17.2	<mgmtCmd> resource specific procedure on CRUD operations	120

7.4.17.2.0	Introduction	120

7.4.17.2.1	Create	120

7.4.17.2.2	Retrieve	120

7.4.17.2.3	Update	120

7.4.17.2.4	Delete	121

7.4.18	Resource Type <execInstance>	122

7.4.18.1	Introduction	122

7.4.18.2	<execInstance> resource specific procedure on CRUD oerations	123

7.4.18.2.0	Introduction	123

7.4.18.2.1	Update (Cancel)	123

7.4.18.2.2	Retrieve	124

7.4.18.2.3	Delete	124

7.4.19	Resource Type <node>	124

7.4.19.1	Introduction	124

7.4.19.2	<node> resource specific procedure on CRUD operations	125

7.4.19.2.1	Create	125

7.4.19.2.2	Retrieve	125

7.4.19.2.3	Update	126

7.4.19.2.4	Delete	126

7.4.20	Resource Type <m2mServiceSubscriptionProfile>	126

7.4.20.1	Introduction	126

7.4.20.2	<m2mServiceSubscriptionProfile> resource specific procedure on CRUD operations	127

7.4.20.2.0	Introduction	127

7.4.20.2.1	Create	127

7.4.20.2.2	Retrieve	127

7.4.20.2.3	Update	127

7.4.20.2.4	Delete	127

7.4.21	Resource Type <serviceSubscribedNode>	127

7.4.21.1	Introduction	127

7.4.21.2	<serviceSubscribedNode> resource specific procedure on CRUD operations	128

7.4.21.2.0	Introduction	128

7.4.21.2.1	Create	128

7.4.21.2.2	Retrieve	128

7.4.21.2.3	Update	129

7.4.21.2.4	Delete	129

7.4.22	Resource Type <pollingChannel>	129

7.4.22.1	Introduction	129

7.4.22.2	<pollingChannel> resource specific procedure on CRUD operations	130

7.4.22.2.0	Introduction	130

7.4.22.2.1	Create	130

7.4.22.2.2	Retrieve	130

7.4.22.2.3	Update	130

7.4.22.2.4	Delete	130

7.4.23	Resource Type <pollingChannelURI>	130

7.4.23.1	Introduction	130

7.4.23.2	<pollingChannelURI> resource specific procedure on CRUD operations	131

7.4.23.2.0	Introduction	131

7.4.23.2.1	Create	131

7.4.23.2.2	Retrieve	131

7.4.23.2.3	Update	131

7.4.23.2.4	Delete	131

7.4.24	Resource Type <statsConfig>	131

7.4.24.1	Introduction	131

7.4.24.2	<statsConfig> resource-specific procedure on CRUD operations	132

7.4.24.2.1	Create	132

7.4.24.2.2	Retrieve	132

7.4.24.2.3	Update	132

7.4.24.2.4	Delete	133

7.4.25	Resource Type <eventConfig>	133

7.4.25.1	Introduction	133

7.4.25.2	<eventConfig> resource-specific procedure on CRUD operations	134

7.4.25.2.1	Create	134

7.4.25.2.2	Retrieve	135

7.4.25.2.3	Update	135

7.4.25.2.4	Delete	135

7.4.26	Resource Type <statsCollect>	135

7.4.26.1	Introduction	135

7.4.26.2	<statsCollect> resource-specific procedure on CRUD operations	136

7.4.26.2.1	Create	136

7.4.26.2.2	Retrieve	137

7.4.26.2.3	Update	137

7.4.26.2.4	Delete	137

7.4.27	Announced resource type	137

7.4.27.1	Introduction	137

7.4.27.2	Resource specific procedure on CRUD operations	138

7.4.27.2.1	Introduction	138

7.4.27.2.2	Create	138

7.4.27.2.3	Retrieve	139

7.4.27.2.4	Update	139

7.4.27.2.5	Delete	139

7.4.28	Resource Type latest	139

7.4.28.1	Introduction	139

7.4.28.2	<latest> Resource Specific Procedure on CRUD Operations	139

7.4.28.2.1	Introduction	139

7.4.28.2.2	Create	140

7.4.28.2.3	Retrieve	140

7.4.28.2.4	Update	140

7.4.28.2.5	Delete	140

7.4.29	Resource Type oldest	141

7.4.29.1	Introduction	141

7.4.29.2	<oldest> Resource Specific Procedure on CRUD Operations	141

7.4.29.2.1	Introduction	141

7.4.29.2.2	Create	141

7.4.29.2.3	Retrieve	141

7.4.29.2.4	Update	141

7.4.29.2.5	Delete	142

7.4.30	Resource Type <serviceSubscribedAppRule>	142

7.4.30.1	Introduction	142

7.4.30.2	<serviceSubscribedAppRule> resource specific procedure on CRUD operations	143

7.4.30.2.1	Introduction	143

7.4.30.2.2	Create	143

7.4.30.2.3	Retrieve	143

7.4.30.2.4	Update	143

7.4.30.2.5	Delete	143

7.5	Primitive-specific procedures and definitions	143

7.5.1	Notification data object and procedures	143

7.5.1.1	Notification data object	143

7.5.1.2	Notification procedures	144

7.5.1.2.1	Introduction	144

7.5.1.2.2	Notification for modification of subscribed resources	144

7.5.1.2.3	Subscription Verification during Subscription Creation	146

7.5.1.2.4	Notification for Subscription Deletion	147

7.5.1.2.5	Notification for Asynchronous Non-blocking Request	147

7.5.1.2.6	Notification for subscription via group	147

7.5.2	Elements contained in the Content primitive parameter	148

8	Representation of primitives in data transfer	149

8.1	Introduction	149

8.2	Short names	149

8.2.1	Introduction	149

8.2.2	Primitive parameters	150

8.2.3	Resource attributes	150

8.2.4	Resource types	155

8.2.5	Complex data types members	156

8.3	XML serialization	158

8.3.1	Method	158

8.3.2	Examples	158

8.4	JSON serialization	159

8.4.1	Terminology	159

8.4.2	Method	159

8.4.3	Examples	160

Annex A (informative):	 Binding Mch to Diameter for Charging	161

A.1	Introduction	161

A.2	Diameter Commands on Mch	161

A.2.1	Accounting Request Command	161

A.2.2	Accounting Answer Command	161

A.3	Mapping of M2M Recorded Information Elements to AVPs	161

A.4	Summary of AVPs used	162

A.5	oneM2M Specific AVP Usage	164

A.5.1	Access-Network-Identifier AVP	164

A.5.2	Acct-Application-Id AVP	164

A.5.3	Accounting-Record-Type AVP	164

A.5.4	Application-Entity-ID AVP	164

A.5.5	Control-Memory-Size AVP	164

A.5.6	Current-Number-Members AVP	164

A.5.7	Data-Memory-Size AVP	164

A.5.8	External-ID AVP	164

A.5.9	Group-Name AVP	164

A.5.10	Hosting-CSE-ID AVP	165

A.5.11	Originator AVP	165

A.5.12	Maximum-Number-Members AVP	165

A.5.13	M2M-Event-Record-Timestamp AVP	165

A.5.14	M2M-Information AVP	165

A.5.15	Node-ID AVP	165

A.5.16	Occupancy AVP	165

A.5.17	Protocol-Type AVP	166

A.5.18	Rating-Group AVP	166

A.5.19	Receiver AVP	166

A.5.20	Request-Body-Size AVP	166

A.5.21	Request-Headers-Size AVP	166

A.5.22	Request-Operation AVP	166

A.5.23	Response-Body-Size AVP	166

A.5.24	Response-Headers-Size AVP	166

A.5.25	Response-Status-Code AVP	166

A.5.26	Service-Context-Id AVP	167

A.5.27	Service-Information AVP	167

A.5.28	Subgroup-Name AVP	167

A.5.29	Subscription-Id AVP	167

A.5.30	Subscription-Id-Data AVP	167

A.5.31	Subscription-Id-Type AVP	167

A.5.32	Target-ID AVP	167

Annex B (normative):	Device triggering	169

B.1	Providing device triggering service by means of 3GPP networks	169

B.1.1	Introduction	169

B.1.2	Device action request command	169

B.1.3	Device action answer command	169

B.1.4	Device notification request command	169

B.1.5	Device notification answer command	169

Annex C(informative):	XML examples	170

C.1	XML schema for container resource type	170

C.2	Container resource that conforms to the Schema given above (see Annex C.1)	171

Annex D (normative):	<mgmtObj> Resource specializations	172

D.1	Introduction	172

D.2	Resource [firmware]	172

D.2.1	Introduction	172

D.2.2	Resource specific procedure on CRUD operations	172

D.2.2.0	Introduction	172

D.2.2.1	Create	172

D.2.2.2	Update	173

D.2.2.3	Retrieve	173

D.2.2.4	Delete	173

D.3	Resource [software]	173

D.3.1	Introduction	173

D.3.2	Resource specific procedure on CRUD operations	174

D.3.2.0	Introduction	174

D.3.2.1	Create	174

D.3.2.2	Update	174

D.3.2.3	Retrieve	175

D.3.2.4	Delete	175

D.4	Resource [memory]	175

D.4.1	Introduction	175

D.4.2	Resource specific procedure on CRUD operations	175

D.4.2.0	Introduction	175

D.4.2.1	Create	175

D.4.2.2	Update	176

D.4.2.3	Retrieve	176

D.4.2.4	Delete	176

D.5	Resource [areaNwkInfo]	176

D.5.1	Introduction	176

D.5.2	Resource specific procedure on CRUD operations	177

D.5.2.0	Introduction	177

D.5.2.1	Create	177

D.5.2.2	Update	177

D.5.2.3	Retrieve	177

D.5.2.4	Delete	177

D.6	Resource [areaNwkDeviceInfo]	177

D.6.1	Introduction	177

D.6.2	Resource specific procedure on CRUD operations	178

D.6.2.0	Introduction	178

D.6.2.1	Create	178

D.6.2.2	Update	178

D.6.2.3	Retrieve	178

D.6.2.4	Delete	179

D.7	Resource [battery]	179

D.7.1	Introduction	179

D.7.2	Resource specific procedure on CRUD operations	179

D.7.2.0	Introduction	179

D.7.2.1	Create	179

D.7.2.2	Update	179

D.7.2.3	Retrieve	180

D.7.2.4	Delete	180

D.8	Resource [deviceInfo]	180

D.8.1	Introduction	180

D.8.2	Resource specific procedure on CRUD operations	181

D.8.2.0	Introduction	181

D.8.2.1	Create	181

D.8.2.2	Update	181

D.8.2.3	Retrieve	181

D.8.2.4	Delete	181

D.9	Resource [deviceCapability]	181

D.9.1	Introduction	181

D.9.2	Resource specific procedure on CRUD operations	182

D.9.2.0	Introduction	182

D.9.2.1	Create	182

D.9.2.2	Update	182

D.9.2.3	Retrieve	183

D.9.2.4	Delete	183

D.10	Resource [reboot]	183

D.10.1	Introduction	183

D.10.2	Resource specific procedure on CRUD operations	184

D.10.2.0	Introduction	184

D.10.2.1	Create	184

D.10.2.2	Update	184

D.10.2.3	Retrieve	184

D.10.2.4	Delete	184

D.11	Resource [eventLog]	185

D.11.1	Introduction	185

D.11.2	Resource specific procedure on CRUD operations	185

D.11.2.1	Introduction	185

D.11.2.2	Create	185

D.11.2.3	Update	186

D.11.2.4	Retrieve	186

D.11.2.5	Delete	186

D.12	Resource [cmdhPolicy]	186

D.12.0	Introduction	186

D.12.1	Resource [activeCmdhPolicy]	187

D.12.2	Resource [cmdhDefaults]	187

D.12.3	Resource [cmdhDefEcValue]	188

D.12.4	Resource [cmdhEcDefParamValues]	188

D.12.5	Resource [cmdhLimits]	189

D.12.6	Resource [cmdhNetworkAccessRules]	190

D.12.7	Resource [cmdhNwAccessRule]	191

D.12.8	Resource [cmdhBuffer]	192

Annex E (informative):	Procedures for accessing resources	193

E.1	Accessing resources in CSEs - blocking requests	193

E.2	Accessing Resources in CSEs - non-blocking requests	194

E.2.1	Non-blocking models	194

E.2.2	Synchronous case	194

Annex F (informative):	Guidelines for oneM2M resource type XSD	199

Annex G (normative):	Location request	201

G.1	Introduction	201

G.2	Location request by means of OMA-REST-NetAPI-TerminalLocation interface	201

G.2.1	Introduction	201

G.2.2	Resource structure of OMA NetAPI for terminal location	201

G.2.3	Procedures for terminal location	203

G.2.3.1	Request in a single query toward a location server	203

G.2.4	Subscribe to notifications for periodic location updates	204

G.2.5	Subscribe to notifications for area updates	205

Annex H (normative):	CMDH message processing	207

H.1	Pre-requisites	207

H.2	CMDH processing: processing request or response messages requiring the receiver CSE to forward information to another CSE	208

H.2.1	Applicability of CMDH processing	208

H.2.2	Partitioning of CMDH processing	208

H.2.3	CMDH message validation procedure	210

H.2.4	CMDH message forwarding procedure	215

H.2.5	Establishment of Mcc communication connection to another CSE	222

Annex I (informative):	Guidelines for using XSD files in AE and CSE code	224

I.1	Usage of the oneM2M developed XSD files	224

I.2	Example AE/CSE implementation featuring mapping between short and long names for XML serialization	224

I.3	Example AE/CSE implementation featuring mapping between short and long names for JSON serialization	226

List of tables and figures	228

History	229

# Intellectual Property Rights

IPRs essential or potentially essential to the present document may have been declared to ETSI. The information pertaining to these essential IPRs, if any, is publicly available for ETSI members and non-members, and can be found in ETSI SR 000 314: "Intellectual Property Rights (IPRs); Essential, or potentially Essential, IPRs notified to ETSI in respect of ETSI standards", which is available from the ETSI Secretariat. Latest updates are available on the ETSI Web server (https://ipr.etsi.org/).

Pursuant to the ETSI IPR Policy, no investigation, including IPR searches, has been carried out by ETSI. No guarantee can be given as to the existence of other IPRs not referenced in ETSI SR 000 314 (or the updates on the ETSI Web server) which are, or may be, or may become, essential to the present document.

# Foreword

This Technical Specification (TS) has been produced by ETSI Partnership Project oneM2M (oneM2M).

# 1	Scope

The present document specifies the communication protocol(s) for oneM2M compliant Systems, M2M Applications, and/or other M2M systems.

The present document also specifies the common data formats, interfaces and message sequences to support reference points(s) defined by oneM2M.

# 2	References

## 2.1	Normative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

Referenced documents which are not found to be publicly available in the expected location might be found at http://docbox.etsi.org/Reference.

NOTE:	While any hyperlinks included in this clause were valid at the time of publication, ETSI cannot guarantee their long term validity.

The following referenced documents are necessary for the application of the present document.

[1]	W3C, Extensible Markup Language (XML) 1.0 (Fifth Edition), W3C Recommendation 26 November 2008.

[2]	IETF RFC 3986: "Uniform Resource Identifier (URI): Generic Syntax".

[3]	W3C XMLSchemaP2: "W3C Recommendation (2004), XML Schema Part 2:Datatypes Second Edition.".

[4]	Void.

[5]	Void.

[6]	ETSI TS 118 101: "oneM2M; Functional Architecture (oneM2M TS-0001)".

[7]	ETSI TS 118 103: "oneM2M; Security solutions (oneM2M TS-0003)".

[8]	IEEE 754-2008: "IEEE. IEEE Standard for Floating-Point Arithmetic", 29 August 2008.

NOTE: Available at http://ieeexplore.ieee.org/servlet/opac?punumber=4610933.

[9]	IETF RFC 3548: "The Base16, Base32, and Base64 Data Encodings".

[10]	IETF RFC 2045: "Multipurpose Internet Mail Extensions (MIME) Part One: Format of Internet Message Bodies".

[11]	IETF RFC 3987:" Internationalized Resource Identifiers (IRIs)".

[12]	IETF BCP 47: "Best Current Practices 47". Concatenation of IETF RFC 4646:” Tags for Identifying Languages" "(2006) and RFC 4647: "Matching of Language Tags" (2006).

[13]	IETF RFC 3588: "Diameter Base Protocol".

[14]	IETF RFC 6733: "Diameter Base Protocol".

[15]	ETSI TS 123 682: "Digital celluar telecommunication system (Phase 2+); Universal Mobile Telecommunication System (UMTS); LTE; Architecture enhancements to facilitate communications with packet data networks and applications" (3GPP TS 23.682 Release 11)".

[16]	ETSI TS 129 368: "Universal Mobile Telecommunication System (UMTS); LTE; Tsp interface protocol between the MTC Interworking Function (MTC-IWF) and Service Capability Server (SCS)" (3GPP TS 29.368 Release 11)".

[17]	ETSI TS 123 003: "Digital cellular telecommunication system (Phase 2+); Universal Mobile Telecommunication System (UMTS); Numbering, addressing and identification" (3GPP TS 23.003).

[18]	Void.

[19]	IETF RFC 7159: "The JavaScript Object Notation (JSON) Data Interchange Format".

[20]	IETF RFC 4234: "Augmented BNF for Syntax Specifications: ABNF"

[21]	IETF RFC 3629: " UTF-8, a transformation format of ISO 10646".

[22]	ETSI TS 118 108: "oneM2M; CoAP Protocol Binding (oneM2M TS-0008)".

[23]	ETSI TS 118 109: "oneM2M; HTTP Protocol Binding (oneM2M TS-0009)".

[24]	ETSI TS 118 110: "oneM2M; MQTT Protocol Binding (oneM2M TS-0010)".

[25]	ETSI TS 118 111: "oneM2M; Common Terminology (oneM2M TS-0011)".

[26]	IETF RFC 6837: "Media Type Specifications and Registration Procedures".

[27]	ISO 8601:2004; "Data elements and interchange formats -- Information interchange -- Representation of dates and times".

[28]	OMA-TS-REST-NetAPI_TerminalLocation-V1_0-20130924-A: "RESTful Network API for Terminal Location", Version 1.0.

[29]	IETF RFC 4632: " Classless Inter-domain Routing (CIDR): The Internet Address Assignment and Aggregation Plan".

[30]	IETF RFC 5952: "A Recommendation for IPv6 Address Text Representation".

## 2.2	Informative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

NOTE:	While any hyperlinks included in this clause were valid at the time of publication, ETSI cannot guarantee their long term validity.

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	oneM2M Drafting Rules.

NOTE:	Available at http://ftp.onem2m.org/Others/Rules_Pages/oneM2M-Drafting-Rules-V1_0_1.doc.

[i.2]	Fielding, Roy Thomas (2000): "Architectural Styles and the Design of Network-based Software Architectures", Doctoral dissertation, University of California, Irvine.

[i.3]	OMA-NetAPI-NotificationChannel: "RESTful Network API for Notification Channel", Open Mobile Alliance™, OMA-TS-REST_NetAPI_NotificationChannel-V1_0.

[i.4]	OMA-MLP: "Mobile Location Protocol", Version 3.4 (OMA-TS-MLP-V3_4-20130226), Open Mobile Alliance.

[i.5]	ETSI TS 132 299: "digital cellular telecommunication system (Phase 2+); Universal Mobile Telecommunication System (UMTS); LTE; Telecommunication management;Charging management; Diameter charging applications" (3GPP TS 32.299 Release 11)".

[i.6]	IETF RFC 4006: " Diameter Credit-Control Application".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the following terms and definitions and those given in ETSI TS 118 111 Common Terminology  [25] apply:

Complex Data Types: is a data type that has a child element.

Enumeration Type: is a data type that enables for a variable to be a set of predefined constants.

Group Hosting CSE: CSE where the addressed group resource resides.

Hosting CSE: CSE where the addressed resource is hosted.

Location Server: is a server offering location capabilities.

M2M Area Network: a network provides connectivity between Application Service Nodes or Application Dedicated Nodes and Middle Nodes in the field domain.

Mca: reference point for M2M Communication with AE.

Mcc: reference point for M2M Communication with CSE.

Mcc': reference point for M2M Communication with CSE of different M2M Service Provider.

Originator: For single-hop case, the Originator is the entity that sends a Request. For multi-hop case, the Originator is the entity that sends the first Request in a sequence of requests.

NOTE:	An Originator can either be an AE or a CSE.

Receiver: is the entity that receives the Request.

Receiver CSE: is any CSE that receives a request.

Registrar CSE: CSE is the CSE where an Application or another CSE has registered.

Registree/Registrar CSE: is the CSE that registers with another CSE.

Request: is the message sent from the Originator to the Receiver.

Response: is the message replied to the Request from the Receiver to the Originator.

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations and those given in ETSI TS 118 111 Common Terminology [25] apply:

3GPP2	3rd Generation Partnership Project 2

ACP	AccessControlPolicy

AD	Anno Domini

AE-ID	Application Entity Identifier

ARC	Architecture

ASN-CSE	Application Entity that is registered with the CSE at Application Service Node

BCP	Best Current Practices

CDT	Common Data Type

CIDR	Classless Inter-Domain Routing

CMDH	Communication Management and Delivery Handling

CoAP	Constrained Application Protocol

CRUD	Create Retrieve Update Delete

CRUD+N	Create Retrieve Update Delete Notification

CSE-ID	Common Service Entity Identifier

CUDN	Create Update Delete NotifyDAA	Device Action Answer

DAR	Device-Action-Request

DNA	Device Notification Answer

DNR	Device Notification Request

DTLS	Datagram Transport Layer Security

FFS	For Further Study

FQDN	Fully Qualified Domain Name

GPS	Global Positioning System

HTTP	HyperText Transfer Protocol

IANA	Internet Assigned Numbers Authority

ID	IDentifier

IEEE	Institute of Electrical and Electronics Engineers

IETF	Internet Engineering Task Force

IN-AE	Application Entity that is registered with the CSE in the Infrastructure Node

IN-CSE 	CSE which resides in the Infrastructure Node

IRI	Internationalized Resource Identifier

ISO	International Organization for Standardization

JSON 	JavaScript Object Notation

MA	Mandatory Announced

MIME	Multipurpose Internet Mail Extension

MN-CSE	Reference Point for M2M Communication with CSE of different M2M Service Provider

MQTT	Message Queue Telemetry Transport

MTC-IWF 	MachinetType Communications - InterWorking Function

NP	Not Present

OA	Optional Announced

OMA-DM	Open Mobile Alliance Device Management

RD	Retrieve DeleteRFC	Request For Comment

RPC	Remote Procedure Call

RSC	Response Status Codes

RUD	Retrieve Update Delete

SCS 	Services Capability Server

SP	Service Provider

SP-ID	Service Provider Identifier

TBD	To Be Determined

TCP	Transmission Control Protocol

TLS	Transport Layer Security

UDP	User Datagram Protocol

URI	Uniform Resource Identifier

URL	Uniform Resource Locator

UTC	Coordinated Universal Time

UTF	UCS Transformation Format

UUID	Universally Unique Identifier

XML	eXtensible Markup Language

XSD	XML Schema Definition

WLAN	Wireless Local Area Network

# 4	Conventions

The key words "Shall", "Shall not", "May", "Need not", "Should", "Should not" in the present document are to be interpreted as described in the oneM2M Drafting Rules [i.1].

To improve readability:

- The information elements of oneM2M Request/Response messages will be referred to as parameters. Parameter names will be written in bold italic.

- The information elements of resources will be referred to as attributes and child resources. Attributes will be written in italics.

- Abbreviated short names for information elements (see clause 8) will be written in bold italic.

# 5	Protocol design principles and requirements

## 5.1	General introduction

The following clauses contain the design principles and requirements for the oneM2M protocol.

## 5.2	Introduction

### 5.2.0	General

The oneM2M architecture is resource-based (ETSI TS 118 101 Functional Architecture [6]). The functionality of the system is exposed by means of APIs over all reference points specified in ETSI TS 118 101 Functional Architecture [6]. Operations upon resources hosted by a CSE are carried over an established channel that constitutes the communication on the reference points Mca and Mcc. All resource operations could be fulfilled with the considerations in terms of scalability, extensibility, fault tolerance and robustness, energy efficiency, and self-operation.

Offline Charging using the Mch reference point is described in [6] and specified in ClauseA

Each resource operation comprises a pair of primitives: Request and Response. The Request and Response primitives can be represented as XML documents or JSON texts. This process of representing a primitive as XML documents or JSON texts is denoted serialization in the present document. Serialization translates primitives into a format that can be stored or exchanged between network entities. This is exploited when transmitting primitives over communication protocols such as HTTP, CoAP or MQTT.

In order to provide a well-defined interface for the reference points in ETSI TS 118 101 [6] Functional Architecture, the following aspects need to be provided:

the collection of primitives carried over a specific reference point; and

the definitions and procedures of resource types in relation to the underlying protocols and reference points involved.

The current document provides:

protocol design principles and requirements

data type definitions and representations;

primitive format and generic procedures;

common operations of originators and receivers;

resource type-specific procedures ; and

XML definitions and schema.

NOTE:	The actual binding of the interface to a specific protocol is not part of the present document, but is specified in separate Technical Specifications [22], [23],[24].

In accordance with the oneM2M architecture, each reference point is applicable to a wide range of underlying network technologies and transport protocols. oneM2M defines a set of bindings for specific underlying network technologies and transport protocols, these bindings are not limiting the applicability of the interfaces when used in other underlying networks and transport protocols. However, the behaviour of the interface needs to be respected in accordance to the present document and ETSI TS 118 101 Functional Architecture [6].

### 5.2.1	Interfaces to the underlying networks

The CSEs access the network service functions provided by the underlying networks such as 3GPP, 3GPP2 and WLAN via Mcn reference point. The following services are provided by the underlying networks:

Device triggering (see Annex B)

Location request (see Annex G)

Device Management (see clause 7.3.4)

## 5.3	API design guidelines

The following are the guidelines for designing APIs:

APIs shall follow the principle of RESTful architecture, as described in [i.2].

APIs shall indicate which features are supported and not supported over the reference points specified in ETSI TS 118 101 [6].

APIs shall define how to address resources and how to manipulate resources, in accordance with ETSI TS 118 101 [6]; the resource is identified by a Universal Resource Identifier (URI), [2].

APIs shall provide the format and syntax of the operation primitives for all resources defined in ETSI TS 118 101 Functional Architecture [6] . In case that for a particular protocol binding an operation cannot be supported it has to be clearly stated in the specific protocol binding technical specification.

Resource has a representation (see [i.2]) that shall be transferred and manipulated with the verbs. These verbs are identified as operations in ETSI TS 118 101 Functional Architecture [6]: CREATE. RETRIEVE, UPDATE, DELETE and NOTIFY.

All primitives as well as the way that those primitives are sent shall be defined. The functionality of the primitives shall be compliant to the resource type specific procedure as specified in ETSI TS 118 101 Functional Architecture [6], clause 10.2.

Primitives shall include attributes in accordance with ETSI TS 118 101 Functional Architecture [6] for a specific resource.

Primitive shall be self-descriptive and contain all the information needed for the receiver of the primitives to handle the primitives.

Primitive should be idempotent operations which mean no matter how many times the primitive is sent, the result does not' change, in accordance to [i.2].

Primitives shall be mapped on the transport layer protocols.

## 5.4	Primitives

### 5.4.1	Introduction

Primitives are common service layer messages exchanged over the Mca, Mcc and Mcc' reference points.

There are two use cases:

communication between an Originator and a Receiver which are collocated on the same M2M Node (e.g. ASN or MN) in the Common Service layer,

communication between an Originator and a remote Receiver via an Underlying Network.

In the first use case the primitives may be exchanged directly between the Originator and Receiver processes.

In case of using an IP-based Underlying Network as illustrated in Figure 5.4.1-1, the primitives are mapped to application  layer communication protocols such as HTTP, CoAP or MQTT which use TCP or UDP on the transport layer. The specification of primitives, however, is independent of underlying communication protocols and allow introduction of bindings to other communication protocols.

Figure 5.4.1-1: Communication model using Request and Response primitives over an IP-based Underlying Network

A single primitive in the common service layer may be mapped to zero or more transport messages by the communication protocol.

The Originators shall send requests to Receivers through primitives. The Originator and Receiver may be represented by either an AE or a CSE. The CRUD request primitive addresses a resource residing in a CSE. The Notify request primitive may address an AE or CSE.

Each CRUD+N operation consists of request and response primitives.

### 5.4.2	Primitives modelling

Primitives are modelled as follows.

A primitive is represented in form of  a data structure  which defines with appropriate parameters specific procedures to be executed by both originator and receiver entities.

The data structure of a primitive consists of two parts:

a control part, which contains parameters specifying the processing of the primitive; and

an optional content part, which represents resource data, either the complete resource or only part of the resource (i.e. values of one or more resource attributes) in the partial addressing case.

Figure 5.4.2-1: Primitives modelling

### 5.4.3	Primitive principles

Execution of one primitive shall finish completely before execution of a subsequent primitives starts that affects the same resource.

When creating or updating the resource, its representation (full or partial) shall be contained in the content part of the primitive. Based on the representation of the resource, the Hosting CSE can create or update the entire resource without need for further information.

The operations on resources triggered by primitives shall be idempotent. This means no matter how many times the same primitive is targeted to the same resource, the resource does not change after the first execution of this primitive, with the exception of the creation of child resources.

### 5.4.4	Serialization of primitives

When transferred over a oneM2M reference point while using a communication protocol such as HTTP, CoAP or MQTT,  the way oneM2M Request and Response primitives are represented shall be defined by a  specific oneM2M protocol binding that is being used for the message transfer. The originator and receiver of each primitive use the same binding, and thus they will be using compatible serialization and deserialization techniques. Clause 8 of the present document defines canonical approaches for serializing primitives as JSON objects or XML documents used by oneM2M protocol bindings.

## 5.5	Design principles

### 5.5.1	Introduction

The following clauses present the design principles which could wrap up the perspectives and ways in terms of definitions and procedures of APIs and resources for the oneM2M core protocol specified in the present document. These design principles shall cover all characteristics and advantages of oneM2M protocols including specifications of bindings to transport protocols such as HTTP, CoAP, and MQTT.

The design of oneM2M protocols consider and mitigate the risk of unintended consequences, such as extensibility and interoperability issues, operational problems, or efficiency.

### 5.5.2	Extensibility

The oneM2M protocols are designed to allow continued development and to facilitate changes by means of standardized extensions.

The impact of the extensibility on the existing oneM2M protocol functions shall be minimized.

Extensibility can be related to one or more of the following aspects:

handling a wide range of transport protocols as well as a different number of devices,

adding, removing or modifying oneM2M protocol functionality,

new oneM2M protocol routines,

new primitives and data types.

### 5.5.3	Scalability

For provisioning scalability as a requirement in the design of oneM2M protocols, one or several of the following mechanisms are used:

Ensuring direct addressability to the CSEs hosting target resources, to minimize network hops.

Asynchrony in terms of data processing, with the objective of minimizing the number of discarded packets.

Caching mechanisms that allow all the received packets to be processed.

Efficient load distribution to avoid bottlenecks and data loss.

Data compression and/or aggregation, in order to reduce the amount of data sent through the network.

### 5.5.4	Fault tolerance and robustness

One or more of the following mechanisms in terms of link availability can be exploited in the design of oneM2M protocols to account for a variety of exception conditions.

To provide reliable transmission of data packets, packet recovery will be dealt with by using mechanisms appropriate for the operating environment (e.g., constrained devices, unreliable networks).

When oneM2M protocols are employed over unreliable links, multiple data dissemination paths can be provided and maintained.

### 5.5.5	Efficiency

oneM2M protocols are designed with consideration of efficiency for networking involved resource-constrained devices.

As energy consumption directly affects the overall system performance, oneM2M protocols should consider energy efficiency, especially in resource constrained environments with battery-powered oneM2M devices.

Energy efficient oneM2M protocols aims at reducing the overall energy consumption while maintaining the performance required by the oneM2M Applications.

### 5.5.6	Inter-operability

API inter-operability between different protocol stacks is expected. For example, oneM2M API over HTTP/TCP/IP needs to inter-operate with CoAP/UDP in a local network using oneM2M API. oneM2M protocols are specified to provision the API inter-operability.

### 5.5.7	Self-operation and self-management

Devices employing the oneM2M API inter-work with established management protocols (e.g. security, discovery, bootstrapping, etc). The inter-working with legacy management protocols via the oneM2M API shall be carried out in self-operation methods.

# 6	oneM2M protocols/API overview

## 6.1	Introduction

The present document describes message formats and procedures to communicate with oneM2M compliant M2M Platform System.

The present document describes:

Data representation for communication protocol messages.

Normal and exceptional procedure.

Status codes.

Guidelines for drafting APIs.

For wide acceptance by industrial markets, the present document describes structured and non-structured data for oneM2M Protocol using XML Schema Definition (XSD) language  [3].

The actual format of data in request and response messages partially depends on the applied protocol binding. Mapping rules between the data formats defined in the present document types and  protocol-specific native data formats are specified in the protocol binding specifications ETSI TS 118 108, ETSI TS 118 109 and ETSI TS 118 110.

Any data types of XML elements defined for use in oneM2M protocols shall use the namespace:

- http://www.onem2m.org/xml/protocols.

The present document, and any XML or XML Schema Documents produced by oneM2M shall use the prefix m2m: to refer to that namespace.

The XSD files referenced in the present document shall serve following purposes:

- Provide an unambiguous definition of XML element names and data types used for

resource representations,

resource attributes,

Request and Response primitives (including Notification primitive),

parameters used in Request and Response primitives.

- Help to identify and avoid that equivalent data types are defined multiple times with different names.

- Provide a testable definition of the value range of data elements (e.g. allowed number range, allowed characters or character patterns, allowed enumeration values).

- Provide a testable definition of the presence of mandatory elements (“minOccurs=1”) and of cardinality not larger than a given limit (e.g. “maxOccurs=1”) in XML representations of data objects (i.e. resource instantiations and primitive parameters).

- Provide a testable definition of the correct sequence of occurrence of each element of a data object (where correct sequence is required).

- Enable the use of development tools that generate executable code for data object processing from the XSD.

- Enable the use of XML development tools which allow automatic generation of valid templates for XML and JSON objects, and validation of the compliance of any XML or JSON objects with the XSD.

Parameters and resource representations exchanged in primitives between oneM2M entities shall comply with data formats defined in the present document based on the referred XSD documents. The present document defines procedures for validation of received messages and the error handling in case of reception of non-compliant message content.

- NOTE: M2M implementations are required to validate the data received in incoming primitives in accordance with the present document, but the present document does not intend to impose restrictions on implementation of the validation procedures.  In particular the validation procedure is not required to use the XSD documents directly.

## 6.2	Addressing

### 6.2.1	Introduction

This clause describes the method of addressing oneM2M entities (e.g. AE or CSE) and oneM2M resources using identifiers described in the ETSI TS 118 101 Functional Architecture [6].

### 6.2.2	Summary of oneM2M Identifiers

Table 6.2.2-1 shows the summary of M2M Identifiers defined in ETSI TS 118 101 Functional Architecture [6].

Table 6.2.2-1: M2M Identifiers

| Identifier | Data Type | Description |
| --- | --- | --- |
| M2M-SP-ID | m2m:ID (see clause 6.3.3) | A globally unique ID as specified in [6] |
| App-ID | m2m:ID (see clause 6.3.3) | The identifier is specified in[6] |
| AE-ID | m2m:ID (see clause 6.3.3) | A globally unique ID as specified in [6] |
| CSE-ID | m2m:ID (see clause 6.3.3) | A globally unique ID as specified in [6] |
| M2M-Node-ID | m2m:nodeID (see clause 6.3.3) | A globally unique ID as specified in[6] |
| M2M-Sub-ID | m2m:ID (see clause 6.3.3) | A globally unique ID as specified in [6] |
| M2M-Request-ID | m2m:requestID (see clause 6.3.3) | A unique ID as specified in [6] |
| M2M-Ext-ID | m2m:externalID (see clause 6.3.5) | The identifier is specified in [6] |
| UNetwork-ID | m2m:ID (see clause 6.3.3) | A unique ID as specified in [6] |
| Trigger-Recipient-ID | m2m:triggerRecipientID | The identifier is specified in [6] |

### 6.2.3	oneM2M Entity Addressing

The oneM2M entities (e.g. AE or CSE) are identified and addressable using M2M Identifiers. Since an M2M Identifier is protocol independent,  an IN-CSE shall accommodate address resolution functionality to get actual PoA addresses for communicating with other M2M entities using a specific protocol binding.

The present document assumes each oneM2M entity has the CSE-PoA address of its Registrar CSE in advance.

When the oneM2M entity is communicating to another oneM2M entity, the address appearing in the oneM2M primitive (e.g. From or To parameter) shall be the absolute form of AE-ID or CSE-ID defined in ETSI TS 118 101 [6].

The CSE-ID shall be assigned by M2M Service Provider. The syntax of CSE-ID is defined by following ABNF notation[20].

CSE-ID =  absolute-CSE-ID / SP-relative-CSE-ID

absolute-CSE-ID = M2M-SP-ID  SP-relative-CSE-ID

M2M-SP-ID = "//" 1*unreserved

SP-relative-CSE-ID = "/" 1*unreserved

unreserved = ALPHA / DIGIT / "-" / "." / "_" / "~"

EXAMPLE 1 Starts:

EXAMPLE: //myoperator.com/cse1

This is an example of an absolute-CSE-ID, "//myoperator.com" is the M2M-SP-ID and "/cse1" is the SP-relative CSE-ID.

EXAMPLE 1 Ends:

The AE-ID is either assigned by the M2M Service Provider (S-type AE-ID Stem), or by the AE'''s Registrar CSE (C-Type Stem).

The syntax of AE-ID in ABNF notation [20] is as follows:

AE-ID = absolute-AE-ID / SP-relative-AE-ID

absolute-AE-ID = M2M-SP-ID SP-relative-AE-ID

SP-relative-AE-ID = (SP-relative-CSE-ID "/" C-AE-ID-Stem ) / ("/" S-AE-ID-Stem )

S-AE-ID-Stem = "S" SP-assigned-AE-ID-Stem

C-AE-ID-Stem = "C" CSE-assigned-AE-ID-Stem

SP-assigned-AE-ID-Stem = 1*unreserved

CSE-assigned-AE-ID-Stem = 1*unreserved

unreserved = ALPHA / DIGIT / "-" / "." / "_" / "~"

EXAMPLE 2 Starts:

EXAMPLE: //myoperator.com/S563423

This is an example of an absolute-AE-ID that was assigned by the M2M-SP (//myoperator.com).

EXAMPLE: //myoperator.com/cse2/C3532ea3

This is an example of an absolute AE-ID, which registered on the Registrar CSE //myoperator.com/cse2. ''C3532ea3'' is the AE-ID-Stem which is assigned by //myoperator.com/cse2.

EXAMPLE: /cse2/C3532ea3

This is the SP-relative version of the absolute AE-ID that is shown above.

EXAMPLE 2 Ends:

### 6.2.4	oneM2M Resource Addressing

Authorized oneM2M entities can operate on a oneM2M Resource by specifying the Resource Identifier as the target address.

- Structured Resource ID (used in Hierarchical Addressing): the ID is constructed as a relative path from the CSEBase resource via parent resources.

Unstructured Resource ID (used in Non-hierarchical Addressing): the ID uniquely identifies the resource in the domain of its Hosting CSE.

Furthermore each of these forms can be expressed either

- relative to the Hosting CSE; or

- relative to the M2M Service Provider; or

- in absolute form.

A single attribute of the targeted oneM2M resource shall be addressable adding the sub-address (targeted-attribute-name) following a "#" character after the resource address.

The address appearing in 'ChildResourceRef' may be used as in M2M-SP relative form, if it is obvious which M2M-SP or CSE-ID should be added.

The syntax of the oneM2M Resource Identifier (resource-ID) in ABNF notion [20] is as follows:

resource-identifier = (hierarchical-resource-address / non-hierarchical-resource-address) [ "#" targeted-attribute-name ]

hierarchical-resource-address = [ CSE-ID] 1*("/" resource-name)

non-hierarchical-resource-address = [ CSE-ID] "/" unstructured-ID

unstructured-ID = 1*unreserved

resource-name = 1*unreserved

When including hierarchical resource IDs into the Content parameter of response primitives (cf. clause 7.5.2), the resource Hosting CSE shall select the appropriate format of the ID depending on the location of the Originator. If the Originator of the request primitive is registered to the Hosting CSE, the CSE-relative format of hierarchical resource ID applies. If the Originator of the request primitive is not registered to the Hosting CSE but belongs to the same M2M-SP domain, the CSE-relative format applies. If the Originator of the request primitive belongs to a different M2M-SP domain, the Absolute format of hierarchical resource ID applies.

## 6.3	Common data types

### 6.3.1	Introduction

The following sub-clauses define the data format of resource attributes and parameters used in primitives.

### 6.3.2	Simple data types incorporated from XML schema

The following 'built-in data types' are incorporated from XML Schema definition [3].

Note that name space identifier for 'http://www.w3.org/2001/XMLSchema' shall be referred to using the prefix xs: in the present document.

Table 6.3.2-1: Data Types incorporated from XML Schema

| Data Type | Description | Notes |
| --- | --- | --- |
| xs:anyType | A special complex type definition whose name is anyType in the XSD namespace, is present in each XSD schema. The definition of anyType serves as default type definition for element declarations whose XML representation does not specify one. |  |
| xs:anySimpleType | The anySimpleType is considered to have an unconstrained lexical space for all built-in simple datatypes. |  |
| xs:string | The string data type represents character strings in XML |  |
| xs:boolean | boolean represents the values of two-valued logic. |  |
| xs:decimal | decimal represents a subset of the real numbers, which can be represented by decimal numerals. The value space of decimal is the set of numbers that can be obtained by dividing an integer by a non-negative power of ten, i.e. expressible as i / 10 n where i and n are integers and n ≥ 0. Precision is not reflected in this value space; the number 2.0 is not distinct from the number 2.00. The order relation on decimal is the order relation on real numbers, restricted to this subset. |  |
| xs:float | The float data type is patterned after the IEEE single-precision 32-bit floating point data type IEEE 754-2008 [8]. Its value space is a subset of the rational numbers. Floating point numbers are often used to approximate arbitrary real numbers. |  |
| xs:double | The double data type is patterned after the IEEE double-precision 64-bit floating point data type IEEE 754-2008 [8]. Each floating point data type has a value space that is a subset of the rational numbers. Floating point numbers are often used to approximate arbitrary real numbers. |  |
| xs:duration | duration is a data type that represents durations of time. |  |
| xs:hexBinary | hexBinary represents arbitrary hex-encoded binary data. |  |
| xs:base64Binary | base64Binary represents arbitrary Base64-encoded binary data. For base64Binary data the entire binary stream is encoded using the Base64 Encoding defined in IETF RFC 3548 [9], which is derived from the encoding described in IETF RFC 2045 [10]. |  |
| xs:anyURI | anyURI represents an Internationalized Resource Identifier Reference (IRI). An anyURI value can be absolute or relative, and may have an optional fragment identifier (i.e. it may be an IRI Reference). This type should be used when the value fulfils the role of an IRI, as defined in IETF RFC 3987 [11] or its successor(s) in the IETF Standards Track. |  |
| xs:normalizedString | normalizedString represents white space normalized strings. The ·value space· of normalizedString is the set of strings that do not contain the carriage return (#xD), line feed (#xA) nor tab (#x9) characters. The lexical space· of normalizedString is the set of strings that do not contain the carriage return (#xD), line feed (#xA) nor tab (#x9) characters. The base type of normalizedString is string. |  |
| xs:token | token represents tokenized strings. The ·value space· of token is the set of strings that do not contain the carriage return (#xD), line feed (#xA) nor tab (#x9) characters, that have no leading or trailing spaces (#x20) and that have no internal sequences of two or more spaces. The lexical space· of token is the set of strings that do not contain the carriage return (#xD), line feed (#xA) nor tab (#x9) characters, that have no leading or trailing spaces (#x20) and that have no internal sequences of two or more spaces. The base type·of token is normalizedString. |  |
| xs:NCName | The·value space of NCName is the set of all strings which can be used as XML element names, omitting strings that contain : characters. |  |
| xs:language | language represents formal natural language identifiers, as defined by IETF BCP 47[12]. |  |
| xs:integer | integer is derived from decimal by fixing the value of fractionDigits·to be 0 and disallowing the trailing decimal point. This results in the standard mathematical concept of the integer numbers. The ·value space· of integer is the infinite set {...,-2,-1,0,1,2,...}. The ·base type of integer is decimal. |  |
| xs:nonNegativeInteger | nonNegativeInteger has a lexical representation consisting of an optional sign followed by a non-empty finite-length sequence of decimal digits (#x30-#x39). If the sign is omitted, the positive sign ('+') is assumed. If the sign is present, it shall be "+" except for lexical forms denoting zero, which may be preceded by a positive ('+') or a negative ('-') sign. For example: 1, 0, 12678967543233, +100000. |  |
| xs:positiveInteger | positiveInteger is ·derived· from nonNegativeInteger by setting the value of minInclusive·to be 1. This results in the standard mathematical concept of the positive integer numbers. The ·value space· of positiveInteger is the infinite set {1,2,...}. The base type·of positiveInteger is nonNegativeInteger. |  |
| xs:unsignedLong | unsignedLong is derived· from nonNegativeInteger by setting the value of ·maxInclusive· to be 18446744073709551615. The base type·of unsignedLong is nonNegativeInteger. |  |
| xs:unsignedInt | unsignedInt is ·derived·from unsignedLong by setting the value of·maxInclusive·to be 4294967295. The base type·of unsignedInt is unsignedLong. |  |
| xs:unsignedShort | unsignedShort is ·derived· from unsignedInt by setting the value of maxInclusive· to be 65535. The ·base type·of unsignedShort is unsignedInt. |  |

### 6.3.3	oneM2M simple data types

Table 6.3.3-1 describes oneM2M-specific simple data type definitions. XML Schema data type definitions for these data types can be found in the XSD file called CDT-commonTypes-v1_6_0.xsd contained in ts_118004v010100p0.zip.

The types in table 6.3.3-1 are either:

- Atomic data types derived from XML Schema data types by restrictions other than enumeration

- List data types constructed from other XML Schema or oneM2M-defined atomic data types.

The oneM2M-defined enumeration data types are defined in clause 6.3.4.

Table 6.3.3-1: oneM2M Simple Data Types

| XSD type name | Type Name | Examples | Description |
| --- | --- | --- | --- |
| m2m:ID | Generic ID | //globalm2m.org | Used to represent generic IDs generated and used within oneM2M (M2M-SP-ID) |
|  |  | //globalm2m.org/C190XX7T | (CSE-ID) |
|  |  | //globalm2m.org/CSE1/123A38ZZY | (AE-ID) |
| m2m:nodeID | Node ID | urn:gsma:imei:90420156-025763-0;svn=42 | Used for Node IDs. The constraints on this type are different from those on Generic IDs (IMEI as node ID) |
| m2m:deviceID | Device ID | urn:dev:ops:012345-Set%2DTop%2DBox-0123456789 | A Device ID identifies a device using a Universally Unique IDentifier (UUID). A valid hex digit character string of UUID and the format of the URN is one of OPS URN, OS URN, IMEI URN, ESN URN, or MEID URN. |
| m2m:externalID | M2M-EXT-ID | urn:gsma:imei:90420156-025763-0;vers=0 | The External Identifier allows the Underlying Network to identify the M2M Device (e.g. ASN, MN) associated with the CSE-ID.In 3GPP case, the accessID is mapped to External Identifier as specified in TS 23.003 [17]. |
| m2m:requestID | Request ID | ab3f124a, CSE1/98821 | Used for Request IDs. This type may include the ID of the target CSE as well as a part that varies for each ID |
| m2m:nhURI | Non Hierarchical Identifier | /CSE090112/ C190XX7T | Used where a resourceID is required to be non-hierarchical |
| m2m:acpType | List of ACP Resource IDs | //IN-CSEID.m2m.myoperator.org/93405 | Used to represent a list of AccessControlPolicy identifiers. |
| m2m:labels | list of xs:token | printers networkwifi1 home_energy | A list of tokens used as keys for discovering resources (searching wifi connected printer from vendor 1) |
| m2m:triggerRecipientID | Trigger Recipient Identifier | 3010 | Used when device triggering services are requested from the Underlying Network, to identify an instance of an ASN/MN-CSE on an execution environment, to which the trigger is routed. Defined as port number in the range 0 to 65535. |
| m2m:listOfM2MID | List of M2M identifiers |  | xs:list of elements of data type m2m:ID |
| m2m:listOfMinMax | List of Time Limits | 10 2560 | xs:list of two xs:long values defining min and max limits of time intervals in units of milliseconds (value -1 representing infinite time) |
| m2m:backOffParameters | List of Backoff Parameters | 100 100 2000 | Ordered sequence of 3 values of data type xs:nonNegativeInteger representing backoffTime, backoffTimeIncrement, maximumBackoffTime (in units of milliseconds) |
| m2m:ipv4 | IPv4 address string with optional CIDR suffix | 10.125.0.0/16,122.77.12.1 | Used in m2m:accessControlRules specified in the section 6.3.5.27 |
| m2m:ipv6 | IPv6 address string with optional CIDR suffix | ::/0, Fadf:ddd0::/32, abcd:ffff:abb0:aaaa::/64 | Used in m2m:accessControlRules specified in the section 6.3.5.27 |
| m2m:countryCode | Country Code | KR | 2-character country code as defined by ISO-3166 |
| m2m:poaList | List of PointOfAccess strings | http://172.25.0.10:8080, coap://m2m.sp.com:5683 mqtt://172.25.0.10:1883 | list of xs:string. Each pointOfAccess entry in list is represented as a string containing the underlying transport protocol as well as the IP address and port (or an FQDN). |
| m2m:timestamp | Time stamp string | 20141003T112032 | DateTime string using 'Basic Format' specified in ISO8601 [27]. Time zone shall be interpreted as UTC timezone. See below for more details. |
| m2m:absRelTimestamp | absolute or relative time stamp string | 20141003T112032 (absolute time),or 3600000 (relative time) | defined as xs:union of m2m:timestamp and xs:duration data types |
| m2m:typeOfContent | Type of Content | application/xml | The media type shall be an IANA registered Media Types name, or an experimental Media Type (See [26]) ':' |
| m2m:serializations | Serialization types | application/xml application/json | A list of IANA registered media types that can be used for serialization of primitives. The permitted values are - application/xml - application/json |
| m2m:contentInfo | Content Information | application/xml:2 | A string consisting of a media type optionally followed by a m2m:encoding separated by '':'' character. See note 1 |
| m2m:eventCat | Event Category | 2 | Either - One of the values from m2m:stdEventCats or, - A user-defined category in the range 100-999 |
| m2m:eventCatWithDef | Event Category with default | 0 | Either - A value from m2m:eventCat , or - The value 0 which has the special meaning “default” |
| m2m:listOfEventCat | List of (applicable) Event Categories | 1 101 | xs:list of elements of data type m2m:eventCat |
| m2m:listOfEventCatWithDef | List of m2m:eventCatWithDef | 0 1 101 |  |
| m2m:scheduleEntry | Schedule Entry | * 0-5 2,6,10 * * * * | The string is used to describe a duration of enablement. The string format is described in clause 7.4.10.1 |
| m2m:attributeList | List of xs:NCName | poa rr | Used for the Content parameter of Retrieve request primitives. Attributes represented with their short names. |
| m2m:serviceRoles | List of SRole-IDs | “01-001” (see note 2) NOTE: This is an enumeration of String value) | Used to represent a list of SRole-IDs. |
| NOTE 1: The encoding in m2m:contentInfo may be omitted when the value was ''0 (plain)". But since default value of encoding is not allowed in future releases, it is recommended not to omit the encoding. NOTE 2: This is an enumeration of String value. |  |  |  |

The m2m:timestamp datatype uses ISO 8601 [27] Complete Representation using the Basic Format as described here:

The timestamp shall be a string containing Year, Month, Day, Hours, Minutes and Seconds components using the format YYYYMMDDThhmmss as defined in [27]. In this representation the character “T” is to indicate the start of the time of day portion.

All these components shall appear in the string; reduced representations are not permitted.

The Seconds component may optionally contain a decimal fraction. In this case the string shall contain two integer digits, followed by a comma and then one or more fractional digits, up to a maximum of six. For example YYYYMMDDThhmmss,ssssss

The timestamp string shall not contain Timezone information. All timestamps shall be interpreted as being in UTC.

A receiving or Hosting CSE shall accept a timestamp that contains fractional seconds, but it need not act on a timestamp with the level of  precision that is implied by its fractional part. For example it is acceptable for a Hosting CSE to round up an expiration time when interpreting it.

NOTE 1:	Care should be taken when developing an application that compares timestamps. This is because AE’s and CSE’s are not required to have their clocks synchronized.

NOTE 2:	As the m2m:timestamp is expressed in UTC, an AE has to be aware of the Timezone in which it is operating if it is to be able to relate the timestamp to its local time.

### 6.3.4	oneM2M enumerated data types

#### 6.3.4.1	Introduction

The oneM2M Enumeration Types are defined as extension from ''enumeration type'' which is defined in XML Schema definition [3]. The oneM2M Enumeration Types are based on <xs:integer>, and the numeric values are interpreted as specified in clause 6.3.4.2. Table 6.3.4.1-1 shows the example of Enumeration Type definition for m2m:enumFooType.

Table 6.3.4.1-1: Example of oneM2M Enumeration Type Definition

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | Interpretation-1 |  |
| 2 | Interpretation-2 |  |
| 3 | Interpretation-3 |  |
| NOTE: See clause x.x.x “title of clause” |  |  |

The oneM2M Enumeration Type definition shall be implemented as part of 
CDT-enumeration-v1_6_0.xsd. Figure 6.3.4.1-1 shows the example of XSD representation of ''m2m:enumFooType''.

<xs:simpleType name="enumFooType">

<xs:restriction base="xs:integer">

<xs:enumeration value="1"/>

<xs:enumeration value="2"/>

<xs:enumeration value="3"/>

</xs:restriction>

</xs:simpleType>

Figure 6.3.4.1-1: Example of XSD version of oneM2M Enumeration Type

#### 6.3.4.2	Enumeration type definitions

##### 6.3.4.2.1	m2m:resourceType

Table 6.3.4.2.1-1: Interpretation of resourceType

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | accessControlPolicy |  |
| 2 | AE |  |
| 3 | container |  |
| 4 | contentInstance |  |
| 5 | CSEBase |  |
| 6 | delivery |  |
| 7 | eventConfig |  |
| 8 | execInstance |  |
| 9 | group |  |
| 10 | locationPolicy |  |
| 11 | m2mServiceSubscriptionProfile |  |
| 12 | mgmtCmd |  |
| 13 | mgmtObj |  |
| 14 | node |  |
| 15 | pollingChannel |  |
| 16 | remoteCSE |  |
| 17 | request |  |
| 18 | schedule |  |
| 19 | serviceSubscribedAppRule |  |
| 20 | serviceSubscribedNode |  |
| 21 | statsCollect |  |
| 22 | statsConfig |  |
| 23 | subscription |  |
| 10001 | accessControlPolicyAnnc |  |
| 10002 | AEAnnc |  |
| 10003 | containerAnnc |  |
| 10004 | contentInstanceAnnc |  |
| 10009 | groupAnnc |  |
| 10010 | locationPolicyAnnc |  |
| 10013 | mgmtObjAnnc |  |
| 10014 | nodeAnnc |  |
| 10016 | remoteCSEAnnc |  |
| 10018 | scheduleAnnc |  |
| NOTE: See clause 6.4.1 “Request message parameter data types”. |  |  |

##### 6.3.4.2.2	m2m:cseTypeID

Used for cseType attribute of <CSEBase> resource.

Table 6.3.4.2.2-1: Interpretation of cseTypeID

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | IN_CSE |  |
| 2 | MN_CSE |  |
| 3 | ASN_CSE |  |
| NOTE: See clause 7.4.5 “Resource Type remoteCSE”. |  |  |

##### 6.3.4.2.3	m2m:locationSource

Used for locationSource attribute of <locationPolicy> resource.

Table 6.3.4.2.3-1: Interpretation of locationSource

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | Network_based |  |
| 2 | Device_based |  |
| 3 | Sharing_based |  |
| NOTE: See clause 7.4.11 “Resource Type locationPolicy” |  |  |

##### 6.3.4.2.4	m2m:stdEventCats

Used for ec parameter in request and eventCat attribute of <delivery> resource and cmdh policy resource types.

Table 6.3.4.2.4-1: Interpretation of stdEventCats

| Value | Interpretation | Note |
| --- | --- | --- |
| 2 | Immediate |  |
| 3 | BestEffort |  |
| 4 | Latest |  |
| NOTE: See clause 7.4.12 “Resource Type delivery” and clauseD.12 “Resource cmdhPolicy” |  |  |

##### 6.3.4.2.5	m2m:operation

Used for Operation parameter in request and operation attribute in <request> resource as well as operationMonitor.

Table 6.3.4.2.5-1: Interpretation of operation

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | Create |  |
| 2 | Retrieve |  |
| 3 | Update |  |
| 4 | Delete |  |
| 5 | Notify |  |
| NOTE: See clause 6.4.1 “Request message parameter data types” |  |  |

##### 6.3.4.2.6	m2m:responseType

Used for Response Type parameter (as a part of responseTypeInfo, See Clause 6.3.5.30) in request.

Table 6.3.4.2.6-1: Interpretation of responseType

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | nonBlockingRequestSynch |  |
| 2 | nonBlockingRequestAsynch |  |
| 3 | blockingRequest |  |
| NOTE: See clause 6.4.1 “Request message parameter data types” |  |  |

##### 6.3.4.2.7	m2m:resultContent

Used for Result Content parameter in request.

Table 6.3.4.2.7-1: Interpretation of resultContent

| Value | Interpretation | Note |
| --- | --- | --- |
| 0 | nothing |  |
| 1 | attributes |  |
| 2 | hierarchical address |  |
| 3 | hierarchical address and attributes |  |
| 4 | attributes and child resources |  |
| 5 | attributes and child resource references |  |
| 6 | child resource references |  |
| 7 | original resource |  |
| NOTE: See clause 6.4.1 “Request message parameter data types” |  |  |

##### 6.3.4.2.8	m2m:discResType

Used in metaInformation attribute in <request> resource

Table 6.3.4.2.8-1: Interpretation of discResType

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | structured |  |
| 2 | unstructured |  |
| NOTE: See clause 6.4.1 “Request message parameter data types” |  |  |

##### 6.3.4.2.9	m2m:responseStatusCode

See clause 6.6.3 "Current Response Status Codes"

Table 6.3.4.2.9-1: Interpretation of responseStatusCode

| Value | Interpretation | Note |
| --- | --- | --- |
| ('Numeric Code' in Clause 6.6.3) | ('Description' in clause 6.6.3) |  |

##### 6.3.4.2.10	m2m:requestStatus

Used for requestStatus attribute in <request> resource.

Table 6.3.4.2.10-1: Interpretation of requestStatus

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | COMPLETED |  |
| 2 | FAILED |  |
| 3 | PENDING |  |
| 4 | FORWARDED |  |
| NOTE: See clause 7.4.13 “Resource Type request” |  |  |

##### 6.3.4.2.11	m2m:memberType

Used for memberType attribute in <group> resource.

Table 6.3.4.2.11-1: Interpretation of memberType

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | accessControlPolicy |  |
| 2 | AE |  |
| 3 | container |  |
| 4 | contentInstance |  |
| 5 | CSEBase |  |
| 6 | delivery |  |
| 7 | eventConfig |  |
| 8 | execInstance |  |
| 9 | group |  |
| 10 | locationPolicy |  |
| 11 | m2mServiceSubscription |  |
| 12 | mgmtCmd |  |
| 13 | mgmtObj |  |
| 14 | node |  |
| 15 | pollingChannel |  |
| 16 | remoteCSE |  |
| 17 | request |  |
| 18 | schedule |  |
| 19 | serviceSubscribedAppRule |  |
| 20 | serviceSubscribedNode |  |
| 21 | statsCollect |  |
| 22 | statsConfig |  |
| 23 | subscription |  |
| 10001 | accessControlPolicyAnnc |  |
| 10002 | AEAnnc |  |
| 10003 | containerAnnc |  |
| 10004 | contentInstanceAnnc |  |
| 10009 | groupAnnc |  |
| 10010 | locationPolicyAnnc |  |
| 10013 | mgmtObjAnnc |  |
| 10014 | nodeAnnc |  |
| 10016 | remoteCSEAnnc |  |
| 10018 | scheduleAnnc |  |
| 24 | mixed | A mixture of the resource types from 1 to 23, 10001 to 10004, 10009 to 10010, 10013 to 10014 and 10016 to 10018 as listed above. |
| NOTE: See clause 7.4.14 “Resource Type group” |  |  |

##### 6.3.4.2.12	m2m:consistencyStrategy

Used for consistencyStrategy attribute in <group> resource.

Table 6.3.4.2.12-1: Interpretation of consistencyStrategy

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | ABANDON_MEMBER |  |
| 2 | ABANDON_GROUP |  |
| 3 | SET_MIXED |  |
| NOTE: See clause 7.4.14 “Resource Type group” |  |  |

##### 6.3.4.2.13	m2m:cmdType

Used for cmdType attribute in <mgmtCmd> resource.

Table 6.3.4.2.13-1: Interpretation of cmdType

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | RESET |  |
| 2 | REBOOT |  |
| 3 | UPLOAD |  |
| 4 | DOWNLOAD |  |
| 5 | SOFTWAREINSTALL |  |
| 6 | SOFTWAREUNINSTALL |  |
| 7 | SOFTWAREUPDATE |  |
| NOTE: See clause 7.4.17 “Resource Type mgmtCmd” |  |  |

##### 6.3.4.2.14	m2m:execModeType

Used for execModeType attribute in <mgmtCmd> and <execInstance> resource.

Table 6.3.4.2.14-1: Interpretation of execModetType

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | IMMEDIATEONCE |  |
| 2 | IMMEDIATEREPEAT |  |
| 3 | RANDOMONCE |  |
| 4 | RANDOMREPEAT |  |
| NOTE: See clause 7.4.17 “Resource Type mgmtCmd” and clause 7.4.18 “Resource Type execInstance” |  |  |

##### 6.3.4.2.15	m2m:execStatusType

Used for execStatusType attribute in <execInstance> resource.

Table 6.3.4.2.15-1: Interpretation of execStatusType

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | INITIATED |  |
| 2 | PENDING |  |
| 3 | FINISHED |  |
| 4 | CANCELLING |  |
| 5 | CANCELLED |  |
| 6 | STATUS_NON_CANCELLABLE |  |
| NOTE: See clause 7.4.18 “Resource Type execInstance” |  |  |

##### 6.3.4.2.16	m2m:execResultType

Used for execStatusType attribute in <execInstance> resource.

Table 6.3.4.2.16-1: Interpretation of execResultType

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | STATUS_REQUEST_UNSUPPORTED |  |
| 2 | STATUS_REQUEST DENIED |  |
| 3 | STATUS_CANCELLATION_DENIED |  |
| 4 | STATUS_INTERNAL_ERROR |  |
| 5 | STATUS_INVALID_ARGUMENTS |  |
| 6 | STATUS_RESOURCES_EXCEEDED |  |
| 7 | STATUS_FILE_TRANSFER_FAILED |  |
| 8 | STATUS_FILE_TRANSFER_SERVER_AUTHENTICATION_FAILURE |  |
| 9 | STATUS_UNSUPPORTED_PROTOCOL |  |
| 10 | STATUS_UPLOAD_FAILED |  |
| 11 | STATUS_FILE_TRANSFER_FAILED_MULTICAST_GROUP_UNABLE_JOIN |  |
| 12 | STATUS_FILE_TRANSFER_FAILED_SERVER_CONTACT_FAILED |  |
| 13 | STATUS_FILE_TRANSFER_FAILED_FILE_ACCESS_FAILED |  |
| 14 | STATUS_FILE_TRANSFER_FAILED_DOWNLOAD_INCOMPLETE |  |
| 15 | STATUS_FILE_TRANSFER_FAILED_FILE_CORRUPTED |  |
| 16 | STATUS_FILE_TRANSFER_FILE_AUTHENTICATION_FAILURE |  |
| 19 | STATUS_FILE_TRANSFER_WINDOW_EXCEEDED |  |
| 20 | STATUS_INVALID_UUID_FORMAT |  |
| 21 | STATUS_UNKNOWN_EXECUTION_ENVIRONMENT |  |
| 22 | STATUS_DISABLED_EXECUTION_ENVIRONMENT |  |
| 23 | STATUS_EXECUTION_ENVIRONMENT_MISMATCH |  |
| 24 | STATUS_DUPLICATE_DEPLOYMENT_UNIT |  |
| 25 | STATUS_SYSTEM_RESOURCES_EXCEEDED |  |
| 26 | STATUS_UNKNOWN_DEPLOYMENT_UNIT |  |
| 27 | STATUS_INVALID_DEPLOYMENT_UNIT_STATE |  |
| 28 | STATUS_INVALID_DEPLOYMENT_UNIT_UPDATE_DOWNGRADE_DISALLOWED |  |
| 29 | STATUS_INVALID_DEPLOYMENT_UNIT_UPDATE_UPGRADE_DISALLOWED |  |
| 30 | STATUS_INVALID_DEPLOYMENT_UNIT_UPDATE_VERSION_EXISTS |  |
| NOTE: See clause 7.4.17 “Resource Type mgmtCmd” |  |  |

##### 6.3.4.2.17	m2m:pendingNotification

This is used for pendingNotification attribute in <subscription> resource.

Table 6.3.4.2.17-1: Interpretation of pendingNotification

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | sendLatest |  |
| 2 | sendAllPending |  |
| NOTE: See clause 7.4.9 “Resource Type subscription” |  |  |

##### 6.3.4.2.18	m2m:notificationContentType

Table 6.3.4.2.18-1: Interpretation of notificationContentType

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | All Attributes |  |
| 2 | Modefied Attributes |  |
| 3 | ResourceID |  |
| NOTE: See clause 7.4.9 “Resource Type subscription” |  |  |

##### 6.3.4.2.19	m2m:notificationEventType

Used for eventNotificationCriteria conditions.

Table 6.3.4.2.19-1: Interpretation of notificationEventType

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | Update_of_Resource | Default |
| 2 | Delete_of_Resource |  |
| 3 | Create_of_Direct_Child_Resource |  |
| 4 | Delete_of_Direct_Child_Resource |  |

##### 6.3.4.2.20	m2m:status

This is used for [software], [firmware] resources.

Table 6.3.4.2.20-1: Interpretation of status

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | Successful |  |
| 2 | Failure |  |
| 3 | In_Process |  |
| NOTE: See clause D.2, D.3 firmware and software management. |  |  |

##### 6.3.4.2.21	m2m:batteryStatus

This is used for [battery] resource.

Table 6.3.4.2.21-1: Interpretation of batteryStatus

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | NORMAL | The battery is operating normally and not on power. |
| 2 | CHARGING | The battery is currently charging. |
| 3 | CHARGING_COMPLETE | The battery is fully charged and still on power. |
| 4 | DAMAGED | The battery has some problem. |
| 5 | LOW_BATTERY | The battery is low on charge. |
| 6 | NOT_INSTALLED | The battery is not installed. |
| 7 | UNKNOWN | The battery information is not available. |
| NOTE: See clause D.7 battery management. |  |  |

##### 6.3.4.2.22	m2m:mgmtDefinition

This is used for <mgmtObj> resource.

Table 6.3.4.2.22-1: Interpretation of mgmtDefinition

| Value | Interpretation | Note |
| --- | --- | --- |
| 1001 | [firmware] |  |
| 1002 | software |  |
| 1003 | memory |  |
| 1004 | areaNwkInfo |  |
| 1005 | areaNwkDeviceInfo |  |
| 1006 | battery |  |
| 1007 | deviceInfo |  |
| 1008 | deviceCapability |  |
| 1009 | reboot |  |
| 1010 | eventLog |  |
| 1011 | cmdhPolicy |  |
| 1012 | activeCmdhPolicy |  |
| 1013 | cmdhDefaults |  |
| 1014 | cmdhDefEcValue |  |
| 1015 | cmdhEcDefParamValues |  |
| 1016 | cmdhLimits |  |
| 1017 | cmdhNetworkAccessRules |  |
| 1018 | cmdhNwAccessRule |  |
| 1019 | cmdhBuffer |  |
| 0 | Unspecified | Permits vendor-specific extensions |
| NOTE: See clause 7.4.16 mgmtObj |  |  |

##### 6.3.4.2.23	m2m:logTypeId

Used for the logTypeId attribute of [eventLog] Management Resource.

Table 6.3.4.2.23-1: Interpretation of logTypeId

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | System |  |
| 2 | Security |  |
| 3 | Event |  |
| 4 | Trace |  |
| 5 | Panic |  |

##### 6.3.4.2.24	m2m:logStatus

Used for the logStatus attribute of [eventLog] Management Resource.

Table 6.3.4.2.24-1: Interpretation of logStatus

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | Started | the logging activity is started |
| 2 | Stopped | the logging activity is stopped |
| 3 | Unknown | the current status of the logging activity is unknown. |
| 4 | NotPresent | the log data is not present and the logData attribute shall be ignored. |
| 5 | Error | error conditions for the logging activities, and the logging is stopped. |

##### 6.3.4.2.25	m2m:eventType

Used for eventType attribute in <eventConfig> resource.

Table 6.3.4.2.25-1: Interpretation of eventType

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | DATAOPERATION |  |
| 2 | STORAGEBASED |  |
| 3 | TIMERBASED |  |
| NOTE: See clause 7.4.25 "Resource Type eventConfig" |  |  |

##### 6.3.4.2.26	m2m:statsRuleStatusType

Used for statsRuleStatusType attribute in <statsCollect> resource.

Table 6.3.4.2.26-1: Interpretation of statsRuleStatusType

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | ACTIVE |  |
| 2 | INACTIVE |  |
| NOTE: See clause 7.4.26 "Resource Type statsCollect" |  |  |

##### 6.3.4.2.27	m2m:statModelType

Used for statModelType attribute in <statsCollect> resource.

Table 6.3.4.2.27-1: Interpretation of statModelType

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | EVENTBASED |  |
| NOTE: See clause 7.4.26 "Resource Type statsCollect" |  |  |

##### 6.3.4.2.28	m2m:encodingType

Used for describing encoding type which is applied on the content attribute of the contentInstance resource.

Table 6.3.4.2.28-1: Interpretation of encodingType

| Value | Interpretation | Note |
| --- | --- | --- |
| 0 | Plain - no transfer encoding is applied |  |
| 1 | base64 encoding (see [9]) is applied on string data |  |
| 2 | base64 encoding (see [9]) is applied on binary data |  |

##### 6.3.4.2.29	m2m:accessControlOperations

Used for accessControlPolicys.

Table 6.3.4.2.29-1: Interpretation of accessControlOperations

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | CREATE |  |
| 2 | RETRIEVE |  |
| 4 | UPDATE |  |
| 8 | DELETE |  |
| 16 | NOTIFY |  |
| 32 | DISCOVERY |  |
| NOTE: Combinations of these values are specified by adding them together. For example the value 5 is interpreted as "CREATE and UPDATE" |  |  |

##### 6.3.4.2.30	m2m:SRole-ID

Used for <m2mServiceSubscriptionProfile>

Table 6.3.4.2.30-1: Interpretation of SRole-ID

| Value | Interpretation | Note |
| --- | --- | --- |
| "01-001" | Software Management |  |
| "02-001" | Device Configuration |  |
| "02-002" | Device Diagnostics and Management |  |
| "02-003" | Device Firmware Management |  |
| "02-004" | Device Topology |  |
| "03-001" | Location |  |
| "04-001" | Basic Data |  |
| "05-001" | Onboarding |  |
| "06-001" | Security Administration |  |
| "07-001" | Groups Management |  |
| "08-001" | Event Collection |  |
| NOTE: This is an enumeration of String values |  |  |

##### 6.3.4.2.31	m2m:filterUsage

Used in m2m:filterCriteria.

Table 6.3.4.2.31-1: Interpretation of filterUsage

| Value | Interpretation | Note |
| --- | --- | --- |
| 1 | Discovery Criteria |  |
| 2 | Conditional Retrieval | This is the default value when the filterUsage condition is not present in a Retrieve request. |

### 6.3.5	Complex data types

#### 6.3.5.1	Introduction

The present clause defines structured information for specific use in oneM2M protocol.  These types are defined to be xs:sequence complex types, unless specified otherwise. XML Schema data type definitions for these data types can be found in the XSD file called  CDT-commonTypes-v1_6_0.xsd contained in ts_118004v010100p0.zip.

In addition, each oneM2M resource has a corresponding complex data type. These are described in clause 6.5.

#### 6.3.5.2	m2m:deliveryMetaData

Used for deliveryMetaData attribute in <delivery> resource.

Table 6.3.5.2-1: Type Definition of m2m:deliveryMetadata

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| tracingOption | xs:boolean | 1 |  |
| tracingInfo | m2m:listOfM2MID | 0..1 |  |

#### 6.3.5.3	m2m:aggregatedRequest

Used for aggregatedRequest attribute in <delivery> resource.

Table 6.3.5.3-1: Type Definition of m2m:aggregatedRequest

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| request | (anonymous) | 1..n |  |
| request/operation | m2m:operation | 1 | See clause 6.3.4.2.5 |
| request/to | xs:anyURI | 1 |  |
| request/from | m2m:ID | 1 | See clause 6.3.3 |
| request/requestIdentifier | m2m:requestID | 1 | See clause 6.3.3 |
| request/primitiveContent | m2m:primitiveContent | 0..1 | See clause 6.3.5.5 |
| request/metaInformation | m2m:metaInformation | 0..1 | See clause 6.3.5.4 |

#### 6.3.5.4	m2m:metaInformation

Used for metaInformation attribute in <request> resource, and m2m:aggregatedRequest data type.

Table 6.3.5.4-1: Type Definition of m2m:metaInformation

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| resourceType | m2m:resourceType | 0..1 | See clause 6.3.4.2.1 |
| originatingTimestamp | m2m:timestamp | 0..1 |  |
| requestExpirationTimestamp | m2m:absRelTimestamp | 0..1 |  |
| resultExpirationTimestamp | m2m:absRelTimestamp | 0..1 |  |
| operationExecutionTime | m2m:absRelTimestamp | 0..1 |  |
| responseType | m2m:responseTypeInfo | 0..1 | See clause 6.3.4.2.6 |
| resultPersistence | m2m:absRelTimestamp | 0..1 |  |
| resultContent | m2m:resultContent | 0..1 | See clause 6.3.4.2.7 |
| eventCategory | m2m:eventCat | 0..1 | See clause 6.3.3 |
| deiveryAggregation | xs:boolean | 0..1 |  |
| groupRequestIdentifier | xs:string | 0..1 |  |
| filterCriteria | m2m:filterCriteria | 0..1 | See clause 6.3.5.8 |
| discoveryResultType | m2m:discResType | 0..1 | See clause 6.3.4.2.8 |

#### 6.3.5.5	m2m:primitiveContent

Used for Content parameter in request/response primitive and the content attribute in <request> resource.

See clause 7.2.1.1 and 7.2.1.2 .

#### 6.3.5.6	m2m:batchNotify

Used for batchNotify attribute in <subscription> resource.

Table 6.3.5.6-1: Type Definition of m2m:batchNotify

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| number | xs:nonNegativeInteger | 1 |  |
| duration | xs:duration | 1 | If the duration is not given by the Originator, the Hosting CSE shall set this with the default duration value as given by the M2M Service Provider. |

#### 6.3.5.7	m2m:eventNotificationCriteria

Used for eventNotificationCriteria of a <subscription> resource.

Table 6.3.5.7-1:Type Definition of m2m:eventNotificationCriteria

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| createdBefore | m2m:timestamp | 0..1 |  |
| createdAfter | m2m:timestamp | 0..1 |  |
| modifiedSince | m2m:timestamp | 0..1 |  |
| unmodifiedSince | m2m:timestamp | 0..1 |  |
| stateTagSmaller | xs:positiveInteger | 0..1 |  |
| stateTagBigger | xs:nonNegativeInteger | 0..1 |  |
| expireBefore | m2m:timestamp | 0..1 |  |
| expireAfter | m2m:timestamp | 0..1 |  |
| sizeAbove | xs:nonNegativeInteger | 0..1 |  |
| sizeBelow | xs:positiveInteger | 0..1 |  |
| operationMonitor | m2m:operation | 0..5 |  |
| attribute | m2m:attribute | 0..n |  |
| notificationEventType | m2m:notificationEventType | 0..4 |  |

#### 6.3.5.8	m2m:filterCriteria

Used indirectly in the <request> resource and for the  Filter Criteria parameter in a request.

Table 6.3.5.8-1: Type Definition of m2m:filterCriteria

| Element Path | Element Data Type | Mutiplicity | Note |
| --- | --- | --- | --- |
| createdBefore | m2m:timestamp | 0..1 |  |
| createdAfter | m2m:timestamp | 0..1 |  |
| modifiedSince | m2m:timestamp | 0..1 |  |
| unmodifiedSince | m2m:timestamp | 0..1 |  |
| stateTagSmaller | xs:positiveInteger | 0..1 |  |
| stateTagBigger | xs:nonNegativeInteger | 0..1 |  |
| expireBefore | m2m:timestamp | 0..1 |  |
| expireAfter | m2m:timestamp | 0..1 |  |
| labels | m2m:labels | 0..1 |  |
| resourceType | list of m2m:resourceType | 0..1 |  |
| sizeAbove | xs:nonNegativeInteger | 0..1 |  |
| sizeBelow | xs:positiveInteger | 0..1 |  |
| contentType | m2m:typeOfContent | 0..n |  |
| attribute | m2m:attribute | 0..n |  |
| filterUsage | m2m:filterUsage | 0..1 |  |
| limit | xs:nonNegativeInteger | 0..1 |  |

#### 6.3.5.9	m2m:attribute

Used in m2m:eventNotificationCriteria and m2m:filterCriteria.

Table 6.3.5.9-1: Type Definition of m2m:attribute

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| name | xs:NCName | 1 |  |
| value | xs:anyType | 1 |  |

#### 6.3.5.10	(void)

#### 6.3.5.11	m2m:scheduleEntries

Table 6.3.5.11-1: Type Definition of m2m:scheduleEntries

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| scheduleEntry | m2m:scheduleEntry | 1..n |  |

#### 6.3.5.12	m2m:aggregatedNotification

Used in the Notification Data Object.

Table 6.3.5.12-1: Type Definition of m2m:aggregatedNotification

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| notification | m2m:notification | 1..n |  |

#### 6.3.5.13	m2m:notification

Table 6.3.5.13-1: Type Definition of m2m:notification

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| notificationEvent | (anonymous) | 0..1 |  |
| notificationEvent/representation | xs:anyType | 0..1 | Representation of resource modification in XML/JSON representation. |
| notificationEvent/operationMonitor | (anonymous) | 0..1 |  |
| notificationEvent/operationMonitor/operation | m2m:operation | 1 | m2m:operation This element shall only be present if the operationMonitor parent element is present. Otherwise it shall not. |
| notificationEvent/operationMonitor/originator | m2m:ID | 1 | m2m:ID This element shall only be present if the operationMonitor parent element is present. Otherwise it shall not. |
| notificationEvent/notificationEventType | m2m:notificationEventType | 1 |  |
| verificationRequest | xs:boolean | 0..1 |  |
| subscriptionDeletion | xs:boolean | 0..1 |  |
| subscriptionReference | xs:anyURI | 1 |  |
| creator | m2m:ID | 0..1 |  |
| notificationForwardingURI | xs:anyURI | 0..1 |  |

#### 6.3.5.14	m2m:actionStatus

Table 6.3.5.14-1: Type Definition of m2m:actionStatus

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| action | xs:anyURI | 0..1 | Reference to the action (represented by a resource attribute) being performed |
| status | m2m:status | 0..1 | Indicates the status of the operation is successful, failure or in process. See table 6.3.4.2.20 |

#### 6.3.5.15	m2m:anyArgType

Table 6.3.5.15-1: Type Definition of m2m:anyArgType

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| name | xs:NCName |  |  |
| value | xs:anyType |  |  |

#### 6.3.5.16	m2m:resetArgsType

Table 6.3.5.16-1: Type Definition of m2m:resetArgsType

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| anyArg | m2m:anyArgType | 0..n |  |

#### 6.3.5.17	m2m:rebootArgsType

Table 6.3.5.17-1: Type Definition of m2m:rebootArgsType

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| anyArg | m2m:anyArgType | 0..n |  |

#### 6.3.5.18	m2m:uploadArgsTypes

Table 6.3.5.18-1: Type Definition of m2m:uploadArgsType

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| fileType | xs:string | 1 |  |
| URL | xs:anyURI | 1 |  |
| username | xs:string | 1 |  |
| password | xs:string | 1 |  |
| anyArg | m2m:anyArgType | 0..n |  |

#### 6.3.5.19	m2m:downloadArgsType

Table 6.3.5.19-1: Type Definition of m2m:downloadArgsType

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| fileType | xs:string | 1 |  |
| URL | xs:anyURI | 1 |  |
| username | xs:string | 1 |  |
| password | xs:string | 1 |  |
| filesize | xs:positiveInteger | 1 |  |
| targetFile | xs:string | 1 |  |
| delaySeconds | xs:positiveInteger | 1 |  |
| successURL | xs:anyURI | 1 |  |
| startTime | m2m:timestamp | 1 |  |
| completeTime | m2m:timestamp | 1 |  |
| anyArg | m2m:anyArgType | 0..n |  |

#### 6.3.5.20	m2m:softwareInstallArgsType

Table 6.3.5.20-1: Type Definition of m2m:softwareInstallArgsType

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| URL | xs:anyURI | 1 |  |
| UUID | xs:string | 1 |  |
| username | xs:string | 1 |  |
| password | xs:string | 1 |  |
| executionEnvRef | xs:string | 1 |  |
| anyArg | m2m:anyArgType | 0..n |  |

#### 6.3.5.21	m2m:softwareUpdateArgsType

Table 6.3.5.21-1: Type Definition of m2m:softwareUpdateArgsType

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| UUID | xs:string | 1 |  |
| version | xs:string | 1 |  |
| URL | xs:anyURI | 1 |  |
| username | xs:string | 1 |  |
| password | xs:string | 1 |  |
| executionEnvRef | xs:string | 1 |  |
| anyArg | m2m:anyArgType | 0..n |  |

#### 6.3.5.22	m2m:softwareUninstallArgsType

Table 6.3.5.22-1: Type Definition of m2m:softwareUninstallArgsType

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| UUID | xs:string | 1 |  |
| version | xs:string | 1 |  |
| executionEnvRef | xs:string | 1 |  |
| anyType | m2m:anyArgType | 0..n |  |

#### 6.3.5.23	m2m:execReqArgsListType

Table 6.3.5.23-1: Type Definition of m2m:execReqArgsListType

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| reset | m2m:resetArgsType | 0..n |  |
| reboot | m2m:rebootArgsType | 0..n |  |
| upload | m2m:uploadArgsType | 0..n |  |
| download | m2m:downloadArgsType | 0..n |  |
| softwareInstall | m2m:softwareInstallArgsType | 0..n |  |
| softwareUpdate | m2m:softwareUpdateType | 0..n |  |
| softwareUninstall | m2m:softwareUninstallArgsType | 0..n |  |

This type is an xs:choice.  It shall contain elements from no more than one row listed in the table above.

#### 6.3.5.24	m2m:mgmtLinkRef

Table 6.3.5.24-1: Type Definition of m2m:mgmtLinkRef

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| (base content) | xs:anyURI | 1 | URI (of type xs:anyURI) with name and type attributes. |
| @name | xs:NCName | 1 | The name attribute represents the name of the referenced resource instance. |
| @type | m2m:mgmtDefinition | 1 | The type attribute is restricted to the allowed specializations of resource type <mgmtObj>. |

In the above table, names of XML schema attributes are prefixed with a “@” character to differentiate these from Resource attribute names. The “@” character is not part of the actual attribute name.

#### 6.3.5.25	m2m:resourceWrapper

This data type is used for the m2m:resource root element of the Content parameter of response primitives as defined in clause 7.5.2.

Table 6.3.5.25-1: Type Definition of m2m:resourceWrapper

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| m2m:<resourceType> | (anonymous) | 1 | A representation of a Resource with specific type as described in clause 7.4 |
| URI | xs:anyURI | 1 | Hierarchical URI of the resource. |

#### 6.3.5.26	m2m:setOfAcrs

Table 6.3.5.26-1: Type Definition of m2m:setOfAcrs

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| accessControlRules | m2m:accessControlRule | 0..n | Data type of privileges and selfPrivileges attributes. |

#### 6.3.5.27	m2m:accessControlRule

Table 6.3.5.27-1: Type Definition of m2m:accessControlRule

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| accessControlOriginators | list of xs:anyURI | 1 | Reserved character '*' represents any Originators qualify the accessControlOriginators. |
| accessControlOperations | m2m:accessControlOperations | 1 |  |
| accessControlContexts |  | 0..n |  |
| accessControlContexts/accessControlWindow | m2m:scheduleEntry | 0..n |  |
| accessControlContexts/accessControlIpAddresses |  | 0..1 |  |
| accessControlContexts/accessControlIpAddresses/ipv4Addresses | list of m2m:ipv4 | 0..1 | List of IPv4 addresses. |
| accessControlContexts/accessControlIpAddresses/ipv6Addresses | list of m2m:ipv6 | 0..1 | List of IPv6 addresses. |
| accessControlContexts/accessControlLocationRegions | m2m:locationRegion | 0..1 |  |
| NOTE: Some of the above elements are defined in clause 9.6.2 of ETSI TS 118 101 [6] with slightly different names as follows (name in parenthesis used in ETSI TS 118 101): accessControlWindow (accessControlTimeWindow), accessControlIpAddresses (accessControlIpAddress). |  |  |  |

The accessControlContexts/accessControlIpAddresses element may include either the ipv4Addresses element, ipv6Addresses element, or both elements.

Each individual IPv4 address of data type m2m:ipv4 in the list of IPv4 addresses is represented in dotted-decimal notation with optional Classless Inter-Domain Routing (CIDR) suffix in accordance with IETF RFC 4632 [29]. Each individual IPv6 address of data type m2m:ipv6 in the list of IPv6 addresses is represented in colon separated groups of hexadecimal digits with optional network prefix in accordance with IETF RFC 5952 [30]. Example IPv4 and IPv6 addresses which comply with data types m2m:ipv4 and m2m:ipv6, respectively, are given in table 6.3.2-1.

#### 6.3.5.28	m2m:locationRegion

Table 6.3.5.28-1: Type Definition of m2m:locationRegion

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| circRegion | List of 3 xs:float | 0..1 | The values represent latitude (+/-90 degrees), longitude (+/-180 degrees), and radius (metres). |
| countryCode | list of m2m:countryCode | 0..1 |  |

This is an xs:choice. A  locationRegion shall contain either:

- A countryCode element, in which case circRegion shall not appear, or

A circRegion element, in which case countryCode shall not appear

#### 6.3.5.29	m2m:childResourceRef

Table 6.3.5.29-1: Type Definition of m2m:childResourceRef

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| (base content) | xs:anyURI | 1 | URI of the child resource. |
| @name | xs:NCName | 1 | Gives the name of the child resource pointed to by the URI. |
| @type | m2m:resourceType | 1 | Gives the resourceType of the child resource pointed to by the URI. |

In the above table, names of XML schema attributes are prefixed with a “@” character to differentiate these from Resource attribute names. The “@” character is not part of the actual attribute name.

#### 6.3.5.30	m2m:responseTypeInfo

Table 6.3.5.30-1: Type Definition of m2m:responseTypeInfo

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| responseTypeValue | m2m:responseType | 1 | See clause 6.3.4.2.6. |
| notificationURI | List of xs:anyURI | 0..1 | This element may be included only when the responseType is set to "2" (nonBlockingRequestAsynch). Empty list in this element shall be allowed. See Clause 7.5.1.2.5. |

#### 6.3.5.31	m2m:rateLimit

Used in <subscription>.

Table 6.3.5.31-1: Type Definition of m2m:rateLimit

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| maxNrOfNotify | xs:nonNegativeInteger | 0..1 |  |
| timeWindow | xs:duration | 0..1 |  |

#### 6.3.5.32	m2m:operationResult

Used for operationResult attribute in <request> resource.

NOTE: This data type corresponds to the sequence of elements in the response primitive defined in clause 6.4.2.

Table 6.3.5.32-1: Type Definition of m2m:operationResult

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| response Status Code | m2m:responseStatusCode | 1 | See clause 6.3.4.2.9. |
| request Identifier | m2m:requestID | 1 | See clause 6.3.3. |
| primitiveContent | m2m:primitiveContent | 0..1 | See clause 6.3.5.5. |
| to | xs:anyURI | 0..1 |  |
| from | m2m:ID | 0..1 | See clause 6.3.3. |
| originating Timestamp | m2m:timestamp | 0..1 |  |
| result Expiration Timestamp | m2m:absRelTimestamp | 0..1 |  |
| event Category | m2m:eventCat | 0..1 | See clause 6.3.3. |

#### 6.3.5.33	m2m:aggregatedResponse

Used when aggregating responses by a group.

Table 6.3.5.33-1: Type Definition of m2m:aggregatedResponse

| Element Path | Element Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| responsePrimitive | See Table 6.4.2-1 for detail. | 1..n |  |

### 6.3.6	Universal and Common attributes

ETSI TS 118 101 Functional Architecture [6] defines a number of Universal Attributes (which appear in all resources) and Common Attributes (which appear in more than one resource and have the same meaning whenever they do appear). The type and values shall be supported according to the description given inTable 6.3.6-1.

If a Resource is represented as an XML document then the resource attributes (if present) appear in the order listed in this table. They appear before any resource-specific attributes.

Table 6.3.6-1: Universal and Common Attributes

| Attribute Name | Data Type | Value restrictions and Notes |
| --- | --- | --- |
| resourceType | m2m:resourceType | This attribute is only determined at creation time by the hosting CSE. |
| resourceID | m2m:ID | This attribute is determined at creation time by the hosing CSE and used for non hierarchical addressing method. |
| parentID | m2m:nhURI | This attribute is determined by the hosting CSE and specified in all resource types.For <CSEBase> however, the value of this attribute shall be an empty string. |
| creationTime | m2m:timestamp | This attribute is determined by the hosting CSE when the resource is locally created. |
| lastModifiedTime | m2m:timestamp | This attribute is determined by the hosting CSE when the addressed resource is modified by means of the UPDATE operation. |
| labels | m2m:labels | Absence of this attribute means there are no labels. |
| accessControlPolicyIDs | m2m:acpType | accessControlPolicyIDs. |
| expirationTime | m2m:timestamp | expirationTime. |
| link | xs:anyURI | Absence of this attribute means that this is not an announced resource. |
| announceTo | list of xs:anyURI | Absence of this attribute means that this is not an announced resource. |
| announcedAttribute | list of xs:NCName | Absence of this attribute means that this is not an announced resource. |
| stateTag | xs:nonNegativeInteger | This attribute is determined by the hosting CSE. When a resource is created this counter is set to '0' and it will be incremented on every modification of the resource. |
| resourceName | xs:NCName |  |

Table 6.3.6-2 describes some complex types that group together the universal and common attributes, to be used by Resource Type definitions. Note that stateTag  only appears in four resource types, and so is not included in these definitions, instead it is declared in the XSD files of the resources that need it.

Table 6.3.6-2: Complex Data Types declaring groups of resource common attributes

| XSD type name | Child Elements | Child Element Datatype | Multiplicity | Description |
| --- | --- | --- | --- | --- |
| m2m:resource | @resourceName | xs:NCName | 1 |  |
|  | resourceType | m2m:resourceType | 1 |  |
|  | resourceID | m2m:ID | 1 |  |
|  | parentID | m2m:nhURI | 1 |  |
|  | creationTime | m2m:timestamp | 1 |  |
|  | lastModifiedTime | m2m:timestamp | 1 |  |
|  | labels | m2m:labels | 0..1 |  |
| m2m:regularResource | @resourceName | xs:NCName | 1 | Declares the universal / common attributes included in the non-announceable resource types. |
|  | resourceType | m2m:resourceType | 1 |  |
|  | resourceID | m2m:ID | 1 |  |
|  | parentID | m2m:nhURI | 1 |  |
|  | accessControlPolicyIDs | m2m:acpType | 0..1 |  |
|  | creationTime | m2m:timestamp | 1 |  |
|  | expirationTime | m2m:timestamp | 1 |  |
|  | lastModifiedTime | m2m:timestamp | 1 |  |
|  | stateTag | xs:nonNegativeInteger | 1 |  |
|  | Labels | m2m:labels | 0..1 |  |
|  | accessControlPolicyIDs | m2m:acpType | 0..1 |  |
|  | expirationTime | m2m:timestamp | 1 |  |
| m2m:announceableResource | @resourceName | xs:NCName | 1 | Declares the universal / common attributes included in the majority of announceable resource types. |
|  | resourceType | m2m:resourceType | 1 |  |
|  | resourceID | m2m:ID | 1 |  |
|  | parentID | xs:anyURI | 1 |  |
|  | accessControlPolicyIDs | m2m:acpType | 0..1 |  |
|  | creationTime | m2m:timestamp | 1 |  |
|  | expirationTime | m2m:timestamp | 1 |  |
|  | lastModifiedTime | m2m:timestamp | 1 |  |
|  | stateTag | xs:nonNegativeInteger | 1 |  |
|  | Labels | m2m:labels | 0..1 |  |
|  | Link | xs:anyURI | 0..1 |  |
|  | announceTo | list of xs:anyURI | 0..1 |  |
|  | announcedAttribute | list of xs:NCName | 0..1 |  |
| m2m:announcedResource | @resourceName | xs:NCName | 1 | Declares the universal / common attributes in the announced variant of the preceding resources. |
|  | resourceType | m2m:resourceType | 1 |  |
|  | resourceID | m2m:ID | 1 |  |
|  | parentID | m2m:nhURI | 1 |  |
|  | creationTime | m2m:timestamp | 1 |  |
|  | lastModifiedTime | m2m:timestamp | 1 |  |
|  | labels | m2m:labels | 0..1 |  |
|  | accessControlPolicyIDs | m2m:acpType | 0..1 |  |
|  | expirationTime | m2m:timestamp | 1 |  |
|  | Link | xs:anyURI | 1 |  |
| m2m:announceableSubordinateResource | @resourceName | xs:NCName | 1 | Declares the universal / common attributes used by resource types that are subordinate children of other resources. |
|  | resourceType | m2m:resourceType | 1 |  |
|  | resourceID | m2m:ID | 1 |  |
|  | parentID | xs:anyURI | 1 |  |
|  | creationTime | m2m:timestamp | 1 |  |
|  | lastModifiedTime | m2m:timestamp | 1 |  |
|  | labels | m2m:labels | 0..1 |  |
|  | expirationTime | m2m:timestamp | 1 |  |
|  | announceTo | list of xs:anyURI | 0..1 |  |
|  | announcedAttribute | list of xs:NCName | 0..1 |  |
| m2m:announcedSubordinateResource | @resourceName | xs:NCName | 1 | Declares the common / universal attributes used in the announced variants of the subordinate resource types. |
|  | resourceType | m2m:resourceType | 1 |  |
|  | resourceID | m2m:ID | 1 |  |
|  | parentID | m2m:nhURI | 1 |  |
|  | creationTime | m2m:timestamp | 1 |  |
|  | lastModifiedTime | m2m:timestamp | 1 |  |
|  | labels | m2m:labels | 0..1 |  |
|  | expirationTime | m2m:timestamp | 1 |  |
|  | link | xs:anyURI | 1 |  |
| NOTE: In the above table, names of XML schema attributes are prefixed with a "@" character to differentiate these from Resource attribute names. The "@" character is not part of the actual attribute name. |  |  |  |  |

## 6.4	Message parameter data types

### 6.4.1	Request primitive parameter data types

The data types of request primitive parameters are specified in this clause.

Detailed request primitive parameter descriptions and usage can be found in clause 8.1.2 of the ETSI TS 118 101 Functional Architecture [6]. Further details on the representation of primitives are specified in clauses 7.2.1.1 and 8.

Table 6.4.1-1: Data Types for Request primitive parameters

| Primitive Parameter | Data Type | Multiplicity | Default Handling (see note 1) | Note |
| --- | --- | --- | --- | --- |
| Operation | m2m:operation | 1 | Not applicable | See clause 6.3.4.2.5 |
| To | xs:anyURI | 1 | Not applicable |  |
| From | m2m:ID | 0..1 | Not applicable | See clause 6.3.3. Also see note 2 below. |
| Request Identifier | m2m:requestID | 1 | Not applicable | See clause 6.3.3 |
| Resource Type | m2m:resourceType | 0..1 | No default | See clause 6.3.4.2.1 |
| Content | m2m:primitiveContent | 0..1 | No default | See clause 6.3.5.5 |
| Role | xs:anyType See NOTE | 0..1 |  | The Role optional parameter is required when role based access control is applied. It shall be used by the Receiver to check the Access Control privileges of the Originator. As described in oneM2M TS 0003 [7], clause 7.1.2. The use of this parameter is reserved for future use. The exact data type is not specified in this release |
| Originating Timestamp | m2m:timestamp | 0..1 | No default |  |
| Request Expiration Timestamp | m2m:absRelTimestamp | 0..1 | Can be given by CMDH policy (clauseD.12) | "Result Expiration Timestamp" shall be later than "Request Message Expiration Timestamp" |
| Result Expiration Timestamp | m2m:absRelTimestamp | 0..1 | Can be given by CMDH policy (clauseD.12) |  |
| Operation Execution Time | m2m:absRelTimestamp | 0..1 | Can be given by CMDH policy (clauseD.12) |  |
| Response Type | m2m:responseTypeInfo | 0..1 | Use 'blockingRequest' | See clause 6.3.5.30 |
| Result Persistence | m2m:absRelTimestamp | 0..1 | Can be given by CMDH policy (clause D.12) |  |
| Result Content | m2m:resultContent | 0..1 | Use 'attributes' | See clause 6.3.4.2.7 |
| Event Category | m2m:eventCat | 0..1 | No default | See clause 6.3.3 |
| Delivery Aggregation | xs:boolean | 0..1 | Can be given by CMDH policy (ClauseD.12), otherwise FALSE |  |
| Group Request Identifier | xs:string | 0..1 | No default |  |
| Filter Criteria | m2m:filterCriteria | 0..1 | No default | See clause 6.3.5.8 |
| Discovery Result Type | m2m:discResType | 0..1 | Use 'structured' | See clause 6.3.4.2.8 |
| NOTE 1: Default handling is the request handling procedure on a Transit/Hosting CSE when the request parameter is not included in a request primitive. This is not applicable for mandatory parameters which are marked as 'M' in tableTable 7.2.1.1-1. NOTE 2: From parameter shall be present for all requests except for AE CREATE where it is optional. |  |  |  |  |

### 6.4.2	Response primitive parameter data types

The data types of response primitive parameters are specified in this clause.

Detailed response message parameter descriptions and usage can be found in clause 8.1.3 of  ETSI TS 118 101 [6]. Further details on the representation of primitives are specified in clauses 7.2.1.2 and 8.

Table 6.4.2-1: Data Types for Response primitive parameters

| Primitive Parameter | Data Type | Multiplicity | Note |
| --- | --- | --- | --- |
| Response Status Code | m2m:responseStatusCode | 1 | See clause 6.3.4.2.9 |
| Request Identifier | m2m:requestID | 1 | See clause 6.3.3 |
| Content | m2m:primitiveContent | 0..1 | See clause 6.3.5.5 |
| To | m2m:ID | 0..1 | See clause 6.3.3 |
| From | m2m:ID | 0..1 |  |
| Originating Timestamp | m2m:timestamp | 0..1 | See Table 6.3.3-1 |
| Result Expiration Timestamp | m2m:absRelTimestamp | 0..1 | See Table 6.3.3-1 |
| Event Category | m2m:eventCat | 0..1 | See clause 6.3.3 |

## 6.5	Resource data types

### 6.5.1	Description

Each oneM2M Resource Data Type is defined using XML Schema (XSD), and supplied as a separate XSD document.  This XML Schema defines the attributes of the Resource in accordance with ETSI TS 118 101 Functional Architecture [6]. It represents an entire resource. In other words if and only if a requestor retrieves an entire resource in XML format, the XML that is returned shall be valid with respect to the schema for that resource. Note that the payload of a Create or Update request primitive does not necessarily have to be valid according to the schema, as this payload is not required to contain values for all the resource attributes. In particular a resource might contain mandatory read-only primitives, and these would not appear in a Create or Update request.

Each Resource Type , along with its Announced variant (if there is one) is defined in a separate XSD file. The name of that file should be prefixed with 'CDT-' and followed by the resource type name and version of the the present document.

The individual Resource Types inherit from a set of base resource types. These definitions, which can be found in the file CDT-commonTypes- v1_6_0.xsd, contain definitions for the common and universal attributes, and establish an inheritance hierarchy shown in Figure 6.5.1-1.

Figure 6.5.1-1: Resource Types

### 6.5.2	resource

#### 6.5.2.1	Description

This XSD type definition includes the six universal attributes that are present in all oneM2M resource type definitions. It forms the root of the resource inheritance hierarchy.

#### 6.5.2.2	Reference

See Table 6.3.6-2.

#### 6.5.2.3	Usage

This type is used indirectly by all resource types. It is used directly just by the <CSEBase> resource type.

### 6.5.3	regularResource

#### 6.5.3.1	Description

This type definition includes the universal and common attributes used by the non-annouceable M2M resources.

#### 6.5.3.2	Reference

See Table 6.3.6-2.

#### 6.5.3.3	Usage

This type is used by the following resource types:

<delivery>, <eventConfig>, <execInstance>, <m2mServiceSubscriptionProfile>, <mgmtCommand>, <pollingChannel>, <request>, <serviceSubscribedNode>, <statsCollect>, <statsConfig>, <subscription>, <serviceSubscribedAppRule>

### 6.5.4	announceableResource

#### 6.5.4.1	Description

This type definition includes the universal and common attributes used by M2M resource types that are capable of being announced.  In addition to the attributes of a regularResource, it includes (as optional) the common attributes that are used by the announcement mechanism.

#### 6.5.4.2	Reference

See Table 6.3.6-2.

#### 6.5.4.3	Usage

This type is used by the following resource types:

AE>, <container>, <group>, <locationPolicy>, <node>, <remoteCSE>

It is also used by the specializations of <mgmtObj>.

### 6.5.5	announcedResource

#### 6.5.5.1	Description

This type definition includes the universal and common attributes used by a resource that is announcing an announceable resource. In addition to the attributes of a regularResource, it includes (as optional) the link common attribute.

#### 6.5.5.2	Reference

See Table 6.3.6-2.

#### 6.5.5.3	Usage

This type is used by the following resource types:

<AEAnnc>, <containerAnnc>, <groupAnnc>, <locationPolicyAnnc>, <nodeAnnc>, <remoteCSEAnnc>

It is also used by the xxxAnnc variants of the <mgmtObj> specializations.

### 6.5.6	announceableSubordinateResource

#### 6.5.6.1	Description

This type definition includes the common attributes used by resource types that are subordinate children of other resource types. It excludes attributes like accessControlPolicyIDs, as this attribute is defined for such resources.

#### 6.5.6.2	Reference

See Table 6.3.6-2.

#### 6.5.6.3	Usage

This type is used by the following resource types:

<AEAnnc>, <containerAnnc>, <groupAnnc>, <locationPolicyAnnc>, <nodeAnnc>, <remoteCSEAnnc>

It is also used by the xxxAnnc variants of the <mgmtObj> specializations.

### 6.5.7	announcedSubordinateResource

#### 6.5.7.1	Description

This type definition includes the common attributes used by the Announced variants of the resource types that are subordinate children of other resource types.

#### 6.5.7.2	Reference

See Table 6.3.6-2

#### 6.5.7.3	Usage

This type is used by the following resource types:

<accessControlPolicyAnnc>, <contentInstanceAnnc>, <scheduleAnnc>.

## 6.6	Response status codes

### 6.6.1	Introduction

The present clause specifies the assignment of oneM2M Response Status Code (RSC) values, which are  returned in the Response Status Code parameter of Response primitive.

The RSC may be delivered as oneM2M defined structured data, or the mapped native status code for transport protocol binding (e.g. HTTP, CoAP, MQTT).

### 6.6.2	RSC framework overview

The RSCs are categorised as one of 6 classes:

Table 6.6.2-1: Definition of Response Status Code class

| Status Class | Codeclass | Interpretation |
| --- | --- | --- |
| Informational | 1xxx | The request is successfully received, but the request is still on process. |
| Success | 2xxx | The request is successfully received, understood, and accepted. |
| Redirection | 3xxx | (Not used in present release) |
| Originator Error | 4xxx | The request was malformed by the Originator and, is rejected. |
| Receiver Error | 5xxx | The requested operation cannot be performed due to an error condition at the Receiver CSE. |
| Network Service Error | 6xxx | The requested operation cannot be performed due to an error condition at the Network Serivce Entity. |

### 6.6.3	Definition of Response Status Codes

#### 6.6.3.1	Overview

The tables in the following clauses specify the RSCs for oneM2M releases. Each RSC includes: a response status represented as numeric code. The supplemental information may be returned when it is needed.

#### 6.6.3.2	Informational response class

Table 6.6.3.2-1 specifies the RSCs for acknowledgement responses for each release.

Table 6.6.3.2-1: Informational Responses class

| Numeric Code | Description |
| --- | --- |
| 1000 | ACCEPTED |

#### 6.6.3.3	Successful response class

Table 6.6.3.3-1 specifies the RSCs for Successful responses.

Table 6.6.3.3-1: RSCs for Successful response class

| Numeric Code | Description |
| --- | --- |
| 2000 | OK |
| 2001 | CREATED |
| 2002 | DELETED |
| 2004 | UPDATED |
| 2100 | CONTENT_EMPTY |

#### 6.6.3.4	Redirection response class

In this release, no values in this response class are defined.

Table 6.6.3.4-1: RSCs for Redirection response class

| Numeric Code | Description |
| --- | --- |

#### 6.6.3.5	Originator Error response class

Table 6.6.3.5-1 specify the RSCs for Originator Error responses.

Table 6.6.3.5-1: RSCs for Originator Error response class

| Numeric Code | Description |
| --- | --- |
| 4000 | BAD_REQUEST |
| 4004 | NOT_FOUND |
| 4005 | OPERATION_NOT_ALLOWED |
| 4008 | REQUEST_TIMEOUT |
| 4101 | SUBSCRIPTION_CREATOR_HAS_NO_PRIVILEGE |
| 4102 | CONTENTS_UNACCEPTABLE |
| 4103 | ACCESS_DENIED |
| 4104 | GROUP_REQUEST_IDENTIFIER_EXISTS |
| 4105 | CONFLICT |

#### 6.6.3.6	Receiver Error response class

Table 6.6.3.6-1 specifies the RSCs for Receiver Error  responses.

Table 6.6.3.6-1: RSCs for Receiver Error response class

| Numeric Code | Description |
| --- | --- |
| 5000 | INTERNAL_SERVER_ERROR |
| 5001 | NOT_IMPLEMENTED |
| 5103 | TARGET_NOT_REACHABLE |
| 5105 | NO_PRIVILEGE |
| 5106 | ALREADY_EXISTS |
| 5203 | TARGET_NOT_SUBSCRIBABLE |
| 5204 | SUBSCRIPTION_VERIFICATION_INITIATION_FAILED |
| 5205 | SUBSCRIPTION_HOST_HAS_NO_PRIVILEGE |
| 5206 | NON_BLOCKING_REQUEST_NOT_SUPPORTED |
| 5207 | NOT_ACCEPTABLE |

#### 6.6.3.7	Network System Error response class

Table 6.6.3.7-1 specifies the RSCs for when the External System reported some errors.

Table 6.6.3.7-1: RSCs for Network Service Error response class

| Numeric Code | Description |
| --- | --- |
| 6003 | EXTERNAL_OBJECT_NOT_REACHABLE |
| 6005 | EXTERNAL_OBJECT_NOT_FOUND |
| 6010 | MAX_NUMBER_OF_MEMBER_EXCEEDED |
| 6011 | MEMBER_TYPE_INCONSISTENT |
| 6020 | MGMT_SESSION_CANNOT_BE_ESTABLISHED |
| 6021 | MGMT_SESSION_ESTABLISHMENT_TIMEOUT |
| 6022 | INVALID_CMDTYPE |
| 6023 | INVALID_ARGUMENTS |
| 6024 | INSUFFICIENT_ARGUMENTS |
| 6025 | MGMT_CONVERSION_ERROR |
| 6026 | MGMT_CANCELLATION_FAILED |
| 6028 | ALREADY_COMPLETE |
| 6029 | MGMT_COMMAND_NOT_CANCELLABLE |

## 6.7	oneM2M specific MIME media types

The present sub-clause defines oneM2M specific MIME media types which may be used by protocol bindings.

The oneM2M specific MIME media types are defined under the vendor tree of "application" mediate type which is prefixed with 'application/vnd.onem2m-'.

Table 6.7-1: oneM2M specific MIME media types

| oneM2M specific MIME subtype | mapped oneM2M data type | Note |
| --- | --- | --- |
| vnd.onem2m-res+xml | m2m:resource | For oneM2M resource operation.The type of oneM2M resource in content shall be indicated by "ty" parameter. XML serialization rule is applied. (See clause 7.5.2). |
| vnd.onem2m-res+json | m2m:resource | Same information of above. JSON serialization rule is applied. (See clause 7.5.2). |
| vnd.onem2m-ntfy+xml | m2m:notification or m2m:aggregatedNotification | For Notify operation for resource subscription. XML serialization rule is applied. (See clause 7.5.1). |
| vnd.onem2m-ntfy+json | m2m: notification or m2m:aggregatedNotification | Same information of above. JSON serialization rule is applied. (See clause 7.5.1). |
| vnd.onem2m-preq+xml | m2m:requestPrimitive | For exchanging serialized oneM2M request primitive. XML serialization rule is applied (See clause 6.4.1 and 7.2.1.1). |
| vnd.onem2m-preq+json | m2m:requestPrimitive | Same information of above. JSON serialization rule is applied. (See clause 6.4.1 and 7.2.1.1). |
| vnd.onem2m-prsp+xml | m2m:responsePrimitive | For exchanging Response parameters. XML serialization rules is applied. (See clause 6.4.2 and 7.2.1.2). |
| vnd.onem2m-prsp+json | m2m:responsePrimitive | Same information of above. JSON serialization rule is applied. (See clause 6.4.2 and 7.2.1.2). |

## 6.8	Virtual Resources

A virtual resource is used to trigger processing and/or retrieve results, but does not have a permanent representation in a CSE. Table 6.8-1 lists the Virtual Resources

Table 6.8-1: Virtual Resources

| Virtual Resource Type | resourceName | Parent Resource | Notes |
| --- | --- | --- | --- |
| <latest> | latest | <container> | See clause 7.4.28 |
| <oldest> | oldest | <container> | See clause 7.4.29 |
| <fanOutPoint> | fanOutPoint | <group> | See clause 7.4.15 |
| <pollingChannelURI> | pollingChannelURI | <pollingChannel> | See clause 7.4.23 |

Each resource instance listed in “Parent Resource” column of Table 6.8-1has one virtual resource child of each type listed against it in the table. These child resource instances have fixed resourceNames, as shown in the second column.

The parent resources contain named references, whose names match  the virtual child'''s resourceNames. Each reference is a URI to the corresponding virtual resource. In the <container> case, there are two such references, one called latest and one called oldest. The URI returned stays valid for the lifetime of the virtual resource.

A virtual resource can also be addressed using a hierarchical URI formed by taking the hierarchical URI of the parent resource and appending a / followed by the resourceName of the virtual resource.

# 7	oneM2M procedures

## 7.1	Introduction

The following clauses describe prerequisites such as primitive format and procedure outline with three generic scenarios that are Originator, Receiver, and Resource Handling in accordance with CRUD+N operations. In addition, for specific resource type they provide common or resource specific attributes, data type definition for the attributes, and child resources as well as they explain resource specific procedures on CRUD operations to communicate with oneM2M compliant M2M Platform System by oneM2M protocols and APIs as follows:

Primitive formats and generic procedures

Common operations

Resource type-specific definitions and procedures

Notification definition and procedures

## 7.2	Primitive format and generic procedure

### 7.2.1	Primitive format

#### 7.2.1.1	Request primitive format

Table 7.2.1.1-1 summarizes the primitive parameters of  the Request primitive, indicating their presence depending on the C, R, U, D or N operations. "M" indicates mandatory, "O" indicates optional, "NP" indicates not present.

Refer to clause 8.1.2 of the ETSI TS 118 101 [6] for additional information on the request primitive parameters.

Table 7.2.1.1-1: Request Primitive Parameters

| Primitive Parameter | CREATE | RETRIEVE | UPDATE | DELETE | NOTIFY |
| --- | --- | --- | --- | --- | --- |
| Operation | M | M | M | M | M |
| To | M | M | M | M | M |
| From | O* See note 1 | M | M | M | M |
| Request Identifier | M | M | M | M | M |
| Resource Type | M | NP | NP | NP | NP |
| Content | M | O | M | NP | M |
| Role | O | O | O | O | O |
| Originating Timestamp | O | O | O | O | O |
| Request Expiration Timestamp | O | O | O | O | O |
| Result Expiration Time | O | O | O | O | O |
| Operation Execution Time | O | O | O | O | O |
| Response Type | O | O | O | O | O |
| Result Persistence | O | O | O | O | NP |
| Result Content | O | O | O | O | NP |
| Event Category | O | O | O | O | O |
| Delivery Aggregation | O | O | O | O | O |
| Group Request Identifier | O | O | O | O | O |
| Filter Criteria | NP | O | O | O | NP |
| Discovery Result Type | NP | O | NP | NP | NP |
| NOTE 1: The From parameter is Mandatory for all requests except for AE CREATE. For AE CREATE, it is Optional. |  |  |  |  |  |

The Content parameter in a Request shall contain one of the following:

- A partial Resource. This applies to Create and Update request primitives. In the case of a Create request the Content parameter shall contain a single root element whose name is the name of the Resource and whose content consists of one or more attributes, child Resources or childResource references. In the case of an Update request primitive, the Content parameter shall contain the attributes and their new values. Attributes to be deleted from the resource shall be indicated without a value. In both cases the resource type is as defined in clause 7.4, however since a partial resource is being transferred it is not required to be valid according to the XSD for that resource. If an attribute is present with a value, however, the value shall comply to the data type defined in the XSD of that resource.

A Notification Data Object. This applies to Notification request primitives. The data type of the data object is named <m2m:notification> and is described in Clause 7.5.1

An Aggregated Notification. This applies to Notification request primitives. The data type of the data object is named <m2m:aggregatedNotification> and contains multiple <m2m:notification> objects. This is described in clause 7.5.1.

An AttributeList element, as described in clause 7.5.2. This is used in partial retrieve request primitives to indicate a list of attribute names whose values shall be retrieved in the response.

A ResponsePrimitive object as described in clause 7.5.1. This applies to Notification request primitives which are sent when accessing resources in asynchronous non-blocking mode.

#### 7.2.1.2	Response primitive format

Table7.2.1.2-1 summarizes the primitive parameters for Response primitive, indicating their presence depending on the C, R, U, D or N operations of the associated Request primitive and whether this operation was successful or caused an error. "M" indicates mandatory, "O" indicates optional, "NP" indicates not present.

Refer to clause 8.1.3 of ETSI TS 118 101 [6] for additional information on the request primitive parameters.

NOTE: Response Code and Status Code parameters are merged into the Response Status Code parameter.

Table7.2.1.2-1 : Response Primitive Parameters

| Primitive parameter | Ack | CREATE Success | RETRIEVE Success | UPDATE Success | DELETE Success | NOTIFY Success | Error |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Response Status Code | M | M | M | M | M | M | M |
| Request Identifier | M | M | M | M | M | M | M |
| Content | O | O | M | O | O | O | O |
| To | O | O | O | O | O | O | O |
| From | O | O | O | O | O | O | O |
| Originating Timestamp | O | O | O | O | O | O | O |
| Result Expiration Timestamp | O | O | O | O | O | O | O |
| Event Category | O | O | O | O | O | O | O |

The Content parameter in a Response shall contain one of the following:

- A complete or partial Resource. This applies to a response primitive sent in reply to create and retrieve request message. In this case the Content parameter shall contain a single root element whose name is the name of the resource and whose content consists of one or more attributes, child resources or childResource references. In this case the resource type is as defined in clause 7.4.However if a partial resource is being transferred, it is not required to be valid according to the XSD for that resource, in terms of the presence of resource attributes. Any attribute that is present, however, shall comply to the data type defined in the XSD of that resource.

The URI of a resource. This shall be included in an element called m2m:URI defined in clause 7.5.2.

A partial resource and its hierarchical URI. These are included in an element called m2m:resource defined in clause 7.5.2. The URI is included as an element of m2m:resource.

A list of URIs. This can  be used for transferring the childResource URIs only or in a Discovery response. These are included in an element called m2m:URIList defined in clause 7.5.2.

An Aggregated Response. This is sent as a result of a Group operation. This uses the element m2m:aggregatedResponse defined in clause 7.5.2.

Raw data.

### 7.2.2	Description of generic procedures

#### 7.2.2.1	Generic resource request procedure for originator

A generic resource Request procedure shall be comprised of the following actions. Additional actions specific to individual procedures are listed in the respective sections by referencing these actions and providing additional steps. The Originator shall execute the following steps in order:

Figure 7.2.2.1-1: Generic procedure of Originator

Orig-1.0 "Compose of a Request primitive": Please refer to clause 7.3.1.1 for details.

Orig-2.0 "Send a Request to the Receiver CSE": The Request primitive shall be included mandatory parameters which are "Operation", "To", "From", and "Request Identifier" parameter. Please refer to clause 7.3.1.2 for details.

Orig-3.0 "Check Response Type": In this step, the Originator checks communication method either blockingRequest, nonBlockingRequestSynch or nonBlockingRequestAsynch by using Response Type parameter (see detail in clause 8.1.2 in the ETSI TS 118 101 [6]). If Response Type parameter does not exist, communication method is 'blocking Request' as specified at clause 6.4.1.

If the Response Type is blockingRequest it waits for "Response" primitive and goes to step Orig-4.0. If the Response Type is nonBlockingRequestSync, it waits for acknowledgement of "Response" primitive and goes to step Orig-4.1. If the Response Type is nonBlockingRequestAsynch, it waits for acknowledgement of Response primitive and goes to step Orig-4.1.

Orig-4.0 and Orig-4.1 "Wait for response primitive": Please refer to clause 7.3.1.3 for details.

Orig-5.0 "Send a Request primitive with op=R": The "Request" primitive shall be included mandatory parameters which are "Operation", "To", "From", and "Request Identifier" parameter. See clause 7.3.1.4 for details.

Orig-5.1 "Receive a Response primitive from the Hosting CSE": The Originator shall receive mandatory parameters which are Response Status Code, Request Identifier and Content parameter. A Request Identifier shall be identical to the Orig-5.0. An information of Content parameter is the result of the Orig-2.0 when the Receiver completed handling of Request primitive of Orig-2.0.

Orig-5.2 "Completion of operation by Response Status Code parameter": When the Response Status Code is successful and Content parameter exist, it goes to Orig-5.3. When the Response Status Code is acknowledgement which indicates processing at the Receiver, it goes to Orig-5.0.  When the Response Status Code is error such as Originator error (4XXX) or Receiver error (5XXX) or Network error (6XXX) or absence of Content parameter, it goes to finish with error.

Orig-5.3 "Extract a result form Response primitive of Orig-5.1": The information of operationResult attribute of the <request> resource in Content parameter from Orig-5.1 is extracted from Response primitive which is included Request Identifier, Response Status Code and optional Content parameter. The <request> resource shall be included mandatory attributes as specified in clause 9.6.12 of ETSI TS 118 101 [6]. The Request Identifier in operationResult attribute shall be identical of Orig-2.0.

Orig-6.0 "Process Response primitive": A "Request Identifier" shall be identical to the Orig-2.0. The Originator processes the result of the response.

Orig-7.0 "Receive a Request primitive with op=N": The Originator receives Request primitive with mandatory parameters which are Operation, To, From, Request Identifier and Content parameter.  An Operation parameter shall be Notify.A Content parameter is the notification information as specified in clause 7.5.1.1.

Orig-8.0 "Create a Response primitive with op=N": The Originator creates Response primitive with mandatory parameters which are Response Status Code and Request Identifier parameter. A Request Identifier shall be identical to the Orig-7.0.

Orig-9.0 "Send a Response primitive with op=N": The Response primitive which is created at Orig-8.0 shall be sent to the Receiver. Please refer to clause 7.3.2.3 for details.

Orig-9.1 “Extract Response primitive of Orig-2.0 from Orig-7.0”: The information of operationResult attribute in <request> resource from Orig-7.0 in Response primitive is included Request Identifier, Response Status Code and optional Content parameters. The <request> resource shall be included mandatory attributes as specified in clause 9.6.12 of the ETSI TS 118 101 [6]. The Request Identifier in operationResult attribute shall be identical of Orig-2.0.

#### 7.2.2.2	Generic request procedure for receiver

The Receiver shall execute the following steps in order. In case of error in any of the steps below, the Receiver shall execute "Create an error response" (refer to clause 7.3.3.13 for details) and then "Send Response primitive" (refer to clause 7.3.2.4for details). The corresponding Response code shall be included in the Response primitive.

Recv-1.0: “Check the validity of received request primitive”

Recv-2.0: Communication method?

Recv-6.0: Resource handling procedures

Recv-3.0: “Create <request> resource locally”

Recv-4.0: “Create a success Response”

Recv-5.0: “Send Response primitive”

nonBlockingRequestAsynch

nonBlockingRequestSynch

Recv-6.0: Resource handling procedures

Recv-7.0: “Update <request> resource”

Finish

Start

Recv-3.0: “Create <request> resource locally”

Recv-4.0: “Create a success Response”

Recv-5.0: “Send Response primitive”

Recv-6.0: Resource handling procedures

Recv-7.0: “Update <request> resource”

Recv-8.0: “Send Notification”

Recv-9.0: “Wait for Response primitive”

blockingRequest

Figure 7.2.2.2-1: Generic procedure of Receiver

Recv-1.0 "Check the validity of received request primitive": See clause 7.3.2.1 for details.

Recv-2.0 "Communication method?": The Receiver CSE checks whether a received request is blockingRequest, nonBlockingRequestSynch or nonBlockingRequestAsynch by using Response Type parameter (see detail in clause 8.1.2 in ETSI TS 118 101 [6]). If the request is blockingRequest or Response Type parameter is not included, it goes to step Recv-6.0 "Resource handling procedure". If the request is nonBlockingRequestSynch, it goes to step Recv-3.0 "Create <request> resource locally"  If the request is nonBlockingRequestAsynch, it goes to step Recv-3.0 "Create <request> resource locally".

Recv-3.0 "Create <request> resource locally": Please refer to clause 7.3.2.2 for details.

Recv-4.0 "Create a successResponse": Please refer to clause 7.3.2.2 for details.

Recv-5.0 "Send Response Primitive": Please refer to clause 7.3.2.4 for details.

Recv-6.0 "Resource handling procedure": Please refer to Figure 6.3.4.2.31-2 for details.

Recv-7.0 "Update <request> resource": Please refer to clause 7.3.2.5 for details. This step is only valid when the request is non-blocking.

Recv-8.0 "Send Notification": Please refer to clause 7.5.1.2.5 for details.

Recv-9.0 "Wait for a Response primitive": Please refer to clause 7.3.1.3 for details.

Recv-6.10: “Queue request primitive and execute CMDH message forwarding procedure”

Recv-6.1: Hosting CSE of the targeted resource?

Start

Recv-6.3: “Check authorization of the Originator”

Recv-6.4: “Check validity of resource representation for the given resource type”

Recv-6.2: “Check existence of the addressed resource”

Recv-6.5: “Create/Update/Retrieve/Delete/Notify operation is performed”

Recv-6.6: “Announce/De-announce the resource”

Finish

Yes

No

Recv-6.7: “Create a success response”

Recv-6.8: “Send Response Primitive”

Recv-6.9: CMDH processing supported?

Recv-6.11: “Forwarding”

No

Yes

Recv-6.0.1: Receiver is Registrar CSE & Originator is AE & operation is create

Recv-6.0.2: “Check Service Subscription Profile”

Yes

No

Recv-6.6.1: “Communication Method?”

Else

blockingRequest

Figure 6.3.4.2.31-2: Resource handling procedure

The above figure describes the generic procedure to resource handling procedures.

Recv-6.0.1 "Receiver is Registrar CSE, Originator is AE and operation is create?": The step checks if the receiver is Registrar CSE, the Originator is AE, and operation is create. If the receiver is Registrar CSE and Originator is an AE, goes to Recv-6.0.2 "Check Service Subscription Profile". Otherwise, goes to Recv-6.1.

Recv-6.0.2 "Check Service Subscription Profile": Please refer to clause 7.3.2.7 for details.

Recv-6.1 "Hosting CSE of the targeted resource?": The step checks if the receiver is a transit CSE or the Hosting CSE of the received Request by examining the To parameter of the Request primitive. If the receiver hosts the resource that the address in the To parameter represents, the receiver is the Hosting CSE (goes to Recv-6.2"Check existence of the addressed resource", Yes branch). Otherwise, the receiver is the Transit CSE (goes to Recv-6.9 "Queue request primitive and execute CMDH message forwarding procedure", No branch).

Recv-6.2 "Check existence of the addressed resource": Please refer to clause 7.3.3.1 for details.

Recv-6.3 "Check authorization of the Originator": Please refer to clause 7.3.3.15 for details.

Recv-6.4 "Check validity of resource representation": Please refer to clause 7.3.3.3 and clause 7.3.3.4 for details. Notify is not applicable for this step.

Recv-6.5 "Create/Update/Retrieve/Delete/Notify operation is performed": The step represents five common operations which are "Create the resource (clause 7.3.3.5)", "Retrieve the resource (clause 7.3.3.6)", "Update the resource (clause 7.3.3.7)", "Delete the resource (clause 7.3.3.8)" and "Notify re-targeting (clause 7.3.3.9)". Notify re-targeting is performed for the Create, Update, Retrieve, Delete, or Notify operation respectively.

Recv-6.6 "Announce/De-announce the resource": The step represents two common operations which are "Announce the resource" and "De-announce the resource". Please refer to clause 7.3.3.10 and clause 7.3.3.11 for details. Notify is not applicable for this step.

Recv-6.6.1 "Communication method?": The Receiver CSE checks whether a received request is blockingRequest or not by using Response Type parameter (see detail in clause 8.1.2 in ETSI TS 118 101 Functional Architecture [6]). If the request was blockingRequest or Response Type parameter was not included, it goes to step Recv-6.7 "Create a success response".Otherwise, it goes back to the generic procedure of the receiver (Figure 7.1.1.2.2-1).

Recv-6.7 "Create a success response": Please refer to clause 7.3.3.12 for details.

Recv-6.8 "Send Response Primitive": Please refer to clause 7.3.3.16 for details. If the Receiver is Hosting CSE, after this step, the procedure is terminated.

Recv-6.9 "CMDH processing supported?": This step checks whether the Receiver supports the CMDH processing.

Recv-6.10 "Queue request primitive and execute CMDH message forwarding procedure": If CMDH message is supported, the Receiver CSE shall queue the received request primitive and execute the "CMDH message forwarding procedure". Please refer to clause H.2.4. for details.

Recv-6.11 "Forwarding": If CMDH processing is not supported, carry out message forwarding as defined in clause 7.3.2.6.

## 7.3	Common operations

### 7.3.1	Originator actions

#### 7.3.1.1	Compose request primitive

The originator shall compose a Request message that shall be mapped to a specific protocol.

The Request shall include the mandatory parameters which is Originator, To, From and Request Identifier.

The Request may include the time related parameters which is Originating Timestamp, Request Expiration Timestamp, Result Expiration Timestamp, and Operation Execution Time.

The Request may include the other parameters as specified in (Table 7.2.1.1-1: Request Primitive Parameters).

When including a resource representation in the request indication for create and update, the originator shall take into account the validation rules as specified in "Check validity for resource representation for create" and "Check validity for resource representation for update" respectively.

EXAMPLE:	Any attributes marked with NP shall not be present in the resource representation for the corresponding request indication.

#### 7.3.1.2	Send a request to the receiver CSE

The originator shall determine the receiver CSE.

The receiver of the Request shall be the registrar CSE of the originator in case the originator is not IN-CSE.

If the originator is the IN-CSE, the receiver of the Request shall be the CSE whose identifier is the prefix of the To parameter of the Request.

If this results in no matching CSE, then the request is rejected with a Response Status Code indicating "NOT_FOUND" error.

If this results in multiple CSEs, the request is rejected with a Response Status Code indicating "INTERNAL_SERVER_ERROR" error.

#### 7.3.1.3	Wait for response primitive

The originator shall wait for the Response primitive from the receiver that corresponds to the Request primitive that was sent by the originator. Correlation between the Request and the corresponding Response is handled by the transport layer or by Request Identifier parameter of the primitive.

If no Response primitive is received within a certain time, specified by server policy and/or by the underlying transport technology, this shall be handled as if a Response primitive with a Response Status Code indicating "REQUEST_TIMEOUT" error was received.

#### 7.3.1.4	Retrieve the <request> resource

When the Originator needs to retrieve information about an associated previously issued non-blocking request, the Originator shall request to Retrieve the attributes of the <request> resource. The Originator shall compose the Request primitive with the following parameters and send the Request to the Receiver CSE. See clauses 7.3.1.1 and 7.3.1.2.

NOTE: The Originator may include optional parameters described in clause 8.1.2 of ETSI TS 118 101 Functional Architecture [6].

Table 7.3.1.4-1: Request primitive parameter settings

| Parameter Name | Value |
| --- | --- |
| Operation | Retrieve |
| To | This shall be set to the URI of the <request> resource received in the response (acknowledgement) to the previously issued non-blocking request. |
| From | Id of the Originator |
| Request Identifier | The identifier of this request message. |
| Content | Optionally includes the name of attributes that needs to be retrieved. |

### 7.3.2	Receiver CSE actions

#### 7.3.2.1	Check the validity of received request primitive

The validity checking of the message carrying the received request primitive is specified by the protocol mapping Technical Specifications (CoAP binding [22], HTTP binding [23],  and MQTT binding [24]). The received resource representation (e.g. in plain XML, binary XML or JSON) shall be validated against the provided schema definitions.

If the Request Expiration Timestamp is given in the request and expired, the Receiver CSE shall reject the request with an "REQUEST_TIMEOUT" Response Status Code parameter value.

If the received request is communicated within an established Security Association (ETSI TS 118 103 [7]), and

the Receiver knows that the Registree using the established Security Association is an AE; and

the Receiver knows the AE-ID(s) of the Registree using the established Security Association; and

the From parameter does not match the allowed AE-ID(s) of the Registree using the established Security Association,

then the request shall be rejected with an "ACCESS_DENIED" Response Status Code parameter value.

If the received request is communicated within an established Security Association, and

the Receiver knows that the Registree using the established Security Association is a CSE; and

the Receiver knows the CSE -ID of the Registree using the established Security Association; and

if one of the following applies:

the From parameter is an CSE-ID that matches one of the Receiver''s Registree CSE''s CSE-ID other than the CSE-ID of the Registree using the established Security Association, or

the From parameter is an CSE-Relative C-Type AE-ID-Stem, or

the From parameter is an SP-Relative AE-ID or Absolute AE-ID with a  C-Type AE-ID-Stem, and the CSE-ID portion of the From parameter matches one of the Receiver''s Registree CSE''s CSE-ID other than the CSE-ID of the Registree for the established Security Association,

then the request shall be rejected with an "ACCESS_DENIED" Response Status Code parameter value .

NOTE: An SP-Relative-AE-ID or Absolute AE-ID with a C-Type AE-ID-Stem always includes a CSE-ID portion (see ETSI TS 118 101 [6]).

If the received request is communicated outside of an established Security Association, and

if the From parameter includes an AE-ID, and

the request is not a CREATE <AE> Request, and

the From parameter does not match the AE-ID of an AE currently registered to the Receiver

then the request shall be rejected with a "ACCESS_DENIED" Response Status Code parameter value.

If the received request is communicated outside of an established Security Association, and the From parameter includes a CSE-ID, then the request shall be rejected with an "ACCESS_DENIED" Response Status Code parameter value.

If a received request needs to be forwarded to another CSE and if CMDH processing is supported, then in addition, the "CMDH message validation procedure" defined in clause H.2.3. shall be carried out.

If the message is not valid, the request shall be rejected with a Response Status Code indicating "BAD_REQUEST" error.

If the receiver does not support the content format (i.e. type of serialization) requested by the originator, the request shall be rejected with a Response Status Code indicating "NOT_ACCEPTABLE" error.

#### 7.3.2.2	Create <request> resource locally

Creation of a <request> resource can only be done on a Receiver CSE implicitly. When the Receiver CSE receives a request for targeting any other resource type or requesting a notification in non-blocking mode, i.e. the Response Type parameter of the request is set to either 'nonBlockingRequestSynch' or 'nonBlockingRequestAsynch', and if the Receiver CSE supports the <request> resource type as indicated by the 'supportedResourceType' attribute of the <CSEBase> resource, the Receiver CSE shall create an instance of <request> resource based on the following steps. If the Receiver CSE does not support the <request> resource type, the 'nonBlockingRequestSynch' request shall be rejected with a Response Status Code indicating "NON_BLOCKING_REQUEST_NOT_SUPPORTED" error. For the 'nonBlockingRequestAsynch'request, a Receiver CSE that does not support the <request> resource type shall be able to respond to an acceptable request with a response containing an Acknowledgement without a reference to a resource containing the context of the request.

The Receiver CSE of a non-blocking request is the Hosting CSE for the <request> resource that shall be associated with the non-blocking request.

- Assign a value to the common attributes of <request> resource according to the following table:

Table 7.3.2.2-1: Common attributes settings for <request> resource

| Attribute Name | Value |
| --- | --- |
| resourceType | Request |
| resourceID | Hosting CSE shall assign a value to this attribute. |
| expirationTime | The value of the expirationTime shall be chosen dependent on the Request Expiration Timestamp, Result Expiration Timestamp, Operation Execution Time and Result Persistence parameters associated with the original request. If the value consistent with the Request Expiration Timestamp, Result Expiration Timestamp, Operation Execution Time and Result Persistence parameters is too long, the Hosting CSE shall reject the request. |
| parentID | The parent resource of a <request> resource shall be the <CSEBase> resource of the Hosting CSE. |
| creationTime | Date/time of creation of this resource. |
| lastModifiedTime | Date/time which is equal to the creationTime. |
| accessControlPolicyIDs | Populate with one ID of an <accessControlPolicy> that contains the following: In the 'privileges' attribute - Allow RUD operations to the Hosting CSE - Allow RD operations to the Originator, i.e. the value of the parameter From in the associated non-blocking request In the 'selfPrivileges' attribute - Allow U operations the parent <accessControlPolicy> resource to the Originator, i.e. the value of the parameter From in the associated non-blocking request |
| Labels | Originator ID |
| stateTag | 0 |
| resourceName | Hosting CSE shall assign a value to this attribute. |

Assign a value to the resource-specific attributes of <request> resource according to the following table:

Table 7.3.2.2-2: Resource-specific attributes settings for <request> resource

| Attribute Name | Value |
| --- | --- |
| operation | The value of the parameter Operation in the associated non-blocking request. |
| Target | The value of the parameter To in the associated non-blocking request. |
| originator | The value of the parameter From in the associated non-blocking request. |
| requestID | The value of the parameter Request Identifier in the associated non-blocking request. |
| metaInformation | The content of this attribute is set to information in optional parameters described in clause 8.1.2 of [6] given in the associated non-blocking request. |
| Content | The value of the parameter Content, if any, in the associated non-blocking request. |
| requestStatus | The Receiver CSE shall set this to "PENDING". |
| operationResult | Empty |

#### 7.3.2.3	Create a success response (acknowledgement)

The Receiver CSE shall create a Response primitive. The Receiver CSE shall include the following parameters in the Response primitive.

Table 7.3.2.3-1: Response primitive parameter settings

| Parameter Name | Value |
| --- | --- |
| Response Status Code | "ACCEPTED" |
| Request Identifier | The value of the parameter Request Identifier in the associated non-blocking request. |
| Originating Timestamp | Timestamp when this message was built |
| Content | Reference to the <request> of the associated non-blocking request only if <request> resource is supported. |

#### 7.3.2.4	Send response primitive (acknowledgement)

A Response primitive shall be sent back to the originator.

#### 7.3.2.5	Update <request> resource

Changes in the attributes of a <request> resource shall be done by the Hosting CSE implicitly due to changes of the status (requestStatus) of the associated non-blocking request or due to the reception of an operation result (operationResult) in response to the associated non-blocking request. The Receiver CSE shall update attributes of an instance of <request> resource based on the following steps.

- Update a value to the common attributes of <request> resource according to the following table:

Table 7.3.2.5-1: Common attributes settings for <request> resource

| Attribute Name | Value |
| --- | --- |
| lastModifiedTime | Date/time of the last modification. |
| stateTag | This value is incremented on every modification. |

Update a value to the resource-specific attributes of <request> resource according to the following table:

Table 7.3.2.5-2: Resource-specific attributes settings for <request> resource

| Attribute Name | Value |
| --- | --- |
| requestStatus | If the Receiver CSE is a Transit CSE and the previously received request has been successfully forwarded to the next hop, this shall be set to "FORWARDED". If the Receiver CSE is a Transit CSE and the previously received request has been rejected by the next hop, this shall be set to "FAILED". If the Receiver CSE is the Target CSE (i.e. To parameter in the request message starts with the CSEBase URI of the Receiver CSE) and the originally requested operation has been completed, this shall be set to "COMPLETED". |
| operationResult | Hosting CSE shall contain the response message of the originally requested operation – if any – in line with the rc parameter in the associated non-blocking request. |

#### 7.3.2.6	Forwarding

If the To parameter in the request does not start with the CSEBase URI of the receiver, the receiver CSE shall forward the request or shall serve the request locally (see below).

If the To parameter in the request starts with the CSEBase URI of the receiver, then the receiver CSE shall handle the request locally.

Acting as an originator the CSE shall perform the following procedures:

- "Send a Request to the receiver CSE".

"Wait for Response primitive".

When the Response is received the receiver CSE shall:

- Primitive specific procedure: Forward the Response to the original CSE.

#### 7.3.2.7	Check Service Subscription Profile

The receiver shall check if the originator''s <serviceSubscriptionProfile> has S-RoleID in serviceRoles attribute that allows to create resource type specified in resource type parameter. The receiver shall find a proper <serviceSubscribedNode> including CSE-ID of the receiver and determine serviceRoles from the parent resource of the <serviceSubscribedNode> which corresponds to <serviceSubscriptionProfile> resource.

### 7.3.3	Hosting CSE actions

#### 7.3.3.1	Check the supported resource types

If the request is a valid request, but the Hosting CSE does not implement the requested resource type, then the Hosting CSE shall reject the request and return an error response with Response Status Code indicating "NOT IMPLEMENTED".

#### 7.3.3.2	Check existence of the addressed resource

If the Request Expiration Timestamp is given in the request and expired, the Hosting CSE shall reject the request with an "REQUEST_TIMEOUT" Response Status Code parameter value.

The Hosting CSE shall check if the resource addressed by the To parameter exists in the repository. If the resource does not exist, the Hosting CSE shall reject the request with a Response Status Code indicating "NOT_FOUND" error.

The Hosting CSE shall also check if the conditions specified in the Filter Criteria parameter in the Retrieve/Update/Delete operation are met. If the condition check fails, the Hosting CSE shall reject the request with a Response Status Code indicating "NOT_FOUND" error.

If the Filter Criteria parameter is included in the Create request, the Hosting CSE shall reject the request with a Response Status Code indicating "BAD_REQUEST" error.

If the Hosting CSE does not support the content format (i.e. type of serialization) requested by the originator, the request shall be rejected with a Response Status Code indicating "NOT_ACCEPTABLE" error.

#### 7.3.3.3	Check validity of resource representation for CREATE

The handling below shall apply to each attribute in the resource for CREATE request primitives and the handling depends on the "presence in CREATE request" column of the resource table. If the request is rejected based on the rules below, then the other attributes do not have to be checked.

If no resource representation is present in the CREATE request, then the request is rejected with a Response Status Code indicating "BAD_REQUEST" error.

If the expirationTime attribute is present in the resource representation, but its value indicates a time in the past, then the request shall be rejected with a Response Status Code indicating "BAD_REQUEST" error.

There are three cases where the hosting CSE shall configure or overrride an expirationTime value that differs from the value specified in the resource representation (if present).

- The Originator does not specify an expirationTime

The Originator requests an expirationTime that is later than expirationTime of the parent

The hosting CSE determines the expirationTime requested by the Originator doesn’t meet its requirements (E.g. based on a local policy)

In each of these cases, the hosting CSE shall configure an expirationTime into the resource that is less than or equal to  the expirationTime of the parent resource. In addition, the hosting CSE shall communicate the modified value back to the originator in the response if the Result Content parameter permits this.

M attribute

If the attribute is present in the resource representation in the CREATE request, the hosting CSE shall check if the value is acceptable according to internal policies.

If the provided value is not accepted,  the hosting CSE shall reject the request with a Response Status Code indicating "BAD_REQUEST" error.

If the attribute is not present in the resource representation in the CREATE request the hosting CSE shall reject the request with a Response Status Code indicating "BAD_REQUEST" error.

O attribute

If the attribute is present in the resource representation in the CREATE request, the hosting CSE shall check if the value is acceptable according to internal policies.

If the provided value is not accepted then the hosting CSE shall reject the request with a Response Status Code indicating "BAD_REQUEST" error.

NP attribute

If the attribute is present in the resource representation in the CREATE request, the hosting CSE shall reject the request with a Response Status Code indicating "BAD_REQUEST" error.

#### 7.3.3.4	Check validity of resource representation for UPDATE

The handling below shall apply to each attribute in the resource for UPDATE request primitives and the handling depends on the "presence in UPDATE request" column of the resource table. If the request is rejected based on the rules below, then the other attributes do not have to be checked.

If the expirationTime attribute is present in the resource representation, but its value indicates a time in the past, then the request shall be rejected with a Response Status Code indicating "BAD_REQUEST" error.

M attribute

If the attribute is present in the resource representation in the UPDATE request, the hosting CSE shall check if the value is acceptable according to internal policies.

If the provided value is not accepted, the hosting CSE shall reject the request with a Response Status Code indicating "BAD_REQUEST" error.

If the attribute is not present in the resource representation in the UPDATE request, the hosting CSE shall reject the request with a Response Status Code indicating "BAD_REQUEST" error.

O attribute

If the attribute is present in the resource representation in the UPDATE request, the hosting CSE shall check if the value is acceptable according to internal policies.

If the provided value is not accepted, the hosting CSE shall reject the request with a Response Status Code indicating "BAD_REQUEST " error.

NP attribute

If the attribute is present in the resource representation in the UPDATE request, the hosting CSE shall reject the request with a Response Status Code indicating "BAD_REQUEST" error.

#### 7.3.3.5	Create the resource

If the Operation Execution Time is given in the request, the Hosting CSE should perform the following procedures at the time and shall not perform the procedures before the time.

A new resource shall be created and correlated to the addressed and existing parent resource. As the result of the resource creation, the lastModifiedTime attribute of the parent resource shall be set to the same value as the creationTime attribute of the created resource. The following rules shall be applied.

The URI of the created resource shall be the URI of its parent resource with the resourceName appended. (e.g. http://CSEbase.operator.org/myAppID, for an application resource with resourceName "myAppID" created in the parent resource http://CSEbase.operator.org).

When configuring the resourceName attribute of the new resource, the hosting CSE shall use the name provided in the resourceName attribute within the content of the request. The hosting CSE shall first check for the presence of any resources having a resourceName attribute that matches the one specified in the request and that have the same parent as the new resource being created.   If such a resource exists, then the hosting CSE shall reject the request with a Response Status Code indicating "CONFLICT" error. If the resourceName is not provided in the request, the Hosting CSE shall generate and assign a name to the resourceName attribute of the new resource.

If expirationTime attribute is present in the resource representation of the to be created resource and the expirationTime is set to a non-negative time, then an expiration timer shall be started by the hosting CSE. At timer expiration the related resource is deleted by "Delete the addressed resource".

For setting the attributes in the resource representation the following rules shall apply in CREATE request primitives:

M attribute

If the provided value is acceptable, the server shall use the provided value in the resource representation of the created resource.

O attribute

If a value is provided and accepted, then the server shall use the provided value in the resource representation of the created resource.

If the attribute is not provided or accepted, but the multiplicity of the attribute is "1" in the resource, the hosting CSE shall assign default value or assign value based on local policy, or the value of specified in clause 7.4.

If the attribute is not present in the resource representation in the CREATE request and the multiplicity of the attribute is "0..1" in the resource, the hosting CSE shall create the resource without the attribute.

NP attribute

If the attribute is not present in the resource representation in the CREATE request, and the multiplicity of the attribute is "1" in the resource, then the hosting CSE shall create the resource with the default value.

#### 7.3.3.6	Retrieve the resource

If the Operation Excution Time is given in the request, the Hosting CSE should perform the following procedures at the time and shall not perform the procedures before the time.

When the resource is read to provide a response to Retrieve request primitives:

Full retrieve request: the request target is a resource given in the To parameter

The content of the Response to the Retrieve Request shall comply to the Result Content parameter in the Request. If the Result Content is not provided in the Request, the representation of the resource which includes all the attributes shall be returned.

Partial retrieve request: there are two cases:

Case 1) 	the request target is a resource given in the To parameter and specific attribute names are provided in the Content parameter:

The values of the resource attribute(s) provided in the Content parameter shall be retrieved.

Case 2)	the request target is a resource given in the To parameter, the resource attribute is provided in the To parameter as a fragment identifier component of URI following ''#'' character [2]. The resource attribute shall be represented as a short name and shall belong to short name list in Table 8.2.3-1 to Table 8.2.3-5

The resource attribute provided in the To parameter shall be retrieved.

#### 7.3.3.7	Update the resource

If the Operation Execution Time parameter is given in the request, the Hosting CSE should perform the following procedures at that time and shall not perform the procedures before that time.

Attributes that are not included in the Content parameter of the addressed resource shall not be changed by the hosting CSE. For attributes provided in the Content parameter, their content shall be updated while the following rules apply:

If the announceTo attribute or announcedAttribute attribute of the resource is requested to be updated, the hosting CSE shall update the attribute as described in the "announce the resource or attribute" and "de-announce the resource or attribute" procedures as specified in the clause 7.3.3.10 and clause 7.3.3.11, respectively.

M attribute

If the provided attribute value is accepted, the server shall use the provided value in the resource representation of the updated resource.

O attribute

If an attribute value is provided in the Content parameter and the value is accepted, the server shall use the provided value in the resource representation of the updated resource.

If the attribute is not provided in the Content parameter, but the attribute exists in the target resource, , the hosting CSE shall leave the value of that attribute unchanged.

If this attribute is provided in the Content parameter and does not exist in the target resource, the hosting CSE shall create such attribute with the provided value.

If this attribute is set to NULL in the Content parameter and exists in the target resource, the hosting CSE shall delete such attribute if the deletion of the attribute is allowed by the local policy.

NP attribute

If the attribute is not present in the resource representation in the UPDATE request and the multiplicity of the attribute is "1" in the resource, then the hosting CSE shall not update the attribute value. There is 2 exceptions to this rule and they  are the lastModifiedTime attribute and stateTag attribute. The hosting CSE shall set the lastModifiedTime to the current time whenever an update primitive is received. The hosting CSE shall change the stateTag each time an update primitive is received.

If the attribute is present in the resource representation in the UPDATE request the presented value shall be ignored, i.e. the hosting CSE shall never update its resource representation based on the presence of an NP attribute value in an update.

If the expirationTime attribute is present and modified by the procedure and it is set to a non-negative time, then an expiration timer shall be re-started by the hosting CSE. At timer expiration the related resource is deleted by "Delete the addressed resource".

#### 7.3.3.8	Delete the resource

If the Operation Excution Time is given in the request, the Hosting CSE should perform the following procedures at the time and shall not perform the procedures before the time.

The addressed resource with all its attributes shall be deleted. Any expiration timer shall be stopped. This same procedure shall be invoked (recursively) for each child resource of the deleted resource in case the child resource is only linked to the deleted resource.

The parent resource of the addressed resource shall be updated to remove the reference to the deleted resource. If the parent resource has a lastModificationTime attribute then this attribute shall be set to the time of the deletion.

If the resource is announced, the CSE shall try to de-announce the resource correspondingly.

#### 7.3.3.9	Notify re-targeting

If the Operation Excution Time is given in the request, the Hosting CSE should perform the following procedures at the time and shall not perform the procedures before the time.

When the Hosting CSE receives a Notify request primitive targeting (i.e., To parameter) its <AE> resource, the Hosting CSE re-targets the primitive to the AE if the <AE> resource does not have any <pollingChannel> resource as a child.

- Get pointOfAccess attribute value of the corresponding <AE> resource. If there is no available pointOfAccess address then the Hosting CSE shall send the Notify response primitive with a Response Status Code indicating "TARGET_NOT_REACHABLE" error.

Forward the Notify request primitive to the first address retrieved from pointOfAccess value

If the forwarding is failed due to "Target not reachable", iterate 2) with the next address.

- If the Hosting CSE cannot forward it in the end, then it send the Notify response primitive with a Response Status Code indicating "TARGET_NOT_REACHABLE" error.

#### 7.3.3.10	Announce the resource or attribute

If CREATE request that contains an announceTo attribute is received,

Compose the CREATE Request primitive as follows:

Link is set to the URI of the original resource.

If accessControlPolicyIDs of the original resource is not present, accessControlPolicyIDs is set to the same value with the parent resource or from the local policy of the original resource.

Attributes marked with MA and attributes marked with OA that are included in the announcedAttribute attribute. Such attributes shall be present in the original resource and set to same value as the original resource.

Send a CREATE Request to the CSE(s) represented by exact URI(s) or CSE-ID(s) in the announceTo of the request.

Wait for Response primitive

Add the URI of successfully announced resource to the announceTo attribute of the resource

Include updated announceTo attribute in the Content parameter in the Response to the received CREATE Request.

If UPDATE request that adds the URI or CSE-ID into the announceTo attribute is received,

Compose the CREATE Request primitive as follows:

Link is set to the URI of the original resource.

If accessControlPolicyIDs of the original resource is not present, accessControlPolicyIDs is set to the same value with the parent resource or from the local policy of the original resource.

Attributes marked with MA and attributes marked with OA that are included in the announcedAttribute attribute. Such attributes shall be present in the original resource and set to same value as the original resource.

Send a CREATE Request to the CSE(s) represented by exact URI(s) or CSE-ID(s) in the announceTo of the request, which is not included in the announceTo attribute of the original resource.

Wait for Response primitive

Add the URI of successfully announced resource to the announceTo attribute of the resource

Include updated announceTo attribute in the Content parameter in the Response to the received UPDATE Request.

If UPDATE request that adds the attribute name into the announcedAttribute attribute is received,

Compose the UPDATE Request. The UPDATE Request shall provide the attribute name for the attribute to be announced, and the initial value for the attribute in the Content parameter. The initial value shall be the same with the value from the original resource. The attribute that will be announced shall be marked as OA.

Send UPDATE Requests to all announced resources listed in the announceTo attribute.

Wait for Response primitive.

Add the attribute name of the successfully announced attribute to the announcedAttribute attribute.

Include updated announcedAttribute attribute in the Content parameter in the Response to the received UPDATE Request.

If an attribute(s) specified as MA (See ETSI TS 118 101 [6]) or an attribute(s) included in the announcedAttribute attribute is updated:

Compose an UPDATE Request primitive by including the updated attribute(s) with its associated updated value.

Send the UPDATE Request to all CSE(s) represented by the URI(s) in the announceTo attribute of the original resource.

If an attribute(s) specified as MA (See ETSI TS 118 101 [6]) or an attribute(s) included in the announcedAttribute attribute is deleted:

Compose an UPDATE Request primitive by including the updated attribute(s) with its value set to NULL.

Send the UPDATE Request to all CSE(s) represented by the URI(s) in the announceTo attribute of the original resource.

#### 7.3.3.11	De-announce the resource or attribute

If UPDATE Request that deletes the URI from the announceTo attribute is received:

Compose the DELETE Request primitive.

Send a DELETE Request to the CSE(s) represented by URI(s) in the announceTo attribute of the resource, which is not included in the announceTo of the request. The To parameter in the DELETE Request shall be set to the URI for the announced resource that will be deleted.

Wait for Response primitive.

Remove the URI of successfully de-announced resource from the announceTo attribute of the resource.

Include updated announceTo attribute in the Content parameter in the Response to the UPDATE Request of the original resource.

If DELETE Request is received:

Compose the DELETE Request primitive.

Send DELETE Requests to all announced resources addressed by the URI(s) in the announceTo attribute of the resource.

Wait for Response primitive.

If UPDATE request that deletes the attribute name from the announcedAttribute attribute is received:

Compose the UPDATE Request primitive. The To parameter in the UPDATE Request shall be set to the URI for the announced resource. The UPDATE Request shall set the attribute that will be de-announced (i.e. to be deleted) in the Content parameter to NULL. The attribute that will be de-announced shall be marked as OA.

Send UPDATE Requests to all announced resources listed in the announceTo attribute of the original resource.

Wait for Response primitive.

Delete the attribute name of the successfully de-announced attribute from the announcedAttribute attribute.

Include updated announcedAttribute attribute in the Content parameter in the Response to the received UPDATE Request.

#### 7.3.3.12	Create a success response

The Hosting CSE shall create a success response primitive with a Response Status Code indicating:

" CREATED" in case of Create operation. If the Hosting CSE assigned attribute(s) not provided or modified any of the provided attributes as provided in the Request, the Content parameter shall include the assigned and/or modified attributes;

"OK" in case of Retrieve operation;

"UPDATED" in case of Update operation;

"DELETED" in case of Delete operation; and

"OK" in case of Notify operation.

The Hosting CSE shall include Request Identifier parameter in the response primitive.

The Hosting CSE shall include Content parameter with:

the address and/or attributes(assigned and modified by the Hosting CSE) of the created reseource depending on Result Content parameter (i.e., attributes,  hierarchical-address, hierarchical-address+attributes) in the request primitive. This shall apply for Create operation;

the retrieved attributes and/or child resource references depending on Result Content parameter (i.e., attributes, attributes+child-resources, attributes+child-resource-references, child-resource-references, original-resource) in the request primitive. This shall apply for Retrieve operation; and

the modified/created/deleted attributes. This shall apply for Update operation.

More details can be found in clause 7.2.1.2 (Response primitive format).

NOTE: If Result Content parameter is not given in the request primitive, the default value is attributes. How to deal with each Result Content value is described in clause 8.1.2 [6]).

The Hosting CSE may include To, From, Originating Timestamp, Result Expiration Timestamp, Event Category parameters.

#### 7.3.3.13	Create an error response

The receiver shall create an error response primitive with a Response Status Code indicating the detected error condition.

NOTE:	Possible error codes and its error handling is described in resource specific procedure.

#### 7.3.3.14	Resource discovery procedure

If the Operation Excution Time is given in the request, the Hosting CSE should perform the following procedures at the time and shall not perform the procedures before the time.

A resource discovery is used to discover resources in a CSE. A Resource discovery request is done by sending Retrieve request with filterUsage, one of the filterCriteria parameters, configured as "discovery" and the request may include other filterCriteria parameters as well. A resource discovery request procedure shall be comprised of the following actions.

Originator:

The Originator shall follow the steps from Orig-1.0 to Orig-6.0 specified in clause 7.2.2.1 Generic Resource Request Procedure for Originator.

In addition to Orig-1.0, the following steps shall be performed.

The To parameter in the Retrieve Request indicates the root of where the discovery begins.

The Retrieve Request shall include filterUsage parameter in filterCriteria.

The Retrieve Request may include other parameters of filterCriteria.

Receiver:

The Receiver shall follow the steps from Recv-1.0 to Recv-7.0 specified in clause 7.2.2.2 Generic Resource Request Procedure for Receiver.

Hosting CSE shall not perform steps from Recv-6.3 to Recv-6.6 and perform the following steps instead.

The Receiver shall find resources, which match all the configured filterCriteria and which the Originator has "Discover" access right, under the addressed resource".

In Recv-6.7, the Receiver shall include addresses for all the found resources.

The Receiver shall perform Recv-6.8 and the procedure is terminated.

#### 7.3.3.15	Check authorization of the originator

Depending on the target resource type, the Hosting CSE shall use accessControlPolicyIDs of the different resources.

For <schedule> resource, the Hosting CSE shall evaluate the accessControlPolicyIDs of the parent resource.

For <latest>, <oldest> and <contentInstance> resource, the Hosting CSE shall evaluate the accessControlPolicyIDs of the parent <container> resource.

For <m2mServiceSubscriptionProfile> and <serviceSubscriedNode> resource, if it has no accessControlPolicyIDs value, the Hosting CSE shall evaluate the accessControlPolicyIDs of the parent resource.

For other resources, the Hosting CSE shall evaluate the accessControlPolicyIDs of the resource.

The evaluation procedure shall be performed as following:

- The Hosting CSE retrieves the access control rules from privilege attribute of the <accessControlPolicy> which is linked as the accessControlPolicyIDs. If the target is <accessControlPolicy> resource, it retrieves the rules from selfPrivilege attribute instead.

- The Hosting CSE checks the following conditions for the access control rules. If there is any rule satisfying all conditions then the evaluation is successful, otherwise it is failed. For more details, see the clause 7.1.5 in ETSI TS 118 103 [7].

accessControlOriginators of the rule includes the Originator information.

accessControlContexts of the rule includes the request context, if the rule includes the accessControlContexts

accessControlOperations of the rule matches the operation type of the request.

If the evaluation failed, then authorization failure information shall be returned to the Originator.

#### 7.3.3.16	Send response primitive

A Response primitive shall be sent back to the Originator. If the primitive is successful response and the Result Expiration Timestamp is given in the request, then the Hosting CSE should send the primitive before the Result Expiration Timestamp.

#### 7.3.3.17	Using Filter Criteria for identification of target resources

##### 7.3.3.17.0	Introduction

When the Filter Criteria primitive parameter is present in a request primitive, it shall be applied for identification of the applicable target resources of the respective operation. This may apply to Retrieve, Delete and Discovery operations as specified in clauses 7.3.3.6, 7.3.3.8 and 7.3.3.14, respectively.

The Filter Criteria primitive parameter defines conditions on resource attributes. Resources matching the conditions shall be selected as target of the operation. Table 7.3.3.17-1 summarizes the various filter criteria and conditions. Each row in the table represents a different filter condition type.

If multiple conditions of different type (i.e. different condition tags) are present in the Filter Criteria parameter, these shall be satisfied all to pass the overall combined filter condition, i.e. the combined condition shall be derived by applying Boolean AND operation across each individual condition.

If multiple conditions of the same type (i.e. same condition tag) are present in the Filter Criteria parameter, these shall be combined by applying Boolean OR operation. This applies to condition tags labels, resourceType, contentType or attribute for multiplicity n > 1.

Table 7.3.3.17-1: Summary on Filter conditions

| Condition Tag | Multiplicity | Targeted Resource Attribute | Matching Condition |
| --- | --- | --- | --- |
| createdBefore, | 0..1 | creationTime | creationTime < createdBefore, see clause 7.3.3.17.1. |
| createdAfter | 0..1 |  | createdAfter ≤ creationTime , see clause 7.3.3.17.1. |
| lastModifiedBefore | 0..1 | lastModifiedTime | lastModifiedTime < lastModifiedBefore, see clause 7.3.3.17.2. |
| lastModifiedAfter | 0..1 |  | lastModifiedAfter ≤ lastModifiedTime, see clause 7.3.3.17.2. |
| stateTagSmaller | 0..1 | stateTag | stateTagSmaller < stateTag, see clause 7.3.3.17.3. |
| stateTagBigger | 0..1 |  | stateTag ≤ stateTagBigger, see clause 7.3.3.17.3. |
| expireBefore | 0..1 | expirationTime | expirationTime < expireBefore, see clause 7.3.3.17.4. |
| expireAfter | 0..1 |  | expireAfter ≤ expirationTime , see clause 7.3.3.17.4. |
| Labels | 0..n | labels | see clause 7.3.3.17.5. |
| resourceType | 0..n | resourceType | see clause 7.3.3.17.6. |
| sizeBelow | 0..1 | contentSize | contentSize < sizeBelow, see clause see clause 7.3.3.17.7. |
| sizeAbove | 0..1 |  | sizeAbove ≤ contentSize, see clause see clause 7.3.3.17.7. |
| typeOfContent | 0..n | contentInfo | matched with typeOfContent component in contentInfo, see clause 7.3.3.17.8. |
| Attribute | 0..n | (variable) | name and value of Filter Criteria attribute matches resource attribute, see clause 7.3.3.17.9. |
| Limits | 0..1 | (not applicable) | Constraint on maximum number of targeted resources, see clause 7.3.3.17.10. |
| filterUsage | 0..1 | (not applicable) | Indicator specifying the use case of Filter Criteria parameters. |

##### 7.3.3.17.1	Conditions on the creationTime attribute

The Filter Criteria elements createdBefore and createdAfter define a time interval which is tested against the creationTime attribute of the applicable resources.

This filter criterion shall be satisfied if any of the following three conditions is fulfilled:

- only createdBefore given in Filter Criteria: 
creationTime <  createdBefore

only createdAfter given in Filter Criteria:
createdAfter ≤ creationTime)

both, createdBefore and createdAfter given in Filter Criteria: 
(createdAfter ≤ creationTime) AND (creationTime < createdBefore)

NOTE:  In case 3) the Filter Criteria will only generate a match if createdAfter < createdBefore.

##### 7.3.3.17.2	Conditions on the lastModifiedTime attribute

The Filter Criteria elements lastModifiedBefore and lastModifiedAfter define a time interval which is tested against the lastModifiedTime attribute of the applicable resources.

This filter criterion shall be satisfied if any of the following three conditions is fulfilled:

- only lastModifiedBefore given in Filter Criteria: 
lastModifiedTime <  lastModifiedBefore

only lastModifiedAfter given in Filter Criteria: 
lastModifiedAfter ≤ lastModifiedTime

both, lastModifiedBefore and lastModifiedAfter given in Filter Criteria: 
(lastModifiedAfter ≤ lastModifiedTime) AND (lastModifiedTime < lastModifiedBefore))

NOTE:  In case 3) the Filter Criteria will only generate a match if lastModifiedAfter < lastModifiedBefore.

##### 7.3.3.17.3	Conditions on stateTag attribute

The Filter Criteria elements stateTagSmaller and stateTagBigger define a number range which is tested against the stateTag attribute of the applicable resources.

This filter criterion shall be satisfied if any of the following three conditions is fulfilled:

- only stateTagSmaller given in Filter Criteria: 
stateTag <  stateTagSmaller

only stateTagBigger given in Filter Criteria: 
stateTagBigger ≤ stateTag

both, stateTagSmaller and stateTagBigger given in Filter Criteria:
(stateTagBigger ≤ stateTag) AND (stateTag < stateTagSmaller)

NOTE:  In case 3) the Filter Criteria will only generate a match if stateTagBigger < stateTagSmaller

##### 7.3.3.17.4	Conditions on expirationTime attribute

The Filter Criteria elements expireBefore and expireAfter define a time interval which is tested against the expirationTime attribute of the applicable resources.

This filter criterion shall be satisfied if any of the following three conditions is fulfilled:

- only expireBefore given in Filter Criteria: 
expirationTime <  expireBefore

only expiredAfter given in Filter Criteria: 
expireAfter ≤ expirationTime

both, expireBefore and expireAfter given in Filter Criteria:
(expireAfter ≤ expirationTime) AND (expirationTime < expireBefore)

NOTE:  In case 3) the Filter Criteria will only generate a match if expireAfter < expireBefore.

##### 7.3.3.17.5	Conditions on labels attribute

The Filter Criteria element labels defines a list of labels which is tested against the labels attribute of the applicable resource instances.

This filter criterion shall be satisfied if any of the labels in Filter Criteria matches any of the labels in the respective resource attribute.

##### 7.3.3.17.6	Conditions on resourceType attribute

The Filter Criteria element resourceType defines a list of resource types which is tested against the resourceType attribute of the resource.

This filter criterion shall be satisfied if any of the numbers in the resourceType Filter Criteria element matches the resourceType attribute.

##### 7.3.3.17.7	Conditions on contentSize attribute

The Filter Criteria elements sizeBelow and sizeAbove define a number range which is tested against the value of the contentSize attribute of applicable <contentInstance> resources.

This filter criterion shall be satisfied if any of the following three conditions is fulfilled:

- only sizeBelow given in Filter Criteria: 
contentSize <  sizeBelow

only sizeAbove given in Filter Criteria: 
sizeAbove ≤ contentSize

both, sizeBelow and sizeAbove given in Filter Criteria:
(sizeAbove ≤ contentSize) AND (contentSize < sizeBelow)

NOTE:  In case 3) the Filter Criteria will only generate a match if sizeAbove < sizeBelow.

##### 7.3.3.17.8	Conditions on typeOfContent of contentInfo attribute

The Filter Criteria element typeOfContent defines a string (or multiple such strings) which is compared against the contentInfo attribute of applicable <contentInstance> resources.

One or multiple such typeOfContent elements may be included in the Filter Criteria parameter.

This filter criterion shall be satisfied if any of the typeOfContent elements in Filter Criteria matches the typeOfContent part of the contentInfo attribute in a <contentInstance> resource.

##### 7.3.3.17.9	Conditions on attribute name and value pairs

The Filter Criteria elements attribute defines one or more pairs (attribute/name, attribute/value, see clause 6.3.4.8) that are compared against all applicable resource representations which include a resource attribute with a name as given in 'attribute/name'.

This filter criterion shall be satisfied if any of the attribute elements in the Filter Criteria parameter matches both, attribute/name and attribute/value in any applicable resource representation.

Attribute names used in any of the above specific conditions shall be omitted from this attribute name and value pair condition, i.e. the following attributes shall not be used for this condition as it may conflict with the explicit criteria defined for those attributes: creationTime, lastModifiedTime, stateTag, expirationTimelabels, resourceType, contentSize and contentInfo.

##### 7.3.3.17.10	Constraint on number of retrieved resources by limit element

The limit element of the Filter Criteria parameter does not represent a filter condition. It imposes a limit on the number of resources that should be retrieved with the given Retrieve request primitive.

The number of resources retrieved with the request primitive (and to be included into the corresponding response primitive) shall not exceed the number indicated in the limit element of the Filter Criteria parameter.

##### 7.3.3.17.11	Filter Usage request parameter

The filterUsage element of the Filter Criteria parameter does not represent a filter condition. It indicates how the Filter Criteria parameter shall be used. If this parameter is not provided, the Retrieve request primitive which includes this element triggers a generic retrieve operation. The data type of filterUsage is defined in clause 6.3.3.2.31.

### 7.3.4	Management common operations

#### 7.3.4.1	Identify the managed entity and the technology specific protocol

The Hosting CSE shall identify the managed entity to be managed via the <node> resource which is the parent resource in case of an addressed <mgmtObj> resource. In case of a <mgmtCmd> resource the entity to be managed is indicated in the execTarget attribute which addresses either a <node> resource or a group of resources of type <node>. Hence, in all cases the managed entity is ultimately identified through the <node> resource, from which the identifier of the device can be retrieved.

Then the Hosting CSE shall determine the technology specific protocol to be used for communicating with the managed entity based on the objectID of the addressed <mgmtObj> resource. If the managed entity cannot be identified, the Hosting CSE shall reject the request with the Response Status Code indicating "EXTERNAL_OBJECT_NOT_REACHABLE" in the Response primitive.

#### 7.3.4.2	Locate the technology specific data model objects to be managed on the managed entity

The Hosting CSE shall locate the technology specific data model object to be managed on the managed entity by the objectPaths attribute of the <mgmtObj> resource addressed by the URI provided in the To primitive parameter. In the case that the To addresses an [objectAttribute], the Hosting CSE shall locate the technology specific data model object on the managed entity through the objectPaths attribute of the <mgmtObj> resource of the addressed [objectAttribute], combined with their relative position in the technology specific data model object tree. If the technology specific data model object cannot be located, the Hosting CSE shall reject the request with the Response Status Code indicating "EXTERNAL_OBJECT_NOT_FOUND" in the Response primitive.

In the case that the management server is external to the Hosting CSE, the Hosting CSE shall identify the management server that is capable of performing the operation on the technology specific data model object. If the management server cannot be identified, the Hosting CSE shall reject the request with the Response Status Code indicating "EXTERNAL_OBJECT_NOT_REACHABLE" in the Response primitive.

#### 7.3.4.3	Establish a management session with the managed entity or management server

In the case that the management server is embedded with the CSE, if there is no existing management session between the Hosting CSE and the managed entity, the Hosting CSE shall also trigger the managed entity to establish a management session with the Hosting CSE by sending triggering message to the managed entity using the determined technology specific protocol in case such triggering mechanism is supported by the technology specific protocol. If the triggering mechanism is not supported by the technology specific protocol, the Hosting CSE shall reject the request with the Response Status Code indicating "MGMT_SESSION_CANNOT_BE_ESTABLISHED". If the management session cannot be established with the managed entity, the Hosting CSE shall reject the request with the Response Status Code indicating "MGMT_SESSION_CANNOT_BE_ESTABLISHED". If the management session cannot be established within a limited time span as per local policy, the Hosting CSE shall reject the request with the Response Status Code indicating "MGMT_SESSION_ESTABLISHMENT_TIMEOUT" in the Response primitive.

In the case that the management server is external to the Hosting CSE, if there is no existing management session between the Hosting CSE and the management server that manages the technology specific data model objects, the Hosting CSE shall establish a session with the managed entity with the necessary access control privileges to perform the technology specific request on the technology specific protocol. If the management session cannot be established with the management server, the Hosting CSE shall reject the request with Response Status Code indicating "MGMT_SESSION_CANNOT_BE_ESTABLISHED". If the management session cannot be established within a limited time span as per local policy, the Hosting CSE shall reject the request with Response Status Code indicating "MGMT_SESSION_ESTABLISHMENT_TIMEOUT" in the Response primitive.

#### 7.3.4.4	Send the management request(s) to the managed entity corresponding to the received Request primitive

The Hosting CSE shall send the management request(s) to the managed entity or management server in the established management session in order to perform the management operation as requested by the received Request primitive. The management request shall address the technology specific data model object on the managed entity as determined in clause 7.3.4 or in the primitive specific clauses. The technology specific  request being used is specific to the technology specific protocol according to a pre-defined mapping relationship with the Request primitive.

The internal data structure of the technology specific data model object addressed by the technology specific request shall be determined based on the mapping relationship of the <mgmtObj>, or <mgmtCmd> resources and the technology specific data model objects or based on the generic mapping rule as specified in ETSI TS 118 101 [6] clauses, 9.6.15, 9.6.16, and 9.6.17. The Hosting CSE shall extract the management results received from the managed entity or management server in order to prepare a Response primitive to be sent to the originator later. Unless explicitly stated, if the management request cannot be performed successfully, the Hosting CSE shall reject the Request primitive with the management server in the Response primitive according to the mapping relationship with the technology specific protocol.

## 7.4	Resource type-specific procedures and definitions

### 7.4.1	Introduction

The reference point applicability of each resource-specific procedure for the following sub-clauses is described in the corresponding procedure specification in clause 10.2(Resource Type-Specific Procedures) [6]. E.g., Applicable reference points of <container> resource creation procedure (clause 7.4.7.2.1) in present specification is described as Mca, Mcc and Mcc' in clause 10.2.4.1(create <container> procedure) [6].

### 7.4.2	Resource type specification conventions

#### 7.4.2.0	Introduction

This clause describes how to understand the following clauses for resource type-specific procedures and definitions.

#### 7.4.2.1	Resource type definition conventions

The following table includes the information of XSD data type definition files for the corresponding resource type.

Table 7.4.2.1-1: Data type definition of <resourceType>

| Data Type ID | File Name | Note |
| --- | --- | --- |
| Actual Data Type ID | XSD file name |  |

The following table includes the information of universal/common attributes of the resource type. Request optionality information means inclusion of the attribute name and its value in the request primitive is Mandatory(M)/Optional(O)/NP(Not Present). The value shall be empty in case of creator attribute to be set on CREATing a resource.This is applicable for Create and Update operation only. For Retrieve operation, attribute names are optionally included in the request, but not with any values. For Delete operation, any attribute names or their values cannot be included in the request.

Universal/common attributes do not have any default value, however, have value restrictions and notes (see Table 6.3.6-1).

Table 7.4.2.1-2: Universal/Common Attributes of <resourceType> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| Universal/common attribute name | M/O/NP | O/NP |

The following table includes the information of resource specific attributes of the resource type. Convention for request optionality is the same as the universal/common attribute table above.

Table 7.4.2.1-3: Resource Specific Attributes of <resourceType> resource

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| resource specific attribute name | M/O/NP | O/NP |  |  |

The following table includes the information of child resources of the resource type.

Table 7.4.2.1-4: Child resources of <resourceType> resource

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to Resource Type Definition |
| --- | --- | --- | --- |
| <resourceType> | "[variable]" or fixed name | Multiplicity of child resource instances in the corresponding resource type | Reference to the resource type definition in the present document. |

#### 7.4.2.2	Resource type-specific procedure conventions

This clause describes resource type specific procedures referring generic procedures defined in clause 7.2.2. Each operation specific procedure describes procedures for the Originator and the Receiver. If the resource and operation specific procedure is the same as the generic procedure, the Originator and Receiver procedure refer to them. Otherwise, the deviation/addition is clearly described with related procedure numbers (e.g., Recv 6.1) in clause 7.2.2.

If a deviation/addition procedure includes sub-procedures in one more level(s), proper numbering is used to show the levels (e.g., "1)",  "a)"). If sub-procedures do not care the order, bullets are used instead of numbers

### 7.4.3	Resource type <accessControlPolicy>

#### 7.4.3.1	Introduction

The <accessControlPolicy> resource is comprised of  privileges and selfPrivileges attributes which represent a set of access control rules  defining which entities (defined as accessControlOriginators) have the privilege to perform  certain operations (defined as accessContolOperations) within specified contexts (defined as accessControlContexts) and are used by the CSEs in making access decision to specific resources.

The detailed description can be found in clause 9.6.2 in ETSI TS 118 101 [6].

Table 7.4.3.1-1: Data type defintion of <accessControlPolicy> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| accessControlPolicy | CDT-accessControlPolicy-v1_6_0.xsd |  |

Table 7.4.3.1-2: Universal/Common Attributes of <accessControlPolicy> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| expirationTime | O | O |
| labels | O | O |
| creationTime | NP | NP |
| lastModifiedTime | NP | NP |
| announceTo | O | O |
| announcedAttribute | O | O |

Table 7.4.3.1-3: Resource Specific Attributes of <accessControlPolicy> resource

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| Privileges | M | O | m2m:setOfAcrs | No default |
| selfPrivileges | M | O | m2m:setOfAcrs | No default |

The following table includes the information of child resources of the resource type.

Table 7.4.3.1-4: Child Resources of <accessControlPolicy> resource

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to Resource Type Definition |
| --- | --- | --- | --- |
| <subscription> | [variable] | 0..n | clause 7.4.9 |

#### 7.4.3.2	accessControlPolicy resource specific procedure on CRUD operations

##### 7.4.3.2.0	Introduction

This sub-clause describes accessControlPolicy resource specific behaviour for CRUD operations.

##### 7.4.3.2.1	Create

Originator:

No changes from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

##### 7.4.3.2.2	Retrieve

Originator:

No changes from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

##### 7.4.3.2.3	Update

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

##### 7.4.3.2.4	Delete

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

### 7.4.4	Resource Type <CSEBase>

#### 7.4.4.1	Introduction

A <CSEBase> resource shall represent a CSE. This <CSEBase> resource shall be the root for all the resources that are residing on the CSE. The detailed description can be found in clause 9.6.3 in ETSI TS 118 101 [6]).

Table 7.4.4.1-1: Data type definition of <CSEBase> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| CSEBase | CDT-CSEBase-v1_6_0.xsd |  |

The value of the parentID attribute for the <CSEBase> resource shall be  an empty string since the <CSEBase> resource does not have a parent.

Table 7.4.4.1-2: Resource Specific Attributes of <CSEBase> resource

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
| cseType | NP | NP | m2m:cseTypeID | No default |
| CSE-ID | NP | NP | m2m:ID | No default |
| supportedResourceType | NP | NP | list of m2m:resourceType | No default |
| pointOfAccess | NP | NP | m2m:poaList | No default |
| nodeLink | NP | NP | xs:anyURI | No default |

Table 7.4.4.1-3: Child resources of <CSEBase> resource

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to Resource Type Definition |
| --- | --- | --- | --- |
| <remoteCSE> | [variable] | 0..n | Clause 7.4.5 |
| <node> | [variable] | 0..n | Clause 7.4.19 |
| <AE> | [variable] | 0..n | Clause 7.4.6 |
| <container> | [variable] | 0..n | Clause 7.4.7 |
| <group> | [variable] | 0..n | Clause 7.4.14 |
| <accessControlPolicy> | [variable] | 0..n | Clause 7.4.3 |
| <subscription> | [variable] | 0..n | Clause 7.4.9 |
| <mgmtCmd> | [variable] | 0..n | Clause 7.4.17 |
| <locationPolicy> | [variable] | 0..n | Clause 7.4.11 |
| <statsConfig> | [variable] | 0..n | Clause 7.4.24 |
| <statsCollect> | [variable] | 0..n | Clause 7.4.26 |
| <request> | [variable] | 0..n | Clause 7.4.13 |
| <delivery> | [variable] | 0..n | Clause 7.4.12 |
| <schedule> | [variable] | 0..1 | Clause 7.4.10 |
| <m2mServiceSubscriptionPolicy | [variable] | 0..n | Clause 7.4.20 |
| <serviceSubscribedAppRule> | [variable] | 0..n | Clause 7.4.30 |

#### 7.4.4.2	<CSEBase> resource specific procedure on CRUD operations

##### 7.4.4.2.1	Create

Originator:

The <CSEBase> resource shall not be created via API.

Receiver:

Primitive specific operation on Recv-1.0 "Check the syntax of received message":

- If the request is received, the Receiver CSE shall execute the following steps in order.

- "Create an unsuccessful Response primitive" with the Response Status Code indicating "OPERATION_NOT_ALLOWED" error.

- "Send the Response primitive".

##### 7.4.4.2.2	Retrieve

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

##### 7.4.4.2.3	Update

Originator:

The <CSEBase> resource shall not be updated via API.

Receiver:

Primitive specific operation on Recv-1.0 "Check the syntax of received message":

- If the request is received, the Receiver CSE shall execute the following steps in order.

- "Create an unsuccessful Response primitive" with the Response Status Code indicating "OPERATION_NOT_ALLOWED" error .

"Send the Response primitive".

##### 7.4.4.2.4	Delete

Originator:

The <CSEBase> resource shall not be DELETEed via API.

Receiver:

Primitive specific operation on Recv-1.0 "Check the syntax of received message":

- If the request is received, the Receiver CSE shall execute the following steps in order.

- "Create an unsuccessful Response primitive" with the Response Status Code indicating "OPERATION_NOT_ALLOWED" error.

"Send the Response primitive".

### 7.4.5	Resource Type <remoteCSE>

#### 7.4.5.1	Introduction

A <remoteCSE> resource shall represent a remote CSE that is registered to the Registrar CSE. <remoteCSE> resources shall be located directly under the <CSEBase>.

Conversely each registered CSE shall also be represented as a sub-set of <remoteCSE> resource in the registering CSE's <CSEBase>.

The detailed description can be found in clause 9.6.4 in Architecture TS.

Table 7.4.5.1-1: Data type definition of <remoteCSE> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| remoteCSE | CDT-remoteCSE-v1_6_0.xsd |  |

Table 7.4.5.1-2: Universal/Common Attributes of <remoteCSE> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| labels | O | O |
| announceTo | O | O |
| announcedAttribute | O | O |

Table 7.4.5.1-3: Resource Specific Attributes of <remoteCSE> resource

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| cseType | O | NP | m2m:cseTypeID | No default |
| pointOfAccess | O | O | m2m:poaList | No default |
| CSEBase | M | NP | xs:anyURI | No default |
| CSE-ID | M | NP | m2m:ID | No default |
| M2M-Ext-ID | O | O | m2m:externalID | No default |
| Trigger-Recipient-ID | O | O | m2m:triggerRecipientID | No default |
| requestReachability | M | O | xs:boolean | No default |
| nodeLink | O | O | xs:anyURI | No default |

Table 7.4.5.1-4: Child resources of <remoteCSE> resource

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to Resource Type Definition |
| --- | --- | --- | --- |
| <container> | [variable] | 0..n | Clause 7.4.7 |
| <group> | [variable] | 0..n | Clause 7.4.14 |
| <accessControlPolicy> | [variable] | 0..n | Clause 7.4.3 |
| <subscription> | [variable] | 0..n | Clause 7.4.9 |
| <pollingChannel> | [variable] | 0..1 | Clause 7.4.22 |
| <schedule> | [variable] | 0..n | Clause 7.4.10 |

#### 7.4.5.2	<remoteCSE> resource specific procedure on CRUD operations

##### 7.4.5.2.0	Introduction

The entire CSE registration procedure including <CSE> resource creation procedure below is defined in 10.1.1.2.1 [6] ("CSE registration procedure").

##### 7.4.5.2.1	Create

Originator:

No change from the generic procedures in clause 7.2.2.1 with the following exception:

An AE shall not originate a Create <remoteCSE> resource request.

Receiver:

- Primitive specific operation on Recv-1.0 "Check the syntax of received message":If the request is received over the Mca reference point, the Receiver CSE shall execute the following steps in order.

- "Create an unsuccessful Response primitive" with the response status code 'OPERATION_NOT_ALLOWED'.

- "Send the Response primitive"

NOTE: Determination of the reference point is to the discretion of the Receiver CSE implementation.

##### 7.4.5.2.2	Retrieve

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.1.

##### 7.4.5.2.3	Update

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.1.

##### 7.4.5.2.4	Delete

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.1.

### 7.4.6	Resource Type <AE>

#### 7.4.6.1	Introduction

The <AE> resource represents information about an Application Entity known to a given Common Services Entity.

The detailed description can be found in clause 9.6.5 in ETSI TS 118 101 [6].

Table 7.4.6.1-1: Data type definition of <AE> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| AE | CDT-AE-v1_6_0.xsd | XSD schema for AE resource |

Table 7.4.6.1-2: Universal/Common Attributes of <AE> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| labels | O | O |
| announceTo | O | O |
| announcedAttribute | O | O |

Table 7.4.6.1-3: Resource Specific Attributes of <AE> resource

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| appName | O | O | xs:string | No default |
| App-ID | M | NP | xs:string | No default |
| AE-ID | NP | NP | m2m:ID | No default |
| pointOfAccess | O | O | m2m:poaList | No default |
| ontologyRef | O | O | xs:anyURI | No default |
| nodeLink | NP | NP | xs:anyURI | No default |
| requestReachability | M | O | xs:boolean | No default |
| contentSerialization | O | O | m2m:serializations | No default |

Table 7.4.6.1-4: Child resources of <AE> resource

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to Resource Type Definition |
| --- | --- | --- | --- |
| <subscription> | [variable] | 0..n | Clause 7.4.9 |
| <container> | [variable] | 0..n | Clause 7.4.7 |
| <group> | [variable] | 0..n | Clause 7.4.14 |
| <accessControlPolicy> | [variable] | 0..n | Clause 7.4.3 |
| <pollingChannel> | [variable] | 0..n | Clause 7.4.22 |
| <schedule> | [variable] | 0..n | Clause 7.4.10 |

#### 7.4.6.2	<AE> resource specific procedure on CRUD+N operations

##### 7.4.6.2.1	Introduction

This clause describes AE resource specific behaviour for CRUD+N operations.

The entire AE registration procedure including <AE> resource creation procedure below is defined in clause 10.1.1.2.2 of the ETSI TS 118 101 [6] ("Application Entity registration procedure").

##### 7.4.6.2.2	Create

Originator:

No change from the generic procedures in clause 7.2.2.1 with the with the following exception:

A CSE shall not originate a Create <AE> resource request.

Receiver:

Primitive specific operation on Recv-1.0 "Check the syntax of received message":

- If the request is received over Mcc or Mcc' reference point, the Receiver CSE shall execute the following steps in order.

- "Create an unsuccessful Response primitive" with the Response Status Code 'OPERATION_NOT_ALLOWED'.

- "Send the Response primitive".

NOTE:Determination of the reference point is to the discretion of the Receiver CSE implementation.

- Otherwise,

- No change from the generic procedures in clause 7.2.2.2.

##### 7.4.6.2.3	Retrieve

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

##### 7.4.6.2.4	Update

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

##### 7.4.6.2.5	Delete

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

##### 7.4.6.2.6	Notify

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

### 7.4.7	Resource Type <container>

#### 7.4.7.1	Introduction

This resource represents a container for data instances. It is used to share information among other entities and potentially to track the data. A <container> resource has no associated content, only attributes and child resources.

The detailed description can be found in clause 9.6.6 in ETSI TS 118 101 [6].

Table 7.4.7.1-1: Data type definition of <container> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| Container | CDT-container-v1_6_0.xsd |  |

Table 7.4.7.1-2: Universal/Common Attributes of <container> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| stateTag | NP | NP |
| labels | O | O |
| announceTo | O | O |
| announcedAttribute | O | O |

Table 7.4.7.1-3: Resource Specific Attributes of <contianer> resource

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| creator | O | NP | m2m:ID | No default |
| maxNrOfInstances | O | O | xs:nonNegativeInteger | No default |
| maxByteSize | O | O | xs:nonNegativeInteger | No default |
| maxInstanceAge | O | O | xs:nonNegativeInteger | No default |
| currentNrOfInstances | NP | NP | xs:nonNegativeInteger | No default (This is generated by the hosting CSE and limited by the maxNrOfInstances) |
| currentByteSize | NP | NP | xs:nonNegativeInteger | No default (This is generated by the hosting CSE and limited by the maxByteSize) |
| locationID | O | O | xs:anyURI | No default |
| ontologyRef | O | O | xs:anyURI | No default |

Table 7.4.7.1-4: Child resources of <container> resource

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to in Resource Type Definition |
| --- | --- | --- | --- |
| <contentInstance> | [variable] | 0..n | Clause 7.4.8 |
| <subscription> | [variable] | 0..n | Clause 7.4.9 |
| <container> | [variable] | 0..n | Clause 7.4.7 |
| <latest> | latest | 1 | Clause 7.4.28 |
| <oldest> | oldest | 1 | Clause 7.4.29 |

#### 7.4.7.2	<container> resource specific procedure on CRUD operations

##### 7.4.7.2.0	Introduction

This clause describes container resource specific behaviour for CRUD operations.

##### 7.4.7.2.1	Create

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

##### 7.4.7.2.2	Retrieve

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

##### 7.4.7.2.3	Update

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

.

##### 7.4.7.2.4	Delete

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

### 7.4.8	Resource Type <contentInstance>

#### 7.4.8.1	Introduction

The <contentInstance> resource represents a data instance in the container.

The detailed description can be found in clause 9.6.7 in ETSI TS 118 101 [6].

Table 7.4.8.1-1: Data type definition of <contentInstance> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| contentInstance | CDT-contentInstance-v1_6_0.xsd |  |

Table 7.4.8.1-2: Universal/Common Attributes of <contentInstance> resource

| Attribute Name | Request Optionality |
| --- | --- |
|  | Create |
| @resourceName | O |
| resourceType | NP |
| resourceID | NP |
| parentID | NP |
| expirationTime | O |
| creationTime | NP |
| lastModifiedTime | NP |
| stateTag | NP |
| labels | O |
| announceTo | O |
| announcedAttribute | O |

Table 7.4.8.1-3: Resource Specific Attributes of <contentInstance> resource

| Attribute Name | Request Optionality | Data Type | Default Value and Constraints |
| --- | --- | --- | --- |
|  | Create |  |  |
| creator | O | m2m:ID |  |
| contentInfo | O | m2m:contentInfo | No default. |
| contentSize | NP | xs:nonNegativeInteger | No default. |
| ontologyRef | O | xs:anyURI | No default. |
| content | M | xs:anySimpleType | No default (Transfer encoding may be applied, and indicated applied encoding as part of the contentInfo attribute). |

The contentInfo attribute shall provide meta information about the stored data in content. m2m:encodingType (0;plain, 1:base64 encoded string, 2:base64 encoded binary), and is optional.

#### 7.4.8.2	<contentInstance> resource specific procedure on CRUD operations

##### 7.4.8.2.1	Create

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

Primitive specific operation on Recv-6.5 "Create/Update/Retrieve/Delete/Notify operation is performed":

- The hosting CSE shall set the contentSize to the size in bytes of the content attribute.

- currentNrOfInstances and currentByteSize of direct parent <container> resorce shall be updated. If currentNrOfInstances and/or currentByteSize exceeds maxNrOfInstances and/or maxByteSize of direct parent <container> resource respectively, the hosting CSE shall return the response primitive with a Response Status Code indicating " NOT_ACCEPTABLE" error.

No other changes from the generic procedures in clause 7.2.2.2. The Originator may omit the name of the <contentInstance> resource unless the Originator need to refer specific content later.

##### 7.4.8.2.2	Retrieve

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2. The Originator may omit the name of the targeted <contentInstance> resource when the latest version of stored content is requested.

##### 7.4.8.2.3	Update

Originator:

The <contentInstance> resource shall not be Updated via API.

Receiver:

Primitive specific operation on Recv-1.0 "Check the syntax of received message":

If the request is received, the Receiver CSE shall execute the following steps in order.

- "Create an unsuccessful Response primitive" with the Response Status Code indicating "OPERATION_NOT_ALLOWED" error.

- "Send the Response primitive".

##### 7.4.8.2.4	Delete

Originator:

Primitive specific operation on Recv-6.5 "Create/Update/Retrieve/Delete/Notify operation is performed":

- currentNrOfInstances and currentByteSize of direct parent <container> resorce shall be updated.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

### 7.4.9	Resource Type <subscription>

#### 7.4.9.1	Introduction

The <subscription> resource contains subscription information for its subscribed-to resource. The subscription resource is a child of the subscribed to resource.

The detailed description can be found in clause 9.6.8 in ETSI TS 118 101 [6].

Table 7.4.9.1-1: Data type definition of <subscription> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| subscription | CDT-subscription-v1_6_0.xsd |  |

Table 7.4.9.1-2: Universal/Common Attributes of <subscription> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| labels | O | O |

Table 7.4.9.1-3: Resource Specific Attributes of <subscription> resource

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| eventNotificationCriteria | O | O | m2m:eventNotificationCriteria | Default behaviour is notification on Update_of_Resource |
| expirationCounter | O | O | xs:positiveInteger | No default |
| notificationURI | M | O | list of xs:anyURI | No default |
| groupID | O | O | xs:anyURI | No default |
| notificationForwardingURI | O | O | xs:anyURI | No default |
| batchNotify | O | O | m2m:batchNotify | No default |
| rateLimit | O | O | m2m:rateLimit | No default |
| preSubscriptionNotify | O | NP | xs:positiveInteger | No default |
| pendingNotification | O | O | m2m:pendingNotification | No default |
| notificationStoragePriority | O | O | xs:positiveInteger | No default |
| latestNotify | O | O | xs:boolean | No default |
| notificationContentType | O | O | m2m:notificationContentType | No default |
| notificationEventCat | O | O | m2m:eventCat | No default |
| creator | O | NP | m2m:ID | No default |
| subscriberURI | O | NP | xs:anyURI | No default |

Table 7.4.9.1-4: Reference of child resources

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to in Resource Type Definition |
| --- | --- | --- | --- |
| <schedule> | notificationSchedule | 0..1 | Clause 7.4.10 |

#### 7.4.9.2	<subscription> resource specific procedure on CRUD operations

##### 7.4.9.2.1	Create

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

The following are additional Hosting CSE procedures to the generic resource handling procedures (Figure 7.2.2.2-1 in clause 7.2.2.2). The additional procedures shall be inserted from Recv-6.2 to Recv-6.8 as below.

The resource handling procedure for the Hosting CSE which receives <subscription> CREATE request shall perform the following procedures in order:

- Recv-6.2

- Recv-6.3

- Check if the subscribed-to resource, addressed in To parameter in the Request, is subscribable. Subscribable resource types are defined in ETSI TS 118 101 [6], they have <subscription> resource types as their child resources.

If it is not subscribable, the Hosting CSE shall return the Notify response primitive with a Response Status Code indicating "TARGET_NOT_SUBSCRIBABLE" error.

- Check if the Originator has privileges for retrieving the subscribed-to resource.

If the Originator does not have the privilege, the Hosting CSE shall return the Notify response primitive with Response Status Code indicating "NO_PRIVILEGE" error.

- If the notificationURI is not the Originator, the Hosting CSE should send a Notify request primitive to the notificationURI with verificationRequest parameter set as TRUE (See clause 7.5.1.2.3).

  - If the Hosting CSE cannot send the Notify request primitive, the Hosting CSE shall return the Notify response primitive with a Response Status Code indicating "SUBSCRIPTION_VERIFICATION_INITIATION_FAILED" error.

  - If the Hosting CSE sent the primitive, the Hosting CSE shall check if the Notify response primitive contains a Response Status Code indicating "SUBSCRIPTION_CREATOR_HAS_NO_PRIVILEGE" or "SUBSCRIPTION_HOST_HAS_NO_PRIVILEGE error. If so, the Hosting CSE shall return the Create response primitive with a Response Status Code indicating the same error from the Notify response primitive  to the Originator.

- Recv-6.4

- Recv-6.5

If the notificationURI is not the Originator, the Hosting CSE shall store Originator ID to creator attribute.

- Recv-6.6

- Recv-6.7

- Recv-6.8

##### 7.4.9.2.2	Retrieve

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

##### 7.4.9.2.3	Update

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

##### 7.4.9.2.4	Delete

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

### 7.4.10	Resource Type <schedule>

#### 7.4.10.1	Introduction

The <schedule> resource shall represent scheduling information in the context of its parent resource. If a <schedule> resource is not present as a child resource then there are no time-constraints on the context of its parent resource. An Originator shall have the same access control privileges to the <schedule> resource as it has to its parent resource.

The detailed <schedule> resource description can be found in clause 9.6.9 of the ETSI TS 118 101 [6].

Table 7.4.10.1-1: Data type definition of <schedule> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| schedule | CDT-schedule-v1_6_0.xsd |  |

Table 7.4.10.1-2: Universal/Common Attributes of <schedule> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| labels | O | O |
| announceTo | O | O |
| announcedAttribute | O | O |

Table 7.4.10.1-3: Resource Specific Attributes of <schedule> resource

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| scheduleElement | M | O | m2m:scheduleEntries | No Default and shall not be blank. |

The scheduleElement attribute represents the list of scheduled tasks with the time of the execution.

The each entry of the scheduleElelement attribute shall consist of a line with 6 field values (See Table 7.3.8.1-4).

The time to be matched with the schedule pattern shall be interpreted in UTC timezone.

Table 7.4.10.1-4:  Definition of m2m:scheduleEntry string format

| Field Name | Range of values | Note |
| --- | --- | --- |
| Second | 0 to 59 |  |
| Minute | 0 to 59 |  |
| Hour | 0 to 23 |  |
| Day of the month | 1 to 31 |  |
| Month of the year | 1 to 12 |  |
| Day of the week | 0 to 6 | 0 means Sunday |

Each field value can be either an asterisk ('*': matching all valid values), an element, or an elements separated by commas(',').

An element shall be either a number or two numbers separated by a hyphen ('-': matching between two values).

The task which shall be executed is depending on the parent resource of the <schedule> resource (see Table 7.3.8.1-5).

Table 7.3.8.1-5: The task to be executed

| Parent resource | Task to be executed | Note |
| --- | --- | --- |
| <remoteCSE> | Establish connection to the remoteCSE | Timing of disconnection is up to implementation in present release. |
| <subscription> | Flash spooled notifications |  |

EXAMPLE 1:

EXAMPLE: * 0-5 2,6,10 * * *

In case of parent resource was <remoteCSE>, the CSE will be establish connection on 2:00-2:05, 6:00-6:05, and 10:00-10:05 every day.

End of EXAMPLE 1:

EXAMPLE 2:

EXAMPLE: * * 8-20 * * *

In case of the parent resource was <subscription>, the notification for the subscribed event will be suspended between from 20:00 to 8:00 on weekend.

End of EXAMPLE 2:

Table 7.4.10.1-5: Child resources of <schedule > resource

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to in Resource Type Definition |
| --- | --- | --- | --- |
| Subscription | [variable] | 0..n | Clause 7.4.9 |

#### 7.4.10.2	<schedule> resource specific procedure on CRUD operations

##### 7.4.10.2.0	Introduction

This sub-clause describes <schedule> resource specific behaviour for CRUD operations.

##### 7.4.10.2.1	Create

Originator:

No change from the generic procedures in clause 7.2.2.1.

If <schedule> is created then scheduleElement (L) shall be created.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

##### 7.4.10.2.2	Retrieve

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

##### 7.4.10.2.3	Update

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

##### 7.4.10.2.4	Delete

Originator:

No change from the generic procedures in clause 7.2.2.1.

If <schedule> is deleted then scheduleElement (L) shall be deleted.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

### 7.4.11	Resource Type <locationPolicy>

#### 7.4.11.1	Introduction

The <locationPolicy> resource represents the method for obtaining and managing geographical location information of an M2M Node. The detailed description can be found in the clause 9.6.10 in ETSI TS 118 101 [6] .

Table 7.4.11.1-1: Data type definition of <locationPolicy> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| locationPolicy | CDT-locationPolicy-v1_6_0.xsd |  |

Table 7.4.11.1-2: Universal/Common Attributes of <locationPolicy> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| expirationTime | O | O |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| lastModifiedTime | NP | NP |
| labels | O | O |
| announceTo | O | O |
| announcedAttribute | O | O |

Table 7.4.11.1-3: Resource Specific Attributes of <locationPolicy> resource

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| locationSource | M | NP | m2m:locationSource | No default |
| locationUpdatePeriod | O | O | xs:duration | No default |
| locationTargetID | O | NP | m2m:nodeID | No default |
| locationServer | O | NP | xs:anyURI | No default |
| locationContainerID | NP | NP | xs:anyURI | No default |
| locationContainerName | O | O | xs:string | No default |
| locationStatus | NP | NP | xs:string | No default |

Table 7.4.11.1-4: Child resources of <locationPolicy> resource

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to Resource Type Definition |
| --- | --- | --- | --- |
| <subscription> | [variable] | 0..n | Clause 7.4.9 |

#### 7.4.11.2	<locationPolicy> resource specific procedure on CRUD Operations

##### 7.4.11.2.0	Introduction

This clause describes <locationPolicy> resource specific primitive behaviour for CRUD operations.

##### 7.4.11.2.1	Create

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

The following <locationPolicy> resource type specific procedures shall be performed after Recv-6.5 and before Recv-6.6 generic procedures.

- After the successful creation of <locationPolicy> resource, the Hosting CSE shall create <container> resource where the actual location information will be stored and the resource shall contain cross-references for the both resources, locationContainerID attribute for the <locationPolicy> resource and locationID attribute for the <container> resource. The name of the created <container> resource shall be determined by the locationContainerID attribute if it is applicable.

Check the locationSource and locationUpdatePeriod attributes:

- If the period is set as '0' or NULL, then the each positioning procedure as defined in 3), 4) and 5) shall be performed when the <latest> child resource of <container> resource, which links to the <locationPolicy> resource, is retrieved. The newly acquired location information shall be stored as the <contentInstance> child resource of the the <container> resource.

- If the locationSource attribute is set by 'Network Based' and locationUpdatePeriod attribute is set by any duration value (higher than 0 second), then continue with the step 3.

- If the locationSource attribute is set by 'Device Based' and locationUpdatePeriod attribute is set by any duration value (higher than 0 second), then continue with the step 4.

- If the locationSource attribute is set by 'Sharing Based' and locationUpdatePeriod attribute is set by any duration value (higher than 0 second), then continue with the step 5.

The Hosting CSE shall retrieve the locationTargetID and locationServer attributes from the stored <location Policy> resource.

In case either the locationTargetID or locationServer attribute cannot be obtained, the hosting CSE shall reject the request with the Response Status Code indicating "BAD_REQUEST" error. Then, the Hosting CSE shall transform the location-acquisition request into Location Server request [28], using the attributes stored in <locationPolicy> resource. The Hosting CSE shall also provide default values for other required parameters (e.g. quality of position) in the Location Server request according to local policies.

The Hosting CSE shall send this Location Server request to the location server using, for example, OMA Mobile Location Protocol [i.4] and OMA RESTful NetAPI for Terminal Location [28]. The location server performs positioning procedure based upon the Location Server request. Then continue with step 6.

Based on the period information, locationUpdatePeriod attribute, this step can be periodically repeated or the location server can only notify the Hosting CSE of location information that performs periodically.

NOTE 1:	The location server performs the privacy control and only responds successfully if the positioning procedure is permitted.

NOTE 2:	The detail information on how the Location Server request message is converted into OMA RESTful NetAPI for Terminal Location message is described in Annex G.

The Hosting CSE shall perform positioning procedure using location determination modules and technologies (e.g. GPS). Then continue with step 6.

Based on the period information, locationUpdatePeriod attribute, this step can be periodically repeated.

NOTE 3:	The Hosting CSE can utilize the internal interface (e.g. System Call) to communicate with the modules and technologies. The detailed procedure is out of scope.

The Hosting CSE shall collect information of topology of M2M Area Network using <node> resource and find the closest Node from the Originator that has registered with the Hosting CSE and has location information. The closest Node is determined by the minimum hop based on the collected topology information.

- If the Hosting CSE can find the closest Node from the Originator, the location information of the closest Node shall be stored as the location information of the Originator into a <contentInstance> resource under the created <container> resource.

If the Hosting CSE cannot find the closest Node from the Originator, the location information of the Hosting CSE shall be stored as the location information of the Originator into a <contentInstance> resource under the created <container> resource.

The Hosting CSE shall receive the corresponding response and transform it into a Response primitive.

- If the positioning procedure is failed, the Hosting CSE shall store a statusCode based on the error code in the locationStatus attribute in the created <locationPolicy> resource.

If the positioning procedure is successfully complete which means that the Hosting CSE acquires the location information, The Hosting CSE shall store the acquired location information into a <contentInstance> resource under the created <container> resource.

##### 7.4.11.2.2	Retrieve

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

##### 7.4.11.2.3	Update

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2 with the following addidtion to Recv-6.5:

If the value of locationUpdatePeriod attribute is updated to 0 or NULL, the Hosting CSE shall stop periodical positioning procedure and perform the procedures as specified in 2) of Receiver in clause 7.2.10.2.1 (<locationPolicy>  create procedure).

If the value of locationUpdatePeriod attribute is updated to bigger than 0 (e.g., 1 hour) from 0 or NULL, the Hosting CSE shall start periodical positioning procedure as specified in 2) of Receiver in clause 7.2.10.2.1 (<locationPolicy>  create procedure).

##### 7.4.11.2.4	Delete

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

The procedure of the Receiver written in the clause 7.2.2.2 (from Rcv-D-1.0 to Rcv-D-10.0) shall be the same as initial steps. A following step is the <locationPolicy> resource type specific procedure for DELETE operation.

- Once the <locationPolicy> resource is deleted, the Receiver shall delete the associated resources (e.g. <container>, <contentInstance> resources). If the locationSource attribute and the locationUpdatePeriod attribute of the <locationPolicy> resource has been set with appropriate value, the Receiver shall tear down the session. The specific mechanism used to tear down the session depends on the support of the Underlying Network and other factors.

### 7.4.12	Resource Type <delivery>

#### 7.4.12.1	Introduction

In order to be able to initiate and manage the execution of data delivery in a resource-based manner, resource type delivery is defined. This resource type shall be used for forwarding requests from one CSE to another CSE when the Delivery Aggregation parameter in the request is set to ON. The detailed description can be found in clause 9.6.11 in ETSI TS 118 101 [6].

Table 7.4.12.1-1: Data type definition of <delivery> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| delivery | CDT-delivery-v1_6_0.xsd |  |

Table 7.4.12.1-2: Universal/Common Attributes of <delivery> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| expirationTime | O | O |
| parentID | NP | NP |
| creationTime | NP | NP |
| lastModifiedTime | NP | NP |
| accessControlPolicyIDs | O | O |
| labels | O | O |
| stateTag | NP | NP |

Table 7.4.12.1-3: Resource Specific Attributes of <delivery> resource

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| source | M | NP | m2m:ID | No default |
| target | M | NP | m2m:ID | No default |
| lifespan | M | O | m2m:timestamp | No default |
| eventCat | M | O | m2m:eventCat | No default |
| deliveryMetaData | M | O | m2m: deliveryMetaData | No default |
| aggregatedRequest | O | O | m2m:aggregatedRequest | No default |

Table 7.4.12.1-4: Child resources of <delivery> resource

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to Resource Type Definition |
| --- | --- | --- | --- |
| <subscription> | variable | 0..n | Clause 7.4.9 |

#### 7.4.12.2	<delivery> resource specific procedure on CRUD operations

##### 7.4.12.2.0	Introduction

This clause describes <delivery> resource specific behaviour for CRUD operations.

##### 7.4.12.2.1	Create

Originator:

An AE shall not originate a Create <delivery> resource request.

Primitive specific operation on Orig-1.0 "Compose Request primitive":

- The Originator shall use a blocking request (i.e. Response Type=blockingRequest).

- The Originator shall provide the content of the <delivery> resource.

No change for the remaining steps from the generic procedures in clause 7.2.2.1.

Receiver:

Primitive specific operation on Recv-1.0 "Check the syntax of received message":

- If the request is received over Mca reference point, the Receiver CSE shall execute the following steps in order.

  - "Create an unsuccessful Response primitive" with a Response Status Code indicating 'OPERATION_NOT_ALLOWED' error.

  - "Send the Response primitive".

Otherwise,

  - No change from the generic procedures in clause 7.2.2.2.

NOTE: Determination of the reference point is to the discretion of the Receiver CSE implementation.

##### 7.4.12.2.2	Retrieve

Originator:

Primitive specific operation on Orig-1.0 "Compose Request primitive":

- The Originator shall use a blocking request (i.e. Response Type=blockingRequest).

No change for the remaining steps from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

##### 7.4.12.2.3	Update

Originator:

An AE shall not originate a Create <delivery> resource request.

Primitive specific operation on Orig-1.0 "Compose Request primitive":

- The Originator shall use a blocking request (i.e. Response Type=blockingRequest).

- The Originator shall provide the content of the <delivery> resource.

No change for the remaining steps from the generic procedures in clause 7.2.2.1.

Receiver:

Primitive specific operation on Recv-1.0 "Check the syntax of received message":

- If the request is received over Mca reference point, the Receiver CSE shall execute the following steps in order.

  - "Create an unsuccessful Response primitive" with a Response Status Code indicating 'OPERATION_NOT_ALLOWED' error.

  - "Send the Response primitive".

Otherwise,

  - No change from the generic procedures in clause 7.1.2.2.

NOTE: Determination of the reference point is to the discretion of the Receiver CSE implementation.

##### 7.4.12.2.4	Delete

Originator:

An AE shall not originate a Create <delivery> resource request.

Primitive specific operation on Org-1.0 "Compose Request primitive":

- The Originator shall use a blocking request (i.e. Response Type=blockingRequest).

No change for the remaining steps from the generic procedures in clause 7.2.2.1.

Receiver:

Primitive specific operation on Recv-1.0 "Check the syntax of received message":

- If the request is received over Mca reference point, the Receiver CSE shall execute the following steps in order.

  - "Create an unsuccessful Response primitive" with a Response Status Code indicating 'OPERATION_NOT_ALLOWED' error.

  - "Send the Response primitive".

Otherwise,

  - No change from the generic procedures in clause 7.2.2.2

NOTE: Determination of the reference point is to the discretion of the Receiver CSE implementation.

### 7.4.13	Resource Type <request>

#### 7.4.13.1	Introduction

The <request> resource is used to represent information on locally issued requests (i.e. issued by an AE or CSE internal). This allows for robust synchronous and asynchronous request processing coping with various constraints on maximum blocking time. When an AE or CSE issues a request for targeting any other resource type or requesting a notification in non-blocking mode , i.e. the Response Type parameter of the request is set to either 'nonBlockingRequestSynch' or 'nonBlockingRequestAsynch', and if the Registrar CSE of the Originator supports the <request> resource type as indicated by the supportedResourceType attribute of the <CSEBase> resource representing the Registrar CSE of the Originator, the Registrar CSE of the Originator shall create an instance of <request> to capture and expose the context of the associated non-blocking request. The detailed description can be found in clause 9.6.12 in ETSI TS 118 101 Architecture TS[6].

Table 7.4.13.1-1: Data type definition of <request> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| request | CDT-request-v1_6_0.xsd |  |

Table 7.4.13.1-2: Universal/Common Attributes of <request> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | NP | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| expirationTime | NP | NP |
| parentID | NP | NP |
| creationTime | NP | NP |
| lastModifiedTime | NP | NP |
| accessControlPolicyIDs | NP | NP |
| labels | NP | NP |
| stateTag | NP | NP |

Table 7.4.13.1-3: Resource Specific Attributes of <request> resource

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| operation | NP | NP | m2m:operation | No default |
| target | NP | NP | xs:anyURI | No default |
| originator | NP | NP | m2m:ID | No default |
| requestID | NP | NP | m2m:requestID | No default |
| metaInformation | NP | NP | m2m:metaInformation | No default |
| primitiveContent | NP | NP | m2m:primitiveContent | No default |
| requestStatus | NP | NP | m2m:requestStatus | No default |
| operationResult | NP | NP | m2m:operationResult | No default |

Table 7.4.13.1-4 : Reference of child resources

| Child Resource Type Name | Child Resource Name | Multiplicity | Ref. to in Resource Type Definition |
| --- | --- | --- | --- |
| <subscription> | [variable] | 0..n | Clause 7.4.9 |

#### 7.4.13.2	<request> resource specific procedure on CRUD operations

##### 7.4.13.2.0	Introduction

This clause describes request resource specific procedure on Resource Hosting CSE for CRUD operations.

##### 7.4.13.2.1	Create

Originator:

The <request> resource shall not be created via API. See clause 7.3.2.2 Create <request> resource locally.

Receiver:

Primitive specific operation on Recv-1.0 "Check the syntax of received message":

- "Create an unsuccessful Response primitive" with a Response Status Code indicating 'OPERATION_NOT_ALLOWED' error.

"Send the Response primitive".

##### 7.4.13.2.2	Retrieve

Originator: the procedure of the Originator is the same as the clause 7.2.2.1.

Receiver: the procedure of the Receiver is the same as the clause 7.2.2.2.

##### 7.4.13.2.3	Update

Originator:

The <request> resource shall not be updated via API. See clause 7.3.2.5 Update <request> resource.

Receiver:

Primitive specific operation on Recv-1.0 "Check the syntax of received message":

- "Create an unsuccessful Response primitive" with a Response Status Code indicating 'OPERATION_NOT_ALLOWED' error.

"Send the Response primitive".

##### 7.4.13.2.4	Delete

Originator: the procedure of the Originator is the same as the clause 7.2.2.1

Receiver: the procedure of the Receiver is the same as the clause 7.2.2.2.

### 7.4.14	Resource Type <group>

#### 7.4.14.1	Introduction

The <group> resource represents a group of resources of the same or mixed types. The <group> resource can be used to do bulk manipulations on the resources represented by the memberIDs attribute. The <group> resource contains an attribute that represents the members of the group and a virtual resource (the <fanOutPoint>) that allows operations to be applied to the resources represented by those members. The detailed description can be found in clause 9.6.13 in ETSI TS 118 101 [6].

Table 7.4.14.1-1: Data type definition of <group> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| group | CDT-group-v1_6_0.xsd |  |

Table 7.4.14.1-2: Universal/Common Attributes of <group> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| labels | O | O |
| announceTo | O | O |
| announcedAttribute | O | O |

Table 7.4.14.1-3: Resource Specific Attributes of <group> resource

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| creator | O | NP | m2m:ID |  |
| memberType | O | NP | m2m:memberType | Default value is set to 'MIXED' |
| currentNrOfMembers | NP | NP | xs:positiveInteger | No default (This is generated by the hosting CSE and limited by the maxNrOfMembers attribute of the <group> resource) |
| maxNrOfMembers | M | O | xs:positiveInteger | No default |
| memberIDs | M | O | list of xs:anyURI | No default |
| membersAccessControlPolicyIDs | O | O | list of xs:anyURI | No default |
| memberTypeValidated | NP | NP | xs:boolean | No default (This is generated by the hosting CSE) |
| consistencyStrategy | O | NP | m2m:consistencyStrategy | Default value is set to 'ABANDON_MEMBER' |
| groupName | O | O | xs:string | No default |

Table 7.4.14.1-4: Child resources of <group> resource

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to in Resource Type Definition |
| --- | --- | --- | --- |
| <subscription> | [variable] | 0..n | Clause 7.4.9 |
| <fanOutPoint> | fanOutPoint | 1 | Clause 7.4.15 |

#### 7.4.14.2	<group> resource specific procedure on CRUD operations

##### 7.4.14.2.1	Introduction

This clause describes <group> resource specific procedure on Resource Hosting CSE for CRUD operations.

##### 7.4.14.2.2	Create

Primitive specific operation after Recv-C-6.4 "Check validity of resource representation for the given resource type" and before Recv-C-6.5 "Create/Update/Retrieve/Delete/Notify operation is performed ". See clause 7.2.2.2.

- Primitive specific operation: Validate the provided attributes. It shall also check whether the number of URIs present in the memberIDs attribute of the group resource representation does not exceed the maximum as specified by the attribute maxNrOfMembers. If the maximum is exceeded, the request shall be rejected with a Response Status Code indicating "MAX_NUMBER_OF_MEMBER_EXCEEDED" error.
If the memberType attribute of the <group> resource is not "MIXED", the hosting CSE shall also verify that all the member URIs including sub-groups in the attribute memberIDs of the <group> resource representation provided in the request shall conform to the memberType of the group resource.

In the case that the <group> resource contains sub-group member resources, the receiver shall retrieve the memberType of the sub-group member resources to validate the memberType. If the memberType cannot be retrieved due to lack of privilege, the request shall be rejected with a Response Status Code indicating "NO_PRIVILEGE" error. If the sub-group member resources are temporarily unreachable, the receiver shall set the memberTypeValidated attribute of the <group> resource to FALSE and return the result to the originator in the response of the request. As soon as any unreachable sub-group resource becomes reachable, the receiver shall perform the memberType validation procedure. The originator may get to know the validation result by subscribe to the created resource if the memberTypeValidated attribute is FALSE. Upon unsuccessful validation, the receiver shall delete the <group> resource if the consistencyStrategy of the <group> resource is ABANDON_GROUP, or remove the inconsistent members from the <group> resource if the consistencyStrategy attribute is ABANDON_MEMBER, or set the memberType attribute of the <group> resource to "MIXED" if the consistencyStrategy attribute is SET_MIXED.
The memberTypeValidated attribute shall be set to TRUE if all the members have been validated successfully.

##### 7.4.14.2.3	Retrieve

No primitive specific operations.

##### 7.4.14.2.4	Update

- Primitive specific operation after Recv-6.4 "Check validity of resource representation for the given resource type" and before Recv-6.5 "Create/Update/Retrieve/Delete/Notify operation is performed ". See clause 7.2.2.2.Primitive specific operation: If the memberType attribute of the <group> resource is not "MIXED", the hosting CSE shall verify that all the member URIs including sub-groups in the attribute memberIDs of the <group> resource representation provided in the request shall conform to the memberType of the <group> resource.

In the case that the <group> resource contains sub-group member resources, the receiver shall retrieve the memberType of the sub-group member resource to validate the memberType. If the memberType cannot be retrieved due to lack of privilege, the request shall be rejected with a Response Status Code indicating "NO_PRIVILEGE" error. If the sub-group member resources are temporarily unreachable, the receiver shall set the memberTypeValidated attribute of the <group> resource to FALSE and return the result to the originator in the response of the request. As soon as any unreachable sub-group resource becomes reachable, the receiver shall perform the memberType validation procedure. The originator may get to know the validation result by subscribe to the created resource if the memberTypeValidated attribute is FALSE. Upon unsuccessful validation, the receiver shall delete the <group> resource if the consistencyStrategy of the <group> resource is ABANDON_GROUP, or remove the inconsistent members from the <group> resource if the consistencyStrategy attribute is ABANDON_MEMBER, or set the memberType attribute of the <group> resource to "MIXED" if the consistencyStrategy attribute is SET_MIXED.
The memberTypeValidated attribute shall be set to TRUE if all the members have been validated successfully.

Primitive specific operation: The hosting CSE shall check whether the number of provided memberIDs in the attribute members exceeds the limitation of maxNrOfMembers. The hosting CSE shall also check whether the value provided in maxNrOfMmembers is lesser than the currentNrOfMembers attribute value. If it exceeds, the hosting CSE shall reject the request with Response Status Code indicating "NOT_ALLOWED". error

##### 7.4.14.2.5	Delete

No primitive specific operations.

### 7.4.15	Resource Type <fanOutPoint>

#### 7.4.15.1	Introduction

The <fanOutPoint> resource is a virtual resource because it does not have a representation. It is the child resource of a <group> resource. Whenever the request is sent to the <fanOutPoint> resource, the request is fanned out to each of the members of the <group> resource indicated by the memberIDs attribute of the <group> resource. The responses (to the request) from each member are then aggregated and returned to the Originator. The detailed description can be found in clause 9.6.14 in ETSI TS 118 101 [6].

There are no common attributes, resource specific attributes or xsd file to <fanOutPoint> resource because it''s a virtual resource.

A <fanOutPoint> can be addressed in one of two ways:

- Using the URI retrieved from its parent <group> resource;  or

- Using a hierarchical URI formed by taking the hierarchical URI of the parent <group> and appending the string /fanOutPoint to that URI

This hierarchical URI can be extended by appending further path elements beyond the place where /fanOutPoint/ occurs. A request sent to such a URI is not fanned out to the group members, but instead it is fanned out to the resources located by taking the hierarchical URI of each group member in turn and then appending the additional path elements to that URI.

For example, if  /IN-CSE-0001/myGroup were a group with members

- /IN-CSE-0001/m1 and

- /IN-CSE-0001/m2

then a request sent to /IN-CSE-0001/myGroup/fanOutPoint/x/y would be fanned out to

- /IN-CSE-0001/m1/x/y and

- /IN-CSE-0001/m2/x/y

The additional path elements can reference virtual resources, for example if m1 and m2 were both <container> resources then a request sent to /IN-CSE-0001/myGroup/fanOutPoint/latest  would be fanned out to the most recent <contentInstance> child resource of both m1 and m2.

As a final example consider the case where the members m1 and m2 are themselves also <group> resources. In this case a request sent to  /IN-CSE-0001/myGroup/fanOutPoint/fanOutPoint will be fanned out to all the members of m1 and all members of m2.

#### 7.4.15.2	<fanOutPoint> operations

##### 7.4.15.2.1	Validate the type of  resource to be created

If this is a CREATE request and the memberType attribute of the addressed parent group resource is not "MIXED", the group hosting CSE may check whether the type of resource to be created is a valid and compatible child resource type of the group members.  If they are not consistent, the request shall be rejected with a Response Status Code indicating "MEMBER_TYPE_INCONSISTENT" error.  If the To parameter includes …/fanOutPoint without any additional appended relative address, then the type of resource specified by the memberType attribute of the parent group resource shall be checked to ensure that it is compatible with the type of child resource to be created.  If  the To parameter includes an additional appended relative address after the fanOutPoint element and the Hosting CSE is able to determine the corresponding resource type (e.g. relative address corresponds to a virtual resource having a fixed name and known type), then this type shall be checked to ensure that is compatible with the type of child resource to be created.   Otherwise if the hosting CSE is not able to determine the type of the resource targeted by the relative address it shall not perform the validation.

##### 7.4.15.2.2	Sub-group creation for members residing on the same CSE

The group hosting CSE shall obtain URIs of addressed resources from the attribute memberIDs of the parent <group> resource. The group hosting CSE may determine that multiple member resources belong to the same remote member hosting CSE, and may perform as an Originator to request to create a sub-group containing the specific multiple member resources in that member hosting CSE. This sub-group is created in the member hosting CSE as described in clause 7.4.14.2.2. The To parameter of this group Create request may be <memberHosting cseBase>/ <groupHosting remoteCse>/ or <memberHosting cseBase>/ etc. The group hosting CSE shall also provide From parameter (i.e. group hosting CSE) and sub-group resource representation that contains a memberIDs attribute with all the members residing on the addressed member Hosting CSE. The sub-group representation may include the attribute accessControlPolicyIDs, so that the group hosting CSE has the access right to this sub-group. The ID of the sub-group may be proposed by the group hosting CSE and determined by the member hosting CSE or it may be given by the member hosting CSE.
If there is already a sub-group resource defined in the remote member hosting CSE, then the group hosting CSE may utilize the existing sub-group resource.

##### 7.4.15.2.3	Assign URI for aggregation of notification

If the request is a request to create a <subscription> resource, the group hosting CSE shall validate the request to check whether it contains a notificationForwardingURI attribute or not. If it does not, the group hosting CSE shall forward it to the group memebers.     If it does,  the group hosting CSE shall assign a new URI to the notificationURI attribute of the <subscription> in the requests before forwarding it to the group members. This is so the group hosting CSE can receive and aggregate Notifications from those subscriptions.

##### 7.4.15.2.4	Fanout Request to each member

For each member hosting CSE, the group hosting CSE shall perform the following steps:

a)	The primitive parameters From and To shall be mapped to the primitive parameters of the corresponding Request to be sent out to each member of the group. The primitive parameter From shall be directly used. The prefix of primitive parameter To i.e. <URI of group resource>/fanOutPoint shall be replaced by hierarchical URIs derived from the attribute memberIDs of the group resource, but excluding the member resources which construct a sub-group. In addition, any additional relative address that was appended to .../fanOutPoint in the original Request shall be appended to each To URI.  For these member resources contained in a sub-group, the primitive To of the composed Request shall be <URI of sub-group resource>/fanOutPoint plus any additional appended relative address including in the original Request. The group hosting CSE shall execute "Compose Request primitives". In addition, the group hosting CSE shall generate a unique group request identifier, add it as a primitive parameter to the Request and locally store the group request identifier as per the local policy.

b)	"Send the Request to the receiver CSE".

c)	"Wait for Response primitives".

The procedures between group hosting CSE and member hosting CSEs shall comply with the corresponding creation procedures as described in clause 7. The detailed procedures are according to the type of Resource provided in the Request primitive. During fanOutPoint manipulation, the member hosting CSE receiving a Request send from the group hosting CSE shall check if the Request contains a Group Request Identifier parameter. If the Request contains a Group Request Identifier parameter, the member hosting CSE shall compare the Group Request Identifier parameter to the Group Request Identifier locally stored. If a match is found, the member hosting CSE shall reject the request with the Response Status Code indicating "GROUP_REQUEST_IDENTIFIER_EXISTS" error in the Response primitive. Otherwise, the member hosting CSE shall continue with the operations according to the Request and locally store the Group Request Identifier parameter.

#### 7.4.15.3	<fanOutPoint> resource specific procedure on CRUD operations

##### 7.4.15.3.1	Introduction

This clause describes <fanOutPoint> resource specific behaviour for CRUD operations.

##### 7.4.15.3.2	Create

A Create operation sent to a <fanOutPoint> is fanned out to the members (if any) of the parent <group>. It is equivalent to sending a Create to each member and therefore results in new resources being created as children of these existing members.

If the Create is sent to a hierarchical URI containing a fanOutPoint and an additional path relative to that fanOutPoint then the new resources are not created as immediate children of the members, rather they are created as children of descendents of those members (as determined by the relative path).

Originator:

Primitive specific operation after Orig-1.0 "Compose Request primitive" and before Orig-2.0 "Send the Request to the receiver CSE": In the case the Originator wants to subscribe to all the member resources of the group and the originator wants the group hosting CSE to aggregate all the notifications come from its member hosting CSEs, the Originator shall include notificationForwardingURI attribute in the <subscription> resource.

Receiver:

Primitive specific operation after Recv-6.2 "Check existence of the addressed resource" and before Recv-6.3 "Check authorization of the Originator".

Primitive specific operation additional to Recv-6.3 "Check authorization of the Originator": The Group Hosting CSE shall check the authorization of the Originator based on the membersAccessControlPolicyIDs of the parent <group> resource. In the case the membersAccessControlPolicyIDs is not provided, the accessControlPolicyIDs of the parent <group> resource shall be used.

Primitive specific operation to replace Recv-6.5"Create/Update/Retrieve/Delete/Notify operation is performed" and Recv-6.6"Announce/De-announce the resource" in the generic procedure:

- Validate the type of of resource to be created, refer to clause 7.4.15.2.1.

- Sub-group creation for members residing on the same CSE, refer to clause 7.4.15.2.2.

- Assign URI for aggregation of notification, refer to clause 7.4.15.2.3.

- Fanout Request to each member, refer to clause 7.4.15.2.4.

5)	The group hosting CSE shall aggregate the Responses after receiving responses from its member resources and sub-groups and aggregate the Responses:

Primitive specific operation additional to Recv-6.7 "Create a success response", the Response shall include the aggregated Responses.

##### 7.4.15.3.3	Retrieve

Originator:

No primitive specific operations.

Receiver:

Primitive specific operation after Recv-6.2 "Check existence of the addressed resource" and before Recv-6.3 "Check authorization of the Originator".

Primitive specific operation additional to Recv-6.3 "Check authorization of the Originator": The Group Hosting CSE shall check the authorization of the Originator based on the membersAccessControlPolicyIDs of the parent group resource. In the case the membersAccessControlPolicyIDs is not provided, the accessControlPolicyIDs of the parent group resource shall be used.

Primitive specific operation to replace Recv-6.5"Create/Update/Retrieve/Delete/Notify operation is performed" and Recv-6.6 "Announce/De-announce the resource" in the generic procedure:

- Sub-group creation for members residing on the same CSE, refer to clause 7.4.15.2.2.

- Fanout Request to each member, refer to clause 7.4.15.2.4.

3)	The group hosting CSE shall aggregate the Responses after receiving responses from its member resources and sub-groups and aggregate the Responses:

Primitive specific operation additional to Recv-6.7 "Create a success response", the Response shall include the aggregated Responses.

##### 7.4.15.3.4	Update

Originator:

No primitive specific operations.

Receiver:

Primitive specific operation after Recv-6.2 "Check existence of the addressed resource" and before Recv-6.3 "Check authorization of the Originator".

Primitive specific operation additional to Recv-6.3 "Check authorization of the Originator": The Group Hosting CSE shall check the authorization of the Originator based on the membersAccessControlPolicyIDs of the parent group resource. In the case the membersAccessControlPolicyIDs is not provided, the accessControlPolicyIDs of the parent group resource shall be used.

Primitive specific operation to replace Recv-6.5 "Create/Update/Retrieve/Delete/Notify operation is performed" and Recv-6.6"Announce/De-announce the resource" in the generic procedure:

- Sub-group creation for members residing on the same CSE , refer to clause 7.4.15.2.2.

- Fanout Request to each member. See Clause 7.4.15.2.4.

- The group hosting CSE shall aggregate the Responses after receiving responses from its <member> resources and sub-groups and aggregate the Responses:

Primitive specific operation additional to Recv-6.7 "Create a success response", the Response shall include the aggregated Responses.

##### 7.4.15.3.5	Delete

The primitive deletes the member resources and their child resources belonging to an existing <group> resource.

Originator:

No primitive specific operations.

Receiver:

Primitive specific operation after Recv-6.2 "Check existence of the addressed resource" and Recv-6.3 "Check authorization of the Originator": The To parameter consists of the URI of the group resource plus a suffix consisting of /fanOutPoint or /fanOutPoint/plus any additional appended relative address.

Primitive specific operation additional to Recv-6.3 "Check authorization of the Originator": The Group Hosting CSE shall check the authorization of the Originator based on the membersAccessControlPolicyIDs of the parent group resource. In the case the membersAccessControlPolicyIDs is not provided, the accessControlPolicyIDs of the parent group resource shall be used.

Primitive specific operation to replace Recv-6.5"Create/Update/Retrieve/Delete/Notify operation is performed" and Recv-6.6"Announce/De-announce the resource" in the generic procedure:

- Sub-group creation for members residing on the same CSE , refer to clause 7.4.15.2.2

- Fanout Request to each member. See clause 7.4.15.2.4

- The group hosting CSE shall aggregate the Responses after receiving responses from its <member> resources and sub-groups and aggregate the Responses:

Primitive specific operation additional to Recv-6.7 "Create a success response", the Response shall include the aggregated Responses.

### 7.4.16	Resource Type <mgmtObj>

#### 7.4.16.1	Introduction

The <mgmtObj> resource contains management data which represents individual M2M management functions. It represents a general structure to map to technology specific data models. There are multiple specializations of  <mgmtObj>; these are defined in the Annex D. Each of these specializations has its own schema file. There is no separate schema file just for <mgmtObj>, however the XML schema types for the specializations all conform to the pattern described in this clause.

Table 7.4.16.1-1: Universal/Common Attributes of <mgmtObj> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| Labels | O | O |

Table 7.4.16.1-2: Resource Specific Attributes of <mgmtObj> resource

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| mgmtDefinition | O | NP | m2m:mgmtDefinition | No default |
| objectIDs | O | NP | list of xs:anyURI | No default |
| objectPaths | O | NP | list of xs:anyURI | No default |
| description | O | O | xs:string | No default |
| mgmtLink | O | O | m2m:mgmtLinkRef | No default |

Table 7.4.16.1-3: Child resources of <mgmtObj> resource

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to Resource Type Definition |
| --- | --- | --- | --- |
| <subscription> | [variable] | 0..n | Clause 7.4.9 |

#### 7.4.16.2	<mgmtObj> resource specific procedure on CRUD operations

##### 7.4.16.2.0	Introduction

This clause describes <mgmtObj> resource specific procedure on resource Hosting CSE for CRUD operations.

The procedures are defined for management when technology specific protocols are used. When service layer management is performed, generic procedures defined in clause 7.2.2 shall comply for resource creation, update, retrieval and deletion. Procedures additional to resource manipulations to perform the management are further defined in Annex D.

##### 7.4.16.2.1	Create

Primitive specific operation before Orig-C-1.0 "Compose Request primitive":

- Primitive specific operation: If the originator is the managed entity, it shall generate the <mgmtObj> resource representation based on the technology specific data model object of the managed entity to be exposed. The objectIDs and objectPaths attributes may be set with the Request.

Primitive specific operation after Recv-6.5 "Create/Update/Retrieve/Delete/Notify operation is performed" and before Recv-6.6 "Announce/De-announce the resource" if the originator is an IN-AE:

"Identify the managed entity and the technology specific protocol".

Primitive specific operation: the receiver shall generate the technology specific data model object to be added to the managed entity based on the <mgmtObj> resource representation provided in the Request primitive. The receiver may determine the target location on the managed entity where the generated technology specific data model object shall be added based on the objectIDs and objectPaths provided in the request primitive and the technology specific data model being used. The receiver may also choose to let the managed entity decide the target location where the generated technology specific data model  object shall be added using technology specific mechanism.

"Establish a management session with the managed entity".

"Send the management request(s) to the managed entity corresponding to the received Request primitive ". If the receiver receives an error response from the managed entity because the technology specific data model  object to be added already exists on the managed entity, the receiver shall check (by using e.g. OMA-DM Get command or TR069 GetParameterValues/GetParameterAttributes command) if the existing technology specific data model object is the same as the one to be added, then it shall consider the requested primitive as successfully performed instead of sending an error response primitive; otherwise, it shall reject the request with the Response Status Code indicating "ALREADY_EXISTS" error in the Response primitive. The receiver shall also record the location where the technology specific data model  object is added to the managed entity in the successful case. The objectIDs and objectPaths attributes may be set with the Request.

The receiver may repeat Step 4 in order to add to the managed entity the technology specific data model  objects that are mapped from the mandatory sub-resources (including any descendants) that are required to be created automatically with the default attribute values.

##### 7.4.16.2.2	Retrieve

Primitive specific operation after Recv-6.5 "Create/Update/Retrieve/Delete/Notify operation is performed" and before Recv-6.6 "Announce/De-announce the resource" if the originator is an IN-AE:

- "Identify the managed entity and the technology specific protocol".

"Locate the technology specific data model objects to be managed on the managed entity".

"Establish a management session with the managed entity".

"Send the management request(s) to the managed entity corresponding to the received Request primitive". The receiver may also update the <mgmtObj> resource representation with the retrieved technology specific data model object if required according to the local policy.

##### 7.4.16.2.3	Update

The Update primitive is used for the update of the resource as well as the execution of a management procedure. The execution is performed using an Update primitive which without any content as the payload part of the primitive by addressing specific attribute to start the management procedure.

Primitive specific operation after Recv-6.5 "Create/Update/Retrieve/Delete/Notify operation is performed" and before Recv-6.6 "Announce/De-announce the resource" if the originator is IN-AE.

- "Identify the managed entity and the technology specific protocol".

- "Locate the technology specific data model objects to be managed on the managed entity".

- "Establish a management session with the managed entity".

- "Send the management request(s) to the managed entity corresponding to the received Request primitive". The receiver may also update the <mgmtObj> resource representation with the retrieved technology specific data model object if required according to the local policy.

##### 7.4.16.2.4	Delete

Primitive specific operation after Recv-6.5 "Create/Update/Retrieve/Delete/Notify operation is performed" and before Recv-6.6 "Announce/De-announce the resource" if the originator is IN-AE.

- "Identify the managed entity and the technology specific protocol".

"Locate the technology specific data model objects to be managed on the managed entity".

"Establish a management session with the managed entity".

"Send the management request(s) to the managed entity corresponding to the received Request primitive". The receiver may also update the <mgmtObj> resource representation with the retrieved technology specific data model object if required according to the local policy.

### 7.4.17	Resource Type <mgmtCmd>

#### 7.4.17.1	Introduction

The <mgmtCmd> resource shall contain the following attributes and child resource as illustrated in Table 7.4.17.1-2, Table 7.4.17.1-3, and Table 7.4.17.1-4. The data type and default value of these attributes and child resources are included in the tables.

Table 7.4.17.1-1: Data type definition of <mgmtCmd> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| mgmtCmd | CDT-mgmtCmd-v1_6_0.xsd |  |

Table 7.4.17.1-2: Universal/Common Attributes of <mgmtCmd> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| Labels | O | O |

Table 7.4.17.1-3: Resource Specific Attributes of <mgmtCmd> resource

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| description | O | O | xs:string | size: 256 No default |
| cmdType | M | O | m2m:cmdType | RESET, REBOOT, UPLOAD, DOWNLOAD, SOFTWAREINSTALL, SOFTWAREUPDATE, SOFTWAREUNINSTALL No default |
| execReqArgs | O | O | m2m:execReqArgsListType | A list of entries which are dependent on cmdType: If cmdType=RESET, execReqArgsList=resetArgsType. If cmdType=REBOOT, execReqArgsList=rebootArgsType. If cmdType=UPLOAD, execReqArgsList=uploadArgsType. If cmdType=DOWNLOAD, execReqArgsList=downloadArgsType. If cmdType=SOFTWAREINSTALL, execReqArgsList=softwareInstallArgsType. If cmdType=SOFTWAREUPDATE, execReqArgsList=softwareUpdateArgsType. If cmdType= SOFTWAREUNINSTALL, execReqArgsList=softwareUninstallArgsType. No default |
| execEnable | O | O | xs:boolean | No default |
| execTarget | M | O | m2m:nodeID | No default |
| execMode | M | O | m2m:execModeType | IMMEDIATEONCE, IMMEDIATEREPEAT, RANDOMONCE, RANDOMREPEAT Default=IMMEDIATEONCE |
| execFrequency | O | O | xs:duration | No default |
| execDelay | O | O | xs:duration | Default=0 |
| execNumber | O | O | xs:nonNegativeInteger | Default=1 |

Table 7.4.17.1-4: Child resources of <mgmtCmd> resource

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to Resource Type Definition |
| --- | --- | --- | --- |
| <subscription> | [variable] | 0..n | clause 7.4.9 |
| <execInstance> | [variable] | 0..n | clause 7.4.18 |

The <mgmtCmd> shall be executed for the following modes:

If execMode is IMMEDIATEONCE, <mgmtCmd> shall be executed immediately and only once. In this mode, execFrequency, execDelay, and execNumber shall not be used.

If execMode is IMMEDIATEREPEAT, <mgmtCmd> shall be executed immediately and repeated multiple times as determined by execNumber and the time interval between each execution is specified by execFrequency. In this mode, execDelay shall not be used.

If execMode is RANDOMONCE, <mgmtCmd> shall be executed only once at a delayed time which is specified by execDelay. In this mode, execFrequency and execNumber shall not be used.

If execMode is RANDOMREPEAT, <mgmtCmd> shall be executed multiple times as specified by execNumber but the first execution shall be executed at a delayed time. execDelay specifies the delayed time. The time interval between each execution is specified by execFrequency.

#### 7.4.17.2	<mgmtCmd> resource specific procedure on CRUD operations

##### 7.4.17.2.0	Introduction

This clause describes <mgmtCmd> resource specific procedures for CRUD operations.

##### 7.4.17.2.1	Create

This procedure shall use the Create common operations detailed in clause 7.2.2.1 without primitive specific actions. The Originator shall use the steps Orig-C-1.0, Orig-C-2.0, and Orig-C-3.0 as described in clause 7.2.2.1. The Receiver shall use the steps Rcv-C-1.0 to Rcv-C-11.0 as described in clause 7.2.2.1.

The Originator shall provide the <mgmtCmd> resource representation to the Receiver (e.g. IN-CSE). The Receiver may generate one of the following status codes and send it to the Originator.

If the Originator provides an invalid cmdType value in the Create primitive, the Receiver shall generate a Response Status Code indicating "INVALID_CMDTYPE" error.

If the name/value entry in execReqArgs does not match the value of cmdType in the Create primitive, the Receiver shall generate a Response Status Code indicating "INVALID_ARGUMENTS" error.

If the name/value entries in execReqArgs do not contain mandatory arguments as required by cmdType, the Receiver shall generate a Response Status Code indicating "INSUFFICIENT_ARGUMENTS" error.

##### 7.4.17.2.2	Retrieve

This procedure shall use the Retrieve common operations detailed in clause 7.3 without primitive specific actions. The Originator shall use the steps Orig-R-1.0, Orig-R-2.0, and Orig-R-3.0 as described in clause 7.3. The Receiver shall use the steps Rcv-R-1.0 to Rcv-R-9.0 as described in clause 7.3.

##### 7.4.17.2.3	Update

7.3.17.2.3.1	Update (Normal)

If the Update primitive does not address the execEnable attribute of the <mgmtCmd>, it results in update of all or part of the information of an existing <mgmtCmd> resource with the new attribute values. The procedure uses the common Update operations detailed in clause 7.3, without primitive specific actions.

The Originator shall use the steps Orig-U-1.0, Orig-U-2.0, and Orig-U-3.0 as described in clause 7.3. The Receiver shall use the steps Rcv-U-1.0 to Rcv-U-11.0 as described in clause 7.3.

If the Originator attempts to update attributes resourceType, resourceID or cmdType, the Receiver shall generate a Response Status Code indicating "NO_PRIVILEGE" error.

If the Originator attempts to update attributes execTarget, execMode, but the <mgmtCmd> has child resource <execInstance> already created, the Receiver shall generate a Response Status Code indicating "CONTENTS_UNACCEPTABLE" error.

If the Originator attempts to update attributes any attribute with a value which is not allowed, the Receiver shall generate a Response Status Code indicating "CONTENTS_UNACCEPTABLE" error.

If the Update primitive for <mgmtCmd> does address the execEnable attribute of the <mgmtCmd>, it effectively triggers an Execute <mgmtCmd> procedure, see clause 7.3.15.2.3.2.

7.3.17.2.3.2	Update (Execute)

The execute operation is triggered by an Update primitive, if the primitive addresses the execEnable attribute of the <mgmtCmd>. The procedure uses the Update common operations detailed in clause 7.3 with the following primitive specific operation after Rcv-U-4.0 and before Rcv-U-5.0:

- The Receiver shall identify the managed entity and the management protocol. The execTarget attribute of <mgmtCmd> indicates the managed entity.

The Receiver shall automatically create an <execInstance> based on the <mgmtCmd> resource. If the execTarget attribute addresses a <group> resource, the Receiver shall create multiple <execInstance> sub-resources based on the value of currentNrOfMembers attribute.

The Receiver shall copy the following attributes from <mgmtCmd> to each <execInstance> created:  execMode, execFrequency, execDelay, execNumber, and execReqArgs. The execStatus of <execInstance> is set as INITIATED. The Receiver shall set the execTarget attribute of each <execInstance> sub-resource to the URI of each target <node> resource.

The Receiver shall determine if the <mgmtCmd> shall be executed immediately or postponed according to the combination of execMode, execFrequency, execDelay, and execNumber. If the <mgmtCmd> shall be executed immediately (e.g. execMode is IMMEDIATEONCE), the following steps shall be performed; otherwise the following steps shall be postponed and skipped until the delay is expired (e.g. as indicated by execDelay).

The Receiver shall establish a management session with the identified managed entity.

The Receiver shall perform management command conversion and execution and set the execStatus attribute of <execInstance> to PENDING. If the Receiver cannot perform the command conversion successfully (e.g. execReqArgs does not have sufficient name/value pairs), the Receiver shall generate a Response Status Code indicating "MGMT_CONVERSION_ERROR" error.

After receiving completion response from the managed entity, the Receiver shall set execStatus attribute of corresponding <execInstance> to FINISHED.

If the Update primitive for <mgmtCmd> does not address the execEnable attribute of the <mgmtCmd>, it effectively triggers an Update <mgmtCmd> procedure, see clause 7.4.17.2.3.

##### 7.4.17.2.4	Delete

This procedure is based on the Delete common operations detailed in clause 7.3.

The Receiver shall determine:

If there are related management operations pending on the managed entity by checking if the execStatus attribute of all <execInstance> sub-resources are PENDING.

If the related management operations are cancellable by checking the cmdType attribute of <mgmtCmd>.

If there are no management commands pending on the remote entity the Receiver shall delete the addressed <mgmtCmd> resource and send a success response to the Originator.

If there are cancellable management commands still pending on any remote entity, the Receiver shall perform the following steps:

- The Receiver shall identify the managed entity and the management protocol. The execTarget attribute of each <execInstance> sub-resource which has execStatus of PENDING indicates the managed entity.

- The Receiver shall establish a management session with each managed entity.

- The Receiver shall perform management command conversion and execution resulting in cancellation of the commands which are pending on the managed entity.

- For each successful cancellation RPC the execStatus attribute of the corresponding <execInstance> is set to CANCELLED. For each un-successful cancellation RPCs the execStatus attribute of the corresponding <execInstance> is determined from the reported fault codes for the unsuccessful RPCs.

- Upon completion of all the cancellation operations, if any fault codes are returned by the managed entity, an error response to the Delete primitive with a Response Status Code indicating "CANCELLATION_FAILED" error is returned, and the <mgmtCmd> resource is not deleted. If all cancellation operations are successful on the managed entity, a success response to the Delete primitive is returned and the <mgmtCmd> resource is deleted.

If there are non-cancellable management commands still pending on the remote entity, the Receiver shall send an error response to the Delete request to the Originator, with a Response Status Code indicating "MGMT_COMMAND_NOT_CANCELLABLE" error. The execStatus attribute of the specific <execInstance> sub-resource is changed to STATUS_NON_CANCELLABLE.

### 7.4.18	Resource Type <execInstance>

#### 7.4.18.1	Introduction

The <execInstance> resource shall contain the following child resource and attributes.

Table 7.4.18.1-1: Data type definition of <execInstance> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| execInstance | CDT-execInstance-v1_6_0.xsd |  |

Table 7.4.18.1-2: Universal/Common Attributes of <execInstance> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| labels | O | O |

Table 7.4.18.1-3: Resource Specific Attributes of <execInstance> resource

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| execStatus | NP | O | m2m:execStatusType | INITIATED, PENDING, FINISHED, CANCELLING, CANCELLED STATUS_NON_CANCELLABLEL Default=INITIATED |
| execResult | NP | O | xs:execResultType | No default |
| execDisable | NP | O | xs:boolean | No default |
| execTarget | O | O | m2m:nodeID | No default |
| execMode | O | O | m2m:execModeType | IMMEDIATEONCE, IMMEDIATEREPEAT, RANDOMONCE, RANDOMREPEAT Default=IMMEDIATEONCE |
| execFrequency | O | O | xs:duration | No default |
| execDelay | O | O | xs:duration | Default=0 |
| execNumber | O | O | xs:nonNegativeInteger | Default=1 |
| execReqArgs | O | O | m2m:execReqArgsListType | No default |

Table 7.4.18.1-4: Child Resources of <execInstance> resource

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to Resource Type Definition |
| --- | --- | --- | --- |
| <subscription> | [variable] | 0..n | Clause 7.4.9 |

#### 7.4.18.2	<execInstance> resource specific procedure on CRUD oerations

##### 7.4.18.2.0	Introduction

This clause describes <execInstance> resource specific procedures for CRUD operations.

##### 7.4.18.2.1	Update (Cancel)

The <execInstance> Cancel operation is triggered by an Update primitive, if the primitive addresses the execDisable attribute. The procedure is based on Update common operations detailed in clause 7.3.

The Receiver shall determine:

- If there are related management operations pending on the managed entity by checking the execStatus attribute of the addressed <execInstance> sub-resource is PENDING.

- If the related management operations are cancellable by checking the cmdType attribute of the parent <mgmtCmd> resource.

If there are no management commands still pending on the remote entity, an error response to the Update primitive with a Response Status Code indicating "ALREADY_COMPLETE" error is returned to the Originator.

If there are cancellable management commands still pending on the remote entity, the Receiver shall perform the following steps:

- The Receiver shall identify the managed entity and the management protocol. The execTarget attribute of the addressed <execInstance> indicates the managed entity.

- The Receiver shall establish a management session with the managed entity.

- The Receiver shall perform management command conversion and execution resulting in cancellation of the commands which are pending on the managed entity.

- If the cancellation is successfully executed on the managed entity, the Receiver shall return a success response to the Originator and shall set execStatus of <execInstance> to CANCELLED.

- If the cancellation is unsuccessful on the managed entity, the Receiver shall return an error response to the Originator with a Response Status Code indicating "CANCELLATION_FAILED" error. The execStatus attribute is determined from the fault codes reported by the managed entity.

If there are non-cancellable management commands still pending on the remote entity, the Receiver shall return an error response to the Originator with a Response Status Code indicating "NOT_CANCELLABLE_COMMAND" error, and the execStatus attribute is changed to STATUS_NON_CANCELLABLE.

##### 7.4.18.2.2	Retrieve

This procedure shall use the Retrieve common operations detailed in clause 7.3, without primitive specific actions. The Originator shall use the steps Orig-R-1.0, Orig-R-2.0, and Orig-R-3.0 as described in clause 7.2.2.1. The Receiver shall use the steps Rcv-R-1.0 to Rcv-R-9.0 as described in clause 7.3.

##### 7.4.18.2.3	Delete

This procedure is based on the Delete common operations detailed in clause 7.3.

The Receiver shall determine:

- If there are related management operations pending on the managed entity by checking the execStatus attribute of the addressed <execInstance> sub-resource is PENDING.

- If the related management operations are cancellable by checking the cmdType attribute of the parent <mgmtCmd> resource.

If there are no management commands still pending on the remote entity, the Receiver shall delete the addressed resource and send a success response to the Originator.

If there are cancellable management commands still pending on the remote entity, the Receiver shall perform the following steps:

- The Receiver shall identify the managed entity and the management protocol. The execTarget attribute of the addressed <execInstance> indicates the managed entity.

- The Receiver shall establish a management session with the managed entity.

- The Receiver shall perform management command conversion and execution resulting in cancellation of the commands which are pending on the managed entity.

- If the cancellation is successfully executed on the managed entity, the Receiver shall return a success response to the Delete request to the Originator and shall delete the <execInstance> resource.

- If the cancellation is unsuccessful on the managed entity, the Receiver shall return an error response to the Delete request to the Originator with a Response Status Code indicating "CANCELLATION_FAILED" error. The execStatus attribute is determined from the fault codes reported by the managed entity.

If there are non-cancellable management commands still pending on the remote entity, the Receiver shall return an error response to the Delete request to the Originator with a Response Status Code indicating "NOT_CANCELLABLE_COMMAND". The execStatus attribute is set to STATUS_NOT_CANCELLABLE.

### 7.4.19	Resource Type <node>

#### 7.4.19.1	Introduction

The <node> resource represents specific information that provides properties of an oneM2M Node that can be utilized by other oneM2M operations. The <node> resource has <mgmtObj> as its child resources.

Table 7.4.19.1-1: Data type definition of <node> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| node | CDT-node- v1_6_0.xsd |  |

Table 7.4.19.1-2: Universal/Common Attributes of <node> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| labels | O | O |

Table 7.4.19.1-3: Resource Specific Attributes of <node> resource

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| nodeID | M | O | m2m:nodeID |  |
| hostedCSELink | O | NP | m2m:ID |  |

Table 7.4.19.1-4: Child resources of <node> resource

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to Resource Type Definition |
| --- | --- | --- | --- |
| <mgmtObj> | [variable] | 0..n | 7.4.16, See Annex D |
| <subscription> | [variable] | 0..n | 7.4.9 |

#### 7.4.19.2	<node> resource specific procedure on CRUD operations

##### 7.4.19.2.1	Create

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

##### 7.4.19.2.2	Retrieve

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

##### 7.4.19.2.3	Update

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2

##### 7.4.19.2.4	Delete

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

### 7.4.20	Resource Type <m2mServiceSubscriptionProfile>

#### 7.4.20.1	Introduction

The <m2mServiceSubscriptionProfile> resource represents an M2M Service Subscription Profile. It is used to represent all data pertaining to the M2M Service Subscription Profile, i.e., the technical part of the contract between an M2M Application Service Provider and an M2M Service Provider.

The detailed description can be found in clause 9.6.19 in ETSI TS 118 101 [6].

Table 7.4.20.1-1: Data type definition of <m2mServiceSubscriptionProfile> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| m2mServiceSubscriptionProfile | CDT-m2mServiceSubscriptionProfile-v1_6_0.xsd |  |

Table 7.4.20.1-2: Universal/Common Attributes of <m2mServiceSubscriptionProfile>

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| expirationTime | O | O |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| labels | O | O |
| lastModifiedTime | NP | NP |

Table 7.4.20.1-3: Resource Specific Attributes of <m2mServiceSubscriptionProfile>

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| serviceRoles | O | O | m2m:serviceRoles |  |

Table 7.4.20.1-4: Child resources of <m2mServiceSubscriptionProfile>

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to Resource Type Definition |
| --- | --- | --- | --- |
| <subscription> | [variable] | 0..n | Clause 7.4.9 |
| <serviceSubscribedNode > | [variable] | 0..n | Clause 7.4.21 |

#### 7.4.20.2	<m2mServiceSubscriptionProfile> resource specific procedure on CRUD operations

##### 7.4.20.2.0	Introduction

This clause describes <m2mServiceSubscriptionProfile> resource specific behaviour for CRUD operations.

##### 7.4.20.2.1	Create

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

##### 7.4.20.2.2	Retrieve

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

##### 7.4.20.2.3	Update

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

##### 7.4.20.2.4	Delete

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

### 7.4.21	Resource Type <serviceSubscribedNode>

#### 7.4.21.1	Introduction

The <serviceSubscribedNode> resource represents M2M Node information that is needed as part of the M2M Service Subscription resource. It shall contain information about the M2M Node as well as application identifiers of the Applications running on that Node.

The detailed description can be found in clause 9.6.20 in ETSI TS 118 101 [6].

Table 7.4.21.1-1: Data type definition of <serviceSubscribedNode> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| serviceSubscribedNode | CDT-serviceSubscribedNode-v1_6_0.xsd |  |

Table 7.4.21.1-2: Universal/Common Attributes of <serviceSubscribedNode> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| expirationTime | O | O |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| labels | O | O |
| lastModifiedTime | NP | NP |

Table 7.4.21.1-3: Resource Specific Attributes of <serviceSubscribedNode> resource

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| nodeID | M | NP | m2m:nodeID |  |
| CSE-ID | O | NP | m2m:ID |  |
| deviceIdentifier | O | NP | list of m2m:deviceID |  |
| ruleLinks | O | O | list of xs:anyURI |  |

Table 7.4.21.1-4: Child resources of <serviceSubscribedNode> resource

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to Resource Type Definition |
| --- | --- | --- | --- |
| <subscription> | [variable] | 0..n | 7.4.9 |

#### 7.4.21.2	<serviceSubscribedNode> resource specific procedure on CRUD operations

##### 7.4.21.2.0	Introduction

This clause describes <serviceSubscribedNode> resource specific behaviour for CRUD operations.

##### 7.4.21.2.1	Create

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

##### 7.4.21.2.2	Retrieve

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

##### 7.4.21.2.3	Update

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

##### 7.4.21.2.4	Delete

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

### 7.4.22	Resource Type <pollingChannel>

#### 7.4.22.1	Introduction

The <pollingChannel> resource is used to perform service layer long polling when an AE/CSE cannot receive a request from other entities, however it can get a request as a response to a long polling request. Actual long polling can be performed on the <pollingChannelURI> resource which is the child resource of the <pollingChannel> resource.

The detailed description can be found in clause 9.6.21 in ETSI TS 118 101 [6].

Table 7.4.22.1-1: Data type definition of <pollingChannel> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| pollingChannel | CDT-pollingChannel-v1_6_0.xsd |  |

Table 7.4.22.1-2: Universal/Common Attributes of <pollingChannel> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| creationTime | NP | NP |
| lastModifiedTime | NP | NP |
| expirationTime | O | O |
| accessControlPolicyIDs | O | O |
| labels | O | O |

Table 7.4.22.1-3: Child resources of <pollingChannel> resource

| Child Resource Type | Name | Multiplicity | Ref. to Resource Type Definition |
| --- | --- | --- | --- |
| <pollingChannelURI> | pollingChannelURI | 1 | Clause 7.4.23 |

#### 7.4.22.2	<pollingChannel> resource specific procedure on CRUD operations

##### 7.4.22.2.0	Introduction

This clause describes <pollingChannel> resource specific behaviour for CRUD operations.

##### 7.4.22.2.1	Create

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

Same as the generic procedures in clause 7.2.2.2 except one addition:

After Recv-6.3 procedure, the Hosting CSE shall check if the Originator ID is the same as the AE-ID or CSE-ID of the target <AE> resource or <remoteCSE> resource, respectively. If the check is failed, then the Hosting CSE shall return response primitive with a Response Status Code indicating "NO_PRIVILEGE" error.

##### 7.4.22.2.2	Retrieve

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

##### 7.4.22.2.3	Update

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

##### 7.4.22.2.4	Delete

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

### 7.4.23	Resource Type <pollingChannelURI>

#### 7.4.23.1	Introduction

The <pollingChannelURI> resource is the virtual child resource which is automatically generated during the parent <pollingChannel> resource creation. The detailed description can be found in clause 9.6.22 in ETSI TS 118 101 [6].

There is no data type definition for <pollingChannelURI> resource because it is' a virtual resource type.

#### 7.4.23.2	<pollingChannelURI> resource specific procedure on CRUD operations

##### 7.4.23.2.0	Introduction

This clause describes <pollingChannelURI> resource specific behaviour for the Retrieve operation as a service layer long polling request. CUDN requests to the <pollingChannelURI> resource shall be rejected.

##### 7.4.23.2.1	Create

The present document does not define Create operation on a <pollingChannelURI> resource. A Create request for the resource shall be rejected.

A <pollingChannelURI> virtual resource shall only be created during its parent <pollingChannel> resource creation procedure.

##### 7.4.23.2.2	Retrieve

Originator: shall execute Originator actions in clause 7.2.2.1 as a service layer long polling request. It is' the Originator's responsibility to initiate this procedure after it gets long polling response either successful or unsuccessful. The Originator shall send this Retrieve request as blocking request (clause 8.2.1 in ETSI TS 118 101 [6]).

Receiver: shall execute the following steps in order and these are modifications to the generic procedure from Recv-6.3 to Recv-6.5 in clause 7.2.2.2:

Recv-6.3 Check if the request Originator is the creator of the parent <pollingChannel> resource. If it is not the creator, the Hosting CSE shall send response primitive with a Response Status Code indicating "ACCESS_DENIED" error.

Recv-6.4	No change from the generic procedure.

Recv-6.5

If there is a pending request(s) to be sent to the Originator

Create a Response primitive by setting the Content  parameter with pending request(s).

Else

Wait for a request for the Originator until the Request Expiration Timestamp of the Originator's request. If a request is available before the Request Expiration Timestamp timeout, create a Response primitive including Pending Requests primitive parameter. Otherwise, create a response primitive with a Response Status Code indicating "REQUEST_TIMEOUT" error.

##### 7.4.23.2.3	Update

The present document does not define Update operation on a <pollingChannelURI> resource. An Update request for the resource shall be rejected.

##### 7.4.23.2.4	Delete

The present document does not define Delete operation on a <pollingChannelURI> resource. A Delete request for the resource shall be rejected.

### 7.4.24	Resource Type <statsConfig>

#### 7.4.24.1	Introduction

The <statsConfig> resource is used to store configuration data for collecting statistics for AEs. The <eventConfig> child resource is a mechanism for defining events that trigger statistics collection activity. Additional description of the <statsConfig> resource is contained in clauses 9.6.22 and 10.2.15 of ETSI TS 118 101 [6].

Table  7.4.24.1-1: Data type definition of <statsConfig>

| Data Type ID | File Name | Note |
| --- | --- | --- |
| statsConfig | CDT-statsConfig-v1_6_0.xsd |  |

Table 7.4.24.1-2: Universal/Common Attributes of <stateConfig> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| labels | O | O |

Table 7.4.24.1-3: Resource Specific Attributes of <stateConfig> resource

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| creator | O | NP | m2m:ID |  |

Table  7.4.24.1-4: Child resources of <statsConfig> resource

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to Resource Type Definition |
| --- | --- | --- | --- |
| <eventConfig> | [variable] | 0..n | 7.4.25 |
| <subscription> | [variable] | 0..n | 7.4.9 |

#### 7.4.24.2	<statsConfig> resource-specific procedure on CRUD operations

##### 7.4.24.2.1	Create

Originator:

No change from the generic procedure in clause 7.2.2.1

Receiver:

This procedure follows the Generic Request Procedure for Receiver specified in clause 7.2.2.1 with the following <statsConfig> resource-specific updates.

Resource-specific operation before Recv-6.2:

- If the To primitive parameter addresses a receiver CSE that is not an IN-CSE, then the request shall be rejected with a Response Status Code indicating "BAD_REQUEST" error.

##### 7.4.24.2.2	Retrieve

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

##### 7.4.24.2.3	Update

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

##### 7.4.24.2.4	Delete

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

### 7.4.25	Resource Type <eventConfig>

#### 7.4.25.1	Introduction

The <eventConfig> resource defines events that trigger statistics collection activity on an IN-CSE. Additional description of the <eventConfig> resource is contained in clauses 9.6.23 and 10.2.15 of ETSI TS 118 101 [6].

Table 7.4.25.1-1: Data type definition of <eventConfig>

| Data Type ID | File Name | Note |
| --- | --- | --- |
| eventConfig | CDT-eventConfig-v1_6_0.xsd |  |

Table 7.4.25.1-2: Universal/Common Attributes of <eventConfig> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| labels | O | O |

Table 7.4.25.1-3: Resource Specific Attributes of <eventConfig> resource

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| creator | O | NP | m2m:ID | No default |
| eventID | NP | NP | xs:string | Uniquely identifies a configurable event No default |
| eventType | M | O | m2m:eventType | DATAOPERATION STORAGEBASED TIMERBASED No default |
| eventStart | O | O | m2m:timestamp | No default (present only when eventType is set to TIMERBASED) |
| eventEnd | O | O | m2m:timestamp | No default (present only when eventType is set to TIMERBASED) |
| operationType | O | O | list of m2m:operation | CREATE RETRIEVE UPDATE DELETE NOTIFY No default (present only when eventType is set to DATAOPERATION) |
| dataSize | O | O | xs:nonNegativeInteger | No default (present only when eventType is set to STORAGEBASED) |

Table  7.4.25.1-4: Child Resources of <eventConfig> resource

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to Resource Type Definition |
| --- | --- | --- | --- |
| <subscription> | [variable] | 0..n | Clause 7.4.9 |

#### 7.4.25.2	<eventConfig> resource-specific procedure on CRUD operations

##### 7.4.25.2.1	Create

Originator:

This procedure follows the Generic Resource Request Procedure for Originator specified in clause 7.2.2.1, with the following <eventConfig> resource-specific updates.

Resource-specific operation before Orig-1.0 "Compose Request primitive":

- If event-based statistics collection will be used, the Originator shall generate the representation of the <eventConfig> child resource instance to produce the desired trigger condition for the intended event. For example, one representation of <eventConfig> could have eventType set to DATA OPERATION and operationType set to Retrieve. In another example, a representation could have eventType set to TIMER-BASED, eventStart set to midnight tomorrow and eventEnd set to midnight of the day after tomorrow. See Table 7.4.25.1-3 for value restrictions and default settings pertaining to the attributes of <eventConfig>.

Receiver:

No change from the generic procedure in clause 7.2.2.2.

##### 7.4.25.2.2	Retrieve

Originator:

No change from the generic procedure in clause 7.2.2.1.

Receiver:

No change from the generic procedure in clause 7.2.2.2.

##### 7.4.25.2.3	Update

Originator:

No change from the generic procedure in clause 7.2.2.1.

Receiver:

No change from the generic procedure in clause 7.2.2.2.

##### 7.4.25.2.4	Delete

Originator:

No change from the generic procedure in clause 7.2.2.1.

Receiver:

No change from the generic procedure in clause 7.2.2.2.

### 7.4.26	Resource Type <statsCollect>

#### 7.4.26.1	Introduction

The <statsCollect> resource controls the collection of statistics information on an IN-CSE. Information in an associated <eventConfig> resource shall be used by the IN-CSE or IN-AE to define specific event-related triggers. Additional description of the <statsCollect> resource is contained in clauses 9.6.24 and 10.2.15 of ETSI TS 118 101 [6].

Table 7.4.26.1-1: Data type definition of <statsCollect>

| Data Type ID | File Name | Note |
| --- | --- | --- |
| statsCollect | CDT-statsCollect-v1_6_0.xsd |  |

Table 7.4.26.1-2: Universal/Common Attributes of <statsCollect> resource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | O | O |
| lastModifiedTime | NP | NP |
| labels | O | O |

Table 7.4.26.1-3: Resource Specific Attributes of <statsCollect> resource

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| creator | O | NP | m2m:ID | No default |
| statsCollectID | NP | NP | xs:string | Unique ID (within SP domain) for each instance of collected statistics No default |
| collectingEntityID | M | NP | m2m:ID | Unique ID of entity (e.g., IN-AE, IN-CSE) requesting the collection of statistics No default |
| collectedEntityID | M | NP | m2m:ID | Unique ID of entity (e.g., AE, CSE) for which statistics will be collected No default |
| statsRuleStatus | M | O | m2m:statsRuleStatusType | ACTIVE INACTIVE No default |
| statModel | M | O | m2m:statModelType | EVENTBASED Default=EVENTBASED |
| collectPeriod | O | O | m2m:scheduleEntries | No default (see Table 7.4.10.1-3 for string format) |
| eventID | O | O | xs:string | Uniquely identifies a configurable event No default (present when statModel is set to EVENTBASED; corresponds to an eventID attribute in an <eventConfig> resource that defines a specific event for collection) |

Table 7.4.26.1-4: Child Resources of <statsCollect> resource

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to Resource Type Definition |
| --- | --- | --- | --- |
| <subscription> | [variable] | 0..n | Clause 7.4.9 |

#### 7.4.26.2	<statsCollect> resource-specific procedure on CRUD operations

##### 7.4.26.2.1	Create

Originator:

This procedure follows the Generic Resource Request Procedure for Originator specified in clause 7.2.2.1, with the following <statsCollect> resource-specific updates.

Resource-specific operation before Orig-1.0:

- The Originator shall generate and populate a representation of the <statsCollect> resource to produce the desired collection scenario, with the exception of statsCollectID (which is populated by the IN-CSE). If statModel is set to EVENT-BASED then the Originator shall provide a value for eventID that corresponds to an eventID value stored in an <eventConfig> resource (which defines the event triggers to be used). See Table 7.4.26.1-3 for value restrictions and default settings pertaining to the attributes of <statsCollect>.

Receiver:

This procedure follows the Generic Request Procedure for Receiver specified in clause 7.2.2.2, with the following <statsCollect> resource-specific updates.

Resource-specific operation before Recv-6.2:

- If the to primitive parameter addresses a receiver CSE that is not an IN-CSE, then the request shall be rejected with a Response Status Code indicating "BAD_REQUEST" error.

Resource-specific operation before Recv-6.6 and after Recv-6.5:

- The receiver IN-CSE shall generate and store a unique (within the Service Provider domain) value for statsCollectID.

If status is set to ACTIVE, the IN-CSE shall begin monitoring the conditions defined by the <statsCollect> resource and generating Service Statistics Collection Records as the conditions are met.

##### 7.4.26.2.2	Retrieve

Originator:

This procedure follows the Generic Resource Request Procedure for Originator specified in clause 7.2.2.1.

Receiver:

This procedure follows the Generic Request Procedure for Receiver specified in clause 7.2.2.2.

##### 7.4.26.2.3	Update

Originator:

This procedure follows the Generic Resource Request Procedure for Originator specified in clause 7.2.2.1.

Receiver:

This procedure follows the Generic Request Procedure for Receiver specified in clause 7.2.2.2, with the following <statsCollect> resource-specific updates.

Resource-specific operation before Recv-6.6 and after Recv-6.5:

- If status is set to ACTIVE, the IN-CSE shall begin monitoring the conditions defined by the <statsCollect> resource and generating Service Statistics Collection Records as the conditions are met.

- If status is set to INACTIVE, the IN-CSE shall stop monitoring the conditions defined by the <statsCollect> resource.

##### 7.4.26.2.4	Delete

Originator:

This procedure follows the Generic Resource Request Procedure for Originator specified in clause 7.2.2.1.

Receiver:

This procedure follows the Generic Request Procedure for Receiver specified in clause 7.2.2.1.

### 7.4.27	Announced resource type

#### 7.4.27.1	Introduction

A resource can be announced to one or more remote CSEs to inform the remote CSEs of the existence of the original resource. An announced resource can have a limited set of attributes and a limited set of child resources from the original resource.  The announced resource includes a link to the original resource hosted by the original resource-hosting CSE.

All announced resources have the same procedures regardless of the announced resource types.

Table 7.4.27.1-1: Data type definition of announced Resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| Actual Data Type ID | CDT-accessControlPolicy-v1_6_0.xsd CDT-remoteCSE-v1_6_0.xsd CDT-AE-v1_6_0.xsd CDT-container-v1_6_0.xsd CDT-contentInstance-v1_6_0.xsd CDT-schedule-v1_6_0.xsd CDT-locationPolicy-v1_6_0.xsd CDT-group-v1_6_0.xsd CDT-node-v1_6_0.xsd |  |

Table 7.4.27.1-2: Universal/Common Attributes of announcedResource

| Attribute Name | Request Optionality |  |
| --- | --- | --- |
|  | Create | Update |
| @resourceName | O | NP |
| resourceType | NP | NP |
| resourceID | NP | NP |
| parentID | NP | NP |
| accessControlPolicyIDs | O | O |
| creationTime | NP | NP |
| expirationTime | M | O |
| lastModifiedTime | NP | NP |
| labels | O | O |
| link | M | O |

Each announced resource type has the resource specific attributes that is the subset of the original resource.

Table 7.4.27.1-3: Resource Specific Attributes of announcedResource

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| Name of attribute specified as MA | M | O | The same data type defined at the original resource | This attribute shall be set to the same value with the attribute at the original resource |
| Name of attribute specified as OA | O | O | The same data type defined at the original resource | This attribute shall be set to the same value with the attribute at the original resource |

#### 7.4.27.2	Resource specific procedure on CRUD operations

##### 7.4.27.2.1	Introduction

This clause describes announced resource specific procedure for CRUD operations.

The original resource hosting CSE shall create/update/delete the announced resource as specified at the clause 7.3.3.10 and clause 7.2.2.2.

##### 7.4.27.2.2	Create

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

##### 7.4.27.2.3	Retrieve

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

In case of the Result Content information is set to the "original-resource", the Recv-6.5 shall be changed as follows:

Recv-6.5	"Read the original resource whose address is provided by the link attribute of the announced resource"

##### 7.4.27.2.4	Update

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2, except that Recv-6.3 is replaced as follows:

Recv-6.3 “Check if the value of the From paramether in Request message is identical to the CSE-ID included in the link attribute in the announced resource”

##### 7.4.27.2.5	Delete

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2, except that Recv-6.3 is replaced as follows:

Recv-6.3 “Check if the value of the From paramether in Request message is identical to the CSE-ID included in the link attribute in the announced resource”

### 7.4.28	Resource Type latest

#### 7.4.28.1	Introduction

The <latest> resource is a virtual resource because it does not have a representation. It is a child resource of the <container> resource. Whenever a request addresses the <latest> resource, the Hosting CSE shall apply the request to the latest <contentInstance> resource among all existing <contentInstance> resources of the <container> resource.

#### 7.4.28.2	<latest> Resource Specific Procedure on CRUD Operations

##### 7.4.28.2.1	Introduction

This sub-clause describes <latest> resource specific behaviour for operations. Among operations, only Retrieve and Delete operations shall be allowed for the <latest> resource.

##### 7.4.28.2.2	Create

Originator:

The <latest> resource shall not be created via API.

Receiver:

Primitive specific operation on Recv-1.0 "Check the syntax of received message":

- If the request is received, the Receiver CSE shall execute the following steps in order.

  - "Create an unsuccessful Response primitive" with the Response Status Code indicating "OPERATION_NOT_ALLOWED" error.

  - "Send the Response primitive".

##### 7.4.28.2.3	Retrieve

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.1 except the following modification:

Recv-6.2	Check the existence of the latest <contentInstance> resource among all existing <contentInstance> resources in the parent <container> resource. If the resource exists, the subsequent procedures of the Receiver (i.e., after Recv-6.2) shall be performed for the resource. If the resource does not exist, the Hosting CSE shall reject the request with a Response Status Code indicating "NOT_FOUND" error.

##### 7.4.28.2.4	Update

Originator:

The <latest> resource shall not be updated via API.

Receiver:

Primitive specific operation on Recv-1.0 "Check the syntax of received message":

- If the request is received, the Receiver CSE shall execute the following steps in order.

  - "Create an unsuccessful Response primitive" with the Response Status Code indicating "OPERATION_NOT_ALLOWED" error.

  - "Send the Response primitive".

##### 7.4.28.2.5	Delete

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.1 except the following modification:

Recv-6.2	Check the existence of the latest <contentInstance> resource among all existing <contentInstance> resources in the parent <container> resource. If the resource exists, the subsequent procedures of the Receiver (i.e., after Recv-6.2) shall be performed for the resource. If the resource does not exist, the Hosting CSE shall reject the request with a Response Status Code indicating "NOT_FOUND" error.

### 7.4.29	Resource Type oldest

#### 7.4.29.1	Introduction

The <oldest> resource is a virtual resource because it does not have a representation. It is a child resource of the <container> resource. Whenever a request addresses the <oldest> resource, the Hosting CSE shall apply the request to the oldest <contentInstance> resource among all existing <contentInstance> resources of the <container> resource.

#### 7.4.29.2	<oldest> Resource Specific Procedure on CRUD Operations

##### 7.4.29.2.1	Introduction

Among operations, only Retrieve and Delete operations shall be allowed for the <oldest> resource.

##### 7.4.29.2.2	Create

Originator:

The <oldest> resource shall not be created via API.

Receiver:

Primitive specific operation on Recv-1.0 "Check the syntax of received message":

- If the request is received, the Receiver CSE shall execute the following steps in order.

  - "Create an unsuccessful Response primitive" with the Response Status Code indicating "OPERATION_NOT_ALLOWED" error.

  - "Send the Response primitive".

##### 7.4.29.2.3	Retrieve

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.1 except the following modification:

Recv-6.2	Check the existence of the oldest <contentInstance> resource among all existing <contentInstance> resources in the parent <container> resource. If the resource exists, the subsequent procedures of the Receiver (i.e., after Recv-6.2) shall be performed for the resource.  If the resource does not exist, the Hosting CSE shall reject the request with a Response Status Code indicating "NOT_FOUND" error.

##### 7.4.29.2.4	Update

Originator:

The <oldest> resource shall not be updated via API.

Receiver:

Primitive specific operation on Recv-1.0 "Check the syntax of received message":

- If the request is received, the Receiver CSE shall execute the following steps in order.

  - "Create an unsuccessful Response primitive" with the Response Status Code indicating "OPERATION_NOT_ALLOWED" error.

  - "Send the Response primitive".

##### 7.4.29.2.5	Delete

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.1 except the following modification:

Recv-6.2	Check the existence of the oldest <contentInstance> resource among all existing <contentInstance> resources in the parent <container> resource. If the resource exists, the subsequent procedures of the Receiver (i.e., after Recv-6.2) shall be performed for the resource. If the resource does not exist, the Hosting CSE shall reject the request with a Response Status Code indicating "NOT_FOUND" error.

### 7.4.30	Resource Type <serviceSubscribedAppRule>

#### 7.4.30.1	Introduction

The <serviceSubscribedAppRule> resource represents a rule that defines allowed App-ID and AE-ID combinations that are acceptable for registering an AE on a Registrar CSE. The detailed description can be found in the clause 9.6.29 in ETSI TS 118 101 [6].

Table 7.4.30.1-1: Data type definition of <serviceSubscribedAppRule> resource

| Data Type ID | File Name | Note |
| --- | --- | --- |
| serviceSubscribedAppRule | CDT-serviceSubscribedAppRule-v1_6_0.xsd |  |

Table 7.4.30.1-2: Universal/Common Attributes of <serviceSubscribedAppRule> resource

| Attribute Name | Request Optionality |  | Resource Specific Note |
| --- | --- | --- | --- |
|  | Create | Update |  |
| resourceType | NP | NP |  |
| resourceID | NP | NP |  |
| parentID | NP | NP |  |
| expirationTime | O | O |  |
| accessControlPolicyIDs | O | O |  |
| creationTime | NP | NP |  |
| lastModifiedTime | NP | NP |  |
| Labels | O | O |  |

Table 7.4.30.1-3: Resource Specific Attributes of <serviceSubscribedAppRule> resource

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| applicableCredIDs | O | O | m2m:listOfM2MID |  |
| allowedApp-IDs | O | O | m2m:listOfM2MID |  |
| allowedAEs | O | O | m2m:listOfM2MID |  |

Table 7.4.30.1-4: Child resources of <serviceSubscribedAppRule> resource

| Child Resource Type | Child Resource Name | Multiplicity | Ref. to Resource Type Definition |
| --- | --- | --- | --- |
| <subscription> | [variable] | 0..n | Clause 7.4.9 |

#### 7.4.30.2	<serviceSubscribedAppRule> resource specific procedure on CRUD operations

##### 7.4.30.2.1	Introduction

This clause describes <serviceSubscribedAppRule> resource specific primitive behaviour for CRUD operations.

##### 7.4.30.2.2	Create

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

##### 7.4.30.2.3	Retrieve

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

##### 7.4.30.2.4	Update

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

##### 7.4.30.2.5	Delete

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

## 7.5	Primitive-specific procedures and definitions

### 7.5.1	Notification data object and procedures

#### 7.5.1.1	Notification data object

Notification procedures represent a special case of the generic procedures defined in clause 7.2.2, where the Operation parameter of the request primitive is set to value "N" (Notify). In this case, the request primitive is referred to as Notify request primitive, and the associated response primitive is denoted as Notify response primitive.

A Notify request primitive shall convey a special notification data object in its Content parameter. This notification data object has no resource type representation in the ETSI TS 118 101 [6], since it does not represent a resource accessible by any M2M entities. The data type of the notification data object is defined in the tables below. The first column of Figure 6.3.4.2.31-2 defines the permitted names the root element the notification data object can take with the data type listed in the third column.

Table 7.5.1.1-1: Data Type Definition of notification data object

| Data Type ID | File Name | Note |
| --- | --- | --- |
| notification | CDT-notification-v1_6_0.xsd |  |

Table 7.5.1.1-2: Data Types for notification data objects

| Root Element Name | Request Optionality | Data Type | Default Value and Constraints |
| --- | --- | --- | --- |
|  | N |  |  |
| notification | O | m2m:notification |  |
| aggregatedNotification | O | m2m:aggregatedNotification |  |
| responsePrimitive | O | m2m:responsePrimitive |  |

When an Originator sends a Notify primitive to an AE Receiver, it shall use one of the serializations specified in that AE’s contentSerialization attribute. If the AE has no contentSerialization attribute, the Originator is free to choose the serialization format itself.

#### 7.5.1.2	Notification procedures

##### 7.5.1.2.1	Introduction

Notification procedures shall be employed for the following use cases:

to notify Receiver(s) of modifications of a resource for an associated <subscription> resource,

to request Receiver(s) to perform resource subscription verification,

to notify deletion of the <subscription> resource,

to notify Receiver(s) for Asynchronous Non-blocking Request,

to notify Receiver(s) of modifications of a resource when the subscription relationship is established through the <group> resource.

The following clauses specify the notification procedures for each of the above use cases.

##### 7.5.1.2.2	Notification for modification of subscribed resources

When the notification message is forwarded or aggregated by transit CSEs, the Originator or a transit CSE shall check whether there are notification policies to enforce between subscription resource Hosting CSE and the notification target. In that case, the transit CSE as well as the Originator shall process Notify request primitive(s) by using the corresponding policy and send processed Notify request primitive(s) to the next CSE with notification policies related to the enforcement so that the transit CSE is able to enforce the policy defined by the subscriber. The notification policies related to the enforcement at this time is verified by using the subscription reference in the Notify request primitive. In the notification policies, the latestNotify attribute is only enforced in the transit CSE as well as the Originator.

If Event Category parameter is set to ''latest' in the notification request primitive, the transit CSE as well as Originator shall cache the most recent Notify request. That is, if a new Notify request is received by the CSE with a subscription reference that has already been buffered for a pending Notify request, the newer Notify request will replace the buffered older Notify request.

Originator: When an event is generated, the Originator shall execute the following steps in order:

Step 1.0	Check the eventNotificationCriteria attribute of the <subscription> resource associated with the modified resource:

If the eventNotificationCriteria attribute is set, then the Originator shall check whether the corresponding event matches with the event criteria. If notificationEventType is not set within the eventNotificationCriteria attribute, the Originator shall use the default setting of Update_of_Resource to compare against the event. If the event matches, go to the step 2.0. Otherwise, the Originator shall discard the corresponding event

If the eventNotificationCriteria attribute is not configured, the Originator shall use the default setting of Update_of_Resource for notificationEventType and then continue with the step 2.0

Step 2.0	The Originator shall check the notification policy as described in the below steps, but the notification policy may be checked in different order. After checking the notification policy in step 2.0 (i.e., from step 2.1to step 2.6), then continue with step 3.0

Step 2.1	The Originator shall determine the type of the notification per the notificationContentType attribute. The possible values of for notificationContentType attribute are 'Modified Attributes', 'All Attributes', and or optionally 'ResourceID'. This attribute may be used joint with eventType attribute in the eventNotificationCriteria to determine if it is the attributes of the subscribed-to resource or the attributes of the child resource of the subscribed-to resource that shall be returned in the notification.

If the value of notificationContentType is set to 'Modified Attribute', the Notify request primitive shall include the modified attribute(s) only

If the value of notificationContentType is set to 'All Attributes', the Notify request primitive shall include the whole subscribed-to resource

If the value of notificationContentType is set to 'ResourceID', the Notify request primitive shall include the resourceID of the subscribed-to resource

If the notificationContentType attribute is not configured, the default value is set to 'All Attributes'

Step 2.2	Check the notificationEventCat attribute:

If the notificationEventCat attribute is set, the Notify request primitive shall employ the Event Category parameter as given in the notificationEventCat attribute. Then continue with the next step

If the notificationEventCat attribute is not configured,then continue with other step

Step 2.3	Check the latestNotify attribute:

If the latestNotify attribute is set, the Originator shall assign Event Category parameter of value 'latest' of the notifications generated pertaining to the subscription created. Then continue with other step

- NOTE: The use of some attributes such as rateLimit, batchNotify and preSubscriptionNotify is not supported in this release of the document.

Step 3.0	The Originator shall check the notification and reachability schedules, but the notification schedules may be checked in different order.

If the <subscription> resource associated with the modified resource includes a <notificationSchedule> child resource, the Originator shall check the time periods given in the scheduleElement attribute of the <notificationSchedule> child resource.

Also, the Originator shall check the reachability schedule associated with the Receiver by exploring its <schedule> resource. If reachability schedules are not present in a Node then that Entity is considered to be always reachable

If notificationSchedule and reachability schedule indicate that message transmission is allowed, then proceed with step 5.0. Otherwise, proceed with step 4.0

In particular, if the notificationEventCat attribute is set to ''immediate'' and the <notificationSchedule> resource does not allow transmission, then go to step 5.0 and send the corresponding Notify request primitive by temporarily ignoring the Originator''s notification schedule

Step 4.0	Check the pendingNotification attribute:

If the pendingNotification attribute is set, then the Originator shall cache pending Notify request primitives according to the pendingNotification attribute. The possible values are ''sendLatest'' and ''sendAllPending''. If the value of pendingNotification is set to ''sendLatest'', the most recent Notify request primitive shall be cached by the Originator and it shall set the Event Category parameter to ''latest''. If pendingNotification  is set to ''sendAllPending'', all Notify request primitives shall be cached by the Originator. If the pendingNotification attribute is not configured, the Originator shall discard the corresponding Notify request primitive. The processed Notify request primitive by the pendingNotification attribute is sent to the Receiver after the reachability recovery (see the step 6.0)

Step 5.0	Check the expirationCounter attribute:

If the expirationCounter attribute is set, then it shall be decreased by one when the Originator successfully sends the Notify request primitive. If the counter equals to  zero('0'), the corresponding <subscription> resource shall be deleted. Then end the 'Compose Notify Request Primitive' procedure

If the expirationCounter attribute is not configured, then end the 'Compose Notify Request Primitive' procedure

Originator: After reachability recovery, the Originator shall execute the following steps in order:

Step 6.0	If the pendingNotification attribute is set, the Originator shall send the processed Notify request primitive by the pendingNotification attribute and, then continue with the step 7.0

Step 7.0	Check the expirationCounter attribute:

If the expirationCounter attribute is set, then its value shall be decreased by one when the Originator successfully sends the Notify request primitive. If the counter meets zero, the corresponding <subscription> resource shall be deleted. Then end the 'Compose Notify Request Primitive' procedure.

If the expirationCounter attribute is not configured, then end the 'Compose Notify Request Primitive' procedure

Receiver: When the Hosting CSE receives a Notify request primitive, the Hosting CSE shall check validity of the primitive parameters. In case the Receiver is a transit CSE which forwards or aggregates Notify request primitives before sending to the subscriber or other transit CSEs, upon receiving the Notify request primitive with the Event Category parameter set to 'latest', the Receiver shall identify the latest Notify request primitive with the same subscription reference while storing Notify request primitives locally. When the Receiver as a transit CSE needs to send pending Notify request primitives, it shall send the latest Notify request primitive.

##### 7.5.1.2.3	Subscription Verification during Subscription Creation

Originator:

When the Originator is triggered to perform subscription verification (clause 7.4.9.2.1) during <subscription> creation procedure, it performs the following steps in order.

- Set the verificationRequest element of the notification data object as TRUE in the Notify request primitive.

Set the creator element of the notification data object as the Originator ID of the <subscription> creation in the primitive.

Set the to parameter as notificationURI in the primitive. If the notificationURI contains more than one value, then set the other value to the duplicated primitives from step 2).

Send the Notify request primitive(s).

Receiver:

When the Hosting CSE receives a Notify request primitive which includes verificationRequest element of the notification data object set as TRUE, the Hosting CSE shall check if the creator and the Originator have NOTIFY privilege to the notificationURI.

If it fails, the Hosting CSE shall return a Response Status Code indicating "SUBSCRIPTION_CREATOR_HAS_NO_PRIVILEGE" or "SUBSCRIPTION_HOST_HAS_NO_PRIVILEGE" error, respectively, with the Notify response primitive. Otherwise, it shall return successful response primitive.

##### 7.5.1.2.4	Notification for Subscription Deletion

Originator:

When the <subscription> resource is deleted and subscriberURI of the <subscription> resource is configured, the Originator shall send a Notify request primitive with subscriptionDeletion element of the notification data object set as TRUE and subscriptionReference element set as the URI of the <subscription> resource to the entity indicated in subscriberURI.

##### 7.5.1.2.5	Notification for Asynchronous Non-blocking Request

Originator:

When the requested operation for a nonBlockingAsynch request is completed, the Originator (=hosting CSE of the resource) shall send a Notify request primitive to inform the final result of requested operation against the oneM2M resource. When the notificationURI was present and empty in the Response Type parameter in the previously received nonBlockingAsync request, no notification with the result of the requested operation shall be sent at all by the Originator. Otherwise, the Originator shall send a Notify request primitive as follows: (If the notificationURI was present and contains multiple entries, then the Originator shall repeat the following steps for each entry in the notificationURI list.)

- The Originator shall compose a Request primitive with following parameter settings:

  - The From parameter shall be set to the ID of the Originator (i.e. Hosting CSE which hosts the resource targeted by the previously received nonBlockingAsynch request).

  - If the notificationURI was not present in the Response Type parameter in the previously received nonBlockingAsync request, then the To parameter shall be set to the Originator of the previously received nonBlockingAsynch request.
If the notificationURI was present and not empty in the Response Type parameter in the previously received nonBlockingAsync request, then the To parameter shall be set to the next notificationURI list entry.

  - The Response Type : If the Originator chooses to send the Notification in nonBlockingAsynch mode, the Originator shall include a notificationURI in the Response Type and set it to empty .

  - The Content parameter shall be set to the response to the previously received nonBlockingAsynch request as m2m:responsePrimitive.

The Originator shall send the Request primitive. See clause 7.3.1.2 for detail.

Receiver:

No change from the generic procedure in clause 7.2.2.2.

##### 7.5.1.2.6	Notification for subscription via group

Whenever the subscribed to resources' modification triggers a notification procedure as definedin clause 7.5.1.2.2 and the subscription relationship is established through group resource, the following procedure shall be performed.

The Member hosting CSE shall perform the steps defined in 7.5.1.2.2.

The Group hosting CSE shall perform the following steps in order:

- Validate if the notification is sent from its own member resources when it gets a notification at the notificationURI. The group hosting CSE shall return a response primitive with the Response Status Code indicating "ACCESS_DENIED" error if the validation fails.

Upon successful validation, the group hosting CSE shall collect notification requests targeted at the same subscriber according to the notificationForwardingURI element of each notification data obeject. The group hosting CSE shall aggregate the notification requests into an aggregatedNotification element of the notification data object. The timing of aggregation is done as per the group hosting CSE''s local policy which is out of scope of the present document.

Send the aggregated notification to the notificationURI according to the notificationForwardingURI element in the notification data object. In case the group hosting CSE is member of another group hosting CSE through which the subscription is created, the notification request shall be sent according to the mapping of the notificationURI of the two group hosting CSEs. When aggregating the notification requests, the group hosting CSE may utilize the Request Expiration Timestamp parameter of the notification request primitive to determine the time by which the aggregated notifications need to be sent.

"Wait for Response primitive" procedure.

Upon receiving the response, the group hosting CSE shall send the response separately to each individual member hosting CSEs to respond their corresponding notify request.

The group hosting CSE may also stop aggregating notification requests depending on its own policy. The group hosting CSE shall not stop aggregating notification requests before the corresponding subscription expires.

The Subscriber shall perform the following steps in order:

- Extract each notification from the aggregated notification;

- Treat the notification as if it is sent from the original subscribed-to resource;

- "Create a success response" procedure;

- "Send the Response primitive" procedure.

### 7.5.2	Elements contained in the Content primitive parameter

Clauses 7.2.1.1 and 7.2.1.2 enumerate the forms that the Content primitive parameter takes in various Request and Response cases. Note that the Content primitive parameter is denoted as primitiveContent in both, CDT-requestPrimitive-v1_6_0.xsd and CDT-responsePrimitive-v1_6_0.xsd.

This clause details the Objects (elements) used in some of these cases in the tables below.

The following elements are defined for use in the Content parameter of a request:

Table 7.5.2-1: Elements used for request content

| Element Name | Applicable Operations | Data Type | Defined in |
| --- | --- | --- | --- |
| m2m:<resourceType> | C U | m2m:<resourceType> | CDT-<resourceType>-v1_6_0.xsd |
| m2m:notification | N | m2m:notification | CDT-notification-v1_6_0.xsd |
| m2m:aggregatedNotification | N | m2m:aggregatedNotification | CDT-notification-v1_6_0.xsd |
| m2m:attributeList | R | m2m:attributeList | CDT-requestPrimitive-v1_6_0.xsd |
| m2m:responsePrimitive | N | m2m:responsePrimitive | CDT-responsePrimitive-v1_6_0.xsd |

The following elements are defined for use in the Content parameter of a response sent in reply to a request message with Operation and Result Content (rcn) parameters as given in the column "Applicable Operations" (the settings of the Result Content parameters are defined in clause 6.3.3.2.7; NP means the rcn parameter is not present)::

Table 7.5.2-2: Elements used for response content

| Element Name | Applicable Operations/rcn | Data Type | Element is Defined in |
| --- | --- | --- | --- |
| m2m:<resourceType> | C/1, 7,NP R/1,4,5,6,7,NP U/1,NP D/1,NP See NOTE 1 | m2m:<resourceType> | CDT-<resourceType>-v1_6_0.xsd |
| m2m:resource | C/3 | m2m:resourceWrapper | CDT-responsePrimitive-v1_6_0.xsd |
| m2m:URIList | R/NP See NOTE 2 | m2m:listOfURIs | CDT-responsePrimitive-v1_6_0.xsd |
| m2m:aggregatedResponse | C R U D See NOTE 3 | m2m:aggregatedResponse | CDT-responsePrimitive-v1_6_0.xsd |
| m2m:URI | C/2 See NOTE 4 | xs:anyURI | CDT-responsePrimitive-v1_6_0.xsd |
| Raw data | See NOTE 5 |  |  |
| NOTE 1: The case rcn = 7 applies to Retrieve operation only (R/7). It retrieves the original resource in case the To parameter points to an announced resource. NOTE 2: This applies to ordinary retrieve operation when the Result Content parameter is not present and for discovery operation. For discovery, the format of the URIList elements (structured, unstructured) depends on the Discovery Result Type parameter setting (see clause 6.3.3.2.8). NOTE 3: This applies to CRUD operations on a <fanOut> child resource of a <group> parent resource. The Content parameter of each response primitive included in aggregatedResponse is set as given in one of the other rows of this table. NOTE 4: This also applies to the response ("acknowledgement") to non-blocking requests in asynchronous mode for any CRUD operation NOTE 5: Raw data could e.g. be included as debugging information into error responses |  |  |  |

# 8	Representation of primitives in data transfer

## 8.1	Introduction

This clause defines the representation of request and response primitives as XML documents or JSON texts. The process of translating objects (i.e. primitives in the present context) into a format that can be stored or exchanged between network entities is commonly denoted as serialization or marshalling.

The serialization described here is used in two places:

- It can be used when transmitting primitives over communication protocols such as HTTP, CoAP or MQTT. When applying a particular protocol binding, it is permitted to adapt the serialization approach, in order to make use of protocol-specific features. For example, a particular protocol binding may require that one or more primitive parameters  be mapped to protocol-specific header fields rather than being included in the protocol-specific serialized JSON or XML which represents the message body.

Certain instances of resource types, e.g. instances of the <delivery> resource, include serialized primitives embedded in one of their resource attributes.

In order to enable efficient communication, the short names introduced in clause 8.2 shall be applied in XML and JSON serializations to identify primitive parameters and resource attribute names. This implies that short names are applied in any communication over the Mca, Mcc and Mcc' reference points.

## 8.2	Short names

### 8.2.1	Introduction

XML and JSON representations require the explicit encoding of the names of primitive parameters, resource attributes, (in the case of XML) resource types and complex data types members. Whenever a protocol binding transfers such a name over a oneM2M reference point, it shall use a shortened form of that name, rather than the full name that is used elsewhere in this and other oneM2M specifications. Short names enable payload reduction on involved telecommunication interfaces.

The mapping between the full names and their shortened form is given in the clauses that follow.

### 8.2.2	Primitive parameters

In protocol bindings primitive parameter names shall be translated into short names of Table 8.2.2-1.

Table 8.2.2-1: Primitive parameter short names

| Parameter Name | XSD long name | Occurs in | Short Name |
| --- | --- | --- | --- |
| Operation | operation | Request | op |
| To | to | Request, Response | to |
| From | from | Request, Response | fr |
| Request Identifier | requestIdentifier | Request, Response | rqi |
| Resource Type | resourceType | Request | ty |
| Content | primitiveContent | Request, Response | pc |
| Role | role | Request | rol |
| Originating Timestamp | originatingTimestamp | Request, Response | ot |
| Request Expiration Timestamp | requestExpirationTimestamp | Request | rqet |
| Result Expiration Timestamp | resultExpirationTimestamp | Request, Response | rset |
| Operation Execution Time | operationExecutionTime | Request | oet |
| Response Type | responseType | Request | rt |
| Result Persistence | resultPersistence | Request | rp |
| Result Content | resultContent | Request | rcn |
| Event Category | eventCategory | Request, Response | ec |
| Delivery Aggregation | deliveryAggregation | Request | da |
| Group Request Identifier | groupRequestIdentifier | Request | gid |
| Filter Criteria | filterCriteria | Request | fc |
| Discovery Result Type | discoveryResultType | Request | drt |
| Response Status Code | responseStatusCode | Response | rsc |

XML serialized representations of primitives employ root element names to differentiate between request and response primitive types (see clause 8.3). These root element names shall be translated into short names as in Table 8.2.2-2.

Table 8.2.2-2: Primitive root element short names

| Root Element Name | Occurs in | Short Name |
| --- | --- | --- |
| requestPrimitive | Request | rqp |
| responsePrimitive | Response | rsp |

### 8.2.3	Resource attributes

In protocol bindings resource attributes names shall be translated into short names of Table 8.3-1.

Table 8.2.3-1: Resource attribute short names (1/5)

| Attribute Name | Occurs in | Short Name |
| --- | --- | --- |
| accessControlPolicyIDs | All except accessControlPolicy, contentInstance | acpi |
| announcedAttribute | accessControlPolicy, AE, container, contentInstance, group, locationPolicy, mgmtObj, node, remoteCSE, schedule | aa |
| announceTo | accessControlPolicy, AE, container, contentInstance, group, locationPolicy, mgmtObj, node, remoteCSE, schedule | at |
| creationTime | All | ct |
| expirationTime | All except contentInstance, CSEBase | et |
| labels | All (optional) | lbl |
| lastModifiedTime | All | lt |
| link | All | lnk |
| parentID | All | pi |
| resourceID | All | ri |
| resourceType | All | ty* |
| stateTag | container, contentInstance, delivery, request | st |
| resourceName | All | rn |
| privileges | accessControlPolicy | pv |
| selfPrivileges | accessControlPolicy | pvs |
| App-ID | AE | api |
| AE-ID | AE | aei |
| appName | AE | apn |
| pointOfAccess | AE, CSEBase, remoteCSE | poa |
| ontologyRef | AE, container, contentInstance | or |
| nodeLink | AE, CSEBase, remoteCSE | nl |
| contentSerialization | AE | csz |
| creator | container, contentInstance,eventConfig, group, pollingChannel, statsCollect, statsConfig, subscription | cr |
| maxNrOfInstances | container | mni |
| maxByteSize | container | mbs |
| maxInstanceAge | container | mia |
| currentNrOfInstances | container | cni |

Table 8.2.3-2: Resource attribute short names (2/5)

| Attribute Name | Occurs in | Short Name |
| --- | --- | --- |
| currentByteSize | container | cbs |
| locationID | container | li |
| contentInfo | contentInstance | cnf |
| contentSize | contentInstance | cs |
| primitiveContent | request | pc* |
| content | contentInstance | con |
| cseType | CSEBase, remoteCSE | cst |
| CSE-ID | CSEBase, remoteCSE, service SubscribedNode | csi |
| supportedResourceType | CSEBase | srt |
| notificationCongestionPolicy | CSEBase | ncp |
| source | delivery | sr |
| target | delivery, request | tg |
| lifespan | delivery | ls |
| eventCat | delivery | eca* |
| deliveryMetaData | delivery | dmd |
| aggregatedRequest | delivery | arq |
| eventID | eventConfig, statsCollect | evi |
| eventType | eventConfig | evt |
| eventStart | eventConfig | evs |
| eventEnd | eventConfig | eve |
| operationType | eventConfig | opt |
| dataSize | eventConfig | ds |
| execStatus | execInstance | exs |
| execResult | execInstance | exr |
| execDisable | execInstance | exd |
| execTarget | execInstance, mgmtCmd | ext |
| execMode | execInstance, mgmtCmd | exm |
| execFrequency | execInstance, mgmtCmd | exf |
| execDelay | execInstance, mgmtCmd | exy |
| execNumber | execInstance, mgmtCmd | exn |
| execReqArgs | execInstance, mgmtCmd | exra |
| execEnable | mgmtCmd | exe |
| memberType | group | mt |
| currentNrOfMembers | group | cnm |
| maxNrOfMembers | group | mnm |
| memberIDs | group | mid |
| membersAccessControlPolicyIDs | group | macp |
| memberTypeValidated | group | mtv |
| consistencyStrategy | group | csy |
| groupName | group, subscription | gn |
| locationSource | locationPolicy | los |
| locationUpdatePeriod | locationPolicy | lou |
| locationTargetID | locationPolicy | lot |
| locationServer | locationPolicy | lor |
| locationContainerID | locationPolicy | loi |
| locationContainerName | locationPolicy | lon |
| locationStatus | locationPolicy | lost |
| serviceRoles | m2mServiceSubscriptionProfile | svr |
| description | mgmtCmd, mgmtObj, all management resources from firmware | dc |
| cmdType | mgmtCmd | cmt |
| mgmtDefinition | mgmtObj, all management resources from firmware | mgd |
| objectIDs | mgmtObj | obis |

Table 8.2.3-3: Resource attribute short names (3/5)

| Attribute Name | Occurs in | Short Name |
| --- | --- | --- |
| objectPaths | mgmtObj | obps |
| nodeID | node | ni |
| hostedCSELink | node | hcl |
| CSEBase | remoteCSE | cb* |
| M2M-Ext-ID | remoteCSE | mei |
| Trigger-Recipient-ID | remoteCSE | tri |
| requestReachability | remoteCSE | rr |
| originator | request | og |
| metaInformation | request | mi |
| requestStatus | request | rs |
| operationResult | request | ol |
| operation | request | op* |
| requestID | request | rid |
| scheduleElement | schedule | se |
| deviceIdentifier | serviceSubscribedNode | di |
| ruleLinks | serviceSubscribedNode | rlk |
| statsCollectID | statsCollect | sci |
| collectingEntityID | statsCollect | cei |
| collectedEntityID | statsCollect | cdi |
| devStatus | areaNwkDeviceInfo | ss |
| statsRuleStatus | statsCollect | srs |
| statModel | statsCollect | sm |
| collectPeriod | statsCollect | cp |
| eventNotificationCriteria | subscription | enc |
| expirationCounter | subscription | exc |
| notificationURI | subscription | nu |
| groupID | subscription | gpi |
| notificationForwardingURI | subscription | nfu |
| batchNotify | subscription | bn |
| rateLimit | subscription | rl |
| preSubscriptionNotify | subscription | psn |
| pendingNotification | subscription | pn |
| notificationStoragePriority | subscription | nsp |
| latestNotify | subscription | ln |
| notificationContentType | subscription | nct |
| notificationEventCat | subscription | nec |
| subscriberURI | subscription | su |
| version | firmware, software | vr |
| URL | firmware, software | url |
| update | firmware | ud |
| updateStatus | firmware | uds |
| install | software | in |
| uninstall | software | un |
| installStatus | software | ins |
| activate | software | act |
| deactivate | software | dea |
| activeStatus | software, areaNwkInfo | acts |
| memAvailable | memory | mma |
| memTotal | memory | mmt |

Table 8.2.3-4: Resource attribute short names (4/5)

| Attribute Name | Occurs in | Short Name |
| --- | --- | --- |
| areaNwkType | areaNwkInfo | ant |
| listOfDevices | areaNwkInfo | ldv |
| devID | areaNwkDeviceInfo | dvd |
| devType | areaNwkDeviceInfo | dvt |
| areaNwkId | areaNwkDeviceInfo | awi |
| sleepInterval | areaNwkDeviceInfo | sli |
| sleepDuration | areaNwkDeviceInfo | sld |
| listOfNeighbors | areaNwkDeviceInfo | lnh |
| batteryLevel | battery | btl |
| batteryStatus | battery | bts |
| deviceLabel | deviceInfo | dlb |
| manufacturer | deviceInfo | man |
| Model | deviceInfo | mod |
| deviceType | deviceInfo | dty |
| fwVersion | deviceInfo | fwv |
| swVersion | deviceInfo | swv |
| hwVersion | deviceInfo | hwv |
| capabilityName | deviceCapability | can |
| Attached | deviceCapability | att |
| capabilityActionStatus | deviceCapability | cas |
| Enable | deviceCapability | ena |
| Disable | deviceCapability | dis |
| currentState | deviceCapability | cus |
| Reboot | reboot | rbo |
| factoryReset | reboot | far |
| logTypeId | eventLog | lgt |
| logData | eventLog | lgd |
| logStatus | eventLog | lgst |
| logStart | eventLog | lga |
| logStop | eventLog | lgo |
| firmwareName | firmware | fwnnam |
| softwareName | software | swn |
| cmdhPolicyName | cmdhPolicy | cpn |
| mgmtLink | cmdhPolicy, activeCmdhPolicy, cmdhDefaults, cmdhNetworkAccessRules, cmdhNwAccessRule | cmlk |
| activeCmdhPolicyLink | activeCmdhPolicy | acmlk |
| Order | cmdhDefEcValue, cmdhLimits | od |
| defEcValue | cmdhDefEcValue | dev |
| requestOrigin | cmdhDefEcValue, cmdhLimits | ror |
| requestContext | cmdhDefEcValue, cmdhLimits | rct |
| requestContextNotification | cmdhDefEcValue, cmdhLimits | rctn |
| requestCharacteristics | cmdhDefEcValue, cmdhLimits | rch |
| applicableEventCategories | cmdhNetworkAccessRules | aecs |
| applicableEventCategory | cmdhEcDefParamValues, cmdhBuffer | aec |
| defaultRequestExpTime | cmdhEcDefParamValues | dqet |
| defaultResultExpTime | cmdhEcDefParamValues | dset |
| defaultOpExecTime | cmdhEcDefParamValues | doet |
| defaultRespPersistence | cmdhEcDefParamValues | drp |
| defaultDelAggregation | cmdhEcDefParamValues | dda |
| limitsEventCategory | cmdhLimits | lec |
| limitsRequestExpTime | cmdhLimits | lqet |
| limitsResultExpTime | cmdhLimits | lset |
| limitsOpExecTime | cmdhLimits | loet |
| limitsRespPersistence | cmdhLimits | lrp |
| limitsDelAggregation | cmdhLimits | lda |
| targetNetwork | cmdhNwAccessRule | ttn |

Table 8.2.3-5: Resource attribute short names (5/5)

| Attribute Name | Occurs in | Short Name |
| --- | --- | --- |
| minReqVolume | cmdhNwAccessRule | mrv |
| backOffParameters | cmdhNwAccessRule | bop |
| otherConditions | cmdhNwAccessRule | ohc |
| maxBufferSize | cmdhBuffer | mbfs |
| storagePriority | cmdhBuffer | sgp |
| applicableCredIDs | serviceSubscribedAppRule | apci |
| allowedApp-IDs | serviceSubscribedAppRule | aai |
| allowedAEs | serviceSubscribedAppRule | aae |
| NOTE: marked short names have been already assigned in Table 8.2.2-1. |  |  |

### 8.2.4	Resource types

In protocol bindings resource type names shall be translated into short names of Table 8.2.4-1.

Table 8.2.4-1: Resource and specialization type short names

| Resource Type Name | Short Name |
| --- | --- |
| accessControlPolicy | acp |
| accessControlPolicyAnnc | acpA |
| AE | ae |
| AEAnnc | aeA |
| container | cnt |
| containerAnnc | cntA |
| latest | la |
| oldest | ol |
| contentInstance | cin |
| contentInstanceAnnc | cinA |
| CSEBase | cb |
| delivery | dlv |
| eventConfig | evcg |
| execInstance | exin |
| fanOutPoint | fopt |
| group | grp |
| groupAnnc | grpA |
| locationPolicy | lcp |
| locationPolicyAnnc | lcpA |
| m2mServiceSubscriptionProfile | mssp |
| mgmtCmd | mgc |
| mgmtObj | mgo |
| mgmtObjAnnc | mgoA |
| node | nod |
| nodeAnnc | nodA |
| pollingChannel | pch |
| pollingChannelURI | pcu |
| remoteCSE | csr |
| remoteCSEAnnc | csrA |
| request | req |
| schedule | sch |
| scheduleAnnc | schA |
| serviceSubscribedAppRule | asar |
| serviceSubscribedNode | svsn |
| statsCollect | stcl |
| statsConfig | stcg |
| subscription | sub |
| firmware | fwr |
| firmwareAnnc | fwrA |
| software | swr |
| softwareAnnc | swrA |
| memory | mem |
| memoryAnnc | memA |
| areaNwkInfo | ani |
| areaNwkInfoAnnc | aniA |
| areaNwkDeviceInfo | andi |
| areaNwkDeviceInfoAnnc | andiA |
| battery | bat |
| batteryAnnc | batA |
| deviceInfo | dvi |
| deviceInfoAnnc | dviA |
| deviceCapability | dvc |
| deviceCapabilityAnnc | dvcA |
| reboot | rbo * |
| rebootAnnc | rboA |
| eventLog | evl |
| eventLogAnnc | evlA |
| cmdhPolicy | cmp |
| activeCmdhPolicy | acmp |
| cmdhDefaults | cmdf |
| cmdhDefEcValue | cmdv |
| cmdhEcDefParamValues | cmpv |
| cmdhLimits | cml |
| cmdhNetworkAccessRules | cmnr |
| cmdhNwAccessRule | cmwr |
| cmdhBuffer | cmbf |

NOTE: * marked short names have been already assigned in attribute Tables 8.2.3-1 to 8.2.3-5.

### 8.2.5	Complex data types members

In protocol bindings complex data types member names shall be translated into short names of Table 8.2.5-1.

Table 8.2.5-1: Complex data type member short names

| Member Name | Occurs in | Short Name |
| --- | --- | --- |
| createdBefore | filterCriteria, eventNotificationCriteria | crb |
| createdAfter | filterCriteria, eventNotificationCriteria | cra |
| modifiedSince | filterCriteria, eventNotificationCriteria | ms |
| unmodifiedSince | filterCriteria, eventNotificationCriteria | us |
| stateTagSmaller | filterCriteria, eventNotificationCriteria | sts |
| stateTagBigger | filterCriteria, eventNotificationCriteria | stb |
| expireBefore | filterCriteria, eventNotificationCriteria | exb |
| expireAfter | filterCriteria, eventNotificationCriteria | exa |
| labels | filterCriteria, eventNotificationCriteria | lbl * |
| resourceType | filterCriteria | ty * |
| sizeAbove | filterCriteria, eventNotificationCriteria | sza |
| sizeBelow | filterCriteria, eventNotificationCriteriay | szb |
| contentType | filterCriteria | cty |
| limit | filterCriteria | lim |
| attribute | filterCriteria, eventNotificationCriteria | atr |
| notificationEventType | eventNotificationCriteria | net |
| operationMonitor | eventNotificationCriteria, notificationEvent | om |
| representation | notificationEvent | rep |
| filterUsage | filterCriteria | fu |
| eventCatType | eventCat | ect |
| eventCatNo | eventCat | ecn |
| number | batchNotify | num |
| duration | batchNotify | dur |
| notification | aggregatedNotification, Request Primitive Content | sgn |
| notificationEvent | notification | nev |
| verificationRequest | notification | vrq |
| subscriptionDeletion | notification | sud |
| subscriptionReference | notification | sur |
| creator | notification | cr* |
| notificationForwardingURI | notification | nfu* |
| operation | operationMonitor | op* |
| originator | operationMonitor | or* |
| accessId | externalID | aci |
| MSISDN | externalID | msd |
| action | actionStatus | acn |
| status | actionStatus | sus |
| childResource | All except execInstance, announced resource, management resources from firmware | ch |
| accessControlRule | privileges, selfPrivileges | acr |
| accessControlOriginators | accessControlRule | acor |
| accessControlOperations | accessControlRule | acop |
| accessControlContexts | accessControlRule | acco |
| accessControlWindow | accessControlContexts | actw |
| accessControlIpAddresses | accessControlContexts | acip |
| ipv4Addresses | accessControlIpAddress | ipv4 |
| ipv6Addresses | accessControlIpAddress | ipv6 |
| accessControlLocationRegion | accessControlContexts | aclr |
| countryCode | accessControlLocationRegion | accc |
| circRegion | accessControlLocationRegion | accr |
| name | attribute, anyArgType, mgmtLinkRef, childResourceRef | nm* |
| value | attribute | val |
| type | anyArgType | typ |
| maxNrOfNotify | rateLimit | mnn |
| timeWindow | rateLimit | tww |
| scheduleEntry | scheduleElement | sce |
| aggregatedNotification | Request Primitive Content | agn |
| attributeList | Request Primitive Content | atrl |
| aggregatedResponse | Response Primitive Content | agr |
| resource | Response Primitive Content | rce |
| URIList | Response Primitive Content | uril |
| anyArg | resetArgsType, rebootArgsType, uploadArgsType, downloadArgsType, softwareInstallArgsType softwareUpdateArgsType, softwareUninstallArgsType, execReqArgsListType | any |
| fileType | downloadArgsType | ftyp |
| URI | resourceWrapper | uri |
| URL | downloadArgsType | url* |
| username | uploadArgsType, downloadArgsType, softwareUpdateArgsType, softwareUninstallArgsType, | unm |
| password | uploadArgsType, downloadArgsType, softwareUpdateArgsType, softwareUninstallArgsType, | pwd |
| filesize | downloadArgsType | fsi |
| targetFile | downloadArgsType | tgf |
| delaySeconds | downloadArgsType | dss |
| successURL | downloadArgsType | surl |
| startTime | downloadArgsType | stt |
| completeTime | downloadArgsType | cpt |
| UUID | softwareInstallArgsType softwareUpdateArgsType, softwareUninstallArgsType, | uuid |
| executionEnvRef | softwareInstallArgsType softawareUpdateArgsType, softwareUninstallArgsType, | eer |
| version | softwareUninstallArgsType | vr* |
| reset | execReqArgsListType | rst |
| reboot | execReqArgsListType | rbo* |
| upload | execReqArgsListType | uld |
| download | execReqArgsListType | dld |
| softwareInstall | execReqArgsListType | swin |
| softwareUpdate | execReqArgsListType | swup |
| softwareUninstall | execReqArgsListType | swun |
| tracingOption | dekiveryMetaData | tcop |
| tracingInfo | dekiveryMetaData | tcin |
| responseTypeValue | responseTypeInfo | rtv |
| notificationURI | responseTypeInfo | nu* |

NOTE: * marked short names have been already assigned in attribute Table 8.2.3-1.

## 8.3	XML serialization

### 8.3.1	Method

XML serialization of request or response primitives refers to the process of representing the primitive as an XML document.

The XML document shall be a well-formed XML document compliant with W3C XML 1.0 [1]. It shall be restricted to Unicode characters and encoded using UTF-8 as described in RFC 3629 [21].

The structure and data types of XML serialized request and response primitives shall be consistent with the XSD defined in CDT-requestPrimitive-v1_6_0.xsd and CDT-responsePrimitive-v1_6_0.xsd, respectively. The data types used in these XSD files comply with the definitions in clause 6 and clause 7 of the present document.

Note that the XSD files included in the present release employ the long names for primitive parameters and other XML elements and attributes, but the primitive serialization is required to use the corresponding short names (as defined clause 8.2 of the present document).

NOTE: XML Schema files that use short names might be made available at a future date.

The primitive Content parameter is serialized just like any other element of complex type. Generally, the Content parameter may include only a partial set of attributes specified for the resource type as indicated in the Resource Type parameter, e.g. for partial Update or Retrieve Request procedures. For Notification Request primitives, the Content parameter includes a Notification data object as defined in clause 7.5.1.1 and the datatype definition given in CDT-notification-v1_6_0.xsd.

### 8.3.2	Examples

An example that shows a request primitive serialized into an XML document is shown below. This example shows the create request for an instance of a <contentInstance> resource. Only mandatory primitive parameters and resource attributes are shown.

<?xml version="1.0" encoding="UTF-8"?>
<m2m:rqp xmlns:m2m="http://www.onem2m.org/xml/protocols"
 >
    <op>1</op>
    <to>//cse1.mym2msp.org/</to>
    <fr>/cse1234/app567</fr>
    <rqi>0002bf63</rqi>
    <ty>4</ty>
    <pc>
        <m2m:cin>
            <cnf>application/xml:1</cnf>
            <con>PHRpbWU+MTc4ODkzMDk8L3RpbWU+PHRlbXA+MjA8L3RlbXA+DQo=</con>
        </m2m:cin>
    </pc>
</m2m:rqp>

The XML elements have the following meaning:

- rqp:  Root element of the Request primitive, which includes a reference to an XSD file which defines its datatype.

- op: 	Operation parameter of datatype m2m:operation:  in this example value = 1 indicates a "Create" operation.

- to: 	To parameter of type m2m:anyURI: URI of the target resource.

- fr: 	From parameter of type m2m:ID: ID of the Originator (either AE-ID or CSE-ID).

- rqi: 	Request Identifier parameter of type m2m:requestID: this could e.g. represent a counter number.

- ty: 	Resource Type parameter of datatype m2m:resourceType: indicating type of the resource to be created (value = 4 indicates that a <contentInstance> resource shall be created).

- pc: 	Content parameter of datatype m2m:primitiveContent: the attributes of the resource to be provided by the Originator.

- cin:  Root element of the <contentInstance> resource of datatype m2m:contentInstance: this includes the mandatory attributes  (and optional attributes not shown in this example) supplied by the request Originator. In this example, the cn parameter includes an instance of a <contentInstance> resource which consists of two attributes: contentInfo (cnf) – which specifies base64 encoding - and the content (con) itself.

## 8.4	JSON serialization

### 8.4.1	Terminology

The following conventions are used in the clause that follows.

- The italicized terms object, member, name, array, number, string, boolean and null are to be interpreted as in RFC 7159 [19]

- The italicized term element is to be interpreted to encompass oneM2M Primitive Parameters, Resource Attributes and other elements or attributes used inside oneM2M complex type definitions

### 8.4.2	Method

The primitive shall be encoded as a JSON object, conforming to the requirements of RFC 7159 [19]. This JSON object shall be restricted to Unicode characters defined in The Unicode Standard and encoded using UTF-8 as described in IETF RFC 3629 [21].  The names in each object in the JSON shall be unique.

The structure of the top-level primitive object shall be determined by the data type definitions in clause 6 and clause 7 of the present document, as follows:

- All member''s names shall be the short name defined in clause 8.2.

- If an element is defined in the present document as having a complex type, then it is serialized in the JSON member as an object and its children are recursively serialized as members of that object, using short names as defined in clause 8.2.

- The membership of each nested object shall respect the cardinality constraints from the corresponding XSD complex type definition,

- If an element is defined in the present document as having an atomic data type that is numeric in nature (e.g. xs:integer or a type derived from it) then its value is serialized into the JSON member as a number.

- If an element is defined as having an atomic data type that is non-numeric then its value is serialized into the JSON member as a string.

- If an element is defined as xs:boolean (or a type derived from xs:boolean) then it is serialized in the JSON member as a boolean.

- If an element is defined as having an xs:list type in the corresponding XSD then it is serialized in the JSON member as an array.

- If an element instance has a null value then it is serialized into the JSON member as a null, regardless of the data type that it has in the corresponding XSD.

- If an element is defined as having maxOccurs > 1 in the corresponding XSD then its parent JSON member is serialized as an array.

- If an element has an XSD data type that is a simple type with XML attributes, then it is serialized in the JSON member as an object. The XML attributes appear as members of that object (using their short names) and the value of the element is serialized as a member of that object with the special name "val".

- The members (at each level) may be serialized in any order. The order in which they appear in the corresponding XSD file is immaterial.

The Content parameter is treated just like any other parameter of complex type. It is serialized as an object and its members are the attributes and/or child resource references of the Resource that is being transferred. The Content parameter is not required to contain all the attributes of the Resource. The JSON representation of the Content parameter shall be encapsulated by a member name as defined in the first column of  Table 7.5.2-1 and Table 7.5.2-2.

### 8.4.3	Examples

Here is an example that shows the payload of a request message serialized using JSON:

{"op": "1", "fr": "//xxxxx/2345", "to": "//xxxxx/99", "rqi": "A1234", "pc": {"m2m:sch": {"se": "* 0-5 2,6,10 * * * *"}}, "ty": 18}

- op: operation (in this case it is Create)

- fr: ID of the Originator (either the AE or CSE)

- to: URI of the target resource

- rqi: request identifier (this is a string)

- pc: attributes of the <schedule> resource with member name "m2m:sch" to be provided by Originator. This is serialized as a nested JSON object

- ty: type of resource to be created (in this case a Schedule resource). This is a number.

Note that the Operation (op) parameter is present only in Request primitives. The presence of this parameter in JSON serialized primitive representations allows to differentiate Request primitives from Response primitives.

Annex A (informative): 
Binding Mch to Diameter for Charging

# A.1	Introduction

Present clause provides Diameter binding of Mch for information.

# A.2	Diameter Commands on Mch

## A.2.1	Accounting Request Command

The ACR command is sent from the Charging Function (CHF included within the SCA CSF) embedded within the M2M IN to the Charging Server using the Mch reference point. This command issused for Event Based requests.

The ACR message format is defined according to the Diameter Base Protocol in IETF RFC 3588 [13]  as follows:

<ACR> ::= < Diameter Header: 271, REQ, PXY >

< Session-Id >

{ Origin-Host }

{ Origin-Realm }

{ Destination-Realm }

{ Accounting-Record-Type }

{ Accounting-Record-Number }

[ Acct-Application-Id ]

[ Destination-Host ]

[ Origin-State-Id ]

[ Event-Timestamp ]

* [ Proxy-Info ]

* [ Route-Record ]

[ Service-Context-Id ]

[ Service-Information ]

* [ AVP ]

## A.2.2	Accounting Answer Command

The ACR command is sent from the Charging Server to the Charging Function (CHF included within the SCA CSF) embedded within the M2M IN in response to the ACR command and is used to acknowledge reception of the charging data. This command is used for Event Based responses.

The ACA message format is defined according to the Diameter Base Protocol in RFC 3588 [13] as follows:

<ACA> ::= < Diameter Header: 271, PXY >

< Session-Id >

{ Result-Code }

{ Origin-Host }

{ Origin-Realm }

{ Accounting-Record-Type }

{ Accounting-Record-Number }

[ Acct-Application-Id ]

[ User-Name ]

[ Origin-State-Id ]

[ Event-Timestamp ]

* [ Proxy-Info ]

* [ AVP ]

# A.3	Mapping of M2M Recorded Information Elements to AVPs

The following table describes the mapping of the M2M Recorded Information Elements identified in ETSI TS 118 101 to the Diameter AVPs.

Table A.3-1: Mapping of M2M Recorded Information Elements to Diameter AVPs

| M2M Recorded Information Elements | Diameter AVP |
| --- | --- |
| M2M Service Subscription Identifier | Subscription-Id |
| Application Entity ID | Application-Entity-ID |
| External ID | External-ID |
| Receiver | Receiver |
| Originator | Originator |
| Hosting CSE-ID | Hosting-CSE-ID |
| Target ID | Target-ID |
| Protocol Type | Protocol-Type |
| Request Operation | Request-Operation |
| Request Headers size | Request-Headers-Size |
| Request Body size | Request-Body-Size |
| Response Headers size | Response-Headers-Size |
| Response Body size | Response-Body-Size |
| Response Status Code | Response-Status-Code |
| Time Stamp | M2M-Event-Record-Timestamp |
| M2M-Event-Record-Tag | Rating-Group |
| Control Memory Size | Control-Memory-Size |
| Data Memory Size | Data-Memory-Size |
| Access Network Identifier | Access-Network-Identifier |
| Additional Information | AVP |
| Occupancy | Occupancy |
| Group Name | Group-Name |
| maxNrOfMembers | Maximum-Number-Members |
| currentNrOfMembers | Current-Number-Members |
| Subgroup Name | Subgroup-Name |
| M2M-Node-Id | Node-Id |
| M2M Service Subscription Identifier | Subscription-Id |
| Application Entity ID | Application-Entity-ID |

# A.4	Summary of AVPs used

The following table lists the Diameter AVPs specifically used for the offline charging interface.

In Table A.3-1, columns "Used in ACR" and "Used in ACA" identify at a protocol level if the AVP is mandatory, optional, or not allowed. When identified as optional here, an AVP may be considered mandatory for certain conditions as identified in Table 12.1.2.2-1 of ETSI TS 118 101 [6].

AVPs defined for oneM2M specific usage are assigned Vendor-Id of 45687. The formats and usage of oneM2M specific AVPs are defined in the present document in clause A.5.

The table contains the following information:

AVP Name: The name used in Diameter.

AVP Vendor ID: The entity defining the AVP code in the next column.

AVP Code: The AVP Code used in the Diameter AVP Header.

Used in ACR: Indicates if it is mandatory, optional or not used in the ACR command.

Used in ACA: Indicates if it is mandatory, optional or not used in the ACA command.

Used in CCR: Not in this release.

Used in CCA: Not in this release.

AVP Defined: A reference to where this AVP is defined.

Value Type: The Diameter format of the AVP data as defined in Basic or Derived AVP Data Format.

AVP Flag Rules: The rules for how the AVP Flags in the AVP Header may be set.

May Encrypt: Indicates if the AVP may be encrypted or not.

Table A.4-1: Use Of Diameter AVPs

| AVP Name | AVP Vendor Id | AVP Code | Used in |  |  |  | Reference | Value Type | AVP Flag rules |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ACR | ACA | CCR | CCA |  |  | Must | May | Should not | Must not | May Encr. |
| Access-Network-Identifier | 45687 | 1000 | O | - | - | - | [A.5.1. ] | Unsigned32 | M | P | - | V | Y |
| Acct-Application-Id | 0 | 259 | O | O | - | - | [5.5.1] | Unsigned32 | M | P | - | V | N |
| Accounting-Record-Number | 0 | 485 | M | M | - | - | IETF RFC 3588 [13] | Unsigned32 | M | P | - | V | Y |
| Accounting-Record-Type | 0 | 480 | M | M | - | - | [5.4.2] | Enumerated | M | P | - | V | Y |
| Application-Entity-ID | 45687 | 1001 | O | - | - | - | [A.5.2. ] | UTF8String | M | P | - | V | Y |
| AVP | * | * | O | - | - | - | * | * |  |  |  |  |  |
| Control-Memory-Size | 45687 | 1002 | O | - | - | - | [A.5.5. ] | Unsigned32 | M | P | - | V | Y |
| Current-Number-Members | 45687 | 1003 | O | - | - | - | [A.5.6. ] | Unsigned32 | M | P | - | V | Y |
| Data-Memory-Size | 45687 | 1004 | O | - | - | - | [A.5.7. ] | Unsigned32 | M | P | - | V | Y |
| Destination-Host | 0 | 293 | O | - | - | - | IETF RFC 3588 [13] | DiamIdent | M | P | - | V | N |
| Event-Timestamp | 0 | 55 | O | O | - | - | IETF RFC 3588 [13] | Time | M | P | - | V | N |
| External-ID | 45687 | 1005 | O | - | - | - | [A.5.8. ] | UTF8String | M | P | - | V | Y |
| Group-Name | 45687 | 1006 | O | - | - | - | [A.5.9. ] | UTF8String | M | P | - | V | Y |
| Hosting-CSE-ID | 45687 | 1007 | O | - | - | - | [A.5.10. ] | UTF8String | M | P | - | V | Y |
| Originator | 45687 | 1008 | M | - | - | - | [A.5.11. ] | UTF8String | M | P | - | V | Y |
| Maximum-Number-Members | 45687 | 1009 | O | - | - | - | [A.5.12. ] | Unsigned32 | M | P | - | V | Y |
| M2M-Event-Record-Timestamp | 45687 | 1010 | M | - | - | - | [A.5.13. ] | Time | M | P | - | V | Y |
| M2M-Information | 45687 | 1011 | M | - | - | - | [A.5.14. ] | Grouped | M | P | - | V | Y |
| Node-Id | 10415 | 2064 | M | - | - | - | TS 32.299 [i.5] | UTF8String | V,M | P | - | - | N |
| Occupancy | 45687 | 1012 | O | - | - | - | [A.5.16. ] | Unsigned32 | M | P | - | V | Y |
| Origin-Host | 0 | 264 | M | M | - | - | IETF RFC 3588 [13] | DiamIdent | M | P | - | V | N |
| Origin-Realm | 0 | 296 | M | M | - | - | IETF RFC 3588 [13] | DiamIdent | M | P | - | V | N |
| Origin-State-Id | 0 | 278 | O | O | - | - | IETF RFC 3588 [13] | Unsigned32 | M | P | - | V | N |
| Protocol-Type | 45687 | 1013 | O | - | - | - | [A.5.17. ] | Enumerated | M | P | - | V | Y |
| Proxy-Info | 0 | 284 | O | O | - | - | IETF RFC 3588 [13] | Grouped | M | - | - | P,V | N |
| Rating-Group | 0 | 432 | O | - | - | - | IETF RFC 4006 [i.6] | Unsigned32 | M | P | - | V | Y |
| Receiver | 45687 | 1014 | O | - | - | - | [A.5.19. ] | UTF8String | M | P | - | V | Y |
| Request-Body-Size | 45687 | 1015 | O | - | - | - | [A.5.20. ] | Unsigned32 | M | P | - | V | Y |
| Request-Headers-Size | 45687 | 1016 | O | - | - | - | [A.5.21. ] | Unsigned32 | M | P | - | V | Y |
| Request-Operation | 45687 | 1017 | O | - | - | - | [A.5.22. ] | Enumerated | M | P | - | V | Y |
| Response-Body-Size | 45687 | 1018 | O | - | - | - | [A.5.23. ] | Unsigned32 | M | P | - | V | Y |
| Response-Headers-Size | 45687 | 1019 | O | - | - | - | [A.5.24. ] | Unsigned32 | M | P | - | V | Y |
| Response-Status-Code | 45687 | 1020 | O | - | - | - | [A.5.25. ] | Enumerated | M | P | - | V | Y |
| Result-Code | 0 | 268 | - | M | - | - | IETF RFC 3588 [13] | Unsigned32 | M | P | - | V | N |
| Route-Record | 0 | 282 | O | - | - | - | IETF RFC 3588 [13] | DiamIdent | M | - | - | P,V | N |
| Service-Context-Id | 0 | 461 | O | - | - | - | [A.0.26. ] | Grouped | M | P | - | V | N |
| Service-Information | 10415 | 873 | O | - | - | - | TS 32.299 [i.5] | Grouped | M | P | - | V | N |
| Session-Id | 0 | 263 | M | M | - | - | RFC 3588 [13] | UTF8String | M | P | - | V | Y |
| Subgroup-Name | 45687 | 1021 | O | - | - | - | [A.0.28. ] | UTF8String | M | P | - | V | Y |
| Subscription-Id | 0 | 443 | M | - | - | - | RFC 4006 [i.6] | Grouped | M | P | - | V | Y |
| Subscription-Id-Data | 0 | 444 | M | - | - | - | RFC 4006 [i.6] | UTF8String | M | P | - | V | Y |
| Subscription-Id-Type | 0 | 450 | M | - | - | - | RFC 4006 [i.6] | Enumerated | M | P | - | V | Y |
| Target-ID | 45687 | 1022 | O | - | - | - | [A.0.32. ] | UTF8String | M | P | - | V | Y |

# A.5	oneM2M Specific AVP Usage

## A.5.1	Access-Network-Identifier AVP

The Access-Network-Identifier AVP (AVP Code 1000) is of type Unsigned32 and identifies the access network associated with the request triggering the M2M Event Record. The IN-CSE detects the link on which a request came from or was  sent to and that link maps to a specific Network and locally configured identifier.

## A.5.2	Acct-Application-Id AVP

Since the protocol used on Mch is Diameter Accounting, this AVP shall contain the value of 3 as defined in IETF RFC 3588 [13].

## A.5.3	Accounting-Record-Type AVP

The Accounting-Record-Type AVP (AVP Code 480) is of type Enumerated and contains the type of accounting record being sent. The following value is currently defined for the Accounting-Record-Type AVP: EVENT_RECORD (value 1) for an Event Based request.

## A.5.4	Application-Entity-ID AVP

The Application-Entity-ID AVP (AVP Code 1001) is of type UTF8String and represents the identity of the M2M Application Entity when it is applicable. The format of the AE-ID is specified in clause 6.2.3.

## A.5.5	Control-Memory-Size AVP

The Control-Memory-Size AVP (AVP Code 1002) is of type Unsigned32 and represents the storage memory (in bytes) used to store control related information associated with the M2M event record (excludes data storage associated with container related operations).

## A.5.6	Current-Number-Members AVP

The Current-Number-Members AVP (AVP Code 1003) is of type Unsigned32 and represents the current number of members in a group as determined by the responses to a request transmitted to a group. This is the same as the attribute "currentNrOfMembers" for the group as described in Table 7.4.14.1-3.

## A.5.7	Data-Memory-Size AVP

The Data-Memory-Size AVP (AVP Code 1004) is of type Unsigned32 and represents the storage memory in bytes, where applicable, to store data  associated with container related operations.

## A.5.8	External-ID AVP

The External-ID AVP (AVP Code 1005) is of type UTF8String and contains the external ID used to communicate over Mcn where applicable. The format is the same as the M2M-Ext-ID in clause Addressing.

## A.5.9	Group-Name AVP

The Group-Name AVP (AVP Code 1006) is of type UTF8String and identifies the group associated with a request. It shall be included when the IN initiates a fanning operation. This is the same as the attribute "groupName" for the group as described in Table 7.4.14.1-3.

## A.5.10	Hosting-CSE-ID AVP

The Hosting-CSE-ID AVP (AVP Code 1007) is of type UTF8String and represents the identity of the hosting CSE for the request in case the receiver is not the host. The format of the CSE-ID is specified in clause 6.2.3.

## A.5.11	Originator AVP

The Originator AVP (AVP Code 1008) is of type UTF8String and identifies the originator (i.e., from party) of the M2M request. This can be any M2M Node with format as per clause 6.2.3.

## A.5.12	Maximum-Number-Members AVP

The Maximum-Number-Members AVP (AVP Code 1009) is of type Unsigned32 and represents the maximum number of members of the group for the Create and Update operations. This is the same as the attribute "maxNrOfMembers" for the group as described in Table 7.4.14.1-3

## A.5.13	M2M-Event-Record-Timestamp AVP

The M2M-Event-Record-Timestamp AVP (AVP code 1010) is of type Time and represents the time for recording the M2M event record.

## A.5.14	M2M-Information AVP

The M2M-Information AVP (AVP code 1011) is of type Grouped. Its purpose is to allow the transmission of service information elements used for OneM2M specific charging.

It has the following ABNF grammar:

M2M-Information :: =  < AVP Header: 1011>

[ Application-Entity-ID ]

[ External-ID ]

[ Receiver ]

[ Originator ]

[ Hosting-CSE-ID ]

[ Target-ID ]

[ Protocol-Type ]

[ Request-Operation ]

[ Request-Headers-Size ]

[ Request-Body-Size ]

[ Response-Headers-Size ]

[ Response-Body-Size ]

[ Response-Status-Code ]

[ Rating-Group ]

[ M2M-Event-Record-Timestamp ]

[ Control-Memory-Size ]

[ Data-Memory-Size ]

[ Access-Network-Identifier ]

[ Occupancy ]

[ Group-Name ]

[ Maximum-Number-Members ]

[ Current-Number-Members ]

[ Subgroup-Name ][ Node-Id ]

* [ AVP ]

## A.5.15	Node-ID AVP

The Node-Id AVP (AVP Code 2064) is of type UTF8String and includes an optional, operator configurable identifier string for the node generating the Accounting-Record-Number for the Diameter ACR.

## A.5.16	Occupancy AVP

The Occupancy AVP (AVP Code 1012) is of type Unsigned32 and represents the overall size (in bytes) of the containers generated by a set of AEs identified by the M2M Service Subscription Identifier

## A.5.17	Protocol-Type AVP

The Protocol-Type AVP (AVP Code 1013) is of type Enumerated and indicates the protocol used for the request. The values are given below:

0	HTTP

1	CoAP

2	MQTT

3 .. 99	Reserved for OneM2M defined protocol types

100 .. 199	Operator and vendor specific protocol types

## A.5.18	Rating-Group AVP

The Rating-Group AVP (AVP Code 432) is of type Unsigned32 and represents a classification of M2M event records for charging purposes. This is assigned by the IN and is M2M SP specific.

## A.5.19	Receiver AVP

The Receiver AVP (AVP Code 1014) is of type UTF8String and identifies the receiver (i.e., to party) of the M2M request. This can be any M2M Node with format as per clause 6.2.3.

## A.5.20	Request-Body-Size AVP

The Request-Body-Size AVP (AVP Code 1015) is of type Unsigned32 and represents the number of bytes of the body transported in the Request.

## A.5.21	Request-Headers-Size AVP

The Request-Headers-Size AVP (AVP Code 1016) is of type Unsigned32 and represents the number of bytes in the control information header in the Request.

## A.5.22	Request-Operation AVP

The Request-Operation AVP (AVP Code 1017) is of type Enumerated and identifies the type of operation requested. The values are defined in Table 6.3.4.2.5-1.

## A.5.23	Response-Body-Size AVP

The Response-Body-Size AVP (AVP Code 1018) is of type Unsigned32 and represents the number of bytes of the body transported in the Response.

## A.5.24	Response-Headers-Size AVP

The Response-Headers-Size AVP (AVP Code 1019) is of type Unsigned32 and represents the number of bytes in the control information header in the Response.

## A.5.25	Response-Status-Code AVP

The Response-Status-Code AVP (AVP Code 1020) is of type Enumerated and identifies the value of returned in the Response Status Code parameter of the Response. The values are defined in clause 6.6.3.

## A.5.26	Service-Context-Id AVP

This AVP is of type UTF8String and contains a unique identifier of the Diameter charging specific document that applies the request. This is an identifier allocated by the service provider, by the service element manufacturer, or by a standardization body, and shall uniquely identify a given Diameter charging specific document.

The format of the Service-Context-Id is:

"extensions"."Release"."service-context" "@" "domain"

The OneM2M specific values "service-context" "@" "domain" are:

ts0004@oneM2M.org for OneM2M charging

The "Release" indicates the OneM2M Release the service specific document is based upon e.g. 1 for Release 1.

The "extensions" is operator specific information to any extensions in a service specific document.

## A.5.27	Service-Information AVP

The Service-Information AVP (AVP code 873) is of type Grouped. Its purpose is to allow the transmission of additional OneM2M specific information elements.

The complete ABNF syntax is defined and maintained in ETSI TS 132 299 [i.5]. The group structure includes zero or more occurrences of the Subscription-Id AVP and the M2M-Information AVP.

The format and content of the M2M-Information AVP which includes the OneM2M specific AVPs are specified in the present document.

## A.5.28	Subgroup-Name AVP

The Subgroup-Name AVP (AVP Code 1021) is of type UTF8String and identifies the subgroup associated with a request. It shall be included when the IN initiates a fanning operation and one of the members of the group is a. This is the same as the attribute "groupName" for the subgroup as described in Table 7.4.14.1-3.

## A.5.29	Subscription-Id AVP

The Subscription-Id AVP (AVP Code 443) is of type Grouped with structure defined in RFC 4006 [30]. The Subscription-Id AVP includes a Subscription-Id-Data AVP that holds the identifier and a Subscription-Id-Type AVP that defines the identifier type.

For M2M, this identifies the M2M Service Subscription ID associated with the request. This is determined by association maintained by the M2M SP as per clause 12.1.3 in ETSI TS 118 101 [6].

## A.5.30	Subscription-Id-Data AVP

The Subscription-Id-Data AVP (AVP Code 444) is of type UTF8String as defined in RFC 4006 [i.6]. The Subscription-Id-Data is used to identify the M2M Service Subscription. The Subscription-Id-Type AVP defines which type of identifier is used.

## A.5.31	Subscription-Id-Type AVP

The Subscription-Id-Type AVP (AVP Code 450) is of type Enumerated as defined in RFC 4006 [i.6]. It is used to determine which type of identifier is carried by the Subscription-Id AVP. The type(s) to be supported  is(are) determined by the M2M SP.

## A.5.32	Target-ID AVP

The Target-ID AVP (AVP Code 1022) is of type UTF8String and identifies the target URL for the M2M request if available.

Alternatively the Target-ID AVP can identify the target resource identifier with format defined in clause 6.3.4.

Annex B (normative):
Device triggering

# B.1	Providing device triggering service by means of 3GPP networks

## B.1.1	Introduction

3GPP Underlying Network has defined a dedicated interface for requesting device triggering. The normative references for applicable interfaces are as follows: ETSI TS 123 682 [15]. The specification for the interface Tsp is described in ETSI TS 129 368 [16]. Tsp interface uses Diameter Base Protocol as specified in IETF RFC 3588 [13], in order to use such an interface the CSE shall act as a Diameter client as described in IETF RFC 6733 [14].

Editor''s Note: IETF RFC 3588 Reference needs to be checked to determine that it is current.

Before the CSE initiates the device triggering, the CSE and MTC-IWF shall execute the procedures once as specified in ETSI TS 129 368 [16].

## B.1.2	Device action request command

When a CSE needs to issue a device triggering request to the MTC-IWF, the CSE shall send a Device-Action-Request (DAR) command (for detail, see ETSI TS 129 368 [16]). The following list provides the parameters mapping between the oneM2M and 3GPP.

Either External-Id or MSISDN:  the CSE maps it to the M2M-Ext-ID, see clause 6.2.

SCS identifier: the CSE maps it to the CSE-ID, see clause 6.2.

Application Port Identifier: the CSE maps it to Trigger-Recipient-ID, see clause 6.2.

## B.1.3	Device action answer command

As a result of device triggering request to MTC-IWF, the CSE receives a Device-Action-Answer (DAA) command (for detail, see ETSI TS 129 368 [16]).

## B.1.4	Device notification request command

As a report of the result for device triggering delivery by 3GPP network, the CSE receives a Device-Notification-Request (DNR) command (for detail, see ETSI TS 129 368 [16]).

## B.1.5	Device notification answer command

As a result of device notification request to MTC-IWF, the CSE sends a Device-Notification-Answer (DNA) command (for detail, see ETSI TS 129 368 [16]).

Annex C(informative):
XML examples

# C.1	XML schema for container resource type

<?xml version="1.0" encoding="UTF-8"?>

<!--

Copyright Notification

The oneM2M Partners authorize you to copy this document, provided that you retain all copyright and other proprietary notices

contained in the original materials on any copies of the materials and that you comply strictly with these terms.

This copyright permission does not constitute an endorsement of the products or services, nor does it encompass the granting of

any patent rights. The oneM2M Partners assume no responsibility for errors or omissions in this document.

© 2015, oneM2M Partners Type 1 (ARIB, ATIS, CCSA, ETSI, TIA, TTA, TTC). All rights reserved.

Notice of Disclaimer & Limitation of Liability

The information provided in this document is directed solely to professionals who have the appropriate degree of experience to understand

and interpret its contents in accordance with generally accepted engineering or other professional standards and applicable regulations.

No recommendation as to products or vendors is made or should be implied.

NO REPRESENTATION OR WARRANTY IS MADE THAT THE INFORMATION IS TECHNICALLY ACCURATE OR SUFFICIENT OR CONFORMS TO ANY STATUTE,

GOVERNMENTAL RULE OR REGULATION, AND FURTHER, NO REPRESENTATION OR WARRANTY IS MADE OF MERCHANTABILITY OR FITNESS FOR ANY

PARTICULAR PURPOSE OR AGAINST INFRINGEMENT OF INTELLECTUAL PROPERTY RIGHTS.

NO oneM2M PARTNER TYPE 1 SHALL BE LIABLE, BEYOND THE AMOUNT OF ANY SUM RECEIVED IN PAYMENT BY THAT PARTNER FOR THIS DOCUMENT, WITH RESPECT TO

ANY CLAIM, AND IN NO EVENT SHALL oneM2M BE LIABLE FOR LOST PROFITS OR OTHER INCIDENTAL OR CONSEQUENTIAL DAMAGES.

oneM2M EXPRESSLY ADVISES ANY AND ALL USE OF OR RELIANCE UPON THIS INFORMATION PROVIDED IN THIS DOCUMENT IS AT THE RISK OF THE USER.

-->

<xs:schema xmlns="http://www.w3.org/2001/XMLSchema"

targetNamespace="http://www.onem2m.org/xml/protocols"  xmlns:m2m="http://www.onem2m.org/xml/protocols"

elementFormDefault="unqualified"

<xs:include schemaLocation="CDT-commonTypes-v1_6_0.xsd" />

<xs:include schemaLocation="CDT-contentInstance-v1_6_0.xsd" />

<xs:include schemaLocation="CDT-subscription-v1_6_0.xsd" />

<xs:element name="container">

<xs:complexType>

<xs:complexContent>

<!-- Inherit Common Attributes from announceableResource -->

<xs:extension base="m2m:announceableResource">

<!-- Resource Specific Attributes -->

<xs:sequence>

<xs:element name="stateTag" type="xs:nonNegativeInteger" />

<xs:element name="creator" type="m2m:ID" />

<xs:element name="maxNrOfInstances" type="xs:nonNegativeInteger"

minOccurs="0" />

<xs:element name="maxByteSize" type="xs:nonNegativeInteger"

minOccurs="0" />

<xs:element name="maxInstanceAge" type="xs:nonNegativeInteger"

minOccurs="0" />

<xs:element name="currentNrOfInstances" type="xs:nonNegativeInteger" />

<xs:element name="currentByteSize" type="xs:nonNegativeInteger" />

<xs:element name="locationID" type="xs:anyURI"

minOccurs="0" />

<xs:element name="ontologyRef" type="xs:anyURI"

minOccurs="0" />

<!-- Child Resources -->

<xs:element name="latest" type="xs:anyURI" minOccurs="0" />

<xs:element name="oldest" type="xs:anyURI" minOccurs="0" />

<xs:choice minOccurs="0" maxOccurs="1" >

<xs:element name="childResource" type="m2m:childResourceRef"

minOccurs="1" maxOccurs="unbounded" />

<xs:choice minOccurs="1" maxOccurs="unbounded" >

<xs:element ref="m2m:container" />

<xs:element ref="m2m:contentInstance" />

<xs:element ref="m2m:subscription" />

</xs:choice>

</xs:choice>

</xs:sequence>

</xs:extension>

</xs:complexContent>

</xs:complexType>

</xs:element>

</xs:schema>

# C.2	Container resource that conforms to the Schema given above (see Annex C.1)

<?xml version="1.0" encoding="UTF-8"?>

<m2m:container xmlns:m2m="http://www.onem2m.org/xml/protocols"

xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance"

xsi:schemaLocation=

"http://www.onem2m.org/xml/protocols CDT-container-v1_6_0.xsd"

resourceName="12xx">

<resourceType>3</resourceType>

<resourceID>//IN-CSEID.m2m.myoperator.org/96719</resourceID>

<parentID>//IN-CSEID.m2m.myoperator.org/96734</parentID>

<creationTime>20141003T112032</creationTime>

<lastModifiedTime>20141003T112032</lastModifiedTime>

<labels>label1 label2</labels>

<accessControlPolicyIDs >

<accessControlPolicyID>l//IN-CSEID.m2m.myoperator.org/93405</accessControlPolicyID>

</accessControlPolicyID/>

<expirationTime>20141130T120000</expirationTime>

<stateTag>0 </stateTag>

<creator>//IN-CSEID.m2m.myoperator.org/9125</creator>

<maxNrOfInstances>5</maxNrOfInstances>

<maxByteSize>104857600</maxByteSize>

<maxInstanceAge>3600</maxInstanceAge>

<currentNrOfInstances>2</currentNrOfInstances>

<currentByteSize>6</currentByteSize>

<latest>//IN-CSEID.m2m.myoperator.org/96739</latest>

<locationID>//IN-CSEID.m2m.myoperator.org/1112</locationID>

<ontologyRef>http://tempuri.org/ontologies/xyz</ontologyRef>

<latest>//IN-CSEID.m2m.myoperator.org/96739</latest>

<oldest>//IN-CSEID.m2m.myoperator.org/34722</oldest>

<childResource name="instance1234" type="4">//IN-CSEID.m2m.myoperator.org/1722</childResource>

<childResource name="instance1235" type="4">//IN-CSEID.m2m.myoperator.org/34722</childResource>

<childResource name="1923" type="23">//IN-CSEID.m2m.myoperator.org/2323</childResource>

</m2m:container>

Annex D (normative):
<mgmtObj> Resource specializations

# D.1	Introduction

The Annex defines the structure and procedure for  the <mgmtObj> resource specializations. The resource specializations specified in the following clauses of this Annex shall be created on the IN-CSE when the management request is performed using technology specific protocols. The IN-CSE further interacts with the management server to perform management requests towards the managed entity. If the management request is performed solely over the M2M Service Layer, the <mgmtObj> resource specializations are created on the managed entity if the managed entity is equipped with a CSE. If the managed entities are non-oneM2M Nodes, the resources are created on the MN-CSE of the managed entity. The details can be found in the ETSI TS 118 101 [6].

# D.2	Resource [firmware]

## D.2.1	Introduction

The detailed description of the [firmware] resource can be found in clause D.2 of the ETSI TS 118 101 [6].

Table D.2-1: Data Type Definition of [firmware]

| Data Type ID | File Name | Note |
| --- | --- | --- |
| firmware, firmwareAnnc | CDT-firmware-v1_6_0.xsd |  |

Table D.2-2: Resource specific attributes of [firmware]

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| mgmtDefinition | M | NP | See clause 7.4.16 | 1001 (firmware) |
| objectID | O | NP | See clause 7.4.16 |  |
| objectPaths | O | NP | See clause 7.4.16 |  |
| description | O | O | See clause 7.4.16 |  |
| version | M | O | xs:string |  |
| firmwareName | M | O | xs:string |  |
| URL | M | O | xs:anyURI |  |
| update | M | O | xs:boolean |  |
| updateStatus | NP | O | m2m:actionStatus |  |

## D.2.2	Resource specific procedure on CRUD operations

### D.2.2.0	Introduction

When management is performed using technology specific protocols, the procedures defined in 7.4.16.2 <mgmtObj> specific procedures shall be used. The following clauses define additional procedures besides the generic procedure defined in 7.2.2.

### D.2.2.1	Create

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

Primitive specific step after generic procedure defined in clause 7.2.2.2.

May start to download the firmware image from the location indicated by attribute URL in the firmware resource.

### D.2.2.2	Update

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

Primitive specific operation additional to Recv-6.5 "Create/Update/Retrieve/Delete/Notify operation is performed":

When the attribute update of the firmware resource is updated to TRUE, use the downloaded firmware image to update the current using firmware. The Receiver may need to update the fwVersion attribute of the [deviceInfo] resource if needed.

### D.2.2.3	Retrieve

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

### D.2.2.4	Delete

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

Primitive specific step after generic procedure defined in clause 7.2.2.2:

Delete the downloaded firmware image locally.

# D.3	Resource [software]

## D.3.1	Introduction

The detailed description of the [software] resource  can be found in clause D.3 of ETSI TS 118 101 [6].

Table D.3-1: Data Type Definition of [software]

| Data Type ID | File Name | Note |
| --- | --- | --- |
| software, softwareAnnc | CDT-software-v1_6_0.xsd |  |

Table D.3-2: Resource specific attributes of [software]

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| mgmtDefinition | M | NP | See clause 7.4.16 | 1002 (software) |
| objectID | O | NP | See clause 7.4.16 |  |
| objectPaths | O | NP | See clause 7.4.16 |  |
| description | O | O | See clause 7.4.16 |  |
| version | M | O | xs:string |  |
| softwareName | M | O | xs:string |  |
| URL | M | O | xs:anyURI |  |
| install | NP | O | xs:boolean |  |
| uninstall | NP | O | xs:boolean |  |
| installStatus | NP | NP | m2m:actionStatus |  |
| activate | NP | O | xs:boolean |  |
| deactivate | NP | O | xs:boolean |  |
| activeStatus | NP | NP | m2m:actionStatus |  |

## D.3.2	Resource specific procedure on CRUD operations

### D.3.2.0	Introduction

When management is performed using technology specific protocols, the procedures defined in 7.4.16.2 <mgmtObj> resource specific procedures shall be used. The following clauses define additional procedures besides the generic procedure defined in 7.2.2.

### D.3.2.1	Create

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

May start to download the software package from the location indicated by attribute URL in the software resource.

### D.3.2.2	Update

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

Primitive specific operation additional to Recv-6.5 "Create/Update/Retrieve/Delete/Notify operation is performed":

When the attribute install of the [software] resource is updated to TRUE, install the software package downloaded from the address indicated by attribute URL of the [software] resource.

When the attribute uninstall of the [software] resource is updated to TRUE, uninstall the corresponding software of the [software] resource.

When the attribute activate of the [software] resource is updated to TRUE, activate the corresponding software of the [software] resource.

When the attribute deactivate of the [software] resource is updated to TRUE, deactivate the corresponding software of the [software] resource.

The Receiver may need to update the swVersion attribute of the [deviceInfo] resource if needed.

### D.3.2.3	Retrieve

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

### D.3.2.4	Delete

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

Primitive specific step after generic procedure defined in clause 7.2.2.2.

Delete the downloaded software package locally.

# D.4	Resource [memory]

## D.4.1	Introduction

The detailed description of the [memory] resource can be found in clause D.4 of ETSI TS 118 101 [6].

Table D.4-1: Data Type Definition of [memory]

| Data Type ID | File Name | Note |
| --- | --- | --- |
| memory, memoryAnnc | CDT-memory-v1_6_0.xsd |  |

Table D.4-2: Resource specific attributes of [memory]

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| mgmtDefinition | M | NP | See clause 7.4.16 | 1003 (memory) |
| objectID | O | NP | See clause 7.4.16 |  |
| objectPaths | O | NP | See clause 7.4.16 |  |
| description | O | O | See clause 7.4.16 |  |
| memAvailable | M | O | xs:unsignedLong | Unit: Byte. |
| memTotal | M | O | xs:unsignedLong | Unit: Byte. |

## D.4.2	Resource specific procedure on CRUD operations

### D.4.2.0	Introduction

When management is performed using technology specific protocols, the procedures defined in 7.4.16.2 <mgmtObj> specific procedures shall be used. The following clauses define additional procedures besides the generic procedure defined in 7.2.2.

### D.4.2.1	Create

.Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

### D.4.2.2	Update

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

### D.4.2.3	Retrieve

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

### D.4.2.4	Delete

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

# D.5	Resource [areaNwkInfo]

## D.5.1	Introduction

The detailed description of the [areaNwkInfo] resource can be found in clause D.5 of ETSI TS 118 101 [6].

Table D.5-1: Data Type Definition of [areaNwkInfo]

| Data Type ID | File Name | Note |
| --- | --- | --- |
| areaNwkInfo, areaNwkInfoAnnc | CDT-areaNwkInfo-v1_6_0.xsd |  |

Table D.5-2: Resource specific attributes of [areaNwkInfo]

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| mgmtDefinition | M | NP | See clause 7.4.16 | 1004 (areaNwkInfo) |
| objectID | O | NP | See clause 7.4.16 |  |
| objectPaths | O | NP | See clause 7.4.16 |  |
| description | O | O | See clause 7.4.16 |  |
| areaNwkType | M | O | xs:string |  |
| listOfDevices | M | O | list of xs:anyURI |  |

## D.5.2	Resource specific procedure on CRUD operations

### D.5.2.0	Introduction

When management is performed using technology specific protocols, the procedures defined in 7.4.16.2 <mgmtObj> specific procedures shall be used. The following clauses define additional procedures besides the generic procedure defined in 7.2.2.

### D.5.2.1	Create

.Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

### D.5.2.2	Update

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

### D.5.2.3	Retrieve

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

### D.5.2.4	Delete

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

# D.6	Resource [areaNwkDeviceInfo]

## D.6.1	Introduction

The detailed description of the [areaNwkDeviceInfo] resource can be found in clause D.6 of ETSI TS 118 101 [6].

Table D.6-1: Data Type Definition of [areaNwkDeviceInfo]

| Data Type ID | File Name | Note |
| --- | --- | --- |
| areaNwkDeviceInfo, areaNwkDeviceInfoAnnc | CDT-areaNwkDeviceInfo-v1_6_0.xsd |  |

Table D.6-2: Resource specific attributes of [areaNwkDeviceInfo]

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| mgmtDefinition | M | NP | See clause 7.4.16 | 1005 (areaNwkDeviceInfo) |
| objectID | O | NP | See clause 7.4.16 |  |
| objectPaths | O | NP | See clause 7.4.16 |  |
| description | O | O | See clause 7.4.16 |  |
| devID | M | O | xs:string |  |
| devType | M | O | xs:string |  |
| areaNwkId | M | O | xs:anyURI |  |
| sleepInterval | O | O | xs:nonNegativeInteger | Unit: second |
| sleepDuration | O | O | xs:nonNegativeInteger | Unit: second |
| devStatus | O | O | xs:string |  |
| listOfNeighbors | M | O | list of xs:anyURI |  |

## D.6.2	Resource specific procedure on CRUD operations

### D.6.2.0	Introduction

When management is performed using technology specific protocols, the procedures defined in 7.4.16.2 <mgmtObj> specific procedures shall be used. The following clauses define additional procedures besides the generic procedure defined in 7.2.2.

### D.6.2.1	Create

.Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

### D.6.2.2	Update

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

Primitive specific operation additional to Recv-6.5 "Create/Update/Retrieve/Delete/Notify operation is performed":

When the attribute listOfNeighbors of the [areaNwkDeviceInfo] resource is updated, the receiver shall modify the corresponding connection relationship among devices in the M2M Area Network by sending signals to non-oneM2M Nodes which is out of scope of oneM2M. According to the response from the non-oneM2M nodes of the modify signal, the receiver shall corresponding update the [areaNwkDeviceInfo] resource which may include the update of the listOfNeighbors and the devType attribute. The modification may include change of the attach point of the device or removal from the area network.

### D.6.2.3	Retrieve

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

### D.6.2.4	Delete

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

# D.7	Resource [battery]

## D.7.1	Introduction

The detailed description of the [battery] resource can be found in clause D.7 of Architecture ETSI TS 118 101 [6].

Table D.7-1: Data Type Definition of [battery]

| Data Type ID | File Name | Note |
| --- | --- | --- |
| battery, batteryAnnc | CDT-battery-v1_6_0.xsd |  |

Table D.7-2: Resource specific attributes of [battery]

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| mgmtDefinition | M | NP | See clause 7.4.16 | 1006 (battery) |
| objectID | O | NP | See clause 7.4.16 |  |
| objectPaths | O | NP | See clause 7.4.16 |  |
| description | O | O | See clause 7.4.16 |  |
| batteryLevel | M | O | xs:unsignedInt | Range: 0-100 Unit: percent |
| batteryStatus | M | O | m2m:batteryStatus |  |

## D.7.2	Resource specific procedure on CRUD operations

### D.7.2.0	Introduction

When management is performed using technology specific protocols, the procedures defined in 7.4.16.2 <mgmtObj> specific procedures shall be used. The following clauses define additional procedures besides the generic procedure defined in 7.2.2.

### D.7.2.1	Create

.Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

### D.7.2.2	Update

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

### D.7.2.3	Retrieve

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

### D.7.2.4	Delete

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

# D.8	Resource [deviceInfo]

## D.8.1	Introduction

The resource [deviceInfo] is used to provide information regarding the device.

The detailed description of the [deviceInfo] resource can be found in clause D.8 of Architecture ETSI TS 118 101 [6].

Table D.8-1: Data Type Definition of [deviceInfo]

| Data Type ID | File Name | Note |
| --- | --- | --- |
| deviceInfo, deviceInfoAnnc | CDT-deviceInfo-v1_6_0.xsd |  |

Table D.8-2: Resource specific attributes of [deviceInfo]

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| mgmtDefinition | M | NP | See clause 7.4.16 | 1007 (deviceInfo) |
| objectID | O | NP | See clause 7.4.16 |  |
| objectPaths | O | NP | See clause 7.4.16 |  |
| description | O | O | See clause 7.4.16 |  |
| deviceLabel | M | O | xs:string |  |
| manufacturer | M | O | xs:string |  |
| model | M | O | xs:string |  |
| deviceType | M | O | xs:string |  |
| fwVersion | M | O | xs:string |  |
| swVersion | M | O | xs:string |  |
| hwVersion | M | O | xs:string |  |

## D.8.2	Resource specific procedure on CRUD operations

### D.8.2.0	Introduction

When management is performed using technology specific protocols, the procedures defined in 7.4.16.2 <mgmtObj> specific procedures shall be used. The following clauses define additional procedures besides the generic procedure defined in 7.2.2.

### D.8.2.1	Create

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

### D.8.2.2	Update

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

### D.8.2.3	Retrieve

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

### D.8.2.4	Delete

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

# D.9	Resource [deviceCapability]

## D.9.1	Introduction

The resource [deviceCapability] is used to provide information regarding the device.

The detailed description of the [deviceCapability] resource can be found in clause D.9 of ETSI TS 118 101 [6].

Table D.9-1: Data Type Definition of [deviceCapability]

| Data Type ID | File Name | Note |
| --- | --- | --- |
| deviceCapability, deviceCapabilityAnnc | CDT-deviceCapability-v1_6_0.xsd |  |

Table D.9-2: Resource specific attributes of [deviceCapability]

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| mgmtDefinition | M | NP | See clause 7.4.16 | 1008 (deviceCapability) |
| objectID | O | NP | See clause 7.4.16 |  |
| objectPaths | O | NP | See clause 7.4.16 |  |
| description | O | O | See clause 7.4.16 |  |
| capabilityName | M | O | xs:string |  |
| attached | M | O | xs:boolean | 1. true: currently attached to the device 2. false: currently detached to the device |
| capabilityActionStatus | M | O | m2m:actionStatus | The action (i.e., enable, disable) and the related status. See the Table 6.3.2.3 1 |
| currentState | M | O | xs:boolean | - true: the device capability is enabled - false: the device capability is disabled |
| enable | O | O | xs:boolean | the value of this attribute is always "true" |
| disable | O | O | xs:boolean | the value of this attribute is always "true" |

## D.9.2	Resource specific procedure on CRUD operations

### D.9.2.0	Introduction

When management is performed using technology specific protocols, the procedures defined in 7.4.16.2 <mgmtObj> specific procedures shall be used. The following clauses define additional procedures besides the generic procedure defined in 7.2.2.

### D.9.2.1	Create

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

### D.9.2.2	Update

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

Primitive specific operation additional to Recv-6.5 "Create/Update/Retrieve/Delete/Notify operation is performed":

When the attribute enable of the [deviceCapability] resource is updated to TRUE, enable the device capability of the [deviceCapability] resource.

When the attribute disable of the [deviceCapability] resource is updated to TRUE, disable the device capability of the [deviceCapability] resource.

### D.9.2.3	Retrieve

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

### D.9.2.4	Delete

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

# D.10	Resource [reboot]

## D.10.1	Introduction

The resource [reboot] is used to provide information regarding the device.

The detailed description of the [reboot] resource can be found in clause D.10 of ETSI TS 118 101 [6].

Table D.10-1: Data Type Definition of [reboot]

| Data Type ID | File Name | Note |
| --- | --- | --- |
| reboot, rebootAnnc | CDT-reboot-v1_6_0.xsd |  |

Table D.10-2: Resource specific attributes of [reboot]

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| mgmtDefinition | M | NP | See clause 7.4.16 | 1009 (reboot) |
| objectID | O | NP | See clause 7.4.16 |  |
| objectPaths | O | NP | See clause 7.4.16 |  |
| description | O | O | See clause 7.4.16 |  |
| reboot | O | O | xs:boolean | the value of this attribute is always "True" |
| factoryReset | O | O | xs:boolean | the value of this attribute is always "True" |

## D.10.2	Resource specific procedure on CRUD operations

### D.10.2.0	Introduction

When management is performed using technology specific protocols, the procedures defined in 7.4.16.2 <mgmtObj> specific procedures shall be used. The following clauses define additional procedures besides the generic procedure defined in 7.2.2.

### D.10.2.1	Create

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

### D.10.2.2	Update

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

Primitive specific operation additional to Recv-6.5 "Create/Update/Retrieve/Delete/Notify operation is performed":

When the attribute reboot of the [reboot] resource is updated to TRUE, reboot the corresponding node.

When the attribute factoryReset of the [reboot] resource is updated to TRUE, factory reset the corresponding node shall be applied.

### D.10.2.3	Retrieve

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

### D.10.2.4	Delete

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

# D.11	Resource [eventLog]

## D.11.1	Introduction

The Resource [eventLog] is used to provide information regarding the device.

The detailed description of the [eventLog] resource can be found in clause D.11 of ETSI TS 118 101 [6].

Table D.11-1: Data Type Definition of [eventLog]

| Data Type ID | File Name | Note |
| --- | --- | --- |
| eventLog, eventLogAnnc | CDT-eventLog-v1_6_0.xsd |  |

Table D.11-2: Resource specific attributes of [eventLog]

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| mgmtDefinition | M | NP | See clause 7.4.16 | 1010 (eventLog) |
| objectID | O | NP | See clause 7.4.16 |  |
| objectPaths | O | NP | See clause 7.4.16 |  |
| description | O | O | See clause 7.4.16 |  |
| logTypeId | M | O | m2m:logTypeId | See Table 6.3.4.2.23-1 |
| logData | M | O | xs:string | the content and format of this attribute is out of the present document. |
| logStatus | M | O | m2m:logStatus | See Table 6.3.4.2.24-1 |
| logStart | O | O | xs:boolean | the value of this attribute is always "True" |
| logStop | O | O | xs:boolean | the value of this attribute is always "True" |

## D.11.2	Resource specific procedure on CRUD operations

### D.11.2.1	Introduction

When management is performed using technology specific protocols, the procedures defined in 7.4.16.2 <mgmtObj> specific procedures shall be used. The following clauses define additional procedures besides the generic procedure defined in 7.2.2.

### D.11.2.2	Create

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

### D.11.2.3	Update

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

Primitive specific operation additional to Recv-6.5 "Create/Update/Retrieve/Delete/Notify operation is performed":

When the attribute logStart of the [eventLog] resource is updated to TRUE, start the logging.

When the attribute logStop of the [eventLog] resource is updated to TRUE, stop the logging.

### D.11.2.4	Retrieve

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

### D.11.2.5	Delete

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

# D.12	Resource [cmdhPolicy]

## D.12.0	Introduction

The resource [cmdhPolicy] represents a set of rules associated with a specific CSE that govern the behaviour of that CSE regarding rejecting, buffering and sending request or response messages via the Mcc reference point.

The detailed description can be found in clause D.12 of ETSI TS 118 101 [6].

Table D.12-1: Data Type Definition of [cmdhPolicy]

| Data Type ID | File Name | Note |
| --- | --- | --- |
| cmdhPolicy | CDT-cmdhPolicy-v1_6_0.xsd |  |

Note that the optional <subscription> child resources are not used for CMDH policies.

Table D.12-2: Resource specific attributes of [cmdhPolicy]

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| mgmtDefinition | M | NP | See clause 7.4.16 | 1011 (cmdhPolicy) |
| objectID | O | NP | See clause 7.4.16 |  |
| objectPaths | O | NP | See clause 7.4.16 |  |
| description | O | O | See clause 7.4.16 |  |
| cmdhPolicyName | M | O | xs:string | None |
| mgmtLink | M | O | m2m:mgmtLinkRef | 1 link to [cmdhDefaults] resource instance, 1 or more link(s) to [cmdhLimits] resource instance(s), 1 or more link(s) to [cmdhNetworkAccessRules] resource instance(s), 1 or more link(s) to [cmdhBuffer] resource instance(s) |

The Resource Specific Procedure on CRUD Operations as specified in clause 7.4.16 for the generic <mgmtObj> resource type apply.

## D.12.1	Resource [activeCmdhPolicy]

The resource [activeCmdhPolicy] provides a link to the currently active set of CMDH policies.

The detailed description can be found in clause D.12.1 of ETSI TS 118 101 [6].

Table D.12.1-1: Data Type Definition of [activeCmdhPolicy]

| Data Type ID | File Name | Note |
| --- | --- | --- |
| activeCmdPolicy | CDT-activeCmdhPolicy-v1_6_0.xsd |  |

Table D.12.1-2: Resource specific attributes of [activeCmdhPolicy]

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| mgmtDefinition | M | NP | See clause 7.4.16 | 1012 (activeCmdhPolicy) |
| objectID | O | NP | See clause 7.4.16 |  |
| objectPaths | O | NP | See clause 7.4.16 |  |
| description | O | O | See clause 7.4.16 |  |
| activeCmdhPolicyLink | M | O | m2m:ID | The resource ID of the [cmdhPolicy] resource instance containing the CMDH policies that are currently active for the associated CSE. |

## D.12.2	Resource [cmdhDefaults]

The resource [cmdhDefaults] defines which CMDH related parameters will be used by default when a request or response message contains the Event Category parameter but not any other CMDH related parameters and which default Event Category parameter shall be used when none is given in the request or response message. The detailed description can be found in clause D.12.2 of ETSI TS 118 101 [6].

Table D.12.2-1: Data Type Definition of [cmdhDefaults]

| Data Type ID | File Name | Note |
| --- | --- | --- |
| cmdhDefaults | CDT-cmdhDefaults-v1_6_0.xsd |  |

Table D.12.2-2: Resource specific attributes of [cmdhDefaults]

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| mgmtDefinition | M | NP | See clause 7.4.16 | 1013 (cmdhDefaults) |
| objectID | O | NP | See clause 7.4.16 |  |
| objectPaths | O | NP | See clause 7.4.16 |  |
| description | O | O | See clause 7.4.16 |  |
| mgmtLink | M | O | m2m:mgmtLinkRef | 1 or more link(s) to [cmdhDefEcValue] resource instance(s); 1 or more link(s) to [cmdhEcDefParamValues] resource instance(s) |

## D.12.3	Resource [cmdhDefEcValue]

The resource [cmdhDefEcValue] represents a default value for the Event Category parameter of an incoming request or response message. This default Event Category becomes applicable when certain conditions are matched which are defined by the other attributes of this resource. The detailed description can be found in clause D.12.3 of ETSI TS 118 101 [6].

Table D.12.3-1: Data Type Definition of [cmdhDefEcValue]

| Data Type ID | File Name | Note |
| --- | --- | --- |
| cmdhDefEcValue | CDT-cmdhDefEcValue-v1_6_0.xsd |  |

Table D.12.3-2: Resource specific attributes of [cmdhDefEcValue]

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| mgmtDefinition | M | NP | See clause 7.4.16 | 1014 (cmdhDefEcValue) |
| objectID | O | NP | See clause 7.4.16 |  |
| objectPaths | O | NP | See clause 7.4.16 |  |
| description | O | O | See clause 7.4.16 |  |
| order | M | O | xs:positiveInteger | None |
| defEcValue | M | O | m2m:eventCat | None |
| requestOrigin | M | O | m2m:listOfM2MID | None |
| requestContext | O | O | xs:anyType | None |
| requestContextNotification | O | O | xs:boolean | None |
| requestCharacteristics | O | O | xs:anyType | None |

## D.12.4	Resource [cmdhEcDefParamValues]

The resource [cmdhEcDefParamValues] represents a specific set of default values for the CMDH related parameters Request Expiration Timestamp, Result Expiration Timestamp, Operational Execution Time, Result Persistence and Delivery Aggregation that are applicable for a given Event Category if these parameters are not specified in the message. The detailed description can be found in clause D.12.4 of ETSI TS 118 101 [6].

Table D.12.4-1: Data Type Definition of [cmdhEcDefParamValues]

| Data Type ID | File Name | Note |
| --- | --- | --- |
| cmdhEcDefParamValues | CDT-cmdhEcDefParamValues-v1_6_0.xsd |  |

Table D.12.4-2: Resource specific attributes of [cmdhEcDefParamValues]

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| mgmtDefinition | M | NP | See clause 7.4.16 | 1015 (cmdhEcDefParamValues) |
| objectID | O | NP | See clause 7.4.16 |  |
| objectPaths | O | NP | See clause 7.4.16 |  |
| description | O | O | See clause 7.4.16 |  |
| applicableEventCategory | M | O | list of m2m:eventCatWithDef | Exactly one instance of this [cmdhEcDefParamValues] resource shall be provisioned which contains a value "0" (default) setting for this attribute. |
| defaultRequestExpTime | M | O | xs:long | -1 means infinity, unit: ms |
| defaultResultExpTime | M | O | xs:long | -1 means infinity, unit: ms |
| defaultOpExecTime | M | O | xs:long | -1 means infinity, unit: ms |
| defaultRespPersistence | M | O | xs:long | -1 means infinity, unit: ms |
| defaultDelAggregation | M | O | xs:boolean | None |

## D.12.5	Resource [cmdhLimits]

The resource [cmdhLimits] represents limits for CMDH related parameter values. The detailed description can be found in clause D.12.5 of ETSI TS 118 101 [6].

Table D.12.5-1: Data Type Definition of [cmdhLimits]

| Data Type ID | File Name | Note |
| --- | --- | --- |
| cmdhLimits | CDT-cmdhLimits-v1_6_0.xsd |  |

Table D.12.5-2: Resource specific attributes of [cmdhLimits]

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| mgmtDefinition | M | NP | See clause 7.4.16 | 1016 (cmdhLimits) |
| objectID | O | NP | See clause 7.4.16 |  |
| objectPaths | O | NP | See clause 7.4.16 |  |
| description | O | O | See clause 7.4.16 |  |
| order | M | O | xs:positiveInteger | None |
| requestOrigin | M | O | m2m:listOfM2MID | None |
| requestContext | O | O | xs:anyType | None |
| requestContextNotification | O | O | xs:boolean | None |
| requestCharacteristics | O | O | xs:anyType | None |
| limitsEventCategory | M | O | list of m2m:eventCat | None |
| limitsRequestExpTime | M | O | m2m:listOfMinMax | -1 means infinity, unit: ms |
| limitsResultExpTime | M | O | m2m:listOfMinMax | -1 means infinity, unit: ms |
| limitsOpExecTime | M | O | m2m:listOfMinMax | -1 means infinity, unit: ms |
| limitsRespPersistence | M | O | m2m:listOfMinMax | -1 means infinity, unit: ms |
| limitsDelAggregation | M | O | restricted list of xs:boolean | This attribute defines the permitted settings of the DeliveryAggregation parameter of request primitives. '0' means 'False' '1' means 'True' '0 1' means 'False' or 'True' |

## D.12.6	Resource [cmdhNetworkAccessRules]

The resource [cmdhNetworkAccessRules] defines the usage of underlying networks for forwarding information to other CSEs during processing of CMDH-related requests in a CSE. The detailed description can be found in clause D.12.6 of ETSI TS 118 101 [6].

Table D.12.6-1: Type Definition of [cmdhNetworkAccessRules]

| Data Type ID | File Name | Note |
| --- | --- | --- |
| cmdhNetworkAccessRules | CDT-cmdhNetworkAccessRules-v1_6_0.xsd |  |

Table D.12.6-2: Resource specific attributes of [cmdhNetworkAccessRules]

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| mgmtDefinition | M | NP | See clause 7.4.16 | 1017 (cmdhNetworkAccessRules) |
| objectID | O | NP | See clause 7.4.16 |  |
| objectPaths | O | NP | See clause 7.4.16 |  |
| description | O | O | See clause 7.4.16 |  |
| applicableEventCategories | M | O | list of m2m:eventCatWithDef | Exactly one instance of this [cmdhNetworkAccessRules] resource shall be provisioned which contains a value "0" (default) setting for this attribute. |
| mgmtLink | O | O | m2m:mgmtLinkRef | Zero or more links to [cmdhNwAccessRule] resource instance(s) |

## D.12.7	Resource [cmdhNwAccessRule]

The resource [cmdhNwAccessRule] defines limits in usage of specific underlying networks for forwarding information to other CSEs during processing of CMDH-related requests. The detailed description can be found in clause D.12.7 of ETSI TS 118 101 [6].

Table D.12.7-1: Data Type Definition of [cmdhNwAccessRule]

| Data Type ID | File Name | Note |
| --- | --- | --- |
| cmdhNwAccessRule | CDT-cmdhNwAccessRule-v1_6_0.xsd |  |

Table D.12.7-2: Resource specific attributes of [cmdhNwAccessRule]

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| mgmtDefinition | M | NP | See clause 7.4.16 | 1018 (cmdhNwAccessRule) |
| objectID | O | NP | See clause 7.4.16 |  |
| objectPaths | O | NP | See clause 7.4.16 |  |
| description | O | O | See clause 7.4.16 |  |
| targetNetwork | M | O | m2m:listOfM2MID | None |
| minReqVolume | M | O | xs:nonNegativeInteger | Unit: byte |
| backOffParameters | M | O | m2m: backOffParameters | Ordered sequence of 3 values: backoffTime, backoffTimeIncrement, maximumBackoffTime, Unit: ms |
| otherConditions | O | O | xs:anyType | None |
| mgmtLink | M | O | m2m:mgmtLinkRef | Link to an instance "allowedSchedule" of a <schedule> resource |

## D.12.8	Resource [cmdhBuffer]

The resource [cmdhBuffer] represents limits in usage of buffers for temporarily storing information that needs to be forwarded to other CSEs during processing of CMDH-related requests in a CSE. The detailed description can be found in clause D.12.8 of ETSI TS 118 101 Functional Architecture [6].

Table D.12.8-1: Data Type Definition of [cmdhBuffer]

| Data Type ID | File Name | Note |
| --- | --- | --- |
| cmdhBuffer | CDT-cmdhBuffer-v1_6_0.xsd |  |

Table D.12.8-2: Resource specific attributes of [cmdhBuffer]

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| mgmtDefinition | M | NP | See clause 7.4.16 | 1019 (cmdhBuffer) |
| objectID | O | NP | See clause 7.4.16 |  |
| objectPaths | O | NP | See clause 7.4.16 |  |
| description | O | O | See clause 7.4.16 |  |
| applicableEventCategory | M | O | list of m2m:eventCatWithDef | Exactly one instance of this [cmdhBuffer] resource shall be provisioned which contains a value "0" (default) setting for this attribute. |
| maxBufferSize | M | O | xs:nonNegativeInteger | Unit: byte |
| storagePriority | M | O | xs:positiveInteger | The range of storage priority is from 1 to 10. |

Annex E (informative):
Procedures for accessing resources

# E.1	Accessing resources in CSEs - blocking requests

The result of a Request is sent back to the Originator as part of the Response of the Request.This communication mode could  result in long blocking times.

The interaction employing blocking involves the following steps in this order:

Figure E.1-1: Blocking access to resource

- The Originator sends a request to accessing a resource. The Response Type parameter of the request is set  to 'blockingRequest'. The Response Type parameter can be omitted in this case since 'blockingRequest' is its default value.

- The Hosting CSE receives the request, and it completes the requested processing of resources.

- The Hosting CSE responds to Originator, the response contains the requested results in resource content, and the Response Status Code parameter of response is set to "OK".

# E.2	Accessing Resources in CSEs - non-blocking requests

## E.2.1	Non-blocking models

If the Originator chooses the Blocking mode described in clauseE.1, it might have to wait a long time for a response from the Receiver. To avoid this possibility it can choose a Non-Blocking mode. In Non-blocking modes, the Receiver sends an Acknowledgement of the request, which provides a reference to the result of the requested operation. The Originator can retrieve the result at a later time.

There are two forms of Non-blocking mode: Synchronous and Asynchronous.

## E.2.2	Synchronous case

The Originator asks for non-Blocking Communication by setting  the Response Type parameter of the Request  to 'nonBlockingRequestSynch'. The Receiver  CSE responds after acceptance with an Acknowledgement confirming, that it will process the Request further. The Receiver CSE creates a local <request> resource pertaining to the Request received and returns a reference to this created <request> resource as the Content of the acknowledgement Response. Then the Receiver needs to forward the Request to the next CSE if the Receiver CSE is not the Hosting CSE of the addressed resource. Or the Hosting CSE needs to start handling the Request if the Receiver CSE is the Hosting CSE of the addressed resource.

The Originator of the Request may retrieve the <request> resource afterwards to check on the status of its Request and to inspect the final result of the Request when this is available.

Figure E.2.2-1 illustrates the steps involved in a synchronous  non-blocking interaction. In this example the Receiver CSE is the CSE that hosts the resource that is the target of the Originator''s request.

Figure E.2.2-1: non-Blocking access to resource in synchronous mode (no hop)

- The originator sends a request to access a resource, setting the Response Type parameter of request to 'nonblockingRequestSynch'.

- If the Receiver CSE supports non-blocking synchronous  interactions (this is indicated by its support for the <request> resource), it creates an instance of <request> resource. The requestStatus attribute of the <request> resource is set to " ACCEPTED".  Please refer to Table 7.1.2.2.4-1 and Table 7.1.2.2.4-2for other attributes.

- The Hosting CSE sends a response to the Originator, the Response Status  Code parameter of its response is set to "ACCEPTED" , and a reference to the <request> resource is provided in the Content.

- The Hosting CSE processes the resource according to the requested operation . When the requested operation has finished, the Hosting CSE will UPDATE the <request> resource, putting the results of the operation into the operationResult attribute,  and updating the value of requestStatus to "COMPLETED", also the values of stateTag and lastModifiedTime.

- The Originator requests to RETRIEVE the original requested results by addressing the <request> resource.

- The Hosting CSE responds to Originator. The response contains the <request> resource as  its Content, and the Originator can examine the <request> resource''s requestStatus attribute to check that the operation has completed and retrieve its results from the operationResult attribute.

A variation of synchronous case is depicted in the following clauses. In this variation it is assumed that the addressed resource is not stored in the Registrar CSE, then the Registrar CSE needs to be a Transit CSE to forward the request to the Hosting CSE.

Figure E.2.2-2 illustrates this case. :

Figure E.2.2-2: non-Blocking access to resource in synchronous mode (one hop)

- The Originator sends a request to its Registrar CSE (this is a Transit CSE, not the Hosting CSE), setting the Response Type parameter of the request to 'nonblockingRequestSynch'.2. If the Transit CSE supports non-blocking synchronouse interactions (this is indicated by its support for the <request> resource), it creates an instance of <request> resource. The requestStatus attribute of the <request> resource is set to "ACCEPTED". The Please refer to Table 7.4.13.1-3 for other attributes.

- The Transit  CSE sends a response to the Originator, the Response Status Code parameter of its response is set to acknowledgement, and a reference to the <request> resource is provided in the Content.

- The Transit CSE forwards the original request to the Hosting CSE.

- If the Hosting  CSE supports non-blocking synchronouse interactions (this is indicated by its support for the <request> resource), it creates an instance of <request> resource. The requestStatus attribute of the <request> resource is set to "ACCEPTED". Please refer to Table 7.1.2.2.4-1 and Table 7.1.2.2.4-2for other attributes.

- The Hosting CSE sends a response to the Transit CSE, the Response Status Code parameter of its response is set to"ACCEPTED" and a reference to the <request> resource is provided in the Content.

- The Hosting CSE processes the resource according to the requested operation. When the requested operation has finished, the Hosting CSE will UPDATE the <request> resource, putting the results of the operation into the operationResult attribute, and updating the values of requestStatus to "COMPLETED", also the values of stateTag and lastModifiedTime.

- The Transit CSE requests to RETRIEVE the original requested results by addressing the <request> resource

- The Hosting CSE sends a response to the Transit CSE. The response contains the <request> resource as  its Content.

- The Transit CSE UPDATEs its <request> resource, copying  the operationResult from the response that it received from the Hosting CSE. It also updates the values of requestStatus, stateTag and lastModifiedTime.

- The Originator requests to RETRIEVE the original requested results by addressing the <request> resource.

- The Transit CSE responds to Originator. The response contains the <request> resource as  its Content, and the Originator can examine the <request> resource''s requestStatus attribute to check that the operation has completed and retrieve its results from the operationResult attribute.

Annex F (informative):
Guidelines for oneM2M resource type XSD

- This Annex contains rules to be followed when creating XML Schemas Definition (XSD files to represent the oneM2M resources). The XSD files themselves form part of the oneM2M protocol specification, but the rules used to construct them do not, hence this Annex is informative, although it contains normative language.

- The purpose of these rules is:

- To keep a consistent style between the schemas for different resources

- To keep the XSD simple

- To allow individual resource schemas to be authored and maintained separately, while minimising the risk of conflict when they are all used together

- Each XSD file should include a schema element with following namespace declaration:

<xs:schema xmlns:xs="http://www.w3.org/2001/XMLSchema" 	
            targetNamespace="http://www.onem2m.org/xml/protocols" 
            xmlns:m2m=http://www.onem2m.org/xml/protocols
            xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance"
            elementFormDefault="unqualified" attributeFormDefault="unqualified" >

This defines the prefix xs: for the XML Schema namespace, a target namespace http://www.onem2m.org/xml/protocols, and the prefix m2m: as equivalent for the target namespace. The xsi: namespace can be omitted if the resource has no nillable attributes (see below).
Locally declared elements and attributes shall be unqualified (elementFormDefault and attributeFormDefault declarations are not strictly required since "unqualified" is the default value setting).

- Each Resource XSD file will contain a Global Element Declaration whose name is the name of the Resource Type in accordance with ETSI TS 118 101 Functional Architecture [6] . This means that the root element of a Resource (when represented as an XML instance) contains an m2m: (or equivalent) namespace prefix. If the Resource is announceable, the XSD file will contain a second Global Element Declaration  that is used for the Announced variant of the resource. The name of that element will be formed by adding the suffix Annc to the name of the first Global Element. The XSD should not contribute anything to the m2m: namespace other than these root elements.

The root element of each resource shall have a required attribute called "resourceName" which gives an identifier for that particular resource instance. A URI to the resource instance can be constructed by taking the URI of its parent and appending /<name> where <name> is the value of the resourceName attribute.

Each resource attribute of the Resource Type in accordance with ETSI TS 118 101 [6] is represented as a child element of the top level element. It shall be declared as an element that is local to the resource that contains it, and so does not have a namespace prefix in any XML instance representation of the resource.

Each child resource shall be represented as a child element of the top level element by referring to the global element definition of the child Resource (this allows the child Resource representation to be returned inline). The resource schemas will also include – as an alternative – an element called 'childResource' which is used to return a non-hierarchical URI for the associated child resource, if this has been requested. This element shall have two attributes (in XSD) : a) type; Data type ID of instances, b) name; the name of a child resource instance.

Each Resource attribute shall be declared to use one of the following data types:

  - A data type listed in clause 6.3.2 or 6.3.3.

  - A list of one of the data types listed in clause 6.3.2 or 6.3.3. If the list type is not already included in 6.3.3 it may be defined inside the XSD file for the resource, but if so it will be defined as an anonymous type in the attribute declaration itself.

  - A data type derived by restriction from one of the types listed in clause 6.3.2 or 6.3.3. This may be added to clause 6.3.3, or defined inside the XSD file for the resource, but in the latter case it will be defined as an anonymous type in the attribute declaration itself.

  - An anonymous complex type defined as part of the attribute declaration (inside the XSD file for the resource). The complex type should only be composed out of the types listed in clause 6.3.2 or 6.3.3.

If a data type is used by more than one attribute (either in the same resource or in two different resources) it will be included in 6.3.3, and referenced by each attribute that uses it. Options 6b, 6c, 6d should only be used in cases where the type is only used by one attribute.

All Resource types will extend one of the XML complex types described in clause 6.5 and included in the file CDT-commonTypes-v1_6_0.xsd contained in ts_118004v010100p0.zip.

The resource-specific attributes and child resources shall appear as a sequence of elements in the XSD file, with their order being determined by the order shown in the tables in clause 7.4.

Each XSD file shall include an XML comment that contains a oneM2M Copyright Notification Notice of Disclaimer & Limitation of Liability, and a change history. The change history is to be filled in only after the initial release.

To enable distinction between element names used for resource attributes and their data types in the m2m: namespace, it shall be avoided to use identical names. It is recommended to use the text suffix 'Type' in data type names. Example:    	<xs:element name="status" type="m2m:statusType />

Each mgmtLink shall be represented as a child element 'mgmtLink' which is used to return a non-hierarchical URI for the associated management resource. This element has two attributes (in XSD): a) type; Data type ID of instances, b) name; the name of a child resource instance.

Annex G (normative):
Location request

# G.1	Introduction

Location Request is a means by which a CSE requests the geographical or physical location information of a target Node to the location server located in the Underlying Network over Mcn reference point. This annex describes only the case of location request when the attribute locationSource of <locationPolicy> resource type is set to Network Based. Please see clause 7.4.11.

The specific interface used for this request depends on the capabilities of the Underlying Network and other factors. This annex provides the interfaces for location request used for the communication between the CSE and the location server.

# G.2	Location request by means of OMA-REST-NetAPI-TerminalLocation interface

## G.2.1	Introduction

This OMA REST Network API for Terminal Location specification v1.0 [28] is generally used to open up service capabilities, especially location capability, in the underlying network toward applications. This clause introduces the resources structure and procedures to handle the oneM2M-specified location request. In addition, since this OMA Network API uses only HTTP as underlying message protocol, some binding mapping are mentioned in the procedures in the clause G.2.3. .

## G.2.2	Resource structure of OMA NetAPI for terminal location

When a CSE needs to request the geographical or physical location information of a target CSE or AE hosted in a M2M Node toward a location server located in the Underlying Network over Mcn reference point. The CSE shall request Terminal Location Query following Procedures for Terminal Location (see clause G.2.3. ).

The OMA REST NetAPI for Terminal Location allows CSE to obtain information about geographical location of a terminal (e.g. Node in oneM2M architecture ETSI TS 118 101 [6]). In order to obtain location information, CSE shall use one of two services of the Terminal Location API:

request the current Terminal Location in a single query toward a Location Server

subscribe to notifications of periodic Terminal Location updates.

Additionally, in order to track the terminal''s movement in relation to the geographic area (circle), crossing in and out (more detail usage is defined in the clause E of ETSI TS 118 103) it is also proposed to use a service of the Terminal Location API:

subscribe to notification of area updates

Since oneM2M system utilizes the three services mentioned above, this clause introduces the capabilities that is related to the services from OMA REST NetAPI for Terminal Location [28].

A CSE and a Node shall act as an application and a terminal respectively as described in [28].

Figure G.2.2-1: Resource Structure defined by NetAPI for Terminal Location

The two capabilities used for oneM2M system location request are 'Terminal location'. 'Periodic location notification subscriptions' and 'area notification subscriptions'. The table below describes the URL structure, data structure and mapping with CRUD operation of each resource.

Table G.2.2-3: Applicable NetAPI for Terminal Location

| Capability | URL Base URL: | Resource Type | Operations |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | C | R | U | D |
| Terminal location | /location | TerminalLocation | no | return current location of the terminal | no | no |
| Periodic location notification subscriptions | /periodic | PeriodicNotificationSubscription (used for CREATE) | create new subscription | return all subscriptions | no | No |
| Area notification subscription | /area/circle | CircleNotificationSubscription (used for CREATE) | create a new subscription | return all subscriptions | No | no |

Based on the table above, three resource types, TerminalLocation, PeriodicNotificationSubscription and CircleNotificationSubscription shall be used for the location request specified in the oneM2M system. The resource types are described in the tables below. The table also contains the relevant attributes column that is correlated with either <locationPolicy> or <accessControlPolicy> resource type defined (ETSI TS 123 003 [17]). Only attributes that may be utilized by oneM2M system are described. For the detailed information, see the [28].

Table G.2.2-4: Resource Type Definition – TerminalLocation

| Attributes | OMA NetAPI Defined Type | Description | Relevant Attribute defined by oneM2M |
| --- | --- | --- | --- |
| Address | xsd:anyURI | Address of the terminal to which the location information applies | locationTargetID in the <locationPolicy> resource type |
| locationRetrievalStatus | common:RetrievalStatus | Status of retrieval for this terminal address. | locationStatus in the <locationPolicy> resource type |
| currentLocation | LocationInfo | Location of terminal. | Content in the <contentInstance> resource type |

Table G.2.2-5: Resource Type Definition – PeriodicNotificationSubscription

| Attributes | OMA NetAPI Defined Type | Description | Relevant Attribute defined by oneM2M |
| --- | --- | --- | --- |
| address | xsd:anyURI | Addresses of terminals to monitor | locationTargetID in the <locationPolicy> resource type |
| frequency | xsd:int | Maximum frequency (in seconds) of notifications (can also be considered minimum time between notifications) per subscription. | locationUpdatePeriod in the <locationPolicy> resource type |
| duration | xsd:int | Period of time (in seconds) notifications are provided for. If set to "0" (zero), a default duration time, which is specified by the service policy, will be used. If the parameter is omitted, the notifications will continue until the maximum duration time, which is specified by the service policy, unless the notifications are stopped by deletion of subscription for notifications. | locationUpdatePeriod in the <locationPolicy> resource type |

Table G.2.2-6: Resource Type Definition – CircleNotificationSubscription

| Attributes | OMA NetAPI Defined Type | Description | Relevant Attribute defined by oneM2M |
| --- | --- | --- | --- |
| Latitude | xsd:float | Latitude of center point. | accessControlLocationRegion in the <accessControlPolicy> resource type |
| longitude | xsd:float | Longitude of center point. | accessControlLocationRegion in the <accessControlPolicy> resource type |
| Radius | xsd:float | Radius of circle around center point in meters. | accessControlLocationRegion in the <accessControlPolicy> resource type |
| checkImmediate | xsd:boolean | Check location immediately after establishing subscription. |  |

## G.2.3	Procedures for terminal location

### G.2.3.1	Request in a single query toward a location server

This procedure shows how to request and return location for a M2M Node.

Figure G.2.3.1-1: Single Query Toward Location Server

- A Hosting CSE requests location for a single terminal (Node) by means of OMA REST NetAPI for terminal location API. This request message shall contain terminal address and Request URL with the address of Location Server using RETRIEVE operation.

In this step, the TerminalLocation resource type described in Table G.2.2-3 shall be used with RETRIEVE operation.

NOTE: GET operation shall be used for this RETRIEVE operation.

- The Location Server shall retrieve the location information of the terminal.

- After the successful retrieve, the Hosting CSE receives the location information.

## G.2.4	Subscribe to notifications for periodic location updates

This procedure shows how to control subscriptions for periodic notifications about terminal location.

Figure G.2.4-1: Subscribe to Notification for Periodic Location Updates

- A Hosting CSE shall create a new periodic notification subscription for obtaining location information of a terminal periodically.

In this step, the PeriodicNotificationSubscription resource type described in Table G.2.2-3 shall be used with CREATE operation.

NOTE: POST operation shall be used for this CREATE operation.

- After the successful creation of subscription, the Hosting CSE shall receive the response.

- When the set up timer is expires, the location server shall notify the application of current location information.

In this step, the notification message shall be used as NOTIFY operation.

NOTE: Alternatively, the hosting CSE obtains the notifications using a Notification Channel [i.3]. This is repeated at specific frequency (periodic information) when the CSE is not reachable.

NOTE: POST operation shall be used for this NOTIFY operation

- After the successful receiver of notification, the Hosting CSE shall send a response to the location server.

- Based upon the location configuration change by the Hosting CSE, it updates an individual subscription for periodic location notification.

In this step, the PeriodicNotificationSubscription resource type described in the Table G.2.2-3  shall be used with UPDATE operation.

NOTE: PUT operation shall be used for this UPDATE operation.

## G.2.5	Subscribe to notifications for area updates

This procedure shows how to subscribe to area update notification.

Figure G.2.5-1: Subscribe to Notification for Area Updates

- A Hosting CSE shall create a new area notification subscription to track the terminal''s movement in relation to the geographical area (circle), crossing in and out. In this step, the CircleNotificationSubscription resource type described in the table-G.1-3 shall be used with CREATE operation.

NOTE: POST operation shall be used for this CREATE operation.

- After the successful creation of subscription, the Hosting CSE shall receive the response.

- When the target terminal crosses in or out the specified area (circle), the location server shall notify the application of current location information.

In this step, the notification message shall be used as NOTIFY operation.

NOTE: Alternatively, the hosting CSE obtains the notifications using a Notification Channel [i.3].

NOTE: POST operation shall be used for this NOTIFY operation

- After the successful receiver of notification, the Hosting CSE shall send a response to the location server.

- Based upon the location configuration change by the Hosting CSE, it updates an individual subscription for area location notification.

In this step, the CircleNotificationSubscription resource type described in the table-G.1-3 shall be used with UPDATE operation.

NOTE: PUT operation shall be used for this UPDATE operation.

Annex H (normative):
CMDH message processing

# H.1	Pre-requisites

The scope of CMDH processing is to decide at which time and via which communication path to forward request or response messages from a receiver CSE to another CSE. A number of message parameters impact the CMDH processing. CMDH-related request message parameters are:

- Event Category ('ec')

- Request Expiration Timestamp ('rqet')

- Result Expiration Timestamp ('rset')

- Operation Execution Time ('oet')

- Result Persistence ('rp')

- Originating Timestamp ('ot')

- Delivery Aggregation ('da')

CMDH-related response message parameters are:

- Event Category ('ec')

  - 'ec' is needed for response messages as well since response messages can go over multiple hops and CMDH needs to know how to handle them.

- Result Expiration Timestamp ('rset')

- Delivery Aggregation ('da')

  - When a request message was carried inside a <delivery> resource type, also the corresponding response message shall be carried in a <delivery> resource, i.e. the CSE requested to carry out an operation indicated in a request message that reached that CSE via a <delivery> resource, shall also send the response within a <delivery> resource.

The details on how those parameters impact the CMDH processing are described in the next clauses. This annex uses the short names as listed above to refer to request and response parameters.

In the following description it is assumed that the CSE behaviour for CMDH processing is governed by CMDH policies that are represented by [cmdhPolicy] resources and their child resources which are effective for the respective CSE. If legacy device management technologies are used to provision these policies, the information represented by the effective [cmdhPolicy] resources and their child resources may not be available as oneM2M defined resources on the field nodes hosting the respective CSE. This CMDH related policy information may only be available in form of managed objects specific to the used device management technology. In that case the mapping from oneM2M specified [cmdhPolicy] resources and their child resources to equivalent objects of the deployed legacy device management technology shall be used to substitute the respective information contained in [cmdhPolicy] resources and their child resources in the description below. Therefore, whenever reference to [cmdhPolicy] resources, child resources thereof or any attributes of [cmdhPolicy] resources and their children are used in the description of CMDH processing below, they shall be read as a placeholder for the equivalent objects provided by legacy device management technologies on field nodes that are provisioned with such legacy device management technologies.

For a CSE that is processing request or response messages in CMDH, exactly one set of policies represented by a [cmdhPolicy] resource shall be active, as defined by the [activeCmdhPolicy] child resource of the <node> resource that represents the node which hosts the respective CSE. In case of field nodes that are managed via legacy device management technologies, the active CMDH policy can be represented by management objects of that device management technology. For the sake of simplicity, the term 'active [cmdhPolicy]' is used in this and the following clauses to refer to the active CMDH policy information even if no oneM2M specified resources are used to represent CMDH policies. Before any provisioning of CMDH policies has occurred, the 'active [cmdhPolicy]' and its corresponding managed objects defined for legacy device management technologies shall contain the specified default values as described in the [cdmhPolicy] specific procedures and procedures specific for all its child resources. For that reason, it can be assumed that information for an 'active [cmdhPolicy]' is always present on a CMDH capable CSE.

In addition, the active [cmdhPolicy] can have at least one or more [cmdhLimits] child resources and the active [cmdhPolicy] hosting CSE shall lookup all [cmdhLimits] child resources. If the attribute requestContextNotification of any of found [cmdhLimits] resources is present and set to true, the CSE shall establish a subscription to the dynamic context information of the CSE defined in requestContext attribute of the found [cmdhLimits] as well as subscription to this [cmdhLimits] resource for all AEs corresponding to the AE-ID or an App-ID appearing in the requestOrigin attribute. The subscription(s) shall be established when the [cmdhPolicy] is provisioned or pre-provisioned and any of found [cmdhLimits] child resource has the attribute requestContextNotification that is set to true. Hence, both this policy establishment and changes of the context information and the [cmdhLimits] resource shall be notified to the respective AEs and the notification shall contain the limits for CMDH related parameter values defined in [cmdhLimits], context information and subscription reference ID. After this, the AEs which received the notification shall send only allowed 'ec' messages if 'ec' is specified by the AEs.

# H.2	CMDH processing: processing request or response messages requiring the receiver CSE to forward information to another CSE

## H.2.1	Applicability of CMDH processing

If a request or response message that is targeting an entity or a resource in the 'to' parameter that is not among any of

- the receiver CSE itself,

- an AE registered with the receiver CSE,

- a resource hosted on the receiver CSE,

and if the message is not a response message with an acknowledgement response code, the receiver CSE of that message needs to forward the message to another CSE via CMDH processing, see also the description in Clause 7.2.2. Description of Generic Procedures of the present docuement. For forwarding a message to the target CSE indicated by the 'to' parameter of the message, the receiver CSE shall determine to which CSE the message needs to be forwarded next. In the following clauses this CSE is referred to as the 'next CSE'. CMDH processing shall be carried out as described in the following clauses.

## H.2.2	Partitioning of CMDH processing

The CMDH processing consists of two parts:

- CMDH message validation: This includes message parameter pre-processing, deciding on acceptance for transporting the message, and buffering of messages.
This procedure defines how incoming request or response messages that need to be forwarded to other CSE(s) shall be pre-processed, how a decision on acceptance of the message for forwarding to another CSE shall be derived and how the messages shall be queued up before the actual forwarding can happen. Details of CMDH validation are defined in clause H.2.3.

- CMDH message forwarding: This includes selecting buffered messages and communication path for forwarding the message to another CSE.
This procedure defines how to select among the messages buffered for forwarding to other CSEs the ones that need to be transported at a certain time and how to select an appropriate communication path for transporting the message(s). Details of CMDH message forwarding are defined in clauseH.2.4.

CMDH message validation (Part A) will be carried out for each incoming new message for which CMDH processing is applicable.

If CMDH message validation is successful, the received message shall be queued up for the CMDH message forwarding process (Part B) including the associated storagePriority attribute  as defined in the applicable [cmdhBuffer] resource (see details in the CMDH message validation procedure).

If the queued message was a request message and it was done in non-blocking mode then:

- if the Receiver CSE supports the <request> resource type, it shall create a <request> resource representing the pending non-blocking request

- the Receiver CSE shall send an acknowledgement response message to the entity that sent the request message directly via Mca or Mcc to the receiver CSE indicating the acceptance of the request

- if the receiver CSE supports the <request> resource type it shall provide a reference to the created <request> resource in the 'cn' parameter of the response.

After successful forwarding of such a request message, any incoming response message matching with the Request-ID and the Originator in the <request> resource shall be parsed to update the corresponding attributes of the <request> resource. In case a non-blocking synchronous request was forwarded successfully and a response with acknowledgement was received, it is the responsibility of the CSE that forwarded the message to periodically poll the status of the <request> resource created on the next CSE and update the locally created <request> resource accordingly. When the locally created <request> resource expires the hosting CSE can remove it. Details on <request> resource specific procedures for polling results are defined in clause 7.3.1.4.

If the queued message was a request message and it was done in blocking mode then memorize the open blocking request by storing its Request-ID and Originator and set a timer for a timeout until which a matching response message with the same Request-ID and Originator shall be received by the CSE processing this message. If no matching response is received when the timeout expires, the receiver CSE shall send a response message to the entity that sent the request to the Receiver CSE indicating unsuccessful processing of the request, unless the Receiver CSE and the Originator are the same. If Receiver CSE and Originator are the same, the Originator can decide internally whether to retry forwarding of the message.

If CMDH message validation is not successful, then the received message shall either get ignored – in case the received message is a response message – or a new error response message shall be sent back to the entity that sent the message to the Receiver CSE – in case the received message is a request message and the Originator is not the Receiver CSE. If Receiver CSE and Originator are the same, the Originator can decide internally whether to create a new request message.

The CMDH message forwarding process (Part B) will handle all queued up messages that shall be forwarded to another CSE. This process shall always be carried out when messages are pending for forwarding to another CSE.

The flow of CMDH processing is depicted in Figure H.2.2-1:

Figure H.2.2-1: CMDH Processing

## H.2.3	CMDH message validation procedure

In CMDH message validation, pre-processing of CMDH related parameters of a message for which CMDH-processing applies, deriving the decision on acceptance of a message and the buffering of that messages shall be carried out in line with the following steps. A summary of this processing is depicted in the flow chart at the end of this clause.

- Filling in missing CMDH-related parameters:

  - Determine the value that shall be used for the 'ec' parameter of the processed message

    - If the message contains an 'ec' parameter: Use the value of the 'ec' parameter provided in the message.

    - If the message does not contain an 'ec' parameter:

      - Lookup all [cmdhDefEcValue] child resources of the [cmdhDefaults] resource that is a child resource of the provisioned active [cmdhPolicy] resource.

      - If the message is a request message and any of the attributes requestContext, and requestCharacteristics are present in the found [cmdhDefEcValue] resources, discard all [cmdhDefEcValue] resources from the list of found items for which the context conditions or the request characteristics at time of processing the request message are not met, respectively.

      - Among the remaining found [cmdhDefEcValue] resources do the following selection:

        - If present, select the [cmdhDefEcValue] resource containing the AE-ID in the list defined by the requestOrigin attribute which matches with the 'fr' parameter in case of a request message or with the 'to' parameter in case of a response message. If multiple [cmdhDefEcValue] resources match, select the one with the lowest value in the order attribute. Continue processing with step 1.1.2.4

        - If present, select the [cmdhDefEcValue] resource containing the App-ID in the list defined by the requestOrigin attribute which matches with the 'fr' parameter in case of a request message or with the 'to' parameter in case of a response message. If multiple [cmdhDefEcValue] resources match, select the one with the lowest value in the order attribute. Continue processing with step 1.1.2.4

        - If present, select the [cmdhDefEcValue] resource containing the string 'localAE' in the list defined by the requestOrigin attribute in case of processing a message where the 'fr' parameter is the AE-ID of an AE registered with the CSE processing this message. If multiple [cmdhDefEcValue] resources match, select the one with the lowest value in the order attribute. Continue processing with step 1.1.2.4

        - If present, select the [cmdhDefEcValue] resource containing the string 'thisCSE' in the list defined by the requestOrigin attribute in case of processing a message where the 'fr' parameter is the CSE-ID of the CSE processing this message. If multiple [cmdhDefEcValue] resources match, select the one with the lowest value in the order attribute. Continue processing with step 1.1.2.4

        - Select the [cmdhDefEcValue] resource containing the string 'default' in the list defined by the requestOrigin attribute in case of processing a message where no other matches were found.

      - If a [cmdhDefEcValue] resource has been selected in steps 1.1.2.3.1 through 1.1.2.3.4: Use the value of the defEcValue attribute of the selected [cmdhDefEcValue] resource as the value for the 'ec' parameter of the message. Else use the enumeration value of 'bestEffort' for the 'ec' parameter of the message.

  - Filling in values that shall be used for the remaining CMDH-related parameters of messages

    - If the message contains any of the CMDH-related parameters 'rqet', 'rset', 'oet', 'rp': The provided values of the respective parameters in the message shall be used. No filling in is needed for those parameters. If any of the parameters 'rqet', 'rset', 'oet', 'rp' present in the message is represented in relative time format (i.e. as a duration in units of milliseconds), the receiving CSE shall translate the values of those parameters into absolute time format by adding the duration to the originating timestamp in the 'ot' parameter of the message. This 'ot' parameter is an optional message parameter and in case it is not present in a message, it shall be filled in by the first receiving CSE of a message using the time when the message was received.

    - If the message parameter 'ec' has a value corresponding to 'bestEffort', use the following values for any missing CMDH-related parameters: For a request message use 'rqet' =  -1('infinite'), 'rset' = -1 ('infinite'), 'oet' = 0 ('now'), 'rp' = 0 ('none'), 'da' = 'True'. For a response message use 'rset' = -1 ('infinite'), 'da' = 'True'. Continue with step 2.

    - If the message parameter 'ec' has a value corresponding to 'immediate', do not fill in any remaining missing CMDH-related parameters and continue with step 2.

    - For any of the missing CMDH-related parameters fill in values as follows:

      - Lookup all [cmdhEcDefParamValues] child resources of the [cmdhDefaults] resource that is a child resource of the provisioned active [cmdhPolicy] resource.

      - Among the found [cmdhEcDefParamValues]  resources do the following selection:

        - If present, select the [cmdhEcDefParamValues] resource containing the value of the 'ec' parameter of the message in the list defined by the applicableEventCategory attribute. If a match is found, continue processing with step 1.2.4.3

        - Select the [cmdhEcDefParamValues] resource that contains the string 'default' in the list defined by the applicableEventCategory attribute.

      - Use the following attributes of the selected [cmdhEcDefParamValues] resource to fill in any missing CMDH-related message parameters: Fill in the value of the attribute defaultRequestExpTime for the parameter 'rqet' if it is missing. Fill in the value of the attribute defaultResultExpTime for the parameter 'rset' if it is missing. Fill in the value of the attribute defaultOpExecTime for the parameter 'oet' if it is missing. Fill in the value of the attribute defaultRespPersistence for the parameter 'rp' if it is missing. Fill in the value of the attribute defaultDelAggregation for the parameter 'da' if it is missing. Convert the values of 'rqet', 'rset',  'oet' and 'rp' into absolute time representations if they were filled in during this step, by adding the respective durations to the 'ot' parameter value. In case where the time duration of the default parameter indicates 'infinity', the absolute time representation of the corresponding primitive parameter shall be set to the largest possible date "99993112T000000".

- Compare CMDH parameters with allowed CMDH parameter limits:
Check if CMDH-related parameters effective for the message are with allowed limits.

  - Lookup all [cmdhLimits] child resources of the provisioned active [cmdhPolicy] resource.

  - If the message is a request message and any of the attributes requestContext, and requestCharacteristics are present in the found  [cmdhLimits] resources, discard all [cmdhLimits] resources from the list of found items for which the context conditions or the request characteristics at time of processing the request message are not met, respectively.

  - Among the remaining found [cmdhLimits] resources do the following selection:

    - If present, select the [cmdhLimits] resource(s) containing the AE-ID in the list defined by the requestOrigin attribute which matches with the 'fr' parameter in case of a request message or with the 'to' parameter in case of a response message. If multiple [cmdhLimits] resources match, select the one with the lowest value in the order attribute. Continue processing with step 2.4

    - If present, select the [cmdhLimits] resource(s) containing the App-ID in the list defined by the requestOrigin attribute which matches with the 'fr' parameter in case of a request message or with the 'to' parameter in case of a response message. If multiple [cmdhLimits] resources match, select the one with the lowest value in the order attribute. Continue processing with step 2.4

    - If present, select the [cmdhLimits] resource(s) containing the string 'localAE' in the list defined by the requestOrigin attribute in case of processing a message where the 'fr' parameter is the AE-ID of an AE registered with the CSE processing this message. If multiple [cmdhLimits] resources match, select the one with the lowest value in the order attribute. Continue processing with step 1.1.2.4

    - If present, select the [cmdhLimits] resource(s) containing the string 'thisCSE' in the list defined by the requestOrigin attribute in case of processing a message where the 'fr' parameter is the CSE-ID of the CSE processing this message. If multiple [cmdhLimits] resources match, select the one with the lowest value in the order attribute. Continue processing with step 2.4

    - Select the [cmdhLimits] resource containing the string 'default' in the list defined by the requestOrigin attribute in case of processing a message where no other matches were found.

  - Validate if 'ec' parameter is within allowed range:
If the 'ec' parameter of the message is not within the list defined by the limitsEventCategory attribute of the selected [cmdhLimits] resource, mark CMDH message validation for this message as not successful and exit CMDH message validation.

  - Validate if 'rqet' parameter is within allowed range:
If the 'rqet' parameter is present in the message and if it is not within the range defined by the 'ot' parameter and limitsRequestExpTime attribute of the selected [cmdhLimits] resource, mark CMDH message validation for this message as not successful and exit CMDH message validation.

  - Validate if 'rset' parameter is within allowed range:
If the 'rset' parameter is present in the message and if it is not within the range defined by the 'ot' parameter and limitsResultExpTime attribute of the selected [cmdhLimits] resource, mark CMDH message validation for this message as not successful and exit CMDH message validation.

  - Validate if 'oet' parameter is within allowed range:
If the 'oet' parameter is present in the message and if it is not within the range defined by the 'ot' parameter and limitsOpExecTime attribute of the selected [cmdhLimits] resource, mark CMDH message validation for this message as not successful and exit CMDH message validation.

  - Validate if 'rp' parameter is within allowed range:
If the 'rp' parameter is present in the message and if it is not within the range defined by the 'ot' parameter and limitsRespPersistence attribute of the selected [cmdhLimits] resource, mark CMDH message validation for this message as not successful and exit CMDH message validation.

  - Validate if 'da' parameter is within allowed range:
If the 'da' parameter is present in the message and if it is not within the list of allowed values defined by the limitsDelAggregation attribute of the selected [cmdhLimits] resource, mark CMDH message validation for this message as not successful and exit CMDH message validation.

- Check if message complies with network access rules and buffer limits:

  - Check if 'ec' parameter has enumeration value for 'immediate':
If the 'ec' parameter of the message is 'immediate' bypass any checks on buffering or access network usage rules. Mark the CMDH message validation for this message as successful and end CMDH message validation.

  - Check if delivering the message is possible within the boundaries of access network usage rules in CMDH policies:

    - Lookup all [cmdhNetworkAccessRules] child resources of the provisioned active [cmdhPolicy] resource.

    - Among the all found [cmdhNetworkAccessRules] resources do the following selection:

      - If present, select the [cmdhNetworkAccessRules] resource containing the value of the 'ec' parameter of the message in the list defined by the applicableEventCategory attribute. If a match is found, continue processing with step 3.2.3

      - Select the [cmdhNetworkAccessRules] resource that contains the enumeration value for 'default' in the list defined by the applicableEventCategory attribute.

    - Lookup all [cmdhNwAccessRule] child resources of the selected [cmdhNetworkAccessRules] resource

    - Among all found [cmdhNwAccessRule] resources find at least one for which the <schedule> child resource 'allowedSchedule' is allowing usage of the corresponding target network consistent with the 'rqet' parameter in case of a request message being processed or in line with the 'rset' parameter in case of a response message being processed. If no matching [cmdhNwAccessRule] resource is found, mark CMDH validation for this message as not successful due to lack of scheduling opportunities and end CMDH message validation. Otherwise continue.

  - Check if delivering the message is possible within the boundaries of buffer usage rules in CMDH policies:

    - Lookup all [cmdhBuffer] child resources of the provisioned active [cmdhPolicy] resource.

    - Among the all found [cmdhBuffer] resources do the following selection:

      - If present, select the [cmdhBuffer] resource containing the value of the 'ec' parameter of the message in the list defined by the applicableEventCategory attribute. If a match is found, continue processing with step 3.3.3

      - Select the [cmdhBuffer] resource that contains the enumeration value for 'default' in the list defined by the applicableEventCategory attribute.

    - Check if the amount of memory needed to buffer the message being validated in addition to the already buffered messages matching with the same buffer usage policy in the selected [cmdhBuffer] resource would exhaust the limit defined by the maxBufferSize attribute of the selected [cmdhBuffer] resource or if the available memory for CMDH forwarding on the receiver CSE would get exhausted even when purging buffered messages with lower storage priority.

      - If the check is negative, mark the CMDH message validation for the message being validated as successful, assign the storage priority defined in the storagePriority attribute of the selected [cmdhBuffer] resource to the validated message, and end CMDH message validation

      - If the check is positive, mark the CMDH message validation for the message being validated as not successful and end CMDH message validation.

Figure H.2.3-1: CMDH message validation procedure

## H.2.4	CMDH message forwarding procedure

The high-level sequence of processing steps for the CMDH message forwarding process is depicted in the flow chart below. Note that this flow chart only represents the reference flow for implementing a standard compliant behavior. Other standard compliant implementations may be possible as long as the events defined below will result in the same normative message exchanges via reference points.

Occurrence of the following events shall trigger processing in the CMDH message forwarding:

- One or more new message(s) get(s) queued up for CMDH message forwarding

- Any of the underlying networks becomes usable for message forwarding due to transition(s) in allowed schedule(s) or due to establishing of availability of connectivity (e.g. cable plugged-in, coverage established)

- Any of the underlying networks becomes unusable for message forwarding due to transition(s) in allowed schedule(s) or due to loss of  availability of connectivity (e.g. cable unplugged, coverage lost)

- Any message buffered for CMDH forwarding expires

Figure H.2.4-1: CMDH message forwarding procedure

When a new message is getting queued up for CMDH message forwarding, carry out the following:

If the 'ec' parameter of the messages has the value 'immediate':

Forward message as soon as possible to the next CSE. The processing in this situation is described by the flow chart in Figure H.2.4-2

  - If a Mcc communication connection to the next CSE for forwarding the message is already established, continue with step 1.3.

  - If no Mcc communication connection to the next CSE for forwarding the message is established pick one underlying network among all underlying networks that can provide communication to the next CSE and establish a Mcc communication connection to the next CSE in line with the rules outlined in clause H.2.5. If establishment of a Mcc communication connection to the next CSE was not successful before the message expires, continue with step 1.4.

  - Determine whether delivery aggregation or forwarding of the message itself shall be used:

    - If the message contains a 'da' parameter set to the value 'True', the Receiver CSE shall forward this message by creation of a <delivery> resource on the next CSE as outlined in clause 7.4.12. The receiver CSE can combine the forwarded message in the same <delivery> resource with other messages for which the 'da' parameter set to 'True' and which need to be forwarded to the same target CSE.

    - If the message is not forwarded using a <delivery> resource, the receiver CSE shall forward the message as is to the next CSE via the established Mcc communication connection.

  - If the message could not be forwarded successfully to the next CSE before it expired (e.g. due to repeated unsuccessful attempts to establish a Mcc communication connection or due to the lack of usable underlying networks), the receiver CSE shall carry out the following:

    - If the message was a response message, ignore the message. End this cycle of CMDH message forwarding and wait for new triggering events.

    - If the message was a request message:

      - If the request was a blocking request:
Send an error response to the pending blocking request with a matching Request-ID and Originator indicating the reason for failure and close the blocking request.  End this cycle of CMDH message forwarding and wait for new triggering events.

      - If the request was a non-blocking request:
Update the associated <request> resource with matching Request-ID and Originator using an error response code indicating the reason for failure.  If the non-blocking request was made in asynchronous mode, send a notification with the error response to the notification target(s) of the request. End this cycle of CMDH message forwarding and wait for new triggering events.

  - Else, i.e. if the message was forwarded successfully to the next CSE:

    - If the message was a response and the Receiver CSE has an open blocking request context with a matching Request-ID and matching Originator, mark the open blocking request as closed, end this cycle of CMDH message forwarding and wait for new triggering events.

    - If the message was a request message:

      - If the request was a blocking request:
Keep the context of the pending blocking request with matching Request-ID and matching Originator open and wait for an incoming response message with the same Request-ID and Originator.  End this cycle of CMDH message forwarding and wait for new triggering events.

    - If the request was a non-blocking request:
Wait for a response to the forwarded request (e.g. response with acknowledgement or error response). Update the associated <request> resource with the matching Request-ID and Originator using a response code that reflects the status of the forwarded request (e.g. accepted by next CSE, unsuccessful). If the next CSE responded with an error response message and the request was in non-blocking asynchronous mode, send a notification request message to the Originator of the forwarded request containing the error response of the next CSE. End this cycle of CMDH message forwarding and wait for new triggering events.

- Else, i.e. when the 'ec' parameter of the messages does not have the value corresponding to 'immediate':

    - Buffer the message to be forwarded in the CMDH forwarding buffer:
The processing in this situation is described by the flow chart in Figure H.2.4.2. If the message is a request message and the 'ec' parameter of the messages has the value corresponding to 'latest':

      - If the request message is a notification triggered by a subscription:

        - Find any buffered request message that is a notification triggered by a subscription with the same subscription reference.

      - Else, i.e. if the request message is not a notification triggered by a subscription:

        - Find any buffered request message that has the same values in the ('fr', 'to', 'op' ) parameters as the message being processed

      - If any request message was found in steps 2.1.1.1.1 or 2.1.1.2.1, purge the found message from the CMDH forwarding buffer.

    - If there is not enough memory available to buffer the message being processed in the CMDH forwarding buffer:

      - Find any buffered messages with storage priority values lower than the one assigned to the message being processed.

      - If any messages are found:
Purge enough messages among the found messages so that the message being processed can be buffered in the CMDH forwarding buffer. Messages which entered the buffer later shall be purged first. In case any request messages need to be purged, carry out the following:

        - In case of purging a non-blocking request messages:
Update the associated <request> resource with the same Request-ID as the purged request message with a status indicating unsuccessful completion. If the purged message was made in asynchronous mode, send a response to the notification target(s) of the pending non-blocking request

        - In case of purging a blocking request message:
Send an error response to the open blocking request with the same Request-ID as in the purged request message and close the blocking request.

      - Due to the checking of sufficient memory in CMDH message forwarding buffer during CMDH message validation, there should be enough memory available to accommodate the message to be buffered at this point. If that is still not the case, then do the following:

        - In case the message to be buffered is a response message:
Ignore the message to be buffered. End this cycle of CMDH message forwarding and wait for new triggering events.

        - In case the message to be buffered is a non-blocking request message:
Update the associated <request> resource with the same Request-ID as the request message to be buffered with a status indicating unsuccessful completion. If the request message to be buffered was made in asynchronous mode, send a response to the notification target(s) of the pending non-blocking request. End this cycle of CMDH message forwarding and wait for new triggering events.

        - In case the message to be buffered is a blocking request message:
Respond with an error response message to the open blocking request with the same Request-ID as in the request message to be buffered and close the blocking request. End this cycle of CMDH message forwarding and wait for new triggering events.

    - Store the message to be buffered with its assigned storage priority in the CMDH forwarding buffer. Include it in future evaluations for possible message forwarding.

  - Evaluate if any message forwarding is currently allowed:

    - For all buffered messages that are pending in CMDH message forwarding carry out the following evaluation steps:

      - Among all [cmdhNetworkAccessRules] child resources of the provisioned active [cmdhPolicy] resource do the following selection:

        - If present, select the [cmdhNetworkAccessRules] resource containing a value in the list defined by the applicableEventCategory attribute that is equal to the value of the 'ec' parameter of the buffered message to be evaluated for forwarding. If a match is found, continue processing with step 2.2.1.2.

        - Select the [cmdhNetworkAccessRules] resource that contains the string 'default' in the list defined by the applicableEventCategory attribute.

      - Lookup all [cmdhNwAccessRule] child resources of the selected [cmdhNetworkAccessRules] resource

      - If the attribute otherConditions is present in any of the found  [cmdhNwAccessRule] resources, discard all [cmdhNwAccessRule] resources from the list of found items for which the conditions expressed by otherConditions at time of evaluation of the message for forwarding are not met, respectively.

      - Among the all remaining found [cmdhNwAccessRule] resources find those for which 
   - the <schedule> child resource allowedSchedule is currently allowing usage of the corresponding target network, and
   - for which the corresponding target network could be used to reach the next CSE for forwarding the message under evaluation.
If any allowed target network was found, memorize the message under evaluation as an allowed message and the allowed target network(s) for the message under evaluation and continue with the next evaluation of buffered messages

    - When all buffered messages have been evaluated, remove from the memorized list of allowed messages and their allowed target networks those target networks where the amount of data to be forwarded – accumulated over all allowed messages of the same event category – is less than the amount of data indicated in the minReqVolume attribute of the corresponding [cmdhNwAccessRule] resource.

    - Remove any messages from the list of allowed messages for forwarding if no allowed target network is left for that message after the previous step.

  - Process messages allowed for forwarding to the next CSE:
If any messages can be forwarded, i.e. if any evaluation of step 2.2 was positive, apply the following steps:

    - Reuse already established Mcc communication connections or – if needed – establish new Mcc communication connection(s) so that all the messages that are allowed to be forwarded to their next CSE can be forwarded. Some messages may be allowed on the same target network.  Follow the procedure outlined in clause H.2.5. for setting up a Mcc communication connection to another CSE via a particular target network. If no usable Mcc communication connection could be established for forwarding a particular allowed message before the message expires, execute step 1.4 in this clause above for that message.

    - For all messages allowed for forwarding and for which Mcc communication connections are established, apply steps 1.3 through 1.5 in this clause above.

  - Else, i.e. currently no message forwarding is allowed:
End this cycle of CMDH message forwarding and wait for new triggering events.

When any of the underlying networks becomes usable for message forwarding due to transition(s) in allowed schedule(s) or due to establishing of availability of connectivity (e.g. cable plugged-in, coverage established), carry out the processing above in this clause starting with step 2.2.

When any of the underlying networks becomes unusable for message forwarding due to transition(s) in allowed schedule(s) or due to loss of availability of connectivity (e.g. cable unplugged, coverage lost), complete – if at all possible – any ongoing message forwarding procedures. End this cycle of CMDH message forwarding and wait for new triggering events.

When any message buffered for CMDH forwarding expires, carry out step 1.4 in this clause above. End this cycle of CMDH message forwarding and wait for new triggering events.

Figure H.2.4-2: Forwarding of messages with 'ec' = 'immediate'.

Figure H.2.4-3: Buffering of messages for CMDH message forwarding.

## H.2.5	Establishment of Mcc communication connection to another CSE

When a Mcc communication connection shall be established via a specific target network for forwarding a message of a specific event category indicated by the 'ec' parameter of the message, the process of establishing the Mcc communication connection shall be governed by values contained in the backOffParameters attribute of the [cmdhNwAccessRule] resource that was used to evaluate whether the message was allowed to be forwarded, as defined in step 2.2 in the procedure outlined in clause H.2.4.

When connectivity via the selected target network to reach the next CSE has not already been established for other reasons, then the CSE that is trying to forward a message buffered for CMDH message forwarding shall establish a new Mcc communication connection via the selected target network for transporting oneM2M messages to the next CSE via a new Mcc instance. This communication connection shall be established following the procedures for authentication and security association using TLS or DTLS as defined in the ETSI TS 118 103 Security Solutions [7] taking into account provisioned security settings. The protocol mapping for transporting oneM2M specified messages via this instance of Mcc shall be selected according to the capabilities of the two end-points of the Mcc instance.

If establishing the Mcc communication connection via the selected target network fails, a new attempt to establish that communication connection shall only be made after waiting for a back-off time according to the value given in the 'back-off time' component of the backOffParameters attribute of the effective [cmdhNwAccessRule] resource.

When establishing the Mcc communication connection via the selected target network still fails, for each subsequent new attempt to establish the Mcc communication connection without any successful attempts in-between, the back-off time shall be increased by the value given in the 'back-off time increment' component of the backOffParameters attribute of the effective [cmdhNwAccessRule] resource.

The back-off time for waiting before making any new attempt to establish the Mcc communication connection via the selected target network shall not exceed the value given by the 'maximum back-off time' component of the backOffParameters attribute of the effective [cmdhNwAccessRule] resource.

When the next CSE is hosted on a node for which a usable Mcc communication connection for forwarding a message to the next CSE can only be established by the next CSE itself, device triggering mechanisms as defined in the ETSI TS 118 101 [6] shall be used.

In case the next CSE can only be reached via communication connections originating from the node that hosts the next CSE, while it is capable of processing incoming oneM2M messages, it is assumed that such a CSE establishes a polling channel as defined in the ETSI TS 118 101[6] in order to effectively receive unsolicited oneM2M messages.

Annex I (informative):
Guidelines for using XSD files in AE and CSE code

# I.1	Usage of the oneM2M developed XSD files

The primary purpose of the XSD files developed by oneM2M is described in clause 6.1. This informative Annex provides an example of potential usage of the XSD in practical implementations of oneM2M entities (AE and CSE).

As has been specified in clause 8, to enable efficient communication, the short names introduced in clause 8.2 shall be applied in XML and JSON serializations of request and response primitives to identify primitive parameters, and to identify resource names, resource attribute names and their complex data type members when included in the Content primitive parameter. This implies that short names are applied in any communication over the Mca, Mcc and Mcc' reference points. Nevertheless, the XSD files included in the present release employ the long names for primitive parameters and any other XML elements and attributes.

This annex provides a possible use case of the oneM2M developed XSD files for information.

# I.2	Example AE/CSE implementation featuring mapping between short and long names for XML serialization

Figure I.2-1 shows an example where the oneM2M defined XSD files are used as input to a code generator. Such code generators are available for most object-oriented programming languages such as e.g. Java, C++  and Python. The following descriptions include some code examples given in Python syntax. However, corresponding expressions in C++ or Java look very similar.

Code generators generate a library of XSD binding classes corresponding to each of the data types defined in the input XSD files. This library can then be imported into the source code of the respective programming language which implements an AE or CSE.

For example, if this library is denoted schemaLib, instances of a request primitive and of a resource type <contentInstance>, denoted in the Python source code fragment below as reqPrimInstance (internal representation of m2m:requestPrimitive) and contentInstance1 (as internal representation of m2m:contentInstance), respectively, can simply be generated as follows:

import schemaLib

…

reqPrimInstance = schemaLib.requestPrimitive()

contentInstance1 = schemaLib.contentInstance()

Each of the instances created in this way represents a data object reflecting the same tree structure as defined in the XSD files that served as input to the code generator.

Any request primitive parameter in reqPrimInstance as defined above can be addressed and assigned values as follows:

reqPrimInstance.operation = operation     #e.g. operation = 1 for CREATE

reqPrimInstance.to = path                 #path = address of target resource

reqPrimInstance.from_ = originator  #originator=identifier representing the originator

reqPrimInstance.requestIdentifier = str(requestIDCounter) #counter in string format

reqPrimInstance.resourceType = resourceType #e.g. resourceType = 4 for <contentInstance>

Parameters defined as complex type in the XSD such as e.g. the Filter Criteria primitive parameter can be assigned values as follows:

reqPrimInstance.filterCriteria.createdBefore = '20161201T000000'

reqPrimInstance.filterCriteria.createdAfter = '20150501T123000'

reqPrimInstance.filterCriteria.labels = 'label1 label2 label3'

reqPrimInstance.filterCriteria.attribute.append(pyxb.BIND())

reqPrimInstance.filterCriteria.attribute[0] = schemaLib.attribute("name0","value0")

Note that the class attribute names in the source code are identical with the XML element or attribute names as used in the XSD files  (sometimes minor exceptions can occur, for instance in case that a name used in the XSD represents a reserved name in the source code. In such case the code generator typically would append a special suffix to the name, e.g. "_").  Since the XSD uses the long parameter and resource attribute names, these also appear as class attributes in the source code.  From an implementation perspective, this is preferable compared to using short names. Using short names in the XSD would result in short names in the source code. However, these short names have essentially lost their semantics and are therefore more difficult do memorize. Any misspelling in the code may easily result in another well-defined short name such that identifying errors in the source code becomes more difficult.

A code generator as considered here, typically also provides a set of class methods and utility functions which allow to generate code objects from a given XML representation, and inversely, to generate XML representations from a code object. For example, consider that a string variable, denote reqPrimXML represents a serialized request primitive as follows (note that this representation corresponds to the example given in clause 8.3.2 but with long names used here):

reqPrimXML =

'<?xml version="1.0" encoding="UTF-8"?>
     <m2m:requestPrimitive xmlns:m2m="http://www.onem2m.org/xml/protocols"
      >
        <operation>1</operation>
        <to>//cse1.mym2msp.org/cse8976/container123</to>
        <from>//cse1234/app567</from>
        <requestIdentifier>0002bf63</requestIdentifier >
        <resourceType>4</resourceType>
        <primitiveContent>
            <contentInstance>
               <contentInfo>application/xml:1</contentInfo>
               <content>PHRpbWU+MTc4ODkzMDk8L3RpbWU+PHRlbXA+MjA8L3RlbXA+DQo=</content>
            </contentInstance>
        </ primitiveContent>
    </m2m:requestPrimitive>'

Assuming that the auto-generated library schemaLib includes a utility function createFromDocument(), the following code statement creates an instance reqPrimInstance from the XML serialized request primitive in the string variable reqPrimXML:

reqPrimInstance = schemaLib.createFromDocument(reqPrimXML)

The root element of the XML string (i.e. m2m:requestPrimitive in this example) identifies the template (class) that need to be used to create the data object reqPrimInstance. All value settings of the parameters are taken from the XML string, e.g. reqPrimInstance.operation is set to 1.

The reverse operation, i.e. generation of an XML string from the data object reqPrimInstance is typically possible with a class method toxml() as follows:

reqPrimXML = reqPrimInstance.toxml()

If any value settings of reqPrimInstance have not been changed in the given code, the above statement generates the same XML string as given above. Both operations, createFromDocument() and toxml(), also allow to verify the compliance of the XML representations with the XSD that was used as input when generating the schemaLib source code.

The question arises, if there is a way to generate XML or JSON representations that include the short names as defined in clause 8.2 when employing XSD with the long names as described above.

The following outlines two possible ways to resolve this issue.

The first straightforward approach is to use a text parser which replaces the long names used in XML or JSON strings with their corresponding short names, or vice-versa. We denote such functions as map_L2S() and map_S2L(). This approach is illustrated in the box labelled "AE or CSE source code" in Figure I.2-1 for an XML serialized string.

Given a string reqPrimXML representing an XML serialized request primitive with long names as described above, the statement

reqPrimXML_sh = map_L2S(reqPrimXML)

would produce an XML string that includes the short names as shown in the representation already given in clause 8.3.2.

The reverse operation, generating an XML representation with long names from a representation with short names could be done with

reqPrimXML = map_S2L(reqPrimXML_sh)

Both mapping functions require a mapping table which includes all long names and their associated short names. The required mapping table can be derived from Tables 8.2.2-1, 8.2.2-2, 8.2.3-1 to 8.2.3-5, 8.2.4-1 and 8.2.5-1.

In order to work in both mapping directions, the mapping table must represent a one-to-one relationship between short and long names.

The second approach is essentially a code-optimized variant of the above first approach.

The source code of the described   createFromDocument() and toxml() functions could be extended by the programmer by including the functionality of map_S2L() directly into createFromDocument() and including the functionality of map_L2S() directly into toxml(). An additional function argument could be included which allows to enable and disable the mapping function.

Set of

XSD files

Code

Generator

AE or CSE source code

library of XSD binding classes and functions

map_S2L()

map_L2S()

createFromDocument()ent

toxml()

processing

objects

with long names

messages including

serialized XML with short names

messages including

serialized XML with short names

Figure I.2-1: Example AE or CSE implementation: processing based on long names, XML representations using short names

# I.3	Example AE/CSE implementation featuring mapping between short and long names for JSON serialization

Figure I.3-1 shows an example implementation which employs JSON serialization. The core of this example implementation is identical with the one described above for XML serialization. In the example it is assumed that for producing a JSON representation which is valid against its associated XSD, an XML file is generated first by means of the toxml() function described in clause I.2 above. In this case the mapping from long to short names can be accomplished also with the map_L2S() function used in the XML serialization example. This XML file can then be converted into a structured data representation that allows direct conversion into JSON. When using Python programming language, the most suitable representation is the dictionary format. In Figure I.3-1, the function denoted as xml2dict(), generates a Python dictionary object which in the final operation step is serialized into the XSD-valid JSON representation by means of the json.dumps() function.  In order to comply with the requirements for the JSON representation as defined in clause 8.4, it is necessary to adjust the data type of numeric and list-type elements.

At the receiving side of the described implementation example, received JSON data is converted into a Python dictionary object by means of the json.loads() function. This dictionary object is unparsed by means of a function  denoted dict.unparse() in Figure I.3-1 which generates directly an instance of the class applicable to the received data which is defined in SchemaLib. During the unparse operation, the mapping is accomplished between the short names included in the received JSON data object and the long names employed in the class definition included in SchemaLib. The unparse operation also implements validation of the compliance of the received JSON data with the XSD.

JSON

with

short

names

Code

Generator

AE or CSE source code

json.loads()

map_L2S()

dict.unparse()

toxml()

processing

objects

with long names

xml2dict()

json.dumps()

JSON

with

short

names

Set of

XSD files

library of XSD binding classes and functions

Figure I.3-1: Example AE or CSE implementation with processing based on long names

# List of tables and figures

Figure 5.4.1-1: Communication model using Request and Response primitives over an IP-based Underlying Network	20

Figure 5.4.2-1: Primitives modelling	20

Figure 6.3.4.1-1: Example of XSD version of oneM2M Enumeration Type	32

Figure 6.5.1-1: Resource Types	56

Figure 7.2.2.1-1: Generic procedure of Originator	66

Figure 7.2.2.2-1: Generic procedure of Receiver	68

Figure 6.3.4.2.31-2: Resource handling procedure	70

Figure E.1-1: Blocking access to resource	203

Figure E.2.2-1: non-Blocking access to resource in synchronous mode (no hop)	205

Figure E.2.2-2: non-Blocking access to resource in synchronous mode (one hop)	207

Figure G.2.2-1: Resource Structure defined by NetAPI for Terminal Location	212

Figure G.2.3.1-1: Single Query Toward Location Server	214

Figure G.2.4-1: Subscribe to Notification for Periodic Location Updates	215

Figure G.2.5-1: Subscribe to Notification for Area Updates	216

Figure H.2.2-1: CMDH Processing	220

Figure H.2.3-1: CMDH message validation procedure	224

Figure H.2.4-1: CMDH message forwarding procedure	225

# History

| Document history |  |  |
| --- | --- | --- |
| V1.0.0 | February 2015 | Publication |
| V1.1.0 | March 2016 | Publication |