	Doc# PRO-2016-0271-fanout_response_aggregation(R2).doc
Change Request
	[image: image1.png]






	


	CHANGE REQUEST

	Meeting:*
	PRO#23.6

	Source:*
	KETI(Korea Electronics Technology Institute)

	Date:*
	2016-07-08

	Contact:*
	SeungMyeong JEONG, KETI, sm.jeong@keti.re.kr
Jaeho Kim, KETI, jhkim@keti.re.kr

Jaeseok Yun, KETI, jaeseok@keti.re.kr

	Reason for Change/s:*
	See the introduction 

	CR  against:  Release*
	Release 2

	CR  against:  WI*
	 FORMCHECKBOX 
 Active <Work Item number>  

 FORMCHECKBOX 
 MNT maintenace / < Work Item number(optional)>
 FORMCHECKBOX 
 STE Small Technical Enhancements / < Work Item number (optional)>
Only ONE of the above shall be ticked

	CR  against:  TS/TR*
	TS-0004 v2.6.0

	Clauses/Sub Clauses*
	6.2.4 oneM2M resource addressing

	Type of change: *
	 FORMCHECKBOX 
 Editorial change
 FORMCHECKBOX 
 Bug Fix or Correction
 Change to existing feature or functionality
 New feature or functionality
Only ONE of the above shall be ticked

	Post Freeze checking:*
	This CR contains only essential changes and corrections?  YES  FORMCHECKBOX 
  NO  FORMCHECKBOX 

This CR may break backwards compatibility with the last approved version of the TS?       YES  FORMCHECKBOX 
  NO  FORMCHECKBOX 

This CR is a mirror CR? YES  FORMCHECKBOX 
  if YES, please indicate the document number of the original CR: PRO-2016-0270-fanout_response_aggregation(R1) : NO  FORMCHECKBOX 
  

	Template Version:27 May 2015 (Dot not modify)


oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.
In case of a correction, and the change apply to previous releases, a separated “mirror CR” should be posted at the same time of this CR
Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.
Follow the drafting rules.
All pictures must be editable.
Check spelling and grammar to the extent practicable.
Use Change bars for modifications.
The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete sections need not show surrounding clauses as long as the proposed section number clearly shows where the new section is proposed to be located.
Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.
When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text. 
Introduction
1. added member resource ID information into each member response when the group host aggregates them. the reason is also written.

2. added error handling when there’s no response received by Hosting CSE in time.

3. suggested to relocate MEMBER_TYPE_INCONSISTENT from network error category to Originator error category, since it is obviously the Originator’s fault.

Once we reach an agreement for RSC update, then respective protocol binding CRs shall be followed.
-----------------------Start of change 1-------------------------------------------
7.3.1.1 <fanOutPoint> operations

7.4.14.2.5 Aggregation of member responses
After receiving the member responses from the member hosting CSEs, the group hosting CSE shall respond to the Originator with an aggregated response. To indicate which response is generated by which member resource, the Hosting CSE shall add member resource ID, which is corresponding to the response, into From response parameter in each member response.

If the group Hosting CSE gets no response before the Result Expiration Timestamp expiray, then the Hosting CSE shall return error with the Response Status Code parameter set as “MEMBERS_NOT_RESPONDED”.

If Response Type, Result Expiration Time or Result Persistence were set in the request, these affect the behaviour of the group hosting CSE as follows:

If Response Type is set to blockingRequest, the group hosting CSE shall respond only once with the aggregated response. It shall do this before the time indicated by the Result Expiration Time is reached. The group hosting CSE shall discard any member responses received after this time.

If Response Type is set to nonBlockingRequestSynch, the group hosting CSE shall create a <request> resource locally and respond the Originator with the address of this <request> resource. Until the Result Expirtation Time is reached, the group hosting CSE shall aggregate the member responses and include this aggregated response in the operationResult of the <request> resource. 
If Response Type is set to nonBlockingRequestAsynch, the group hosting CSE shall notify the Originator or the notification targets with  aggregated responses before the Result Expiration Time expires. The group hosting CSE may notify the Originator more than once during the period until the Result Expiration Time expires. Each notification shall contain different member responses.
If Response Type is set to flexBlocking, the group hosting CSE shall keep aggregating the member responses until the group hosting CSE determines that it is time to send a response – this depends on the properties of the group hosting CSE related to the <group> resource (the number of aggregated responses or the time period of the aggregation). By that time, if the aggregated response contains all the member responses, the group hosting CSE shall respond with the aggregated response. However if only some of the member responses have been received , the group hosting CSE shall create a <request> resource from the received request, and respond to the Originator with the reference to the created <request> resource as well as the currently aggregated responses.  Until the time specified in Result Expiration Time is reached, the group hosting CSE shall keep aggregating the remaining member responses and updating the aggregated response in the operationResult of the <request> resource. If notificationTarget is provided in the request, the group hosting CSE shall notify the Originator with the aggregated response. Each notification shall contain different member responses.

If the group hosting CSE supports <request> resource, in the nonBlockingRequestAsynch, nonBlockingSynch and flexBlocking case, it shall set the requestStatus of the <request> resource to PARTIALLY_COMPLETED if some of the member responses are received.  If the group hosting CSE has aggregated all the member responses, it shall set the operationResult to COMPLETED.

In any of the cases above, member responses received after the Result Expiration Time shall be discarded. After the time specified in Result Persistence, the aggregated response shall not be retrievable.
If any of the parameter mentioned above are missing from the request, the group hosting CSE shall determine the time to respond using its local Policy.
-----------------------End of change 1---------------------------------------------

-----------------------Start of change 2-------------------------------------------

6.4.1.1 Originator Error response class

Table 6.6.3.5-1 specify the RSCs for Originator Error responses.
Table 6.6.3.5‑1: RSCs for Originator Error response class

	Numeric Code
	Description

	4000
	BAD_REQUEST

	4004
	NOT_FOUND

	4005
	OPERATION_NOT_ALLOWED

	4008
	REQUEST_TIMEOUT

	4101
	SUBSCRIPTION_CREATOR_HAS_NO_PRIVILEGE

	4102
	CONTENTS_UNACCEPTABLE

	4103
	ACCESS_DENIED

	4104
	GROUP_REQUEST_IDENTIFIER_EXISTS

	4105
	CONFLICT

	x
	MEMBER_TYPE_INCONSISTENT


6.4.1.2 Receiver Error response class

Table 6.6.3.6-1 specifies the RSCs for Receiver Error  responses.
Table 6.6.3.6‑1: RSCs for Receiver Error response class

	Numeric Code
	Description

	5000
	INTERNAL_SERVER_ERROR

	5001
	NOT_IMPLEMENTED

	5103
	TARGET_NOT_REACHABLE

	5105
	NO_PRIVILEGE

	5106
	ALREADY_EXISTS

	5203
	TARGET_NOT_SUBSCRIBABLE

	5204
	SUBSCRIPTION_VERIFICATION_INITIATION_FAILED

	5205
	SUBSCRIPTION_HOST_HAS_NO_PRIVILEGE

	5206
	NON_BLOCKING_REQUEST_NOT_SUPPORTED

	5207
	NOT_ACCEPTABLE

	5208
	DISCOVERY_DENIED_BY_IPE

	y
	MEMBERS_NOT_RESPONDED


6.4.1.3 Network System Error response class

Table 6.6.3.7-1 specifies the RSCs for when the External System reported some errors.

Table 6.6.3.7‑1: RSCs for Network Service Error response class

	Numeric Code
	Description

	6003
	EXTERNAL_OBJECT_NOT_REACHABLE

	6005
	EXTERNAL_OBJECT_NOT_FOUND

	6010
	MAX_NUMBER_OF_MEMBER_EXCEEDED

	
	

	6020
	MGMT_SESSION_CANNOT_BE_ESTABLISHED

	6021
	MGMT_SESSION_ESTABLISHMENT_TIMEOUT

	6022
	INVALID_CMDTYPE

	6023
	INVALID_ARGUMENTS

	6024
	INSUFFICIENT_ARGUMENTS

	6025
	MGMT_CONVERSION_ERROR

	6026
	MGMT_CANCELLATION_FAILED

	6028
	ALREADY_COMPLETE

	6029
	MGMT_COMMAND_NOT_CANCELLABLE


-----------------------End of change 2---------------------------------------------

CHECK LIST

· Does this change request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?
· Does this CR contain changes related to only one particular issue/problem?
· Have any mirror crs been posted?
· Does this change request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable?
· Does this change request follow the drafting rules?
· Are all pictures editable?
· Have you checked the spelling and grammar?
· Have you used change bars for all modifications?
· Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete sections need not show surrounding clauses as long as the proposed section number clearly shows where the new section is proposed to be located.)
· Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?
© 2016 oneM2M Partners
                                                                                                   Page 1 (of 5)



[image: image1.png]