	Doc# PRO-2017-0103R01 MQTT Sending a Request on Rel-2
Change Request
	[image: image1.png]






	


	CHANGE REQUEST

	Meeting ID:*
	PRO 29

	Source:*
	Francisco Sang-Eon Kim, KT, kim.sangeon@kt.com

	Date:*
	2017-05-12

	Reason for Change/s:*
	Developers may have difficult to understand specification.

	CR  against:  Release*
	Release 2

	CR  against:  WI*
	 FORMCHECKBOX 
 Active <Work Item number>  

 FORMCHECKBOX 
 MNT maintenance / WI-0049
Is this a mirror CR? Yes  FORMCHECKBOX 
 No  FORMCHECKBOX 

mirror CR number: 
 FORMCHECKBOX 
 STE Small Technical Enhancements / < Work Item number (optional)>
Only ONE of the above shall be ticked

	CR  against:  TS/TR*
	TS-0010-V2.5.0

	Clauses *
	6.4.2 Sending a Request

	Type of change: *
	 FORMCHECKBOX 
 Editorial change

 FORMCHECKBOX 
 Bug Fix or Correction

 Change to existing feature or functionality

 New feature or functionality
Only ONE of the above shall be ticked

	Impacted other TS/TR(s)
	<TS/TR number>, <Version Number>, and <Description on which aspect should be reflected in this TS/TR>

	Post Freeze checking:*
	This CR contains only essential changes and corrections?  YES  FORMCHECKBOX 
  NO  FORMCHECKBOX 

This CR may break backwards compatibility with the last approved version of the TS?       YES 
  NO 


	Template Version: January 2017 (Do not modify)


oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.
In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR
Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).
Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.
Follow the drafting rules.
All pictures must be editable.
Check spelling and grammar to the extent practicable.
Use Change bars for modifications.
The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.
Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.
When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text. 
Introduction
This contribution proposes to use formal term of ‘MQTT PUBLISH Packet’ as specifiec in OASIS MQTT Version 3.1.1  instead of MQTT PUBLISH protocol packet, 

The Consolas font means ‘name of MQTT fields’.

The contents of this clause is about Topic Name, Therefore, this contribution proposes to change a title to Topic Name for oneM2M Originator.

TS-0004 specifies serialization for data format of message body: XML, JSON, CBOR at clause 8.3, 8.4 and 8.5 respectively. 

-----------------------Start of change 1-------------------------------------------
6.4.2
Topic Name for Requests
A request is transmitted by sending it as an MQTT PUBLISH Packet to the MQTT Server. The MQTT PUBLISH Packet  uses a Topic Name that identifies both the Originator and the Receiver of the request as follows:

· oneM2M/req/<originator>/<receiver>/<type>

· "oneM2M" is a literal string identifying the topic as being used by oneM2M.

· <originator> is the SP-relative-AE-ID or SP-relative-CSE-ID of the entity that sends the request on this hop, omitting any leading "/"’s and replacing any other "/" characters with ":" characters.

· <receiver> is the SP-relative-AE-ID or SP-relative-CSE-ID of the Receiver  (AE, Transit CSE or Hosting CSE) on this hop, omitting any leading "/"’s and replacing any other "/" characters with ":" characters.

· "req" is a literal string identifying this as a request.

· <type> is "xml" or "json" or "cbor" indicating the MQTT payload data type as described in clause 6.5.4.


-----------------------End of change 1---------------------------------------------

CHECK LIST

· Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?
· Does this CR contain changes related to only one particular issue/problem?
· Have any mirror CRs been posted?
· Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?
· Are all pictures editable?
· Have you checked the spelling and grammar?
· Have you used change bars for all modifications?
· Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)
· Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?
© 2017 oneM2M Partners
                                                                                                   Page 3 (of 3)



[image: image1.png]