	Doc# PRO-2017-0119-ack_content_for_non-blocking (rel-2).doc
Change Request
	[image: image1.png]






	


	CHANGE REQUEST

	Meeting ID:*
	PRO#29

	Source:*
	SeungMyeong JEONG, KETI, sm.jeong@keti.re.kr 
Il-Yeup Ahn, KETI, iyahn@keti.re.kr

	Date:*
	2017-05-18

	Reason for Change/s:*
	To clarify that non-blocking “sync” mode also sends an address of <request> resource in ACK

	CR  against:  Release*
	2

	CR  against:  WI*
	 FORMCHECKBOX 
 Active <Work Item number>  

 FORMCHECKBOX 
 MNT maintenance / < Work Item number(optional)>
Is this a mirror CR? Yes  FORMCHECKBOX 
 No  FORMCHECKBOX 

mirror CR number: (Note to Rapporteur - use latest agreed revision)
 FORMCHECKBOX 
 STE Small Technical Enhancements / < Work Item number (optional)>
Only ONE of the above shall be ticked

	CR  against:  TS/TR*
	TS-0004 

	Clauses *
	7.5.2

	Type of change: *
	 FORMCHECKBOX 
 Editorial change
 FORMCHECKBOX 
 Bug Fix or Correction
 Change to existing feature or functionality
 New feature or functionality
Only ONE of the above shall be ticked

	Impacted other TS/TR(s)
	

	Post Freeze checking:*
	This CR contains only essential changes and corrections?  YES  FORMCHECKBOX 
  NO  FORMCHECKBOX 

This CR may break backwards compatibility with the last approved version of the TS?       YES 
  NO 

	Template Version: January 2017 (Do not modify)


oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.
In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR
Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).
Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.
Follow the drafting rules.
All pictures must be editable.
Check spelling and grammar to the extent practicable.
Use Change bars for modifications.
The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.
Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.
When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text. 
Introduction
In the generic procedure to create ACK, when a Receiver CSE supports <request> resource, the Content response parameter shall contain the resource address of the <request> resource. It does not matter whether the request was non-blocking synch or asynch mode.
See the below for ACK generating procedure.
7.3.2.3 Create a success response (acknowledgement) 
The Receiver CSE shall create a Response primitive. The Receiver CSE shall include the following parameters in the Response primitive.

Table 7.3.2.3‑1: Response primitive parameter settings
	Parameter Name
	Value 

	Response Status Code
	"ACCEPTED"

	Request Identifier
	The value of the parameter Request Identifier in the associated non-blocking request.

	Originating Timestamp
	Timestamp when this message was built

	Content
	Reference to the <request> of the associated non-blocking request only if <request> resource is supported.


This CR proposes to add missing non-blcking “synch” mode in the table of response Content parameter definition.

NOTE: once this contribution becomes agreeable, then the mirrors will be prepared to get approvals for them all.
-----------------------Start of change 1-------------------------------------------
7.5.2 Elements contained in the Content primitive parameter

Clauses 7.2.1.1 and 7.2.1.2 enumerate the forms that the Content primitive parameter takes in various Request and Response cases. Note that the Content primitive parameter is denoted as primitiveContent in both, CDT-requestPrimitive-v2_11_0.xsd and CDT-responsePrimitive-v2_11_0.xsd.

This clause details the Objects (elements) used in some of these cases. In the tables below.

The following elements are defined for use in the Content parameter of a request:

Table 7.5.2‑1: Elements used for request content
	Element Name
	Applicable Operations
	Data Type
	Defined in 

	m2m:<resourceType>

{other namespace identifier}:<resourceType>
	C U
	m2m:<resourceType>

{other namespace identifier}:<resourceType>
	CDT-<resourceType>-v2_11_0.xsd

	m2m:notification
	N
	m2m:notification
	CDT-notification-v2_11_0.xsd

	m2m:aggregatedNotification
	N
	m2m:aggregatedNotification
	CDT-notification-v2_11_0.xsd

	m2m:securityInfo
	N
	m2m:securityInfo
	CDT-notification-v2_11_0.xsd

	m2m:attributeList
	R
	m2m:attributeList
	CDT-requestPrimitive-v2_11_0.xsd

	m2m:responsePrimitive
	N
	m2m:responsePrimitive
	CDT-responsePrimitive-v2_11_0.xsd


The following elements are defined for use in the Content parameter of a response sent in reply to a request message with Operation and Result Content (rcn) parameters as given in the column “Applicable Operations” (the settings of the Result Content parameters are defined in clause 6.3.4.2.7; NP means the rcn parameter is not present).
Table 7.5.2‑2: Elements used for response content
	Element Name
	Applicable Operations/rcn
	Data Type
	Element is Defined in 

	m2m:<resourceType>

{other namespace identifier}:<resourceType>

NOTE 6
	C/1,9,NP

R/1,4,5,6,7,8,NP

U/1,9,NP

D/1

See NOTE1
	m2m:<resourceType>

{other namespace identifier}:<resourceType>
	CDT-<resourceType>-v2_11_0.xsd

	m2m:resource
	C/3
	m2m:resourceWrapper
	CDT-responsePrimitive-v2_11_0.xsd

	m2m:URIList
	R/NP

See NOTE 2
	m2m:listOfURIs
	CDT-responsePrimitive-v2_11_0.xsd

	m2m:resourceRefList
	R/6

See NOTE 2
	m2m:listOfChildResourceRef 
	CDT-responsePrimitive-v2_11_0.xsd

	m2m:aggregatedResponse
	C R U D

See NOTE 3
	m2m:aggregatedResponse
	CDT-responsePrimitive-v2_11_0.xsd

	m2m:URI
	C/2

See NOTE 4
	xs:anyURI
	CDT-responsePrimitive-v2_11_0.xsd

	m2m:debugInfo
	See NOTE 5
	xs:string
	CDT-responsePrimitive-v2_11_0.xsd

	m2m:securityInfo
	N/NP
	m2m:securityInfo
	CDT-notification-v2_11_0.xsd

	NOTE 1:
The case rcn = 7 applies to Retrieve operation only (R/7). It retrieves the original resource in case the To parameter points to an announced resource. The case R/NP applies to Retrieve operation (Non-Discovery) only.

NOTE 2:
This applies to discovery operation only. For discovery, the format of the address (structured, unstructured) depends on the Discovery Result Type parameter setting (see clause 6.3.4.2.8).

NOTE 3:
This applies to CRUD operations on a <fanOutPoint> child resource of a <group> parent resource. The Content parameter of each response primitive included in aggregatedResponse is set as given in one of the other rows of this table.

NOTE 4:
This also applies to the response ("acknowledgement") to non-blocking requests in asynchronous and synchronous modes for any CRUD operation. 
NOTE 5:
This is a plain text messages which can optionally be included as debugging information in error responses. The language and content of the message is determined by the Service Provider.

NOTE 6:
"{other namespace identifier}" refers to a namespace other than m2m.


The XML schema definition of the Content primitive parameter (i.e. datatype m2m:primitiveContent) allows to include XML wildcard elements. An XML representation of the Content primitive parameter shall include a root element which is associated with an XSD Global Element. The root element shall be prefixed with a namespace prefix identifier (e.g. m2m:) specified in the associated XSD which defines the respective Global Element. The Content primitive parameter allows to include namespaces other than m2m.
-----------------------End of change 1---------------------------------------------

CHECK LIST

· Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?
· Does this CR contain changes related to only one particular issue/problem?
· Have any mirror CRs been posted?
· Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?
· Are all pictures editable?
· Have you checked the spelling and grammar?
· Have you used change bars for all modifications?
· Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)
· Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?
© 2017 oneM2M Partners
                                                                                                   Page 1 (of 6)



[image: image1.png]