	Doc# ARC-2017-0229-Resource de-announcement correction.doc
Change Request
	[image: image1.png]






	


	CHANGE REQUEST

	Meeting ID:*
	PRO 31

	Source:*
	Hao Wu, ZTE, wu.hao18@zte.com.cn
Kun Yang, ZTE, yang.kun15@zte.com.cn
Jingting Hu, ZTE, hu.jingting@zte.com.cn

	Date:*
	2017-09-18

	Reason for Change/s:*
	Correct the attribute de-announcement procedure in section 7.3.3.11.

	CR  against:  Release*
	Release2

	CR  against:  WI*
	 FORMCHECKBOX 
 Active <Work Item number>  

FORMCHECKBOX
 MNT maintenance / < Work Item number(optional)>

Is this a mirror CR? Yes  FORMCHECKBOX 
 No  FORMCHECKBOX 

mirror CR number: PRO-2017-0228
FORMCHECKBOX
 STE Small Technical Enhancements / < Work Item number (optional)>
Only ONE of the above shall be ticked

	CR  against:  TS/TR*
	TS-0004 V3.3.0

	Clauses *
	7.3.3.11 De-announce the resource or attribute

	Type of change: *
	 FORMCHECKBOX 
 Editorial change

FORMCHECKBOX
 Bug Fix or Correction

FORMCHECKBOX
 Change to existing feature or functionality

 FORMCHECKBOX 
 New feature or functionality
Only ONE of the above shall be ticked

	Impacted other TS/TR(s)
	<TS/TR number>, <Version Number>, and <Description on which aspect should be reflected in this TS/TR>

	Post Freeze checking:*
	This CR contains only essential changes and corrections?  YES FORMCHECKBOX
  NO  FORMCHECKBOX 

This CR may break backwards compatibility with the last approved version of the TS?       YES  FORMCHECKBOX 
  NO FORMCHECKBOX



	Template Version: January 2017 (Do not modify)


oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).
Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.
Follow the drafting rules.

All pictures must be editable.
Check spelling and grammar to the extent practicable.
Use Change bars for modifications.
The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.
Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.
When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text. 

Introduction

In the de-announcement procedure, when attribute(s) included in the announcedAttribute attribute is deleted, the attribute should also be deleted from the attribute list in the announcedAttribute attribute. This process already has been covered in TS-0001 as follows with yellow highlight. So, this contribution is trying to add this process in section 7.3.3.11.
10.2.13.10
Procedure for original resource Hosting CSE for De-Announcing Attributes

This clause describes procedure that shall be used by the original resource Hosting CSE to remove announced attributes at remote announced resources (i.e. the attribute de-announcement).

The Originator of this Request shall be the original resource Hosting CSE.

Table 10.2.13.10-1: Original Resource Hosting CSE to De-Announce Attribute: UPDATE

	Attribute De-Announcement: UPDATE

	Associated Reference Point
	Mcc.

	Information in Request message
	Information described for the Originator of the UPDATE Request as in clause 10.1.4.

Content: Parameter includes the names of the attributes to be deleted (de-announced) with their values set to NULL.

	Processing at the Originator before sending Request
	The Originator shall request to delete the announced attributes by using the UPDATE Request as specified in clause 10.1.4. Only attributes marked as OA can be de-announced:

Content: Parameter in the UPDATE Request shall provide the names of the attributes to be de-announced by setting their values set to NULL.

	Processing at the Receiver
	If the value of the From parameter in Request message is identical with the CSE-ID included in the link attribute in the announced resource, the Receiver (CSE hosting announced resource) shall grant the Request after successful validation of the Request. The Receiver shall perform as follows:

· Delete the de-announced attributes identified by the Content parameter in the UPDATE Request as per procedures in clause 10.1.4.
· Respond to the Originator with the appropriate UPDATE Response as in clause 10.1.4.

	Information in Response message
	Parameters defined in table 8.1.3-1 that are applicable.

	Processing at Originator after receiving Response
	The Originator after receiving the Response from the Receiver shall perform the following steps:

· If the attributes have been successfully removed, the announcedAttribute attribute shall be updated so as to remove the attribute names for the successfully de-announced attributes.

	Exceptions
	All exceptions described in the basic procedures (clause 10.1.4) are applicable.


-----------------------Start of change 1-------------------------------------------

7.3.1.1 De-announce the resource or attribute

If UPDATE Request that deletes the URI from the announceTo attribute is received:

· Compose the DELETE Request primitive.

· Send a DELETE Request to the CSE(s) represented by URI(s) in the announceTo attribute of the resource, which is not included in the announceTo of the request. The To parameter in the DELETE Request shall be set to the URI for the announced resource that will be deleted.

· Wait for Response primitive.

· Remove the URI of successfully de-announced resource from the announceTo attribute of the resource.

· Include updated announceTo attribute in the Content parameter in the Response to the UPDATE Request of the original resource.

If DELETE Request is received:

· Compose the DELETE Request primitive.

· Send DELETE Requests to all of the CSE(s) represented by the URI(s) in the announceTo attribute of the resource.

· Wait for Response primitive.

If UPDATE Request that deletes the attribute name from the announcedAttribute attribute is received:

· Compose the UPDATE Request primitive. The To parameter in the UPDATE Request shall be set to the URI for the announced resource. The UPDATE Request shall set the attribute that will be de-announced (i.e. to be deleted) in the Content parameter to NULL. The attribute that will be de-announced shall be marked as OA.

· Send UPDATE Requests to all announced resources listed in the announceTo attribute of the original resource.

· Wait for Response primitive.

· Delete the attribute name of the successfully de-announced attribute from the announcedAttribute attribute.

· Include updated announcedAttribute attribute in the Content parameter in the Response to the received UPDATE Request.
If an attribute(s) specified as MA (See TS-0001 [6]) or an attribute(s) included in the announcedAttribute attribute is deleted:

· Compose an UPDATE Request primitive by including the deleted attribute(s) with its value set to NULL.
· Send the UPDATE Request to all CSE(s) represented by the URI(s) in the announceTo attribute of the original resource.
· Wait for Response primitive.
· Delete the attribute name of the successfully de-announced attribute from the announcedAttribute attribute if it was present.
-----------------------End of change 1---------------------------------------------

CHECK LIST

· Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

· Does this CR contain changes related to only one particular issue/problem?
· Have any mirror CRs been posted?
· Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?
· Are all pictures editable?
· Have you checked the spelling and grammar?
· Have you used change bars for all modifications?
· Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)
· Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?
© 2017 oneM2M Partners
                                                                                                   Page 5 (of 5)



[image: image1.png]