| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | PRO 31 |
| Source:* | Anupama, C-DOT, anupama@cdot.in Poornima, C-DOT, poornima@cdot.in Giribabu naik, C-DOT, moode@cdot.in |
| Date:* | 2017-09-05 |
| Reason for Change/s:* | See the introduction |
| CR against: Release* | Release 2 |
| CR against: WI* | Active <Work Item number> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: PRO-2017-0206 (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0004 Version 2.13.0 |
| Clauses * | D.3, D.9,D.10,D.11 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | <TS/TR number>, <Version Number>, and <Description on which aspect should be reflected in this TS/TR> |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

There are attributes in <mgmtObj> on which update of one starts the actions and update of other stops the actions e.g. activate, deactivate or install, uninstall in [software] resource, logStart, logStop in [eventLog] resource. An originator may specify both the attributes in UPDATE request. So there should be some handling at CSE to decide what actions to take when both the attributes are simultaneously set to TRUE.

The CR proposes that when in a UPDATE request, when both such attributes are set to TRUE then CSE should reject such request as it is causing ambiguous behavior.

### -----------------------Start of change 1-------------------------------------------

Resource [software]

    - Introduction

The detailed description of the [software] resource can be found in clause D.3 of TS-0001 [6].

Table D.3.1-1: Data Type Definition of [software]

| Data Type ID | File Name | Note |
| --- | --- | --- |
| software, softwareAnnc | CDT-software-v2_13_0.xsd |  |

Table D.3.1-2: Resource specific attributes of [software]

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| mgmtDefinition | M | NP | See clause 7.4.15 | 1002 (software) |
| objectIDs | O | NP | See clause 7.4.15 |  |
| objectPaths | O | NP | See clause 7.4.15 |  |
| description | O | O | See clause 7.4.15 |  |
| version | M | O | xs:string |  |
| softwareName | M | O | xs:string |  |
| URL | M | O | xs:anyURI |  |
| install | NP | O | xs:boolean |  |
| uninstall | NP | O | xs:boolean |  |
| installStatus | NP | NP | m2m:actionStatus |  |
| activate | NP | O | xs:boolean |  |
| deactivate | NP | O | xs:boolean |  |
| activeStatus | NP | NP | m2m:actionStatus |  |

Resource specific procedure on CRUD operations

- D.3.2.0.	Introduction

When management is performed using technology specific protocols, the procedures defined in clause 7.4.15.2 <mgmtObj> resource specific procedures shall be used. The following clauses define additional procedures besides the generic procedure defined in clause 7.2.2.

      - Create

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

May start to download the software package from the location indicated by attribute URL in the software resource.

Update

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

Primitive specific operation additional to Recv-6.5 "Create/Update/Retrieve/Delete/Notify operation is performed":

When the attribute install of the [software] resource is updated to TRUE, install the software package downloaded from the address indicated by attribute URL of the [software] resource.

When the attribute uninstall of the [software] resource is updated to TRUE, uninstall the corresponding software of the [software] resource.

When the attribute install and uninstall of the [software] resource are simultaneously set to TRUE in request, the CSE shall reject the request with a Response Status Code indicating "BAD_REQUEST" error.

When the attribute activate of the [software] resource is updated to TRUE, activate the corresponding software of the [software] resource.

When the attribute deactivate of the [software] resource is updated to TRUE, deactivate the corresponding software of the [software] resource.

When the attribute activate and deactivate of the [software] resource are simultaneously set to TRUE in request, the CSE shall reject the request with a Response Status Code indicating "BAD_REQUEST" error.

The Receiver may need to update the swVersion attribute of the [deviceInfo] resource if needed.

Retrieve

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

Delete

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

Primitive specific step after generic procedure defined in clause 7.2.2.2.

Delete the downloaded software package locally.

### -----------------------End of change 1---------------------------------------------

### -----------------------Start of change 2---------------------------------------------

  - Resource [deviceCapability]

    - Introduction

The resource [deviceCapability] is used to provide information regarding the device.

The detailed description of the [deviceCapability] resource can be found in clause D.9 of oneM2M TS-0001 [6].

Table D.9.1-1: Data Type Definition of [deviceCapability]

| Data Type ID | File Name | Note |
| --- | --- | --- |
| deviceCapability, deviceCapabilityAnnc | CDT-deviceCapability-v2_13_0.xsd |  |

Table D.9.1-2: Resource specific attributes of [deviceCapability]

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| mgmtDefinition | M | NP | See clause 7.4.15 | 1008 (deviceCapability) |
| objectIDs | O | NP | See clause 7.4.15 |  |
| objectPaths | O | NP | See clause 7.4.15 |  |
| description | O | O | See clause 7.4.15 |  |
| capabilityName | M | O | xs:string |  |
| attached | M | O | xs:boolean | 1. true: currently attached to the device 2. false: currently detached to the device |
| capabilityActionStatus | M | O | m2m:actionStatus | The action (i.e. enable, disable) and the related status. See Table 6.3.5.14-1. |
| currentState | M | O | xs:boolean | - true: the device capability is enabled - false: the device capability is disabled |
| enable | O | O | xs:boolean | the value of this attribute is always "true" |
| disable | O | O | xs:boolean | the value of this attribute is always "true" |

Resource specific procedure on CRUD operations

      - Introduction

When management is performed using technology specific protocols, the procedures defined in clause 7.4.15.2 <mgmtObj> specific procedures shall be used. The following clauses define additional procedures besides the generic procedure defined in clause 7.2.2.

Create

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

Update

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

Primitive specific operation additional to Recv-6.5 "Create/Update/Retrieve/Delete/Notify operation is performed":

When the attribute enable of the [deviceCapability] resource is updated to TRUE, enable the device capability of the [deviceCapability] resource.

When the attribute disable of the [deviceCapability] resource is updated to TRUE, disable the device capability of the [deviceCapability] resource.

When the attribute enable and disable of the [deviceCapability] resource are simultaneously set to TRUE in request, the CSE shall reject the request with a Response Status Code indicating "BAD_REQUEST" error.

Retrieve

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

Delete

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

### -----------------------End of change 2---------------------------------------------

### -----------------------Start of change 3-------------------------------------------

Resource [reboot]

    - Introduction

The resource [reboot] is used to provide information regarding the device.

The detailed description of the [reboot] resource can be found in clause D.10 of oneM2M TS-0001 [6].

Table D.10.1-1: Data Type Definition of [reboot]

| Data Type ID | File Name | Note |
| --- | --- | --- |
| reboot, rebootAnnc | CDT-reboot-v2_13_0.xsd |  |

Table D.10.1-2: Resource specific attributes of [reboot]

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| mgmtDefinition | M | NP | See clause 7.4.15 | 1009 (reboot) |
| objectIDs | O | NP | See clause 7.4.15 |  |
| objectPaths | O | NP | See clause 7.4.15 |  |
| description | O | O | See clause 7.4.15 |  |
| reboot | O | O | xs:boolean | the value of this attribute is always "True" |
| factoryReset | O | O | xs:boolean | the value of this attribute is always "True" |

Resource specific procedure on CRUD operations

- D.10.2.0.	Introduction

When management is performed using technology specific protocols, the procedures defined in clause 7.4.15.2 <mgmtObj> specific procedures shall be used. The following clauses define additional procedures besides the generic procedure defined in clause 7.2.2.

      - Create

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

Update

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

Primitive specific operation additional to Recv-6.5 "Create/Update/Retrieve/Delete/Notify operation is performed":

When the attribute reboot of the [reboot] resource is updated to TRUE, reboot the corresponding node.

When the attribute factoryReset of the [reboot] resource is updated to TRUE, factory reset the corresponding node shall be applied.

When the attribute reboot and factoryReset of the [reboot] resource are simultaneously set to TRUE in request, the CSE shall reject the request with a Response Status Code indicating "BAD_REQUEST" error.

Retrieve

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

Delete

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

### -----------------------End of change 3-------------------------------------------

### -----------------------Start of change 4-------------------------------------------

Resource [eventLog]

    - Introduction

The Resource [eventLog] is used to provide information regarding the device.

The detailed description of the [eventLog] resource can be found in clause D.11 of oneM2M TS-0001 [6].

Table D.11.1-1: Data Type Definition of [eventLog]

| Data Type ID | File Name | Note |
| --- | --- | --- |
| eventLog, eventLogAnnc | CDT-eventLog-v2_13_0.xsd |  |

Table D.11.1-2: Resource specific attributes of [eventLog]

| Attribute Name | Request Optionality |  | Data Type | Default Value and Constraints |
| --- | --- | --- | --- | --- |
|  | Create | Update |  |  |
| mgmtDefinition | M | NP | See clause 7.4.15 | 1010 (eventLog) |
| objectIDs | O | NP | See clause 7.4.15 |  |
| objectPaths | O | NP | See clause 7.4.15 |  |
| description | O | O | See clause 7.4.15 |  |
| logTypeId | M | O | m2m:logTypeId | See Table 6.3.4.2.23-1 |
| logData | M | O | xs:string | The content and format of this attribute is out of the present document. |
| logStatus | M | O | m2m:logStatus | See Table 6.3.4.2.24-1 |
| logStart | O | O | xs:boolean | the value of this attribute is always "True" |
| logStop | O | O | xs:boolean | the value of this attribute is always "True" |

Resource specific procedure on CRUD operations

- D.11.2.0.	Introduction

When management is performed using technology specific protocols, the procedures defined in clause 7.4.15.2 <mgmtObj> specific procedures shall be used. The following clauses define additional procedures besides the generic procedure defined in clause 7.2.2.

      - Create

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

Update

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

Primitive specific operation additional to Recv-6.5 "Create/Update/Retrieve/Delete/Notify operation is performed":

When the attribute logStart of the [eventLog] resource is updated to TRUE, start the logging.

When the attribute logStop of the [eventLog] resource is updated to TRUE, stop the logging.

When the attribute logStart and logStop of the [eventLog] resource are simultaneously set to TRUE in request, the CSE shall reject the request with a Response Status Code indicating "BAD_REQUEST" error.

Retrieve

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

Delete

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

No change from the generic procedures in clause 7.2.2.2.

### -----------------------End of change 4-------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?