| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | PRO 32 |
| Source:* | Bob Flynn, Convida Wireless; Flynn.bob@convidawireless.com |
| Date:* | 2017-11-15 |
| Reason for Change/s:* | Release Version Procedures |
| CR against: Release* | Release 2 |
| CR against: WI* | Active <Work Item number> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0009 Version 2.10.0 |
| Clauses * |  |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | <TS/TR number>, <Version Number>, and <Description on which aspect should be reflected in this TS/TR> |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

Adds HTTP bind for new request/response parameter, Release Version Indicator (rvi)

### -----------------------Start of change 1-------------------------------------------

### 6.4.22	X-M2M-RVI

The X-M2M-RVI header shall be mapped to the Release Version Indicator parameter of request and response primitives, and vice versa.

### -----------------------End of change 1-------------------------------------------

### -----------------------Start of change 2-------------------------------------------

# A.1	<container> resource creation

Figure A.1-1 is HTTP mapping of procedure described in clause 7.4.7.2.1. Note the example shown in the figure applies under the following assumptions:

"CSE1" is the name (i.e. value of the resourceName attribute) of the <CSEBase> resource of the registrar CSE

"cont1" is the name of the created <container> resource chosen by the registrar CSE

Figure A.1-1: oneM2M HTTP Binding Example - container creation

### -----------------------End of change 2-------------------------------------------

### -----------------------Start of change 3-------------------------------------------

### 6.3.2	Status-Code

The Response Status Code parameter of response primitives shall be mapped to the HTTP Status-Code. Since the Response Status Code parameter values have been defined with more detailed information than HTTP status codes, one or more Response Status Code value may be mapped to the same HTTP Status-Code. The original Response Status Code parameter value shall be carried in the X-M2M-RSC header(see clause 6.4.14).

The mapping of Response Status Code parameter value of oneM2M request primitive to Status-Code of HTTP request messages is specified in table 6.3.2-1.

Table 6.3.2-1: Status Code Mapping

| oneM2M Response Status Codes | HTTP Status Codes |
| --- | --- |
| 2000 (OK) | 200 (OK) |
| 2002 (DELETED) |  |
| 2004 (UPDATED) |  |
| 2001 (CREATED) | 201 (Created) |
| 1000 (ACCEPTED) | 202 (Accepted) |
| 4000 (BAD_REQUEST) | 400 (Bad Request) |
| 4001 (RELEASE_VERSION_NOT_SUPPORTED) |  |
| 4102 (CONTENTS_UNACCEPTABLE) |  |
| 4110 (GROUP_MEMBER_TYPE_INCONSISTENT) |  |
| 6010 (MAX_NUMBER_OF_MEMBER_EXCEEDED) |  |
| 6022 (INVALID_CMDTYPE) |  |
| 6023 (INVALID_ARGUMENTS) |  |
| 6024 (INSUFFICIENT_ARGUMENTS) |  |
| 6028 (ALREADY_COMPLETE) |  |
| 6029 (MGMT_COMMAND_NOT_CANCELLABLE) |  |
| 4101 (SUBSCRIPTION_CREATOR_HAS_NO_PRIVILEGE) | 403 (Forbidden) |
| 4103 (ORIGINATOR_HAS_NO_PRIVILEGE) |  |
| 5105 (RECEIVER_HAS_NO_PRIVILEGE) |  |
| 5106 (ALREADY_EXISTS) |  |
| 5203 (TARGET_NOT_SUBSCRIBABLE) |  |
| 5205 (SUBSCRIPTION_HOST_HAS_NO_PRIVILEGE) |  |
| 4106 (ORIGINATOR_HAS_NOT_REGISTERED) |  |
| 4107 (SECURITY_ASSOCIATION_REQUIRED) |  |
| 4108 (INVALID_CHILD_RESOURCE_TYPE) |  |
| 4109 (NO_MEMBERS) |  |
| 4111 (ESPRIM_UNSUPPORTED_OPTION) |  |
| 4112 (ESPRIM_UNKNOWN_KEY_ID) |  |
| 4113 (ESPRIM_UNKNOWN_ORIG_RAND_ID) |  |
| 4114 (ESPRIM_UNKNOWN_RECV_RAND_ID) |  |
| 4115 (ESPRIM_BAD_MAC) |  |
| 4116 (ESPRIM_IMPERSONATION_ERROR) |  |
| 5208 (DISCOVERY_DENIED_BY_IPE) |  |
| 4004 (NOT_FOUND) | 404 (Not Found) |
| 5103 (TARGET_NOT_REACHABLE) |  |
| 6003 (EXTERNAL_OBJECT_NOT_REACHABLE) |  |
| 6005 (EXTERNAL_OBJECT_NOT_FOUND) |  |
| 4005 (OPERATION_NOT_ALLOWED) | 405 (Method Not Allowed) |
| 5207 (NOT_ACCEPTABLE) | 406 (Not Acceptable) |
| 4008 (REQUEST_TIMEOUT) | 408 (Request Timeout) |
| 4104 (GROUP_REQUEST_IDENTIFIER_EXISTS) | 409 (Conflict) |
| 4105 (CONFLICT) |  |
| 5000 (INTERNAL_SERVER_ERROR) | 500 (Internal Server Error) |
| 5204 (SUBSCRIPTION_VERIFICATION_INITIATION_FAILED) |  |
| 5209 (GROUP_MEMBERS_NOT_RESPONDED) |  |
| 5210 (ESPRIM_DECRYPTION_ERROR) |  |
| 5211 (ESPRIM_ENCRYPTION_ERROR) |  |
| 5212 (SPARQL_UPDATE_ERROR) |  |
| 6020 (MGMT_SESSION_CANNOT_BE_ESTABLISHED) |  |
| 6021 (MGMT_SESSION_ESTABLISHMENT_TIMEOUT) |  |
| 6025 (MGMT_CONVERSION_ERROR) |  |
| 6026 (MGMT_CANCELLATION_FAILED) |  |
| 5001 (NOT_IMPLEMENTED) | 501 (Not Implemented) |
| 5206 (NON_BLOCKING_REQUEST_NOT_SUPPORTED) |  |

### -----------------------End of change 3-------------------------------------------

### -----------------------Start of change 4-------------------------------------------

### -----------------------End of change 4-------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?