	Doc# PRO-2018-0087-successful_RSC_for_group_fan-out_(R3).doc
Change Request
	[image: image1.png]






	


	CHANGE REQUEST

	Meeting ID:*
	PRO#34

	Source:*
	SeungMyeong JEONG, KETI, sm.jeong@keti.re.kr 
NakMyoung Sung, KETI, nmsung@keti.re.kr  

	Date:*
	2010-03-08

	Reason for Change/s:*
	

	CR  against:  Release*
	3

	CR  against:  WI*
	 FORMCHECKBOX 
 Active
 FORMCHECKBOX 
 MNT maintenance / < Work Item number(optional)>
Is this a mirror CR? Yes  FORMCHECKBOX 
 No  FORMCHECKBOX 

mirror CR number: PRO-2018-0086-successful_RSC_for_group_fan-out_(R2)
 FORMCHECKBOX 
 STE Small Technical Enhancements / < Work Item number (optional)>
Only ONE of the above shall be ticked

	CR  against:  TS/TR*
	TS-0004 v3.6.0

	Clauses *
	7.4.14.2.5
Aggregation of member responses

	Type of change: *
	 FORMCHECKBOX 
 Editorial change
 FORMCHECKBOX 
 Bug Fix or Correction
 Change to existing feature or functionality
 FORMCHECKBOX 
 New feature or functionality
Only ONE of the above shall be ticked

	Impacted other TS/TR(s)
	n/a

	Post Freeze checking:*
	This CR contains only essential changes and corrections?  YES  FORMCHECKBOX 
  NO  FORMCHECKBOX 

This CR may break backwards compatibility with the last approved version of the TS?       YES 
  NO 

	Template Version: January 2017 (Do not modify)


oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.
In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR
Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).
Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.
Follow the drafting rules.
All pictures must be editable.
Check spelling and grammar to the extent practicable.
Use Change bars for modifications.
The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.
Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.
When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text. 
Introduction
In the current TS-0004, it is not defined which successful RSC is sent for fan-out requests having different operations. For example, when there’s one or more response from members to Create fan-out request from the Originator, the response from the group Hosting CSE will have 2001 as the successful RSC value. 
The contributor suggests the discussion on the following situations:

1. Should the group Hosting CSE send back another RSC syntax saying “fanned-out” to one or more in the current successful case definition instead of “OK”, “Created”, “Deleted” that we have?

2. In case there’s one or more member response having unsuccessful RSC, should the RSC of the aggregated response be unsuccessful code? This means the group Hosting CSE shall look into each response from members. What about half created and half not created case? Should the group Host say “Created”? or newly “partially created” or something else?
Initially this CR is to open up the discussion on those points, and the change request in the initial version is an one of the options for discussion.
-----------------------Start of change 1-------------------------------------------

7.4.14.2.5 Aggregation of member responses
After receiving the member responses from the member hosting CSEs, the group hosting CSE shall respond to the Originator with an aggregated response. To indicate which response is generated by which member resource, the Hosting CSE shall add member resource ID, which is corresponding to the response, into From response parameter in each member response.


If Response Type, Result Expiration Time or Result Persistence were set in the request, these affect the behaviour of the group hosting CSE as follows:

If Response Type is set to blockingRequest, the group hosting CSE shall respond only once with the aggregated response. It shall do this before the time indicated by the Result Expiration Time is reached. The group hosting CSE shall discard any member responses received after this time.

If Response Type is set to nonBlockingRequestSynch, the group hosting CSE shall create a <request> resource locally and respond the Originator with the address of this <request> resource. Until the Result Expirtation Time is reached, the group hosting CSE shall aggregate the member responses and include this aggregated response in the operationResult of the <request> resource. 
If Response Type is set to nonBlockingRequestAsynch, the group hosting CSE shall notify the Originator or the notification targets with aggregated responses before the Result Expiration Time expires. The group hosting CSE may notify the Originator more than once during the period until the Result Expiration Time expires. Each notification shall contain different member responses.
If Response Type is set to flexBlocking, the group hosting CSE shall keep aggregating the member responses until the group hosting CSE determines that it is time to send a response – this depends on the properties of the group hosting CSE related to the <group> resource (the number of aggregated responses or the time period of the aggregation). By that time, if the aggregated response contains all the member responses, the group hosting CSE shall respond with the aggregated response. However if only some of the member responses have been received , the group hosting CSE shall create a <request> resource from the received request, and respond to the Originator with the reference to the created <request> resource as well as the currently aggregated responses. Until the time specified in Result Expiration Time is reached, the group hosting CSE shall keep aggregating the remaining member responses and updating the aggregated response in the operationResult of the <request> resource. If notificationTarget is provided in the request, the group hosting CSE shall notify the Originator with the aggregated response. Each notification shall contain different member responses.

If the group hosting CSE supports <request> resource, in the nonBlockingRequestAsynch, nonBlockingSynch and flexBlocking case, it shall set the requestStatus of the <request> resource to PARTIALLY_COMPLETED if some of the member responses are received. If the group hosting CSE has aggregated all the member responses, it shall set the operationResult to COMPLETED.

In any of the cases above, member responses received after the Result Expiration Time shall be discarded. After the time specified in Result Persistence, the aggregated response shall not be retrievable.
If the group Hosting CSE gets no response before the Result Expiration Timestamp expiry, then the Hosting CSE shall return error with the Response Status Code parameter set as "GROUP_MEMBERS_NOT_RESPONDED". Otherwise, the group Hosting CSE shall return successful the Response Status Code parameter value “OK” regardless of the requested operation. Note that the successful Response Status Code parameter is set regardless of the Response Status Code parameter value in each response primitive from the group member(s).
If any of the parameter mentioned above are missing from the request, the group hosting CSE shall determine the time to respond using its local Policy.
7.4.14.3 <fanOutPoint> resource specific procedure on CRUD operations 

7.4.14.3.1 Introduction

This sub-clause describes <fanOutPoint> resource specific behaviour for CRUD operations.

7.4.14.3.2 Create

A Create operation sent to a <fanOutPoint> is fanned out to the members (if any) of the parent <group>. It is equivalent to sending a Create to each member and therefore results in new resources being created as children of these existing members. 

If the Create is sent to a hierarchical URI containing a fanOutPoint and an additional path relative to that fanOutPoint then the new resources are not created as immediate children of the members, rather they are created as children of descendents of those members (as determined by the relative path).

Originator:

Primitive specific operation after Orig-1.0 "Compose Request primitive" and before Orig-2.0 "Send the Request to the receiver CSE": In the case the Originator wants to subscribe to all the member resources of the group and the originator wants the group hosting CSE to aggregate all the notifications come from its member hosting CSEs, the Originator shall include notificationForwardingURI attribute in the <subscription> resource.
If the Response Status Code in individual responses are other than CREATED then the Originator may issue a new CREATE request to the group Hosting CSE with Group Request Target Members parameter set in request containing list of members for which CREATE has failed.
Receiver:

Primitive specific operation after Recv-6.2 "Check existence of the addressed resource" and before Recv-6.3 "Check authorization of the Originator".

Primitive specific operation additional to Recv-6.3 "Check authorization of the Originator": The Group Hosting CSE shall check the authorization of the Originator based on the membersAccessControlPolicyIDs of the parent <group> resource. In the case the membersAccessControlPolicyIDs is not provided, the accessControlPolicyIDs of the parent <group> resource shall be used.

Primitive specific operation to replace Recv-6.5"Create/Update/Retrieve/Delete/Notify operation is performed" and Recv-6.6"Announce/De-announce the resource" in the generic procedure:

1) Validate the type of resource to be created, refer to clause 7.4.14.2.1.

2) Sub-group creation for members residing on the same CSE, refer to clause 7.4.14.2.2.

3) Assign URI for aggregation of notification, refer to clause 7.4.14.2.3.

4) Fanout Request to each member, refer to clause 7.4.14.2.4.

5) Aggregation of member responses, refer to clause 7.4.14.2.5.
7.4.14.3.3 Retrieve

Originator:

No primitive specific operations.
If the Response Status Code in individual responses is other than OK then the Originator may issue a new RETRIEVE request to the group Hosting CSE with Group Request Target Members parameter set in request containing list of members for which RETRIEVE has failed.

Receiver:
Primitive specific operation after Recv-6.2 "Check existence of the addressed resource" and before Recv-6.3 "Check authorization of the Originator".

Primitive specific operation additional to Recv-6.3 "Check authorization of the Originator": The Group Hosting CSE shall check the authorization of the Originator based on the membersAccessControlPolicyIDs of the parent group resource. In the case the membersAccessControlPolicyIDs is not provided, the accessControlPolicyIDs of the parent group resource shall be used.

Primitive specific operation to replace Recv-6.5"Create/Update/Retrieve/Delete/Notify operation is performed" and Recv-6.6 "Announce/De-announce the resource" in the generic procedure:

1) Sub-group creation for members residing on the same CSE, refer to 7.4.14.2.2.
2) Fanout Request to each member, refer to 7.4.14.2.4.
3) Aggregation of member responses, refer to clause 7.4.14.2.5.
7.4.14.3.4 Update

Originator:

No primitive specific operations.
If the Response Status Code in individual responses is other than UPDATED then the Originator may issue a new UPDATE request to the group Hosting CSE with Group Request Target Members parameter set in request containing list of members for which UPDATE has failed.
Receiver:

Primitive specific operation after Recv-6.2 "Check existence of the addressed resource" and before Recv-6.3 "Check authorization of the Originator".

Primitive specific operation additional to Recv-6.3 "Check authorization of the Originator": The Group Hosting CSE shall check the authorization of the Originator based on the membersAccessControlPolicyIDs of the parent group resource. In the case the membersAccessControlPolicyIDs is not provided, the accessControlPolicyIDs of the parent group resource shall be used.

Primitive specific operation to replace Recv-6.5 "Create/Update/Retrieve/Delete/Notify operation is performed" and Recv-6.6"Announce/De-announce the resource" in the generic procedure:
1) Sub-group creation for members residing on the same CSE , refer to 7.4.14.2.2.
2) Fanout Request to each member. See Clause 7.4.14.2.4.

3) Aggregation of member responses, refer to clause 7.4.14.2.5.
7.4.14.3.5 Delete

The primitive deletes the member resources and their child resources belonging to an existing <group> resource.

Originator:

No primitive specific operations.
If the Response Status Code in individual responses is other than DELETED then the Originator may issue a new DELETE request to the group Hosting CSE with Group Request Target Members parameter set in request containing list of members for which DELETE has failed.
Receiver:

Primitive specific operation after Recv-6.2 "Check existence of the addressed resource" and Recv-6.3 "Check authorization of the Originator": The To parameter consists of the URI of the group resource plus a suffix consisting of /fanOutPoint or /fanOutPoint/plus any additional appended relative address.

Primitive specific operation additional to Recv-6.3 "Check authorization of the Originator": The Group Hosting CSE shall check the authorization of the Originator based on the membersAccessControlPolicyIDs of the parent group resource. In the case the membersAccessControlPolicyIDs is not provided, the accessControlPolicyIDs of the parent group resource shall be used.

Primitive specific operation to replace Recv-6.5"Create/Update/Retrieve/Delete/Notify operation is performed" and Recv-6.6"Announce/De-announce the resource" in the generic procedure:
1) Sub-group creation for members residing on the same CSE , refer to 7.4.14.2.2
2) Fanout Request to each member. See Clause 7.4.14.2.4
3) Aggregation of member responses, refer to clause 7.4.14.2.5.
4) A Delete operation targeting a <fanOutPoint> does not remove members from the parent <group>.
-----------------------End of change 1-------------------------------------------
CHECK LIST

· Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?
· Does this CR contain changes related to only one particular issue/problem?
· Have any mirror CRs been posted?
· Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?
· Are all pictures editable?
· Have you checked the spelling and grammar?
· Have you used change bars for all modifications?
· Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)
· Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?
© 2018 oneM2M Partners
                                                                                                   Page 2 (of 7)



[image: image1.png]