| oneM2M Technical Report |  |
| --- | --- |
| Document Number | oneM2M-TR-0026-V-0.5.0 |
| Document Name: | Vehicular Domain Enablement |
| Date: | 2016-Dec.-05 |
| Abstract: | This oneM2M Technical Report examines how the current oneM2M System can be used in the Vehicular Domain and includes a study of advanced features which the future oneM2M release(s) could support for this vertical domain. |
| Template Version: 08 September 2015 (Dot not modify) |  |

This Specification is provided for future development work within oneM2M only. The Partners accept no liability for any use of this Specification.

The present document has not been subject to any approval process by the oneM2M Partners Type 1.  Published oneM2M specifications and reports for implementation should be obtained via the oneM2M Partners' Publications Offices.

About oneM2M

The purpose and goal of oneM2M is to develop technical specifications which address the need for a common M2M Service Layer that can be readily embedded within various hardware and software, and relied upon to connect the myriad of devices in the field with M2M application servers worldwide.

More information about oneM2M may be found at:  http//www.oneM2M.org

Copyright Notification

© 2015, oneM2M Partners Type 1 (ARIB, ATIS, CCSA, ETSI, TIA, TSDSI, TTA, TTC).

All rights reserved.

The copyright and the foregoing restriction extend to reproduction in all media.

Notice of Disclaimer & Limitation of Liability

The information provided in this document is directed solely to professionals who have the appropriate degree of experience to understand and interpret its contents in accordance with generally accepted engineering or other professional standards and applicable regulations. No recommendation as to products or vendors is made or should be implied.

NO REPRESENTATION OR WARRANTY IS MADE THAT THE INFORMATION IS TECHNICALLY ACCURATE OR SUFFICIENT OR CONFORMS TO ANY STATUTE, GOVERNMENTAL RULE OR REGULATION, AND FURTHER, NO REPRESENTATION OR WARRANTY IS MADE OF MERCHANTABILITY OR FITNESS FOR ANY PARTICULAR PURPOSE OR AGAINST INFRINGEMENT OF INTELLECTUAL PROPERTY RIGHTS. NO oneM2M PARTNER TYPE 1 SHALL BE LIABLE, BEYOND THE AMOUNT OF ANY SUM RECEIVED IN PAYMENT BY THAT PARTNER FOR THIS DOCUMENT, WITH RESPECT TO ANY CLAIM, AND IN NO EVENT SHALL oneM2M BE LIABLE FOR LOST PROFITS OR OTHER INCIDENTAL OR CONSEQUENTIAL DAMAGES. oneM2M EXPRESSLY ADVISES ANY AND ALL USE OF OR RELIANCE UPON THIS INFORMATION PROVIDED IN THIS DOCUMENT IS AT THE RISK OF THE USER.

Contents

Contents	3

1	Scope	10

2	References	10

2.1	Normative references	10

2.2	Informative references	10

3	Definitions, symbols and abbreviations	10

3.1	Definitions	10

3.2	Symbols	11

3.3	Abbreviations	11

4	Conventions,	11

5	Introduction to Vehicular Domain	11

5.1	Vehicular Domain Overview	11

5.2	Technology Trends in Vehicular Domain	12

5.3	The focus of oneM2M in Vehicular Domain	13

6	Vehicular Domain Use Cases	14

6.1	Vehicle Diagnostic & Maintenance Report	14

6.1.1	Description	14

6.1.2	Source	14

6.1.3	Actors	14

6.1.4	Pre-conditions	14

6.1.5	Triggers	14

6.1.6		Normal Flow	15

6.1.7	Alternative Flow	15

6.1.8	Post-conditions	15

6.1.9	High Level Illustration	16

6.1.10	Potential Requirements	16

6.2	Use Case on Remote Maintenance Services	16

6.2.1	Description	16

6.2.2	Source	17

6.2.3	Actors	17

6.2.4	Pre-conditions	17

6.2.5	Triggers	17

6.2.6	Normal Flow	17

6.2.7		Alternative Flow	18

6.2.8	Post-conditions	18

6.2.9	High Level Illustration	18

6.2.10	Potential Requirements	19

6.3	Traffic Accident Information Collection	19

6.3.1	Description	19

6.3.2	Source	19

6.3.3	Actors	19

6.3.4	Pre-conditions	19

6.3.5	Triggers	20

6.3.6	Normal Flow	20

6.3.7	Alternative Flow	21

6.3.8	Post-conditions	21

6.3.9	High Level Illustration	21

6.3.10	Potential Requirements	21

6.4	Fleet Management Service using DTG (Digital Tachograph)	22

6.4.1	Description	22

6.4.2	Source	22

6.4.3	Actors	22

6.4.4	Pre-conditions	22

6.4.5	Triggers	23

6.4.6	Normal Flow	23

6.4.7	Alternative Flow	25

6.4.8	Post-conditions	25

6.4.9	High Level Illustration	26

6.4.10	Potential Requirements	26

6.5	Use cases for Electronic Toll Collection (ETC) service	26

6.5.1	Description	26

6.5.2	Source	27

6.5.3	Actors	27

6.5.4	Pre-conditions	27

6.5.5	Triggers	27

6.5.6	Normal Flow	28

6.5.7	Alternative Flow	29

6.5.8	Post-conditions	29

6.5.9	High Level Illustration	29

6.5.10	Potential Requirements	29

6.6	Use cases for Taxi Advertisement	30

6.6.1	Description	30

6.6.2	Source	30

6.6.3	Actors	30

6.6.4	Pre-conditions	30

6.6.5	Triggers	30

6.6.7	Normal Flow	30

6.6.7	Alternative Flow	30

6.6.8	Post-conditions	30

6.6.9	High Level Illustration	30

6.6.10	Potential Requirements	31

6.7	Use Case on Vehicle Data Service	31

6.7.1	Description	31

6.7.2	Source	31

6.7.3	Actors	31

6.7.4	Pre-conditions	32

6.7.5	Triggers	32

6.7.6	Normal Flow	32

6.7.7	Alternative Flow	34

6.7.8	Post-conditions	34

6.7.9	High Level Illustration	35

6.7.10	Potential requirements	35

6.8	Smart Automatic Driving	36

6.8.1	Description	36

6.8.2	Source	36

6.8.3	Actors	36

6.8.4	Pre-conditions	36

6.8.5	Triggers	37

6.8.6	Normal Flow	37

6.8.7	Alternative flow	37

6.8.8	Post-conditions	37

6.8.9	High Level Illustration	37

6.8.10	Potential requirements	38

6.9	Use Case on Vehicle Data Wipe Service	38

6.9.1	Description	38

6.9.2	Source	38

6.9.3	Actors	39

6.9.4	Pre-conditions	39

6.9.5	Triggers	39

6.9.6	Normal Flow	39

6.9.7	Alternative flow	39

6.9.8	Post-conditions	39

6.9.9	High Level Illustration	40

6.9.9.1	Data Request and Response	40

6.9.9.2		Issue of Bigger Data	41

6.9.9.3		Pre-condition of Data Wipe (and Post-condition of Data Request and Data Response)	41

6.9.9.4		Data Wipe	42

6.9.9.5		Data Wipe with Authentication	42

6.9.9.6		Post Condition of Data Wipe	43

6.9.10	Potential requirements	43

6.10	Vehicle Management based on Geo-Fence	43

6.10.1	Description	43

6.10.2	Source	43

6.10.3	Actors	43

6.10.4	Pre-conditions	44

6.10.5	Triggers	44

6.10.6	Normal Flow	44

6.10.7	Alternative flow	45

6.10.8	Post-conditions	45

6.10.9	High Level Illustration	45

6.10.10	Potential requirements	45

6.11	Use Case on Secure Over-The-Air Firmware Update for Automotive ECUs	46

6.11.1	Description	46

6.11.2	Source	46

6.11.3	Actors	46

6.11.4	Pre-conditions	47

6.11.5	Triggers	47

6.11.6	Normal Flow	47

6.11.7	Alternative flow	48

6.11.8	Post-conditions	50

6.11.9	High Level Illustration	50

6.11.10	Potential requirements	50

6.12	Car/Bicycle Sharing Services	51

6.12.1	Description	51

6.12.2	Source	51

6.12.3	Actors	51

6.12.4	Pre-conditions	52

6.12.5	Triggers	52

6.12.6		Normal Flow	52

6.12.7	Alternative Flow	55

6.12.8	Post-conditions	55

6.12.9	High Level Illustration	56

6.12.10	Potential Requirements	56

6.13	Smart Parking	56

6.13.1	Description	56

6.13.2	Source	56

6.13.3	Actors	57

6.13.4	Pre-conditions	57

6.13.5	Triggers	57

6.13.6	Normal Flow	57

6.13.7		Alternative Flow	59

6.13.8	Post-conditions	60

6.13.9	High Level Illustration	61

6.13.10	Potential Requirements	61

6.14	Vehicle Broadcasting without Registration	61

6.14.1	Description	61

6.14.2	Source	61

6.14.3	Actors	61

6.14.4	Pre-conditions	62

6.14.5	Triggers	62

6.14.6	Normal Flow	62

6.14.7		Alternative Flow	62

6.14.8	Post-conditions	62

6.14.9	High Level Illustration	63

6.14.10	Potential Requirements	63

6.15	Vehicle location privacy protection	63

6.15.1	Description	63

6.15.2	Source	63

6.15.3	Actors	63

6.15.4	Pre-conditions	64

6.15.5	Triggers	64

6.15.6	Normal Flow	64

6.15.7	Alternative flow	65

6.15.8	Post-conditions	65

6.15.9	High Level Illustration	65

6.15.10	Potential requirements	65

6.16	Vehicle Domain service continuity	66

6.16.1 Description	66

6.16.2 Source	66

6.16.3  Actors	66

6.16.4 Pre-conditions	66

6.16.5 Triggers	66

6.16.6 Normal Flow	66

6.16.7 Alternative flow	67

6.16.8 Post-conditions	68

6.16.9 High Level Illustration	69

6.16.10 Potential requirements	69

7	Overview of Potential Requirements	69

8	High Level Architecture	73

9	oneM2M Release 2 Enablement for Vehicular Domain	75

9.1	xxx	75

9.2	xxx	75

10	 Potential enhancements to oneM2M	75

Proforma copyright release text block	75

Annexes	76

Annex <y>: Bibliography	76

History	77

1	Scope

This oneM2M Technical Report examines how the current oneM2M System can be used in the Vehicular Domain and includes a study of advanced features which the future oneM2M release(s) could support for this vertical domain. The Technical Report also analyzes use cases and the potential requirements pertaining to the use cases with regard to vehicular domain.

2	References

The following text block applies.

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

2.1	Normative references

As a Technical Report (TR) is entirely informative it shall not list normative references.

The following referenced documents are necessary for the application of the present document.

Not applicable.

2.2	Informative references

Clause 2.2 shall only contain informative references which are cited in the document itself.

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	oneM2M Drafting Rules (http://www.onem2m.org/images/files/oneM2M-Drafting-Rules.pdf)

3	Definitions, symbols and abbreviations

Delete from the above heading the word(s) which is/are not applicable.

3.1	Definitions

Clause numbering depends on applicability.

A definition shall not take the form of, or contain, a requirement.

The form of a definition shall be such that it can replace the term in context. Additional information shall be given only in the form of examples or notes (see below).

The terms and definitions shall be presented in alphabetical order.

For the purposes of the present document, the [following] terms and definitions [given in ... and the following] apply:

Definition format

<defined term>: <definition>

If a definition is taken from an external source, use the format below where [N] identifies the external document which must be listed in Section 2 References.

<defined term>[N]: <definition>

example 1: text used to clarify abstract rules by applying them literally

NOTE:	This may contain additional information.

3.2	Symbols

Clause numbering depends on applicability.

For the purposes of the present document, the [following] symbols [given in ... and the following] apply:

Symbol format

<symbol>	<Explanation>

<2nd symbol>	<2nd Explanation>

<3rd symbol>	<3rd Explanation>

3.3	Abbreviations

Abbreviations should be ordered alphabetically.

Clause numbering depends on applicability.

For the purposes of the present document, the [following] abbreviations [given in ... and the following] apply:

Abbreviation format

ARIB	Association of Radio Industries and Businesses

ATIS	Alliance for Telecommunications Industry Solutions

CCSA	China Communications Standards Association

ETSI	European Telecommunications Standards Institute

TIA	Telecommunications Industry Association,

TSDSI	Telecommunications Standards Development Society

TTA	Telecommunications Technology Association

TTC	Telecommunication Technology Committee

<ABBREVIATION1>	<Explanation>

<ABBREVIATION2>	<Explanation>

<ABBREVIATION3>	<Explanation>

4	Conventions,

The key words “Shall”, ”Shall not”, “May”, ”Need not”, “Should”, ”Should not” in this document are to be interpreted as described in the oneM2M Drafting Rules [i.1]

5	Introduction to Vehicular Domain

5.1	Vehicular Domain Overview

Existing ITS (Intelligent Transport Systems) services in automotive industry have been provided through the architecture which is composed of connected vehicles to various infrastructures such as the ETC (Electronic Toll Collection) service, VICS (Vehicle Information and Communication System) or telematics service centers. Those systems have been growing by improving mobility convenience for drivers. However, the infrastructure systems are required  to change  and become more collaborative , in order to meet rising social demands such as energy saving, traffic congestion resolution and fatal accidents avoidance.

The changing trends are enabledby improvements of IT technologies such as V2V/V2I (Vehicular to Vehicular/ Vehicular to Infrastructure) communication, cloud systems or OSS (Open Source Software). The M2M architecture for vehicle domain described in this TR accelerates wide collaboration between not only auto manufacturers or suppliers but also between telecommunication providers or government, which results in improvement of safety, comfortability and eco-friendliness. Figure 5.1-1 is an example of reference architecture.

Figure 5.1-1 Vehicular Domain Architecture

In this architecture, vehicles can connect to various services via M2M systems. Gateways which collect sensing data could be located  both inside (left of figure) and outside of the vehicle (centre of figure). A vehicle may act as a gateway and collect also data generated by sensors located outside of a vehicle (right of figure). Furthermore vehicles may connect to each other and a vehicle may connect to services via other vehicles.

This vehicular domain architecture provides the basic functions for analyzing huge amounts of data collected from large number of vehicles which makes it possibleto create future services as a result of collaboration with other industries.

Furthermore, multiple security levels according to features of collected data make it possible to provide critical control and management for vehicles such as remote maintenance or automated driving control via communication networks.

5.2	Technology Trends in Vehicular Domain

In order to develop vehicular domain architecture as ITS  platform, many worldwide organizations are discussing technology and standardization related to this field.

ISO (International Organization for Standardization) is an international standard-setting body which standardized technologies for ITS systems through the work of TC (Technical Committee)  204 (ITS) and 22 (Road Vehicles). In recent years,  ISO has focused on CITS (Cooperative ITS) using V2V/V2I communication for some new services such as automated driving and Urban ITS which improves mobility in urban area. Furthermore, it has made an effort to develop gateways between in-vehicle area network and nomadic devices like smartphones.

ITU-T (International Telecommunication Union Telecommunication Standardization Sector) is one of the three sectors of the ITU and it coordinates standards for telecommunications. In SG16 (multimedia) of  ITU-T, VGP (Vehicle Gateway Platform) has been standardized in terms of telecommunication. In particular, reference architecture and functional architecture based on supposed applications and interface protocol to vehicles or ICT devices are discussed.

ETSI (European Telecommunications Standards Institute) is a standardization organization in the telecommunications industry (equipment makers and network operators) in Europe. In TC (Technical Committee) ITS, standards, specifications and other deliverables to support the development and implementation of ITS Service provision across the network have been developed. The scope includes communication media, and associated physical layer, transport layer, network layer, security, lawful intercept and the provision of generic web services.

The W3C (World Wide Web Consortium) is the main international standards organization for the World Wide Web. In 2013, it has established the Automotive and Web Platform Business Group and prepared a draft of Vehicle Information Access and Vehicle Data Spec. Currently these topics are in discussion to standardize in the Automotive Working Group. Respond to the growing needs of web services for a connected car, the W3C focuses on interfaces for application vendors to accsess vehicle data using a standard and secure method.

In reference to V2V/V2I commnication technologies which support M2M services in vehicular domain,  DSRC (Dedicated Short Range Communications) has been discussed in ITU-R (International Telecommunication Union Radiocommunications Sector) and IEEE (The Institute of Electrical and Electronics Engineers). Moreover cellular communication for vehicular domain is also discussed in 3GPP (Third Generation Partnership Project). These wireless communication technologies are expected to resolve some problems such as cost, coverage, latency and power consumption.,  while an  unified communication standard is expected to accelerate the growth of M2M services in business.

Table 5.2-1 Vehicular Domain Standards

| No | Organization | Sector | Focus point | Major topics |
| --- | --- | --- | --- | --- |
| 1 | ISO | TC204, TC22 | ITS services | Cooperative System, In-vehicular gateway |
| 2 | ITU-T | SG16 | Telecommunication | Vehicle Gateway Platform, Communication protocol |
| 3 | ETSI | TC ITS | Network system, Radio Technique | Cooperative ITS, DSRC |
| 4 | W3C | Automotive WG | Web services | Web-API for vehicles |
| 5 | ITU-R | SG5/WP5A | Wireless communication | V2V/V2I communication (DSRC) |
| 6 | IEEE | 802.11p + P1609 | Wireless communication | V2V/V2I communication (DSRC) |
| 7 | 3GPP | - | Wireless communication | V2V/V2I communication (Cellular) |

5.3	The focus of oneM2M in Vehicular Domain

In the context of progress achieved through various standardization efforts, oneM2M is continuing to develop a common platform which can be used for information transport functions between vehicles and backend servers to satisfy vehicular domain requirements. The following capabilities are of key importance to the vehicular domain.

Data management

Since vehicles may move fast, the operation and management in this domain will be dynamic, complex and difficult. For example, if the nodes located on roadside act as gateway, status of a connection between device and gateway is varying frequently. Therefore, management of data (including sensing and device information, data streams) as well as filtering and preprocessing functions (at devices or gateways) are required in order to provide  efficient analysis services in the complicated enviroments.

Communication management

M2M services in vehicular domain need to deal with many and widespread devices. On the other hand, the target devices of the operation are depending on the services. For instance, it is expected that the content is distributed to the vehicles in a specific geographical area. Therefore, the communication management function is needed to select appropriate communication protocols and modes (e.g. unicast, multicast or broadcast), according to features of data, for efficient utilization of network resources.

Location

Geographical location information is important in vehicular services. To improve safety, up-to-date location information is important for collision avoidance. Therefore, accurate location providing/managing functions are needed in this vehicular domain. Furthermore, the inherent node mobility in the vehicular domain creates greater variability in the services available along the way. Mobility management functionality is required to allow maintaining the vehicular services as the node moves, and to provide means for selection and migration of services between nodes.

Security

To mitigate the life-threatening risk resulting from malicious control and management of vehicles via network, components of vehicular services should support strict authentication functionality. Furthermore  integrity and encryption functionality for communication are required,  with considerration for on system  scalability and resource constraints  at the devices.

6	Vehicular Domain Use Cases

  - 6.1	Vehicle Diagnostic & Maintenance Report

    - 6.1.1	Description

The Vehicle Service Centre wants to help the vehicle owner to be aware of the status of the vehicle and remind them to maintain the vehicle in a timely manner to avoid any damages.

Hence the Vehicle Service Centre needs to obtain and analyse data from the vehicle periodically. Based on the analysis result, it will notify to the vehicle owner showing what's going on with the vehicle —in simple language and images together with some maintenance suggestions.

    - 6.1.2	Source

oneM2M-REQ-2012-0067R03 Vehicle Stolen and Vehicle Diagnostics

    - Actors

- Vehicle Owner: 
By reading the Vehicle Diagnostic & Maintenance Report sent from the Vehicle Service Centre, the vehicle owner would decide whether to maintain his/her vehicle.

- Vehicle Service Centre: 
It operates a service platform for diagnostics and maintenance of vehicles, obtains and analyzes the diagnostics data from the vehicle. It will also send vehicle Diagnostic & Maintenance Report in e-mail together with maintenance suggestions to the vehicle owner.

- Mobile Communication Network Operator:
As the transmission medium, it supports the network services between Vehicle Service Centre and Vehicle for the information transmission.

- M2M Device: 
It is embedded in a vehicle, which is used to send information to Vehicle Service Centre and implement diagnostics function from Vehicle Service Centre.

    - 6.1.4	Pre-conditions

- The vehicle supports the diagnostics pre-configured to report the diagnostics data collected from sensors within the vehicle periodically.

- The vehicle is already ignited

    - 6.1.5	Triggers

None

    - 6.1.6		Normal Flow

Figure 6.1-1 Vehicle Diagnostics Normal Flow

- The vehicle collects the diagnostics data from sensors within the vehicle and sends it to the Vehicle Service Centre. The diagnostics data includes information from Engine and Transmission System, Stability Control System, Air Bag System, Emission System, Antilock Brake System and so on. The information includes tyre pressure, odometer data, life of engine oil, engine and gear-box status, antilock braking system status etc.

- The Vehicle Service Centre sends the diagnostics data to the “Vehicle Detection M2M Application”. This M2M application receives and analyzes the diagnostics data.

- The “Vehicle Detection M2M Application” finds that the Brake pads need to be replaced. It queries the maintenance services provided by “Vehicle Resolution M2M Application” and gets the information of the company who can provide the components.

- The “Vehicle Detection M2M Application” finally sends the Diagnostic & Maintenance Report to the vehicle owner together with the suggested component providers either by email or alert message displayed in the vehicle terminal.

- The vehicle owner will decide whether to maintain his/her vehicle based on the Diagnostic & Maintenance Report.

    - 6.1.7	Alternative Flow

None

    - 6.1.8	Post-conditions

For normal flow, the vehicle owner maintains his/her vehicle according to the Diagnostic & Maintenance report in time.

    - 6.1.9	High Level Illustration

Figure 6.1-2 Vehicle diagnostics Normal Flow

    - 6.1.10	Potential Requirements

- The M2M application System shall enable the M2M Devices to exchange M2M application to diagnostic data periodically with the M2M Application in the network domain.

- The M2M System shall enable the M2M Application to configure the notification interval in the M2M Devices.

- The M2M system shall support a mechanism to describe the syntax and semantics format of the M2M application diagnostics data exchanged between the M2M Devices and the M2M Application in the network domain.

  - 6.2	Use Case on Remote Maintenance Services

    - 6.2.1	Description

This use case introduces a remote maintenance service for the automobiles (cars).

Because integrity of the cars is a matter of human life, the remote maintenance service of the car (treated as M2M Gateway in this use case) should be strongly secured.

Therefore, the integrity measurements both before and after the remote maintenance operation should also be severely performed.

One of the methods to endorse the measurement process might be guaranteed by HSM (Hardware Security Modules) in the M2M Gateway. This method provides the higher reliability level than that by the software emulator, the decision on the level of security is based on the information sent to the center. In the HSM method, this case, the integrity measurement report is can be made by HSM through an internal the mechanism in the HSM and put in the electronic signature/ by the key in the HSM.

This use case is derived from the automobiles, but the similar case of the remote maintenance services could be considered with Medical equipment, Household applications, financial transaction terminals and Industrial control and machinery.

    - 6.2.2	Source

oneM2M-REQ-2013-0188R06 Use Case Remote Maintenance

    - 6.2.3	Actors

Relevant to the name in the figure in clause 6.2.9, High Level Illustration:

- Car: the machine works as a M2M Gateway in which M2M Device(s) is implemented as the parts of it.

- Center: the M2M Platform which provides remote maintenance.

- The Hardware Security Module (HSM): a module in the M2M Gateway (e.g. Trusted Platform Module) that helps determining the level of security functions to endorse the integrity measurement process and holds the electronic signature key.

- A white list: data base which is accessed by the center may be used for verifying the integrity measurement report from the M2M Gateway (car), using a secure communication protocol e.g. Trusted Network Connect TNC protocol.

- Support software: installable software module to check the integrity of the Car assisted by TPM or the emulator and to support the newly implemented M2M Device(s) (i.e. sensor(s)).

    - 6.2.4	Pre-conditions

Center recognizes the software which is installed in the Car to shall be updated.

    - 6.2.5	Triggers

None

    - 6.2.6	Normal Flow

- Mutual authentication between the Car (M2M Gateway) and the Center (M2M Platform) is performed.

- Center requests the Car to report the integrity check on that Car.

- Support software which is installed in the Car runs integrity check of the Car assisted by TPM or the emulator.

- Generated integrity status/configuration information report is endorsed by the hardware key which is protected by TPM. This report may contain a detection of the newly implemented sensor(s) (M2M Device(s)).

- Support software sends the report based on TNC (Trusted Network Connect, which is application level secure communication protocol) to the Center.

- Center verifies the report securely based on the White list which is based outside the M2M network.

- Center determines whether the Car contains the software which shall be updated.

- Center selects corresponding software modules.

- Center delivers the support software module to the Car.

- The support software is applied at the Car.

- The applied result endorsed by the device key (actual process is done by TPM or the emulator) is reported to the Center.

- Center side confirms the completion of delivery/embedding.

- Center side stores the sequence of operations log as certifiable evidence for indemnity.

    - 6.2.7		Alternative Flow

None

    - 6.2.8	Post-conditions

Newly installed software/sensor(s) is correctly identified as authorized part(s) on the Car, and working correctly with installed support software. The Car’s integrity status/configuration information data which is endorsed by the hardware key which is protected by TPM or the emulator is sent to the Center side.

    - 6.2.9	High Level Illustration

Figure 6.2-1 Remote Maintenance Flow

Figure 6.2-2 Remote Maintenance High Level Illustration

    - 6.2.10	Potential Requirements

- The M2M service SHALL be able to provide the mechanism for authorization for integrity-checking and installing processes of software/hardware/firmware component(s) on M2M Device(s).

- The M2M system SHALL be able to support authentication using device key on the integrity check for M2M Device(s).

- The M2M Device SHALL be able to support HSM (Hardware Security Module) to protect its integrity depending on the security level requirement.

  - 6.3	Traffic Accident Information Collection

    - 6.3.1	Description

The Intelligent Transportation System (ITS) is mainly used for avoiding collision of vehicles. If doing some extension, an ITS can also be used for other purposes such as electronic payment of road tolls, traffic information collection and broadcast, local service advertisements, etc.

It is for sure that the ITS will save a lot of lives, but some traffic accidents will occur any way. So we still need rescue teams to go to the accident sites to help the victims and police to ease the traffic jam caused by the accident. A rescue team can make a more proper rescue plan if they are able to see the scene of accident. Similarly police can make a better traffic control plan if they are able to get an overview of traffic situation near the accident site.

This use case will show how the M2M technologies can help people to timely access to the detailed information of a traffic accident.

    - 6.3.2	Source

oneM2M-REQ-2013-0264R05 Use Case Traffic Accident Information Collection

    - 6.3.3	Actors

M2M Platform: It stores M2M data and runs M2M applications. It provides various M2M services to M2M service subscribers.

ITS Center: It is responsible for managing ITS on M2M Platform. It decides what service is provided to an ITS service subscriber.

Police Station: It is a subscriber of ITS service on M2M platform and responsible for controlling the traffic.

Rescue Center: It is a subscriber of ITS service on M2M platform and responsible for carrying out rescue missions.

ITS-Station (ITS-S): It is a kind of M2M Device installed in vehicles. It broadcast its travel status in a fixed interval in order to inform other ITS-S where it is. The ITS-S is equipped with a digital camera used for taking pictures according to the command given by a driver, ITS center or ITS-S itself. The ITS-S is able to communicate with M2M Platform through wireless network or a RSU using Dedicated Short Range Communications (DSRC).

Road Side Unit (RSU): It is a kind of M2M Gateway installed at roadside. The RSU is able to communicate with ITS-S using DSRC and communicate with M2M Platform through wired or wireless network.

    - 6.3.4	Pre-conditions

The ITS-Ss are equipped with a digital camera.

The ITS-Ss nearby the accident site are able to connect to M2M platform through either the wireless network or a RSU.

Police Station and Rescue Center are the subscribers of ITS services.

    - 6.3.5	Triggers

There are two ways to start an accident reporting process. One is the ITS-S involved in an accident detects the crash and then starts an accident reporting process automatically; the other is a driver in a passing by vehicle manually starts an accident reporting process through giving a command to the ITS-S in his vehicle.

    - 6.3.6	Normal Flow

- The ITS-S in the vehicle that is directly involved in an accident detects a crash has happened, and then starts an accident reporting process automatically.

- An accident reporting process may also be started manually. For example, a driver of a vehicle that is passing by the accident site stops and then manually starts an accident reporting process through giving a command to the ITS-S in his vehicle.

- The ITS-S first takes some pictures with its digital camera, and then uses these pictures together with current time and geographical coordinates to generate an accident report. This report shall be signed by the ITS-S.

- The ITS-S tries to connect to M2M Platform and then sends the accident report to the M2M Platform. (step 1 in figure 6.3-1)

- There are two ways for an ITS-S to connect to the M2M Platform. One is through wireless network; the other is through a nearby RSU using DSRC.

- The M2M Platform receives and verifies the accident report, and then does some necessary analysis. The analysis result will be pushed to the subscribers, i.e. the Police Station and the Rescue Center.

- The subscribers receive, verify and parse the information coming from M2M platform, and then do some necessary analysis. Based on different situation the subscribers may ask the M2M Platform to provide further information.

- In this scenario the Police Station asks the M2M Platform to provide an overview of the traffic situation near the accident site, and the Rescue Center asks the M2M Platform to provide more visual information about the accident. These service requirements are submitted to the M2M Platform.

- The M2M Platform receives and verifies the service requirements from Police Station and Rescue Center, and then sends data collection commands to the ITS-S that originally sends the accident report. (step 2 in figure 6.3-1)

- The command generated for Police Station requires the ITS-Ss near the accident site to report their travel status.

- The command generated for Rescue Center requires the ITS-Ss around the accident site to provide pictures.

- The ITS-S that originally sent the accident report receives the commands sent from the M2M Platform. It verifies and parses the commands, and then broadcasts the commands that should be broadcasted. (step 3 in figure 6.3-1)

- In this scenario the broadcasted commands are generated by the M2M platform for Police Station and Rescue Center respectively.

- The ITS-Ss nearby the accident site receive, verify, parse and execute received commands, i.e. take pictures, get current travel status, generate reports, sign the reports and upload signed reports to M2M Platform. These reports could be sent anonymously. (step 4 in figure 6.3-1)

- Some commands need to be rebroadcasted within a predetermined area and predetermined period of time. (step 5 in figure 6.3-1)

- In this scenario the command generated for the Police Station needs to be rebroadcasted. The ITS-Ss receive this command will only report their travel status. (step 6 in figure 6.3-1)

- M2M Platform accumulates and verifies the reports uploaded by the ITS-Ss, and then generates a report contain visual information about the accident scene for the Rescue Center and a report about traffic situation near the accident site. These reports will be pushed to Rescue Center and Police Station respectively.

- The Rescue Center analyzes the report about the accident scene, and then makes a proper rescue plan. The Police Station analyzes the report about traffic situation, and then makes a proper travel control plan.

    - 6.3.7	Alternative Flow

None

    - 6.3.8	Post-conditions

Based on the detailed information provided by the ITS service on the M2M platform, the rescue team can make a proper rescue plan, and the police can make a proper travel control plan.

    - 6.3.9	High Level Illustration

Figure 6.3-1 High Level Illustration of Traffic Accident Information Collection

    - 6.3.10	Potential Requirements

- A M2M System shall support communication between M2M Platform and a M2M device either directly or via a gateway.

- A M2M System shall be able to exchange information between M2M applications via M2M Platform.

- A M2M System shall be able to take actions according to the received service requests from M2M Applications.

- A M2M system shall be able to support service requests from M2M applications for communication with QoS requirement, such as, higher delivery priority, reliable delivery, etc.

- A M2M System shall support mutual-authentication among M2M device, M2M gateway, M2M platform and M2M Application.

- The information sent by a M2M device or the M2M platform or a M2M application shall use cryptographic technology to ensure information authentication and information integrity.

- A M2M system shall permit information being provided in anonymous way.

- A command issued by a M2M System shall be able to have time expiration or geography restriction.

  - 6.4	Fleet Management Service using DTG (Digital Tachograph)

    - 6.4.1	Description

“DTG-based fleet management service” is the fleet management services utilizing DTG data and related service, to facilitate extensive service features of fleet management.

DTG provides vehicle data such as driving speed, RPM (Revolution Per Minute), brake’s status, and mileage, etc.

DTG data management service, based on M2M gateway and DTG data management server, reports and manages DTG data in real-time to store it in the memory of M2M device in vehicle at a certain rate (i.e. one second in this case) to submit it to the national authority or transfer it to central office managing the data in a server.

The fleet management service utilizing the above mentioned service functionality provides advanced service features such as the precise quest of vehicles based on location and the tracking of cargo along with the route of the carrier vehicle, by means of the capability of remote monitoring and control of vehicle status provided by the DTG data management service.

    - 6.4.2	Source

oneM2M-REQ-2013-0219R01 Use case – Fleet management using DTG

    - 6.4.3	Actors

- DTG device manufacturer to provide DTG devices and DTG management system

- M2M device manufacturer to provide M2M gateway and related functionalities

- The service provider for fleet management service using DTG

- The network provider supporting the communication for fleet management service

- The national agency that manages and operates DTG data (in case of Korea)

    - 6.4.4	Pre-conditions

- The DTG device records the DTG data occasionally or periodically to transfer it to an application server through M2M Gateway.

- M2M service gateway delivers the DTG data, useful to fleet management service, from terminal system to the application server.

- Application server provides fleet management service, using DTG data, to customer.

- A taxi call service provider operates fleet management service using DTG data, such as for reporting the taxi location and passenger status and for call arrangement.

- A bus traffic service provider operates fleet management service using DTG data, including for providing guide information on bus arrival/estimated time, bus schedules on web-site, and status information such as route and air pressure of tire, etc.

- A fleet management service provider of truck operates fleet management service based on vehicle information (location, route, gas, tire pressure etc.) and peripheral device information (temperature, humidity, door lock and goods weight etc.)

    - 6.4.5	Triggers

The following triggers could initiate the information exchanging process according to the flows described hereafter followings:

- Creation of DTG data that M2M device occasionally or periodically transfers to an application server.

- Arrangement of taxi service calls delivered to a DTG device.

- Report of information about vehicle location and route to application server.

    - 6.4.6	Normal Flow

DTG service (Common service)

- DTG data is periodically (normally once in a second in this case) transferred and stored into DTG management server, and when in case of an event of accident, the data is stored at an immediate mode. (within 10ms in this case).

- The DTG data stored in DTG device will be transferred to DTG management server periodically, and once after the engine stopped.

- The DTG management server stores DTG data and accident event file which is to be posted onto the web site of national agency.

- Analysis of DTG data and accident data to provide driving behavioural habits (quick start/stop, excessive speed) or the accident causes.

Figure 6.4-1 Taxi Call Service Normal Flow

Taxi call service

- Terminal system occasionally or periodically reports location, passenger status information to application server (FMS server).

- Customer requests taxi call service to the taxi call center through a phone call or smart phone application.

- Taxi call center sends call request to a terminal system in the taxi through the application server.

- The taxi driver accepts the call request through the terminal system, and then the taxi will come to the customer’s location.

Figure 6.4-2 Normal Flow - Fleet Management Service (Truck)

Fleet Management Service (Truck)

- Terminal system occasionally or periodically reports the vehicle status information including the location, current route, ignition status, terminal version, and driver information to application server (FMS server).

- When the application server receives the information, it delivers it to logistic management center.

- Terminal system also reports the peripheral information (air pressure of tire, gas gauge, temperature, humidity, door lock etc.) to the logistics management center through the application server.

- Logistic management center can request the information about vehicle itself or peripheral device, to enforce possible controls to them when it is needed.

- Terminal system reports the emergency events, such as fire in car, unlocked doors when unattended, and puncture while driving, etc. to FMS server

Figure 6.4-3 Normal Flow - Fleet Management Service (Bus)

Fleet Management Service (Bus)

- When the application server receives the vehicle information (engine ignition, terminal version, car S/N, and driver ID, etc.) from terminal system, it provides the received information to the BTS management server.

- BTS management server sends time schedule, route of bus and the fare information to terminal system through the application server (FMS server).

- Terminal system sets the time schedule, the route, and the fare information. And then it occasionally or periodically reports its location and the driving route to application server.

- Terminal system also reports the information about peripheral devices such as air pressure of tire, gas gauge level, and bus fare status to BTS management server occasionally or periodically.

- BTS management server provides an arrival/estimated time and a bus schedule on web-site.

    - 6.4.7	Alternative Flow

None

    - 6.4.8	Post-conditions

None

    - 6.4.9	High Level Illustration

Figure 6.4-4 High Level Illustration Fleet Management

    - 6.4.10	Potential Requirements

- Provisioning, installation, configuration and registration method of terminal system

  - Especially for the case of overlapping two different system for DTG management system (owns and manages the device) and the application system using DTG data (utilising the data from the device).

- DTG/FMS data storing method and delivery protocol

  - There is no dominant standard specifying data formats and protocols for vehicle related applications.

- Vehicle location based service method

  - M2M service platform is expected to provide the service capability supporting location based service.

- Control, configuration, error logging, and management method for the terminal system Over The Air.

  - M2M service platform is expected to provide the service capability supporting the Over The Air management.

  - 6.5	Use cases for Electronic Toll Collection (ETC) service

    - 6.5.1	Description

ETC is an important part of Intelligent Transportation System (ITS). It is vigorously promoted in many countries.

ETC aims to eliminate the delay on toll roads by collecting tolls electronically. ETC determines whether the vehicles passing are enrolled in the program, alerts enforcers for those that are not, and electronically debits the accounts of registered vehicle owners without requiring them to stop.

In the ETC Use Case, On Board Unit (OBU) is a dedicated device located in the vehicle and it can communicate with the local RSUs only. Stated differently, the communication between OBU and ETC platform is via the RSUs. With a vehicle moving, the OBU would connect to the next RSU and release connection with the previous RSU. It is necessary for the OBU to 'register' with ETC Service Platform for receiving M2M system services. The term 'registration' here refers to an OBU having its contextual information available at the ETC Service Platform.  Such registration information is for the duration of an OBU being subscribed to services from the ETC Service Platform. At the same time, OBU 'registers' with local RSUs also, as OBU connects to different RSUs. Such registration context at the RSUs is temporary, and gets released as the OBU moves out of the range of the RSU and connects to the next RSU. The OBU uses its connectivity with local RSUs for achieving communication with ETC Service Platform.

As regards the 'registration context' between the OBU and the ETC Service Platform, the contextual information needs to reflect the complete profile of the OBU at the ETC Service Platform. Such information can include OBU identity, credentials, service subscription information, payment history, account balance etc. This is referred to here as 'full registration'.

As regards the 'registration context' between the OBU and the RSU, such contextual information is a subset of the contextual information at the ETC Service Platform for the said OBU. The information in this subset needs to be sufficient for the RSU to identify the OBU to the ETC Service Platform for the OBU to receive desired services without compromising any sensitive information to the RSU. This is referred to here as 'lightweight registration'

    - 6.5.2	Source

REQ-2014-0431R03 Use cases for Electronic Toll Collection (ETC) service

REQ-2014-0449R02 Use cases for Electronic Toll Collection (ETC) service

    - 6.5.3	Actors

- Vehicle Owner enrolls for ETC service.

- On Board Unit (OBU) is a M2M device used to store information such as identifier of the vehicle. The OBU typically does not communicate with the ETC Service Platform directly while receiving services. Direct communication between OBU and the ETC Service Platform may however be supported, out of band, for example for subscribing for ETC services.

- Road Side Unit (RSU) is a device which is an intermediate entity and is available to connect from OBU to the ETC Service Platform. Typical role of RSU can be implemented as an M2M gateway, and it may provide other functions as well (e.g., traffic light control, barrier control etc.).

- ETC Service Platform is responsible for collecting the information regarding the OBU via the RSU, including information such as location information from the RSU.

- ETC Service Provider provides its own M2M services for the user (OBU) through the ETC Service Platform.

    - 6.5.4	Pre-conditions

- Vehicle owner enrolls his/her vehicle for ETC services.

- All RSUs have their respective context (registration information) available at the ETC Service Platform.

    - 6.5.5	Triggers

A vehicle equipped with OBU drives close to an electronic road charging station which equipped with RSU.

    - 6.5.6	Normal Flow

Figure 6.5-1 Normal Flow for Electronic Toll Collection (ETC) service

- Vehicle owner subscribes to ETC services and the OBU is 'registered' with the ETC Service Platform. All RSUs are also registered with the ETC Service Platform.

- Note: The term 'registration' refers to communication and resulting contextual information of the registering entity at the registered entity.

- When the vehicle goes to a highway entrance (location A), OBU registers with RSU-A.

- The Vehicle (OBU) sends a request to ETC Service Platform via RSU-A asking for permission to pass through.

- RSU-A forwards the request from the OBU along with its own identifying information e.g. location information, to the ETC Service Platform.

- ETC Service Platform processes the request for the OBU, received from RSU-A, and responds to RSU-A. The processing at ETC Service Platform can include charging the 'toll' to OBU account and updating account balance etc., for the OBU. The response to RSU-A includes the recommended action for the OBU for pass through.

- RSU-A decides whether vehicle can pass through according to the response from the ETC Service Platform.

- The vehicle drives to the highway exit at location B, and OBU registers to RSU at location B (RSU-B).

- The vehicle sends a request to ETC Service Platform via RSU-B asking for permission for pass through.

- RSU-B forwards the request for the OBU along with its own information e.g. location information to the ETC Service Platform.

- ETC Service Platform processes the request for the OBU received from RSU-B. The processing at ETC Service Platform can include charging the 'toll' to OBU and updating account balance etc. for the OBU. The ETC Service Platform may communicate with the ETC Service Provider, as needed, while processing the request from the OBU.

- ETC Service Platform responds to RSU-B by including information such as the recommended action for the OBU to pass through.

- RSU-B decides whether vehicle can pass through according to the response from the ETC Service Platform.

    - 6.5.7	Alternative Flow

None

    - 6.5.8	Post-conditions

None

    - 6.5.9	High Level Illustration

Figure 6.5-2 High Level Illustration for Electronic Toll Collection (ETC) service

    - 6.5.10	Potential Requirements

- The M2M system shall provide the capability for an M2M device to maintain registration with multiple entities simultaneously.

- The registration shall be able to include information that identify the peer entity, and other information such as its management privilege, subscription etc., that are necessary for the conduct of the respective peer relationships.

- It shall be possible for some registrations to hold the complete set of information context about the peer entity. This is refereed to as "full registration".

- It shall be possible for some registrations to hold only a subset of information context about the peer entity. This is referred to as "lightweight registration".

- It shall be possible for "lightweight registration" at different entities that pertain to a common peer entity, to hold different sets of information, if needed, about the common peer entity.

- It shall be possible to correlate the "full registration" and the "lightweight registration" that pertain to a common peer entity.

- It shall be possible to distinguish the "full registrations" and the "lightweight registrations" that pertain to a common peer entity.

  - 6.6	Use cases for Taxi Advertisement

    - 6.6.1	Description

For the taxi advertisement device, it does NOT need to access to the network during the day when the network is busy (e.g. from 8:00 to 23:00), but need to access the network at night when the network is idle (e.g. from 23:00 to 8:00) to download a large number of advertisement data.

    - 6.6.2	Source

REQ-2014-0467R02 Use case for taxi advertisement

    - 6.6.3	Actors

- Taxi advertisement device, which can download advertisement data from advertisement data server through M2M service platform and show to passengers.

- The M2M service platform, which can control the taxi advertisement device and its access to the network

- Advertisement data server, which can provide advertisement data for the advertisement device to download

    - 6.6.4	Pre-conditions

The taxi advertisement device and the advertisement data server registered to the M2M service platform.

    - 6.6.5	Triggers

The taxi advertisement device accesses to the advertisement data server though M2M platform to download data.

    - 6.6.7	Normal Flow

- The taxi advertisement device accesses to the advertisement data server through M2M platform.

- The M2M service platform checks the time policy. If current time is permitted, it allows the device to access the advertisement data server. Otherwise, it denies access and instructs the taxi advertisement device not to access the advertisement data server until the time when access is permitted.

- When access is permitted the taxi advertisement device downloads data from advertisement data server.

    - 6.6.7	Alternative Flow

None

    - 6.6.8	Post-conditions

None

    - 6.6.9	High Level Illustration

Figure 6.6-1 High Level Illustration of Taxi advert use case

    - 6.6.10	Potential Requirements

- The M2M service platform shall be able to support the time-based policies to access the Underlying network.

  - 6.7	Use Case on Vehicle Data Service

    - 6.7.1	Description

This use case introduces several services based on various data collected by sensor devices via smart vehicle (vehicle with on-board communication system) which is regarded as M2M gateway.

The sensor devices may be located in the vehicle but may also be located outside the vehicle, e.g. on the road side. Some sensor devices are equipped with M2M area network module and measure individual data. The smart vehicle connects to the sensor devices and collects data from sensor devices by using the M2M area network technology such as Wireless LAN, ZigBee, Bluetooth, etc., and sends the data to application server in infrastructure domain via mobile network.

Management server and Application Server in the M2M infrastructure domain connect to the smart vehicle via a mobile network in order to control its configurations updating software and exchanging M2M data (e.g. updating a map).

It is important to observe that the smart vehicle as M2M gateway has mobility. For instance, there are possibilities for a mobile device to simultaneously connect to many sensor devices, and to newly connect to sensor devices which have never been connected before.

This use case illustrates potential requirements from the use case of services utilizing mobile device.

    - 6.7.2	Source

REQ-2014-0472R06: Use Case on Vehicle Data Services

    - 6.7.3	Actors

- M2M Device: In-vehicle sensor device and outdoor sensor device. In-vehicle sensor includes on-board (built-in) sensor used for monitoring of machine health and also sensor on user carry-on device, such as smart phone and wearable device like Fitbit™, Nike Fuelband™, Sony SmartBand™ for example. In-vehicle sensor may monitor status of machine health (diagnosis), mobility, passenger’s health and environment. Outdoor sensor device may be located outdoor and monitor roadway infrastructure, agriculture, property (surveillance) and utility (telemetering), for example. It may be equipped with several kind of communication protocol.

- M2M Area Network: Area network which connects M2M Device with M2M Gateway and also provides connectivity among M2M Gateways. It may include Wireless LAN, Bluetooth, ZigBee and Ethernet.

- M2M Gateway: On-board communication system equipped on the smart vehicle, which communicates with M2M Devices and other M2M Gateways via M2M Area Network and also communicates with Application Servers and Management Servers via Mobile Network.

- Mobile Network: Network which has functions to transfer data and control messages between M2M Gateway and M2M Application Server/Management Server. It may include cellular base station.

- M2M Management Server: Server which manages M2M applications in M2M Gateway and M2M Device by installing, uninstalling and updating them.

- M2M Application Server: Server which maintains database and provides the data access services such as accepting data publication from and issuing data subscription to M2M Gateway/Device. This server also manages non-M2M applications such as navigation system as well as contents such as map data in M2M Gateway and M2M Device by installing, uninstalling and updating them.

    - 6.7.4	Pre-conditions

- It is possible to establish a connection among M2M Application Server, M2M Management Server and Smart Vehicle via Mobile Network.

- It is possible to establish a connection between Smart Vehicle and M2M Device and among Smart Vehicles via M2M Area Network.

- The M2M Gateway has been configured by the M2M Management Server.

- The M2M Device has not yet been configured by the M2M Management Server.

    - 6.7.5	Triggers

Subject to capabilities of the M2M Area Network, the M2M Device detects a M2M Gateway that can be associated, or vice versa. The association may require explicit operation (permission) for that association by users (e.g. owners of Gateway and/or Device) or administrator (a manufacturer of Gateway and/or Device and/or Management Server). Examples are below:

- Sensors are built into the smart vehicle at the factory (Permanent Association, no triggering is needed).

- Sensors are equipped to the smart vehicle after market, e.g. at auto dealers or auto parts stores. Triggering occurs when the smart vehicle is being equipped with the new sensor

- Users bring sensors with user carry-on devices into the smart vehicle.

- The smart vehicle detects nearby outdoor sensors.

- The smart vehicle detects other smart vehicles nearby.

    - 6.7.6	Normal Flow

M2M Configuration step: Management Server configures M2M Device

Upon triggering, i.e. when the M2M Device detects a M2M Gateway that can be associated or the M2M Gateway detects a M2M Device that can be associated the following sequence is initiated:

M2M Gateway establishes a connection to M2M Device (or the M2M Device establishes the connection to the M2M Gateway) via the M2M Area Network.

M2M Device sends its attribute information (e.g. type of device, service certificates of the device, required application software, etc.) to the M2M Gateway.

M2M Gateway establishes a connection to the M2M Management Server via Mobile Network.

M2M Gateway relays the attribute information to Management Server.

M2M Management Server provides M2M Gateway with the appropriate software and configuration data for the M2M Device.

M2M Gateway relays the software and configuration data to M2M Device.

M2M Device configures itself according to the software and configuration data.

Transmission of data between M2M Device and M2M Application Server

- Data Publication of M2M Device

- M2M Gateway (re-)establishes a connection to M2M Device via M2M Area Network.

- M2M Device publishes measured data to M2M Gateway.

- M2M Gateway establishes a connection to M2M Application Server via Mobile Network.

- M2M Gateway relays the data to M2M Application Server.

- M2M Application Server receives the data.

Data Subscription from M2M Application Server

- M2M Gateway establishes a connection to M2M Application Server via Mobile Network.

- M2M Application Server sends M2M Gateway attribute information of data in need.

- M2M Gateway (re-)establishes a connection to M2M Device via M2M Area Network.

- M2M Gateway relays the attribute information to M2M Device.

- M2M Device publishes measured data that meets the attribute information to M2M Gateway.

- M2M Gateway relays the data to Application Server.

- Application Server receives the data.

Update of non-M2M Application 1: M2M Application Server and M2M Gateway

- M2M Gateway establishes a connection to M2M Application Server via Mobile Network.

- M2M Gateway sends its attribute information to M2M Application Server.

- M2M Application Server provides M2M Gateway with the appropriate software, its configuration data and contents such as map.

- M2M Gateway configures itself according to the software, configuration data, etc.

Update of non-M2M Application 2: M2M Application Server and M2M Device

- M2M Device establishes a connection to M2M Gateway via M2M Area Network.

- M2M Device sends its attribute information to M2M Gateway.

- M2M Gateway establishes a connection to M2M Application Server via Mobile Network.

- M2M Gateway relays the attribute information to M2M Application Server.

- M2M Application Server provides M2M Gateway with the appropriate software and configuration data.

- M2M Gateway relays the configuration software and data to M2M Device.

- M2M Device configures itself according to the software and configuration data.

    - 6.7.7	Alternative Flow

Alternative Flow 1

This alternative flow may occur in the case where the M2M Gateway only occasionally connects to devices and servers – e.g. via M2M Area networks that can only occasionally be used.

- M2M Gateway Opportunistic Communication (Store and Forward)

  - M2M Gateway may store data that are destined to M2M Device, M2M Management Server and M2M Application Server.

  - M2M Gateway may send the stored data to other M2M Gateway.

  - Both, the originator and the other M2M Gateway may deliver the stored data to the destination when connected.

  - M2M Gateway may erase the stored data that has been already sent to the destination.

Alternative Flow 2

This variant flow may occur in the case where the M2M Gateway processes the data flow between M2M

Gateway and M2M Application Server.

- M2M Gateway Data Processing

  - M2M Gateway may aggregate, statistically summarize (e.g. average) and/or erase data based on criteria that are indicated in the data subscription or the configuration data from M2M Application Server.

Alternative Flow 3

This variant flow may occur in cases where the M2M Gateway broadcasts its interest to subscribe to specific data to all M2M Devices in its vicinity.

- M2M Gateway Cross Layer Optimization

  - M2M Gateway may indicate a full or part of data subscription (attribute information of data in need) in wireless pre-association information such as beacon.

  - Smart Vehicle and Wi-Fi Hotspot may periodically broadcast beacon that contains the subscription in order to prevent sensor devices from establishing unnecessary connection and wasting radio resources as well as battery power.

Note:  M2M Application Data Security (TBD)

    - 6.7.8	Post-conditions

- M2M Application Server stores data and provides data access service via API for user applications.

- M2M Devices and M2M Gateways are well maintained by M2M Management Server and M2M Application Server.

    - 6.7.9	High Level Illustration

    - 6.7.10	Potential requirements

- The oneM2M System shall enable discovery of M2M Application Servers, M2M Management Servers and M2M Devices available to an M2M Gateway for data exchange.

- The oneM2M System shall enable discovery of M2M Gateways available to a M2M Management Server and an M2M Device for data exchange.

- The oneM2M System shall be able to support the capabilities for data repository (i.e. to collect/store) and for data transfer from one or more M2M Devices or M2M Gateways, for delivery to one or more M2M Gateways via M2M Area Network without any assistance or instruction of M2M Management Servers and M2M Application Servers.

- Upon request from M2M Application Server, an M2M Gateway shall enable functions that pre-process (e.g. average) M2M data before providing them to the recipient.

- Upon request, an M2M Gateway shall enable functions that erase M2M data (e.g. that have been sent or could not be sent to the recipient within a certain time) based on criteria from an M2M Application Server.

- An M2M Gateway and/or an M2M Device shall be able to broadcast to all M2M Devices and/or M2M Gateways in the vicinity its need to receive/deliver specific data.

- M2M Gateway and/or M2M Device shall be able to establish a connection to each other if it is able to receive/deliver the required specific data.

  - 6.8	Smart Automatic Driving

    - 6.8.1	Description

An attention to an automatic driving is increasing. An automatic driving technology normally superimposes dynamic information gathered by car-mounted sensors and/or cameras on a static high-definition map. It makes possible to calculate the way which is appropriate for the vehicle to take.

Because an automatic driving vehicle gathers dynamic surrounding circumstances by sensors and/or cameras, the coverage of the vehicle’s perception is normally almost a few hundreds of meters. Therefore, the automatic driving vehicle is forced to make a sharp lane change, a sudden stop or a return of its control to a driver at the worst in a case of unusual state such as a crash, a road work or a dropping on the road.

In order to make a smooth lane change, a gradual slow down or a fully prepared return of its control to a driver even in such unusual states, the system which can collect current road conditions and feedback credible information to relevant vehicles is required. The information can be used by each vehicle to calculate the driving way such as a lane, path or speed.

    - 6.8.2	Source

REQ-2015-0554-Smart Automatic Driving

    - 6.8.3	Actors

- Vehicle Driving Support Centre:

It distributes high-definition maps to vehicles supporting an automatic driving. Furthermore, it collects unusual states from vehicles on the road, validates its credibility and feeds back credible information to relevant vehicles. If there are devices/GWs located at the roadside, some of the functions of vehicle driving support center may be provided via these devices/GWs.

- OneM2M System:

It connects between the vehicle driving support centre and vehicles. Furthermore, the devices/GWs located at the roadside may provide some of the functions of vehicle driving support center.

- Vehicles:

The automatic driving vehicles have multiple on-board sensors and cameras for the automatic driving which superimposes dynamic information gathered by the sensors and/or cameras on a static high-definition map distributed by the vehicle driving support centre. Furthermore, the automatic driving vehicles use unusual states information from the centre to calculate the driving way such as a lane, path or speed.

    - 6.8.4	Pre-conditions

- Some vehicles support an automatic driving.

- The automatic driving vehicles have a mobile communication module and a function to notice their own location and on-board camera image to the vehicle driving support centre.

    - 6.8.5	Triggers

The automatic vehicles report occurrence of an unusual state such as a crash, a road work or a dropping on the road.

    - 6.8.6	Normal Flow

- The automatic driving vehicles reaching the location where the unusual state occurs detect the state by on-board sensors and cameras. As a result, the vehicles change a lane, slow down, stop or return the control to the driver, which become sudden operations depending on those situations. Additionally, the vehicles report the unusual state to the vehicle driving support centre. The state is reported as on-board camera image with additional information such as current time and locations of the vehicle.

- The centre receiving those reports identifies the location where the unusual state occurs, compares the reports which seem to be sent from same location and validates its credibility.

- The centre notices the unusual state to the vehicles which are reaching the location of the unusual state after finishing the credibility validation. The notice is protected by an appropriate way such as digital signature, because this information is used for each vehicle to calculate the way of the automatic driving.

- The automatic driving vehicles receiving the notice from the centre make a smooth lane change, a gradual slow down or a fully prepared return of its control to the driver before reaching the location of the unusual state.

    - 6.8.7	Alternative flow

None.

    - 6.8.8	Post-conditions

None.

    - 6.8.9	High Level Illustration

Figure 6.8-1 High Level Illustration of smart automatic driving

    - 6.8.10	Potential requirements

- OneM2M System shall be able to send the information to intended vehicles by unicast, multicast and/or broadcast.

- OneM2M System shall be able to securely transfer the information by using an appropriate way such as digital signature.

- OneM2M System shall be able to transfer the information on real-time basis for feeding back current road states to automatic driving control. The feedback time should be less than a few seconds (the distance between vehicles is normally worth of a few seconds) to avoid unnecessary speed down/stop of following vehicles.

- OneM2M system shall be able to guarantee its reliability in order to receive/feedback massages from/to related vehicles.

- oneM2M System shall enable sharing of service information between devices/GWs based on proximity.

- oneM2M System shall enable sending and receiving of service information between devices/GWs with minimized interruption.

  - 6.9	Use Case on Vehicle Data Wipe Service

    - 6.9.1	Description

This use case introduces vehicle data wipe services in addition to the use case on vehicle data services as REQ-2014-0472R06.

Background: A data center on the cloud collects sensor data from vehicles using the mobile network (e.g. cellular and wireless LAN). The data may relate to diagnosis, mobility and context of vehicles. The diagnostic data are useful for vehicle design improvement and the mobility data for dynamic route guidance services. The contextual data are captured by stereo cameras and radar scanners in the automated driving system. And then those data may constitute the complete 3D roadway map which is also essential for the automated driving system in turn. In the use case of REQ-2014-0472R06, the data center, that is the M2M application server, requests data by sending a meta-data to vehicles. Then vehicles prepare data which meet the criteria of the meta-data and publish it to the server. The meta-data may describe attributes of requested data such as time period, geographical area, data type, statistic process options and so forth. The figure in section 1.1.9.1 shows how this simply works.

Problem statement: Though a certain volume of data (big data) obtains utility value, the bigger data gradually show less increase of the value (as is shown in the figure in section 1.1.9.2.) For instance, no more samples are required for statistical analysis once the number of samples gets sufficient. Duplicated collections of collected pieces of map are also redundant. Such wasteful data deliveries consume multiple expensive resources of vehicles, mobile network, network backhaul and servers. Data in the vehicle may also lose its value when it gets delivered, obsolete, false or even malicious.

Solution: Such the unwanted data collection needs to be cut off in order to spare those resources for other useful data delivery. In addition, unwanted data in a limited capacity of the vehicle storage need to be wiped out. These operations should be carried on under the right authorization.

    - 6.9.2	Source

REQ-2015-0589R04-Usecase_on_vehicle_data_wipe_service

    - 6.9.3	Actors

- M2M Device: Data source node such as sensors in vehicles.

- M2M Gateway: Data relay node such as a vehicle on-board communication system with storage.

- M2M Application Server: Data requesting node such as a data center on the cloud.

    - 6.9.4	Pre-conditions

The M2M Application Server has disseminated a data request with the meta-data to vehicle(s) (as is shown in the figure in section 1.1.9.3.)

Note: the meta-data may describe attributes of requested data such as time period, geographical area, data type, statistic process options and so forth.

    - 6.9.5	Triggers

The M2M Application Server decides to cancel the data request and erase the data in the vehicles. (One of the reason for this could be that the data has been collected.)

    - 6.9.6	Normal Flow

- The M2M Application Server transmits a wipe request with the same meta-data in the target data request to the vehicle(s).

- On arrival of the wipe request, the vehicle deletes the meta-data and data which meet the criteria of meta-data from in its storage.

    - 6.9.7	Alternative flow

- If it is additionally instructed in the wipe request, the wipe request can be delivered from the vehicle to other vehicles.

- If it is additionally instructed in the wipe request, the vehicle may only stop publishing data which meets the criteria of meta-data, instead of deleting.

- The M2M Application Server and vehicles may share a credential and use the credential to authenticate the wipe request (as is shown in the figure in section 1.1.9.5.) The credential could be pre-shared key or could be based on public key infrastructure. The credential could also be delivered with the data request.

    - 6.9.8	Post-conditions

There is no data relating to the cancelled data request in the vehicle storage.

    - 6.9.9	High Level Illustration

      - 6.9.9.1	Data Request and Response

      - 6.9.9.2		Issue of Bigger Data

      - 6.9.9.3		Pre-condition of Data Wipe (and Post-condition of Data Request and Data Response)

      - 6.9.9.4		Data Wipe

      - 6.9.9.5		Data Wipe with Authentication

      - 6.9.9.6		Post Condition of Data Wipe

    - 6.9.10	Potential requirements

- The oneM2M System shall enable the cancellation of continuous data collection and/or the deletion of collected data when pre-defined conditions are met.

- The oneM2M System shall enable pre-defined conditions to be protected from unauthorized modification.

- The oneM2M System shall enable the deletion of M2M data produced/stored by the M2M Devices/Gateways based on request from an authorized entity.

  - 6.10	Vehicle Management based on Geo-Fence

    - 6.10.1	Description

Since a vehicle is a nomadic object, localization of vehicles and collecting information for management are crucial features for vehicle management.

From the holistic viewpoint, vehicle management is a generic term and may address various vehicle and smart car services such as diagnostic, maintenance and so on.

Specifically, in this use case, vehicle management based geo-fence feature is about how to monitor the location and movement of a target vehicle efficiently (e.g. judge where the movement is illegal or legal for example)

    - 6.10.2	Source

REQ-2016-0002R01 Vehicle Management based on Geo-fence

    - 6.10.3	Actors

- Vehicle:

Vehicle is a nomadic object which is monitored by oneM2M infrastructure node or Location Server and it equips hardware devices for localization. (e.g. GPS, Cellular modem)

- oneM2M Infrastructure Node:

A system exposes the service functions for vehicle management such as Location, Device management and potentially additional services.

- Location Server:

A Location Server can localize the target device(s) using collected measurements (e.g., RSSI, AP ID) and detect Geo-Fence event based on set configurations.

- LBS/IoT Application:

An Application requests Geo-Fence based location service toward oneM2M Infrastructure Node.

    - 6.10.4	Pre-conditions

N/A

    - 6.10.5	Triggers

A LBS/IoT Application requires to set Geo-Fence-based Location-Based services depending its demand.

    - 6.10.6	Normal Flow

- A vehicle Device registers with oneM2M Infrastructure.

- A LBS/IoT Application requests the Geo-Fence Service to oneM2M Infrastructure. The Application set the configuration regarding Geo-Fence Service such as Geo-Fence area, event criteria, quality of event detection and so on.

- oneM2M Infrastructure acquires Measurement and/or Position fix from the Vehicle devices.

- The Measurement and/or Position fix are/is forwarded to Location Server by oneM2M infrastructure.

- Location Server runs Geo-Fence detection engine and analyzes whether the Geo-Fence event has occurred or not. The detection is operated following the configuration of application.

- When any event has occurred, the event reports to the LBS/IoT Application.

    - 6.10.7	Alternative flow

N/A

    - 6.10.8	Post-conditions

N/A

    - 6.10.9	High Level Illustration

- Vehicle Devices can gather the signal measurement and/or position fix

- The Devices are registered and connected to oneM2M Infrastrucutre

- GeoFence Server can detect any movement event based on the Application’s configuration

    - 6.10.10	Potential requirements

- The oneM2M System shall support reporting of Geo-Fence based Location event of the target M2M Device to the M2M Application based on the Application’s configuration.

- The oneM2M System shall support the M2M Application setting the configuration for Geo-Fence based location service.

  - 6.11	Use Case on Secure Over-The-Air Firmware Update for Automotive ECUs

    - 6.11.1	Description

This use case introduces secure remote Over-The-Air (OTA) Firmware Update scheme for M2M Devices such as ECUs (Electronic Control Units) through on-board M2M Gateway in automobiles.  An automobile which has an on-board M2M GW is called “Smart Vehicle”.

Each M2M Device is connected to the on-board M2M Gateway Unit by M2M Area Network such as Controller Area Network (CAN).

M2M Management Server in Infrastructure Domain collects information on version of currently installed firmware in ECUs and distributes updated firmware dedicated for each M2M Device through the M2M Gateway using Mobile Network.

    - 6.11.2	Source

REQ-2016-0012R01 Use Case on OTA Firmware Update for ECUs

    - 6.11.3	Actors

- M2M Device: Electronic Control Unit (ECU) in an automobile is assumed as an M2M Device. There are many kinds of on-board ECUs in automobile, e.g. Power train ECU, Transmission ECU, Brake ECU, etc.

- M2M Gateway (M2M GW): M2M GW is connected with M2M Devices to collect M2M Data and distribute firmware through multiple M2M Area Networks depending on kinds of functionality or security requirements. M2M GW is equipped with a data communication module to communicate with M2M Management Servers and external servers in Infrastructure Domain. Also, it has a display device in order to confirm user’s authorization for the firmware update operation.

- M2M Area Network: CAN (Controller Area Network) or LIN (Local Interconnect Network) is used as wired M2M Area Network in an automobile.

- Mobile Network: Mobile Network is assumed to transfer M2M Data and messages between M2M GW and M2M Management Server.

- M2M Management Server：This server is utilized to conduct firmware update operation and management of  version of firmware currently installed in M2M Devices.

- OEM：Original Equipment Manufacturer means automobile manufacturer who is ultimately responsible for the product (e.g. firmware for ECU).

- Firmware Developer: A developer of firmware dedicated for on-board M2M Device (ECU).

- User：A person who makes a decision for conducting firmware update of ECU, when a request for the update is notified from M2M Management Server. This assumes to be an owner of automobile.

Figure 6.11-1 High Level Structure for Over-The-Air Firmware Update for Automotive ECUs

    - 6.11.4	Pre-conditions

- M2M GW is connected to M2M Management Server via Mobile Network.

- M2M GW is connected to M2M Device through CAN.

- M2M GW has capability to manage status of on-board equipments in Smart Vehicle.

    - 6.11.5	Triggers

- Trigger from Server

  - When a new firmware becomes available, M2M Management Server notifies it to M2M GW and M2M GW notifies it to User via a display device of M2M GW.

- Trigger from User

  - User can operate M2M GW to check availability of updated firmware for ECUs in M2M Management Server.

    - 6.11.6	Normal Flow

- A: Generation of updated firmware

- Firmware Developer generates updated firmware for an ECU and submits it to OEM for approval.

- OEM verifies the updated firmware and attaches OEM’s Digital Signature to it as token of approval.

- OEM uploads the updated firmware with the Digital Signature to M2M Management Server.

- B: Authentication

M2M Management Server and M2M GW conduct mutual authentication. It is possible to avoid unintentional outflow of firmware by distributing it to the right M2M GW using the authentication mechanism. M2M GW can conduct firmware update operation based on request from the trusted Management Server. Widely used authentication procedures such as Challenge and Response using shared key cryptosystem and public key cryptosystem are applicable.

- C: Establishment of Secure Channel

Encryption of communication channel between M2M Management Server and M2M GW is carried out after the mutual authentication. Encryption procedure such as SSL is applicable.

- D: Verification of current version of Firmware installed in M2M Device

- M2M Management Server sends a request to M2M GW in order to verify version information of firmware currently installed in M2M Device.

- M2M GW transfers the verification request to M2M Device.

- M2M Device sends version information of firmware currently installed back to M2M GW together with its own Digital Signature.

- M2M GW transfers the version information with the Digital Signature of M2M Device to M2M Management Server.

- Management Server verifies the version information with the Digital Signature and checks presence of firmware to be updated for the M2M Device.

- E: Distribution of updated Firmware

- M2M Management Server distributes the updated firmware with OEM’s Digital Signature to M2M GW.

- M2M GW carries out verification of the Digital Signature of the firmware.

- F: User Authorization for Firmware Update

M2M GW sends a message to User in order to confirm permission from the User to conduct firmware update operation, with information regarding firmware update operation.

- G: Firmware Update Implementation

- M2M GW transfers the updated firmware to M2M Device.

- M2M Device verifies Digital Signature of the firmware and gets updated with the firmware if it is the right firmware.

- H: Notification of Firmware Update completion

- M2M Device sends version information of the updated firmware with its Digital Signature when reboot with the updated firmware is completed.

- M2M GW transfers the firmware version information with Digital Signature of M2M Device to M2M Management Server.

- M2M Management Server verifies the Digital Signature of M2M Device and then the operation of Firmware Update is finally completed.

    - 6.11.7	Alternative flow

(1)  User Initiated Firmware Update Operation

- Steps A through C are the same as shown in Normal Flow.

- Step D : User Verification of  Presence of Firmware to be updated

- User operates Control Display Device of M2M GW in order to check availability of updated firmware  for M2M Device.

- M2M GW sends a request to M2M Device in order to verify version information of firmware currently installed in M2M Device..

- M2M Device sends version information of firmware currently installed back to M2M GW together with its own Digital Signature.

- M2M GW transfers the version information with the Digital Signature of M2M Device to M2M Management Server.

- Management Server verifies the version information with the Digital Signature and checks presence of firmware to be updated for the M2M Device.

- If there is firmware to be updated, follow Steps E – H of Normal Flow.

- If there is no firmware to be updated, M2M GW notifies User of it on the display.

(2) Constrained Device (ECU)

This alternative flow is provided for Constrained Device which is unable to conduct attachment of its own Digital Signature or verification of Digital Signature of other servers and devices by itself due to limitation of capability. In this case, it is possible to conduct Firmware Update operation by trusting M2M GW on the assumption that M2M GW and M2M Device are mutually trusted.

- Steps A through C are the same as shown in Normal Flow.

- D: Verification of current version of Firmware installed in M2M Device

- M2M Management Server sends a request to M2M GW in order to verify version information of firmware currently installed in M2M Device.

- M2M GW transfers the verification request to M2M Device.

- M2M Device sends version information of firmware currently installed back to M2M GW.

- M2M GW transfers the version information with its Digital Signature to M2M Management Server.

- M2M Management Server verifies the Digital Signature and version information, and checks presence of firmware to be updated for the M2M Device.

- E: Distribution of updated Firmware（same as Step E in Normal Flow）

- F: User Authorization for Firmware Update（same as in Step F in Normal Flow）

- G: Firmware Update Implementation

- M2M GW transfers updated firmware to M2M Device.

- M2M Device gets updated with the firmware trusting M2M GW.

- H: Notification of Firmware Update completion

- M2M Device sends version information of the updated firmware when reboot with the updated firmware is completed.

- M2M GW transfers the firmware version information with its Digital Signature to M2M Management Server.

- M2M Management Server verifies the Digital Signature and then the operation of Firmware Update is finally completed.

    - 6.11.8	Post-conditions

- M2M Management Server has to verify the status of M2M Device (firmware version, number of update failures, error information, etc.) and manage these types of information elements.

- M2M Device has to have rollback function in order to restore to the original state, when the Firmware Update operation fails.

    - 6.11.9	High Level Illustration

    - 6.11.10	Potential requirements

- The oneM2M System shall be able to prevent unauthorized modification of the firmware of M2M Device.

- The oneM2M System shall be able to detect unauthorized modification of the firmware of M2M Device.

- The oneM2M System shall be able to stop operation of M2M device when it is updated with wrong firmware.

- The oneM2M System shall be able to support security mechanisms to protect their cryptographic keys and cryptographic operations by using tamper proof devices such as TPM (Trusted Platform Module), HSM (Hardware Security Module) and SIM (Subscriber Identity Module).

- The oneM2M System shall be able to prevent malfunction of M2M Device caused by receiving unsolicited messages or information.

  - 6.12	Car/Bicycle Sharing Services

    - 6.12.1	Description

As seen clearly, automation already penetrates all aspects of life even in our urban life. The goal of this use case is to describe several automation services which are occurred in different urban space in different life style, bicycle/car sharing services.

Brief Features of Services

- Car Sharing Service

Car Sharing is to offer a new service model for automobile transportation. Simply, Car Sharing is a self-service, on-demand alternative to car ownership; a service that is offered to urban residents (B2C) and businesses (B2B).

This service is mainly designed around a particular user profile – first of all, people who live in cities but do not drive a car every day and secondly tourists who live in cities but do not own a car. Thus, people who need a car at short notice but take an alternative to car ownership.

The brief procedure of this service is 1) joining the membership, 2) unlocking the car door, 3) driving away, 4) parking to any reserved spot provided by the service provider and/or public, and 5) paying as you drive (including gas, insurance, and etc.).

- Bicycle Sharing Service

Bicycle sharing service is also a new service in which bicycle are made available for shared use to individuals who do not own a bicycle. Generally, bicycle sharing service is run by government agencies.

The procedure of this service is similar to the car sharing service, but the different type of services such as healthcare service can be combined.

    - 6.12.2	Source

oneM2M-REQ-2012-0132R01 Use Case: Car/Bicycle Sharing Services

    - 6.12.3	Actors

- User

A user who takes the ownership of the shared things which are car and bicycle.

- Sensors (or Sensor Devices)

Sensor Devices can be various based on its usage, and do not have any direct communication interfaces to the M2M Service Platform.

  - For Car Sharing Service – Door Control Sensor, Tire Pressure Sensor, Fuel Indication Sensor, GPS.

  - For Bicycle Sharing Service – Lock Control Sensor, Accelerometer, Tire Pressure Sensor, Heart-rate Sensor.

- Smartphone

A device which is an intermediate entity and is available to connect from sensors to a M2M Service Platform. The basic role is similar to the general M2M gateway, but it has some sensors and some applications (navigation) itself used by services.

- M2M Service Platform

In charge of providing common functionalities for the M2M services. It is mainly in charge of collecting the status and configuration information of sensors and controlling them via the smartphone and/or M2M gateway.

- M2M Service Providers

Companies which provide its own M2M services for the user through the M2M Service Platform. The M2M Service Providers can be various according to the types of services.

The providers include Car Sharing Service Provider, Insurance Company, Gas Station, Bicycle Sharing Service Provider, and Healthcare Service Provider.

    - 6.12.4	Pre-conditions

See sub-case flows.

    - 6.12.5	Triggers

See sub-case flows.

    - 6.12.6		Normal Flow

Sub use case 1 – Car Sharing Case

Trigger:

A user wants to take an ownership of the car.

Pre-conditions:

The user preliminary joins a membership of the Car Sharing Service.

Sensors built in the car are required to periodically (normal) and non-periodically (urgent) send sensor data to the M2M Service Platform based on the trigger defined by the M2M Service Providers.

The M2M Service Platform collects and manages data and configurations related to the services. Generally, each service has its own data and configuration set, simply called resources.

The M2M Service Providers in the service domain have a service agreement each other for unified services.

The Smartphone has a navigation and car sharing application.

Figure 6.12-1 Car Sharing Normal Flow

Detailed Flow Descriptions:

- The Applications of each Service Provider in the service domain register and subscribe to changes of resources (or information) about the Car Sharing Service in the M2M Service Platform.

- Since each resource in the M2M Service Platform is owned by the Car Sharing Provider, Insurance Company and Gas Station, if an application needs to access another resource, it shall request proper access right of the resources and grant that request if appropriate and based on the service agreement.

- As the user finds a shared car, opens the car door and turns on the ignition using interfaces of the Smartphone such as Bluetooth and NFC, if the user is authorized.

- The Sensors report the changed status to the M2M Service Platform via the Smartphone as a gateway when the specific condition is triggered. (Car is just being used)

- The M2M Service Platform notifies the Car Sharing Service Provider of the changed status. (Note: The Car Sharing Service Provider can update the situation that the car is being used on its website)

- (Normal Reporting Case for managing the Service) The Sensors report the changed status to the M2M Service Platform via the Smartphone when the specific condition is triggered. (Periodic location reporting and car health check for maintenance reasons)

- The M2M Service Platform notifies the Car Sharing Service Provider of the changed status. (Note: Agreement on privacy policy of location is preliminary confirmed)

- (Urgent Reporting Case for handling any emergency) The Sensors report the changed status to the M2M Service Platform via the smartphone as a gateway when the specific condition is triggered. (The fuel is low)

- The M2M Service Platform immediately notifies the Car Sharing Service Provider of the changed status.

- The Car Sharing Service Provider finds out the nearest Gas Station according to the received location information and a service agreement between the Car Sharing Service Provider and the Gas Station, and the Provider sends the route information to M2M Service Platform.

- The M2M Service Platform notifies the Smartphone of the route information.

- After filling the fuel, the user virtually pays the fuel fee by using the Smartphone’s NFC tag. The payment information is reported to the M2M Service Platform.

- The M2M Service Platform notifies the Car Sharing Provider and the Gas Station of the payment information. (Note: This procedure is for the Car Sharing Provider to pay Gas Station the fuel fee instead of the user)

- Afterwards, due to the low battery of the Smartphone (less than 30% remain), the Smartphone reports the changed status to the M2M Service Platform.

- The M2M Service Platform automatically changes the subscription and reporting attributes of the Sensors and the Car Sharing Service Provider. (For example, if the Platform changes the subscription attributes to “only emergency case”, only emergency subscription case will be notified. The others cannot be notified, but at the end of service, batch-mode)

- As the user arrives at the destination, and turns off the ignition, the sensors report the accumulated information, normal event subscription information, to the M2M Service Platform via smartphone.

- The M2M Service Platform notifies the Car Sharing Provides and Insurance Company of the usage of the shared car.

- The Insurance Company stores the insurance fee by writing onto the Car Sharing Service Provider’s resource in the M2M Service Platform, in this case the Insurance Company preliminary acquires proper access right to write.

- The M2M Service Platform notifies the Car Sharing Provides of the insurance fee.

Post-conditions:

- The User will pay as him/her drive according to the recorded data.

- The Car Sharing Service Provider can update the position and status of the car on its website using the recorded data. Thus, next users can make use of the Car Sharing Service.

Sub Use Case 2 – Bicycle Sharing Service

Trigger:

A user wants to take an ownership of the bicycle.

Pre-conditions:

The user preliminary joins a membership of the Bicycle Sharing Service.

The sensors built in the car and in the smartphone are required to periodically (normal) and non-periodically (urgent) send sensor data to the M2M Service Platform based on the trigger defined by the Service Provider.

The M2M Service Platform collects and manages data and configurations related to the services. Generally, each service has its own data and configuration set, simply called resources.

The Smartphone has a navigation and bicycle sharing application.

The M2M Service Providers in the service domain have a service agreement each other for unified services.

Figure 6.12-2 Bicycle Sharing Normal Flow

Detailed Flow Descriptions:

- The Applications in the service domain register the service and subscribe to changes of information about the Bicycle Sharing Service.

- Since each resource in the M2M Service Platform is owned by the Bicycle Sharing Service Provider and Health Service Provider, if an Application needs to access another resource, it shall request proper access right of the resources and grant that request if appropriate and based on the service agreement.

- To unlock the bicycle, the user tags the locker of the bicycle through the NFC interface.

- The Sensors report the changed status to the M2M Service Platform via the smartphone as a gateway when the specific condition is triggered (for example, the bicycle is being used).

- The M2M Service Platform notifies the Bike Sharing Service Provider of the changed status. (Note: The Bicycle Sharing Service Provider can record the situation on its web-site that the car is being used).

- (Normal Reporting Case for managing the Service) The heart-rate of the user is continuously collected by the heart-rate sensor on the handlebar, and the health-related information such as heart-rate, location, time is reported periodically to the Service Operator.

- The M2M Service Platform notifies the Bicycle Sharing Service Provider and the Health Service Provider of the health Service information.

- (Urgent Reporting Case for handling any emergency) While riding the bicycle, the tire pressure sensor detects the low pressure of the front tire, the information is immediately sent to the M2M Service Platform via the Smartphone with location information.

- The M2M Service Platform notifies the Bicycle Sharing Service Provider of the changed status.

- The Bicycle Sharing Service Provider finds out the nearest bike repair shop according to the received location information, and the Provider sends the route information to M2M Service Platform.

- The M2M Service Platform forwards the route information to the Smartphone which has a navigation application.

- Afterwards, due to the low battery of the Smartphone (less than 30% remain), The Smartphone reports the changed status to the Service Operator. (Low battery indication)

- The M2M Service Platform automatically changes the subscription attributes of sensors and the Bicycle Service Provider such as delay tolerance to reduce battery-consumption. (For example, if the Platform changes the subscription attributes to “only emergency case”, only emergency subscription case will be notified. The others cannot be notified, but at the end of service, batch-mode).

- As the user arrives at the destination and parks to the reserved spot, the Sensor reports the accumulated information and normal event subscription information to the M2M Service Platform via the Smartphone.

- The M2M Service Platform notifies Bicycle Sharing Service Provide and Healthcare Service Provider of the usage of the shared bicycle.

Post-conditions:

- The User may pay as him/her drive according to the recorded data. (not for the public service case)

- The Bicycle Sharing Service Provider can update the position and status of the car on its website using the recorded data. Thus, next users can make use of the Bicycle Sharing Service.

    - 6.12.7	Alternative Flow

None

    - 6.12.8	Post-conditions

See sub-case flows.

    - 6.12.9	High Level Illustration

Figure 6.12-3 Car/Bicycle Sharing High Level Illustration

    - 6.12.10	Potential Requirements

- The M2M System shall support mobile/portable M2M Gateway and/or Device.

- The M2M System shall support to distinguish the event levels for reporting and handle it differentially.

- Note: For example, the event levels may be divided into normal and urgent event.

- Based on the condition of the M2M Gateway and/or Device, the M2M System shall change the reporting (or subscription) mechanisms and/or configurations related to a service.

- The M2M System shall support to process access right requests of a resource and grant the requests if the requests

  - 6.13	Smart Parking

    - 6.13.1	Description

Smart parking helps one of the biggest problems on driving in urban areas; finding empty parking spaces and controlling illegal parking. Those parking spaces are wide spread and owned by different providers so that it is not easy to access at one place/time.

With smart parking service, drivers can easily find available parking spaces, pay parking fees and even can make reservations. Making parking reservations would be available for limited people such as VIPs or the disabled, since ordinary parking service needs to satisfy first-come-first-served rule.

In this use case, law enforcement authority is also included as an actor. This implies M2M technologies aims rightness/safety as well as convenience.

    - 6.13.2	Source

oneM2M-REQ-2013-0169R03 Use Case Smart Parking

    - 6.13.3	Actors

- M2M Service Platform

This is a platform that interacts with M2M Gateways/Devices and M2M Application Service Providers.

- Smartphone

This is a M2M Device acts as a car navigator and a wallet to pay parking fee by connecting parking meters.

- On-street Parking Meter

This is a M2M Device installed near parking slots to charge drivers parking fees.

- In-building Parking Sensor

This is a M2M Device with a small camera that can recognize a plate on cars, and is installed near disabled-only parking spaces.

- Parking Provider

This is a M2M Application Service Provider who owns parking lots, in this use case there are two parking providers; in the mall and on street.

- Billing Provider

This is a M2M Application Service Provider (e.g. credit card company) who provides billing service to M2M Users such as parking fee. When it issues a bill, coupons from M2M Application Service Providers are used as tokens for compensation schemes. They also can charge fines issued by police center on their bills.

- Police Center

This is a law enforcement authority, one of M2M Application Service Providers, who charges fine to whom break laws.

- User

This is a M2M service user who drives a car. In the second sub case, dedicated parking space, there are two users. One originally makes a reservation who is handicapped, and the other who illegally parks a car on disabled-only parking area.

    - 6.13.4	Pre-conditions

See sub-case below.

    - 6.13.5	Triggers

See sub-case below.

    - 6.13.6	Normal Flow

- Finding Space, Parking Car & Paying Bill

Pre-condition

User sets a destination as a shopping mall using the smartphone navigator, also checks availability of parking space in the building before or while driving.

Triggers

The car approaches near the destination.

Figure 6.13-1 Normal Flow - Finding Space, Parking Car & Paying Bill

Normal Flow

- Since the user set the mall as the destination before, when the user is near the mall, the navigator sends the location to the mall parking provider automatically.

- The mall parking provider informs the navigator that there’s no empty parking space now.

- Based on the car’s location, which is near the mall, the mall parking provider inquires availability of other parking spaces through M2M service platform.

- There are empty spaces on street, so the mall parking provider recommends that parking space.

- The user touches the smartphone on a parking meter to start parking. Then the street parking provider is noticed, and also the mall parking provider is.

- The mall parking provider offers discount coupon for parking outside as a compensation.

- The user touches the smartphone on the meter to finish parking.

- The street parking provider bills parking fee. The bill with discount coupon is sent to billing provider through M2M service platform.

- Dedicated Parking Space

Pre-condition

Before driving, the user (user A) makes a parking reservation for a slot in a shopping mall, which is especially for the disabled. It is normally assured because there will be fines for illegal parking on this dedicated parking space

Triggers

None

Figure 6.13-2 Normal Flow 1 - Finding Dedicated Parking Space

Normal Flow

- The other user (user B) parks a car on the parking lot, which is already reserved by user A before.

- User B’s illegal parking on the disabled-only parking area is reported to police center.

- Police center charges fine on user B.

- User A approaches the mall and is noticed that reserved parking space is taken and only choice now is normal parking slots.

- User A parks on a normal parking slot instead of the reserved one.

- The parking provider offers parking discount coupon to the user A as a compensation.

- After shopping, user A leaves the building and finish parking.

- The parking provider bills parking fee for user A, adopting the parking coupon.

    - 6.13.7		Alternative Flow

Alternative Flow 1 – Dedicated Parking Space

- Pre-condition

  - Before driving, the user (user A) makes a parking reservation for a slot in a shopping mall, which is especially for the disabled. It is normally assured because there will be fines for illegal parking on this dedicated parking space.

- Triggers

  - None

Figure 6.13-3 Alternative Flow 1 - Finding Dedicated Parking Space

- The other user (user B) parks a car on the parking lot, which is already reserved by user A before.

- The mall parking provider inquires plate number of the car to a CCTV near the parking space.

- User B’s illegal parking on the disabled-only parking area is reported to police center.

- Police center charges fine on user B.

- User A approaches the mall and is noticed that reserved parking space is taken and only choice now is normal parking slots.

- User A parks on a normal parking slot instead of the reserved one.

- The parking provider offers parking discount coupon to the user A as a compensation.

- After shopping, user A leaves the building and finish parking.

- The parking provider bills parking fee for user A, adopting the parking coupon.

    - 6.13.8	Post-conditions

None

    - 6.13.9	High Level Illustration

Figure 6.13-4 High Level Illustration of Smart Parking

    - 6.13.10	Potential Requirements

- The M2M System shall support mechanisms to correlate charging data/records from different M2M Application Service Providers.

- The M2M System shall support triggering M2M Devices to report on-demand regarding collected data from other M2M Devices.

  - 6.14	Vehicle Broadcasting without Registration

    - 6.14.1	Description

This use case consists for any vehicle driving fast to signal its emergency state which can be for example hard braking state or abnormal vehicle state to other following vehicles or infrastructure, such as a road side unit in its vicinity

By broadcasting time limited periodic messages which indicate vehicle’s emergency state triggered by an event, the vehicle can warn unspecified entities of a sudden danger so limiting the risk of collision. The infrastructure can collect those information and relay them to other entities or servers in the distance.

Since a vehicle is a very quick-moving object and the transferred message is a time-critical information, the transmission speed is crucial for this use case.

    - 6.14.2	Source

REQ-2016-0031R03 Vehicle BroadCasting without Registration TR-0026

    - 6.14.3	Actors

- Source Vehicle:

A Source Vehicle is a moving object which provides communications functions necessary to support connected vehicle operations. It can detect its emergency state and send warning message by broadcasting.

- Target Vehicle

A Target Vehicle is a moving object which provides communications functions necessary to support connected vehicle operations for communicating with the Source Vehicle. After receiving warning message, it tries to avoid the potential danger.

- Infrastructure Node:

An Infrastructure Node is located by a road and provides communications functions necessary to support connected vehicle operations. It can collect information generated in a certain area and partially process it. It is also possible to convey it to the server or other vehicles in a far distance.

    - 6.14.4	Pre-conditions

N/A

    - 6.14.5	Triggers

When a vehicle application detects its emergency state, it should trigger broadcasting specific information.

    - 6.14.6	Normal Flow

- Each Vehicle and Infrastructure Node do not register with each other.

- Source Vehicle runs event detection engine and analyzes whether the safety-related event has occurred or not.

- Source Vehicle send the safety-related information to unspecified entities by broadcasting

  - Infrastructure Node gathers the broadcasted information and locally processes and relays to other vehicles or servers in a distance if needed

  - Target Vehicle receives the safety-related information and re-acts to avoid the potential collision.

    - 6.14.7		Alternative Flow

N/A

    - 6.14.8	Post-conditions

N/A

    - 6.14.9	High Level Illustration

- Source Vehicle can send time-critical information to unspecified entities

- Infrastructure Node can relay this information to the server which can also re-use that information.

    - 6.14.10	Potential Requirements

- The oneM2M system shall enable the M2M Infrastructure to facilitate direct communication between two or more different M2M devices without having registered with one another.

  - 6.15	Vehicle location privacy protection

    - 6.15.1	Description

The privacy protection is an issue that must be considered for locating in human to human communications. For example, in 3GPP network, privacy protection is implemented by an independent entity such as a privacy profile register (PPR) or a gateway mobile location center (GMLC). In the vehicle domain, the location information of the vehicle represents the location of the vehicle user. Therefore an issue of privacy protection is also closely related to the vehicle location information. The privacy protection refers to that when a vehicle is connected to the M2M system, the user of the vehicle has permission to specify when and where a third party application is allowed to access to the vehicle location information.

After the M2M platform receives a message from a third part application, requesting for the location information of a certain vehicle, the M2M platform determines the entity that should performs the privacy inspection, according to the source of the location information. For example, if the location information of the vehicle is network based and the underlying network is 3GPP, then the entity that performs the privacy inspection should be the 3GPP location server. On the other hand, if the location information of the vehicle is device based, then it should be the M2M platform that performs the privacy inspection.

    - 6.15.2	Source

REQ-2016-0040R02-TR-0026-Vehicle_location_privacy_protection

    - 6.15.3	Actors

- M2M Device: It is embedded in a vehicle, which is used to connecting to the M2M platform.

- Vehicle: Vehicle connects to the M2M platform via the embedded M2M device and it equips hardware devices for localization. (e.g. GPS, Cellular modem)

- Vehicle Owner: The owner of the vehicle. The owner has privacy agreement with the 3GPP network location server or the M2M platform.

- 3GPP network location server: When the vehicle connects to the M2M platform via the 3GPP underlying network, the 3GPP network location server can perform the location service and provide the location information to the M2M platform. In this case the 3GPP network location server also performs the privacy inspection function. The 3GPP network location server is configured with location privacy policies according to the privacy agreement with the vehicle owner and the network operator policy. The location privacy policies refer to when and where a third part application is allowed to access to the vehicle location information.

- M2M Platform: It connects, manages M2M devices and exposes location services to the M2M application. The M2M platform can collect the vehicle location information either from the underlying network, e.g. 3GPP network location server, or from the device itself, e.g. the vehicle is equipped with any location capable modules or technologies (e.g. GPS) and is able to position itself. The M2M platform is configured with location privacy policies according to the privacy agreement with the vehicle owner. The location privacy policies refer to when and where a M2M application is allowed to access to the vehicle location information.

- M2M Application server: The M2M application server requests for the location information of the vehicle for any possible application usage, e.g. fleet management or traffic monitoring.

    - 6.15.4	Pre-conditions

- The vehicle owner already has privacy protection agreement with the operator of the 3GPP network or the M2M platform.

    - 6.15.5	Triggers

The M2M application server requests to access to the location information of the vehicle.

    - 6.15.6	Normal Flow

Scenario 1: The vehicle No.1 connects to the M2M platform via the 3GPP network and the location information is collected by the 3GPP network location server:

- The M2M application sends message to the M2M platform to request for the location information of vehicle No.1.

- The M2M platform checks the location information source of the vehicle. if the location information of the vehicle comes from the underlying network location server, e.g. 3GPP network location server, the M2M platform sends privacy inspection requests, including the M2M application identifier which can be recognized by the 3GPP network, to 3GPP network location server to verify whether the M2M application is allowed to access to the location information.

- The 3GPP network location server is configured with the privacy policies of vehicle No.1. When the 3GPP network location server receives the access right verify request, it performs the privacy inspection,  checking the privacy policies with the M2M application identifier and other information, e.g. the time and the lasted location of the vehicle. The 3GPP network location server returns the checking result to the M2M platform, that is whether the M2M application is allowed to access to the location information.

- If the M2M application is allowed to access to the location information, after the M2M platform receives the responses from 3GPP network location server, it sends the location information request message to the 3GPP network location server.

- The 3GPP network location server performs the location updating procedure inside the 3GPP network and collects the latest location information of the vehicle. Then the 3GPP network location server returns the location information to  the M2M platform.

- The M2M platform forwards the location information to the M2M application.

Scenario 2: The vehicle No.2 connects to the M2M platform via the 3GPP network. It is equipped with location capable modules or technologies (e.g. GPS) and is able to position itself. The M2M platform can collect the location information of the vehicle directly without the help of any other location server.

- The M2M application sends message to the M2M platform to request for the location information of vehicle No.2.

- The M2M platform checks the location information source of the vehicle. if the location information of the vehicle is device based and is collected by the M2M platform directly from the vehicle itself, then the M2M platform checks the privacy policies configured in the M2M platform.

- If the M2M application is allowed to access to the location information, then the M2M platform returns the location information to the M2M application. If the M2M application is not allowed to access to the location information, then the M2M platform sends failure response to the M2M application with appropriate error code.

    - 6.15.7	Alternative flow

N/A

    - 6.15.8	Post-conditions

N/A

    - 6.15.9	High Level Illustration

M2M application

Vehicle1

Vehicle2

GPS module

underlying network

Underlying network location server

privacy

M2M platform

privacy

    - 6.15.10	Potential requirements

1.	The oneM2M system shall be able to support the enforcement and management (e.g. update) of the privacy policies for vehicle location information.

2.	The oneM2M system shall be able to apply the privacy policies configured in the oneM2M system to those vehicles whose location information is obtained by the oneM2M system itself.

3.	The oneM2M system shall be able to request the underlying network to perform, on behalf of the oneM2M system, the privacy policy decision concerning the location information if the location information is obtained by the underlying network location service.

  - 6.16	Vehicle Domain service continuity

    - 6.16.1 Description

Autonomous or self-driving cars have been gaining attention as early versions of such vehicles have become available.  A gateway on such a vehicle, may be responsible for movement control (braking, turning, etc.) as well as delivering external services. Such a scenario requires real time monitoring of sensors and fast access to various actuators for accident avoidance. For example, time series data may be gathered which triggers specific actions when data is lost. In such cases service continuity and data transfer of time-critical information are essential.

    - 6.16.2 Source

REQ-2016-0052R02-TR-0026_service_use_case

    - 6.16.3  Actors

- Vehicle:

A Source Vehicle is a moving object which provides support for services necessary for vehicle operations.

- Road-Side Unit (RSU):

A Road–side unit is located along vehicular paths and provides localized support for services necessary for multiple vehicle operations. This support is provided for vehicles within the RSU’s communication range.

- Infrastructure Node:

An Infrastructure Node provides centralized support for services provided by a Services Provider to a large number and variety of devices.

    - 6.16.4 Pre-conditions

- All RSUs are registered to the Infrastructure Node.

    - 6.16.5 Triggers

- When a vehicle moves it loses connectivity to one road-side unit, and then re-connects to different road-side units along the way

    - 6.16.6 Normal Flow

The Normal Flow depicts the case where a moving vehicle receives notifications

Figure 1.1-1 Normal Flow for Vehicular Domain Service Continuity

- Vehicle establishes communication with RSU 1.

- RSU 1 creates a subscription to a resource hosted on the Infrastructure Node on behalf of the Vehicle.

- A notification corresponding to the subscription in step 1 is generated and sent to the Vehicle via RSU 1

- The vehicle moves, and establishes communication with RSU 2.

- Supporting interactions between RSUs and Infrastructure Node.

- A notification corresponding to the subscription in step 1 is generated. The notification is sent to the Vehicle via RSU 2

    - 6.16.7 Alternative flow

The Alternative Flow depicts the case where a moving vehicle reports observed road/traffic conditions (e.g. vehicles in proximity and their speed, distance to curb, etc.) and receives collision avoidance commands. The RSUs collect and store the information from the vehicles, and also exchange information with each other. This allows the RSUs to better identify the trajectory of vehicles and to help predict potential collisions. As collision avoidance decisions are extremely time sensitive, all decisions are made locally at the RSU. This avoids any unnecessary delays incurred with communication to the infrastructure node.

Figure 1.1-2 Alternative Flow for Vehicular Domain Service Continuity

- Vehicle establishes communication with RSU 1.

- Vehicle sends road/traffic data to RSU 1.

- The vehicle moves, and establishes communication with RSU 2.

- RSU 2 retrieves traffic data related to vehicle from RSU 1.

- Vehicle sends road/traffic data to RSU 2.

- Collision avoidance algorithm determines that a collision is imminent (based on vehicle data from RSU 2 and from RSU 1)

- RSU 2 sends a collision avoidance command to the vehicle.

    - 6.16.8 Post-conditions

N/A

    - 6.16.9 High Level Illustration

Figure 1.1-3 High Level Illustration Vehicular Domain Service Continuity

    - 6.16.10 Potential requirements

- The oneM2M system shall enable continuity of services to M2M devices as they move across various geographic points in the oneM2M system.

6.17. Optimal Speed Recommendation

    - 6.17.1 Description

The target vehicle is approaching to the traffic light that is currently displaying red.

The central server connected with vehicles, traffic lights and other entities such as weather server, map server and remote vehicles collects the vehicle's precise location and the traffic light phase schedule as well as additional information such as weather, road shape, emergency situation and etc. of the front road where the vehicle is heading to.

The server then calculates a specific optimal speed for the vehicle based on the collected information to reach the traffic light at the beginning of the next green phase and if necessary to avoid potential dangers.

The result speed is finally delivered to the driver who can avoid the unnecessary stop at the traffic light and unforeseeable hazards.

    - 6.17.2 Source

REQ-2016-0067R02-new_Vehicle_usecase_TR-0026.DOC

    - 6.17.3 Actors

- Target Vehicle:

A Target Vehicle is a moving object which periodically provides its movement information such as time, location, velocity, heading, lateral and longitudinal acceleration to the central server so that it can utilize the information to calculate the optimal speed. After the calculation in the server, the Target Vehicle receives back the result directly from the server.

- Traffic Light:

A Traffic Light is located by a road and provides its phase schedule necessary for the server to calculate the optimal speed of the target vehicle.

- Central Server:

A Central Server calculates the optimal speed for the target vehicle based on the gathered necessary information from various sources including the target vehicle, the traffic light, other information servers and remote vehicles.

- Information Servers:

There could be many kinds of additional Information Servers that can be connected with and utilized by the central server. For example, a weather server provides weather data such as time, location, external air temperature, snow, rain and etc. and it is possible to detect potential road hazards via a map server.

- Remote Vehicles:

Remote vehicles automatically or manually report emergency situation to the server.

    - 6.17.4 Pre-conditions

N/A

    - 6.17.5 Triggers

- When the central server detects that the target vehicle has approached a few hundred meters (It should be more than 300 meters in order to safely adjust the current speed) in front of the traffic light, the server calculates the optimal speed and sends it to the target vehicle.

    - 6.17.6 Normal Flow

- As the first step, associated entities register to the central server except remote vehicle which does not necessarily need to register because it is moving object and this scenario works based on location information. It is enough to know just the location of the remote vehicle and what happens around it.

- The server gathers information to calculate the optimal speed of the target vehicle.

2.1. The target vehicle periodically provides its movement information including heading, time, location, velocity, lateral and longitudinal acceleration.

2.2. The traffic light periodically provides its phase schedule when to be red, yellow or green.

2.3. Additional information server provides further useful data as requested by the central server. Information such as slippery zone, high accident frequency location, or sharp curve could be examples.

2.4. Remote vehicles can also provide more information to the server in case accident or something dangerous happens. This can be done automatically as programmed or manually by drivers. Note that the server does not subscribe remote vehicles since it’s impossible to predict which vehicles may have necessary information. So remote vehicles should initiate to communicate with the server.

- When the target vehicle has reached a specific area (It could be 300m in front of the traffic light) ahead of the traffic light, the central server starts calculating the optimal speed for the target vehicle to pass through the traffic light with no stop based on the collected information. If any additional information indicates there could be safety risk, the server should consider them as well for the calculation.

- The final outcome from the server is delivered to the target vehicle and shown to the driver who can follow the speed recommendation.

    - 6.17.7 Alternative flow

N/A

    - 6.17.8 Post-conditions

N/A

    - 6.17.9 High Level Illustration

- Solid lines in red are mandatory while dashed lines in gray are optional for this use case.

    - 6.17.10 Potential requirements

- The oneM2M system shall support real time data transmission for moving object to have enough time to react to the received information.

- The oneM2M system shall verify geographical location information from moving objects regardless of information accuracy.

- The oneM2M system shall verify time synchronization among multiple sources that provide inputs for a single output.

- The oneM2M system shall be able to guarantee network stability and reliability for applications that are related to safety

- The oneM2M system shall apply high-level security(prevention from malicious hacker) for applications that are related to safety.

- The oneM2M system shall provide privacy protection mechanism at the central server.

7	Overview of Potential Requirements

Potential requirements from all vehicular domain use cases collected in this technical report are summarized as follows,

- The M2M application System shall enable the M2M Devices to exchange M2M application to diagnostic data periodically with the M2M Application in the network domain.

Note: These requirements addresses the use case 6.1 “Vehicle Diagnostic & Maintenance Report”.

- The M2M System shall enable the M2M Application to configure the notification interval in the M2M Devices.

Note: These requirements addresses the use case 6.1.

- The M2M system shall support a mechanism to describe the syntax and semantics format of the M2M application diagnostics data exchanged between the M2M Devices and the M2M Application in the network domain.

Note: These requirements addresses the use case 6.1.

- The M2M service SHALL be able to provide the mechanism for authorization for integrity-checking and installing processes of software/hardware/firmware component(s) on M2M Device(s) (SER-013).

Note: These requirements addresses the use case 6.2 “Use Case on Remote Maintenance Services”.

- The M2M system SHALL be able to support authentication using device key on the integrity check for M2M Device(s).

Note: These requirements addresses the use case 6.2.

- The M2M Device SHALL be able to support HSM (Hardware Security Module) to protect its integrity depending on the security level requirement.

Note: These requirements addresses the use case 6.2.

- A M2M System shall support communication between M2M Platform and a M2M device either directly or via a gateway.

Note: These requirements addresses the use case 6.3 “Traffic Accident Information Collection”.

- A M2M System shall be able to exchange information between M2M applications via M2M Platform.

Note: These requirements addresses the use case 6.3.

- A M2M System shall be able to take actions according to the received service requests from M2M Applications.

Note: These requirements addresses the use case 6.3.

- A M2M system shall be able to support service requests from M2M applications for communication with QoS requirement, such as, higher delivery priority, reliable delivery, etc.

Note: These requirements addresses the use case 6.3.

- A M2M System shall support mutual-authentication among M2M device, M2M gateway, M2M platform and M2M Application (SER-040).

Note: These requirements addresses the use case 6.3.

- The information sent by a M2M device or the M2M platform or a M2M application shall use cryptographic technology to ensure information authentication and information integrity.

Note: These requirements addresses the use case 6.3.

- A M2M system shall permit information being provided in anonymous way.

Note: These requirements addresses the use case 6.3.

- A command issued by a M2M System shall be able to have time expiration or geography restriction.

Note: These requirements addresses the use case 6.3.

- Provisioning, installation, configuration and registration method of terminal system

  - Especially for the case of overlapping two different system for DTG management system (owns and manages the device) and the application system using DTG data (utilising the data from the device).

- Note: These requirements addresses the use case 6.4 “Fleet Management Service using DTG (Digital Tachograph)”.

- DTG/FMS data storing method and delivery protocol

  - There is no dominant standard specifying data formats and protocols for vehicle related applications.

- Note: These requirements addresses the use case 6.4.

- Vehicle location based service method

  - M2M service platform is expected to provide the service capability supporting location based service.

- Note: These requirements addresses the use case 6.4.

- Control, configuration, error logging, and management method for the terminal system Over The Air.

  - M2M service platform is expected to provide the service capability supporting the Over The Air management.

- Note: These requirements addresses the use case 6.4.

- The M2M system shall provide the capability for an M2M device to maintain registration with multiple entities simultaneously.

Note: These requirements addresses the use case 6.5 “Use cases for Electronic Toll Collection (ETC) service”.

- The registration shall be able to include information that identify the peer entity, and other information such as its management privilege, subscription etc., that are necessary for the conduct of the respective peer relationships.

Note: These requirements addresses the use case 6.5.

- It shall be possible for some registrations to hold the complete set of information context about the peer entity. This is refereed to as "full registration".

Note: These requirements addresses the use case 6.5.

- It shall be possible for some registrations to hold only a subset of information context about the peer entity. This is referred to as "lightweight registration".

Note: These requirements addresses the use case 6.5.

- It shall be possible for "lightweight registration" at different entities that pertain to a common peer entity, to hold different sets of information, if needed, about the common peer entity.

Note: These requirements addresses the use case 6.5.

- It shall be possible to correlate the "full registration" and the "lightweight registration" that pertain to a common peer entity.

Note: These requirements addresses the use case 6.5.

- It shall be possible to distinguish the "full registrations" and the "lightweight registrations" that pertain to a common peer entity.

Note: These requirements addresses the use case 6.5.

- The M2M service platform shall be able to support the time-based policies to access the Underlying network.

Note: This requirement addresses the use case 6.6 “Use cases for Taxi Advertisement”.

- The oneM2M System shall enable discovery of M2M Application Servers, M2M Management Servers and M2M Devices available to an M2M Gateway for data exchange (OSR-056).

Note: This requirement addresses the use case 6.7 “Use Case on Vehicle Data Service”.

- The oneM2M System shall enable discovery of M2M Gateways available to a M2M Management Server and an M2M Device for data exchange (OSR-057).

Note: This requirement addresses the use case 6.7.

- The oneM2M System shall be able to support the capabilities for data repository (i.e. to collect/store) and for data transfer from one or more M2M Devices or M2M Gateways, for delivery to one or more M2M Gateways via M2M Area Network without any assistance or instruction of M2M Management Servers and M2M Application Servers(OSR-019).

Note: This requirement addresses the use case 6.7.

- Upon request from M2M Application Server, an M2M Gateway shall enable functions that pre-process (e.g. average) M2M data before providing them to the recipient.

Note: This requirement addresses the use case 6.7.

- Upon request, an M2M Gateway shall enable functions that erase M2M data (e.g. that have been sent or could not be sent to the recipient within a certain time) based on criteria from an M2M Application Server.

Note: This requirement addresses the use case 6.7.

- An M2M Gateway and/or an M2M Device shall be able to broadcast to all M2M Devices and/or M2M Gateways in the vicinity its need to receive/deliver specific data.

Note: This requirement addresses the use case 6.7.

- M2M Gateway and/or M2M Device shall be able to establish a connection to each other if it is able to receive/deliver the required specific data.

Note: This requirement addresses the use case 6.7.

- OneM2M System shall be able to send the information to intended vehicles by unicast, multicast and/or broadcast.

Note: This requirement addresses the use case 6.8 “Smart Automatic Driving”.

- OneM2M System shall be able to securely transfer the information by using an appropriate way such as digital signature.

Note: This requirement addresses the use case 6.8.

- OneM2M System shall be able to transfer the information on real-time basis for feeding back current road states to automatic driving control. The feedback time should be less than a few seconds (the distance between vehicles is normally worth of a few seconds) to avoid unnecessary speed down/stop of following vehicles.

Note: This requirement addresses the use case 6.8.

- OneM2M system shall be able to guarantee its reliability in order to receive/feedback massages from/to related vehicles.

Note: This requirement addresses the use case 6.8.

- The oneM2M System shall enable the cancellation of continuous data collection and/or the deletion of collected data when pre-defined conditions are met. (OSR-089)

Note: This requirement addresses the use case 6.9 “Use Case on Vehicle Data Wipe Service”.

- The oneM2M System shall enable pre-defined conditions to be protected from unauthorized modification (SER-050).

Note: This requirement addresses the use case 6.9.

- The oneM2M System shall enable the deletion of M2M data produced/stored by the M2M Devices/Gateways based on request from an authorized entity (SER-051).

Note: This requirement addresses the use case 6.9.

- The oneM2M System shall support reporting of Geo-Fence based Location event of the target M2M Device to the M2M Application based on the Application’s configuration(OSR-047).

Note: This requirement addresses the use case 6.10 “Vehicle Management based on Geo-Fence”.

- The oneM2M System shall support the M2M Application setting the configuration for Geo-Fence based location service.

Note: This requirement addresses the use case 6.10.

- The oneM2M System shall be able to prevent unauthorized modification of the firmware of M2M Device.

Note: This requirement addresses the use case 6.11 “Use Case on Secure Over-The-Air Firmware Update for Automotive ECUs”.

- The oneM2M System shall be able to detect unauthorized modification of the firmware of M2M Device.

Note: This requirement addresses the use case 6.11.

- The oneM2M System shall be able to stop operation of M2M device when it is updated with wrong firmware.

Note: This requirement addresses the use case 6.11.

- The oneM2M System shall be able to support security mechanisms to protect their cryptographic keys and cryptographic operations by using tamper proof devices such as TPM (Trusted Platform Module), HSM (Hardware Security Module) and SIM (Subscriber Identity Module).

Note: This requirement addresses the use case 6.11.

- The oneM2M System shall be able to prevent malfunction of M2M Device caused by receiving unsolicited messages or information.

Note: This requirement addresses the use case 6.11.

8	High Level Architecture

## 8.1	Introduction

The fifteen use cases in the vehicular domain discussed in the present document are listed in table 8.1-1.

Table 8.1-1: Use cases in the vehicular domain

| Use case No. | Title | Description |
| --- | --- | --- |
| 1 | Vehicular Diagnostic & Maintenance Report | See clause 6.1 |
| 2 | Use Case on Remote Maintenance Services | See clause 6.2 |
| 3 | Traffic Accident Information Collection | See clause 6.3 |
| 4 | Fleet Management Service using DTG (Digital Tachograph) | See clause 6.4 |
| 5 | Use cases for Electronic Toll Collection (ETC) service | See clause 6.5 |
| 6 | Use cases for Taxi Advertisement | See clause 6.6 |
| 7 | Use Case on Vehicle Data Service | See clause 6.7 |
| 8 | Smart Automatic Driving | See clause 6.8 |
| 9 | Use Case on Vehicle Data Wipe Service | See clause 6.9 |
| 10 | Vehicle Management based on Geo-Fence | See clause 6.10 |
| 11 | Use Case on Secure Over-The-Air Firmware Update for Automotive ECUs | See clause 6.11 |
| 12 | Car/Bicycle Sharing Services | See clause 6.12 |
| 13 | Smart Parking | See clause 6.13 |
| 14 | Vehicle Broadcasting without Registration | See clause 6.14 |
| 15 | Vehicle location privacy protection | See clause 6.15 |

Though these use cases are focusing on the vehicular domain, those ways of functional deployment are different. The following clauses provid three types of the high level oneM2M archtecture mapping for these use cases.

## 8.2	Vehicular Architecture Type 1

Figure 8.2-1 illustrates the first type of high level oneM2M architecture in vehicular domain. In this type, vehicular domain applications and the M2M service platform described in each use case are mapped to the IN (Infrastructure Node). Each vehicle equips a communication module which supports cellular network or other wireless communication technologies, and the vehicle can connect to a service platform via the communication module. Alternatively, an external unit such as the smartphone acts as both a communication module and a GW. In this case, the external unit would connect to in-vehicle network via some network technologies such as Bluetooth.

The GW which provides some functions defined in oneM2M acts as the MN or the ASN. The GW would be the in-vehicle unit such as an ECU (Electronic Control Unit) or the external unit such as the smartphone. The GW is connecting to in-vehicle units and external sensors via an in-vehicle network such as CAN (Controller Area Network) or other communication technology such as Bluetooth.

The in-vehicle units such as ECUs or external sensors act as the ADN or the non-oneM2M device node.

The use case 1 to 3, 6 to 11, 13 and 15 are categorized into this type.

Figure 8.2-1: High Level Architecute Type 1 in Vehicular Domain

## 8.3	Vehicular Architecture Type 2

Figure 8.3-1illustrates the second type architecture. Same as the first type, vehicular domain applications and the M2M service platform described in each use case are mapped to the IN. Each vehicle equips a communication module to connect to the a road side unit.

The raod side unit which provides some functions defined in oneM2M acts as the MN.

In this type, the GW in vehicle acts as the MN or the ASN. The in-vehicle units such as ECUs or external sensors act as the ADN or the non-oneM2M device node.

The use case 3 to 5 and 12 are  categorized into this type.

Figure 8.3-1: High Level Architecute Type 2 in Vehicular Domain

## 8.4	Vehicular Architecture Type 3

Figure 8.4-1 illustrates the third type. In this type, a vehicle to vehicle communication technology is supported in each vehicle. To support the vehicule to vehicle communication in the oneM2M architecture, the GW acts as the MN.

The use case 3 and 14 are categorized into this type.

Figure 8.4-1: High Level Architecute Type 3 in Vehicular Domain

9		Key Issues for Enablement for Vehicular Domain

Editor’s Note: This clause summarizes key issues resulting from the usecases and requirements identified. Each key issue is captured in a separate clause.

## 9.1	Key Issues 1: Location

Geographical location information is important in vehicular services, because a node in vehicular domain moves dynamically. As shown in use cases in clause 6, location is the key factor of vehicular services. For instance, the procedures of vehicular services could be triggered by the change of location. In other cases, the vehicular service would provide to the vehicles located in the dedicated area.

In oneM2M Release 2, Location CSF (LOC CSF) gives the following location functions.

- A location server in the Underlying Network

- A GPS module in an M2M device

- Information for inferring location stored in other Nodes

Through the analysis of existing use cases in this technical report, the following descriptions discuss the consideration points in location for vehicular domain.

    - 9.1.1	Accuracy of geographical location

In the vehicular domain, each vehicle would generally be equipped with a device to obtain geographical location (geo-locaiton) information. Through the location device, CSE in a vehicle periodically updates its geo-location and forward it to the oneM2M platform. The accuracy of geo-location information and the applicable use cases are dependent on the capability of the geo-location information source. Table 9-1-1 summarizes the required accuracy of the geo-location information, and the methods to obtain the required information accurately.

Table 9.1-1: Required position accuracy of location

| Title | AREA | ROAD | LANE |
| --- | --- | --- | --- |
| Accuracy of geo-location (margin of error) | Less than dozens of meters | Less than a few meters | Less than dozens of centimeters |
| Example of methods | An Underlying Network (e.g. Cell-ID from 3GPP, ITS-G5) Information for inferring location stored in other Nodes | GPS Assisted GPS Underlying Network (e. g. Cell-ID from RSUs) Information for inferring location stored in other Nodes | RTK-GPS Recent GNSS/RNSS |
| Use cases | 6.8, 6.10 | 6.4, 6.12, 6.13 | 6.x |

Figure 9.1-2: AREA

To deliver a content to vehicles located in a dedicated area (a few hundred meters), the “AREA level” accuracy of geo-location would be required. Figure 9.1-2 illustrates the concept of “AREA” use case. In this case, an acceptable error range should be less than dozens of meters. For instance, an underlying network can provide location information with the required accuracy of geo-location based on Cell-ID.

Geo-Fence is also applicable for the AREA level accuracy of geo-location. In oneM2M Release 2, Geo-Fence is utilizable as the resource type <locationPolicy>. For instance, in use case 6.8, when a traffic accident occurs, oneM2M platform sets Geo-Fence around the accident point. Then the oneM2M platform can provide the content to the vehicles in the Geo-Fence.

Figure 9.1-3: ROAD

To deliver content to vehicles located in a dedicated road, the “ROAD level” accuracy of geo-location would be required. In this case, navigation systems or services can be utilized and its acceptable error range should be less than a few meters. Some use cases provide route navigation for vehicles based on their location. Figure 9.1-3 illustrates the reason why “ROAD” accuracy of geo-location is required. As shown in the figure, the service cannot provide accurate route navigation when the location contains larger error. To obtain the location information with the required accuracy of geo-location level, GPS (Global Positioning System) or Assisted GPS are applicable. Furthermore, when the underlying network consists of RSUs, if the RSUs are located every a few meters to dozens of meters, the oneM2M platform could obtain ROAD level accuracy of geo-location.

To deliver content to vehicles located in a dedicated lane, the “LANE level” accuracy of geo-location would be required. At this time, no use case needs thislevel of accuracy, but the future use cases will require this accuracy. In this case, an acceptable error range should be less than dozens of centimeters. To obtain the location information with the required accuracy level, a device in a vehicle and an infrastructure should support the current positioning systems such as RTK-GPS (Real Time Kinematic GPS) or the latest GNSS (Global Navigation Satellite System) /RNSS (RadioNavigation Satellite System).

    - 9.1.2	Latency

Since a node in vehicular domain moves fast and dynamically, the latency (e. g., network latency) causes the difference between the actual vehicle position and the assumed position in application implemented in the center. For instance, one second latency causes dozens of meters difference. Therefore, if the service requires higher position accuracy and time-sensitive operation, the service should take the effect of latency into consideration (for instance, extrapolating the location considering the latency).

9.2 	Key Issue 2: Maintaining AE contact information

Clauses 6.3, 6.5, 6.8, and 6.16. as well as  requirement 1 in clause 6.16.10 require that  a moving M2M device uses the remote services offered by an M2M platform as well as the local services offered by a local M2M gateway. As a device AE moves along its path, it moves out of communication range of one MN-CSE and enters the communication range of another MN-CSE.

Assuming a Release 2 implementation, if the AE wishes to use the services provided by these MN-CSEs, the AE will perform a re-registration. At each (re)registration, a new <AE> resource is created at the new MN-CSE.

AEs are reached through their registrar CSE, by contacting the <AE> resource created during registration. This is referred to as the AE contact information. A number of oneM2M services rely on having up-to-date AE contact information in order to function properly. For these services, the remote CSE providing this service stores the URI of the <AE> resource as an attribute. A quick search of TS0004 reveals that an <AE> resource URI can be used in the following services: notification list of subscribe/notify, link associated with an announced resource, notification list of a non-blocking asynchronous request, and member list of a group resource.

This results in the following issue:

If an AE changes its registration point, and by consequence its AE contact information, the oneM2M services highlighted above, will have URIs that point to the wrong Registrar CSE – that is they have stale AE contact information.

## 9.3	Key Issue 2: Registration management

Clauses 6.3, 6.5, 6.8, and 6.16. as well as  requirement 1 in clause 6.16.10 require that  a moving M2M device uses the remote services offered by an M2M platform as well as the local services offered by a local M2M gateway. As a device AE moves along its path, it moves out of communication range of one MN-CSE and enters the communication range of another MN-CSE.

Assuming a Release 2 implementation, if the AE wishes to use the services provided by these MN-CSEs, the AE is required to perform a re-registration to every new MN-CSE it encounters. However, as the AE moves out of communication range of an ‘old’ MN-CSE, it will not be able to delete its registration hosted on that CSE.

This results in the following potential issue:

The AE might end up with multiple registrations in the oneM2M system. Although the old registration will eventually expire and be removed, there is a period where both registrations are active. In fact, the duration of this period may be quite long depending on the expiration time associated with the registration.

During this period, the old MN-CSE may be consuming processing and storage resources to manage the old AE registration. For example, the old MN-CSE may be monitoring a  <timeSeries> resource associated with the AE, in order to track, detect and report missing data in a Time Series. Or, the old MN-CSE may receive a discovery request and it will search under the old registration <AE> resource, for resources matching the filter criteria.

10	 Potential Solutions for the Key Issues

Editor’s Note: This clause summarizes solutions for the Key Issues. Each solution is captured in a separate clause.

10.1 Solution A

10.1.1 Solution Description

Editor’s Note: This clause provides a description of the solution

In order to address the Key Issues 1 and 2 we can rely on the IN-CSE to propagate the updates throughout the oneM2M system and to proactively delete the AE registration in the old MN-CSE.  The basic procedure is described below:

Step 1: The IN-CSE determines that an AE mobility event has occurred. An AE mobility event occurs when an AE has moved and has re-registered to a new ASN-CSE or MN-CSE.

For cases where the AE-ID is assigned by the IN-CSE (AE-ID-Stem starts with the letter ‘S’) the IN-CSE may examine the re-registration <AE> announcement. If this announcement is using a previously allocated AE-ID, but coming from a different ASN-CSE or MN-CSE, the IN-CSE can make the determination that this registration is as a result of an AE mobility event.

For cases where the AE-ID is assigned by a local CSE (an ASN-CSE or MN-CSE), then the AE-ID-Stem starts with the letter ‘C’. The local CSE may examine the registration request. If the AE supplies an AE-ID-Stem and the local CSE is unaware of this AE, then the CSE can make the determination that this registration is as a result of an AE mobility event. Once this determination is made, the local CSE needs to inform the IN-CSE. This can be achieved through a Notify request, which includes in its Content the old AE contact information (old URI) and new AE contact information (new URI).

Step 2: The IN-CSE examines all its services that have the old AE contact information, and it updates these with the new AE contact information. Basically this would involve looking in the attributes of all <group>, <subscription>, and <announce> resources, and replacing the old URI with the new URI.

Step 3: The IN-CSE informs MN-CSEs and ASN-CSEs that there has been an AE mobility event for this AE.  This can be achieved through a Notify request, which includes in its Content the old AE contact information (old URI) and new AE contact information (new URI).

Step 4: The MN-CSEs and ASN-CSEs examine their services to determine if they are impacted by the AE mobility event. If so, they update the old AE contact information with the new AE contact information. The Old MN-CSE will use this as an indication that it may delete the old <AE> registration.

10.1.2 Solution Applicability

Editor’s Note: This clause specifies which Key Issues are resolved using this solution

This solution applies to Key Issue 2 and Key Issue 3

Proforma copyright release text block

This text box shall immediately follow after the heading of an element (i.e. clause or annex) containing a proforma or template which is intended to be copied by the user. Such an element shall always start on a new page.

Notwithstanding the provisions of the copyright clause related to the text of the present document, oneM2M grants that users of the present document may freely reproduce the <proformatype> proforma in this {clause|annex} so that it can be used for its intended purposes and may further publish the completed <proformatype>.

<PAGE BREAK>

Annexes

Each annex shall start on a new page (insert a page break between annexes A and B, annexes B and C, etc.).

Use the Heading 9 style for the title and the Normal style for the text.

Annex <A>:
Title of annex (style H9)

<Text>

<PAGE BREAK>

Annex <B>:
Title of annex (style H9)

<Text>

B.1	First clause of the annex (style H1)

<Text>

B.1.1	First subdivided clause of the annex (style H2)

<Text>

<PAGE BREAK>

Annex <y>:
Bibliography

The annex entitled "Bibliography" is optional.

It shall contain a list of standards, books, articles, or other sources on a particular subject which are not mentioned in the document itself.

It shall not include references mentioned in the document.

Use the Heading 9 style for the title and B1+ or Normal for the text.

<Publication>: "<Title>".

OR

<Publication>: "<Title>".

<PAGE BREAK>

History

This clause shall be the last one in the document and list the main phases (all additional information will be removed at the publication stage).

| Publication history |  |  |
| --- | --- | --- |
| V.1.1.1 | <dd Mmm yyyy> | <Milestone> |

| Draft history (to be removed on publication) |  |  |
| --- | --- | --- |
| V.0.1.0 | 29-Oct-2015 | Skeleton draft |
| V.0.2.0 | 14-Mar-2016 | Merges of agreed contributions at REQ#22 and editorial updates |
| V.0.3.0 | 18-July-2016 | Merges of agreed contributions at REQ#23 and editorial updates |
| V.0.4.0 | 17-Oct-2016 | Merges of agreed contributions at REQ#24 and editorial updates |
| V.0.5.0 | 05-Dec-2016 | Merges of agreed contributions at REQ#25, ARC#25 and editorial updates |