| Doc# oneM2M-SEC-2014-0047-Key_hierarchy Input Contribution |  |
| --- | --- |

| INPUT CONTRIBUTION |  |
| --- | --- |
| Group Name:* | WG4 (SEC) Meeting during TP#9 |
| Title:* | SEC CSF key hierarchy |
| Source:* | Morpho Cards GmbH, Gemalto |
| Contact: | Oleg Krainioukov (oleg.krainioukov@morpho.com) Mireille Pauliac (mireille.pauliac@gemalto.com) |
| Date:* | 2014-02-03 |
| Abstract:* | This contribution proposes to add Key Hierarchy for oneM2M SEC CSF (Chapter 5.3). R01 is based on Version 0.3.0 of TS-0003 as available on the portal. |
| Agenda Item:* | TBD |
| Work item(s): | WI 0007 - oneM2M Security Analysis |
| Document(s) Impacted* | oneM2M-TS-0003 Security Solutions TS |
| Intended purpose of document:* | Decision Discussion Information Other <specify> |
| Decision requested or recommendation:* | Agree to include the changes proposed in revision marks in this input contribution to the Security Solutions TS |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

## Addition of a new text to section 5.3:

### 5.3.x	Key hierarchy and realisation

#### 5.3.x.1	Description of oneM2M keys

The following describes keys used for different levels of Authentication and Authorization in Security CSF:

M2M Root Key

- This key shall be used for mutual authentication and key agreement between the CSEs, for example between IN-CSE and MN-CSE. M2M Root Key is also used for deriving an M2M Connection Key, see below, through authentication and key agreement between the CSEs. M2M Root Key shall be coupled with a unique CSE an CSE-ID identifier, and may be bootstrapped in the CSE in various different ways, depending on the trust relationships between different stakeholders in the M2M ecosystem. One M2M Root Key exists for each set of provisioned CSE credentials. IN IN-CSE, M2M Root Key shall be stored within MAS.

- In scenarios where the M2M Service Provider and the Access Network Provider have a trust relationship (including the case that they are actually the same entity), access network credential may be used as M2M Root Key.

M2M Connection Key

- This key shall be derived from M2M Root Key, upon successful mutual authentication of the CSEs, as explained above. Upon derivation, oneM2M Connection Key shall be delivered from the MAS to the IN-CSE. M2M Connection Key shall expire upon termination of the corresponding Service Connection. Lifetime of oneM2M Connection Key shall be less than or equal to the lifetime of M2M Root Key. A different M2M Connection Key shall be generated for every new M2M Security association procedure of the CSE with the same or a different IN-CSE. M2M Connection Key is used as symmetric shared secret for setting up secure data sessions between CSEs.

M2M Application Key

- This key is derived from M2M Root Key. There shall be one M2M Application Key per AE. M2M Application Key is shared between AE and respective CSE. M2M Application Key should be used for authentication of AE by the Security CSF in the CSE and for protection of application data traffic.

#### 5.3.x.2	M2M Root Key Provisioning

The following scenarios are supported by the oneM2M architecture:

- 1)	Pre-provisioning

- M2M Root Key may be securely provisioned during manufacturing or deployment. This key could be securely stored inside the M2M device or one of its components (e.g., in UICC independently from the Network Access Application). In these cases, the M2M Service Provider is responsible for ensuring that M2M Nodes are provided with necessary M2M Root Keys using mechanisms that are not specified in the present document.

2)	Access Network based Bootstrapping

Sec CSF may leverage key material that is derived from Access Network Credentials, and use that key material to provision the M2M Root Key in Security CSF of a CSE.

3)	Access Network independent Bootstrapping

M2M Root Key may be provisioned in Security CSF in an access-network independent procedure. This scenario is applicable when the Access Network Operator and the M2M Service Provider do not share a business relationship and/or do not wish to use Access Network Credentials for bootstrapping of Service Layer credentials.

For options 1 and 3 there may be no need for Access Network Credentials, depending on the access network technology used.