Doc# SEC-2014-0278R01-TS-0003_DM_Remote_Administration

	INPUT CONTRIBUTION

	Group Name:*
	WG2

	Title:*
	TS-0003 DM Remote Administration

	Source:*
	Tim Carey, ALU, timothy.carey@alcatel-lucent.com

	Contact:
	Tim Carey, ALU, timothy.carey@alcatel-lucent.com

	Date:*
	2014-05-16

	Abstract:*
	This contribution proposes clarifications to the Remote admin clause and introduces DM for administration of the SE and sensitive data.

	Agenda Item:*
	Security TS

	Work item(s):
	WI 00-7

	Document(s) 

Impacted*
	Security TS (TS-0003)

	Intended purpose of

document:*
	 FORMCHECKBOX 
 Decision

 FORMCHECKBOX 
 Discussion

 Information

 Other <specify>

	Decision requested or recommendation:*
	Include in TS-0006


oneM2M Notice
The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.
-------------------------------Justification-------------------------------------------------------------------

In many devices in the field domain, the device doesn’t have a secure environment in the sense of a TEE or SIM card. In these scenarios, service providers many times utilize device management to administer the secure environment and sensitive data of the M2M Service Layer.
Note: TS-0003 suggests something called “sensitive data” without definition – need editors note to define sensitive data.

-----------------------Start of Change 1-------------

6.3.4.1.1
Interworking with SE in 3GPP and 3GPP2

In case the SE of the underlying network is used and in case the underlying network is a 3GPP or 3GPP2 network, remote administration mechanisms as specified in [7] and [8], and its extensions [9], [10] for 3GPP underlying networks or [11] and [12] for 3GPP2 underlying networks shall be used to securely administrate both the SE and the sensitive data of the M2M Service Layer.

Editors Note: Sensitive data of the M2M Service Layer needs definition to what specifically is considered as sensitive data that is not part of the SE.
---------------------- End of Change 1-------------

-----------------------Start of Change 2-------------

6.3.4.1.x
Remote Administration of SE Using Device Management
In case the where environment the M2M field domain nodes doesn’t utilize remote administration mechanisms in clause 6.3.4.1.1, Device Management  technologies [a], [b], [c] may be used to securely administrate the sensitive data of the M2M Service Layer.
---------------------- End of Change 2-------------

-----------------------Start of Changes to References Section -------------

2.1
Normative references

The following referenced documents are necessary for the application of the present document.
[a]
TR-069: “CPE WAN Management Protocol Issue”: 1 Amendment 5, November 2013, Broadband Forum  

[b]
OMA-DM: “OMA Device Management Protocol”, Version 1.3, Open Mobile Alliance
[c]
LWM2M: "OMA LightweightM2M”, Version 1.0, Open Mobile Alliance 
-----------------------End of Changes to References  -------------

© 2014 oneM2M Partners

Page 1 (of 3)

