| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SEC #33 |
| Source:* | Chengfang Fang, Huawei, fang.chengfang@huawei.com Jie Shi, Huawei, shi.jie1@huawei.com |
| Date:* | 2018-01-11 |
| Reason for Change/s:* | Add RSPF for IBC-Based Credential to support IBC-Based SAEF |
| CR against: Release* | Release 4 |
| CR against: WI* | Active <WI-0066> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS_0003 and v3.7.0 |
| Clauses * | Clauses 8 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) |  |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

The IBC-Based Security Association Establishment Frameworks in TR-0041 is proposed to be added into TS-0003

### -----------------------Start of change 1-------------------------------------------

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in oneM2M TR-0004 [i.2] and the following abbreviations apply:

(D)TLS-PSK	(D)TLS Pre-Shared Key (ciphersuites)

3GPP2	3rd Generation Partnership Project 2

AAA	Authentication, Authorization and Accounting

ABAC	Attribute Based Access Control

ACP	AccessControlPolicy Instance

AEAD	Authenticated Encryption with Associated Data

AE-ID	Application Entity Identifier

App-ID	Application Identifier

ASE	Asymmetric Secure Element

ASN-CSE	CSE which resides in the Application Service Node

AuthorSignReqInfo	Authorization Signature Request Information

AuthorSign	Authorization Signature

AuthorRelMapRecord	Authorization Relationship Mapping Record

AuthorRelIndicator	Authorization Relationship Indicator

AuthorSignIndicator Authorization Signature Indicator

BSF	Bootstrapping Server Function

B-TID	Bootstrapping Transaction Identifier

CA	Certification Authority or Certificate Authority

CIDR	Classless Inter-Domain Routing

CoAP	Constrained Application Protocol

CSE-ID	Common Service Entity Identifier

CSR	Certificate Signing Request

DTLS	Datagram Transport Layer Security (Protocol)

(D)TLS-PSK	(D)TLS Pre-Shared Key (ciphersuites)

ECC	Elliptic Curve Cryptography

EKU	Extended Key Usage

ESCertKE	End-to-End Certificate-based Key Establishment

Enrolee-ID	Enrolee Identity

ESData	End-to-End Security of Data

ESF	End-to-End Security Function

ESPrim	End-to-end Security of Primitives

EST	Enrolment over Secure Transport

ETSI	European Telecommunications Standards Institute

FQDN	Fully Qualified Domain Name

GBA_ME	ME-based GBA

GBA_U	GBA with UICC-based enhancements

GUSS	GBA User Security Settings

HLR	Home Location Register

HSS	Home Subscriber System

HTTP	HyperText Transfer Protocol

HW	Hardware

ID	Identifier

IdA	Identifier for entity A

IdB	Identifier for entity B

IN-CSE	CSE which resides in the Infrastructure Node

IPv4	Internet Protocol version 4

IPv6	Internet Protocol version 6

IV	Initialization Vector

Kc	M2M Secure Connection Key

KcID	M2M Secure Connection Key Identifier

Ke	Enrolment Key

KeID	Enrolment Key Identifier

Ker	Enrolment Re-Authentication Key

Km	Master Credential

KmID	Master Credential Identifier

Kpm	pre-provisioned credential for Master Credential provisioning

KpmID	pre-provisioned credential for Master Credential provisioning Identifier

Kpsa	provisioned credential for M2M Security Association Establishment

KpsaID	provisioned credential for M2M Security Association Establishment Identifier

Ks	temporary Key material referred to in GBA

Ks..NAF	Abbreviation of Ks_(int/ext)_NAF

Ks_(ext/int)_NAF	Derived key in GBA_ME or Derived key in GBA_U which remains on UICC

Ks_ext_NAF	Derived key in GBA_U sent to the ME

Ks_int_NAF	Derived key in GBA_U which remains on UICC

Ks_NAF	Derived key in the ME

M2M-SP	M2M Service Provider

MAF	M2M Authentication Function

MAF-ID	M2M Authentication Function Identifier

Mca	Reference Point for M2M Communication with AE

Mcc	Reference Point for M2M Communication with CSE

Mcc'	Reference Point for M2M Communication with CSE of different M2M Service Provider

Mcn	Reference Point for M2M Communication with NSE

MEF	M2M Enrolment Function

MIC	Message Integrity Code

MN-CSE	CSE which resides in the Middle Node

MTE	M2M Trust Enabler

NAF	Network Application Function

OAEP	Optimal Asymmetric Encryption Padding

OCSP	Online Certificate Status Protocol

PDP	Policy Decision Point

PEP	Policy Enforcement Point

PII	Personally Identifiable Information

PIN	Personal Identification Number

PIP	Policy Information Point

PKI	Public Key Infrastructure

PRP	Policy Retrieval Point

RA	Registration Authority

RSA	Rivest, Shamir and Adleman

RSAES			RSA Encryption Scheme

RSASSA			RSA Signature Scheme Algorithm

RSPF	Remote Security Provisioning Framework

SAEF	Security Association Establishment Framework

SCEP	Simple Certificate Enrolment Protocol

SE	Secure Environment

SUID	Security Usage Identifier

SW	Software

T&C	Terms and Conditions

TEE	Trusted Execution Environment

TEF	Trust Enabling Function

TLS	Transport Layer Security (Protocol)

UE	(3GPP) User Equipment

UNSP	Underlying Network Service Provider

URI	Uniform Resource Identifier

USS	User Security Settings

XACML	eXtensible Access Control Markup Language

### -----------------------End of change 1---------------------------------------------

### -----------------------Start of change 2-------------------------------------------

## 8.3	Remote Security Provisioning Frameworks

### 8.3.1	Overview on Remote Security Provisioning Frameworks

#### 8.3.1.1	Purpose of Remote Security Provisioning Frameworks

Remote Security Provisioning Frameworks (RSPFs) provision credentials to an Enrolee, which is a security principal in a Node or CSE or AE, as part of the Enrolment of the Enrolee to an M2M SP or M2M Trust Enabler. The MEF provides its services on behalf of administrating stakeholders such as M2M SPs or third party M2M Trust Enablers (MTE). An administrating stakeholder authorizes the MEF Service Provider to provide services to MEF clients, and oversees authorizing the management of credentials.

The credentials are either:

- A symmetric key shared by the Enrolee and an Enrolment Target, which may be a MAF or Node or CSE or AE:

If the Enrolment target is an MAF, then the credential can be used for MAF-based SAEF, MAF-based ESPrim and MAF-based ESData Protection Options, with the provisioned symmetric key used for mutual authentication of the Enrolee and the MAF.

If the Enrolment target is a Node or CSE or AE, then the credential can be used for only one of PSK-based SAEF or PSK- based ESPrim or PSK-based ESData Protection Options. The provisioned symmetric key used for mutual authentication of the Enrolee and the other Node or CSE or AE.

NOTE:	This case should be employed only in cases where the Enrolee is expected to require a symmetric key with relatively few CSE or AE.

- Certificate(s) for which the Enrolee knows the corresponding private key, and a set of trust anchors for authenticating the M2M SP or MTE's MAF or other entities enrolled with the M2M SP or MTE. These credentials can be used for:

Securing communication directly with other Nodes or CSEs or AEs using Certificate-Based SAEF, Direct End-to-End Key Establishment using Certificates (ESCertKE), and certificate-based ESData protection options. The other Nodes or CSEs or AEs would authenticate themselves using their own certificate(s), chaining to a provisioned trust anchor CA certificate, in these security frameworks.

MAF-based SAEF, MAF-based ESPrim, and MAF-based ESData protection options, with the certificate used for authentication of the Enrolee to the MAF. The MAF would authenticate using its own certificate chaining to a provisioned trust anchor CA certificate.

The oneM2M specifications also support provisioning of credentials using the device configuration mechanisms specified in oneM2M TS-0022 [57], and pre-provisioning; that is, provisioning using means other than a Remote Security Provisioning Framework. The method for pre-provisioning can be deployment dependent. Interoperable frameworks enabling pre-provisioning are described in annex D for UICC and in Annex L for independent hardware based secure environments supporting asymmetric cryptography.

- IBC credentials (ID, SSK, PVT, KPAK) including: an ID which is the identity of the Enrolee, a Secret Signing Key (SSK), a corresponding Public Validation Token (PVT) and a KMS Public Authentication Key (KPAK). These credentials can be used for:

Securing communication directly with other Nodes or CSEs or AEs using IBC-Based SAEF. The other Nodes or CSEs or AEs would derive a shared secret from their respective SSK, their respective peers’ ID and peers’ PVT together with the KPAK.

NOTE:	The RSPFs are specified to provide an interoperable interface for Field Domain entities to interact with an MEF. Use of the specified RSPFs are recommended for use by Field Domain entities because they have been reviewed by the security experts of oneM2M. The RSPFs can also be used by Infrastructure Domain entities (Nodes, AEs, CSEs and MAFs) for interacting with an MEF. It is expected that the MEF may include additional “backend” interfaces, not specified by oneM2M, for coordination of information with administrating stakeholders and MAF Service Providers.

### -----------------------End of change 2---------------------------------------------

### -----------------------Start of change 3--------------------------------------------

### 8.3.4	Enrolment Exchange

#### 8.3.4.1	Enrolment Exchange Procedures

The following procedures may occur within an Enrolment Exchange:

- MEF Client Registration procedures;

- Symmetric Key Provisioning procedures;

- Certificate Provisioning procedure;

- IBC-based Credential Provisioning procedures;

- Device Configuration procedures, per oneM2M TS-0022 [57] can be applied, with the MEF interacting with a DM Server and MEF Client interacting with the DM Client on the Managed Entity;

- MEF Client Command Procedures (i.e. CRUD procedures targeting at a <mefClientCmd> resource), which enable the MEF to control the sequence of Enrolment Exchange procedures.

The clauses below describe triggering mechanisms specific to each set of procedures. Alternatively, other mechanisms, not specified by oneM2M, can be used to trigger any Enrolment Exchange procedure, with the condition that such mechanisms provide a satisfactory level of security. Example mechanisms include pre-configuration and manual configuration.

#### 8.3.4.2	MEF Client Registration

MEF Client Registration procedures are specified in clauses 8.3.5.2.3, 8.3.5.2.4, 8.3.5.2.5, and 8.3.5.2.6.

MEF Client Registration procedures can only be performed within an Enrolment Exchange.

MEF Client Registration procedures can be triggered by the following oneM2M-specified mechanisms:

- Procedures triggered using Device Configuration. Device Configuration, specified in oneM2M TS-0022 [57], can trigger MEF Registration Procedures:

  - Adding a [MEFClientRegCfg] MO triggers the MEF Client to perform the MEF Client Registration Procedure, specified in clause 8.3.5.2.3.

  - Deleting a [MEFClientRegCfg] MO triggers the MEF Client to stop using the associated MEF Client registration, delete any credentials associated with that MEF Client registration and end the associated MEF Client registration on the MEF.  The MEF achieves the final step by performing the MEF Client De-Registration Procedure, specified in clause 8.3.5.2.6.

#### 8.3.4.2	MEF Client Registration

MEF Client Registration procedures are specified in clauses 8.3.5.2.3, 8.3.5.2.4, 8.3.5.2.5, and 8.3.5.2.6.

MEF Client Registration procedures can only be performed within an Enrolment Exchange.

MEF Client Registration procedures can be triggered by the following oneM2M-specified mechanisms:

- Procedures triggered using Device Configuration. Device Configuration, specified in oneM2M TS-0022 [57], can trigger MEF Registration Procedures:

  - Adding a [MEFClientRegCfg] MO triggers the MEF Client to perform the MEF Client Registration Procedure, specified in clause 8.3.5.2.3.

  - Deleting a [MEFClientRegCfg] MO triggers the MEF Client to stop using the associated MEF Client registration, delete any credentials associated with that MEF Client registration and end the associated MEF Client registration on the MEF.  The MEF achieves the final step by performing the MEF Client De-Registration Procedure, specified in clause 8.3.5.2.6.

#### 8.3.4.3	Symmetric Key Provisioning

Symmetric Key Provisioning procedures are specified in clauses 8.3.5.2.7, 8.3.5.2.8, 8.3.5.2.9, and 8.3.5.2.10.

These procedures can only be performed within an Enrolment Exchange.

These procedures can be triggered by the following oneM2M-specified mechanisms:

- Procedures triggered using  a “MO_Node” MEF Client Command: Device Provisioning (oneM2M TS-0022 [57]) can be used to configure MEF Client with an [authenticationProfile] MO which has a child [MEFClientRegCfg] MO node to instruct the MEF Client that Symmetric Key Provisioning will be used for credentials used in that [authenticationProfile] MO. If a MEF Client receives of an “MO_NODE” MEF Client Command matching the path of such a [authenticationProfile] MO, then this can trigger a Symmetric Key Provisioning procedure according to the information elements in the MEF Client Command and the current values of the parameters in these MO nodes.

NOTE:   Using Device Configuration to update or delete the [authenticationProfile] MO and/or its child [MEFClientRegCfg] MO node does not implicitly trigger a Symmetric Key Provisioning procedure. The update or delete will not take effect until a Symmetric Key Provisioning procedure is trigger by some other mechanisms.

- Procedures triggered by an expiry of a MEF Key Registration. If the MEF Client previously (successfully) executed an MEF Key Registration procedure under the control of an [authenticationProfile] MO on the MEF Client, and the current time is greater than the expirationTime of the [authenticationProfile] resource, and if the current time is close to or greater than the expirationTime of the most recent MEF Key Registration, then this can trigger the MEF Client to perform MEF Key Registration. The criteria for being “close to the expirationTime” is left up to the implementation of the MEF Client.

- Procedures triggered by receiving, within a oneM2M security protocol, a symmetric key identifier whose FQDN matches the MEF's FQDN. If a Target MEF Client receives, within a oneM2M security protocol, a symmetric key identifier whose FQDN matches the MEF's FQDN, then this can trigger the Target MEF Client to execute the MEF Key Retrieval Procedure specified in clause 8.3.5.2.8. See steps 6 and 7 in clause 8.3.5.1.

#### 8.3.4.4	Certificate Provisioning

Certificate Provisioning procedures are specified in clause 8.3.6.

These procedures can only be performed within an Enrolment Exchange.

These procedures can be triggered by the following oneM2M-specified mechanisms:

- Procedures triggering using MEF Client Command Procedure: If the MEF Client receives a MEF Client Command identifying a Certificate Provisioning Procedure, then this triggers the MEF Client to execute the Certificate Provisioning procedure using the information elements included in the command.

#### 8.3.4.5	IBC Credential Provisioning

IBC Credential Provisioning procedures are specified in clause 8.3.7.

These procedures can only be performed within an Enrolment Exchange.

These procedures can be triggered by the following oneM2M-specified mechanisms:

- Procedures triggering using MEF Client Command Procedure: If the MEF Client receives a MEF Client Command identifying a IBC Credential Provisioning Procedure, then this triggers the MEF Client to execute a IBC Credential Provisioning procedure using the information elements included in the command.

#### 8.3.4.6	Device Configuration

Device Configuration is specified in oneM2M TS-0022 [57].

Device Configuration can be performed within an Enrolment Exchange with a MEF, or in a DM session with other DM servers (separate from an Enrolment Exchange). Clause 8.3.8 specifies use of Device Configuration within an Enrolment Exchange with a MEF.

Device Configuration can be triggered by the following oneM2M-specified mechanisms:

- Procedures triggered using MEF Client Command Procedure: If the MEF Client receives a MEF Client Command identifying the Device Configuration Procedure, then this trigger the MEF Client to execute a Device Configuration session using the information elements included in the command.

#### 8.3.4.7	MEF Client Command

MEF Client Command procedures are specified in clause 8.3.9.

MEF Client Command procedures can only be performed within an Enrolment Exchange.

MEF Client Command procedures can be triggered by the following oneM2M-specified mechanisms:

- Procedures triggered following MEF Client Registration Procedure

  - A MEF Client Command Retrieve shall be executed following an MEF Client Registration procedure (other than MEF Client De-registration).

- Procedures triggered according to retryDuration

  - When the MEF issues a NO_MORE_COMMANDS MEF Client Command, then the cmdArgs includes a retryDuration providing the duration after which the MEF Client attempts MEF Client Command Retrieve. A retryDuration is cancelled whenever the MEF Client successfully interacts with the MEF prior to this time. For further details see clause.8.3.9.6.

- Procedures triggered following an attempt to perform an issued MEF Client Command

  - If the MEF Client has attempted executing a previously issued MEF Client Command, then the MEF Client shall perform the MEF Client Command Update procedure to report on the status of that execution. The MEF can issue a MEF Client Command in the response.

An Example of a MEF Client Command procedure is illustrated in figure 8.3.4.6-1.

Figure 8.3.4.6-1: Example MEF Client Command procedure

- The MEF client sends an MEF Client Registration request.

- The MEF creates a <mefClientReg> resource.

- If the MEF wants to issue a MEF Client Command it creates a <mefClientCmd> resource as child of the <mefClientReg> resource

- The MEF sends the MEF Client Registration response which includes a representation of the <mefClientReg> resource, including the childResource reference, whose value represents the resource ID of a <mefClientCmd> resource.

- The presence of the childResource reference triggers the MEF client to retrieve the <mefClientCmd> resource. The MEF Client sends a MEF Client Command Retrieve request to the MEF

- The MEF forms the response.

- The MEF returns a MEF Client Command Retrieve response which includes the <mefClientCmd> resource.

- The MEF client parses the received response and executes the command included therein.

- After execution of the command, the MEF client reports the result to the MEF by a MEF Client Command Update Request

- The MEF updates the <mefClientCmd>. If the MEF has a new command for the MEF Client it indicates a trigger in the representation of the <mefClientCmd> resource.

- The MEF sends the MEF Client Command Update Response. If the received response includes another MEF Client Command, steps 8 to 11 are repeated.

### -----------------------End of change 4---------------------------------------------

### -----------------------Start of change 5-------------------------------------------

### 8.3.7	IBC Credential Provisioning Procedure Details

#### 8.3.7.1	Introduction

Clause 8.3.7 describes the details of the IBC Credential Provisioning Procedures.

The IBC Credential Provisioning procedures include the following actors:

- MEF Client:  a Security Principal requesting provisioning of an MEF-Provisioned credential. The MEF Client uses the MEF-Provisioned credential for subsequent authentication of itself to the MEF. The Security Principal can use the MEF-Provisioned credential for subsequent authentication of itself in other oneM2M Security Principals.

- KMS: issuing MEF-Provisioned IBC credential. The KMS is configured with master public key and master private key. The KMS accepts an entity’s identity and generates a corresponding IBC private key[18][i.29].

- MEF: serving requests from the MEF Client, and forward IBC credentials generation requests towards the KMS.

#### 8.3.7.2	Initial IBC Credential Provisioning procedure

Purpose: Enabling an MEF Client to request its first IBC credential from the MEF.

Pre-Conditions:

- The MEF Client is provided with the IBCBasedURI whose FQDN shall match the FQDN of the MEF, and is triggered to perform IBC Credential Provisioning procedure.

- The MEF Client and MEF have successfully performed a MEF Handshake and the MEF associates an identifier with the MEF Client. For the Initial IBC Credential Provisioning procedure, one of the following RSPFs shall be used.

- PPSK-Based RSPF as described in clause 8.3.2.1.

- Certificate-Based RSPF as described in clause 8.3.2.2.

Procedure Description:

- The MEF Client shall send a request with its identity ID (CSE-ID or AE-ID) to the MEF;

- The MEF shall generate a new ID_2 for the enrolee based on the FQDN, the received identity ID and the expiration time for this IBC credential (i.e. ID_2 = FQDN@hexBinary([ID])@[expirationTime]);

NOTE 1: hexBinary([ID]) denotes the hexadecimal representation of the binary value of the received identity ID. The generated ID_2 shall be unique.

NOTE 2: The MEF forwards the ID_2 to the Key Management Service(KMS). The KMS generates a private key SSK and a Public Validation Token (PVT) for the received ID_2 with the master public KPAK and the master secret key KSAK[i.29]; and sends the SSK, PVT and KPAK back to the MEF.

- The MEF shall send (ID_2, SSK, PVT, KPAK) to the Enrolee.

##### 8.3.7.3	IBC Credential Re-Provisioning procedure

### Purpose: Enabling an MEF Client to renew/rekey a currently valid Enrolled IBC credential.

Pre-Conditions:

- The MEF Client has previously performed the Initial IBC Credential Provisioning procedure or IBC Credential Re-Provisioning Procedure with the MEF, and the MEF Client has installed its IBC credential from the most recent such procedure.

- The MEF Client is provided with the IBCBasedURI whose FQDN shall match the FQDN of the MEF, and is triggered to perform IBC Credential Provisioning procedure.

- The MEF Client and MEF have successfully performed a MEF Handshake and the MEF associates an identifier with the MEF Client. As in pre-condition B of clause 8.3.7.2.

Procedure Description:

- The MEF Client shall send a request with its ID_1 = FQDN@hexBinary([ID’])@[expirationTime]to the MEF;

- The MEF shall generate a new ID_2 for the enrolee based on the FQDN, the MEF Client’s identity ID’(CSE-ID or AE-ID) and the new expiry time (i.e. ID_2 = FQDN@hexBinary([ID’])@[expirationTime’]);

NOTE 3: hexBinary([ID’]) denotes the hexadecimal representation of the binary value of ID’; The generated ID_2 shall be unique.

NOTE 4: The MEF forwards the new ID_2 to the KMS. The KMS generates corresponding SSK and PVT, and sends the SSK, PVT and KPAK back to the MEF. As in NOTE 2 of clause 8.3.7.2.

- The MEF shall send (ID_2, SSK, PVT, KPAK) to the Enrolee.

### -----------------------End of change 5---------------------------------------------

### -----------------------Start of change 6-------------------------------------------

### 8.3.8	MEF Client Configuration Details

### -----------------------End of change 6---------------------------------------------

### -----------------------Start of change 7-------------------------------------------

### 8.3.9	Profile for Device Configuration within an Enrolment Exchange

### -----------------------End of change 7---------------------------------------------

### -----------------------Start of change 8-------------------------------------------

### 8.3.10	MEF Client Command Processing

### -----------------------End of change 8---------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?