| oneM2M Technical Specification |  |
| --- | --- |
| Document Number | TS-0007 -V0.8.1 |
| Document Name: | Service Components |
| Date: | 2015-May-05 |
| Abstract: | This document specifies the M2M Services provided by the M2M Services Platform, the integration and interworking of the M2M Services functional architecture of the oneM2M Services Platform and informatively illustrates the use of the M2M Services within the context of complex business services. |

This Specification is provided for future development work within oneM2M only. The Partners accept no liability for any use of this Specification.

The present document has not been subject to any approval process by the oneM2M Partners Type 1. Published oneM2M specifications and reports for implementation should be obtained via the oneM2M Partners' Publications Offices.

About oneM2M

The purpose and goal of oneM2M is to develop technical specifications which address the need for a common M2M Service Layer that can be readily embedded within various hardware and software, and relied upon to connect the myriad of devices in the field with M2M application servers worldwide.

More information about oneM2M may be found at: http//www.oneM2M.org

Copyright Notification

No part of this document may be reproduced, in an electronic retrieval system or otherwise, except as authorized by written permission.

The copyright and the foregoing restriction extend to reproduction in all media.

© 2015, oneM2M Partners Type 1 (ARIB, ATIS, CCSA, ETSI, TIA, TTA, TTC).

All rights reserved.

Notice of Disclaimer & Limitation of Liability

The information provided in this document is directed solely to professionals who have the appropriate degree of experience to understand and interpret its contents in accordance with generally accepted engineering or other professional standards and applicable regulations. No recommendation as to products or vendors is made or should be implied.

NO REPRESENTATION OR WARRANTY IS MADE THAT THE INFORMATION IS TECHNICALLY ACCURATE OR SUFFICIENT OR CONFORMS TO ANY STATUTE, GOVERNMENTAL RULE OR REGULATION, AND FURTHER, NO REPRESENTATION OR WARRANTY IS MADE OF MERCHANTABILITY OR FITNESS FOR ANY PARTICULAR PURPOSE OR AGAINST INFRINGEMENT OF INTELLECTUAL PROPERTY RIGHTS. NO oneM2M PARTNER TYPE 1 SHALL BE LIABLE, BEYOND THE AMOUNT OF ANY SUM RECEIVED IN PAYMENT BY THAT PARTNER FOR THIS DOCUMENT, WITH RESPECT TO ANY CLAIM, AND IN NO EVENT SHALL oneM2M BE LIABLE FOR LOST PROFITS OR OTHER INCIDENTAL OR CONSEQUENTIAL DAMAGES. oneM2M EXPRESSLY ADVISES ANY AND ALL USE OF OR RELIANCE UPON THIS INFORMATION PROVIDED IN THIS DOCUMENT IS AT THE RISK OF THE USER.

Contents

1	Scope	27

2	References	27

2.1	Normative references	27

2.2	Informative references	27

3	Definitions and abbreviations	27

3.1	Definitions	27

3.2	Abbreviations	28

4	Conventions	29

5	M2M Services Architecture	29

5.1	Introduction	29

5.2	Reference Points	31

5.2.1	Introduction	31

5.2.2	Mca Reference Point	31

5.2.3	Msc Reference Point	31

5.2.3.1	Message Exchange Patterns	31

5.2.3.2	M2M Identifiers	32

5.2.3.2.1	M2M Service Capability Identifier (M2M-Serv-Cap-ID)	32

5.2.4	Mcc' Reference Point	33

5.2.5	Mcn Reference Point	33

5.3	Configurations support by M2M Service Architecture	34

6	M2M Services	34

6.1	Introduction	34

6.2	Service Subscription	34

6.2.1	Overview	34

6.2.2	M2M Service Subscription Entities	34

6.2.2.1	Overview	34

6.2.2.2	M2M Service Subscription Broker Entities	35

6.2.2.3	M2M Service Subscription Management Adapter Entities	35

6.2.2.4	M2M Service Subscription Transport Adapter Entities	36

6.2.3	Service Capabilities	37

6.2.3.1	validateServiceSubscription	37

6.2.3.1.1	Description	37

6.2.3.1.2	Pre-Conditions	37

6.2.3.1.3	Signature - validateServiceSubscription	37

6.2.3.1.4	Post-Conditions	37

6.2.3.1.5	Exceptions	37

6.2.3.1.6	Policies for Use	37

6.2.3.1.7	oneM2M Resource Interworking	37

6.2.3.2	getAE	37

6.2.3.2.1	Description	37

6.2.3.2.2	Pre-Conditions	38

6.2.3.2.3	Signature – getAE	38

6.2.3.2.4	Service Interactions	38

6.2.3.2.5	Post-Conditions	38

6.2.3.2.6	Exceptions	38

6.2.3.2.7	Policies for Use	38

6.2.3.2.8	oneM2M Resource Interworking	39

6.2.3.3	getBroker	39

6.2.3.3.1	Description	39

6.2.3.3.2	Pre-Conditions	39

6.2.3.3.3	Signature – getBroker	39

6.2.3.3.4	Post-Conditions	39

6.2.3.3.5	Exceptions	39

6.2.3.3.6	Policies for Use	39

6.2.3.3.7	oneM2M Resource Interworking	39

6.2.3.4	getManagementAdapter	39

6.2.3.4.1	Description	39

6.2.3.4.2	Pre-Conditions	40

6.2.3.4.3	Signature - getManagementAdapter	40

6.2.3.4.4	Post-Conditions	40

6.2.3.4.5	Exceptions	40

6.2.3.4.6	Policies for Use	40

6.2.3.4.7	oneM2M Resource Interworking	40

6.2.3.5	getTransportAdapter	41

6.2.3.5.1	Description	41

6.2.3.5.2	Pre-Conditions	41

6.2.3.5.3	Signature - getTransportAdapter	41

6.2.3.5.4	Post-Conditions	41

6.2.3.5.5	Exceptions	41

6.2.3.5.6	Policies for Use	41

6.2.3.5.7	oneM2M Resource Interworking	41

6.2.3.6	associateAEWithServiceSubscription	41

6.2.3.6.1	Description	41

6.2.3.6.2	Pre-Conditions	41

6.2.3.6.3	Signature – associateAEWithServiceSubscription	42

6.2.3.6.4	Service Interactions	42

6.2.3.6.5	Post-Conditions	42

6.2.3.6.6	Exceptions	42

6.2.3.6.7	Policies for Use	42

6.2.3.6.8	oneM2M Resource Interworking	43

6.2.3.7	disassociateAEFromServiceSubscription	43

6.2.3.7.1	Description	43

6.2.3.7.2	Pre-Conditions	43

6.2.3.7.3	Signature - disassociateAEFromServiceSubscription	43

6.2.3.7.4	Service Interactions	43

6.2.3.7.5	Post-Conditions	43

6.2.3.7.6	Exceptions	43

6.2.3.7.7	Policies for Use	43

6.2.3.7.8	oneM2M Resource Interworking	44

6.2.3.8	refreshAEAssociationWithServiceSubscription	44

6.2.3.8.1	Description	44

6.2.3.8.2	Pre-Conditions	44

6.2.3.8.3	Signature – refreshAEAssociationWithServiceSubscription	44

6.2.3.8.4	Service Interactions	44

6.2.3.8.5	Post-Conditions	45

6.2.3.8.6	Exceptions	45

6.2.3.8.7	Policies for Use	45

6.2.3.8.8	oneM2M Resource Interworking	45

6.3	Authorization	45

6.3.1	Overview	45

6.3.2	Service Capabilities	46

6.3.2.1	authorizeServiceCapability	46

6.3.2.1.1	Description	46

6.3.2.1.2	Pre-Conditions	46

6.3.2.1.3	Common M2M Service Capability Parameters for Request Authentication and Authorization	46

6.3.2.1.4	Service Interactions	46

6.3.2.1.5	Post-Conditions	46

6.3.2.1.6	Exceptions	46

6.3.2.1.7	Policies for Use	46

6.3.2.1.8	oneM2M Resource Interworking	46

6.4	Data Exchange	47

6.4.1	Overview	47

6.4.2	Data Exchange Entities	47

6.4.2.1	Subscribe-Publish-Notify Data Exchange	47

6.4.2.1.1	Overview	47

6.4.2.1.2	Supporting Rules	47

6.4.2.1.2.1	Overview	47

6.4.2.1.1.2	Publication Resources	47

6.4.2.1.1.3	Delivery Policy	47

6.4.2.1.1.4	Message Retainment Policy	47

6.4.2.1.1.5	Service Subscription Integration	48

6.4.2.2	Request-Response Data Exchange	48

6.4.2.2.1	Overview	48

6.4.2.2.2	Supporting Rules	48

6.4.2.2.2.1	Overview	48

6.4.2.2.2.2	Delivery Policy	48

6.4.2.2.2.3	Service Subscription Integration	48

6.4.3	Service Capabilities	48

6.4.3.1	subscribe	48

6.4.3.1.1	Description	48

6.4.3.1.2	Pre-Conditions	49

6.4.3.1.3	Signature – subscribe	49

6.4.3.1.4	Service Interactions	49

6.4.3.1.5	Post-Conditions	50

6.4.3.1.6	Exceptions	50

6.4.3.1.7	Policies for Use	50

6.4.3.1.8	oneM2M Resource Interworking	50

6.4.3.2	publish	51

6.4.3.2.1	Description	51

6.4.3.2.2	Pre-Conditions	51

6.4.3.2.3	Signature - publish	51

6.4.3.2.4	Service Interactions	51

6.4.3.2.5	Post-Conditions	52

6.4.3.2.6	Exceptions	52

6.4.3.2.7	Policies for Use	52

6.4.3.2.8	oneM2M Resource Interworking	52

6.4.3.3	notify	53

6.4.3.3.1	Description	53

6.4.3.3.2	Pre-Conditions	53

6.4.3.3.3	Signature - notify	53

6.4.3.3.4	Post-Conditions	53

6.4.3.3.5	Exceptions	53

6.4.3.3.6	Policies for Use	53

6.4.3.3.7	oneM2M Resource Interworking	53

6.4.3.4	sendMessage	53

6.4.3.4.1	Desription	53

6.4.3.4.2	Pre-Conditions	54

6.4.3.4.3	Signature - sendMessage	54

6.4.3.4.4	Service Interactions	54

6.4.3.4.5	Post-Conditions	55

6.4.3.4.6	Exceptions	55

6.4.3.4.7	Policies for Use	55

6.4.3.4.8	oneM2M Resource Interworking	56

6.5	Broker	56

6.5.1	Overview	56

6.5.2	Service Capabilities	56

6.5.2.1	subscribe	56

6.5.2.1.1	Description	56

6.5.2.1.2	Pre-Conditions	56

6.5.2.1.3	Signature - subscribe	56

6.5.2.1.4	Post-Conditions	56

6.5.2.1.5	Exceptions	56

6.5.2.1.6	Policies for Use	56

6.5.2.2	publish	56

6.5.2.2.1	Description	56

6.5.2.2.2	Pre-Conditions	57

6.5.2.2.3	Signature - publish	57

6.5.2.3.4	Post-Conditions	57

6.5.2.3.5	Exceptions	57

6.5.2.3.6	Policies for Use	57

6.5.2.3.7	oneM2M Resource Interworking	57

6.5.2.3	notify	57

6.5.2.3.1	Description	57

6.5.2.3.2	Pre-Conditions	58

6.5.2.3.3	Signature - notify	58

6.5.2.3.4	Service Interactions	58

6.5.2.3.5	Post-Conditions	59

6.5.2.3.6	Exceptions	59

6.5.2.3.7	Policies for Use	59

6.5.2.3.8	oneM2M Resource Interworking	60

6.5.2.4	sendMessage	60

6.5.2.4.1	Description	60

6.5.2.4.2	Pre-Conditions	60

6.5.2.4.3	Signature - sendMessage	60

6.5.2.4.4	Post-Conditions	60

6.5.2.4.5	Exceptions	60

6.5.2.4.6	Policies for Use	60

6.5.2.4.6	oneM2M Resource Interworking	60

6.6	Device Management	60

6.6.1	Overview	60

6.6.2	Device Management Entities	61

6.6.2.1	Overview	61

6.6.2.2	Supporting Rules	61

6.6.2.2.1	Report Policy Type	61

6.6.2.2.2	Batch Report Type	61

6.6.2.2.3	Firmware Report Type	61

6.6.2.2.4	Orchestration Rule Type	62

6.6.2.2.5	Firmware Info Type	62

6.6.2.2.6	Service Subscription Integration	62

6.6.2.2.7	Device Info Type	62

6.6.2.2.8	Memory Type	63

6.6.2.2.9	Battery Type	63

6.6.2.2.10	DeviceCapability Type	63

6.6.2.2.11	BatteryStatus Enum	63

6.6.2.2.12	DeviceCapabilityName Enum	64

6.6.2.2.13	LockStatus Enum	64

6.6.2.2.14	Area Network Info Type	64

6.6.2.2.15	Area Network Device Type	65

6.6.2.2.16	Area Network Device Info Type	65

6.6.2.2.17	LogTypeId Enum	65

6.6.2.2.18	LogStatus Enum	65

6.6.2.2.19	LogInfo Type	65

6.6.2.2.20	TroubleshootingReport Type	66

6.6.2.2.21	Log Type	66

6.6.2.2.22	Log Filter Criteria	66

6.6.2.2.23	Action Enum	66

6.6.2.2.24	Status Enum	67

6.6.2.2.25	ActionStatus Type	67

6.6.2.2.26	SoftwareReport Type	67

6.6.2.2.27	PendingReport Enum	67

6.6.2.2.28	OperationMonitor Enum	68

6.6.3	Service Capabilities	68

6.6.3.1	downloadFirmware	68

6.6.3.1.1	Description	68

6.6.3.1.2	Pre-Conditions	68

6.6.3.1.3	Signature - downloadFirmware	68

6.6.3.1.4	Service Interactions	68

6.6.3.1.5	Post-Conditions	69

6.6.3.1.6	Exceptions	69

6.6.3.1.7	Policies for Use	69

6.6.3.1.8	oneM2M Resource Interworking	69

6.6.3.2	installFirmware	69

6.6.3.2.1	Description	69

6.6.3.2.2	Pre-Conditions	69

6.6.3.2.3	Signature - installFirmware	70

6.6.3.2.4	Service Interactions	70

6.6.3.2.5	Post-Conditions	70

6.6.3.2.6	Exceptions	70

6.6.3.2.7	Policies for Use	70

6.6.3.2.8	oneM2M Resource Interworking	71

6.6.3.3	getFirmwareInformation	71

6.6.3.3.1	Description	71

6.6.3.3.2	Pre-Conditions	71

6.6.3.3.3	Signature - getFirmwareInformation	71

6.6.3.3.4	Service Interactions	71

6.6.3.3.5	Post-Conditions	71

6.6.3.3.6	Exceptions	72

6.6.3.3.7	Policies for Use	72

6.6.3.3.8	oneM2M Resource Interworking	72

6.6.3.4	getFirmwareExecStatus	72

6.6.3.4.1	Description	72

6.6.3.4.2	Pre-Conditions	72

6.6.3.4.3	Signature – getFirmwareExecStatus	72

6.6.3.4.4	Service Interactions	72

6.6.3.4.5	Post-Conditions	73

6.6.3.4.6	Exceptions	73

6.6.3.4.7	Policies for Use	73

6.6.3.4.8	oneM2M Resource Interworking	73

6.6.3.5	deviceManagementReport	73

6.6.3.5.1	Description	73

6.6.3.5.2	Pre-Conditions	73

6.6.3.5.3	Signature – deviceManagementReport	74

6.6.3.5.4	Post-Conditions	74

6.6.3.5.5	Exceptions	74

6.6.3.5.6	Policies for Use	74

6.6.3.57	oneM2M Resource Interworking	74

6.6.3.6	upgradeFirmware	74

6.6.3.6.1	Description	74

6.6.3.6.2	Pre-Conditions	74

6.6.3.6.3	Signature - upgradeFirmware	75

6.6.3.6.4	Service Interactions	75

6.6.3.6.5	Post-Conditions	76

6.6.3.6.6	Exceptions	76

6.6.3.6.7	Policies for Use	76

6.6.3.6.8	oneM2M Resource Interworking	76

6.6.3.7	getDeviceInformation	76

6.6.3.7.1	Description	76

6.6.3.7.2	Pre-Conditions	76

6.6.3.7.3	Signature - getDeviceInformation	76

6.6.3.7.4	Service Interactions	76

6.6.3.7.5	Post-Conditions	77

6.6.3.7.6	Exceptions	77

6.6.3.7.7	Policies for Use	77

6.6.3.7.8	oneM2M Resource Interworking	77

6.6.3.8	getDeviceCapabilities	77

6.6.3.8.1	Description	77

6.6.3.8.2	Pre-Conditions	77

6.6.3.8.3	Signature – getDeviceCapabilities	78

6.6.3.8.4	Service Interactions	78

6.6.3.8.5	Post-Conditions	78

6.6.3.8.6	Exceptions	78

6.6.3.8.7	Policies for Use	78

6.6.3.8.8	oneM2M Resource Interworking	78

6.6.3.9 	enableDeviceCapability	78

6.6.3.9.1	Description	78

6.6.3.9.2	Pre-Conditions	79

6.6.3.9.3	Signature –enableDeviceCapability	79

6.6.3.9.4	Service Interactions	79

6.6.3.9.5	Post-Conditions	79

6.6.3.9.6	Exceptions	79

6.6.3.9.7	Policies for Use	79

6.6.3.9.8	oneM2M Resource Interworking	80

6.6.3.10	disableDeviceCapability	80

6.6.3.10.1	Description	80

6.6.3.10.2	Pre-Conditions	80

6.6.3.10.3	Signature –disableDeviceCapability	80

6.6.3.10.4	Service Interactions	80

6.6.3.10.5	Post-Conditions	80

6.6.3.10.6	Exceptions	81

6.6.3.10.7	Policies for Use	81

6.6.3.10.8	oneM2M Resource Interworking	81

6.6.3.11	getAreaNetworks	81

6.6.3.11.1	Description	81

6.6.3.11.2	Pre-Conditions	81

6.6.3.11.3	Signature - getAreaNetworks	81

6.6.3.11.4	Service Interactions	81

6.6.3.11.5	Post-Conditions	82

6.6.3.11.6	Exceptions	82

6.6.3.11.7	Policies for Use	82

6.6.3.11.8	oneM2M Resource Interworking	82

6.6.3.12	updateDeviceForAreaNetwork	82

6.6.3.12.1	Description	82

6.6.3.12.2	Pre-Conditions	82

6.6.3.12.3	Signature – updateDeviceForAreaNetwork	82

6.6.3.12.4	Service Interactions	82

6.6.3.12.5	Post-Conditions	83

6.6.3.12.6	Exceptions	83

6.6.3.12.7	Policies for Use	83

6.6.3.12.8	oneM2M Resource Interworking	83

6.6.3.13	rebootDevice	83

6.6.3.13.1	Description	83

6.6.3.13.2	Pre-Conditions	83

6.6.3.13.3	Signature - rebootDevice	84

6.6.3.13.4	Service Interactions	84

6.6.3.13.5	Post-Conditions	84

6.6.3.13.6	Exceptions	84

6.6.3.13.7	Policies for Use	84

6.6.3.13.8	oneM2M Resource Interworking	84

6.6.3.14	resetDevice	85

6.6.3.14.1	Description	85

6.6.3.14.2	Pre-Conditions	85

6.6.3.14.3	Signature - resetDevice	85

6.6.3.14.4	Service Interactions	85

6.6.3.14.5	Post-Conditions	85

6.6.3.14.6	Exceptions	86

6.6.3.14.7	Policies for Use	86

6.6.3.14.8	oneM2M Resource Interworking	86

6.6.3.15	uploadDeviceLog	86

6.6.3.15.1	Description	86

6.6.3.15.2	Pre-Conditions	86

6.6.3.15.3	Signature – uploadDeviceLog	86

6.6.3.15.4	Service Interactions	86

6.6.3.15.5	Post-Conditions	87

6.6.3.15.6	Exceptions	87

6.6.3.15.7	Policies for Use	87

6.6.3.15.8	oneM2M Resource Interworking	87

6.6.3.16	getDeviceLogs	87

6.6.3.16.1	Description	87

6.6.3.16.2	Pre-Conditions	87

6.6.3.16.3	Signature - getDeviceLogs	87

6.6.3.16.4	Service Interactions	88

6.6.3.16.5	Post-Conditions	88

6.6.3.16.6	Exceptions	88

6.6.3.16.7	Policies for Use	88

6.6.3.16.8	oneM2M Resource Interworking	88

6.6.3.17	getDeviceLogInformation	88

6.6.3.17.1	Description	88

6.6.3.17.2	Pre-Conditions	88

6.6.3.17.3	Signature – getDeviceLogInformation	89

6.6.3.17.4	Service Interactions	89

6.6.3.17.5	Post-Conditions	89

6.6.3.17.6	Exceptions	89

6.6.3.17.7	Policies for Use	89

6.6.3.17.8	oneM2M Resource Interworking	89

6.6.3.18	getSoftwareInformation	90

6.6.3.18.1	Description	90

6.6.3.18.2	Pre-Conditions	90

6.6.3.18.3	Signature – getSoftwareInformation	90

6.6.3.18.4	Service Interactions	90

6.6.3.18.5	Post-Conditions	90

6.6.3.18.6	Exceptions	91

6.6.3.18.7	Policies for Use	91

6.6.3.18.8	oneM2M Resource Interworking	91

6.6.3.19	downloadSoftware	91

6.6.3.19.1	Description	91

6.6.3.19.2	Pre-Conditions	91

6.6.3.19.3	Signature –downloadSoftware	91

6.6.3.19.4	Service Interactions	91

6.6.3.19.5	Post-Conditions	92

6.6.3.19.6	Exceptions	92

6.6.3.19.7	Policies for Use	92

6.6.3.19.8	oneM2M Resource Interworking	92

6.6.3.20	installSoftware	92

6.6.3.20.1	Description	92

6.6.3.20.2	Pre-Conditions	92

6.6.3.20.3	Signature –installSoftware	93

6.6.3.20.4	Service Interactions	93

6.6.3.20.5	Post-Conditions	93

6.6.3.20.6	Exceptions	93

6.6.3.20.7	Policies for Use	93

6.6.3.20.8	oneM2M Resource Interworking	93

6.6.3.21	activateSoftware	94

6.6.3.21.1	Description	94

6.6.3.21.2	Pre-Conditions	94

6.6.3.21.3	Signature –activateSoftware	94

6.6.3.21.4	Service Interactions	94

6.6.3.21.5	Post-Conditions	94

6.6.3.21.6	Exceptions	94

6.6.3.21.7	Policies for Use	95

6.6.3.21.8	oneM2M Resource Interworking	95

6.6.3.22	deactivateSoftware	95

6.6.3.22.1	Description	95

6.6.3.22.2	Pre-Conditions	95

6.6.3.22.3	Signature –deactivateSoftware	95

6.6.3.22.4	Service Interactions	95

6.6.3.22.5	Post-Conditions	96

6.6.3.22.6	Exceptions	96

6.6.3.22.7	Policies for Use	96

6.6.3.22.8	oneM2M Resource Interworking	96

6.6.3.23	removeSoftware	96

6.6.3.23.1	Description	96

6.6.3.23.2	Pre-Conditions	96

6.6.3.23.3	Signature – removeSoftware	97

6.6.3.23.4	Service Interactions	97

6.6.3.23.5	Post-Conditions	97

6.6.3.23.6	Exceptions	97

6.6.3.23.7	Policies for Use	97

6.6.3.23.8	oneM2M Resource Interworking	97

6.7	Management Adapter	98

6.7.1	Overview	98

6.7.2	Service Capabilities	98

6.7.2.1	downloadFirmware	98

6.7.2.1.1	Description	98

6.7.2.1.2	Pre-Conditions	98

6.7.2.1.3	Signature - downloadFirmware	98

6.7.2.1.4	Post-Conditions	98

6.7.2.1.5	Exceptions	98

6.7.2.1.6	Policies for Use	98

6.7.2.1.7	oneM2M Resource Interworking	98

6.7.2.2	installFirmware	98

6.7.2.2.1	Description	98

6.7.2.2.2	Pre-Conditions	99

6.7.2.2.3	Signature –installFirmware	99

6.7.2.2.4	Post-Conditions	99

6.7.2.2.5	Exceptions	99

6.7.2.2.6	Policies for Use	99

6.7.2.2.7	oneM2M Resource Interworking	99

6.7.2.3	getFirmwareInformation	99

6.7.2.3.1	Description	99

6.7.2.3.2	Pre-Conditions	99

6.7.2.3.3	Signature –getFirmwareInformation	100

6.7.2.3.4	Post-Conditions	100

6.7.2.3.5	Exceptions	100

6.7.2.3.6	Policies for Use	100

6.7.2.3.7	oneM2M Resource Interworking	100

6.7.2.4	getFirmwareExecStatus	100

6.7.2.4.1	Pre-Conditions	100

6.7.2.4.2	Pre-Conditions	100

6.7.2.4.3	Signature –getFirmwareExecInstance	100

6.7.2.4.4	Post-Conditions	100

6.7.2.4.5	Exceptions	101

6.7.2.4.6	Policies for Use	101

6.7.2.4.7	oneM2M Resource Interworking	101

6.7.2.5	deviceManagementReport	101

6.7.2.5.1	Description	101

6.7.2.5.2	Pre-Conditions	101

6.7.2.5.3	Signature –deviceManagementReport	101

6.7.2.5.4	Service Interactions	101

6.7.2.5.5	Post-Conditions	102

6.7.2.5.6	Exceptions	102

6.7.2.5.7	Policies for Use	102

6.7.2.5.7	oneM2M Resource Interworking	102

6.7.2.6	getDeviceInformation	102

6.7.2.6.1	Description	102

6.7.2.6.2	Pre-Conditions	102

6.7.2.6.3	Signature –getDeviceInformation	102

6.7.2.6.4	Post-Conditions	103

6.7.2.6.5	Exceptions	103

6.7.2.6.6	Policies for Use	103

6.7.2.6.7	oneM2M Resource Interworking	103

6.7.2.7	getDeviceCapabilities	103

6.7.2.7.1	Description	103

6.7.2.7.2	Pre-Conditions	103

6.7.2.7.3	Signature –getDeviceCapabilities	103

6.7.2.7.4	Post-Conditions	103

6.7.2.7.5	Exceptions	103

6.7.2.7.6	Policies for Use	103

6.7.2.7.7	oneM2M Resource Interworking	103

6.7.2.8	enableDeviceCapability	104

6.7.2.8.1	Description	104

6.7.2.8.2	Pre-Conditions	104

6.7.2.8.3	Signature –enableDeviceCapability	104

6.7.2.8.4	Post-Conditions	104

6.7.2.8.5	Exceptions	104

6.7.2.8.6	Policies for Use	104

6.7.2.8.7	oneM2M Resource Interworking	104

6.7.2.9	disableDeviceCapability	104

6.7.2.9.1	Description	104

6.7.2.9.2	Pre-Conditions	104

6.7.2.9.3	Signature –disableDeviceCapability	105

6.7.2.9.4	Post-Conditions	105

6.7.2.9.5	Exceptions	105

6.7.2.9.6	Policies for Use	105

6.7.2.9.7	oneM2M Resource Interworking	105

6.7.2.10	getAreaNetworks	105

6.7.2.10.1	Description	105

6.7.2.10.2	Pre-Conditions	105

6.7.2.10.3	Signature –getAreaNetworks	105

6.7.2.10.4	Post-Conditions	105

6.7.2.10.5	Exceptions	105

6.7.2.10.6	Policies for Use	105

6.7.2.10.7	oneM2M Resource Interworking	106

6.7.2.11	updateDeviceForAreaNetwork	106

6.7.2.11.1	Description	106

6.7.2.11.2	Pre-Conditions	106

6.7.2.11.3	Signature –updateDeviceForAreaNetwork	106

6.7.2.11.4	Post-Conditions	106

6.7.2.11.5	Exceptions	106

6.7.2.11.6	Policies for Use	106

6.7.2.11.7	oneM2M Resource Interworking	106

6.7.2.12	rebootDevice	106

6.7.2.12.1	Description	106

6.7.2.12.2	Pre-Conditions	106

6.7.2.12.3	Signature – rebootDevice	107

6.7.2.12.4	Post-Conditions	107

6.7.2.12.5	Exceptions	107

6.7.2.12.6	Policies for Use	107

6.7.2.12.7	oneM2M Resource Interworking	107

6.7.2.13	resetDevice	107

6.7.2.13.1	Description	107

6.7.2.13.2	Pre-Conditions	107

6.7.2.13.3	Signature – resetDevice	107

6.7.2.13.4	Post-Conditions	107

6.7.2.13.5	Exceptions	108

6.7.2.13.6	Policies for Use	108

6.7.2.13.7	oneM2M Resource Interworking	108

6.7.2.14	uploadDeviceLog	108

6.7.2.14.1	Description	108

6.7.2.14.2	Pre-Conditions	108

6.7.2.14.3	Signature – uploadDeviceLog	108

6.7.2.14.4	Post-Conditions	108

6.7.2.14.5	Exceptions	108

6.7.2.14.6	Policies for Use	108

6.7.2.14.7	oneM2M Resource Interworking	108

6.7.2.15	getDeviceLogs	108

6.7.2.15.1	Description	108

6.7.2.15.2	Pre-Conditions	109

6.7.2.15.3	Signature –getDeviceLogs	109

6.7.2.15.4	Post-Conditions	109

6.7.2.15.5	Exceptions	109

6.7.2.15.6	Policies for Use	109

6.7.2.15.7	oneM2M Resource Interworking	109

6.7.2.16	getDeviceLogInformation	109

6.7.2.16.1	Description	109

6.7.2.16.2	Pre-Conditions	109

6.7.2.16.3	Signature – getDeviceLogInformation	109

6.7.2.16.4	Post-Conditions	109

6.7.2.16.5	Exceptions	110

6.7.2.16.6	Policies for Use	110

6.7.2.16.7	oneM2M Resource Interworking	110

6.7.2.17	getSoftwareInformation	110

6.7.2.17.1	Description	110

6.7.2.17.2	Pre-Conditions	110

6.7.2.17.3	Signature –getSoftwareInformation	110

6.7.2.17.4	Post-Conditions	110

6.7.2.17.5	Exceptions	110

6.7.2.17.6	Policies for Use	110

6.7.2.17.7	oneM2M Resource Interworking	110

6.7.2.18	downloadSoftware	111

6.7.2.18.1	Description	111

6.7.2.18.2	Pre-Conditions	111

6.7.2.18.3	Signature –downloadSoftware	111

6.7.2.18.4	Post-Conditions	111

6.7.2.18.5	Exceptions	111

6.7.2.18.6	Policies for Use	111

6.7.2.18.7	oneM2M Resource Interworking	111

6.7.2.19	installSoftware	111

6.7.2.19.1	Description	111

6.7.2.19.2	Pre-Conditions	111

6.7.2.19.3	Signature –installSoftware	112

6.7.2.19.4	Post-Conditions	112

6.7.2.19.5	Exceptions	112

6.7.2.19.6	Policies for Use	112

6.7.2.19.7	oneM2M Resource Interworking	112

6.7.2.20	activateSoftware	112

6.7.2.20.1	Description	112

6.7.2.20.2	Pre-Conditions	112

6.7.2.20.3	Signature – activateSoftware	112

6.7.2.20.4	Post-Conditions	112

6.7.2.20.5	Exceptions	113

6.7.2.20.6	Policies for Use	113

6.7.2.20.7	oneM2M Resource Interworking	113

6.7.2.21	deactivateSoftware	113

6.7.2.21.1	Description	113

6.7.2.21.2	Pre-Conditions	113

6.7.2.21.3	Signature –deactivateSoftware	113

6.7.2.21.4	Post-Conditions	113

6.7.2.21.5	Exceptions	113

6.7.2.21.6	Policies for Use	113

6.7.2.21.7	oneM2M Resource Interworking	113

6.7.2.22	removeSoftware	113

6.7.2.22.1	Description	113

6.7.2.22.2	Pre-Conditions	114

6.7.2.22.3	Signature –removeSoftware	114

6.7.2.22.4	Post-Conditions	114

6.7.2.22.5	Exceptions	114

6.7.2.22.6	Policies for Use	114

6.7.2.22.7	oneM2M Resource Interworking	114

6.8	Service Administration	114

6.8.1	Overview	114

6.8.2	Service Administration Entities	114

6.8.2.1	Overview	114

6.8.2.2	M2M Service Entity	115

6.8.2.3	M2M Service Filter Criteria	115

6.8.3	Service Capabilities	115

6.8.3.1	createM2MService	115

6.8.3.1.1	Description	115

6.8.3.1.2	Pre-Conditions	116

6.8.3.1.3	Signature – createM2MService	116

6.8.3.1.4	Service Interactions	116

6.8.3.1.5	Post-Conditions	116

6.8.3.1.6	Exceptions	116

6.8.3.1.7	Policies for Use	116

6.8.3.1.8	oneM2M Resource Interworking	117

6.8.3.2	deleteM2MService	117

6.8.3.2.1	Description	117

6.8.3.2.2	Pre-Conditions	117

6.8.3.2.3	Signature – deleteM2MService	117

6.8.3.2.4	Service Interactions	117

6.8.3.2.5	Post-Conditions	118

6.8.3.2.6	Exceptions	118

6.8.3.2.7	Policies for Use	118

6.8.3.2.8	oneM2M Resource Interworking	118

6.8.3.3	addRoleToM2MService	118

6.8.3.3.1	Description	118

6.8.3.3.2	Pre-Conditions	118

6.8.3.3.3	Signature – addRoleToM2MService	118

6.8.3.3.4	Service Interactions	118

6.8.3.3.5	Post-Conditions	119

6.8.3.3.6	Exceptions	119

6.8.3.3.7	Policies for Use	119

6.8.3.3.8	oneM2M Resource Interworking	119

6.8.3.4	deleteRoleFromM2MService	119

6.8.3.4.1	Description	119

6.8.3.4.2	Pre-Conditions	119

6.8.3.4.3	Signature – deleteRoleFromM2MService	119

6.8.3.4.4	Service Interactions	119

6.8.3.4.5	Post-Conditions	120

6.8.3.4.6	Exceptions	120

6.8.3.4.7	Policies for Use	120

6.8.3.4.8	oneM2M Resource Interworking	120

6.8.3.5	getM2MService	120

6.8.3.5.1	Description	120

6.8.3.5.2	Pre-Conditions	120

6.8.3.5.3	Signature – getM2MService	120

6.8.3.5.4	Service Interactions	120

6.8.3.5.5	Post-Conditions	121

6.8.3.5.6	Exceptions	121

6.8.3.5.7	Policies for Use	121

6.8.3.5.8	oneM2M Resource Interworking	121

6.8.3.6	addServiceCapabilityToRole	121

6.8.3.6.1	Description	121

6.8.3.6.2	Pre-Conditions	121

6.8.3.6.3	Signature – addServiceCapabilityToRole	121

6.8.3.6.4	Service Interactions	121

6.8.3.6.5	Post-Conditions	122

6.8.3.6.6	Exceptions	122

6.8.3.6.7	Policies for Use	122

6.8.3.6.8	oneM2M Resource Interworking	122

6.8.3.7	deleteServiceCapabilityFromRole	122

6.8.3.7.1	Description	122

6.8.3.7.2	Pre-Conditions	122

6.8.3.7.3	Signature – deleteServiceCapabilityFromRole	122

6.8.3.7.4	Service Interactions	122

6.8.3.7.5	Post-Conditions	123

6.8.3.7.6	Exceptions	123

6.8.3.7.7	Policies for Use	123

6.8.3.7.8	oneM2M Resource Interworking	123

6.8.3.8	getServiceCapability	123

6.8.3.8.1	Description	123

6.8.3.8.2	Pre-Conditions	123

6.8.3.8.3	Signature – getServiceCapability	123

6.8.3.8.4	Service Interactions	124

6.8.3.8.5	Post-Conditions	124

6.8.3.8.6	Exceptions	124

6.8.3.8.7	Policies for Use	124

6.8.3.8.8	oneM2M Resource Interworking	124

6.9	Service Subscription Administration	124

6.9.1	Overview	124

6.9.2	Service Subscription Entities	125

6.9.3.1	Service Subscription Entity	125

6.9.2.2	Service Subscription Filter Criteria	125

6.9.2.3	M2M Node Entity for Service Subscription	125

6.9.2.4	M2M Node Filter Criteria	125

6.9.2.5	Application Rule	126

6.9.2.6	Application Rule Filter Criteria	126

6.9.2.7	Supporting Rules	126

6.9.2.7.1	Schedule Type	126

6.9.3	Service Capabilities	127

6.9.3.1	createServiceSubscription	127

6.9.3.1.1	Description	127

6.9.3.1.2	Pre-Conditions	127

6.9.3.1.3	Signature – createServiceSubscription	127

6.9.3.1.4	Service Interactions	127

6.9.3.1.5	Post-Conditions	128

6.9.3.1.6	Exceptions	128

6.9.3.1.7	Policies for Use	128

6.9.3.1.8	oneM2M Resource Interworking	128

6.9.3.2	deleteServiceSubscription	128

6.9.3.2.1	Description	128

6.9.3.2.2	Pre-Conditions	128

6.9.3.2.3	Signature – deleteServiceSubscription	128

6.9.3.2.4	Service Interactions	128

6.9.3.2.5	Post-Conditions	129

6.9.3.2.6	Exceptions	129

6.9.3.2.7	Policies for Use	129

6.9.3.2.8	oneM2M Resource Interworking	129

6.9.3.3	updateServiceSubscription	129

6.9.3.3.1	Description	129

6.9.3.3.2	Pre-Conditions	130

6.9.3.3.3	Signature – updateServiceSubscription	130

6.9.3.3.4	Service Interactions	130

6.9.3.3.5	Post-Conditions	130

6.9.3.3.6	Exceptions	130

6.9.3.3.7	Policies for Use	130

6.9.3.3.8	oneM2M Resource Interworking	131

6.9.3.4	addRoleToServiceSubscription	131

6.9.3.4.1	Description	131

6.9.3.4.2	Pre-Conditions	131

6.9.3.4.3	Signature – addRoleToServiceSubscription	131

6.9.3.4.4	Service Interactions	131

6.9.3.4.5	Post-Conditions	132

6.9.3.4.6	Exceptions	132

6.9.3.4.7	Policies for Use	132

6.9.3.4.8	oneM2M Resource Interworking	132

6.9.3.5	deleteRoleFromServiceSubscription	132

6.9.3.5.1	Description	132

6.9.3.5.2	Pre-Conditions	132

6.9.3.5.3	Signature – deleteRoleFromServiceSubscription	132

6.9.3.5.4	Service Interactions	132

6.9.3.5.5	Post-Conditions	133

6.9.3.5.6	Exceptions	133

6.9.3.5.7	Policies for Use	133

6.9.3.5.8	oneM2M Resource Interworking	133

6.9.3.6	getServiceSubscription	133

6.9.3.6.1	Description	133

6.9.3.6.2	Pre-Conditions	133

6.9.3.6.3	Signature – getServiceSubscription	133

6.9.3.6.4	Service Interactions	133

6.9.3.6.5	Post-Conditions	134

6.9.3.6.6	Exceptions	134

6.9.3.6.7	Policies for Use	134

6.9.3.6.8	oneM2M Resource Interworking	134

6.9.3.7	addNodeToServiceSubscription	134

6.9.3.7.1	Description	134

6.9.3.7.2	Pre-Conditions	134

6.9.3.7.3	Signature – addNodeToServiceSubscription	134

6.9.3.7.4	Service Interactions	134

6.9.3.7.5	Post-Conditions	135

6.9.3.7.6	Exceptions	135

6.9.3.7.7	Policies for Use	135

6.9.3.7.8	oneM2M Resource Interworking	135

6.9.3.8	deleteNodesFromServiceSubscription	135

6.9.3.8.1	Description	135

6.9.3.8.2	Pre-Conditions	135

6.9.3.8.3	Signature – deleteNodesFromServiceSubscription	135

6.9.3.8.4	Service Interactions	136

6.9.3.8.5	Post-Conditions	136

6.9.3.8.6	Exceptions	136

6.9.3.8.7	Policies for Use	136

6.9.3.8.8	oneM2M Resource Interworking	136

6.9.3.9	getNodesForServiceSubscription	136

6.9.3.9.1	Description	136

6.9.3.9.2	Pre-Conditions	136

6.9.3.9.3	Signature – getNodesForServiceSubscription	136

6.9.3.9.4	Service Interactions	137

6.9.3.9.5	Post-Conditions	137

6.9.3.9.6	Exceptions	137

6.9.3.9.7	Policies for Use	137

6.9.3.9.8	oneM2M Resource Interworking	137

6.9.3.10	createApplicationRule	137

6.9.3.10.1	Description	137

6.9.3.10.2	Pre-Conditions	138

6.9.3.10.3	Signature – createApplicationRule	138

6.9.3.10.4	Service Interactions	138

6.9.3.10.5	Post-Conditions	138

6.9.3.10.6	Exceptions	138

6.9.3.10.7	Policies for Use	138

6.9.3.10.8	oneM2M Resource Interworking	139

6.9.3.11	deleteApplicationRules	139

6.9.3.11.1	Description	139

6.9.3.11.2	Pre-Conditions	139

6.9.3.11.3	Signature - deleteApplicationRules	139

6.9.3.11.4	Service Interactions	139

6.9.3.11.5	Post-Conditions	139

6.9.3.11.6	Exceptions	139

6.9.3.11.7	Policies for Use	139

6.9.3.11.8	oneM2M Resource Interworking	140

6.9.3.12	getApplicationRules	140

6.9.3.12.1	Description	140

6.9.3.12.2	Pre-Conditions	140

6.9.3.12.3	Signature – getApplicationRules	140

6.9.3.12.4	Service Interactions	140

6.9.3.12.5	Post-Conditions	140

6.9.3.12.6	Exceptions	140

6.9.3.12.7	Policies for Use	140

6.9.3.12.8	oneM2M Resource Interworking	141

6.9.3.13	updateNodeForServiceSubscription	141

6.9.3.13.1	Description	141

6.9.3.13.2	Pre-Conditions	141

6.9.3.13.3	updateNodeForServiceSubscription	141

6.9.3.13.4	Service Interactions	141

6.9.3.13.5	Post-Conditions	142

6.9.3.13.6	Exceptions	142

6.9.3.13.7	Policies for Use	142

6.9.3.13.8	oneM2M Resource Interworking	142

6.9.3.14	updateApplicationRule	142

6.9.3.14.1	Description	142

6.9.3.14.2	Pre-Conditions	142

6.9.3.14.3	Signature – updateApplicationRule	142

6.9.3.14.4	Service Interactions	142

6.9.3.14.5	Post-Conditions	143

6.9.3.14.6	Exceptions	143

6.9.3.14.7	Policies for Use	143

6.9.3.14.8	oneM2M Resource Interworking	143

6.9.3.15	addReachabilitySchedule	143

6.9.3.15.1	Description	143

6.9.3.15.2	Pre-Conditions	143

6.9.3.15.3	Signature – addReachabilitySchedule	143

6.9.3.15.4	Service Interactions	143

6.9.3.15.5	Post-Conditions	144

6.9.3.15.6	Exceptions	144

6.9.3.15.7	Policies for Use	144

6.9.3.15.8	oneM2M Resource Interworking	144

6.9.3.16	updateReachabilitySchedule	144

6.9.3.16.1	Description	144

6.9.3.16.2	Pre-Conditions	144

6.9.3.16.3	Signature –updateReachabilitySchedule	144

6.9.3.16.4	Service Interactions	145

6.9.3.16.5	Post-Conditions	145

6.9.3.16.6	Exceptions	145

6.9.3.16.7	Policies for Use	145

6.9.3.16.8	oneM2M Resource Interworking	145

6.9.3.17	getReachabilitySchedule	145

6.9.3.17.1	Description	145

6.9.3.17.2	Pre-Conditions	146

6.9.3.17.3	Signature – getReachabilitySchedule	146

6.9.3.17.4	Service Interactions	146

6.9.3.17.5	Post-Conditions	146

6.9.3.17.6	Exceptions	146

6.9.3.17.7	Policies for Use	146

6.9.3.17.8	oneM2M Resource Interworking	146

6.9.3.18	deleteReachabilitySchedule	147

6.9.3.18.1	Description	147

6.9.3.18.2	Pre-Conditions	147

6.9.3.18.3	Signature – deleteReachabilitySchedule	147

6.9.3.18.4	Service Interactions	147

6.9.3.18.5	Post-Conditions	147

6.9.3.18.6	Exceptions	147

6.9.3.18.7	Policies for Use	147

6.9.3.18.8	oneM2M Resource Interworking	147

6.10	Event Collection	148

6.10.1	Overview	148

6.10.2	Event Collection Entities	148

6.10.2.1	Event Collection Types	148

6.10.2.1.1	Event Collection Record	148

6.10.2.1.2	Event Configuration Type	148

6.10.2.1.3	getEventCollectionPolicy Filter Criteria	148

6.10.2.1.4 	getEventCollectionTriggers Filter Criteria	149

6.10.2.1.5 	getEventRecords Filter Criteria	149

6.10.3	Service Capabilities	149

6.10.3.1	setEventCollectionPolicy	149

6.10.3.1.1	Description	149

6.10.3.1.2	Pre-Conditions	149

6.10.3.1.3	Signature – setEventCollectionPolicy	149

6.10.3.1.4	Post-Conditions	150

6.10.3.1.5	Exceptions	150

6.10.3.1.6	Policies for Use	150

6.10.3.1.7	Service Interactions	150

6.10.3.1.8	oneM2M Resource Interworking	150

6.10.3.2	getEventCollectionPolicy	150

6.10.3.2.1	Description	150

6.10.3.2.2	Pre-Conditions	151

6.10.3.2.3	Signature – getEventCollectionPolicy	151

6.10.3.2.4	Post-Conditions	151

6.10.3.2.5	Exceptions	151

6.10.3.2.6	Policies for Use	151

6.10.3.2.7	Service Interactions	151

6.10.3.2.8	oneM2M Resource Interworking	152

6.10.3.3	setEventCollectionTriggers	152

6.10.3.3.1	Description	152

6.10.3.3.2	Pre-Conditions	153

6.10.3.3.3	Signature - setEventCollectionTriggers	153

6.10.3.3.4	Post-Conditions	153

6.10.3.3.5	Exceptions	153

6.10.3.3.6	Policies for Use	153

6.10.3.3.7	Service Interactions	153

6.10.3.3.8	oneM2M Resource Interworking	154

6.10.3.4	getEventCollectionTriggers	154

6.10.3.4.1	Description	154

6.10.3.4.2	Pre-Conditions	154

6.10.3.4.3	Signature – getEventCollectionTriggers	154

6.10.3.4.4	Post-Conditions	154

6.10.3.4.5	Exceptions	154

6.10.3.4.6	Policies for Use	155

6.10.3.4.7	Service Interactions	155

6.10.3.4.8	oneM2M Resource Interworking	155

6.10.3.5	recordEvent	155

6.10.3.5.1	Description	155

6.10.3.5.2	Pre-Conditions	156

6.10.3.5.3	Signature - recordEvent	156

6.10.3.5.4	Post-Conditions	156

6.10.3.5.5	Exceptions	156

6.10.3.5.6	Policies for Use	156

6.10.3.5.7	Service Interactions	156

6.10.3.5.8	oneM2M Resource Interworking	157

6.10.3.6	getEventRecords	157

6.10.3.6.1	Description	157

6.10.3.6.2	Pre-Conditions	157

6.10.3.6.3	Signature – getEventRecords	157

6.10.3.6.4	Post-Conditions	157

6.10.3.6.5	Exceptions	157

6.10.3.6.6	Policies for Use	157

6.10.3.6.7	Service Interactions	157

6.10.3.6.8	oneM2M Resource Interworking	158

6.11	Registration	158

6.11.1	Overview	158

6.11.2	Service Capabilities	159

6.11.2.1	registerAE	159

6.11.2.1.1	Description	159

6.11.2.1.2	Pre-Conditions	159

6.11.2.1.3	Signature – registerAE	159

6.11.2.1.4	Services Interaction	159

6.11.2.1.5	Post-Conditions	160

6.11.2.1.6	Exceptions	160

6.11.2.1.7	Policies for Use	160

6.11.2.1.8	oneM2M Resource Interworking	160

6.11.2.2	refreshAERegistration	161

6.11.2.2.1	Description	161

6.11.2.2.2	Pre-Conditions	161

6.11.2.2.3	Signature – refreshAERegistration	161

6.11.2.2.4	Service Interactions	161

6.11.2.2.5	Post-Conditions	162

6.11.2.2.6	Exceptions	162

6.11.2.2.7	Policies for Use	162

6.11.2.2.8	oneM2M Resource Interworking	162

6.11.2.3	deregisterAE	162

6.11.2.3.1	Description	162

6.11.2.3.2	Pre-Conditions	162

6.11.2.3.3	Signature - deregisterAE	162

6.11.2.3.4	Service Interactions	162

6.11.2.3.5	Post-Conditions	163

6.11.2.3.6	Exceptions	163

6.11.2.3.7	oneM2M Resource Interworking	163

6.12	Registration Administration	163

6.12.1	Overview	163

6.12.2	Service Capabilities	164

6.12.2.1	getRegistrationStatus	164

6.12.2.1.1	Description	164

6.12.2.1.2	Pre-Conditions	164

6.12.2.1.2	Signature - getRegistrationStatus	164

6.12.2.1.4	Service Interaction	164

6.12.2.1.5	Post-Conditions	164

6.12.2.1.6	Exceptions	164

6.12.2.1.7	Policies for Use	164

6.12.2.1.8	oneM2M Resource Interworking	165

6.12.2.2	revokeAERegistration	165

6.12.2.2.1	Description	165

6.12.2.2.2	Pre-Conditions	165

6.12.2.2.3	Signature - revokeAERegistration	165

6.12.2.2.4	Service Interaction	165

6.12.2.2.5	Post-Conditions	166

6.12.2.2.6	Exceptions	166

6.12.2.2.7	Policies for Use	166

6.12.2.2.8	oneM2M Resource Interworking	166

6.12.2.3	subscribeInitialAERegistrationEvent	166

6.12.2.3.1	Description	166

6.12.2.3.2	Pre-Conditions	166

6.12.2.3.3	Signature - subscribeInitialAERegistrationEvent	166

6.12.2.3.4	Service Interaction	166

6.12.2.3.5	Post-Conditions	167

6.12.2.3.6	Exceptions	167

6.12.2.3.7	Policies for Use	167

6.12.2.3.8	oneM2M Resource Interworking	167

6.12.2.4	unsubscribeInitialAERegistrationEvent	167

6.12.2.4.1	Description	167

6.12.2.4.2	Pre-Conditions	167

6.12.2.4.3	Signature – unsubcribeInitialAERegistrationEvent	167

6.12.2.4.4	Service Interaction	168

6.12.2.4.5	Post-Conditions	168

6.12.2.4.6	Exceptions	168

6.12.2.4.7	Policies for Use	168

6.12.2.4.8	oneM2M Resource Interworking	168

7	M2M Service Components	169

7.1	Introduction	169

7.2	Service Component Interaction Cross Reference	169

7.3	Infrastructure Component (INF)	169

7.3.1	introduction	169

7.3.2	INF to Service Cross Reference	169

7.4	Service Subscription Component (SSUB)	169

7.4.1	Introduction	169

7.4.2	SSUB to Service Cross Reference	170

7.5	Transport Adapter (TRA)	170

7.5.1	Introduction	170

7.5.2	TRA to Service Cross Reference	170

7.6	Accounting (ACC)	170

7.6.1	Introduction	170

7.6.2	Accounting to Service Cross Reference	170

7.7	Service Exposure (SE)	170

7.7.1	Introduction	170

7.7.2	Service Exposure to Service Cross Reference	170

7.8	Management Adapter (MA)	171

7.8.1	Introduction	171

7.8.2	Management Adapter to Service Cross Reference	171

7.9	Device Management (DM)	171

7.9.1	Introduction	171

7.9.2	Device Management to Service Cross Reference	171

Annex A (informative): Common Request Processing	172

A.1	Overview	172

A.2	Mca Common Request Processing	172

A.2.1	Introduction	172

A.2.2	Mca Common Request Parameters (normative)	172

A.2.3	Authentication and Authorization of Requests	172

A.2.3.1	Description	172

A.2.3.2	Pre-Conditions	173

A.2.3.2	Common M2M Service Capability Parameters for Request Authentication and Authorization	173

A.2.3.3	Service Interactions	173

A.2.3.4	Post-Conditions	174

A.2.3.5	Exceptions	174

A.2.3.6	Policies for Use	174

A.3	Msc Common Request Processing	174

A.3.1	Introduction	174

A.3.2	Msc Common Request Data Types (normative)	174

A.4	Common Entity Attributes	175

A.4.1	Introduction	175

A.4.2	System Generated Attributes (normative)	175

A.4.3	User Defined Discovery Attributes (normative)	175

A.5	Using Filter Criteria	175

Annex B (informative): Data Exchange Services	176

B.1	Overview	176

B.2	Supporting Services	176

B.2.1	Subscribe-Publish-Notify Message Exchange	176

B.2.1.1	Overview	176

B.2.1.2	Service Capabilities	176

B.2.1.2.1	subscribeRequest	176

B.2.1.2.1.1	Description	176

B.2.1.2.1.2	Pre-Conditions	176

B.2.1.2.1.3	Signature – subscribeRequest	176

B.2.1.2.1.4	Service Interactions	176

B.2.1.2.1.5	Post-Conditions	177

B.2.1.2.1.6	Exceptions	177

B.2.1.2.1.7	Policies for Use	177

B.2.1.2.2	subscribeComplete	177

B.2.1.2.2.1	Description	177

B.2.1.2.2.2	Pre-Conditions	178

B.2.1.2.2.3	Signature – subscribeComplete	178

B.2.1.2.2.4	Service Interactions	178

B.2.1.2.2.5	Post-Conditions	178

B.2.1.2.2.6	Exceptions	178

B.2.1.2.2.7	Policies for Use	178

B.2.1.2.3	publishRequest	178

B.2.1.2.3.1	Description	178

B.2.1.2.3.2	Pre-Conditions	179

B.2.1.2.3.3	Signature – publishRequest	179

B.2.1.2.3.4	Service Interactions	179

B.2.1.2.3.5	Post-Conditions	180

B.2.1.2.3.6	Exceptions	180

B.2.1.2.3.7	Policies for Use	180

B.2.1.2.4	publishComplete	180

B.2.1.2.4.1	Description	180

B.2.1.2.4.2	Pre-Conditions	181

B.2.1.2.4.3	Signature – publishComplete	181

B.2.1.2.4.4	Service Interactions	181

B.2.1.2.4.5	Post-Conditions	181

B.2.1.2.4.6	Exceptions	181

B.2.1.2.4.7	Policies for Use	181

B.2.1.2.5	notifyRequest	181

B.2.1.2.5.1	Description	181

B.2.1.2.5.2	Pre-Conditions	182

B.2.1.2.5.3	Signature – notifyRequest	182

B.2.1.2.5.4	Service Interactions	182

B.2.1.2.5.5	Post-Conditions	183

B.2.1.2.5.6	Exceptions	183

B.2.1.2.5.7	Policies for Use	183

B.2.1.2.6	notifyComplete	183

B.2.1.2.6.1	Description	183

B.2.1.2.6.2	Pre-Conditions	183

B.2.1.2.6.3	Signature - notifyComplete	183

B.2.1.2.6.4	Service Interactions	183

B.2.1.2.6.5	Post-Conditions	183

B.2.1.2.6.6	Exceptions	183

B.2.1.2.6.7	Policies for Use	183

B.2.2	Request-Response Message Exchange	184

B.2.2.1	Overview	184

B.2.2.2	Service Capabilities	184

B.2.2.2.1	sendMessageRequest	184

B.2.2.2.1.1	Description	184

B.2.2.2.1.2	Pre-Conditions	184

B.2.2.2.1.3	Signature – sendMessageRequest	184

B.2.2.2.1.4	Service Interactions	184

B.2.2.2.1.5	Post-Conditions	185

B.2.2.2.1.6	Exceptions	185

B.2.2.2.1.7	Policies for Use	185

B.2.2.2.2	sendMessageComplete	185

B.2.2.2.2.1	Description	185

B.2.2.2.2.2	Pre-Conditions	186

B.2.2.2.2.3	Signature - sendMessageComplete	186

B.2.2.2.2.4	Service Interactions	186

B.2.2.2.2.5	Post-Conditions	186

B.2.2.2.2.6	Exceptions	186

B.2.2.2.2.7	Policies for Use	186

Annex C (informative): Service Subscription Administration Services	187

C.1	Overview	187

C.2	Supporting Services	187

C.2.1	Overview	187

C.2.2	Service Capabilities	187

C.2.2.1	getServiceSubscription	187

C.2.2.1.1	Description	187

C.2.2.1.2	Pre-Conditions	187

C.2.2.1.3	Signature – getServiceSubscription	187

C.2.2.1.4	Service Interactions	187

C.2.2.1.5	Post-Conditions	188

C.2.2.1.6	Exceptions	188

C.2.2.1.7	Policies for Use	188

C.2.2.2	addNodeToServiceSubscription	188

C.2.2.2.1	Description	188

C.2.2.2.2	Pre-Conditions	189

C.2.2.2.3	Signature – addNodeToServiceSubscription	189

C.2.2.2.4	Service Interactions	189

C.2.2.2.5	Post-Conditions	190

C.2.2.2.6	Exceptions	190

C.2.2.2.7	Policies for Use	190

C.2.2.3	deleteNodesFromServiceSubscription	190

C.2.2.3.1	Description	190

C.2.2.3.2	Pre-Conditions	190

C.2.2.3.3	Signature – deleteNodesFromServiceSubscription	190

C.2.2.3.4	Service Interactions	190

C.2.2.3.5	Post-Conditions	191

C.2.2.3.6	Exceptions	191

C.2.2.3.7	Policies for Use	191

C.2.2.4	getNodesForServiceSubscription	191

C.2.2.4.1	Description	191

C.2.2.4.2	Pre-Conditions	192

C.2.2.4.3	Signature – getNodesForServiceSubscription	192

C.2.2.4.4	Service Interactions	192

C.2.2.4.5	Post-Conditions	193

C.2.2.4.6	Exceptions	193

C.2.2.4.7	Policies for Use	193

C.2.2.5	createApplicationRule	193

C.2.2.5.1	Description	193

C.2.2.5.2	Pre-Conditions	193

C.2.2.5.3	Signature – createApplicationRule	193

C.2.2.5.4	Service Interactions	193

C.2.2.5.5	Post-Conditions	194

C.2.2.5.6	Exceptions	194

C.2.2.5.7	Policies for Use	194

C.2.2.6	deleteApplicationRules	194

C.2.2.6.1	Description	194

C.2.2.6.2	Pre-Conditions	195

C.2.2.6.3	Signature – deleteApplicationRules	195

C.2.2.6.4	Service Interactions	195

C.2.2.6.5	Post-Conditions	196

C.2.2.6.6	Exceptions	196

C.2. 2.6.7	Policies for Use	196

C.2.2.7	getApplicationRules	196

C.2.2.7.1	Description	196

C.2.2.7.2	Pre-Conditions	196

C.2.2.7.3	Signature – getApplicationRules	196

C.2.2.7.4	Service Interactions	196

C.2.2.7.5	Post-Conditions	197

C.2.2.7.6	Exceptions	197

C.2.2.7.7	Policies for Use	197

C.2.2.8	updateApplicationRule	197

C.2.2.8.1	Description	197

C.2.2.8.2	Pre-Conditions	198

C.2.2.8.3	Signature – updateApplicationRule	198

C.2.2.8.4	Service Interactions	198

C.2.2.8.5	Post-Conditions	199

C.2.2.8.6	Exceptions	199

C.2.2.8.7	Policies for Use	199

C.2.2.9	updateNodeForServiceSubscription	199

C.2.2.9.1	Description	199

C.2.2.9.2	Pre-Conditions	200

C.2.2.9.3	Signature – updateNodeForServiceSubscription	200

C.2.2.9.4	Service Interactions	200

C.2.2.9.5	Post-Conditions	201

C.2.2.9.6	Exceptions	201

C.2.2.9.7	Policies for Use	201

Annex D (informative): Device Management Services	202

D.1	Overview	202

D.2	Supporting Services	202

D.2.1	Overview	202

D.2.2	Service Capabilities	202

D.2.2.1	downloadFirmware	202

D.2.2.1.1	Description	202

D.2.2.1.2	Pre-Conditions	202

D.2.2.1.3	Signature – downloadFirmware	202

D.2.2.1.4	Service Interactions	202

D.2.2.1.5	Post-Conditions	203

D.2.2.1.6	Exceptions	203

D.2.2.1.7	Policies for Use	203

D.2.2.2	installFirmware	203

D.2.2.2.1	Description	203

D.2.2.2.2	Pre-Conditions	203

D.2.2.2.3	Signature – installFirmware	204

D.2.2.2.4	Service Interactions	204

D.2.2.2.5	Post-Conditions	204

D.2.2.2.6	Exceptions	204

D.2.2.2.7	Policies for Use	204

D.2.2.3	getFirmwareInformation	205

D.2.2.3.1	Description	205

D.2.2.3.2	Pre-Conditions	205

D.2.2.3.3	Signature – getFirmwareInformation	205

D.2.2.3.4	Service Interactions	205

D.2.2.3.5	Post-Conditions	206

D.2.2.3.6	Exceptions	206

D.2.2.3.7	Policies for Use	206

D.2.2.4	getFirmwareExecStatus	206

D.2.2.4.1	Description	206

D.2.2.4.2	Pre-Conditions	206

D.2.2.4.3	Signature – getFirmwareExecStatus	206

D.2.2.4.4	Service Interactions	206

D.2.2.4.5	Post-Conditions	207

D.2.2.4.6	Exceptions	207

D.2.2.4.7	Policies for Use	207

D.2.2.5	deviceManagementReport	207

D.2.2.5.1	Description	207

D.2.2.5.2	Pre-Conditions	207

D.2.2.5.3	Signature –deviceManagementReport	208

D.2.2.5.4	Service Interactions	208

D.2.2.5.5	Post-Conditions	208

D.2.2.5.6	Exceptions	209

D.2.2.5.7	Policies for Use	209

D.2.2.6	upgradeFirmware	209

D.2.2.6.1	Description	209

D.2.2.6.2	Pre-Conditions	209

D.2.2.6.3	Signature – upgradeFirmware	209

D.2.2.6.4	Service Interactions	209

D.2.2.6.5	Post-Conditions	210

D.2.2.6.6	Exceptions	210

D.2.2.6.7	Policies for Use	210

D.2.2.7	getDeviceInformation	210

D.2.2.7.1	Description	210

D.2.2.7.2	Pre-Conditions	211

D.2.2.7.3	Signature – getDeviceInformation	211

D.2.2.7.4	Service Interactions	211

D.2.2.7.5	Post-Conditions	211

D.2.2.7.6	Exceptions	212

D.2.2.7.7	Policies for Use	212

D.2.2.8	getDeviceCapabilities	212

D.2.2.8.1	Description	212

D.2.2.8.2	Pre-Conditions	212

D.2.2.8.3	Signature – getDeviceCapabilities	212

D.2.2.8.4	Service Interactions	212

D.2.2.8.5	Post-Conditions	213

D.2.2.8.6	Exceptions	213

D.2.2.8.7	Policies for Use	213

D.2.2.9	enableDeviceCapability	213

D.2.2.9.1	Description	213

D.2.2.9.2	Pre-Conditions	213

D.2.2.9.3	Signature – enableDeviceCapability	214

D.2.2.9.4	Service Interactions	214

D.2.2.9.5	Post-Conditions	214

D.2.2.9.6	Exceptions	214

D.2.2.9.7	Policies for Use	214

D.2.2.10	disableDeviceCapability	215

D.2.2.10.1	Description	215

D.2.2.10.2	Pre-Conditions	215

D.2.2.10.3	Signature – disableDeviceCapability	215

D.2.2.10.4	Service Interactions	215

D.2.2.10.5	Post-Conditions	216

D.2.2.10.6	Exceptions	216

D.2.2.10.7	Policies for Use	216

D.2.2.11	getAreaNetworks	216

D.2.2.11.1	Description	216

D.2.2.11.2	Pre-Conditions	216

D.2.2.11.3	Signature – getAreaNetworks	216

D.2.2.11.4	Service Interactions	216

D.2.2.11.5	Post-Conditions	217

D.2.2.11.6	Exceptions	217

D.2.2.11.7	Policies for Use	217

D.2.2.12	updateDeviceForAreaNetwork	217

D.2.2.12.1	Description	217

D.2.2.12.2	Pre-Conditions	217

D.2.2.12.3	Signature –updateDeviceForAreaNetwork	218

D.2.2.12.4	Service Interactions	218

D.2.2.12.5	Post-Conditions	218

D.2.2.12.6	Exceptions	218

D.2.2.12.7	Policies for Use	219

D.2.2.13	rebootDevice	219

D.2.2.13.1	Description	219

D.2.2.13.2	Pre-Conditions	219

D.2.2.13.3	Signature – rebootDevice	219

D.2.2.13.4	Service Interactions	219

D.2.2.13.5	Post-Conditions	220

D.2.2.13.6	Exceptions	220

D.2.2.13.7	Policies for Use	220

D.2.2.14	resetDevice	220

D.2.2.14.1	Description	220

D.2.2.14.2	Pre-Conditions	220

D.2.2.14.3	Signature –resetDevice	220

D.2.2.14.4	Service Interactions	221

D.2.2.14.5	Post-Conditions	221

D.2.2.14.6	Exceptions	221

D.2.2.14.7	Policies for Use	221

D.2.2.15	uploadDeviceLog	221

D.2.2.15.1	Description	221

D.2.2.15.2	Pre-Conditions	221

D.2.2.15.3	Signature –uploadDeviceLog	222

D.2.2.15.4	Service Interactions	222

D.2.2.15.5	Post-Conditions	222

D.2.2.15.6	Exceptions	222

D.2.2.15.7	Policies for Use	222

D.2.2.16	getDeviceLogs	223

D.2.2.16.1	Description	223

D.2.2.16.2	Pre-Conditions	223

D.2.2.16.3	Signature – getDeviceLogs	223

D.2.2.16.4	Service Interactions	223

D.2.2.16.5	Post-Conditions	224

D.2.2.16.6	Exceptions	224

D.2.2.16.7	Policies for Use	224

D.2.2.17	getDeviceLogInformation	224

D.2.2.17.1	Description	224

D.2.2.17.2	Pre-Conditions	224

D.2.2.17.3	Signature – getDeviceLogInformation	224

D.2.2.17.4	Service Interactions	224

D.2.2.17.5	Post-Conditions	225

D.2.2.17.6	Exceptions	225

D.2.2.17.7	Policies for Use	225

D.2.2.18	getSoftwareInformation	225

D.2.2.18.1	Description	225

D.2.2.18.2	Pre-Conditions	225

D.2.2.18.3	Signature – getSoftwareInformation	226

D.2.2.18.4	Service Interactions	226

D.2.2.18.5	Post-Conditions	226

D.2.2.18.6	Exceptions	226

D.2.2.18.7	Policies for Use	227

D.2.2.19	downloadSoftware	227

D.2.2.19.1	Description	227

D.2.2.19.2	Pre-Conditions	227

D.2.2.19.3	Signature – downloadSoftware	227

D.2.2.19.4	Service Interactions	227

D.2.2.19.5	Post-Conditions	228

D.2.2.19.6	Exceptions	228

D.2.2.19.7	Policies for Use	228

D.2.2.20	installSoftware	228

D.2.2.20.1	Description	228

D.2.2.20.2	Pre-Conditions	228

D.2.2.20.3	Signature –installSoftware	229

D.2.2.20.4	Service Interactions	229

D.2.2.20.5	Post-Conditions	229

D.2.2.20.6	Exceptions	229

D.2.2.20.7	Policies for Use	229

D.2.2.21	activateSoftware	230

D.2.2.21.1	Description	230

D.2.2.21.2	Pre-Conditions	230

D.2.2.21.3	Signature – activateSoftware	230

D.2.2.21.4	Service Interactions	230

D.2.2.21.5	Post-Conditions	231

D.2.2.21.6	Exceptions	231

D.2.2.21.7	Policies for Use	231

D.2.2.22	deactivateSoftware	231

D.2.2.22.1	Description	231

D.2.2.22.2	Pre-Conditions	231

D.2.2.22.3	Signature – deactivateSoftware	231

D.2.2.22.4	Service Interactions	231

D.2.2.22.5	Post-Conditions	232

D.2.2.22.6	Exceptions	232

D.2.2.22.7	Policies for Use	232

D.2.2.23	removeSoftware	232

D.2.2.23.1	Description	232

D.2.2.23.2	Pre-Conditions	232

D.2.2.23.3	Signature – removeSoftware	233

D.2.2.23.4	Service Interactions	233

D.2.2.23.5	Post-Conditions	233

D.2.2.23.6	Exceptions	233

D.2.2.23.7	Policies for Use	233

Annex E (informative): Device On-boarding Service	235

E.1	Overview	235

E.2	Supporting Services	235

E.2.1	Remote Administration	235

E.2.1.1	Overview	235

E.2.1.2	Service Capabilities	235

E.2.1.1.1	notifyRegistrationContact	235

E.2.1.1.1.1	Description	235

E.2.1.1.1.2	Pre-Conditions	235

E.2.1.1.1.3	Signature – notifyRegistrationContact	235

E.2.1.1.1.4	Service Interactions	235

E.2.1.1.1.5	Post-Conditions	236

E.2.1.1.1.6	Exceptions	236

E.2.1.1.1.7	Policies for Use	236

Annex F (informative): Registration Services	237

F.1	Overview	237

F.2	Mca Registration Service Processing and Supporting Services	237

F.2.1	Overview	237

F.2.2	Service Capabilities	237

F.2.2.1	registerAE	237

F.2.2.1.1	Description	237

F.2.2.1.2	Pre-Conditions	237

F.2.2.1.3	Signature – registerAE	237

F.2.2.1.4	Service Interactions	237

F.2.2.1.4	Post-Conditions	238

F.2.2.1.5	Exceptions	238

F.2.2.1.6	Policies for Use	238

F.2.2.2	refreshAERegistration	238

F.2.2.2.1	Description	238

F.2.2.2.2	Pre-Conditions	239

F.2.2.2.3	Signature – refreshAERegsitration	239

F.2.2.2.4	Service Interactions	239

F.2.2.2.5	Post-Conditions	240

F.2.2.2.6	Exceptions	240

F.2.2.2.7	Policies for Use	240

F.2.2.3	deregisterAE	240

F.2.2.3.1	Description	240

F.2.2.3.2	Pre-Conditions	240

F.2.2.3.3	Signature – deregisterAE	240

F.2.2.3.4	Service Interactions	240

F.2.2.3.5	Post-Conditions	241

F.2.2.3.6	Exceptions	241

F.2.2.3.6	Policies for Use	241

Annex G (informative): M2M Service Capability Template	242

G.1	Introduction	242

G.2	<serviceCapabilityName>	242

G.2.1	Description	242

G.2.2	Pre-Conditions	242

G.2.3	Signature	242

G.2.4	Service Interactions	242

G.2.5	Post-Conditions	242

G.2.6	Exceptions	243

G.2.7	Policies for Use	243

G.2.8	oneM2M Resource Interworking	243

Annex H (informative): oneM2M Service Requirements	244

History	245

# 1	Scope

The present document describes the M2M Services provided by the oneM2M Services Platform, the integration and interworking of the M2M Services functional architecture of the oneM2M Services Platform and informatively illustrates the use of the M2M Services within the context of complex business services.

# 2	References

## 2.1	Normative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the reference document (including any amendments) applies.

The following referenced documents are necessary for the application of the present document.

[1]	oneM2M TS-0002: "oneM2M Requirements".

[2]	oneM2M TS-0001: "oneM2M Functional Architecture".

## 2.2	Informative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the reference document (including any amendments) applies.

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	oneM2M Drafting Rules.

NOTE:	Available at http://member.onem2m.org/Static_pages/Others/Rules_Pages/oneM2M-Drafting-Rules-V1_0.doc.

[i.2]	W3C WD-wsdl20-patterns-20040326:"Web Services Description Language (WSDL) Version 2.0 Part 2: Message Exchange Patterns".

[i.3]	OASIS soa-rm: "Reference Model for Service Oriented Architecture 1.0".

[i.4]	oneM2M TS-0003: "oneM2M Security Solutions".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions apply:

M2M Service: A Service that creates added-value to the providing M2M Service Platform and is consumed by one or more Supporting Services.

NOTE: 	The Data Exchange Service is an example of a M2M Service.

M2M Service Capability: Service Capability that is identified within the M2M Service Platform

M2M Service Registration: when a Service is deployed into a SOA environment; the SOA environment's Service Registry is updated so that the Service can be accessed by other Services

Proprietary Reference Point: Reference Point as defined in Functional Architecture [2] where the realization of the communication flows are implementation specific

Service: mechanism to enable access to one or more Service Capabilities, where the access is provided using a prescribed interface and is exercised consistent with constraints and policies as specified by the service description [i.3]

Service Capability: real-world effect that a service provider is able to provide to a service consumer [i.3]

Service Cluster: collection of distributed and related Services that are gathered to solve a business problem

Service Component: entity of the M2M Services Architecture which may contain one or more M2M Services

Service Execution Environment: Execution Environment is a logical entity that represents an environment capable of running Services

Service Exposure Component: Service Component that provides an enforcement point for specific oneM2M Reference Points (e.g. Mca, Mcn, Mcc')

Service Registry: component of an underlying SOA environment that maintains information about Services and provides facilities to publish and discover Services to entities that would utilize the Service

Supporting Service: A Service that creates added-value to an organization and is relevant to the business process of the organization that consumes Services from the M2M Services Platform.

NOTE: 	Supporting Services are typically complex, orchestrated Services which consume M2M Services among other Services. A Device Onboarding Service is an example of a Supporting Service.

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

ACC			Accounting

AE			Application Entity

AE-ID			Application Entity Identifier

AMQP			Advanced Message Queuing Protocol

ASP			Application Service Provider

BSS			Business Support System

CDMA			Code Division Mupltiple Access

CSE			Common Service Entity

DM			Device Management

EDGE			Enhanced Data Rates for Global Evolution

ESB			Enterprise Service Bus

GPS			Global Positioning System

GNSS			Global Navigation Satellite System

INF			Infrastructure

JMS			Java Message Service

LAN			Local Area Network

MA			Management Agent

M2M			Machine-to-Machine

M2M-Serv-Cap-ID	M2M Service Capability Identifier

MEP			Message Exchange Pattern

MQTT			Message Queuing Telemetry Transport

NFC			Near Field Communication

NMS			Network Management System

NSE			Network Service Exposure

OSS			Operational Support System

POA			Point of Attachment

SEE			Service Execution Environment

SE			Service Exposure

SEA			Security Association Endpoint

SOA			Service Oriented Architecture

SP			Service Provider

SSUB			Service Subscription

TRA		    	Transport Adapter

UPA			Universal Powerline Association

URL			Universal Resource Language

USB			Universal Serial Bus

WCDMA			Wideband Code Division Mupltiple Access

WLAN			Wireless Local Area Network

WSDL			Web Service Description Language

XMPP			Extensible Messaging and Presence Protocol

# 4	Conventions

The keywords "Shall", "Shall not", "May", "Need not", "Should", "Should not" in the present document are to be interpreted as described in the oneM2M Drafting Rules [i.1].

## 5	M2M Services Architecture5.1	Introduction

This clause describes the architecture the M2M Services within the context of the M2M Services Platform.

The architecture of the M2M Service Platform as defined in the oneM2M Functional Architecture [2] includes an entity described as the Common Service Entity (CSE). The CSE comprises a set of service functions that are common to the M2M environment and are exposed through the Mca and Mcc' Reference Points. These Reference Points are described in the oneM2M Functional Architecture [2]. The M2M Service Architecture described in this specification is primarily suitable for the Infrastructure Domain where the CSE is viewed as a set of Service Components.

The M2M Service Architecture augments the oneM2M Functional Architecture by specifying M2M Services provided to M2M Application and M2M Service Providers.

These M2M Services are consumed by:

AEs across the Mca Reference Point via the Service Exposure Component

Other Infrastructure CSEs across the Mcc' Reference Point via the Remote Service Exposure Component

Other Service Components across the Msc Reference Point

These M2M Services utilize services of the:

Underlying Network across the Mcn Reference Point via the Network Service Utilization Component

Figure 5.1-1: oneM2M Services Architecture

The oneM2M Services architecture in Figure 5.1-1 comprises of the following entities:

Application Entity (AE): Defined by the oneM2M Functional Architecture [2], the Application Entity provides Application logic for the end-to-end M2M solutions.

Common Services Entity (CSE): Defined by the oneM2M Functional Architecture [2], a Common Services Entity comprises the set of "service functions" that are common to the M2M environments and specified by oneM2M. For oneM2M Services, this definition of a CSE is a logical representation where "service functions" that are exposed through the Mca and Mcc' Reference Points via the corresponding Service Exposure and Remote Service Exposure Components and the Network Service Utilization Component utilizes services of the Underlying Network through the Mcn referent point. In addition Service Components consume and provide M2M Services with other Service Components.

- As a logical representation of loosely coupled Service Components, the CSE is entity that in itself is identifiable but not directly addressable. Instead the addressable entities are the corresponding M2M Service Capabilities within the Service Exposure Components of the Reference Points.

Service Exposure Component: The Service Exposure Component exposes M2M Service Capabilities to AEs.

Network Service Utilization Component: The Network Service Utilization Component consumes Service Capabilities from the NSE.

Remote Service Exposure Component: The Remote Service Exposure connects M2M Service Capabilities from different M2M environments.

- The Service Exposure, Network Service Utilization and Remote Service Exposure Components follow the CSE Public Domain Names convention described in clause 6.5.1.1 of the Functional Architecture [2] but are extended as a sub-domain of the Infrastructure Node public domain name.

## 5.2	Reference Points

### 5.2.1	Introduction

The M2M Service Architecture exposes M2M Services across the Mca and Msc Reference Points. The enhancement of the Mcc and Mcc' Reference Point to expose M2M Services through the Remote Service Exposure Component is FFS. Likewise the Service Capabilities consumed from the NSE are FFS.

### 5.2.2	Mca Reference Point

The Mca Reference Point is defined within the oneM2M Functional Architecture [2]. This specification defines a set of service oriented capabilities that is used for the communication flow between the M2M Service Platform and the AE.

NOTE: 	In this specification, when an AE uses the Data Exchange M2M Service Capabilities defined in this specification across the Mca Reference Point, the CSE transmits the payload through the M2M Service Platform; offering additional M2M Services (e.g. Authorization, Event Collection) associated with the transmittal of the payload.

### 5.2.3	Msc Reference Point

The Msc Reference Point is a Proprietary Reference Point that specifies set of interactions between the Service Capabilities of different Service Components.

### 5.2.3.1	Message Exchange Patterns

Communication between M2M Service Components which pass over the Msc Reference Point utilizes a web services approach, e.g. Web Services Message Exchange Patterns (MEP) defined by WSDL [i.2]:

In-Only

Robust In-Only

In-Out

In-Optional-Out

Out-Only

Robust Out-Only

Out-In

Out-Optional-In

Figure 5.2.3.1-1: Message Exchange Patterns

The MEP(s) utilized by a M2M Service Capability is documented within the Service Capabilities Policy clause.

NOTE: 	A subset of the MEPs provided by a M2M Service Capability would be exposed by the Service Exposure Component or the Remote Service Exposure Component.

#### 5.2.3.2	M2M Identifiers

##### 5.2.3.2.1	M2M Service Capability Identifier (M2M-Serv-Cap-ID)

Service Components contain multiple M2M Services (M2M-Serv-ID). Each M2M Service contains multiple M2M Service Capabilities. The M2M Service Capability Identifier is assigned to each M2M Service Capability.

Table 5.2.3.2.1-1: M2M Service Capability Identifier

| Identifier | Assigned by | Assigned to | Assigned during | Lifetime | Uniqueness | Used during | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M2M Service Capability Identifier | M2M SP | M2M Service Capability | M2M Service Registration. This is the identifier from the Services Registry. | Life of the M2M Service Registration. | Global | Message exchange between Service Components across the Msc and Mcc'. Message exchange across the Mca. |  |

### 5.2.4	Mcc' Reference Point

The Mcc' Reference Point is defined within the oneM2M Functional Architecture [2]. The oneM2M Service Architecture utilizes communications over the Mcc' Reference Point to construct a Service Cluster for service sharing and capacity expansion. A messaging exchange protocol (e.g. AMQP, XMPP, JMS) between two service execution environments (SEE) (e.g. ESB, Apache Service Mix) accommodates the SEEs with same and different ownership.

NOTE: 	The M2M Service Capabilities that are exposed across the Mcc' Reference Point is FFS. Likewise, the protocol used for communication between SEEs with different ownership may require additional security capabilities and is FFS.

### 5.2.5	Mcn Reference Point

The Mcn Reference Point is defined within the oneM2M Functional Architecture [2].

NOTE: 	The Service Capabilities that are consumed across the Mcn Reference Point is FFS.

## 5.3	Configurations support by M2M Service Architecture

M2M Services exposed across the Mca, Mcn, Mcc' and Msc reference are supported for Infrastructure Nodes as defined in the oneM2M Functional Architecture [2].

## 6	M2M Services6.1	Introduction

This clause describes the M2M Services provided by the M2M Services Platform.. The M2M Services defined in this clause are utilized by the Supporting Services described in the Annexes of the present document.

Figure 6.1-1: M2M Services

## 6.2	Service Subscription

### 6.2.1	Overview

The Service Subscription service provides the ability to:

Validate the M2M Service Capabilities (e.g. requests, notifications) that go across the Mca Reference Point by ensuring M2M Service requested by the originator is permitted within the context of the M2M Service Subscription

Retrieve the Application Entity and associated M2M Service Subscription for the Application Entity

Assist in returning the Broker and Transport Adapter service instances needed to exchange data for data exchange service capabilities

Assist in returning the Management Adapter service instance needed to manage the device

### 6.2.2	M2M Service Subscription Entities

#### 6.2.2.1	Overview

The following diagram describes the associations between the AE, Service Roles and the M2M Service Capabilities to obtain an Application Entity and validate the M2M Service Subscription:

The M2M Service Subscription(s) for a specified M2M Service Capability is validated through the associations the Service Role has to the M2M Service Capability and M2M Service Subscription

The Application Entity and associated M2M Service Subscription is obtained through the Application Entity's to the M2M Service Subscription that is created during registration of the Application Entity

Figure 6.2.2.1-1: Service Subscription Entity Relationships

#### 6.2.2.2	M2M Service Subscription Broker Entities

In the present document. Brokers are used to exchange data between AEs where the AEs use the publish-subscribe data exchange pattern. Brokers are typically implemented using types of Message Queue technologies (e.g. MQTT, XMPP).

The following diagram describes the associations between the AE, M2M Service Subscription, M2M Service Capability and the Broker resource in order to obtain the Broker used to exchange the data:

The Broker is obtained through the association of the Broker Resource and the M2M Service Capability.

The Broker Resource is associated with Applications which are themselves associated with the Application Entity in the context of a M2M Service Subscription.

Figure 6.2.2.2-1: M2M Service Subscription for Broker Entity Relationships

#### 6.2.2.3	M2M Service Subscription Management Adapter Entities

The following diagram describes the associations between the M2M Service Subscription, M2M Nodes associated with the M2M Service Subscription and M2M Service Capability in order to obtain the Management Adapter.

The Management Adapter is obtained through the association of the M2M Node and the M2M Service Capability.

The M2M Node is associated with the M2M Service Subscription.

Figure 6.2.2.3-1: M2M Service Subscription For Management Adapter Entity Relationships

#### 6.2.2.4	M2M Service Subscription Transport Adapter Entities

In the present document. Transport Adapters for request-reply message exchange patterns. Transport Adapters are typically implemented using types of Message technologies (e.g. WSDL, REST).

The following diagram describes the associations between the M2M Service Subscription, AE and M2M Service Capability in order to obtain the Transport Adapter.

The Transport Adapter is obtained through the association of the Application and the M2M Service Capability

The Application is associated with the M2M Service Subscription

The AE is associated with an Application and M2M Service Subscription AE registration

Figure 6.2.2.4-1: M2M Service Subscription For Transport Adapter Entity Relationships

### 6.2.3	Service Capabilities

#### 6.2.3.1	validateServiceSubscription

##### 6.2.3.1.1	Description

This service capability ensures that a valid M2M Service Subscription exists for the requested M2M Service Capability requested that is either received by the M2M Service Platform from an AE or is transmitted to the AE from the M2M Service Platform.

##### 6.2.3.1.2	Pre-Conditions

A correlation between the M2M Service Capability, M2M Service Role(s) and the M2M Service Subscription has been defined in order to validate the M2M Service Subscription.

##### 6.2.3.1.3	Signature - validateServiceSubscription

Table 6.2.3.1.3-1: Service Subscription - validateServiceSubscription capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceSubscriptionId | IN | NO | The M2M Service Subscription Identifier (M2M-Service-Profile-ID) |
| serviceCapId | IN | NO | The M2M Service Capability Identifier (M2M-Serv-Cap-ID) |
| responseType | OUT | NO | Response types: - M2M Service Capability does not exist for requested M2M Service Subscription - M2M Service Capability does exist for the requested M2M Service Subscription |

##### 6.2.3.1.4	Post-Conditions

Not Applicable.

##### 6.2.3.1.5	Exceptions

Not Applicable.

##### 6.2.3.1.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.2.3.1.7	oneM2M Resource Interworking

This service capability interworks with the following oneM2M Resource operations that are interworked are:

RETRIEVE <m2MServiceSubscriptionProfile> resource retrieves the associated Service Roles. A M2M Service Capability is associated with the M2M Service Roles. The M2M Service Subscription is validated through the Service Roles associated with the M2M Service Capability.

#### 6.2.3.2	getAE

##### 6.2.3.2.1	Description

This service capability provides the ability to retrieve the Application Entity information and associated M2M Service Subscription Identifier associated with the Application Entity Identifier.

##### 6.2.3.2.2	Pre-Conditions

Not Applicable.

##### 6.2.3.2.3	Signature – getAE

Table 6.2.3.2.3-1: Service Subscription – getAE capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| aeId | IN | NO | The Application Entity Id (AE-ID) |
| applicationId | OUT | NO | The Application Identifier (App-ID) for the AE |
| serviceSubscriptionId | OUT | YES | The M2M Service Subscription (M2M-Service-Profile-ID) for the AE |
| externalIds | OUT | YES | The external identifiers of the device that hosts the AE |
| reachabilitySchedule | OUT | YES | The contact reachability schedule information of the AE associated with the device node. The absence of this parameter implies the AE associated with the device node is always contact reachable. Type Schedule, see 6.9.2.7.1. |
| responseType | OUT | YES | Unique response types for this service. AE does not exist |

##### 6.2.3.2.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the service capability

Figure 6.2.3.2.4-1: Service Subscription – getAE Diagram

##### 6.2.3.2.5	Post-Conditions

Not Applicable.

##### 6.2.3.2.6	Exceptions

Not Applicable.

##### 6.2.3.2.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.2.3.2.8	oneM2M Resource Interworking

When an AE is registered using a CREATE operation on the <AE> resource, the AE is associated with the M2M Service Subscription which is returned along with the correlated AE information (e.g. Application, Device).

#### 6.2.3.3	getBroker

##### 6.2.3.3.1	Description

This service capability obtains the Broker instance that will be used to subscribe and publish requests based on the requested M2M Service Capability (e.g. publishRequest, subscribeRequest), AE, Resource and M2M Service Subscription.

For the publishRequest service capability the AE is the from AE-ID and the resource is the toResource representing a leaf node from the publication resource.

For the subscribeRequest service capability the AE is the from AE-ID and the resource is the publication resource.

##### 6.2.3.3.2	Pre-Conditions

A correlation between a M2M Service Subscription, AE, Service Capability, Resource and Broker exist.

##### 6.2.3.3.3	Signature – getBroker

Table 6.2.3.3.3-1: Service Subscription - getBroker capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceSubscriptionId | IN | NO | The M2M Service Subscription Identifier (M2M-Service-Profile-ID). |
| serviceCapId | IN | NO | The M2M Service Capability Identifier (M2M-Serv-Cap-ID). |
| aeId | IN | NO | The identifier of the AE |
| resource | IN | NO | The identifier of the resource that is associated with the AE and M2M Service Subscription |
| broker | OUT | NO | The returned instance of the Broker service associated with the M2M Service Subscription. |
| responseType | OUT | NO | Response types: Broker does not exist for requested M2M Service Capability, M2M Service Subscription and Resource |

##### 6.2.3.3.4	Post-Conditions

Not Applicable.

##### 6.2.3.3.5	Exceptions

Not Applicable.

##### 6.2.3.3.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.2.3.3.7	oneM2M Resource Interworking

This service capability interworks with the following oneM2M Resource operations that are interworked are:

Not Applicable.

#### 6.2.3.4	getManagementAdapter

##### 6.2.3.4.1	Description

This service capability obtains the Management Adapter instance that will be used to manage device based on the requested M2M Service Capability (e.g. downloadFirmware, installFirmware) and device.

##### 6.2.3.4.2	Pre-Conditions

A correlation between a Management Adapter, the M2M Service Capability and device exist.

##### 6.2.3.4.3	Signature - getManagementAdapter

Table 6.2.3.4.3-1: Service Subscription – getManagementAdapter capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceSubscriptionId | IN | YES | The M2M Service Subscription Identifier (M2M-Service-Profile-ID), is optional when the deviceId is unique across M2M Service Subscription. |
| serviceCapId | IN | NO | The M2M Service Capability Identifier (M2M-Serv-Cap-ID). |
| deviceId | IN | YES | The unique device identifier in the context of the M2M Service Subscription. |
| managementAdapter | OUT | NO | The returned instance of the Management Adapter service associated with the subscription. |
| responseType | OUT | NO | Response types: Management Adapter does not exist |

##### 6.2.3.4.4	Post-Conditions

Not Applicable.

##### 6.2.3.4.5	Exceptions

Not Applicable.

##### 6.2.3.4.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.2.3.4.7	oneM2M Resource Interworking

This service capability interworks with the following oneM2M Resource operations that are interworked are:

Not Applicable.

#### 6.2.3.5	getTransportAdapter

##### 6.2.3.5.1	Description

This service capability obtains the Transport adapter instance that will be used to send requests based on the requested M2M Service Capability, AE and M2M Service Subscription.

##### 6.2.3.5.2	Pre-Conditions

A correlation between a M2M Service Subscription, AE and type of transport adapter (e.g. Broker) service exist.

##### 6.2.3.5.3	Signature - getTransportAdapter

Table 6.2.3.5.3-1: Service Subscription – getTransportAdapter capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceSubscriptionId | IN | NO | The M2M Service Subscription Identifier (M2M-Service-Profile-ID)' |
| serviceCapId | IN | NO | The M2M Service Capability Identifier (M2M-Serv-Cap-ID). |
| aeId | IN | NO | The identifier of the AE that is to target of the request ("to") (AE-ID) |
| transportAdapter | OUT | NO | The returned instance of the service (e.g. Broker) associated with the M2M Service Subscription. |
| responseType | OUT | NO | Response types: Transport Adapter does not exist for requested M2M Service Capability, M2M Service Subscription and target |

##### 6.2.3.5.4	Post-Conditions

Not Applicable.

##### 6.2.3.5.5	Exceptions

Not Applicable.

##### 6.2.3.5.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.2.3.5.7	oneM2M Resource Interworking

Not Applicable.

#### 6.2.3.6	associateAEWithServiceSubscription

##### 6.2.3.6.1	Description

This service capability provides the ability to associate an AE with a M2M Service Subscription. This service capability shall be restricted to the Msc Reference Point.

##### 6.2.3.6.2	Pre-Conditions

One of the following combinations has to be unique within the M2M System for the AE to be associated with an M2M Service Subscription:

Identifier of Application Entity (AE-ID); or

Identifier of the M2M Application (App-ID) and the credential identifier for the M2M Application.

The M2M System is pre-provisioned with the Application Rule (clause 6.9.2.5) that includes one of the above combinations in order for a M2M Service Subscription to the returned.

##### 6.2.3.6.3	Signature – associateAEWithServiceSubscription

Table 6.2.3.6.3-1: Service Subscription – associateAEWithServiceSubscription capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| aeId | IN | NO | The Application Entity Identifier (AE-ID). |
| credentialId | IN | YES | The Application Credential Identifier |
| applicationId | IN | YES | The Application Identifier (App-ID) |
| serviceSubscriptionId | OUT | NO | The M2M Service Subscription Identifier (M2M-Service-Profile-ID) |
| responseType | OUT | YES | Unique response types for this service. M2M Service Subscription does not exist. Application Entity Identifier is not unique within the context of the M2M System. |

##### 6.2.3.6.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue this capability

Figure 6.2.3.6.4-1: Service Subscription - associateAEWithServiceSubscription Diagram

##### 6.2.3.6.5	Post-Conditions

The Application Entity Identifier (AE-ID) provided as the input parameter is associated with the M2M Service Subscription.

##### 6.2.3.6.6	Exceptions

Not Applicable.

##### 6.2.3.6.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.2.3.6.8	oneM2M Resource Interworking

This service capability associates the AE-ID with the M2M Service Subscription Profile. Currently TS-0001 [2] does not define how an AE is associated with a M2M Service Subscription Profile as part of a registration flow.

#### 6.2.3.7	disassociateAEFromServiceSubscription

##### 6.2.3.7.1	Description

This service capability provides the ability to remove the association of an AE from a M2M Service Subscription. This service capability shall be restricted to the Msc Reference Point.

##### 6.2.3.7.2	Pre-Conditions

An AE has successfully registered with the M2M system.

##### 6.2.3.7.3	Signature - disassociateAEFromServiceSubscription

Table 6.2.3.7.3-1: Service Subscription - disassociateAEFromServiceSubscription capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| aeId | IN | NO | The Application Entity Identifier (AE-ID). |
| serviceSubscriptionId | IN | NO | The M2M Service Subscription Identifier (M2M-Service-Profile-ID) |
| responseType | OUT | YES | Unique response types for this service. - M2M Service Subscription does not exist. - Application Entity Identifier is not associated with the M2M Service Subscription. |

##### 6.2.3.7.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue this capability

Figure 6.2.3.7.4-1: Service Subscription - disassociateAEFromServiceSubscription Diagram

##### 6.2.3.7.5	Post-Conditions

The Application Entity registration association is removed from the M2M Service Subscription.

##### 6.2.3.7.6	Exceptions

Not Applicable.

##### 6.2.3.7.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.2.3.7.8	oneM2M Resource Interworking

This service capability removes the association of the AE-ID from the M2M Service Subscription Profile. Currently TS-0001 [2] does not define how an AE is disassociated from a M2M Service Subscription Profile as part of a AE lifecycle flow.

#### 6.2.3.8	refreshAEAssociationWithServiceSubscription

##### 6.2.3.8.1	Description

This service capability provides the ability to refresh an existing registration association between an Application Entity and a M2M Service Subscription. This service capability shall be restricted to the Mca and Msc Reference Points.

##### 6.2.3.8.2	Pre-Conditions

An AE has successfully registered with the M2M system

##### 6.2.3.8.3	Signature – refreshAEAssociationWithServiceSubscription

Table 6.2.3.8.3-1: Service Subscription – refreshAEAssociationFWithServiceSubscription capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| aeId | IN | NO | The Application Entity Identifier (AE-ID). |
| serviceSubscriptionId | IN | NO | The M2M Service Subscription Identifier (M2M-Service-Profile-ID) |
| responseType | OUT | YES | Unique response types for this service. - M2M Service Subscription does not exist. - Application Entity Identifier is not associated with the M2M Service Subscription. |

##### 6.2.3.8.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue this capability

Figure 6.2.3.8.4-1: Service Subscription - refreshAEAssociationWithServiceSubscription Diagram

##### 6.2.3.8.5	Post-Conditions

Not Applicable.

##### 6.2.3.8.6	Exceptions

Not Applicable.

##### 6.2.3.8.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.2.3.8.8	oneM2M Resource Interworking

This service capability refreshes the association of the Application Entity (AE-ID) with the M2M Service Subscription Profile. Currently TS-0001 [2] does not define how an AE registration association is refreshed with a M2M Service Subscription Profile as part of a AE lifecycle flow.

## 6.3	Authorization

### 6.3.1	Overview

The Authorization service provides the ability to:

Authorize the Originator for the service capability

NOTE:	The determination of how the entity instance based access management is configured in order to ensure no cross ASP access to other ASPs data is FFS.

### 6.3.2	Service Capabilities

#### 6.3.2.1	authorizeServiceCapability

##### 6.3.2.1.1	Description

This service capability authenticates an Originator is a valid Originator of the request and then authorizes the M2M Service Capability within the context of a M2M Service Subscription between the from AE-ID and to AE-ID.

##### 6.3.2.1.2	Pre-Conditions

The M2M Identifiers for the request Originator, M2M Service Subscription from AE-ID and to AE-ID are assigned to the M2M Service Subscription.

A correlation between the M2M Service Capability, M2M Service and the M2M authorization event has been defined in order to authorize the M2M Service Capability.

##### 6.3.2.1.3	Common M2M Service Capability Parameters for Request Authentication and Authorization

Table 6.3.2.1.3-1: Common M2M Service Capability Parameters for 
Request Authentication and Authorization

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| originator | IN | NO | The identifier of the Originator that is issuing the request. |
| from | IN | NO | The identifier of the Originator of the request (AE-ID) |
| to | IN | NO | The identifier of the AE that is to target of the request (AE-ID) |
| serviceCapId | IN | NO | The identifier of the M2M Service Capability (M2M-Serv-Cap-ID) |
| serviceSubscriptionId | IN | NO | The M2M Service Subscription Identifier (M2M-Service-Profile-ID) |
| responseType | OUT | YES | Response types that are relevant to the Authentication and Authorizations of requests from AEs across the Mca Reference Point. Originator is not authenticated Originator does not have a M2M Service Subscription Originator not authorized for the M2M Service Capability |

##### 6.3.2.1.4	Service Interactions

Not Applicable.

##### 6.3.2.1.5	Post-Conditions

Not Applicable.

##### 6.3.2.1.6	Exceptions

Not Applicable.

##### 6.3.2.1.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.3.2.1.8	oneM2M Resource Interworking

This service capability interworks with the following oneM2M Resource operations that are interworked are:

FFS – The End-to-End Security Framework is expected to develop APIs that can be issued for this purposed.

## 6.4	Data Exchange

### 6.4.1	Overview

The Data Exchange service provides the ability to exchange payloads between AEs across the Mca Reference Point using supported data exchange patterns. This service utilizes an underlying transport adapter to provide the transportation functions.

The supported data exchange patterns are:

Subscribe-Publish-Notify

Request-Response

### 6.4.2	Data Exchange Entities

#### 6.4.2.1	Subscribe-Publish-Notify Data Exchange

##### 6.4.2.1.1	Overview

This data exchange pattern permits AEs to subscribe to publication resources. When an AE publishes data to a specific publication resource, the AEs that have subscribed to the publication resource are notified of the publication by receiving the published data.

##### 6.4.2.1.2	Supporting Rules

###### 6.4.2.1.2.1	Overview

A publication resource is an entity that represents the mechanism used by underlying publish and subscribe protocols which represent one or more queues in the underlying protocol. A publication resource in the data exchange service provides capabilities to reference one or more queues in the underlying protocol using a tree mechanism where wildcards are placed to represent all elements of the tree branch or leaf.

###### 6.4.2.1.1.2	Publication Resources

When a AE publishes a payload to a publication resource, the publication resource is evaluated to obtain the leaf node of the tree. The AEs that have subscribed to leaf nodes of the tree receive the message via the notify M2M Service Capability.

###### 6.4.2.1.1.3	Delivery Policy

As part of the notify M2M Service Capability the Service Layer attempts to honour the delivery policy of the subscriber and publisher. The delivery policy is the attempt by the Service Layer to provide a reliable delivery of the payload that was published using the mechanisms of the underlying Broker. The reliable delivery of a payload is defined in the deliveryPolicy type.

Table 6.4.2.1.1.3-1: Type: deliveryPolicy

| Attribute | Description |
| --- | --- |
| retryLimit | The number of retries by the Broker to connected subscribers before assuming failure of the notification. |

###### 6.4.2.1.1.4	Message Retainment Policy

When a message is published, the M2M Service Layer, using the resources of the underlying Broker, attempts to retain messages according to a retainment policy. Retainment of messages permits AEs that are subscribed to the publication resource but not connected to the underlying Broker to receive messages, up to the retainLimit once the AE connects to the Broker. The message retainment policy is defined in the retainmentPolicy type.

Table 6.4.2.1.1.4-1: Type: retainmentPolicy

| Attribute | Description |
| --- | --- |
| retainLimit | The maximum number of messages retained by the Broker for a subscriber that is not connected to the underlying Broker. |

###### 6.4.2.1.1.5	Service Subscription Integration

In order for the Data Exchange Service to take advantage of the underlying transport, there shall be an association between the AE Resource and the identifier of the underlying transport within the context of the M2M Service Subscription.

#### 6.4.2.2	Request-Response Data Exchange

##### 6.4.2.2.1	Overview

This data exchange pattern permits AEs to synchronously send messages to other AEs utilizing the underlying transport.

##### 6.4.2.2.2	Supporting Rules

###### 6.4.2.2.2.1	Overview

Messages are sent to from an AE to another AE via the Service Layer. When sending a message the originating AE can provide a delivery policy to be used by the transport adapter when sending the message to the target AE.

###### 6.4.2.2.2.2	Delivery Policy

As part of the message sending capability the Service Layer attempts to honour the delivery policy of the message Originator. The delivery policy is the attempt by the Service Layer to provide a reliable delivery of the payload that was sent using the mechanisms of the transport adapter. The reliable delivery of a payload is defined in the deliveryPolicy type.

Table 6.4.2.2.2.2-1: Type: deliveryPolicy

| Attribute | Description |
| --- | --- |
| retryLimit | The number of retries by the transport adapter to target AEs before assuming failure of the notification. |

###### 6.4.2.2.2.3	Service Subscription Integration

In order for the Data Exchange Service to take advantage of the underlying transport, there shall be an association between the target AE and the identifier of the underlying transport within the context of the M2M Service Subscription.

### 6.4.3	Service Capabilities

#### 6.4.3.1	subscribe

##### 6.4.3.1.1	Description

This service capability provides the ability for AEs to subscribe to receive payloads based on a publication resource.

As part of the subscription, the subscribing AE can provide delivery and retainment policies to enhance the robustness of the notification to AEs that have subscribed to the publication resource identified in the request.

##### 6.4.3.1.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a M2M Service Subscription, publication resource, subscribing AE and Broker exist.

##### 6.4.3.1.3	Signature – subscribe

Table 6.4.3.1.3-1: Data Exchange Service – subscribe capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| publicationResource | IN | NO | The publication resource. See 6.4.2.1.1.2 |
| deliveryPolicy | IN | YES | The delivery policy when notifying the subscriber. See 6.4.2.1.1.3 |
| retainmentPolicy | IN | YES | The retainment policy for unconnected subscribers. See 6.4.2.1.1.4 |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Exception: Request may not have been completed |

##### 6.4.3.1.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the subscribe request to the Supporting Service to validate the request and obtain the Broker instance for the subscription request.

Issue the request to the broker to subscribe the AE-ID for the publication resource

Issue the subscribe complete subscription request to the Supporting Service

Figure 6.4.3.1.4-1: Data Exchange Service – subscribe capability

##### 6.4.3.1.5	Post-Conditions

Not Applicable.

##### 6.4.3.1.6	Exceptions

Not Applicable.

##### 6.4.3.1.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.4.3.1.8	oneM2M Resource Interworking

This service capability interworks with the following oneM2M Resource operations that are interworked are:

Not Applicable.

#### 6.4.3.2	publish

##### 6.4.3.2.1	Description

This service capability provides the ability for AEs to publish payloads to a leaf node of a publication resource.

As part of the publication, the publishing AE can provide a delivery policy to enhance the robustness of the publication to AEs that have subscribed to the Resource identified in the request.

##### 6.4.3.2.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a M2M Service Subscription, resource to publish the payload, publishing AE and Broker exist.

##### 6.4.3.2.3	Signature - publish

Table 6.4.3.2.3-1: Data Exchange Service – publish capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| toResource | IN | NO | The leaf node of a publication resource. See 6.4.2.1.1.2 |
| deliveryPolicy | IN | YES | The delivery policy when notifying the subscriber. See 6.4.2.1.1.3 |
| payload | IN | NO | The payload to published. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Exception: Request may not have been completed |

##### 6.4.3.2.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the publish request to the Supporting Service to validate the request

- Issue the request to the broker to publish the payload from the publishing AE to the resource in the request

- Issue the complete publication request to the Supporting Service

Figure 6.4.3.2.4-1: Data Exchange Service –publish (In-Out) capability

##### 6.4.3.2.5	Post-Conditions

Not Applicable.

##### 6.4.3.2.6	Exceptions

Not Applicable.

##### 6.4.3.2.7	Policies for Use

Message Exchange Patterns: In-Out, In-Only

The In-Only message exchange pattern would return a transport layer acknowledgement if supported by the underlying transport layer protocol. Response types or Exceptions are not returned in this message exchange pattern.

Transaction Pattern: Participation allowed

##### 6.4.3.2.8	oneM2M Resource Interworking

This service capability interworks with the following oneM2M Resource operations that are interworked are:

Not Applicable.

#### 6.4.3.3	notify

##### 6.4.3.3.1	Description

This service capability provides the ability to notify AEs that have subscribed to the publication resource identified in a publish request.

##### 6.4.3.3.2	Pre-Conditions

A correlation between a M2M Service Subscription, subscribed AE, publication resource and Broker exist.

##### 6.4.3.3.3	Signature - notify

Table 6.4.3.33-1: Data Exchange Service – notify capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| fromResource | IN | NO | The leaf node of a publication resource. See 6.4.2.1.1.2 |
| deliveryPolicy | IN | YES | The delivery policy when notifying the subscriber. See 6.4.2.1.1.3 |
| payload | IN | NO | The Payload of the message to notify to the AE |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Exception: Request may not have been completed |

##### 6.4.3.3.4	Post-Conditions

Not Applicable.

##### 6.4.3.3.5	Exceptions

Not Applicable.

##### 6.4.3.3.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.4.3.3.7	oneM2M Resource Interworking

This service capability interworks with the following oneM2M Resource operations that are interworked are:

Not Applicable.

#### 6.4.3.4	sendMessage

##### 6.4.3.4.1	Desription

This service capability provides the ability for AEs to send messages to a target AE.

As part of the message send request, the originating AE can provide a delivery policy to enhance the robustness of the message to the AE.

##### 6.4.3.4.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a M2M Service Subscription, Originating AE and Transport Adapter exist.

##### 6.4.3.4.3	Signature - sendMessage

Table 6.4.3.4.3-1: Data Exchange Service – sendMessage capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deliveryPolicy | IN | YES | The delivery policy when sending to the target AE. See 6.4.2.2.2.2 |
| payload | IN | NO | The payload to be sent. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Exception: Request may not have been completed |

##### 6.4.3.4.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the sendMessage request to the Supporting Service to validate the request

- Issue the request to the transport adapter (in this case the Broker service) to send the payload from the Originator to the target AE.

- Issue the complete sendMessage request to the Supporting Service

Figure 6.4.3.4.4-1: Data Exchange Service –sendMessage capability

##### 6.4.3.4.5	Post-Conditions

Not Applicable.

##### 6.4.3.4.6	Exceptions

Not Applicable.

##### 6.4.3.4.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.4.3.4.8	oneM2M Resource Interworking

Not Applicable.

## 6.5	Broker

### 6.5.1	Overview

The Broker service provides the ability to adapt the Publish-Subscribe-Notify and Request Response data exchange patterns to the underlying transport protocol and deployment configuration.

The Broker maintains the subscriptions to the publication resources

### 6.5.2	Service Capabilities

#### 6.5.2.1	subscribe

##### 6.5.2.1.1	Description

This service capability provides the ability for AEs to subscribe to receive payloads based on a publication resource.

As part of the publication, the publishing AE can provide delivery and retainment policies to enhance the robustness of the notification to AEs that have subscribed to the Resource identified in the request.

##### 6.5.2.1.2	Pre-Conditions

Not Applicable.

##### 6.5.2.1.3	Signature - subscribe

Table 6.5.2.1.3-1: Broker - subscribe capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| publicationResource | IN | NO | The publication resource. See 6.4.2.1.1.2 |
| deliveryPolicy | IN | YES | The delivery policy when notifying the subscriber. See 6.4.2.1.1.3 |
| retainmentPolicy | IN | YES | The retainment policy for unconnected subscribers. See 6.4.2.1.1.3 |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Exception: Request may not have been completed |

##### 6.5.2.1.4	Post-Conditions

Not Applicable.

##### 6.5.2.1.5	Exceptions

Not Applicable.

##### 6.5.2.1.6	Policies for Use

Message Exchange Patterns: In-Out

The In-Only message exchange pattern would return a transport layer acknowledgement if supported by the underlying Transaction Pattern: Participation allowed

#### 6.5.2.2	publish

##### 6.5.2.2.1	Description

This service capability provides the ability for AEs to publish payloads to a leaf node of a publication resource.

As part of the publication, the publishing AE can provide a delivery policy to enhance the robustness of the publication to AEs that have subscribed to the Resource identified in the request.

##### 6.5.2.2.2	Pre-Conditions

Not Applicable.

##### 6.5.2.2.3	Signature - publish

Table 6.5.2.2.3-1: Broker - publish capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| toResource | IN | NO | The leaf node of a publication resource. See 6.4.2.1.1.2. |
| deliveryPolicy | IN | YES | The delivery policy when notifying the subscriber. See 6.4.2.1.1.3. |
| payload | IN | NO | Payload to be published. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Exception: Request may not have been completed |

##### 6.5.2.3.4	Post-Conditions

Not Applicable.

##### 6.5.2.3.5	Exceptions

Not Applicable.

##### 6.5.2.3.6	Policies for Use

Message Exchange Patterns: In-Out

The In-Only message exchange pattern would return a transport layer acknowledgement if supported by the underlying Transaction Pattern: Participation allowed

##### 6.5.2.3.7	oneM2M Resource Interworking

This service capability interworks with the following oneM2M Resource operations that are interworked are:

Not Applicable.

#### 6.5.2.3	notify

##### 6.5.2.3.1	Description

This service capability provides the ability for the Broker to notify AEs that have subscribed to the publication resource identified in the request.

##### 6.5.2.3.2	Pre-Conditions

The Broker maintains the AEs (to AE-ID, clientId) that have subscribed to publicationResources.

The Broker maintains the M2M Service Subscription (serviceSubscriptionId) for AE subscriptions to publicationResources.

##### 6.5.2.3.3	Signature - notify

Table 6.5.2.3.3-1: Broker – notify capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| payload | IN | NO | The payload to be transferred |
| fromResource | IN | NO | The leaf node of a publication resource. See 6.4.2.1.1.2 |
| deliveryPolicy | IN | YES | The delivery policy when notifying the subscriber. See 6.4.2.1.1.3 |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Originator does not have a Broker for the requested resource Originator has requested an invalid delivery policy Exception: Request may not have been completed |

##### 6.5.2.3.4	Service Interactions

For each AE subscribed to the resource perform the interactions of service capabilities required for this service capability:

- Issue the notify request to the Supporting Service to validate the request

- Issue the request to the SE: Data Exchange

- Issue the complete request to the Supporting Service

Figure 6.5.2.3.4-1: Broker –notify (In-Out) capability

##### 6.5.2.3.5	Post-Conditions

Not Applicable.

##### 6.5.2.3.6	Exceptions

Not Applicable.

##### 6.5.2.3.7	Policies for Use

Message Exchange Patterns: In-Out, In-Only

The In-Only message exchange pattern would return a transport layer acknowledgement if supported by the underlying transport layer protocol. Response types or Exceptions are not returned in this message exchange pattern.

Transaction Pattern: Participation allowed

##### 6.5.2.3.8	oneM2M Resource Interworking

This service capability interworks with the following oneM2M Resource operations that are interworked are:

Not Applicable.

#### 6.5.2.4	sendMessage

##### 6.5.2.4.1	Description

This service capability provides the ability for AEs to send message payloads to a target AE.

As part of the send message, the originating AE can provide delivery to enhance the robustness of the publication to AEs that have subscribed to the Resource identified in the request.

##### 6.5.2.4.2	Pre-Conditions

The Broker service has connectivity to the target AE where the target AE has subscribed to the Broker to receive send message requests from the Broker.

##### 6.5.2.4.3	Signature - sendMessage

Table 6.5.2.4.3-1: Broker - sendMessage capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deliveryPolicy | IN | YES | The delivery policy when notifying the subscriber. See 6.4.1.1.2.2 |
| payload | IN | NO | Payload to be published. |
| responseType | OUT | YES | Unique response types for this service. Target AE is not subscribed with the Broker to receive messages Originator has requested an invalid delivery policy NOTE: Consumed services also provide response types. Exception: Request may not have been completed |

##### 6.5.2.4.4	Post-Conditions

Not Applicable.

##### 6.5.2.4.5	Exceptions

Not Applicable.

##### 6.5.2.4.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.5.2.4.6	oneM2M Resource Interworking

Not Applicable.

## 6.6	Device Management

### 6.6.1	Overview

The Device Management service provides for the capability for an AE to manage devices across the Mca Reference Point using supported device management message exchange patterns. This service utilizes management adapters to provide the management functions.

The supported device management message exchanges patterns are:

Request-Response

Request- Asynchronous Report

### 6.6.2	Device Management Entities

#### 6.6.2.1	Overview

The Device Management service permits AEs to request management operations (e.g. device configuration operations, device troubleshooting operations, device firmware management operations, and application software management operations) and provide the result or report/retrieve the status of the operations to the originating AE.

AEs have the option to request to manage a targeted set of devices that include:

Individual device

Multiple devices through the use of lists of devices

A group that designates as list of devices using the groupId

AEs have option to request management operations to be performed based on a schedule.

AEs have the option to request that responses to requests be delivered synchronous (the request is held until all responses are received) or asynchronously (Reports of the operation are transmitted to the AE when an operation has completed) to the request submission. For asynchronous requests, reports are transmitted based on a report policy configured as part of the original request.

AEs also have the capability to request to manage devices by complex operations (e.g. upgradefirmware), where the M2M service platform can orchestrate management operations (e.g. installfirmware, downloadware) according to the business process.

#### 6.6.2.2	Supporting Rules

##### 6.6.2.2.1	Report Policy Type

The report policy is used to determine how a report is sent by the M2M System to the AE. There is a choice of parameters used for batchReport, pendingReport, latestReport.

Table 6.6.2.2.1-1: Type:ReportPolicy

| Parameter | Optional | Description |
| --- | --- | --- |
| groupId | YES | The group identifier. |
| operationMonitorList | YES | List of monitored operations. Enum OperationMonitor, see 6.6.2.2.29. |
| batchReport | YES | This parameter indicates that AE wants to receive batches of reports rather than receiving them one at a time. This attribute e may include two values: the number of reports to be batched for delivery and a duration. If batchReport is used simultaneously with latestReport, only the latest report shall be sent. Type BatchReport, see 6.6.2.2.2. |
| pendingReport | YES | This parameter indicates how missed reports due to a period of connectivity. Enum PendingReport, see 6.6.2.2.28. |
| latestReport | YES | This parameter indicates if AE wants only the latest report. |

##### 6.6.2.2.2	Batch Report Type

The BatchReport is complex type that defines the batch for reports.

Table 6.6.2.2.2-1: Type: BatchReport

| Parameter | Optional | Description |
| --- | --- | --- |
| number | YES | The number of reports to be batched. |
| duration | YES | The duration in which all the reports to be batched. |

##### 6.6.2.2.3	Firmware Report Type

The FirmwareReport is complex type that defines the current state of a firmware management operation for a specific device.

Table 6.6.2.2.3-1: Type: FirmwareReport

| Parameter | Optional | Description |
| --- | --- | --- |
| groupId | YES | The group identifier. |
| deviceId | NO | The unique device identifier in the context of the M2M Service Subscription. |
| operation | YES | The firmware management operation.(e.g. installFirmware, downloadFirmware) |
| operationResult | YES | The firmware management operation execution result. |
| operationStatus | YES | The firmware management operation execution status. It can be Initiated, Started, Finished, Cancelled or Deleted. |

##### 6.6.2.2.4	Orchestration Rule Type

The OrchestrationRule is a complex type that describes the target and associated schedule of the request for the execution of the management operation.

Table 6.6.2.2.4-1: Type: OrchestrationRule

| Parameter | Optional | Description |
| --- | --- | --- |
| groupId | YES | The group identifier. |
| deviceId | NO | The unique device identifier in the context of the M2M Service Subscription. |
| operation | NO | The orchestrated operation. (e.g. installFirmware, downloadFirmware) |
| schedule | YES | The time when to execute firmware operation. If it is not provided, the operation shall be executed immediately. |

##### 6.6.2.2.5	Firmware Info Type

The FirmwareInfo is a complex type that describes the version, name, URL of the device firmware.

Table 6.6.2.2.5-1: Type: FirmwareInfo

| Parameter | Optional | Description |
| --- | --- | --- |
| version | YES | The version of the firmware. |
| name | YES | The name of the firmware to be used on the device. |
| url | YES | The URL from which the firmware image can be downloaded. |

##### 6.6.2.2.6	Service Subscription Integration

In order for the Device Management Service to take advantage of the Management Adapter, there shall be association between the device associated with a Service Subscription and the service capability to the Management Adapter.

##### 6.6.2.2.7	Device Info Type

The DeviceInfo is a complex type that describes the device label, manufacturer, model, device type, firmware version, software version, hardware version.

Table 6.6.2.2.7-1: Type: DeviceInfo

| Parameter | Optional | Description |
| --- | --- | --- |
| deviceLabel | YES | Unique device label assigned by the manufacturer. The uniqueness may be global or only valid within a certain domain (e.g. vendor-wise or for a certain deviceType). |
| manufacturer | YES | The name/identifier of the device manufacturer. |
| model | YES | The name/identifier of the device mode assigned by the manufacturer. |
| deviceType | YES | The type (e.g. cell phone, photo frame, smart meter) or product class (e.g. X-series) of the device. |
| fwVersion | YES | The firmware version of the device. NOTE: If the device only supports one kind of Software this is identical to softwareVersion. |
| swVersion | YES | The software version of the device. |
| hwVersion | YES | The hardware version of the device. |

##### 6.6.2.2.8	Memory Type

The Memory is a complex type that describes the current available amount of memory and the total amount of memory.

Table 6.6.2.2.8-1: Type: Memory

| Parameter | Optional | Description |
| --- | --- | --- |
| memAvailable | YES | The current available amount of memory in bytes. |
| memTotal | YES | The total amount of memory in bytes. |

##### 6.6.2.2.9	Battery Type

The Battery is a complex type that describes the current battery level and the status of the battery.

Table 6.6.2.2.9-1: Type: Battery

| Parameter | Optional | Description |
| --- | --- | --- |
| level | YES | The current battery level as a percent of the battery capacity |
| status | YES | Indicates the status of the battery. Enum BatteryStatus, see 6.6.2.2.11. |

##### 6.6.2.2.10	DeviceCapability Type

The DeviceCapability is a complex type that describes the name of the capability, attach label, capability action status.

Table 6.6.2.2.10-1: Type: DeviceCapability

| Parameter | Optional | Description |
| --- | --- | --- |
| name | YES | The name of the capability. Enum DeviceCapabilityName, see 6.6.2.2.12. |
| attached | YES | Indicates whether the capability is attached to the device or not. |
| state | NO | Indicates if the capability is enabled or disabled |

##### 6.6.2.2.11	BatteryStatus Enum

Table 6.6.2.2.11-1: Enum: BatteryStatus

| Battery Status | Description |
| --- | --- |
| Normal | The battery is operating normally and not on power. |
| Charging | The battery is currently charging. |
| Charge Complete | The battery is fully charged and still on power. |
| Damaged | The battery has some problem. |
| Low Battery | The battery is low on charge. |
| Not Installed | The battery is not installed. |
| Unknown | The battery information is not available. |

##### 6.6.2.2.12	DeviceCapabilityName Enum

Table 6.6.2.2.11-1: Enum: DeviceCapabilityName

| Device Capability Name | Description |
| --- | --- |
| Hardware: ExternalMemory | Used to enable/disable the removable card memory or storage disk. |
| Hardware: Display | Used to enable/disable the display screen. |
| Hardware: Camera | Used to enable/disable the camera. |
| Hardware: Speaker | Used to enable/disable the speaker. |
| I/O: Tethering | Used to enable/disable the device to be attached to other devices. |
| I/O: AudioInputEncoder | Used to enable/disable the audio input encoder. |
| I/O: AttachedDevice | Used to enable/disable the capability to allow other devices to be attached to this device. |
| I/O: Keyboard | Used to enable/disable the keyboard |
| I/O: InputPeripheral | Used to enable/disable the input peripheral. |
| I/O: OutputPeripheral | Used to enable/disable the output peripheral. |
| I/O: USB | Used to enable/disable the USB port. |
| I/O: SerialPort | Used to enable/disable the serial port. |
| I/O: ParallelPort | Used to enable/disable the Parallel port. |
| I/O: GPS | Used to enable/disable GPS capability |
| I/O: GNSS | Used to enable/disable GNSS capability |
| Connectivity: Bluetooth | Used to enable/disable the Bluetooth connectivity. |
| Connectivity: WLAN | Used to enable/disable the WLAN connectivity. |
| Connectivity: Infrared | Used to enable/disable the Infrared connectivity. |
| Connectivity: WCDMA | Used to enable/disable the WCDMA connectivity |
| Connectivity: GPRS | Used to enable/disable the GPRS connectivity |
| Connectivity: EDGE | Used to enable/disable the EDGE connectivity |
| Connectivity: CDMA | Used to enable/disable the CDMA connectivity |
| Connectivity: WiMAX | Used to enable/disable the WiMAX connectivity |
| Connectivity: LTE | Used to enable/disable the LTE connectivity |
| Connectivity: NFC | Used to enable/disable the NFC connectivity |
| Connectivity: TD-SCDMA | Used to enable/disable the TD-SCDMA connectivity |
| LAN Interfaces: USB | Used to enable/disable the USB Interface |
| LAN Interfaces: Wi-Fi | Used to enable/disable the Wi-Fi Radio |
| LAN Interfaces: HomePlug | Used to enable/disable the HomePlug Interface |
| LAN Interfaces: MoCA | Used to enable/disable the MoCA Interface |
| LAN Interfaces: UPA | Used to enable/disable the UPA Interface |
| Hardware: SmartCardReader | Used to enable/disable the SmartCardReader |

##### 6.6.2.2.13	LockStatus Enum

Table 6.6.2.2.13-1: Enum: LockedStatus

| Lock Status | Description |
| --- | --- |
| locked | The device is locked. |
| unlocked | The device is unlocked. |

##### 6.6.2.2.14	Area Network Info Type

The AreaNwkInfo is a complex type that describes the information about a M2M area network. Information includes the area network identifier, area network name, area network type, and the list of devices in the M2M Area Network.

Table 6.6.2.2.14-1: Type: AreaNwkInfo

| Parameter | Optional | Description |
| --- | --- | --- |
| areaNwkId | NO | The unique area network identifier. |
| areaNwkName | YES | The Area Network name. |
| areaNwkType | YES | The areaNwkType is an implementation-chosen string that indicates the type of M2M Area Network. |
| listOfDevices | YES | Indicates the list of devices in the M2M Area Network. Type AreaNwkDevice, see 6.6.2.2.15. From listOfDevices, the topology of the area network can be discovered and retrieved. |

##### 6.6.2.2.15	Area Network Device Type

The AreaNwkDevice is a complex type that describes the device information for a device in the context of a M2M Area Network. Information includes the device identifier, area network identifier and the area network device specific information.

Table 6.6.2.2.15-1: Type: AreaNwkDevice

| Parameter | Optional | Description |
| --- | --- | --- |
| deviceId | NO | The unique device identifier in the context of the M2M Service Subscription. |
| areaNwkDeviceInfo | YES | The area network device information. Type AreaNwkDeviceInfo, see 6.6.2.2.16. |

##### 6.6.2.2.16	Area Network Device Info Type

The AreaNwkDeviceInfo is a complex type that describes information about the device in the context of the M2M Area Network. Information includes the type of device, the interval of time between two sleep cycles, the time duration of each sleep cycle, the status of the device and any neighbor devices.

Table 6.6.2.2.16-1: Type: AreaNwkDeviceInfo

| Parameter | Optional | Description |
| --- | --- | --- |
| deviceType | YES | The type (e.g. cell phone, photo frame, smart meter) or product class (e.g. X-series) of the device. |
| sleepInterval | YES | The interval between two sleep cycles. |
| sleepDuration | YES | The time duration of each sleep cycle. |
| status | YES | The status of the device (sleeping or waked up). |
| listOfNeighbors | YES | Indicates the neighbour devices of the same area network. When modified, the connection relationship of the devices shall be modified accordingly. |

##### 6.6.2.2.17	LogTypeId Enum

Table 6.6.2.2.17-1: Enum: LogTypeId

| LogTypeId | Description |
| --- | --- |
| system | System log |
| security | Security log |
| event | Event log |
| trace | Trace log |
| panic | Panic log |

##### 6.6.2.2.18	LogStatus Enum

Table 6.6.2.2.18-1: Enum: LogStatus

| LogStatus | Description |
| --- | --- |
| Started | the logging activity is started |
| Stopped | the logging activity is stopped |
| Unknown | the current status of the logging activity is unknown. |
| NotPresent | the log data is not present and the logData attribute shall be ignored. |
| Error | error conditions for the logging activities, and the logging is stopped. |

##### 6.6.2.2.19	LogInfo Type

The LogInfo is a complex type that describes the log type, diagnostic data, log action status.

Table 6.6.2.2.19-1: Type: LogInfo

| Parameter | Optional | Description |
| --- | --- | --- |
| logTypeId | NO | The type of the log. E.g. security log, system log. Enum LogTypeId, see 6.6.2.2.17. |
| logData | NO | Diagnostic data logged upon event of interests defined by the diagnostic function. |
| logActionStatus | NO | Indicates the status of the Action. E.g. Started, Stopped. Enum LogStatus, see 6.6.2.2.18. |

##### 6.6.2.2.20	TroubleshootingReport Type

The TroubleshootingReport is complex type that defines the current state of a troubleshooting operation for a specific device.

Table 6.6.2.2.20-1: Type: TroubleshootingReport

| Parameter | Optional | Description |
| --- | --- | --- |
| groupId | YES | The group identifier. |
| deviceId | NO | The unique device identifier in the context of the M2M Service Subscription. |
| operation | YES | The troubleshooting operation.(e.g. rebootDevice, resetDevice, uploadDeviceLog) |
| operationResult | YES | The troubleshooting operation execution result. |
| operationStatus | YES | The troubleshooting operation execution status. It can be Initiated, Started, Finished, Cancelling, or Cancelled. |

##### 6.6.2.2.21	Log Type

The Log is a complex type that describes the log information, log URL.

Table 6.6.2.2.21-1: Type: Log

| Parameter | Optional | Description |
| --- | --- | --- |
| logInfo | NO | The log information. Type LogInfo, see 6.6.2.2.19. |
| logURL | NO | The URL from which the log can be accessed. |

##### 6.6.2.2.22	Log Filter Criteria

Table 6.6.2.2.22-1: Log Filter Criteria

| Criterion name | Description |
| --- | --- |
| logTypeId | The type of the log. E.g. security log, system log. Enum LogTypeId, see 6.6.2.2.17. |
| startTime | The start time of log. |
| endTime | The end time of log. |

##### 6.6.2.2.23	Action Enum

Table 6.6.2.2.23-1: Enum: Action

| Software Action | Description |
| --- | --- |
| Install | The action that installs the software. |
| Uninstall | The action that un-installs the software. |
| Activate | The action that activates software previously installed. |
| Deactivate | The action that deactivates software. |

##### 6.6.2.2.24	Status Enum

Table 6.6.2.2.24-1: Enum: Status

| Status | Description |
| --- | --- |
| Successful | The status is successful. |
| Failure | The status is failure. |
| In-Process | The status is in process. |

##### 6.6.2.2.25	ActionStatus Type

The ActionStatus is a complex type that describes the action and related status.

Table 6.6.2.2.25-1: Type: ActionStatus

| Parameter | Optional | Description |
| --- | --- | --- |
| action | NO | The action being performed Enum Action, see 6.6.2.2.24. |
| status | NO | Indicates the status of the operation is successful, failure or in process. Enum Status, see 6.6.2.2.25. |

##### 6.6.2.2.26	SoftwareReport Type

The SoftwareReport is complex type that defines the current state of an application software management operation for a specific device.

Table 6.6.2.2.26-1: Type: SoftwareReport

| Parameter | Optional | Description |
| --- | --- | --- |
| groupId | YES | The group identifier. |
| deviceId | NO | The unique device identifier in the context of the M2M Service Subscription. |
| operation | YES | The application software management operation.(e.g. downloadSoftware, installSoftware, activateSoftware, deactivateSoftware, removeSoftware) |
| operationResult | YES | The application software management operation execution result. |
| installStatus | YES | Indicates the status of the install. Enum ActionStatus, see 6.6.2.2.26. |
| activeStatus | YES | The status of active or deactivate action. Enum ActionStatus, see 6.6.2.2.26. |

##### 6.6.2.2.27	PendingReport Enum

Table 6.6.2.2.27-1: Enum: PendingReport

| PendingReport | Description |
| --- | --- |
| SendLatest | Send the latest report. |
| SendAllPending | Send all pending report. |

##### 6.6.2.2.28	OperationMonitor Enum

Table 6.6.2.2.28-1: Enum: OperationMonitor

| Monitored Operation | Description |
| --- | --- |
| DownloadFirmware | The downloadFirmware operation. |
| InstallFirmware | The installFirmware operation. |
| RebootDevice | The rebootDevice operation. |
| ResetDevice | The resetDevice operation. |
| UploadDeviceLog | The uploadDeviceLog operation. |
| DownloadSoftware | The downloadSoftware operation. |
| InstallSoftware | The installSoftware operation. |
| ActiveSoftware | The activeSoftware operation. |
| DeactivateSoftware | The deactivateSoftware operation. |
| RemoveSoftware | The removeSoftware operation. |

### 6.6.3	Service Capabilities

#### 6.6.3.1	downloadFirmware

##### 6.6.3.1.1	Description

This service capability provides the ability for AEs to download device firmware.

##### 6.6.3.1.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the service capability and device exist.

##### 6.6.3.1.3	Signature - downloadFirmware

Table 6.6.3.1.3-1: Device Management Service –downloadFirmware capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| firmwareInfo | IN | NO | The device firmware information. Type FirmwareInfo, see 6.6.2.2.5 |
| deviceId | IN | YES | The unique device identifier in the context of the M2M Service Subscription. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. Type ReportPolicy, see 6.6.2.2.1. |
| requestId | OUT | NO | The M2M Request Identifier (M2M-Request-ID) |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Issuer does not have a Management Adapter for the requested operation Report policy does not supported by the Management Adapter Firmware download failure,e.g. firmware miss dependency, firmware is too large for available program or data memory |

##### 6.6.3.1.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.6.3.1.4-1: downloadFirmware Diagram

##### 6.6.3.1.5	Post-Conditions

The Management Adapter has submitted a request to the Management Server to download firmware.

##### 6.6.3.1.6	Exceptions

Not Applicable.

##### 6.6.3.1.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.3.1.8	oneM2M Resource Interworking

This service capability is used to download firmware. The service capability aligns with the <firmware> resource and maps to the UPDATE procedure for the attribute update of the resource. The service capability also aligns with the <mgmtCmd> resource and maps to the EXECUTE procedure for the resource.

#### 6.6.3.2	installFirmware

##### 6.6.3.2.1	Description

This service capability provides the ability for AEs to install firmware on the device.

##### 6.6.3.2.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the service capability and device exist.

The firmware to be installed exists on the device.

##### 6.6.3.2.3	Signature - installFirmware

Table 6.6.3.2.3-1: Device Management Service –installFirmware capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| firmwareInfo | IN | NO | The device firmware information. Type FirmwareInfo, see 6.6.2.2.5 |
| deviceId | IN | YES | The unique device identifier in the context of the M2M Service Subscription. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. Type ReportPolicy, see 6.6.2.2.1. |
| requestId | OUT | NO | The M2M Request Identifier (M2M-Request-ID) |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Issuer does not have a Management Adapter for the requested operation Report policy does not supported by the Management Adapter Firmware install failure, e.g. firmware is missing dependency on the device, firmware is too large for available program or data memory |

##### 6.6.3.2.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.6.3.2.4-1: installFirmware Diagram

##### 6.6.3.2.5	Post-Conditions

The Management Adapter has submitted a request to the Management Server to install firmware.

##### 6.6.3.2.6	Exceptions

Not Applicable.

##### 6.6.3.2.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.3.2.8	oneM2M Resource Interworking

This service capability is used to install firmware. The service capability aligns with the <firmware> resource and maps to the UPDATE procedure for the attribute update of the resource. The service capability also aligns with the <mgmtCmd> resource and maps to the EXECUTE procedure for the resource.

#### 6.6.3.3	getFirmwareInformation

##### 6.6.3.3.1	Description

This service capability provides the ability for AEs to get device firmware information.

##### 6.6.3.3.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the service capability and device exist.

##### 6.6.3.3.3	Signature - getFirmwareInformation

Table 6.6.3.3.3-1: Device Management Service –getFirmwareInformation capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | YES | The unique device identifier in the context of the M2M Service Subscription. |
| requestId | OUT | NO | The M2M Request Identifier (M2M-Request-ID) |
| firmwareInfo | OUT | YES | The device firmware information. Type FirmwareInfo, see 6.6.2.2.5 |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Issuer does not have a Management Adapter for the requested operation |

##### 6.6.3.3.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.6.3.3.4-1: getFirmwareInformation Diagram

##### 6.6.3.3.5	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get firmware information.

##### 6.6.3.3.6	Exceptions

Not Applicable.

##### 6.6.3.3.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.3.3.8	oneM2M Resource Interworking

This service capability is used to get firmware information. The service capability aligns with the <firmware> resource and maps to the RETRIEVE procedure for the attribute version, name, URL of the resource.

#### 6.6.3.4	getFirmwareExecStatus

##### 6.6.3.4.1	Description

This service capability provides the ability for AEs to get state of firmware management operation for a specific device.

##### 6.6.3.4.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and previously submitted firmware request exist.

##### 6.6.3.4.3	Signature – getFirmwareExecStatus

Table 6.6.3.4.3-1: Device Management Service –getFirmwareExecStatus capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| operationRequestId | IN | NO | The M2M Request Identifier of previously submitted firmware request (M2M-Request-ID) |
| deviceId | IN | YES | The unique device identifier in the context of the M2M Service Subscription. |
| requestId | OUT | NO | The M2M Request Identifier (M2M-Request-ID) |
| firmwareReport | OUT | YES | The firmware management operation execution result or status. Type FirmwareReport, see 6.6.2.2.3. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Issuer does not have a Management Adapter for the requested operation The previously submitted firmware request does not exist |

##### 6.6.3.4.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.6.3.4.4-1: getFirmwareExecStatus Diagram

##### 6.6.3.4.5	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get firmware state of management operation.

##### 6.6.3.4.6	Exceptions

Not Applicable.

##### 6.6.3.4.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.3.4.8	oneM2M Resource Interworking

This service capability is used to get state of firmware management operation. The service capability aligns with the <firmware> resource and maps to the RETRIEVE procedure for the attribute updateStatus of the resource. The service capability also aligns with the <execInstance> resource and maps to the RETRIEVE procedure for the attribute execStatus, execResult of the resource.

#### 6.6.3.5	deviceManagementReport

##### 6.6.3.5.1	Description

This service capability provides the ability to report the device management operation execution result or status information to the AE.

##### 6.6.3.5.2	Pre-Conditions

A correlation between a Management Adapter, the M2M Service Capability and previously submitted request exist.

##### 6.6.3.5.3	Signature – deviceManagementReport

Table 6.6.3.5.3-1: Device Management Service –deviceManagementReport capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| isLastReport | IN | NO | Boolean, whether it is the last report. |
| sequenceNumber | IN | NO | The report sequence number. |
| firmwareReportList | IN | YES | Array of firmwareReport. Type FirmwareReport, see 6.6.2.2.3. |
| troubleshootingReportList | IN | YES | Array of troubleshootingReport. Type TroubleshootingReport, see 6.6.2.2.20. |
| softwareReportList | IN | YES | Array of softwareReport. Type SoftwareReport, see 6.6.2.2.27. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. The target AE does not exist. |

##### 6.6.3.5.4	Post-Conditions

The AE has received a report of firmware operation execution status or result. The M2M System shall maintain the state of the report (i.e. sequence number) according to the report policy.

##### 6.6.3.5.5	Exceptions

Not Applicable.

##### 6.6.3.5.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.3.57	oneM2M Resource Interworking

Not Applicable.

#### 6.6.3.6	upgradeFirmware

##### 6.6.3.6.1	Description

This service capability is a complex operation for AEs to upgrade firmware on one or more devices. The service capability utilizes orchestration rules for the related management operations such as downloadFirmware, installFirmware, getFirmwareInformation, getFirmwareExecInstance and reportFirmwareStatus according to the business process.

This service capability permits AEs to upgrade the firmware on individual device, multiple devices or a group of devices. In addition the upgrade of the firmware is permitted based on a schedule for each of the operations.

##### 6.6.3.6.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between Management Adapter, the service capability and devices or device group exist.

##### 6.6.3.6.3	Signature - upgradeFirmware

Table 6.6.3.6.3-1: Device Management Service –upgradeFirmware capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| orchestrationRuleList | IN | NO | List of OrchestrationRule. Type OrchestrationRule, see clause 6.6.2.2.4. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. Type ReportPolicy, see 6.6.2.2.1. |
| requestId | OUT | NO | The M2M Request Identifier (M2M-Request-ID) |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Report policy is not supported by the Management Adapter The Orchestration Rules are not supported by the Management Adapter Operation exception: The device's reachability schedule is inconsistent with the schedule of the Orchestration rule. |

##### 6.6.3.6.4	Service Interactions

The interactions of service capabilities required for this service capability:

- 1)	Issue the request to the Supporting Service to perform the operation

Figure 6.6.3.6.4-1: upgradeFirmware Diagram

##### 6.6.3.6.5	Post-Conditions

The Management Adapter has submitted a set of requests to the Management Server to manage firmware.

##### 6.6.3.6.6	Exceptions

Not Applicable.

##### 6.6.3.6.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.3.6.8	oneM2M Resource Interworking

Not Applicable.

#### 6.6.3.7	getDeviceInformation

##### 6.6.3.7.1	Description

This service capability provides the ability for AEs to get device information.

##### 6.6.3.7.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

##### 6.6.3.7.3	Signature - getDeviceInformation

Table 6.6.3.7.3-1: Device Management Service – getDeviceInformation capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| deviceInfo | OUT | NO | The device information. Type DeviceInfo, see 6.6.2.2.7. |
| memory | OUT | YES | The memory information. Type Memory, see 6.6.2.2.8. |
| battery | OUT | YES | The battery information. Type Battery, see 6.6.2.2.9. |
| lockStatus | OUT | YES | The device lock status. Enum LockStatus, see 6.6.2.2.13. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Issuer does not have a Management Adapter for the requested operation |

##### 6.6.3.7.4	Service Interactions

The interactions of service capabilities required for this service capability:

- 1)	Issue the request to the Supporting Service to perform the operation

Figure 6.6.3.7.4-1: getDeviceInformation Diagram

##### 6.6.3.7.5	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get device information.

Based on the capabilities supported by the device, the memory, battery and lock status information may or may not be returned.

##### 6.6.3.7.6	Exceptions

Not Applicable.

##### 6.6.3.7.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.3.7.8	oneM2M Resource Interworking

This service capability is used to get device information. The service capability aligns with the <deviceInfo> resource and maps to the Retrieve procedure for the resource. The service capability aligns with the <memory> resource and maps to the Retrieve procedure for the resource. The service capability aligns with the <battery> resource and maps to the Retrieve procedure for the resource.

#### 6.6.3.8	getDeviceCapabilities

##### 6.6.3.8.1	Description

This service capability provides the ability for AEs to get the device capabilities.

##### 6.6.3.8.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

##### 6.6.3.8.3	Signature – getDeviceCapabilities

Table 6.6.3.8.3-1: Device Management Service – getDeviceCapabilities capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| deviceCapabilities | OUT | YES | List of deviceCapability. Type DeviceCapability, see 6.6.2.2.10. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Issuer does not have a Management Adapter for the requested operation |

##### 6.6.3.8.4	Service Interactions

The interactions of service capabilities required for this service capability:

- 1)	Issue the request to the Supporting Service to perform the operation

Figure 6.6.3.8.4-1: getDeviceCapabilities Diagram

##### 6.6.3.8.5	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get device capabilities.

##### 6.6.3.8.6	Exceptions

Not Applicable.

##### 6.6.3.8.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.3.8.8	oneM2M Resource Interworking

This service capability is used to get device capabilities. The service capability aligns with the<deviceCapability> resource and maps to the RETRIEVE procedure for the resource.

#### 6.6.3.9 	enableDeviceCapability

##### 6.6.3.9.1	Description

This service capability provides the ability for AEs to enable device capability.

##### 6.6.3.9.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

##### 6.6.3.9.3	Signature –enableDeviceCapability

Table 6.6.3.9.3-1: Device Management Service –enableDeviceCapability capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| Name | IN | NO | The name of the capability. Enum DeviceCapabilityName, see 6.6.2.2.12. |
| state | OUT | NO | Indicates if the capability is enabled or disabled. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Issuer does not have a Management Adapter for the requested operation |

##### 6.6.3.9.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.6.3.9.4-1: enableDeviceCapability Diagram

##### 6.6.3.9.5	Post-Conditions

The Management Adapter has submitted a request to the Management Server to enable device capability.

##### 6.6.3.9.6	Exceptions

Not Applicable.

##### 6.6.3.9.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.3.9.8	oneM2M Resource Interworking

This service capability is used to enable device capability. The service capability aligns with the <deviceCapability> resource and maps to the UPDATE procedure for the attribute enable of the resource.

#### 6.6.3.10	disableDeviceCapability

##### 6.6.3.10.1	Description

This service capability provides the ability for AEs to disable device capability.

##### 6.6.3.10.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the service capability and device exist.

##### 6.6.3.10.3	Signature –disableDeviceCapability

Table 6.6.3.10.3-1: Device Management Service –disableDeviceCapability capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| name | IN | NO | The name of the capability. Enum DeviceCapabilityName, see 6.6.2.2.12. |
| state | OUT | NO | Indicates if the capability is enabled or disabled. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Issuer does not have a Management Adapter for the requested operation |

##### 6.6.3.10.4	Service Interactions

The interactions of service capabilities required for this service capability:

- 1)	Issue the request to the Supporting Service to perform the operation

Figure 6.6.3.10.4-1: disableDeviceCapability Diagram

##### 6.6.3.10.5	Post-Conditions

The Management Adapter has submitted a request to the Management Server to disable device capability.

##### 6.6.3.10.6	Exceptions

Not Applicable.

##### 6.6.3.10.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.3.10.8	oneM2M Resource Interworking

This service capability is used to disable device capability. The service capability aligns with the <deviceCapability> resource and maps to the UPDATE procedure for the attribute disable of the resource.

#### 6.6.3.11	getAreaNetworks

##### 6.6.3.11.1	Description

This service capability provides the ability for AEs to get area networks information.

##### 6.6.3.11.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and area network exist.

##### 6.6.3.11.3	Signature - getAreaNetworks

Table 6.6.3.11.3-1: Device Management Service –getAreaNetworks capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| areaNwks | OUT | YES | Array of area network. Type AreaNwkInfo, see 6.6.2.2.14. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Issuer does not have a Management Adapter for the requested operation |

##### 6.6.3.11.4	Service Interactions

The interactions of service capabilities required for this service capability:

- 1)	Issue the request to the Supporting Service to perform the operation

Figure 6.6.3.11.4-1: getAreaNetworks Diagram

##### 6.6.3.11.5	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get area networks information.

##### 6.6.3.11.6	Exceptions

Not Applicable.

##### 6.6.3.11.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.3.11.8	oneM2M Resource Interworking

This service capability is used to get area networks information. The service capability aligns with the <areaNwkInfo> resource and maps to the RETRIEVE procedure for the resource. The service capability aligns with the <areaNwkDeviceInfo> resource and maps to the Retrieve procedure for the resource.

#### 6.6.3.12	updateDeviceForAreaNetwork

##### 6.6.3.12.1	Description

This service capability provides the ability for AEs to update device information for area network.

##### 6.6.3.12.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability, the device and area network exist.

##### 6.6.3.12.3	Signature – updateDeviceForAreaNetwork

Table 6.6.3.12.3-1: Device Management Service – updateDeviceForAreaNetwork capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| areaNwkId | IN | NO | The unique area network identifier. |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| areaNwkDeviceInfo | IN | YES | The existing areaNwkDeviceInfo are replaced with the information in this parameter. Type AreaNwkDeviceInfo, see 6.6.2.2.16. |
| lastModifiedTime | OUT | NO | The modified time. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Issuer does not have a Management Adapter for the requested operation |

##### 6.6.3.12.4	Service Interactions

The interactions of service capabilities required for this service capability:

- 1)	Issue the request to the Supporting Service to perform the operation

Figure 6.6.3.12.4-1: updateDeviceForAreaNetwork Diagram

##### 6.6.3.12.5	Post-Conditions

The Management Adapter has submitted a request to the Management Server to update device information for area network.

##### 6.6.3.12.6	Exceptions

Not Applicable.

##### 6.6.3.12.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.3.12.8	oneM2M Resource Interworking

The service capability aligns with the <areaNwkDeviceInfo> resource and maps to the Update procedure for attribute devType, sleepInterval, sleepDuration, status, listOfNeighbors of the resource.

#### 6.6.3.13	rebootDevice

##### 6.6.3.13.1	Description

This service capability provides the ability for AEs to reboot device.

##### 6.6.3.13.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

##### 6.6.3.13.3	Signature - rebootDevice

Table 6.6.3.13.3-1: Device Management Service –rebootDevice capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. Type ReportPolicy, see 6.6.2.2.1. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Issuer does not have a Management Adapter for the requested operation report policy does not supported by the Management Adapter |

##### 6.6.3.13.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.6.3.13.4-1: rebootDevice Diagram

##### 6.6.3.13.5	Post-Conditions

The Management Adapter has submitted a request to the Management Server to reboot device.

##### 6.6.3.13.6	Exceptions

Not Applicable.

##### 6.6.3.13.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.3.13.8	oneM2M Resource Interworking

This service capability is used to reboot device. The service capability aligns with the <reboot> resource and maps to the Execute procedure for the attribute reboot of the resource.

#### 6.6.3.14	resetDevice

##### 6.6.3.14.1	Description

This service capability provides the ability for AEs to reset device.

##### 6.6.3.14.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

##### 6.6.3.14.3	Signature - resetDevice

Table 6.6.3.14.3-1: Device Management Service –resetDevice capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. Type ReportPolicy, see 6.6.2.2.1. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Issuer does not have a Management Adapter for the requested operation report policy does not supported by the Management Adapter |

##### 6.6.3.14.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.6.3.14.4-1: resetDevice Diagram

##### 6.6.3.14.5	Post-Conditions

The Management Adapter has submitted a request to the Management Server to reset device.

##### 6.6.3.14.6	Exceptions

Not Applicable.

##### 6.6.3.14.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.3.14.8	oneM2M Resource Interworking

This service capability is used to reset device. The service capability aligns with the <reboot> resource and maps to the Execute procedure for the attribute factoryReset of the resource.

#### 6.6.3.15	uploadDeviceLog

##### 6.6.3.15.1	Description

This service capability provides the ability for AEs to upload device log.

##### 6.6.3.15.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

##### 6.6.3.15.3	Signature – uploadDeviceLog

Table 6.6.3.15.3-1: Device Management Service – uploadDeviceLog capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| logInfo | IN | NO | The log information. Type LogInfo, see 6.6.2.2.19. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.2.2.1. |
| logURL | OUT | YES | The URL from which the log can be uploaded. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Issuer does not have a Management Adapter for the requested operation report policy does not supported by the Management Adapter |

##### 6.6.3.15.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.6.3.15.4-1: uploadDeviceLog Diagram

##### 6.6.3.15.5	Post-Conditions

The Management Adapter has submitted a request to the Management Server to upload device log.

##### 6.6.3.15.6	Exceptions

Not Applicable.

##### 6.6.3.15.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.3.15.8	oneM2M Resource Interworking

This service capability is used to upload device log. The service capability aligns with the <eventLog> resource and maps to the Update procedure for the resource.

#### 6.6.3.16	getDeviceLogs

##### 6.6.3.16.1	Description

This service capability provides the ability for AEs to get all logs of a device.

##### 6.6.3.16.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

##### 6.6.3.16.3	Signature - getDeviceLogs

Table 6.6.3.16.3-1: Device Management Service – getDeviceLogs capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| logList | OUT | YES | Array of log. Type Log, see 6.6.2.2.21. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Issuer does not have a Management Adapter for the requested operation |

##### 6.6.3.16.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.6.3.16.4-1: getDeviceLogs Diagram

##### 6.6.3.16.5	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get all logs of a device.

##### 6.6.3.16.6	Exceptions

Not Applicable.

##### 6.6.3.16.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.3.16.8	oneM2M Resource Interworking

This service capability is used to get all logs of a device. The service capability aligns with the <eventLog> resource and maps to the Retrieve procedure for the resource.

#### 6.6.3.17	getDeviceLogInformation

##### 6.6.3.17.1	Description

This service capability provides the ability for AEs to get a device log information.

##### 6.6.3.17.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

##### 6.6.3.17.3	Signature – getDeviceLogInformation

Table 6.6.3.17.3-1: Device Management Service – getDeviceLogInformation capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| filterCriteria | IN | YES | See Table 6.6.2.2.22 |
| logURL | IN | YES | The URL from which the log can be accessed. |
| log | OUT | YES | Type Log, see 6.6.2.2.21. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Issuer does not have a Management Adapter for the requested operation |

##### 6.6.3.17.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.6.3.17.4-1: getDeviceLogInformation Diagram

##### 6.6.3.17.5	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get a device log information.

##### 6.6.3.17.6	Exceptions

Not Applicable.

##### 6.6.3.17.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.3.17.8	oneM2M Resource Interworking

This service capability is used to a device log information. The service capability aligns with the <eventLog> resource and maps to the Retrieve procedure for the resource.

#### 6.6.3.18	getSoftwareInformation

##### 6.6.3.18.1	Description

This service capability provides the ability for AEs to get application software information.

##### 6.6.3.18.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

##### 6.6.3.18.3	Signature – getSoftwareInformation

Table 6.6.3.18.3-1: Device Management Service –getSoftwareInformation capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| version | OUT | YES | The version of the software. |
| name | OUT | YES | The name of the software. |
| URL | OUT | YES | The URL from which the software package can be downloaded. |
| installStatus | OUT | YES | Indicates the status of the install. Enum ActionStatus, see 6.6.2.2.26. |
| activeStatus | OUT | YES | The status of active or deactivate action. Enum ActionStatus, see 6.6.2.2.26. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Issuer does not have a Management Adapter for the requested operation |

##### 6.6.3.18.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.6.3.18.4-1: getSoftwareInformation Diagram

##### 6.6.3.18.5	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get application software information.

##### 6.6.3.18.6	Exceptions

Not Applicable.

##### 6.6.3.18.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.3.18.8	oneM2M Resource Interworking

This service capability is used to get application software information. The service capability aligns with the <software> resource and maps to the Retrieve procedure for the resource.

#### 6.6.3.19	downloadSoftware

##### 6.6.3.19.1	Description

This service capability provides the ability for AEs to download application software.

##### 6.6.3.19.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

##### 6.6.3.19.3	Signature –downloadSoftware

Table 6.6.3.19.3-1: Device Management Service –downloadSoftware capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| version | IN | NO | The version of the software. |
| name | IN | NO | The name of the software. |
| URL | IN | NO | The URL from which the software package can be downloaded. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.2.2.1. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Issuer does not have a Management Adapter for the requested operation report policy does not supported by the Management Adapter |

##### 6.6.3.19.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.6.3.19.4-1: downloadSoftware Diagram

##### 6.6.3.19.5	Post-Conditions

The Management Adapter has submitted a request to the Management Server to download application software.

##### 6.6.3.19.6	Exceptions

Not Applicable.

##### 6.6.3.19.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.3.19.8	oneM2M Resource Interworking

This service capability is used to download application software. The service capability aligns with the <software> resource and maps to the UPDATE procedure for the attribute install of the resource. The service capability also aligns with the <mgmtCmd> resource and maps to the EXECUTE procedure for the resource.

#### 6.6.3.20	installSoftware

##### 6.6.3.20.1	Description

This service capability provides the ability for AEs to install application software.

##### 6.6.3.20.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

##### 6.6.3.20.3	Signature –installSoftware

Table 6.6.3.20.3-1: Device Management Service –installSoftware capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| version | IN | NO | The version of the software. |
| name | IN | NO | The name of the software. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.2.2.1. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Issuer does not have a Management Adapter for the requested operation report policy does not supported by the Management Adapter |

##### 6.6.3.20.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.6.3.20.4-1: installSoftware Diagram

##### 6.6.3.20.5	Post-Conditions

The Management Adapter has submitted a request to the Management Server to install application software.

##### 6.6.3.20.6	Exceptions

Not Applicable.

##### 6.6.3.20.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.3.20.8	oneM2M Resource Interworking

This service capability is used to install application software. The service capability aligns with the <software> resource and maps to the UPDATE procedure for the attribute install of the resource. The service capability also aligns with the <mgmtCmd> resource and maps to the EXECUTE procedure for the resource.

#### 6.6.3.21	activateSoftware

##### 6.6.3.21.1	Description

This service capability provides the ability for AEs to activate software previously installed.

##### 6.6.3.21.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

##### 6.6.3.21.3	Signature –activateSoftware

Table 6.6.3.21.3-1: activateSoftware capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| version | IN | NO | The version of the software. |
| name | IN | NO | The name of the software. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.2.2.1. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Issuer does not have a Management Adapter for the requested operation report policy does not supported by the Management Adapter |

##### 6.6.3.21.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.6.3.21.4-1: activateSoftware Diagram

##### 6.6.3.21.5	Post-Conditions

The Management Adapter has submitted a request to the Management Server to activate software previously installed.

##### 6.6.3.21.6	Exceptions

Not Applicable.

##### 6.6.3.21.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.3.21.8	oneM2M Resource Interworking

This service capability is used to activate software previously installed. The service capability aligns with the <software> resource and maps to the UPDATE procedure for the attribute activate of the resource. The service capability also aligns with the <mgmtCmd> resource and maps to the EXECUTE procedure for the resource.

#### 6.6.3.22	deactivateSoftware

##### 6.6.3.22.1	Description

This service capability provides the ability for AEs to deactivates software.

##### 6.6.3.22.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and the device exist.

##### 6.6.3.22.3	Signature –deactivateSoftware

Table 6.6.3.22.3-1: deactivateSoftware capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| version | IN | NO | The version of the software. |
| name | IN | NO | The name of the software. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.2.2.1. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Issuer does not have a Management Adapter for the requested operation report policy does not supported by the Management Adapter |

##### 6.6.3.22.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.6.3.22.4-1: deactivateSoftware Diagram

##### 6.6.3.22.5	Post-Conditions

The Management Adapter has submitted a request to the Management Server to deactivates software.

##### 6.6.3.22.6	Exceptions

Not Applicable.

##### 6.6.3.22.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.3.22.8	oneM2M Resource Interworking

This service capability is used to deactivates software. The service capability aligns with the <software> resource and maps to the UPDATE procedure for the attribute deactivate of the resource. The service capability also aligns with the <mgmtCmd> resource and maps to the EXECUTE procedure for the resource.

#### 6.6.3.23	removeSoftware

##### 6.6.3.23.1	Description

This service capability provides the ability for AEs to uninstall the software.

##### 6.6.3.23.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and the device exist.

##### 6.6.3.23.3	Signature – removeSoftware

Table 6.6.3.23.3-1: Device Management Service –removeSoftware capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| version | IN | NO | The version of the software. |
| name | IN | NO | The name of the software. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.2.2.1. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Issuer does not have a Management Adapter for the requested operation report policy does not supported by the Management Adapter |

##### 6.6.3.23.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.6.3.23.4-1: removeSoftware Diagram

##### 6.6.3.23.5	Post-Conditions

The Management Adapter has submitted a set of requests to the Management Server to uninstall the software.

##### 6.6.3.23.6	Exceptions

Not Applicable.

##### 6.6.3.23.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.6.3.23.8	oneM2M Resource Interworking

This service capability is used to install application software. The service capability aligns with the <software> resource and maps to the UPDATE procedure for the attribute uninstall of the resource. The service capability also aligns with the <mgmtCmd> resource and maps to the EXECUTE procedure for the resource.

## 6.7	Management Adapter

### 6.7.1	Overview

The Management Adapter service provides the ability to adapt the M2M Service Layer operations to the technology specific operations of the Management Server.

### 6.7.2	Service Capabilities

#### 6.7.2.1	downloadFirmware

##### 6.7.2.1.1	Description

This service capability provides the ability to download the specific device firmware.

##### 6.7.2.1.2	Pre-Conditions

A correlation between a Management Server, the service capability and device exist.

##### 6.7.2.1.3	Signature - downloadFirmware

Table 6.7.2.1.3-1: Management Adapter –downloadFirmware capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| firmwareInfo | IN | NO | The device firmware information. Type FirmwareInfo, see 6.6.2.2.5 |
| deviceId | IN | YES | The unique device identifier in the context of the M2M Service Subscription. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.2.2.1. |
| requestId | IN | NO | The M2M Request Identifier (M2M-Request-ID) |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Report policy is not supported by the Management Adapter Firmware download failure, e.g. firmware is missing dependency on the device, firmware is too large for available program or data memory |

##### 6.7.2.1.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to download the firmware.

##### 6.7.2.1.5	Exceptions

Not Applicable.

##### 6.7.2.1.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.1.7	oneM2M Resource Interworking

This service capability is used to download firmware. The service capability aligns with the <firmware> resource and maps to the UPDATE procedure for the attribute update of the resource. The service capability also aligns with the <mgmtCmd> resource and maps to the EXECUTE procedure for the resource.

#### 6.7.2.2	installFirmware

##### 6.7.2.2.1	Description

This service capability provides the ability to install firmware on a device.

##### 6.7.2.2.2	Pre-Conditions

A correlation between a Management Server, the M2M Service Capability and device exist.

##### 6.7.2.2.3	Signature –installFirmware

Table 6.7.2.2.3-1: Management Adapter –installFirmware capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| firmwareInfo | IN | NO | The device firmware information. Type FirmwareInfo, see 6.6.2.2.5 |
| deviceId | IN | YES | The unique device identifier in the context of the M2M Service Subscription. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. |
| requestId | IN | NO | The M2M Request Identifier (M2M-Request-ID) |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Report policy does not supported by the Management Adapter Firmware install failure, e.g. firmware is missing dependency on the device, firmware is too large for available program or data memory |

##### 6.7.2.2.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to install firmware.

##### 6.7.2.2.5	Exceptions

Not Applicable.

##### 6.7.2.2.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.2.7	oneM2M Resource Interworking

This service capability is used to install firmware. The service capability aligns with the <firmware> resource and maps to the UPDATE procedure for the attribute update of the resource. The service capability also aligns with the <mgmtCmd> resource and maps to the EXECUTE procedure for the resource.

#### 6.7.2.3	getFirmwareInformation

##### 6.7.2.3.1	Description

This service capability provides the ability to retrieve the information related to the firmware on a device.

##### 6.7.2.3.2	Pre-Conditions

A correlation between a Management Server, the M2M Service Capability and device exist.

##### 6.7.2.3.3	Signature –getFirmwareInformation

Table 6.7.2.3.3-1: Management Adapter –getFirmwareInformation capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | YES | The unique device identifier in the context of the M2M Service Subscription. |
| requestId | IN | NO | The M2M Request Identifier (M2M-Request-ID) |
| firmwareInfo | OUT | YES | The device firmware information. Type FirmwareInfo, see 6.6.2.2.5 |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Exception: Request may not have been completed |

##### 6.7.2.3.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get the firmware information.

##### 6.7.2.3.5	Exceptions

Not Applicable.

##### 6.7.2.3.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.3.7	oneM2M Resource Interworking

This service capability is used to get firmware information. The service capability aligns with the <firmware> resource and maps to the RETRIEVE procedure for the attribute version, name, URL of the resource.

#### 6.7.2.4	getFirmwareExecStatus

##### 6.7.2.4.1	Pre-Conditions

This service capability provides the ability to get state of firmware management operation for a specific device.

##### 6.7.2.4.2	Pre-Conditions

A correlation between a Management Server, the M2M Service Capability and previously submitted firmware request exist.

##### 6.7.2.4.3	Signature –getFirmwareExecInstance

Table 6.7.2.4.3-1: Management Adapter –getFirmwareExecStatus capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| operationRequestId | IN | NO | The M2M Request Identifier of previously submitted firmware request (M2M-Request-ID) |
| deviceId | IN | YES | The unique device identifier in the context of the M2M Service Subscription. When provided the FirmwareReport type is generated for only the specified device. |
| firmwareReport | OUT | YES | The firmware management operation execution result or status. Type FirmwareReport, see 6.6.2.2.3. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. The previously submitted firmware request does not exist |

##### 6.7.2.4.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get the firmware operation execution status or result for each device in the operation request.

##### 6.7.2.4.5	Exceptions

Not Applicable.

##### 6.7.2.4.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.4.7	oneM2M Resource Interworking

This service capability is used to get state of firmware management operation. The service capability aligns with the <firmware> resource and maps to the RETRIEVE procedure for the attribute updateStatus of the resource. The service capability also aligns with the <execInstance> resource and maps to the RETRIEVE procedure for the attribute execStatus, execResult of the resource.

#### 6.7.2.5	deviceManagementReport

##### 6.7.2.5.1	Description

This service capability provides the ability for the Management Adapter to report the device management operation execution result or status information. The Management Adapter can send one or multiple reports according to the report policy.

##### 6.7.2.5.2	Pre-Conditions

A correlation between a Management Server, the M2M Service Capability and previously submitted requests exist.

##### 6.7.2.5.3	Signature –deviceManagementReport

Table 6.7.2.5.3-1: Management Adapter –deviceManagementReport capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| to | IN | NO | The identifier of the AE that is to receive the data(M2M-AppInst-ID) |
| isLastReport | IN | NO | Boolean, whether it is the last report. |
| sequenceNumber | IN | NO | The report sequence number. |
| firmwareReportList | IN | YES | Array of firmwareReport. Type FirmwareReport, see 6.6.2.2.3. |
| troubleshootingReportList | IN | YES | Array of troubleshootingReport. Type TroubleshootingReport, see 6.6.2.2.20. |
| softwareReportList | IN | YES | Array of softwareReport. Type SoftwareReport, see 6.6.2.2.27. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. The target AE does not exist. |

##### 6.7.2.5.4	Service Interactions

The interactions of service capabilities required for this service capability:

- 1)	Issue the request to the Supporting Service to perform the operation

Figure 6.7.2.5.4-1: deviceManagementReport Diagram

##### 6.7.2.5.5	Post-Conditions

The M2M System maintains the state of the report (i.e. sequence number) according to the report policy.

##### 6.7.2.5.6	Exceptions

Not Applicable.

##### 6.7.2.5.7	Policies for Use

Message Exchange Patterns: Not Applicable

Transaction Pattern: Not Applicable

##### 6.7.2.5.7	oneM2M Resource Interworking

Not Applicable.

#### 6.7.2.6	getDeviceInformation

##### 6.7.2.6.1	Description

This service capability provides the ability to get the specific device information.

##### 6.7.2.6.2	Pre-Conditions

A correlation between a Management Server, the M2M Service Capability and device exist.

##### 6.7.2.6.3	Signature –getDeviceInformation

Table 6.7.2.6.3-1: Management Adapter –getDeviceInformation capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| deviceInfo | OUT | YES | The device information. Type DeviceInfo, see 6.6.2.2.7. |
| memory | OUT | YES | The memory information. Type Memory, see 6.6.2.2.8. |
| battery | OUT | YES | The battery information. Type Battery, see 6.6.2.2.9. |
| lockStatus | OUT | YES | The device lock status. Enum LockStatus, see 6.6.2.2.13. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. - Exception: Request may not have been completed |

##### 6.7.2.6.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get the device information.

##### 6.7.2.6.5	Exceptions

Not Applicable.

##### 6.7.2.6.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.6.7	oneM2M Resource Interworking

This service capability is used to get device information. The service capability aligns with the <deviceInfo> resource and maps to the RETRIEVE procedure for the resource. The service capability aligns with the <memory> resource and maps to the Retrieve procedure for the resource. The service capability aligns with the <battery> resource and maps to the Retrieve procedure for the resource.

#### 6.7.2.7	getDeviceCapabilities

##### 6.7.2.7.1	Description

This service capability provides the ability to get device capabilities.

##### 6.7.2.7.2	Pre-Conditions

A correlation between a Management Server, the M2M Service Capability and device exist.

##### 6.7.2.7.3	Signature –getDeviceCapabilities

Table 6.7.2.7.3-1: Management Adapter –getDeviceCapabilities capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| deviceCapabilities | OUT | YES | Array of deviceCapability. Type DeviceCapability, see 6.6.2.2.10. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Exception: Request may not have been completed |

##### 6.7.2.7.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get device capabilities.

##### 6.7.2.7.5	Exceptions

Not Applicable.

##### 6.7.2.7.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.7.7	oneM2M Resource Interworking

This service capability is used to get device capabilities. The service capability aligns with the<deviceCapability> resource and maps to the RETRIEVE procedure for the resource.

#### 6.7.2.8	enableDeviceCapability

##### 6.7.2.8.1	Description

This service capability provides the ability to enable device capability.

##### 6.7.2.8.2	Pre-Conditions

A correlation between a Management Server, the M2M Service Capability and device exist.

##### 6.7.2.8.3	Signature –enableDeviceCapability

Table 6.7.2.8.3-1: Management Adapter –enableDeviceCapability capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| name | IN | NO | The name of the capability. Enum DeviceCapabilityName, see 6.6.2.2.12. |
| state | OUT | NO | Indicates if the capability is enabled or disabled. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Exception: Request may not have been completed |

##### 6.7.2.8.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to enable device capability.

##### 6.7.2.8.5	Exceptions

Not Applicable.

##### 6.7.2.8.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.8.7	oneM2M Resource Interworking

This service capability is used to enable device capability. The service capability aligns with the <deviceCapability> resource and maps to the UPDATE procedure for the attribute enable of the resource.

#### 6.7.2.9	disableDeviceCapability

##### 6.7.2.9.1	Description

This service capability provides the ability to disable device capability.

##### 6.7.2.9.2	Pre-Conditions

A correlation between a Management Server, the M2M Service Capability and device exist.

##### 6.7.2.9.3	Signature –disableDeviceCapability

Table 6.7.2.9.3-1: Management Adapter –disableDeviceCapability capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| name | IN | NO | The name of the capability. Enum DeviceCapabilityName, see 6.6.2.2.12. |
| state | OUT | NO | Indicates if the capability is enabled or disabled. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Exception: Request may not have been completed |

##### 6.7.2.9.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to disable device capability.

##### 6.7.2.9.5	Exceptions

Not Applicable.

##### 6.7.2.9.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.9.7	oneM2M Resource Interworking

This service capability is used to disable device capability. The service capability aligns with the <deviceCapability> resource and maps to the UPDATE procedure for the attribute disable of the resource.

#### 6.7.2.10	getAreaNetworks

##### 6.7.2.10.1	Description

This service capability provides the ability to get area networks information.

##### 6.7.2.10.2	Pre-Conditions

A correlation between a Management Server, the M2M Service Capability and area network exist.

##### 6.7.2.10.3	Signature –getAreaNetworks

Table 6.7.2.10.3-1: Management Adapter –getAreaNetworks capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| AreaNwks | OUT | YES | Array of area network. Type AreaNwkInfo, see 6.6.2.2.13. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Exception: Request may not have been completed |

##### 6.7.2.10.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get area networks information.

##### 6.7.2.10.5	Exceptions

Not Applicable.

##### 6.7.2.10.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.10.7	oneM2M Resource Interworking

This service capability is used to get area networks information. The service capability aligns with the <areaNwkInfo> resource and maps to the RETRIEVE procedure for the resource. The service capability aligns with the <areaNwkDeviceInfo> resource and maps to the Retrieve procedure for the resource.

#### 6.7.2.11	updateDeviceForAreaNetwork

##### 6.7.2.11.1	Description

This service capability provides the ability for the Management Adapter to update device information for area network.

##### 6.7.2.11.2	Pre-Conditions

A correlation between a Management Server, the M2M Service Capability, the device and area network exists.

##### 6.7.2.11.3	Signature –updateDeviceForAreaNetwork

Table 6.7.2.11.3-1: Management Adapter –updateDeviceForAreaNetwork capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| areaNwkId | IN | NO | The unique area network identifier. |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| areaNwkDeviceInfo | IN | YES | The existing areaNwkDeviceInfo are replaced with the information in this parameter. Type AreaNwkDeviceInfo, see 6.6.2.2.15. |
| lastModifiedTime | OUT | NO | The modified time. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Exception: Request may not have been completed |

##### 6.7.2.11.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to update device information for area network.

##### 6.7.2.11.5	Exceptions

Not Applicable.

##### 6.7.2.11.6	Policies for Use

Message Exchange Patterns: Not Applicable

Transaction Pattern: Not Applicable

##### 6.7.2.11.7	oneM2M Resource Interworking

The service capability aligns with the <areaNwkDeviceInfo> resource and maps to the Update procedure for attribute devType, sleepInterval , sleepDuration , status , listOfNeighbors of the resource.

#### 6.7.2.12	rebootDevice

##### 6.7.2.12.1	Description

This service capability provides the ability to reboot the specific device.

##### 6.7.2.12.2	Pre-Conditions

A correlation between a Management Server, the M2M Service Capability and device exist.

##### 6.7.2.12.3	Signature – rebootDevice

Table 6.7.2.12.3-1: Management Adapter – rebootDevice capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.2.2.1. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Exception: Request may not have been completed |

##### 6.7.2.12.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to reboot the device.

##### 6.7.2.12.5	Exceptions

Not Applicable.

##### 6.7.2.12.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.12.7	oneM2M Resource Interworking

This service capability is used to reboot device. The service capability aligns with the <reboot> resource and maps to the Execute procedure for the attribute reboot of the resource.

#### 6.7.2.13	resetDevice

##### 6.7.2.13.1	Description

This service capability provides the ability to reset device.

##### 6.7.2.13.2	Pre-Conditions

A correlation between a Management Server, the M2M Service Capability and device exist.

##### 6.7.2.13.3	Signature – resetDevice

Table 6.7.2.13.3-1: Management Adapter – resetDevice capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.2.2.1. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Exception: Request may not have been completed |

##### 6.7.2.13.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to reset device.

##### 6.7.2.13.5	Exceptions

Not Applicable.

##### 6.7.2.13.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.13.7	oneM2M Resource Interworking

This service capability is used to reset device. The service capability aligns with the <reboot> resource and maps to the Execute procedure for the attribute factoryReset of the resource.

#### 6.7.2.14	uploadDeviceLog

##### 6.7.2.14.1	Description

This service capability provides the ability to upload device log.

##### 6.7.2.14.2	Pre-Conditions

A correlation between a Management Server, the M2M Service Capability and device exist.

##### 6.7.2.14.3	Signature – uploadDeviceLog

Table 6.7.2.14.3-1: Management Adapter – uploadDeviceLog capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| logInfo | IN | NO | The log information. Type LogInfo, see 6.6.2.2.19. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.2.2.1. |
| logURL | OUT | YES | The URL from which the log can be uploaded. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Exception: Request may not have been completed |

##### 6.7.2.14.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to upload device log.

##### 6.7.2.14.5	Exceptions

Not Applicable.

##### 6.7.2.14.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.14.7	oneM2M Resource Interworking

This service capability is used to upload device log. The service capability aligns with the <eventLog> resource and maps to the Update procedure for the resource.

#### 6.7.2.15	getDeviceLogs

##### 6.7.2.15.1	Description

This service capability provides the ability to get all logs of a device.

##### 6.7.2.15.2	Pre-Conditions

A correlation between a Management Server, the M2M Service Capability and device exist.

##### 6.7.2.15.3	Signature –getDeviceLogs

Table 6.7.2.15.3-1: Management Adapter –getDeviceLogs capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| logList | OUT | YES | Array of log. Type Log, see 6.6.2.2.21. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Exception: Request may not have been completed |

##### 6.7.2.15.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get all logs of a device.

##### 6.7.2.15.5	Exceptions

Not Applicable.

##### 6.7.2.15.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.15.7	oneM2M Resource Interworking

This service capability is used to get all logs of a device. The service capability aligns with the <eventLog> resource and maps to the Retrieve procedure for the resource.

#### 6.7.2.16	getDeviceLogInformation

##### 6.7.2.16.1	Description

This service capability provides the ability to get a device log information.

##### 6.7.2.16.2	Pre-Conditions

A correlation between a Management Server, the M2M Service Capability and device exist.

##### 6.7.2.16.3	Signature – getDeviceLogInformation

Table 6.7.2.16.3-1: Management Adapter – getDeviceLogInformation capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| filterCriteria | IN | YES | See Table 6.6.2.2.22 |
| logURL | IN | YES | The URL from which the log can be accessed. |
| log | OUT | YES | Type Log, see 6.6.2.2.21. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Exception: Request may not have been completed |

##### 6.7.2.16.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get a device log information.

##### 6.7.2.16.5	Exceptions

Not Applicable.

##### 6.7.2.16.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.16.7	oneM2M Resource Interworking

This service capability is used to get device log information. The service capability aligns with the <eventLog> resource and maps to the Retrieve procedure for the resource.

#### 6.7.2.17	getSoftwareInformation

##### 6.7.2.17.1	Description

This service capability provides the ability to get application software information.

##### 6.7.2.17.2	Pre-Conditions

A correlation between a Management Server, the M2M Service Capability and device exist.

##### 6.7.2.17.3	Signature –getSoftwareInformation

Table 6.7.2.17.3-1: Management Adapter –getSoftwareInformation capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| version | OUT | YES | The version of the software. |
| name | OUT | YES | The name of the software. |
| url | OUT | YES | The URL from which the software package can be downloaded. |
| installStatus | OUT | YES | Indicates the status of the install. Enum ActionStatus, see 6.6.2.2.26. |
| activeStatus | OUT | YES | The status of active or deactivate action. Enum ActionStatus, see 6.6.2.2.26. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Exception: Request may not have been completed |

##### 6.7.2.17.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get application software information.

##### 6.7.2.17.5	Exceptions

Not Applicable.

##### 6.7.2.17.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.17.7	oneM2M Resource Interworking

This service capability is used to get application software information. The service capability aligns with the <software> resource and maps to the Retrieve procedure for the resource.

#### 6.7.2.18	downloadSoftware

##### 6.7.2.18.1	Description

This service capability provides the ability to download application software.

##### 6.7.2.18.2	Pre-Conditions

A correlation between a Management Server, the M2M Service Capability and device exist.

##### 6.7.2.18.3	Signature –downloadSoftware

Table 6.7.2.18.3-1: Management Adapter –downloadSoftware capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| version | IN | NO | The version of the software. |
| name | IN | NO | The name of the software. |
| url | IN | NO | The URL from which the software package can be downloaded. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.2.2.1. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Exception: Request may not have been completed |

##### 6.7.2.18.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to download application software.

##### 6.7.2.18.5	Exceptions

Not Applicable.

##### 6.7.2.18.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.18.7	oneM2M Resource Interworking

This service capability is used to download application software. The service capability aligns with the <software> resource and maps to the UPDATE procedure for the attribute install of the resource. The service capability also aligns with the <mgmtCmd> resource and maps to the EXECUTE procedure for the resource.

#### 6.7.2.19	installSoftware

##### 6.7.2.19.1	Description

This M2M Service Capability provides the ability to install application software.

##### 6.7.2.19.2	Pre-Conditions

A correlation between a Management Server, the service capability and device exist.

##### 6.7.2.19.3	Signature –installSoftware

Table 6.7.2.19.3-1: Management Adapter –installSoftware capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| version | IN | NO | The version of the software. |
| name | IN | NO | The name of the software. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.2.2.1. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Exception: Request may not have been completed |

##### 6.7.2.19.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to install application software.

##### 6.7.2.19.5	Exceptions

Not Applicable.

##### 6.7.2.19.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.19.7	oneM2M Resource Interworking

This service capability is used to install application software. The service capability aligns with the <software> resource and maps to the UPDATE procedure for the attribute install of the resource. The service capability also aligns with the <mgmtCmd> resource and maps to the EXECUTE procedure for the resource.

#### 6.7.2.20	activateSoftware

##### 6.7.2.20.1	Description

This service capability provides the ability to activate software previously installed.

##### 6.7.2.20.2	Pre-Conditions

A correlation between a Management Server, the M2M Service Capability and device exist.

##### 6.7.2.20.3	Signature – activateSoftware

Table 6.7.2.20.3-1: Management Adapter – activateSoftware capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| version | IN | NO | The version of the software. |
| name | IN | NO | The name of the software. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.2.2.1. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Exception: Request may not have been completed |

##### 6.7.2.20.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to activate software previously installed.

##### 6.7.2.20.5	Exceptions

Not Applicable.

##### 6.7.2.20.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.20.7	oneM2M Resource Interworking

This service capability is used to activate software previously installed. The service capability aligns with the <software> resource and maps to the UPDATE procedure for the attribute activate of the resource. The service capability also aligns with the <mgmtCmd> resource and maps to the EXECUTE procedure for the resource.

#### 6.7.2.21	deactivateSoftware

##### 6.7.2.21.1	Description

This service capability provides the ability for the Management Adapter to deactivates software.

##### 6.7.2.21.2	Pre-Conditions

A correlation between a Management Server, the M2M Service Capability and device exist.

##### 6.7.2.21.3	Signature –deactivateSoftware

Table 6.7.2.21.3-1: Management Adapter –deactivateSoftware capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| version | IN | NO | The version of the software. |
| name | IN | NO | The name of the software. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.2.2.1. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Exception: Request may not have been completed |

##### 6.7.2.21.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to deactivates software.

##### 6.7.2.21.5	Exceptions

Not Applicable.

##### 6.7.2.21.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.21.7	oneM2M Resource Interworking

This service capability is used to deactivates software. The service capability aligns with the <software> resource and maps to the UPDATE procedure for the attribute deactivate of the resource. The service capability also aligns with the <mgmtCmd> resource and maps to the EXECUTE procedure for the resource.

#### 6.7.2.22	removeSoftware

##### 6.7.2.22.1	Description

This service capability provides the ability for the Management Adapter to uninstall the software.

##### 6.7.2.22.2	Pre-Conditions

A correlation between a Management Server, the M2M Service Capability and device exist.

##### 6.7.2.22.3	Signature –removeSoftware

Table 6.7.2.22.3-1: Management Adapter –removeSoftware capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| deviceId | IN | NO | The unique device identifier in the context of the M2M Service Subscription. |
| version | IN | NO | The version of the software. |
| name | IN | NO | The name of the software. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.2.2.1. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Exception: Request may not have been completed |

##### 6.7.2.22.4	Post-Conditions

The Management Adapter has submitted a request to the Management Server to uninstall the software.

##### 6.7.2.22.5	Exceptions

Not Applicable.

##### 6.7.2.22.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.7.2.22.7	oneM2M Resource Interworking

This service capability is used to install application software. The service capability aligns with the <software> resource and maps to the UPDATE procedure for the attribute uninstall of the resource. The service capability also aligns with the <mgmtCmd> resource and maps to the EXECUTE procedure for the resource.

## 6.8	Service Administration

### 6.8.1	Overview

The Service Administration service provides the capability to administer the following across the Msc Reference Points:

Administration of M2M Service attributes

Association of M2M Services to corresponding Service Roles

Association of Service Roles to M2M Service Capabilities

### 6.8.2	Service Administration Entities

#### 6.8.2.1	Overview

The information elements that relate M2M Services to Service Roles and Service Roles to M2M Service Capabilities is depicted below:

Each instance of an M2M Service contains zero or instances of a Service Role

The M2M Service can be obtained from the Service Role. The Service Roles from the M2M Service can be obtained from the M2M Service

Each instance of a Service Role is associated with zero or more instances of a M2M Service Capability

The M2M Service can be obtained from the Service Role. The Service Roles associated with the M2M Service can be obtained from the M2M Service

Each instance of a M2M Service Capability is associated with zero or more instances of a Service Role

The M2M Service Capabilities that are associated with a Service Role can be obtained from the Service Role. Likewise the Service Roles associated with the M2M Service Capability can be obtained from the M2M Service Capability

Each operation that is defined within the Service Exposure Component is assigned to exactly one M2M Service Capability

Figure 6.8.1-1: M2M Service Entity Relationships

#### 6.8.2.2	M2M Service Entity

The Service is a primary entity that provides the attributes to identify the M2M Service and associated Service Roles within the oneM2M System. These M2M Services and associated Service Roles shall be maintained by the M2M Service Provider.

Table 6.8.2.2-1: M2M Service Entity

| Attribute name | Description |
| --- | --- |
| serviceId | The M2M Service Identifier (M2M-Serv-ID). |
| serviceRoleIds | List of Service Role Identifiers (SRole-ID) associated with the Service. |

##### 6.8.2.3	M2M Service Filter Criteria

Table 6.8.2.3-1: M2M Service Filter Criteria

| Criterion name | Description |
| --- | --- |
| serviceIds | List of the Service Identifiers (M2M-Serv-ID). |
| serviceRoleIds | One or more Service Role Identifiers (SRole-ID) associated with the M2M Service entity. |

### 6.8.3	Service Capabilities

#### 6.8.3.1	createM2MService

##### 6.8.3.1.1	Description

This service capability provides the ability to create a M2M Service.

##### 6.8.3.1.2	Pre-Conditions

Not Applicable.

##### 6.8.3.1.3	Signature – createM2MService

Table 6.8.3.1.3-1: Service Administration – createM2M Service capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| name | IN | NO | The unique name of the M2M Service. |
| description | IN | YES | The description of the M2M Service. |
| serviceId | OUT | NO | The identifier of the created M2M Service (M2M-Serv-ID) |
| responseType | OUT | YES | Unique response types for this service. M2M Service exists for the name parameter M2M Service Role exists for M2M Service |

##### 6.8.3.1.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue this capability

Figure 6.8.3.1.4-1: Service Administration – createM2MService Diagram

##### 6.8.3.1.5	Post-Conditions

Not Applicable.

##### 6.8.3.1.6	Exceptions

Not Applicable.

##### 6.8.3.1.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.8.3.1.8	oneM2M Resource Interworking

This service capability creates the M2M Service. The M2M Service entity is described in the Annex G of the oneM2M Functional Architecture [2]. The M2M-Serv-ID created using this capability shall map to the M2M-Serv-ID of the oneM2M Functional Architecture [2].

#### 6.8.3.2	deleteM2MService

##### 6.8.3.2.1	Description

This service capability provides the ability to delete a M2M Service. This service capability shall be restricted to the Msc Reference Point.

##### 6.8.3.2.2	Pre-Conditions

The M2M Service's Service Roles are not associated with any M2M Service Subscriptions.

##### 6.8.3.2.3	Signature – deleteM2MService

Table 6.8.3.2.3-1: Service Administration – deleteM2MService capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceId | IN | NO | The M2M Service Identifier (M2M-Serv-ID). |
| responseType | OUT | YES | Unique response types for this service. M2M Service does not exist M2M Service has Service Roles associated with M2M Service Subscriptions |

##### 6.8.3.2.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue this capability

Figure 6.8.3.2.4-1: Service Administration – deleteM2MService Diagram

##### 6.8.3.2.5	Post-Conditions

The M2M Service is deleted along with any associations has to the Service Roles. The Node information associated with the M2M Service is also deleted.

##### 6.8.3.2.6	Exceptions

Not Applicable.

##### 6.8.3.2.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.8.3.2.8	oneM2M Resource Interworking

This service capability deletes the M2M Service. The M2M Service entity is described in the Annex G of the oneM2M Functional Architecture [2].

#### 6.8.3.3	addRoleToM2MService

##### 6.8.3.3.1	Description

This service capability provides the ability to add new Service Roles to a M2M Service. This service capability shall be restricted to the Msc Reference Point.

##### 6.8.3.3.2	Pre-Conditions

M2M Service exists.

##### 6.8.3.3.3	Signature – addRoleToM2MService

Table 6.8.3.3.3-1: Service Administration – addRoleToM2MService capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceId | IN | NO | The M2M Service Identifier (M2M-Serv-ID). |
| serviceRoleIds | IN | NO | List of Service Role Identifiers (SRole-ID) to be associated with the M2M Service. If a SRole-ID in the parameter is already associated with the M2M Service, nothing is done for the SRole-ID as it already is associated. |
| responseType | OUT | YES | Unique response types for this service. M2M Service does not exist |

##### 6.8.3.3.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue this capability

Figure 6.8.3.3.4-1: Service Administration – addRoleToM2MService Diagram

##### 6.8.3.3.5	Post-Conditions

The associations that M2M Service has to the Service Role(s) are updated.

##### 6.8.3.3.6	Exceptions

Not Applicable.

##### 6.8.3.3.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.8.3.3.8	oneM2M Resource Interworking

This service capability updates the M2M Service with the respective M2M Service Roles. The M2M Service entity is described in the Annex G of the oneM2M Functional Architecture [2].

#### 6.8.3.4	deleteRoleFromM2MService

##### 6.8.3.4.1	Description

This service capability provides the ability to delete existing Service Roles from a M2M Service. This service capability shall be restricted to the Msc Reference Point.

##### 6.8.3.4.2	Pre-Conditions

The Service Role is not associated with any M2M Service Subscriptions.

##### 6.8.3.4.3	Signature – deleteRoleFromM2MService

Table 6.8.3.4.3-1: Service Administration – deleteRoleFromM2MService capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceId | IN | NO | The M2M Service Identifier (M2M-Serv-ID). |
| serviceRoleIds | IN | NO | List of Service Role Identifiers (SRole-ID) to be deleted from the association with the M2M Service. |
| responseType | OUT | YES | Unique response types for this service. M2M Service does not exist Service Role does not exist Service Role is associated with a M2M Service Subscription |

##### 6.8.3.4.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue this capability

Figure 6.8.3.4.4-1: Service Administration – deleteRoleFromM2MService Diagram

##### 6.8.3.4.5	Post-Conditions

The M2M Service is updated with associations of the identified Service Roles deleted.

##### 6.8.3.4.6	Exceptions

Not Applicable.

##### 6.8.3.4.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.8.3.4.8	oneM2M Resource Interworking

This service capability updates the M2M Service with the respective Service Roles removed. The M2M Service entity is described in the Annex G of the oneM2M Functional Architecture [2].

#### 6.8.3.5	getM2MService

##### 6.8.3.5.1	Description

This service capability provides the ability to retrieve the existing M2M Service. This service capability shall be restricted to the Msc Reference Points.

##### 6.8.3.5.2	Pre-Conditions

Not Applicable.

##### 6.8.3.5.3	Signature – getM2MService

Table 6.8.3.5.2-1: Service Administration – getM2MService capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| filterCriteria | IN | NO | See M2M Service Filter Criteria Table 6.8.2.3-1 |
| services | OUT | NO | The resulting M2M Service entities in Table 6.8.2.2-1. |
| responseType | OUT | YES | Unique response types for this service. None |

##### 6.8.3.5.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue this capability

Figure 6.8.3.5.4-1: Service Administration – getM2MService Diagram

##### 6.8.3.5.5	Post-Conditions

Not Applicable.

##### 6.8.3.5.6	Exceptions

Not Applicable.

##### 6.8.3.5.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.8.3.5.8	oneM2M Resource Interworking

This service capability retrieves the M2M Service. The M2M Service entity is described in the Annex G of the oneM2M Functional Architecture [2].

#### 6.8.3.6	addServiceCapabilityToRole

##### 6.8.3.6.1	Description

This service capability adds a M2M Service Capability to one or more Service Roles.

##### 6.8.3.6.2	Pre-Conditions

Service Roles exist.

##### 6.8.3.6.3	Signature – addServiceCapabilityToRole

Table 6.8.3.6.3-1: Service Administration – addServiceCapabilityToRole capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceCapId | IN | NO | The M2M Service Capability Identifier (M2M-Serv-Cap-ID). |
| serviceRoleIds | IN | NO | List of Service Role Identifiers (SRole-ID) to be associated with the M2M Service Capability. |
| responseType | OUT | YES | Response types: Service Role does not exist M2M Service Capability is not part of the capabilities of M2M Service associated with the Service Role |

##### 6.8.3.6.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue this capability

Figure 6.8.3.6.4-1: Service Administration – addServiceCapabilityToRole Diagram

##### 6.8.3.6.5	Post-Conditions

Not Applicable.

##### 6.8.3.6.6	Exceptions

Not Applicable.

##### 6.8.3.6.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.8.3.6.8	oneM2M Resource Interworking

Not Applicable.

#### 6.8.3.7	deleteServiceCapabilityFromRole

##### 6.8.3.7.1	Description

This service capabilitydelete a M2M Service Capability from one or more Service Roles.

##### 6.8.3.7.2	Pre-Conditions

The association between the M2M Service Capability and Service Role exists.

##### 6.8.3.7.3	Signature – deleteServiceCapabilityFromRole

Table 6.8.3.7.3-1: Service Administration – deleteServiceCapabilityFromRole capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceCapId | IN | NO | The M2M Service Capability Identifier (M2M-Serv-Cap-ID). |
| serviceRoleIds | IN | NO | List of Service Role Identifiers (SRole-ID) to be associated with the M2M Service Capability to be removed |
| responseType | OUT | YES | Response types: Service Role does not exist Service Role is not associated with a M2M Service Capability |

##### 6.8.3.7.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue this capability

Figure 6.8.3.7.4-1:Service Administration – deleteServiceCapabilityFromRole Diagram

##### 6.8.3.7.5	Post-Conditions

The association of the M2M Service Capability and Service Role is deleted.

##### 6.8.3.7.6	Exceptions

Not Applicable.

##### 6.8.3.7.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.8.3.7.8	oneM2M Resource Interworking

Not Applicable.

#### 6.8.3.8	getServiceCapability

##### 6.8.3.8.1	Description

This service capability retrieves the M2M Service Capability and associated Service Roles for a specified requested operation.

##### 6.8.3.8.2	Pre-Conditions

There is a correlation to the service capabilities' operation (signature) and the M2M Service Capability. In many deployments the service capabilities' signature serves as the M2M Service Capability.

##### 6.8.3.8.3	Signature – getServiceCapability

Table 6.8.3.8.3-1: Service Administration – getServiceCapability capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| operation | IN | NO | Request operation – Common to all requests |
| serviceCapId | OUT | NO | The M2M Service Capability Identifier (M2M-Serv-Cap-ID). |
| serviceRoleIds | OUT | NO | List of the M2M Service Roles Identifiers (SRole-ID) associated with the M2M Service Capabilities. |
| responseType | OUT | YES | Response types: Unable to determine M2M Service Capability from the operation |

##### 6.8.3.8.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue this capability

Figure 6.8.3.8.4-1: Service Administration – getServiceCapability Diagram

##### 6.8.3.8.5	Post-Conditions

Not Applicable.

##### 6.8.3.8.6	Exceptions

Not Applicable.

##### 6.8.3.8.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.8.3.8.8	oneM2M Resource Interworking

Not Applicable.

## 6.9	Service Subscription Administration

### 6.9.1	Overview

The Service Subscription Administration service provides the capability to administer the following across the Mca and Msc Reference Points:

Maintenance of M2M Services Subscriptions

Association of M2M Nodes and Application Rules to the M2M Service Subscription

The M2M Service Subscription defines the technical part of the contract between a M2M Subscriber (typically an M2M Application Service Provider) and a M2M Service Provider. Each M2M Service Subscription has a unique identifier (M2M-Service-Profile-ID).

A M2M Service Subscription establishes an association between one or more M2M Application Rules and one or more M2M Nodes. In addition, the M2M Service Subscription is associated with one or more Service Roles.

A M2M Service Subscription shall be used for the following purposes:

Serve as a basis for authorization for service capabilities

Serve as the basis for charging

Identify which devices and M2M Applications are part of this M2M Service Subscription

### 6.9.2	Service Subscription Entities

#### 6.9.3.1	Service Subscription Entity

Table 6.9.3.1-1: Service Subscription Entity

| Attribute name | Description |
| --- | --- |
| serviceSubscriptionId | The M2M Service Subscription Identifier (M2M-Service-Profile-ID). |
| serviceRoleIds | List of Service Role Identifiers (SRole-ID) associated with the M2M Service Subscription. |

#### 6.9.2.2	Service Subscription Filter Criteria

Table 6.9.3.2-1: Service Subscription Filter Criteria

| Criterion name | Description |
| --- | --- |
| serviceSubscriptionIds | List of the M2M Service Subscription Identifiers (M2M-Service-Profile-ID). |
| serviceRoleIds | List of Service Role Identifiers (SRole-ID) associated with the M2M Service Subscription entity. |

#### 6.9.2.3	M2M Node Entity for Service Subscription

Table 6.9.2.3-1: M2M Node Entity for Service Subscription

| Attribute name | Description |
| --- | --- |
| serviceSubscriptionId | The M2M Service Subscription Identifier (M2M-Service-Profile-ID) for the Device. |
| nodeId | The unique M2M Node identifier in the context of the M2M Service Subscription. |
| externalIds | List of URNs that represent the external identifiers associated with this M2M Node. |
| applicationRuleIds | List of Application Rules Identifiers associated with the M2M Node. |

#### 6.9.2.4	M2M Node Filter Criteria

Table 6.9.2.4-1: M2M Node Filter Criteria

| Criterion name | Description |
| --- | --- |
| serviceSubscriptionIds | List of the M2M Service Subscription Identifier (M2M-Service-Profile-ID) for the Device. |
| nodeIds | List of the unique M2M Node identifier in the context of the M2M Service Subscription. |
| externalIds | List of URNs that represent the external identifiers associated with this M2M Node. |
| applicationRuleIds | List of Application Rule identifiers associated with the M2M Node. |

#### 6.9.2.5	Application Rule

Table 6.9.2.5-1: Application Rule Entity

| Criterion name | Description |
| --- | --- |
| applicationRuleId | The unique Application Rule identifier in the context of the M2M Service Subscription. |
| credentialIds | List of credential identifiers for which this Application Rule is applicable, i.e. for registration requests coming into a CSE via a Security Association Endpoint (SEA) [i.4], that was authenticated using credentials that match with any of these credential identifiers, the current Application Rule applies. This can contain a '*' for any credential identifier or 'None' for not authenticated case. Also Wildcards within an element of this list are possible (e.g. 'C123*X' for any credential identifier that starts with 'C123' and ends with 'X') to define sets or ranges of credential identifiers. |
| allowedApplicationIds | List of M2M Application Identifiers (App-ID) that shall be considered to be allowed for AE registration requests received via Security Association Endpoint (SEA) [i.4]. This can contain '*' for any App-ID. Also Wildcards within an element of this list are possible (e.g. 'C123*X' for any App-ID that starts with 'C123' and ends with 'X') to define sets or ranges of App-IDs. |
| allowedAEIds | List of M2M Application Entity Identifiers (AE-ID) that shall be considered to be allowed for AE registration requests. This can contain zero or more specific AE identifier values, 'S*' for any Service Provider AE-ID, 'C*' for any system assigned AE-ID, or '*' for any AE-ID. Also Wildcards within an element of this list are possible (e.g. 'C123*X' for any AE-ID that starts with 'C123' and ends with 'X') to define sets or ranges of AE-ID. |

#### 6.9.2.6	Application Rule Filter Criteria

Table 6.9.2.6-1: Application Rule Filter Criteria

| Criterion name | Description |
| --- | --- |
| applicationRuleIds | List of the unique Application Rule identifier in the context of the M2M Service Subscription. |
| credentialIds | List of credential identifiers for which this Application Rule is applicable, i.e. for registration requests coming into the M2M System via a Security Association Endpoint (SEA) [i.1], that was authenticated using credentials that match with any of these credential identifiers, the current Application Rule applies. This can contain a '*' for any credential identifier or 'None' for not authenticated case. Also Wildcards within an element of this list are possible (e.g. 'C123*X' for any credential identifier that starts with 'C123' and ends with 'X') to define sets or ranges of credential identifiers. |
| allowedApplicationIds | List of M2M Application Identifiers (App-ID) that shall be considered to be allowed for AE registration requests received via Security Association Endpoint (SEA) [i.2]. This can contain '*' for any App-ID. Also Wildcards within an element of this list are possible (e.g. 'C123*X' for any App-ID that starts with 'C123' and ends with 'X') to define sets or ranges of App-IDs. |
| allowedAEIds | List of M2M Application Entity Identifiers (AE-ID) that shall be considered to be allowed for AE registration requests. This can contain zero or more specific AE identifier values, 'S*' for any Service Provider AE-ID, 'C*' for any system assigned AE-ID, or '*' for any AE-ID. Also Wildcards within an element of this list are possible (e.g. 'C123*X' for any AE-ID that starts with 'C123' and ends with 'X') to define sets or ranges of AE-ID. |

#### 6.9.2.7	Supporting Rules

##### 6.9.2.7.1	Schedule Type

The Schedule is a simple type that describes the schedule information.

Table 6.9.2.7.1-1: Type: Schedule

| Parameter | Optional | Description |
| --- | --- | --- |
| scheduleElement | NO | Expresses time periods defined by second, minute, hour day of month, month, and year. Supports repeating periods, and wildcards expressed as a list. The definition of a schedule is defined in clause 7.3.9 of TS-0004. |

### 6.9.3	Service Capabilities

#### 6.9.3.1	createServiceSubscription

##### 6.9.3.1.1	Description

This service capability provides the ability to create a M2M Service Subscription. This service capability shall be restricted to the Msc Reference Point.

##### 6.9.3.1.2	Pre-Conditions

Not Applicable.

##### 6.9.3.1.3	Signature – createServiceSubscription

Table 6.9.3.1.3-1: Service Subscription Administration - createServiceSubscription capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceSubscriptionId | IN-OUT | YES | The M2M Service Subscription Identifier (M2M-Service-Profile-ID). If the attribute is not provided on input, the oneM2M System shall assign the M2M-Service-Profile-ID. The M2M-Service-Profile-ID is unique for the oneM2M System. |
| serviceRoleIds | IN | NO | List of Service Role Identifiers (SRole-ID) to be associated with the M2M Service Subscription. |
| responseType | OUT | YES | Unique response types for this service. M2M Service Subscription exists Service Role does not exist |

##### 6.9.3.1.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the createServiceSubscription capability

Figure 6.9.3.1.4-1: Service Subscription Administration – createServiceSubscription Diagram

##### 6.9.3.1.5	Post-Conditions

The M2M Service Subscription is created and any Service Roles are associated with the M2M Service Subscription.

##### 6.9.3.1.6	Exceptions

Not Applicable.

##### 6.9.3.1.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.3.1.8	oneM2M Resource Interworking

This service capability creates the M2M Service Subscription with the identifier (M2M-Service-Profile-ID) and maps to the CREATE procedure for the <m2mServiceSubscriptionProfile> resource.

This service capability utilizes the access management services with the privileges associated with the associated Service Role. As such the accessControlPolicyIDs attribute of the <m2mServiceSubscriptionProfile> resource is not used within this service capability.

#### 6.9.3.2	deleteServiceSubscription

##### 6.9.3.2.1	Description

This service capability provides the ability to delete a M2M Service Subscription. This service capability shall be restricted to the Msc Reference Point.

##### 6.9.3.2.2	Pre-Conditions

Not Applicable.

##### 6.9.3.2.3	Signature – deleteServiceSubscription

Table 6.9.3.2.3-1: Service Subscription Administration – deleteServiceSubscription capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceSubscriptionId | IN | NO | The M2M Service Subscription Identifier (M2M-Service-Profile-ID). |
| responseType | OUT | YES | Unique response types for this service. M2M Service Subscription does not exist |

##### 6.9.3.2.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the deleteServiceSubscription capability

Figure 6.9.3.2.4-1: Service Subscription Administration – deleteServiceSubscription Diagram

##### 6.9.3.2.5	Post-Conditions

The M2M Service Subscription is deleted along with any associations of the M2M Service Subscription has to the Service Roles. The Node information (e.g. devices, M2M Applications) associated with the M2M Service Subscription is also deleted.

##### 6.9.3.2.6	Exceptions

Not Applicable.

##### 6.9.3.2.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.3.2.8	oneM2M Resource Interworking

This service capability deletes the M2M Service Subscription with the identifier (M2M-Service-Profile-ID) and maps to the DELETE procedure for the <m2mServiceSubscriptionProfile> resource.

#### 6.9.3.3	updateServiceSubscription

##### 6.9.3.3.1	Description

This service capability provides the ability to update a M2M Service Subscription. This service capability shall be restricted to the Msc Reference Point.

##### 6.9.3.3.2	Pre-Conditions

Not Applicable.

##### 6.9.3.3.3	Signature – updateServiceSubscription

Table 6.9.3.3.3-1: Service Subscription Administration – updateServiceSubscription capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceSubscriptionId | IN | NO | The M2M Service Subscription Identifier (M2M-Service-Profile-ID). |
| serviceRoleIds | IN | YES | List of Service Role Identifiers (SRole-ID) to be associated with the M2M Service Subscription. If supplied, SRole-IDs in the list that are not already associated with the M2M Service Subscription are added. SRole-IDs that are associated with the M2M Service Subscription but not in the list are deleted. |
| roleIdResult | OUT | YES | If the parameter serviceRoleIds is present, this parameter provides the results (added, deleted) for each affected Service Roles. |
| responseType | OUT | YES | Unique response types for this service. M2M Service Subscription does not exist Service Role does not exist for a Supporting Service |

##### 6.9.3.3.4	Service Interactions

- The interactions of service capabilities required for this service capability:

- Issue the updateServiceSubscription capability

Figure 6.9.3.3.4-1: Service Subscription Administration - updateServiceSubscription Diagram

##### 6.9.3.3.5	Post-Conditions

The M2M Service Subscription is updated along with any associations of the M2M Service Subscription has to the Service Roles.

##### 6.9.3.3.6	Exceptions

Not Applicable.

##### 6.9.3.3.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.3.3.8	oneM2M Resource Interworking

This service capability updates the M2M Service Subscription with the identifier (M2M-Service-Profile-ID) and maps to the UPDATE procedure for the <m2mServiceSubscriptionProfile> resource.

#### 6.9.3.4	addRoleToServiceSubscription

##### 6.9.3.4.1	Description

This service capability provides the ability to add new Service Roles to a M2M Service Subscription. This service capability shall be restricted to the Msc Reference Point.

##### 6.9.3.4.2	Pre-Conditions

Not Applicable.

##### 6.9.3.4.3	Signature – addRoleToServiceSubscription

Table 6.9.3.4.3-1: Service Subscription Administration – addRoleToServiceSubscription capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceSubscriptionId | IN | NO | The M2M Service Subscription Identifier (M2M-Service-Profile-ID). |
| serviceRoleIds | IN | NO | List of Service Role Identifiers (SRole-ID) to be associated with the M2M Service Subscription. If a SRole-ID in the parameter is already associated with the M2M Service Subscription, nothing is done for the SRole-ID as it already is associated. |
| responseType | OUT | YES | Unique response types for this service. M2M Service Subscription does not exist Service Role does not exist for a Supporting Service |

##### 6.9.3.4.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the addRoleToServiceSubscription capability

Figure 6.9.3.4.4-1: Service Subscription Administration – addRoleToServiceSubscription Diagram

##### 6.9.3.4.5	Post-Conditions

The M2M Service Subscription is updated with any associations of the M2M Service Subscription has to the Service Roles.

##### 6.9.3.4.6	Exceptions

Not Applicable.

##### 6.9.3.4.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.3.4.8	oneM2M Resource Interworking

This service capability updates the M2M Service Subscription with the identifier (M2M-Service-Profile-ID) and maps to the UPDATE procedure for the <m2mServiceSubscriptionProfile> resource for the serviceRoles attribute.

#### 6.9.3.5	deleteRoleFromServiceSubscription

##### 6.9.3.5.1	Description

This service capability provides the ability to delete existing Service Roles from a M2M Service Subscription. This service capability shall be restricted to the Msc Reference Point.

##### 6.9.3.5.2	Pre-Conditions

Not Applicable.

##### 6.9.3.5.3	Signature – deleteRoleFromServiceSubscription

Table 6.9.3.5.3-1: Service Subscription Administration – deleteRoleFromServiceSubscription capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceSubscriptionId | IN | NO | The M2M Service Subscription Identifier (M2M-Service-Profile-ID). |
| serviceRoleIds | IN | NO | List of Service Role Identifiers (SRole-ID) to be deleted from the association with the M2M Service Subscription. |
| responseType | OUT | YES | Unique response types for this service. M2M Service Subscription does not exist Service Role does not exist for a Supporting Service |

##### 6.9.3.5.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the deleteRoleFromServiceSubscription capability

Figure 6.9.3.5.4-1: Service Subscription Administration – deleteRoleFromServiceSubscription Diagram

##### 6.9.3.5.5	Post-Conditions

The M2M Service Subscription is updated with associations of the identified Service Roles deleted.

##### 6.9.3.5.6	Exceptions

Not Applicable.

##### 6.9.3.5.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.3.5.8	oneM2M Resource Interworking

This service capability updates the M2M Service Subscription with the identifier (M2M-Service-Profile-ID) and maps to the UPDATE procedure for the <m2mServiceSubscriptionProfile> resource for the serviceRoles attribute.

#### 6.9.3.6	getServiceSubscription

##### 6.9.3.6.1	Description

This service capability provides the ability to retrieve the existing M2M Service Subscription. This service capability shall be restricted to the Msc and Mca Reference Points.

##### 6.9.3.6.2	Pre-Conditions

Not Applicable.

##### 6.9.3.6.3	Signature – getServiceSubscription

Table 6.9.3.6.3-1: Service Subscription Administration – getServiceSubscription capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| filterCriteria | IN | NO | See Table 6.9.2.2-1 |
| serviceSubscriptions | OUT | NO | The resulting M2M Service Subscription entities in Table 6.9.2.3-1. |
| responseType | OUT | YES | Unique response types for this service. None |

##### 6.9.3.6.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the getServiceSubscription capability

Figure 6.9.3.6.4-1: Service Subscription Administration – getServiceSubscription Diagram

##### 6.9.3.6.5	Post-Conditions

Not Applicable.

##### 6.9.3.6.6	Exceptions

Not Applicable.

##### 6.9.3.6.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.3.6.8	oneM2M Resource Interworking

This service capability retrieves the M2M Service Subscription for the specified filter criteria and maps to the RETRIEVE procedure for the <m2mServiceSubscription> resource.

#### 6.9.3.7	addNodeToServiceSubscription

##### 6.9.3.7.1	Description

This service capability provides the ability to add a M2M Node to the existing M2M Service Subscription. This service capability shall be restricted to the Msc and Mca Reference Points.

##### 6.9.3.7.2	Pre-Conditions

Not Applicable.

##### 6.9.3.7.3	Signature – addNodeToServiceSubscription

Table 6.9.3.7.3-1: Service Subscription Administration – addNodeToServiceSubscription capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceSubscriptionId | IN | NO | The M2M Service Subscription (M2M-Service-Profile-ID) to add the Device |
| externalIds | IN | YES | A List of URNs that represent the external identifiers associated with this M2M Node. |
| applicationRuleIds | IN | YES | A list of Application Rules associated with this Node. |
| nodeId | OUT | NO | The unique M2M Node identifer for this entity. |
| responseType | OUT | YES | Unique response types for this service. M2M Service Subscription does not exist. |

##### 6.9.3.7.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the addNodeToServiceSubscription capability

Figure 6.9.3.7.4-1: Service Subscription Administration – addNodeToServiceSubscription Diagram

##### 6.9.3.7.5	Post-Conditions

Not Applicable.

##### 6.9.3.7.6	Exceptions

Not Applicable.

##### 6.9.3.7.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.3.7.8	oneM2M Resource Interworking

This service capability adds the M2M Node information for the M2M Service Subscription. This capability maps to the CREATE procedure of the <serviceSubscribedNode> resource.

#### 6.9.3.8	deleteNodesFromServiceSubscription

##### 6.9.3.8.1	Description

This service capability provides the ability to delete M2M Nodes from the existing M2M Service Subscription. This service capability shall be restricted to the Msc and Mca Reference Points.

##### 6.9.3.8.2	Pre-Conditions

Not Applicable.

##### 6.9.3.8.3	Signature – deleteNodesFromServiceSubscription

Table 6.9.3.8.3-1: Service Subscription Administration - deleteNodesFromServiceSubscription capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceSubscriptionId | IN | NO | The M2M Service Subscription (M2M-Service-Profile-ID) for the Devices |
| nodeIds | IN | NO | List of M2M Node identifiers to be deleted from the M2M Service Subscription. |
| responseType | OUT | YES | Unique response types for this service. M2M Service Subscription does not exist. M2M Node does not exist for this M2M Service Subscription |

##### 6.9.3.8.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the deleteNodesFromServiceSubscription capability

Figure 6.9.3.8.4-1: Service Subscription Administration – deleteNodesFromServiceSubscription Diagram

##### 6.9.3.8.5	Post-Conditions

Not Applicable.

##### 6.9.3.8.6	Exceptions

Not Applicable.

##### 6.9.3.8.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.3.8.8	oneM2M Resource Interworking

This service capability removes the M2M Node from the M2M Service Subscription. This capability maps to the DELETE procedure of the <serviceSubscribedNode> resource.

#### 6.9.3.9	getNodesForServiceSubscription

##### 6.9.3.9.1	Description

This service capability provides the ability to retrieve M2M Nodes for an existing M2M Service Subscription. This service capability shall be restricted to the Msc and Mca Reference Points.

##### 6.9.3.9.2	Pre-Conditions

Not Applicable.

##### 6.9.3.9.3	Signature – getNodesForServiceSubscription

Table 6.9.3.9.3-1: Service Subscription Administration – getNodesForServiceSubscription capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| filterCriteria | IN | NO | See Table 6.9.2.4-1 |
| nodes | OUT | NO | The resulting M2M Node entities in Table 6.9.2.3-1. |
| responseType | OUT | YES | Unique response types for this service. None |

##### 6.9.3.9.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the getNodesForServiceSubscription capability

Figure 6.9.3.9.4-1: Service Subscription Administration – getNodesForServiceSubscription Diagram

##### 6.9.3.9.5	Post-Conditions

Not Applicable.

##### 6.9.3.9.6	Exceptions

Not Applicable.

##### 6.9.3.9.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.3.9.8	oneM2M Resource Interworking

This service capability retrieves the devices for a M2M Service Subscription for the specified filter criteria and maps to the RETRIEVE procedure for the <serviceSubscribedNode> resource.

#### 6.9.3.10	createApplicationRule

##### 6.9.3.10.1	Description

This service capability provides the ability to create a M2M Application Rule to be associated with one or more M2M Nodes. This service capability shall be restricted to the Msc and Mca Reference Points.

##### 6.9.3.10.2	Pre-Conditions

Not Applicable.

##### 6.9.3.10.3	Signature – createApplicationRule

Table 6.9.3.10.3-1: Service Subscription Administration – createApplicationRule capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| credentialIds | IN | YES | List of credential identifiers for which this Application Rule is applicable, i.e. for registration requests coming into the M2M System via a Security Association Endpoint (SEA) [i.3], that was authenticated using credentials that match with any of these credential identifiers, the current Application Rule applies. |
| allowedApplicationIds | IN | YES | List of M2M Application Identifiers (App-ID) that shall be considered to be allowed for AE registration requests received via Security Association Endpoint (SEA) [i.4]. |
| allowedAEIds | IN | YES | List of M2M Application Entity Identifiers (AE-ID) that shall be considered to be allowed for AE registration requests. |
| applicationRuleId | OUT | NO | The unique Application Rule identifier in the context of the M2M Service Subscription. |
| responseType | OUT | YES | Unique response types for this service. None |

##### 6.9.3.10.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the createApplicationRule capability

Figure 6.9.3.10.4-1: Service Subscription Administration – createApplicationRule Diagram

##### 6.9.3.10.5	Post-Conditions

Not Applicable.

##### 6.9.3.10.6	Exceptions

Not Applicable.

##### 6.9.3.10.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.3.10.8	oneM2M Resource Interworking

This service capability adds an Application Rule. This capability maps to the CREATE procedure of the <serviceSubscribedAppRule> resource.

#### 6.9.3.11	deleteApplicationRules

##### 6.9.3.11.1	Description

This service capability provides the ability to delete Application Rules and any references to the Application Rule made by M2M Nodes to the Application Rules. This service capability shall be restricted to the Msc and Mca Reference Points.

##### 6.9.3.11.2	Pre-Conditions

Not Applicable.

##### 6.9.3.11.3	Signature - deleteApplicationRules

Table 6.9.3.11.3-1: Service Subscription Administration – deleteApplicationRules capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| applicationRuleIds | IN | NO | A list of Application Rule identifiers. |
| responseType | OUT | YES | Unique response types for this service. Application Rule does not exist |

##### 6.9.3.11.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the deleteApplicationRules capability

Figure 6.9.3.11.4-1: Service Subscription Administration – deleteApplicationRules Diagram

##### 6.9.3.11.5	Post-Conditions

Not Applicable.

##### 6.9.3.11.6	Exceptions

Not Applicable.

##### 6.9.3.11.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.3.11.8	oneM2M Resource Interworking

This service capability adds Application Rules. This capability maps to the DELETE procedure of the <serviceSubscribedAppRule> resource.

#### 6.9.3.12	getApplicationRules

##### 6.9.3.12.1	Description

This service capability provides the ability to retrieve the Application Rules. This service capability shall be restricted to the Msc and Mca Reference Points.

##### 6.9.3.12.2	Pre-Conditions

Not Applicable.

##### 6.9.3.12.3	Signature – getApplicationRules

Table 6.9.3.12.3-1: Service Subscription Administration – getApplicationRules capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| filterCriteria | IN | NO | See Table 6.9.2.6-1 |
| applicationRules | OUT | NO | The resulting M2M Node entities in Table 6.9.2.5-1. |
| responseType | OUT | YES | Unique response types for this service. None |

##### 6.9.3.12.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the getApplicationRules capability

Figure 6.9.3.12.4-1: Service Subscription Administration – getApplicationRules Diagram

##### 6.9.3.12.5	Post-Conditions

Not Applicable.

##### 6.9.3.12.6	Exceptions

Not Applicable.

##### 6.9.3.12.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.3.12.8	oneM2M Resource Interworking

This service capability retrieves Application Rules. This capability maps to the RETRIEVE procedure of the <serviceSubscribedAppRule> resource.

#### 6.9.3.13	updateNodeForServiceSubscription

##### 6.9.3.13.1	Description

This service capability provides the ability to update the Application Rules and External Identifiers to be associated to a Node within the context of a M2M Service Subscription. This service capability shall be restricted to the Msc and Mca Reference Points.

##### 6.9.3.13.2	Pre-Conditions

Not Applicable.

##### 6.9.3.13.3	updateNodeForServiceSubscription

Table 6.9.3.13.3-1: Service Subscription Administration – updateNodeForServiceSubscription capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceSubscriptionId | IN | NO | The M2M Service Subscription (M2M-Service-Profile-ID) for the device |
| nodeId | IN | NO | The unique M2M Node identifier in the context of the M2M Service Subscription. |
| applicationRuleIds | IN | YES | A list of Application Rule Identifier. When present this list will overlay the list of Applications associated with the Node. |
| externalIds | IN | YES | List of URNs that represent the external identifiers associated with this device. When present this list will overlay the list of external identifiers associated with the Node. |
| responseType | OUT | YES | Unique response types for this service. M2M Service Subscription does not exist M2M Node does not exist for the M2M Service Subscription |

##### 6.9.3.13.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the updateNodeForServiceSubscription capability

Figure 6.9.3.13.4-1: Service Subscription Administration – updateNodeForServiceSubscription Diagram

##### 6.9.3.13.5	Post-Conditions

When parameters are present, the M2M Applications and External Identifiers are replaced for the M2M Node.

##### 6.9.3.13.6	Exceptions

Not Applicable.

##### 6.9.3.13.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.3.13.8	oneM2M Resource Interworking

This service capability updates the M2M Application and/or External Identifier list(s) for the Node in the context of the M2M Service Subscription Profile. This capability maps to the UPDATE procedure of the <serviceSubscribedNode> resource.

#### 6.9.3.14	updateApplicationRule

##### 6.9.3.14.1	Description

This service capability provides the ability to update an Application Rule. This service capability shall be restricted to the Msc and Mca Reference Points.

##### 6.9.3.14.2	Pre-Conditions

Not Applicable.

##### 6.9.3.14.3	Signature – updateApplicationRule

Table 6.9.3.14.3-1: Service Subscription Administration – updateApplicationRule capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| applicationRuleId | IN | NO | The unique Application Rule identifier in the context of the M2M Service Subscription. |
| credentialIds | IN | YES | List of credential identifiers for which this Application Rule is applicable, i.e. for registration requests coming into the M2M System via a Security Association Endpoint (SEA) [i.3], that was authenticated using credentials that match with any of these credential identifiers, the current Application Rule applies. When present this list will overlay the list of credential identifiers. |
| allowedApplicationIds | IN | YES | List of M2M Application Identifiers (App-ID) that shall be considered to be allowed for AE registration requests received via Security Association Endpoint (SEA) [i.3]. When present this list will overlay the list of allowed App-IDs. |
| allowedAEIds | IN | YES | List of M2M Application Entity Identifiers (AE-ID) that shall be considered to be allowed for AE registration requests. When present this list will overlay the list of allowed AE-IDs. |
| responseType | OUT | YES | Unique response types for this service. Application Rule does not exist. |

##### 6.9.3.14.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the updateApplicationRule capability

Figure 6.9.3.14.4-1: Service Subscription Administration – updateApplicationRule Diagram

##### 6.9.3.14.5	Post-Conditions

When parameters are present, the Allowed Application, AEs and Credentials are replaced for the Application Rule.

##### 6.9.3.14.6	Exceptions

Not Applicable.

##### 6.9.3.14.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.3.14.8	oneM2M Resource Interworking

This service capability updates an Application Rule. This capability maps to the UPDATE procedure of the <serviceSubscribedAppRule> resource.

#### 6.9.3.15	addReachabilitySchedule

##### 6.9.3.15.1	Description

This service capability provides the ability to add a reachability schedule to an AE. This service capability shall be restricted to the Msc and Mca Reference Points.

##### 6.9.3.15.2	Pre-Conditions

Not Applicable.

##### 6.9.3.15.3	Signature – addReachabilitySchedule

Table 6.9.3.15.3-1: Service Subscription Administration – addReachabilitySchedule capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| aeId | IN | NO | The Application Entity Identifier (AE-ID). |
| reachabilitySchedule | IN | YES | The contact reachability schedule information of the AE associated with the device node. The absence of this parameter implies the AE associated with the device node is always contact reachable. Type Schedule, see 6.9.2.7.1. |
| responseType | OUT | YES | Unique response types for this service. AE successfully added |

##### 6.9.3.15.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the addReachabilitySchedule capability

Figure 6.9.3.15.4-1: addReachabilitySchedule Diagram

##### 6.9.3.15.5	Post-Conditions

When parameters are present, the reachability schedule is added for the AE.

##### 6.9.3.15.6	Exceptions

Not Applicable.

##### 6.9.3.15.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.3.15.8	oneM2M Resource Interworking

This service capability is used to add AE reachability schedule information. The service capability aligns with the <schedule> resource and maps to the CREATE procedure. However the linkage between the <schedule> resource and the AE does not exist.

#### 6.9.3.16	updateReachabilitySchedule

##### 6.9.3.16.1	Description

This service capability provides the ability to update an AE's reachability schedule information. This service capability shall be restricted to the Msc and Mca Reference Points.

##### 6.9.3.16.2	Pre-Conditions

Not Applicable.

##### 6.9.3.16.3	Signature –updateReachabilitySchedule

Table 6.9.3.16.3-1: Service Subscription Administration –updateReachabilitySchedule capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| aeId | IN | NO | The Application Entity Identifier (AE-ID). |
| reachabilitySchedule | IN | YES | The contact reachability schedule information of the AE associated with the device node. The absence of this parameter implies the AE associated with the device node is always contact reachable. Type Schedule, see 6.9.2.7.1. |
| responseType | OUT | YES | Unique response types for this service. AE successfully updated |

##### 6.9.3.16.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the updateReachabilitySchedule capability

Figure 6.9.3.16.4-1: updateReachabilitySchedule Diagram

##### 6.9.3.16.5	Post-Conditions

When parameters are present, the reachability schedule is updated for the AE.

##### 6.9.3.16.6	Exceptions

Not Applicable.

##### 6.9.3.16.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.3.16.8	oneM2M Resource Interworking

This service capability is used to update AE's reachability schedule information. The service capability aligns with the <schedule> resource and maps to the UPDATE procedure. The reference between an AE and a <schedule> resource does not exist as an oneM2M Resource attribute.

#### 6.9.3.17	getReachabilitySchedule

##### 6.9.3.17.1	Description

This service capability provides the ability to get an AE's reachability schedule information. This service capability shall be restricted to the Msc and Mca Reference Points.

##### 6.9.3.17.2	Pre-Conditions

Not Applicable.

##### 6.9.3.17.3	Signature – getReachabilitySchedule

Table 6.9.3.17.3-1: Service Subscription Administration – getReachabilitySchedule capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| aeId | IN | NO | The Application Entity Identifier (AE-ID). |
| reachabilitySchedule | OUT | YES | The contact reachability schedule information of the AE associated with the device node. The absence of this parameter implies the AE associated with the device node is always contact reachable. Type Schedule, see 6.9.2.7.1. |
| responseType | OUT | YES | Unique response types for this service. not successfully |

##### 6.9.3.17.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the getReachabilitySchedule capability

Figure 6.9.3.17.4-1: getReachabilitySchedule Diagram

##### 6.9.3.17.5	Post-Conditions

When parameters are present, the reachability schedule is gotten from the AE.

##### 6.9.3.17.6	Exceptions

Not Applicable.

##### 6.9.3.17.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.3.17.8	oneM2M Resource Interworking

This service capability is used to get an AE's reachability schedule information. The service capability aligns with the <schedule> resource and maps to the RETRIEVE procedure. The reference between an AE and a <schedule> resource does not exist as an oneM2M Resource attribute.

#### 6.9.3.18	deleteReachabilitySchedule

##### 6.9.3.18.1	Description

This service capability provides the ability to delete a reachability schedule for an AE. This service capability shall be restricted to the Msc and Mca Reference Points.

##### 6.9.3.18.2	Pre-Conditions

Not Applicable.

##### 6.9.3.18.3	Signature – deleteReachabilitySchedule

Table 6.9.3.18.3-1: Service Subscription Administration – deleteReachabilitySchedule capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| aeId | IN | NO | The Application Entity Identifier (AE-ID). |
| responseType | OUT | YES | Unique response types for this service. AE successfully deleted |

##### 6.9.3.18.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the deleteReachabilitySchedule capability

Figure 6.9.3.18.4-1: deleteReachabilitySchedule Diagram

##### 6.9.3.18.5	Post-Conditions

When parameters are present, the reachability schedule is deleted from the AE.

##### 6.9.3.18.6	Exceptions

Not Applicable.

##### 6.9.3.18.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.9.3.18.8	oneM2M Resource Interworking

This service capability is used to delete an AE's reachability schedule information. The service capability aligns with the <schedule> resource and maps to the DELETE procedure. The reference between an AE and a <schedule> resource does not exist as an oneM2M Resource attribute.

## 6.10	Event Collection

### 6.10.1	Overview

The Event Collection service provides the capability to record events for accounting purposes.

### 6.10.2	Event Collection Entities

#### 6.10.2.1	Event Collection Types

##### 6.10.2.1.1	Event Collection Record

Table 6.10.1.1-1: Type: Event Record Template

| Information Element | Optional | Description |
| --- | --- | --- |
| eventRecordID | NO | It is the unique ID that identifies a specific event record. |
| eventCollectionTriggerID | NO | It is the unique ID that identifies a specific event collection triggering scenario, which triggers information recording for a specific event. |
| collectingEntityID | NO | This is the unique ID of the entity that collects the statistics. It can be an AE-ID or M2M Service Capability Identifier (M2M-Serv-Cap-ID) |
| collectedEntityID | NO | This is the unique ID of the entity whose service layer operation statistics are being collected. It can be an AE-ID or M2M Service Capability Identifier (M2M-Serv-Cap-ID). |
| serviceProfileID | YES | An M2M Service Profile Identifier defines M2M Services and/or M2M Service Roles and AEs applicable to M2M Nodes. |
| eventDescription | NO | Describes the event triggered at the service. It is a subset of the eventConfig data structure. The specific structure depends on the specific event defined by the Event Collection Trigger. |
| Vendor Specific Information | YES | Defines Vendor specific information |

##### 6.10.2.1.2	Event Configuration Type

Table 6.10.2.1.2-1 Type: Event Collection – eventConfig

| Parameter name | Optional | Description |
| --- | --- | --- |
| eventType | NO | This attribute indicates the type of the event, such as "subscribeComplete", "downloadFirmware ", "timer based",, "storage based", etc. The eventType can be a combination of multiple sub-types, for example, it can be based on both "subscribeComplete" and a timer. |
| serviceRoleIds | YES | Defines the specific service role. |
| eventStart | YES | This attribute indicates the start time of the event. It is mandatory if the eventType includes "timer based". |
| eventEnd | YES | This attribute indicates the end time of the event. It is mandatory if the eventType includes "timer based". |
| dataSize | YES | This attribute defines the data size if an event is triggered when the stored data exceeds a certain size. It is mandatory if the eventType is "storage based". |

##### 6.10.2.1.3	getEventCollectionPolicy Filter Criteria

Table 6.10.2.1.3-1: getEventCollectionPolicy Filter Criteria

| Criterion name | Description |
| --- | --- |
| eventType | See Table 6.10.2.1.2-1 |
| eventStart | See Table 6.10.2.1.2-1. This criterion may only be used when the eventType criterion is set to "timer based". |
| eventEnd | See Table 6.10.2.1.2-1. This criterion may only be used when the eventType criterion is set to "timer based". |
| dataSize | See Table 6.10.2.1.2-1. This criterion may only be used when the eventType criterion is set to "storage based". |

##### 6.10.2.1.4 	getEventCollectionTriggers Filter Criteria

Table 6.10.2.1.4-1: getEventCollectionTriggers Filter Criteria

| Criterion name | Description |
| --- | --- |
| eventCollectionPolicyID | See Table 6.10.3.3.2-1. |
| collectingEntityID | See Table 6.10.3.3.2-1. |
| collectedEntityID | See Table 6.10.3.3.2-1. |
| eventReceiverID | See Table 6.10.3.3.2-1. |
| serviceID | See Table 6.10.3.3.2-1. |
| triggerStatus | See Table 6.10.3.3.2-1. |

##### 6.10.2.1.5 	getEventRecords Filter Criteria

Table 6.10.2.1.5-1: getEventRecords Filter Criteria

| Criterion name | Description |
| --- | --- |
| eventCollectionTriggerID | See Table 6.10.1.1-1. |
| collectingEntityID | See Table 6.10.1.1-1. |
| collectedEntityID | See Table 6.10.1.1-1. |
| serviceProfileID | See Table 6.10.1.1-1. |
| eventDescription | See Table 6.10.1.1-1. In the filterCriteria, the eventDescription can be a string that partially matches the eventDescription parameter in the record. |

### 6.10.3	Service Capabilities

#### 6.10.3.1	setEventCollectionPolicy

##### 6.10.3.1.1	Description

This service capability provides the ability for AEs to configure event collection policies for statistics and charging purposes. This service capability applies to the Mca Reference Point.

##### 6.10.3.1.2	Pre-Conditions

The Originator can be an AE, and the Originator wants to configure event collection policies at a receiving service capability. The receiving service capability conducts event collection according to the policies. The receiving service capability may establish its own event collection policies.

The Originator, if different from the receiving service capability, is subscribed and registered to the receiving service capability.

##### 6.10.3.1.3	Signature – setEventCollectionPolicy

Table 6.10.3.1.3-1: Event Collection – setEventCollectionPolicy capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| serviceId | IN | NO | The M2M Service Identifier (M2M-Serv-ID) |
| eventConfig | IN | NO | The configuration of the event collection policy. Defined in Table 6.10.2.1.2-1. |
| eventCollectionPolicyID | OUT | NO | The receiving service capability generates a unique policy ID for each policy. |
| responseType | OUT | YES | Unique response types for this service. Exception: Policy already exists |

##### 6.10.3.1.4	Post-Conditions

Not Applicable.

##### 6.10.3.1.5	Exceptions

The Originator has no access right to create the policy.

##### 6.10.3.1.6	Policies for Use

Message Exchange Patterns: In-Out

##### 6.10.3.1.7	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.10.3.1.7-1: setEventCollectionPolicy Diagram

##### 6.10.3.1.8	oneM2M Resource Interworking

The service capability aligns with the <eventConfig> resource and maps to the CREATE procedure for the resource.

#### 6.10.3.2	getEventCollectionPolicy

##### 6.10.3.2.1	Description

This service capability provides the ability for entities, such as an AE or a service capability, to retrieve the existing policies stored at a service capability. This service capability applies to the Mca and Msc Reference Point.

##### 6.10.3.2.2	Pre-Conditions

The Originator can be an AE or a service capability, and the Originator wants to retrieve event collection policies at a receiving service capability.

The Originator, if different from the receiving service capability, is subscribed and registered to the receiving service capability. The Originator is only allowed to retrieve with the proper access right.

##### 6.10.3.2.3	Signature – getEventCollectionPolicy

Table 6.10.3.2.3-1: Event Collection – getEventCollectionPolicy capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| eventCollectionPolicyID | IN | NO | See Table 6.10.3.1.2-2. The eventCollectionPolicyID specifies the specific event collection policy to retrieve, if the Originator has such information. If the Originator does not have the information regarding eventCollectionPolicyID, it indicates eventCollectionPolicyID with a specific value that represents "any IDs", and uses the filterCriteria to filter selected policies. |
| filterCriteria | IN | YES | See Table 6.10.2.1.3-1. |
| responseType | OUT | YES | Unique response types for this service. Exception: eventCollectionPolicyID does not exist |

##### 6.10.3.2.4	Post-Conditions

Not Applicable.

##### 6.10.3.2.5	Exceptions

The Originator has no access right to retrieve the policy

##### 6.10.3.2.6	Policies for Use

Message Exchange Patterns: In-Out

##### 6.10.3.2.7	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.10.3.2.7-1: getEventCollectionPolicy Diagram

##### 6.10.3.2.8	oneM2M Resource Interworking

The service capability aligns with the <eventConfig> resource and maps to the RETRIEVE procedure for the resource.

#### 6.10.3.3	setEventCollectionTriggers

##### 6.10.3.3.1	Description

This service capability provides the ability for AEs and service capabilities to configure specific triggers for event collection, based on the Event Collection Policy. This service capability applies to the Mca and Msc Reference Point.

##### 6.10.3.3.2	Pre-Conditions

The Originator can be an AE or a service capability, and the Originator wants to configure event collection triggers based on existing event collection policy available at the collecting service capability.

##### 6.10.3.3.3	Signature - setEventCollectionTriggers

Table 6.10.3.3.3-1: Event Collection – setEventCollectionTriggers capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| eventCollectionPolicyID | IN | NO | This attribute specify the policy used to define the specific event collection trigger. |
| collectingEntityID | IN | NO | This is the ID of the entity where the event is being collected. If not specified, the default collecting entity is the receiving service capability. |
| collectedEntityID | IN | NO | This is the ID of the entity that triggered the event collection. If not specified, the default is all entities triggered the event at the collecting entity. |
| eventReceiverID | IN | YES | In cases when there is another entity, other than the collecting entity (such as a service capability), wants to receive the collected event, this attribute provides the unique ID of this entity. |
| serviceRoleIds | IN | YES | Specifies the service roles that trigger the event collection. |
| serviceId | IN | YES | Specifies the services that trigger the event collection. |
| triggerStatus | IN | YES | This attribute indicates if the trigger is activated or not. If not defined, the trigger is activated. |
| eventCollectionTriggerID | OUT | NO | The receiving service capability generates a unique ID for each event collection trigger. |
| responseType | OUT | YES | Unique response types for this service. Exception: eventCollectionPolicyID is invalid collectingEntityID is invalid collectedEntityID is invalid eventReceiverID is invalid triggerStatus is invalid |

##### 6.10.3.3.4	Post-Conditions

After the successful creation of event collection triggers, when the defined event happens at the collectingEntity, and when the event collection trigger status is ACTIVE, the collectingEntity shall collect the event. The supporting service shall send a recordEvent message to the Event Collection entity.

##### 6.10.3.3.5	Exceptions

The Originator has no access right to create the event collection triggers.

##### 6.10.3.3.6	Policies for Use

Message Exchange Patterns: In-Out

##### 6.10.3.3.7	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.10.3.3.7-1: setEventCollectionTriggers Diagram

##### 6.10.3.3.8	oneM2M Resource Interworking

The service capability aligns with the <statsCollect> resource and maps to the CREATE procedure for the resource.

#### 6.10.3.4	getEventCollectionTriggers

##### 6.10.3.4.1	Description

This service capability provides the ability for AEs and service capabilities to retrieve event collection triggers at a receiving service capability. This service capability applies to the Mca and Msc Reference Point.

##### 6.10.3.4.2	Pre-Conditions

Originating AEs and service capabilities are subscribed and registered to the receiving service capability. Originator has the access right to retrieve.

##### 6.10.3.4.3	Signature – getEventCollectionTriggers

Table 6.10.3.4.3-1: Event Collection – getEventCollectionTriggers capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| eventCollectionTriggerID | IN | YES | This attribute specifies which event collection triggers to retrieve. If not specified, any event collection trigger matching the filter criteria should be retrieved. If the Originator does not have the information regarding eventCollectionTriggerID, it indicates with a specific value that represents "any IDs", and uses the filterCriteria to filter selected event triggers. |
| filterCriteria | IN | YES | See Table 6.10.2.1.4-1. |
| responseType | OUT | YES | Unique response types for this service. Exception: eventCollectionTriggerID does not exist |

##### 6.10.3.4.4	Post-Conditions

Not Applicable.

##### 6.10.3.4.5	Exceptions

The Originator has no access right to retrieve the event collection triggers.

##### 6.10.3.4.6	Policies for Use

Message Exchange Patterns: In-Out

##### 6.10.3.4.7	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue the request to the Supporting Service to perform the operation

Figure 6.10.3.4.7-1: getEventCollectionTriggers Diagram

##### 6.10.3.4.8	oneM2M Resource Interworking

The service capability aligns with the <statsCollect> resource and maps to the RETRIEVE procedure for the resource.

#### 6.10.3.5	recordEvent

##### 6.10.3.5.1	Description

This service capability provides the ability for a service (such as Data Exchange service) to trigger the event collection service to record an event. This service applies to the Msc Reference Point.

##### 6.10.3.5.2	Pre-Conditions

Event Collection Triggers have been created by the setEventCollectionTriggers capability.

##### 6.10.3.5.3	Signature - recordEvent

Table 6.10.3.5.3-1: Event Collection – recordEvent capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| eventCollectionPolicyID | IN | YES | This attribute specify the policy used to define the specific event collection trigger. |
| eventCollectionTriggerID | IN | NO | The service where the trigger happens provides the eventCollectionTriggerID being used. |
| eventDescription | IN | NO | This attribute describes the event triggered at the service. It is a subset of the eventConfig data structure. It contains one or more parameters defined in the eventConfig data type. |
| recordID | OUT | NO | The event collection service generates a unique ID. |
| responseType | OUT | YES | Unique response types for this service. Exception:. Event recording cannot be completed |

##### 6.10.3.5.4	Post-Conditions

Not Applicable.

##### 6.10.3.5.5	Exceptions

Not Applicable.

##### 6.10.3.5.6	Policies for Use

Message Exchange Patterns: In-Out

##### 6.10.3.5.7	Service Interactions

The request comes from the service where the event was triggered to the Event Collection entity.

Figure 6.10.3.5.7-1: recordEvent Diagram

##### 6.10.3.5.8	oneM2M Resource Interworking

Not Applicable.

#### 6.10.3.6	getEventRecords

##### 6.10.3.6.1	Description

This service capability provides the ability for AEs and service capabilities to retrieve the recorded events for statistical or charging purposes. This service capability applies to the Mca and Msc Reference Point.

##### 6.10.3.6.2	Pre-Conditions

Originating AEs and service capabilities are subscribed and registered to the receiving service capability.

##### 6.10.3.6.3	Signature – getEventRecords

Table 6.10.3.6.3-1: Event Collection – getRecords capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| eventRecordID | IN | NO | This attribute specifies which record to retrieve. If not specified, any event records matching the filter criteria should be retrieved. If the Originator does not have the information regarding the eventRecordID, it indicates with a specific value that represents "any IDs", and use the filterCriteria to filter selected records of events. |
| filterCriteria | IN | YES | See Table 6.10.2.1.5-1 |
| responseType | OUT | YES | Unique response types for this service. Exception: The eventRecordID |

##### 6.10.3.6.4	Post-Conditions

Not Applicable.

##### 6.10.3.6.5	Exceptions

Not Applicable.

##### 6.10.3.6.6	Policies for Use

Message Exchange Patterns: In-Out

##### 6.10.3.6.7	Service Interactions

The Originator sends the request to the Event Collection entity to obtain the event records that it is

interested in.

Figure 6.10.3.6.7-1: getEventRecords Diagram

##### 6.10.3.6.8	oneM2M Resource Interworking

Not Applicable.

## 6.11	Registration

### 6.11.1	Overview

The Registration service provides the ability for:

Initial Registration of an AE

Refresh an existing Registration of an AE

AE initiated termination of an existing Registration

Removal of subscriptions to AE Registration events

### 6.11.2	Service Capabilities

#### 6.11.2.1	registerAE

##### 6.11.2.1.1	Description

This service capability enables an AE with the proper authorization to register with the M2M System. This service capability shall be restricted across the Mca Reference Point.

##### 6.11.2.1.2	Pre-Conditions

The AE has not registered with the M2M System. An AE identifier may have been already allocated to the registering AE and acquired by the AE by means outside the scope of this specification.

##### 6.11.2.1.3	Signature – registerAE

Table 6.11.2.1.3-1: Registration – registerAE capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| pointOf Access | IN | NO | The point of Access of the registered AE. |
| applicationId | IN | NO | The Application Identifier (App-ID). |
| credentialId | IN | YES | Application Credential-ID used to identify the Application Entity for the Application. It is used to retrieve the corresponding M2M Service Subscription. |
| expirationTime | IN | NO | The expiration time of the registration as requested by the Originator. |
| reachabilitySchedule | IN | YES | The contact reachability schedule information of the AE associated with the device node. The absence of this parameter implies the AE associated with the device node is always contact reachable. Type Schedule, see 6.9.2.7.1. |
| aeId | IN-OUT | Optional for IN only-Not Optional for OUT | Application Entity (AE-ID) is provided back in the response. |
| responseType | OUT | NO | Unique response types for this service: AE successfully registered AE not successfully registered M2M Service Subscription does not exist Invalid Application Credential Identifier. |

##### 6.11.2.1.4	Services Interaction

The interactions of service capabilities required for this service capability:

- Associate the AE with the M2M Service Subscription

- Issue the request to the Supporting Service.

- Send a notification to the Device Onboarding Supporting Service for a first-time application registration in the event there is a subscription of the first contact event by the Supporting Service.

Figure 6.11.2.1.4-1: Registration – registerAE Diagram

##### 6.11.2.1.5	Post-Conditions

AE is registered and can start using service capabilities

The returned AE-ID shall be used in any subsequent operation related to that application

##### 6.11.2.1.6	Exceptions

Not Applicable.

##### 6.11.2.1.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.11.2.1.8	oneM2M Resource Interworking

This service capability maps to the CREATE procedure of the <AE> resource.

#### 6.11.2.2	refreshAERegistration

##### 6.11.2.2.1	Description

This service capability enables an AE to refresh an existing AE registration with the M2M System. This service capability shall be restricted to the Mca Reference Points.

##### 6.11.2.2.2	Pre-Conditions

The AE has successfully registered with the M2M System.

##### 6.11.2.2.3	Signature – refreshAERegistration

Table 6.11.2.2.3-1: Registration - refreshAERegistration capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| aeId | IN | NO | The Application Entity Identifier (AE-ID) of the request originator. |
| pointOfAccess | IN | YES | The point of Access of the registered AE. POA is optional only if identical to the one in the refreshed registration |
| expirationTime | IN | NO | The expiration time of the registration as requested by the Originator. |
| reachabilitySchedule | IN | YES | The contact reachability schedule information of the AE associated with the device node. The absence of this parameter implies the AE associated with the device node is always contact reachable. Type Schedule, see 6.9.2.7.1. |
| responseType | OUT | NO | Unique response types for this service: AE successfully refreshed Registration does not exist |

##### 6.11.2.2.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Issue a request to refresh the AE association with the M2M Service Profile

- Issue the request to the Supporting Service

Figure 6.11.2.2.4-1: Registration – refreshAERegistration Diagram

##### 6.11.2.2.5	Post-Conditions

The Application Entity registration is refreshed and can continue using service capabilities.

##### 6.11.2.2.6	Exceptions

Not Applicable.

##### 6.11.2.2.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.11.2.2.8	oneM2M Resource Interworking

This service capability maps to the UPDATE procedure of the <AE> resource.

#### 6.11.2.3	deregisterAE

##### 6.11.2.3.1	Description

This service capability enables an AE to deregister from the M2M System. This service capability shall be restricted to the Mca Reference Points.

##### 6.11.2.3.2	Pre-Conditions

The AE successfully registered with the M2M System.

##### 6.11.2.3.3	Signature - deregisterAE

Table 6.11.2.3.3-1: Registration – deregisterAE capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| aeId | IN | NO | The Application Identifier (AE-ID) |
| responseType | OUT | NO | Unique response types for this service: AE successfully De-registered Registration does not exist |

##### 6.11.2.3.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Disassociate the AE with Service Subscription

- Issue the request to Supporting Services.

- Send a notification to device onboarding service supporting service for application de-registration in the event there is a subscription

Figure 6.11.2.3.4-1: Registration – deregisterAE Diagram

##### 6.11.2.3.5	Post-Conditions

AE is no longer registered and cannot use any services.

##### 6.11.2.3.6	Exceptions

Not Applicable.

Transaction Pattern: Participation allowed

##### 6.11.2.3.7	oneM2M Resource Interworking

This service capability maps to the DELETE procedure of the <AE> resource.

## 6.12	Registration Administration

### 6.12.1	Overview

The Registration administration service provides the ability for:

Retrieve the AE's Registration status

Revoke an AE existing Registration

Permit subscriptions to AE Registration events

### 6.12.2	Service Capabilities

#### 6.12.2.1	getRegistrationStatus

##### 6.12.2.1.1	Description

This service capability enables a Supporting Service authorized for this operation, to retrieve the registration status for an AE. This service capability is restricted to the Msc Reference Point.

##### 6.12.2.1.2	Pre-Conditions

Not Applicable.

##### 6.12.2.1.2	Signature - getRegistrationStatus

Table 6.12.2.1.2-1: Registration Administration – getRegistrationStatus capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| aeId | IN | NO | The Application Entity Identifier (AE-ID) |
| responseType | OUT | NO | Unique response types for this service: AE is registered AE registration does not exist |

##### 6.12.2.1.4	Service Interaction

The interactions of service capabilities required for this service capability:

- Retrieve the Application Entity information

Figure 6.12.2.1.4-1: Registration Administration – getRegistrationStatus Diagram

##### 6.12.2.1.5	Post-Conditions

Not Applicable.

##### 6.12.2.1.6	Exceptions

Not Applicable.

##### 6.12.2.1.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.12.2.1.8	oneM2M Resource Interworking

This service capability maps to the RETRIEVE procedure of the <AE> resource.

#### 6.12.2.2	revokeAERegistration

##### 6.12.2.2.1	Description

This service capability enables a Supporting Service authorized for this operation to revoke an existing AE registration. This service capability shall be restricted to the Msc Reference Points.

##### 6.12.2.2.2	Pre-Conditions

The AE is successfully registered with the M2M System.

##### 6.12.2.2.3	Signature - revokeAERegistration

Table 6.12.2.2.3-1: Registration Administration – revokeAERegistration capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| aeId | IN | NO | Application Entity Identifier (AE-ID) |
| responseType | OUT | NO | Unique response types for this service: AE successfully De-registered Registration does not exist |

##### 6.12.2.2.4	Service Interaction

The interactions of service capabilities required for this service capability:

- Issue a request to the Service Subscription Administration to fetch the service profile.

- Disassociate the AE from the M2M Service Subscription

Figure 6.12.2.2.4-1: Registration Administration – revokeAERegistration Diagram

##### 6.12.2.2.5	Post-Conditions

The Application Entity is no longer registered and cannot use any services

##### 6.12.2.2.6	Exceptions

Not Applicable.

##### 6.12.2.2.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.12.2.2.8	oneM2M Resource Interworking

This service capability maps to the DELETE procedure of the <AE> resource.

#### 6.12.2.3	subscribeInitialAERegistrationEvent

##### 6.12.2.3.1	Description

This service capability enables subscription to the initial (first time only) registration of an AE. This capability shall be restricted to the Msc Reference Point.

##### 6.12.2.3.2	Pre-Conditions

Not Applicable.

##### 6.12.2.3.3	Signature - subscribeInitialAERegistrationEvent

6.12.2.3.3-1: Registration Administration – subscribeInitialAERegistrationEvent capability

| Parameter name | Direction |  | Optional | Description |
| --- | --- | --- | --- | --- |
| subscriptionId |  | OUT | NO | Return subscriptionId in case of a success. |
| responseType |  | OUT | NO | Unique response types for this service: Successfully subscribed Not successfully subscribed |

##### 6.12.2.3.4	Service Interaction

The interactions of service capabilities required for this service capability:

- Subscribe the requesting AE for the initial registration event.

Figure 6.12.2.3.4-1: Registration Administration – subscribeInitialAERegistrationEvent Diagram

##### 6.12.2.3.5	Post-Conditions

A Subscription is created for the Originator to receive initial contact events.

##### 6.12.2.3.6	Exceptions

Not Applicable.

##### 6.12.2.3.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.12.2.3.8	oneM2M Resource Interworking

Not Applicable.

#### 6.12.2.4	unsubscribeInitialAERegistrationEvent

##### 6.12.2.4.1	Description

This service capability enables a Supporting Service for this operation to remove an existing subscription against an AE initial registration. This capability shall be restricted to the Msc Reference Point.

##### 6.12.2.4.2	Pre-Conditions

Not Applicable.

##### 6.12.2.4.3	Signature – unsubcribeInitialAERegistrationEvent

Table 6.12.2.4.3-1: unsubscribeInitialAERegistrationEvent capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| subscriptionId | IN | NO | The subscriptionId for the AE initial registration event. |
| responseType | OUT | NO | Unique response types for this service: - Successfully unsubscribed - Subscription does not exist |

##### 6.12.2.4.4	Service Interaction

The interactions of service capabilities required for this service capability:

- Remove the subscription

Figure 6.12.2.4.4-1: Registration Administration – unsubscribeInitialAERegistrationEvent Diagram

##### 6.12.2.4.5	Post-Conditions

Subscription to initial registration event is removed.

##### 6.12.2.4.6	Exceptions

Not applicable.

##### 6.12.2.4.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

##### 6.12.2.4.8	oneM2M Resource Interworking

Not applicable.

# 7	M2M Service Components

## 7.1	Introduction

This clause describes the M2M Service Components provided by the M2M Services Platform.

An M2M Service Component defines the set of M2M Services that are related in the sense that the M2M Services that comprise the M2M Service Component are typically deployed together in order to support one or more Supporting Services.

The M2M Service Architecture augments the oneM2M Functional Architecture by specifying M2M Services provided to M2M Service Providers. These M2M Services are exposed to the AE across the Mca Reference Point as well as other Service Components across the Msc Reference Point.

## 7.2	Service Component Interaction Cross Reference

Table 7.1.1-1: Service Component Cross Reference

## 7.3	Infrastructure Component (INF)

### 7.3.1	introduction

The INF Component provides services that are exposed to other M2M Service Components across the Msc Reference Point. The Services within the INF Component are common and utility type Services needed to enable other M2M Services.

### 7.3.2	INF to Service Cross Reference

Authorization: All Service Capabilities

Registration Services: All Service Capabilities

## 7.4	Service Subscription Component (SSUB)

### 7.4.1	Introduction

The SSUB Component provides services that are necessary to maintain the M2M Service, the associations (e.g. Services Roles) to the M2M Service, M2M Service Subscriptions, the associations (e.g. AE, M2M Nodes) to the M2M Service Subscription and any external identifiers associated with AE and M2M Nodes. In addition the services used to manage devices and AEs are part of this component.

### 7.4.2	SSUB to Service Cross Reference

Service Subscription: All Service Capabilities

Service Administration: All Service Capabilities

Service Subscription Administration: All Service Capabilities

## 7.5	Transport Adapter (TRA)

### 7.5.1	Introduction

The TRA component provides services are used to integrate or adapt Data Exchange requests to the underlying Data Exchange transport (e.g. XMPP or MQTT Broker).

### 7.5.2	TRA to Service Cross Reference

Broker: All Service Capabilities

## 7.6	Accounting (ACC)

### 7.6.1	Introduction

The ACC Component provides services that are record events necessary to charge for the Services provided to AEs.

### 7.6.2	Accounting to Service Cross Reference

Event Collection: All Service Capabilities

## 7.7	Service Exposure (SE)

### 7.7.1	Introduction

The SE Component acts as the primary interface for AEs to interact with the M2M Service Layer across the Mca Reference Point.

### 7.7.2	Service Exposure to Service Cross Reference

Data Exchange: subscribe, publish, sendMessage

Service Subscription Administration: getServiceSubscription, addNodeToServiceSubscription, deleteNodesFromServiceSubscription, getNodesForServiceSubscription, updateNodeForServiceSubscription, addApplicationRule, deleteApplicationRules, getApplicationRules, updateApplicationRule, addReachabilitySchedule, updateReachabilitySchedule, getReachabilitySchedule, deleteReachabilitySchedule

Device Management: downloadFirmware, installFirmware, getFirmwareInformation, getFirmwareExecStatus, upgradeFirmware, getDeviceInformation, getDeviceCapabilities, enableDeviceCapability, disableDeviceCapability, getAreaNetworks, updateDeviceForAreaNetwork, rebootDevice, resetDevice, uploadDeviceLog, getDeviceLogs, getDeviceLogInformation, getSoftwareInformation, downloadSoftware, installSoftware, activateSoftware, deactivateSoftware, removeSoftware

Event Collection: setEventCollectionPolicy, getEventCollectionPolicy, setEventCollectionTriggers, getEventCollectionTriggers, getEventRecords

Registration Services: registerAE, refreshAERegistration, deregisterAE

## 7.8	Management Adapter (MA)

### 7.8.1	Introduction

The Management Adapter component provides services that are used to integrate or adapt Device Management requests to the Management Servers of existing device management technologies(e.g. TR-069, OMA-DM, and LWM2M).

### 7.8.2	Management Adapter to Service Cross Reference

Management Adapter: All Service Capabilities

Device Management: deviceManagementReport

## 7.9	Device Management (DM)

### 7.9.1	Introduction

The Device Management component provides services to manage devices without consideration of the technology of Underlying Network.

### 7.9.2	Device Management to Service Cross Reference

Device Management: All Service Capabilities

# Annex A (informative): Common Request Processing

## A.1	Overview

This annex illustrates the usage of the common services described in this technical specification for implementation requests received from AEs across the Mca Reference Point.

## A.2	Mca Common Request Processing

### A.2.1	Introduction

AEs communicate with the M2M System over the Mca Reference Point through the SE component. The SE component accepts the request from the AE and provides the request to the Supporting Service in order allow the M2M Service Provider to ensure the request is properly authorized and recorded. In addition, the M2M Service Provider is able to orchestrate other capabilities that have not been specified.

### A.2.2	Mca Common Request Parameters (normative)

Each request that is received by the SE component across the Mca Reference Point shall have a common set of input parameters as described in Table A.2.2-1 Mca: Common Request Input Parameters.

Table A.2.2-1: Mca: Common Request Input Parameters

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| from | IN | NO | The identifier of the Originator of the request (AE-ID) |
| requestId | IN-OUT | YES | The request identifier. When supplied by the Originator, the same value is returned in the response if the response is applicable for the message exchange pattern. If not supplied by the Originator, the M2M System supplied a globally unique value in the response if the response is applicable for the message exchange pattern. |
| operation | IN | NO | The signature of the operation that is received or transmitted across the Mca reference point. |

Service capabilities in the SE component whose message exchange patterns require a response shall have a common set of output parameters as described in Table A.2.2-2 Mca: Common Request Output Parameters.

Table A.2.2-2: Mca: Common Request Output Parameters

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| responseType | OUT | YES | Response types that are relevant to requests from AEs across the Mca Reference Point. |

In addition to the parameters in Table A.2.2-1, each request that addresses a target AE (e.g. Data Exchange) that is received by the SE component across the Mca Reference Point shall have a common set of input parameters as described in Table A.2.2-3 Mca: Common Request Input Parameters For Target AEs.

Table A.2.2-3: Mca: Common Request Input Parameters For Target AEs

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| to | IN | NO | The identifier of the AE that is the target of the request (AE-ID) |

### A.2.3	Authentication and Authorization of Requests

#### A.2.3.1	Description

Upon reception of requests across the Mca reference, the request Originator is authenticated and the requested M2M Service Capability is authorized when the:

Request Originator has a M2M Service Subscription for the requested operation.

Request Originator is authorized for the requested operation.

#### A.2.3.2	Pre-Conditions

The external identifiers for the from parameter's AE-ID are assigned to a M2M Service Subscription (M2M-Service-Profile-ID).

A correlation between the service capability and the M2M authorization event has been defined in order to authorize the service capability.

#### A.2.3.2	Common M2M Service Capability Parameters for Request Authentication and Authorization

Table A.2.2.2-1: Common M2M Service Capability Parameters for Request Authentication and Authorization

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Mca common request input parameters | IN | NO | The Mca common request input parameters defined in Table A.2.2-1. |
| responseType | OUT | YES | Response types that are relevant to the Authentication and Authorizations of requests from AEs across the Mca Reference Point. Originator is not authenticated Originator does not have a M2M Service Subscription Originator not authorized for the M2M Service Capability |

#### A.2.3.3	Service Interactions

The interactions of service capabilities required to authenticate and authorize a request:

- Obtain the M2M Service Capability from the requested operation

- Obtain the Application Entity and associated M2M Service Subscription

- Validate the M2M Service Subscription for the Service Capability.

- Authorize the use of the M2M Service Capability

Figure A.2.3.3-1: Mca Common Request Authorization Diagram

#### A.2.3.4	Post-Conditions

Success case: The request is permitted.

Failure case: The request is not permitted and an optional response type is transmitted back the request Originator.

#### A.2.3.5	Exceptions

Not Applicable.

#### A.2.3.6	Policies for Use

Message Exchange Patterns: In-Out, In-Only, Robust In-Only, In-Optional-Out

Transaction Pattern: Participation allowed

## A.3	Msc Common Request Processing

### A.3.1	Introduction

Service Components communicate with each other across the Msc Reference Point. Additional input or output parameters are not necessary when invoking service capabilities across the Msc Reference Point. As such signature of the service capability is sufficient for implementing the service capability in clause 6 when the capability is invoked across the Msc Reference Point.

### A.3.2	Msc Common Request Data Types (normative)

Each request that is received by the M2M Component across the Msc Reference Point shall have a common set of input parameters as described in Table A.3.1-1 Msc: Common Request Input Parameters.

Table A.3.2-1: Msc: Common Request Input Parameters

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| from | IN | NO | The identifier of the Originator of the request. This is typically the Service Capability identifier of the M2M System. |
| requestId | IN-OUT | NO | The request identifier. When supplied by the Originator, the same value is returned in the response if the response is applicable for the message exchange pattern. If not supplied by the Originator, the M2M System supplied a globally unique value in the response if the response is applicable for the message exchange pattern. |

## A.4	Common Entity Attributes

### A.4.1	Introduction

Information Entities (e.g. M2M Service Subscription) that are maintained using the M2M Service Capabilities have a set of common attributes that are associated with the entity when the information entity is created, updated and retrieved.

### A.4.2	System Generated Attributes (normative)

When an information entity is created or updated the system shall update the common attributes as described in Table A.4.2-1. When an information entity is retrieved the system shall return the common attributes as described in Table A.4.2-1.

Table A.4.2-1: System Generated Attributes

| Parameter name | Description |
| --- | --- |
| creationTime | The date and time that the system created the information entity. |
| lastModifiedTime | The date and time that the system modified the information entity. The system modifies this attribute when the information entity is created or updated. |

### A.4.3	User Defined Discovery Attributes (normative)

Information entities are retrieved through the system's service capabilities. Some service capabilities allow for discovery of information elements using user defined discovery attributes.

Table A.4.3-1: User Defined Discovery Attributes

| Parameter name | Description |
| --- | --- |
| labels | The list of labels that a user can add or modify in creation and update requests for information entities. When a service capability is used to retrieve a list of information elements, the service capability shall have this attribute as a common request input attribute. |

## A.5	Using Filter Criteria

Service Capabilities have the ability to define a filter criteria entity as part of the Service Capability's input parameters. Filter criteria within the system shall be evaluated using the following rules:

different elements of the filter criteria entity shall use the "AND" logical operation

same elements of the filter criteria entity shall use the "OR" logical operation

# Annex B (informative): Data Exchange Services

## B.1	Overview

This annex illustrates the usage of the services for requests to exchange data from AEs across the Mca Reference Point.

## B.2	Supporting Services

### B.2.1	Subscribe-Publish-Notify Message Exchange

#### B.2.1.1	Overview

This service provides the capability to exchange data using the publish and subscribe message patterns where AEs:

Request to receive payloads from a publication resource by subscribing to the publication resource.

Request to publish a payload to a resource.

#### B.2.1.2	Service Capabilities

##### B.2.1.2.1	subscribeRequest

###### B.2.1.2.1.1	Description

This service capability provides the ability to validate a subscription request from an AE.

###### B.2.1.2.1.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a M2M Service Subscription, subscribing AE, publication resource and Broker exist.

###### B.2.1.2.1.3	Signature – subscribeRequest

Table B.2.1.2.1.3-1: Data Exchange Service – subscribeRequest capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input parameters | - | - | See Table A.2.2-3 |
| publicationResource | IN | NO | The publication resource. See 6.4.2.1.1.1 |
| deliveryPolicy | IN | YES | The delivery policy when notifying the subscriber. See 6.4.2.1.1.2 |
| retainmentPolicy | IN | YES | The retainment policy for unconnected subscribers. See 6.4.2.1.1.3 |
| transportAdapter | OUT | YES | The instance of the transport adapter service (broker) |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Originator does not have a Broker for the requested Resource Delivery policy not supported by the transport adapter Retainment policy not supported by the transport adapter |

###### B.2.1.2.1.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Determine the Broker to be used for the originating AE and publication resource

- Perform the Common Request Services for requests across the Mca Reference Point

- Validate the delivery and retainment policies

Figure B.2.1.2.1.4-1: subscribeRequest and subscribeComplete Diagram

###### B.2.1.2.1.5	Post-Conditions

Success case: The request is permitted.

Failure case: The request is not permitted and a response type is transmitted back to the AE.

###### B.2.1.2.1.6	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### B.2.1.2.1.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 250 ms

##### B.2.1.2.2	subscribeComplete

###### B.2.1.2.2.1	Description

This service capability provides the ability to complete the actions required once the subscription request has been completed in the Broker.

###### B.2.1.2.2.2	Pre-Conditions

Not Applicable.

###### B.2.1.2.2.3	Signature – subscribeComplete

Table B.2.1.2.2.3-1: Data Exchange Service – subscribeComplete capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input parameters | - | - | See Table A.2.2-3 |
| publicationResource | IN | NO | The publication resource. See 6.4.2.1.1.1 |
| deliveryPolicy | IN | YES | The delivery policy when notifying the subscriber. See 6.4.2.1.1.2 |
| retainmentPolicy | IN | YES | The retainment policy for unconnected subscribers. See 6.4.2.1.1.3 |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Exception: Request may not have been completed |

###### B.2.1.2.2.4	Service Interactions

The interactions of service capabilities required for this service capability as shown in Figure B.2.1.2.1.4-1:

- Upon a successful subscribe by the Broker, record the event for accounting purposes

###### B.2.1.2.2.5	Post-Conditions

Success case: The request is permitted.

Failure case: The request is not permitted and a response type is transmitted back to the AE.

###### B.2.1.2.2.6	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### B.2.1.2.2.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 50 ms

##### B.2.1.2.3	publishRequest

###### B.2.1.2.3.1	Description

This service capability provides the ability to validate a publication request from an AE..

As part of the publication, the publishing AE can provide a delivery policy to enhance the robustness of the publication to AEs that have subscribed to the Resource identified in the request.

###### B.2.1.2.3.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a M2M Service Subscription, publishing AE, resource to publish the payload and Broker exist.

###### B.2.1.2.3.3	Signature – publishRequest

Table B.2.1.2.3.3-1: Data Exchange Service – publishRequest capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input parameters | - | - | See Table A.2.2-3 |
| toResource | IN | NO | The leaf node of a publication resource. See 6.4.2.1.1.1 |
| deliveryPolicy | IN | YES | The delivery policy when notifying the subscriber. See 6.4.2.1.1.2 |
| transportAdapter | OUT | YES | The instance of the transport adapter service (broker) |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Originator does not have a Broker for the requested resource Originator has requested an invalid deliveryPolicy Originator has requested an invalid retainmentPolicy Delivery policy not supported by the transport adapter Exception: Request may not have been completed |

###### B.2.1.2.3.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Determine the Broker to be used for the originating AE and resource

- Perform the Common Request Services for requests across the Mca Reference Point

- Validate the delivery policy for the Broker

- Upon a successful publication to the Broker, record the event for accounting purposes

Figure B.2.1.2.3.4-1: publishRequest and publishComplete Diagram

###### B.2.1.2.3.5	Post-Conditions

Success case: The request is permitted.

Failure case: The request is not permitted and a response type is transmitted back to the AE.

###### B.2.1.2.3.6	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### B.2.1.2.3.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300 ms

##### B.2.1.2.4	publishComplete

###### B.2.1.2.4.1	Description

This service capability provides the ability to complete the actions required once the publication request has been completed in the Broker.

###### B.2.1.2.4.2	Pre-Conditions

Not Applicable.

###### B.2.1.2.4.3	Signature – publishComplete

Table B.2.1.2.4.3-1: Data Exchange Service – pubishComplete capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input parameters | - | - | See Table A.2.2-3 |
| toResource | IN | NO | The leaf node of a publication resource. See 6.4.2.1.1.1 |
| deliveryPolicy | IN | YES | The delivery policy when notifying the subscriber. See 6.4.2.1.1.2 |
| retainmentPolicy | IN | YES | The retainment policy for unconnected subscribers. See 6.4.2.1.1.3 |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Exception: Request may not have been completed |

###### B.2.1.2.4.4	Service Interactions

The interactions of service capabilities required for this service capability as shown in Figure B.2.1.2.3.4-1:

- Upon a successful publish by the Broker, record the event for accounting purposes

###### B.2.1.2.4.5	Post-Conditions

Success case: The request is permitted.

Failure case: The request is not permitted and a response type is transmitted back to the AE.

###### B.2.1.2.4.6	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### B.2.1.2.4.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 50 ms

##### B.2.1.2.5	notifyRequest

###### B.2.1.2.5.1	Description

This service capability provides the ability to validate a notification request to an AE.

As part of the notification, delivery policy that is requested to be used is validated and adjusted if necessary.

###### B.2.1.2.5.2	Pre-Conditions

A correlation between a M2M Service Subscription, subscribing AE, subscribed resource and Broker exist.

The Pre-Conditions for Mca Received Requests are met.

###### B.2.1.2.5.3	Signature – notifyRequest

Table B.2.1.2.5.3-1: Data Exchange Service – notifyRequest capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input parameters | - | - | See Table A.2.2-1 |
| fromResource | IN | NO | The leaf node of a publication resource. See 6.4.2.1.1.1 |
| deliveryPolicy | IN-OUT | YES | The delivery policy when notifying the subscriber. See 6.4.2.1.1.2 |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Originator does not have a Broker for the requested resource Originator has requested an invalid delivery policy Exception: Request may not have been completed |

###### B.2.1.2.5.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca Reference Point

- Validate that the delivery policy requested by the Broker is allowed for the subscribing AE and M2M Service Subscription

Figure B.2.1.2.5.4-1: notifyRequest and notifyComplete Diagram

###### B.2.1.2.5.5	Post-Conditions

Success case: The request is permitted.

Failure case: The request is not permitted and a response type is transmitted back to the transport adapter.

###### B.2.1.2.5.6	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### B.2.1.2.5.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300 ms

##### B.2.1.2.6	notifyComplete

###### B.2.1.2.6.1	Description

This service capability provides the ability to complete the actions required once the notification request has been completed in the Data Exchange service.

###### B.2.1.2.6.2	Pre-Conditions

Not Applicable.

###### B.2.1.2.6.3	Signature - notifyComplete

Table B.2.1.2.6.3-1: Data Exchange Service – notifyComplete capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input parameters | - | - | See Table A.2.2-3 |
| fromResource | IN | NO | The leaf node of a publication resource. See 6.4.2.1.1.1 |
| deliveryPolicy | IN | YES | The delivery policy when notifying the subscriber. See 6.4.2.1.1.2 |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Exception: Request may not have been completed |

###### B.2.1.2.6.4	Service Interactions

The interactions of service capabilities required for this service capability as shown in Figure B.2.1.2.5.4-1:

- Upon a successful notification by the Data Exchange, record the event for accounting purposes

###### B.2.1.2.6.5	Post-Conditions

Success case: The request is permitted.

Failure case: The request is not permitted and a response type is transmitted back to the transport adapter.

###### B.2.1.2.6.6	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### B.2.1.2.6.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 50 ms

### B.2.2	Request-Response Message Exchange

#### B.2.2.1	Overview

This service provides the capability to exchange data using the synchronous request-response message pattern where AEs request to send a message to an AE and wait for a response to the message.

#### B.2.2.2	Service Capabilities

##### B.2.2.2.1	sendMessageRequest

###### B.2.2.2.1.1	Description

This service capability provides the ability to validate a send message request from an AE.

###### B.2.2.2.1.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a M2M Service Subscription, originating AE and Transport Adapter exist.

###### B.2.2.2.1.3	Signature – sendMessageRequest

Table B.2.2.2.1.3-1: Data Exchange Service - sendMessageRequest capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input parameters | - | - | See Table A.2.2-3 |
| deliveryPolicy | IN | YES | The delivery policy when sending a request. See 6.4.2.1.1.2 |
| transportAdapter | OUT | YES | The instance of the transport adapter service. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Originator does not have a transport adaptor for the requested AE. Delivery policy not supported by the underlying transport adapter. |

###### B.2.2.2.1.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Determine the transport adapter to be used for the originating AE and target AE. This example shows the use of the Broker service for the transport adapter.

- Perform the Common Request Services for requests across the Mca Reference Point

- Validate the delivery policy

Figure B.2.2.2.1.4-1: sendMessageRequest and sendMessageComplete Diagram

###### B.2.2.2.1.5	Post-Conditions

Success case: The request is permitted.

Failure case: The request is not permitted and a response type is transmitted back to the AE.

###### B.2.2.2.1.6	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### B.2.2.2.1.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 250 ms

##### B.2.2.2.2	sendMessageComplete

###### B.2.2.2.2.1	Description

This service capability provides the ability to complete the actions required once the request has been completed in the Broker.

###### B.2.2.2.2.2	Pre-Conditions

Not Applicable.

###### B.2.2.2.2.3	Signature - sendMessageComplete

Table B.2.2.2.2.3-1: Data Exchange Service - sendMessageComplete capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input parameters | - | - | See Table A.2.2-3 |
| deliveryPolicy | IN | YES | The delivery policy when notifying the subscriber. See 6.4.2.1.1.2 |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. Exception: Request may not have been completed |

###### B.2.2.2.2.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Upon a successful message send and received from the transport adapter, record the event for accounting purposes

###### B.2.2.2.2.5	Post-Conditions

Success case: The request is permitted.

Failure case: The request is not permitted and a response type is transmitted back to the AE.

###### B.2.2.2.2.6	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

###### B.2.2.2.2.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 50 ms

# Annex C (informative): Service Subscription Administration Services

## C.1	Overview

This annex illustrates the usage of the services for requests to administer M2M Service Subscriptions from AEs across the Mca Reference Point.

## C.2	Supporting Services

### C.2.1	Overview

This service provides the capability for AEs to administer the M2M Service Subscriptions that have been previously created by the M2M Service Provider. The following capabilities are provided.

Retrieval of M2M Service Subscriptions based on a set of filter criteria.

Administration of the M2M Nodes associated with a M2M Service Subscriptions as well as the Application Rules associated with the M2M Node.

### C.2.2	Service Capabilities

#### C.2.2.1	getServiceSubscription

##### C.2.2.1.1	Description

This service capability provides the ability for an AE to retrieve the M2M Service Subscription across the Mca Reference Point.

##### C.2.2.1.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

##### C.2.2.1.3	Signature – getServiceSubscription

Table C.2.2.1.3-1: Service Subscription Administration – getServiceSubscription capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Mca Common request input parameters | - | - | See Table A.2.2-1 |
| filterCriteria | IN | NO | See Table 6.9.2.2-1 |
| serviceSubscriptions | OUT | NO | The resulting M2M Service Subscription entities in Table 6.9.2.3-1. |
| responseType | OUT | YES | Unique response types for this service. None |

##### C.2.2.1.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the common request authorization

- Retrieve the M2M Service Subscriptions for the supplied criteria

- Account for the event

Figure C.2.2.1.4-1: getServiceSubscription Diagram

##### C.2.2.1.5	Post-Conditions

Not Applicable.

##### C.2.2.1.6	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

##### C.2.2.1.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300 ms

#### C.2.2.2	addNodeToServiceSubscription

##### C.2.2.2.1	Description

This service capability adds a M2M Node to a M2M Service Subscription.

##### C.2.2.2.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

##### C.2.2.2.3	Signature – addNodeToServiceSubscription

Table C.2.2.2.3-1: Service Subscription Administration – addDeviceToServiceSubscription capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Mca Common request input parameters | - | - | See Table A.2.2-1 |
| externalIds | IN | YES | A List of URNs that represent the external identifiers associated with this M2M Node. |
| applicationRuleIds | IN | YES | A list of Application Rules associated with this Node. |
| nodeId | OUT | NO | The unique M2M Node identifer for this entity. |
| responseType | OUT | YES | Unique response types for this service. M2M Service Subscription does not exist. |
| applicationRuleIds | IN | YES | A list of Application Rules associated with this Node. |

##### C.2.2.2.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the common request authorization

- Add the M2M Nodes to the M2M Service Subscription

- Account for the event

Figure C.2.2.2.4-1: addNodeToServiceSubscription Diagram

##### C.2.2.2.5	Post-Conditions

The M2M Node is added to the M2M Service Subscription returning the oneM2M System.

##### C.2.2.2.6	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

##### C.2.2.2.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300 ms

#### C.2.2.3	deleteNodesFromServiceSubscription

##### C.2.2.3.1	Description

This service capability deletes M2M Nodes from a M2M Service Subscription.

##### C.2.2.3.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

##### C.2.2.3.3	Signature – deleteNodesFromServiceSubscription

Table C.2.2.3.3-1: Service Subscription Administration – deleteNodesFromServiceSubscription capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input parameters | - | - | See Table A.2.2-1 |
| nodeIds | IN | NO | List of M2M Node identifiers to be deleted from the M2M Service Subscription. |
| responseType | OUT | YES | Unique response types for this service. M2M Service Subscription does not exist. M2M Node does not exist for this M2M Service Subscription |

##### C.2.2.3.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the common request authorization

- Delete the M2M Nodes from the M2M Service Subscription

- Account for the event

Figure C.2.2.3.4-1: deleteNodesFromServiceSubscription Diagram

##### C.2.2.3.5	Post-Conditions

The M2M Nodes are deleted from the M2M Service Subscription.

##### C.2.2.3.6	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

##### C.2.2.3.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300 ms

#### C.2.2.4	getNodesForServiceSubscription

##### C.2.2.4.1	Description

This service capability retrieves the M2M Nodes for a M2M Service Subscription.

##### C.2.2.4.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

##### C.2.2.4.3	Signature – getNodesForServiceSubscription

Table C.2.2.4.3-1: Service Subscription Administration – getNodesForServiceSubscription capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input parameters | - | - | See Table A.2.2-1 |
| filterCriteria | IN | NO | See Table 6.9.2.4-1 |
| nodes | OUT | NO | The resulting M2M Node entities in Table 6.9.2.3-1. |
| responseType | OUT | YES | Unique response types for this service. None |

##### C.2.2.4.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the common request authorization

- Retrieve the M2M Nodes with a filter condition

- Account for the event

Figure C.2.2.4.4-1: getNodesForServiceSubscription Diagram

##### C.2.2.4.5	Post-Conditions

Not Applicable.

##### C.2.2.4.6	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

##### C.2.2.4.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300 ms

#### C.2.2.5	createApplicationRule

##### C.2.2.5.1	Description

This service capability provides the ability to create a M2M Application Rule to be associated with one or more M2M Nodes.

##### C.2.2.5.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

##### C.2.2.5.3	Signature – createApplicationRule

Table C.2.2.5.3-1: Service Subscription Administration – createApplciationRule capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input parameters | - | - | See Table A.2.2-1 |
| credentialIds | IN | YES | List of credential identifiers for which this Application Rule is applicable, i.e. for registration requests coming into the M2M System via a Security Association Endpoint (SEA) [i.3], that was authenticated using credentials that match with any of these credential identifiers, the current Application Rule applies. |
| allowedApplicationIds | IN | YES | List of M2M Application Identifiers (App-ID) that shall be considered to be allowed for AE registration requests received via Security Association Endpoint (SEA) [i.3]. |
| allowedAEIds | IN | YES | List of M2M Application Entity Identifiers (AE-ID) that shall be considered to be allowed for AE registration requests. |
| applicationRuleId | OUT | NO | The unique Application Rule identifier in the context of the M2M Service Subscription. |
| responseType | OUT | YES | Unique response types for this service. None |

##### C.2.2.5.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the common request authorization

- Create the Application Rule

- Account for the event

Figure C.2.2.5.4-1: createApplicationRule Diagram

##### C.2.2.5.5	Post-Conditions

The Application Rule is created.

##### C.2.2.5.6	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

##### C.2.2.5.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300 ms

#### C.2.2.6	deleteApplicationRules

##### C.2.2.6.1	Description

This service capability provides the ability to delete Application Rules and any references to the Application Rule made by M2M Nodes to the Application Rules.

##### C.2.2.6.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

##### C.2.2.6.3	Signature – deleteApplicationRules

Table C.2.2.6.2-1: Service Subscription Administration – deleteApplicationRules capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input parameters | - | - | See Table A.2.2-1 |
| applicationRuleIds | IN | NO | A list of Application Rule identifiers. |
| responseType | OUT | YES | Unique response types for this service. Application Rule does not exist |

##### C.2.2.6.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the common request authorization

- Delete the Application Rules

- Account for the event

Figure C.2.2.6.4-1: deleteApplicationRules Diagram

##### C.2.2.6.5	Post-Conditions

Application Rules and any references to the Application Rule made by M2M Nodes to the Application Rules are deleted.

##### C.2.2.6.6	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

##### C.2. 2.6.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300 ms

#### C.2.2.7	getApplicationRules

##### C.2.2.7.1	Description

This service capability provides the ability to retrieve the Application Rules.

##### C.2.2.7.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

##### C.2.2.7.3	Signature – getApplicationRules

Table C.2.2.7.3-1: Service Subscription Administration – getApplicationRulescapability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input parameters | - | - | See Table A.2.2-1 |
| filterCriteria | IN | NO | See Table 6.9.2.6-1 |
| applicationRules | OUT | NO | The resulting M2M Node entities in Table 6.9.2.5-1. |
| responseType | OUT | YES | Unique response types for this service. None |

##### C.2.2.7.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the common request authorization

- Retrieve the M2M Applications of the M2M Service Subscription

- Account for the event

Figure C.2.2.7.4-1: getApplicationRules Diagram

##### C.2.2.7.5	Post-Conditions

Not Applicable.

##### C.2.2.7.6	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

##### C.2.2.7.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300 ms

#### C.2.2.8	updateApplicationRule

##### C.2.2.8.1	Description

This service capability provides the ability to update an Application Rule.

##### C.2.2.8.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

##### C.2.2.8.3	Signature – updateApplicationRule

Table C.2.2.8.3-1: Service Subscription Administration – updateApplicationForDevice capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Mca Common request input parameters | - | - | See Table A.2.2-1 |
| applicationRuleId | IN | NO | The unique Application Rule identifier in the context of the M2M Service Subscription. |
| credentialIds | IN | YES | List of credential identifiers for which this Application Rule is applicable, i.e. for registration requests coming into the M2M System via a Security Association Endpoint (SEA) [i.5], that was authenticated using credentials that match with any of these credential identifiers, the current Application Rule applies. When present this list will overlay the list of credential identifiers. |
| allowedApplicationIds | IN | YES | List of M2M Application Identifiers (App-ID) that shall be considered to be allowed for AE registration requests received via Security Association Endpoint (SEA) [i.6]. When present this list will overlay the list of allowed App-IDs. |
| responseType | OUT | YES | Unique response types for this service. Application Rule does not exist. |

##### C.2.2.8.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the common request authorization

- Update the Application Rule.

- Account for the event

Figure C.2.2.8.4-1: updateApplicationRule Diagram

##### C.2.2.8.5	Post-Conditions

The Application Rule is updated.

##### C.2.2.8.6	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

##### C.2.2.8.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300 ms

#### C.2.2.9	updateNodeForServiceSubscription

##### C.2.2.9.1	Description

This service capability provides the ability to update the Application Rules and External Identifiers to be associated to a Node within the context of a M2M Service Subscription.

##### C.2.2.9.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

##### C.2.2.9.3	Signature – updateNodeForServiceSubscription

Table C.2.2.9.3-1: Service Subscription Administration – updateNodeForServiceSubscription capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Mca Common request input parameters | - | - | See Table A.2.2-1 |
| nodeId | IN | NO | The unique M2M Node identifier in the context of the M2M Service Subscription. |
| applicationRuleIds | IN | YES | A list of Application Rule Identifier. When present this list will overlay the list of Applications associated with the Node. |
| externalIds | IN | YES | List of URNs that represent the external identifiers associated with this device. When present this list will overlay the list of external identifiers associated with the Node. |
| responseType | OUT | YES | Unique response types for this service. M2M Service Subscription does not exist M2M Node does not exist for the M2M Service Subscription |

##### C.2.2.9.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the common request authorization

- Update the M2M Node information for the M2M Service Subscription

- Account for the event

Figure C.2.2.9.4-1: updateNodeForServiceSubscription Diagram

##### C.2.2.9.5	Post-Conditions

The M2M Node for the M2M Service Subscription is updated.

##### C.2.2.9.6	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

##### C.2.2.9.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300 ms

# Annex D (informative): Device Management Services

## D.1	Overview

This annex illustrates the usage of the services for requests to manage device from AEs across the Mca Reference Point.

## D.2	Supporting Services

### D.2.1	Overview

This service provides the capability to manage device using the request-response or request-asynchronous report message patterns where AEs:

Request to manage device using device specific service capabilities or more complex service capabilities that the Support Service will orchestrate into the individual service capabilities according to their business process.

Request to obtain management operation execution results or status using the reporting capability. Reports can be generated or batched by the report policy.

### D.2.2	Service Capabilities

#### D.2.2.1	downloadFirmware

##### D.2.2.1.1	Description

This service capability provides the ability to execute a downloadFirmware request from an AE.

##### D.2.2.1.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

##### D.2.2.1.3	Signature – downloadFirmware

Table D.2.2.1.3-1: Device Management Service –downloadFirmware capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input attributes | - | - | See Table A.2.2-1 |
| firmwareInfo | IN | NO | The device firmware information. Type FirmwareInfo, see 6.6.2.2.5 |
| deviceId | IN | YES | The unique device identifier in the context of the M2M Service Subscription. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.2.2.1. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. |

##### D.2.2.1.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca Reference Point

- Retrieves the Management Adapter to be used for the device and M2M Service Subscription

- Issues the request to the Management Adapter to download the firmware

- Records the event

Figure D.2.2.1.4-1: downloadFirmware Diagram

##### D.2.2.1.5	Post-Conditions

The Management Adapter has submitted a request to the Management Server to download firmware.

The event has been recorded.

##### D.2.2.1.6	Exceptions

Not Applicable.

##### D.2.2.1.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300 ms

#### D.2.2.2	installFirmware

##### D.2.2.2.1	Description

This service capability provides the ability to execute a installFirmware request from an AE.

##### D.2.2.2.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

##### D.2.2.2.3	Signature – installFirmware

Table D.2.2.2.3-1: Device Management Service –installFirmware capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input attributes | - | - | See Table A.2.2-1 |
| firmwareInfo | IN | NO | The device firmware information. Type FirmwareInfo, see 6.6.2.2.5 |
| deviceId | IN | YES | The unique device identifier in the context of the M2M M2M Service Subscription. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. |

##### D.2.2.2.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca Reference Point

- Retrieves the Management Adapter to be used for the device and M2M Service Subscription

- Issues the request to the Management Adapter to install the firmware

- Records the event

Figure D.2.2.2.4-1: installFirmware Diagram

##### D.2.2.2.5	Post-Conditions

The Management Adapter has submitted a request to the Management Server to install firmware.

The event has been recorded.

##### D.2.2.2.6	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

##### D.2.2.2.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300 ms

#### D.2.2.3	getFirmwareInformation

##### D.2.2.3.1	Description

This service capability provides the ability to execute a getFirmwareInformation request from an AE.

##### D.2.2.3.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

##### D.2.2.3.3	Signature – getFirmwareInformation

Table D.2.2.3.3-1: Device Management Service – getFirmwareInformation capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input attributes | - | - | See Table A.2.2-1 |
| deviceId | IN | YES | The unique device identifier in the context of the M2M M2M Service Subscription. |
| firmwareInfo | OUT | YES | The device firmware information. Type FirmwareInfo, see 6.6.2.2.5 |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. |

##### D.2.2.3.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca Reference Point

- Retrieves the Management Adapter to be used for the device and M2M Service Subscription

- Issues the request to the Management Adapter to get the firmware information

- Records the event

Figure D.2.2.3.4-1: getFirmwareInformation Diagram

##### D.2.2.3.5	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get firmware information.

The event is recorded.

##### D.2.2.3.6	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

##### D.2.2.3.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300 ms

#### D.2.2.4	getFirmwareExecStatus

##### D.2.2.4.1	Description

This service capability provides the ability to retrieve the execution status or result of a requested firmware operation.

##### D.2.2.4.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and previously submitted firmware request exist.

##### D.2.2.4.3	Signature – getFirmwareExecStatus

Table D.2.2.4.3-1: Device Management Service – getFirmwareExecStatus capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input attributes | - | - | See Table A.2.2-1 |
| operationRequestId | IN | NO | The M2M Request Identifier of previously submitted firmware request (M2M-Request-ID) |
| deviceId | IN | YES | The unique device identifier in the context of the M2M M2M Service Subscription. |
| firmwareReport | OUT | YES | The firmware management operation execution result or status. Type FirmwareReport, see 6.6.2.2.3. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. The previously submitted firmware request does not exist |

##### D.2.2.4.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca Reference Point

- Retrieves the Management Adapter to be used for the device and M2M Service Subscription

- Issues the request to the Management Adapter to retrieve the firmware operations execution status or result

- Records the event

Figure D.2.2.4.4-1: getFirmwareExecStatus Diagram

##### D.2.2.4.5	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get firmware operation execution status or result.

Record the event.

##### D.2.2.4.6	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

##### D.2.2.4.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300 ms

#### D.2.2.5	deviceManagementReport

##### D.2.2.5.1	Description

This service capability provides the ability to report an AE about the status or result of management operations for previously submitted operation requests.

##### D.2.2.5.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and previously submitted management requests exist.

##### D.2.2.5.3	Signature –deviceManagementReport

Table D.2.2.5.3-1: Device Management Service –deviceManagementReport capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input attributes | - | - | See Table A.2.2-1 |
| isLastReport | IN | NO | Boolean, whether it is the last report. |
| sequenceNumber | IN | NO | The report sequence number. |
| firmwareReportList | IN | NO | Array of firmwareReport. Type FirmwareReport, see 6.6.2.2.3. |
| troubleshootingReportList | IN | YES | Array of troubleshootingReport. Type TroubleshootingReport, see 6.6.2.2.20. |
| softwareReportList | IN | YES | Array of softwareReport. Type SoftwareReport, see 6.6.2.2.27. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. |

##### D.2.2.5.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca Reference Point

- Record the event

Figure D.2.2.5.4-1: deviceManagementReport Diagram

##### D.2.2.5.5	Post-Conditions

The AE has received a report of management operation execution status or result.

The event is recorded.

##### D.2.2.5.6	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

##### D.2.2.5.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300 ms

#### D.2.2.6	upgradeFirmware

##### D.2.2.6.1	Description

This service capability permits AEs to upgrade the firmware on individual device, multiple devices or a group of devices. In addition the upgrade of the firmware is permitted based on a schedule for each of the operations

##### D.2.2.6.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between Management Adapter, the M2M Service Capability and devices or device group exist.

##### D.2.2.6.3	Signature – upgradeFirmware

Table D.2.2.6.3-1: Device Management Service –upgradeFirmware capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input attributes | - | - | See Table A.2.2-1 |
| orchestrationRuleList | IN | NO | List of OrchestrationRule. Type OrchestrationRule, see clause 6.6.2.2.4. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.2.2.1. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. |

##### D.2.2.6.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca Reference Point

- Retrieves the Management Adapter to be used for the device and M2M Service Subscription

- Issues a series of operations to the Management Adapter to upgrade the firmware on a the devices in the orchestration rule list. Each entry in the orchestration rule list contains the operation, the devices and schedule for the operation.

- Records the event

Figure D.2.2.6.4-1: upgradeFirmware Diagram

##### D.2.2.6.5	Post-Conditions

The Management Adapter has submitted a set of requests to the Management Server to upgrade the firmware.

The event is recorded.

##### D.2.2.6.6	Exceptions

The reachability schedule of the device does not match the schedule of the orchestration rule.

##### D.2.2.6.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300 ms

#### D.2.2.7	getDeviceInformation

##### D.2.2.7.1	Description

This service capability provides the ability for an AE to retrieve the information about a device across the Mca Reference Point.

##### D.2.2.7.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

##### D.2.2.7.3	Signature – getDeviceInformation

Table D.2.2.7.3-1: Device Management Service –getDeviceInformation capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input attributes | - | - | See Table A.2.2-1 |
| deviceId | IN | NO | The unique device identifier in the context of the M2M M2M Service Subscription. |
| deviceInfo | OUT | NO | The device information. Type DeviceInfo, see 6.6.2.2.7. |
| memory | OUT | YES | The memory information. Type Memory, see 6.6.2.2.8. |
| battery | OUT | YES | The battery information. Type Battery, see 6.6.2.2.9. |
| lockStatus | OUT | YES | The device lock status. Enum LockStatus, see 6.6.2.2.13. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. |

##### D.2.2.7.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca Reference Point

- Retrieves the Management Adapter to be used for the device and M2M Service Subscription

- Issues the request to the Management Adapter to get device information

- Records the event

Figure D.2.2.7.4-1: getDeviceInformation Diagram

##### D.2.2.7.5	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get device information.

Based on the capabilities supported by the device, the memory, battery and lock status information may or may not be returned.

The event has been recorded.

##### D.2.2.7.6	Exceptions

Not Applicable.

##### D.2.2.7.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300 ms

#### D.2.2.8	getDeviceCapabilities

##### D.2.2.8.1	Description

This service capability provides the ability to execute a getDeviceCapabilities request from an AE.

##### D.2.2.8.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

##### D.2.2.8.3	Signature – getDeviceCapabilities

Table D.2.2.8.3-1: Device Management Service –getDeviceCapabilities capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input attributes | - | - | See Table A.2.2-1 |
| deviceId | IN | NO | The unique device identifier in the context of the M2M M2M Service Subscription. |
| deviceCapabilities | OUT | YES | Array of deviceCapability. Type DeviceCapability, see 6.6.2.2.10. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. |

##### D.2.2.8.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca Reference Point

- Retrieves the Management Adapter to be used for the device and M2M Service Subscription

- Issues the request to the Management Adapter to get device capabilities

- Records the event

Figure D.2.2.8.4-1: getDeviceCapabilities Diagram

##### D.2.2.8.5	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get device capabilities.

The event has been recorded.

##### D.2.2.8.6	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

##### D.2.2.8.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300 ms

#### D.2.2.9	enableDeviceCapability

##### D.2.2.9.1	Description

This service capability provides the ability to execute a enableDeviceCapability request from an AE.

##### D.2.2.9.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

##### D.2.2.9.3	Signature – enableDeviceCapability

Table D.2.2.9.3-1: Device Management Service –enableDeviceCapability capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input attributes | - | - | See Table A.2.2-1 |
| deviceId | IN | NO | The unique device identifier in the context of the M2M M2M Service Subscription. |
| name | IN | NO | The name of the capability. Enum DeviceCapabilityName, see 6.6.2.2.12. |
| state | OUT | NO | Indicates if the capability is enabled or disabled. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. |

##### D.2.2.9.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca Reference Point

- Retrieves the Management Adapter to be used for the device and M2M Service Subscription

- Issues the request to the Management Adapter to enable device capability

- Records the event

Figure D.2.2.9.4-1: enableDeviceCapability Diagram

##### D.2.2.9.5	Post-Conditions

The Management Adapter has submitted a request to the Management Server to enable device capability.

The event is recorded.

##### D.2.2.9.6	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

##### D.2.2.9.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300 ms

#### D.2.2.10	disableDeviceCapability

##### D.2.2.10.1	Description

This service capability provides the ability to execute a disableDeviceCapability request from an AE.

##### D.2.2.10.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

##### D.2.2.10.3	Signature – disableDeviceCapability

Table D.2.2.10.3-1: Device Management Service – disableDeviceCapability capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input attributes | - | - | See Table A.2.2-1 |
| deviceId | IN | NO | The unique device identifier in the context of the M2M M2M Service Subscription. |
| name | IN | NO | The name of the capability. Enum DeviceCapabilityName, see 6.6.2.2.12. |
| state | OUT | NO | Indicates if the capability is enabled or disabled. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. |

##### D.2.2.10.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca Reference Point

- Retrieves the Management Adapter to be used for the device and M2M Service Subscription

- Issues the request to the Management Adapter to disable device capability

- Records the event

Figure D.2.2.10.4-1: disableDeviceCapability Diagram

##### D.2.2.10.5	Post-Conditions

The Management Adapter has submitted a request to the Management Server to disable device capability.

Record the event.

##### D.2.2.10.6	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

##### D.2.2.10.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300 ms

#### D.2.2.11	getAreaNetworks

##### D.2.2.11.1	Description

This service capability provides the ability to execute a getAreaNetworks request from an AE.

##### D.2.2.11.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and area network exist.

##### D.2.2.11.3	Signature – getAreaNetworks

Table D.2.2.11.3-1: Device Management Service – getAreaNetworks capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input attributes | - | - | See Table A.2.2-1 |
| areaNwks | OUT | YES | Array of area network. Type AreaNwkInfo, see 6.6.2.2.13. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. |

##### D.2.2.11.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca Reference Point

- Retrieves the Management Adapter to be used for the device and M2M Service Subscription

- Issues the request to the Management Adapter to get area networks information

- Records the event

Figure D.2.2.11.4-1: getAreaNetworks Diagram

##### D.2.2.11.5	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get area networks information.

The event has been recorded.

##### D.2.2.11.6	Exceptions

Not Applicable.

##### D.2.2.11.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300 ms

#### D.2.2.12	updateDeviceForAreaNetwork

##### D.2.2.12.1	Description

This service capability provides the ability to execute a updateDeviceForAreaNetwork request from an AE.

##### D.2.2.12.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability, the device and area network exist.

##### D.2.2.12.3	Signature –updateDeviceForAreaNetwork

Table D.2.2.12.3-1: Device Management Service – updateDeviceForAreaNetwork capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input attributes | - | - | See Table A.2.2-1 |
| areaNwkId | IN | NO | The unique area network identifier. |
| deviceId | IN | NO | The unique device identifier in the context of the M2M M2M Service Subscription. |
| areaNwkDeviceInfo | IN | YES | The existing areaNwkDeviceInfo are replaced with the information in this parameter. Type AreaNwkDeviceInfo, see 6.6.2.2.15. |
| lastModifiedTime | OUT | NO | The modified time. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. |

##### D.2.2.12.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca Reference Point

- Retrieves the Management Adapter to be used for the device and M2M Service Subscription

- Issues the request to the Management Adapter to update device information for area network

- Records the event

Figure D.2.2.12.4-1: updateDeviceForAreaNetwork Diagram

##### D.2.2.12.5	Post-Conditions

The Management Adapter has submitted a request to the Management Server to update device information for area network.

The event is recorded.

##### D.2.2.12.6	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

##### D.2.2.12.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300 ms

#### D.2.2.13	rebootDevice

##### D.2.2.13.1	Description

This service capability provides the ability to execute a rebootDevice request from an AE.

##### D.2.2.13.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

##### D.2.2.13.3	Signature – rebootDevice

Table D.2.2.13.3-1: Device Management Service – rebootDevice capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input attributes | - | - | See Table A.2.2-1 |
| deviceId | IN | NO | The unique device identifier in the context of the M2M M2M Service Subscription. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.2.2.1. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. |

##### D.2.2.13.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca Reference Point

- Retrieves the Management Adapter to be used for the device and M2M Service Subscription

- Issues the request to the Management Adapter to reboot device

- Records the event

Figure D.2.2.13.4-1: rebootDevice Interaction Diagram

##### D.2.2.13.5	Post-Conditions

The Management Adapter has submitted a request to the Management Server to reboot device.

The event has been recorded.

##### D.2.2.13.6	Exceptions

Not Applicable.

##### D.2.2.13.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300 ms

#### D.2.2.14	resetDevice

##### D.2.2.14.1	Description

This service capability provides the ability to execute a resetDevice request from an AE.

##### D.2.2.14.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

##### D.2.2.14.3	Signature –resetDevice

Table D.2.2.14.3-1: Device Management Service –resetDevice capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input attributes | - | - | See Table A.2.2-1 |
| deviceId | IN | NO | The unique device identifier in the context of the M2M M2M Service Subscription. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.2.2.1. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. |

##### D.2.2.14.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca Reference Point

- Retrieves the Management Adapter to be used for the device and M2M Service Subscription

- Issues the request to the Management Adapter to reset device

- Records the event

Figure D.2.2.14.4-1: resetDevice Interaction Diagram

##### D.2.2.14.5	Post-Conditions

The Management Adapter has submitted a request to the Management Server to reset device.

The event has been recorded.

##### D.2.2.14.6	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

##### D.2.2.14.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300 ms

#### D.2.2.15	uploadDeviceLog

##### D.2.2.15.1	Description

This service capability provides the ability to execute a uploadDeviceLog request from an AE.

##### D.2.2.15.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

##### D.2.2.15.3	Signature –uploadDeviceLog

Table D.2.2.15.3-1: Device Management Service –uploadDeviceLog capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input attributes | - | - | See Table A.2.2-1 |
| deviceId | IN | NO | The unique device identifier in the context of the M2M M2M Service Subscription. |
| logInfo | IN | NO | The log information. Type LogInfo, see 6.6.2.2.19. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.2.2.1. |
| logURL | OUT | YES | The URL from which the log can be uploaded. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. |

##### D.2.2.15.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca Reference Point

- Retrieves the Management Adapter to be used for the device and M2M Service Subscription

- Issues the request to the Management Adapter to upload device log

- Records the event

Figure D.2.2.15.4-1: uploadDeviceLog Interaction Diagram

##### D.2.2.15.5	Post-Conditions

The Management Adapter has submitted a request to the Management Server to upload device log.

The event is recorded.

##### D.2.2.15.6	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

##### D.2.2.15.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300 ms

#### D.2.2.16	getDeviceLogs

##### D.2.2.16.1	Description

This service capability provides the ability to execute a getDeviceLogs request from an AE.

##### D.2.2.16.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

##### D.2.2.16.3	Signature – getDeviceLogs

Table D.2.2.16.3-1: Device Management Service – getDeviceLogs capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input attributes | - | - | See Table A.2.2-1 |
| deviceId | IN | NO | The unique device identifier in the context of the M2M M2M Service Subscription. |
| logList | OUT | YES | Array of log. Type Log, see 6.6.2.2.21. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. |

##### D.2.2.16.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca Reference Point

- Retrieves the Management Adapter to be used for the device and M2M Service Subscription

- Issues the request to the Management Adapter to get all logs of a device

- Records the event

Figure D.2.2.16.4-1: getDeviceLogs Interaction Diagram

##### D.2.2.16.5	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get all logs of a device.

The event is recorded.

##### D.2.2.16.6	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

##### D.2.2.16.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300 ms

#### D.2.2.17	getDeviceLogInformation

##### D.2.2.17.1	Description

This service capability provides the ability to execute a getDeviceLogInformation request from an AE.

##### D.2.2.17.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

##### D.2.2.17.3	Signature – getDeviceLogInformation

Table D.2.2.17.3-1: Device Management Service – getDeviceLogInformation capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input attributes | - | - | See Table A.2.2-1 |
| deviceId | IN | NO | The unique device identifier in the context of the M2M M2M Service Subscription. |
| filterCriteria | IN | YES | See Table 6.6.2.2.22 |
| logURL | IN | YES | The URL from which the log can be accessed. |
| log | OUT | YES | Type Log, see 6.6.2.2.21. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. |

##### D.2.2.17.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca Reference Point

- Retrieves the Management Adapter to be used for the device and M2M Service Subscription

- Issues the request to the Management Adapter to get a device log information

- Records the event

Figure D.2.2.17.4-1: getDeviceLogInformation Interaction Diagram

##### D.2.2.17.5	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get a device log information.

The event is recorded.

##### D.2.2.17.6	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

##### D.2.2.17.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300 ms

#### D.2.2.18	getSoftwareInformation

##### D.2.2.18.1	Description

This service capability provides the ability to execute a getSoftwareInformation request from an AE.

##### D.2.2.18.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

##### D.2.2.18.3	Signature – getSoftwareInformation

Table D.2.2.18.3-1: Device Management Service – getSoftwareInformation capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input attributes | - | - | See Table A.2.2-1 |
| deviceId | IN | NO | The unique device identifier in the context of the M2M M2M Service Subscription. |
| version | OUT | YES | The version of the software. |
| name | OUT | YES | The name of the software. |
| URL | OUT | YES | The URL from which the software package can be downloaded. |
| installStatus | OUT | YES | Indicates the status of the install. Enum ActionStatus, see 6.6.2.2.26. |
| activeStatus | OUT | YES | The status of active or deactivate action. Enum ActionStatus, see 6.6.2.2.26. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. |

##### D.2.2.18.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca Reference Point

- Retrieves the Management Adapter to be used for the device and M2M Service Subscription

- Issues the request to the Management Adapter to get application software information

- Records the event

Figure D.2.2.18.4-1: getSoftwareInformation Interaction Diagram

##### D.2.2.18.5	Post-Conditions

The Management Adapter has submitted a request to the Management Server to get application software information.

The event has been recorded.

##### D.2.2.18.6	Exceptions

Not Applicable.

##### D.2.2.18.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300 ms

#### D.2.2.19	downloadSoftware

##### D.2.2.19.1	Description

This service capability provides the ability to execute a downloadSoftware request from an AE.

##### D.2.2.19.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

##### D.2.2.19.3	Signature – downloadSoftware

Table D.2.2.19.3-1: Device Management Service – downloadSoftware capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input attributes | - | - | See Table A.2.2-1 |
| deviceId | IN | NO | The unique device identifier in the context of the M2M M2M Service Subscription. |
| version | IN | NO | The version of the software. |
| name | IN | NO | The name of the software. |
| URL | IN | NO | The URL from which the software package can be downloaded. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.2.2.1. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. |

##### D.2.2.19.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca Reference Point

- Retrieves the Management Adapter to be used for the device and M2M Service Subscription

- Issues the request to the Management Adapter to download application software

- Records the event

Figure D.2.2.19.4-1: downloadSoftware Interaction Diagram

##### D.2.2.19.5	Post-Conditions

The Management Adapter has submitted a request to the Management Server to download application software.

The event has been recorded.

##### D.2.2.19.6	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

##### D.2.2.19.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300 ms

#### D.2.2.20	installSoftware

##### D.2.2.20.1	Description

This service capability provides the ability to execute a installSoftware request from an AE.

##### D.2.2.20.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

##### D.2.2.20.3	Signature –installSoftware

Table D.2.2.20.3-1: Device Management Service –installSoftware capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input attributes | - | - | See Table A.2.2-1 |
| deviceId | IN | NO | The unique device identifier in the context of the M2M M2M Service Subscription. |
| version | IN | NO | The version of the software. |
| name | IN | NO | The name of the software. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.2.2.1. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. |

##### D.2.2.20.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca Reference Point

- Retrieves the Management Adapter to be used for the device and M2M Service Subscription

- Issues the request to the Management Adapter to install application software

- Records the event

Figure D.2.2.20.4-1: installSoftware Interaction Diagram

##### D.2.2.20.5	Post-Conditions

The Management Adapter has submitted a request to the Management Server to install application software.

The event is recorded.

##### D.2.2.20.6	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

##### D.2.2.20.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300 ms

#### D.2.2.21	activateSoftware

##### D.2.2.21.1	Description

This service capability provides the ability to execute an activateSoftware request from an AE.

##### D.2.2.21.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

##### D.2.2.21.3	Signature – activateSoftware

Table D.2.2.21.31: Device Management Service – activateSoftware capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input attributes | - | - | See Table A.2.2-1 |
| deviceId | IN | NO | The unique device identifier in the context of the M2M M2M Service Subscription. |
| version | IN | NO | The version of the software. |
| name | IN | NO | The name of the software. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.2.2.1. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. |

##### D.2.2.21.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca Reference Point

- Retrieves the Management Adapter to be used for the device and M2M Service Subscription

- Issues the request to the Management Adapter to activate software previously installed

- Records the event

Figure D.2.2.21.4-1: activateSoftware Interaction Diagram

##### D.2.2.21.5	Post-Conditions

The Management Adapter has submitted a request to the Management Server to activate software previously installed.

Record the event.

##### D.2.2.21.6	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

##### D.2.2.21.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300 ms

#### D.2.2.22	deactivateSoftware

##### D.2.2.22.1	Description

This service capability provides the ability to execute a deactivateSoftware request from an AE.

##### D.2.2.22.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

##### D.2.2.22.3	Signature – deactivateSoftware

Table D.2.2.22.3-1: Device Management Service – deactivateSoftware capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input attributes | - | - | See Table A.2.2-1 |
| deviceId | IN | NO | The unique device identifier in the context of the M2M M2M Service Subscription. |
| version | IN | NO | The version of the software. |
| name | IN | NO | The name of the software. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.2.2.1. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. |

##### D.2.2.22.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca Reference Point

- Retrieves the Management Adapter to be used for the device and M2M Service Subscription

- Issues the request to the Management Adapter to deactivates software

- Records the event

Figure D.2.2.22.4-1: deactivateSoftware Interaction Diagram

##### D.2.2.22.5	Post-Conditions

The Management Adapter has submitted a request to the Management Server to deactivates software.

The event is recorded.

##### D.2.2.22.6	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

##### D.2.2.22.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300 ms

#### D.2.2.23	removeSoftware

##### D.2.2.23.1	Description

This service capability provides the ability to execute a removeSoftware request from an AE.

##### D.2.2.23.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a Management Adapter, the M2M Service Capability and device exist.

##### D.2.2.23.3	Signature – removeSoftware

Table D.2.2.23.3-1: Device Management Service – removeSoftware capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Common request input attributes | - | - | See Table A.2.2-1 |
| deviceId | IN | NO | The unique device identifier in the context of the M2M M2M Service Subscription. |
| version | IN | NO | The version of the software. |
| name | IN | NO | The name of the software. |
| reportPolicy | IN | YES | The policy used to report the state of the operation to the originating AE. See clause 6.6.2.2.1. |
| responseType | OUT | YES | Unique response types for this service. NOTE: Consumed services also provide response types. |

##### D.2.2.23.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca Reference Point

- Retrieves the Management Adapter to be used for the device and M2M Service Subscription

- Issues the request to the Management Adapter to uninstall the software

- Records the event

Figure D.2.2.23.4-1 removeSoftware Interaction Diagram

##### D.2.2.23.5	Post-Conditions

The Management Adapter has submitted a set of requests to the Management Server to uninstall the software.

The event is recorded.

##### D.2.2.23.6	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

##### D.2.2.23.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300 ms

# Annex E (informative): Device On-boarding Service

## E.1	Overview

This annex illustrates the usage of services to onboard a device or AE when the M2M Node or AE contacts the M2M System.

The Device On-boarding supporting service is a process by which a M2M Node is provided the artifacts (e.g. configuration information, software and firmware) needed to successfully register an AE with the M2M Service Layer and invoke or receive the authorized service capabilities.

## E.2	Supporting Services

### E.2.1	Remote Administration

#### E.2.1.1	Overview

This service provides remote administration of the device and/or AE when an AE registers or registers with the M2M System. The mechanism is implemented using the first contact subscription and notification services provided/used by the Registration Service.

#### E.2.1.2	Service Capabilities

##### E.2.1.1.1	notifyRegistrationContact

###### E.2.1.1.1.1	Description

This service capability provides the capability to invoke a process to update the M2M Service Layer artifacts (e.g. credentials, device firmware, and application software).

###### E.2.1.1.1.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

A correlation between a M2M Service Subscription and the contacting entity (M2M Node, AE) exist.

The Supporting Service subscribed to receive registration events using the subscribeInitialAERegistrationEvent service capability.

###### E.2.1.1.1.3	Signature – notifyRegistrationContact

Table E.2.1.1.1.3-1: Device On-boarding Service – notifyRegistrationContact capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Mca Common request input parameters | - | - | See Table A.2.2-1 |
| aeId | IN | NO | The AE-ID of the registered AE. |
| event | IN | NO | The event that caused the registration contact. Enumeration of: FirstContact |

###### E.2.1.1.1.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca Reference Point

- Determine the Application information (Device, Subscription, and Application Id) for the registering AE.

- Determine from the Device information if the device need be updated

- Determine from the Application information if Applications need updated on the Device

- Record the event for accounting purposes.

Figure E.2.1.1.1.4-1: notifyRegistrationContact Diagram

###### E.2.1.1.1.5	Post-Conditions

Success case: The request is permitted and the associated.

Failure case: The request is recorded as a failure.

###### E.2.1.1.1.6	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are recorded by this service capability.

###### E.2.1.1.1.7	Policies for Use

Message Exchange Patterns: In-Only

Transaction Pattern: Creates Transaction

Maximum Response: Not applicable for the In-Only message exchange pattern

# Annex F (informative): Registration Services

## F.1	Overview

This annex illustrates the usage of the services for requests to register AEs across the Mca Reference Point and between M2M Service Components across the Msc Reference Point.

## F.2	Mca Registration Service Processing and Supporting Services

### F.2.1	Overview

AEs communicate with the M2M System over the Mca Reference Point through the SE component. The SE component accepts the request from the AE and provides the request to the Supporting Service in order allow the M2M Service Provider to ensure the request is properly authorized and recorded. In addition, the M2M Service Provider is able to orchestrate other capabilities that have not been specified.

### F.2.2	Service Capabilities

#### F.2.2.1	registerAE

##### F.2.2.1.1	Description

This service capability enables an AE to register with the M2M System. This service capability shall be restricted across the Mca Reference Point

##### F.2.2.1.2	Pre-Conditions

The AE has not registered with the M2M System.

The Pre-Conditions for Mca Received Requests are met.

##### F.2.2.1.3	Signature – registerAE

Table F.2.2.1.3-1: Registration Services – registerAE capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Mca Common request input parameters | - | - | See Table A.2.2-1 |
| aeId | IN | NO | The Application Entity Identifier (AE-ID) to be refreshed. |
| pointOfAccess | IN | YES | The point of Access of the registered AE. POA is optional only if identical to the one in the refreshed registration |
| expirationTime | IN | NO | The expiration time of the registration as requested by the Originator. |
| reachabilitySchedule | IN | YES | The contact reachability schedule information of the AE associated with the device node. The absence of this parameter implies the AE associated with the device node is always contact reachable. Type Schedule, see 6.9.2.7.1. |
| responseType | OUT | NO | Unique response types for this service: AE successfully refreshed Registration does not exist |

##### F.2.2.1.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Mca Reference Point

- Record the event

Figure F.2.2.1.4-1: Registration Services – registerAE Diagram

##### F.2.2.1.4	Post-Conditions

Event is recorded.

##### F.2.2.1.5	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

##### F.2.2.1.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 250 ms

#### F.2.2.2	refreshAERegistration

##### F.2.2.2.1	Description

This service capability enables an AE to refresh an existing registration with the M2M System

##### F.2.2.2.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

The AE is registered and allocated an aeId by the M2M System.

##### F.2.2.2.3	Signature – refreshAERegsitration

Table F.2.2.2.3-1: Registration Services – refreshAERegistration capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Mca Common request input parameters | - | - | See Table A.2.2-1 |
| aeId | IN | NO | The Application Entity Identifier (AE-ID) to be refreshed. |
| pointOfAccess | IN | YES | The point of Access of the registered AE. POA is optional only if identical to the one in the refreshed registration |
| expirationTime | IN | NO | The expiration time of the registration as requested by the Originator. |
| reachabilitySchedule | IN | YES | The contact reachability schedule information of the AE associated with the device node. The absence of this parameter implies the AE associated with the device node is always contact reachable. Type Schedule, see 6.9.2.7.1. |
| responseType | OUT | NO | Unique response types for this service: AE successfully refreshed Registration does not exist |

##### F.2.2.2.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Msc Reference Point

- Record the event

Figure F.2.2.2.4-1: Registration Services – refreshAERegistration Diagram

##### F.2.2.2.5	Post-Conditions

Success case: The request is permitted.

Failure case: The request is not permitted and a response type is transmitted to the AE

##### F.2.2.2.6	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

##### F.2.2.2.7	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 50 ms

#### F.2.2.3	deregisterAE

##### F.2.2.3.1	Description

This service capability enables an AE to deregister from the M2M System.

##### F.2.2.3.2	Pre-Conditions

The Pre-Conditions for Mca Received Requests are met.

The Application Entity is registered.

##### F.2.2.3.3	Signature – deregisterAE

Table F.2.2.3.3-1: RegistrationServcies – deregisterAE capability

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Mca Common request input parameters | - | - | See Table A.2.2-1 |
| aeId | IN | NO | The Application Entity Identifier (AE-ID) to be deregistered. |
| responseType | OUT | NO | Unique response types for this service: AE successfully De-registered Registration does not exist |

##### F.2.2.3.4	Service Interactions

The interactions of service capabilities required for this service capability:

- Perform the Common Request Services for requests across the Msc Reference Point

- Record the event

Figure F.2.2.3.4-1: Registration Services – deregisterAE Diagram

##### F.2.2.3.5	Post-Conditions

Success case: The request is permitted.

Failure case: The request is not permitted and a response type is transmitted back to the AE

##### F.2.2.3.6	Exceptions

No unique exceptions for this service capability.

Consumed services may throw exceptions which are forwarded by this service capability.

##### F.2.2.3.6	Policies for Use

Message Exchange Patterns: In-Out

Transaction Pattern: Participation allowed

Maximum Response: 300 ms

# Annex G (informative): M2M Service Capability Template

## G.1	Introduction

The M2M Service Capability template is used as a specification that defines the contract between the consumer of the service capability and the producer of the service capability. The specification contract includes documenting the provided Signature, constraints (Pre-Conditions, post-conditions, exceptions), policies and runtime expectations.

## G.2	<serviceCapabilityName>

<Document introductory description of the service capability>

The <serviceCapabilityName> reflects the operational functionality provided by the M2M Service Capability. M2M Service Capabilities are named using a construct [verb][noun][context] where the:

verb: Used to indicate what will happen to the noun

noun: Is the target of the noun

context: Is optional and further defines the service capability

### G.2.1	Description

The description of the service capability.

### G.2.2	Pre-Conditions

<Document the Pre-Conditions that may only be valid to run provided some testable prior business rule check or other value to be changed is actually present and available for the service capability to act upon. Pre-Conditions are documented that are in addition to that provided by the internal resource operations and procedures>

### G.2.3	Signature

Table G.2.3-1: M2M Service Capability Signature

| Parameter name | Direction | Optional | Description |
| --- | --- | --- | --- |
| Name of the parameter. | IN, OUT, IN-OUT | YES, NO | Description of the parameter in the context of the service capability. |

Direction: The direction is relative to the entity that provides (implements) the M2M Service Capability. The value "IN" means that the entity expects to receive a value for the parameter from the consumer (sender) of the M2M Service Capability request. The value "Out" means that the entity will send a value for the parameter to the consumer (sender) of the M2M Service Capability request. A value of "IN-OUT" means that the entity will receive the value for the parameter from the consumer and then send a value (not necessarily the same value) for the parameter back to the consumer.

Optional: Indicates if the parameter is required in requests (Direction: IN, IN-OUT) or responses (Direction: IN-OUT, OUT). A value of YES means that the parameter is optional. A value of NO means that the parameter is mandatory.

### G.2.4	Service Interactions

<Document the detail procedure for <serviceCapabilityName>. The detail procedure for the service capability is written as an Diagram in a time order and step-by-step. The Originator and what the Originator carries out (a task, function, accessing to resources or other services) are defined in each step.

### G.2.5	Post-Conditions

<Document the post-conditions of the <serviceCapabilityName>. If the service capability has no defined output that can be validated except a notification of success (or failure) to set a value in the target of the service capability, a testable post condition of the service capability is documented. Post-conditions are documented that are in addition to that provided by the internal resource operations and procedures.>

### G.2.6	Exceptions

<Document the exception or error conditions that may be returned by the <serviceCapabilityName>. Exceptions are documented that are in addition to that provided by the internal resource operations and procedures.>

### G.2.7	Policies for Use

Message Exchange Patterns : See clause 5.2.3.1Message Exchange Patterns for allowable patterns.

Transaction Pattern:

Creates Transaction: This service capability starts and completes a transaction

Participation allowed: This service capability is able to participate in a transaction without violating the ongoing transaction's properties (i.e. Atomicity, Consistency, Isolation, Durability)

Participation not allowed: This service capability is unable to participate in a transaction

Maximum Response: The maximum time in milliseconds permitted to complete this service capability.

### G.2.8	oneM2M Resource Interworking

<Document the related oneM2M Resources operations to <serviceCapabilityName> here. Interworking rules are defined for each Resource operation. If there is no related internal resource, this resource list can be empty (e.g. the service capability is 'sendTrigger').>.

# Annex H (informative): oneM2M Service Requirements

The primary motivation of the present documentis to integrate oneM2M Service capabilities with existing M2M deployments without affecting that existing M2M solution's underlying deployment framework and protocols. For example if an existing M2M solution uses a XMPP based framework to communicate between applications in the network and devices, then M2M Service capabilities are efficiently exposed to the XMPP based framework without requiring modifications to the existing M2M solution's deployment framework or protocol. A secondary motivation for the present documentis to offer M2M Service capabilities to M2M service providers who utilize service oriented architectural frameworks.

This annex defines requirements that have not been captured in the oneM2M Requirements [1].

| Requirement ID | Description |
| --- | --- |
| SCA-001 | The M2M System shall be capable of integrating oneM2M service capabilities with existing M2M deployments without affecting the existing M2M solution's underlying deployment framework and protocols. |
| SCA-002 | The M2M System shall be capable of offering oneM2M service capabilities to SOA architectural frameworks. |

# History

| History (to be removed on publication) |  |  |
| --- | --- | --- |
| V0.0.1 | 19-Feb-2014 | Rapporteur Input – Draft Skeleton |
| V.0.1.0 | 22-Feb-2014 | Added agreed contributions from TP9 ARC-2014-0081R02 |
| V.0.2.0 | 15-May-2014 | Added agreed contributions from TP10 and interim conference calls ARC-2014-1259-TS-0007-Reference_Update ARC-2014-1260R02-TS-0007-Mca-Request_Services ARC-2014-1324-TS-0007_Attribute_Alignment ARC-2014-1261R02-TS-0007-Mca-Sub-Pub-Notify |
| V.0.3.0 | 17-June-2014 | Added agreed contributions from TP11 ARC-2014-1390-TS-0007_IN-OUT_DeSignation ARC-2014-1391-TS-0007_Alignment_with_TS-0001_terms ARC-2014-1398-TS-0007_Clause-Role-Interworking-Removal ARC-2014-1401R02-TS-0007_Response_Code_Cleanup ARC-2014-1404-TS-0007-Service_Adminstration ARC-2014-1405-TS-0007-Service_Subscription_Adminstration |
| V.0.4.0 | 04-August-2014 | Added agreed contributions from TP12 ARC-2014-1411R02-TS-0007-Mca_Device_Firmware_management ARC-2014-1473R01-TS-0007-Mca_Device_Configuration_service ARC-2014-1474R01-TS-0007-Mca_Device_Topology_Management_service |
| V.0.5.0 | 01-October-2014 | Added agreed contributions from TP12 conference calls ARC-2014-1519R01-TS-0007-Request-Response-Data-Exchange-Pattern Added agreed contributions from TP13 Updated Normative Reference text ARC-2014-1534-TS-0007-Annex-A-Rework ARC-2014-1523R03-TS-0007-Device-Onboarding ARC-2014-1548R02-TS_0007_Event_Collection_service ARC-2014-1554-TS-0007-Section_7_DM_update ARC-2014-1560R01-TS-0007-Mca_Device_Troubleshooting_service ARC-2014-1555-TS-0007-Mca_Application_Software_Management_Service ARC-2014-1540R01-TS0007-Registration-Service ARC-2014-1541R01-TS0007-Supporting-Service-Registration-Services |
| V.0.5.1 | 17-October-2014 | Added agreed contributions from TP13 conference calls ARC-2014-1600-TS-0007-_ServiceCapabilities_CommonRequest_Alignment |
| V.0.6.0 | 24-November-2014 | Added agreed contributions from TP13.4 conference call (email) and TP14 ARC-2014-1615-TS-0007-M2M_Service_Capability_Alignment ARC-2014-1643-TS-0007-downloadFirmware&installFirmware_update ARC-2014-1644R01-TS-0007-Section_6_getManagementAdapter_update |
| V.0.6.1 | 05-January-2015 | Added agreed contributions from TP14.2 conference call ARC-2014-1688R02-TS-0007-Service_Subscription_Alignment ARC-2014-1689R06-TS0007-Registration_Alignment_with_Service_Subscription |
| V0.7.0 | 27-January-2015 | Added agreed contributions from TP15 plenary ARC-2015-0001-Msc-TS0007-Common-Request-Parameters ARC-2015-0002-TS-0007-Section_3_5_Editor_Notes_Cleanup ARC-2015-0004R02-TS_0007_clean_up_-_event_collection_filterCriteria ARC-2015-0008R04-TS-0007-DM_editor_notes_cleanup |
| V0.8.0 | 30-March-2015 | Added agreed contributions from TP16 plenary ARC-2015-1737-TS-0007-Non-DM-MSC-Notation_correction ARC-2015-1742-TS-0007-DM-MSC-Notation_correction ARC-2015-1743-TS-0007-DM_editor_note_cleanup |
| V0.8.1 | May 2015 | Pre-processing done before TB approval e-mail: mailto:edithelp@etsi.org |