| CHANGE REQUEST |  |
| --- | --- |
| Meeting:* | MAS#21 |
| Source:* | InterDigital |
| Date:* | 2016-01-09 |
| Contact:* | Qing Li, InterDigital, Qing.Li@InterDigital.com Chonggang Wang, InterDigital, Chonggang.Wang@InterDigital.com Catalina Mladin, InterDigital, Catalina.Mladin@InterDigital.com Martin Bauer, NEC, Martin.Bauer@neclab.eu |
| Reason for Change/s:* | R01: added more details based on the feedback from MAS-2016-0029R01 discussion. R02: 1) added intro text in each section; 2) moved definition and info reference into different sections. R03: moved Data Entity and Semantics Entity into definition subclause. |
| CR against: Release* | Rel-2 |
| CR against: WI* | Active WI-5 MNT Maintenance / < Work Item number(optional)> STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TR-0007 V.2.8.0 |
| Clauses/Sub Clauses* |  |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO This CR is a mirror CR? YES if YES, please indicate the document number of the original CR: <Document Number) : NO |
| Template Version:27 May 2015 (Dot not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separated “mirror CR” should be posted at the same time of this CR

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete sections need not show surrounding clauses as long as the proposed section number clearly shows where the new section is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

It’s an action item from TP20 to address the Access Control Policy management issue.

### -----------Start of change 1 - Reference section--------------------------------

## 2.2	Informative references

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	ETSI TS 102 690: "Machine-to-Machine communications (M2M); Functional architecture".

[i.2]	ETSI TR 101 584: "Machine-to-Machine communications (M2M); Study on Semantic support for M2M Data".

[i.3]	Open Geospatial Consortium.

NOTE:	Available at http://www.opengeospatial.org/.

[i.4]	Open Geospatial Consortium, Sensor Web Enablement DWG.

NOTE:	Available at http://www.opengeospatial.org/projects/groups/sensorwebdwg.

[i.5]	Open Geospatial Consortium, Sensor Web Enablement architecture, August 2008.

[i.6]	Open Geospatial Consortium, Observations and Measurements.

NOTE:	Available at http://www.opengeospatial.org/standards/om.

[i.7]	Open Geospatial Consortium, OpenGIS Sensor Model Language (SensorML).

NOTE:	Available at http://www.opengeospatial.org/standards/sensorml.

[i.8]	Open Geospatial Consortium, OpenGIS Transducer Markup Language (TML) Encoding Specification.

NOTE:	Available at http://www.opengeospatial.org/standards/tml.

[i.9]	Open Geospatial Consortium, Sensor Observation Service.

NOTE:	Available at http://www.opengeospatial.org/standards/sos.

[i.10]	Open Geospatial Consortium, Sensor Planning Service (SPS).

NOTE:	Available at http://www.opengeospatial.org/standards/sps.

[i.11]	Open Geospatial Consortium, Sensor Alert Service.

NOTE:	Available at http://www.opengeospatial.org/standards/requests/44.

[i.12]	W3C Semantic Sensor Network Incubator Group.

NOTE:	Available at http://www.w3.org/2005/Incubator/ssn/wiki/Main_Page.

[i.13]	Open Geospatial Consortium, Sensor Web Interface for IoT SWG.

NOTE:	Available at http://www.opengeospatial.org/projects/groups/sweiotswg.

[i.14]	Open Geospatial Consortium Best Practice, Semantic annotations in OGC standards.

NOTE:	Available at http://www.opengeospatial.org/node/1790.

[i.15]	Open Geospatial Consortium, Geography Markup Language.

NOTE:	Available at http://www.opengeospatial.org/standards/gml.

[i.16]	Semantic Sensor Network XG Final Report, W3C Incubator Group Report 28 June 2011.

NOTE:	Available at http://www.w3.org/2005/Incubator/ssn/XGR-ssn/.

[i.17]	Open Geospatial Consortium, Sensor Web Enablement DWG.

NOTE:	Available at http://www.opengeospatial.org/projects/groups/sensorwebdwg.

[i.18]	SSN Suggested Key Ontology Intro Attributes.

NOTE:	Available at http://www.w3.org/2005/Incubator/ssn/wiki/SSN_Suggested_Key_Ontology_Intro_Attributes.

[i.19]	W3C Semantic Sensor Network Incubator Group, Semantic Sensor Network Ontology.

NOTE	Available at http://www.w3.org/2005/Incubator/ssn/ssnx/ssn.

[i.20]	DUL ssn.

NOTE:	Available at http://www.w3.org/2005/Incubator/ssn/wiki/DUL_ssn.

[i.21]	HGI02029: "Smart Home Architecture and System Requirements".

[i.22]	W3C OWL Working Group, OWL 2 Web Ontology Language Document Overview.

NOTE:	Available at http://www.w3.org/TR/owl2-overview/.

[i.23]	W3C SPARQL Working Group, SPARQL 1.1 Overview.

NOTE:	Available at http://www.w3.org/TR/sparql11-overview/.

[i.24]	Jack Rusher, TripleStore, Semantic Web Advanced Development for Europe (SWAD-Europe), Workshop on Semantic Web Storage and Retrieval - Position Papers.

NOTE:	Available at http://www.w3.org/2001/sw/Europe/events/20031113-storage/positions/rusher.html.

[i.25]	oneM2M-TR-0001-UseCase: "oneM2M Use cases collection".

[i.26]	Resource Description Framework (RDF): "Concepts and Abstract Syntax".

NOTE:	Available at http://www.w3.org/TR/rdf-concepts/.

[i.27]	RDF Vocabulary Description Language 1.0: "RDF Schema".

NOTE:	Available at http://www.w3.org/TR/rdf-schema/.

[i.28]	SPARQL Query Language for RDF.

NOTE:	Available at http://www.w3.org/TR/rdf-sparql-query/.

[i.29]	Web Ontology Language (OWL).

NOTE:	Available at http://www.w3.org/2001/sw/wiki/OWL.

[i.30]	Dbpedia.

NOTE:	Available at http://dbpedia.org/About.

[i.31]	Turtle: Terse RDF Triple Language. Eric Prud'hommeaux and Gavin Carothers. W3C Last Call Working Draft, 10 July 2012.

NOTE:	Available at http://www.w3.org/TR/2012/WD-turtle-20120710/.
Latest version available at http://www.w3.org/TR/turtle/.

[i.32]	OWL Syntaxes.

NOTE:	Available at http://ontogenesis.knowledgeblog.org/88?kblog-transclude=2.

[i.33]	Rule Interchange Format (RIF) overview (second version).

NOTE:	Available at http://www.w3.org/TR/2013/NOTE-rif-overview-20130205/.

[i.34]	SWRL: A Semantic Web Rule Language Combining OWL and RuleML (Version 0.5).

NOTE:	Aavailable at http://www.daml.org/2003/11/swrl/.

[i.35]	SPIN - Overview and Motivation.

NOTE:	Available at http://www.w3.org/Submission/spin-overview/.

[i.36]	oneM2M Drafting Rules.

NOTE:	Available at http://member.onem2m.org/Static_pages/Others/Rules_Pages/oneM2M-Drafting-Rules-V1_0.doc.

[i.37]	oneM2M TS 0002: "onM2M Requirements".

[i.38]	Broadband Forum TR-069: "CPE WAN Management Protocol v1.1".

[i.39]	oneM2M TS 0001: "oneM2M Functional Architecture".

[i.40]	D-S4 - SMART 2013-0077 – Smart Appliances -  Final Study Report v1.0

[i.41]	An exemplar Data Annotation process: http://nemo.nic.uoregon.edu/wiki/Data_Annotation

NOTE:	Available at https://docs.google.com/file/d/0B2nnxMhTMGh4WTVsSVRsb01ha3c/edit

### ----------- End of change 1- Reference section -----------------------------

### ----------- Start of change 2 - Definition section -----------------------------

## 3.1	Definitions

For the purposes of the present document, the following terms and definitions apply:

abstraction: process of mapping between a set of Device Application Information Models and an Abstract Application Information Model according to a specified set of rules

attribute: see definition in oneM2M TS 0001 [i.39].

Data Annotation: process of providing and inserting data into a semantic structure specified by an ontology. Therefore, Data Annotation is the mapping from unstructured raw data into structured semantic data, i.e. from reality to ontology. An example of the Data Annotation process is described in [i.41].

Data Entity: a logical entity which contains data resources and related functionalities. It can also contain semantic resources and related functionalities, e.g. a temporary semantic graph store. The semantic functionalities are used to support the data functionalities locally.

Semantic Entity: a logical entity which contains semantic functionalities, e.g. it can contain a central graph store and related semantic reasoning and analytic functions, such as semantic annotation, semantic mash-up, etc.

ontology: formal specification of a conceptualization, that is defining Concepts as Objects with their properties and relationships versus other Concepts

physical entity: tangible element that is intrinsic to the environment, and that is not specific to a particular M2M application in this environment. Depending on the environment, the physical entity may be a smart phone, a camera, a smart TV/audio, a piece of furniture, somebody, a room of a building, a car, a street of a city, etc.

NOTE:	To be part of the M2M/IoT architecture, a physical entity does not need to be connected through a direct network interface, or even to be identified through a universal identification scheme such as RFID/EPC global, provided it can be sensed by sensors that are supposed to be deployed in this environment, and possibly acted upon by actuators.

relation: (also called "interrelation" or "property") stating a relationship among Concepts

EXAMPLE:	"is-part-of", "is-subtype-of".

thing: element of the environment that is individually identifiable in the M2M system

thing representation: it is the instance of the informational model of the Thing in the M2M System

NOTE:	A Thing Representation provides means for applications to interact with the Thing.

### ----------- End of change 2- Definition section -----------------------------

### -----------------------Start of change 3-------------------------------------------

8.5.5.5	Access Control for Data Resources and Semantic Triples

Access control for semantic information provided as semantic triples is discussed in this section. As the efficiency of access control depends also on the underlying implementation,  architectural and implementation perspectives are presented.

8.5.5.5.1	Access Control in a Hierarchically Layered Architecture

While the current oneM2M architectural view is exclusively based on resources, these can be implemented in different ways. Especially for managing semantic triples, triple stores are the straightforward choice. This has implications for how the access control to resources and triples is handled. In this section different implementation options are discussed assuming a hierarchical structuring into a semantic layer and a data layer.

As shown in Figure 8.5.5.1-1 and Figure 8.5.5.1-2, data resources and semantic triples may be integrated in a hierarchically layered architecture with a Data Layer containing the data resources and related functions and a Semantic Layer containing semantic triples and related semantic functions.

The upper layer, i.e. the Data Layer in Figure 8.5.5.1-1 and the Semantic Layer in Figure 8.5.5.1-2, controls and manages the Access Control Policies (ACPs). The lower layer, i.e. the Semantic Layer in Figure 8.5.5.1-1 and the Data Layer in Figure 8.5.5.1-2, supports the upper layer with semantic graphs or raw data respectively.

The layers may reside on different CSEs, but integration on the same CSE may be more performance efficient.

Figure 8.5.5.1-1 Access Control in a Hierarchically Layered Structure - Controlled by the Data Layer

Figure 8.5.5.1-2 Access Control in a Hierarchically Layered Structure - Controlled by the Semantic Layer

Figure 8.5.5.1-1 shows a typical data resource driven scheme for M2M scenarios. Resource Discovery through the data resource tree may be supported by the semantic leaves (i.e. distributed graph stores) in the Semantic Layer. The ACPs are maintained under <semanticDescriptor> resources in the Data Layer.

Figure 8.5.5.1-2 shows a typical semantics driven scheme for Semantic Web scenarios. Semantic Query may be conducted in the Semantic Layer with the return of the URI or URL of the data resources in the Data Layer. Semantic Resource Discovery may also be realized with the return of the data resources in the Data Layer via proper mapping between these two layers. The triples in the Semantic Layer are associated with their specific ACPs. A data resource in the Data Layer is addressed by a triple (e.g. via its URI or URL) associated with a ACP.

8.5.5.5.2	 Access Control in a Parallel Architecture

Looking beyond the currently supported semantic functionality, a more advanced architecture may be needed. In the following the concepts of Data Entity and Semantic Entity are introduced that may be used in different configurations, supporting more advanced semantic functionality like semantic mash-up and the possible interaction with other semantic platforms like the semantic web.

Figure 8.5.5.2-1 shows an exemplary scheme for intelligent IoT scenarios, which has more advanced data and semantics features or functions. As illustrated in Figure 8.5.5.2-1, a parallel architecture may have a Data Entity and a Semantic Entity.

A Data Entity and/or a Semantic Entity may each have its own Access Control Policies (ACPs) for managing the access control within its scope.

The semantic triples or data resources in the Data Entity may be exposed to the Semantic Entity with specific ACPs associated. For example,

- a Semantic Publication function in the Data Entity may expose semantic triples to the Central Graph Store in Semantic Entity with the corresponding ACPs associated with the triples; or

- a Data Annotation function in the Data Entity may expose data and associated ACPs to the Central Graph Store via the local temporary or caching Relational Data Base Management System (RDBMS) and Semantic Reasoning and Mapping functions in Semantic Entity.

The data resources or semantic triples in the Semantic Entity may also be exposed to the Data Entity with specific ACPs associated. For example,

- a Semantic Mash-up function in the Semantic Entity may expose new data resources and related ACPs from the semantic mash-up to the Data Entity; or

- a Semantic Annotation function in the Semantic Entity may expose semantic triples and related ACPs to the <semanticDescriptor> resources in the Data Entity.

Figure 8.5.5.2-1 Access Control in a Parallel Structure

Figure 8.5.5.2-1 shows that a Data Entity and a Semantic Entity may reside on different CSEs. But a Data Entity and a Semantic Entity may also reside on the same CSE.  Figure 8.5.5.2-2 shows a logical resource tree with both a Data Entity and a Semantic Entity.

Figure 8.5.5.2-2 Logic Resource Tree with both a Data Entity and a Semantic Entity

### ----------------------- End of change 3-------------------------------------------