| oneM2M Technical Specification |  |
| --- | --- |
| Document Number | Revised TS-0012-V3.0.0 |
| Document Name: | Base Ontology |
| Date: | 2016-October-28 |
| Abstract: | oneM2M's base ontology constitutes a basis framework for specifying the semantics of data that are handled in oneM2M. Sub-classes of some of its concepts are expected to be defined by other bodies in order to enable semantic interworking. In particular interworking with non-oneM2M systems (e.g. Area Networks and their devices) should be facilitated. |
| Template Version:23 February 2015 (Dot not modify) |  |

This Specification is provided for future development work within oneM2M only. The Partners accept no liability for any use of this Specification.

The present document has not been subject to any approval process by the oneM2M Partners Type 1.  Published oneM2M specifications and reports for implementation should be obtained via the oneM2M Partners' Publications Offices.

About oneM2M

The purpose and goal of oneM2M is to develop technical specifications which address the need for a common M2M Service Layer that can be readily embedded within various hardware and software, and relied upon to connect the myriad of devices in the field with M2M application servers worldwide.

More information about oneM2M may be found at:  http//www.oneM2M.org

Copyright Notification

© 2016, oneM2M Partners Type 1 (ARIB, ATIS, CCSA, ETSI, TIA, TSDSI, TTA, TTC).

All rights reserved.

The copyright extends to reproduction in all media.

Notice of Disclaimer & Limitation of Liability

The information provided in this document is directed solely to professionals who have the appropriate degree of experience to understand and interpret its contents in accordance with generally accepted engineering or other professional standards and applicable regulations. No recommendation as to products or vendors is made or should be implied.

NO REPRESENTATION OR WARRANTY IS MADE THAT THE INFORMATION IS TECHNICALLY ACCURATE OR SUFFICIENT OR CONFORMS TO ANY STATUTE, GOVERNMENTAL RULE OR REGULATION, AND FURTHER, NO REPRESENTATION OR WARRANTY IS MADE OF MERCHANTABILITY OR FITNESS FOR ANY PARTICULAR PURPOSE OR AGAINST INFRINGEMENT OF INTELLECTUAL PROPERTY RIGHTS. NO oneM2M PARTNER TYPE 1 SHALL BE LIABLE, BEYOND THE AMOUNT OF ANY SUM RECEIVED IN PAYMENT BY THAT PARTNER FOR THIS DOCUMENT, WITH RESPECT TO ANY CLAIM, AND IN NO EVENT SHALL oneM2M BE LIABLE FOR LOST PROFITS OR OTHER INCIDENTAL OR CONSEQUENTIAL DAMAGES. oneM2M EXPRESSLY ADVISES ANY AND ALL USE OF OR RELIANCE UPON THIS INFORMATION PROVIDED IN THIS DOCUMENT IS AT THE RISK OF THE USER.

Contents

1	Scope	6

2	References	6

2.1	Normative references	6

2.2	Informative references	6

3	Definitions and abbreviations	7

3.1	Definitions	7

3.2	Abbreviations	7

4	Conventions	7

5	General information on the oneM2M Base Ontology (informative)	8

5.1	Motivation and intended use of the ontology	8

5.1.1	Why using ontologies in oneM2M?	8

5.1.1.1	Introduction to ontologies	8

5.1.1.2	The purpose of the oneM2M Base Ontology	9

5.1.1.2.0 	Introduction	9

5.1.1.2.1	Syntactic interoperability	9

5.1.1.2.2	Semantic interoperability	9

5.1.2	How are the Base Ontology and external ontologies used?	10

5.1.2.1	Overview	10

5.1.2.2	Introduction to usage of classes, properties and restrictions	10

5.1.2.3	Methods for jointly using the Base Ontology and external ontologies	10

5.2	Insights into the Base Ontology	11

5.2.1	General design principles of the Base Ontology	11

5.2.1.1	General Principle	11

5.2.1.2	Essential Classes and Properties of the Base Ontology	12

5.2.2	void	16

6	Description of Classes and Properties	16

6.1	Classes	16

6.1.1	Class: Thing	16

6.1.2	Class: ThingProperty	17

6.1.3	Class: Aspect	18

6.1.4	Class: MetaData	19

6.1.5	Class: Device	20

6.1.6	Class: InterworkedDevice	22

6.1.7	Class: AreaNetwork	23

6.1.8	Class: Service	24

6.1.9	Class: Function	26

6.1.9.0	General description	26

6.1.9.1	Class: ControllingFunction	27

6.1.9.2	Class: MeasuringFunction	27

6.1.10	Class: Operation	28

6.1.10.0	General description	28

6.1.10.1	Class: GET_InputDataPoint	30

6.1.10.2	Class: SET_OutputDataPoint	30

6.1.11	Class: Command	31

6.1.12	Class: OperationInput	32

6.1.13	Class: OperationOutput	34

6.1.14	Class: OperationState	35

6.1.15	Class: InputDataPoint	36

6.1.16	Class: OutputDataPoint	38

6.1.17	Class: Variable	39

6.1.18	Class: SimpleTypeVariable	41

6.2	Object Properties	42

6.2.1	Void	42

6.2.2	Void	42

6.2.3	Object Property: consistsOf	42

6.2.4	Object Property: describes	42

6.2.5	Object Property: exposesCommand	42

6.2.6	Object Property: exposesFunction	43

6.2.7	Object Property: hasCommand	43

6.2.8	Object Property: hasFunction	43

6.2.9	Object Property: hasInput	43

6.2.10	Object Property: hasInputDataPoint	44

6.2.11	Object Property: hasMetaData	44

6.2.12	Void	44

6.2.13	Object Property: hasOperation	44

6.2.14	Object Property: hasOperationState	44

6.2.15	Void	45

6.2.16	Object Property: hasOutput	45

6.2.17	Object Property: hasOutputDataPoint	45

6.2.18	Object Property: hasService	45

6.2.19	Object Property: hasSubStructure	45

6.2.20	Object Property: hasThingProperty	46

6.2.21	Object Property: hasThingRelation	46

6.2.22	Void	46

6.2.23	Void	46

6.2.24	Void	46

6.2.25	Object Property: isPartOf	46

6.2.26	Object Property: refersTo	46

6.3	Data Properties	47

6.3.1	Data Property: hasDataType	47

6.3.2	Data Property: hasDataRestriction	48

6.3.2.0	General description	48

6.3.2.1	Data Property: hasDataRestriction_minInclusive	49

6.3.2.2	Data Property: hasDataRestriction_maxInclusive	49

6.3.2.3	Data Property: hasDataRestriction_minExclusive	49

6.3.2.4	Data Property: hasDataRestriction_maxExclusive	49

6.3.2.5	Data Property: hasDataRestriction_length	49

6.3.2.6	Data Property: hasDataRestriction_minLength	49

6.3.2.7	Data Property: hasDataRestriction_maxLength	49

6.3.2.8	Data Property: hasDataRestriction_pattern	49

6.3.2.9	Data Property: hasDataRestriction_langRange	49

6.3.3	Data Property: hasValue	49

6.3.4	Data Property: netTechnologyCommunicationProtocol	50

6.3.5	Data Property: netTechnologyPhysicalStandard	50

6.3.6	Data Property: netTechnologyProfile	50

6.3.7	Data Property: oneM2MTargetURI	50

6.3.8	Data Property: oneM2MAttribute	51

6.3.9	Data Property: oneM2MMethod	51

6.3.10	Data Property: hasThingAnnotation	51

6.4	Annotation Properties	52

6.4.1	Annotation Property: resourceDescriptorLink	52

7	Instantiation of the Base Ontology and external ontologies to the oneM2M System	52

7.1	Instantiation rules for the Base Ontology	52

7.1.1	Instantiation of classes of the oneM2M Base Ontology and derived external ontologies in the oneM2M System:	52

7.1.1.1	General on instantiating classes of the Base Ontology in the oneM2M System	52

7.1.1.2	Instantiation of individual classes of the Base Ontology	53

7.1.2	Instantiation of Object Properties	58

7.1.3	Instantiation of Data Properties	59

7.1.4	Instantiation of Annotation Properties	59

7.2	Common mapping principles between the Base Ontology and external ontologies	59

8	void	60

9	void	60

Annex A (normative): OWL representation of Base Ontology	61

Annex B (informative): Mappings of selected external ontologies to the Base Ontology	62

B.1	Mapping of SAREF	62

B.1.1	Introduction to SAREF	62

B.1.3	Mapping SAREF to oneM2M resource structure	66

B.1.3.1	Introduction	66

B.1.3.2	Mapping rules	66

B.1.3.3	Example showing the uses of the semanticDescriptor resource and instantiation in the oneM2M resource structure	66

History	72

# 1	Scope

The present document contains the specification of the oneM2M base ontology. A formal OWL representation of the base ontology can be found at http://www.onem2m.org/ontology/Base_Ontology. 
The present document also specifies an instantiation of the base ontology in oneM2M resources which is required for generic interworking.

In addition the present document contains the functional specification for an Interworking Proxy Application Entity (IPE), the oneM2M resources and their usage for generic interworking.

Finally an example is given how external ontologies can be mapped to the base ontology. The example uses the Smart Appliances REFerence (SAREF) ontology (http://ontology.tno.nl/saref).

# 2	References

## 2.1	Normative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the reference document (including any amendments) applies.

The following referenced documents are necessary for the application of the present document.

[1]	oneM2M TS-0011: "Common Terminology".

[2]	oneM2M TS-0001: "Functional Architecture".

[3]	W3C Recommendation: "RDF 1.1 Concepts and Abstract Syntax".

## 2.2	Informative references

References are either specific (identified by date of publication and/or edition number or version number) or non-specific. For specific references, only the cited version applies. For non-specific references, the latest version of the reference document (including any amendments) applies.

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	oneM2M Drafting Rules.

NOTE:	Available at http://www.onem2m.org/images/files/oneM2M-Drafting-Rules.pdf.

[i.2]	The Smart Appliances REFerence (SAREF) ontology.

NOTE:	Available at http://ontology.tno.nl/saref/.

[i.3]	Open-source ontology editor PROTÉGÉ.

NOTE:	Available at http://protege.stanford.edu/.

[i.4]	W3C OWL Working Group: "OWL 2 Web Ontology Language Document Overview".

NOTE:	Available at http://www.w3.org/TR/owl2-overview/.

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in oneM2M TS-0011 [1] and the following apply:

annotation property: property that can be used to add information (metadata/data about data) to classes, individuals and Object/Data Properties

class: OWL standard ontology language from the World Wide Web Consortium (W3C) (see [i.4]), Concepts are called "Classes"

concept: entity of an Ontology that has an agreed, well defined, meaning within the domain of interest of that ontology

NOTE:	A Concept is conceptually grouping a set of Individuals.

data property: property that relates an individual of a Class to data of a stecified type and range

interworked device: non-oneM2M device (NoDN) for which communication with oneM2M entities can be achieved via an Interworking Proxy Application Entity (IPE)

ontology: formal specification of a conceptualization, that is defining Concepts as objects with their properties and relationships versus other Concepts

generic interworking: generic interworking allows interworking with many types of non- oneM2M Area Networks and Devices that are described in the form of a oneM2M compliant ontology which is derived from the oneM2M Base Ontology

NOTE:	Generic interworking supports the interworking variant "full mapping of the semantic of the non-oneM2M data model to Mca" as indicated in clause F.2 of oneM2M TS-0001 [2].

object property: property that relates an individual of  a domain Class to an individual of a range Class

property: in OWL standard ontology language Properties represent relations among individuals

NOTE:	Properties can be sub-categorized as Object Properties, Data Properties and Annotation Properties.

proxied device: virtual Device (i.e. a set of oneM2M resources together with an IPE) that represents the Interworked Device in the oneM2M System

relation: (also called "interrelation" or "property") stating a relationship among individuals

restriction: describes a class of individuals based on the relationships that members of the class participate in

NOTE:	Restrictions can be sub-categorized as: existential Restrictions, universal Restrictions, Cardinality restrictions and hasValue Restrictions.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in oneM2M TS-0011 [1] and the following apply:

AE	Application Entity

OWL	Web Ontology Language

SAREF	Smart Appliances REFerence ontology

SPARQL	SPARQL Protocol and RDF Query Language

# 4	Conventions

The key words "Shall", "Shall not", "May", "Need not", "Should", "Should not" in the present document are to be interpreted as described in the oneM2M Drafting Rules [i.1].

# 5	General information on the oneM2M Base Ontology (informative)

## 5.1	Motivation and intended use of the ontology

### 5.1.1	Why using ontologies in oneM2M?

#### 5.1.1.1	Introduction to ontologies

In a nutshell an ontology is a vocabulary with a structure. The vocabulary applies to a certain domain of interest (e.g. metering, appliances, medicine, etc.) and it contains concepts that are used within that domain of interest, similar to the "defined terms" in clause 3, "Definitions".

An ontology should:

Capture a shared understanding of a domain of interest.

Provide a formal and machine manipulable model of the domain.

The ontology lists and denominates these concepts which have agreed, well defined, meanings within the domain of interest (e.g. the concept of "Device" has an agreed, well defined, meaning within the scope of the Smart Appliances REFerence (SAREF) ontology see [i.2]).

Concepts do not identify individuals but they identify classes of individuals. Therefore, in the OWL standard ontology language from the World Wide Web Consortium (W3C) (see [3]), concepts are called "Classes".

The structure part of the ontology is introduced through agreed, well defined, relationships between its concepts. Such a relationship - in OWL called "Object Property" - links a subject concept to an object concept.

subject concept  relationship  object concept

in OWL:

domain Class  Object Property  range Class

EXAMPLE 1:	In SAREF an Object Property "accomplishes" relates the "Device" class to the "Task" class:

Device  accomplishes  Task

Also the relationships/Object Properties of an ontology have agreed, well defined, meanings within the domain of interest. In the example above the "accomplishes" part of the relationship is well documented as part of SAREF (see [i.2]).

A second type of Properties in OWL is called "Data Properties". A Data Property is linking a subject Class to a data. These data may be typed or untyped.

EXAMPLE 2:	in SAREF the Data Property "hasManufacturer" links the class "Device with data of datatype "Literal":

Device  hasManufacturer  Literal

Again, the Data Properties of an ontology have agreed, well defined, meanings within the domain of interest.
In the example 2, the Data Property "hasManufacturer" indicates that the Literal, that is linked via this Data Property will indicate the manufacturer of the Device.

Data Properties can be considered similar to attributes in oneM2M.

A third type of Properties in OWL is called "AnnotationProperties". An Annotation Property is used to provide additional information about ontology elements like classes and instances, which typically are external to the ontology and would not be used for reasoning. Example usages for such additional information are for providing a creator, a version or a comment. The object of an annotation property  is either a data literal, a URI reference, or an individual.

In general, an individual of a certain Class may or may not have a particular relation (Object Property, Data Property or Annotation Property) that is defined by the ontology. However, if such a relation exists for the individual then that relation should be used with the meaning specified by the ontology.

One additional, crucial aspect differentiates an ontology from a vocabulary with a structure. An ontology enables specified, allowed constructs (based on predicate logic) and can be represented in a formal, machine interpretable form e.g. by the OWL standard ontology language. This allows the creation of queries (e.g. through the SPARQL query language) that search for individuals of specified classes, having specified relationships, etc.

The OWL flavour OWL-DL (where DL stands for "Description Logic"), that is used in the present document and that is supported by the ontology-editing tool "Protégé" (see [i.3]), has the additional advantage that it is underpinned by a description logic. For ontologies that fall into the scope of OWL-DL a reasoner can be used to automatically check the consistency of classes, take what has explicitly stated in the ontology and use it to infer new information. OWL-DL ensures that queries are decidable.

Additionally, OWL-DL allows the creation of Intersection, Union and Complement classes,  restrictions (e.g. on the required/allowed number of relationships for any individual of the Class along this property) an other useful constructs.

#### 5.1.1.2	The purpose of the oneM2M Base Ontology

##### 5.1.1.2.0 	Introduction

Ontologies and their OWL representations are used in oneM2M to provide syntactic and semantic interoperability of the oneM2M System with external systems. These external systems are expected to be described by ontologies.

The only ontology that is specified by oneM2M is the oneM2M Base Ontology, as described in the present document. However, external organizations and companies are expected to contribute their own ontologies that can be mapped (e.g. by sub-classing, equivalence..) to the oneM2M Base Ontology. 
Such external ontologies might describe specific types of devices (as e.g. in the SAREF ontology) or, more generally, they might describe real-world "Things" (like buildings, rooms, cars, cities.) that should be represented in a oneM2M implementation. The value for external organizations and companies to provide their ontologies to oneM2M consists in supplementing oneM2M data with information on the meaning/purpose of these data. The OWL representation of that ontology provides a common format across oneM2M.

The oneM2M Base Ontology is the minimal ontology (i.e. mandating the least number of conventions) that is required such that other ontologies can be mapped into oneM2M.

##### 5.1.1.2.1	Syntactic interoperability

Syntactic interoperability is mainly used for interworking with non-oneM2M devices in Area Networks. In this case an ontology - represented as an OWL file - that contains the Area Network specific types of communication parameters (names of operations, input/output parameter names, their types and structures, etc.) is used to configure an Interworking Proxy Entity (IPE). 
With the help of this OWL file the IPE is able to allocate oneM2M resources (AEs, containers) that are structured along the Area Network specific parameters and procedures. This enables oneM2M entities to read/write from/into these resources such that the IPE can serialize the data and send/receive them from/to the devices in the Area Network.

The semantic meaning of these resources is implicitly given by the interworked Area Network technology.

Each ontology that describes a specific type of interworked Area Network needs to be derived from the oneM2M Base Ontology. In particular the device types of an ontology of an interworked Area Network need to be mapped (e.g. by sub-typing) into the concept "Interworked Device" of the oneM2M Base Ontology.

##### 5.1.1.2.2	Semantic interoperability

Semantic interoperability is mainly used to describe functions provided by oneM2M compliant devices (M2M Devices). 
For example, different, oneM2M compliant types of washing machines may all perform a Function like "washing-function" ,"drying-function", "select wash temperature"…, however the oneM2M resources (containers), through which these functions can be accessed, can have different resourceNames, child-structures and type of content.
In this case an ontology - represented as an OWL file -contains the specific types of the M2M Application Service and/or Common Service of the M2M Device (e.g. CRUD operation, resourceNames, child-structures and type of content, etc.) together with the Function of that service (e.g. "washing-function").

Each ontology that describe a specific type of M2M Device needs to be derived from the oneM2M Base Ontology. In particular the device type needs to be mapped (e.g. by sub-typing) into the concept "Device" of the oneM2M Base Ontology.

### 5.1.2	How are the Base Ontology and external ontologies used?

#### 5.1.2.1	Overview

This clause describes how an external ontology that is compatible with the Base Ontology can be used in a joint fashion.

NOTE: 	Further use of external ontologies is left to subsequent releases.

#### 5.1.2.2	Introduction to usage of classes, properties and restrictions

An ontology consists of  Properties and Classes.

Properties represent relationships, and link individuals from the specified domain (a class) to individuals from the specified  range (another class). There are two main types of properties in the Base Ontology, object properties and data properties. An object property describes a relationship between two object individuals. A data properties describes a relationship between an object individuals and a concrete data value that may be typed or untyped.

Classes are interpreted as sets of individuals, and sometimes classes are also seen as a concrete representation of concepts. In the Base Ontology, a Class can be directly defined by the class name and class hierarchy or defined by the properties characteristics of  the individuals in the class. The latter method is known as restriction. The classes defined by restriction can be anonymous, which contains all of the individuals that satisfy the restriction.

In the Base Ontology, the restrictions can be divided as existential restrictions, universal restrictions and cardinality restrictions:

Existential restrictions describe classes of individuals that participate in at least one (some) relationship along a given property to individuals that are members of the class, e.g. since a Device (Class: Device) has at least one function (Object Property: hasFunction) (Class: Function) that this device accomplishes, then (Class: Device) is a subclass of the anonymous class of (Object Property: hasFunction) some (Class: Function).

Universal restrictions describe classes of individuals that for a given property only have relationships along this property to individuals that are members of  the class. For example, since a subclass "Watervalve" of (Class: Device) only has a Function (Object Property: hasFunction) subclass "Open_or_Close_Valve" of (Class: Function), then (Class:Watervale) is a superclass of the anonymous class of (Object Property: hasFunction) only (Class: Open_or_Close_Valve).

Cardinality restrictions describe classes of individuals that, for a given property, only have a specified number of relationships along this property to individuals that are members of  the class.

#### 5.1.2.3	Methods for jointly using the Base Ontology and external ontologies

If the Base Ontology is available and the external ontologies are compatible with the Base Ontology, the Base Ontology and the external ontologies can be jointly used in the following ways.

Classes and properties mapping:

The names of the class and properties in different ontologies may be totally different, but the meanings of these class and properties can be relevant. Classes and proporties mapping is used to link the relevant classes and properties in different ontologies.

The descriptions for the classes and  properties mapping  relationship of the Base Ontology and external ontologies can be given in an ontology or a semantic rule depending on the frequency of the usage. For the frequent cases, it is better to give the mapping description in an ontology, even in the Base Ontology.

The classes and properties mapping can be based on the properties defined in OWL and RDFs, e.g. rdfs:subClassOf, owl:equivalentClass, for classifying the hierarchy of the classes and properties in Base Ontology and external ontologies. The inheritance from upper properties and classes will be implied according to the mapped hierarchy. For example, when a class A in an external ontology is mapped as a subclass of  the class B in the Base Ontology, it implies that the properties of class B in the Base Ontology will be inherited by the class A in the external ontology.

- Table 1 gives a simple example for  classes and properties mapping between two ontologies.

Table 1: An example for classes and properties mapping between two ontologies

| Properties mapping |  |  | Classes mapping |  |  |
| --- | --- | --- | --- | --- | --- |
| property I | mapping relationship | property II | class I | mapping relationship | class II |
| OntologyB: hasSwitch | rdfs:subPropertyOf | OntologyA: hasOperation | OntologyB: appliance | rdfs:subClassOf | OntologyA:device |
| OntologyA: hasPower | owl:equivalentProperty | OntologyB: hasPower | OntologyB:lamp | owl:equivalentClass | OntologyA:light |
| OntologyA: hasVendor | owl:equivalentProperty | OntologyB: hasManufacturer | OntologyB:Switch | rdfs:subClassOf | OntologyA:Operation |

Individual annotation across multiple ontologies:

Though the names of the class and properties in different ontologies may be totally different, the semantic annotation for individuals can be done based on these different ontologies respectively and  independently. In this way, the knowledge from different ontologies are used together to describe the individuals.

- Table 2 gives a simple example for individual annotation across two ontologies.

Table 2: An example for individual annotation across two ontologies

| Individuals | Semantic annotation based on Ontology A |  | Semantic annotation based on Ontology B |  |
| --- | --- | --- | --- | --- |
|  | Properties | classes | properties | Classes |
| Light A | rdf:type | Ontology A: Light | rdf:type | Ontology B:ledLight |
|  | OntologyA: hasOperation | Ontology A:Open | OntologyB: hasColor | rdf:datatype="&xsd;string">'red'< |
|  | OntologyA: hasStatus | rdf:datatype="&xsd;boolean">true< | OntologyB: hasSwitch | OntologyB:Switch |
| NOTE: The two methods can be used jointly or independentlly. |  |  |  |  |

The compatibility of two ontologies depends on their class hierarchies. When the class hierarchy of one ontology can be mapped as a part or an external part of  the class hierarchy of the other ontology, they are compatible. When multiple ontologies are pairwise compatible, they are compatible.

## 5.2	Insights into the Base Ontology

### 5.2.1	General design principles of the Base Ontology

#### 5.2.1.1	General Principle

The Base Ontology has been designed with the intent to provide a minimal number of concepts, relations and restrictions that are necessary for semantic discovery of entities in the oneM2M System. To make such entities discoverable in the oneM2M System they need to be be semantically described as classes (concepts) in a - technology/vendor/other-standard specific - ontology and these classes (concepts) need to be related to some classes of the Base Ontology as sub-classes.
Additionally, the Base Ontology enables non-oneM2M technologies to build derived ontologies that describe the data model of the non-oneM2M technology for the purpose of interworking with the oneM2M System.

The Base Ontology only contains Classes and Properties but not instances because the Base Ontology and derived ontologies are used in oneM2M to only provide a semantic description of the entities they contain. 
Instantiation (i.e. data of individual entities represented in the oneM2M System - e.g. devices, things, etc.) is done via oneM2M resources

The Base Ontology is available at the web page:

http://www.onem2m.org/ontology/Base_Ontology;

which contains the latest version of the ontology and individual versions of the ontology (see Annex A)

#### 5.2.1.2	Essential Classes and Properties of the Base Ontology

Figure 1: The oneM2M Base Ontology

Figure 1 shows the essential Classes and Properties of the Base Ontology. The nodes (bubbles) denote Classes whereas edges (arrows) denote Object Properties.

The graph in figure 1 can be read as follows:

A Thing in oneM2M (Class: Thing) is an entity that can be identified in the oneM2M System. 
A Thing may have properties (Object Property: hasThingProperty).
A Thing can have relations to other things (Object Property: hasThingRelation).
E.g. A room that is modelled in oneM2M would be a Thing that could have a room-temperature as a ThingProperty (via hasThingProperty) and could have a hasThingRelation "isAdjacentTo" to another room.
In general it isassumed that a Thing is not able to conmmunicate electronically with its environment. However, the sub-class of Thing that is able to interact electronically is called a "Device".

A ThingProperty (Class: ThingProperty) denotes a property of a Thing. A Thing can be described with (the values of) ThingProperties, but in general the Thing cannot influence that value or being influenced by it. A human or a computer or a device could set the value of a Thing's ThingProperty and possibly read it. A ThingProperty be can be retrieved or updated by an entity of the oneM2M System.
E.g. the indoor temperature of the room could be a Value of a Thing "room", or the manufacturer could be a ThingProperty of a Thing "car".
A ThingProperty of a thing can describe a certain Aspect, e.g. the indoor temperature describes the Aspect "Temperature" that could be measured by a temperature sensor.
A ThingProperty of a Thing can have meta data

Variable (Class: Variable) constitutes a super class to the following classes: ThingProperty, OperationInput, OperationOutput, OperationState, InputDataPoint, OutputDataPoint. Its members are entities that have some data (e.g. integers, text, etc., or structured data) that can change over time. These data of the Variable usually describe some real-world Aspects (e.g. a temperature) and can have MetaData (e.g. units, precision).
A Variable can be structured, i.e. it can consist of (sub-) Variables.

One sub-class is defined in the base ontology:

SimpleTypeVariable (Class: SimpleTypeVariable) is a sub-class of Variable that only consists of Variables of simple xml types like xsd:integer, xsd:string…, potentially including restrictions.

MetaData (Class: MetaData) contain data (like units, precision-ranges, etc.) about the Values of a Thing or about an Aspect. 
E.g. the indoor temperature could have meta data: "Degrees Celsius".

A Device (Class: Device) is a Thing (a sub-class of class:Thing) that is designed to accomplish a particular task via the Functions the Device performs. 
A Device can be able to interact electronically with its environment via a network
. 
A Device contains some logic and is producer and/or consumer of data that are exchanged via its Services with other entities (Devices, Things) in the network. A Device interacts through the DataPoints and/or Opertions of its Services.
In the context of oneM2M a Device is always assumed to be capable of communicating electronically via a network (oneM2M or interworked non-oneM2M network):

In order to accomplish its task, the device performs one or more Functions (Object Property: hasFunction) (Class: Function). 
These Functions are exposed in the network as Services of the Device.

A Device can be composed of several (sub-) Devices (Object Property: consistsOf) (Class: Device).
=> consistsOf only Device.

Each Device (including sub-Devices) needs to be individually addressable in the network.

E.g. a "lightswitch" would be a device, a combined fridge/freezer would be a device that consists of a sub-device fridge and a sub-device freezer.

A Function (Class: Function) represents the Function necessary to accomplish the task for which a Device is designed. A device can be designed to perform more than one Function.
The Class: Function exhibits the - human understandable - meaning what the device "does":

A Function refers to (e.g. observes or influences) a certain Aspect.

E.g. considering a "light switch" then a related Function could be "Controlling_ON_OFF".
These Functions would refer to an Aspect "lighting", that is influenced by the device "light switch".

Two sub-classes of class Function are defined in the base ontology:

ControllingFunction (Class: ControllingFunction) is a sub-class of Function that only controls/influences real world Aspects that the Function relates to.

MeasuringFunction (Class: MeasuringFunction) is a sub-class of Function that only measures/senses real world Aspects that the Function relates to.

An Aspect (Class: Aspect) describes the real-world aspect that a Function relates to. Aspect is also used to describe a quality or kind of OperationInput- or OperationOutput variables. The Aspect could be a (physical or non-physical) entity or it could be a quality.

A Command (Class: Command) represents an action that can be performed to support the Function. An Operation exposes a Command to the network. OperationInput and OperationOutput of the related Operation can parameterize the command.
e.g. the Function "Dimming-Function" could have a Command "setPercentage", with a parameter that has values 0 - 100.

A Service (Class: Service) is a representation of a Function to a network that makes the Function discoverable, registerable, remotely controllable in the network. A Service can represent one or more Functions. A Service is offered by a device that wants (a certain set of) its Functions to be discoverable, registerable, remotely controllable by other devices in the network:

While a Function describes the meaning of the device's Function the Service (Class: Service) is used to describe how such Function is represented in a communication network and is therefore dependent on the technology of the network.

E.g. the Function: "turn_light_On_or_Off" could be exposed in the network by a Service "Binary Value Actuator".

A Service may be composed of smaller, independent (sub)Services, e.g. re-usable servicemodules.

An OutputDataPoint (class: OutputDataPoint) is a Variable of a Service that is set by a RESTful Device in its environment and that provides state information about the Service. The Device updates the OutputDataPoint autonomously (e.g. at periodic times). To enable a third party to retrieve the current value of a OutputDataPoint (out of schedule) devices often also offer a SET_OutputDataPoint Operation to trigger the device to update the data of the OutputDataPoint.

An InputDataPoint (class: InputDataPoint) is a Variable of a Service that is set by a RESTful Device in its environment and that the Device readsout autonomously (e.g. at periodic times). To enable a third party to instruct the device to retrieve (out of schedule) the current value of a InputputDataPoint devices often also offer a GET_InputDataPoint Operation to trigger the device to retrieve the data from the InputDataPoint.

NOTE 1:	Input- and Output DataPoints are usually used by Devices (AEs) that communicate in a RESTful way, while Operations are the procedures that are used for remote procedure based communication. Operations are, however, also needed in RESTful systems to correlate output, that is produced by a device, to the input that triggered the production of that output.

An Operation (Class: Operation)  is the means of a Service to communicate in a procedure-type manner over the network (i.e. transmit data to/from other devices). 
An Operation is a representation of a Command to a network:

An Operation can have OperationInput (data consumed by the Device) and OperationOutput (Data produced by the Device), as well as a Method that describes how the Operation is invoked over the network.

An Operation shall have a Data Property "OperationState" that indicates how the operation has progressed in the device.

An Operation is transient. I.e. an Operation can be invoked, possibly produces output and is finished.

An Operation correlates the output data of the Operation to the input data that were used at Operation invokation.

Two sub-classes of class Operation are defined in the base ontology:

GET_InputDataPoint (Class: GET_InputDataPoint) is a sub-class of Operation that may be offered by a Device to trigger the device to retrieve the data of an InputDataPoint.
(e.g. outside of the schedule when the device normally retrieves that DataPoint).

SET_OutputDataPoint (Class: SET_OutputDataPoint) is a sub-class of Operation that may be offered by a Device to trigger the device to update the data of an OutputDataPoint.
(e.g. outside of the schedule when the device normally updates that DataPoint).

OperationInput (Class: OperationInput) describes the type of input of an Operation to a service of the device. The OperationInput class represents all possible values for that input (data types and -ranges or a list of enumerated individuals). An Operation can have multiple OperationInputs and/or OperationOutputs. If an instance of an Operation is executed then the input value to that Operation  is an instance of its OperationInput classes (e.g. enumerated instances like "ON" or "OFF" for an OperationInput class that sets the state of a switch or a real number within a certain range for a "Temperature" OperationInput class for a thermostat).

OperationOutput (Class: OperationOutput) describes the type of output of an Operation from a service of the device. The OperationOutput class represents all possible values for that OperationOutput (data types and -ranges or a list of enumerated individuals). An Operation can have multiple OperationInputs and/or OperationOutputs. If an instance of an Operation is executed then the output values of that Operation are instances of its OperationOutput classes.

OperationState (Class: OperationState) describes the current state during the lifetime of an Operation. The OperationState class represents all possible values for that state (enumerated individuals). The OperationState is set during the progress of the operation by the entity invoking the operation, the entity that is the target of the operation, e.g. a device (or for interworked devices by the IPE) and the CSE. It takes values like 
"data_received_by_application",  "operation_ended", "operation_failed", "data_transmitted_to_interworked_device".

Area Network (Class: AreaNetwork):

An Area Network is characterized by its technology:

physical properties (e.g. IEEE_802_15_4_2003_2_4GHz); its

communication protocol (e.g. ZigBee_1_0); and

potentially a profile (e.g. ZigBee_HA).

Interworked Device (Class: InterworkedDevice):

Is part of an AreaNetwork.

NOTE 2:	An Interworked Device is not a oneM2M Device and can be only accessed from the oneM2M System by communicating with a "proxied" (virtual) device that has been created by an Interworking Proxy Entity (IPE).
The InterworkedDevice class describes the "proxied" (virtual) device that is represented in the oneM2M System as an individual <AE> resource or a child resource of the <AE> of its IPE.

### 5.2.2	void

# 6	Description of Classes and Properties

## 6.1	Classes

### 6.1.1	Class: Thing

Figure 3: Thing

Description

A Thing in oneM2M (Class: Thing) is an entity that can be identified in the oneM2M System. 
A Thing that is not a Device is not able to conmmunicate electronically with its environment. However, the sub-class of Thing that is able to interact electronically is called a "Device".
A Thing may have ThingProperties (Object Property: hasThingProperty). A Thing can have relations to other things (Object Property: hasThingRelation).
Since a Thing that is not a Device is not able to conmmunicate electronically it cannot influence the value of its ThingProperties or being influenced by it. Similarly a Thing cannot document its - real-world - relationships (via hasThingRelation) to other Things.

E.g. A room that is modelled in oneM2M would be a Thing that could have a room-temperature as a ThingProperty and could have a relationship "isAdjacentTo" to another room.

Object Properties

This Class is the domain Class of Object Property:

hasThingProperty (range Class: ThingProperty)

hasThingRelation (range Class: Thing)

This Class is the range Class of Object Property:

hasThingRelation (domain Class: Thing)

Data Properties

hasThingAnnotation (range datatype: rdfs:Literal)

Superclass-subclass Relationships

This Class is subclass of:

none

This Class is superclass of:

device

Restrictions

This Class is anonymous sub-class of:

hasThingRelation only Thing
(Universal restriction: a Thing can only have a relationship "hasThingRelation" to other Things)

hasThingProperty only ThingProperty

### 6.1.2	Class: ThingProperty

Figure 4: ThingProperty

Description

A ThingProperty (Class: ThingProperty) denotes a property of a Thing. A ThingProperty can e.g. be observed or influenced by devices, or it constitutes static data about a Thing.
E.g. the indoor temperature of the room could be a ThingProperty of a Thing "room".
A ThingProperty of a thing can describe a certain Aspect, e.g. the indoor temperature describes the Aspect "Temperature" that could be measured by a temperature sensor.
A ThingProperty of a Thing can have meta data.

The class ThingProperty is a sub-class of the Variable class.

Object Properties

This Class is the domain Class of Object Property:

describes (range Class: Aspect)
(inherited from class: Variable)

hasMetaData (range Class: MetaData) 
(inherited from class: Variable)

This Class is the range Class of Object Property:

hasThingProperty (domain Class: Thing)

Data Properties

This Class is part of the domain Class of Data Property:

Inherited from class: Variable and possibly Class SimpleTypeVariable.
(see clauses 6.1.17 and 6.1.18.)

Superclass-subclass Relationships

This Class is sub-class of:

Variable

This Class is super-class of:

None

Restrictions

None

### 6.1.3	Class: Aspect

Figure 5: Aspect

Description

An Aspect (Class: Aspect) describes the real-world aspect that a Function relates to. Aspect is also used to describe the quality or kind of a Variable.
The Aspect could be a (physical or non-physical) entity or it could be a quality.

Object Properties

This Class is the domain Class of Object Property:

hasMetaData (range Class: MetaData)

This Class is the range Class of Object Property:

refersTo (domain Class: Function)

describes (domain Class: Variable)

Data Properties

none

Superclass-subclass Relationships

This Class is sub-class of:

none

This Class is super-class of:

none

Restrictions

none

### 6.1.4	Class: MetaData

Figure 6: MetaData

Description

MetaData (Class: MetaData) contain data (like units, precision-ranges …) about a Variable or about an Aspect. 
E.g. the indoor temperature could have as meta data an individual "Celsius_Scale" that specifies that the temperature needs to be understood as degrees Celsius.

Object Properties

This Class is the domain Class of Object Property:

none

This Class is the range Class of Object Property:

hasMetaData (domain Class: Variable)

hasMetaData (domain Class: Aspect)

Data Properties

none

Superclass-subclass Relationships

This Class is sub-class of:

none

This Class is super-class of:

none

Restrictions

none

### 6.1.5	Class: Device

Figure 7: Device

Description

A Device (Class: Device) is a Thing (a sub-class of class:Thing) that is designed to accomplish a particular task via the Functions the Device performs. 
A Device can be able to interact electronically with its environment via a network. A Device contains some logic and is producer and/or consumer of data that are exchanged via its Services with other oneM2M entities (Devices, Things) in the network. A Device may be a physical or non-physical entity.
A Device interacts through the DataPoints and/or Operations of its Services:

In order to accomplish its task, the device performs one or more Functions.

These Functions are exposed in the network as Services of the Device.

A Device can be composed of several (sub-) Devices.

Each Device (including sub-Devices) needs to be individually addressable in the network.

Object Properties

This Class is the domain Class of Object Property:

consistsOf (range Class: Device)

hasService (range Class: Service)

hasFunction (range Class: Function)

hasThingProperty (range Class: ThingProperty)
(inherited from Class:Thing)

This Class is the range Class of Object Property:

consistsOf (domain Class: Device)

Data Properties

none

Superclass-subclass Relationships

This Class is sub-class of:

Thing

This Class is super-class of:

InterworkedDevice

Restrictions

This Class is anonymous sub-class of:

consistsOf only Device
(Universal restriction: a Device can have a relationship "consistsOf" only to other Devices)

hasFunction min 1 Function
(Universal restriction: a Device must have a relationship " hasFunction " to at least 1 Function)

hasService only Service
(Universal restriction: a Device can have a relationship " hasService " only to Services)

### 6.1.6	Class: InterworkedDevice

Figure 8: InterworkedDevice

Description

An InterworkedDevice (Class: InterworkedDevice) is a Device - e.g. in an Area Network - that does not support oneM2M interfaces and can only be accessed from the oneM2M System by communicating with a "proxied" (virtual) device that has been created by an Interworking Proxy Entity.

Object Properties

This Class is the domain Class of Object Property:

isPartOf (range Class: AreaNetwork)

This Class is the range Class of Object Property:

none

Data Properties

none

Superclass-subclass Relationships

This Class is sub-class of:

Device

This Class is super-class of:

none

Restrictions

none

### 6.1.7	Class: AreaNetwork

Figure 9: AreaNetwork

Description

An AreaNetwork (Class: AreaNetwork) is a Network that provides data transport services between an Interworked Device and the oneM2M System. Different area Networks can use heterogeneous network technologies that may or may not support IP access.

Object Properties

This Class is the domain Class of Object Property:

none

This Class is the range Class of Object Property:

isPartOf (domain Class: InterworkedDevice)

Data Properties

netTechnologyPhysicalStandard (range datatype: rdf:PlainLiteral) which serves for Identification of the physical properties of a Area Network technology (e.g. IEEE_802_15_4_2003_2_4GHz)

netTechnologyCommunicationProtocol (range datatype: rdf:PlainLiteral) which serves for Identification of a communication protocol (e.g. ZigBee_1_0)

netTechnologyProfile (range datatype: rdf:PlainLiteral) which serves for Identification of a profile (e.g. ZigBee_HA) of a Area Network technology

Superclass-subclass Relationships

This Class is sub-class of:

none

This Class is super-class of:

none

Restrictions

none

### 6.1.8	Class: Service

Figure 10: Service

Description

A Service (Class: Service) is a electronic representation of a Function in a network. The Service exposes the Function to the network and makes it discoverable, registerable and remotely controllable in the network. 
A Service is offered by a device that wants (a certain set of) its Functions to be discoverable, registerable, remotely controllable by other devices in the network.
A Service can expose one or more Functions and a Function can be exposed by one or more Services.

The Input- and Output DataPoints and Operations of a Service may have the same names as for a different Service, however the Service to which they belong differentiates how they are addressed in the Device (e.g. via a port specific to the Service).

NOTE:	While a Function  describes the - human understandable - meaning of a Service of the device the Service is used to describe how such Function is represented in a communication network and can be accessed by electronic means. The Service and its Operations is therefore dependent on the technology of the network, hard- and software of the device.

E.g. the Function: "turn_light_On_or_Off" could be exposed in the network by a Service "UPDATE Binary Value".

Object Property "hasSubService" is expresses the fact that Services can be composed of independent (sub)Services.
E.g. a Service could thus be composed out of multiple (reusable) service modules. A Dimmer could contain a module "binaryActuator" to turn on/off and additionally "setInteger0-255Actuator" to set the dimming level.

Object Properties

This Class is the domain Class of Object Property:

exposesFunction (range Class: Service)

hasOperation (range Class: Operation)

hasInputDataPoint (range Class: InputDataPoint)

hasOutputDataPoint (range Class: OutputDataPoint)

hasSubService (range Class: Service)

If for a Service an Operation, Input-, OutputDataPoint or sub-Service is mandatory then the related Object Property (hasOperation, hasInputDataPoint, hasOutputDataPoint, hasSubService) shall have a Property Restriction with cardinality “min 1”.

This Class is the range Class of Object Property:

hasService  (domain Class: Device)

hasSubService (domain Class: Service)

Data Properties

none

Superclass-subclass Relationships

This Class is sub-class of:

none

This Class is super -class of:

none

Restrictions

hasSubService only Service
(Universal restriction: a Service can have a relationship "hasSubService" only to other Service)

hasOperation only Operation

exposesFunction some Function
(Universal restriction: at least one of the relationship " exposesFunction "of a Service must point o a Function)

hasInputDataPoint only InputDataPoint

hasOutputDataPoint only OutputDataPoint

### 6.1.9	Class: Function

#### 6.1.9.0	General description

Figure 11: Function

Description

A Function (Class: Function) represents a particular function necessary to accomplish the task for which a Device is designed. A device can be designed to perform more than one Function.
The Function exhibits the - human understandable - meaning what the device "does".

A Function refers to (e.g. observes or influences) some real-world aspect(s), that can be modelled as a Class: Aspect.

E.g. considering a "light switch" then a related Function could be "Controlling_ON_OFF" or "Controlling Brightness". These Functions would refer to an Aspect "light-control".

A Function of a Device can be influenced/observed by a human user through the Commands that this Function has and that are offered to the user.

Object Properties

This Class is the domain Class of Object Property:

hasCommand (range Class: Command)

refersTo (range Class: Aspect)

This Class is the range Class of Object Property:

exposesFunction (domain Class: Service)

hasFunction  (domain Class: Device)

Data Properties

none

Superclass-subclass Relationships

This Class is sub-class of:

none

This Class is super-class of:

ControllingFunction

MeasuringFunction

Restrictions

none

#### 6.1.9.1	Class: ControllingFunction

Description

A ControllingFunction (Class: ControllingFunction) represents a Function that has impacts on the real world, but does not gather data. In general a ControllingFunction has Commands (and/or Operations of its related Services) that receive input data.

E.g. a thermostat would have "temperature-adjustment" as a ControllingFunction.

Object Properties

This Class is the domain Class of Object Property:

none

This Class is the range Class of Object Property:

none

Data Properties

none

Superclass-subclass Relationships

This Class is sub-class of:

Function

This Class is super-class of:

none

Restrictions

none

#### 6.1.9.2	Class: MeasuringFunction

Description

A MeasuringFunction (Class: MeasuringFunction) represents a Function that has no impacts on the real world, but only gathers data. In general a MeasuringFunction has Commands (and/or Operations of its related Services) that generate output data.

E.g. a temperature sensor would have "temperature-sensing" as a MeasuringFunction.

Object Properties

This Class is the domain Class of Object Property:

none

This Class is the range Class of Object Property:

none

Data Properties

none

Superclass-subclass Relationships

This Class is sub-class of:

Function

This Class is super-class of:

none

Restrictions

none

### 6.1.10	Class: Operation

#### 6.1.10.0	General description

Figure 12: Operation

Description

An Operation (Class: Operation)  is the means of a Service to communicate in a procedure-type manner over the network (i.e. transmit data to/from other devices). It is the -machine interpretable- exposure of a -human understandable- Command to a network.
An Operation is transient. I.e. an Operation can be invoked, possibly produces output and is finished:

A non-oneM2M Device or a oneM2M entity (e.g. an AE) can invoke an Operation of the Device (oneM2M Device or InterworkedDevice) and that invocation can trigger some action in the Device. If an Operation has input data it may receive input data from:

InputDataPoints (persistent entities); and/or

OperationInput (transient entities, that are deleted when the Operation finishes);

and potentially produce output data into:

OutputDataPoints (persistent entities) and/or

OperationOutput (transient entities, that are deleted when the Operation finishes)

An Operation correlates the output data of the Operation to the input data that were used at Operation invokation.

An Operation has an OperationState that allows a oneM2M entity to get informed on the progress of that operation.

NOTE:	The OperationState - which provides information of an ongoing or finished operation - should not be confused with state information about the Device or Service, which potentially could be obtained as output data of some operation.

Object Properties

This Class is the domain Class of Object Property:

exposesCommand (range Class: Command)

hasInput (range Class: OperationInput)

hasInputDataPoint (range Class: InputDataPoint)

hasOutput (range Class: OperationOutput)

hasOutputDataPoint (range Class: OutputDataPoint)

hasOperationState (range Class: OperationState)

- If for an Operation an OperationInput, OperationOutput, Input-, or OutputDataPoint is mandatory then the related Object Property (hasInput, hasOutput, hasInputDataPoint, hasOutputDataPoint) shall have a Property Restriction with cardinality “min 1”.

- If for an Operation the OperationState is mandatory then the related Object Property (hasOperationState) shall have a Property Restriction with cardinality “exactly 1”.

This Class is the range Class of Object Property:

hasOperation (range Class: Service)

Data Properties

oneM2MMethod (range data type: rdf:PlainLiteral)

oneM2MTargetURI (range data type: rdfs: Literal)

Superclass-subclass Relationships

This Class is sub-class of:

none

This Class is super-class of:

GET_InputDataPoint

SET_OutputDataPoint

Restrictions

exposesCommand only Command

hasInput only OperationInput

hasOperationState only OperationState

hasOutput only OperationOutput

#### 6.1.10.1	Class: GET_InputDataPoint

Description

GET_InputDataPoint (Class: GET_InputDataPoint) is an Operation that may be offered by a Device to trigger the device to retrieve the data of an InputDataPoint 
(e.g. outside of the schedule when the device normally retrieves data from that DataPoint)

Object Properties

This Class is the domain Class of Object Property:

hasInputDataPoint (range Class: InputDataPoint)

This Class is the range Class of Object Property:

none

Data Properties

none

Superclass-subclass Relationships

This Class is sub-class of:

Operation

This Class is super-class of:

none

Restrictions

none

#### 6.1.10.2	Class: SET_OutputDataPoint

Description

SET_OutputDataPoint (Class: SET_OutputDataPoint) is an Operation that may be offered by a Device to trigger the device to update the data of an OutputDataPoint 
(e.g. outside of the schedule when the device normally updates that DataPoint)

Object Properties

This Class is the domain Class of Object Property:

hasOutputDataPoint (range Class: OutputDataPoint)

This Class is the range Class of Object Property:

none

Data Properties

none

Superclass-subclass Relationships

This Class is sub-class of:

Operation

This Class is super-class of:

none

Restrictions

none

### 6.1.11	Class: Command

Figure 13: Command

Description

A Command (Class: Command) represents an action that can be performed to support the Function. A Command is the -human understandable - name of that action that is invoked in a device or is reported by the device. An Operation exposes a Command to the network. OperationInput and OperationOutput of the related Operation can parameterize the command.
e.g. the Function "dimming-Function" of a light switch that remotely controls a light could have a Command "setLightIntensity", with a parameter that has values 0 - 100 %.

Also InputDataPoints and OutputDataPoints expose Commands to the network. When a Device communicates in a RESTful way then changing (UPDATEing) an InputDataPoint triggers an action in the Device once the Device has read out the data from the InputDataPoint.
Similarly, when a Device sets the data of an OutputDataPoint then it provides state information about the Device.

NOTE:	In RESTful systems the names of Input- and OutputDataPoints are usually chosen in such a way that they express the Command, i.e. the human-understandable meaning (e.g. a binary InputDataPoint of a lightswitch could have a name "Set_Light_Status"). Updating a DataPoint can be interpreted as executing a Command.

Object Properties

This Class is the domain Class of Object Property:

isExposedByOperation (range Class: Operation)

hasInput (range Class: OperationInput)

hasOutput (range Class: OperationOutput)

This Class is the range Class of Object Property:

hasCommand (domain Class: Function)

exposesCommand (domain Class: Operation OR InputDataPoint OR OutputDataPoint)

Data Properties

none

Superclass-subclass Relationships

This Class is sub-class of:

none

This Class is super-class of:

none

Restrictions

hasInput only OperationInput
(Universal restriction: a Command can have a relationship " hasInput " only to OperationInput)

hasOutput only OperationOutput

hasInputDataPoint only InputDataPoint

hasOutputDataPoint only OutputDataPoint

### 6.1.12	Class: OperationInput

Figure 14: OperationInput

Description

OperationInput (Class: OperationInput) describes an input of an Operation of a Service. OperationInput also describes the input of a Command:

OperationInput is transient. An instance of OperationInput is deleteted when the instance of its Operation is deleted.

An Operation/Command may have multiple OperationInputs and/or OperationOutputs. If an instance of an Operation is invoked then the input value to that Operation shall be an instance of its OperationInput class.

Object Properties

This Class is the domain Class of Object Property:

hasMetaData (range Class: MetaData)
(inherited from class:Variable)

describes (range Class: Aspect)
(inherited from class:Variable)

This Class is the range Class of Object Property:

hasInput (domain Class: Operation)

hasInput (domain Class: Command)

Data Properties

This Class is part of the domain Class of Data Property:

Inherited from class: Variable and possibly Class SimpleTypeVariable
(see clauses 6.1.17 and 6.1.18)

Superclass-subclass Relationships

This Class is sub-class of:

Variable

NOTE:	Since class:SimpleTypeVariable is a sub-class of class:Variable a specific instance of OperationInput may also be a SimpleTypeVariable.

This Class is super-class of:

none

Restrictions

none

### 6.1.13	Class: OperationOutput

Figure 15: OperationOutput

Description

OperationOutput (Class: OperationOutput) describes an output of an Operation. OperationOutput also describes the output of a Command.

OperationOutput is transient. An instance of OperationOutput is deleteted when the instance of its Operation is deleted.

An Operation/Command may have multiple OperationInputs and/or OperationOutputs.

Object Properties

This Class is the domain Class of Object Property:

hasMetaData (range Class: MetaData)
(inherited from class:Variable)

describes (range Class: Aspect)
(inherited from class:Variable)

This Class is the range Class of Object Property:

hasOutput (domain Class: Operation)

hasOutput (domain Class: Command)

Data Properties

This Class is part of the domain Class of Data Property:

Inherited from class: Variable and possibly Class SimpleTypeVariable
(see clauses 6.1.17 and 6.1.18)

Superclass-subclass Relationships

This Class is sub-class of:

Variable

NOTE:	Since class:SimpleTypeVariable is a sub-class of class:Variable a specific instance of OperationOutput may also be a SimpleTypeVariable

This Class is super-class of:

none

Restrictions

none

### 6.1.14	Class: OperationState

Figure 16: OperationState

Description

OperationState (Class: OperationState) describes the current state of an Operation. The OperationState class represents all possible values for that state (enumerated individuals). The OperationState is set during the progress of the operation by the CSE and, optionally, the entity that is the target of the operation, e.g. a device (or for interworked devices by the IPE).

This class contains a text string that is provided by the AE (e.g. an IPE). Values for that text that are specified in oneM2M are:

"data_received_by_application"

"operation_ended"

"operation_failed"

"data_transmitted_to_interworked_device"

Additional values for the text string of the operationState attribute are permissible.

Object Properties

This Class is the range Class of Object Property:

hasOperationState (domain Class: Operation)

Data Properties

This Class is part of the domain Class of Data Property:

Inherited from class: Variable
(see clause 6.1.17)

hasDataType (range data type: xsd:string)
(inherited from class: SimpleTypeVariable, see clause 6.1.18)

hasDataRestriction_Pattern (range data type: xsd:string{"data received by application", "operation ended", "operation failed", "data transmitted to interworked device"})
(inherited from class: SimpleTypeVariable, see clause 6.1.18)

Superclass-subclass Relationships

This Class is sub-class of:

SimpleTypeVariable

This Class is super-class of:

none

Restrictions

hasDataRestriction_pattern exactly 1 xsd:string

(hasDataRestriction_pattern value "data_received_by_application"^^xsd:string) or (hasDataRestriction_pattern value "data_transmitted_to_interworked_device"^^xsd:string) or (hasDataRestriction_pattern value "operation_ended"^^xsd:string) or (hasDataRestriction_pattern value "operation_failed"^^xsd:string)

Note: A OperationState has a Data Property “hasDataRestriction” with a pattern of exactly 1 xsd:string. 
The value of that xsd:string can be: "data_received_by_application", "data_transmitted_to_interworked_device", "operation_ended" or "operation_failed".

### 6.1.15	Class: InputDataPoint

Figure 17: InputDataPoint

Description

InputDataPoint (Class: InputDataPoint) is a Variable of a Service that is accessed by a RESTful Device in its environment and that the Device reads out autonomously (e.g. at periodic times). To enable a third party to instruct the device to retrieve (out of schedule) the current value of a InputputDataPoint devices often also offer a GET_InputDataPoint Operation to trigger the device to retrieve the data from the InputDataPoint:

An InputDataPoint is a persistent entity.

NOTE 1:	Input- and Output DataPoints are usually used by Devices (AEs) that communicate in a RESTful way, while Operations are the procedures that are used for remote procedure based communication. Operations are, however, also needed in RESTful systems to correlate output, that is produced by a device, to the input that triggered the production of that output.

Object Properties

This Class is the domain Class of Object Property:

hasMetaData (range Class: MetaData)
(inherited from class:Variable)

describes (range Class: Aspect)
(inherited from class:Variable)

This Class is the range Class of Object Property:

hasInputDataPoint (domain Class: Operation)

hasInputDataPoint (domain Class: Command)

Data Properties

This Class is part of the domain Class of Data Property:

Inherited from class: Variable and possibly Class SimpleTypeVariable
(see clauses 6.1.17 and 6.1.18)

Superclass-subclass Relationships

This Class is sub-class of:

Variable

NOTE 2:	Since class:SimpleTypeVariable is a sub-class of class:Variable a specific instance of InputDataPoint may also be a SimpleTypeVariable.

This Class is super-class of:

none

Restrictions

none

### 6.1.16	Class: OutputDataPoint

Figure 18: OutputDataPoint

Description

OutputDataPoint (Class: OutputDataPoint) is a Variable of a Service that is set by a RESTful Device in its environment and that the Device updates autonomously (e.g. at periodic times). To enable a third party to instruct the device to update (out of schedule) the current value of a OutputputDataPoint devices often also offer a SET_OutputDataPoint Operation to trigger the device to update the data of the OutputDataPoint:

An OutputDataPoint is a persistent entity.

NOTE 1:	Input- and Output DataPoints are usually used by Devices (AEs) that communicate in a RESTful way, while Operations are the procedures that are used for remote procedure based communication. Operations are, however, also needed in RESTful systems to correlate output, that is produced by a device, to the input that triggered the production of that output.

Object Properties

This Class is the domain Class of Object Property:

hasMetaData (range Class: MetaData)
(inherited from class:Variable)

describes (range Class: Aspect)
(inherited from class:Variable)

This Class is the range Class of Object Property:

hasOutputDataPoint (domain Class: Operation)

hasOutputDataPoint (domain Class: Command)

Data Properties

This Class is part of the domain Class of Data Property:

Inherited from class: Variable and possibly Class SimpleTypeVariable
(see clauses 6.1.17 and 6.1.18)

Superclass-subclass Relationships

This Class is sub-class of:

Variable

NOTE 2:	Since class:SimpleTypeVariable is a sub-class of class:Variable a specific instance of OutputDataPoint may also be a SimpleTypeVariable.

This Class is super-class of:

none

Restrictions

none

### 6.1.17	Class: Variable

Figure 19: Variable

Description

A Variable (Class: Variable) constitutes a super class to the following classes: ThingProperty, OperationInput, OperationOutput, OperationState, InputDataPoint, OutputDataPoint, SimpleTypeVariable. Its members are entities that store some data (e.g. integers, text, etc., or structured data) that can change over time. 
These data of the Variable usually describe some real-world Aspects (e.g. a temperature) and can have MetaData (e.g. units, precision, etc.)

Object Properties

This Class is the domain Class of Object Property:

hasMetaData (range Class: MetaData)

describes  (range Class: Aspect)

hasSubStructure (range Class: Variable)

- If a Variable has a sub structure for which certain parts (i.e. Variables of the sub-straucture) are mandatory then the related Object Property (hasSubStructure) shall have a Property Restriction with cardinality “min 1”.

This Class is the range Class of Object Property:

hasSubStructure (domain Class: Variable)

Data Properties

oneM2MMethod (range datatype:  rdf:PlainLiteral)
This data property contains a oneM2M Method through which the oneM2M instantiation of the value of the Variable can be manipulated by the communicating entity:

It contains the string "RETRIEVE" for retrieving the variable when the oneM2M resource is of type <container> or <flexContainer>. This applies to sub-classes: OperationOutput, OutputDatapoint, ThingProperty and OperationState.

It contains the string "CREATE" for updating the variable when the oneM2M resource is of type <container>. This applies to sub-classes: OperationInput, InputDatapoint, ThingProperty.

It contains the string "UPDATE" for updationg the variable when the oneM2M resource is of type <flexContainer>. This applies to sub-classes: OperationInput, InputDatapoint, ThingProperty.

oneM2MTargetURI (range data type: rdfs: Literal)
This data property contains the URI of a oneM2M resource (<container> or <flexContainer>) through which the oneM2M instantiation of the value of the Variable can be manipulated by the communicating entity. It can contain an absolute address or an address relative to the <semanticDescriptor> resource that holds the RDF description of the Variable. 
That address could be e.g. the value of the parentID for the <container> or <flexContainer> of a Input- or OutputDataPoint which has child-resource of type <semanticDescriptor> that holds the RDF description of the DataPoint.

Superclass-subclass Relationships

This Class is sub-class of:

none

This Class is super-class of:

ThingProperty

OperationInput, OperationOutput

OperationState

InputDataPoint

OutputDataPoint

SimpleTypeVariable

Restrictions

hasSubStructure only Variable

### 6.1.18	Class: SimpleTypeVariable

Figure 20: SimpleTypeVariable

Description

SimpleTypeVariable (Class: SimpleTypeVariable) is a sub-class of class:Variable that only consists of Variables of simple xml types like xsd:integer, xsd:string, etc., potentially including restrictions

The simple datatypes and -restrictions contained in "OWL 2 Web Ontology Language Structural Specification and Functional-Style Syntax (Second Edition)" [i.4] are supported.

Object Properties

This Class is the domain Class of Object Property:

none

This Class is the range Class of Object Property:

none

Data Properties

This Class is part of the domain Class of Data Property:

Inherited from class: Variable. 
(see clause 6.1.17)

hasValue (range data type: rdfs: Literal)
This data property contains the value of the Variable if that value is part of the semantic description and is not contained in a different resource (identified by the oneM2MTargetURI data property). Storing the value of a Variable in a semantic description (i.e. as part of the RDF description in the semanticDescriptor resource) is useful for values that are relatively static (e.g. the name of the manufacturer):

Data properties "hasValue" and "oneM2MTargetURI" are mutually exclusive. Only one of the two shall be instantiated.

oneM2MAttribute (range data: rdf:PlainLiteral)
This Data Property contains the name of the attribute of the oneM2M resource (of type <container> or <flexContainer>) that is referenced with the oneM2MTargetURI and  that stores the value of the SimpleTypeVariable

hasDataType (range datatype: rdf:PlainLiteral)
This Data Property contains the datatype of the SimpleTypeVariable as text string

hasDataRestriction (range datatype: rdf:PlainLiteral)
This Data Property contains a restriction of value of the the SimpleTypeVariable

Superclass-subclass Relationships

This Class is sub-class of:

Variable

This Class is super-class of:

OperationState

Restrictions

hasDataType exactly 1 rdf:PlainLiteral

hasDataRestriction only rdf:PlainLiteral

hasSubStructure exactly 0 Variable

## Note: A Simple Type Variable has a data type but no other variables as sub-stucture

## 6.2	Object Properties

### 6.2.1	Void

### 6.2.2	Void

### 6.2.3	Object Property: consistsOf

Description

A Device can consist of (i.e. be composed) of several (sub-) Devices

Domain Class

Device

Range Class

Device

### 6.2.4	Object Property: describes

Description

A Variable describes an Aspect (a quality or kind)

Domain Class

Variable

Range Class

Aspect

### 6.2.5	Object Property: exposesCommand

Description

A -machine interpretable- Operation or an Input/OutputDataPoint of a Service exposes a -human understandable- Command to a network.

Domain Class

Operation

Range Class

Command

### 6.2.6	Object Property: exposesFunction

Description

A Service exposes a Function to the network and makes it discoverable, registerable and remotely controllable in the network.

Domain Class

Service

Range Class

Function

### 6.2.7	Object Property: hasCommand

Description

A Function of a Device can be influenced/observed by a human user through the Commands that this Function has and that are offered to the user

Domain Class

Function

Range Class

Command

### 6.2.8	Object Property: hasFunction

Description

In order to accomplish its task, a Device performs one or more Functions

Domain Class

Device

Range Class

Function

### 6.2.9	Object Property: hasInput

Description

An Operation of a Service of the Device or a Command of a Function of the Device can have OperationInput data.

Domain Class

Operation

Command

Range Class

OperationInput

### 6.2.10	Object Property: hasInputDataPoint

Description

A Service or an Operation of a Service of the Device can have InputDataPoints. Communicating entities write data into InputDataPoints and the Device retrieves the data at times according to an internal schedule.
An InputDataPoint can also contain the data that are used as input to an Operation.

Domain Class

Operation

Service

Range Class

InputDataPoint

### 6.2.11	Object Property: hasMetaData

Description

A Variable can have MetaData (like units, precision-ranges, etc.)

Domain Class

Variable

Range Class

MetaData

### 6.2.12	Void

### 6.2.13	Object Property: hasOperation

Description

A Service communicates by means of Operations over the network to transmit data to/from other devices

Domain Class

Service

Range Class

Operation

### 6.2.14	Object Property: hasOperationState

Description

An Operation may have an OperationState that is exposed

Domain Class

Operation

Range Class

OperationState

### 6.2.15	Void

### 6.2.16	Object Property: hasOutput

An Operation of a Service of the Device or a Command of a Function of the Device can have OperationOutput data

Domain Class

Operation

Command

Range Class

OperationOutput

### 6.2.17	Object Property: hasOutputDataPoint

Description

A Service or an Operation of a Service of the Device can have OutputDataPoints. The Device writes data into OutputDataPoints at times according to an internal schedule and the communicating entitis retrieves the data.
An OutputDataPoint can also contain the data that are created as ouput of an Operation.

Domain Class

Operation

Service

Range Class

OutputDataPoint

### 6.2.18	Object Property: hasService

Description

The Functions of a Device are exposed in the network as Services of the Device

Domain Class

Device

Range Class

Service

### 6.2.19	Object Property: hasSubStructure

Description

A structured Variable can be composed of (sub-)Variables

Domain Class

Variable

Range Class

Variable

### 6.2.20	Object Property: hasThingProperty

Description

A Thing may have properties that can be described by Values

Domain Class

Thing

Range Class

Value

### 6.2.21	Object Property: hasThingRelation

Description

A Thing may have relations to itself or to other Things

Domain Class

Thing

Range Class

Thing

### 6.2.22	Void

### 6.2.23	Void

### 6.2.24	Void

### 6.2.25	Object Property: isPartOf

Description

An InterworkedDevice consist a part of an AreaNetwork

Domain Class

InterworkedDevice

Range Class

AreaNetwork

### 6.2.26	Object Property: refersTo

Description

A Function of a Device can refer to a certain Aspect (a quality or kind) that is measured or controlled by that Function. 
e.g. a temperature sensor would refer to the Aspect "Temperature" that it measures

Domain Class

Function

Range Class

Aspect

## 6.3	Data Properties

### 6.3.1	Data Property: hasDataType

Note that in the pesent document the name space identifier for:

- 'http://www.w3.org/2001/XMLSchema' shall be referred to using the prefix: xsd

- 'http://www.w3.org/2002/07/owl' shall be referred to using the prefix: owl

- 'http://www.w3.org/1999/02/22-rdf-syntax-ns' shall be referred to using the prefix: rdf

Description

This Data Property specifies the data type  of the SimpleTypeVariable as URI

Domain Class

SimpleTypeVariable

Range Datatype

xsd:anyURI

Permissible URIs are:

for Numbers types 
(possible restrictions: xsd:minInclusive, xsd:maxInclusive, xsd:minExclusive, xsd:maxExclusive):

owl:real

owl:rational

xsd:decimal

xsd:integer

xsd:nonNegativeInteger

xsd:nonPositiveInteger

xsd:positiveInteger

xsd:negativeInteger

xsd:long

xsd:int

xsd:short

xsd:byte

xsd:unsignedLong

xsd:unsignedInt

xsd:unsignedShort

xsd:unsignedByte

for PlainLiteral - contains all String types  
(possible restrictions: xsd:length, xsd:minLength, xsd:maxLength, xsd:pattern, rdf:langRange):

rdf:PlainLiteral

for String types
(possible restrictions: xsd:length, xsd:minLength, xsd:maxLength, xsd:pattern):

xsd:string

xsd:normalizedString

xsd:token

xsd:language

xsd:Name

xsd:NCName

xsd:NMTOKEN

for Boolean Values (no restrictions):

xsd:boolean

for Binary Data types
(possible restrictions: xsd:minLength, xsd:maxLength, xsd:length):

xsd:hexBinary

xsd:base64Binary

for IRIs
(possible restrictions: xsd:minLength, xsd:maxLength, xsd:length, xsd:pattern):

xsd:anyURI

- for Time Instants
(possible restrictions: sd:minInclusive, xsd:maxInclusive, xsd:minExclusive,  xsd:maxExclusive):

xsd:dateTime

xsd:dateTimeStamp

for Literals:

rdf:XMLLiteral

### 6.3.2	Data Property: hasDataRestriction

#### 6.3.2.0	General description

Description

This Data Property specifies the restrictions on the data type of the SimpleTypeVariable.

Domain Class

SimpleTypeVariable

Range Datatype

rdf:PlainLiteral

The Data Property "hasDataRestriction" shall always be sub-classed as one of the following specializations.

The range of a sub-classed Data Property "hasDataRestriction" shall be instantiated as the value restricting the data. E.g. a value 100 in the range of Data Property: hasDataRestriction_minInclusive specifies that the SimpleTypeVariable can only take values greater or equal to 100.

#### 6.3.2.1	Data Property: hasDataRestriction_minInclusive

For applicability of this sub-class of data property: hasDataRestriction see clause 6.3.1	Data Property: hasDataType

#### 6.3.2.2	Data Property: hasDataRestriction_maxInclusive

For applicability of this sub-class of data property: hasDataRestriction see clause 6.3.1	Data Property: hasDataType

#### 6.3.2.3	Data Property: hasDataRestriction_minExclusive

For applicability of this sub-class of data property: hasDataRestriction see clause 6.3.1	Data Property: hasDataType

#### 6.3.2.4	Data Property: hasDataRestriction_maxExclusive

For applicability of this sub-class of data property: hasDataRestriction see clause 6.3.1	Data Property: hasDataType

#### 6.3.2.5	Data Property: hasDataRestriction_length

For applicability of this sub-class of data property: hasDataRestriction see clause 6.3.1	Data Property: hasDataType

#### 6.3.2.6	Data Property: hasDataRestriction_minLength

For applicability of this sub-class of data property: hasDataRestriction see clause 6.3.1	Data Property: hasDataType

#### 6.3.2.7	Data Property: hasDataRestriction_maxLength

For applicability of this sub-class see of data property: hasDataRestriction clause 6.3.1	Data Property: hasDataType

#### 6.3.2.8	Data Property: hasDataRestriction_pattern

For applicability of this sub-class of data property: hasDataRestriction see clause 6.3.1	Data Property: hasDataType

#### 6.3.2.9	Data Property: hasDataRestriction_langRange

For applicability of this sub-class of data property: hasDataRestriction see clause 6.3.1	Data Property: hasDataType

### 6.3.3	Data Property: hasValue

Description

This data property contains the value of the Variable if that value is part of the semantic description and is not contained in a different resource (identified by the oneM2MTargetURI data property). Storing the value of a Variable in a semantic description (i.e. as part of the RDF description in the semanticDescriptor resource) is useful for values that are relatively static (e.g. the name of the manufacturer).

Data properties "hasValue" and "oneM2MTargetURI" are mutually exclusive. Only one of the two shall be instantiated.

Domain Class

SimpleTypeVariable

Range Datatype

rdfs: Literal

### 6.3.4	Data Property: netTechnologyCommunicationProtocol

Description

Identifies a communication protocol (e.g. ZigBee_1_0)

Domain Class

AreaNetwork

Range Datatype

netTechnologyCommunicationProtocol rdf:PlainLiteral

### 6.3.5	Data Property: netTechnologyPhysicalStandard

Description

netTechnologyPhysicalStandardIdentification of the physical properties of a Area Network technology (e.g. IEEE_802_15_4_2003_2_4GHz).

Domain Class

AreaNetwork

Range Datatype

rdf:PlainLiteral

### 6.3.6	Data Property: netTechnologyProfile

Description

netTechnologyProfileIdentification of a profile (e.g. ZigBee_HA) of a Area Network technology

Domain Class

AreaNetwork

Range Datatype

rdf:PlainLiteral

### 6.3.7	Data Property: oneM2MTargetURI

Description

oneM2MTargetURI (range data type: rdfs: Literal)
This data property contains the URI of a oneM2M resource (<container> or <flexContainer>) through which the oneM2M instantiation of the value of the Variable can be manipulated by the communicating entity. It can contain an absolute address or an address relative to the <semanticDescriptor> resource that holds the RDF description of the Variable. 
That address could be e.g.:

The value of the parentID for the <container> or <flexContainer> of a Input- or OutputDataPoint which has child-resource of type <semanticDescriptor> that holds the RDF description of the DataPoint.

Domain Class

Operation

Variable

Range Datatype

rdfs:Literal

### 6.3.8	Data Property: oneM2MAttribute

Description

This Data Property contains the name of the attribute of the oneM2M resource of type <flexContainer> or the the child resource of the oneM2M resource of type <container>  that is referenced with the oneM2MTargetURI and  that stores the value of the SimpleTypeVariable:

if the resource-type of the  oneM2M resource that is referenced with the oneM2MTargetURI is <container> then this Data Property shall contain the text string "#latest".

Domain Class

SimpleTypeVariable

Range Datatype

rdf:PlainLiteral

### 6.3.9	Data Property: oneM2MMethod

Description

This data property contains a oneM2M CRUD Method through which the oneM2M instantiation of the value of the Variable can be manipulated by the communicating entity:

It contains the string "RETRIEVE" for retrieving the variable when the oneM2M resource is of type <container> or <flexContainer>. This applies to sub-classes: OperationOutput, OutputDatapoint, ThingProperty and OperationState.

It contains the string "CREATE" for updating the variable when the oneM2M resource is of type <container>. This applies to sub-classes: OperationInput, InputDatapoint, ThingProperty.

It contains the string "UPDATE" for updationg the variable when the oneM2M resource is of type <flexContainer>. This applies to sub-classes: OperationInput, InputDatapoint, ThingProperty.

Domain Class

Variable

Operation

Range Datatype

rdf:PlainLiteral

### 6.3.10	Data Property: hasThingAnnotation

Description

This data property contains a description of a Thing.

Note: Data Property: hasThingAnnotation should be used in cases where the annotation data do not change very often and/or where they are usually not the subject of semantic operations  (e.g. semantic discovery). 
Otherwise Object Propety: hasThingPropery should be used

- E.g. a certain type of Device could have the model (as a numeric description) or the name of the manufacturer (as a textual annotation)

Domain Class

Thing

Range Datatype

rdfs: Literal

## 6.4	Annotation Properties

### 6.4.1	Annotation Property: resourceDescriptorLink

Description

The resourceDescriptorLink annotation property is used to refer to a semanticDescriptor resource that contains more information about its subject. Its subject may be any individual and the range shall be the data literal or URI reference that represents the address of the semanticDescriptor:

For a oneM2M instantiation of the Base Ontology the resourceDescriptorLink annotation property is used to annotate instances that appear in the range of object properties. The URI points to the semanticDescriptor that contains more information about the instance of that class.

NOTE:	In OWL DL it is not allowed to define property axioms on annotation properties, i.e. it is not possible to define a domain and a range within the ontology itself.

Domain Class

owl:Thing

Range Datatype

xsd:anyURI

# 7	Instantiation of the Base Ontology and external ontologies to the oneM2M System

## 7.1	Instantiation rules for the Base Ontology

### 7.1.1	Instantiation of classes of the oneM2M Base Ontology and derived external ontologies in the oneM2M System:

#### 7.1.1.1	General on instantiating classes of the Base Ontology in the oneM2M System

Clause 7.1.1 describes how the Base Ontonlogy shall be instantiated in the oneM2M Sytem.

NOTE 1:	Apart from semantically describing oneM2M Solutions a standardized oneM2M instantiation of the Base Ontology is also needed for the purpose of Interworking with full semantic mapping when the non-oneM2M data model is described by a oneM2M compliant ontology as described in clause F.5 of oneM2M TS-0001 [2]

NOTE 2:	Other instantiations of the Base Ontology are permissible if interworking according to clause F.5 of oneM2M TS-0001 [2] is not required.

Every instantiation of a class of the Base Ontology (or a sub-class thereof) shall be instantiated in a descriptor attribute of a oneM2M resource of type <semanticDescriptor>. A <semanticDescriptor> resource may instantiate multiple classes.

That <semanticDescriptor>  resource shall:

- Contain instantiations of classes in the RDF data [3] of its descriptor attribute:

Every instance of a class shall be globally identified within the oneM2M Solution using the rdf:about attribute that contains a URI (e.g. based on a MAC address of a Device) that is unique within the oneM2M Solution.

NOTE 3:	The choice of a suitable unique URI is out of scope of oneM2M.

- Contain an Ontology-Ref attribute that identifies the class whose instantiation is described in the descriptor attribute. Depending on the instantiation this is a class in the Base Ontology or a class of another ontology that is a sub-class of (includes equals to) a class in the Base Ontology as defined in clause 5.1.2.2.

The <semanticDescriptor> shall also contain:

- The instantiated Object Properties for which the instantiated class is the domain class.

- The instantiated Data Properties for which the instantiated class is the domain class.

NOTE 4:	Instantiations of the domain class and the range class of an object property may be contained in <semanticDescriptor>different <semanticDescriptor> resources.

If the range class of an object property is instantiated in a <semanticDescriptor> resource that is different to the <semanticDescriptor> resource in which the domain class is instantiated then the <semanticDescriptor> of the instance of the domain class shall contain:

- an instance of the resourceDescriptorLink annotation property that contains the URI of the semanticDescriptor of the instance of the range class.

For specific classes that are indicated in clause 7.1.1.2 (in particular class:OperationState and classes that are derived from class:Variable) the values of Data Properties may be stored in the parent resource of that <semanticDescriptor> instead of the RDF data [3] of the descriptor attribute.

Any class of a derived external ontology shall be either:

a sub-class of a class of the Base Ontology; or

the range class of some Object Property whose domain class is a class (e.g. class:Thing) or sub-class of the Base Ontology.

If a class of a derived external ontology is a sub-class of a class of the Base Ontology then it shall be instantiated in the same way as the class of the Base Ontology.

If a class of a derived external ontology is not a sub-class of the Base Ontology but is the range class of some Object Property whose domain class is a class or sub-class of the Base Ontology then it shall be instantiated in the data of the descriptor attribute of the <semanticDescriptor> child resource of the oneM2M resource that instantiates the sub-class of the Base Ontology.

#### 7.1.1.2	Instantiation of individual classes of the Base Ontology

An overview of the oneM2M resources for instantiating the classes of the oneM2M Base Ontology is shown in the figure 21. Different colours indicate different resource types.

Figure 21: oneM2M instantiation of the Base Ontology

The Device class of the oneM2M Base Ontology (or a sub-class thereof that is not an InterworkedDevice) shall be instantiated in the data of the descriptor attribute of a resource of type  <semanticDescriptor> that is a child resource of an <AE>.

The Device instance is identified using the rdf:about attribute that contains a URI (e.g. the MAC address) that is unique within the oneM2M Solution.

The application logic (identified by its APP-ID of the Device instance is provided by the Application Entity (AE) of that Device.

NOTE 1:	The The resourceID of a <node> resource that stores the node specific information where this AE resides is contained in the nodeLink attribute of the <AE> of the Device

The InterworkedDevice class of the oneM2M Base Ontology (or a sub-class) shall be instantiated in the data of the descriptor attribute of a a resource of type <semanticDescriptor> that is a child resource of:

an <AE> resource of its Interworking Proxy Application Entity (IPE); or, alternatively

a <container> or <flexContainer> resource that is a child resource of the <AE> resource of its Interworking Proxy Application Entity (IPE).

The InterworkedDevice instance is identified using the rdf:about attribute that contains a URI (e.g. the device identifier of the device in the interworked system) that is unique within the oneM2M Solution.

The APP-ID of the <AE> that is the parent (or grand-parent) of the <semanticDescriptor> which contains an instance of an InterworkedDevice, shall be the APP-ID of InterworkedDevice's IPE.

NOTE 2:	The reason for the different instantiation of Device and InterworkedDevice is the following:

Case 1 – native oneM2M device
A oneM2M device (ASN, ADN - and potentially MN) always has an AE and an <AE> resource. Therefore the Device class for a oneM2M device needs to be instantiated in the descriptor attribute of the  <semanticDescriptor> child resource of that <AE>.

Case 2 – interworked (non-oneM2M) device
In the case of an interworked (non-oneM2M) device the IPE may

  - Instatiate the InterworkedDevice class for the interworked (non-oneM2M) devices in the descriptor attribute of the  <semanticDescriptor> child resource of the IPE’s <AE>.

  - The IPE can create multiple <AE> resources, one for each interworked device.
In this case each interworked device needs to be instantiated in the descriptor attribute of the  <semanticDescriptor> child resource of that newly created <AE>.

  - The IPE can create multiple <container> or <flexContainer> resources as child resources of the <AE> resource of its Interworking Proxy Application Entity (IPE) , one for each interworked device.
In this case each interworked (non-oneM2M) device needs to be instantiated in the descriptor attribute of the  <semanticDescriptor> child resource of that <container> or <flexContainer>.

The AreaNetwork class (or a sub-class) shall be instantiated in the data of the descriptor attribute of the <semanticDescriptor> child resource of the oneM2M resource that instantiates the InterworkedDevice class:

The Data Properties "anTechnologyCommunicationProtocol", "anTechnologyPhysicalStandard" and "anTechnologyProfile" are instantiated in the descriptor attribute of the <semanticDescriptor> child resource of the oneM2M resource that instantiates the InterworkedDevice class.

The Service class (or a sub-class) shall be instantiated in the data of the descriptor attribute of the <semanticDescriptor> child resource of a genericInterworkingService (specialization of <flexContainer>) resource.

The instance is identified using the rdf:about attribute that contains the URI of the Device concatenated with the letter "*" and the class name of the Service (e.g. 00:11:2F:74:2C:8F*MyService).

The genericInterworkingService resource shall be a child resource of the (<AE>, <container> or <flexContainer>) resource that contains the <semanticDescriptor> which instantiates the Device class. It contains references to the <container> or <flexContainer> resources that represent Input- and/or OutputDatapoints of the Service.

The Function class (or sub-class) shall be instantiated in the data of the descriptor attribute of the <semanticDescriptor> child resource of the oneM2M resource that instantiates the Device class.

The instance is identified using the rdf:about attribute that contains the URI of the Device concatenated with the letter "*" and the class name of the Function (e.g. 00:11:2F:74:2C:8F*MyFunction).

The Command class (or sub-class) shall be instantiated in the data of the descriptor attribute of the <semanticDescriptor> child resource of the oneM2M resource that instantiates the Device class.

The instance is identified using the rdf:about attribute that contains the URI of the Device concatenated with the letter "*" and the class name of the Command (e.g. 00:11:2F:74:2C:8F*MyCommand).

The Operation class (or sub-classes) shall be instantiated in the descriptor attribute of the <semanticDescriptor> child resource of a genericInterworkingOperationInstance (specialization of <flexContainer>) resource.

The instance is identified using the rdf:about attribute that contains the URI of the Device concatenated with the letter "*" and the class name of the Service, concatenated with the letter "*" and a combination of the class name of the Operation with a number that makes the instance unique within its Service instance during the operation's  lifetime 
(e.g. at a certain point in time a Service instance might have Operation instances with OperationInstances with IDs:

"00:11:2F:74:2C:8F*MyService*MyOperation1", "00:11:2F:74:2C:8F*MyService*MyOperation5",

"00:11:2F:74:2C:8F*MyService*MyOtherOperation1",

"00:11:2F:74:2C:8F*MyService*MyThirdOperation8").

The genericInterworkingOperationInstance resource shall be a child resource of the genericInterworkingService resource that contains the <semanticDescriptor> which instantiates the Service class:

The range instance of Object Property "hasOperation"that links the instance of the Service to the instance of the Operation shall be be annotated with an Annotation Property: resourceDescriptorLink <semanticDescriptor>which shall contain a reference to the resource of type <semanticDescriptor> that instantiates the Operation.

The OperationInput and OperationOutput class (or sub-class) shall be instantiated in the data of the descriptor attribute of the <semanticDescriptor> child resource of a <container> or <flexContainer>.

The instance is identified using the rdf:about attribute that contains the URI of the OperationInstance concatenated with the letter "*" and the class name of the OperationInput and OperationOutput
(e.g. 00:11:2F:74:2C:8F*MyService*MyThirdOperation8*MyOperationOutput).

The <container> or <flexContainer>, whose the <semanticDescriptor> child resource contains the instance of the OperationInput or OperationOutput shall be a child resource of the genericInterworkingOperationInstance resource:

The range of an instantiation of Object Properties "hasInput" and  "hasOutput" that link the instance of the Operation to the instance of the OperationInput and OperationOutput shall be be annotated with an Annotation Property: resourceDescriptorLink which shall contain a reference to the resource of type <semanticDescriptor> that instantiates the OperationInput and OperationOutput.

The InputDataPoint and OutputDataPoint class (or sub-class) shall be instantiated in the data of the descriptor attribute of the <semanticDescriptor> child resource of a <container> or <flexContainer>.

The instance is identified using the rdf:about attribute that contains the URI of the Device concatenated with the letter "*" and the class name of the InputDataPoint or OutputDataPoint (e.g. 00:11:2F:74:2C:8F*MyInputDataPoint)

The <container> or <flexContainer> resource shall be a child resource of the (<AE>, <container> or <flexContainer>) resource that contains the <semanticDescriptor> which instantiates the Device class or InterworkedDevice class.

The range of an instantiation of Object Properties "hasInputDataPoint" and  "hasOutputDataPoint" that link the instance of a Service or Operation to the instance of the InputDataPoint and OutputDataPoint shall be be annotated with an Annotation Property: resourceDescriptorLink which shall contain a reference to the resource of type <semanticDescriptor> that instantiates the InputDataPoint and OutputDataPoint.

<semanticDescriptor>The OperationState class (or sub-class) shall be instantiated in the data of the descriptor attribute of the <semanticDescriptor> child resource of the genericInterworkingOperationInstance resource that is related via the "hasOperationState" Object Property.

The instance is identified using the rdf:about attribute that contains the URI of the OperationInstance concatenated with the letter "*" and "OperationState" (i.e. the class name of the OperationState). 
(e.g. 00:11:2F:74:2C:8F*MyService*MyThirdOperation8*OperationState)

The data property "oneM2MTargetURI" shall contain the URI of the genericInterworkingOperationInstance resource

The data property "oneM2MAttribute" shall ontain the value "OperationState" (i.e. the name of the Attribute of the OperationState in the genericInterworkingOperationInstance resource)

<semanticDescriptor>The Aspect class (or sub-classes) may be instantiated in the data of the descriptor attribute of the <semanticDescriptor> child resource of any resource type that allows a <semanticDescriptor> child resource.

The instance is identified using the using the rdf:about attribute that contains a URI that is unique within the oneM2M Solution.

NOTE 3:	The choice of a suitable unique URI is out of scope of oneM2M.

The Thing class (or sub-classes) may be instantiated in the data of the descriptor attribute of the <semanticDescriptor> child resource of any resource type that allows a <semanticDescriptor> child resource:

The instance is identified using the using the rdf:about attribute that contains a URI that is unique within the oneM2M Solution.

NOTE 4:	The choice of a suitable unique URI is out of scope of oneM2M.

The range of an instantiation of Object Property "hasThingRelation" that links the instance of the Thing to the instance of a second Thing shall be be annotated with an Annotation Property: resourceDescriptorLink which shall contain a reference to the resource of type <semanticDescriptor> that instantiates the second Thing

NOTE 5:	This reference could refer to Thing, or a Device (as a sub-class of Thing).

The range of an instantiation of an Object Property "hasThingProperty" that links the instance of the Thing to an instance of a ThingProperty shall be be annotated with an Annotation Property: resourceDescriptorLink which shall contain a reference to the resource of type <semanticDescriptor> that instantiates the ThingProperty<semanticDescriptor>.

The TingProperty class (or sub-class) shall be instantiated in the data of the descriptor attribute of the <semanticDescriptor> of the Thing  or of a separate <container> or <flexContainer>.

The instance is identified using the rdf:about attribute that contains the URI of the Thing concatenated with the letter "*" and the class name of the ThingProperty (e.g. [some out of scope Thing URI]*MyThingProperty):

If the TingProperty is a SimpleTypeVariable and contains in its data property "hasValue" the value of the ThingProperty then it can be instantiated in the data of the descriptor attribute of the <semanticDescriptor> of the Thing.

Otherwise the TingProperty shall be instantiated in the data of the descriptor attribute of a separate <container> or <flexContainer> which shall be a child resource of the parent resource of <semanticDescriptor> which instantiates the Thing class. 
In this case the The range of an Object Property "hasThingProperty" that links an instance of a Thing to the instance of the ThingProperty shall be be annotated with an Annotation Property: resourceDescriptorLink which shall contain a reference to the resource of type <semanticDescriptor> that instantiates the ThingProperty.

The sub-classes of the Variable class shall be instantiated in the data of the descriptor attribute of the <semanticDescriptor> child resource of resources of a <container> or  <flexContainer>. 
If the Variable has a structure (if it is composed of (sub-)Variables) - i.e. it is not a SimpleTypeVariable then:

The <semanticDescriptor> shall have instances of object property "hasSubStructure" and each instance contains in its range an instance of a (sub-) Variable. If that (sub-)Variable is part of a different <semanticDescriptor> resource then the The range of an object property "hasSubStructure" shall be annotated with an annotation property: "resourceDescriptorLink" which shall contain a reference to the resource of type <semanticDescriptor> that instantiates the (sub-)Variable.

The<semanticDescriptor> shall contain an instance of data property "oneM2MTargetURI" which shall contain the URI of the parent <container> or  <flexContainer> resource.

The<semanticDescriptor> shall have instances of the data property "oneM2MMethod" which indicates a oneM2M CRUD Method through which the oneM2M instance of the value of the Variable can be manipulated by the communicating entity:

It contains the string "RETRIEVE" for retrieving the variable when the oneM2M resource is of type <container> or <flexContainer>. This applies to sub-classes: OperationOutput, OutputDatapoint, ThingProperty and OperationState.

It contains the string "CREATE" for updating the variable when the oneM2M resource is of type <container>. This applies to sub-classes: OperationInput, InputDatapoint, ThingProperty.

It contains the string "UPDATE" for updating the variable when the oneM2M resource is of type <flexContainer>. This applies to sub-classes: OperationInput, InputDatapoint, ThingProperty.

The sub-classes of the SimpleTypeVariable class shall be instantiated in the data of the descriptor attribute of the <semanticDescriptor> child resource of resources of a <container> or  <flexContainer>. 
The data properties are the same as for instances of the Variable class. 
In addition:

The data property "hasValue" contains the value of the Variable if that value is part of the semantic description and is not contained in a different resource (identified by the oneM2MTargetURI data property).

NOTE 6:	Storing the value of a Variable in a semantic description (i.e. as part of the RDF description in the <semanticDescriptor> resource) is useful for values that are relatively static (e.g. the name of the manufacturer).

NOTE 7:	Data properties "hasValue" and "oneM2MTargetURI" are mutually exclusive. Only one of the two shall be instantiated for a SimpleTypeVariable.

If data property "oneM2MTargetURI" is instantiated then the data property "oneM2MAttribute" shall also be instantiated and contain:

In the case of a <flexContainer> the name of the Attribute of the SimpleTypeVariable in the <flexContainer> resource).

In the case of a <container> the value "#latest".

The MetaData class (or sub-classes) may be instantiated in the data of the descriptor attribute of the <semanticDescriptor> child resource of any resource type that allows a <semanticDescriptor> child resource.

The instance is identified using the using the rdf:about attribute that contains a URI that is unique within the oneM2M Solution.

### 7.1.2	Instantiation of Object Properties

Object properties relate an instance of domain class to an instance of the range class.
They shall be instantiated in the data of the descriptor attribute of the <semanticDescriptor> resource that instantiates the domain class of the object property.

If the range class of an Object Property is instantiated in a different resource than the instantiation of the domain class then the The range class of an Object Property shall be annotated with the Annotation Property: resourceDescriptorLink which shall contain a reference to that resource.

### 7.1.3	Instantiation of Data Properties

Data properties shall be instantiated in the data of the descriptor attribute of the <semanticDescriptor> resource that instantiates the domain class of the data property.

### 7.1.4	Instantiation of Annotation Properties

Annotation properties may be instantiated in the data of the descriptor attribute of any <semanticDescriptor> resource.

For a oneM2M instantiation of the Base Ontology the resourceDescriptorLink annotation property is used to annotate the range of object properties with the URI of the <semanticDescriptor> that contains the instance  (that holds the RDF description of the instance) of the range class of the object property.

The resourceDescriptorLink annotation property is not part of the semantic description but is used to refer to a <semanticDescriptor> resource that contains more information about its subject. It is resolved by combining the content of the descriptor attributes of the <semanticDescriptor> resources before a semantic operation (e.g. SPARQL query) is performed.

## 7.2	Common mapping principles between the Base Ontology and external ontologies

The base ontology can be mapped to other external ontologies (e.g., SAREF, SSN, etc.).  The following priciples are applied for the mapping between ontologies:

Principle 1 - Classes Mapping (owl:equivalentClass):

Making the statement X owl:equivalentClass Y essentially means that two named classes are synonymous, i.e. that all instances of class X are instances of class Y and vice versa.

Using this principle, two classes specified in different ontologies are declared to be equivalent.

Principle 2 - Properties Mapping (owl:equivalentProperty):

The owl:equivalentProperty construct can be used to state that two properties have the same property extension. Syntactically, owl:equivalentProperty is a built-in OWL property with rdf:Property as both its domain and range.

Using this principle, if two properties are declared to be equivalent, two properties have the same semantics or meaning.

Principle 3 - Class Instances Mapping (owl:sameAs):

The property owl:sameAs is used to state that two individuals (i.e. class instances) are the same.

Using this principle, two class instances specified in different ontologies are declared to be equivalent.

Principle 4 - SubClass Mapping (rdfs:subClassOf):

The property rdfs:subClassOf is used to state that the class extension of a class description is a subset of the class extension of another class description.

Making the statement X rdfs:subClassOf Y essentially means that all instances of class X are instances of class Y.

Principle 5 – SubProperties Mapping (rdfs:subPropertyOf):

  - The property rdfs:subPropertyOf  is used to state that one property is the subproperty of another property. Syntactically, the domain and range of rdfs:subPropertyOf are both of type rdf:Property.

  - Making the statement X rdfs:subPropertyOf Y essentially means that all resources related by property X are also related by property Y.

When using the above principles for the mapping between the Base Ontology and an external ontology, the hierarchy and relations of the classes and properties defined in these two ontologies should be carefully considered since there may exist some incompatible points after mapping due to the inheritance of the properties.

# 8	void

# 9	void

Annex A (normative):
OWL representation of Base Ontology

The OWL representation of Base Ontology is provided in a separate document.

The Base Ontology is available at the web page:

http://www.onem2m.org/ontology/Base_Ontology

which contains the latest version of the (RDF/XML) OWL representation of the ontology under the URL:

http://www.onem2m.org/ontology/Base_Ontology/oneM2M_base_ontology.owl

and individual versions of the (RDF/XML) OWL representation of the ontology under the URL:

http://www.onem2m.org/ontology/Base_Ontology/oneM2M_base_ontology-vx_y_z.owl 
(where x,y,z signify the version numbering for major- minor- and editorial changes of the base ontology).

e.g. http://www.onem2m.org/ontology/Base_Ontology/oneM2M_base_ontology-v0_5_0.owl.

Annex B (informative):
Mappings of selected external ontologies to the Base Ontology

# B.1	Mapping of SAREF

## B.1.1	Introduction to SAREF

The following description of the SAREF ontology is copied from the following site: https://sites.google.com/site/smartappliancesproject/ontologies/reference-ontology.

It provides an introduction to the scope and objectives of SAREF ontology:

"The Smart Appliances REFerence (SAREF) ontology is a shared model of consensus that facilitates the matching of existing assets (standards/protocols/datamodels/etc.) in the smart appliances domain. The SAREF ontology provides building blocks that allow separation and recombination of different parts of the ontology depending on specific needs.

The starting point of SAREF is the concept of Device (e.g., a switch). Devices are tangible objects designed to accomplish one or more functions in households, common public buildings or offices. The SAREF ontology offers a lists of basic functions that can be eventually combined in order to have more complex functions in a single device. For example, a switch offers an actuating function of type "switching on/off". Each function has some associated commands, which can also be picked up as building blocks from a list. For example, the "switching on/off" is associated with the commands "switch on", "switch off" and "toggle". Depending on the function(s) it accomplishes, a device can be found in some corresponding states that are also listed as building blocks.

A Device offers a Service, which  is a representation of a Function to a network that makes the function discoverable, registerable and remotely controllable by other devices in the network. A Service can represent one or more functions. A Service is offered by a device that wants (a certain set of) its function(s) to be discoverable, registerable, remotely controllable by other devices in the network. A Service must specify the device that is offering the service, the function(s) to be represented, and the (input and output) parameters necessary to operate the service.

A Device in the SAREF ontology is also characterized by an (Energy/Power) Profile that can be used to optimize the energy efficiency in a home or office that are part of a building."

A formal description of the ontology is provided via this link: http://ontology.tno.nl/saref/

The following table B.1 provides a list of SAREF concepts - source http://www.etsi.org/images/files/Events/2015/201502_SMARTAPP/D-S4 - SMART 2013-0077 - Smart Appliances - Final Study Report_v1.0.pdf.

Table B.1: List of SAREF concepts

| Concept | Definition |
| --- | --- |
| Building Object | A Building Object is an object in the building that can be controlled by devices, such as a door or a window that can be automatically opened or closed by an actuator |
| Building Space | According to FEIMSER, a Building Space in SAREF defines the physical spaces of the building. A building space contains devices or building objects. |
| Command | A Command is a directive that a device should support to perform a certain function. A command may act upon a state, but does not necessarily act upon a state. For example, the ON command acts upon the ON/OFF state, but the GET command does not act upon any state, since it gives a directive to retrieve a certain value with no consequences on states. |
| Commodity | A Commodity is a marketable item for which there is demand, but which is supplied without qualitative differentiation across a market. SAREF refers to energy commodities such as electricity, gas, coal and oil. |
| Device | A Device in the context of the Smart Appliances study is a tangible object designed to accomplish a particular task in households, common public buildings or offices. In order to accomplish this task, the device performs one or more functions. For example, a washing machine is designed to wash (task) and to accomplish this task it performs the start and stop function. |
| Device Category | A Device Category provides a way to classify devices according to a certain point of view, for example, the point of view of the user of the device vs. the device's manufacturer, or the domain in which the device is used (e.g., smart appliances vs. building domain vs. smart grid domain), etc. |
| Function | A Function represents the particular use for which a Device is designed. A device can be designed to perform more than one function. |
| Function Category | A Function Category provides a way to classify functions according to a certain point of view, for example, considering the specific application area for which a function can be used (e.g., light, temperature, motion, heat, power, etc.), or the capability that a function can support (e.g., receive, reply, notify, etc.), and so forth. |
| Profile | A Profile caracterizes a device for the purpose to optimize the energy efficiency in the home or office in which the device is located. The saref:Profile class allows to describe the energy (or power) production and consumption of a certain device using the saref: hasProduction and saref:hasConsumption properties. This production and consumption can be calculated over a time span (the saref:hasTime property) and, eventually, associated to some costs (the saref:hasPrice property). |
| Property | A Property is anything that can be sensed, measured or controlled in households, common public buildings or offices. |
| Service | A Service is a representation of a function to a network that makes the function discoverable, registerable, remotely controllable by other devices in the network. A service can represent one or more functions. A Service is offered by a device that wants (a certain set of) its function(s) to be discoverable, registerable, remotely controllable by other devices in the network. A Service should specify the device that is offering the service, the function(s) to be represented, and the (input and output) parameters 144 necessary to operate the service. |
| State | A State represents the state in which a device can be found, e.g. ON/OFF/STANDBY, or ONLINE/OFFLINE, etc. |
| Task | A Task represents the goal for which a device is designed (from a user perspective). For example, a washing machine is designed for the task of cleaning |
| Unit of Measure | The Unit of Measure is a standard for measurement of a quantity, such as a Property. For example, Power is a property and Watt is a unit of power that represents a definite predetermined power: when 10 Watt is mentioned, it actually means 10 times the definite predetermined power called "watt". Our definition is based on the definition of unit of measure in the Ontology of units of Measure (OM). A list of some units of measure that are relevant for the purpose of the Smart Appliances ontology is proposed here, but this list can be extended. |

### B.1.2	Class mapping relationship between SAREF and the Base Ontology

This section provides an example on how the principle specified in Section 7.2 can be applied to the mapping between the Base Ontology and SAREF.

Using two differnet ontologies (i.e., Base_Ontology.owl and SAREF.owl), a new ontology can be created using the mapping principles,  i.e. adding mapping relationships into the union of these two ontologies as a new mapped ontology.

As a simple case, the following illustrates the subClass mappig between oneM2M:Device and saref:Device. The following shows that saref:Device is a subclass of oneM2M:Device.

There are two popular OWL syntax: OWL/XML or RFD/XML.

OWL/XML syntax for sub-class mapping is:

<Import>http://www.onem2m.org/ontology/Base_Ontology</Import>

<Import>http://ontology.tno.nl/saref</Import>

<SubClassOf>

<Class IRI="http://ontology.tno.nl/saref#Device"/>

<Class IRI="http://www.onem2m.org/ontology/Base_Ontology#Device"/>

</SubClassOf>

RFD/XML syntax for subclass mapping is:

<rdf:Description rdf:about="http://ontology.tno.nl/saref#Device">

<rdfs:subClassOf rdf:resource="http://www.onem2m.org/ontology/Base_Ontology#Device"/>

</rdf:Description>

Then, all instances of saref:Device are instances of oneM2M:Device.

Figure B.1 shows the class hierarchy of the new mapped ontology with mapping saref:Device as subClass of  oneM2M:Device.

Figure B.1: Class hierarchy of a mapped ontologywith mapping saref:Device as subClass of oneM2M:Device

Considering both the definition and the relations of the classes in SAREF and the base ontology, Table B.2 gives the possible subclass mapping between SAREF and the base ontology.

Table B.2: Sub-class mapping between SAREF and the base ontology

| Class in SAREF | Mapping relationship | Class in Base Ontology |
| --- | --- | --- |
| saref:BulidingObject | rdfs:subClassOf | oneM2M:Thing |
| saref: BulidingSpace | rdfs:subClassOf | oneM2M:Thing |
| saref :Commodity | rdfs:subClassOf | oneM2M:Thing |
| saref :Property | rdfs:subClassOf | oneM2M:InputDataPoint OR oneM2M:OutputDataPoint |
| saref :UnitOfMeasure | rdfs:subClassOf | oneM2M:MetaData |
| saref :MeteringFunction | rdfs:subClassOf | oneM2M:MesuringFunction |
| saref :State | rdfs:subClassOf | oneM2M:InputDataPoint OR oneM2M:OutputDataPoint |
| saref:Profile, | rdfs:subClassOf | oneM2M:Thing |
| saref:Task | rdfs:subClassOf | oneM2M:ThingProperty |
| saref:DeviceCategory | rdfs:subClassOf | oneM2M:ThingProperty |
| saref:FunctionCategory | rdfs:subClassOf | oneM2M:Aspect |

Note: Mapping SAREF classes into sub-classes of the Base Ontology is only relevant for the oneM2M System as this sub-class relationship enables a standardized instantiation of these SAREF classes as oneM2M resources.

For the purpose of semantic discovery also sub-classing some Base Ontology classes to SAREF classes can be done.
Thus oneM2M Devices, Functions, Services … for which instances (oneM2M resources) exist in a oneM2M Solution can be discovered as SAREF instances if these resources are annotated with their semantic description in the appropriate <semanticDescriptor> child resources and if the semantic description is made available to SAREF.

Mutual sub-class relationships (rdfs:subClassOf) result in equivalent classes (owl:equivalentClass), i.e. any instance of one class is also an instance of the equivalent class.

Table B.3: Equivalence-class mapping between SAREF and the base ontology

| Class in SAREF | Mapping relationship | Class in Base Ontology |
| --- | --- | --- |
| saref:Device | owl:equivalentClass | oneM2M:Device |
| saref:Command | owl:equivalentClass | oneM2M:Command |
| saref:Function | owl:equivalentClass | oneM2M:Function |
| saref:Service | owl:equivalentClass | oneM2M:Service |
| saref:ActuatingFunction | owl:equivalentClass | oneM2M:ControllingFunction |
| saref:SensingFunction | owl:equivalentClass | oneM2M:MesuringFunction |

Object Properties of the base ontology, for which domain- and range classes have equivalent classes in the SAREF ontology, can have equivalent Object Properties in SAREF.

Table B.4: Equivalent Property mapping between SAREF and the base ontology

| Object Propery in SAREF | Mapping relationship | Object Propery in Base Ontology |
| --- | --- | --- |
| saref:offers | owl:equivalentProperty | oneM2M:hasService |
| saref:consistsOf | owl:equivalentProperty | oneM2M:consistsOf |
| saref:represents | owl:equivalentProperty | oneM2M:exposesFunction |
| saref:hasCommand | owl:equivalentProperty | oneM2M:hasCommand |

## B.1.3	Mapping SAREF to oneM2M resource structure

### B.1.3.1	Introduction

Mapping an ontology to oneM2M describes how an instance of that ontology may be represented under oneM2M resource structure. This clause proposes a recommended way to map SAREF to oneM2M.

### B.1.3.2	Mapping rules

Mapping ontologies to the oneM2M resource structure may be provided through a list of mapping rules. The oneM2M Base Ontology instantiation rules in clause 7.1 apply. Currently no additional mapping rules apply.

### B.1.3.3	Example showing the uses of the semanticDescriptor resource and instantiation in the oneM2M resource structure

This clause gives an example of how oneM2M resources and their semantic annotations based on the Smart Appliance REFerence Ontology (SAREF) [i.2] can be used to describe a device representing a smart appliance.
It assumes that the Instantiation rules in clause 7 and the sub-class mapping relationship between SAREF and the Base Ontology (clause B.1.2) apply.

The example taken from SAREF is a (simplified) washing machine:

- The washing machine has been manufactured by manufacturer XYZ.

- XYZ describes this type of washing machine as "Very cool Washing Machine"

- The model of the type of washing machine is XYZ_Cool

- The washing machine has an actuating function: WashingFunction which has three commands:

  - ON_Command

  - OFF_Command

  - Toggle_Command

- The related service of the washing machine that represents that actuating function is of class: SwitchOnService from SAREF. It has

  - an InputDataPoint: BinaryInput (to expose command ON_Command and OFF_Command) and

  - an Operation: ToggleBinary (to expose command Toggle_Command)

- The washing machine has also a function: MonitoringFunction that informs the user about the current state of the washing machine.

- The state of the washing machine: WashingMachineStatus can take the values "WASHING" or "STOPPED" or "ERROR".

- This state WashingMachineStatus is updated as an the OutputDataPoint of a service MonitorService of the washing machine that monitors the washing machine’s behaviour.

- The washing machine is located at My_Bathroom

NOTE: "InputDataPoint", "OutputDataPoint" and "Operation" are not specified in SAREF, they are classes of the oneM2M Base Ontology.

This example will identify the specific washing machine by the URI: "WASH_XYZ_123". WASH_XYZ_123 that is an instance of class: XYZ_Cool which is contained in XYZ's ontology: http://www.XYZ.com/WashingMachines. 
The ontology "http://www.XYZ.com/WashingMachines” that contains the model type "XYZ_Cool" is compliant to SAREF, which is turn is compliant to the oneM2M Base Ontology (see clause B.1.2).

The following figure B.2 shows some sub-classing relationships between XYZ_Cool, SAREF and the oneM2M Base Ontology.

Figure B.2: Sub-classing relationships between XYZ_Cool, SAREF and the oneM2M Base Ontology

According to clause 7.1.1.2 the washing machine – as a sub-class of the oneM2M:Device class – shall be instantiated in the data of the descriptor attribute of a resource of type  <semanticDescriptor> that is a child resource of an <AE>.

For example:

- that <AE> resource could have resourceID = "00000001” and have a resourceName "My-WashingMachine”

- It’s CSE-relative address would be:
Non-Hierarchical:

  - "00000001”

Hierarchical:

  - "./My-WashingMachine”

Figure B.3: Resource structure of smart washing machine <AE> and its child-resources

Figure B.3 shows the resource structure of a <AE> resource representing the smart washing machine. It consists of an ontologyRef attribute, which contains the URI of the ontology concept of the smart washing machine, e.g. "http://www.XYZ.com/WashingMachines#XYZ_Cool" (not shown in the figure: the ontologyRef attributes in the semanticDescriptors of the child resources, e.g. http://www.XYZ.com/WashingMachines#WashingMachineStatus). The ontology http://www.XYZ.com/WashingMachines needs to include - and creates sub-classing relationships with – SAREF and the oneM2M Base Ontology.

The <AE> resource representing the smart washing machine contains as child-resources:

- a <semanticDescriptor> resource that contains the rdf description of the washing machine WASH_XYZ_123 in its descriptor attribute

- two <genericInterworkingService> child-resources and their <semanticDescriptor>s are used for modelling the services SwitchOnService and MonitorService

- the SwitchOnService in turn has a child resource of type <genericInterworkingOperationInstance> which is created whenever a ToggleBinary Operation is invoked.

- two <container> child-resources and their <semanticDescriptor>s are used for holding the values for InputDataPoint BinaryInput and OutputDataPoint WashingMachineStatus for their respective services.

The RDF in the following tables shows the semantic annotation stored in the semanticDescriptor resources related to the washing machine.

Table B.5: RDF annotation contained in the descriptor attribute of the <semanticDescriptor> resource of a SAREF washing machine <AE> resource

<rdf:RDF <rdf:Description rdf:about="WASH_XYZ_123"> <rdf:type rdf:resource="http://www.XYZ.com/WashingMachines#XYZ_Cool"/> <saref:hasManufacturer>XYZ</saref:hasManufacturer> <saref:hasDescription>Very cool Washing Machine</saref:hasDescription> <saref:hasState rdf:resource="WASH_XYZ_123*WashingMachineStatus"/> <saref:hasFunction rdf:resource="WASH_XYZ_123*WashingFunction"/> <saref:hasService rdf:resource="WASH_XYZ_123*SwitchOnService"/> <saref:hasService rdf:resource="WASH_XYZ_123*MonitorService"/> <saref:isLocatedIn rdf:resource="My_Bathroom"/> </rdf:Description> <rdf:Description rdf:about="WASH_XYZ_123*WashingMachineStatus"> <oneM2M:resourceDescriptorLink rdf:resource= "./My-WashingMachine/WashingMachineStatus/semanticDescriptor"/> </rdf:Description> <rdf:Description rdf:about="WASH_XYZ_123*WashingFunction"> <rdf:type rdf:resource="https://w3id.org/saref#ActuatingFunction"/> <saref:hasCommand rdf:resource="WASH_XYZ_123*WashingFunction*ON_Command"/> <saref:hasCommand rdf:resource="WASH_XYZ_123*WashingFunction*OFF_Command"/> <saref:hasCommand rdf:resource="WASH_XYZ_123*WashingFunction*Toggle_Command"/> </rdf:Description> <rdf:Description rdf:about="WASH_XYZ_123*WashingFunction*ON_Command"> <rdf:type rdf:resource="https://w3id.org/saref#OnCommand"/> </rdf:Description> <rdf:Description rdf:about="WASH_XYZ_123*WashingFunction*OFF_Command"> <rdf:type rdf:resource="https://w3id.org/saref#OffCommand"/> </rdf:Description> <rdf:Description rdf:about="WASH_XYZ_123*WashingFunction*Toggle_Command"> <rdf:type rdf:resource="https://w3id.org/saref#ToggleCommand"/> </rdf:Description> <rdf:Description rdf:about="WASH_XYZ_123*SwitchOnService"> <oneM2M: resourceDescriptorLink rdf:resource= "./My-WashingMachine/SwitchOnService/semanticDescriptor"/> </rdf:Description> <rdf:Description rdf:about="WASH_XYZ_123*MonitorService"> <oneM2M: resourceDescriptorLink rdf:resource= "./My-WashingMachine/MonitorService/semanticDescriptor"/> </rdf:Description> <rdf:Description rdf:about="My_Bathroom"> <rdf:type rdf:resource="https://w3id.org/saref#BuildingSpace"/> <oneM2M: resourceDescriptorLink rdf:resource= "//m2m.service.com/SomeIN-CSE/ResourceName_of_My_Bathroom/semanticDescriptor "/> </rdf:Description> </rdf:RDF>

Table B.6: RDF annotation contained in the descriptor attribute of the <semanticDescriptor> resource of the WashingMachineStatus <container> resource

<rdf:RDF <rdf:Description rdf:about="WASH_XYZ_123*WashingMachineStatus"> <rdf:type rdf:resource="https://w3id.org/saref#State"/> <oneM2M:oneM2MTargetURI rdf:resource= "./My-WashingMachine/WashingMachineStatus"/> <oneM2M:oneM2MAttribute>#latest</oneM2M:oneM2MAttribute> <oneM2M:hasDataType>xsd:string</oneM2M:hasDataType> <oneM2M:oneM2MMethod>RETRIEVE</oneM2M:oneM2MMethod> </rdf:Description> </rdf:RDF>

Table B.7: RDF annotation contained in the descriptor attribute of the <semanticDescriptor> resource of the BinaryInput <container> resource

<rdf:RDF <rdf:Description rdf:about="WASH_XYZ_123*SwitchOnService*BinaryInput"> <rdf:type rdf:resource="https://w3id.org/saref#Property"/> <oneM2M:exposesCommand rdf:resource="WASH_XYZ_123*WashingFunction*ON_Command"/> <oneM2M:exposesCommand rdf:resource="WASH_XYZ_123*WashingFunction*OFF_Command"/> <oneM2M:oneM2MTargetURI rdf:resource= "./My-WashingMachine/BinaryInput"/> <oneM2M:oneM2MAttribute>#latest</oneM2M:oneM2MAttribute> <oneM2M:hasDataType> xsd:hexBinary</oneM2M:hasDataType> <oneM2M:oneM2MMethod> CREATE</oneM2M:oneM2MMethod> </rdf:Description> <rdf:Description rdf:about="WASH_XYZ_123*WashingFunction*ON_Command"> <oneM2M:resourceDescriptorLink rdf:resource= "./My-WashingMachine/semanticDescriptor"/> </rdf:Description> <rdf:Description rdf:about="WASH_XYZ_123*WashingFunction*OFF_Command"> <oneM2M:resourceDescriptorLink rdf:resource= "./My-WashingMachine/semanticDescriptor"/> </rdf:Description> </rdf:RDF>

Table B.8: RDF annotation contained in the descriptor attribute of the <semanticDescriptor> resource of the MonitorService <genericInterworkingService> resource

<rdf:RDF <rdf:Description rdf:about="WASH_XYZ_123*MonitorService"> <rdf:type rdf:resource="https://w3id.org/saref#Service"/> <saref:represents rdf:resource="WASH_XYZ_123*MonitoringFunction"/> <oneM2M:hasOutputDataPoint rdf:resource="WASH_XYZ_123*WashingMachineStatus"/> </rdf:Description> <rdf:Description rdf:about="WASH_XYZ_123*WashingMachineStatus"> <oneM2M:resourceDescriptorLink rdf:resource= "./My-WashingMachine/WashingMachineStatus/semanticDescriptor"/> </rdf:Description> <rdf:Description rdf:about="WASH_XYZ_123*MonitoringFunction"> <oneM2M:resourceDescriptorLink rdf:resource= "./My-WashingMachine/semanticDescriptor"/> </rdf:Description> </rdf:RDF>

Table B.9: RDF annotation contained in the descriptor attribute of the <semanticDescriptor> resource of the SwitchOnService <genericInterworkingService> resource

<rdf:RDF <rdf:Description rdf:about="WASH_XYZ_123*SwitchOnService"> <rdf:type rdf:resource="https://w3id.org/saref#SwitchOnService"/> <saref:represents rdf:resource="WASH_XYZ_123*WashingFunction"/> <oneM2M:hasOutDataPoint rdf:resource="WASH_XYZ_123*SwitchOnService*BinaryInput"/> <oneM2M:hasOperation rdf:resource="WASH_XYZ_123*SwitchOnService*ToggleBinary"/> </rdf:Description> <rdf:Description rdf:about="WASH_XYZ_123*SwitchOnService*BinaryInput"> <oneM2M:resourceDescriptorLink rdf:resource= "./My-WashingMachine/BinaryInput/semanticDescriptor"/> </rdf:Description> <rdf:Description rdf:about="WASH_XYZ_123*SwitchOnService*ToggleBinary"> <oneM2M:resourceDescriptorLink rdf:resource= "./My-WashingMachine/SwitchOnService/ToggleBinary/semanticDescriptor"/> </rdf:Description> <rdf:Description rdf:about="WASH_XYZ_123*WashingFunction"> <oneM2M:resourceDescriptorLink rdf:resource= "./My-WashingMachine/semanticDescriptor"/> </rdf:Description> </rdf:RDF>

Table B.10: RDF annotation contained in the descriptor attribute of the <semanticDescriptor> resource of the ToggleBinary <genericInterworkingOperationInstance> resource

<rdf:RDF <rdf:Description rdf:about="WASH_XYZ_123*SwitchOnService*ToggleBinary"> <rdf:type rdf:resource="http://www.onem2m.org/ontology/Base_Ontology/base_ontology#Operation"/> <oneM2M:exposesCommand rdf:resource="WASH_XYZ_123*WashingFunction*Toggle_Command"/> </rdf:Description> <rdf:Description rdf:about="WASH_XYZ_123*WashingFunction*Toggle_Command"> <oneM2M:resourceDescriptorLink rdf:resource= "./My-WashingMachine/semanticDescriptor"/> </rdf:Description> </rdf:RDF>

# History

| Publication history |  |  |
| --- | --- | --- |
| V2.0.0 | 30-Aug-2016 | Release 2 publication |
| V3.0.0 | 28-Oct-2016 | Including CRs of TP#25: MAS-2016-0217 (mirror of MAS-2016-0216), MAS-2016-0244R01 (mirror of MAS-2016-0230R02) MAS-2016-0218 |