|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2017-2020 |  | SG20-C.n |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | STUDY GROUP 20 |  |  |
|  |  |  | Original: English |  |  |
| Question(s): |  | Q3/20 |  | Geneva, 4-15 September 2017 |  |
| CONTRIBUTION |  |  |  |  |  |
| Source: |  | Nokia |  |  |  |
| Title: |  | ITU-T Y.oneM2M.DG.DM "oneM2M-TR 0035 Developer guide of device management " |  |  |  |
| Purpose: |  | Proposal |  |  |  |
| Contact: |  | Shane HE Nokia |  |  | Tel: Email: shane.he@nokia.com |
| Contact: |  | Jiaxin Yin Huawei Technologies Co., Ltd. China |  |  | Tel: Fax: Email: yinjiaxin@huawei.com |

| Keywords: | Device management, developer guide, oneM2M |
| --- | --- |
| Abstract: | This document provides example of device management in oneM2M |

Recommendation ITU-T <No.>

< oneM2M-TR 0035 Developer guide of device management >

Summary

The document provides developer guide for device management using oneM2M.

–	Device management using oneM2M protocol

–	Device management using oneM2M and other management technologies.

Keywords

Device management, developer guide, oneM2M

Introduction

<Optional – This clause should appear only if it contains information different from that in Scope and Summary>

1	Scope

The present document gives example of message flow and procedures on how device management will be performed using oneM2M defined primitives and procedures.

2	References

-

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

Not applicable.

3	Definitions

None

3.1	Terms defined elsewhere

None

3.2	Terms defined in this Recommendation

None

4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

ARIB	Association of Radio Industries and Businesses

ATIS	Alliance for Telecommunications Industry Solutions

CCSA	China Communications Standards Association

ETSI	European Telecommunications Standards Institute

TIA	Telecommunications Industry Association,

TSDSI	Telecommunications Standards Development Society

TTA	Telecommunications Technology Association

TTC	Telecommunication Technology Committee

5	Conventions

The key words “Shall”, ”Shall not”, “May”, ”Need not”, “Should”, ”Should not” in this document are to be interpreted as described in the oneM2M Drafting Rules [b-oneM2M DR]

6	Device management over the service layer

6.1	Introduction

oneM2M uses <mgmtObj> resource for device management. For device management over the service layer, the <mgmtObj> that is used for the management of the managed entity is located at different places in case of the characteristic of the managed entity. In either of the case, there exists one driver or client on the managed entity that observes the update of the <mgmtObj> resource to perform management operation.

For ASN, MN and IN which has CSE, the <mgmtObj> is hosted on the corresponding ASN-CSE, MN-CSE and IN-CSE as child resource of the <node> resource that represents the ASN, MN and IN. The managed entity observes the <mgmtObj> directly to perform the management operation.

For ADN, the <mgmtObj> resource is hosted on the registrar CSE of the ADN-AE as child resource of the <node> resource that represents the ADN. The ADN-AE in this case should subscribe the <mgmtObj> resources under the corresponding <node> resource to receive notifications on any update of the <mgmtObj> resource. The ADN which is the managed entity in this case further perform the management operation based on the notifications received.

For NoDN, the <mgmtObj> resource is hosted on the CSE which is NoDN is connected to. As NoDN is not an oneM2M defined entity, the connection between NoDN and the CSE is out of scope of oneM2M and is implementation specific. In this case, the <mgmtObj> resource on the CSE under the corresponding <node> resource should be monitored to determine if any management operation need to be performed. This part is implementation specific and is out of scope of oneM2M as well.

6.2	Use case

An application want to retrieve the current available memory of the device. In this case, the specialization of <mgmtObj> memory is used Annex D.4 of [b-oneM2M TS-0001]. In the memory specialization, memAvailable is the attribute that contains the required information. The application can get to know the available memory by retrieving the resource.

Figure: 6.2-1: Use case for management

6.3	Architecture

6.3.1	Management of ASN, MN and IN

Figure: 6.3.1-1: Architecture for management of IN,MN,ASN

In the architecture for management of IN, MN and ASN. As those Managed Entities have their own CSEBase resource, they have the capability to host oneM2M resources. When using device management over the service layer, the [memory] resource is directly hosting under the <node> resource of the CSEBase. In this case, the <node> resource represents the IN, MN, ASN themselves.

The Driver is the software that interacts with the memory of the device that acquires the total memory and available memory from the system.

In this case, the [memory] specialization is modified by Driver using internal interface. The modification may be done periodically or some other policies which is out of scope of oneM2M standard.

Whenever the AE issues retrieve to the [memory] specialization, the value from the resource is returned.

6.3.2	Management of ADN

Figure: 6.3.2-1: Architecture for management of ADN

In the architecture of management of ADN, the [memory] specialization is hosted on the Registrar CSE of the ADN. According to the supported configuration of oneM2M architecture, the Registrar CSE could be IN-CSE or MN-CSE.

In this case, the <node> resource hosted under the CSEBase of the Registrar CSE represents the ADN which is the managed entity. The driver gets the memory information using internal interfaces and make the ADN-AE updates the memory value to the [memory] specialization over the Mca reference point. The update over Mca is triggered periodically or based on the policy of the ADN which is implementation specific.

6.3.3	Management of NoDN

Figure: 6.3.3-1: Architecture for management of NoDN

In the architecture of management of NoDN, the [memory] specialization is hosted on the connection point of NoDN. The connection point is the entity that the NoDN is connected to. The entity has the adaptor that shares the same network protocol with NoDN.

In this case, the <node> resource hosted under the CSEBase represents the NoDN. And the connection between Driver and the CSE is out of scope of oneM2M. It may be Bluetooth, ZigBee or even proprietary.

The update of the memory information happens over the interface between driver and CSE which is out of scope of oneM2M.

6.4	Procedures

6.4.1	Management of ASN, MN and IN

Figure: 6.4.1-1: Procedures for management of IN, MN, ASN

001: The AE send an oneM2M Retrieve primitive to the ResourceID of [memory] resource.

002: The CSE receives the Retrieve primitive and triggers the Driver to acquire the current available memory value, update the [memory] resource.

003: The CSE responds the AE with the representation of the [memory] resource.

Note: The AE could also subscribe the [memory] resource to get the notification on the update of the [memory] resource.

The [memory] resource is this case is locally created before hand by the Hosting CSE using internal interfaces.

6.4.2	Management of ADN

Figure: 6.4.2-1: Procedures for management of ADN

001: The AE sends Retrieve request to Registrar CSE of ADN-AE. Requesting the [memory] resource.

003: The Registrar CSE responds with the representation of the [memory] resource.

002: The ADN internally acquires the available memory information.

004: The ADN-AE updateds the [memory] resource using Mca reference point.

005: Update successful

Note that 001, 003 and 002, 004 don’t have a specific order. The ADN is updating the [memory] resource periodically. In 003, the Registrar CSE is responding always the current resource.

Subscritpion and notification could also be used by the AE to monitor the current status of the [memory] resource.

The [memory] resource in this case is created by ADN-AE.

6.4.3	Management of NoDN

Figure: 6.4.3-1: Procedures for management of NoDN

001: The AE sends Retrieve request to connecton point of NoDN. Requesting the [memory] resource.

003: The CSE of the connection point responds with the representation of the [memory] resource.

002: The NoDN internally acquires the available memory information.

004: The NoDN updateds the [memory] resource using protocol that is non-oneM2M.

Note that 001, 003 and 002, 004 don’t have a specific order. The NoDN is updating the [memory] resource periodically. In 003, the Registrar CSE is responding always the current resource.

The [memory] resource in this case is created by NoDN via non oneM2M interface.

6.5	Implementation

6.5.1	Introduction

In this implementation,  the current document is using HTTP binding and JSON serialization as examples, and SP-relative-structured resource addressing format is used.

Following resource ID and/or entity ID is used:

- Memory Resource

  - Resource Name "memory001"

  - Resource ID "m343245334adf" represented in CSE-relative Unstructured-Resource-ID of the [memory] resource.

  - Parent Resource ID "n893051036jdg"

- AE-ID "C324352841DAS" of the managed entity

- Registrar/Hosting CSE

  - SP-relative CSE-ID "/CSE0034234"

  - CSE Resource Name "server"

- Host name of the Registrar/Hosting CSE "in.management.server.com".

6.5.2	Management of ASN, MN and IN

| Step | Message Example |
| --- | --- |
| 001 | HTTP Request GET /~/CSE0034234/server/memory001 HTTP/1.1 Host : in.management.server.com X-M2M-RI : 1234 X-M2M-Origin: C324352841DAS Accept : application/json |
| 002 | HTTP Response 200 OK X-M2M-RI : 1234 X-M2M-RSC : 2000 { "m2m:mgo" : { "ri":"m343245334adf", "pi":"n893051036jdg", "ct":"20161129T121550", "et":"20181231T235959", "lt":"20161130T152341", "ty":13, "rn":"memory001", "mgd":1003, "mma":800, "mmt":1024 } } |

6.5.3	Management of ADN

| Step | Message Example |
| --- | --- |
| 001 | Same as 001 in 5.5.2 |
| 002 | Same as 002 in 5.5.2 |
| 004 | HTTP Request PUT /~/CSE0034234/server/memory001?rcn=0 HTTP/1.1 Host : in.management.server.com X-M2M-RI : 1235 Content-Type: application/json X-M2M-Origin: C324352841DAS { "m2m:mgo":{ "mma":600 } } |
| 005 | HTTP Response 200 OK X-M2M-RI : 1235 X-M2M-RSC : 2004 |

6.5.4	Management of NoDN

001 and 003 is the same as in clause 6.5.2.

002 and 004is protocol specific and even proprietary, thus it’s out of scope of oneM2M.

7	Device management using external management technologies

7.1	Introduction

In the case of device management using external management technologies such as OMA DM, LWM2M or BBF TR069. All management is done through IN-CSE, since only the IN-CSE has the capability to talk to the Management Server. Therefore, in this case, all <node> resource that represents the managed entity is hosted under the <CSEBase> of IN-CSE. The manged entity may be IN, MN, ASN, ADN or NoDN. Then the <mgmtObj> that is used for management is hosted as the child resource of <node> resource. The attribute objectIDs and objectPaths of the <mgmtObj> are set in this case to map to the external management objects.

7.2	Use case

An application want to retrieve the current  avaiable memory of the device. In this case, the specialization of <mgmtObj> memory is used Annex D.4 of [b-oneM2M TS-0001]. In the memory specialization, memAvailable is the attribute that contains the required information. The application can get to know the available memory by retrieving the resource.

Figure: 7.2-1: Use case for management

7.3	Architecture

Figure: 7.3-1: Architecture for management using external technology

In the case of management using external management technology, all management related resources are hosted on the IN-CSE. IN-CSE interacts with Management Server through ms interface and triggers the Management Server to send management commands to Managed Entity through the mc interface. mc and ms interface is defined by external management technology or implementation specific which is out of scope of oneM2M.

The Management Server may be deployed together with the IN-CSE, in this case, the IN-CSE directly sends out management commands through mc interface to the Managed Entity. The Management Server may also be deployed separately from the IN-CSE, in this case, the IN-CSE need to send commands to Management Server to trigger tell the Management Server to send out management commands to the Managed Entity. ms interface may be implementation specific.

7.4	Procedures

Figure: 7.4-1: Procedures for management to ADN

001: AE retrieves [memory] resource specialization from IN-CSE over mca interface.

002: On receiving the request, the IN-CSE sends management command to managed entity using external management technology implementation.

003: On the managed entity, the driver aquires the memory information.

004: The managed entity responds with aquired memory information over the mc interface.

005: The IN-CSE updates the [memory] resource and respond the RETRIEVE request with the representation of [memory] resource.

7.5	Implementation

In the implementation, the document is using LWM2M as the external management technology. Resource ID and/or entity ID that are defined in Section 5.5.1 is also applicable to current section.

In addition, host name of the managed entity is named as "adn.managed.entity.com", the memory resource name is "memory002", the memory resource ID is "m463245245afg", and the parent resource ID of the memory resource is  "n993051028asd". The request URL for the managed entity is assumed as "coap://adn.managed.entity.com/adn001/memory".

| Step | Message Example |
| --- | --- |
| 001 | HTTP Request GET /~/CSE0034234/server/memory002 HTTP/1.1 Host : in.management.server.com X-M2M-RI : 3454 X-M2M-Origin: C324352841DAS Accept : application/json |
| 002 | CoAP Request Method : 0.01 (GET) Uri-Host : adn.managed.entity.com Uri-Path : adn001 Uri-Path : memory oneM2M-FR : C324352841DAS oneM2M-RQI: 4553 Payload : empty |
| 004 | CoAP Response X-M2M-RSC : 2.05 (OK) oneM2M-RQI : 4553 Content-Format: 50 (application/json) Payload : {memory_object_serialized_in_JSON} |
| 005-2 | HTTP Response X-M2M-RI : 3454 X-M2M-RSC : 2000 { "m2m:mgo" : { "ri":"m463245245afg", "pi":"n993051028asd", "ct":"20161129T121550", "et":"20181231T235959", "lt":"20161130T152341", "ty":13, "rn":"memory002", "mgd":1003, "mma":800, "mmt":1024 } } |

Annex A

oneM2M Specification Update and Maintenance Control Procedure

(This annex forms an integral part of this Recommendation.)

This Annex describes the update and maintenance control procedure that shall be followed for oneM2M specifications that are transposed as ITU-T Recommendations, and it applies explicitly to this Recommendation and any other Recommendation making normative reference to this Annex.

The oneM2M Specifications developed by oneM2M are updated on a regular basis. Continued alignment of the specifications in this Recommendation and the oneM2M Specifications is fundamental to ensure compatibility amongst devices and services available on the market and to avoid confusion amongst users and implementers.

In summary, any and all improvements, modifications or changes to this Recommendation are developed in oneM2M, and are submitted on regular basis to ITU, to be integrated into a future in-force edition of this Recommendation.

The following procedure defines how improvements, modifications or changes to the oneM2M specifications are acknowledged and integrated into this Recommendation.

  - Any and all improvements, modifications or changes to this Recommendation, upon agreement by the oneM2M, shall be submitted by oneM2M member(s) (e.g. Partner Type 1, Member Company), that is also an ITU-T Member, as input to ITU-T SG20.

- Submission of any and all improvements, modifications or changes to this Recommendation will clearly indicate applicable page, clause, table, figure and paragraph affected. NOTE – Full text submission is preferred than delta change instructions, to avoid ambiguities and change implementation mistakes.

- The submission will contain reference information to indicate the transposition of oneM2M Specifications as oneM2M Partner Type 1 standards.

  - Upon receiving the submission, the ITU-T study group concerned will take action for processing towards a revised edition of this Recommendation (as per [ITU-T A.8]).

  - IPR disclosures and licensing declarations associated with oneM2M specifications are made pursuant to the IPR policies and procedures of oneM2M Partner Organizations.  For more information, please see the oneM2M Partner Agreement.

Bibliography

-

[b-oneM2M DR]			oneM2M Drafting Rules.

NOTE:	Available at http://www.onem2m.org/images/files/oneM2M-Drafting-Rules.pdf.

[b-oneM2M TS-0001]	oneM2M Technical Specification TS-0001, Functional Architecture