| CHANGE REQUEST |  |
| --- | --- |
| Meeting:* | TDE #49.2 |
| Source:* | TF-oneM2M |
| Date:* | 2021-01-18 |
| Contact:* | Miguel Angel Reina Ortega, ETSI, MiguelAngel.ReinaOrtega@etsi.org |
| Reason for Change/s:* | New TPs for cross-resource notification release 4 |
| CR against: Release* | Rel-4 |
| CR against: WI* | Active <Work Item number> MNT Maintenace / < Work Item number(optional)> STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0018-V-4.2.0 |
| Clauses/Sub Clauses* |  |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR is a mirror CR? YES NO if YES, please indicate the document number of the original CR: |
| Template Version:23 February 2015 (Dot not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete sections need not show surrounding clauses as long as the proposed section number clearly shows where the new section is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

New TPs for cross-resource notification release 4.

R01 – Including corrections as discussed during the last call.

R02 – Including some small corrections to TPs (test objective enhancement and parent release) and two new TPs

----------------------- Start of change 1 -----------------------

##### 7.2.2.4.4	NOTIFY Operation

TP/oneM2M/CSE/SUB/NTF/012

| TP Id |  | TP/oneM2M/CSE/SUB/NTF/012 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT successfully sends a cross-resource notification to the notificationURI when notifications from resources in regularResourcesAsTarget are received within required TIME_LIMIT window |  |
| Reference |  | TS-0001 [1], clause 9.6.58, TS-0004 [2], clause 7.5.1.2.18 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 containing a container resource on CONTAINER_RESOURCE_ADDRESS_1 a container resource on CONTAINER_RESOURCE_ADDRESS_2 and the IUT having created a <crossResourceSubsription> resource on CROSS_RESOURCE_SUBS_ADDRESS containing regularResourcesAsTarget attribute containing CONTAINER_RESOURCE_ADDRESS_1 and CONTAINER_RESOURCE_ADDRESS_2 and timeWindowType attribute set to TIME_WINDOW_TYPE and timeWindowSize attribute set to DURATION and notificationURI attribute set to AE2_RESOURCE_ADDRESS and the IUT having registered the AE2 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request (Req1) from AE1 contaning To set to CONTAINER_RESOURCE_ADDRESS_1 and From set to AE_ID and Content containing container resource containing valid labels attribute and the IUT receives a valid UPDATE Request (Req2) from AE1 contaning To set to CONTAINER_RESOURCE_ADDRESS_2 and From set to AE_ID and Content containing container resource containing valid labels attribute and timer has not reached TIME_LIMIT } |  | IUT  AE1 IUT  AE1 |
|  | then { the IUT sends a valid Response to AE1 for Req1 and the IUT sends a valid Response to AE1 for Req2 and the IUT sends a NOTIFY Request to AE2 generated from crossResourceSubsription on CROSS_RESOURCE_SUBS_ADDRESS containing Content containing notification message containing subscriptionReference attribute set to CROSS_RESOURCE_SUBS_ADDRESS } |  | IUT  AE1 IUT  AE1 IUT AE2 |

| TP Id | TIME_WINDOW_TYPE |
| --- | --- |
| TP/oneM2M/CSE/SUB/NTF/012_TWT/1 | 1 (PERIODICWINDOW) |
| TP/oneM2M/CSE/SUB/NTF/012_TWT/2 | 2 (SLIDINGWINDOW) |

TP/oneM2M/CSE/SUB/NTF/013

| TP Id |  | TP/oneM2M/CSE/SUB/NTF/013 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT does not send a cross-resource notification to the notificationURI when a notification from resource in regularResourcesAsTarget is not received within required TIME_LIMIT window |  |
| Reference |  | TS-0001 [1], clause 9.6.58, TS-0004 [2], clause 7.5.1.2.18 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 containing a container resource on CONTAINER_RESOURCE_ADDRESS_1 a container resource on CONTAINER_RESOURCE_ADDRESS_2 and the IUT having created a <crossResourceSubsription> resource on CROSS_RESOURCE_SUBS_ADDRESS containing regularResourcesAsTarget attribute containing CONTAINER_RESOURCE_ADDRESS_1 and CONTAINER_RESOURCE_ADDRESS_2 and timeWindowType attribute set to TIME_WINDOW_TYPE and timeWindowSize attribute set to DURATION and notificationURI attribute set to AE2_RESOURCE_ADDRESS and the IUT having registered the AE2 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request (Req1) from AE1 contaning To set to CONTAINER_RESOURCE_ADDRESS_1 and From set to AE_ID and Content containing container resource containing valid labels attribute and the IUT receives a valid UPDATE Request (Req2) from AE1 contaning To set to CONTAINER_RESOURCE_ADDRESS_2 and From set to AE_ID and Content containing container resource containing valid labels attribute and timer has reached TIME_LIMIT } |  | IUT  AE1 IUT  AE1 |
|  | then { the IUT sends a valid Response to AE1 for Req1 and the IUT sends a valid Response to AE1 for Req2 and the IUT does not send a NOTIFY Request to AE2 } |  | IUT  AE1 IUT  AE1 |

| TP Id | TIME_WINDOW_TYPE |
| --- | --- |
| TP/oneM2M/CSE/SUB/NTF/013_TWT/1 | 1 (PERIODICWINDOW) |
| TP/oneM2M/CSE/SUB/NTF/013_TWT/2 | 2 (SLIDINGWINDOW) |

TP/oneM2M/CSE/SUB/NTF/014

| TP Id |  | TP/oneM2M/CSE/SUB/NTF/014 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a cross-resource notification to the notificationURI when notifications from resources in regularResourcesAsTarget are received within required TIME_LIMIT window having different NOTIFICATION_EVENT_TYPE values for each of the target resources |  |
| Reference |  | TS-0001 [1], clause 9.6.58, TS-0004 [2], clause 7.5.1.2.18 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 containing a container resource on CONTAINER_RESOURCE_ADDRESS_1 a container resource on CONTAINER_RESOURCE_ADDRESS_2 and the IUT having created a <crossResourceSubsription> resource on CROSS_RESOURCE_SUBS_ADDRESS containing regularResourcesAsTarget attribute containing CONTAINER_RESOURCE_ADDRESS_1 and CONTAINER_RESOURCE_ADDRESS_2 and timeWindowType attribute set to TIME_WINDOW_TYPE and timeWindowSize attribute set to DURATION and eventNotificationCriteriaSet containing EVENT_NOTIFICATION_CRITERIA_1 set to 1 (Update_of_resource) and EVENT_NOTIFICATION_CRITERIA_2 set to 3 (Create_of_Direct_Child_Resource) and notificationURI attribute set to AE2_RESOURCE_ADDRESS and the IUT having registered the AE2 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request (Req1) from AE1 contaning To set to CONTAINER_RESOURCE_ADDRESS_1 and From set to AE_ID and Content containing container resource containing valid labels attribute and the IUT receives a valid CREATE Request (Req2) from AE1 contaning To set to CONTAINER_RESOURCE_ADDRESS_2 and From set to AE_ID and Content containing a valid container resource representation and timer has not reached TIME_LIMIT } |  | IUT  AE1 IUT  AE1 |
|  | then { the IUT sends a valid Response to AE1 for Req1 and the IUT sends a valid Response to AE1 for Req2 and the IUT sends a NOTIFY Request to AE2 generated from crossResourceSubscription CROSS_RESOURCE_SUBS_ADDRESS containing Content containing notification message containing subscriptionReference attribute set to CROSS_SUBS_RESOURCE_ADDRESS } |  | IUT  AE1 IUT  AE1 IUT  AE2 |

| TP Id | TIME_WINDOW_TYPE |
| --- | --- |
| TP/oneM2M/CSE/SUB/NTF/014_TWT/1 | 1 (PERIODICWINDOW) |
| TP/oneM2M/CSE/SUB/NTF/014_TWT/2 | 2 (SLIDINGWINDOW) |

TP/oneM2M/CSE/SUB/NTF/015

| TP Id |  | TP/oneM2M/CSE/SUB/NTF/015 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT successfully sends a cross-resource notification to the notificationURI when notifications from resources in subscriptionResourcesAsTarget are received within required TIME_LIMIT window |  |
| Reference |  | TS-0001 [1], clause 9.6.58, TS-0004 [2], clause 7.5.1.2.18 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 containing a container resource on CONTAINER_RESOURCE_ADDRESS_1 containing a subscription resource on SUBS_RESOURCE_ADDRESS_1 a container resource on CONTAINER_RESOURCE_ADDRESS_2 containing a subscription resource on SUBS_RESOURCE_ADDRESS_2 and the IUT having created a <crossResourceSubscription> resource on CROSS_RESOURCE_SUBS_ADDRESS containing subscriptionResourcesAsTarget attribute containing SUBS _RESOURCE_ADDRESS_1 and SUBS _RESOURCE_ADDRESS_2 and timeWindowType attribute set to TIME_WINDOW_TYPE and timeWindowSize attribute set to DURATION and notificationURI attribute set to AE2_RESOURCE_ADDRESS and the IUT having registered the AE2 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request (Req1) from AE1 contaning To set to AE1_RESOURCE_ADDRESS_1 and From set to AE_ID and Content containing AE resource containing valid labels attribute and the IUT receives a valid UPDATE Request (Req2) from AE1 contaning To set to AE1_RESOURCE_ADDRESS_2 and From set to AE_ID and Content containing AE resource containing valid labels attribute and timer has not reached TIME_LIMIT } |  | IUT  AE1 IUT  AE1 |
|  | then { the IUT sends a valid Response to AE1 for Req1 and the IUT sends a valid Response to AE1 for Req2 and the IUT sends a NOTIFY Request to AE1 generated from SUBS_RESOURCE_ADDRESS_1 and the IUT sends a NOTIFY Request to AE1 generated from SUBS_RESOURCE_ADDRESS_2 and the IUT sends a NOTIFY Request to AE2 generated from crossResourceSubscription CROSS_RESOURCE_SUBS_ADDRESS containing Content containing notification message containing subscriptionReference attribute set to CROSS_SUBS_RESOURCE_ADDRESS } |  | IUT  AE1 IUT  AE1 IUT  AE1 IUT  AE1 IUT AE2 |

| TP Id | TIME_WINDOW_TYPE |
| --- | --- |
| TP/oneM2M/CSE/SUB/NTF/015_TWT/1 | 1 (PERIODICWINDOW) |
| TP/oneM2M/CSE/SUB/NTF/015_TWT/2 | 2 (SLIDINGWINDOW) |

TP/oneM2M/CSE/SUB/NTF/016

| TP Id |  | TP/oneM2M/CSE/SUB/NTF/016 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT does not send a cross-resource notification to the notificationURI when a notification from resource in subscriptionResourcesAsTarget is not received within required TIME_LIMIT window |  |
| Reference |  | TS-0001 [1], clause 9.6.58, TS-0004 [2], clause 7.5.1.2.18 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 containing a container resource on CONTAINER_RESOURCE_ADDRESS_1 containing a subscription resource on SUBS_RESOURCE_ADDRESS_1 a container resource on CONTAINER_RESOURCE_ADDRESS_2 containing a subscription resource on SUBS_RESOURCE_ADDRESS_2 and the IUT having created a <crossResourceSubscription> resource on CROSS_RESOURCE_SUBS_ADDRESS containing subscriptionResourcesAsTarget attribute containing SUBS _RESOURCE_ADDRESS_1 and SUBS _RESOURCE_ADDRESS_2 and timeWindowType attribute set to TIME_WINDOW_TYPE and timeWindowSize attribute set to DURATION and notificationURI attribute set to AE2_RESOURCE_ADDRESS and the IUT having registered the AE2 } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request (Req1) from AE1 contaning To set to AE1_RESOURCE_ADDRESS_1 and From set to AE_ID and Content containing AE resource containing valid labels attribute and the IUT receives a valid UPDATE Request (Req2) from AE1 contaning To set to AE1_RESOURCE_ADDRESS_2 and From set to AE_ID and Content containing AE resource containing valid labels attribute and timer has reached TIME_LIMIT } |  | IUT  AE1 IUT  AE1 |
|  | then { the IUT sends a valid Response to AE1 for Req1 and the IUT sends a valid Response to AE1 for Req2 and the IUT sends a NOTIFY Request to AE1 generated from SUBS_RESOURCE_ADDRESS_1 and the IUT sends a NOTIFY Request to AE1 generated from SUBS_RESOURCE_ADDRESS_2 and the IUT does not send a NOTIFY Request to AE2 } |  | IUT  AE1 IUT  AE1 IUT  AE1 IUT  AE1 |

| TP Id | TIME_WINDOW_TYPE |
| --- | --- |
| TP/oneM2M/CSE/SUB/NTF/016_TWT/1 | 1 (PERIODICWINDOW) |
| TP/oneM2M/CSE/SUB/NTF/016_TWT/2 | 2 (SLIDINGWINDOW) |

TP/oneM2M/CSE/SUB/NTF/017

| TP Id |  | TP/oneM2M/CSE/SUB/NTF/017 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT successfully sends notification request to the notificationURI when an update request is received for the target subscribed-to resource with notificationEventType set to "Blocking_Update" |  |
| Reference |  | TS-0004 [1], clause 7.5.1.2.19 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and eventNotificationCriteria attribute containing notificationEventType set to 7 (Blocking_Update) and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE1 containing To set to SUBSCRIBED_TO_RESOURCE_ADDRESS and From set to AE1_RESOURCE_ADDRESS and Content containing <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_2 } |  | IUT  AE1 |
|  | then { the IUT sends a NOTIFY Request to AE2 contaning Content containing notification message containing notificationEvent attribute containing representation attribute containing <subscribed-to type> resource containing All attributes } |  | IUT AE2 |

TP/oneM2M/CSE/SUB/NTF/018

| TP Id |  | TP/oneM2M/CSE/SUB/NTF/018 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT successfully sends notification request to the notificationURI when an update request is received for the target subscribed-to resource with notificationEventType set to "Blocking_Update" when the attribute condition tag matches the modified attributes |  |
| Reference |  | TS-0004 [1], clause 7.5.1.2.19 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and eventNotificationCriteria attribute containing notificationEventType set to 7 (Blocking_Update) attribute set to ATTRIBUTE_NAME and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE1 containing To set to SUBSCRIBED_TO_RESOURCE_ADDRESS and From set to AE1_RESOURCE_ADDRESS and Content containing <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_2 } |  | IUT  AE1 |
|  | then { the IUT sends a NOTIFY Request to AE2 contaning Content containing notification message containing notificationEvent attribute containing representation attribute containing <subscribed-to type> resource containing All attributes } |  | IUT AE2 |

TP/oneM2M/CSE/SUB/NTF/019

| TP Id |  | TP/oneM2M/CSE/SUB/NTF/019 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT does not send notification request to the notificationURI when an update request is received for the target subscribed-to resource with notificationEventType set to "Blocking_Update" but the attribute condition tag does not match the modified attributes |  |
| Reference |  | TS-0004 [1], clause 7.5.1.2.19 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME_1 set to VALUE_1 and subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and eventNotificationCriteria attribute containing notificationEventType set to 7 (Blocking_Update) attribute set to ATTRIBUTE_NAME_2 and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE1 containing To set to SUBSCRIBED_TO_RESOURCE_ADDRESS and From set to AE1_RESOURCE_ADDRESS and Content containing <subscribed-to type> resource containing ATTRIBUTE_NAME_1 set to VALUE_2 } |  | IUT  AE1 |
|  | then { the IUT does not send a NOTIFY Request to AE2_RESOURCE_ADDRESS } |  |  |

TP/oneM2M/CSE/SUB/NTF/020

| TP Id |  | TP/oneM2M/CSE/SUB/NTF/020 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT blocks update request for the target subscribed-to resource when an previous update request is received with notificationEventType set to "Blocking_Update" and notification process is still not completed |  |
| Reference |  | TS-0004 [1], clause 7.5.1.2.19 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and eventNotificationCriteria attribute containing notificationEventType set to 7 (Blocking_Update) and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource and the IUT having received a valid UPDATE Request from AE1_RESOURCE_ADDRESS to SUBSCRIBED_TO_RESOURCE_ADDRESS and the IUT having sent a valid NOTIFY Request to AE2_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE1 containing To set to SUBSCRIBED_TO_RESOURCE_ADDRESS and From set to AE1_RESOURCE_ADDRESS and Content containing <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_2 } |  | IUT  AE1 |
|  | then { the IUT does not update SUBSCRIBED_TO_RESOURCE_ADDRESS and the IUT does not send a NOTFIY Request to AE2_RESOURCE_ADDRESS } |  |  |

TP/oneM2M/CSE/SUB/NTF/021

| TP Id |  | TP/oneM2M/CSE/SUB/NTF/021 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT successfully performs update request to the target subscribed-to resource with notificationEventType set to "Blocking_Update" |  |
| Reference |  | TS-0004 [1], clause 7.5.1.2.19 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and eventNotificationCriteria attribute containing notificationEventType set to 7 (Blocking_Update) and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource and the IUT having received a valid UPDATE Request from AE1_RESOURCE_ADDRESS to SUBSCRIBED_TO_RESOURCE_ADDRESS and the IUT having sent the NOTIFY Request to the AE2_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid NOTIFY Response from AE2 containing ResponseStatusCode set to OK (2000) } |  | IUT  AE2 |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) } |  | IUT AE1 |

TP/oneM2M/CSE/SUB/NTF/022

| TP Id |  | TP/oneM2M/CSE/SUB/NTF/022 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects update request to the target subscribed-to resource when notificationEventType is set to "Blocking_Update" and the IUT have received the Notify response containing Response Status Code indicating TARGET_NOT_REACHABLE |  |
| Reference |  | TS-0004 [1], clause 7.5.1.2.19 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and eventNotificationCriteria attribute containing notificationEventType set to 7 (Blocking_Update) and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource and the IUT having received a valid UPDATE Request from AE1_RESOURCE_ADDRESS to SUBSCRIBED_TO_RESOURCE_ADDRESS and the IUT having sent a NOTIFY Request to AE2_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid NOTIFY Response from AE2 containing ResponseStatusCode set to TARGET_NOT_REACHABLE (5103) } |  | IUT  AE2 |
|  | then { the IUT sends a valid Response containing Response Status Code set to 5107 (REMOTE_ENTITY_NOT_REACHABLE) } |  | IUT AE1 |

TP/oneM2M/CSE/SUB/NTF/023

| TP Id |  | TP/oneM2M/CSE/SUB/NTF/023 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects update request to the target subscribed-to resource when notificationEventType is set to "Blocking_Update" and the IUT have received the Notify response containing Response Status Code indicating OPERATION_NOT_ALLOWED |  |
| Reference |  | TS-0004 [1], clause 7.5.1.2.19 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and eventNotificationCriteria attribute containing notificationEventType set to 7 (Blocking_Update) and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource and the IUT having received a valid UPDATE Request from AE1_RESOURCE_ADDRESS to SUBSCRIBED_TO_RESOURCE_ADDRESS and the IUT having sent a NOTIFY Request to AE2_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid NOTIFY Response from AE2 containing ResponseStatusCode set to OPERATION_NOT_ALLOWED (4005) } |  | IUT  AE2 |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4127 (OPERATION_DENIED_BY_REMOTE_ENTITY) } |  | IUT AE1 |

TP/oneM2M/CSE/SUB/NTF/024

| TP Id |  | TP/oneM2M/CSE/SUB/NTF/024 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the creation of a subscription resource with notificationEventType set to "Blocking_Update" when the subscribed-to resource has already a subscription resource with this notificationEventType |  |
| Reference |  | TS-0004 [1], clause 7.5.1.2.19 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing subscription child resource containing eventNotificationCriteria attribute containing notificationEventType set to 7 (Blocking_Update) and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { the IUT receives a valid CREATE Request from AE1 containing To set to SUBSCRIBED_TO_RESOURCE_ADDRESS and Content containing a subscription resource containing eventNotificationCriteria attribute containing notificationEventType set to 7 (Blocking_Update) } |  | IUT  AE1 |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4124 (BLOCKING_SUBSCRIPTION_ALREADY_EXISTS) } |  | IUT AE1 |

## ---------------------- End of change 1 -----------------------

CHECK LIST

- Does this change request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Does this change request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable?

- Does this change request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete sections need not show surrounding clauses as long as the proposed section number clearly shows where the new section is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?