| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS #62 |
| Source:* | Andreas Kraft, DT, Andreas.Kraft@t-systems.com Andreas Neubacher, DT, Andreas.Neubacher@magenta.at |
| Date:* | 2023-10-30 |
| Reason for Change/s:* | TS-0001 – Adding <processManagement> child resource to various resource types |
| CR against: Release* | Release 5 |
| CR against: WI* | Active WI-xxxx MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0001 v.5.3.0 |
| Clauses * | 9.6.1.1, 9.6.3, 9.6.4, 9.6.5 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) |  |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

Introduction

This CR adds the missing resource types <processManagement>, resp. <processManagementAnnc>, to TS-0001, table “9.6.1.1-1: Resource Types”, and to the child resource tables of the <CSEBase>, <remoteCSE>, and <AE> resource type clauses.

In TS-0001 the resource type <processManagement> is defined in clause 9.6.75. In table “9.6.1.1-1: Resource Types” it is listed as well with the resource types <CSEBase>, <remoteCSE>, and <AE> as allowed parent resource types. However, in the entries of these parent resource types the <processManagement>, resp. <processManagementAnnc>, resource types are missing. The respective entries are also missing in the child resource tables of the <CSEBase>, <remoteCSE>, and <AE> resource type clauses.

For the <remoteCSE> resource type entries, only the announced resource type variant (<processManagementAnnc>) is added.

If this CR is agreed a mirror for R4 will be contributed.

R01:

- Add processManagement to remoteCSE in addition to processManagementAnnc.

### **********************  Start of Change 1   *****************************

#### 9.6.1.1	Resource Type Summary

Table 9.6.1.1-1 introduces the normal and virtual resource types and their related child or parent resource types. Details of each resource type follow in the remainder of this clause.

Table 9.6.1.1-1 lists each specified ordinary - i.e. not announced - resource type. An addition of suffix "Annc" to the respective resource type identifier indicates the associated announced resource type. Resource types that can occur as child resources of announced resources are summarized in table 9.6.26.1-1.

Among the resource types listed in table 9.6.1.1-1, the following are termed "Content Sharing Resources" in oneM2M Specifications for the purpose of referring to any of those resource types:

- container;

- contentInstance;

- flexContainer;

- flexContainerInstance;

- timeSeries;

- timeSeriesInstance.

Table 9.6.1.1-1: Resource Types

| Resource Type | Short Description | Child Resource Types | Parent Resource Types | Clause |
| --- | --- | --- | --- | --- |
| accessControlPolicy | Stores a representation of privileges. It is associated with resources that shall be accessible to entities external to the Hosting CSE. It controls "who" is allowed to do "what" and the context in which it can be used for accessing resources | subscription, transaction | AE, AEAnnc, remoteCSE, remoteCSEAnnc, CSEBase, CSEBaseAnnc | 9.6.2 |
| AE | Stores information about the AE. It is created as a result of successful registration of an AE with the Registrar CSE | subscription, container, flexContainer, group, accessControlPolicy, pollingChannel, semanticDescriptor, timeSeries, transaction, transactionMgmt, triggerRequest, crossResourceSubscription, backgroundDataTransfer, semanticMashupInstance, locationPolicy, action, multimediaSession, dynamicAuthorizationConsultation, nwMonitoringReq, timeSyncBeacon, primitiveProfile, softwareCampaign, e2eQosSession, processManagement | CSEBase | 9.6.5 |
| container | Shares data instances among entities. Used as a mediator that buffers data exchanged between AEs and/or CSEs. The exchange of data between AEs (e.g. an AE on a Node in a field domain and the peer-AE on the infrastructure domain) is abstracted from the need to set up direct connections and allows for scenarios where both entities in the exchange do not have the same reachability schedule | container, flexContainer, contentInstance, subscription, latest, oldest，semanticDescriptor, timeSeries, transaction, action | AE, AEAnnc, container, containerAnnc, remoteCSE, remoteCSEAnnc, CSEBase, CSEBaseAnnc, flexContainer, flexContainerAnnc | 9.6.6 |
| contentInstance | Represents a data instance in the <container> resource | semanticDescriptor, transaction | container, containerAnnc | 9.6.7 |
| flexContainer | A template which allows to define specialized (customizable) versions of containers with a flexible and lightweight structure | container, flexContainer, flexContainerInstance, latest, oldest, subscription, semanticDescriptor, timeSeries, transaction, action | AE, AEAnnc, container, containerAnnc, flexContainer, flexContainerAnnc, node, remoteCSE, remoteCSEAnnc, CSEBase, CSEBaseAnnc | 9.6.35 |
| flexContainerInstance | Represents a timestamped Data instance in the <flexContainer> resource | transaction | flexContainer | 9.6.59 |
| CSEBase | The structural root for all the resources that are residing on a CSE. Stores information about the CSE itself | remoteCSE, remoteCSEAnnc, CSEBaseAnnc, node, AE, container, group, accessControlPolicy, subscription, mgmtCmd, locationPolicy, statsConfig, statsCollect, request, delivery, schedule, notificationTargetPolicy, semanticRuleRepository, flexContainer, timeSeries, AEContactList, transaction, transactionMgmt, crossResourceSubscription, backgroundDataTransfer, semanticMashupJobProfile, semanticMashupInstance, action, m2mServiceSubscriptionProfile, dynamicAuthorizationConsultation, localMulticastGroup, serviceSubscribedAppRule, authorizationPolicy, authorizationDecision, authorizationInformation, ontologyRepository, nwMonitoringReq, timeSyncBeacon, primitiveProfile, softwareCampaign, e2eQosSession, semanticRoutingTable, processManagement | None specified | 9.6.3 |
| delivery | Forwards requests from CSE to CSE | subscription, transaction | CSEBase | 9.6.11 |
| eventConfig | Defines events that trigger statistics collection | subscription, transaction | statsConfig | 9.6.24 |
| execInstance | Contains all execution instances of the same Management Command | subscription, transaction | mgmtCmd | 9.6.17 |
| fanOutPoint (V) | Virtual resource containing target for group request It is used for addressing bulk operations to all the resources that belong to a group | None specified | group | 9.6.14 |
| group | Stores information about resources of the same type that need to be addressed as a Group. Operations addressed to a Group resource shall be executed in a bulk mode for all members belonging to the Group | fanOutPoint, subscription, semanticFanOutPoint, semanticDescriptor, transaction, action | AE, AEAnnc, remoteCSE, remoteCSEAnnc, CSEBase, CSEBaseAnnc | 9.6.13 |
| latest (V) | Virtual resource that points to most recently created <contentInstance>, <timeSeriesInstance> or <flexContainerInstance> child resource within a <container>, <timeSeries> or <flexContainer> resource | None specified | container, timeSeries, or flexContainer | 9.6.27 |
| locationPolicy | Includes information to obtain and manage geographical location. It is only referenced within a container, the contentInstances of the container provide location information | subscription, transaction | CSEBase, AE | 9.6.10 |
| mgmtCmd | Management Command resource represents a method to execute management procedures required by existing management protocols | execInstance, subscription, transaction | CSEBase | 9.6.16 |
| mgmtObj | Management Object resource represents management functions that provides an abstraction to be mapped to external management technology. It represents the node and the software installed in the node (see note) | subscription, transaction, semanticDescriptor | node, mgmtObjAnnc | 9.6.15 Annex D |
| m2mServiceSubscriptionProfile | Data pertaining to the M2M Service Subscription | serviceSubscribedNode, subscription, transaction, serviceSubscribedUserProfile | CSEBase | 9.6.19 |
| node | Represents specific Node information | flexContainer, mgmtObj, subscription, semanticDescriptor, schedule, transaction, action | CSEBase | 9.6.18 |
| notificationTargetMgmtPolicyRef | Represents a list of notification targets and the deletion policy | subscription, transaction | subscription | 9.6.31 |
| notificationTargetPolicy | Represents a notification target deletion policy with pre-defined action and deletion rules | subscription, policyDeletionRules, transaction | CSEBase | 9.6.32 |
| notificationTargetSelfReference (V) | Virtual resource used to remove the Notification Target | None specified | subscription | 9.6.34 |
| oldest (V) | Virtual resource that points to first created <contentInstance>, <timeSeriesInstance> or <flexContainerInstance> child resource within a <container>, <timeSeries> or <flexContainer> resource | None specified | container, timeSeries, or flexContainer | 9.6.28 |
| pollingChannel | Represent a channel that can be used for a request-unreachable entity | pollingChannelURI | remoteCSE, AE | 9.6.21 |
| pollingChannelURI (V) | Virtual resource used to perform service layer long polling of a resource Hosting CSE by a request-unreachable entity | None specified | pollingChannel | 9.6.22 |
| policyDeletionRules | Represents a set of rules which is associated with notification target removal policy | subscription, transaction | notificationTargetPolicy | 9.6.33 |
| remoteCSE | Represents a remote CSE for which there has been a registration procedure with the registrar CSE identified by the CSEBase resource | container, containerAnnc, contentInstanceAnnc flexContainer, flexContainerAnnc, group, groupAnnc, accessControlPolicy, accessControlPolicyAnnc, subscription, scheduleAnnc, pollingChannel, timeSeries, timeSeriesAnnc, timeSeriesInstanceAnnc, remoteCSEAnnc, mgmtObjAnnc, nodeAnnc, AEAnnc, locationPolicyAnnc, transaction, crossResourceSubscription, backgroundDataTransfer, semanticDescriptorAnnc, semanticMashupJobProfile, semanticMashupJobProfileAnnc, semanticMashupInstance, semanticMashupInstanceAnnc, action, dynamicAuthorizationConsultation, dynamicAuthorizationConsultationAnnc, nwMonitoringReq, timeSyncBeacon, primitiveProfile, softwareCampaign, e2eQosSession, semanticRoutingTable, processManagement, processManagementAnnc | CSEBase | 9.6.4 |
| request | Expresses/access context of an issued Request | subscription, transaction | CSEBase | 9.6.12 |
| schedule | Contains scheduling information for delivery of messages | subscription, transaction | subscription, CSEBase, node, softwareCampaign | 9.6.9 |
| serviceSubscribedNode | Node information | subscription, transaction | m2mServiceSubscriptionProfile | 9.6.20 |
| statsCollect | Defines triggers for the IN-CSE to collect statistics for applications | subscription, transaction | CSEBase (in IN-CSE) | 9.6.25 |
| statsConfig | Stores configuration of statistics for applications | eventConfig, subscription, transaction | CSEBase (in IN-CSE) | 9.6.23 |
| subscription | Subscription resource represents the subscription information related to a resource. Such a resource shall be a child resource for the subscribed-to resource | schedule, notificationTargetSelfReference, notificationTargetMgmtPolicyRef, transaction | accessControlPolicy, accessControlPolicyAnnc, AE, AEAnnc, container, containerAnnc, CSEBase, delivery, eventConfig, execInstance, group, groupAnnc, locationPolicy, locationPolicyAnnc, mgmtCmd, mgmtObj, mgmtObjAnnc, m2mServiceSubscriptionProfile, node, nodeAnnc, serviceSubscribedNode, remoteCSE, remoteCSEAnnc, CSEBaseAnnc, request, schedule, scheduleAnnc, semanticDescriptor, semanticDescriptorAnnc, statsCollect, statsConfig, flexContainer, flexContainerAnnc, timeSeries, timeSeriesAnnc, nwMonitoringReq, semanticRuleRepository, reasoningRules, reasoningJobInstance, timeSyncBeacon, primitiveProfile, state, processManagement, semanticRoutingTable | 9.6.8 |
| serviceSubscribedAppRule | Represents a rule that defines allowed App-ID and AE-ID combinations that are acceptable for registering an AE on a Registrar CSE | subscription, transaction | CSEBase | 9.6.29 |
| semanticDescriptor | Stores semantic description pertaining to a resource and potentially sub-resources | subscription, transaction | AE, container, contentInstance, group, node, flexContainer, timeSeries, mgmtObj | 9.6.30 |
| semanticFanOutPoint | Virtual resource used as target for semantic discovery aimed at a logical graph distributed over multiple semanticDescriptor resources, which belong to the corresponding group parent resource |  | group | 9.6.14a |
| dynamicAuthorizationConsultation | Represents consultation information used by a CSE when performing consultation-based dynamic authorization | subscription, transaction | AE, AEAnnc, remoteCSE, remoteCSEAnnc, CSEBase, CSEBaseAnnc | 9.6.40 |
| timeSeries | Stores and Shares Time Series Data instances among entities | timeSeriesInstance, subscription, semanticDescriptor, latest, oldest, transaction, action | AE, AEAnnc, remoteCSE, remoteCSEAnnc, CSEBase, CSEBaseAnnc, container, containerAnnc, flexContainer, flexContainerAnnc | 9.6.36 |
| timeSeriesInstance | Represents a Time Series Data instance in the <timeSeries> resource | transaction | timeSeries, timeSeriesAnnc | 9.6.37 |
| role | Represents a role that is assigned to an AE or CSE | subscription, transaction | authorizationInformation | 9.6.38 |
| token | Used for storing a token that is issued to an AE or CSE | subscription, transaction | authorizationInformation | 9.6.39 |
| authorizationDecision | Represents an access control decision point | subscription, transaction | CSEBase | 9.6.41 |
| authorizationPolicy | Represents an access control policy retrieval point | subscription, transaction | CSEBase | 9.6.42 |
| authorizationInformation | Represents an access control information point | role token subscription, transaction | CSEBase | 9.6.43 |
| localMulticastGroup | Stores local multicast group information of member hosting CSE. | transaction | CSEBase | 9.6.44 |
| AEContactList | Contains information about a CSE that has resources that referencing an AE-ID | AEContactListPerCSE, subscription, transaction | CSEBase | 9.6.45 |
| AEContactListPerCSE | Contains information about a CSE that has resources that referencing an AE resource identifier for tracking purposes | None specified | AEContactList | 9.6.46 |
| transactionMgmt |  | subscription | CSEBase, AE, remoteCSE | 9.6.47 |
| transaction |  | action | All non-virtual resource types with the exception of the following: request, delivery, pollingChannel, transactionMgmt, transaction, timeSyncBeacon | 9.6.48 |
| triggerRequest | Used by an AE to initiate, replace or recall a device trigger request | subscription | AE | 9.6.49 |
| ontologyRepository | Represents the collection of the managed ontologies and the semantic validation service | ontology, semanticValidation, subscription, ontologyMapping, ontologyMappingAlgorithmRepository | CSEBase | 9.6.50 |
| ontology | Store the representation of an ontology | subscription | ontologyRepository | 9.6.51 |
| semanticValidation | A virtual resource as the interface to perform semantic validation on the received <semanticDescriptor> resource against the referenced ontology | None specified | ontologyRepository | 9.6.52 |
| ontologyMapping | Represents a mapping operation between two different ontologies | subscription | ontologyRepository | 9.6.70 |
| ontologyMappingAlgorithmRepository | Represents the collection of ontology mapping algorithms | subscription, ontologyMappingAlogrithm | ontologyRepository | 9.6.71 |
| ontologyMappingAlogrithm | Represents an ontology mapping algorithm | None specified | ontologyMappingAlgorithmRepository | 9.6.72 |
| semanticMashupJobProfile | Represents the profile and description of a semantic mashup service | semanticMashupInstance, semanticDescriptor, subscription | CSEBase, remoteCSE | 9.6.53 |
| semanitcMashupInstance | Represents a semantic mashup instance | semanticMashupResult, semanticDescriptor, mashup, subscription | semanticMashupJobProfile, AE, remoteCSE, CSEBase | 9.6.54 |
| mashup | A virtual resource use to trigger the calculation and generation of new mashup result | Not specified | semanticMashupInstance | 9.6.55 |
| semanticMashupResult | Represent semantic mashup results | semanticDescriptor, subscription | semanticMashupInstance | 9.6.56 |
| multimediaSession | Stores a representation of a multimedia session information requested by a registering AE | subscription | AE | 9.6.57 |
| crossResourceSubscription | represents the cross-resource subscription information related to multiple subscribed-to resources. Such a resource shall include a list of subscribed-to resources as its attribute, or shall be created as a child resource of a <group> resource where member resources shall be the subscribed-to resources | schedule, notificationTargetSelfReference, notificationTargetMgmtPolicyRef, transaction | CSEBase, remoteCSE, AE | 9.6.58 |
| backgroundDataTransfer | Stores information for a background data transfer request | None specified | AE, remoteCSE, CSEBase | 9.6.60 |
| action | Specifies the action(s) that is performed whenever an event is triggered at the <dependency> resource | subscription, transaction, dependency | CSEBase, remoteCSE, node, AE, container, flexContainer, group, timeSeries, transaction, state | 9.6.61 |
| dependency | Specifies the condition(s) of a monitored event which triggers the operation(s) specified by the <action> resource | subscription, transaction | action | 9.6.62 |
| e2eQosSession | Specifies the end-to-end (E2E) QoS session requirements for the exchange of oneM2M request and response primitives between oneM2M entities | subscription, transaction | CSEBase, remoteCSE, AE | 9.6.63 |
| nwMonitoringReq | Specifiesed the request that an AE retrieves the Underlying Network information. The resource provides the characteristics of the Underlying Network status in a particular geographic area such as congestion status and number of devices. | subscription | CSEBase, remoteCSE, AE | 9.6.64 |
| semanticRuleRepository | Specifies one or multiple <reasoningRules> child resources to represent different sets of reasoning rules in the oneM2M system | reasoningRules, reasoningJobInstance | CSEBase | 9.6.65 |
| reasoningRules | Store a set of related reasoning rules | subscription | semanticRuleRepository | 9.6.66 |
| reasoningJobInstance | Represents a specific reasoning job instance for enabling one-time or continuous reasoning operations | subscription | semanticRuleRepository | 9.6.67 |
| serviceSubscribedUserProfile | Stores user profile information for a given M2M Service User | subscription, transaction | m2mServiceSubscriptionProfile | 9.6.68 |
| timeSyncBeacon | Specifies criteria that a Hosting CSE uses to generate time synchronization beacon notifications | subscription, transaction | CSEBase, remoteCSE, AE | 9.6.69 |
| primitiveProfile | Stores message parameters and/or resource attributes that can be applied (i.e. added, removed or modified) to a request or a response primitive | subscription, transaction | CSEBase, remoteCSE, AE | 9.6.73 |
| state | Stores status information and actions for a given state of a process that is managed by a Hosting CSE | subscription, transaction, action | processManagement | 9.6.74 |
| processManagement | Specifies a process that a Hosting CSE manages on behalf of an AE that consists of sequence of states defined by <state> resources | state, subscription, transaction | CSEBase, remoteCSE, AE | 9.6.75 |
| softwareCampaign | Provides policies and rules for the dynamic management of a software by a Hosting CSE | subscription, transaction, schedule | CSEBase, remoteCSE, AE | 9.6.76 |
| advancedSemanticDiscovery (V) | Virtual resource containing target used to perform Advanced Semantic Discovery (See TS-0034 “Semantics_Support” [i.14]) | None specified | CSEBase | 9.6.77 |
| semanticRoutingTable | RDF summary of the resources on the Hosting CSE | subscription, transaction | CSEBase, CSEBaseAnnc | 9.6.78 |
| NOTE: See clause 9.6.15 for a summary of specializations of <mgmtObj>. |  |  |  |  |

### ********************* End of Change 1 *********************************

### **********************  Start of Change 2   *****************************

### 9.6.3	Resource Type CSEBase

A <CSEBase> resource shall represent a CSE. The <CSEBase> resource shall be the root for all resources that are residing in the CSE. A CSE shall be represented by only one <CSEBase> resource.

The <CSEBase> resource shall contain the child resources specified in table 9.6.3-1.

Table 9.6.3-1: Child resources of <CSEBase> resource

| Child Resources of <CSEBase> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <cseBaseAnnc> | 0..n | Announced variant of <cseBase>. Resource with CSE-specific information for a CSE that intends to announce resources to another CSE |
| [variable] | <remoteCSE> | 0..n | See clause 9.6.4 |
| [variable] | <remoteCSEAnnc> | 0..n | Announced variant of <remoteCSE>. Resource with CSE-specific information for a CSE that announced itself to another CSE with which it does not have a registration relationship |
| [variable] | <node> | 0..n | See clause 9.6.18 |
| [variable] | <AE> | 0..n | See clause 9.6.5 |
| [variable] | <container> | 0..n | See clause 9.6.6 |
| [variable] | <flexContainer> | 0..n | See clause 9.6.35 |
| [variable] | <group> | 0..n | See clause 9.6.13 |
| [variable] | <accessControlPolicy> | 0..n | See clause 9.6.2 |
| [variable] | <subscription> | 0..n | See clause 9.6.8 |
| [variable] | <mgmtCmd> | 0..n | See clause 9.6.16 |
| [variable] | <locationPolicy> | 0..n | See clause 9.6.10 |
| [variable] | <statsConfig> | 0..n | See clause 9.6.23 |
| [variable] | <statsCollect> | 0..n | See clause 9.6.25 |
| [variable] | <request> | 0..n | See clause 9.6.12 |
| [variable] | <delivery> | 0..n | See clause 9.6.11 |
| [variable] | <schedule> | 0..1 | This resource defines the reachability schedule information of the entity. The absence of this resource implies the entity is always reachable. See clause 9.6.9 |
| [variable] | <role> | 0..n | See clause 9.6.38 |
| [variable] | <token> | 0..n | See clause 9.6.39 |
| [variable] | <m2mServiceSubscriptionProfile> | 0..n | See clause 9.6.19 |
| [variable] | <serviceSubscribedAppRule> | 0..n | See clause 9.6.29 |
| [variable] | <notificationTargetPolicy> | 0..n | See clause 9.6.32 |
| [variable] | <dynamicAuthorizationConsultation> | 0..n | See clause 9.6.40 |
| [variable] | <timeSeries> | 0..n | See clause 9.6.36 |
| [variable] | <authorizationDecision> | 0..n | See clause 9.6.41 |
| [variable] | <authorizationPolicy> | 0..n | See clause 9.6.42 |
| [variable] | <authorizationInformation> | 0..n | See clause 9.6.43 |
| [variable] | <localMulticastGroup> | 0..n | See clause 9.6.44 |
| [variable] | <transactionMgmt> | 0..n | See clause 9.6.47 |
| [variable] | <transaction> | 0..n | See clause 9.6.48 |
| [variable] | <ontologyRepository> | 0..1 | See clause 9.6.50 |
| [variable] | <semanticMashupJobProfile> | 0..n | See clause 9.6.53 |
| [variable] | <semanticMashupInstance> | 0..n | See clause 9.6.54 |
| [variable] | <AEContactList> | 0..n | See clause 9.6.45 |
| [variable] | <e2eQosSession> | 0..1 | See clause 9.6.63 |
| [variable] | <nwMonitoringReq> | 0..n | See clause 9.6.64 |
| [variable] | <semanticRuleRepository> | 0..1 | See clause 9.6.65 |
| [variable] | <softwareCampaign> | 0..n | See clause 9.6.76 |
| [variable] | <action> | 0..n | See clause 9.6.61 |
| [variable] | <processManagement> | 0..n | See clause 9.6.75 |
| srt | <advancedSemanticDiscovery> | 0..1 | See clause 9.6.77 |
| [variable] | <semanticRoutingTable> | 0..n | See clause 9.6.78 |

The <CSEBase> resource shall contain the attributes specified in table 9.6.3-2.

Table 9.6.3-2: Attributes of <CSEBase> resource

| Attributes of <CSEBase> | Multiplicity | RW/ RO/ WO | Description | <CSEBaseAnnc> Attributes |
| --- | --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3. | NA |
| resourceID | 1 | RO | See clause 9.6.1.3. | NA |
| resourceName | 1 | RO | See clause 9.6.1.3. | NA |
| parentID | 1 | RO | See clause 9.6.1.3. Shall be an empty string. | NA |
| creationTime | 1 | RO | See clause 9.6.1.3. | NA |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3. | NA |
| accessControlPolicyIDs | 0..1 (L) | RO | See clause 9.6.1.3. | MA |
| labels | 0..1 (L) | RO | See clause 9.6.1.3. | MA |
| announceTo | 0..1 (L) | RO | See clause 9.6.1.3. | NA |
| announcedAttribute | 0..1 (L) | RO | See clause 9.6.1.3. | NA |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RO | See clause 9.6.1.3. | NA |
| custodian | 0..1 | RW | See clause 9.6.1.3. | NA |
| location | 0..1 | RW | See clause 9.6.1.3. | NA |
| cseType | 0..1 | RO | Indicates the type of CSE represented by the created resource: Mandatory for an IN-CSE, hence multiplicity (1). Its presence is subject to SP configuration in case of an ASN-CSE or a MN-CSE. | NA |
| CSE-ID | 1 | RO | The CSE identifier in SP-relative CSE-ID format (clause 7.2). | NA |
| supportedResourceType | 1 (L) | RO | List of the resource types which are supported in the CSE. This attribute contains subset of resource types listed in clause 9.2. | NA |
| pointOfAccess | 1 (L) | RO | Represents the list of physical addresses to be used by remote CSEs to connect to this CSE (e.g. IP address, FQDN). This attribute is exposed to its Registree. | NA |
| nodeLink | 0..1 | RO | The resource identifier of a <node> resource that stores the node specific information of the node on which the CSE represented by this <CSEBase> resource resides. | NA |
| notificationCongestionPolicy | 0..1 | RO | This attribute applies to CSEs generating subscription notifications. It specifies the rule which is applied when the storage of notifications for each subscriber (an AE or CSE) reaches the maximum storage limit for notifications for that subscriber. E.g. Delete stored notifications of lower notificationStoragePriority to make space for new notifications of higher notificationStoragePriority, or delete stored notifications of older creationTime to make space for new notifications when all notifications are of the same notificationStoragePriority. | NA |
| contentSerialization | 0..1 (L) | RO | The list of supported serializations of the Content primitive parameter for receiving a request from its registrants. (e.g. XML, JSON). The list shall be ordered so that the most preferred format comes first. | NA |
| e2eSecInfo | 0..1 | RO | See clause 9.6.1.3. | MA |
| supportedReleaseVersions | 0..1 (L) | RO | List of oneM2M release versions which are supported by the CSE. Starting with Release 2, this attribute is mandatory for a CSE. For CSEs compliant to older releases, this attribute is optional. For CSEs that do not include this attribute, the default release version shall be Release 1. | MA |
| currentTime | 0..1 | RO | When the CSE receives a retrieve request targeting this resource or attribute, the CSE samples its current time (e.g. makes an OS call to get the system time) and respond with the value in this attribute. An Originator retrieving this attribute can use this time value to adjust and synchronize its time value to the time value of this CSE. | NA |

### ********************* End of Change 2 *********************************

### **********************  Start of Change 3   *****************************

### 9.6.4	Resource Type remoteCSE

A <remoteCSE> resource shall represent a Registree CSE that is registered to the Registrar CSE. <remoteCSE> resources shall be located directly under the <CSEBase> resource of Registrar CSE.

Similarly <remoteCSE> resource shall also represent a Registrar CSE. <remoteCSE> resource shall be located directly under the <CSEBase> resource of Registree CSE.

For example, when CSE1 (Registree CSE) registers with CSE2 (Registrar CSE), there will be two <remoteCSE> resources created: one in CSE1: <CSEBase1>/<remoteCSE2> and one in CSE2: <CSEBase2>/<remoteCSE1>.

Note that the creation of the two resources does not imply mutual registration. The <CSEBase1>/<remoteCSE2> does not mean CSE2 registered with CSE1 in the example above.

The <remoteCSE> resource shall contain the child resources specified in table 9.6.4-1.

Table 9.6.4-1: Child resources of <remoteCSE> resource

| Child Resources of <remoteCSE> | Child Resource Type | Multiplicity | Description | <remoteCSEAnnc> Child Resource Types |
| --- | --- | --- | --- | --- |
| [variable] | <container> | 0..n | See clause 9.6.6 | <container> |
| [variable] | <containerAnnc> | 0..n | Announced variant of <container>. See clause 9.6.6 | <containerAnnc> |
| [variable] | <flexContainer> | 0..n | See clause 9.6.35 | <flexContainer> |
| [variable] | <flexContaineAnnc> | 0..n | Announced variant of <flexContainer>. See clause 9.6.35 | <flexContainerAnnc> |
| [variable] | <group> | 0..n | See clause 9.6.13 | <group> |
| [variable] | <groupAnnc> | 0..n | Announced variant of <group>. See clause 9.6.13 | <groupAnnc> |
| [variable] | <accessControlPolicy> | 0..n | See clause 9.6.2 | <accessControlPolicy> |
| [variable] | <accessControlPolicyAnnc> | 0..n | Announced variant of <accessControlPolicy>. See clause 9.6.2 | <accessControlPolicyAnnc> |
| [variable] | <subscription> | 0..n | See clause 9.6.8 | <subscription> |
| [variable] | <pollingChannel> | 0..1 | See clause 9.6.21. If requestReachability is FALSE, the CSE that created this <remoteCSE> resource should create a <pollingChannel> resource and perform long polling. The <pollingChannel> shall be utilized by the parent resource | None |
| [variable] | <nodeAnnc> | 0..n | Announced variant of <node>. This announced resource is associated with a <node> resource that is hosted on a CSE which is represented by the parent <remoteCSE> or <remoteCSEAnnc> resource. See clause 9.6.18 for <node> | <nodeAnnc> |
| [variable] | <dynamicAuthorizationConsultation> | 0..n | See clause 9.6.40 |  |
| [variable] | <timeSeries> | 0..n | See clause 9.6.36 | <timeSeries> |
| [variable] | <timeSeriesAnnc> | 0..n | Announced variant of <timeSeries>. See clause 9.6.36 | <timeSeriesAnnc> |
| [variable] | <AEAnnc> | 0..n | Announced variant of <AE>. See clause 9.6.5 | <AEAnnc> |
| [variable] | <locationPolicyAnnc> | 0..n | Announced variant of <locationPolicy>. See clause 9.6.10 | <locationPolicyAnnc> |
| [variable] | <transactionMgmt> | 0..n | See clause 9.6.47 | <transactionMgmt> |
| [variable] | <transaction> | 0..n | See clause 9.6.48 | <transaction> |
| [variable] | <ontologyRepositoryAnnc> | 0..1 | Announced variant of <ontologyRepository>. See clause 9.6.50 | <ontologyRepositoryAnnc> |
| [variable] | <ontologyMappingAnnc> | 0..n | Announced variant of <ontologyMapping>. See clause 9.6.70 | <ontologyMappingAnnc> |
| [variable] | <ontologyMappingAlgorithmRepositoryAnnc> | 0..n | Announced variant of <ontologyMappingAlgorithmRepository>. See clause 9.6.71 | <ontologyMappingAlgorithmRepositoryAnnc> |
| [variable] | <semanticMashupJobProfile> | 0..n | See clause 9.6.53 | <semanticMashupJobProfile> |
| [variable] | <semanticMashupJobProfileAnnc> | 0..n | Announced variant of <semanticMashupJobProfile>. See clause 9.6.53 | <semanticMashupJobProfileAnnc> |
| [variable] | <semanticMashupInstance> | 0..n | See clause 9.6.54 | <semanticMashupInstance> |
| [variable] | <semanticMashupInstanceAnnc> | 0..n | Announced variant of <semanticMashupInstance >. See clause 9.6.54 | <semanticMashupInstanceAnnc> |
| [variable] | <action> | 0..n | See clause 9.6.61 | <actionAnnc> |
| [variable] | <e2eQosSession> | 0..1 | See clause 9.6.63 | <e2eQosSession> |
| [variable] | <nwMonitoringReq> | 0..n | See clause 9.6.64 | <nwMonitoringReq> |
| [variable] | <processManagement> | 0..n | See clause 9.6.75 | <processManagement> |
| [variable] | <processManagementAnnc> | 0..n | Announced variant of <processManagement>. See clause 9.6.75 | <processManagementAnnc> |
| [variable] | <softwareCampaign> | 0..n | See clause 9.6.76 | <softwareCampaignAnnc> |
| [variable] | <semanticRoutingTable> | 0..n | See clause 9.6.78 | <semanticRoutingTableAnnc> |

The <remoteCSE> resource shall contain the attributes specified in table 9.6.4-2.

Table 9.6.4-2: Attributes of <remoteCSE> resource

| Attributes of <remoteCSE> | Multiplicity | RW/ RO/ WO | Description | <remoteCSEAnnc> Attributes |
| --- | --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3. | NA |
| resourceID | 1 | RO | See clause 9.6.1.3. | NA |
| resourceName | 1 | WO | See clause 9.6.1.3. | NA |
| parentID | 1 | RO | See clause 9.6.1.3. | NA |
| creationTime | 1 | RO | See clause 9.6.1.3. | NA |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3. | NA |
| expirationTime | 1 | RW | See clause 9.6.1.3. | MA |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3. | MA |
| labels | 0..1 (L) | RW | See clause 9.6.1.3. | MA |
| announceTo | 0..1 (L) | RW | See clause 9.6.1.3. | NA |
| announcedAttribute | 0..1 (L) | RW | See clause 9.6.1.3. | NA |
| announceSyncType | 0..1 | RW | See clause 9.6.1.3. | MA |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See clause 9.6.1.3. | OA |
| location | 0..1 | RW | See clause 9.6.1.3. | OA |
| custodian | 0..1 | RW | See clause 9.6.1.3. | NA |
| cseType | 0..1 | WO | Indicates the type of CSE represented by the created resource. Mandatory for an IN-CSE, hence multiplicity (1). Its presence is subject to SP configuration in case of an ASN-CSE or a MN-CSE. | OA |
| pointOfAccess | 0..1 (L) | RW | For request-reachable remote CSE it represents the list of physical addresses to be used to connect to it (e.g. IP address, FQDN). If this information is not provided and <pollingChannel> resource does exist, the CSE should use <pollingChannel> resource. Then the Hosting CSE can forward a request to the CSE without using the PoA. | OA |
| CSEBase | 1 | WO | The address of the <CSEBase> resource represented by this <remoteCSE> resource. | OA |
| CSE-ID | 1 | WO | The CSE identifier of the remote CSE represented by this <remoteCSE> resource (clause 7.2). | OA |
| M2M-Ext-ID | 0..1 | RW | See clause 7.1.8 where this attribute is described. This attribute is used only for the case of dynamic association of M2M-Ext-ID and CSE-ID. | NA |
| Trigger-Recipient-ID | 0..1 | RW | See clause 7.1.10 where this attribute is described. This attribute is used only for the case of dynamic association of M2M-Ext-ID and CSE-ID. | NA |
| requestReachability | 1 | RW | This attribute is an indication of static capability of the CSE that created this <remoteCSE> resource. If the CSE can receive requests originated at or forwarded by its registrar CSE, this attribute is set to "TRUE" otherwise "FALSE" (see note 1). | OA |
| nodeLink | 0..1 | RW | The resource identifier of a <node> resource that stores the node specific information of the node on which the CSE represented by this <remoteCSE> resource resides. | OA |
| contentSerialization | 0..1 (L) | RW | The list of supported serializations of the Content primitive parameter for receiving a request (e.g. XML, JSON). The list shall be ordered so that the most preferred format comes first. | OA |
| e2eSecInfo | 0..1 | RW | See clause 9.6.1.3. | MA |
| triggerReferenceNumber | 0..1 | RW | This is to identify device trigger procedure request. This attribute is used only for device trigger and assigned by the CSE. | NA |
| descendantCSEs | 0..1(L) | RW | This attribute contains a list of identifiers of descendent CSEs of the Registree CSE represented by this <remoteCSE> resource. A descendant CSE is a CSE that either registers to the CSE represented by this <remoteCSE>, or registers to another CSE which is a descendant CSE of this <remoteCSE>. The Registree CSE represented by this <remoteCSE> shall configure this attribute with a list of descendent CSEs upon creation of the <remoteCSE> resource. The Registree CSE shall update this attribute whenever a new descendent CSE either registers or de-registers. The Registree CSE shall detect when a descendent CSE registers or de-registers by monitoring its <remoteCSE> resources and the descendent CSEs attribute(s) of these <remoteCSE> resources. For a <remoteCSE> resource representing a Registrar CSE this attribute shall not be set. | OA |
| multicastCapability | 0..1 | RW | Indicates the oneM2M node multicast Capability, pre-defined values are: - MBMS. - IP. | OA |
| externalGroupID | 0..1 | RW | It is used by an M2M Service Provider (M2M SP) when services targeted to a group of M2M Devices are requested from the Underlying Network. It is assumed to be a globally unique ID exposed by the underlying network to identify a group of M2M Devices (e.g. ASN, MN) for group related services. | OA |
| triggerEnable | 0..1 | RW | When set to "TRUE", trigger requests may be sent to the CSE represented by this <remoteCSE> resource. When set to "FALSE" trigger requests shall not be sent to this CSE. | OA |
| activityPatternElements | 0..1(L) | RW | This attribute describes the anticipated availability of the CSE for communications. See further description below and table 9.6.4-3. | OA |
| supportedReleaseVersions | 0..1(L) | RW | The oneM2M release versions supported by the CSE represented by this <remoteCSE> resource. Starting with Release 2, this attribute is mandatory for a CSE. For CSEs compliant to older releases, this attribute is optional. For CSEs that do not include this attribute, the default release version shall be Release 1. | MA |
| enableTimeCompensation | 0..1 | RW | Enables time offset compensation functionality. When set to "TRUE", the Registrar CSE peforms time offset compensation for the Registree CSE. If "FALSE", the Registrar CSE does not perform time offset compensation. See clause 10.2.24. Default value is "FALSE". | NA |
| NOTE-1: Even if this attribute is set to "FALSE", it is not meant that the CSE is always unreachable by its registrees. E.g. if the CSE and its registrees are behind the same NAT, then the CSE can receive requests from its registrees. See also pollingChannel description in clause 9.6.21. NOTE-2: For the case of a response, this attribute is applicable if the corresponding request does not contain the serialization format of the Content request parameter to allow a CSE to determine the proper serialization format to use in the response. |  |  |  |  |

The set of activity patterns represented in the activityPatternElements attribute describes the anticipated availability of the CSE for communications. The set provides the anticipated activity timing pattern, and may provide additional information about the anticipated mobility status and expected data size to be exchanged. Each activityPatternElements item is comprised of triples (scheduleElement, stationaryIndication, dataSizeIndicator) with parameters shown and described in table 9.6.4-3.

Table 9.6.4-3: Parameters in activityPatternElements triple

| Name | Description |
| --- | --- |
| scheduleElement | See clause 9.6.9. This parameter shall be composed from seven fields of second, minute, hour, day of month, month, day of week and year. This is a mandatory parameter in the triple. This parameter indicates the times when the entity is available to send and receive primitives. |
| stationaryIndication | It indicates the field node as 'Stationary (Stopping)' or 'Mobile (Moving)' for the traffic pattern. The default value is NULL, denoting that no stationaryIndication is provided. |
| dataSizeIndicator | It indicates the expected data size for the traffic pattern. The default value is NULL, denoting that no dataSizeIndicator is provided. |

### ********************* End of Change 3 *********************************

### **********************  Start of Change 4   *****************************

### 9.6.5	Resource Type AE

An <AE> resource shall represent information about an Application Entity registered to a CSE.

The <AE> resource shall contain the child resources specified in table 9.6.5-1.

Table 9.6.5-1: Child resources of <AE> resource

| Child Resources of <AE> | Child Resource Type | Multiplicity | Description | <AEAnnc> Child Resource Types |
| --- | --- | --- | --- | --- |
| [variable] | <semanticDescriptor> | 0..n | See clause 9.6.30 | <semanticDescriptor>, <semanticDescriptorAnnc> |
| [variable] | <subscription> | 0..n | See clause 9.6.8 | <subscription> |
| [variable] | <container> | 0..n | See clause 9.6.6 | <container> <containerAnnc> |
| [variable] | <flexContainer> | 0..n | See clause 9.6.35 | <flexContainer> <flexContainerAnnc> |
| [variable] | <group> | 0..n | See clause 9.6.13 | <group> <groupAnnc> |
| [variable] | <accessControlPolicy> | 0..n | See clause 9.6.2 | <accessControlPolicy> <accessControlPolicyAnnc> |
| [variable] | <pollingChannel> | 0..1 | See clause 9.6.21 When the AE is request-unreachable, the AE should create this <pollingChannel> resource and perform long polling. The <pollingChannel> shall be utilized by the parent resource | None |
| [variable] | <dynamicAuthorizationConsultation> | 0..n | See clause 9.6.40 | None |
| [variable] | <timeSeries> | 0..n | See clause 9.6.36 | <timeSeries> <timeSeriesAnnc> |
| [variable] | <transactionMgmt> | 0..n | See clause 9.6.47 | <transactionMgmt> |
| [variable] | <transaction> | 0..n | See clause 9.6.48 | <transaction> |
| [variable] | <triggerRequest> | 0..n | See clause 9.6.49 | None |
| [variable] | <multimediaSession> | 0..n | See Clause 9.6.57. This resource holds information describing the established multimedia session | None |
| [variable] | <semanticMashupInstance> | 0..n | See clause 9.6.54 | <semanticMashupInstance> <semanticMashupInstanceAnnc> |
| [variable] | <locationPolicy> | 0..n | See clause 9.6.10 | <locationPolicyAnnc> |
| [variable] | <action> | 0..n | See clause 9.6.61 | <actionAnnc> |
| [variable] | <e2eQosSession> | 0..1 | See clause 9.6.63 | <e2eQosSession> |
| [variable] | <nwMonitoringReq> | 0..n | See clause 9.6.64 | <nwMonitoringReq > |
| [variable] | <processManagement> | 0..n | See clause 9.6.75 | <processManagementAnnc> |
| [variable] | <softwareCampaign> | 0..n | See clause 9.6.76 | <softwareCampaignAnnc> |

The <AE> resource shall contain the attributes specified in table 9.6.5-2.

Table 9.6.5-2: Attributes of <AE> resource

| Attributes of <AE> | Multiplicity | RW/ RO/ WO | Description | <AEAnnc> Attributes |
| --- | --- | --- | --- | --- |
| resourceType | 1 | RO | See clause 9.6.1.3. | NA |
| resourceID | 1 | RO | See clause 9.6.1.3. Contains the AE-ID-Stem of the AE (see clause 7.2 on identifier formats and clause 10.2.2.2 for AE registration procedure). | NA |
| resourceName | 1 | WO | See clause 9.6.1.3. | NA |
| parentID | 1 | RO | See clause 9.6.1.3. | NA |
| expirationTime | 1 | RW | See clause 9.6.1.3. | MA |
| accessControlPolicyIDs | 0..1 (L) | RW | See clause 9.6.1.3. | MA |
| creationTime | 1 | RO | See clause 9.6.1.3. | NA |
| lastModifiedTime | 1 | RO | See clause 9.6.1.3. | NA |
| labels | 0..1 (L) | RW | See clause 9.6.1.3. | MA |
| announceTo | 0..1 (L) | RW | See clause 9.6.1.3. | NA |
| announcedAttribute | 0..1 (L) | RW | See clause 9.6.1.3. | NA |
| announceSyncType | 0..1 | RW | See clause 9.6.1.3. | MA |
| dynamicAuthorizationConsultationIDs | 0..1 (L) | RW | See clause 9.6.1.3. | OA |
| location | 0..1 | RW | See clause 9.6.1.3. | OA |
| custodian | 0..1 | RW | See clause 9.6.1.3. | NA |
| appName | 0..1 | RW | The name of the application, as declared by the application developer (e.g. "HeatingMonitoring"). Several sibling resources may share the appName. | OA |
| App-ID | 1 | WO | The identifier of the Application (see clause 7.1.3). | OA |
| AE-ID | 1 | RO | The identifier of the Application Entity (see clause 7.1.2). | OA |
| M2M-Ext-ID | 0..1 | RW | See clause 7.1.8 where this attribute is described. This attribute is used only for the case of dynamic association of M2M-Ext-ID and AE-ID. | NA |
| trigger-Recipient-ID | 0..1 | RW | See clause 7.1.10 where this attribute is described. This attribute is used only for the case of dynamic association of M2M-Ext-ID and AE-ID. | NA |
| triggerReferenceNumber | 0..1 | RW | This is to identify device trigger procedure request. This attribute is used only for device trigger and assigned by the CSE. | NA |
| pointOfAccess | 0..1 (L) | RW | The list of addresses for communicating with the registered Application Entity over Mca reference point via the transport services provided by Underlying Network (e.g. IP address, FQDN, URI). This attribute shall be accessible only by the AE and the Hosting CSE. If this information is not provided and the <pollingChannel> resource does exist, the AE should use <pollingChannel> resource. Then the Hosting CSE can forward a request to the AE without using the PoA. | OA |
| registrationStatus | 0..1 | RW | Denotes status of the AE registration. If ACTIVE, the <AE> resource and all its child resources may be discoverable. If INACTIVE, the <AE> resource and all its child resources shall not be discoverable. Set to ACTIVE during an AE registration or re-registration. When an AE changes its registration point, the registration at the old registration point is set to INACTIVE. | OA |
| trackRegistrationPoints | 0..1 | RW | Denotes if the Application Entity requests that its Registration Points be tracked. If TRUE, AE requests to be tracked as it changes its Registration Points. If FALSE, the AE requests not to be tracked as it changes its Registration Points. | OA |
| ontologyRef | 0..1 | RW | A URI of the ontology used to represent the information that is managed and understood by the AE. | OA |
| requestReachability | 1 | RW | This attribute is an indication of static capability of the AE that created this <AE> resource. If the AE can receive requests originated at or forwarded by its registrar CSE, this attribute is set to "TRUE" otherwise "FALSE". | OA |
| nodeLink | 0..1 | RW | The resource identifier of a <node> resource that stores the node specific information of the node on which the AE represented by this <AE> resource resides. | OA |
| contentSerialization | 0..1 (L) | RW | The list of supported serializations of the Content primitive parameter for receiving a request and a response from its registrar CSE. (e.g. XML, JSON, CBOR). The list shall be ordered so that the most preferred format comes first. | OA |
| e2eSecInfo | 0..1 | RW | See clause 9.6.1.3. | MA |
| activityPatternElements | 0..1(L) | RW | This attribute describes the anticipated availability of the AE for communications. See further description below and table 9.6.4-3. | OA |
| triggerEnable | 0..1 | RW | When set to "TRUE", trigger requests may be sent to the AE represented by this <AE> resource. When set to "FALSE" trigger requests shall not be sent to this AE. | OA |
| sessionCapabilities | 0..1 (L) | RW | The list of supported session media types (e.g. audio, video, image) and supported session protocols (e.g. RTP, RTP/AVP) as defined by session parameters as defined by the IETF IANA Session Descriptor Protocol (SDP) Parameter Registry. | OA |
| supportedReleaseVersions | 0..1(L) | RW | The oneM2M release versions supported by the Registree AE represented by this <AE> resource. Starting with Release 2, this attribute is mandatory for an AE. For AEs compliant to older releases, this attribute is optional. For AEs that do not include this attribute, the default release version shall be Release 1. | MA |
| externalGroupID | 0..1 | RW | It is used by an M2M Service Provider (M2M SP) when services targeted to a group of M2M Devices are requested from the Underlying Network. It is assumed to be a globally unique ID exposed by the underlying network to identify a group of M2M Devices (e.g. ADN, ASN, MN) for group related services. | OA |
| enableTimeCompensation | 0..1 | RW | Enables time offset compensation functionality. When set to "TRUE", the Registrar CSE peforms time offset compensation for the Registree AE. If "FALSE", the Registrar CSE does not perform time offset compensation. See clause 10.2.24. Default value is "FALSE". | NA |
| NOTE: For the case of a response, this attribute is applicable if the corresponding request does not contain the serialization format of the Content request parameter to allow a CSE to determine the proper serialization format to use in the response. |  |  |  |  |

The set of activity patterns represented in the activityPatternElements attribute describes the anticipated availability of the AE for communications. The set provides the anticipated activity timing pattern and might provide additional information about the anticipated mobility status and expected data size to be exchanged. Each activityPatternElements item is comprised of triples (scheduleElement, stationaryIndication, datasizeIndicator) with parameters shown and described in table 9.6.4-3.

### ********************* End of Change 4 *********************************