| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | RDM#72.2 |
| Source:* | Andreas Kraft |
| Date:* | 2026-01-14 |
| Reason for Change/s:* | Updating short names |
| CR against: Release* | R5 |
| CR against: WI* | Active <#WI#> MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0023 |
| Clauses * | 5.3.1.82, 6.3.2, 6.3.3 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Other TS/TR(s) impacted | None |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2019 (do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

Updating Shortnames

https://git.onem2m.org/specifications/ts/ts-0023/-/merge_requests/69

https://git.onem2m.org/specifications/ts/ts-0023/-/merge_requests/69/diffs?commit_id=fbc880706a700dfb8a4770c42362785e7e6988fe

========= Comment provided by Andreas Kraft at 2026-01-14T12:11:47.700Z: =========

After this CR was accepted another CR will be created that proposes to change the "smokeSensor" (5.3.1.82) attribute "sensorFault" to "smokeSensorSensorFault". This aligns the name of this attribute with similar "...SensorFault" attribute.

## --------------------- Start of change 1 ---------------------

TS-0023-SDT_based_Information_Model_and_Mapping_for_Vertical_Industries.md

---a/TS-0023-SDT_based_Information_Model_and_Mapping_for_Vertical_Industries.md
+++b/TS-0023-SDT_based_Information_Model_and_Mapping_for_Vertical_Industries.md

@@ -1586,6 +1586,8 @@ This ModuleClass provides the capabilities to indicate the detection of smoke an

1586	1586		|none |muteSmokeSensor |none |true |Mute the smoke sensor alarm. |

1587	1587		|none |test |none |true |Testing the alarm |

1588	1588

1589	+	<mark>Editor Note: Perhaps change the name test (too generic).</mark>

1590	+

1589	1591

1590	1592		**Table 5.3.1.82-2: DataPoints of smokeSensor ModuleClass**

1591	1593

@@ -5655,6 +5657,7 @@ In protocol bindings resource type names for module classes shall be translated

5655	5657		|[binaryObject](#53111-binaryobject) |binOt |

5656	5658		|[binarySwitch](#53112-binaryswitch) |binSh |

5657	5659		|[bioElectricalImpedanceAnalysis](#53113-bioelectricalimpedanceanalysis) |bEIAs |

5660	+	|[blender](#531118-blender) |blenr |

5658	5661		|[bodyCompositionAnalyser](#53114-bodycompositionanalyser) |boCAr |

5659	5662		|[boiler](#53115-boiler) |boilr |

5660	5663		|[boilingSwitch](#53112-binaryswitch) |boiSh |

@@ -5676,7 +5679,6 @@ In protocol bindings resource type names for module classes shall be translated

5676	5679		|[cookerHoodJobMode](#53126-cookerhoodjobmode) |cHJMe |

5677	5680		|[credentials](#53127-credentials) |creds |

5678	5681		|[customTemperature](#53187-temperature) |cusTe |

5679		-	|dataGenerationTime |dgt |

5680	5682		|[dehumidifierJobMode](#53128-dehumidifierjobmode) |deJMe |

5681	5683		|[dehumidifierOperationMode](#53157-operationmode) |deOMe |

5682	5684		|[dishWasherJobMode](#53129-dishwasherjobmode) |dWJMe |

@@ -5693,6 +5695,7 @@ In protocol bindings resource type names for module classes shall be translated

5693	5695		|[doorLock](#53152-lock) |dooLk |

5694	5696		|[doorStatus](#53130-doorstatus) |dooSs |

5695	5697		|[electricVehicleConnector](#53131-electricvehicleconnector) |elVCr |

5698	+	|[emergencyHandler](#53102-emergencyhandler) |emeHr |

5696	5699		|[energyConsumption](#53132-energyconsumption) |eneCn |

5697	5700		|[energyGeneration](#53133-energygeneration) |eneGn |

5698	5701		|[faultDetection](#53134-faultdetection) |fauDn |

@@ -5702,12 +5705,18 @@ In protocol bindings resource type names for module classes shall be translated

5702	5705		|[fridgeTemperature](#53187-temperature) |friTe |

5703	5706		|[frozenTemperature](#53187-temperature) |froTe |

5704	5707		|[geoLocation](#53141-geolocation) |geoLn |

5708	+	|[galleryMode](#53137-gallerymode) |galMe |

5709	+	|[gasChargingControl](#53138-gaschargingcontrol) |gaCCl |

5710	+	|[gasMeterAlarm](#53139-gasmeteralarm) |gaMAm |

5711	+	|[gasMeterReportInfo](#53140-gasmeterreportinfo) |gMRIo |

5705	5712		|[glucometer](#53142-glucometer) |glucr |

5706	5713		|[grinder](#53143-grinder) |grinr |

5714	+	|[headingSensor](#531113-headingsensor) |heaSr |

5707	5715		|[heatingZone](#53144-heatingzone) |heaZe |

5708	5716		|[height](#53145-height) |heigt |

5709	5717		|[hotWaterSupply](#53146-hotwatersupply) |hoWSy |

5710	5718		|[impactSensor](#53147-impactsensor) |impSr |

5719	+	|[infraredSensor](#531103-infraredsensor) |infSr |

5711	5720		|[keepWarm](#53148-keepwarm) |keeWm |

5712	5721		|[keypad](#53149-keypad) |keypd |

5713	5722		|[liquidLevel](#53150-liquidlevel) |liqLl |

@@ -5715,6 +5724,7 @@ In protocol bindings resource type names for module classes shall be translated

5715	5724		|[location](#5392-location) |locan |

5716	5725		|[localization](#5393-localization) |loca0 |

5717	5726		|[lock](#53152-lock) |lock |

5727	+	|[magneticSensorParameters](#53161-magneticsensorparameters) |maSPs |

5718	5728		|[mediaInput](#53153-mediaselect) |medIt |

5719	5729		|[mediaOutput](#53153-mediaselect) |medOt |

5720	5730		|[mediaSelect](#53153-mediaselect) |medSt |

@@ -5729,13 +5739,16 @@ In protocol bindings resource type names for module classes shall be translated

5729	5739		|[overcurrentSensor](#53158-overcurrentsensor) |oveSr |

5730	5740		|[oximeter](#53159-oximeter) |oximr |

5731	5741		|[ozoneMeter](#53160-ozonemeter) |ozoMr |

5742	+	|[periodicalReportConfig](#53163-periodicalreportconfig) |peRCg |

5732	5743		|[phoneCall](#53164-phonecall) |phoCl |

5733	5744		|[playerControl](#53165-playercontrol) |plaCl |

5734	5745		|[powerSave](#53166-powersave) |powS0 |

5746	+	|[prePaidCardReader](#531107-prepaidcardreader) |pPCRr |

5735	5747		|[printerRunState](#53175-runstate) |prRSe |

5736	5748		|[printQueue](#53167-printqueue) |priQe |

5737	5749		|[pulsemeter](#53168-pulsemeter) |pulsr |

5738	5750		|[pushButton](#53169-pushbutton) |pusBn |

5751	+	|[rainGauge](#531102-raingauge) |raiGe |

5739	5752		|[recorder](#53170-recorder) |recor |

5740	5753		|[refrigeration](#53171-refrigeration) |refrn |

5741	5754		|[relativeHumidity](#53172-relativehumidity) |relHy |

@@ -5746,7 +5759,13 @@ In protocol bindings resource type names for module classes shall be translated

5746	5759		|[scannerRunState](#53175-runstate) |scRSe |

5747	5760		|[securityMode](#53176-securitymode) |secMe |

5748	5761		|[sessionDescription](#53177-sessiondescription) |sesDn |

5762	+	|[settings](#53103-settings) |setts |

5763	+	|[shoesWasherJobMode](#531119-shoeswasherjobmode)| sWJMe |

5764	+	|[signalPanel](#531114-signalpanel) |sigPl |

5749	5765		|[signalStrength](#53178-signalstrength) |sigSh |

5766	+	|[slcAlarm](#53179-slcalarm) |slcAm |

5767	+	|[slcParameterSetting](#53180-slcparametersetting) |slPSg |

5768	+	|[slcReportInfo](#53181-slcreportinfo) |slRIo |

5750	5769		|[sleepTimer](#53190-timer) |sleTr |

5751	5770		|[smokeSensor](#53182-smokesensor) |smoSr |

5752	5771		|[sphygmomanometer](#53183-sphygmomanometer) |sphyr |

@@ -5757,11 +5776,19 @@ In protocol bindings resource type names for module classes shall be translated

5757	5776		|[temperature](#53187-temperature) |tempe |

5758	5777		|[temperatureAlarm](#53188-temperaturealarm) |temAm |

5759	5778		|[textMessage](#53189-textmessage) |texMe |

5779	+	|[threeDDisplay](#531116-threeddisplay) |thDDy |

5780	+	|[threeDScanner](#531117-threedscanner) |thDSr |

5760	5781		|[timer](#53190-timer) |timer |

5782	+	|[touchScreen](#531106-touchscreen) |touSn |

5761	5783		|[turbo](#53191-turbo) |turbo |

5762	5784		|[uvSensor](#53192-uvsensor) |uveSr |

5763	5785		|[waterFilterInfo](#53135-filterinfo) |waFIo |

5786	+	|[waterFilterType](#531105-waterfiltertype) |waFTe |

5764	5787		|[waterFlow](#53193-waterflow) |watFw |

5788	+	|[waterMeterAlarm](#53194-watermeteralarm)|waMAm |

5789	+	|[waterQualityMonitor](#53198-waterqualitymonitor) |waQMr |

5790	+	|[waterMeterReportInfo](#53195-watermeterreportinfo) |wMRIo |

5791	+	|[waterMeterSetting](#53196-watermetersetting) |waMSg |

5765	5792		|[waterSensor](#53197-watersensor) |watSr |

5766	5793		|[waterStatus](#53151-liquidremaining) |watSs |

5767	5794		|[weight](#53199-weight) |weigt |

@@ -5790,6 +5817,7 @@ In protocol bindings resource type names for actions shall be translated into sh

5790	5817		|[hangup](#53164-phonecall) |hangp |

5791	5818		|[incrementNumberValue](#53155-numbervalue) |inNVe |

5792	5819		|[install](#588-dmpackage) |instl |

5820	+	|[muteSmokeSensor](#53182-smokesensor) |muSSr |

5793	5821		|[nextTrack](#53165-playercontrol) |nexTk |

5794	5822		|[open](#53156-openlevel) |open |

5795	5823		|[previousTrack](#53165-playercontrol) |preTk |

@@ -5801,6 +5829,7 @@ In protocol bindings resource type names for actions shall be translated into sh

5801	5829		|[retrieveLog](#587-dmeventlog)|reLog |

5802	5830		|[start3Dprint](#5311-threedprinter) |staDt |

5803	5831		|[stop3Dprint](#5311-threedprinter) |stoDt |

5832	+	|[test](#53182-smokesensor) |test |

5804	5833		|[toggle](#585-dmfirmware) |togge |

5805	5834		|[uninstall](#588-dmpackage) |uninl |

5806	5835		|[unmount](#5812-dmstorage) |unmot |

@@ -5866,6 +5895,7 @@ In protocol bindings resource attributes names for data points of module classes

5866	5895		|blue |[colour](#53123-colour) |blue |

5867	5896		|bmi |[bodyCompositionAnalyser](#53114-bodycompositionanalyser) |bmi |

5868	5897		|bodyLength |[bodyCompositionAnalyser](#53114-bodycompositionanalyser) |bodLh |

5898	+	|bodyWaterMass |[bodyCompositionAnalyser](#53114-bodycompositionanalyser) |boWMs |

5869	5899		|bone |[bioElectricalImpedanceAnalysis](#53113-bioelectricalimpedanceanalysis) |bone |

5870	5900		|brightness |[brightness](#53117-brightness) |brigs |

5871	5901		|buttonFault |[waterMeterAlarm](#53194-watermeteralarm)|butFt |

@@ -5932,6 +5962,7 @@ In protocol bindings resource attributes names for data points of module classes

5932	5962		|dailyActivityTime |[connectivity](#53125-connectivity) |daATe |

5933	5963		|dailyNumberOfConnections |[connectivity](#53125-connectivity) |dNOCs |

5934	5964		|data |[dmEventLog](#587-dmeventlog) |data |

5965	+	|dataGenerationTime |dgt |

5935	5966		|dataModelType |[dmDataModelIO](#584-dmdatamodelio) |daMTe |

5936	5967		|dataSourceID |[origin](#5394-origin) |daSID |

5937	5968		|dataType |[origin](#5394-origin) |datTe |

@@ -5975,12 +6006,13 @@ In protocol bindings resource attributes names for data points of module classes

5975	6006		|filterLifetime |[filterInfo](#53135-filterinfo) |filLe |

5976	6007		|filterType |[waterFilterType](#531105-waterfiltertype) |filTe |

5977	6008		|fileSystem |[dmStorage](#5812-dmstorage) |filSm |

6009	+	|flexNodeLink |flNLk|

5978	6010		|flowInterval |[waterMeterSetting](#53196-watermetersetting) |floIl |

5979	6011		|foamingStrength |[foaming](#53136-foaming) |foaSh |

5980	6012		|forcedUnmounts |[dmStorage](#5812-dmstorage) |forUs |

5981	6013		|frequency |[energyConsumption](#53132-energyconsumption) |freqy |

5982		-	|friendlyName |[dmDeviceInfo](#583-dmdeviceinfo), [localization](#5393-localization) |friNe |

5983	6014		|friendlyLocation |[location](#5392-location), [localization](#5393-localization) |friLn |

6015	+	|friendlyName |[dmDeviceInfo](#583-dmdeviceinfo), [localization](#5393-localization) |friNe |

5984	6016		|ftu |[waterQualityMonitor](#53198-waterqualitymonitor) |ftu |

5985	6017		|fwVersion |[dmDeviceInfo](#583-dmdeviceinfo)	 |fweVn |

5986	6018		|gaugeHeight |[rainGauge](#531102-raingauge) |gauHt |

@@ -5995,10 +6027,10 @@ In protocol bindings resource attributes names for data points of module classes

5995	6027		|heatingLevel |[heatingZone](#53144-heatingzone) |heaLl |

5996	6028		|height |[height](#53145-height) |heigt |

5997	6029		|hg |[waterQualityMonitor](#53198-waterqualitymonitor) |hg |

5998		-	|highMagneticAlarm |[magneticSensorParameters](#53161-magneticsensorparameters)|hiMAm |

5999	6030		|highFlowAlarm |[waterMeterAlarm](#53194-watermeteralarm)|hiFAm |

6000	6031		|highFlowAlarmThreshold |[waterMeterAlarm](#53194-watermeteralarm)|hFATd |

6001	6032		|highFlowDuration |[waterMeterAlarm](#53194-watermeteralarm)|hiFDn |

6033	+	|highMagneticAlarm |[magneticSensorParameters](#53161-magneticsensorparameters)|hiMAm |

6002	6034		|highPressureAlarm |[waterMeterAlarm](#53194-watermeteralarm)|hiPAm |

6003	6035		|highPressureAlarmThreshold |[waterMeterAlarm](#53194-watermeteralarm)|hPATd |

6004	6036		|highTemperatureAlarm |[temperatureAlarm](#53188-temperaturealarm), [waterMeterAlarm](#53194-watermeteralarm)|hiTAm |

@@ -6020,6 +6052,7 @@ In protocol bindings resource attributes names for data points of module classes

6020	6052		|innerErrorAlarm |[waterMeterAlarm](#53194-watermeteralarm)|inEAm |

6021	6053		|innerHighInternalTemperatureAlarm |[waterMeterAlarm](#53194-watermeteralarm)|iHITm |

6022	6054		|innerTemperatureSensorFault |[waterMeterAlarm](#53194-watermeteralarm)|iTSFt |

6055	+	|inorganicSaltMass |[bodyCompositionAnalyser](#53114-bodycompositionanalyser) |inSMs |

6023	6056		|intensiveReportInterval |[waterMeterSetting](#53196-watermetersetting) |inRIl |

6024	6057		|intensiveReportStartTime |[waterMeterSetting](#53196-watermetersetting) |iRSTe |

6025	6058		|intensiveSampleInterval |[waterMeterSetting](#53196-watermetersetting) |inSIl |

@@ -6038,7 +6071,7 @@ In protocol bindings resource attributes names for data points of module classes

6038	6071		|lightActivePower |[slcReportInfo](#53181-slcreportinfo) |LiAPr |

6039	6072		|lightCount |[slcParameterSetting](#53180-slcparametersetting) |ligCt |

6040	6073		|lightCurrent |[slcReportInfo](#53181-slcreportinfo) |ligC0 |

6041		-	|lightApparentPower |[slcReportInfo](#53181-slcreportinfo) |liAPr |

6074	+	|lightApparentPower |[slcReportInfo](#53181-slcreportinfo) |liAP0 |

6042	6075		|lightDimmingValue |[slcReportInfo](#53181-slcreportinfo) |liDVe |

6043	6076		|lightPolarizationAxis |[slcReportInfo](#53181-slcreportinfo) |liPAs |

6044	6077		|lightPowerFactor |[slcReportInfo](#53181-slcreportinfo) |liPFr |

@@ -6065,7 +6098,8 @@ In protocol bindings resource attributes names for data points of module classes

6065	6098		|lowPressureAlarm |[waterMeterAlarm](#53194-watermeteralarm)|LoPAm |

6066	6099		|lowPressureAlarmThreshold |[waterMeterAlarm](#53194-watermeteralarm)|lPATd |

6067	6100		|lowTemperatureAlarm |[temperatureAlarm](#53188-temperaturealarm), [waterMeterAlarm](#53194-watermeteralarm)|loTAm |

6068		-	|lowTemperatureAlarmThreshold |[temperatureAlarm](#53188-temperaturealarm), [waterMeterAlarm](#53194-watermeteralarm)|LTATd |

6101	+	|lowTemperatureAlarmThreshold |[temperatureAlarm](#53188-temperaturealarm), [waterMeterAlarm](#53194-watermeteralarm)|lTATd |

6102	+	|lowVoltage |[smokeSensor](#53182-smokesensor)|lowVe |

6069	6103		|lqi |[signalStrength](#53178-signalstrength) |lqi |

6070	6104		|machineStates |[runState](#53175-runstate) |macSs |

6071	6105		|magneticDisturb |[gasMeterAlarm](#53139-gasmeteralarm) |magDb |

@@ -6074,6 +6108,7 @@ In protocol bindings resource attributes names for data points of module classes

6074	6108		|manufacturer |[dmDeviceInfo](#583-dmdeviceinfo) |manur |

6075	6109		|manufacturerDetailsLink |[dmDeviceInfo](#583-dmdeviceinfo) |maDLk |

6076	6110		|manufacturingDate |[dmDeviceInfo](#583-dmdeviceinfo) |manDe |

6111	+	|mass |[bioElectricalImpedanceAnalysis](#53113-bioelectricalimpedanceanalysis) |mass |

6077	6112		|material |[battery](#53110-battery) |matel |

6078	6113		|maxHeatingLevel |[heatingZone](#53144-heatingzone) |maHLl |

6079	6114		|maxLength |[textMessage](#53189-textmessage) |maxLh |

@@ -6111,11 +6146,13 @@ In protocol bindings resource attributes names for data points of module classes

6111	6146		|muscleMass |[bodyCompositionAnalyser](#53114-bodycompositionanalyser) |musMs |

6112	6147		|muteEnabled |[audioVolume](#5318-audiovolume) |mutEd |

6113	6148		|name |[dmCapability](#5811-dmcapability), [dmPackage](#588-dmpackage), [dmSoftware](#586-dmsoftware), [dmStorage](#5812-dmstorage) |name |

6149	+	|needsReplacement |[5.3.1.35 filterInfo](#53135-filterinfo)|neeRt |

6114	6150		|network |[threeDprinter](#5311-threedprinter) |netwk |

6115	6151		|negativeCumulativeFlow |[waterMeterReportInfo](#53195-watermeterreportinfo) |neCFw |

6116	6152		|nh3nh4 |[waterQualityMonitor](#53198-waterqualitymonitor) |nh3n4 |

6117	6153		|no2 |[airQualitySensor](#5316-airqualitysensor) |no2 |

6118	6154		|no3n |[waterQualityMonitor](#53198-waterqualitymonitor) |no3n |

6155	+	|nodeLink |N/A |nodLk |

6119	6156		|noise |[airQualitySensor](#5316-airqualitysensor) |noise |

6120	6157		|numberValue |[numberValue](#53155-numbervalue) |numVe |

6121	6158		|o3 |[airQualitySensor](#5316-airqualitysensor) |o3 |

@@ -6161,6 +6198,7 @@ In protocol bindings resource attributes names for data points of module classes

6161	6198		|positionY |[touchScreen](#531106-touchscreen) | posiY |

6162	6199		|postalAddress |[location](#5392-location) |posAs |

6163	6200		|power |[energyConsumption](#53132-energyconsumption) |power |

6201	+	|powerOn |[smokeSensor](#53182-smokesensor)| powOn |

6164	6202		|powerGenerationData |[energyGeneration](#53133-energygeneration) |poGDa |

6165	6203		|powerSaveEnabled |[powerSave](#53166-powersave) |poSEd |

6166	6204		|powerStatus |[dmAgent](#582-dmagent) |powSs |

@@ -6188,6 +6226,7 @@ In protocol bindings resource attributes names for data points of module classes

6188	6226		|rapidFreeze |[refrigeration](#53171-refrigeration) |rapFe |

6189	6227		|readSpeed |[dmStorage](#5812-dmstorage) |reaSd |

6190	6228		|readStatus |[prePaidCardReader](#531107-prepaidcardreader) |reaSs |

6229	+	|rechargeable |[battery](#53110-battery) |reche |

6191	6230		|rechargeCredit |[gasChargingControl](#53138-gaschargingcontrol) |recCt |

6192	6231		|rechargeGas |[gasChargingControl](#53138-gaschargingcontrol) |recGs |

6193	6232		|recipientID |[phoneCall](#53164-phonecall) |recID |

@@ -6201,6 +6240,7 @@ In protocol bindings resource attributes names for data points of module classes

6201	6240		|repetitionCount |[disseminator](#53101-disseminator) |repCt |

6202	6241		|repetitionPeriod |[disseminator](#53101-disseminator) |repPd |

6203	6242		|resistance |[bioElectricalImpedanceAnalysis](#53113-bioelectricalimpedanceanalysis) |resie |

6243	+	|result|N/A | resut |

6204	6244		|retryInterval |[periodicalReportConfig](#53163-periodicalreportconfig) |retIl |

6205	6245		|retryTimes |[periodicalReportConfig](#53163-periodicalreportconfig) |retTs |

6206	6246		|reverseFlowAlarm |[waterMeterAlarm](#53194-watermeteralarm)|reFAm |

@@ -6216,6 +6256,7 @@ In protocol bindings resource attributes names for data points of module classes

6216	6256		|rsrq |[connectivity](#53125-connectivity) |rsrq |

6217	6257		|rssi |[connectivity](#53125-connectivity), [signalStrength](#53178-signalstrength) |rssi |

6218	6258		|runningTime |[timer](#53190-timer) |runTe |

6259	+	|saveAudioVolume |[audioVolume](#5318-audiovolume) |saAVe |

6219	6260		|scanStatus |[cardScanner](#531112-cardscanner) |scaSs |

6220	6261		|sdp |[sessionDescription](#53177-sessiondescription) |sdp |

6221	6262		|secondaryName |[dmFirmware](#585-dmfirmware) |secNe |

@@ -6237,11 +6278,13 @@ In protocol bindings resource attributes names for data points of module classes

6237	6278		|size |[binaryObject](#53111-binaryobject), [features](#5391-features) |size |

6238	6279		|sleepDuration |[dmAreaNwkDeviceInfo](#5810-dmareanwkdeviceinfo) | sleDn |

6239	6280		|sleepInterval |[dmAreaNwkDeviceInfo](#5810-dmareanwkdeviceinfo) | sleIl |

6281	+	|sensorFault | [smokeSensor](#53182-smokesensor) |senFt |

6240	6282		|smokeThreshold |[smokeSensor](#53182-smokesensor) |smoTd |

6241	6283		|so2 |[airQualitySensor](#5316-airqualitysensor) |so2 |

6242	6284		|softLeanMass |[bodyCompositionAnalyser](#53114-bodycompositionanalyser) |soLMs |

6243		-	|softwares | [dmPackage](#588-dmpackage) | softe |

6285	+	|softwares | [dmPackage](#588-dmpackage) | softs |

6244	6286		|soilLevel |[clothesWasherJobModeOption](#53122-clotheswasherjobmodeoption) |soiLl |

6287	+	|somatotype |[bodyCompositionAnalyser](#53114-bodycompositionanalyser) |somae |

6245	6288		|speed |[airFlow](#5314-airflow) |speed |

6246	6289		|speedFactor |[playerControl](#53165-playercontrol) |speFr |

6247	6290		|speedWash |[clothesWasherJobModeOption](#53122-clotheswasherjobmodeoption) |speWh |

@@ -6250,7 +6293,7 @@ In protocol bindings resource attributes names for data points of module classes

6250	6293		|standardreferenceVoltage |[slcAlarm](#53179-slcalarm) |staVe |

6251	6294		|startPause |[operationMode](#53157-operationmode) |staPe |

6252	6295		|state |[dmAgent](#582-dmagent), [dmPackage](#588-dmpackage), [dmFirmware](#585-dmfirmware), [binarySwitch](#53112-binaryswitch) |state |

6253		-	|status |[boiler](#53115-boiler), [disseminator](#53101-disseminator), [dmAreaNwkDeviceInfo](#5810-dmareanwkdeviceinfo), [dmEventLog](#587-dmeventlog), [dmStorage](#5812-dmstorage), [electricVehicleConnector](#53131-electricvehicleconnector), [emergencyHandler](#53102-emergencyhandler), [faultDetection](#53134-faultdetection), filterInf <mark>editor note: correct reference</mark>, [mediaSelect](#53153-mediaselect) |sus |

6296	+	|status |[boiler](#53115-boiler), [disseminator](#53101-disseminator), [dmAreaNwkDeviceInfo](#5810-dmareanwkdeviceinfo), [dmEventLog](#587-dmeventlog), [dmStorage](#5812-dmstorage), [electricVehicleConnector](#53131-electricvehicleconnector), [emergencyHandler](#53102-emergencyhandler), [faultDetection](#53134-faultdetection), [filterInfo](#53135-filterinfo), [mediaSelect](#53153-mediaselect) |sus |

6254	6297		|steamTreat |[clothesWasherJobModeOption](#53122-clotheswasherjobmodeoption) |steTt |

6255	6298		|step |[numberValue](#53155-numbervalue) |step |

6256	6299		|stepsPerPeriod |[cowActivityMonitor](#5371-cowactivitymonitor) |stPPd |

@@ -6260,7 +6303,7 @@ In protocol bindings resource attributes names for data points of module classes

6260	6303		|storageTotal |[dmAgent](#582-dmagent) |stoTl |

6261	6304		|strength |[brewing](#53116-brewing) |streh |

6262	6305		|subModel |[dmDeviceInfo](#583-dmdeviceinfo) |subMl |

6263		-	|sulfide |[waterQualityMonitor](#53198-waterqualitymonitor) |sulfide |

6306	+	|sulfide |[waterQualityMonitor](#53198-waterqualitymonitor) |sulfe |

6264	6307		|supportedHorizontalDirection |[airFlow](#5314-airflow) |suHDn |

6265	6308		|supportedMediaSources |[mediaSelect](#53153-mediaselect) |suMSs |

6266	6309		|supportedMessageValues |[textMessage](#53189-textmessage) |suMVs |

@@ -6321,6 +6364,7 @@ In protocol bindings resource attributes names for data points of module classes

6321	6364		|uri |[printQueue](#53167-printqueue) |ur0 |

6322	6365		|url |[sessionDescription](#53177-sessiondescription), [dmPackage](#588-dmpackage), [dmSoftware](#586-dmsoftware) |url |

6323	6366		|useBlender |[blender](#531118-blender)|useBr |

6367	+	|usedTime |[filterInfo](#53135-filterinfo) |usedTe |

6324	6368		|useGrinder |[grinder](#53143-grinder) |useGr |

6325	6369		|UUID |[dmStorage](#5812-dmstorage) |UUID |

6326	6370		|uvStatus |[uvSensor](#53192-uvsensor) |uvtSs |

@@ -6332,8 +6376,8 @@ In protocol bindings resource attributes names for data points of module classes

6332	6376		|vibrationSensorFault |[waterMeterAlarm](#53194-watermeteralarm)|viSFt |

6333	6377		|visceraFat |[bioElectricalImpedanceAnalysis](#53113-bioelectricalimpedanceanalysis) |visFt |

6334	6378		|voc |[airQualitySensor](#5316-airqualitysensor) |voc |

6335		-	|voltage |[battery](#53110-battery) |volte |

6336		-	|voltage |[energyConsumption](#53132-energyconsumption) |volte |

6379	+	|volumeValue |[audioVolume](#5318-audiovolume) |volVe |

6380	+	|voltage |[battery](#53110-battery), [energyConsumption](#53132-energyconsumption) |volte |

6337	6381		|voltageFrequency |[slcReportInfo](#53181-slcreportinfo) |volFy |

6338	6382		|volumePercentage |[audioVolume](#5318-audiovolume) |volPe |

6339	6383		|washTemp |[clothesWasherJobModeOption](#53122-clotheswasherjobmodeoption) |wasTp |

----------------------- End of change 1 -----------------------