| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS #43 |
| Source:* | Convida Wireless |
| Date:* | 2019-09-16 |
| Contact:* | Dale Seed, Convida, Seed.Dale@convidawireles.com Lu Liu, Convida, Liu.Lu@convidawireless.com Catalina Mladin, Convida, Mladin.Catalina@convidawireles.com |
| Reason for Change/s:* | Provides potential solution affecting process management for action triggering. |
| CR against: Release* | Release 4 |
| CR against: WI* | Active <WI-0093> MNT / < Work Item number(optional)> Is this a companion CR? Yes No Companion CR number: (Note to Rapporteur - use latest agreed revision)Is this a mirror CR? Yes No Mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TR-0060 V 0.0.1 |
| Clauses/Sub Clauses * | Clause 7 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | N/A |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

The following introduces the procedures for state and process management based on the resources introduced in SDS-2019-0222R01.

R01 –

During review we discussed the need to clearly describe the values of various states or conditions after manual (or automatic) process state changes (active, enable, pause, etc)

R02 – clarified the descriptions of the new resources and procedures.

R03 – clarified the descriptions of the new resources and procedures.

### -----------------------Start of change 1-------------------------------------------

# Candidate Solutions

## Solution 1: State and Sequencing for Action Triggering

### 7.2.1 New Resource Type: <state>

An IoT process usually consists of multiple stages or states, and the transitions among the states are driven by various events and the corresponding actions. The <state> resource is proposed to store the information about the IoT process, and to monitor the status of the IoT process and define the actions and transitions in this IoT process.

Table 7.2.1-1: Attributes of <state> resource

| Attributes of <state> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| currentStatus | 1 | RO | The indicator of whether this state is currently active. This attribute may take values from “active” or “inactive”. |
| stateActions | 0..1 (L) | RW | The link(s) to the <action> resource(s) that may be performed in this state. The actions include both the ones that will be performed unconditionally when entering this state, and the ones that may be triggered by certain events/conditions within this state. |
| stateTransitions | 0..1 (L) | RW | The possible transition(s) that may happen from the current state. Each transition is defined as a tuple [transition criteria, next state]: Transition criteria: the event or condition that may trigger state transition; the transition criteria could be defined in a similar way as evalCriteria. Next state: the resourceID of the next <state> resource to transition to. If this state is the last state in the process, this attribute will be empty. |

Once an IoT process transitions to a given state, the value of the currentStatus attribute of the given <state> resource will be changed to “active”. Then, for all the <action> resources that are linked through the stateActions attribute, their evalMode attributes will be changed from “off” to “once”. For an unconditional action, the Hosting CSE will perform the action immediately. For a conditional action, the Hosting CSE will monitor the evalCriteria attribute of each <action> resource to determine if/when the action will be performed. In addition, the Hosting CSE will start to monitor the transition criteria defined in the stateTransitions attribute of the <state> resource. If/when any of the events/conditions defined in the stateTransitions are met, the Hosting CSE will trigger a state transition. When performing a state transition, the currentStatus attribute of the current <state> resource will be changed to “inactive” and all the evalMode attributes of the <action> resources referenced by the current <state> resource will be changed back to “off”. The Hosting CSE will then transition to the next state as indicated by the resource identifier of the next state’s <state> resource defined in the stateTransitions attribute of the current state’s <state> resource.

### 7.2.2 New Resource Type: <processManagement>

The <processManagement> resource is proposed to store the information of the entire process consisting of multiple states.

Table 7.2.2-1: Child Resources of Proposed oneM2M <processManagement> Resource

| Child Resources of <processManagement> | Child Resource Type | Multiplicity | Description |
| --- | --- | --- | --- |
| [variable] | <state> | 0..n | This resource describes the details of a particular state of an IoT process. |
| [variable] | <subscription> | 0..n | See clause 9.6.8 in [x]. |

Table 7.2.2-2: Attributes of <processManagement> resource

| Attributes of <processManagement> | Multiplicity | RW/ RO/ WO | Description |
| --- | --- | --- | --- |
| processStatus | 1 | RO | The status for the entire IoT process. The supported values for this attribute are: Disabled: The IoT process is disabled. Enabled: The IoT process is enabled, and the Hosting CSE will monitor the event/condition defined in the preconditions attribute. If the Hosting CSE detects that the preconditions are met, the Hosting CSE will update the processStatus attribute to “Active”. Active: The IoT process is active (“running”). Paused: The IoT process is paused and will remain in the current state until “Active” again or the exitConditions are met. . Ended: The IoT process has entered the final state or exited through exitConditions. When the processStatus is “Enabled”, “Paused” or “Ended”, the Hosting shall allow child <state> resources to be added, modified of deleted from the process. Otherwise, the Hosting CSE will reject requests to add, modify or delete <state> resources from the process. |
| processControl | 1 | RW | This attribute is used to control and set the status of the process. The supported values for the attribute are: Enable: The Hosting CSE will update processStatus to “Enabled”. Disable: The Hosting CSE will update processStatus to “Disabled”. Activate: The Hosting CSE will update processStatus to “Active”. Pause: The Hosting CSE will update processStatus to “Paused”. End: The Hosting CSE will update processStatus to “Ended”. The <processManagement> resource shall be created with this attribute set to “Disable”. |
| currentState | 1 | RO | The resourceID of the child <state> resource that has currentStatus set to active. If the processStatus is not “Active”, the value of this attribute will be empty. |
| preconditions | 0..1 (L) | RW | This attribute specifies any conditions that must be met for the process to begin. This attribute could be defined in a similar way as evalCriteria. It can be used to trigger the start of the IoT process. When the conditions are met, the Hosting CSE will update processStatus to “Active”, and the Hosting CSE will set currentState to the value defined in initialState. The Hosting CSE will also change the currentStatus of the <state> resource indicated by initialState to “Active”. |
| exitConditions | 0..1 (L) | RW | This attribute specifies what events can end the IoT process. This attribute could be defined in a similar way as evalCriteria. It allows for an asynchronous exit of the process from any state. When the exit conditions are detected by the Hosting CSE, the Hosting CSE will set processStatus to “Ended” and set the status of all child <state> resources to “inactive”. |
| initialState | 1 | RW | The resourceID of the first <state> resource of the process. |
| finalState | 0..1 | RW | The resourceID of the last <state> resource in the process. If the process is a loop, this attribute is empty. Before the Hosting CSE updates the currentState attribute, it checks to see if the current value of currentState matches the value of finalState. If so the Hosting CSE updates processStatus to “Ended”. |

### 7.2.3 State and Process Management

7.2.3.1 Introduction

This clause describes the procedure for managing oneM2M state and process in action triggering via the <state> and <processManagement> resources.

7.2.3.2 Process Management Procedure

This clause describes the procedure a Hosting CSE performs to manage an IoT process.

Step 1:	An AE creates a <processManagement> resource.

Step 2:	An AE creates all the <action> resources that are involved in the IoT process and state transitions.

Step 3:	An AE creates all the <state> resources that are involved in the IoT process as child resources of <processManagement>.

Step 4:	An AE enables the <processManagement> by updating the value of processControl to “Enable”. The Hosting CSE will update the value of processStatus to “Enabled”. The Hosting CSE will start to monitor the event/condition defined in preconditions and exitConditions.

Step 5:	If/when the event/condition defined in preconditions is detected, the Hosting CSE sets the processStatus to “Active”. If preconditions is empty, the process can be manually activated by setting the value of processControl to “Activate” and the Hosting CSE will set the processStatus to “Active”. The Hosting CSE sets the currentState to the value in initialState.

Step 6:	Starting from the initialState, whenever a transition happens, the currentState attribute is updated to the resourceID of the next <state> by the Hosting CSE. The <state> resource corresponding to the current state and cannot be modified, only RETRIEVE operation is allowed.

Step 7:	If an AE updates processControl to “Pause” the Hosting CSE will change the processStatus to “Paused”. The paused IoT process will not transition to another state until processControl is updated to “Activate” or the exitConditions are met.

Step 8:	If an AE updates processControl to “Disable” the Hosting CSE will change processStatus to “Disabled”.   The Hosting CSE will set the currentStatus of all child <state> resources to “inactive”.

Step 9:	If/when the event/condition defined in exitConditions is detected, the Hosting CSE will set processStatus to “Ended”. The Hosting CSE will set the currentStatus of all child <state> resources to “inactive”. The Hosting CSE will set currentState to empty.

Step 10:	If the currentState  transitions to the value of finalState, the processStatus will be set to “Ended” by the Hosting CSE. The Hosting CSE will set the currentStatus of all child <state> resources to “inactive”. The Hosting CSE will set currentState to empty.

7.2.3.3 State Transition Procedure

This clause describes the procedure a Hosting CSE performs to manage an IoT process that has been “Enabled”.

Step 1:	<state> resources are created as child resources of the <processManagement> resource.

Step 2:	The IoT process enters a new state, e.g. after a state transition, or a process is initialized when the pre-conditions are met.

Step 3:	The Hosting CSE sets the value of the currentStatus attribute of the <state> indicated in currentState to “active”.

Step 4:	When a <state> is activated the Hosting CSE shall monitor the stateActions. The Hosting CSE shall set the evalMode attribute of all the <actions> indicated in stateActions attribute to “once”.

For an <action> that has evalCriteria attribute that is empty (an unconditional action) it will be performed immediately.

For a conditional action,

the Hosting CSE will check the evalCriteria attribute of the <action> to determine whether the action will be triggered or not.

Step 5:	The Hosting CSE will start to monitor the conditions specified in stateTransitions.

Step 6:	If one of the events or conditions defined in stateTransitions is detected by the Hosting CSE, the Hosting CSE shall update the currentState of the <processManagement>.

Step 7:	If/when the value of the currentStatus attribute is set to “inactive” by the Hosting CSE.

Step 8:	For all the state specific <actions> that are linked through the stateActions attribute, their evalMode attributes are changed to “off” by the Hosting CSE.

Step 9:	The Hosting CSE transitions to the next state as indicated by the next state ID.

### -----------------------End of change 1 -------------------------------------------

CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?