	SDS-2020-0031R01-TS-0001_User_Based_Charging_Accounting_R4
	[image: image1.png]






	


	CHANGE REQUEST

	Meeting ID:*
	SDS 44

	Source:*
	Dale Seed, Convida

	Date:*
	2020-02-10

	Reason for Change/s:*
	Enhance charging and accounting to add support for M2M Service Users 

	CR  against:  Release*
	Release 4

	CR  against:  WI*
	 FORMCHECKBOX 
 Active – WI-0083 - oneM2M Service Subscribers and Users   

 FORMCHECKBOX 
 MNT maintenance / < Work Item number(optional)>
Is this a mirror CR? Yes  FORMCHECKBOX 
 No  FORMCHECKBOX 

mirror CR number: (Note to Rapporteur - use latest agreed revision)
 FORMCHECKBOX 
 STE Small Technical Enhancements / < Work Item number (optional)>
Only ONE of the above shall be ticked

	CR  against:  TS/TR*
	TS-0001 V4.3.0

	Clauses *
	9.6.25, 10.2.11

	Type of change: *
	 FORMCHECKBOX 
 Editorial change
 FORMCHECKBOX 
 Bug Fix or Correction
 FORMCHECKBOX 
 Change to existing feature or functionality
 New feature or functionality
Only ONE of the above shall be ticked

	Impacted other TS/TR(s)
	TS-0004 Rel-4

	Post Freeze checking:*
	This CR contains only essential changes and corrections?  YES  FORMCHECKBOX 
  NO  FORMCHECKBOX 

This CR may break backwards compatibility with the last approved version of the TS?       YES  FORMCHECKBOX 
  NO  FORMCHECKBOX 



	Template Version: January 2017 (Do not modify)


oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.
In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR
Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).
Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.
Follow the drafting rules.
All pictures must be editable.
Check spelling and grammar to the extent practicable.
Use Change bars for modifications.
The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.
Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.
When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text. 
Introduction
TR-0054 defines an enhancement to the existing Service Charging and Accounting functionality to enable a Hosting CSE with support for collecting statistics for requests originated by a particular M2M Service User.  This is done by expanding the definition of the collectedEntityID attribute to also allow M2M-User-ID.

Likewise, to enable a Hosting CSE with support for collecting statistics on behalf of a particular M2M Service User, the collectingEntityID definition can be extended to also include M2M-User-ID

R01:
· Updated title of contribution from 2019 to 2020
-----------------------Start of change 1-------------------------------------------
9.6.25
Resource Type statsCollect
The <statsCollect> resource shall be used to collect information for AEs or M2M Service Users using the <eventConfig> resource as the triggers in the IN-CSE. Multiple triggers can be established at the IN-CSE for the same AE or M2M Service User. Each trigger may be activated or de-activated independently of others. The <statsCollect> resource shall be located directly under <CSEBase> of the IN‑CSE. 
The <statsCollect> resource shall contain the child resource specified in table 9.6.25-1.

Table 9.6.25-1: Child resources of <statsCollect> resource

	Child Resources of <statsCollect>
	Child Resource Type
	Multiplicity
	Description

	[variable]
	<subscription>
	0..n
	See clause 9.6.8 where the type of this resource is described.

	[variable]
	<transaction>
	0..n
	See clause 9.6.48


The <statsCollect> resource shall contain the attributes specified in table 9.6.25-2.

Table 9.6.25-2: Attributes of <statsCollect> resource

	Attributes of <statsCollect>
	Multiplicity
	RW/

RO/

WO
	Description

	resourceType
	1
	RO
	See clause 9.6.1.3 

	resourceID
	1
	RO
	See clause 9.6.1.3.

	resourceName
	1
	WO
	See clause 9.6.1.3.

	parentID
	1
	RO
	See clause 9.6.1.3.

	accessControlPolicyIDs
	0..1 (L)
	RW
	See clause 9.6.1.3.

	creationTime
	1
	RO
	See clause 9.6.1.3.

	expirationTime
	1
	RW
	See clause 9.6.1.3.

	lastModifiedTime
	1
	RO
	See clause 9.6.1.3.

	labels
	0..1 (L)
	RW
	See clause 9.6.1.3.

	dynamicAuthorizationConsultationIDs
	0..1 (L)
	RW
	See clause 9.6.1.3.

	creator
	0..1
	RO
	See clause 9.6.1.3.

	statsCollectID
	1
	RO
	This is the unique ID to identify a specific statistics collection scenario. It is created by the IN-CSE when the <statsCollect> resource is first created.

	collectingEntityID
	1
	WO
	This is the unique ID of the entity that requests the collection of statistics. For example, it can be an AE-ID, M2M-User-ID or CSE-ID.

	collectedEntityID
	0..1(L)
	WO
	This is the list of unique ID of the entities whose request triggered the configured event for statistics collection. For example, each ID can be an AE-ID, M2M-User-ID or IN-CSE-ID.

If no specific value is provided for this attribute, the IN-CSE interprets it as "any entity".

	statsRuleStatus
	1
	RW
	This attribute indicates whether the rule is "active" or "inactive".

	statModel
	1
	RW
	This attribute indicates the collection model, such as "Subscriber based", "event based", etc.

	collectPeriod
	0..1
	RW
	Expresses time periods defined by second, minute, hour day of month, month, and year. Supports repeating periods, and wildcards expressed as a list.

	eventID
	0..1
	RW
	This attribute refers to the <eventConfig> resource that defines the events that can be collected by the IN-CSE. It is mandatory if the statModel attribute is set to "event based".


-----------------------End of change 1---------------------------------------------

-----------------------Start of change 2-------------------------------------------

10.2.11.14
Service Statistics Collection Record
When the Service Statistics Collection is supported, the Information Elements shall be generated according to table 10.2.11.14-1.

The contents of each Service statistics collection record are decided by the specific collection scenario that triggered the information recording.

Transfer of the Statistics Collection Records over the Mch reference point is not defined in the present document.

Table 10.2.11.14-1: Information Elements for Service Statistics Collection Record

	Information Element
	Mandatory/ optional
	Description

	statsCollectID
	M
	It is the unique ID that identifies a specific statistics collection scenario, which triggers information recording for a specific event.

	collectingEntityID
	M
	This is the unique ID of the entity that collects the statistics. It can be an AE-ID, M2M-User-ID or CSE-ID.

	collectedEntityID
	M
	This is the unique ID of the entity whose service layer operation statistics are being collected. It can be an AE-ID, M2M-User-ID or CSE-ID.

	event
	O
	This indicates a specific event type in each record, such as timer based, data operation, storage triggering. It is only present if the statModel is "event based".

	eventStart
	O
	The start time for the recording the M2M event record.

	eventEnd
	O
	The end time for the recording the M2M event record.

	transactionType
	O
	Specifies the detailed type of a transaction, such as CREATE, RETRIEVE, etc.

	dataSize
	O
	Storage Memory in Kbytes, where applicable, to store data associated events with container related operations.

	Vendor Specific Information
	O
	Defines Vendor specific information.


-----------------------End of change 2---------------------------------------------

CHECK LIST

· Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?
· Does this CR contain changes related to only one particular issue/problem?
· Have any mirror CRs been posted?
· Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?
· Are all pictures editable?
· Have you checked the spelling and grammar?
· Have you used change bars for all modifications?
· Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)
· Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?
© 2020 oneM2M Partners
                                                                                                   Page 1 (of 7)



[image: image1.png]