	


	CHANGE REQUEST

	Meeting ID:*
	SDS 44

	Source:*
	Lu Liu, Convida, Liu.Lu@convidawireless.com 
Dale Seed, Convida, Seed.Dale@convidawireless.com

	Date:*
	2020-02-14

	Reason for Change/s:*
	Add support for state and process management 

	CR  against:  Release*
	Release 4

	CR  against:  WI*
	 FORMCHECKBOX 
 Active – WI-0093 – Action Triggering Enhancements   

 FORMCHECKBOX 
 MNT maintenance / < Work Item number(optional)>
Is this a mirror CR? Yes  FORMCHECKBOX 
 No  FORMCHECKBOX 

mirror CR number: (Note to Rapporteur - use latest agreed revision)
 FORMCHECKBOX 
 STE Small Technical Enhancements / < Work Item number (optional)>
Only ONE of the above shall be ticked

	CR  against:  TS/TR*
	TS-0001 V4.4.0

	Clauses *
	9.6, 10.2 

	Type of change: *
	 FORMCHECKBOX 
 Editorial change
 FORMCHECKBOX 
 Bug Fix or Correction
 FORMCHECKBOX 
 Change to existing feature or functionality
 FORMCHECKBOX 
 New feature or functionality
Only ONE of the above shall be ticked

	Impacted other TS/TR(s)
	TS-0004 Rel-4

	Post Freeze checking:*
	This CR contains only essential changes and corrections?  YES  FORMCHECKBOX 
  NO  FORMCHECKBOX 

This CR may break backwards compatibility with the last approved version of the TS?       YES  FORMCHECKBOX 
  NO  FORMCHECKBOX 



	Template Version: January 2017 (Do not modify)


oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.
In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR
Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).
Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.
Follow the drafting rules.
All pictures must be editable.
Check spelling and grammar to the extent practicable.
Use Change bars for modifications.
The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.
Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.
When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text. 
Introduction
This contribution proposes to add support for the state and process management functionality defined in TR-0060 into TS-0001.  
R01:

· Added <action> resources as child of <state> and added text stating that actions performed in a state shall be child <action> resources of <state>
· Changed “Ended” state to “Completed”
· Add “Activate” processControl to transition process from “Paused” to “Activated” state
· Added text to clarify what happens when no evalCriteria are specified for a child <action> resource of a <state> resource

· Removed the finalState attribute of <processManagement> resource
R02:

- Change stateActions to a singleton rather than a list.
– Clarified that Hosting performs checks on resources an privileges in a process when process transitions from Disabled to Enabled.
-----------------------Start of change 1-------------------------------------------
9.6.x Resource Type state
A process consists of a sequence of states.  Each state in a process is defined by a <state> resource. A <state> resource includes status information maintained for the state and actions performed by the Hosting CSE when a process is in the state. A <state> resource also defines criteria used by a Hosting CSE for transitioning to the next <state> resource in a process.  

The <state> resource shall contain the child resources specified in table 9.6.x-1.

Table 9.6.x-1: Child resources of <state> resource

	Child Resources of <state>
	Child Resource Type
	Multiplicity
	Description
	<stateAnnc> Child Resource Types

	[variable]
	<subscription>
	0..n
	See clause 9.6.8
	<subscription>

	[variable]
	<transaction>
	0..n
	See clause 9.6.48
	<transaction>

	[variable]
	<action>
	0..1
	See clause 9.6.61
	<action>


The <state> resource shall contain the attributes specified in table 9.6.x-2.

Table 9.6.x-2: Attributes of <state> resource

	Attributes of 
<state>
	Multiplicity
	RW/

RO/

WO
	Description
	<stateAnnc> Attributes

	resourceType
	1
	RO
	See clause 9.6.1.3.
	NA

	resourceID
	1
	RO
	See clause 9.6.1.3.
	NA

	resourceName
	1
	WO
	See clause 9.6.1.3.
	NA

	parentID
	1
	RO
	See clause 9.6.1.3.
	NA

	expirationTime
	1
	RW
	See clause 9.6.1.3. 
	MA

	accessControlPolicyIDs
	0..1 (L)
	RW
	See clause 9.6.1.3. 
	MA

	labels
	0..1 (L)
	RW
	See clause 9.6.1.3.
	MA

	creationTime
	1
	RO
	See clause 9.6.1.3.
	NA

	lastModifiedTime
	1
	RO
	See clause 9.6.1.3.
	NA

	announceTo
	0..1 (L)
	RW
	See clause 9.6.1.3.
	NA

	announcedAttribute
	0..1 (L)
	RW
	See clause 9.6.1.3.
	NA

	dynamicAuthorizationConsultationIDs
	0..1 (L)
	RW
	See clause 9.6.1.3.
	OA

	creator
	1
	RO
	See clause 9.6.1.3.
	NA

	stateStatus
	1
	RO
	Indicates whether this state is currently the active state of the process. The supported values for this attribute include “active” and “inactive”. 
	OA

	stateAction
	0..1
	RW
	A resource identifier of a child <action> resource that a Hosting CSE performs when this state is the active state in the process. 
	OA

	stateTransitions
	0..1(L)
	RW
	A list criteria that a Hosting CSE uses to determine if/when to transition from this state to another state in the process.  Each entry in this list is defined as a tuple consisting of the following two elements: 

· evalCriteria: conditions expressed in the same format as evalCriteria defined in Table 9.6.61-3 and where the subject parameter of the evalCriteria is configured with a resource identifier of a subject resource.

· next state: the resource identifier of a <state> resource.

If this <state> resource represents the last state in the process, this attribute shall be empty.
	OA


9.6.y Resource Type processManagement
A <processManagement> resource defines a process that a Hosting CSE manages on behalf of an AE and that consists of a sequence of states. Each state in a process is represented using a <state> resource defined in clause 9.6.x. 

The <processManagement> resource shall contain the child resources specified in table 9.6.y-1.

Table 9.6.y-1: Child resources of <processManagement> resource

	Child Resources of <processManagement>
	Child Resource Type
	Multiplicity
	Description
	<processManagementAnnc> Child Resource Types

	[variable]
	<state>
	0..n
	This resource describes the details of a particular state of a process.
	<state>

	[variable]
	<subscription>
	0..n
	See clause 9.6.8
	<subscription>

	[variable]
	<transaction>
	0..n
	See clause 9.6.48
	<transaction>


The <processManagement> resource shall contain the attributes specified in table 9.6.y-2.

Table 9.6.y-2: Attributes of <processManagement> resource

	Attributes of 
<processManagement>
	Multiplicity
	RW/

RO/

WO
	Description
	<processManagementAnnc> Attributes

	resourceType
	1
	RO
	See clause 9.6.1.3.
	NA

	resourceID
	1
	RO
	See clause 9.6.1.3.
	NA

	resourceName
	1
	WO
	See clause 9.6.1.3.
	NA

	parentID
	1
	RO
	See clause 9.6.1.3.
	NA

	expirationTime
	1
	RW
	See clause 9.6.1.3. 
	MA

	accessControlPolicyIDs
	0..1 (L)
	RW
	See clause 9.6.1.3. 
	MA

	labels
	0..1 (L)
	RW
	See clause 9.6.1.3.
	MA

	creationTime
	1
	RO
	See clause 9.6.1.3.
	NA

	lastModifiedTime
	1
	RO
	See clause 9.6.1.3.
	NA

	announceTo
	0..1 (L)
	RW
	See clause 9.6.1.3.
	NA

	announcedAttribute
	0..1 (L)
	RW
	See clause 9.6.1.3.
	NA

	dynamicAuthorizationConsultationIDs
	0..1 (L)
	RW
	See clause 9.6.1.3.
	OA

	creator
	1
	RO
	See clause 9.6.1.3.
	NA

	processStatus
	1
	RO
	The status of the process. The allowed values include:

· “Disabled”: The process is disabled. The Hosting CSE shall wait until the processControl attribute is updated with a value of “Enabled”  before enabling the process. When a  <processManagement> resource is created and the processControl attribute is not specified, the Hosting CSE shall assign a default value of “Disabled” to processStatus.  The Hosting CSE shall also disable the process and set processStatus to “Disabled” when receiving an update to the processControl attribute with a value of “Disable”.

· “Enabled”: The Hosting CSE has enabled the process after receiving an update to the processControl attribute with a value of “Enable”. Once enabled, the Hosting CSE shall monitor any criteria defined in the activateConditions attribute. If the Hosting CSE detects that the specified activateConditions have been met or that no criteria have been specified, the Hosting CSE shall update the processStatus attribute to “Activated”.

· “Activated”: The Hosting CSE has activated the process. Once activated, the Hosting CSE shall monitor and perform actions based on the <state> resources and the endConditions attribute defined for the process.

· “Paused”: The Hosting CSE has paused the process after recieving an update of the processControl attribute to a value of “Pause”. The Hosting CSE shall keep the process paused in the current state until the processControl attribute is updated with a value of “Activate” to resume the process or “Disable” to exit the process.

· “Completed”: The Hosting CSE has completed the process by processing the last state in the process or detecting specified endConditions have been met.  

· “Aborted”: The Hosting CSE has aborted the process due to detecting an error condition.  
	OA

	processControl
	1
	RW
	This attribute is configured by an AE to instruct the Hosting CSE to enable, disable and pause the processing of the process. The supported values for the attribute are:

· Enable: The Hosting CSE shall enable the process and update processStatus to “Enabled” if the current processStatus is “Disabled”, “Completed” or “Aborted”.

· Disable: The Hosting CSE shall disable the process and update processStatus to “Disabled”.

· Pause: The Hosting CSE shall pause the process and update processStatus to “Paused”. 
· Activate: The Hosting CSE shall reactivate the process and update processStatus to “Activated” if the current processStatus is “Paused”
This attribute shall either not be present in a <processManagement> create request or have a value of “Disable”.
	OA

	currentState
	1
	RO
	The resource identifier of the child <state> resource that has stateStatus set to “active”. If the processStatus is not “Activated” or “Paused”, the value of this attribute shall be NULL.
	OA

	activateConditions
	0..1 (L)
	RW
	This attribute specifies the conditions that shall be met for the Hosting CSE to begin the process. This attribute has the same format as the evalCriteria defined in Table 9.6.61-3 where the subject parameter of the evalCriteria is configured with a resource identifier of a subject resource. 

When the specified activateConditions have been met or if no activateConditions have been specified, the Hosting CSE shall update processStatus to “Activated”, and currentState to the value defined in initialState and start processing the process. The Hosting CSE shall also change the stateStatus attribute of the <state> resource indicated by initialState to “active”.
	OA

	endConditions
	0..1 (L)
	RW
	This attribute specifies early exit criteria that the Hosting CSE shall monitor to detect if/when to stop processing the defined states of the process. This attribute has the same format as the evalCriteria defined in Table 9.6.61-3. where the subject parameter of the evalCriteria is configured with a resource identifier of a subject resource 

This attribute allows for an asynchronous exit of the process from any state. When the exit conditions are detected, the Hosting CSE shall set the processStatus attribute to “Completed” and set the status of all child <state> resources to “inactive”.
If no endConditions are specified, the Hosting CSE shall end the process after processing the last <state> resource of the process (assuming the process is not a loop) or the process is aborted due to an error. 
	OA

	initialState
	1
	RW
	The resource identifier of the first <state> resource of the process.
	OA

	
	
	
	
	


-----------------------End of change 1---------------------------------------------

-----------------------Start of change 2-------------------------------------------

10.2.z
Process Management
10.2.z.1
Introduction
If a Hosting CSE support process management capability, then it shall support the <state> and <processManagement> resource types.  This clause describes the procedure for managing a oneM2M process via the <state> and <processManagement> resources.

A process is defined and managed by an AE creating a <processManagement> resource at a Hosting CSE. The <state> resources corresponding to the states in the process are then created by an AE as child resources of the <processManagement> resource. The <action> resources corresponding to the actions performed by the Hosting CSE with respect to each state are also created as child resources of each <state> resource and linked via the stateAction attribute.  The evalCriteria defined by child <action> resources of each <state> resource are not monitored by a Hosting CSE until the process is activated and the process has transitioned to the state defined by each <state> resource. If no evalCriteria are defined for a child <action> resource of a <state> resource, then the Hosting CSE shall perform the corresponding actions when the process is activated and the process has transitioned to the state defined by the <state> resource. The activateConditions and endConditions attributes are configured by an AE to define the criteria that the Hosting CSE uses to trigger the activation and end of the process. The defined criteria shall be monitored by the Hosting CSE once the process is enabled by the AE. The first state in the process is configured by an AE via the initialState attribute of the <processManagement> resource. The processControl attribute of the <processManagement> resource is updated by an AE to instruct the Hosting CSE to enable, disable or pause the process. While managing the process, the Hosting shall monitor the criteria attributes (e.g. activateConditions, endConditions, stateTransitions) defined within <processManagement> and <state> resources of the process to determine if/when to perform corresponding actions defined by the <action> resources of the process. While managing the process, the Hosting CSE shall update the status attributes (e.g. stateStatus, processStatus, currentState) defined within <processManagement> and <state> resources of the process.  

When entering a state, the Hosting CSE shall update the currentState attribute of the <processManagement> resource to the resource identifier of the <state> resource actively being processed, udpate the value of the stateStatus attribute of this <state> resource to “active”, and check the <action> resource that are defined in the stateAction attribute. For an unconditional action whose evalCriteria is empty, the Hosting CSE shall perform the action immediately when processing of the <state> begins. Otherwise, the Hosting CSE shall monitor the evalCriteria attribute of an <action> resource to determine if/when the action shall be performed. The Hosting CSE shall also monitor the evalCriteria parameter defined in the stateTransitions attribute of the <state> resource. If/when any of the criteria defined in the stateTransitions are met, the Hosting CSE shall perform a state transition. When performing a state transition, the Hosting CSE shall update the stateStatus attribute of the current <state> resource to “inactive and the stateStatus attribute of the next <state> resource (if any) to “active”.  If a next <state> resource is not applicable (e.g. the Hosting CSE has completed processing the last state in the process, the Hosting shall end the process by updating the processStatus attribute of the <processManagement> resource to a value of “Completed”.  While the Hosting CSE is managing a process, it shall also monitor the criteria defined in the endConditions attribute of the <processManagement> resource. If/when any of the criteria defined in the endConditions are met, the Hosting CSE shall end the process by updating the stateStatus attribute of the current <state> resource to “inactive” and the processStatus attribute of the <processManagement> resource to a value of “Completed”.  While the Hosting CSE is managing a process, if it encounters an error condition (e.g. a <state> or <action> resource is not accessible to the Hosting CSE),  the Hosting CSE shall abort the process by updating the stateStatus attribute of the current <state> resource to “inactive” and the processStatus attribute of the <processManagement> resource to a value of “Aborted”.
10.2.z.2
Process Management Procedure

This clause describes the procedure a Hosting CSE performs to manage a process as detailed by the following the steps:

[image: image1.emf]AE

<processManagement> 

Hosting CSE

Referenced resource 

Hosting CSE

Referenced resource 

Hosting CSE

...

1. Create <processManagement> resource

3. Create <action> resource(s)

2. Create <state> resource(s)

4a. processControl = Enable

4b. Check the existence and accessibility of resources related to process

5b. processStatus = Activate;

currentState = initialState;

Monitor endConditions

5a. activateConditions met 

or equal to NULL

4c. processStatus = Enabled;

Monitor activateConditions

6. Transition through states and perform actions related to process

7a. endConditions met

Or

stateTransitions = NULL

7b. processStatus = Completed;

All stateStatus = inactive;

currentState = empty;

Stop monitoring exitConditions


Figure 10.2.z.2-1: Process Management by an AE
Step 1:
An AE creates a <processManagement> resource.

Step 2:
The AE creates all the <state> resources that are involved in the process.  The <state> resources are created as child resources of the <processManagement> resource. 
Step 3:
The AE creates all the <action> resources that are involved in the process. The <action> resources are created as child resources of each corresponding <state> resource. The <action> resources are linked with the corresponding <state> resources via the stateAction attribute of each <state> resource. 
Step 4:
The AE enables the process by updating the processControl attribute of the <processManagement> resource to a value of “Enable”. The Hosting CSE shall validate each of the resources defined in the processto verify if the resource exist and are accessible to creator of the <processManagement> resource. If the validation is successful, the Hosting CSE shall update the value of processStatus to “Enabled”. The Hosting CSE shall then start to monitor the criteria defined in the activateConditions attribute of the <processManagement> resource.

Step 5:
If/when the criteria defined in the activateConditions attribute are met or the activateConditions attribute is NULL, the Hosting CSE shall set the processStatus attribute of the <processManagement> resource to a value of “Activated”. The Hosting CSE shall also set the currentState attribute to the value configured in the initialState attribute. The Hosting CSE shall stop monitoring the criteria defined in the activateConditions attribute and start monitoring the criteria defined in endConditions.

Step 6:
The Hosting shall process each <state> resource in the process starting from the state defined by the initialState attribute of the <processManagement> resource. The Hosting CSE shall evaluate and perform the action that is linked via the stateAction attribute to a state if/when their criteria have been met.  The Hosting CSE shall also monitor the state transition criteria defined in the stateTransitions attribute of the state.  Whenever the Hosting CSE detects that a state transition is required based on the stateTransitions attribute defined within the current <state> resource, the Hosting CSE shall update the currentState attribute with the resource identifier of the next <state> resource. 

· If an AE updates the processControl attribute of the <processManagement> resource to “Pause”, the Hosting CSE shall set the processStatus attribute to “Paused”. Thereafter, the Hosting CSE shall reactivate the process if the processControl attribute is updated to “Enable” or shall end the process if the processControl attribute is updated to “Disable”.

· If an AE updates the processControl attribute of the <processManagement> resource to “Disable”, the Hosting CSE shall set the processStatus attribute to “Disabled”.   The Hosting CSE shall set the stateStatus attributes of all child <state> resources to “inactive”. The Hosting CSE shall set the currentState attribute to NULL. The Hosting CSE shall stop monitoring the criteria defined in the endConditions attribute.

Step 7:
If/when the criteria defined in the endConditions attribute of the <processManagement> resource is met or the Hosting CSE detects that there are no further state transitions defined, the Hosting CSE shall set the processStatus attribute to “Completed”. The Hosting CSE shall set the stateStatus attributes of all child <state> resources to “inactive”. The Hosting CSE shall set the currentState attribute to NULL. The Hosting CSE shall stop monitoring the criteria defined in endConditions attribute. 

· A process that has completed may be restarted by an AE by updating the processControl to “Enable”.

When the processStatus is “Disabled”, the Hosting CSE shall allow child <state> resources to be added, modified of deleted from the process.  Otherwise, the Hosting CSE shall reject requests to add, modify or delete <state> resources from the process.

When the processStatus is “Disabled”, the Hosting CSE shall allow the activateConditions, endConditions and initialState  attributes of the <processManagement> resources to be updated. Otherwise, the Hosting CSE shall reject requests to update these attributes. 

When the processStatus tranisitons from “Disabled” to “Enabled”, The Hosting CSE shall check that all the required resource related to the process are present and the creator of a <processManagement> resource has the necessary privileges to perform the process related operations on these resources. The necessary privileges include RETRIEVE privileges to the resources as well as CREATE, UPDATE and DELETE privileges if/when applicable (e.g. the process includes an <action> resource that results in an UPDATE operation on a targeted resource).    If at any time, the Hosting CSE determines that the creator does not have sufficient privileges, the Hosting CSE shall not enable the process if it is not already enabled and shall abort the process if it is enabled or activated.  In addition to the <processManagement> resource, the following is a list of other potential resources that may be related to the process:

· child <state> resources of the <processManagement> resource;

· child <action> resources of the <state> resources referenced via the stateAction attribute of each <state> resource;

· resources referenced via the stateTransitions attribute of each <state> resource;

· resources referenced via the actionSubjectResources, objectResourceID, subjectResourceID, evalCriteria attributes of each related <action> resource;

· <accessControlPolicy> resources referenced by the <processManagement> resource and all other related resources

When the Hosting CSE is managing an “Enabled” or “Activated” process and encounteres an error condition (e.g. is unable to access a <state> or <action> resource), the Hosting CSE shall abort the process by updating the stateStatus attribute of the current <state> resource to “inactive” and the processStatus attribute of the <processManagement> resource to “Aborted”.

Table 10.2.z.1-1defines the valid processControl values permitted in an update request to a <processManagement> resource based on the current value of the processStatus attribute of the <processManagement> resource. A <processManagement> Hosting CSE shall only allow an update to processControl for the combinations listed.

Table 10.2.z.2-1: Valid <processManagement> processControl update values    

	Current processStatus value
	Valid processControl update values

	Disabled
	Enable

	Enabled
	Disable

	Activated
	Pause, Disable

	Paused
	Enable, Disable, Activate

	Completed
	Enable, Disable

	Aborted
	Enable, Disable


Figure 10.2.z.2-1 defines the legal state transitions of the processStatus attribute of the <processManagement> resource as well as the criteria for each transition.


[image: image2.emf]Enabled

Completed

Disabled

Paused

Activated

processControl 

set to Enable

activateConditions are 

NULL or have been met

currentState equals finalState or 

endConditions have been met

<processManagement> created

Aborted

processControl 

set to Disable

stateTransitions criteria 

have been met

processControl 

set to Activate

processControl 

set to Pause

processControl 

set to Disable

processControl 

set to Enable

processControl 

set to Disable

processControl 

set to Enable


Figure 10.2.z.2-1: <processManagement> processStatus State Diagram 

10.2.z.3
State Transition Procedure

This clause describes the procedure a Hosting CSE shall perform when entering a new state, e.g. after a state transition, or a process is initialized when the activateConditions are met.

Step 1:
The Hosting CSE shall set the value of the stateStatus attribute of the <state> resource indicated in the currentState attribute of the <processManagement> resource to “active”. 

Step 2:
When a <state> resource is activated, the Hosting CSE shall evaluate the stateAction attribute of the <state> resource. 

For an <action> resource that has an evalCriteria attribute that is NULL (i.e. an unconditional action) , the Hosting shall perform the corresponding actions defined by the <action> resource immediately upon transition to the <state> resource. 

For an <action> resource that has an evalCriteria attribute that is not NULL ( i.e. a conditional action), the Hosting CSE shall monitor the evalCriteria attribute of the <action> resource to determine if/when to trigger and perform the action. 
Step 3:
The Hosting CSE shall monitor the criteria specified in the stateTransitions attribute of the <state> resource.

Step 4:
If one of the criteria defined in stateTransitions attribute is detected to have been met by the Hosting CSE, the Hosting CSE shall update the stateStatus of the current <state> resource to “inactive”, and update the currentState of the <processManagement> resource with the resource identifier of the next <state> resource. The Hosting CSE shall then transition to the next state.

10.2.z.4
<state> Resource Procedures

10.2.z.4.1
Create <state>
The Hosting CSE shall use this procedure to create a <state> resource according to the resource identifier of the parent resource.

Table 10.2.z.4.1-1: <state> CREATE
	<state> CREATE 

	Information in Request message
	All parameters defined in table 8.1.2-3 apply with the specific details for:

Content: The resource content shall provide the information as defined in clause 9.6.x

	Processing at Originator before sending Request
	According to clause 10.1.2

	Processing at Receiver
	According to clause 10.1.2
The Receiver shall check the existence and accessibility of the <state> resource(s) defined in the stateTransitions attribute, the <action> resource defined in stateAction, and any resources referenced by these resources. If any of the referenced resources do not exist or are not accessible by the Originator, the Receiver responds with an error.

Once the <state> resource is successfully created, the Receiver shall use the <state> resource per the process management procedure as defined in 10.2.z.2 and 10.2.z.3.

	Information in Response message
	According to clause 10.1.2

	Processing at Originator after receiving Response
	According to clause 10.1.2

	Exceptions
	According to clause 10.1.2


10.2.z.4.2
Retrieve <state>
This procedure shall be used for retrieving the attributes of a <state> resource.

Table 10.2. z.4.2-1: <state> RETRIEVE
	<state> RETRIEVE

	Information in Request message
	All parameters defined in table 8.1.2-2 apply.

	Processing at Originator before sending Request
	According to clause 10.1.3

	Processing at Receiver
	According to clause 10.1.3

	Information in Response message
	All parameters defined in table 8.1.3-1 apply.

	Processing at Originator after receiving Response
	According to clause 10.1.3

	Exceptions
	According to clause 10.1.3


10.2.z.4.3
Update <state>
This procedure shall be used for updating attributes of a <state> resource.

Table 10.2. z.4.3-1: <state> UPDATE
	<state> UPDATE

	Information in Request message


	All parameters defined in table 8.1.2-2 apply.

	Processing at Originator before sending Request
	According to clause 10.1.4

	Processing at Receiver
	According to clause 10.1.4
If the stateStatus attribute of the targeted <state> resource is “active”, then the Receiver responds with an error.

The Receiver shall check the existence and accessibility of the <state> resource(s) defined in stateTransitions, the <action> resource defined in stateAction, and any resources referenced by these resources. If any of the referenced resources do not exist or are not accessible by the Originator, the Receiver responds with an error.

Once the <state> resource is successfully updated, the Receiver shall use the <state> resource per the process management procedure as defined in 10.2.z.2 and 10.2.z.3.

	Information in Response message
	According to clause 10.1.4

	Processing at Originator after receiving Response
	According to clause 10.1.4

	Exceptions
	According to clause 10.1.4


10.2.z.4.4
Delete <state>
This procedure shall be used for deleting a <state> resource.
Table 10.2. z.4.4-1: <state> DELETE
	<state> DELETE

	Information in Request message
	All parameters defined in table 8.1.2-2 apply

	Processing at Originator before sending Request
	According to clause 10.1.5

	Processing at Receiver
	According to clause 10.1.5
If the stateStatus attribute of the targeted <state> resource is “active”, the Receiver responds with an error.



	Information in Response message
	According to clause 10.1.5

	Processing at Originator after receiving Response
	According to clause 10.1.5

	Exceptions
	According to clause 10.1.5


10.2.z.5 
<processManagement> Resource Procedures

10.2.z.5.1 
Create <processManagement>
The Hosting CSE shall use this procedure to create a <processManagement> resource according to the resource identifier of the parent resource.

Table 10.2.z.5.1-1: <processManagement> CREATE
	< processManagement > CREATE 

	Information in Request message
	All parameters defined in table 8.1.2-3 apply with the specific details for:

Content: The resource content shall provide the information as defined in clause 9.6.y

	Processing at Originator before sending Request
	According to clause 10.1.2

	Processing at Receiver
	According to clause 10.1.2
The Receiver shall check the existence and accessibility of the resource(s) referenced in the activateConditions and endConditions attributes, and the <state> resource defined in the initialState attribute. If any of the referenced resources do not exist or are not accessible by the Originator, the Receiver responds with an error.

Once the <processManagement> resource is successfully created, the Receiver shall perform the process management procedure as defined in 10.2.z.2 and 10.2.z.3.

	Information in Response message
	According to clause 10.1.2

	Processing at Originator after receiving Response
	According to clause 10.1.2

	Exceptions
	According to clause 10.1.2


10.2.z.5.2
Retrieve <processManagement>
This procedure shall be used for retrieving the attributes of a <processManagement> resource.

Table 10.2. z.5.2-1: <processManagement> RETRIEVE
	<processManagement> RETRIEVE

	Information in Request message
	All parameters defined in table 8.1.2-2 apply.

	Processing at Originator before sending Request
	According to clause 10.1.3

	Processing at Receiver
	According to clause 10.1.3

	Information in Response message
	All parameters defined in table 8.1.3-1 apply.

	Processing at Originator after receiving Response
	According to clause 10.1.3

	Exceptions
	According to clause 10.1.3


10.2.z.5.3
Update <processManagement>
This procedure shall be used for updating attributes of a <processManagement> resource.

Table 10.2. z.5.3-1: <processManagement> UPDATE
	<processManagement> UPDATE

	Information in Request message


	All parameters defined in table 8.1.2-2 apply.

	Processing at Originator before sending Request
	According to clause 10.1.4

	Processing at Receiver
	According to clause 10.1.4
If the processStatus attribute has a value other than “Disabled” and an attribute other than processControl is requested to be updated, the Receiver responds with an error.

If the processControl is requested to be updated, the Receiver shall check whether it is a legal transition as defined by Figure 10.2.z.2-1. If the transition is not legal, the Receiver responds with an error.

The Receiver shall check the existence and accessibility of the resource(s) referenced in the activateConditions and endConditions attributes, and the <state> resource defined in initialState attribute. If any of the referenced resourcee do not exist or are not accessible by the Originator, the Receiver responds with an error.

After successfully updating the <processManagement> resource, the Receiver shall perform the procedure specified in 10.2.z.2 and 10.2.z.3.

	Information in Response message
	According to clause 10.1.4

	Processing at Originator after receiving Response
	According to clause 10.1.4

	Exceptions
	According to clause 10.1.4


10.2.z.5.4
Delete <processManagement>
This procedure shall be used for deleting a <processManagement> resource.
Table 10.2. z.5.4-1: <processManagement> DELETE
	<processManagement> DELETE

	Information in Request message
	All parameters defined in table 8.1.2-2 apply

	Processing at Originator before sending Request
	According to clause 10.1.5

	Processing at Receiver
	According to clause 10.1.5
If the processStatus is “Activated” or “Paused”, the Receiver responds with an error.



	Information in Response message
	According to clause 10.1.5

	Processing at Originator after receiving Response
	According to clause 10.1.5

	Exceptions
	According to clause 10.1.5


-----------------------End of change 2---------------------------------------------

-----------------------Start of change 3---------------------------------------------
9.6
Resource Types

9.6.1
Overview

9.6.1.1
Resource Type Summary

Table 9.6.1.1-1 introduces the normal and virtual resource types and their related child or parent resource types. Details of each resource type follow in the remainder of this clause.

Table 9.6.1.1-1 lists each specified ordinary – i.e. not announced – resource type. An addition of suffix "Annc" to the respective resource type identifier indicates the associated announced resource type. Resource types that can occur as child resources of announced resources are summarized in Table 9.6.26.1-1 "Announced Resource Types".
Among the resource types listed in Table 9.6.1.1-1, the following are termed "Content Sharing Resources" in oneM2M Specifications for the purpose of referring to any of those resource types:

· container;

· contentInstance;

· flexContainer;
· flexContainerInstance;
· timeSeries;
· timeSeriesInstance.

Table 9.6.1.1-1: Resource Types 

	Resource Type
	Short Description
	Child Resource Types
	Parent Resource Types
	Clause

	accessControlPolicy
	Stores a representation of privileges. It is associated with resources that shall be accessible to entities external to the Hosting CSE. It controls "who" is allowed to do "what" and the context in which it can be used for accessing resources
	subscription, transaction
	AE, AEAnnc, remoteCSE, remoteCSEAnnc, CSEBase
	9.6.2

	AE
	Stores information about the AE. It is created as a result of successful registration of an AE with the Registrar CSE
	subscription, container, 
flexContainer,
group, accessControlPolicy, 
pollingChannel, semanticDescriptor,
timeSeries, transaction, transactionMgmt,

triggerRequest, crossResourceSubscription, backgroundDataTransfer, semanticMashupInstance, locationPolicy, action, multimediaSession, dynamicAuthorizationConsultation, nwMonitoringReq, timeSyncBeacon, primitiveProfile
	CSEBase
	9.6.5

	container
	Shares data instances among entities. Used as a mediator that buffers data exchanged between AEs and/or CSEs. The exchange of data between AEs (e.g. an AE on a Node in a field domain and the peer-AE on the infrastructure domain) is abstracted from the need to set up direct connections and allows for scenarios where both entities in the exchange do not have the same reachability schedule
	container, 
flexContainer, contentInstance, subscription, latest, oldest，semanticDescriptor, timeSeries, transaction, action
	AE, AEAnnc, container, containerAnnc, remoteCSE, remoteCSEAnnc, 
CSEBase,
flexContainer, flexContainerAnnc
	9.6.6

	contentInstance
	Represents a data instance in the <container> resource
	semanticDescriptor, transaction
	Container, containerAnnc
	9.6.7

	flexContainer
	A template which allows to define specialized (customizable) versions of containers with a flexible and lightweight structure 
	container, 

flexContainer, flexContainerInstance, latest, oldest, subscription, semanticDescriptor, timeSeries, transaction, action
	AE, AEAnnc, container, containerAnnc, 

flexContainer, flexContainerAnnc, remoteCSE, remoteCSEAnnc, 
CSEBase
	9.6.35

	flexContainerInstance
	Represents a timestamped Data instance in the <flexContainer> resource
	 transaction, semanticDescriptor
	flexContainer
	9.6.59

	CSEBase
	The structural root for all the resources that are residing on a CSE. Stores information about the CSE itself
	remoteCSE, remoteCSEAnnc, node, AE, container, group, accessControlPolicy, subscription, mgmtCmd, locationPolicy, statsConfig, statsCollect, request, delivery,

schedule,
notificationTargetPolicy, semanticRuleRepository, 
flexContainer,
timeSeries, AEContactList, transaction, transactionMgmt, crossResourceSubscription, backgroundDataTransfer, semanticMashupJobProfile, semanticMashupInstance, action, m2mServiceSubscriptionProfile, dynamicAuthorizationConsultation, localMulticastGroup, serviceSubscribedAppRule, authorizationPolicy, authorizationDecision, authorizationInformation, ontologyRepository, nwMonitoringReq, timeSyncBeacon, primitiveProfile
	None specified
	9.6.3

	delivery
	Forwards requests from CSE to CSE
	subscription, transaction
	CSEBase
	9.6.11

	eventConfig
	Defines events that trigger statistics collection
	subscription, transaction
	statsConfig
	9.6.24

	execInstance
	Contains all execution instances of the same Management Command
	subscription, transaction
	mgmtCmd
	9.6.17

	fanOutPoint (V)
	Virtual resource containing target for group request

It is used for addressing bulk operations to all the resources that belong to a group
	None specified
	group
	9.6.14

	group
	Stores information about resources of the same type that need to be addressed as a Group. Operations addressed to a Group resource shall be executed in a bulk mode for all members belonging to the Group
	fanOutPoint,

subscription,
semanticFanOutPoint, semanticDescriptor, transaction, action
	AE, AEAnnc, remoteCSE, remoteCSEAnnc, CSEBase
	9.6.13

	latest (V)
	Virtual resource that points to most recently created <contentInstance> , <timeSeriesInstance> or <flexContainerInstance> child resource within a <container>,  <timeSeries> or <flexContainer> resource
	None specified
	container, timeSeries, or flexContainer
	9.6.27

	locationPolicy
	Includes information to obtain and manage geographical location. It is only referenced within a container, the contentInstances of the container provide location information
	subscription, transaction
	CSEBase, AE
	9.6.10

	mgmtCmd
	Management Command resource represents a method to execute management procedures required by existing management protocols
	execInstance,

subscription, transaction
	CSEBase
	9.6.16

	mgmtObj
	Management Object resource represents management functions that provides an abstraction to be mapped to external management technology. It represents the node and the software installed in the node (see note)
	subscription, transaction, semanticDescriptor
	node, mgmtObjAnnc
	9.6.15

Annex D

	m2mServiceSubscriptionProfile
	Data pertaining to the M2M Service Subscription
	serviceSubscribedNode,

subscription, transaction, serviceSubscribedUserProfile
	CSEBase 
	9.6.19

	node
	Represents specific Node information
	mgmtObj, 
subscription, semanticDescriptor, schedule, transaction, action
	CSEBase
	9.6.18

	notificationTargetMgmtPolicyRef
	Represents a list of notification targets and the deletion policy
	subscription, transaction
	subscription
	9.6.31

	notificationTargetPolicy
	Represents a notification target deletion policy with pre-defined action and deletion rules
	subscription, policyDeletionRules, transaction
	CSEBase
	9.6.32

	notificationTargetSelfReference (V)
	Virtual resource used to remove the Notification Target
	None specified
	subscription
	9.6.34

	oldest (V)
	Virtual resource that points to first created <contentInstance>, <timeSeriesInstance> or <flexContainerInstance> child resource within a <container>, <timeSeries> or <flexContainer> resource
	None specified
	container, timeSeries, or flexContainer
	9.6.28

	pollingChannel
	Represent a channel that can be used for a request-unreachable entity
	pollingChannelURI
	remoteCSE, AE
	9.6.21

	pollingChannelURI (V)
	Virtual resource used to perform service layer long polling of a resource Hosting CSE by a request-unreachable entity
	None specified
	pollingChannel
	9.6.22

	policyDeletionRules
	Represents a set of rules which is associated with notification target removal policy
	subscription, transaction
	notificationTargetPolicy
	9.6.33

	remoteCSE
	Represents a remote CSE for which there has been a registration procedure with the registrar CSE identified by the CSEBase resource
	container, containerAnnc,
contentInstanceAnnc 
flexContainer, flexContainerAnnc,
group, groupAnnc, accessControlPolicy, accessControlPolicyAnnc, subscription, pollingChannel, 
timeSeries,
timeSeriesAnnc, timeSeriesInstanceAnnc,
mgmtObjAnnc,
nodeAnnc,

AEAnnc,

locationPolicyAnnc, transaction, crossResourceSubscription, backgroundDataTransfer, semanticMashupJobProfile, semanticMashupJobProfileAnnc,  semanticMashupInstance,

semanticMashupInstanceAnnc, 

action, dynamicAuthorizationConsultation, dynamicAuthorizationConsultationAnnc, nwMonitoringReq, timeSyncBeacon, primitiveProfile
	CSEBase
	9.6.4

	request
	Expresses/access context of an issued Request
	subscription, transaction
	CSEBase
	9.6.12

	schedule
	Contains scheduling information for delivery of messages
	subscription, transaction
	subscription, CSEBase, 
· node
	9.6.9

	serviceSubscribedNode
	Node information
	subscription, transaction
	m2mServiceSubscriptionProfile
	9.6.20

	statsCollect
	Defines triggers for the IN-CSE to collect statistics for applications
	subscription, transaction
	CSEBase (in IN‑CSE)
	9.6.25

	statsConfig
	Stores configuration of statistics for applications
	eventConfig,

subscription, transaction
	CSEBase (in IN‑CSE)
	9.6.23

	subscription
	Subscription resource represents the subscription information related to a resource. Such a resource shall be a child resource for the subscribed-to resource
	schedule, notificationTargetSelfReference, notificationTargetMgmtPolicyRef, transaction
	accessControlPolicy, accessControlPolicyAnnc, AE, AEAnnc, container, containerAnnc, CSEBase, delivery, eventConfig, execInstance, group, groupAnnc, locationPolicy, locationPolicyAnnc, mgmtCmd, mgmtObj, mgmtObjAnnc, m2mServiceSubscriptionProfile, node, nodeAnnc, serviceSubscribedNode, remoteCSE, remoteCSEAnnc, request, schedule, scheduleAnnc,
semanticDescriptor, semanticDescriptorAnnc, statsCollect, statsConfig,
flexContainer, flexContainerAnnc,
timeSeries, timeSeriesAnnc, nwMonitoringReq, semanticRuleRepository, reasoningRules, reasoningJobInstance, timeSyncBeacon, primitiveProfile,
state,

processManagement
	9.6.8

	serviceSubscribedAppRule
	Represents a rule that defines allowed App-ID and AE-ID combinations that are acceptable for registering an AE on a Registrar CSE
	subscription, transaction
	CSEBase
	9.6.29

	semanticDescriptor
	Stores semantic description pertaining to a resource and potentially sub-resources.
	subscription, transaction
	AE, container, contentInstance, group, node, flexContainer, timeSeries, mgmtObj
	9.6.30

	semanticFanOutPoint
	Virtual resource used as target for semantic discovery aimed at a logical graph distributed over multiple semanticDescriptor resources, which belong to the corresponding group parent resource
	
	group
	9.6.14a

	dynamicAuthorizationConsultation
	Represents consultation information used by a CSE when performing consultation-based dynamic authorization
	subscription, transaction
	AE, AEAnnc, remoteCSE, remoteCSEAnnc, CSEBase
	9.6.40

	timeSeries
	Stores and Shares Time Series Data instances among entities.
	timeSeriesInstance, subscription, semanticDescriptor,

latest, oldest, transaction, action
	AE, AEAnnc, remoteCSE, remoteCSEAnnc, CSEBase,

container, containerAnnc, flexContainer, flexContainerAnnc
	9.6.36

	timeSeriesInstance
	Represents a Time Series Data instance in the <timeSeries> resource
	 transaction
	timeSeries, timeSeriesAnnc
	9.6.37

	role
	Represents a role that is assigned to an AE or CSE.
	subscription, transaction
	authorizationInformation
	9.6.38

	token
	Used for storing a token that is issued to an AE or CSE.
	subscription, transaction
	authorizationInformation
	9.6.39

	authorizationDecision
	Represents an access control decision point
	subscription, transaction
	CSEBase
	9.6.41

	authorizationPolicy
	Represents an access control policy retrieval point
	subscription, transaction
	CSEBase
	9.6.42

	authorizationInformation
	Represents an access control information point
	role
token
subscription, transaction
	CSEBase
	9.6.43

	localMulticastGroup
	Stores local multicast group information of member hosting CSE.
	 transaction
	CSEBase
	9.6.44

	AEContactList
	Contains information about a CSE that has resources that referencing an AE-ID
	AEContactListPerCSE, subscription, transaction
	CSEBase
	9.6.45

	AEContactListPerCSE
	Contains information about a CSE that has resources that referencing an AE resource identifier for tracking purposes
	None specified
	AEContactList
	9.6.46

	transactionMgmt
	
	subscription
	CSEBase, AE, remoteCSE
	9.6.47

	transaction
	
	action
	All non-virtual resource types with the exception of the following:

request, delivery, pollingChannel, transactionMgmt, transaction, timeSyncBeacon
	9.6.48

	triggerRequest
	Used by an AE to initiate, replace or recall a device trigger request 
	subscription
	AE
	9.6.49

	ontologyRepository
	Represents the collection of the managed ontologies and the semantic validation service
	ontology, semanticValidation, subscription
	CSEBase
	9.6.50

	ontology
	Store the representation of an ontology
	subscription
	ontologyRepository
	9.6.51

	semanticValidation
	A virtual resource as the interface to perform semantic validation on the received <semanticDescriptor> resource against the referenced ontology.
	None specified
	ontologyRepository
	9.6.52

	semanticMashupJobProfile
	Represents the profile and description of a semantic mashup service
	semanticMashupInstance, semanticDescriptor, subscription
	CSEBase, remoteCSE
	9.6.53

	semanitcMashupInstance
	Represents a semantic mashup instance
	semanticMashupResult, semanticDescriptor, mashup, subscription
	semanticMashupJobProfile, AE, remoteCSE, CSEBase
	9.6.54

	mashup
	A virtual resource use to trigger the calculation and generation of new mashup result
	Not specified
	semanticMashupInstance
	9.6.55

	semanticMashupResult
	Represent semantic mashup results
	semanticDescriptor, subscription
	semanticMashupInstance
	9.6.56

	multimediaSession
	Stores a representation of a multimedia session information requested by a registering AE
	subscription 
	AE
	9.6.57

	crossResourceSubscription
	represents the cross-resource subscription information related to multiple subscribed-to resources. Such a resource shall include a list of subscribed-to resources as its attribute, or shall be created as a child resource of a <group> resource where member resources shall be the subscribed-to resources. 
	schedule, notificationTargetSelfReference, notificationTargetMgmtPolicyRef, transaction 
	CSEBase, remoteCSE, AE
	9.6.58

	backgroundDataTransfer
	Stores information for a background data transfer request
	None specified
	AE, remoteCSE, CSEBase
	9.6.60

	action
	Specifies the action(s) that is performed whenever an event is triggered at the <dependency> resource
	dependency
	CSEBase, remoteCSE, node, AE, container, flexContainer, group, timeSeries, transaction, state
	9.6.61

	dependency
	Specifies the condition(s) of a monitored event which triggers the operation(s) specified by the <action> resource
	None specified
	action
	9.6.62

	e2eQosSession
	Specifies the end-to-end (E2E) QoS session requirements for the exchange of oneM2M request and response primitives between oneM2M entities.
	subscription 
	CSEBase, remoteCSE, AE
	9.6.63

	nwMonitoringReq
	Specifiesed the request that an AE retrieves the Underlying Network information. The resource provides the characteristics of the Underlying Network status in a particular geographic area such as congestion status and number of devices.
	subscription 
	CSEBase, remoteCSE, AE
	9.6.64

	semanticRuleRepository
	Specifies one or multiple <reasoningRules> child resources to represent different sets of reasoning rules in the oneM2M system
	reasoningRules, reasoningJobInstance
	CSEBase
	9.6.65

	reasoningRules
	Store a set of related reasoning rules
	subscription
	semanticRuleRepository
	9.6.66

	reasoningJobInstance
	Represents a specific reasoning job instance for enabling one-time or continuous reasoning operations.
	subscription
	semanticRuleRepository
	9.6.67

	serviceSubscribedUserProfile
	Stores user profile information for a given M2M Service User
	subscription, transaction
	m2mServiceSubscriptionProfile
	9.6.68

	timeSyncBeacon
	Specifies criteria that a Hosting CSE uses to generate time synchronization beacon notifications 
	subscription,

transaction
	CSEBase, remoteCSE, AE
	9.6.69

	primitiveProfile
	Stores message parameters and/or resource attributes that can be applied (i.e. added, removed or modified) to a request or a response primitive. 
	subscription, transaction
	CSEBase, remoteCSE, AE 
	9.6.73

	state
	Stores status information and actions for a given state of a process that is managed by a Hosting CSE. 
	subscription, transaction, action
	processManagement
	9.6.x

	processManagement
	Specifies a process that a Hosting CSE manages on behalf of an AE that consists of sequence of states defined by <state> resources. 
	state, subscription, transaction
	CSEBase, remoteCSE, AE
	9.6.y

	NOTE:
See clause 9.6.12 for a summary of specializations of <mgmtObj>.


-----------------------End of change 3---------------------------------------------

CHECK LIST

· Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?
· Does this CR contain changes related to only one particular issue/problem?
· Have any mirror CRs been posted?
· Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?
· Are all pictures editable?
· Have you checked the spelling and grammar?
· Have you used change bars for all modifications?
· Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)
· Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?
[image: image3.png]


AE
<processManagement> Hosting CSE
Referenced resource Hosting CSE
Referenced resource Hosting CSE
...
1. Create <processManagement> resource
3. Create <action> resource(s)
2. Create <state> resource(s)
4a. processControl = Enable
4b. Check the existence and accessibility of resources related to process
5b. processStatus = Activate;
currentState = initialState;
Monitor endConditions
5a. activateConditions met 
or equal to NULL
4c. processStatus = Enabled;
Monitor activateConditions
6. Transition through states and perform actions related to process
7a. endConditions met
Or
stateTransitions = NULL
7b. processStatus = Completed;
All stateStatus = inactive;
currentState = empty;
Stop monitoring exitConditions



Enabled
Completed
Disabled
Paused
Activated
processControl set to Enable
activateConditions are NULL or have been met
currentState equals finalState or endConditions have been met
<processManagement> created
Aborted
processControl set to Disable
stateTransitions criteria have been met
processControl set to Activate
processControl set to Pause
processControl set to Disable
processControl set to Enable
processControl set to Disable
processControl set to Enable



