	Doc# SDS-2020-0068-DAS_procedure_modifications_(R4).doc
Change Request
	[image: image2.png]






	


	CHANGE REQUEST

	Meeting ID:*
	SDS#44

	Source:*
	SeungMyeong JEONG, KETI, sm.jeong@keti.re.kr 
Norihiro Okui, KDDI, no-okui@kddi-research.jp

	Date:*
	2020-02-19

	Reason for Change/s:*
	To modify the CSE-DAS procedurs

	CR  against:  Release*
	4

	CR  against:  WI*
	 FORMCHECKBOX 
 Active
 FORMCHECKBOX 
 MNT maintenance / < Work Item number(optional)>
Is this a mirror CR? Yes  FORMCHECKBOX 
 No  FORMCHECKBOX 

mirror CR number: SDS-2020-0052R01
 FORMCHECKBOX 
 STE Small Technical Enhancements / < Work Item number (optional)>
Only ONE of the above shall be ticked

	CR  against:  TS/TR*
	TS-0004 v4.0.0

	Clauses *
	

	Type of change: *
	 FORMCHECKBOX 
 Editorial change
 FORMCHECKBOX 
 Bug Fix or Correction
 FORMCHECKBOX 
 Change to existing feature or functionality
 FORMCHECKBOX 
 New feature or functionality
Only ONE of the above shall be ticked

	Impacted other TS/TR(s)
	n/a

	Post Freeze checking:*
	This CR contains only essential changes and corrections?  YES  FORMCHECKBOX 
  NO  FORMCHECKBOX 

This CR may break backwards compatibility with the last approved version of the TS?       YES 
  NO 

	Template Version: January 2017 (Do not modify)


oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.
In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR
Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).
Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.
Follow the drafting rules.
All pictures must be editable.
Check spelling and grammar to the extent practicable.
Use Change bars for modifications.
The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.
Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.
When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text. 
Introduction
[change 1]

As per default authorization scheme, <accessControlPolicy> resource based, to get the corresponding access previleges to a request, the targeted resource identifier should be given. However, the current dynamic auth request from a CSE to a DAS does not madate the targeted resource identifier (trid) but targeted resource type (trt).
trt would be good to have as additional info. for DAS to efficiently fetch the matching privilges in its local DB, but does does not qualifies the fact that it is mandatory

[change 2]

As defined in the TS-0001, the different authorization schemes applying sequence says there are other changes, if supported by a CSE, when an Originator gets rejected by one scheme (e.g. <ACP> based). From the flow chart diagram, even if an Originator gets denied per DAS-based authorization, distributed access control can be applied if the system supports. 
One of the procedure immediately sends back error when the conditions are not met by the dynamic ACP info. given by DAS, so it should give another chance.

[image: image1.emf]Authorize using 

selfPrivileges

Target is 

<accessControlPolicy>

resource?

Request Received

Authorized?

accessControlPolicyIDs

 is NULL?

Authorized?

Indirect Dynamic 

Authorization Supported

Request contains 

Token?

Authorize using 

token(s)

Authorized?

Direct Dynamic 

Authorization Supported

dynamicAuthorizationConsultationIDs 

is not NULL?

Authorize by 

consulting DAS

Distributed Authorization 

Supported

(

authorizationDecisionResourceIDs, authorizationPolicyResourceIDs 

and authorizationInformationResourceIDs

are not NULL)

Authorize using 

distributed 

authorization

Authorized?

Authorized?

Access granted –�

process request

Access denied - 

reject request

Return Response

No

Yes

Yes

Yes

No

No

No

Yes

Yes

No

Yes

No

Yes

No

No

Yes

Yes

Access granted –�

process request

No

Yes

Authorized?

No

No

Authorize using 

default access 

privileges

Authorized?

Yes

No

Authorize using 

<accessControlPolicy> 

privileges 

Yes

Target resource supports 

accessControlPolicyIDs 

attribute?

Yes

No

Authorize using parent¶s 

accessControlPolicyIDs 

privileges 


Figure 10.2.3.1-1: Different types of authorization flow chart (TS-0001)
********** Start of change 1 **********

6.3.5.46 m2m:dynAuthDasRequest

Used in m2m:securityInfo.

Table 6.3.5.46‑1: Type Definition of m2m:dynAuthDasRequest
	Element Path
	Element Data Type 
	Multiplicity
	Note

	originator
	m2m:ID
	1
	

	targetedResourceType
	m2m:resourceType
	0..1
	

	operation
	m2m:operation
	1
	

	originatorIP
	(anonymous)
	0..1
	

	originatorIP/ipv4Address
	m2m:ipv4
	0..1
	These elements shall only be present if the originatorIP parent element is present. Exactly one of these elements shall be present

	originatorIP/ipv6Address
	m2m:ipv6
	0..1
	

	originatorLocation
	m2m:locationRegion
	0..1
	

	originatorRoleIDs
	list of m2m:roleID
	0..1
	

	requestTimestamp
	m2m:absRelTimestamp
	0..1
	

	targetedResourceID
	xs:anyURI
	1
	

	proposedPrivilegesLifetime
	m2m:absRelTimestamp
	0..1
	

	roleIDsFromACPs
	list of m2m:roleID
	0..1
	

	tokenIDs
	list of m2m:tokenID
	0..1
	

	authorSignIndicator
	xs:boolean
	0..1
	


********** End of change 1 **********

********** Start of change 2 **********

7.5.1.2.10 Notification for Dynamic Authorization

Dynamic Authorization uses a two-way exchange of information between a Hosting CSE (i.e. Originator) and a Dynamic Authorization System (DAS) Server (i.e. Receiver). This two-way handshake is performed using a Notification request and response. 

Originator:

When the Originator (i.e. Hosting CSE) is triggered to perform dynamic authorization for an incoming request that it receives, then it performs the following steps in order:

1) Configure the To parameter with the address of the corresponding DAS Server associated with the resource targeted by the received request. The Hosting CSE shall use the DAS Server address information configured within the dynamicAuthorizationPoA attribute of the <dynamicAuthorizationConsultation> resource associated with the targeted resource. The Hosting CSE shall determine the corresponding <dynamicAuthorizationConsultation> resource using the dynamicAuthorizationConsultationIDs attribute of the targeted resource. If the attribute is not supported by the targeted resource, or it is not set, or it has a value that does not correspond to a valid <dynamicAuthorizationConsultation> resource(s), or it refers to a <dynamicAuthorizationConsultation> resource(s) that is not reachable, then based on system policies, the dynamicAuthorizationConsultationIDs associated with the parent may apply to the child resource if present, or a system default <dynamicAuthorizationConsultation> may apply if present. If a dynamicAuthorizationConsultationID attribute and corresponding <dynamicAuthorizationConsultation> resource cannot be found or if the dynamicAuthorizationEnabled of a <dynamicAuthorizationConsultation> has a value of false and Distributed Authorization is not supported (see TS-0001 clause 10.2.3), the Hosting CSE shall reject the request by returning an "ORIGINATOR_HAS_NO_PRIVILEGE" Response Status Code to the Originator of the received request and no additional steps shall be performed.

2) Configure the From parameter with the ID of the Hosting CSE which hosts the resource targeted by the received request.

3) Configure the mandatory sub-elements of the securityInfo element of the notification data:

a)
The securityInfoType element shall be configured as "1" (Dynamic Authorization Request) in the Notify request primitive.

b)
The originator element shall be configured with the ID of the Originator of the received request.

c)
The targetedResourceType element shall be configured with the type of resource targeted by the received request.

d)
The operation element shall be configured with the type of operation targeted by the received request.

4) Optionally configure one or more optional sub-elements of the securityInfo element of the notification data:

a)
The originatorIP element may be configured with the IP address of the Originator of the received request.

b)
The originatorLocation may be configured with the location of the Originator of the received request.

c)
The originatorRole may be configured with the role of the Originator of the received request.

d)
The requestTimestamp may be configured with the time at which the request was received.

e)
The targetedResourceID may be configured with the identifier of the targeted resource of the received request.

f)
The proposedPrivilegesLifetime may be configured with a time duration for which the Hosting CSE is requesting privileges be granted to the Originator of the received request.

g)
The rolesFromACPs may be configured with a list of roles specified in the ACPs associated with the resource targeted by the received request.

h)
The tokenIDs may be configured with a list of token identifiers specified by the Originator of the received request.
i)
The authorSignIndicator may be configured with the value specified by the received request.

5) The Hosting CSE shall send the notification request for dynamic authorization to the targeted DAS Server.

Receiver:

When the DAS Server receives a notification request for dynamic authorization, it processes the request and returns a notification response for dynamic authorization to the originating Hosting CSE. 

NOTE 1:
The details of how the DAS Server processes the notification request for dynamic authorization are not visible to the oneM2M system, and are not addressed in the present document.

Originator:

When the Hosting CSE receives a notification response for dynamic authorization, it performs the following steps in order:

1) The Hosting CSE shall verify that the securityInfoType element of the securityInfo element of the notification is configured as "2" (Dynamic Authorization Response). If it is not, the Hosting CSE shall not grant privileges to the Originator of the request for which the Hosting CSE was attempting dynamic authorization. If Distributed Authorization is not supported (see TS-0001 clause 10.2.3), the Hosting CSE shall reject the request by returning an "ORIGINATOR_HAS_NO_PRIVILEGE" Response Status Code to the Originator of the received request and no additional steps shall be performed.

2) The Hosting CSE shall check whether the response contains a dynamicACPInfo element. If present, the Hosting CSE shall create a <accessControlPolicy> child resource under the targeted resource and configure its privileges using the dynamicACPInfo. In this case, the Hosting CSE shall configure the privileges attribute with the grantedPrivileges and the expirationTime attribute with the privilegesLifetime. The Hosting CSE shall also configure the selfPrivileges attribute to allow itself to perform Update/Retrieve/Delete operations on the newly created <accessControlPolicy> resource.

3) The Hosting CSE shall check whether the response contains a tokens element. If present the Hosting CSE shall perform verification and caching of the token as specified in clause 7.3.2 in oneM2M TS-0003 [7] and the Hosting CSE shall check whether an AuthorSignReqInfo is contained in the response. If it is present, the Hosting CSE shall add the AuthorSignReqInfo into the response to the Originator of the incoming request. 

NOTE 2:
The Hosting CSE uses the information in the DAS response for authorization, see clause 7.3.3.15.

********** End of change 2 **********

CHECK LIST

· Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?
· Does this CR contain changes related to only one particular issue/problem?
· Have any mirror CRs been posted?
· Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?
· Are all pictures editable?
· Have you checked the spelling and grammar?
· Have you used change bars for all modifications?
· Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)
· Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?
© 2020 oneM2M Partners
                                                                                                   Page 18 (of 22)



[image: image2.png]_1643199816.vsd
Authorize using selfPrivileges



