	SDS-2020-0113-TS-0003_context_aware_acps_R4
	[image: image1.png]






	


	CHANGE REQUEST

	Meeting ID:*
	SDS 45

	Source:*
	Bob Flynn, Convida, flynn.bob@convidawireless.com 

Dale Seed, Convida, seed.dale@convidawireless.com 

	Date:*
	2020-04-15

	Reason for Change/s:*
	Add support for context aware ACPs

	CR  against:  Release*
	Release 4

	CR  against:  WI*
	 FORMCHECKBOX 
 Active - WI-0077 - Attribute Based Access Control Policy
 FORMCHECKBOX 
 MNT maintenance / < Work Item number(optional)>
Is this a mirror CR? Yes  FORMCHECKBOX 
 No  FORMCHECKBOX 

mirror CR number: (Note to Rapporteur - use latest agreed revision)
 FORMCHECKBOX 
 STE Small Technical Enhancements / < Work Item number (optional)>
Only ONE of the above shall be ticked

	CR  against:  TS/TR*
	TS-0003 V4.1.0

	Clauses *
	7.1.3

	Type of change: *
	 FORMCHECKBOX 
 Editorial change
 FORMCHECKBOX 
 Bug Fix or Correction
 FORMCHECKBOX 
 Change to existing feature or functionality
 New feature or functionality
Only ONE of the above shall be ticked

	Impacted other TS/TR(s)
	TS-0004 Rel-4

	Post Freeze checking:*
	This CR contains only essential changes and corrections?  YES  FORMCHECKBOX 
  NO  FORMCHECKBOX 

This CR may break backwards compatibility with the last approved version of the TS?       YES  FORMCHECKBOX 
  NO  FORMCHECKBOX 



	Template Version: January 2017 (Do not modify)


oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.
In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR
Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).
Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.
Follow the drafting rules.
All pictures must be editable.
Check spelling and grammar to the extent practicable.
Use Change bars for modifications.
The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.
Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.
When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text. 
Introduction
In TS-0001, support has been added for accessControlEvalCriteria and accessControlLimit parameters within the the accessControlContexts attribute of an ACP privilege.

This contribution adds corresponding changes to TS-0003.

-----------------------Start of change 1-------------------------------------------
7.1.3
Format of privileges and selfPrivileges Attributes

The privileges and selfPrivileges attributes exhibit the same data type format which is specified as follows.

Each privileges or selfPrivileges attribute comprises a set of access control rules. In the following, the set of access control rules is denoted as acrs and an individual access control rule in this set as acr. The access control rules in acrs are indexed with the letter k. The number of access control rules in the set is denoted with the letter K:


acrs = { acr(1), acr(2), ..., acr(k), ..., acr(K) }

Each access control rule acr(k) is comprised of three type of components, denoted accessControlOriginators, accessControlOperations and accessControlContexts. The accessControlContext component is an optional parameter.

Hence, an access control rule acr(k) is either represented as a pair:


acr(k) = {acr(k)_accessControlOriginators, acr(k)_accessControlOperations} 

or as a 3-tuple:


acr(k) = {acr(k)_accessControlOriginators, acr(k)_accessControlOperations, acr(k)_accessControlContexts}

The generic term "access-control-rule-tuple" is used when referring to a rule acr(k).

A set acrs of access control rules may consist of a mix of pairs and 3-tuples. For pairs, any context parameters associated with a request message are admissible.

The three component parameters of an access-control-rule-tuple supported in the present document are shown in table 7.1.3-1.

Table 7.1.3-1: Parameters of an access-control-rule-tuple

	Parameter
	Usage Description
	Mandatory/Optional
	Format

	accessControlOriginators
	Set of Originators that can be authorized
	M
	List of CSE-IDs and/or AE‑IDs, or keyword "all" to grant access to all originators

	accessControlOperations
	Set of Operations that can be authorized
	M
	Enumerated list of operations Create Retrieve, Update, Delete, Discover, Notify

	accessControlContexts
	See table 7.1.3-3
	O
	See table 7.1.3-3

	accessControlObjectDetails
	See table 7.1.3-2
	O
	See table 7.1.3-2

	accessControlAuthenticationFlag
	Indicates whether the rule applies only to Originators which are considered to be authenticated by the Hosting CSE 
	O
	Boolean

	accessControlAttributes
	Set of resource attributes for which access can be authorized 
	O
	List of resource attribute name(s). 


The accessControlOriginators parameter comprises a list of SP domain names, CSE-IDs, AE-IDs, resource-IDs of <group> resources and/or Role IDs of any format defined in oneM2M TS‑0001 [1]. If access for all originators is to be allowed, the reserved keyword "all" may be included into the value space of accessControlOriginators.

Using a SP domain name in accessControlOriginators means all AE-IDs and CSE-IDs matching the given domain name can be authorized. 

It is furthermore allowed to use wildcard character "*", in representations of CSE-ID and AE‑ID. The scope of a "*" is terminated by a following "/" character. Table 7.1.3-2 shows examples of using wildcard characters in CSE-IDs and AE-IDs.

Wildcard characters are not applicable to SP domain names, resource-IDs of <group> resources and Role IDs.

Table 7.1.3-2: Examples of using wildcard characters in CSE-IDs and AE-IDs of accessControlOriginators

	
	Form of ID
	Examples
	Meaning

	CSE-ID
	Absolute 
	//m2msp.org/myCSEID 
//*/myCSEID

//*/myCSE*
	Any CSE whose ID matches the wild cards

	
	SP-relative
	/myCSEID
/myCSE*
	Any matching CSE from the SP that is hosting the target resource

	AE-ID
	Absolute
	//m2msp.org/S988
//*/myCSEID/C9886

//*/myCSE*/C9886
	Any AE whose ID matches the wild cards

	
	SP-relative
	/myCSEID/C9886
/myCSEID/C98*
/myCSE*/C98*

/SmyAE*
	Any matching AE from the SP that is hosting the target resource


The data type applicable to accessControlOriginators is defined in oneM2M TS-0004 [4].

The accessControlOperations parameter comprises a list of admissible operations which can be any subset of the following elements: Create, Retrieve, Update, Delete, Discover, and Notify. While Create, Retrieve, Update, Delete, and Notify operation are explicitly indicated in the op parameter of a request message, the Discovery operation is indicated by op = Retrieve in combination with the provisioning of fc and Disrestype parameters in the request message.

The data type applicable to accessControlOperations is defined in oneM2M TS-0004 [4]. 

The accessControlContexts parameters are listed in table 7.1.3-3.

Table 7.1.3-3: Parameters of accessControlContexts

	Parameter
	Usage Description
	Mandatory/Optional
	Formats

	accessControlTimeWindow
	Set of Time Windows that can be authorized
	O
	List of time intervals where access can be granted in extended crontab format

	accessControlLocationRegion
	Set of Location Regions that can be authorized
	O
	1)
Latitude/longitude coordinates, and a radius defining a circular region around the coordinates

2)
Country code

	accessControlIpAddress
	Set of IPv4 and IPv6 addresses that can be authorized
	O
	IPv4: dotted-decimal notation with CIDR suffix

IPv6: colon separated groups of hexadecimal digits with CIDR suffix

	accessControlUserIDs
	Set of M2M Service Users that can be authorized
	O
	List of M2M-User-IDs

	accessControlEvalCriteria
	Set of conditions that are factored into authorization decisions.
	O
	A tuple consisting of a mandatory resource identifier of a subject resource and an set of evaluation criteria applicable to the subject resource.

	accessControlLimit
	Number of times access to a resource can be authorized.
	O
	A number that indicates how many times access can be granted.   


The accessControlTimeWindow parameter represents a list of elements that comply with the extended crontab syntax as defined in clause 7.3.8 of oneM2M TS-0004 [4]. It allows definition of periodically recurring time intervals at which access can be granted, when the rq_time parameter associated with the access request message falls into such interval.

For the elements of accessControlLocationRegion there are two representation choices. These can be represented by a 2‑character country code or a circle with radius R centred at a point defined in terms of longitude and latitude parameters. Refer to Annex F for detailed information. Each element of accessControlLocationRegion defines an admissible location region, which is compared with the rq_loc parameter associated with the access request message.

The data types applicable to accessControlLocationRegion and rq_loc are defined in oneM2M TS-0004 [4].

The accessControlIpAddress parameter represents a list of IPv4 and IPv6 addresses in dotted-decimal notation with CIDR suffix or colon separated groups of hexadecimal digits with CIDR suffix, respectively. If the rq_loc parameter associated with the access request message matches one of these addresses, access may be granted with regard to this criterion.

The data types applicable to accessControlIpAddress and rq_ip are defined in oneM2M TS-0004 [4].

The accessControlUserIDs parameter comprises a list of M2M-User-IDs having a format defined in oneM2M TS‑0001 [1].
This accessControlEvalCriteria parameter represents the conditions determining if the request operation is to be allowed. It allows conditional access to the resource based on conditions not contained in the received request. The accessControlEvalCriteria parameter is a tuple that consists of a mandatory subjectResourceID element as defined in table 9.6.61-2 in oneM2M TS‑0001 [1] and an evalCriteria element defined in table 9.6.61-3 in oneM2M TS-0001 [1].
The accessControlLimit parameter represents the number of times that the policy defined by an access-control-rule-tuple can allow authorization to the requested resource. This attribute maintains the number of remaining accesses allowed. The parameter is decremented each time an access to the requested resource is granted. If the value is greater than zero then access is granted, otherwise access is denied. If the accessControlLimit parameter is not present in an access-control-rule-tuple, then there are no restrictions on the number of times access is granted.  

Using just a SP domain name in accessControlUserIDs means all M2M-User-IDs matching the given domain name can be authorized. For example, “//m2msp.org”.
It is furthermore allowed to use a wildcard character "*" within the SP-Relative-M2M-User-ID portion of a M2M-User-ID. For example, //m2msp.org/homeowner*.  A wildcard character is not permitted within the SP domain name portion of a M2M-User-ID.

The data type applicable to accessControlUserIDs is defined in oneM2M TS-0004 [4].

The accessControlAuthenticationFlag parameter is a Boolean value. If the accessControlAuthenticationFlag parameter is not present, then the value is assumed to be FALSE. If the accessControlAuthenticationFlag parameter is TRUE, then this indicates that the access control rule applies only to Originators considered to have been authenticated by the Hosting CSE. Clause 7.1.2 specifies the criteria used to decide whether or not the Originator is considered to have been authenticated by the Hosting CSE. 

The accessControlObjectDetails parameters are listed in table 7.1.3-4. 

Table 7.1.3-4: Parameters of accessControlObjectDetails

	Parameter
	Usage Description
	Mandatory/Optional
	Formats

	resourceType
	Resource type on which access control rule applies
	O
	Resource type identifier

	specializationID
	Identifier of mgmtDefinition or containerDefinition
	O
	mgmtDefinition or containerDefinition represented as a string.

	childResourceType
	Set of resource type identifiers that can be created under the parent resource.
	O
	Resource type list.


The accessControlObjectDetails attribute specifies a subset of child resource types of the targeted resource to which the access control rule applies. If an access control rule includes accessControlObjectDetails, then childResourceType is specified. An access control rule which does not include any accessControlObjectDetails parameters applies to all child resource types of the target resource. The accessControlObjectDetails parameter is described in table 9.6.2.4-1 of oneM2M TS‑0001 [1]. Child resource types listed in the childResourceType component are subject of access control for the Create operation only. Once a child resource is created, the Access Control Policies assigned directly to it apply. The resourceType and specializationID elements are optional. If either the resourceType or specializationID element is present in accessControlObjectDetails, the CSE matches the type of resource or specialization of the targeted resource with the value specified in the resourceType or specializationID element. Further checking of childResourceType is done only if the resourceType or specializationID match occurs. However, if the resourceType and specializationID elements are not provided, then only childResourceType match is performed.

-----------------------End of change 1---------------------------------------------

CHECK LIST

· Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?
· Does this CR contain changes related to only one particular issue/problem?
· Have any mirror CRs been posted?
· Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?
· Are all pictures editable?
· Have you checked the spelling and grammar?
· Have you used change bars for all modifications?
· Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)
· Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?
© 2020 oneM2M Partners
                                                                                                   Page 1 (of 7)



[image: image1.png]