| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS-45 |
| Source:* | Poornima Shandilya, C-DOT, poornima@cdot.in Anupama Chopra, C-DOT, anupama@cdot.in Suman, C-DOT, ssheoran@cdot.in |
| Date:* | 2020-04-23 |
| Reason for Change/s:* | Refer to Introduction section. |
| CR against: Release* | Release 4 |
| CR against: WI* | Active <Work Item number> MNT maintenance / WI-0083 Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements Only ONE of the above shall be ticked |
| CR against: TS/TR* | TR-0054 Version 0.7.0 |
| Clauses * | 6.1 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Impacted other TS/TR(s) | N.A. |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2017 (Do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar to the extent practicable.

Use Change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

The activation/deactivation date have already been proposed as restriction in <m2mServiceSubscriptionProfile>.

The motivation for this contribution to highlight the importance of <m2mServiceSubscriptionProfile> as a contractual agreement between M2M Service Subscriber and M2M Service Provider and add related limitations in the specification.

Current specification has the resources but lacks the overall detail on what happens when the contract is inactive or has been deactivated or expired or deleted.

TS-0001

The M2M Service Subscription defines the technical part of the contract between an M2M Subscriber (typically an M2M Application Service Provider) and an M2M Service Provider.

Following aspects need to be clearly defined:

- Expiration of contractual agreement between an M2M Service Provider and M2M Application Provider.

  - That is <m2mServiceSubscriptionProfile> reaches its expirationTime as per the current specs handling, the resource shall be deleted.

- Deletion of contractual agreement between an M2M Service Provider and M2M Application Provider.

  - IN receives DELETE request to delete <m2mServiceSubscriptionProfile>

- <m2mServiceSubscriptionProfile> is in ACTIVE/INACTIVE state

  - activation/deactivationTime and status attributes are defined on <m2mServiceSubscriptionProfile>, if agreement is not yet ACTIVE/has been deactivated then how should it be handled.

In current specification, on deletion/expiration of SSP, corresponding <serviceSubscribedNode> resource will be deleted but there will be no impact on the registered applications/nodes from that subscriber.

The input contribution proposes to provide limitations and potentials requirement for same.

### -----------------------Start of change 1---------------------------------------------

## M2M Service Subscription Limitations

## Description

oneM2M currently define an M2M Service Subscription in clause 6.6 of TS-0001[i.1].  It is defined as the technical part of the contract between an M2M Service Subscriber and an M2M Service Provider.  oneM2M defines three resource types in support of the M2M Service Subscription functionality.  These resource types are the <m2mServiceSubscriptionProfile>, <serviceSubscribedNode> and <serviceSubscribedAppRule>.  The relationship between these resources is shown in Figure 6.1.1-1 as defined in clause 9.6.19 of TS-0001 [i.1].

Figure 6.1.1-1: Relationship among M2M Service Subscription related resources

The current M2M Service Subscription functionality defined thus far in oneM2M is limited to the following:

- Defining which CSEs and AEs are hosted on which Nodes

- Defining which AEs are authorized to register to a particular CSE

Currently, the only oneM2M procedure defined that makes use of the M2M Service Subscription functionality is the AE Registration procedure.  When an AE registers, the Registrar CSE can check the applicable <m2mServiceSubscribedNode> and <serviceSubscribedAppRule> resources to determine if the AE is allowed to register.

The M2M Service Subscription functionality is currently not used by any other oneM2M procedures.

The following are some limitations of the existing M2M Service Subscription functionality:

- Lacks the capability to identify a M2M Service Subscriber (i.e. the entity that establishes a M2M Service Subscription with a M2M Service Provider).  This prevents the oneM2M System from supporting M2M service subscriber based functionality such as:

  - M2M Service Subscriber based charging such as defining charging events, collecting statistics and generating charging records per M2M Service Subscribers.

  - M2M Service Subscriber based access control involving access control policy privileges based on M2M Service Subscribers.

  - M2M Service Subscriber based enrolment involving an enrolment of authorized users, devices (i.e. node) and applications (i.e. AEs) associated with a M2M Service Subscriber

  - Support for a profile which defines policies or preferences of the M2M Service Subscriber such as limits on the number and/or types of applications and devices allowed to register, the number of resources that can be created, default access control policies, etc.

- Lacks the capability to identify a M2M Service Subscription (i.e. a unique identifier of the M2M Service Subscription that the M2M Service Subscriber establishes between itself and a M2M Service Provider).

NOTE: 	oneM2M currently defines a M2M Subscription identifier (M2M-Sub-ID ) which could serve as the identifier of a M2M Service Subscription however this identifier is not linked with the existing M2M Service Subscription functionality (i.e. resources and procedures).

Editor’s Note: It is FFS whether both a M2M Service Subscription Identifier and a M2M Service Subscriber Identifier are required

- Lacks the capability to identify an authorized user of a M2M Service Subscriber (e.g. a family member or friend authorized to use a M2M Service Subscriber’s devices, applications and resources).  This prevents the oneM2M System from supporting M2M Service User based functionality such as user based charging, user based access control and user based profiles as defined above for a M2M Service Subscriber.

- Lacks the capability to treat service subscription as a contractual agreement between M2M Service Provider and M2M Application Provider. This prevents the oneM2M System from supporting functionality of

  - Start the oneM2M services for a M2M Service Subscriber following the activation of service subscription.

  - Stop the oneM2M services for a M2M Service Subscriber following the deactivation of service subscription.

  - Delete the M2M Service subscriber subscription and related resources following the DELETE request for service agreement.

  - Delete the M2M Service Subscriber Subscription and related resource on the expiry of service agreement

  - Notify a subscriber when its service subscription reaches expiry or deactivation.

  - Identify resources linked to a service subscription.

## Potential Requirements

- The oneM2M System shall support identification of M2M Service Subscribers and associating a M2M Service Subscriber with a M2M Service Subscription to a M2M Service Provider.

- The oneM2M System shall support identification of M2M Service Users and associating a M2M Service User with a M2M Service Subscriber.

- The oneM2M System shall support charging event detection, statistics collection and charging records generation mechanisms based on M2M Service Subscriber and M2M Service User identification.

- The oneM2M System shall support access control and authorization mechanisms based on  M2M Service Subscriber and M2M Service User identification.

- The oneM2M System shall support M2M Service Subscriber-based enrolment comprised of enrolment of M2M Devices and M2M Applications and M2M Service Users associated with a M2M Service Subscriber.

- The oneM2M System shall support M2M Service Subscriber and M2M Service User profiles specifying their restrictions (e.g. privacy restrictions, max number and/or types of applications and devices the M2M Service Subscriber and its authorized M2M Service Users are allowed to register to the M2M System, the maximum number of resources or bytes of data that the M2M Service Subscriber can store in the M2M System, etc.) and their default configurations (e.g. access control policies, expiration times, max number of content instances, etc.).

- The oneM2M System shall support an activation/deactivation date as a restriction in M2M Service Subscription to allow activation/deactivation of service agreement between M2M Service Provider and M2M Service Subscriber.

- The oneM2M system shall support expiration and deletion of service agreement.

- The oneM2M system shall support notifying a subscriber when the service subscription reaches expiry or deactivation.

- The oneM2M system shall support identifying resources belonging to a service subscription.

### -----------------------End of change 1---------------------------------------------

No mixed AND/OR filter operation will be supported.CHECK LIST

- Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?

- Does this CR contain changes related to only one particular issue/problem?

- Have any mirror CRs been posted?

- Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?

- Are all pictures editable?

- Have you checked the spelling and grammar?

- Have you used change bars for all modifications?

- Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)

- Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?