	Doc# SDS-2020-0200-TS-0001_forwarding_description_R4
Change Request
	[image: image1.png]






	


	CHANGE REQUEST

	Meeting ID:*
	SDS #45.4

	Source:*
	Miguel Angel Reina Ortega, miguelangel.reinaortega@etsi.org 


	Date:*
	2020-18-06

	Reason for Change/s:*
	Forwarding description

	CR  against:  Release*
	Release 4

	CR  against:  WI*
	 FORMCHECKBOX 
 Active <Work Item number>  

 FORMCHECKBOX 
 MNT maintenance / < Work Item number(optional)>
Is this a mirror CR? Yes  FORMCHECKBOX 
 No  FORMCHECKBOX 

mirror CR number: SDS-2020-0195R01-TS-0001_forwarding_description_R3
 FORMCHECKBOX 
 STE Small Technical Enhancements / < Work Item number (optional)>
Only ONE of the above shall be ticked

	CR  against:  TS/TR*
	TS-0001 v4.6.0

	Clauses *
	Clauses 8.2.0

	Type of change: *
	 FORMCHECKBOX 
 Editorial change
 FORMCHECKBOX 
 Bug Fix or Correction
 Change to existing feature or functionality
 New feature or functionality
Only ONE of the above shall be ticked

	Impacted other TS/TR(s)
	

	Post Freeze checking:*
	This CR contains only essential changes and corrections?  YES  FORMCHECKBOX 
  NO  FORMCHECKBOX 

This CR may break backwards compatibility with the last approved version of the TS?       YES 
  NO 


	Template Version: January 2017 (Do not modify)


oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.
In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR
Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).
Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.
Follow the drafting rules.
All pictures must be editable.
Check spelling and grammar to the extent practicable.
Use Change bars for modifications.
The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.
Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.
When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text. 
Introduction
This CR proposes to clarify the description of the forwarding mechanism which seems to be ambiguous with the actual text as a CSE-ID different than the Hosting CSE CSE-ID can appear in the To parameter (targeting a remoteCSE resource). 
-----------------------Start of change 1  -------------------------------------------
8.2.0
Overview
This clause describes the procedures for accessing the resources. The term "hop" in the descriptions here refers to the number of Transit CSEs traversed by a request on its route from the Originator to the Hosting CSE. Traversal implies that the request was forwarded from one CSE to either its Registrar CSE or Registree CSE. For example, when a CSE initiated a request and the Hosting CSE is its Registrar CSE, the hop count is zero.
The Receiver CSE shall forward the received request in the following case:

· The To parameter in the request targets a CSE that is not the Receiver CSE.
The Receiver CSE shall handle the received request in the following cases:

· The To parameter in the request targets the Receiver CSE.
See oneM2M TS-0004 [3] clause 7.3.2.8 for further details.
All the descriptions and message flows in this clause are illustrative for the direction from a Registree acting as an Originator to a Registrar acting as a Receiver only. The flows from a Registrar CSE to a Registree CSE are symmetric with respect to the one described in this clause. Both the IN-CSE and MN-CSE have the ability to route a received request or response messages to one of their Registrees. If the Hosting CSE is not known by an MN-CSE that receives a request or response message, that MN-CSE shall forward the message to its own Registrar CSE by default.
-----------------------End of change 1---------------------------------------------

CHECK LIST

· Does this Change Request include an informative introduction containing the problem(s) being solved, and a summary list of proposals.?
· Does this CR contain changes related to only one particular issue/problem?
· Have any mirror CRs been posted?
· Does this Change Request  make all the changes necessary to address the issue or problem?  E.g. A change impacting 5 tables should not include a proposal to change only 3 tables?Does this Change Request follow the drafting rules?
· Are all pictures editable?
· Have you checked the spelling and grammar?
· Have you used change bars for all modifications?
· Does the change include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change? (Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.)
· Are multiple changes in this CR clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.?
© 2020 oneM2M Partners
                                                                                                   Page 3 (of 5)



[image: image1.png]