	


	CHANGE REQUEST

	Meeting ID:*
	SDS #47

	Source:*
	Dale Seed, Convida, seed.dale @convidawireless.com
Lu Liu, Convida, liu.lu@convidawireless.com

	Date:*
	2020-10-04

	Reason for Change/s:*
	Maintenance on <action> and <dependency> resource functionality

	CR  against:  Release*
	Release 4

	CR  against:  WI*
	 FORMCHECKBOX 
 Active - WI-0093 – Action Triggering Enhancements
 FORMCHECKBOX 
 MNT maintenance / < Work Item number(optional)>
Is this a mirror CR? Yes  FORMCHECKBOX 
 No  FORMCHECKBOX 

mirror CR number: (Note to Rapporteur - use latest agreed revision)
 FORMCHECKBOX 
 STE Small Technical Enhancements / < Work Item number (optional)>
Only ONE of the above shall be ticked

	CR  against:  TS/TR*
	TS-0001 V4.7.0 

	Clauses *
	Various

	Type of change: *
	 FORMCHECKBOX 
 Editorial change
 FORMCHECKBOX 
 Bug Fix or Correction
 Change to existing feature or functionality
 FORMCHECKBOX 
 New feature or functionality
Only ONE of the above shall be ticked

	Impacted other TS/TR(s)
	N/A

	Post Freeze checking:*
	This CR contains only essential changes and corrections?  YES  FORMCHECKBOX 
  NO  FORMCHECKBOX 

This CR may break backwards compatibility with the last approved version of the TS?       YES 
  NO 


	Template Version: January 2017 (Do not modify)


oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.
In case of a correction, and the change apply to previous releases, a separate “mirror CR” should be posted at the same time of this CR
Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).
Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made E.g. A change impacting 5 tables should not only include a proposal to change only 3 tables. Includes any changes to references, definitions, and acronyms in the same deliverable.
Follow the drafting rules.
All pictures must be editable.
Check spelling and grammar to the extent practicable.
Use Change bars for modifications.
The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the new clause is proposed to be located.
Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.
When subsequent changes are made to content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text. 
Introduction
This CR proposes some updates to the <action> and <dependency> to clean-up functionality as part of the Stage 3 work.   

-----------------------Start of change 1-------------------------------------------
9.6.61
Resource Type action
The <action> resource defines an operation (i.e. a oneM2M primitive) performed by the Hosting CSE on a target resource when criteria defined by the <action> resource and any applicable <dependency> resources have been met. 
The <action> resource and its child resources include the following information:

· A subject resource that is monitored by the Hosting CSE to detect if/when the evaluation criteria specified by the <action> resource are met.
· An object resource that the Hosting CSE targets the primitive, defined by the <action> resource, towards if/when the evaluation criteria specified by the <action> resource are met.  
· An input resource that may provide input parameters for the action.
· Conditions that are monitored by the Hosting CSE to determine if the primary event occurs.
· Dependencies that are evaluated by the Hosting CSE to determine if the action is to be conditionally performed.

· Parameters providing priorities of the action and dependencies.
The <action> resource shall contain the child resources specified in table 9.6.61-1. 
Table 9.6.61-1: Child resources of <action> resource

	Child Resources of <action>
	Child Resource Type
	Multiplicity
	Description
	<actionAnnc> Child Resource Types

	[variable]
	<dependency>
	0..n
	See clause 9.6.62
	<dependencyAnnc>

	[variable]
	<subscription>
	0..n
	See clause 9.6.8
	<subscription>

	[variable]
	<transaction>
	0..n
	See clause 9.6.48
	<transaction>


The <action> resource shall contain the attributes specified in table 9.6.61-2. 
Table 9.6.61-2: Attributes of <action> resource

	Attributes of 
<action>
	Multiplicity
	RW/

RO/

WO
	Description
	<actionAnnc> Attributes

	resourceType
	1
	RO
	See clause 9.6.1.3.
	NA

	resourceID
	1
	RO
	See clause 9.6.1.3.
	NA

	resourceName
	1
	WO
	See clause 9.6.1.3.
	NA

	parentID
	1
	RO
	See clause 9.6.1.3.
	NA

	expirationTime
	1
	RW
	See clause 9.6.1.3. 
	MA

	accessControlPolicyIDs
	0..1 (L)
	RW
	See clause 9.6.1.3. 
	MA

	labels
	0..1 (L)
	RW
	See clause 9.6.1.3.
	MA

	creationTime
	1
	RO
	See clause 9.6.1.3.
	NA

	lastModifiedTime
	1
	RO
	See clause 9.6.1.3.
	NA

	
	
	
	
	

	announceTo
	0..1 (L)
	RW
	See clause 9.6.1.3.
	NA

	announcedAttribute
	0..1 (L)
	RW
	See clause 9.6.1.3.
	NA

	announceSyncType
	0..1
	RW
	See clause 9.6.1.3.
	MA

	dynamicAuthorizationConsultationIDs
	0..1 (L)
	RW
	See clause 9.6.1.3.
	OA

	creator
	1
	RO
	See clause 9.6.1.3.
	NA

	owner
	0..1
	RW
	See clause 9.6.1.3.
	NA

	
	
	
	
	

	actionPriority
	0..1
	RW
	Attribute indicating the priority of this action compared with other actions having the same event expression (subjectResourceID and evalCriteria), with highest priority indicated by the lowest value. Local policies are applied for equal or unspecified priorities.
	OA

	subjectResourceID
	0..1
	RW
	The resourceID of the resource that is the subject of monitoring for evaluation of the evalCriteria attribute. The subject can be any resource in the system. If this attribute is not configured, the subject is the parent resource of this <action> resource. 
	OA

	evalCriteria
	1
	RW
	This attribute provides the conditions determining if the action is to be conditionally triggered for the object resource.
The evalCriteria attribute, in conjunction with the subjectResourceID, forms the event expression that is being monitored. See further description below and in table 9.6.61-3.

	OA

	evalMode
	1 
	RW
	Attribute provides the macro control mode of the evaluation of the <action> resource. Some values are:

0 = off

1 = once

2 = periodic

3 = continuous
	OA

	evalControlParam
	0..1
	RW
	When evalMode has a value of “periodic”, evalControlParam represents the periodicity in seconds. 

When evalMode has a value of “continuous” the evaluation is restarted as soon as an action has been triggered. In this case, if evalControlParam is specified, it determines the number of times the Hosting CSE shall trigger the event, otherwise it is repeated indefinitely.

When evalMode has a value of “off” or “once”, this attribute shall be ignored.


	OA

	evalPriority
	0..1 (L)
	RW
	List of priorities to be applied for the evaluation of the dependencies . This is an ordered list of the resourceIDs of child <dependency> resources. 
	OA

	objectResourceID
	1
	RW
	The identifier of a resource or attribute that the Hosting CSE targets the actionPrimitive towards when detecting that the evalCriteria and any dependencies defined in the evalPriority attribute have been met.
	MA

	actionPrimitive 
	1
	RW
	This attribute stores the primitive that is performed on the objectResourceID when the Hosting CSE detects that the evalCriteria and any dependencies defined in the evalPriority attribute have been met.  
If the objectResourceID attribute of the <action> resource is configured, the Hosting CSE shall override the value of the To parameter in the actionPrimitive with the resource identifier defined by objectResourceID and the value of the Resource Type parameter with the resource type of the resource referenced by objectResourceID.

If the input attribute of the <action> resource is configured, the Hosting CSE shall override the value of the Content parameter in the actionPrimitive with the value stored in the input attribute.


· 
· 

	MA

	input
	0..1
	RW
	Input used by the Hosting CSE to override the Content parameter in the actionPrimitive. 
The attribute may be formatted as:

· a string, 

· a resource identifier, or 

· the identifier of a resource attribute.

The Hosting CSE overrides the Content parameter in the actionPrimitive with the following (respectively):

· the given string,
· the resource representation of the referenced resource, 
· the value of the referenced resource attribute 

	OA

	outputResourceID
	0..1
	RW
	The resource identifier of a resource where the Hosting CSE stores a response primitive after performing the actionPrimitive on the objectResourceID.  
	OA

	actionResult
	1
	RW
	After performing the actionPrimitive on the objectResourceID, the Hosting CSE stores the response primitive in this attribute.  
	OA


The conditions represented in the evalCriteria attribute determine if the action is to be conditionally triggered.  

The evalCriteria attribute is comprised of triples (subject, operator, threshold) with the parameters shown and described in table 9.6.61-3.
Table 9.6.61-3: Parameters in evalCriteria triple
	Name
	Description

	subject
	This parameter shall indicate an attribute of the subjectResourceID resource e.g. “tempContainer.content”

	operator
	This parameter is a keyword used to construct the evaluation logic, e.g. ‘equals’, ‘not equals’, ‘greater than’, ‘less than’, ‘greater or equal’, ‘less or equal’, ‘string match’, etc.

	threshold
	This parameter provides a value used to evaluate the criteria. The value type shall match the type of the attribute used as subject. 


9.6.62
Resource Type dependency
The <dependency> resource represents conditions that determine if the action specified in the parent <action> resource is triggered (i.e. a oneM2M primitive performed by the Hosting CSE on a target resource). One or more <dependency> resources may exist to provide complex events for triggering. The <dependency> resources provide conditions that always need to be checked before the action is triggered. The difference between the evalCriteria  specified by an <action> resource and the one specified by a  <dependency> resource is that the former applies to the subject resource and triggers an evaluation, while the latter applies to other resources which provide conditions used in evaluation.
The <dependency> resource shall contain the child resources specified in table 9.6.62-1. 
Table 9.6.62-1: Child resources of <dependency> resource

	Child Resources of <action>
	Child Resource Type
	Multiplicity
	Description
	<dependencyAnnc> Child Resource Types

	[variable]
	<subscription>
	0..n
	See clause 9.6.8
	<subscription>

	[variable]
	<transaction>
	0..n
	See clause 9.6.48
	<transaction>


The <dependency> resource shall contain the attributes specified in table 9.6.62-2.

Table 9.6.62-2: Attributes of <dependency> resource

	Attributes of 
<action>
	Multiplicity
	RW/

RO/

WO
	Description
	<dependencyAnnc> Attributes

	resourceType
	1
	RO
	See clause 9.6.1.3.
	NA

	resourceID
	1
	RO
	See clause 9.6.1.3.
	NA

	resourceName
	1
	WO
	See clause 9.6.1.3.
	NA

	parentID
	1
	RO
	See clause 9.6.1.3.
	NA

	expirationTime
	1
	RW
	See clause 9.6.1.3. 
	MA

	accessControlPolicyIDs
	0..1 (L)
	RW
	See clause 9.6.1.3. If no accessControlPolicyIDs value is configured, the accessControlPolicyIDs of the parent resource shall be applied for privilege checking.
	MA

	labels
	0..1 (L)
	RW
	See clause 9.6.1.3.
	MA

	creationTime
	1
	RO
	See clause 9.6.1.3.
	NA

	lastModifiedTime
	1
	RO
	See clause 9.6.1.3.
	NA

	
	
	
	
	

	announceTo
	0..1 (L)
	RW
	See clause 9.6.1.3.
	NA

	announcedAttribute
	0..1 (L)
	RW
	See clause 9.6.1.3.
	NA

	announceSyncType
	0..1
	RW
	See clause 9.6.1.3.
	MA

	dynamicAuthorizationConsultationIDs
	0..1 (L)
	RW
	See clause 9.6.1.3.
	OA

	creator
	0..1
	RO
	See clause 9.6.1.3.
	NA

	owner
	0..1
	RW
	See clause 9.6.1.3.
	NA

	logic
	1
	RW
	Indicates if the dependency is SUFFICIENT or NECESSARY. The operand of the related parent <action> resource needs to match:

At least one of the SUFFICIENT dependencies OR
All the NECESSARY dependencies
	OA

	evalCriteria
	1
	RW
	Attribute providing conditions determining if the dependency matches the criteria.

The evalCriteria attribute in conjunction with the dependentResource attribute forms the dependency to be evaluated. See further description in table 9.6.61-3.
	OA

	dependentResourceID
	1
	RW
	The resource identifier of the resource for which the evalCriteria is evaluated for the dependency.
	MA


-----------------------End of change 1-------------------------------------------

-----------------------Start of change 2-------------------------------------------

10.2.21
Action Triggering
10.2.21.1
Introduction

This clause describes the procedure for managing oneM2M action triggering via the <action> and <dependency> resources.
A oneM2M <action> resource is created whenever an Originator desires a primitive to be sent in response to the detection of a oneM2M event. An event has a subject resource associated with it – a change on this subject resource causes the Hosting CSE to conditionally trigger the procedure as outlined in 10.2.21.2. This is referred to as “action triggering”. The subject resource is identified by the subjectResourceID attribute of the <action> resource and the evalCriteria attribute of the <action> resource determines if an action is conditionally triggered. Collectively, this combination is known as the event expression and the event is the primary event when it is triggered. 

When an <action> resource is created, the Hosting CSE checks the evalMode attribute to see if an event is enabled for monitoring. The evalMode attribute has one of four valid values: off, once, periodic, or continuous. If evalMode is off, then no event is being monitored by the Hosting CSE. For all other values, the Hosting CSE will begin monitor events . 

When evalMode is once, the Hosting CSE will immediately enable monitoring the event and will only allow the primary event to trigger once. After the event triggers, the Hosting CSE will disable further monitoring. 

When evalMode is either periodic or continuous, the Hosting CSE will also evaluate the evalControlParam attribute to determine how to monitor the event. For periodic evalMode, the evalControlParam attribute determines the periodicity of monitoring the event. The Hosting CSE will start monitoring the event at the indicated periodicity and only allow the event to trigger once within the indicated period. For continuous evalMode, the evalControlParam attribute determines the number of times the Hosting CSE should trigger an event. When in this mode, the Hosting CSE resumes monitoring for the occurrence of the next event upon the triggering of a prior event; hence, there is no lapse in monitoring.

To handle more complex events, one or more  <dependency> resources  may be created as a child resources of the <action> resource. Within the <dependency> resource, the dependentResourceID attribute identifies the resource that the event further depends on. The evalCriteria attribute of the <dependency> resource determines whether the dependency condition is met. The logic attribute determines whether the dependency condition is SUFFICIENT or NECCESARY to the action triggering. Finally, the evalPriority attribute of the <action> resource determines the priorities to the dependency conditions to be applied when evaluating if the action or operation should be triggered based on the primary event. The dependency conditions shall be evaluated by the Hosting CSE in their evalPriority priority order. 

Once a Hosting CSE has determined that an event has triggered, it then uses the following attributes to create the  associated action : actionPrimitive, objectResourceID, and input. The actionPrimitive attribute provides the main content for the primitive that the Hosting CSE sends when the event is triggered. If present, the objectResourceID attribute is used to derive the To and Resource Type parameters which are overwritten in the primitive to be sent. If present, the input attribute is used to derive Content parameter which is overwritten in the primitive to be sent. If these attributes are not present, the primitive sent maintains the parameters provided in the actionPrimitive.
Both the outputResourceID and actionResult attributes of the <action> resource are used by the Hosting CSE to store the response primitive associated with the actionPrimitive. The Hosting CSE shall store the response primitive in the actionResult attribute. If the outputResourceID attribute is configured the Hosting CSE shall also store the response primitive in the resource referenced by the outputResourceID attribute. .  

10.2.21.2
Action Triggering Procedure
This clause describes the procedure a Hosting CSE performs to manage the monitoring of events and, when they occur, the conditional triggering of actions and operations specified, as detailed by the following the steps:

Step 1:
Create <action> resource either as the child resource of a subject resource or elsewhere in the resource tree. The subject resource can be any of the resources defined in oneM2M and is indicated by the subjectResourceID attribute. One or more <dependency> resources may be created as child resources of the <action> resource.

Changes to the subject resource trigger the evaluation of the evalCriteria. 

Step 2:
The content of subject resource is updated 

Step 3:
The Hosting CSE gets the updated result and evaluates if the updated subject resource, and if it matches the criteria defined in the evalCriteria attribute of the <action> resource, the primary event is considered triggered. If the subject resource does not match the criteria defined in the evalCriteria attribute of the <action> resource, the Hosting CSE returns to step 2, waiting for further subject resource updates.

Step 4:
After a primary event is triggered in step 3, if there is at least one <dependency> child resource of the <action> resource, the Hosting CSE performs the following step for each <dependency> child resource(s) in the order provided in the evalPriority attribute, or in an order based on local policy if the attribute is missing. 
For each of the <dependency> child resources, the Hosting CSE uses the evalCriteria attribute and the logic attribute values provided in the current <dependency> resource. The Hosting CSE retrieves the dependent resource specified by the dependentResourceID attribute, evaluates it against the criteria defined in the evalCriteria attribute to check if the dependency is met.

If the dependency is met (i.e. the resource identified by dependentResourceID meets the criteria specified by evalCriteria):
· if the logic attribute is of value SUFFICIENT, the Hosting CSE proceeds to step 5. 

· if the logic attribute is of value NECESSARY, the hosting CSE repeats step 4 for the next <dependency> child resource. 
If the dependency is not met (i.e. the resource identified by dependentResourceID does not meet the criteria specified by evalCriteria), the hosting CSE repeats step 4 for the next <dependency> child resource.  
Step 5:
The Hosting CSE retrieves the input resource if the input attribute of the <action> resource is a resourceID.



NOTE: The procedure outlined in step 4 means that step 5 is reached when, after the primary event is triggered, either:
· the SUFFICIENT dependency with the highest evalPriority priority is met; OR

· all the NECCESARY dependencies are met.

Step 6:
The Hosting CSE composes the request primitive for the object resource for triggering the action by populating it with the content of the actionPrimitive attribute. 

If the objectResource attribute is present, the Hosting CSE determines its type (e.g. by performing a retrieve operation). The Hosting CSE overwrites the To and Resource Type parameters of the primitive to be sent with the values determined from objectResourceID. 
If the input attribute is present and is a string, the Hosting CSE overwrites the Content parameter of the primitive to be sent with the string. If the input attribute is present and is a resource identifier, the Hosting CSE retrieves the resource and overwrites the Content parameter of the primitive to be sent with the retrieved resource representation. If the input attribute is present and is a URI of a resource attribute , the Hosting CSE retrieves the resource attribute and overwrites the Content parameter of the primitive to be sent with the value of the given resource attribute. If the input attribute is not present, the Content parameter of the primitive to be sent remains populated as indicated by the actionPrimitive attribute.
Step 7:
The Hosting CSE sends the composed primitive request to the object resource. 
Step 8:
After receiving the response of the request from step 7, if the outputResourceID attribute of the <action> resource is set, the Hosting CSE stores the response primitive in the resource identified by outputResourceID. The Hosting  and stores the representation as the actionResult attribute. If the outputResourceID attribute is not set the Hosting CSE stores the response primitive received in the actionResult attribute.
10.2.21.3
<action> Resource Procedures
10.2.21.3.1
Create <action>
The hosting CSE shall use this procedure to create an <action> resource according to the resource identifier of the parent resource.
Table 10.2.21.3.1-1: <action> CREATE
	<action> CREATE 

	Information in Request message
	All parameters defined in table 8.1.2-3 apply with the specific details for:
Content: The resource content shall provide the information as defined in clause 9.6.x

	Processing at Originator before sending Request
	According to clause 10.1.2

	Processing at Receiver
	According to clause 10.1.2

After successfully creating the <action> resource, the Receiver shall perform the procedure specified in 10.2.21.2.

	Information in Response message
	According to clause 10.1.2

	Processing at Originator after receiving Response
	According to clause 10.1.2

	Exceptions
	According to clause 10.1.2


10.2.21.3.2
Retrieve <action>
This procedure shall be used for retrieving the attributes of an <action> resource.

Table 10.2.21.3.2-1: <action> RETRIEVE
	<action> RETRIEVE

	Information in Request message
	All parameters defined in table 8.1.2-2 apply.

	Processing at Originator before sending Request
	According to clause 10.1.3

	Processing at Receiver
	According to clause 10.1.3

	Information in Response message
	All parameters defined in table 8.1.3-1 apply.

	Processing at Originator after receiving Response
	According to clause 10.1.3

	Exceptions
	According to clause 10.1.3


10.2.21.3.3
Update <action>
This procedure shall be used for updating attributes of an <action> resource.

Table 10.2.21.3.3-1: <action> UPDATE
	<action> UPDATE

	Information in Request message


	All parameters defined in table 8.1.2-2 apply.

	Processing at Originator before sending Request
	According to clause 10.1.4

	Processing at Receiver
	According to clause 10.1.4
After successfully updating the <action> resource, the Receiver shall perform the procedure specified in 10.2.21.2.

	Information in Response message
	According to clause 10.1.4

	Processing at Originator after receiving Response
	According to clause 10.1.4

	Exceptions
	According to clause 10.1.4


10.2.21.3.4
Delete <action>
This procedure shall be used for deleting an <action> resource.
Table 10.2.21.3.4-1: <action> DELETE
	<action> DELETE

	Information in Request message
	All parameters defined in table 8.1.2-2 apply

	Processing at Originator before sending Request
	According to clause 10.1.5

	Processing at Receiver
	According to clause 10.1.5
After successfully deleting the <action> resource, the Receiver shall stop the event monitoring procedure specified in 10.2.21.2.

	Information in Response message
	According to clause 10.1.5

	Processing at Originator after receiving Response
	According to clause 10.1.5

	Exceptions
	According to clause 10.1.5


10.2.21.4 
<dependency> Resource Procedures
10.2.21.4.1
Create <dependency>
The hosting CSE shall use this procedure to create an <dependency> resource according to the resource identifier of the parent resource.

Table 10.2.21.4.1-1: <dependency> CREATE
	<dependency> CREATE 

	Information in Request message
	All parameters defined in table 8.1.2-3 apply with the specific details for:
Content: The resource content shall provide the information as defined in clause 9.6.x

	Processing at Originator before sending Request
	According to clause 10.1.2

	Processing at Receiver
	According to clause 10.1.2

After successfully creating the <dependency> resource, the Receiver shall perform the procedure specified in 10.2.21.2.

	Information in Response message
	According to clause 10.1.2

	Processing at Originator after receiving Response
	According to clause 10.1.2

	Exceptions
	According to clause 10.1.2


10.2.21.4.2
Retrieve <dependency>
This procedure shall be used for retrieving the attributes of an <dependency> resource.

Table 10.2.21.4.2-1: <dependency> RETRIEVE
	<dependency> RETRIEVE

	Information in Request message
	All parameters defined in table 8.1.2-2 apply.

	Processing at Originator before sending Request
	According to clause 10.1.3

	Processing at Receiver
	According to clause 10.1.3

	Information in Response message
	All parameters defined in table 8.1.3-1 apply.

	Processing at Originator after receiving Response
	According to clause 10.1.3

	Exceptions
	According to clause 10.1.3


10.2.21.4.3
Update <dependency>
This procedure shall be used for updating attributes of an <dependency> resource.

Table 10.2.21.4.3-1: <dependency> UPDATE
	<dependency> UPDATE

	Information in Request message


	All parameters defined in table 8.1.2-2 apply.

	Processing at Originator before sending Request
	According to clause 10.1.4

	Processing at Receiver
	According to clause 10.1.4
After successfully updating the <dependency> resource, the Receiver shall perform the procedure specified in 10.2.21.2.

	Information in Response message
	According to clause 10.1.4

	Processing at Originator after receiving Response
	According to clause 10.1.4

	Exceptions
	According to clause 10.1.4


10.2.21.4.4
Delete <dependency>
This procedure shall be used for deleting an <dependency> resource.
Table 10.2.21.4.4-1: <dependency> DELETE
	<dependency> DELETE

	Information in Request message
	All parameters defined in table 8.1.2-2 apply

	Processing at Originator before sending Request
	According to clause 10.1.5

	Processing at Receiver
	According to clause 10.1.5
After successfully deleting the <dependency> resource, the Receiver shall perform the procedure specified in 10.2.21.2.

	Information in Response message
	According to clause 10.1.5

	Processing at Originator after receiving Response
	According to clause 10.1.5

	Exceptions
	According to clause 10.1.5


-----------------------End of change 2-------------------------------------------

-----------------------Start of change 3-------------------------------------------

9.6.1.1
Resource Type Summary

Table 9.6.1.1-1 introduces the normal and virtual resource types and their related child or parent resource types. Details of each resource type follow in the remainder of this clause.

Table 9.6.1.1-1 lists each specified ordinary – i.e. not announced – resource type. An addition of suffix "Annc" to the respective resource type identifier indicates the associated announced resource type. Resource types that can occur as child resources of announced resources are summarized in Table 9.6.26.1-1 "Announced Resource Types".
Among the resource types listed in Table 9.6.1.1-1, the following are termed "Content Sharing Resources" in oneM2M Specifications for the purpose of referring to any of those resource types:

· container;

· contentInstance;

· flexContainer;
· flexContainerInstance;
· timeSeries;
· timeSeriesInstance.

Table 9.6.1.1-1: Resource Types 

	Resource Type
	Short Description
	Child Resource Types
	Parent Resource Types
	Clause

	accessControlPolicy
	Stores a representation of privileges. It is associated with resources that shall be accessible to entities external to the Hosting CSE. It controls "who" is allowed to do "what" and the context in which it can be used for accessing resources
	subscription, transaction
	AE, AEAnnc, remoteCSE, remoteCSEAnnc, CSEBase
	9.6.2

	AE
	Stores information about the AE. It is created as a result of successful registration of an AE with the Registrar CSE
	subscription, container, 
flexContainer,
group, accessControlPolicy, 
pollingChannel, semanticDescriptor,
timeSeries, transaction, transactionMgmt,

triggerRequest, crossResourceSubscription, backgroundDataTransfer, semanticMashupInstance, locationPolicy, action, multimediaSession, dynamicAuthorizationConsultation, nwMonitoringReq, timeSyncBeacon, primitiveProfile, softwareCampaign, e2eQosSession
	CSEBase
	9.6.5

	container
	Shares data instances among entities. Used as a mediator that buffers data exchanged between AEs and/or CSEs. The exchange of data between AEs (e.g. an AE on a Node in a field domain and the peer-AE on the infrastructure domain) is abstracted from the need to set up direct connections and allows for scenarios where both entities in the exchange do not have the same reachability schedule
	container, 
flexContainer, contentInstance, subscription, latest, oldest，semanticDescriptor, timeSeries, transaction, action
	AE, AEAnnc, container, containerAnnc, remoteCSE, remoteCSEAnnc, 
CSEBase,
flexContainer, flexContainerAnnc
	9.6.6

	contentInstance
	Represents a data instance in the <container> resource
	semanticDescriptor, transaction
	container, containerAnnc
	9.6.7

	flexContainer
	A template which allows to define specialized (customizable) versions of containers with a flexible and lightweight structure 
	container, 

flexContainer, flexContainerInstance, latest, oldest, subscription, semanticDescriptor, timeSeries, transaction, action
	AE, AEAnnc, container, containerAnnc, 

flexContainer, flexContainerAnnc, remoteCSE, remoteCSEAnnc, 
CSEBase
	9.6.35

	flexContainerInstance
	Represents a timestamped Data instance in the <flexContainer> resource
	 transaction
	flexContainer
	9.6.59

	CSEBase
	The structural root for all the resources that are residing on a CSE. Stores information about the CSE itself
	remoteCSE, remoteCSEAnnc, node, AE, container, group, accessControlPolicy, subscription, mgmtCmd, locationPolicy, statsConfig, statsCollect, request, delivery,

schedule,
notificationTargetPolicy, semanticRuleRepository, 
flexContainer,
timeSeries, AEContactList, transaction, transactionMgmt, crossResourceSubscription, backgroundDataTransfer, semanticMashupJobProfile, semanticMashupInstance, action, m2mServiceSubscriptionProfile, dynamicAuthorizationConsultation, localMulticastGroup, serviceSubscribedAppRule, authorizationPolicy, authorizationDecision, authorizationInformation, ontologyRepository, nwMonitoringReq, timeSyncBeacon, primitiveProfile, softwareCampaign, e2eQosSession
	None specified
	9.6.3

	delivery
	Forwards requests from CSE to CSE
	subscription, transaction
	CSEBase
	9.6.11

	eventConfig
	Defines events that trigger statistics collection
	subscription, transaction
	statsConfig
	9.6.24

	execInstance
	Contains all execution instances of the same Management Command
	subscription, transaction
	mgmtCmd
	9.6.17

	fanOutPoint (V)
	Virtual resource containing target for group request

It is used for addressing bulk operations to all the resources that belong to a group
	None specified
	group
	9.6.14

	group
	Stores information about resources of the same type that need to be addressed as a Group. Operations addressed to a Group resource shall be executed in a bulk mode for all members belonging to the Group
	fanOutPoint,

subscription,
semanticFanOutPoint, semanticDescriptor, transaction, action
	AE, AEAnnc, remoteCSE, remoteCSEAnnc, CSEBase
	9.6.13

	latest (V)
	Virtual resource that points to most recently created <contentInstance> , <timeSeriesInstance> or <flexContainerInstance> child resource within a <container>,  <timeSeries> or <flexContainer> resource
	None specified
	container, timeSeries, or flexContainer
	9.6.27

	locationPolicy
	Includes information to obtain and manage geographical location. It is only referenced within a container, the contentInstances of the container provide location information
	subscription, transaction
	CSEBase, AE
	9.6.10

	mgmtCmd
	Management Command resource represents a method to execute management procedures required by existing management protocols
	execInstance,

subscription, transaction
	CSEBase
	9.6.16

	mgmtObj
	Management Object resource represents management functions that provides an abstraction to be mapped to external management technology. It represents the node and the software installed in the node (see note)
	subscription, transaction, semanticDescriptor
	node, mgmtObjAnnc
	9.6.15

Annex D

	m2mServiceSubscriptionProfile
	Data pertaining to the M2M Service Subscription
	serviceSubscribedNode,

subscription, transaction, serviceSubscribedUserProfile
	CSEBase 
	9.6.19

	node
	Represents specific Node information
	mgmtObj, 
subscription, semanticDescriptor, schedule, transaction, action
	CSEBase
	9.6.18

	notificationTargetMgmtPolicyRef
	Represents a list of notification targets and the deletion policy
	subscription, transaction
	subscription
	9.6.31

	notificationTargetPolicy
	Represents a notification target deletion policy with pre-defined action and deletion rules
	subscription, policyDeletionRules, transaction
	CSEBase
	9.6.32

	notificationTargetSelfReference (V)
	Virtual resource used to remove the Notification Target
	None specified
	subscription
	9.6.34

	oldest (V)
	Virtual resource that points to first created <contentInstance>, <timeSeriesInstance> or <flexContainerInstance> child resource within a <container>, <timeSeries> or <flexContainer> resource
	None specified
	container, timeSeries, or flexContainer
	9.6.28

	pollingChannel
	Represent a channel that can be used for a request-unreachable entity
	pollingChannelURI
	remoteCSE, AE
	9.6.21

	pollingChannelURI (V)
	Virtual resource used to perform service layer long polling of a resource Hosting CSE by a request-unreachable entity
	None specified
	pollingChannel
	9.6.22

	policyDeletionRules
	Represents a set of rules which is associated with notification target removal policy
	subscription, transaction
	notificationTargetPolicy
	9.6.33

	remoteCSE
	Represents a remote CSE for which there has been a registration procedure with the registrar CSE identified by the CSEBase resource
	container, containerAnnc,
contentInstanceAnnc 
flexContainer, flexContainerAnnc,
group, groupAnnc, accessControlPolicy, accessControlPolicyAnnc, subscription, pollingChannel, 
timeSeries,
timeSeriesAnnc, timeSeriesInstanceAnnc,
mgmtObjAnnc,
nodeAnnc,

AEAnnc,

locationPolicyAnnc, transaction, crossResourceSubscription, backgroundDataTransfer, semanticMashupJobProfile, semanticMashupJobProfileAnnc,  semanticMashupInstance,

semanticMashupInstanceAnnc, 

action, dynamicAuthorizationConsultation, dynamicAuthorizationConsultationAnnc, nwMonitoringReq, timeSyncBeacon, primitiveProfile, softwareCampaign, e2eQosSession
	CSEBase
	9.6.4

	request
	Expresses/access context of an issued Request
	subscription, transaction
	CSEBase
	9.6.12

	schedule
	Contains scheduling information for delivery of messages
	subscription, transaction
	subscription, CSEBase, node, softwareCampaign
	9.6.9

	serviceSubscribedNode
	Node information
	subscription, transaction
	m2mServiceSubscriptionProfile
	9.6.20

	statsCollect
	Defines triggers for the IN-CSE to collect statistics for applications
	subscription, transaction
	CSEBase (in IN‑CSE)
	9.6.25

	statsConfig
	Stores configuration of statistics for applications
	eventConfig,

subscription, transaction
	CSEBase (in IN‑CSE)
	9.6.23

	subscription
	Subscription resource represents the subscription information related to a resource. Such a resource shall be a child resource for the subscribed-to resource
	schedule, notificationTargetSelfReference, notificationTargetMgmtPolicyRef, transaction
	accessControlPolicy, accessControlPolicyAnnc, AE, AEAnnc, container, containerAnnc, CSEBase, delivery, eventConfig, execInstance, group, groupAnnc, locationPolicy, locationPolicyAnnc, mgmtCmd, mgmtObj, mgmtObjAnnc, m2mServiceSubscriptionProfile, node, nodeAnnc, serviceSubscribedNode, remoteCSE, remoteCSEAnnc, request, schedule, scheduleAnnc,
semanticDescriptor, semanticDescriptorAnnc, statsCollect, statsConfig,
flexContainer, flexContainerAnnc,
timeSeries, timeSeriesAnnc, nwMonitoringReq, semanticRuleRepository, reasoningRules, reasoningJobInstance, timeSyncBeacon, primitiveProfile,

state,

processManagement
	9.6.8

	serviceSubscribedAppRule
	Represents a rule that defines allowed App-ID and AE-ID combinations that are acceptable for registering an AE on a Registrar CSE
	subscription, transaction
	CSEBase
	9.6.29

	semanticDescriptor
	Stores semantic description pertaining to a resource and potentially sub-resources.
	subscription, transaction
	AE, container, contentInstance, group, node, flexContainer, timeSeries, mgmtObj
	9.6.30

	semanticFanOutPoint
	Virtual resource used as target for semantic discovery aimed at a logical graph distributed over multiple semanticDescriptor resources, which belong to the corresponding group parent resource
	
	group
	9.6.14a

	dynamicAuthorizationConsultation
	Represents consultation information used by a CSE when performing consultation-based dynamic authorization
	subscription, transaction
	AE, AEAnnc, remoteCSE, remoteCSEAnnc, CSEBase
	9.6.40

	timeSeries
	Stores and Shares Time Series Data instances among entities.
	timeSeriesInstance, subscription, semanticDescriptor,

latest, oldest, transaction, action
	AE, AEAnnc, remoteCSE, remoteCSEAnnc, CSEBase,

container, containerAnnc, flexContainer, flexContainerAnnc
	9.6.36

	timeSeriesInstance
	Represents a Time Series Data instance in the <timeSeries> resource
	 transaction
	timeSeries, timeSeriesAnnc
	9.6.37

	role
	Represents a role that is assigned to an AE or CSE.
	subscription, transaction
	authorizationInformation
	9.6.38

	token
	Used for storing a token that is issued to an AE or CSE.
	subscription, transaction
	authorizationInformation
	9.6.39

	authorizationDecision
	Represents an access control decision point
	subscription, transaction
	CSEBase
	9.6.41

	authorizationPolicy
	Represents an access control policy retrieval point
	subscription, transaction
	CSEBase
	9.6.42

	authorizationInformation
	Represents an access control information point
	role
token
subscription, transaction
	CSEBase
	9.6.43

	localMulticastGroup
	Stores local multicast group information of member hosting CSE.
	 transaction
	CSEBase
	9.6.44

	AEContactList
	Contains information about a CSE that has resources that referencing an AE-ID
	AEContactListPerCSE, subscription, transaction
	CSEBase
	9.6.45

	AEContactListPerCSE
	Contains information about a CSE that has resources that referencing an AE resource identifier for tracking purposes
	None specified
	AEContactList
	9.6.46

	transactionMgmt
	
	subscription
	CSEBase, AE, remoteCSE
	9.6.47

	transaction
	
	action
	All non-virtual resource types with the exception of the following:

request, delivery, pollingChannel, transactionMgmt, transaction, timeSyncBeacon
	9.6.48

	triggerRequest
	Used by an AE to initiate, replace or recall a device trigger request 
	subscription
	AE
	9.6.49

	ontologyRepository
	Represents the collection of the managed ontologies and the semantic validation service
	ontology, semanticValidation, subscription, ontologyMapping, ontologyMappingAlgorithmRepository
	CSEBase
	9.6.50

	ontology
	Store the representation of an ontology
	subscription
	ontologyRepository
	9.6.51

	semanticValidation
	A virtual resource as the interface to perform semantic validation on the received <semanticDescriptor> resource against the referenced ontology.
	None specified
	ontologyRepository
	9.6.52

	ontologyMapping
	Represents a mapping operation between two different ontologies.
	subscription
	ontologyRepository
	9.6.70

	ontologyMappingAlgorithmRepository
	Represents the collection of ontology mapping algorithms.
	subscription, ontologyMappingAlogrithm
	ontologyRepository
	9.6.71

	ontologyMappingAlogrithm
	Represents an ontology mapping algorithm
	None specified
	ontologyMappingAlgorithmRepository
	9.6.72

	semanticMashupJobProfile
	Represents the profile and description of a semantic mashup service
	semanticMashupInstance, semanticDescriptor, subscription
	CSEBase, remoteCSE
	9.6.53

	semanitcMashupInstance
	Represents a semantic mashup instance
	semanticMashupResult, semanticDescriptor, mashup, subscription
	semanticMashupJobProfile, AE, remoteCSE, CSEBase
	9.6.54

	mashup
	A virtual resource use to trigger the calculation and generation of new mashup result
	Not specified
	semanticMashupInstance
	9.6.55

	semanticMashupResult
	Represent semantic mashup results
	semanticDescriptor, subscription
	semanticMashupInstance
	9.6.56

	multimediaSession
	Stores a representation of a multimedia session information requested by a registering AE
	subscription 
	AE
	9.6.57

	crossResourceSubscription
	represents the cross-resource subscription information related to multiple subscribed-to resources. Such a resource shall include a list of subscribed-to resources as its attribute, or shall be created as a child resource of a <group> resource where member resources shall be the subscribed-to resources. 
	schedule, notificationTargetSelfReference, notificationTargetMgmtPolicyRef, transaction 
	CSEBase, remoteCSE, AE
	9.6.58

	backgroundDataTransfer
	Stores information for a background data transfer request
	None specified
	AE, remoteCSE, CSEBase
	9.6.60

	action
	Specifies the action(s) that is performed whenever an event is triggered at the <dependency> resource
	dependency
	CSEBase, remoteCSE, node, AE, container, flexContainer, group, timeSeries, transaction, state
	9.6.61

	dependency
	Specifies the condition(s) of a monitored event which triggers the operation(s) specified by the <action> resource
	subscription, transaction 
	action
	9.6.62

	e2eQosSession
	Specifies the end-to-end (E2E) QoS session requirements for the exchange of oneM2M request and response primitives between oneM2M entities.
	subscription, transaction 
	CSEBase, remoteCSE, AE
	9.6.63

	nwMonitoringReq
	Specifiesed the request that an AE retrieves the Underlying Network information. The resource provides the characteristics of the Underlying Network status in a particular geographic area such as congestion status and number of devices.
	subscription 
	CSEBase, remoteCSE, AE
	9.6.64

	semanticRuleRepository
	Specifies one or multiple <reasoningRules> child resources to represent different sets of reasoning rules in the oneM2M system
	reasoningRules, reasoningJobInstance
	CSEBase
	9.6.65

	reasoningRules
	Store a set of related reasoning rules
	subscription
	semanticRuleRepository
	9.6.66

	reasoningJobInstance
	Represents a specific reasoning job instance for enabling one-time or continuous reasoning operations.
	subscription
	semanticRuleRepository
	9.6.67

	serviceSubscribedUserProfile
	Stores user profile information for a given M2M Service User
	subscription, transaction
	m2mServiceSubscriptionProfile
	9.6.68

	timeSyncBeacon
	Specifies criteria that a Hosting CSE uses to generate time synchronization beacon notifications 
	subscription,

transaction
	CSEBase, remoteCSE, AE
	9.6.69

	primitiveProfile
	Stores message parameters and/or resource attributes that can be applied (i.e. added, removed or modified) to a request or a response primitive. 
	subscription, transaction
	CSEBase, remoteCSE, AE 
	9.6.73

	state
	Stores status information and actions for a given state of a process that is managed by a Hosting CSE. 
	subscription, transaction, action
	processManagement
	9.6.74

	processManagement
	Specifies a process that a Hosting CSE manages on behalf of an AE that consists of sequence of states defined by <state> resources. 
	state, subscription, transaction
	CSEBase, remoteCSE, AE
	9.6.75

	softwareCampaign
	Provides policies and rules for the dynamic management of a software by a Hosting CSE. 
	subscription, transaction, schedule
	CSEBase, remoteCSE, AE
	9.6.76

	NOTE:
See clause 9.6.12 for a summary of specializations of <mgmtObj>.


-----------------------End of change 3-------------------------------------------

[image: image1.png]


