Doc# SDS-2020-0329-TR-0057_Location_Policy.doc

	Input Contribution

	Meeting ID*
	SDS#48

	Title:*
	TR-0057 – Location Policy

	Source:*
	Laurent Velez, ETSI, laurent.velez@etsi.org

	Date:*
	2020-11-27

	Input related to*
	TR-0057 V0.3.0

	Intended purpose of

document:*
	 FORMCHECKBOX 
 Decision

 FORMCHECKBOX 
 Discussion

 Information

 Other <specify>

	Impacted other TS/TR(s)
	

	Decision requested or recommendation:*
	To incorporate into baseline the new proposed text if agreed.

	Template Version: January 2017 (Do not modify)


oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

Introduction
This contribution proposes a overall and basic description of the location policy function.
-----------------------Start of new text 1-------------------------------------------

6.x
Location policy
6.x.1 Introduction

Geographical location information of M2M entities is one of most significant exposed service capabilities in oneM2M system for services. It allows applications AE to obtain geographical location information of Nodes (e.g. ASN, MN) for Location-based services. Such information could be more than just longitude and the latitude.
In the CSE, the LOC Common Service Function obtains and manages the geographical location information of oneM2M entities. 
On a request of AEs, a dedicated resource  <locationPolicy> is created into the CSE.  This resource does not contain the location information itself of the target nodes. Based on its specific attributes, the  <locationPolicy>  contains the configuration on how to obtain the location information of a target M2M node  (Update period, target ID, location server, target area, etc…)
When a policy is set, a data container(<container> resource) is created and is linked to the <locationPolicy> by its attribute locationContainerID. Then the CSE stores the  location information in the target <container> resource as <contentInstance> resource. The data are updated depending of the policy. Any AE can then get access to the geographical location information.
[image: image1.png]<locationPolicy>

locationSource

locationUpdatePeriod

locationContainer|D.

II [ |$ I|

locationContainerName





Figure 6.x.1-1: <locationPolicy> resource
6.x.2 Obtaining Location Information

The CSE has 3 methods for obtaining location information of an M2M Node . This information is stored in the locationSource attribute of the <locationPolicy>  resource.
· Network-based method: where the CSE on behalf of the AE obtains the target M2M Node's location information from an Underlying Network.

· Device-based method: where the ASN is equipped with any location capable modules or technologies (e.g. GPS) and is able to position itself.

· Sharing-based method: where the ADN has no GPS nor an Underlying Network connectivity. Its location information can be retrieved from either the associated ASN or a MN.

Network-based method: 


[image: image2.emf]AE

CSE

CSE

AE

Location

Requestor

Target M2M Node

(10.213.32.101)

IN

Create <locationPolicy>

Location

Server

<locationPolicy>

locationSource = ‘Network’

locationTargetID = ’10.213.32.101’

locationServer = ‘locserver.lg.org’

Location Response 

Performing Location 

(e.g., Cell-ID, OTDOA)

Location Request 

Ex : OMA API for Terminal Location)

<cont_loc_10.213.32.101>

Content = (x,y)

 Figure 6.x.2-1: Network-based method

Device-based method
[image: image3.png]Location Requestor

Create <|ocatio

Location Acquisition
(e.g., through System Call)

* non oneM2M interface

nPolicy>

<locationPolicy>

I—)( locationSource = ‘Device-Based’ )

<cont_loc>





Figure 6.x.2-2: Device-based method
Sharing-based method


[image: image4.emf]CSE

AE

MN

AE

Location

Requestor – ADN_0 (Location of 

itself)

AE

ADN_1

Location = (x

1

,y

1

)

(Stored in the MN-CSE)

AE

ADN_2

AE

ADN_3

Location = (x

3

,y

3

)

(Stored in the MN-CSE)

Create <locationPolicy>

<locationPolicy>

locationSource = ‘Sharing’

Location information of ADN_1 shall be stored 

as location information of the requestor

Topology Information

(Obtained though DM Technology)

MN

ADN_0

ADN_1 ADN_2

ADN_3

The Closest to 

ADN_0

Figure 6.x.2-3: Sharing-based method

How to link with a <container> resource


URI of the created <container>


Name of the container








For Network-based Location Information


ID of the Target M2M Node


ID of the Location Server located in Underlying Network








Basic Information


Source (Network/Device/Sharing)


Period to obtain location information








© 2019 oneM2M Partners






















Page 1 (of 2)

_1668005628.ppt




AE



CSE



CSE

AE

Location

Requestor

Target M2M Node

(10.213.32.101)

IN



Create <locationPolicy>

Location

Server



<locationPolicy>

locationSource = ‘Network’

locationTargetID = ’10.213.32.101’

locationServer = ‘locserver.lg.org’











Location Response 



Performing Location 

(e.g., Cell-ID, OTDOA)



Location Request 

Ex : OMA API for Terminal Location)



<cont_loc_10.213.32.101>

Content = (x,y)










_1624156463.ppt




CSE

AE

MN



AE

Location

Requestor – ADN_0 (Location of itself)



AE

ADN_1

Location = (x1,y1)

(Stored in the MN-CSE)



AE

ADN_2



AE

ADN_3

Location = (x3,y3)

(Stored in the MN-CSE)









Create <locationPolicy>

<locationPolicy>

locationSource = ‘Sharing’







Location information of ADN_1 shall be stored 

as location information of the requestor

Topology Information

(Obtained though DM Technology)











MN

ADN_0

ADN_1

ADN_2

ADN_3









The Closest to 

ADN_0








