| CHANGE REQUEST |  |
| --- | --- |
| Meeting ID:* | SDS#49 |
| Source:* | Cyrille Bareau, Orange, cyrille.bareau@orange.com Bob Flynn, Exacta, bob.flynn@exactagss.com Andreas Kraft, Deutsche Telekom, Andreas.Kraft@t-systems.com Marianne Mohali, Orange, marianne.mohali@orange.com |
| Date:* | 2021-01-27 |
| Reason for Change/s:* | See the introduction. |
| CR against: Release* | Release 4 |
| CR against: WI* | Active WI MNT maintenance / < Work Item number(optional)> Is this a mirror CR? Yes No mirror CR number: (Note to Rapporteur - use latest agreed revision) STE Small Technical Enhancements / < Work Item number (optional)> Only ONE of the above shall be ticked |
| CR against: TS/TR* | TS-0004 4.3.0 |
| Clauses * | Clauses 7.4.37.2.3 |
| Type of change: * | Editorial change Bug Fix or Correction Change to existing feature or functionality New feature or functionality Only ONE of the above shall be ticked |
| Other TS/TR(s) impacted | N/A |
| Post Freeze checking:* | This CR contains only essential changes and corrections? YES NO This CR may break backwards compatibility with the last approved version of the TS? YES NO |
| Template Version: January 2020 (do not modify) |  |

oneM2M Notice

The document to which this cover statement is attached is submitted to oneM2M.  Participation in, or attendance at, any activity of oneM2M, constitutes acceptance of and agreement to be bound by terms of the Working Procedures and the Partnership Agreement, including the Intellectual Property Rights (IPR) Principles Governing oneM2M Work found in Annex 1 of the Partnership Agreement.

GUIDELINES for Change Requests:

Provide an informative introduction containing the problem(s) being solved, and a summary list of proposals.

Each CR should contain changes related to only one particular issue/problem.

If this is  a correction, and the change applies to previous releases, a separate “mirror CR” should be posted at the same time as this CR

Mirror CR: applies only when the text, including clause numbering are exactly the same.

Companion CR: applies when the change means the same but the baselines differ in some way (e.g. clause number).

Follow the principle of completeness, where all changes related to the issue or problem within a deliverable are simultaneously proposed to be made e.g. a change impacting 5 tables should not only include a proposal to change only 3 tables. Include any changes to references, definitions, and abbreviations in the same deliverable.

Follow the drafting rules.

All pictures must be editable.

Check spelling and grammar.

Use change bars for modifications.

The change should include the current and surrounding clauses to clearly show where a change is located and to provide technical context of the proposed change. Additions of complete clauses need not show surrounding clauses as long as the proposed clause number clearly shows where the proposed new clause is located.

Multiple changes in a single CR shall be clearly separated by horizontal lines with embedded text such as, start of change 1, end of change 1, start of new clause, end of new clause.

When subsequent changes are made to the content of a CR, then the accepted version should not show changes over changes. The accepted version of the CR should only show changes relative to the baseline approved text.

## Introduction

The creation of a <flexContainerInstance> resource is triggered when the parent <flexContainer> resource is updated. However, this currently happens when 2 conditions apply:

- The parent flexContainer owns at least one custom attribute.

- The UPDATE request contains at least one of these custom attributes.

We want to add the possibility to create flexContainerInstances:

- For flexContainers that have no custom attributes. This corresponds to a precise use case involving SDT Actions in TS-0023, that can be mapped to flexContainers with no custom attributes; in this case, the creation of a <flexContainerInstance> will timestamp the triggering of this action.

- On demand, even if none of the current custom attributes are modified (can be used as an optimisation, for instance for historizing a temperature that is stable).

This document is a companion to CR SDS-2021-0002R01-TS-0001_flexContainerInstance_on_empty_UPDATE.

### ********************** Start of change 1   **********************

##### 7.4.37.2.3	Update

Originator:

No change from the generic procedures in clause 7.2.2.1.

Receiver:

Primitive specific operation on Recv-1.0 with the following additional operations.

- The hosting CSE shall validate the received resource representation against the schema value present in the resource containerDefinition attribute. If the received resource is not valid then the Hosting CSE shall return a response primitive with a Response Status Code indicating "BAD_REQUEST" error.

Primitive specific operation on Recv-6.5 "Create/Update/Retrieve/Delete/Notify operation is performed" with the following additional operations:

- If at least one of the maxNrOfInstances, maxByteSize or maxInstanceAge is present in the request with zero value, all children <flexContainerInstances> resources shall be deleted and the currentNrOfInstances, currentByteSize, maxNrOfInstances, maxByteSize and maxInstanceAge attributes shall be deleted.

In the following steps, any occurrence of maxNrOfInstances, maxByteSize or maxInstanceAge will refer to their non-zero value in the updated <flexContainer> resource.

If a custom attribute of the targeted <flexContainer> resource is modified in the request, and maxByteSize attribute is present, the Hosting CSE shall check whether the size in bytes of the custom attributes of the targeted <flexContainer> resource is greater than the maxByteSize of attribute.

- If true, the Hosting CSE shall return the response primitive with a Response Status Code indicating "NOT_ACCEPTABLE" error and skip step 3 below.

- If false, the Hosting CSE shall update the contentSize attribute to the sum of the size in bytes of all of the custom attributes.

3)	If the request’s Content parameter contains either a partial resource with no attribute or at least one custom attribute of the targeted <flexContainer> resource, and at least one of maxNrOfInstances, maxByteSize or maxInstanceAge attributes is present, then after updating the attributes of the targeted <flexContainer> resource:

a)	The Hosting CSE shall create a <flexContainerInstance> resource child of the targeted <flexContainer> resource. The attribute contentSize of the created <flexContainerInstance> resource will have the same value as in the targeted <flexContainer> resource. The custom attributes of the targeted <flexContainer> resource, if any, shall be copied into the created <flexContainerInstance> resource. The originator attribute of the created <flexContainerInstance> resource shall be set to the identifier of the originator of this UPDATE request.

b)	The Hosting CSE shall update the currentNrOfInstances of the targeted <flexContainer > resource with the number of <flexContainerInstance> resources in the targeted <flexContainer > resource. The Hosting CSE shall update the currentByteSize of the targeted <flexContainer > resource with the sum of the contentSize attributes of the <flexContainerInstance> resources in the targeted <flexContainer > resource.

c)	If maxNrOfInstances of the targeted <flexContainer > resource is present then if the currentNrOfInstances when modified to reflect the addition of the new <flexContainerInstance> exceeds maxNrOfInstances, the Hosting CSE shall remove the oldest <flexContainerInstance> resource from the targeted <flexContainer> resource. The attributes currentNrOfInstances and currentByteSize will be updated accordingly.

d)	If maxByteSize of the targeted <flexContainer > resource is present then if the currentByteSize when modified to reflect the addition of the new <flexContainerInstance> exceeds maxByteSize the Hosting CSE shall remove the oldest <flexContainerInstance> resource(s) from the targeted <flexContainer> resource until maxByteSize conditions are met. The attributes currentNrOfInstances and currentByteSize will be updated accordingly.

e)	When removing the oldest <flexContainerInstance> resources, the Hosting CSE shall not generate notifications even if there exists a <subscription> to the targeted <flexContainer> resource and this <subscription> is configured to generate a notification on "Delete_of_Direct_Child_Resource".

f)	If the maxInstanceAge attribute is present in the targeted <flexContainer> resource, then the Hosting CSE shall set the expirationTime attribute in created <flexContainerInstance> child resource such that the time difference between expirationTime and the creationTime of the <flexContainerInstance>. The <flexContainerInstance> child resource shall not exceed the maxInstanceAge of the targeted <flexContainer> resource.

No other changes from the generic procedures in clause 7.2.2.2.

### ********************** End of change 1   **********************